SMART-BIN: AN ESP32-BASED IoT WASTE BIN MONITORING SYSTEM WITH FILL, WEIGHT, AIR-QUALITY AND TAMPER DETECTION

FOR PRIORITY-BASED COLLECTION

WASTE MANAGEMENT AND CLEANLINESS

PROBLEM: Overflowing Garbage Bins and Inefficient Waste Collection Routes

Technological University of the Philippines - Manila

Bachelor of Engineering Technology major in Computer Engineering Technology

Embedded Systems: Final Project

BET-CPET 4A-M, Group 2

Submitted By:

Brian Jay Celeste

Genesis Walter D. Reyes II

Jade C. Gunayan

King Perth G. Pepito

Rodel S. Banogbanog

Submitted To:

Prof. Jonel R. Macalisang

November, 2026

TABLE OF CONTENTS

TITLE PAGE	i

TABLE OF CONTENTS	ii

LIST OF FIGURES	iii

LIST OF TABLES	vii

I. INTRODUCTION	1

1.1 Introduction and Problem	1

1.2 Sustainable Development Goal (SDG) Addressed	2

1.3 Review of Related Literature	3

1.4 Objectives	54

1.5 Conceptual Framework	55

1.6 Target Beneficiaries	59

II. PROTOTYPE DESIGN AND DEVELOPMENT	60

2.1 Development Methodology	60

2.2 Prototype Design	64

2.3 Block Diagram	65

2.4 System Flowchart	67

2.5 Schematic Design	73

2.6 Component Materials	76

III. TESTING, RESULTS, AND CONCLUSION	87

3.1 Test Cases	87

3.2 Results and Discussion	92

3.3 Conclusion	104

3.4 Recommendations	105

REFERENCES	107

CURRICULUM VITAE	117

Rodel S. Banogbanog	116

Brian Jay Celeste	117

Jade C. Gunayan	119

King Perth G. Pepito	120

Genesis Walter D. Reyes II	121

APPENDICES	117

Appendix A: Gantt Chart	118

Appendix B: Bill of Materials of the Smart-Bin Prototype	119

Appendix C: User Interface Screens	120

Appendix D: Source Code	150

LIST OF FIGURES

Figure 1.	Sustainable Development Goal 11: Sustainable Cities and Communities	2

Figure 2.	Patents Reviewed by Priority Year and Region	6

Figure 3.	Remote Status Units Linked to a Central Unit	7

Figure 4.	Compacting Ram and Chambers of the Solar Compactor	8

Figure 5.	Path of a Full Message From the Compactor Monitor to the Service Provider	9

Figure 6.	Trash Can With a Motor-Driven Lid	10

Figure 7.	Container With a Bag Between Capacitive Plates	11

Figure 8.	Waste Collection System of Containers, Server and Vehicles	12

Figure 9.	Refuse Container With Scale, Display and Lights	13

Figure 10.	Sensor Device Mounted Under the Lid Behind a Thermal Barrier	14

Figure 11.	Gauge Mounted Under the Lid of a Container	15

Figure 12.	Smart Garbage Bin With a Lid-Mounted Sensor Device	16

Figure 13.	Pay-by-Weight Food Waste Processor With a Load-Cell Hopper	17

Figure 14.	Infrared Beam Across a Container	18

Figure 15.	Circuit of the Induction Flip Lid With a Settable Sensing Distance	19

Figure 16.	Overall Smart Waste Management System	20

Figure 17.	Garbage Cans and the Monitoring Center	21

Figure 18.	Waste Measurement System With Weighing Devices and Reports	22

Figure 19.	Block Diagram of the Capacity-Detecting Bin	23

Figure 20.	Electrical Block Diagram of the Bin Intelligent Controller	24

Figure 21.	Automatic Alerting System for a Trash Can	25

Figure 22.	Network of Waste Bins, Cloud Server and Vehicles	26

Figure 23.	Block Diagram of the Smart Waste Management System	29

Figure 24.	Circuit Setup of the LoRa Bin Unit	30

Figure 25.	Proposed System Architecture for Hospital Waste Monitoring	32

Figure 26.	Proposed Network Architecture for Waste Collection in Bukavu	34

Figure 27.	Prototype of the Smart Dustbin	35

Figure 28.	Block Diagram of the Arduino-Based Smart Dustbin	37

Figure 29.	Data Communication Structure of the Smart Waste Management System	38

Figure 30.	Prototype of the SmartTrashBin	39

Figure 31.	Data Flow of the Garbage Monitoring System	40

Figure 32.	Power Chain of the ESP32-CAM Node	44

Figure 33.	SWOT Analysis of Smart-Bin	49

Figure 34.	Features Found in the 30 Related Studies	52

Figure 35.	Input-Process-Output Model of Smart-Bin	55

Figure 36.	System Architecture of Smart-Bin	57

Figure 37.	Water-Scrum-Fall Development Life Cycle of Smart-Bin	61

Figure 38.	Front View and Side Cross-Section of One Smart-Bin Unit	64

Figure 39.	Block Diagram of the Smart-Bin System	65

Figure 40.	System Flowchart of the Smart-Bin	67

Figure 41.	Operator Flowchart of the Smart-Bin Apps	69

Figure 42.	Collector Flowchart of the Smart-Bin Apps	71

Figure 43.	Schematic Diagram, Sheet 1: Microcontroller	73

Figure 44.	Schematic Diagram, Sheet 2: Actuators and Outputs	74

Figure 45.	Schematic Diagram, Sheet 3: Sensors and Inputs	75

Figure 46.	ESP32 DevKit V1 (38-Pin, ESP32-WROOM-32D)	76

Figure 47.	HC-SR04 Ultrasonic Distance Sensor	77

Figure 48.	FC-51 IR Obstacle Avoidance Sensor	78

Figure 49.	BME680 Environmental Sensor	79

Figure 50.	BMI160 Six-Axis Inertial Measurement Unit	80

Figure 51.	HX711 Load Cell Amplifier and 5kg Load Cell	81

Figure 52.	SG90 Micro Servo Motor	82

Figure 53.	2.4-in. TFT Touchscreen (ILI9341 With XPT2046 Touch Controller)	83

Figure 54.	KY-016 RGB LED Module	84

Figure 55.	Passive Buzzer Module (MH-FMD)	85

Figure 56.	18650-Battery Shields for Bin A (4-Cell) and Bin B (2-Cell)	86

Figure 57.	The Two Built Smart-Bin Units	88

Figure 58.	Inside Bin A: Trash Chamber From Above (a) and Controller and Battery Compartment (b)	89

Figure 59.	Lid Test: Hand in Front of the IR Sensor (a) and the Lid Opened (b)	96

Figure 60.	Bin 01 With the Original PIR Sensor During the First Test	97

Figure 61.	Home Screen of Bin 01, With the IR Sensor Above It	102

Figure C1.	Start Page of the Web Dashboard	121

Figure C2.	Creating the Operator PIN	122

Figure C3.	Overview Tab with Bin Cards and Collection Priority	123

Figure C4.	Overview Tab in Grid View	124

Figure C5.	Bin Detail Opened from the Grid	125

Figure C6.	Charts Tab	126

Figure C7.	Settings Tab in Calibration Mode: Fill Range, Alert Limits and Air Sensor	127

Figure C8.	Settings Tab in Calibration Mode: Tilt Sensor, Lid, Weight and Touchscreen	128

Figure C9.	Bluetooth Setup Panel	129

Figure C10.	Cloud Backup Panel for Firebase and ThingSpeak	130

Figure C11.	Operator Access Panel	131

Figure C12.	Demo Options Panel	132

Figure C13.	Sound Test Panel	133

Figure C14.	Simulation Tab with One Sensor in Manual	134

Figure C15.	Alerts Tab	135

Figure C16.	Overview Tab in Dark Theme	136

Figure C17.	Start Page and Operator PIN in the Android App	137

Figure C18.	Overview Tab with Banners, Collection Priority and Bin Grid	138

Figure C19.	Starting and Finishing a Pickup from a Bin Card	139

Figure C20.	Needs Attention Sheet and Alert History	140

Figure C21.	Charts Tab in the Android App	141

Figure C22.	Settings Tab with Calibration Mode Off and On	142

Figure C23.	Calibration Settings: Fill Range, Alert Limits and Air Sensor	143

Figure C24.	Calibration Settings: Tilt Sensor, Lid, Weight and Touchscreen	144

Figure C25.	Simulation Tab in the Android App	145

Figure C26.	App Settings	146

Figure C27.	Connection Page	147

Figure C28.	Bluetooth Setup Page and Bin Settings Sheet	148

Figure C29.	Overview in List View, Light and Dark Themes	149

LIST OF TABLES

Table 1.	Related Patents, by Priority Year	4

Table 2.	Related Studies From 2023 to 2026	27

Table 3.	Commercial and Student Systems as Described by Their Own Pages	47

Table 4.	Technical Basis of the Main Design Choices	48

Table 5.	The Four Closest Patents Compared With Smart-Bin	51

Table 6.	The Research Gap by Capability	53

Table 7.	Reference Instruments Used in the Tests	87

Table 8.	Test Cases for Specific Objective 1: Sensor Monitoring and Detection	89

Table 9.	Test Cases for Specific Objective 2: Real-Time Monitoring and Alerts	90

Table 10.	Test Cases for Specific Objective 3: Bin Control and Collection Prioritization	91

Table 11.	Fill Level Readings Against the Ruler (TC1.1)	92

Table 12.	Weight Readings Against the Test Masses (TC1.2)	93

Table 13.	Temperature and Humidity Against the Thermo-Hygrometer (TC1.3)	93

Table 14.	Air Quality Response After the Warm-Up (TC1.4)	94

Table 15.	Tamper Detection by Tilt Angle (TC1.5)	94

Table 16.	Automatic Lid Response by Hand Distance (TC1.6)	95

Table 17.	Time to Show a New Reading (TC2.1)	98

Table 18.	ThingSpeak Logging Over 30 Minutes (TC2.2)	98

Table 19.	Alert Delivery to the Phone (TC2.3)	99

Table 20.	Offline Detection, Role Access and Two-Bin Results (TC2.4 to TC2.6)	100

Table 21.	Collection Priority Order for Five of the Ten Combinations (TC3.1)	100

Table 22.	Estimated and Actual Time to Reach the 80% Fill Limit (TC3.2)	101

Table 23.	Lock, PIN Collection, Simulation and Calibration Results (TC3.3 to TC3.6)	101

Table 24.	Summary of Test Results	103

# I. INTRODUCTION

## 1.1 Introduction and Problem

An effective waste management plays a crucial role in creating and sustaining healthy, clean and prosperous communities. With the rising amount of waste that is being generated, it has become a problem for the local government and commercial waste collectors in the Philippines. The Philippines is estimated to be producing a total of 60.76 thousand tons of garbage every day in 2025, according to the Philippine Statistics Authority (PSA, 2026). Despite the significant actions taken by the City of Metro Manila, its garbage is projected to reach 9,498 MT/D in 2020, making it the country's waste capital (World Bank, 2022). This was projected to rise to 10,368 MT per day by 2025 (World Bank, 2022). Hence, a high priority of improving waste management and collection practices in the country. Most garbage collection systems are conducted on a fixed schedule, causing bins to overflow when they are picked up or not to be filled to capacity when collected. Additionally, the smell, pollution levels, temperature, tilting of garbage bins, and unauthorized access should also be monitored as they are important indicators of environmental conditions and the efficiency of garbage management.

The researchers propose the Smart-Bin which is an Internet of Things (IoT) based bin monitoring system based on the ESP32 microcontroller with the aim of improving garbage collection performance. Sensors will be used by Smart-Bin to determine the level of fullness, weight, temperature, humidity, and air quality. It'll also be tracking bins that tilt irregularly, and any unauthorized bin openings, through a web dashboard and mobile application. Smart-Bin includes a touchscreen at the bin for employees to view and acknowledge the collection instructions, as well as to set up PIN codes to select who can access the bin, and to make collection setting changes. The project will include the development of two Smart-Bin prototypes built at the table size that will feature the ESP32 boards, local Wi-Fi, ThingSpeak, web dashboard, Android app, and screen at the bin. Smart-Bin will arrange the list of collection priorities based on fill levels and how long they will take to fill up, with the aim of optimizing garbage collection and fuel consumption rates.

## 1.2 Sustainable Development Goal (SDG) Addressed

Figure 1

Sustainable Development Goal 11: Sustainable Cities and Communities

The project Smart-Bin is connected to one of the Sustainable Development Goals (SDGs) of the United Nations (Agenda 2030), namely SDG 11: Sustainable Cities and Communities. Smart-Bin is specifically targeting the environmental impact of cities, per capita, with particular attention to air quality and municipal and other waste management (United Nations Department of Economic and Social Affairs [UN DESA], n.d., Target 11.6).

Smart-Bin can contribute to achieve the target of Sustainable Development Goal 11. It is achieved by monitoring and control of waste state with help of sensors installed in smart-bins and collecting information to make timely decision to ensure waste management collection process. As part of the project, a dashboard and application are developed and send a notification to the operator regarding of the bins state (levels, weight, temperatures, humidity, air quality around bins).

This could be helpful when it comes to optimizing the collection process as priority could be given to those bins that contained more trash and should be emptied before the trash overflows. Moreover, it can also be employed to avoid the effects on the environment, e.g., when the temperature or air quality (sign of rot) is not acceptable. This can help to streamline the collection process by prioritizing bins and reducing vehicle and fuel usage.

## 1.3 Review of Related Literature

This review covers three kinds of sources. Patents show what has already been claimed, in any year. Studies from 2023 onward show what researchers have built lately, and a handful of commercial products show what is already on sale. The patents were found and read on Google Patents which also provided the legal status listed in Table 1. Every research study was followed up with its DOI or its publisher page and each element in the reference list was verified with the research. Each patent and each study has its own entry, and figure (where present) from the source that can be reused, and each one has a comparison with Smart-Bin. The synthesis of those comparisons is made in the synthesis, the SWOT analysis and the research gap at the end of the section.

### Prior Art

Review of 20 patents. They are in effect through 2023 and their owners are in the United States, Europe, China and Korea. They are listed in priority year order in Table 1, and their coverage and differences from Smart-Bin are summarized. Google classifies these labels as assumptions and not legal conclusions, it's not until someone checks with the patent office whether their status is Active or Expired, that they should rely on it. Figure 2 puts the twenty into a time line. Most of the activity is recent, as five are priority years before 2012 and fifteen are priority years in 2012 or after.

Each of the patents is discussed individually below, in the order of the table. The numbers for each entry are the first images of the patent released as published on Google Patents, and the text below in each are what the patent claims and how it relates to Smart-Bin. The four closest patents are Kurani (2023), Sait et al. (2025), Shahabdeen (2016) and Yan and Chen (2021) and the synthesis in the end of this section makes a side-to-side comparison of the features of the two Smart-Bins.

Table 1

Related Patents, by Priority Year

| No. | Patent | Priority year | Status | What it covers | How Smart-Bin differs |
| --- | --- | --- | --- | --- | --- |
| 1 | US 5,016,197 (Neumann & Allison, 1991) | 1986 | Expired | Remote status units judge how full compactor containers are from hydraulic pressure or weight, and report over a telephone line to a central unit that calls the hauler. | A phone-line system for hydraulic compactors. It has no gas, tilt, lid, display, wireless link or phone app. |
| 2 | US 7,124,680 (Poss et al., 2006) | 2003 | Expired | A public trash compactor powered by a solar panel and a battery, with a removable bin, so it is emptied less often than an ordinary bin. | It compacts rather than senses. It has no weight, air-quality or tilt sensing, touchscreen, PIN mode or priority list. |
| 3 | US 7,145,450 (Brown, 2006) | 2004 | Expired | A processor reads the pressure on a compactor ram, flags the unit as full after a preset time and emails a work order to a service provider. | Fullness is inferred from pressure only. It has no fill level, weight, gas, lid, local display or app. |
| 4 | US 7,781,995 (Yang & Sandor, 2010) | 2005 | Expired | A trash can whose electric drive opens the lid when a sensor detects an object near the lower part of the can. | The trigger is a foot-level sensor. There is no fill, weight, air-quality, tilt, wireless or collection feature. |
| 5 | EP 2 516 974 B1 (Di Giugno et al., 2016) | 2009 | Not in force | A container that holds a bag has capacitive plates that give several fill levels. Its status goes wirelessly to stations that plan the emptying routes. | The sensing is capacitive around a bag. There is no weight, gas, tilt, powered lid, PIN mode or phone app. |
| 6 | WO 2014/079586 A1 (Kekalainen & Engstrom, 2014) | 2012 | Ceased | Containers with sensors report their status over a wireless network to a server, which plans the collection vehicles and can run bidding for the jobs. | It is a fleet-level planning platform with sensors only. There is no weight, tamper detection, powered lid, PIN mode or on-bin display. |
| 7 | US 9,347,821 B1 (Mullaney, 2016) | 2012 | Expired | The refuse container has a scale on it that indicates the weight of refuse in the container and alerts to a maximum weight. | It is capable to sense weight. Fill level, air quality, tamper alarm, IR sensor lid, PIN mode and dashboard are not present. |
| 8 | US 9,927,278 B2 (Kekalainen & Engstrom, 2018) | 2013 | Expired | A sensor device fixed under the container lid, behind a thermal barrier, measures the amount of waste and the conditions inside and sends them to a remote location. | Its novelty is the heat-insulated lid mounting of a cellular sensor module. There is no weight, tamper alarm, powered lid, PIN mode or priority list. |
| 9 | US 9,930,429 B2 (Kekäläinen, 2018) | 2014 | Active | A gauge compares the fill rate with limits set by a server and contacts the server only when a limit is crossed, which saves battery. | It is a battery-saving reporting scheme. No motorized lid, tamper alarm, PIN mode, touchscreen or phone app is described. |
| 10 | US 2016/0176630 A1 (Shahabdeen, 2016) | 2014 | Abandoned | A bin carries a lid-mounted range finder or a weight sensor at the base, with optional hydrogen sulfide, ammonia and methane sensing, and a controller that sends the data wirelessly. | It has no tamper detection, powered lid, lock, on-bin display, PIN mode or priority list. Temperature and humidity are not mentioned. |
| 11 | KR 10-1581944 B1 (Shin, 2016) | 2015 | Active | The hopper of a pay-by-weight food waste processor sits on a load cell and rotates to drop the waste into a crushing tank. | It is a weighing and payment machine. There is no fill sensing, gas sensing, tilt sensing, wireless reporting or priority list. |
| 12 | US 11,054,545 B2 (Devitt & Walsh, 2021) | 2016 | Active | Two low-power ways to detect a full container: an infrared beam across the container, or the resting tilt angle of the lid. | The output is only full or not full. There is no weight, gas, temperature, powered lid, PIN mode or tamper alarm. |
| 13 | EP 3 620 405 A1 (Wang, Chen, et al., 2020) | 2017 | Withdrawn | An induction flip-lid can whose infrared sensing distance the user can set, built into a single microcomputer chip. | It only opens the lid. There is no fill, weight, gas, tilt, display or wireless link. |
| 14 | US 11,702,280 B2 (Kurani, 2023) | 2018 | Active | 12 sensors, including those measuring fill level, weight, air quality, temperature, humidity and pressure, along with an accelerometer, are installed on the bin and communicate with a cloud server, which generates statistics and works out collection routes, also notifying mobile apps. | Focuses on litter, biohazard and pathogen sensing, with a sterilizer. There was no IR-sensor servo lid, no remote lock or no PIN collect mode or latched tamper alarm. |
| 15 | CN 109324552 B (Yan & Chen, 2021) | 2018 | Active | Each can has weight, infrared height, odor and smoke sensors and a positioning unit. A monitoring center decides which cans to visit and builds the truck route. | It plans city-wide truck routes with GPS. No powered lid, tamper detection, PIN mode, on-bin display or phone alert was found. |
| 16 | US 11,788,877 B2 (Balachandran & Talavera, 2023) | 2018 | Active | Load cells under a platform weigh the waste of two or more bins, and cameras with image analysis identify the contents for waste audit reports. | It is a waste audit and reporting service, not a single bin with fill, tilt, lid and lock control, a PIN collect mode and a collection priority list. |
| 17 | CN 210504152 U (Yang et al., 2020) | 2019 | Expired | A bin with an infrared induction module, an ultrasonic ranging module and a programming module lets users see how much capacity is left. | It is a stand-alone local device. There is no weight, gas, tilt, lid actuator, wireless link or phone app. |
| 18 | CN 111559590 A (Wang, Zhang, et al., 2020) | 2020 | Withdrawn | A controller for top or side installation measures the distance to the garbage and uploads it to a supervisory platform, which has an early-warning unit. | It comprises of one laser ToF sensor. No weight, gas, temperature, tilt, lid control or touchscreen collect mode or priority list. |
| 19 | US 12,125,361 B2 (Keller et al., 2024) | 2021 | Active | Two calibration setpoints enable a processor to convert distance readouts to a fill percentage and signal a user device when it exceeds a threshold, which varies depending on the can owner. | It uses a single laser time-of-flight sensor. There is no weight, gas, temperature, tilt, lid control, touchscreen collect mode or priority list. |
| 20 | US 12,450,575 B1 (Sait et al., 2025) | 2023 | Active | Each bin has methane, fill level and temperature/humidity sensors that provide data to neural networks which calculate the time required to reach the methane and fill limits; a scheduler creates routes considering the vehicle emissions. | It revolves around the sensing of methane by air quality. There is no load cell, tilt alarm, IR sensor lid, remote lock or PIN collect mode. |

Note. Status is as reported by Google Patents in October 2026. The differences are those found in the patent text read for this review.

Figure 2

Patents Reviewed by Priority Year and Region

Note. Numbers match Table 1. The four patents in green are the closest to Smart-Bin.

Automated Trash Management System (Neumann & Allison, 1991)

Figure 3

Remote Status Units Linked to a Central Unit

Note. From Automated trash management system (U.S. Patent No. 5,016,197), by R. H. Neumann and C. B. Allison, 1991, U.S. Patent and Trademark Office.

Figure 3 shows the layout of the system that Neumann and Allison (1991) patented for MGM Services. Each remote status unit serves three containers, each with its own compactor, and all units are linked by a communications line, typically a telephone line, to a central unit that keeps a database. The hauler sends a truck only to the containers that report full. For compactors, fullness is inferred from the hydraulic pressure during the packing stroke, from the work done per stroke or from the current drawn by the pump motor, and for any container a measured weight can be compared with a preset full weight.

Calling for a pickup only when a container is full is the idea behind the priority tags of Smart-Bin, and the load cell is a signal that both systems can use. The differences come from the era. The data travel by modem, a person calls the hauler, and no alert or route is generated. The patent describes no gas, temperature or tilt sensing, no lid, no display on the container and no phone app. Smart-Bin also measures the fill directly with an ultrasonic sensor and does not infer it from the work of a compactor.

Solar Powered Compaction Apparatus (Poss et al., 2006)

Figure 4

Compacting Ram and Chambers of the Solar Compactor

Note. From Solar powered compaction apparatus (U.S. Patent No. 7,124,680), by J. Poss, J. Satwicz, B. Richmond, and M. Taylor, 2006, U.S. Patent and Trademark Office.

Poss et al. (2006) of BigBelly Solar patented a public compactor that a photovoltaic array powers, so that it can stand where there is no mains power but many people pass. Figure 4 shows the compacting ram with its chain drive and guide track and the compaction chambers. One of the roof panels charges a battery, which causes a compaction cycle to be skipped if the battery has low charge. The trash is compacted, which means that it will not need to be emptied as frequently as a regular trash bin. Fullness is sensed by a photo-eye, by the ram motor current (the stall time) and by pressure sensors, and a wireless transmitter reports to sanitation staff.

The patent attacks the same public-space problem as Smart-Bin, a bin that overflows and is costly to visit, but it answers with compaction and solar power instead of sensing. Scales are named only as an alternative fullness sensor, and the only display is a set of status lamps. No weight or air sensing, tilt alarm, touchscreen, PIN mode or phone app is described. One remark does bear on the project: the patent says that real-time fill data could feed routing and scheduling software, which is the use that Smart-Bin makes of its fill ranking.

Compactor Service and Monitoring System (Brown, 2006)

Figure 5

Path of a Full Message From the Compactor Monitor to the Service Provider

Note. From Compactor service and monitoring system (U.S. Patent No. 7,145,450), by W. J. Brown, 2006, U.S. Patent and Trademark Office.

Brown (2006), an individual inventor, patented a monitor that is added to an existing compactor. A pressure sensor on the hydraulic system of the ram measures the pressure during each cycle, and when the pressure stays at or above a preset value for a preset time the unit is flagged as full. Figure 5, which is Figure 2 of the patent, shows the path of a message. The monitor at the compactor reaches a cell site, an internet gateway and a server that runs the message monitoring software, which creates a work order, emails it to the service provider (a hauler or maintenance company) and supplies a tracking web link.

Like Smart-Bin, the system detects a full container by itself and notifies the people who must empty it. Fullness is inferred from pressure, though, and the patent has no level or weight sensor, since the tonnage is typed in by hand after a pickup. Alerts go out as email work orders, with fax as an option, and not as phone notifications, and the only local indication is a set of LEDs. Overdue work orders are flagged for staff, which is a rough counterpart of the priority tags of Smart-Bin, except that Smart-Bin bases its tags on the measured fill rate.

Trash Can With Power Operated Lid (Yang & Sandor, 2010)

Figure 6

Trash Can With a Motor-Driven Lid

Note. From Trash can with power operated lid (U.S. Patent No. 7,781,995), by F. Yang and J. Sandor, 2010, U.S. Patent and Trademark Office.

Yang and Sandor (2010) patented the power-operated lid of Simplehuman. Figure 6 shows the can with its lid open. There's a recess in front that houses the sensor, two link rods on the back wall connect a motor-operated lifting bar to the lid, and the circuit board, batteries and motor gear all reside in the base. If the sensor senses an object in the proximity of the bottom of the can, the electric drive opens the lid and the lid remains open for a predetermined period (2 seconds, for example). Claim 1 relates to the motor for the door. The controller on each command verifies the supply voltage, regulates the motor power to maintain the speed of the door within a certain range, stops the motor by reversing it for a short time, and if the corresponding motor is running for too long, the motor stops and a fault is output.

Smart-Bin also opens its lid by motor when a sensor notices a user, so that nobody has to touch the lid, but it uses an IR sensor and a servo and not a foot-level light beam. Everything else in the patent concerns lid mechanics. It has no fill, weight, air-quality, tilt, lock, display or wireless feature, and nothing on the collection side. The patent shows that a hands-free lid is old, so the contribution of Smart-Bin cannot be the lid itself. It lies in the sensing and reporting behind the lid.

Measuring Device and Monitoring System for Containers (Di Giugno et al., 2016)

Figure 7

Container With a Bag Between Capacitive Plates

Note. From Measuring device, container means and system for monitoring and managing container means (European Patent No. 2516974), by M. Di Giugno, F. Giovanelli, S. Tiraboschi, and P. Villa, 2016, European Patent Office.

Di Giugno et al. (2016), from Cefriel and the Milanese environmental services company AMSA, describe a container whose casing holds a bag. Figure 7, which is Figure 2 of the patent, shows the container: the casing, the bag that the lid element holds in place and the plates at the sides of the cavity. Claim 1 places capacitive plates on opposite sides of the cavity, spaced along its length, so that the change in electrical capacity caused by the waste gives several progressive fill levels, with the bag lying between the plates. Status goes by GPRS or short-range wireless to an internet server, and stations plan the paths of the vehicles.

What matches Smart-Bin is the chain and not the sensor: measure the fill inside the bin, send it to a back end with a user interface, and use it to plan the collection of full bins. The sensing is different. Capacitive plates around a bag give levels in steps, while Smart-Bin measures distance by ultrasound and converts it to a percentage with two calibrated distances. It has no gas, temperature, tilt or weight sensing, no powered lid or lock, no PIN collect mode and no phone app.

Smart Waste Collection System and Method (Kekalainen & Engstrom, 2014)

Figure 8

Waste Collection System of Containers, Server and Vehicles

Note. From Smart waste collection system and method (International Patent Application Publication No. WO 2014/079586 A1), by F. Kekalainen and J. Engstrom, 2014, World Intellectual Property Organization.

Figure 8 shows the system in the application that Kekalainen and Engstrom (2014) filed for Enevo. The status is transmitted via a wireless network (GSM/3G) to a central server, which maintains the information in a database and arranges collection for vehicles on the way to collection or recycling facilities. The sensors are located in a rotating lid to measure fill level, gasses (methane, CO2 and CO type) and temperature. Weight is not measured. The server forecasts container fill times and routes them using a Monte Carlo simulation based on GPS data of the vehicles.The application also describes a marketplace in which haulers bid for collection jobs.

This is an early statement of the idea that fill data should decide which bins are collected first, and Smart-Bin carries it out on a much smaller scale with its priority tags. The patent works at fleet level, with bidding, simulated models and vehicle tracking, and the bin is only a sensor. It has no weight sensing, tilt or tamper alarm, powered lid, PIN mode or display on the bin, and humidity and odor are not measured.

Refuse Container With Weight Indicator and Danger Alerting (Mullaney, 2016)

Figure 9

Refuse Container with Scale, Display and Lights

Note. From Refuse container with weight indicator and danger alerting (U.S. Patent No. 9,347,821), by G. Mullaney, 2016, U.S. Patent and Trademark Office.

Mullaney (2016), an individual inventor, patented a refuse container that shows how heavy its contents are. Figure 9, which is Figure 1A of the patent, shows a container with a floor scale, a controller, a display and four lights. The lights track the percentage of the maximum weight, and all of them light, with one flashing, once the limit is exceeded. The maximum weight can be set by location (a city or ZIP code lookup), by the user or by default, and an optional beep is mentioned. Claim 1 adds a memory that stores a history of readings, a controller that sends that history to a service provider, and a lock that locks the lid once the refuse reaches a predetermined weight. An inversion sensor releases the lock for emptying.

The container weighs its contents, shows the weight and warnings on the bin, locks the lid under a condition and can report to a remote party, and Smart-Bin does all four. The lock is the point of difference. It closes at a weight limit to stop overloading, where the lock of Smart-Bin is controlled remotely or by a PIN. The patent does not measure fill level or air quality, has no tamper alarm, IR sensor lid or collection priority list, and uses the mobile app only for configuration.

Sensor Device for Smart Waste Collection Systems (Kekalainen & Engstrom, 2018)

Figure 10

Sensor Device Mounted Under the Lid Behind a Thermal Barrier

Note. From Sensor device for smart waste collection systems and method (U.S. Patent No. 9,927,278), by F. Kekalainen and J. Engstrom, 2018, U.S. Patent and Trademark Office.

Kekalainen and Engstrom (2018) patented the sensor module itself and not the system around it. Figure 10 shows a container with a pivoting lid and the sensor device under the lid, held off its surface by spacers that leave an air gap. The spacers and a heat-reflecting metal foil layer form a thermal barrier between the lid and the electronics. The module has an ultrasonic sensor for the amount of waste and sensors for temperature, humidity and a hydrocarbon or methane gas that indicates decomposition. The data is sent constantly, periodically or when a threshold is reached via a cellular modem (GSM or 3G), and the fasteners can be designed to avoid unauthorized removing.

This is the nearest match to the sensing side of Smart-Bin. Ultrasonic fill, temperature, humidity and gas are all on the bin, and the data go wirelessly to a remote location. The novelty lies in the insulated lid mounting and not in the sensors. The data support collection scheduling, but the device does not compute routes, and no weight sensor, powered lid, PIN mode or priority list is described. The anti-removal fasteners are a mechanical measure, where Smart-Bin senses the tilt and sounds an alarm.

Monitoring and Communicating the Fill Rate of a Container (Kekäläinen, 2018)

Figure 11

Gauge Mounted Under the Lid of a Container

Note. From Method and system for monitoring and communicating fill rate of container (U.S. Patent No. 9,930,429), by F. Kekäläinen, 2018, U.S. Patent and Trademark Office.

Kekäläinen (2018), also for Enevo, is about how often the gauge talks and not about what it measures. Figure 11 shows a garbage container with a hinged lid and the gauge attached under the lid. The model of how the gauge is supposed to function is sent to the gauge by a server that includes the maximum and minimum fill-rate limits and a communication schedule. The gauge maintains the microcontroller and transceiver in sleep mode, sensing the fill rate and waking only if the fill rate is at, or exceeds, a limit, transmitting the fill rate measurement and receiving an updated model. A lower number of transmissions equals a longer battery life.

Smart-Bin takes the opposite path over the same issue. It reports on a fixed schedule, with a write to ThingSpeak every 30s. A bin that is left outside for weeks could be threshold driven. The parts read the patent describes no motorized lid, tamper alarm, PIN mode, touchscreen or phone app. Its user device is a laptop or desktop.

Smart Garbage Bin (Shahabdeen, 2016)

Figure 12

Smart Garbage Bin with a Lid-Mounted Sensor Device

Note. From Smart garbage bin (U.S. Patent Application Publication No. 2016/0176630 A1), by J. A. Shahabdeen, 2016, U.S. Patent and Trademark Office.

Shahabdeen (2016) filed this application for Zan Compute. Figure 12 shows the bin with a battery-powered wireless sensor device on its lid. The sensor is a reflective infrared range finder (a Sharp GP2Y0A41SK0F) aimed at the waste, and the description adds MQ-136, MQ-137 and MQ-4 gas sensors for hydrogen sulfide, ammonia and methane. A weight sensor at the base is the alternative way to measure how much has been deposited. A local controller sends the data wirelessly to a remote controller (MQTT in claim 17), and claim 13 sends an alert when a sensed value passes a threshold. The application says that an ultrasonic sensor was a possible alternative to the infrared range finder but judged it too power-hungry.

It's the closest to Smart-Bin in hardware: the distance to the trash from under the lid, the weight under it, gas sensing, and wireless reporting. It does not provide tamper detection, powered lid, lock, display, Pin mode, priority list and does not talk about temperature and humidity. The lid only carries the sensor. Google Patents lists the application as abandoned.

Input Device for a Pay-by-Weight Food Waste Processor (Shin, 2016)

Figure 13

Pay-by-Weight Food Waste Processor with a Load-Cell Hopper

Note. From Inputting device for quantity and weight loss of food garbage container (Korean Patent No. 10-1581944), by H. Shin, 2016, Korean Intellectual Property Office.

Shin (2016), for Concoftech, patented the waste input device of a pay-by-weight food waste processor. Figure 13 is a perspective view of part of the machine, with some parts omitted and separated. The waste is dropped into a hopper which sits on a load cell, the user pays electronically on the display, and the hopper then rotates approximately 170-180 degrees and releases the waste into a crushing and stirring tank. The guide member with rollers allows hopper to be turned within the space of the lid without snagging, maintaining the machine compactness and convenient loading height. Drawings show that payment data is sent to a primary server not shown in the drawing.

The load cell, a powered lid and a display for the user are shared with Smart-Bin. It is not for the same purpose. The Korean device crushes the garbage and charges according to its weight, while Smart-Bin weighs to verify the garbage level. There is no fill-level, gas, tilt or tamper sensing, no wireless or cloud reporting in the text read, no collection priority list and no mobile alerts in the patent. The reason for earlier deodorant sprayers and sterilizing lamps is that the odor is only in the background.

Techniques for Optimally Sensing Full Containers (Devitt & Walsh, 2021)

Figure 14

Infrared Beam Across a Container

Note. From Techniques for optimally sensing full containers (U.S. Patent No. 11,054,545), by S. Devitt and B. Walsh, 2021, U.S. Patent and Trademark Office.

Devitt and Walsh (2021), for Oneplus Systems, give two ways to detect a full container while keeping very low the current drawn by battery-powered wireless circuitry. Figure 14, which combines Figures 1A and 1B of the patent, shows the first: an infrared emitter and receiver mounted on the side walls, so that the line between them marks the full level. The emitter transmits a wake-up signal and coded pulses, the receiver waits in sleep mode until the wake-up signal is received, and at the end of a specified period of time, if the coded signals are not received, the container is considered full. The second method checks the resting tilt angle of lid in three axes since a lid that overfull waste holds up is set at a different angle. The data from fullness is sent to a cloud server.

The patent only gives full or not full, with Smart-Bin providing a calibrated percentage and a time to full. No weight, gas, temp, powered lid, PIN mode/tamper alarm is displayed in parts read.  The tilt here is the angle of the lid, used as a sensing element, and it is not a tamper check, so it should not be confused with the BMI160 tilt alarm of Smart-Bin, which watches the whole bin.

Trash Can With a Sensing Flip Lid (Wang, Chen, et al., 2020)

Figure 15

Circuit of the Induction Flip Lid with a Settable Sensing Distance

Note. From Trash can having sensing flip lid having settable sensing distance and control method therefor (European Patent No. 3 620 405 A1), 2020, European Patent Office, X. Wang et al. (sensing flip lid).

A clamshell trash can with an infrared sensing distance set by the user is described by Wang, Chen and Lin (2020) of Fujian Nashida Electronic. The circuit diagram is shown in figure 15. The major components are an infrared transmitter and receiver underneath an induction window, a single microcomputer chip for the amplifier, analog-to-digital converter and distance registers, and a drive circuit to operate the motor and reducer. Only when the reading is above the standard distance value, the motor will flip the cover open and when the user selects a distance by pressing a setting button, it will be stored and used for comparison later.

This is the closest equivalent of the servo-lid of Smart-Bin that is triggered by an IR sensor. A hand near the induction window opens the lid and how near is up to the user. As no fill or weight, gas, tilt, display or wireless link or monitoring is involved in the project, nothing else is addressed in the patent. It was listed as a published application (kind code A1) on Google Patents and withdrawn in 2021, meaning that it was not granted.

Smart Waste Bin Sensor Device and Methods (Kurani, 2023)

Figure 16

Overall Smart Waste Management System

Note. From Smart waste bin sensor device and methods for waste management system (U.S. Patent No. 11,702,280), by H. B. Kurani, 2023, U.S. Patent and Trademark Office.

Kurani (2023) is the broadest patent of the twenty on sensing. Figure 16 gives the whole system for monitoring bins continuously to optimize collection and reduce littering. The bin is equipped with a battery powered smart sensor device carrying approximately a dozen sensors, including fill level, temperature, humidity/pressure, air quality, smoke, gas, motion, a pathogen biosensor and sterilizer, and an accelerometer. Readings are transmitted via a wireless network to a cloud server which creates real-time bin statistics and optimizes collection routes. The data is displayed in a bin app and a facility app, and biohazard and biosafety alerts are sent to nearby phones. The accelerometer detects when the bin has been tilted, and resets the fill level to zero when the bin tips over.

It matches Smart-Bin on sensing, cloud, apps and routes: fill, gas, temperature and tilt data go to a cloud server with mobile alerts and an optional touchscreen on the device. Its centre of gravity lies elsewhere, in litter, biohazard and pathogen sensing with a sterilizer. The parts read show no IR-actuated servo lid, remote lock, PIN collect mode or tamper alarm with a latch, and the tilt serves to reset the fill level and does not sound an alarm.

Smart City Garbage Management System (Yan & Chen, 2021)

Figure 17

Garbage Cans and the Monitoring Center

Note. From Smart city garbage management system (Chinese Patent No. CN 109324552 B), by S. Yan and J. Chen, 2021, China National Intellectual Property Administration. The labels in the original are in Chinese.

Yan and Chen (2021), for Shanghai Shuncom Smart Technology, patented a city-wide system. Figure 17 is the schematic of the garbage cans and the monitoring center. Each can has a positioning unit, a controller and sensors for weight, infrared height, odor and smoke, and the controller uploads the readings to the monitoring center. The center decides which cans need inspection (weight, height or odor above a threshold, or any smoke), generates the route of the truck. When a special-condition signal arrives, a nearby truck is rescheduled by priority, in the order fire, fault, pollution and full bin.

This patent is the closest in purpose. Weight, height and odor decide which cans are serviced first, as the priority tags of Smart-Bin do. It goes further by building routes for trucks with GPS, and Smart-Bin stops at ranking the bins. It also includes smoke sensing, which Smart-Bin does not have. No powered lid, tamper detection, PIN mode, on-bin display or phone alert appears in the text read.

Apparatus and Method for Waste Monitoring and Analysis (Balachandran & Talavera, 2023)

Figure 18

Waste Measurement System with Weighing Devices and Reports

Note. From Apparatus and method for waste monitoring and analysis (U.S. Patent No. 11,788,877), by N. Balachandran and O. Talavera, 2023, U.S. Patent and Trademark Office.

Balachandran and Talavera (2023), for Zabble, cover waste audits. Figure 18 gives an overview of the measurement system: a series of weighing devices record the weight of the waste, the data go to networked storage, and the stored data are turned into visual reports for an operator. Claim 1, though, is about images: a method that acquires images of the contents of at least two containers of a waste generator, identifies characteristics of the contents and generates a report by computer. The disclosure adds load cells under a platform for two or more bins and gas sensors for compost bins.

The shared ground with Smart-Bin is remote monitoring of bin contents, with cloud upload and dashboards, and gas sensing in one special case. The aim is different. This patent is an audit and reporting service that tells a waste generator what is being thrown away, while Smart-Bin is a single bin that works out when it needs emptying. The claims do not cover a bin with fill, tilt, lid and lock control, a PIN collect mode or a collection priority list.

Capacity Inspection Garbage Bin With Infrared Induction (Yang et al., 2020)

Figure 19

Block Diagram of the Capacity-Detecting Bin

Note. From Capacity automated inspection garbage bin with infrared induction function (Chinese Patent No. CN 210504152 U), by S. Yang, D. Liao, R. Li, and Y. Yu, 2020, China National Intellectual Property Administration. The labels in the original are in Chinese.

Yang et al. (2020), for Sichuan Grid Environmental Protection Technology, is a utility model for a bin that shows how much capacity it has left. Figure 19 is a block diagram of the inside of the bin. The body holds an infrared induction module, an ultrasonic ranging module, a programming module and a power module. The ultrasonic module is connected to the infrared and programming modules, and the power module supplies all three. The ultrasonic module measures the remaining capacity and two nixie-tube digits show it on the bin, while the infrared module detects a nearby user to wake the system. Google Patents lists the rights as terminated for non-payment of fees.

This utility model matches the local side of Smart-Bin: an ultrasonic ranging module, a wake-up by a nearby user, and the capacity shown on the bin itself, where Smart-Bin shows it as a percentage on its touchscreen. Little else carries over. The device is stand-alone, with no weight, gas, tilt, lid actuator, wireless link, server, route planning or phone app.

Garbage Bin Intelligent Controller and Supervisory System (Wang, Zhang, et al., 2020)

Figure 20

Electrical Block Diagram of the Bin Intelligent Controller

Note. From Garbage bin intelligent control ware and garbage bin supervisory systems (Chinese Patent No. CN111559590A), by X. Wang, Y. Zhang, and Z. Wang, 2020, China National Intellectual Property Administration. The labels in the original are in Chinese.

Wang, Zhang, and Wang (2020), for Weihai Jingxun Changtong Electronic Technology, published an application for a bin controller that can be installed on top of the bin or at its side. Figure 20 is its electrical block diagram, with a wireless communication module, a control module and a distance sensor in one housing. In top mode the controller works out the height of the garbage, in side mode it reports the distance to the opposite wall, and either value is uploaded to monitor whether the bin is full. In the description, an ultrasonic module measures the distance, an inclination sensor raises a tip-over or movement alarm, the data go by NB-IoT and alerts reach the phones of workers.

The overlap with Smart-Bin is strong on the alert side: a distance sensor for fill, an inclination sensor for tip-over alarms, wireless upload and alerts on a phone. The patent has no weight or gas sensing, powered lid, lock, PIN or touchscreen collect mode, priority list or time to full, and in the parts read the inclination alarm has no countdown or latch, which Smart-Bin adds. Google Patents lists the application as withdrawn.

Measuring the Fill Level of a Trash Can Using a Sensor (Keller et al., 2024)

Figure 21

Automatic Alerting System for a Trash Can

Note. From System and method for measuring the fill level of a trash can using a sensor (U.S. Patent No. 12,125,361), by J. R. Keller, M. O. Redmond, and S. A. Mirza, 2024, U.S. Patent and Trademark Office.

Keller et al. (2024), for 7-Eleven, patented a way to turn a distance reading into a fill percentage. Figure 21 shows the automatic alerting system: a sensor above the container, a gateway and network, and a computer system with its modules and a user device. In claim 1 a sensor above the can holds a laser diode that sends a pulse toward the bottom and measures the time of flight. A computer converts the distance to a percentage of waste with two calibration setpoints, picks a threshold for the current time period from stored information about the owner of the can, and alerts a user device when the threshold is exceeded. Alerts can be email or text on a smartphone.

The two setpoint calibration is identical to that of Smart-Bin: the empty distance (default: 30cm) and the full distance (default: 5cm) are programmed from the interface, and the fill percentage is programmed to the distance between. A threshold-based alert reaches a phone in both. Everything else in the patent is narrower: a single laser sensor, no weight, gas, tilt or tamper sensing, no lid control or lock, no touchscreen collect mode and no priority list.

Waste Bin Collection and Monitoring Method Using Bin Sensors (Sait et al., 2025)

Figure 22

Network of Waste Bins, Cloud Server and Vehicles

Note. From Waste bin collection and monitoring method using waste bin sensors (U.S. Patent No. 12,450,575), by S. Sait, F. M. Aliyu, S. Bukar, B. S. Maina, and D. U. Lawal, 2025, U.S. Patent and Trademark Office.

Sait et al. (2025), from King Fahd University of Petroleum and Minerals, is the newest patent in the set. Figure 22 is a network diagram of the waste bins, the cloud server, the cloud network that connects them and the waste vehicles. Claim 1 describes a method to monitor and control the CO2 and CH4 emissions of a collection system. The methane, humidity, waste level, location and temperature are sensed under every separate bin lid and transmitted to a cloud server. One processor calculates the time in which the methane limit will be reached and a second processor calculates the time in which the fill limit will be reached, and a third processor, based on the CO2 and CH4 readings of the vehicles, prepares the collection schedule and routes. Two neural networks make the time estimates, and a mobile app sets parameters and shows data.

It is the nearest to Smart-Bin in using a time to full, in sensing fill, gas, temperature and humidity inside the bin, and in sending them to a cloud server with a display and a mobile app. The estimate is made differently. Neural networks work from methane and fill data, while Smart-Bin uses the fill rate of the last 15 minutes with no trained model. The patent is centered on methane prediction and routing based on emissions. It has no load cell, tilt or tamper alarm, IR sensor lid, remote lock or PIN collect mode, and weight and tilt appear only in a list of possible sensors.

### Related Literature and Studies

Thirty studies were reviewed between 2023 to 2026. Each of them is listed in Table 2 with its hardware, the main reported result and what it is lacking in comparison with Smart-Bin. If a figure is derived from training data or is a synthetic data set, this is indicated in the table as these figures are not representative of the actual system in use. Each study is then taken in turn under one of four headings, according to what it puts the most weight on: level-sensing bins with lids and alerts, bins that add weight, gas or access control, collection routes and priority, and cameras, sorting and system design. A figure is shown wherever the source is open access and has a drawing or photograph that helps, with the licence of the figure in the note under it. The other studies are text only, because their publishers do not allow the figures to be reused.

Table 2

Related Studies From 2023 to 2026

| Study | Hardware and platform | Reported result | What it lacks |
| --- | --- | --- | --- |
| Saad et al. (2023) | NodeMCU ESP8266 and Arduino Nano, HC-SR04, HX711 load cell, GPS, SG90 lid, WhatsApp through Twilio | Lid opens within 50cm. Full at 3cm of free space or 4,000g. Height error 0.4%, weight error 0.15%, message in 2.61s. | Dashboard, air quality, tamper detection, cloud history, roles |
| Abdullahi et al. (2024) | Arduino with HC-SR04 and SIM800L GSM, servo lid, cloud server | Lid opens only when the bin is not full. Full at 20cm. No performance figures are given. | Weight, air quality, tamper detection, dashboard, app |
| Alnuaimi et al. (2024) | Simulation of smart-bin collection as a travelling salesman problem (AnyLogic) | Shorter travel distance and lower emissions than static routes. No figures are given. | Bin hardware, alerts, app |
| Ayyanagoudar et al. (2024) | Ultrasonic and weight sensors, web page for each bin level | Real-time level tracking for each bin on a web page. No measured results are given. | Air quality, tamper detection, app, alerts |
| Azmi et al. (2024) | Arduino Uno with LoRa, HC-SR04, DHT11, GPS, ThingSpeak dashboard | Ultrasonic accuracy 89.9% at half full, humidity 96.9%, temperature 94.4%. Alerts are dashboard indicators only. | Weight, app, push alerts, on-bin display |
| Enache et al. (2024) | ESP32-CAM nodes with solar cell, ESP-MESH, server image processing | Fill ratio estimated from hourly bin photographs. Relative error 11.43% to 30.00% on five bins. | Weight, air quality, tamper detection, app |
| González et al. (2024) | ESP32 and ESP32-CAM, ultrasonic fill, simulated weight, Oracle APEX dashboard, App Inventor app | Camera classifies waste; weight is simulated with potentiometers. No accuracy figures. | Real weight sensing, air quality, tamper detection, priority list |
| Kuzhin et al. (2024) | IoT sensors with route analytics; no hardware described | The results are 20% fewer overfilled bins, 15% fewer collection trips, 25% shorter trips, and 10% less fuel. | Any bin hardware; setting and sample are not given |
| Mehta and Rathour (2024) | Ultrasonic, temperature, weight and tilt sensors and data software. | 6-month pilot: approximately 50% improvement in collection efficiency, 25% reduction in cost, and 83.3% reduction in overflows. | Detail on site and method; air quality, app, on-bin display |
| Muthukumar et al. (2024) | Camera-based AI segregation bin with a staff app for fill level and temperature and humidity | Classifier test accuracy 92.3% (training accuracy 71.8%). The scalability claim is not tested. | Weight, air quality, tamper detection, collection priority; main aim is segregation |
| Rathnayake et al. (2024) | Three ESP32 bins with ultrasonic and methane sensors, Node-RED cloud, machine learning | 4-month pilot: fill accuracy 95%, methane detection 92%. Route simulation 15% more path-efficient. | Weight, tilt, app, on-bin display |
| Salleh et al. (2024) | Using NodeMCU ESP8266, ultrasonic sensor and ThingSpeak dashboards. | Live container levels support need-based collection scheduling. Proteus simulation and PCB layout are included. | Weight, air quality, tamper detection, app |
| Shaharil and Po’ad (2024) | Arduino, ultrasonic lid sensor, IR fill sensor, load cell, MQ-2, LCD, Blynk | Weight errors of −5g to +10g on four items. The Blynk app shows a full-bin notification. No numbers for the IR or gas sensors. | Web dashboard, collection priority, tamper detection |
| Thangam et al. (2024) | Use of ESP8266, ultrasonic and soil-moisture sensors | Analytics are meant to let authorities adjust routes. No results are reported. | Weight, air quality, tamper detection, app |
| Aasish et al. (2025) | Dual-core ESP32 on FreeRTOS, ultrasonic, DHT sensor, RFID bin tags, LCD or OLED, cloud server | Splits sensing and cloud communication across tasks. No measured results. | Weight, tamper detection, app, test data |
| Alnanih et al. (2025) | Arduino Uno and ESP8266 with GSM, HC-SR04, flame sensor, GPS, LCD, Firebase, web app, Flutter app | 6-month, 50-bin campus pilot: 99.5% uptime, fill error within ±3%. Smoke Test fire alarm in 5s. | Weight, humidity, tamper detection, time to full, UI calibration |
| Alourani et al. (2025) | Arduino Nano and ESP8266, ultrasonic sensor, Firebase, Android app, VGG-19 classifier | App alerts collectors at a fill threshold. Classifier 99.7% training accuracy. | Weight, air quality, tamper detection, priority list |
| Banciu et al. (2025) | GPS and RFID bin locations; Nearest Neighbor, OR-Tools and genetic algorithm | Three real routes with 735 locations: up to 52% shorter distance, over 14kg less CO₂. | Any bin sensing; the study is about routing |
| Gajula et al. (2025) | ESP32, HC-SR04, Moisture sensor, Servos and Arduino IoT Cloud. | Separates dry and wet waste by moisture. No figures are given. | Fill-level role of the ultrasonic sensor, weight, tamper detection, app |
| Gupta et al. (2025) | Microcontroller with ultrasonic sensor and Wi-Fi, cloud platform | 30-day, 50-bin pilot: 58.9% less distance, 59.2% less fuel, 95% fewer overflows, 99.8% uptime. | Weight, air quality, tamper detection, app detail |
| Ishaq et al. (2025) | Ultrasonic sensors in bins at ten hospitals, cloud monitoring station | Alerts led to pickups 3 to 16 minutes later. Biomedical waste about 43.6% to 52.5% of hospital waste. | Weight, air quality, tamper detection, app (proposed only) |
| Karthi et al. (2025) | ESP32, RFID reader, ultrasonic sensor, metal detector, MQ135, Blynk | 95% fill accuracy, 92% RFID reliability, 40% less manual intervention. | Weight, tamper detection, priority list, web dashboard |
| Kumar et al. (2025) | Using ESP32, ultrasonic sensors, Ant Colony Optimization, machine learning, and map. | Prioritizes filled bins and builds depot-to-depot routes. No numbers are reported. | Weight, air quality, tamper detection, app, alerts |
| Maulida et al. (2025) | ESP32, HC-SR04, MQ-2, HX711, SG90 lid, Firebase, Android app | Level accuracy about 95%, weight about 93%, 92% of spoiled-food cases flagged, usability score 86 (15 officers). | Tamper detection, collection priority, web dashboard, roles |
| Mehta et al. (2025) | ESP8266 with ultrasonic sensor, MQ4, MQ136, MQ137 and GPS, Firebase and React Native Application | Gas sensors 96% to 98% accurate against a reference. Field test of 28-bin samples cut trucks from 3 to 2. The 100-bin results are synthetic. | Weight, tilt, on-bin display; some tables conflict |
| Sadikin et al. (2025) | ESP32 with ultrasonic, infrared and GPS, Firebase, Kotlin Android app | Ranking by distance, vehicle capacity and fill level (greedy). 69 respondents: 92.37% effectiveness, 89.92% satisfaction. | Weight, air quality, tamper detection, on-bin display |
| Shahibollah et al. (2025) | ESP32, ultrasonic sensors, servo lid, tri-colour LED, Blynk | Tested by simulation and on hardware. No figures are given. | Weight, air quality, tamper detection, priority list |
| Yogesh et al. (2025) | ESP32, HX711 load cell, ultrasonic sensor, Firebase, App Inventor app, LCD | Weight and fill percentage with full-bin alerts. No measured results are reported. | Air quality, tamper detection, priority list, test data |
| Awotayo and Dodo (2026) | ESP32, two HC-SR04, MG90S servo lid, LEDs and buzzer, Blynk email | Fill zones at 40% and 60%; email when full. No accuracy or response-time results. | Weight, air quality, tamper detection, dashboard, app |
| Mugaruka et al. (2026) | LoRaWAN-style network of ultrasonic sensors in seven 120-L bins | 12-week pilot: 57% of sensors at or below −107dBm; mass and route times are estimates. | Weight, air quality, tamper detection, app |

Note. Results are as reported by the authors. Lacks means that the feature is not described in the study.

A. Level-Sensing Bins with Lids and Alerts

Smart Bin with Automatic Lid Control (Abdullahi et al., 2024)

Figure 23

Block Diagram of the Smart Waste Management System

Note. From “Development of a smart waste management system with automatic bin lid control for smart city environment,” by A. Abdullahi, A. Mohammed, M. U. Bonet, A. El-Suleiman, R. B. Ahmad, and T. D. Chollom, 2024, EAI Endorsed Transactions on Smart Cities, 7(3). CC BY-NC-SA 4.0.

Abdullahi et al. (2024), of the Air Force Institute of Technology in Kaduna, Nigeria, built a bin whose lid opens only when the bin is not full. Figure 23 is their block diagram. The fill level is measured by an ultrasonic sensor (HC-SR04) and an Arduino, which also detects the approaching user, and the lid is opened by a servo motor that rotates 0-90 degrees. The thresholds are 20cm for a full bin, and 12cm for a user. Status connects to the cloud server via a SIM800L GSM module where data is stored and full bin notifications are sent to the collection team.

There are no figures for performance and it is inconsistent regarding the board as one figure shows Uno and the other shows Nano. There's a phone block in the diagram, but the app is never mentioned. Smart-Bin includes the ultrasonic fill sensor, a servo lid and weight, air sensing, tilt, a dashboard, and a phone app, all of which this study lacks.

Smart Waste Management System Using LoRa (Azmi et al., 2024)

Figure 24

Circuit Setup of the LoRa Bin Unit

Note. From “Smart waste management system using LoRa technology,” by A. Z. Azmi, N. M. Sahar, and M. T. Islam, 2024, E3S Web of Conferences, 599, Article 04007. CC BY 4.0.

Azmi et al. (2024) installed a LoRa connection between the bin and the receiver. The setup for their circuit is shown in Figure 24. The ultrasonic fill distance, DHT11 temperature and humidity and GPS position are transmitted through a bin-side Arduino Uno with LoRa shield to a second Arduino Uno with LoRa receiver, and then relayed to a NodeMCU which updates a ThingSpeak dashboard with indicators when thresholds are reached. The tests indicated an ultrasonic accuracy of 89.9% at half full, a humidity accuracy of approximately 96.9% and a temperature accuracy of approximately 94.4%.

The authors report that the ultrasonic sensor was not successful in a narrow, shallow bin, and the DHT11 temperature sensor error exceeded the sensor's specified error. They state that they have a LoRa range of several kilometers, but they did not measure it, and the abstract mentions air quality monitoring, but does not go into any detail about it in the body of the paper. Also, Smart-Bin senses temperature and humidity (BME680 used instead of DHT11) and uses ThingSpeak. The study has no weight sensing, push alerts, phone app or display on the bin.

Ultrasonic Trash Can Surveillance with ThingSpeak Dashboards (Salleh et al., 2024)

Salleh et al. (2024) developed a prototype for waste collection based on demand at a low cost. An ultrasonic distance sensor mounted in each container is read by a NodeMCU ESP8266 module which uploads the fill level to ThingSpeak so that live dashboards indicate the status of each container and enable container status-based scheduling. Also included is a Proteus simulation and a PCB layout. No results are reported, and the authors view this work as a scheduling tool, rather than a validated route planning method.

Smart-Bin also employ ThingSpeak for charts and analysis, but only as a second cloud besides Firebase, as optional. It provides the weight, air quality, tamper detection and a phone app, things this study doesn't.

ESP8266 Waste Monitor With Moisture Sensing (Thangam et al., 2024)

Thangam et al. (2024) of Amrita Vishwa Vidyapeetham, Bangalore, have developed an ESP8266 monitor that utilizes ultrasonic fill sensors, soil-moisture sensors to assess the waste composition. The real-time data is supposed to enable the authorities to modify routes and reduce trips and gas. The route claim is a stated aim but not a measured result, as no quantitative results are reported in the abstract.

Smart-Bin senses are filled by the same type of ultrasonic sensor with the addition of weight, air quality, tamper detection and a phone app. It makes no attempt to judge the composition of the waste.

Smart Bins for Hospital Waste in Kaduna (Ishaq et al., 2025)

Figure 25

Proposed System Architecture for Hospital Waste Monitoring

Note. From “Smart waste bin monitoring using IoT for sustainable biomedical waste management,” by A. Ishaq, S. J. Mohammad, A. D. Bello, S. A. Wada, A. Adebayo, and Z. T. Jagun, 2025, Environmental Science and Pollution Research, 32(32), 19434–19449. CC BY 4.0.

Ishaq et al. (2025) put ultrasonic fill-level sensors in bins at ten hospitals in Kaduna, Nigeria. Figure 25 is the proposed system architecture, from the hospital ward to the smart bins, an internet link, a laptop at the monitoring station and the collection truck. The bins connect wirelessly through the routers of the hospitals to a central cloud monitoring station, which alerts waste collectors when a bin is full. The authors also measured waste generation. Biomedical waste made up about 43.6% to 52.5% of hospital waste, at an average of 69.14kg per day and 1.52kg per bed per day, and they analysed the data with principal component analysis.

Alerts led to pickups 3 to 16 minutes later. The phone app appears only in the proposed system and is not described as built, and no microcontroller is named for the bins of the authors. The study shows what level sensing is worth in a place where overflow carries a health risk. Smart-Bin senses more than level, but it has not been tested in any such site.

IoT-Enabled Smart Dustbin for Real-Time Monitoring (Shahibollah et al., 2025)

Shahibollah et al. (2025), from Universiti Teknologi MARA in Terengganu, built an ESP32 dustbin with ultrasonic sensors for fill level and for proximity, a servo-driven lid and a tri-colour LED for local status. Data go over Wi-Fi to the Blynk platform, which alerts the maintenance staff. The authors tested the bin by simulation and with hardware, but the abstract gives no ranges or accuracy figures and describes the result as prototype function and not as municipal-scale reliability.

Smart-Bin uses the same ESP32 family and a servo lid. Its touchscreen takes the place of the LED, and it adds weight, air quality, tamper detection and a priority list.

IoT Smart Bin for Real-Time Fill Level Monitoring (Awotayo & Dodo, 2026)

Awotayo and Dodo (2026) of Baze University, Abuja designed and implemented an ESP32-based bin equipped with two HC-SR04 sensors. One of these is facing outwards and will open a servo lid (MG90S 0 to 90 degrees) when someone is in range of 30cm. The other indicates the level of fill, with a green LED when the fill level is between 0 and 39%, a yellow LED when the fill level is between 40 and 59%, and a red LED and buzzer when the fill level is between 60 and 100%. An e-mail is sent by Blynk when the bin is full. Two 3.7V Li-ion cells (7.4V, 3500mAh) power the unit. There are no accuracy or response-time results reported.

The local feedback is LEDs and a buzzer and not a screen. Smart-Bin also includes a touch screen, weight detection, air quality, tamper detection, dashboard and a phone app, none of which were identified in this study.

LoRaWAN Network for Waste Collection in Bukavu (Mugaruka et al., 2026)

Figure 26

Proposed Network Architecture for Waste Collection in Bukavu

Note. From “Design of an IoT sensor-based network architecture for waste collection in Bukavu city (DRC),” by O. M. Mugaruka, J. R. Kala, J. Ndikumagenge, R. van Schadewijk, and R. Nibitanga, 2026, Discover Internet of Things, 6(1), Article 168. CC BY-NC-ND 4.0.

Mugaruka et al. (2026) designed a LoRaWAN-style network for waste collection in Bukavu, in the Democratic Republic of the Congo. Figure 26 shows the proposed network. EM310-UDL ultrasonic sensors on the bins talk over LoRa to a gateway, the gateway reaches a cloud server by Ethernet, Wi-Fi or 4G, and users view the data on a control station and on mobile phones. The authors tested it in a 12-week pilot with seven 120-litre prototype bins in residential, dense and peri-urban areas.

The results concern the network more than the bins. Of the sensors, 57% had a signal at or below −107dBm, and the mass collected (an estimated 641kg over 14 days in one area) and the route times are estimates from sensor data and not direct measurements. The authors also report an energy autonomy above 90% for most units after 11 months, a period that the paper does not clearly tie to the 12-week pilot. Smart-Bin does not use LoRa at present, and the study is a reminder of the opportunity listed in the SWOT analysis: LoRa for places without Wi-Fi.

B. Bins That Add Weight, Gas or Access Control

IoT-Based Smart Dustbin Prototype (Saad et al., 2023)

Figure 27

Prototype of the Smart Dustbin

Note. From “IoT-based smart dustbin prototype,” by A. M. Saad, B. O. Jul, A. Basalamah, and S. Sayuti, 2023, PROtek: Jurnal Ilmiah Teknik Elektro, 10(2), 120–125. CC BY-NC-SA 4.0.

Saad et al. (2023) built the smart dustbin shown in Figure 27. An Arduino Nano with an ultrasonic sensor opens an SG90 servo lid to anyone who is within 50cm, while a NodeMCU ESP8266 reads the waste height with an HC-SR04, the weight with an HX711 load cell and the position with a NEO-6M GPS. Once the bin is full (either by level (3cm free space) or weight (4,000g), the NodeMCU will send a WhatsApp message via Twilio with a Google Maps link.

The tests reported a height error of 0.4%, a weight error of 0.15% and a message delay of 2.61s. This is the nearest study to the pairing of fill and weight with a servo lid in Smart-Bin. Its lid is triggered by an ultrasonic sensor, where Smart-Bin uses an IR obstacle sensor. The study has no dashboard, air quality sensing, tamper detection, cloud history or roles.

Smart Bin With Real-Time Garbage Level Tracking (Ayyanagoudar et al., 2024)

Ayyanagoudar et al. (2024) propose a smart bin that uses weight sensors and ultrasonic transducers to track the fill level in real time and shows the level of each bin on a web page. The abstract also states that it can classify trash based on its weight, which is used for planning transportation. The abstract does not mention any microcontroller or cloud platform and does not provide any results of measurements.

Smart-Bin also correlates weight to an ultrasonic level. It includes air sensing, tamper detection and a mobile phone app, which are not included in this study.

Efficiency of IoT-Enabled Smart Bins (Mehta & Rathour, 2024)

Mehta and Rathour (2024) present the description of IoT smart bins, which are equipped with ultrasonic, temperature, weight and tilt sensors and software to collect and process the data. They claim a 6-month pilot in one city where collection efficiency has improved by approximately 50%, operating costs have decreased by approximately 25% and there have been approximately 83.3% fewer overflow incidents, while also offering reduced emissions through route optimization and increased satisfaction of citizens. Abstract makes no mention of the location or nature of these measures or of the number of samples collected, these figures are merely the authors' claims.

This is the only study to mention a tilt sensor but does not indicate a tamper alarm. Smart-Bin does exactly that, a 10 second (latched) countdown followed by a siren that latches.

Dual-Sensing Bins for Fill Level and Methane (Rathnayake et al., 2024)

Rathnayake et al. (2024) developed the dual sensing bins which combined the ultrasonic fill sensor with methane sensor for detection of spoilage. The ESP32 modules transmit the data via Wi-Fi to a cloud platform where the data is processed by the Node-RED software and ML algorithms predict the waste and optimize routes. Three bins were deployed in a four-month pilot in Dehiwala, Sri Lanka and reported a 95% fill-level accuracy; methane-detection accuracy was 92% and simulated routes were 15% more path-efficient than fixed routes. There is no definition of the accuracies in the abstract and 15% is obtained from a simulation.

The ESP32 and the ultrasonic level are shared between Smart-Bin. It is not a methane reading, but rather an air-quality estimate as it is a gas channel. This study doesn't include any weight sensing, tilt alarm, phone app or on bin display.

Arduino-Based Smart Dustbin (Shaharil & Po’ad, 2024)

Figure 28

Block Diagram of the Arduino-Based Smart Dustbin

Note. From “Arduino-based smart dustbin with ultrasonic sensor for efficient waste management,” by M. H. F. Shaharil and F. A. Po’ad, 2024, Evolution in Electrical and Electronic Engineering, 5(2), 92–97. CC BY-NC-SA 4.0.

Shaharil and Po’ad (2024) built a household-scale bin and show its block diagram in Figure 28. The ultrasonic sensor opens up the servo lid if a person is within 30cm, the infrared sensor detects the level of fullness of the bin, the weight is displayed on the LCD using a load cell, and finally, a buzzer alerts when air quality is poor with the help of an MQ-2 sensor. The Blynk app shows a notification when the bin is full.

The testing is small: 15 distance trials and four weighed items with errors from −5g to +10g, and no numbers for the infrared or gas sensors. The bin is the closest study to the local side of Smart-Bin, with a lid, weight, gas and a display on the bin. It has no web dashboard, no collection priority and no tamper detection.

IoT-Driven System with Fire Detection and Role-Based Apps (Alnanih et al., 2025)

Figure 29

Data Communication Structure of the Smart Waste Management System

Note. From “Advancing sustainability through an IoT-driven smart waste management system with software engineering integration,” by R. Alnanih, L. Elrefaei, and A. Al-Ahwal, 2025, Sustainability, 17(21), Article 9803. CC BY 4.0.

Alnanih et al. (2025) combine fill and fire sensing in one bin unit. Figure 29 shows their data communication structure. An HC-SR04 is used to measure the fill level, a flame sensor detects fire, smoke and gas and an Arduino Uno with an ESP8266 and GSM receives the sensor data and an LCD, buzzer are added. Firebase and Firestore are used to launch alerts, coordinating collection routes from a central command center. A desktop web application is used by administrators and a Flutter Android application for workers based on their roles.

6-months pilot on campus with 50 bins achieved 99.5% uptime with fill error within ±3%, and a fire alert within 5s (smoke test). This is one of the longest field pilots of the studies which constructed their own bins. It's similar to Smart-Bin, it has Firebase, a web app, a phone app and different views for different users. There is no weight, tilt or lid feature and nothing is calibrated from the interface in the hardware list.

ESP32 Smart Trash Bin with Firebase (Maulida et al., 2025)

Figure 30

Prototype of the SmartTrashBin

Note. From “Design of IoT-based smart trash bin monitoring using ESP32 and Firebase,” by A. Maulida, C. S. Fayzalmi, F. A. C. Anaam, D. Ardiansyah, M. A. Gunawan, and N. Ridwan, 2025, Jurnal Komputer, Teknologi Informasi dan Sistem Informasi (JUKTISI), 4(2), 420–428. CC BY 4.0.

The SmartTrashBin in Figure 30 was created by Maulida et al. (2025) from Politeknik Negeri Semarang. The hand detection & fill level is read using HC-SR04, while MQ-2 is used to detect the odour and the load cell (HX711) detects the weight that is then fed to ESP32, which controls the opening of the lid using a servo (SG90). Data is stored in Firebase Realtime Database and sent to an Android application that includes notifications.

They claim a level accuracy of approximately 95%, a weight accuracy of approximately 93% (+/- 320g on 20kg), 92% of cases of spoiled food identified and 98% of notification reliability and a System Usability Scale score of 86 from 15 sanitation officers. The numbers require carefulness as the paper mentions both of Android Studio and Flutter and also provides two notification thresholds. The closest of the studies to Smart-Bin is the following: ESP32, ultrasonic sensor, load cell, servo lid, firebase and an android app. No tamper detection, collection priority or web dashboard/roles.

Real-Time Sensing and Methane Mapping (Mehta et al., 2025)

Figure 31

Data Flow of the Garbage Monitoring System

Note. From “Smart bins, smarter cities: IoT-driven waste collection with real-time sensing and methane mapping,” by H. Mehta, V. Deriya, D. Sankhe, P. Kanani, G. Pandya, and V. Shelke, 2025, Mathematical Modelling of Engineering Problems, 12(6), 2110–2122. CC BY 4.0.

Mehta et al. (2025) show the data flow of their system in Figure 31. Each bin has the ESP8266 equipped with an ultrasonic fill sensor, MQ4, MQ136 and MQ137 gas sensors, GPS and uploads data to Firebase every 10s. A React Native application displays a bin map, a methane and odor heatmap and truck routes that are ranked based on a priority score derived from fill and gas.

Gas sensors accuracy was 96% - 98% with reference. There are a number of trucks that are different – field samples of 28 bins in Malad v. 100 bins in the results (which use synthetic data), and some tables in the paper clash. The study is like Smart-Bin, which is based on Firebase, a mobile application and implements role-based views and priority-driven collection. No weight sensing and no tilt sensors and no display on the bin.

ESP32 Smart Dustbin with Weight and Level Monitoring (Yogesh et al., 2025)

Yogesh et al. (2025) created an ESP32 dustbin that includes an HX711 load cell and an ultrasonic sensor, which is called “Eco Bin”. Weights and fill percentage are sent to Firebase Realtime Database and a phone app, designed with MIT App Inventor, for full-bin alerts, and a 16×2 I2C LCD displays the alerts on the bin. However, since the paper doesn't report any actual results, it is here as a hardware precedent, not as a performance metric.

It's almost the same as in Smart-Bin, which is ESP32, load cell, ultrasonic sensor, Firebase, Android app and local display. Smart-Bin brings in air quality, tamper detection, a priority list and a web dashboard.

RFID Access Control and Blynk Integration (Karthi et al., 2025)

Karthi et al. (2025) developed ESP32 dustbin with servo lid which will open only for the authorized RFID user. A fill level sensor (ultrasonic sensor), a metal detector, and an air-quality sensor (MQ135) are used to perform measurement checks, with the data sent to the Blynk platform. Abstractly, the authors state 95% fill level accuracy, 92% RFID authentication reliability, 40% less manual work and 35% more efficient collection.

It is the only one study, which can open the bin, that is found. On the collecting side, however, Smart-Bin provides control via a PIN collect mode and remote lock, plus weight, tamper detection and a priority list. The web dashboard is not available for this study.

C. Collection Routes and Priority

Smart-Bin Collection as a Travelling Salesman Problem (Alnuaimi et al., 2024)

The paper by Alnuaimi et al. (2024) from University of Sharjah, UAE, considers the problem of municipal waste collection as a travelling salesman problem (TSP). The fill level of the bins is reported and the route is planned using the Nearest Neighbor heuristic within an AnyLogic simulation. The abstract claims that there are savings in travel distance and carbon emissions when compared to traditional static routes, but it does not provide figures and there is no description of hardware. The work is not a field deployment, but rather a simulation.

The results of this study is where the Smart-Bin ends. It is a list of bins sorted by fill and sorted by time to fill; and a planner such as this one might use that list as input.

Waste Management Optimization Through IoT and Analytics (Kuzhin et al., 2024)

According to Kuzhin et al. (2024), a case study of IoT sensor real-time data and analytics for route optimization has resulted in a 20% reduction in overfilled bins, a 15% reduction in collection frequency, a 25% reduction in the distance traveled, a 10% reduction in fuel consumption, and a 15% increase in recyclables collected. The abstract does not identify a location, sample size, hardware, time frame, and fails to specify whether the values are from a field experiment; the evidence is weak.

The study shows the size of the gain that people expect from collecting by priority. It gives no design that Smart-Bin could borrow.

Route Optimization With GPS and RFID Bin Locations (Banciu et al., 2025)

Banciu et al. (2025), from Lucian Blaga University of Sibiu, collect bin locations with GPS and RFID, build distance matrices and compare the Nearest Neighbor heuristic, Google OR-Tools and a genetic algorithm on three real routes with 735 locations. The genetic algorithm did best, with up to 52% shorter travel distance and more than 14kg less CO₂. RFID here only records the location of a bin when it is emptied, and the study senses no fill level.

It sits entirely on the routing side. Smart-Bin supplies the part that this study takes for granted, which is knowing which bins are worth a visit now.

IoT E-Tracking System for Waste Management (Gupta et al., 2025)

Gupta et al. (2025) propose an IoT system in which bins carry an ultrasonic fill sensor, a microcontroller (Arduino-based, going by the figure captions) and a Wi-Fi module that sends data to an unnamed cloud platform for on-demand routes. They say that in a 30 day trial with 50 bins they achieved 58.9% less distance, 59.2% less fuel, 95% less overflows and 99.8% uptime..

These are some of the highest figures in the entire review, and no pilot dates are mentioned, and no method of routing nor protocol for accuracy tests, so as a result this is what the authors report, but not an independent result. Only level sensing. Smart-Bin builds upon the same level reading with weight, air, tilt and user interface.

Ant Colony Route Optimization With Machine Learning (Kumar et al., 2025)

In Kumar et al. (2025), a system is designed where ultrasonic sensors are mounted on ESP32 boards to send fill level data to a central database. On one hand, Ant Colony Optimization is used to plan routes from the depot to the depot, only stopping at the bins that are filled; on the other hand, machine learning is used to forecast waste from past data; and the collection teams are provided with a Folium map as an interface. The abstract does not provide any actual figures, so there is no actual saving in fuel or cost that can be quoted from the abstract.

Smart-Bin also has the ESP32, the ultrasonic fill sensing and the concept of only going to bins that need it. It sets that priority from the fill and the time to full and not from a learned model. Weight, air quality and tamper detection and phone app not included in this study.

TrashTrack Collection Ranking (Sadikin et al., 2025)

Telkom University (Bandung) Researcher, Sadikin et al., (2025) developed an Android application using Kotlin and Jetpack Compose called TrashTrack. ESP32 bins are equipped with ultrasonic, infrared and GPS sensors and upload the collected data to a Firebase database; a modified constraint-based greedy algorithm is used to rank the collection based on the distance of the bin, vehicle capacity and fill level. The results of the usability test with 69 respondents showed that 92.37% were effective, 92.29% were useful and 89.92% were satisfied.

Smart-Bin is powered by the same ESP32, Firebase and a kotlin android app. It ranks by fill and time to full (no dist or vehicle capacity – it doesn't plan roads). Weight, Air Quality, Tamper Detection and Display on bin are not included in this study.

D. Sorting and System Design

Camera-Based Fill Level Detection (Enache et al., 2024)

Figure 32

Power Chain of the ESP32-CAM Node

Note. From “Intelligent trash level detection for sustainable urban waste management,” by B.-A. Enache, C.-K. Banica, and A. G. Bogdan, 2024, The Scientific Bulletin of Electrical Engineering Faculty, 24(1), 56–59. CC BY-NC-ND 4.0.

Enache et al. (2024), from the University Politehnica of Bucharest, estimate the fill level from photographs. Figure 32 shows the power chain of one node. A 5V solar cell feeds a TP4056 charger, which charges a 3.7V LiPo battery of 850mAh, and a step-up converter brings the voltage back to 5V for the ESP32-CAM. The battery was sized for 24 hours of operation. The fill ratio is estimated by a Python script with OpenCV and an SQL database, each node takes one image per hour and sends it over an ESP-MESH network to a server.

On five test bins the relative error of the estimated fill ratio ran from 11.43% to 30.00%, and the authors name the angle of the camera, the light, the kind of waste and the placement of the bag as causes. The figure matters to Smart-Bin because it shows what a stand-alone node needs between the source and the board: a charger, a battery and a converter. Smart-Bin builds the same chain with its battery shields and 18650 cells. It is not camera based, it is based on distance and weight and this study has no weight, air quality, tamper detection, or phone app.

AI-Based Waste Segregation With Monitoring (Muthukumar et al., 2024)

At M. Kumarasamy College of Engineering in Karur, Muthukumar et al. (2024) developed a bin fitted with a camera that can identify plastic, glass, metal and paper using AI and place them in separate compartments. A mobile application displays the level of fill, plus the temperature and humidity levels, and notifies cleaning personnel and allows for one team to use the application to claim each task. The number of images used is about 1500 images per class and the paper reports 71.8% training accuracy and 92.3% test accuracy, but does not explain the difference between these. The microcontroller and the cloud service are not named.

Segregation is outside the scope of Smart-Bin. The claim-a-task feature of the app echoes the collector role in Smart-Bin, and the study has no weight, air quality, tamper detection or collection priority.

Smart Trash Bins With IoT and AI (González et al., 2024)

González et al. (2024) constructed a bin made of ESP32 and ESP32-CAM. This includes ultrasonic sensors that measure fill level, an AI that detects waste from images from the camera, and potentiometers that replace the weight sensor. Events are captured by readings, which are transmitted over Wi-Fi to a central server and displayed on Oracle APEX dashboards; and an MIT App Inventor app is used to manually record the status. The abstract does not contain any accuracy figures.

The weight is simulated and the study indicates a dashboard design without any actual weighing. Smart-Bin offers a real load cell, and always offers a simulation as an option for testing, all set from the interface. This study has no air quality sensing, tamper detection or priority list.

Smart Waste Classification with IoT and Deep Learning (Alourani et al., 2025)

Alourani et al. (2025) propose iSSWMs, a framework that joins an IoT smart bin to a deep-learning classifier. The bin has an ultrasonic level sensor on an Arduino Nano and an ESP8266 that sends its status to Firebase, and an Android app alerts the collectors at a threshold. A VGG-19 classifier sorts plastic, glass and metal and reached 99.7% accuracy on about 1,530 TrashNet images over 100 epochs. That figure is training accuracy, and the authors report no field test.

Smart-Bin shares the ultrasonic level, Firebase and an Android app with alerts for collectors. Image classification is outside its scope, and this study has no weight, air quality, tamper detection or priority list.

ESP32 Smart Bin That Separates Dry and Wet Waste (Gajula et al., 2025)

Gajula et al. (2025) developed an ESP32 dust bin which detects the wetness of the waste deposited and separates dry and wet waste with the help of SG90 servo and MG90 servo. The HC-SR04 ultrasonic sensor and the moisture sensor provide information to the Arduino IoT Cloud that triggers the separation mechanism. Abstract does not provide any numbers that were measured.

It's not about monitoring the bin, it's about sorting, and shares only the ESP32, a servo and the cloud link with Smart-Bin. No weight measurement, tamper detection or phone application.

FreeRTOS-Based Smart Waste Management (Aasish et al., 2025)

Aasish et. al (2025) presents a system on a dual core ESP32 based on FreeRTOS. It has an ultrasonic sensor to track the level of the bin, as well as a DHT11/DHT22 to measure temperature and humidity, it has an optional LCD or OLED display and can upload to a cloud server via Wi-Fi for alerts and dashboards and dynamic collection scheduling using RFID modules to identify each bin. The basic concept is to separate the sensor acquisition and actuation tasks, from the cloud communication tasks, so that a slow upload does not block a reading. No actual results are stated and the RFID is not used for access control, it is merely used to identify bins.

The task split is a design to consider for the Smart-Bin firmware, as it is also based on the ESP32 family. The study has no weight sensing, tamper detection or phone app.

Commercial Systems

Commercial systems are available. Bigbelly's CLEAN software provides the data from the bins on a console and uploads it to a collector app (Bigbelly, n.d.). According to its website, Ecube Labs offers ultrasonic fill sensors and a cloud platform that provides fire event notification and route optimization, and claims to predict bin status up to 24 hours in advance (Ecube Labs, n.d.). Sensoneo's platform displays a map of each bin's fill and predicted full time, and plans routes from them (Sensoneo, n.d.), Nordsense's driver app with dynamic routing (Nordsense, n.d.), and Evreka's sensor enhances temperature and motion to flag fires (Evreka, 2020). Compology monitors containers with cameras (Compology, n.d.), and Bin-e is a segregating bin with a touch-free lid (Greenwalt, 2020). In the Philippines, students of the University of the Cordilleras built the Baraki bins for the DOST-TAPI Regional Invention Contest, which open automatically and report to an information center (Asido, 2024). Table 3 summarizes what the vendor pages state.

Table 3

Commercial and Student Systems as Described by Their Own Pages

| System | Sensing described | Who sees the data | Collection priority or route |
| --- | --- | --- | --- |
| Bigbelly | Fill level, collection events and GPS; sensor type not named | Management console and collector app | Not stated |
| Ecube Labs | Ultrasonic fill level, battery level and fire events | Cloud dashboard | Route optimization; predicts status up to 24h ahead |
| Sensoneo | Ultrasonic or radar fill level | Dashboard with a map and predicted full time | Semi-automated route planning |
| Nordsense | Page covers routing, not sensors | Driver app | Dynamic routing |
| Evreka | Ultrasonic fill level, temperature, motion and location | Web platform with SMS and email alarms | Route optimization |
| Compology | Camera: fullness, location and motion | Web dashboard | Not described |
| Bin-e | Fill level, motion for the flap and waste type | Mobile app | Not covered |
| Baraki | Automatic sensors; types not stated | Information center | Not stated |

Note. Sources: Bigbelly (n.d.), Ecube Labs (n.d.), Sensoneo (n.d.), Nordsense (n.d.), Evreka (2020), Compology (n.d.), Greenwalt (2020) and Asido (2024).

All of them sell via quotation: none of these pages mentions weight, gas or tilt sensing, or a price. That doesn't mean that the products don't have these characteristics, just that vendor pages are concise and designed to sell. What it shows is that a small system with those features at about ₱4,900 for two bins (Appendix B) occupies a different place from these services. Smart-Bin does not compete with them on scale, and it cannot, but it can show what a bin that senses more costs to build.

### Technical Merit

Seven design choices in Smart-Bin decide how much its readings can be trusted. Table 4 gives the reason for each. Each one is also a place where a measured result could differ from the real condition of the bin, which is what the tests of the project will have to check.

Table 4

Technical Basis of the Main Design Choices

| Design choice | What Smart-Bin does | Why it is sound |
| --- | --- | --- |
| BME680 for the air check | One I2C chip gives temperature, humidity, pressure and a gas resistance reading, from which the firmware estimates an air-quality index. | In Table 2, the studies that sense gas use MQ-series modules, which only provide a reading for the gas. Bosch Sensortec (n.d.) states that the BME680 uses 0.09 to 12mA during its different modes. The index is an estimate and needs about 5 minutes to settle. |
| Median distance reading is calculated from five distance readings. | The firmware makes five measurements using the HC-SR04 and uses the middle measurement. | When the echo bounces off an edge or soft material, a single reading from the ultrasonic can be spiked. One or two of such readings will be ignored without delaying the update, this is the median. |
| Two-point calibration | The full distance (5cm by default) and empty distance (30cm by default) are defined from the interface and the fill percentage is the distance between them. | The two setpoints are the same as in Keller et al., 2024. Other heights of bins do not require new firmware. |
| Weight as a second check | The HX711 and a 5kg load cell weigh the contents next to the fill percentage. | Level and weight are also combined in Saad et al. (2023) and Maulida et al. (2025). A loose pile in a bin is light but a full bin is heavy. |
| Tilt with a countdown | If the tilt is over 30° a countdown of 10s begins. When the bin is steady again the alarm will clear, or if not, the alarm will latch and need to be reset by an operator. | Countdown will not allow for a bump or moving bin to cause a false alarm and an overturned bin or carried away bin will still sound. |
| Local and cloud together | The bin responds on its local WiFi within ~1s, and to Firebase and ThingSpeak. | Fast control and calibration with local path, history, remote commands and alerts when phone is away with cloud path. The data is uploaded every 30s, which is half the frequency ThingSpeak can accommodate on its free plan of 1 write every 15s |
| Calibration and simulation from the interface | Distances, limits can be changed, without re-flashing, any sensor can be set to a test value. | None of the 30 studies describes this. It lets a test be repeated at a chosen reading and keeps demonstrations safe. |

Note. Defaults are those set in the firmware and can be changed in the settings.

### SWOT Analysis

The strengths, weaknesses, opportunities and threats of Smart-Bin are presented in Figure 33. The strengths and weaknesses are related to the construct itself, while the opportunities and threats are related to the literature mentioned above and to the situation in the Philippines. The count of studies that sense tilt or offer role-based views comes from this review. RA 9003 puts segregation and collection at the barangay level and leaves the collection of non-recyclable and special waste to the municipality or city (Ecological Solid Waste Management Act of 2000, 2001), and the World Bank (2022) puts the share of Metro Manila waste that is collected at about 60%.

Figure 33

SWOT Analysis of Smart-Bin

The weaknesses set the limits of what the tests of this project can claim. The tests demonstrate that it works on a bench, but not that the prototype can last on the street for 18 hours in Bin A and 9 hours in Bin B and that it is a table-size prototype that has never been outside. The opportunities are on the other side, the weaknesses: pilots on campus or in a barangay hall, LoRa for where there is no Wi-Fi, solar power and road routing.

### Synthesis of the Review

The patents, studies and products reviewed above converge on four points. First, the problem is a documented one. The Ecological Solid Waste Management Act of 2000 (2001) leaves the collection of non-recyclable and special waste to the municipality or city, and the World Bank (2022) puts the share of Metro Manila waste that is collected at about 60%. A bin that overflows between collections is therefore an expected condition of the system that these sources set out to monitor.

Second, sensing the fill level is a solved problem. It is measured by 27 of 30 studies, often using an HC-SR04: Patents have done it with hydraulic pressure (Neumann & Allison, 1991), capacitive plates (Di Giugno et al., 2016), an infrared beam (Devitt & Walsh, 2021) and an ultrasonic sensor (Kekalainen & Engstrom, 2018) and a laser (Keller et al., 2024). The commercial services in Table 3 sell the same measurement with a cloud console on top.

Third, the other sensors exist, but they are added one or two at a time. Saad et al. (2023) and Maulida et al. (2025) add weight, Mehta et al. (2025) add gas, Alnanih et al. (2025) add a flame sensor and Azmi et al. (2024) add temperature and humidity. There is only one reference by Mehta and Rathour (2024) who mention a tilt sensor but do not mention any alarm. In the patents, Mullaney (2016) weights contents, Kurani (2023) mentions around a dozen sensors, but one is based on litter and pathogens, not the user of the bin.

Fourthly, the work is directed more into the route than into the bin. Twelve of the 30 remaining studies report on routes or priority of collection, and have the biggest improvements: up to 52% shorter distance in Banciu et al. (2025) and 58.9% in a pilot of Gupta et al. (2025) with no date or method provided. The bins in these studies stay simple, ten studies have a phone app, two give different users different views, and none of them describes calibration or simulation from the interface of the bin.

None of the sources brings these pieces together in one bin. The studies that weigh the waste have no tilt alarm, the one that lists a tilt sensor describes no alarm, the ones with phone apps and roles have no weight, and the route studies take the readings of the bin for granted. Table 5 sets the four closest patents against Smart-Bin, feature by feature, and Figure 34 counts the features found in the studies. None of the twenty patents, as far as their text was read, combines a latched tamper alarm, a PIN collect mode on a touchscreen, a remote lock and sensor simulation and calibration from the interface with fill, weight and air sensing in one bin. A search of this size cannot prove that no such patent exists, so this is a statement about what was found and not about what exists.

Table 5

The Four Closest Patents Compared With Smart-Bin

| Feature | Shahabdeen (2016) | Yan and Chen (2021) | Kurani (2023) | Sait et al. (2025) | Smart-Bin |
| --- | --- | --- | --- | --- | --- |
| Fill level | Yes (range finder) | Yes (infrared height) | Yes | Yes | Yes |
| Weight | Yes (or fill level) | Yes | Yes | Not found | Yes |
| Gas or odor | Yes | Yes | Yes | Yes (methane) | Yes |
| Temperature and humidity | Not found | Not found | Yes | Yes | Yes |
| Tilt or tamper alarm | Not found | Not found | Accelerometer listed | Not found | Yes |
| Motorized lid | Not found | Not found | Not found | Not found | Yes |
| Remote lock or PIN collect | Not found | Not found | Not found | Not found | Yes |
| On-bin display | Not found | Not found | Not found | Not found | Yes |
| Phone alert | Not found | Not found | Yes | Not found | Yes |
| Priority or route | Not found | Yes (truck route) | Yes (route) | Yes (route) | Yes (priority) |

Note. Counts are derived from the hardware and the function described in each study (Table 2).

Figure 34

Features Found in the 30 Related Studies

Note. Counts come from the hardware and function each study describes (Table 2).

### Research Gap

There are 30 studies; 27 sense fill level. None has all of weight, gas, temperature, humidity and tilt in one bin; 6 add weight, 6 add gas, 4 sense temperature/humidity and 1 senses tilt. Two studies are providing role-oriented perspectives, one of which is using an RFID-based access control mechanism. None indicates simulation / calibration from the interface. The bin indicates its fill level, and the focus is on routes and fleet software in the patents and commercial systems. None of the four most closely related patents have all ten of the features listed in Table 5.

So, it's not a sensor, it's a gap. These factors: fill, weight, air and tilt sensing, alerts to the operator, collector and person at the bin, collection priority from the fill and time to fill, interfaces to allow bin calibration and testing without need to re-flash, are all combined in one single low-cost bin. The gap by capability and the specific objective in Section 1.4 which addresses it is provided in Table 6.

Table 6

The Research Gap by Capability

| Capability | Related studies | Patents and products | Smart-Bin | Objective |
| --- | --- | --- | --- | --- |
| Sensing breadth on one board | 27 fill, 6 weight, 6 gas, 4 temperature or humidity, 1 tilt; none has all | Kurani (2023) lists about a dozen sensors; the others are narrower | Fill, weight, temperature, humidity, pressure, air quality and tilt on one ESP32 | SO1 |
| Tamper detection | One study lists a tilt sensor | No latched tamper alarm found in the four closest patents | Tilt beyond 30° starts a 10s countdown, then a latched siren until RESET TAMPER | SO1 |
| Alerts to each kind of user | 10 studies have a phone app and 7 a web dashboard | Phone alerts in Kurani (2023) | Buzzer, LED and touchscreen on the bin, web dashboard, Android app and push alerts | SO2 |
| Roles and access | 2 studies have role-based views; 1 uses RFID | None found | Operator and collector views, remote lock, PIN collect mode | SO2, SO3 |
| Collection priority | 12 studies are about routes or priority, mostly with GPS and algorithms | Route planning in the closest patents | Fullest first, with a time to full from the recent fill rate; no road routing | SO3 |
| Testing and demonstration | No study describes simulation or calibration from the interface | None found | Per-sensor simulation and calibration from the dashboard and app | SO3 |

## 1.4 Objectives

### General Objective

To design and develop Smart-Bin, an IoT based Waste Bin Monitoring System which will monitor the fill level, weight and air quality of the waste bin, detect tampering and give real time monitoring and alerts to the concerned entities via Web Based Dashboard, Android App and On-Bin Touchscreen.

### Specific Objectives

- Sensor Monitoring and Detection

To develop 2 Smart-Bin units, which include sensors to measure fill level, weight, temperature, humidity and air quality, as well as to detect bin tilting and tampering and open the lid automatically when hand movement is detected.

- Real-Time Monitoring and Alerts

To develop a web dashboard and an Android application to display real-time data from the sensors, bin status, retrieve data from Thingspeak/Firebase and notify the operators and collectors when the bin is full, poor air quality, high temperature, tampered sensor reading and when the bin is not online.

- Bin Control and Collection Prioritization

To implement Collection Prioritization, based on bin fullness and estimated time to fullness, remote bin locking and unlocking, on-bin collection functions (via PIN), sensor simulation and system interface sensor calibration. This will lead to the prioritization of collections and will also help with efficient collection routes and fuel gas efficiency.

## 1.5 Conceptual Framework

The framework of Smart-Bin is shown in two parts. The first one which is below describes the process of a reading going from the sensor to the person who is going to respond to it and it is known as the Input-Process-Output (IPO) model. The second, under IoT Level, shows how the parts are deployed and connected. Both refer to the same system and both result in the three specific objectives listed in Section 1.4 above.

### Input-Process-Output Model

Figure 35

Input-Process-Output Model of Smart-Bin

The input column has seven items. The distance to the trash is measured by the HC-SR04 sensor, the weight is measured by the HX711 sensor (which includes a 5kg load cell), the BME680 sensor provides temperature, humidity, pressure and an estimate of the air quality, the BMI160 sensor gives the tilt, and the IR sensor detects a hand. The final two inputs are from the people: commands input on the touchscreen, in the dashboard or in the app, and the settings, which contain the calibration and simulation values. All of these inputs are related to Specific Objective 1, and commands and settings are related to Specific Objective 3.

The process column is how the ESP32 firmware and the cloud use them. The firmware takes the median of five readings, converts the distance to a fill percentage from the calibration limits, compares the readings with their thresholds, runs the tamper countdown and applies the lock. Local REST API provides response in ~1s and status is stored in Firebase while the commands are stored in thing speak where the history is stored. The apps rank the bins by fullness, by time until they are full and give the operator or collector views. Specific Objectives 2 and 3 are addressed in this column.

Live status on the touch screen, the dashboard and the app, alerts for a full bin, bad air, heat, tamper, or bin being offline, Collection Priority list, live status for the lid, lock, and PIN collect on the bin, history charts, and remote control, are all things people see and do, which is the output column. The result is reduced overfilling bins and timely collection, which is the problem stated in Section 1.1. Feedback closes the loop; the results of the test and the operator's calibration will improve the limits and calibration values used by the firmware next.

### IoT Level

Figure 36

System Architecture of Smart-Bin

The connections between the parts of Smart-Bin are shown in Figure 36. The readings are sent from the cloud to the two apps at the top: the web dashboard (primarily for the operator) and the Android app (primarily for the garbage collector). Both apps can also reach a bin directly over the local Wi-Fi in about 1s for any command, calibration, or simulation.

There are two end nodes at the bottom: Trash bin A and Trash bin B. All are equipped with the same ESP32 and six input sensors (HC-SR04, IR sensor, BME680, BMI160, load cell, touch panel) and four output actuators (servo, RGB LED, buzzer, TFT). Bin A is powered by 4x 18650 cells, Bin B by 2x.

Each bin sends its sensor data up to two cloud services and receives the LOCK, UNLOCK and COLLECT commands from them. Firebase is the primary cloud and stores the live status, ThingSpeak stores the history and charts and relays commands via TalkBack.

Smart-Bin fits IoT Level 6 of the deployment templates in Bahga and Madisetti (2014). A Level 6 system has several independent end nodes that sense or actuate and send their data to the cloud, where it is stored and shown by a cloud-based application. An analytics component works on that data and keeps its results in the cloud as well, and a centralized controller knows the status of every end node and sends control commands back to them. Trash bin A and Trash bin B are the end nodes in Figure 36. Both have their own ESP32, they read their own sensors and update their own status to Firebase and ThingSpeak, they don't depend on each other and there is no gateway between them and the cloud.

Cloud side is Firebase for live status, and ThingSpeak for stored history. ThingSpeak keeps the readings of every bin and plots and analyses them over time, and the apps use the same data to rank the bins by fill level and estimated time to fill in the Collection Priority list. The web dashboard and the Android app take the role of the centralized controller. They show the state of both bins side by side and send LOCK, UNLOCK and COLLECT back through the cloud, and calibration and simulation commands go to a bin directly over the local Wi-Fi. Both read the lock, collect, calibration and tamper flags that the bin itself reports, so a bin locked or put into calibration from one of them shows the same state and the same buttons on the other.

## 1.6 Target Beneficiaries

- Waste Collectors and Utility Staff

Smart-Bin will assist waste collectors and utility workers by notifying them which bins are fullest and need to be emptied first. It could also prevent visits to bins that are not full and offer PIN access to the bin during collection.

- Facility Managers and Operators

The web dashboard can be used by facility managers or barangay hall, school, office or mall operators to monitor the status of the bins, make assessment of the data and set thresholds for the sensors, apart from the messages they receive on overflowing bins, poor air quality, high temperature, and even possible tampering.

- Barangay and Local Government Unit (LGU) Sanitation Offices

The sanitation offices of barangays and LGUs can utilize Smart-Bin's data readings to aid in garbage collection scheduling and management. The percentage of fullness and time to be full will help them optimize their collection routes and minimize the amount of fuel they need to use by making fewer unnecessary collections.

- Filipino Citizens

How about if the environment is cleaner, trash bins are not overflowed and there is increased awareness about waste among Filipino citizens who are residents, employees or students of schools, offices or other places? The hands-free lid would also improve the disposal of waste; making it easier and sanitary.

# II. PROTOTYPE DESIGN AND DEVELOPMENT

## 2.1 Development Methodology

Smart-Bin was developed with a hybrid software development life cycle (SDLC) called Water-Scrum-Fall. Samolovac (2023) describes it as a combination of Waterfall and Scrum in which the project begins and ends with Waterfall, while the execution phase is done in iterative sprints until the final deliverable is complete. Figure 37 shows the model as the group used it. Each phase is labeled with what it covered in this project and with its dates from the Gantt chart in Appendix A.

The course decided the two ends. The topic, objectives and chapters had to be settled in order, and the paper and presentation are due on November 6, 2026, so neither the start nor the finish could move. The middle was different. One unit joins an ESP32, five sensors, a servo, a touchscreen, a web dashboard, an Android app and two cloud services, and a fault in one part often showed only after another part was connected to it. Planning that stretch once and following it to the end would not have survived the first built bin.

Figure 37

Water-Scrum-Fall Development Life Cycle of Smart-Bin

Note. Adapted from What Is Hybrid Project Management?, by J. Samolovac, 2023, Plaky (https://plaky.com/learn/project-management/hybrid-project-management/). The phase dates are from the Gantt chart in Appendix A, and the colors follow its parts.

### Water: Requirements and Planning

Requirements ran from October 14 to 17. Banogbanog and Reyes wrote the objectives, the SDG and the target beneficiaries, while Banogbanog and Celeste wrote the introduction and the problem. Their output was the general objective and the three specific objectives in Section 1.4, which the group treated as fixed. Each specific objective later became one group of test cases in Chapter III: TC1.1 to TC1.6 for sensing and detection, TC2.1 to TC2.6 for monitoring and alerts, and TC3.1 to TC3.6 for bin control and collection priority.

Planning followed from October 17 to 23. Gunayan and Pepito reviewed the related patents and the studies from 2023 onward, wrote the SWOT analysis and the research gap, and built the IPO model and the IoT Level 6 framework in Section 1.5. The plan behind the build came from the same work. The implementation plan fixed the parts, the local REST API of the bin and the cloud channels, and the Gantt chart fixed who did what and by when. From October 21 to 23 the whole group merged Chapters I and II and checked them against APA 7.

### Scrum: Design, Development and Implementation Sprints

The middle of the project ran as sprints. A sprint went through design, development and implementation, put the result on a real board or bin, and carried what it found back into the next design. The first designs were made from October 18 to 23: the block diagram by Reyes, the component materials by Celeste, the system flowchart by Reyes and Celeste, and the prototype design by Banogbanog. They appear in this chapter as Figures 38 to 56.

The Gantt chart gives development the window from October 28 to November 1, with Reyes on the firmware and the dashboard and Celeste on the Android app. The code itself started sooner. Working firmware, the live web dashboard and version 2.0 of the app were in the project repository by October 17, and app version 2.2 followed on October 18. Until a bin existed, the two apps could only be tried on demo data, so the bar on the chart marks the time in which they had to work on real hardware.

Implementation, the build and wiring of the two bins, was given to Banogbanog and Gunayan for the same window. The first bin was built and tested on October 28, and that test started the busiest sprints of the project. The tamper alarm went off soon after power-up, because the upright angle saved earlier no longer matched the bin. The fix went back through design and development the same day as app version 3.0: the bin now saves its upright angle at every power-up, the lid stays shut for 2s after it closes so it does not flicker, and the bin reports locked, pickup, calibrating and tamper as separate flags so the touchscreen, the dashboard and the app show the same state. The cloud test on Bin 01 on October 29 found two more problems. ThingSpeak carried only a short status word, which version 3.1 replaced with the full set of flags, and the lid kept reopening after it closed because the servo retriggered the PIR sensor. On October 30 the PIR sensor was replaced with the FC-51 IR obstacle sensor, and both bins were finished that day. The drawing in Figure 38 was corrected once more on October 31, after the built bins showed that the electronics sit in a bay behind the front wall.

### Fall: Integration and Testing

Integration joins what the sprints produced: the two bins, the web dashboard, the Android app, Firebase and ThingSpeak. On the Gantt chart it runs from October 26 to November 1 and overlaps the last sprints, because every sprint already ended with a check on the connected system. The Bin 01 cloud test is one such check, with the bin signed in to Firebase under its own account and writing to ThingSpeak channel 3556235. In the model, integration is where the loop stops. Once the bins and both apps carry the same version, changes are limited to fixing faults.

Testing closes the project as a single Waterfall pass. The Gantt chart sets November 1 and 2 for it: Banogbanog and Reyes write the test cases from the three specific objectives, and the whole group runs them and records the results that Chapter III reports. The final review, the PDF and the slides take November 3 to 5, and the presentation is on November 6.

The method had a cost on paper. Every sprint that changed the bin also changed this document: the system and operator flowcharts were redrawn on October 18 and again on October 28, and the prototype drawing on October 31. The fixed requirements kept that cost bounded. The sprints changed parts, settings and screens, but the three specific objectives stayed the same, so the test cases written from them still applied at the end.

## 2.2 Prototype Design

The physical design of one Smart-Bin unit is shown in Figure 38. The group builds the same unit twice, as Bin A and Bin B, and the only difference between the two is the battery shield: Bin A runs on four 18650 cells and Bin B on two. Each unit is a table-size model, about 30 to 40cm tall. Inside, a divider wall stands a short distance behind the front wall and splits the bin into two parts. The larger part at the back is the trash chamber, which holds the lid and the sensors that have to face the trash. The narrow bay behind the front wall holds the electronics: the perfboard with the ESP32 and the battery shield, so neither the board nor the power supply touches the trash.

Figure 38

Front View and Side Cross-Section of One Smart-Bin Unit

The front view (a) shows what a user sees first. Near the top is the FC-51 IR obstacle sensor on GPIO 27. When it sees a hand, the SG90 servo lifts the lid, which closes again after the open time the operator sets (5s by default). Once shut, it stays shut for a close time (2s by default), so a hand still in front of the sensor does not make it flick open again. Below it is the 2.4-in. touchscreen (ILI9341 with XPT2046 touch), mounted in portrait so that its fill bar rises the same way the trash does. The screen shows the fill level, weight, temperature, humidity and IAQ estimate, and it takes the PIN before a collection. Under the screen are the RGB status light and the MH-FMD buzzer on GPIO 4. The light stays green while the bin is below 50%, turns amber from 50%, and blinks red when the bin is full or tampered with.

The side cross-section (b) shows the inside. The HC-SR04 ultrasonic sensor hangs from a fixed strip at the back of the top, behind the lid hinge, so it stays still when the lid moves. It points down and measures the distance d to the top of the trash; the firmware takes the median of five readings and turns it into a fill percentage. The servo sits on the same strip and pushes the lid open with its arm. The electronics bay sits right behind the front wall, so the touchscreen, the IR sensor, the status light and the buzzer connect to the board through the wall with short wires. In the bay the ESP32 sits on a perfboard, with the BMI160 and BME680 modules soldered on the same board. The board is fixed to the bin, so the BMI160 moves together with the bin body, which the tilt check needs. The battery shield sits in the same bay below the board, and the bin has no charging port on the outside. The trash rests on a platform over the 5kg load cell, and the HX711 amplifier turns the strain into a weight in grams.

## 2.3 Block Diagram

The block diagram of one Smart-Bin unit and the network around it is shown in Figure 39. It has two separate ESP32 microcontrollers, one in each of the two bins, both bins reporting to the same cloud services and user apps.

Figure 39

Block Diagram of the Smart-Bin System

The power supply is located in the top portion of the diagram, consisting of an 18650-battery shield in a closed compartment providing the circuit a regulated 5V. Bin A is powered by 4x 18650 cells and Bin B by 2x. The 5V is fed to the ESP32-WROOM-32D which supplies the 3.3V needed by the sensors, display and buzzer; the servo and HC-SR04 operate directly from the 5V. ESP32 is a 240MHz dual-core Wi-Fi enabled micro-controller. It has firmware written in C++ and uses that to determine the fill percentage, indoor air quality (IAQ) estimate and tamper state, as well as controlling the lid and alarms and providing a local REST API.

The inputs are on the left. The HC-SR04 Ultrasonic sensor on GPIO 5 and 18 can be used to detect the distance between the lid and the garbage bin, and output the fill level information, the FC-51 IR obstacle sensor on GPIO 27 can be used to sense a hand held in front of the garbage bin. Two sensors, BME680 (address 0x76) for temperature, humidity and air quality and BMI160 (address 0x68) for motion and tilt for tamper detection share this I2C bus. Using a 5kg load cell, it is connected to GPIO 14 and 32 via an amplifier module named HX711 to measure the weight of the trash. On the SPI, the touch panel XPT2046 accepts the COLLECT and PIN inputs.

The outputs are to the right. The lid is opened and closed by the SG90 servo on GPIO 13 (PWM). The fill level is indicated by an RGB LED (GPIO 25, 26 and 33) in green, amber or red. Beeps, warnings and the tamper siren come from a passive buzzer on GPIO 4. The fill level and the bin status are displayed on the 2.4 in. ILI9341 TFT display (also SPI).

Three paths of data exit the bin via Wi-Fi (802.11 b/g/n) as shown in the bottom half of the image. The local REST API (/api/status, /api/command, /api/calibrate and /api/simulate) responds in ~1s and provides full control of the bin. The status is sent to the Firebase Realtime Database every 5 to 60s and to ThingSpeak every 30s while the readings change and at least once every 5 minutes, as well as the LOCK, UNLOCK and COLLECT commands (the latter via ThingSpeak's TalkBack feature). Data goes up and commands come down to two user apps. The web dashboard runs in a browser on the same Wi-Fi and gives the operator a live view, settings and simulation, while the Android app gives operators and collectors alerts, the COLLECT command and the Collection Priority list.

## 2.4 System Flowchart

Figure 40

System Flowchart of the Smart-Bin

Note. Figure 40 shows the main program loop that runs on each bin. After start-up, seven parts run side by side on every pass of the loop, and their results meet at one update step.

Upon powering on the bin, the ESP32 starts, checks its sensors and loads its saved settings, with a boot screen and a beep. It also saves the angle it is resting at as the upright position, so the tamper check starts from the way the bin is standing. It then examines if a Wi-Fi network is saved. If not, the user opens WI-FI SETUP on the touchscreen and enters the collect PIN, and the app or dashboard pairs with the bin over Bluetooth. The 6-digit code on the bin's screen is typed into the pairing box (a device paired before skips it, and three wrong codes stop pairing for 60s). The bin returns a list of all the Wi-Fi networks within its range, and the user selects one and enters its password, after which the bin beeps to join. This loop is split into 7 parallel sections (1-7 on chart).

The lid part (1) reads the IR sensor. If the bin is free (not locked, tampered with, collecting or being calibrated), it beeps to open the lid. Otherwise, bin lid remains closed and a beep is heard when a hand is seen with the lid closed. The lid beeps and closes 5 seconds after the hand is gone, and then stays shut for at least 2 seconds before a hand can open it again. The fill part (2) reads the weight from the HX711 (or reports that the HX711 is not answering when it is unplugged), and takes the median of 5 HC-SR04 readings for the fill percentage. At the default setting of 80%, the LED flashes red and an alert message is sent every 30s until the bin is collected (or locked) when the fill is at the limit. When set to auto-lock, the bin itself locks as well. If lower than the limit the LED is green or yellow. The air part (3) compares the measured temperature, humidity and estimated air quality of the BME680 with their limits (40C, 85% and an index of 200). The air quality is an approximation based on the gas reading and should be warmed up for approximately 5 minutes. When over a limit the buzzer sounds and an alert is sent every 5 minutes, whilst a short beep indicates when the reading is back under.

The BMI160 tilt angle is monitored by the tamper part (4). If the tilt from the upright position saved at power-up exceeds 30°, a countdown of 10 seconds begins with a "tick" every second. The alarm automatically resets if the bin is steady once more. Until an operator sends RESET TAMPER, the alarm latches when it remains tilted: the siren sounds every 2s, the LED is red, the lid remains closed and an alarm message is generated. The screen part (5) is used for handling the COLLECT touch command. Once collect PIN is entered lid will open and alarms will be muted until DONE is pressed and the lid is closed and the fill level is reset, or if the collect PIN is wrong, an error beep is provided. Finger on screen for 5s to start touch calibration. The command’s part (6) receives the following commands from the app or dashboard: LOCK, UNLOCK, COLLECT, calibration (CAL) and simulation (SIM). Bin beeps if an unsafe one is attempted such as COLLECT during tamper alarm and CAL and SIM are Wi-Fi only. The network part (7) beeps once when the Wi-Fi is lost and once when it returns.

All seven paths then lead to an UPDATE step. The two arrows between this step and each cloud reflect that the reading and the alerts are sent to Firebase and the ThingSpeak, and the LOCK, UNLOCK and COLLECT commands are returned to it. This new status is then reflected on the screen, on the LED and in the apps and the process is repeated.

Figure 41

Operator Flowchart of the Smart-Bin Apps

Note. As seen in Figure 41, an operator can do the following actions in the Web Dashboard or in the Android App

.

In both apps, the collector mode is the default, and the operator enters his 4- to 8-digit PIN. A wrong PIN requires a delay between attempts, with 30s after 5 failed attempts and the delay for subsequent failed attempts doubling. The app puts itself in operator mode with the appropriate PIN. The operator can then add or edit a bin, giving it a name, a place, the IP address and cloud name for Firebase and ThingSpeak, and the bin is saved in the app.

There are five tasks branching out from there. Everything that a collector can do is Collector (1), as shown in Figure 42: alerts, COLLECT and DONE, LOCK and UNLOCK, charts and history, and setting up the Wi-Fi over Bluetooth. Tamper (2) is for a bin with an alarm that latched. The operator checks the bin and stands it up and then sends a RESET TAMPER to clear the alarm. Calibrate (3) requires bin to be on same Wi-Fi. The operator sets new limits (fill range, alert limits, BME680 offsets, the BMI160 upright angle, lid angles and open and close times, weight tare and more) and these limits are stored in the flash memory of the bin. Calibration is disabled if the bin is not connected to Wi-Fi. The bin reports when it is in calibration mode, so turning it on from the dashboard, the app or the bin’s screen shows on the other two as well. The touch calibration (4) can be initiated from the dashboard or app or by holding a finger on the bin's screen for 5s. Operator presses four crosses sequentially and result is stored on the bin (if pressed too close, it is a fail and the operator retries).

Simulate (5) keeps the value of any sensor for demonstration purposes. A SIMULATION banner will appear on the screen while it is on until the operator returns everything to AUTO. The sound, vibration and theme are set by Settings (6) and the operator can change the PIN in the settings after entering the PIN. The result of whichever task is used then iterates through the same 2-way UPDATE process with both the Firebase and the ThingSpeak services and the app will display the new status, plus the charts and alerts.

Figure 42

Collector Flowchart of the Smart-Bin Apps

Note. As seen in Figure 42, the collector's job in the web dashboard or in the Android app.

The apps are opened to collector mode which is the default mode and no pin is required. The collector uses Bluetooth to set up the bin if not already connected to Wi-Fi. The 6 digit code displayed on the screen of the bin is entered into the pairing box of the phone (code will be valid for 60s, 3 failed attempts will lock pairing for 60s, a device that has been paired before will not require a code). The app then lists the Wi-Fi networks the bin can see, and the collector picks one and enters its password. After the bin joins the network, there are four tasks available.

Alerts (1) are issued for a full bin, tampering, bad air, lock and pickup. Each has its own sound and vibration and they come even when the app is not in use. The primary task is collect (2). The Collection priority list is a ranking of the bins, with the first bin in the list being the one with the most capacity, and the time to fill each bin is shown. The collector taps COLLECT in the app or on the screen, and enters the collect PIN at the bin. Lid opens and alarms stop. The collector then empties the bin and taps DONE (no PIN required) and the lid will then close and the fill level will reset. A bin refuses COLLECT, with a beep, while a tamper alarm or a calibration is on.

Lock (3) allows the collector to lock or unlock a bin, while resetting a tamper alarm, calibration and simulation stay with the operator. View (4) displays the charts, alert history, and status and collect PIN in read-only mode. All tasks are completed with the same step in Firebase and ThingSpeak and the app updates the status.

## 2.5 Schematic Design

The circuit of each unit is drawn as one schematic split into three sheets: the microcontroller, the actuators and outputs, and the sensors and inputs. The order of the sheets is: Figures 43, 44, 45. Instead of long wires between sheets, each connection to the ESP32 has a net label (IO25, etc.) and a bunch of pins with the same net label are grouped together.

Figure 43

Schematic Diagram, Sheet 1: Microcontroller

U1 is the ESP32-WROOM-32D on a 38-pin board. The 4x or 2x 18650-battery shield, BT1, provides the 3.3V rail for the sensors, the display and the buzzer (pin 1) and a regulated 5V to pin 19. C1 and C2 (100nF and 10µF) smooth the 3.3V rail, and C3 and C4 (100nF and 10µF) do the same at the battery input. The ground pins share one GND net. Pins which are not configured in the design (such as EN, TXD, RXD, GPIO0, GPIO2) are identified as "no-connect". According to the note on the sheet, the battery's 5V should not be used while code is being uploaded; rather, it should only be used after the code is uploaded.

Figure 44

Schematic Diagram, Sheet 2: Actuators and Outputs

LED1 is a common cathode RGB LED KY-016. Its red, green and blue legs have 1kΩ resistors connected to IO25, IO26 and IO33. BZ1 is a buzzer module (MH-FMD) that operates on 3.3V and is controlled by IO4. The SG90 micro servo (M1) operates on the 5V power supply, and is controlled by IO13 via PWM, with C5 (100nF) and C6 (1000µF) placed across the power supply to filter current spikes as the servo moves.

U2 is the 2.4 in. ILI9341 touchscreen (240 × 320, SPI). It runs on 3.3V, with C7 (100nF) across its supply. The display and its touch controller share the SPI clock and data lines on IO19, IO23 and IO35, and each has its own chip select: IO17 for the display and IO15 for touch. DC goes to IO16 and the touch interrupt T_IRQ to IO36. R1 (10kΩ) pulls T_IRQ to 3.3V, the line remains high until a touch pulls it down. The RESET and the backlight pin (LED) are connected to 3.3V and the SD card pins are not connected.

Figure 45

Schematic Diagram, Sheet 3: Sensors and Inputs

U3, the FC-51 IR obstacle sensor, runs on 3.3V and sends its active-low output to IO27, with C8 (100nF) across its supply. The HC-SR04 (U4) operates on 5V with the C9 (100nF) from supply and TRIG on IO5. It outputs a 5V pulse to its ECHO pin, which is then divided by R2 (1kΩ) and R3 (2.2kΩ) to produce ~3.4V before reaching IO18, and is thus very close to the 3.3V level that the esp32 input expects for it can tolerate up to 3.6V.

U5 (BME680) and U6 (BMI160) both run on 3.3V, with C10 and C11 (100nF each) across their supplies, and share the I2C bus, with SCL on IO22 and SDA on IO21. Tying the BME680's SDO pin to ground sets its address to 0x76, and tying the BMI160's SA0 pin to ground sets its address to 0x68, so the two do not clash. M2, the 5kg load cell, connects its red, black, white and green wires to the E+, E−, A− and A+ inputs of U7, the HX711. The HX711 runs on 3.3V, with C12 (10µF) and C13 (100nF) across its supply, and sends its data on IO32 with its clock on IO14.

## 2.6 Component Materials

This section describes the main electronic parts used in each Smart-Bin unit. Both bins use the same parts except for the battery shield. Figures 46 to 56 show each part, followed by what it does and how the system uses it.

Figure 46

ESP32 DevKit V1 (38-Pin, ESP32-WROOM-32D)

Note. Image from Sigmanortec (n.d.).

The ESP32 DevKit V1 is a development board built around the ESP32-WROOM-32D module. The module has a dual-core processor that runs at up to 240MHz, with Wi-Fi and Bluetooth on the same chip. The 38-pin board provides an additional USB connector for uploading code and power, a voltage regulator for 3.3V power and pin headers for breadboarding.

There is one board for each bin and each board governs all of the contents of that bin. It reads all six inputs (the HC-SR04, IR sensor, BME680, BMI160, load cell and touchscreen), calculates the fill percentage, air quality reading and tamper state, and controls the servo, LED, buzzer and display. Providing the local REST API over Wi-Fi, it receives the status update command from the dashboard and the Android application and sends status update to Firebase and ThingSpeak.

Figure 47

HC-SR04 Ultrasonic Distance Sensor

Note. Image from SparkFun Electronics (n.d.).

The HC-SR04 is a sound-based distance sensor. It outputs a 40kHz ultrasonic burst when a short pulse is received at its TRIG pin and will keep its ECHO pin high until the echo is received. The distance is determined by the width of that pulse which varies in the range of 2cm to 4m. The sensor runs on 5V.

In Smart-Bin it is installed under the lid and points downward, and thus measures the distance between the lid and the top of the trash. The firmware converts that distance into the fill percentage displayed on the LED, touch screen and the apps and once the fill level reaches the fill limit (80% by default), the full-bin alert is activated. TRIG is wired to IO5, and ECHO is wired to IO18 through a divider of R2 (1kΩ) and R3 (2.2kΩ), which brings the 5V pulse down to about 3.4V, close to the 3.3V input level of the ESP32.

Figure 48

FC-51 IR Obstacle Avoidance Sensor

Note. Drawing of the module sold by Circuitrocks (n.d.-c).

The FC-51 is an infrared obstacle sensor. Its IR LED sends out light, a photodiode picks up what bounces back, and an LM393 comparator pulls the output "low" when something is in front. A blue trimmer sets the range (a few cm to about 30cm). It runs on 3.3V to 5V.

It is used by Smart-Bin to know when a hand is near the bin. When the output on IO27 goes low and the bin is available for opening, the bin beeps and the servo raise the lid and the user never has to touch it. The lid will close 5 seconds (user definable) after the hand leaves, and once closed it stays closed for 2 seconds (also user definable) so that it does not flicker open and shut. During COLLECT mode the sensor is ignored and the lid remains open while the collector is operating.

Figure 49

BME680 Environmental Sensor

Note. Image from Robu.in (n.d.).

The BME680 is a single chip sensor for measuring temperature, humidity, VOC (volatile organic gases) and air pressure. Based on the gas reading, an estimated indoor air quality (IAQ) rating can be obtained. It is powered by 3.3V and communicates via I2C.

In Smart-Bin it monitors the air within the bin. When the temperature, humidity or estimated air quality goes over its limit, the firmware sounds an alarm and sends an alert, even if the bin isn't full yet, because a high temperature may mean a fire or hot material. It is located at I2C address 0x76 (SDO tied to ground).

The purpose of this is using the combination of temperature, humidity, and gas reading  (IAQ) in order to determine by the collector or operator how much rotting trash is in the Smart-Bin for smell-sensitive areas. The IAQ value is an estimate from the gas reading, and it needs about 5 minutes after power-up to settle.

Figure 50

BMI160 Six-Axis Inertial Measurement Unit

Note. Image from ElectroPeak (n.d.).

BMI160 is a 3-axis accelerometer and 3-axis gyroscopes integrated onto one chip. It is an I2C acceleration and rotation sensor that operates on 3.3V.

It is used by Smart-Bin for tamper detection. At every power-up the firmware saves the board’s pitch and roll as the upright position, so a bin that was set down at a slight angle does not start in the tamper alarm. The tilt is then measured from that saved position, and when it goes over the limit (30° by default) the firmware sets the Tamper alarm, activates the buzzer and prevents the lid opening. From the dashboard or the app an operator can see the saved angle, take the current angle as the new upright with USE NOW, type in a pitch and roll, or turn off the power-up capture to keep a fixed angle. It is on the same I2C bus as the BME680 with the address 0x68 by shorting out SA0 to ground. The tamper check is paused during COLLECT mode.

Figure 51

HX711 Load Cell Amplifier and 5kg Load Cell

Note. The HX711 board and the 5kg load cell are shown on a display base. Image from ElectronicsComp.com (n.d.).

A load cell is a metal bar with strain gauges designed so that as the load is applied, the bar's resistance varies slightly. The bar here is a 5kg model, and has a very small voltage output, which is amplified and converted in the HX711 board by a 24 bit analog to digital converter. The result is read by the ESP32 using two pins: DT (IO32) and SCK (IO14).

The two together measure the garbage. Weight is a second check on the fill level: wet waste can be heavy before the bin is full, and a bin full of light waste can be tall without being heavy. The weight is displayed on the dashboard and the app, and stored to the cloud with every measurement. The firmware also checks that the HX711 is connected. An unplugged board gives a flat or out-of-range reading, so after a few of these the bin reports “HX711 not answering” instead of a weight, the same way it flags a missing BME680 or BMI160.

Figure 52

SG90 Micro Servo Motor

Note. Image from The Pi Hut (n.d.).

The SG90 is a small hobby servo which will rotate its output arm to a specific angle (roughly 180 degrees) depending on the width of a PWM pulse. Operates on approximately 5V and can be attached directly to the bin.

Servo is used to open and close the bin lid. It is managed by ESP32, it will open when it is detected people, or when the collector taps COLLECT, it will close after 5s (user settable) and then stay closed for at least 2s (also user settable) when people are not detected or the collector taps DONE. When the bin is locked, full or flagged for tampering, the firmware keeps the servo in the closed position.

Figure 53

2.4-in. TFT Touchscreen (ILI9341 With XPT2046 Touch Controller)

Note. Front, back, and angled views of the module. Image from LCDWIKI (n.d.).

This module is a 2.4 in., 240 × 320 color display driven by the ILI9341 controller over SPI. The same SPI bus also has a separate XPT2046 controller for the resistive touch layer on top. There is also an SD card slot on the board, which isn't used by Smart-Bin.

The bin's local screen is the display. It indicates the fill level, sensor readings and the bin status (LOCKED and TAMPER). The touch layer provides the collector with the buttons COLLECT, PIN and DONE, making it possible to empty a bin without the app.

Figure 54

KY-016 RGB LED Module

Note. Image from Circuitrocks (n.d.-d).

The KY-016 is a small board with a common-cathode RGB LED, a small package containing the red, green and blue chips. When the three colors are combined, other colors are created such as amber (red + green). A 1kΩ resistor is used for each colour to control the current.

The fill level is indicated on the LED, green if there is room in the bin, amber when it is filling up and red when the bin is full or a warning is triggered. It is driven from IO25, IO26 and IO33 and turns green again after the collector taps DONE.

Figure 55

Passive Buzzer Module (MH-FMD)

Note. Image from QuartzComponents (n.d.).

Passive buzzer: No built-in oscillator so it only buzzes if it is pulsed by the controller with a changing signal. This will allow the firmware to produce various tones and patterns, rather than just a single beep.

It is used by Smart-Bin for sound feedback from IO4. It beeps when lid is opened, alerts user if the BME680 senses high temp or humidity or poor air quality, and activates the siren if BMI160 senses tampering. It remains quiet while in COLLECT mode.

Figure 56

18650-Battery Shields for Bin A (4-Cell) and Bin B (2-Cell)

Note. Left: four-cell shield used in Bin A (Circuitrocks, n.d.-b). Right: two-cell shield used in Bin B (Circuitrocks, n.d.-a).

The 18650-battery shield comprises lithium-ion 18650 cells which will provide the electricity needed and convert its voltage to a regulated 5V output. It also features a charging USB port for the cells. Bin A are equipped with a four-cell V9 shield (5V/3A), while Bin B are equipped with a two-cell shield.

The shield supplies power to the entire bin, without the need for a wall outlet, which allows a bin to be set anywhere that Wi-Fi is available. It is placed in a closed compartment and connected to the 5V of the ESP32 which provides 3.3V for the sensors. As mentioned in the schematic, the battery should not be connected during code upload via USB.

# III. TESTING, RESULTS, AND CONCLUSION

## 3.1 Test Cases

The tests in this chapter were run on the two built units, Bin A (bin01) and Bin B (bin02), between October 30 and November 3, 2026. Both ran firmware version 1.0.0 from the main branch of the project repository and were connected to the same 2.4GHz Wi-Fi router. The web dashboard was opened in Chrome on a laptop on that network, and the Android app, version 3.1, ran on a phone that used either the same Wi-Fi or mobile data, depending on the test. Unless a test says otherwise, the bins kept their default settings: an empty distance of 30cm and a full distance of 5cm, which leaves 25cm of usable height, a fill limit of 80%, a temperature limit of 40°C, a humidity limit of 85%, an IAQ limit of 200, a tilt limit of 30°, a lid open time of 5s and a lid close time of 2s.

Each specific objective in Section 1.4 is checked by six test cases, 18 in all, numbered TC1.1 to TC3.6. A test case names what is tested, the steps, the pass criterion and the number of trials. The criteria were fixed before the first trial. Some come straight from the firmware, such as the 10s tamper countdown and the 90s after which the apps call a bin lost. The others were set with related studies in mind, for example the fill error within ±3% reported by Alnanih et al. (2025) and the weight error of 0.15% in Saad et al. (2023), and were loosened a little for a table-size prototype with low-cost sensors. Table 7 lists the instruments used as references, and Figure 57 shows the two built units.

Table 7

Reference Instruments Used in the Tests

| Instrument | Used for | Resolution |
| --- | --- | --- |
| Steel ruler fixed to the inside wall of the chamber | Real fill height in TC1.1 and TC3.2 | 1mm |
| Digital kitchen scale | Checking the test masses before TC1.2 | 1g |
| Test masses (sealed bags of sand) | Known loads of 100, 500, 1000, 2000 and 4000g in TC1.2, and 500g in TC2.1 | 1g |
| Digital thermo-hygrometer | Reference temperature and humidity in TC1.3 | 0.1°C and 1% |
| Inclinometer app on a phone, laid on top of the bin | Tilt angle in TC1.5 | 0.1° |
| Phone camera at 60 frames per second | Response times of the lid, the screens and the apps in TC1.6, TC2.1 and TC3.3 | About 17ms |
| Stopwatch | Warm-up, alert, offline and time-to-full times in TC1.4, TC2.3, TC2.4 and TC3.2 | 0.01s |

Figure 57

The Two Built Smart-Bin Units

Figure 58 shows the inside of Bin A. From above (a), the servo and the HC-SR04 sit together on the strip at the hinge, with the sensor facing down into the chamber. The compartment (b) holds the perfboard with the ESP32, the BME680 and BMI160 modules beside it, and the battery shield with its four 18650 cells behind the board.

Figure 58

Inside Bin A: Trash Chamber From Above (a) and Controller and Battery Compartment (b)

| (a) | (b) |
| --- | --- |

### Specific Objective 1: Sensor Monitoring and Detection

The six test cases for the first objective check that each sensor reads close to a reference and that the lid and the tamper alarm act at the right moment (Table 8). Fill and weight are tested at five points across their range. The air test is a response test, not an accuracy test, because the IAQ value of the BME680 is an estimate made from the gas resistance and the project has no reference instrument for it.

Table 8

Test Cases for Specific Objective 1: Sensor Monitoring and Detection

| ID | Test | Procedure | Pass criterion | Trials |
| --- | --- | --- | --- | --- |
| TC1.1 | Fill accuracy | With the empty and full distances calibrated, fill the bin with crumpled paper to 0, 25, 50, 75 and 100% of the usable height, read on the ruler. At each level, record the fill percentage on the dashboard. | Error of at most 5 percentage points | 5 per level, per bin (50) |
| TC1.2 | Weight accuracy | Tare the bin, then place the test masses of 100, 500, 1000, 2000 and 4000g on the platform one at a time and record the weight on the dashboard. | Error of at most 50g | 5 per mass, per bin (50) |
| TC1.3 | Temperature and humidity | With the default temperature offset of −2°C, place the thermo-hygrometer inside the chamber and compare both readings every 6 minutes for an hour. | Temperature within ±1°C and humidity within ±5% | 10 per bin (20) |
| TC1.4 | Air quality response | Wait for the 5-minute warm-up to end. Place an open container of food scraps in the bin and close the lid. Repeat with an alcohol swab. | IAQ estimate rises past 200 and the air alert appears; no air alert during the warm-up | 3 per source, per bin (12) |
| TC1.5 | Tamper | Tilt the bin to 15, 25, 35 and 45° and set it upright again within 10s. Then hold it at 35° for more than 10s, set it upright and press RESET TAMPER. | No countdown at 15 and 25°. At 35 and 45° the 10s countdown starts and setting the bin upright clears it. A bin left tilted latches the siren, repeated every 2s, until RESET TAMPER | 5 per angle, per bin (40), and 5 held trials per bin (10) |
| TC1.6 | Automatic lid | Hold a hand in front of the IR sensor at 5, 15 and 25cm, then take it away. Watch for a second opening after the lid closes. | Lid opens within 1s, closes 5s ±0.5s after the hand leaves, and does not reopen by itself | 10 per distance, per bin (60) |

### Specific Objective 2: Real-Time Monitoring and Alerts

The second objective is about getting the readings to people. TC2.1 and TC2.2 time the path from a bin to the dashboard, the app and the two clouds. TC2.3 and TC2.4 check the alerts, including the one raised when a bin stops answering, and TC2.5 and TC2.6 check that each role sees the right controls and that two bins are never mixed up (Table 9). In TC2.3 the app is in the background, not swiped away, since Android ends the app's watch service, and with it the alerts, when the app is removed from the recent apps.

Table 9

Test Cases for Specific Objective 2: Real-Time Monitoring and Alerts

| ID | Test | Procedure | Pass criterion | Trials |
| --- | --- | --- | --- | --- |
| TC2.1 | Live latency | Place the 500g mass on the platform and time, on a 60 frames per second video, how long the dashboard and the app take to show the change. Repeat with the phone on mobile data, so that the app reads Firebase. | At most 2s over the local Wi-Fi and at most 10s through Firebase | 10 per path (30) |
| TC2.2 | Cloud logging | Run each bin for 30 minutes: 15 minutes with a reading changing every minute, then 15 minutes untouched. Compare the ThingSpeak entry times with the Firebase status. | An entry every 30s ±5s while readings change and every 5 minutes ±5s while they stay the same; Firebase matches the latest entry | 1 run per bin (66 intervals) |
| TC2.3 | Alert delivery | With the app in the background, raise a full bin, bad air and high temperature from the Simulation tab, and a real tamper by tilting the bin. Time from the event to the phone notification. | Notification on the phone and banner on the dashboard within 20s | 5 per alert type (20) |
| TC2.4 | Offline detection | Switch off one bin and time how long the apps take to call it lost, then switch it on again. | Bin shown as lost with an offline alert about 90s after its last reading, and shown as back after it reconnects | 5 per bin (10) |
| TC2.5 | Role access | Go through every control of the dashboard and the app as a collector, then again after entering the operator PIN. | A collector can see alerts and charts, run COLLECT and DONE, LOCK and UNLOCK and set up Wi-Fi, but cannot reach RESET TAMPER, calibration, touch calibration or simulation. An operator can reach all of them | 1 checklist per interface (2) |
| TC2.6 | Two bins at once | Put Bin A and Bin B in different states, for example Bin A full and Bin B tilted, and check every screen. | Each bin shows its own state with no mix-up | 5 combinations |

### Specific Objective 3: Bin Control and Collection Prioritization

The last six test cases cover what the operator and the collector do with the readings: the order of the Collection Priority list, the time-to-full estimate behind it, remote locking, the PIN collection on the touchscreen, simulation and calibration (Table 10). The priority tags follow the same rules in the dashboard and the app. A bin at or over the fill limit is tagged NOW, then SOON if it will reach the limit in under 2 hours, TODAY if under 8 hours, LATER if it will take longer and IDLE if it is not filling.

Table 10

Test Cases for Specific Objective 3: Bin Control and Collection Prioritization

| ID | Test | Procedure | Pass criterion | Trials |
| --- | --- | --- | --- | --- |
| TC3.1 | Priority order | Set fill pairs for the two bins in the Simulation tab, raising each fill step by step for at least 5 minutes so that the bins have a fill rate, and read the Collection Priority list on both interfaces. | A bin at or over the fill limit is first; otherwise the bin that reaches the limit first is on top, with the right tag | 10 combinations |
| TC3.2 | Time to full | Fill Bin A by hand at a steady rate from about 40%. After 15 minutes, note the estimated time to full, then record when the bin actually reaches 80%. | Error of at most 15% | 3 runs |
| TC3.3 | LOCK and UNLOCK | Lock the bin from the dashboard and hold a hand at the IR sensor, then unlock it and wave again. Repeat from the app on mobile data. Repeat with auto-lock on and the bin filled past 80%. | Lid stays shut while locked; the bin responds within 2s locally and within 10s through Firebase; with auto-lock on, the bin locks itself at the fill limit | 10 per path (20), and 5 auto-lock trials |
| TC3.4 | PIN collect | On the touchscreen, enter a wrong PIN, then the correct 4-digit PIN, empty the bin and press DONE. | Wrong PIN: error and the lid stays shut. Correct PIN: the lid opens and COLLECTING is shown. DONE: the lid closes, the pickup is logged and the bin is back to normal | 10 per bin (20) |
| TC3.5 | Simulation | Set the fill to MANUAL at 90% from the dashboard, then back to AUTO. Repeat for temperature, IAQ and tilt. | Simulation banner and the matching alert appear; the real reading returns on AUTO | 5 per sensor (20) |
| TC3.6 | Calibration | Change the empty and full distances, the fill limit and the lid open time in the settings, check that they apply, then restart the bin. | New values apply without reflashing and are still there after the restart | 5 per bin (10) |

## 3.2 Results and Discussion

The results are reported by objective, with one table for each test case or group of related test cases, and all 18 are brought together in Table 24 at the end. Three measures are used. The mean absolute error (MAE) is the average of |shown − actual| over all trials of a test case. The percent error of a reading is |shown − actual| / actual × 100. The pass rate is the number of trials that met the pass criterion, divided by the number of trials, times 100.

### Results for Specific Objective 1

Table 11 gives the fill readings. Both bins stayed within 3 percentage points at the three middle levels, and the MAE over all 50 trials was 1.8 points, or about 0.45cm of the 25cm usable height. The one failure came at 100% in Bin B, which showed 94% with the chamber full. Crumpled paper leaves gaps at the top, and the HC-SR04 can measure to the bottom of a gap as easily as to the top of a sheet. Pressing the paper down and reading again gave 98%, so the error came from the surface of the trash and not from the sensor or the calibration.

Table 11

Fill Level Readings Against the Ruler (TC1.1)

| Real level (%) | Real distance (cm) | Bin A mean (%) | Bin B mean (%) | Max error (points) | Passed |
| --- | --- | --- | --- | --- | --- |
| 0 | 30.0 | 0.8 | 1.2 | 2 | 10/10 |
| 25 | 23.8 | 26.4 | 23.8 | 3 | 10/10 |
| 50 | 17.5 | 51.6 | 48.2 | 3 | 10/10 |
| 75 | 11.3 | 76.8 | 73.6 | 4 | 10/10 |
| 100 | 5.0 | 97.2 | 96.4 | 6 | 9/10 |

Note. Real distance is from the sensor to the top of the paper. One percentage point is 0.25cm. MAE: Bin A 1.7 points, Bin B 1.8 points, both bins 1.8 points.

The load cell results are in Table 12. Every trial was inside the 50g band. The error grew with the mass, and Bin B read low by about 1% at every mass from 500g up. Its five readings at each mass stayed within 8g of each other, so the offset is in its calibration factor, not in noise. Repeating the weight calibration of Bin B with a heavier known mass would likely remove most of it. The worst trial, at 4000g, was 46g off. That still passes, but it leaves little margin near the 4500g capacity of the bin.

Table 12

Weight Readings Against the Test Masses (TC1.2)

| Mass (g) | Bin A mean (g) | Bin B mean (g) | Max error (g) | Mean percent error (%) | Passed |
| --- | --- | --- | --- | --- | --- |
| 100 | 102 | 97 | 6 | 2.5 | 10/10 |
| 500 | 504 | 493 | 11 | 1.1 | 10/10 |
| 1000 | 1008 | 991 | 14 | 0.9 | 10/10 |
| 2000 | 2013 | 1984 | 22 | 0.7 | 10/10 |
| 4000 | 4027 | 3961 | 46 | 0.8 | 10/10 |

Note. The masses were checked on the kitchen scale before the test. MAE: Bin A 10.8g, Bin B 14.8g, both bins 12.8g.

Table 13 compares the BME680 with the thermo-hygrometer. With the −2°C offset that the firmware applies by default, the temperature stayed within 1°C in all 20 readings. A check without the offset at the start of the hour showed the BME680 about 2°C warm, which is the heat of the ESP32 and the regulator near it. Humidity was within 5% in every reading. Bin B read a little further from the reference on both, but the gap between the bins, 0.2°C and 0.9%, is smaller than the 1% resolution of the reference for humidity.

Table 13

Temperature and Humidity Against the Thermo-Hygrometer (TC1.3)

| Bin | Reference range | Temperature MAE (°C) | Temperature max error (°C) | Humidity MAE (%) | Humidity max error (%) | Passed |
| --- | --- | --- | --- | --- | --- | --- |
| Bin A | 29.6 to 31.2°C, 58 to 66% | 0.4 | 0.8 | 2.1 | 3.9 | 10/10 |
| Bin B | 29.4 to 31.0°C, 59 to 66% | 0.6 | 0.9 | 3.0 | 4.6 | 10/10 |

Note. One reading every 6 minutes for an hour, with the reference placed inside the chamber.

The air test is summarized in Table 14. During the first 5 minutes after power-up, both interfaces showed the air value as warming up and no air alert was raised, even in one extra trial where the swab went into the bin in the second minute. After the warm-up the IAQ estimate rested between 58 and 74. Food scraps took 6 to 8 minutes to push it past 200, and the air alert appeared every time. The alcohol swab crossed the limit in about a minute and pushed the estimate much higher, which fits a gas sensor that reacts strongly to alcohol vapor. This shows that the bin notices a smell building up in a closed chamber within minutes. It does not show what the smell is or how strong it is in standard units, and both apps label the value as an estimate for that reason.

Table 14

Air Quality Response After the Warm-Up (TC1.4)

| Source | Bin | Resting IAQ | Peak IAQ (mean) | Time to pass 200 (s, mean) | Air alert | Passed |
| --- | --- | --- | --- | --- | --- | --- |
| Food scraps | Bin A | 61 | 268 | 370 | 3/3 | 3/3 |
| Food scraps | Bin B | 74 | 241 | 460 | 3/3 | 3/3 |
| Alcohol swab | Bin A | 58 | 412 | 45 | 3/3 | 3/3 |
| Alcohol swab | Bin B | 70 | 389 | 65 | 3/3 | 3/3 |

Note. Times are from closing the lid on the source. The bin was aired out for 10 minutes between trials.

The tamper results are in Table 15. No countdown started at 15 or 25°, and every trial at 35 and 45° started one within 1s of passing the angle. It started a little later at 35° than at 45° because the firmware smooths the tilt reading, and a bin tipped slowly to 35° takes longer to read past the 30° limit. Setting the bin upright inside the 10s always cleared the countdown with the settle beep. In the ten held trials, the siren latched and repeated every 2s, the tamper alert reached the app, and the siren kept going after the bin was upright until RESET TAMPER was pressed. The phone inclinometer and the BMI160 agreed within about 1° once the bin had stopped moving. The bins were switched on standing on the test table, since the upright reference is taken again at every power-up.

Table 15

Tamper Detection by Tilt Angle (TC1.5)

| Tilt | Countdown started | Mean time to start (s) | Outcome | Passed |
| --- | --- | --- | --- | --- |
| 15°, set upright | 0/10 | – | No countdown, as expected | 10/10 |
| 25°, set upright | 0/10 | – | No countdown, as expected | 10/10 |
| 35°, set upright within 10s | 10/10 | 0.8 | Countdown cleared with the settle beep | 10/10 |
| 45°, set upright within 10s | 10/10 | 0.5 | Countdown cleared with the settle beep | 10/10 |
| 35°, held past 10s | 10/10 | 0.8 | Siren latched every 2s, alert in the app, cleared only by RESET TAMPER | 10/10 |

Note. Each row has 5 trials per bin. Angles were read on the phone inclinometer.

Table 16 gives the lid results, and Figure 59 shows one trial: a hand held in front of the IR sensor with the lid still shut (a), and the lid raised by the servo a moment later (b). The lid opened in all 40 trials at 5 and 15cm and in 18 of 20 at 25cm. Both misses were on Bin B, with a dark palm held at an angle near the edge of the range, where too little of the IR light bounced back to the photodiode behind the front panel. The lid took 0.5 to 0.7s to open on average, most of it the servo travel. Measured from the moment the hand left, the lid closed after 5.1 to 5.2s, which is the 5s setting plus the 80ms the firmware waits before it accepts a change on the sensor output. The sensor has no hold time of its own, and the close matched the motion indicator on the dashboard within 0.2s. None of the 60 cycles reopened by itself, which confirms the change described next.

Table 16

Automatic Lid Response by Hand Distance (TC1.6)

| Hand distance | Opened | Mean time to open (s) | Mean close after hand left (s) | Reopened by itself | Passed |
| --- | --- | --- | --- | --- | --- |
| 5cm | 20/20 | 0.5 | 5.1 | 0 | 20/20 |
| 15cm | 20/20 | 0.6 | 5.2 | 0 | 20/20 |
| 25cm | 18/20 | 0.7 | 5.1 | 0 | 18/20 |

Note. Times were read from a 60 frames per second video. Each row has 10 trials per bin.

Figure 59

Lid Test: Hand in Front of the IR Sensor (a) and the Lid Opened (b)

| (a) | (b) |
| --- | --- |

### Lid Reopening Fix

The first cloud test on Bin 01, described under the results for Specific Objective 2, also showed the lid reopening by itself after it closed. The servo that closes the lid retriggered the PIR sensor, which then held its output high for a few seconds, and the firmware read that as a new person at the bin. The PIR was replaced with an FC-51 IR obstacle sensor that reacts only to something in front of it at that moment, so the closing movement no longer starts another cycle. Figure 60 shows Bin 01 during that first test, with the round PIR lens above the touchscreen and LID HOLDING (motion) on the screen while the lid stayed up.

Figure 60

Bin 01 With the Original PIR Sensor During the First Test

### Results for Specific Objective 2

Table 17 gives the time from placing the 500g mass to seeing the new weight. Over the local Wi-Fi both interfaces were inside the 2s limit in every trial. Most of that time is the wait between two requests of the dashboard or the app to the bin, not the bin itself. Through Firebase the app took 4.6s on average. That follows from the firmware, which writes its status to Firebase every 5s while someone is watching, so a change waits up to one write cycle before it reaches the cloud.

Table 17

Time to Show a New Reading (TC2.1)

| Path | Trials | Mean (s) | Max (s) | Limit (s) | Passed |
| --- | --- | --- | --- | --- | --- |
| Dashboard, local Wi-Fi | 10 | 1.1 | 1.8 | 2 | 10/10 |
| Android app, local Wi-Fi | 10 | 1.3 | 1.9 | 2 | 10/10 |
| Android app, Firebase on mobile data | 10 | 4.6 | 7.2 | 10 | 10/10 |

The 30-minute logging runs are in Table 18. While a reading changed every minute, Bin A wrote to ThingSpeak every 30 to 32s, and once the bin was left alone it wrote every 5 minutes, as the firmware intends. Bin B missed one entry. At about minute 9 it lost the Wi-Fi for roughly 20s, reconnected, and wrote on the next turn, which left a 61s gap. Firebase kept the latest status through the drop and matched the newest ThingSpeak entry in all 20 spot checks. The firmware never writes to ThingSpeak more often than once every 15.5s, so the free plan's limit of one write per 15s was never hit.

Table 18

ThingSpeak Logging Over 30 Minutes (TC2.2)

| Bin | Phase | Expected entries | Entries | Interval range (s) | Intervals in limit |
| --- | --- | --- | --- | --- | --- |
| Bin A | Reading changing, 15 minutes | 30 | 30 | 30 to 32 | 30/30 |
| Bin A | Untouched, 15 minutes | 3 | 3 | 300 to 301 | 3/3 |
| Bin B | Reading changing, 15 minutes | 30 | 29 | 30 to 61 | 29/30 |
| Bin B | Untouched, 15 minutes | 3 | 3 | 300 to 302 | 3/3 |

Note. Bin A wrote to ThingSpeak channel 3556235. Firebase was spot-checked 10 times per bin.

### Cloud Connection Test on Bin 01

Firebase and ThingSpeak were tested on the first built bin, Bin 01, running the firmware from the main branch. For Firebase the bin signs in with its own account, bin01@smartbin.test. The operator account is meant only for the dashboard, and Firebase refuses the bin's writes when the operator account is used on the bin. Signed in with its own account, the bin wrote its status, its calibration and its alerts to the Realtime Database, under sb/bins/bin01/status, sb/bins/bin01/cal and sb/alerts/bin01. LOCK and UNLOCK sent from the web dashboard reached the bin within a few seconds.

ThingSpeak channel 3556235 received all eight fields from the same bin: fill, weight, temperature, humidity, IAQ, pressure, tilt and a flags field that packs the motion, lid and status bits into field 8. The bin writes to the channel every 30s while the readings change and at least once every 5 minutes when they stay the same. Bluetooth Wi-Fi setup from the app also worked end to end on Bin 01.

Memory was the main obstacle. The ESP32 does not have enough free RAM to hold two HTTPS connections at once, or to run Bluetooth and an HTTPS connection together. Three changes made both clouds work. ThingSpeak was moved to plain http, which leaves the one HTTPS connection to Firebase. The stack of the web server task was made smaller in build_opt.h. Bluetooth setup now restarts the bin once when it starts and once when it ends, so each part begins with a clean memory, and the screen tells the user that the restart is expected.

Alert times are in Table 19. Every notification reached the phone well inside the 20s limit, at 5.5s on average across the four types. The tamper alert is timed from the moment it latched, after the 10s countdown, which is why it looks faster than the others. These times are longer than the 2.61s message delay that Saad et al. (2023) reported for WhatsApp messages, because the app learns of an alert by reading the bin or Firebase every few seconds rather than receiving a push from a server. For a bin that fills over hours, a delay of a few seconds does not change when a collector is sent.

Table 19

Alert Delivery to the Phone (TC2.3)

| Alert | Trials | Mean (s) | Max (s) | Passed |
| --- | --- | --- | --- | --- |
| Full bin (fill set to 90%) | 5 | 5.8 | 9.4 | 5/5 |
| Bad air (IAQ set to 260) | 5 | 6.1 | 10.2 | 5/5 |
| High temperature (set to 45°C) | 5 | 5.9 | 12.4 | 5/5 |
| Tamper (bin tilted and held) | 5 | 4.3 | 6.8 | 5/5 |

Note. The app was in the background on the phone. The dashboard banner appeared within 2s of the phone notification in every trial.

The remaining three test cases of this objective are pass or fail checks, and Table 20 lists them. A switched-off bin was called lost after 91 to 97s, which is the 90s rule of the apps plus the time since the last reading, and the offline alert sounded each time. After power came back, the bin was shown again 9 to 14s later, the time it needs to boot and join the Wi-Fi. The role checklists matched on both interfaces: a collector could do the daily work and none of the operator settings, and the operator PIN opened everything. In the five two-bin combinations, each card, chart and alert named the right bin.

Table 20

Offline Detection, Role Access and Two-Bin Results (TC2.4 to TC2.6)

| Test | Trials | Result | Passed |
| --- | --- | --- | --- |
| TC2.4 Offline detection | 10 | Lost after 91 to 97s (mean 93s), offline alert every time; shown back 9 to 14s after power-on | 10/10 |
| TC2.5 Role access, dashboard | 1 checklist | Every control matched the role | 1/1 |
| TC2.5 Role access, app | 1 checklist | Every control matched the role | 1/1 |
| TC2.6 Two bins at once | 5 | Each bin showed its own state on every screen | 5/5 |

### Results for Specific Objective 3

All 10 priority combinations put the right bin on top, with the right tags, on both the dashboard and the app. Table 21 shows five of them. The fourth row is the one the feature exists for. Bin A was fuller, at 70%, but Bin B was filling fifteen times faster and was going to reach the limit first, so it was listed first with SOON while Bin A got TODAY. In the fifth row Bin A was half full but not filling, so it dropped below an emptier bin that was.

Table 21

Collection Priority Order for Five of the Ten Combinations (TC3.1)

| No. | Bin A fill, rate | Bin B fill, rate | Expected first | Shown first | Tags (A, B) | Passed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 92%, any | 45%, 10%/h | Bin A | Bin A | NOW, TODAY | Yes |
| 2 | 30%, 6%/h | 85%, any | Bin B | Bin B | LATER, NOW | Yes |
| 3 | 60%, 15%/h | 60%, 5%/h | Bin A | Bin A | SOON, TODAY | Yes |
| 4 | 70%, 2%/h | 40%, 30%/h | Bin B | Bin B | TODAY, SOON | Yes |
| 5 | 50%, 0%/h | 20%, 4%/h | Bin B | Bin B | IDLE, LATER | Yes |

Note. Fill limit 80%. The other five combinations also passed on both interfaces.

The time-to-full estimate is less certain, as Table 22 shows. The firmware works out the fill rate from one fill sample per minute, and the apps divide what is left to the 80% limit by that rate. Runs 1 and 2 came within 15%. Run 3 did not: the paper went in quickly during the first minutes and then more slowly, so the rate at minute 15 was too high and the bin filled later than estimated. The estimate assumes the rate stays the same, and filling by hand does not keep it the same. Real bins fill unevenly too, which is a reason to read the time as a guide to order the bins rather than as a promise. The mean error over the three runs was 12.2%.

Table 22

Estimated and Actual Time to Reach the 80% Fill Limit (TC3.2)

| Run | Fill at minute 15 (%) | Estimated time to full (min) | Actual time to full (min) | Error (%) | Passed |
| --- | --- | --- | --- | --- | --- |
| 1 | 52 | 35 | 38 | 7.9 | Yes |
| 2 | 49 | 52 | 47 | 10.6 | Yes |
| 3 | 58 | 18 | 22 | 18.2 | No |

Note. Each run started at about 40% fill on Bin A. Times are counted from minute 15.

The control tests in Table 23 passed in every trial. A locked bin kept its lid shut against the IR sensor every time, and unlocking took 0.9s on average over the local Wi-Fi and 3.8s through Firebase, where the bin checks for new commands every 2s while someone is watching. With auto-lock on, both bins locked themselves at 80 or 81% and unlocked again after the collector pressed DONE. On the touchscreen a wrong PIN showed the error and left the lid shut, and the right PIN opened it with COLLECTING on the screen. Simulation raised the matching banner and alert in all 20 trials and gave back the real reading on AUTO. The changed settings applied at once and were still there after a restart. Figure 61 shows the home screen of Bin 01. Pressing COLLECT there brings up the PIN pad used in TC3.4.

Table 23

Lock, PIN Collection, Simulation and Calibration Results (TC3.3 to TC3.6)

| Test | Trials | Result | Passed |
| --- | --- | --- | --- |
| TC3.3 LOCK and UNLOCK, local Wi-Fi | 10 | Lid stayed shut while locked; unlock in 0.9s mean, 1.6s max | 10/10 |
| TC3.3 LOCK and UNLOCK, Firebase | 10 | Lid stayed shut while locked; unlock in 3.8s mean, 6.1s max | 10/10 |
| TC3.3 Auto-lock | 5 | Locked itself at 80 to 81%; unlocked after DONE | 5/5 |
| TC3.4 Wrong PIN | 10 | Error shown, lid stayed shut | 10/10 |
| TC3.4 Correct PIN and DONE | 10 | Lid opened with COLLECTING; DONE closed it and logged the pickup | 10/10 |
| TC3.5 Simulation | 20 | Banner and matching alert in every trial; real reading back on AUTO | 20/20 |
| TC3.6 Calibration | 10 | Applied without reflashing; kept after the restart | 10/10 |

Figure 61

Home Screen of Bin 01, With the IR Sensor Above It

### Summary of Results

Table 24 brings the 18 test cases together. Of the 463 trials, 458 met their criterion, a pass rate of 98.9%. Fourteen test cases passed every trial. The four that did not were the fill level at 100% (one trial), the lid at 25cm (two trials), ThingSpeak logging on Bin B (one interval) and the time-to-full estimate (one run). None of the five failed trials came from a fault in the firmware logic. They came from the surface of the trash, the position of the hand, a short Wi-Fi drop and an uneven filling rate, and each of them points to one of the recommendations in Section 3.4.

Table 24

Summary of Test Results

| ID | Test | Trials | Passed | Pass rate (%) | Main measure |
| --- | --- | --- | --- | --- | --- |
| TC1.1 | Fill accuracy | 50 | 49 | 98.0 | MAE 1.8 points |
| TC1.2 | Weight accuracy | 50 | 50 | 100 | MAE 12.8g |
| TC1.3 | Temperature and humidity | 20 | 20 | 100 | MAE 0.5°C and 2.6% |
| TC1.4 | Air quality response | 12 | 12 | 100 | 45 to 460s to pass 200 |
| TC1.5 | Tamper | 50 | 50 | 100 | Countdown within 1s |
| TC1.6 | Automatic lid | 60 | 58 | 96.7 | Opens in 0.6s mean |
| TC2.1 | Live latency | 30 | 30 | 100 | 1.1s local, 4.6s Firebase |
| TC2.2 | Cloud logging | 66 | 65 | 98.5 | One 61s gap |
| TC2.3 | Alert delivery | 20 | 20 | 100 | 5.5s mean |
| TC2.4 | Offline detection | 10 | 10 | 100 | Lost after 93s mean |
| TC2.5 | Role access | 2 | 2 | 100 | All controls matched |
| TC2.6 | Two bins at once | 5 | 5 | 100 | No mix-up |
| TC3.1 | Priority order | 10 | 10 | 100 | Right order and tags |
| TC3.2 | Time to full | 3 | 2 | 66.7 | Mean error 12.2% |
| TC3.3 | LOCK and UNLOCK | 25 | 25 | 100 | 0.9s local, 3.8s Firebase |
| TC3.4 | PIN collect | 20 | 20 | 100 | Wrong PIN always refused |
| TC3.5 | Simulation | 20 | 20 | 100 | Real reading back on AUTO |
| TC3.6 | Calibration | 10 | 10 | 100 | Kept after restart |
|  | Total | 463 | 458 | 98.9 |  |

Note. For TC2.2 the trials are the intervals between ThingSpeak entries.

### Limitations

ThingSpeak's write key travels over plain http, so it can be read by anyone who can see the network traffic between the bin and ThingSpeak. Firebase still uses HTTPS. Bluetooth Wi-Fi setup needs two restarts of the ESP32 for the same memory reason.

The tests themselves have limits as well. They ran indoors, on a table, over five days, with two bins and the people of the group as users. They do not show how the bins behave in rain, in direct sun or in the hands of strangers. Battery life was not measured over a full discharge, so the 18 hours of Bin A and the 9 hours of Bin B remain estimates. The IAQ value had no reference instrument, and the time-to-full estimate was tested only with hand filling.

## 3.3 Conclusion

The first specific objective was met. Both bins measured the fill level with a mean error of 1.8 percentage points and the weight with a mean error of 12.8g, inside the limits set before testing. Temperature and humidity stayed within 1°C and 5% of the reference once the default offset was applied, and the air estimate rose past its limit and raised the alert in every trial after the warm-up. The tamper alarm ignored tilts under 30°, started its countdown within 1s above it and latched until it was reset, and the lid opened for a hand in 58 of 60 trials without ever reopening by itself.

The second specific objective was also met. New readings reached the dashboard and the app in under 2s over the local Wi-Fi and in under 8s through Firebase. ThingSpeak was written every 30s while the readings changed and every 5 minutes when they did not, with one entry missed during a Wi-Fi drop. Alerts for a full bin, bad air, high temperature and tampering reached the phone in 5.5s on average, a bin that went silent was reported lost after about 90s, and each role saw only the controls meant for it.

The third specific objective was met, with one reservation. The Collection Priority list ordered the bins correctly in all 10 combinations, remote LOCK and UNLOCK, the PIN collection, simulation and calibration worked in every trial, and auto-lock closed a full bin by itself. The time-to-full estimate was within 15% in two of three runs, and it is best read as a guide for ordering the bins, not as an exact time.

Taken together, the results show that Smart-Bin does what the general objective set out to do. It monitors the fill level, weight and air of a bin, detects tampering, and alerts the operator and the collector through the web dashboard, the Android app and the on-bin touchscreen. With 458 of 463 trials passed, it is ready to be shown as a working prototype, and the failures that remain point to clear next steps.

## 3.4 Recommendations

The first recommendations come straight from the failed trials. The HC-SR04 should sit higher above the trash, or the firmware should take the median of more readings near the top of the bin, so that gaps in loose trash are not read as empty space. The IR sensor of Bin B should have its range turned up a little or be moved forward in the front panel, since both lid misses came at the edge of its range. The weight of Bin B should be calibrated again with a heavier known mass. A short buffer that keeps the last ThingSpeak reading and sends it as soon as the Wi-Fi returns would close the kind of gap seen in TC2.2.

Security is next. ThingSpeak's write key travels over plain http because the ESP32 cannot hold a second HTTPS connection beside Firebase. Moving to an ESP32 with more memory, such as a module with PSRAM, would let both clouds use HTTPS and would also remove the two restarts that Bluetooth Wi-Fi setup needs.

Range and power limit where the bin can stand. Smart-Bin needs Wi-Fi, so a bin outside a building has no way to report. A low-power long-range link such as LoRa would let a bin sit wherever it is needed, and related studies already use it (Azmi et al., 2024; Mugaruka et al., 2026). The estimated battery life, about 18 hours for Bin A and 9 hours for Bin B, is enough for a demonstration and not for a street. A small solar panel, larger cells, or longer sleep between readings when nobody is watching would stretch it, and a full discharge test should confirm the real figure.

The enclosure has to change for outdoor use. A waterproof, vandal-resistant shell and a heat-proof mounting for the sensors would answer the threats listed in the SWOT analysis, and the tamper alarm could then be tested against real handling instead of a tilted bin on a table.

Collection priority could grow into route planning. Adding GPS and a road map would turn the ranking of the bins into a route for the truck, which is where the largest savings in the related studies were reported, for example 58.9% less distance in Gupta et al. (2025) and up to 52% shorter routes in Banciu et al. (2025). A field pilot with more bins, such as the 50 bins of Alnanih et al. (2025), would show how the system behaves at a scale where the free plans of ThingSpeak and Firebase may no longer be enough.

Two more tests would add evidence this paper does not have. A user evaluation with real operators and collectors, for instance with the System Usability Scale that Maulida et al. (2025) used with 15 sanitation officers, would check how easy the apps are to use. A comparison of the BME680 air estimate with a reference gas sensor would show how far the estimate can be trusted for odors.

The data the bins already send could do more. ThingSpeak keeps every reading, so a weekly report of the fill rate of each bin, by day and by hour, would show the barangay or the campus which bins fill fastest and on which days, and collection days could be planned from that instead of from habit. The Collection Priority list itself ranks the bins by fill alone. A bin with food waste can smell long before it is full, so a later version could raise a bin in the list when its IAQ estimate or its temperature stays high, even if its fill level is still low.

Finally, Bin A and Bin B were built by hand, one at a time. A printed circuit board that carries every sensor header, including the HC-SR04 that was missing from the first board, and a fixed mounting for each sensor would make the two units identical and would let more units be built and calibrated in the same way.

# REFERENCES

Aasish, V. G., Mari Shankar, C., & Shanmugasundaram, M. (2025). FreeRTOS based smart waste management system. In 2025 Innovations in Power and Advanced Computing Technologies (i-PACT) (pp. 1–7). IEEE. https://doi.org/10.1109/i-PACT65952.2025.11307820

Abdullahi, A., Mohammed, A., Bonet, M. U., El-Suleiman, A., Ahmad, R. B., & Chollom, T. D. (2024). Development of a smart waste management system with automatic bin lid control for smart city environment. EAI Endorsed Transactions on Smart Cities, 7(3). https://doi.org/10.4108/eetsc.4385

Alnanih, R., Elrefaei, L., & Al-Ahwal, A. (2025). Advancing sustainability through an IoT-driven smart waste management system with software engineering integration. Sustainability, 17(21), Article 9803. https://doi.org/10.3390/su17219803

Alnuaimi, A. N., Aydin, R., & Al-Shihabi, S. (2024). Optimizing municipal solid waste collection using smart bins: A case study in United Arab Emirates. In 2024 IEEE International Conference on Technology Management, Operations and Decisions (ICTMOD) (pp. 1–6). IEEE. https://doi.org/10.1109/ICTMOD63116.2024.10878234

Alourani, A., Ashraf, M. U., & Aloraini, M. (2025). Smart waste management and classification system using advanced IoT and AI technologies. PeerJ Computer Science, 11, Article e2777. https://doi.org/10.7717/peerj-cs.2777

Asido, D. (2024, January 19). Baguio students develop smart garbage bins. The Post. https://thepost.net.ph/news/campus/baguio-students-develop-smart-garbage-bins/

Awotayo, M. M., & Dodo, U. A. (2026). Design and implementation of an IoT-based smart bin for real-time fill level monitoring. Journal of Science Technology and Education, 14(1), 344–351. https://www.researchgate.net/publication/400748751

Ayyanagoudar, A., Mohankar, A., Gharte, J., Pasnsare, P., Ainapure, B., & Appasani, B. (2024). A smart bin with real-time monitoring and garbage level tracking using IoT. In 2024 11th International Conference on Reliability, Infocom Technologies and Optimization (Trends and Future Directions) (ICRITO) (pp. 1–7). IEEE. https://doi.org/10.1109/ICRITO61523.2024.10522263

Azmi, A. Z., Sahar, N. M., & Islam, M. T. (2024). Smart waste management system using LoRa technology. E3S Web of Conferences, 599, Article 04007. https://doi.org/10.1051/e3sconf/202459904007

Bahga, A., & Madisetti, V. (2014). Internet of things: A hands-on approach.

Balachandran, N., & Talavera, O. (2023). Apparatus and method for waste monitoring and analysis (U.S. Patent No. 11,788,877). U.S. Patent and Trademark Office. https://patents.google.com/patent/US11788877B2

Banciu, C., Florea, A., & Popa, M. (2025). IoT-enabled waste collection: Route optimization and environmental impact reduction using smart technologies. In 2025 29th International Conference on System Theory, Control and Computing (ICSTCC) (pp. 782–788). IEEE. https://doi.org/10.1109/ICSTCC66753.2025.11240462

Bigbelly. (n.d.). CLEAN software. Retrieved October 19, 2026, from https://bigbelly.com/products/clean-software

Bosch Sensortec. (n.d.). BME680 gas sensor (Document No. BST-BME680-DS001) [Data sheet]. Retrieved October 19, 2026, from https://www.bosch-sensortec.com/media/boschsensortec/downloads/datasheets/bst-bme680-ds001.pdf

Brown, W. J. (2006). Compactor service and monitoring system (U.S. Patent No. 7,145,450). U.S. Patent and Trademark Office. https://patents.google.com/patent/US7145450B2

Circuitrocks. (n.d.-a). 2 channel 18650-battery holder protection board. Retrieved October 15, 2026, from https://circuit.rocks/products/2-channel-18650-battery-holder-protection-board

Circuitrocks. (n.d.-b). 4 channel 18650-battery holder protection board. Retrieved October 15, 2026, from https://circuit.rocks/products/4-channel-18650-battery-holder-protection-board

Circuitrocks. (n.d.-c). IR proximity obstacle avoidance sensor module. Retrieved October 30, 2026, from https://circuit.rocks/products/product-2201

Circuitrocks. (n.d.-d). KY-016 RGB 3 color full-color LED module. Retrieved October 15, 2026, from https://circuit.rocks/products/ky-016-rgb-3-color-full-color-led-module

Compology. (n.d.). Image-based container monitoring built for waste. Retrieved October 19, 2026, from https://compology.com/

Devitt, S., & Walsh, B. (2021). Techniques for optimally sensing full containers (U.S. Patent No. 11,054,545). U.S. Patent and Trademark Office. https://patents.google.com/patent/US11054545B2

Di Giugno, M., Giovanelli, F., Tiraboschi, S., & Villa, P. (2016). Measuring device, container means and system for monitoring and managing container means (European Patent No. 2516974). European Patent Office. https://patents.google.com/patent/EP2516974B1

Ecological Solid Waste Management Act of 2000, Republic Act No. 9003 (2001). https://lawphil.net/statutes/repacts/ra2001/ra_9003_2001.html

Ecube Labs. (n.d.). Waste analytics platform: CleanCityNetworks (CCN). Retrieved October 19, 2026, from https://www.ecubelabs.com/waste-analytics-platform/

ElectronicsComp.com. (n.d.). 5 kg load cell with HX711 module shell and 4P DuPont wire kit. Retrieved October 15, 2026, from https://www.electronicscomp.com/5-kg-load-cell-with-hx711-module-shell-and-4p-dupont-wire-kit

ElectroPeak. (n.d.). GY-BMI160 module 6DOF 6 axis angular velocity. Retrieved October 15, 2026, from https://electropeak.com/gy-bmi160-module-6dof-6-axis-angular-velocity

Enache, B.-A., Banica, C.-K., & Bogdan, A. G. (2024). Intelligent trash level detection for sustainable urban waste management. The Scientific Bulletin of Electrical Engineering Faculty, 24(1), 56–59. https://doi.org/10.2478/sbeef-2024-0010

Evreka. (2020). Fill level sensor. Retrieved October 19, 2026, from https://evreka.co/solutions/fill-level-sensor/

Gajula, R., Karanam, M., Vakkapatla, D. C., Endrapu, K. R., Raju, V. V. R., Reddy, S. B., Aljanabi, S. O., Koli, S., & Mahajan, S. (2025). Sustainable waste management solution: ESP32-powered smart bin. AIP Conference Proceedings, 3263(1), Article 140006. https://doi.org/10.1063/5.0261359

González, G. E., Escobar, E. R., Carvajal, I. I., & Bento, A. C. (2024). Smart trash bins: Integration of IoT with AI for efficient waste management. In 2024 IEEE International Humanitarian Technologies Conference (IHTC) (pp. 1–6). IEEE. https://doi.org/10.1109/IHTC61819.2024.10855147

Greenwalt, M. (2020, June 2). Bin-e creates contactless smart bin. Waste360. https://www.waste360.com/recycling/bin-e-creates-contactless-smart-bin

Gupta, S., Gautam, R., & Muneer, A. (2025). IoT-based e-tracking system for waste management. In 2025 12th International Conference on Reliability, Infocom Technologies and Optimization (Trends and Future Directions) (ICRITO) (pp. 1–6). IEEE. https://doi.org/10.1109/ICRITO66076.2025.11241930

Ishaq, A., Mohammad, S. J., Bello, A. D., Wada, S. A., Adebayo, A., & Jagun, Z. T. (2025). Smart waste bin monitoring using IoT for sustainable biomedical waste management. Environmental Science and Pollution Research, 32(32), 19434–19449. https://doi.org/10.1007/s11356-023-30240-1

Karthi, S. P., Aravind, A., Dharineesh, V., Dhayanidhi, J. V., & Dinesh Kumar, J. R. (2025). IoT enabled smart dustbin with RFID-based access control and Blynk integration. In 2025 IEEE First International Conference on Innovations in Engineering and Next-Generation Technologies for Sustainability (ICINVENTS). IEEE. https://doi.org/10.1109/ICINVENTS64613.2025.11402326

Kekäläinen, F. (2018). Method and system for monitoring and communicating fill rate of container (U.S. Patent No. 9,930,429). U.S. Patent and Trademark Office. https://patents.google.com/patent/US9930429B2

Kekalainen, F., & Engstrom, J. (2014). Smart waste collection system and method (International Patent Application Publication No. WO 2014/079586 A1). World Intellectual Property Organization. https://patents.google.com/patent/WO2014079586A1

Kekalainen, F., & Engstrom, J. (2018). Sensor device for smart waste collection systems and method (U.S. Patent No. 9,927,278). U.S. Patent and Trademark Office. https://patents.google.com/patent/US9927278B2

Keller, J. R., Redmond, M. O., & Mirza, S. A. (2024). System and method for measuring the fill level of a trash can using a sensor (U.S. Patent No. 12,125,361). U.S. Patent and Trademark Office. https://patents.google.com/patent/US12125361B2

Kumar, K. R., Pavan, O. S., Azeez, S. A., Suresh, C. V., Sree, K. R., & Reddy, P. U. (2025). IoT enabled route optimization for smart waste management using machine learning. i-manager's Journal on IoT and Smart Automation, 3(2), 11-23. https://doi.org/10.26634/jiot.3.2.21684

Kurani, H. B. (2023). Smart waste bin sensor device and methods for waste management system (U.S. Patent No. 11,702,280). U.S. Patent and Trademark Office. https://patents.google.com/patent/US11702280B2

Kuzhin, M. F., Joshi, A., Mittal, V., Khatkar, M., & Guven, U. (2024). Optimizing waste management through IoT and analytics: A case study using the Waste Management Optimization Test. BIO Web of Conferences, 86, Article 01090. https://doi.org/10.1051/bioconf/20248601090

LCDWIKI. (n.d.). 2.4inch SPI module ILI9341 SKU:MSP2402. Retrieved October 15, 2026, from https://www.lcdwiki.com/2.4inch_SPI_Module_ILI9341_SKU:MSP2402

Leones, J. R. (2025, March 3). Philippines ministerial statement [Paper presentation]. High-Level 12th Regional 3R and Circular Economy Forum in Asia and the Pacific, Jaipur, India. https://uncrd.un.org/sites/uncrd.un.org/files/12th3r_ps1-philippines.pdf

Maulida, A., Fayzalmi, C. S., Anaam, F. A. C., Ardiansyah, D., Gunawan, M. A., & Ridwan, N. (2025). Design of IoT-based smart trash bin monitoring using ESP32 and Firebase. Jurnal Komputer, Teknologi Informasi dan Sistem Informasi (JUKTISI), 4(2), 420-428. https://doi.org/10.62712/juktisi.v4i2.420

Mehta, H., Deriya, V., Sankhe, D., Kanani, P., Pandya, G., & Shelke, V. (2025). Smart bins, smarter cities: IoT-driven waste collection with real-time sensing and methane mapping. Mathematical Modelling of Engineering Problems, 12(6), 2110–2122. https://doi.org/10.18280/mmep.120626

Mehta, S., & Rathour, A. (2024). Innovative waste management: The efficiency of IoT-enabled smart bins. In 2024 7th International Conference on Circuit Power and Computing Technologies (ICCPCT) (pp. 467–471). IEEE. https://doi.org/10.1109/ICCPCT61902.2024.10672653

Mugaruka, O. M., Kala, J. R., Ndikumagenge, J., van Schadewijk, R., & Nibitanga, R. (2026). Design of an IoT sensor-based network architecture for waste collection in Bukavu city (DRC). Discover Internet of Things, 6(1), Article 168. https://doi.org/10.1007/s43926-026-00479-4

Mullaney, G. (2016). Refuse container with weight indicator and danger alerting (U.S. Patent No. 9,347,821). U.S. Patent and Trademark Office. https://patents.google.com/patent/US9347821B1

Muthukumar, K., Siddarth, T., Udhayakumar, K., & Saratha, M. (2024). Smart bin: AI-based waste segregation with monitoring system. International Journal of Innovative Research in Technology, 11(6), 1293–1298. https://ijirt.org/Article?manuscript=169606

Neumann, R. H., & Allison, C. B. (1991). Automated trash management system (U.S. Patent No. 5,016,197). U.S. Patent and Trademark Office. https://patents.google.com/patent/US5016197A

Nordsense. (n.d.). Intelligent routing. Retrieved October 19, 2026, from https://nordsense.com/intelligent-routing/

Philippine Statistics Authority. (2026). Highlights of the Compendium of Philippine Environment Statistics 2016–2025, Component 3: Residuals. https://psa.gov.ph/statistics/environment-statistics/highlights/component-3-residuals

Poss, J., Satwicz, J., Richmond, B., & Taylor, M. (2006). Solar powered compaction apparatus (U.S. Patent No. 7,124,680). U.S. Patent and Trademark Office. https://patents.google.com/patent/US7124680B2

QuartzComponents. (n.d.). Passive buzzer module with PCB. Retrieved October 15, 2026, from https://quartzcomponents.com/products/passive-buzzer-module

Rathnayake, R. M. P. M. D., Chanaka, T. M. M., Dabare, P. T. R., & Hewagamage, K. P. (2024). IoT-enabled dual-sensing smart waste management system: Enhancing urban cleanliness and sustainability in smart cities. In 2024 8th SLAAI International Conference on Artificial Intelligence (SLAAI-ICAI). IEEE. https://doi.org/10.1109/SLAAI-ICAI63667.2024.10844960

Robu.in. (n.d.). BME680 digital temperature humidity pressure sensor CJMCU-680 high altitude sensor module development board. Retrieved October 15, 2026, from https://robu.in/product/bme680-digital-temperature-humidity-pressure-sensor-cjmcu-680-high-altitude-sensor-module-development-board/

Saad, A. M., Jul, B. O., Basalamah, A., & Sayuti, S. (2023). IoT-based smart dustbin prototype. PROtek: Jurnal Ilmiah Teknik Elektro, 10(2), 120–125. https://doi.org/10.33387/protk.v10i2.6035

Sadikin, F. R., Hikmawati, E., & Aulia, S. (2025). TrashTrack: A smart waste monitoring system with constraint-based greedy algorithm for optimized collection routes. In 2025 IEEE International Conference on Artificial Intelligence for Learning and Optimization (ICoAILO) (pp. 262–268). IEEE. https://doi.org/10.1109/ICoAILO66760.2025.11155998

Sait, S., Aliyu, F. M., Bukar, S., Maina, B. S., & Lawal, D. U. (2025). Waste bin collection and monitoring method using waste bin sensors (U.S. Patent No. 12,450,575). U.S. Patent and Trademark Office. https://patents.google.com/patent/US12450575B1

Salleh, A., Mohamad, N. R., & Hashim, N. M. Z. (2024). Optimizing waste collection efficiency by NodeMCU enabled ultrasonic trash can surveillance and ThingSpeak dashboards. International Journal of Latest Technology in Engineering, Management & Applied Science, 12(12), 19-26. https://doi.org/10.51583/ijltemas.2023.121202

Samolovac, J. (2023, January 25). What is hybrid project management? Plaky. https://plaky.com/learn/project-management/hybrid-project-management/

Sensoneo. (n.d.). Waste management system. Retrieved October 19, 2026, from https://www.sensoneo.com/products/waste-management-system/

Shahabdeen, J. A. (2016). Smart garbage bin (U.S. Patent Application Publication No. 2016/0176630 A1). U.S. Patent and Trademark Office. https://patents.google.com/patent/US20160176630A1

Shaharil, M. H. F., & Po’ad, F. A. (2024). Arduino-based smart dustbin with ultrasonic sensor for efficient waste management. Evolution in Electrical and Electronic Engineering, 5(2), 92–97. https://periodical.uthm.edu.my/index.php/eeee/article/view/17147

Shahibollah, R. A., Fadzal, N., Fadzal, A. N., Omar, S., Jaafar, A. N., & Ahmad, N. (2025). IoT-enabled smart dustbin for real-time waste monitoring and scalable hygiene management. In 2025 IEEE 16th Control and System Graduate Research Colloquium (ICSGRC) (pp. 75–80). IEEE. https://doi.org/10.1109/ICSGRC65918.2025.11159745

Shin, H. (2016). Inputting device for quantity and weight loss of food garbage container (Korean Patent No. 10-1581944). Korean Intellectual Property Office. https://patents.google.com/patent/KR101581944B1

Sigmanortec. (n.d.). ESP32-DevKitC ESP32-WROOM-32D WiFi BT 38P development board. Retrieved October 15, 2026, from https://sigmanortec.ro/en/esp32-devkitc-development-board-esp32-wroom-32d-38p

SparkFun Electronics. (n.d.). Ultrasonic distance sensor - 5V (HC-SR04). Retrieved October 15, 2026, from https://www.sparkfun.com/ultrasonic-distance-sensor-hc-sr04.html

Thangam, S., Gurupriya, M., Yadav, C. S. K., Harshith, M., & Kumar, M. S. (2024). A smart waste management system using ESP8266. In 2024 4th International Conference on Sustainable Expert Systems (ICSES) (pp. 1–5). IEEE. https://doi.org/10.1109/ICSES63445.2024.10762967

The Pi Hut. (n.d.). TowerPro servo motor - SG90 analog. Retrieved October 15, 2026, from https://thepihut.com/products/towerpro-servo-motor-sg90-analog

United Nations Department of Economic and Social Affairs. (n.d.). Goal 11: Make cities and human settlements inclusive, safe, resilient and sustainable. https://sdgs.un.org/goals/goal11

Wang, X., Chen, J., & Lin, Z. (2020). Trash can having sensing flip lid having settable sensing distance and control method therefor (European Patent No. 3 620 405 A1). European Patent Office. https://patents.google.com/patent/EP3620405A1

Wang, X., Zhang, Y., & Wang, Z. (2020). Garbage bin intelligent control ware and garbage bin supervisory systems (Chinese Patent No. CN111559590A). China National Intellectual Property Administration. https://patents.google.com/patent/CN111559590A

World Bank. (2022). An assessment of municipal solid waste plans, collection, recycling and disposal of Metro Manila (East Asia and Pacific Region: Marine Plastics Series). https://documents1.worldbank.org/curated/en/099125007222250938/pdf/P170994059e8e308c0a3ec02e0c4f57778b.pdf

Yan, S., & Chen, J. (2021). Smart city garbage management system (Chinese Patent No. CN 109324552 B). China National Intellectual Property Administration. https://patents.google.com/patent/CN109324552B

Yang, F., & Sandor, J. (2010). Trash can with power operated lid (U.S. Patent No. 7,781,995). U.S. Patent and Trademark Office. https://patents.google.com/patent/US7781995B2

Yang, S., Liao, D., Li, R., & Yu, Y. (2020). Capacity automated inspection garbage bin with infrared induction function (Chinese Patent No. CN 210504152 U). China National Intellectual Property Administration. https://patents.google.com/patent/CN210504152U

Yogesh, H. M., Shivalingappa, Suman, M., Vinodh, B., & Malini, V. L. (2025). IoT-enhanced smart dustbin with real-time weight and level monitoring using ESP32. International Journal of Advanced Research in Science, Communication and Technology, 5(10), 227-231. https://doi.org/10.48175/ijarsct-28841

# CURRICULUM VITAE

| RODEL S. BANOGBANOG 1230 Interior 8, Maaliwalas st, Kahilum2, Pandacan, Manila Mobile No.: 09916540971 Email Address: rodel.banogbanog@tup.edu.ph |  |
| --- | --- |

SKILLS

•	Computer Systems Servicing NCII

•	Electrical Installation and Maintenance NCII

•	Basic knowledge C++, Java ang Python Programing

•	Computer Installation and Server Configuration

•	Basic knowledge in Information and Communication Technology

•	Basic knowledge in Computer Configuration

EDUCATIONAL BACKGROUND

•	Bachelor of Engineering Technology	August 2023

Major in Computer Engineering Technology

Technological University of the Philippines – Manila

•	Ignacio Villamor Senior High School	June 2021

2109 San Andres Ext, Sta. Ana Manila

•	Alternative Learning System (ALS) in	June 2019

Sta. Ana Elementary School, M. Roxas St, Sta. Ana

•	Molave Vocational Technical School	June 2009

•	Milligan Elementary School, Molave	June 2004

CONFERENCES/SEMINARS/TRAININGS ATTENDED

•	ROTC Training Service	August 2024

Technological University of the Philippines – Manila

•	Sea gem Training Center	July 2021

•	Kabaka Manpower Training Service	August 2013

CURRICULUM VITAE

| BRIAN JAY CELESTE 829 Teresa St. Brgy. 661 Zone 70 Ermita, Manila 1000 Metro Manila Mobile No.: 09272794551 Email Address: brianjay.celeste@tup.edu.ph |  |
| --- | --- |

SKILLS

•	Adaptable

•	Communicator

•	Critical Thinker

•	Fast Learner

•	Leadership Skills

•	Diligent

•	Computer Literate

•	Programming and Web Development Skills

•	Basic Electrical Circuits and Electronics Knowledge

EDUCATIONAL BACKGROUND

•	Bachelor of Engineering Technology	August 2022 – Present

Major in Computer Engineering Technology

Technological University of the Philippines – Manila

•	Technical-Vocational-Livelihood	June 2021

ICT Strand – Mobile App and Web Development

STI College Bacoor

•	Technical-Vocational-Livelihood	August 2020

ICT Strand – Mobile App and Web Development

STI College Dasmariñas

•	Technical-Vocational-Livelihood	August 2019

ICT Strand – Mobile App and Web Development

STI College Dasmariñas

CONFERENCES/SEMINARS/TRAININGS ATTENDED

•	Beyond Pixels: Android and Visual Studio Mobile Development	August 2024

Technological University of the Philippines – Manila

•	Technobytes Tutorials	August 2021

Introduction to Windows Forms App Development

STI College Bacoor

MEMBERSHIP IN PROFESSIONAL ORGANIZATIONS

•	CIT Student Council – Technical Associate	June 2023

Technological University of the Philippines – Manila

•	Technobytes	June 2022

STI College Bacoor

•	Gamer’s Lounge	June 2021

STI College Dasmariñas

AWARDS AND CERTIFICATIONS

•	C-Language Competition Champion	May 2023

•	1st Place Research Paper	June 2022

•	2nd Place Research Poster	June 2022

•	3rd Place Sci-mulation – Air Vortex Cannon	June 2022

•	JLPT N3 – Japanese Language Proficiency Test Certification	June 2023

WORK EXPERIENCE

•	Technical Support Intern	June 2022

STI College Bacoor

Software and Hardware Troubleshooting

Network Maintenance

•	Aluminum Worker	June 2021

Arami Tousou

Aluminum Processing and Finishing

Japanese–English Interpretation

Technical Support Assistance

CURRICULUM VITAE

| JADE C. GUNAYAN 110 Daisy st. Nbbs proper, Navotas City Metro Manila Mobile No.: 09533713171 Email Address: jade.gunayan@tup.edu.ph |  |
| --- | --- |

SKILLS

•	Adaptable

•	Communicator

•	Critical Thinker

•	Fast Learner

•	Leadership Skills

•	Diligent

EDUCATIONAL BACKGROUND

•	Bachelor Engineering Technology	Present

Major in Computer Engineering Technology

•	Senior High School	2021–2023

San Rafael Technological Vocational High School

•	Junior High School	2017–2021

San Pascual Agricultural High School

•	Elementary	2012–2017

Bongbong Elementary School

CONFERENCES/SEMINARS/TRAININGS ATTENDED

•	CWTS Training Service	August 2024

Technological University of the Philippines – Manila

CURRICULUM VITAE

| KING PERTH G. PEPITO Blk 1. No. 19 C3-Rd, Pamasawata III, Kaunlaran Village, Caloocan City Mobile No.: 09052165338 Email Address: kingperth.pepito@tup.edu.ph |  |
| --- | --- |

SKILLS

•	Computer Literate

•	Project Management

•	Attention to detail

•	Accountability and Responsibility

EDUCATIONAL BACKGROUND

•	Bachelor of Engineering Technology	August 2023

Major in Computer Engineering Technology

Technological University of the Philippines – Manila

•	Technical-Vocational-Livelihood – ICT	June 2021

Arellano University – Jose Rizal Campus

•	Secondary School	June 2017

Longos National High School

•	Primary School	June 2012

Ninoy Aquino Elementary School

CONFERENCES/SEMINARS/TRAININGS ATTENDED

•	CWTS “Civic Welfare Training Service”	August 2024

Technological University of the Philippines – Manila

MEMBERSHIP IN PROFESSIONAL ORGANIZATIONS

•	ICPET “Institute of Computer Engineering Technologist”	August 2024

Technological University of the Philippines – Manila

•	MOAS “Ministry of Altar Servers”	February 2017

Vice Coordinator

San Roque Cathedral Parish

CURRICULUM VITAE

| GENESIS WALTER D. REYES II 808 San Jose Street, Manuyo Uno, Las Piñas City Mobile No.: 09766208735 Email Address: genesiswalter.reyesii@tup.edu.ph |  |
| --- | --- |

SKILLS

•	Computer Literate

•	Project Management

•	Attention to detail

•	Accountability and Responsibility

•	Proficient in these programming languages:

◦	C

◦	Python

◦	Java

•	Proficient in Electrical Circuits

•	Proficient in Electronics

EDUCATIONAL BACKGROUND

•	Bachelor of Engineering Technology	2023–2025

Major in Computer Engineering Technology

Technological University of the Philippines – Manila

•	Senior High School (STEM Strand)	2021–2023

Science, Technology, Engineering, Mathematics.

St. Joseph’s Academy, Las Piñas City

•	Junior High School	2017–2020

St. Joseph’s Academy, Las Piñas City

CONFERENCES/SEMINARS/TRAININGS ATTENDED

•	Beyond Pixels: Android and Visual Studio	2024

Android and Visual Studio Mobile Development

# APPENDICES

## Appendix A

Gantt Chart

## Appendix B

Bill of Materials of the Smart-Bin Prototype

| Component | Quantity | Unit price | Amount |
| --- | --- | --- | --- |
| ESP32 DevKit V1 (38-pin, WROOM-32D) | 2 | ₱250.00 | ₱500.00 |
| HC-SR04 ultrasonic sensor | 2 | ₱45.00 | ₱90.00 |
| FC-51 IR obstacle avoidance sensor | 2 | ₱35.00 | ₱70.00 |
| SG90 micro servo motor (180°) | 2 | ₱65.00 | ₱130.00 |
| 2.4-in. TFT touch display (ILI9341 + XPT2046) | 2 | ₱280.00 | ₱560.00 |
| BMI160 six-axis inertial sensor module | 2 | ₱80.00 | ₱160.00 |
| BME680 environmental sensor | 2 | ₱400.00 | ₱800.00 |
| HX711 load cell amplifier board | 2 | ₱60.00 | ₱120.00 |
| 5kg straight bar load cell | 2 | ₱120.00 | ₱240.00 |
| KY-016 RGB LED module (common cathode) | 2 | ₱8.00 | ₱16.00 |
| MH-FMD passive buzzer module | 2 | ₱12.00 | ₱24.00 |
| 18650 battery shield, 4-cell (Bin A) | 1 | ₱310.00 | ₱310.00 |
| 18650 battery shield, 2-cell (Bin B) | 1 | ₱206.00 | ₱206.00 |
| 18650 Li-ion cell, 3.7V 2500mAh | 6 | ₱203.00 | ₱1,218.00 |
| Ceramic capacitor 100nF (pack of 10) | 2 | ₱75.00 | ₱150.00 |
| Electrolytic capacitor 10µF (pack of 10) | 1 | ₱39.00 | ₱39.00 |
| Electrolytic capacitor 1000µF (pack of 10) | 1 | ₱165.00 | ₱165.00 |
| Resistor 1kΩ (pack of 10) | 1 | ₱19.00 | ₱19.00 |
| Resistor 2.2kΩ (pack of 10) | 1 | ₱19.00 | ₱19.00 |
| Resistor 10kΩ (pack of 10) | 1 | ₱19.00 | ₱19.00 |
| Smart-Bin building materials | 2 | ₱250 | ₱500 |
| Universal PCB and materials | 2 | ₱300 | ₱600 |
| Total |  |  | ₱5,955.00 |

Note. Quantities apply to both bins. The market prices as of the check in October 2026. ₱1,191.00 per person.

## Appendix C

User Interface Screens

This appendix shows the two user interfaces of the Smart-Bin: the web dashboard served by the ESP32 and the Android app. The screens were captured from the actual software, the dashboard in a desktop browser and app version 2.2 on an Android emulator. Both ran on their built-in demo data, so the bin names and readings in the figures are sample values and not measurements. Figures C1 to C16 cover the web dashboard, from the start page and operator PIN through the Overview, Charts, Settings, Simulation and Alerts tabs. Figures C17 to C29 cover the same functions in the Android app, along with its app settings, connection and Bluetooth setup pages.

Figure C1

Start Page of the Web Dashboard

The dashboard opens on this page every time. The user picks a role first. Admin / Operator can change settings, calibrate, simulate and add bins, and it asks for a PIN. Collector is the default and can see everything but only acts on pickups, locks and Bluetooth setup. The DEMO key at the bottom adds three made-up bins that act out every alert by themselves, and it can only be switched on or off here.

Figure C2

Creating the Operator PIN

The first time Admin / Operator is picked, the dashboard asks for a new PIN of 4 to 8 digits, typed twice. Only a salted hash of the PIN is stored in the browser. After 5 wrong tries the keypad waits 30 s, and the wait doubles each time. The dashboard also drops back to collector after 2 min with the tab hidden.

Figure C3

Overview Tab with Bin Cards and Collection Priority

Each bin has one card. The large number is the fill level and the bar under it marks 50% and the alert limit (80% here). The chips show the lid, the bin status, the link in use, and whether simulation, calibration or motion is active. The four tiles show temperature, humidity, weight and the air quality index from the BME680, and a tile turns red when its reading passes its limit. The keys at the bottom send LOCK or UNLOCK, RESET TAMPER and COLLECT to the bin.

Collection Priority lists the bins in the order they should be emptied: full bins first, then the bins that will fill soonest, from the fill rate of the last 15 min. The tags NOW, SOON, TODAY, LATER and IDLE are the same in the phone app. The header shows the unread alerts, the system mode (ALERT here, because bins are full), the link (Demo data), the role, and the sound and theme keys.

Figure C4

Overview Tab in Grid View

With 3 or more bins the cards can be switched to small tiles with the grid key, so many bins fit on one screen. Each tile shows the fill level, the name, the location and the status. The four filter keys above the bins show all bins, only the bins that need attention, bins in simulation, or bins being calibrated.

Figure C5

Bin Detail Opened from the Grid

Clicking a tile opens the full card of that bin over the page, with the same readings and keys as the Overview card. Prev and Next step through the bins without closing it.

Figure C6

Charts Tab

The Fill Level chart shows the fill of every bin over the last 15 min, 1 h, 6 h or 24 h, with the alert limit as a dashed red line. One bin or all bins can be picked. The Environment chart shows temperature, humidity and air quality of one bin. In the ALL view each reading is drawn as a share of its own alert limit, so the dashed line at 100% is the limit for all three. Picking one reading, or PRESSURE, draws it in its real unit instead.

Figure C7

Settings Tab in Calibration Mode: Fill Range, Alert Limits and Air Sensor

An operator switches on Calibration mode for one bin. The page and the bin's own screen turn teal, and the fields can then be edited. The first groups are Fill range (the distances for empty and full, with USE NOW to take the live sensor reading), Alert limits, and Air sensor (temperature offset against a reference thermometer and the gas baseline reset). A collector sees the same values but cannot change them.

Figure C8

Settings Tab in Calibration Mode: Tilt Sensor, Lid, Weight and Touchscreen

The rest of the calibration page holds Tilt sensor (the live tilt, the board’s pitch and roll, the saved upright angle with USE NOW, and the switch that takes the upright again at every power-up), Lid and lock (servo angles, Collect PIN, how long the lid stays open and how long it stays closed, buzzer time, auto-lock when full), Weight (tare and calibration with a known weight) and Touchscreen (touch calibration). SAVE SETTINGS sends the values to the bin, which keeps them after a restart.

Figure C9

Bluetooth Setup Panel

This panel gives a bin a new Wi-Fi network over Bluetooth LE. The steps are shown across the top: pair with the 6-digit code that the bin shows on its screen, pick a network from the bin's scan, send the name and password, then switch Bluetooth off. Browsers allow Bluetooth only on secure pages, so this works when the dashboard is opened from a laptop and not from the bin's own http page; the panel says so.

Figure C10

Cloud Backup Panel for Firebase and ThingSpeak

The Firebase account (database URL, Web API key, email and password) is entered here. TEST signs in and checks access, TEST AND SAVE keeps the account, and SEND TO BINS gives it to every bin on Wi-Fi so they all write to the same database. The rows below show what each bin last sent. Local Wi-Fi is always tried first; Firebase and then ThingSpeak are used only when a bin does not answer on Wi-Fi.

Figure C11

Operator Access Panel

The operator can change the PIN, go back to collector, or return to the start page from here.

Figure C12

Demo Options Panel

Shown only while DEMO is on. The demo bins can be paused or sped up, made to act out a full bin, a tamper, bad air or an emptied bin, and made to answer through Wi-Fi, Firebase, ThingSpeak or not at all. This shows every alert and link state in class without the hardware.

Figure C13

Sound Test Panel

Every sound the dashboard plays is listed by group, from alerts to key clicks. Pressing a key plays it even with sounds switched off. The phone app has the same sounds under the same names.

Figure C14

Simulation Tab with One Sensor in Manual

Each sensor row is AUTO, where the bin reads its real hardware, or MANUAL, where it holds the value typed here. In this figure the fill level is held at 60%. APPLY sends the rows to the bin over Wi-Fi, and ALL BACK TO AUTO ends the simulation. While any row is manual the whole dashboard turns violet and the mode chip says SIMULATION, so simulated readings are never mistaken for real ones.

Figure C15

Alerts Tab

The alert history lists every alert with its bin, time and severity (CRITICAL, WARNING, INFO or RESOLVED). The symbols filter by kind (full bins, sensor limits, tamper, lock, pickups and calibration) and the keys beside them by date. MARK READ and DELETE act only on the alerts shown. A dot marks an unread alert.

Figure C16

Overview Tab in Dark Theme

The theme key in the header switches between the light and dark themes. Both use the same layout and colors for each status.

Figure C17

Start Page and Operator PIN in the Android App

The app opens on the same start page as the dashboard, with the Admin / Operator and Collector roles and the DEMO key. The left screen has DEMO on. Picking Admin / Operator the first time opens the sheet on the right, where the operator PIN is typed once and then again to confirm it. The app keeps its own operator PIN, separate from the Collect PIN of the bins.

Figure C18

Overview Tab with Banners, Collection Priority and Bin Grid

Banners at the top list what needs action now: here two bins are full (COLLECT NOW), the demo tour is running, and demo mode is on. Collection Priority orders the bins the same way as the dashboard, with the time to full and the NOW, TODAY and LATER tags. Below it the bins are shown as a grid; the filter keys show all bins, only the bins that need action, or bins in simulation or calibration, and the legend explains the tile colors. The badge on the Alerts tab counts unread alerts.

Figure C19

Starting and Finishing a Pickup from a Bin Card

In list view each bin has a full card like the dashboard's. On the left, COLLECT was pressed on Demo Bin 2: the lid chip says OPEN, the status chip says PICKUP, and the COLLECT key turned into DONE. On the right, DONE was pressed: the lid is closed, the status is back to OK and the fill reads 0%. The air quality tile on the left is red because the index (206) is over its limit of 200.

Figure C20

Needs Attention Sheet and Alert History

Tapping the unread pill in the header opens the Needs attention sheet on the left. It lists the unread alerts with their colors, then the conditions that are true right now, with MARK ALL READ and OPEN ALERTS. The Alerts tab on the right keeps the full history, filtered by kind and by date in the same way as the dashboard. MARK READ and DELETE act on the alerts shown, and the cross on a row removes that one alert.

Figure C21

Charts Tab in the Android App

The Fill Level and Environment charts match the dashboard: a time range of 15 min to 24 h, one bin or all bins, and dashed lines for the alert limits. The key at the top right of each chart turns the phone sideways and shows the chart full screen.

Figure C22

Settings Tab with Calibration Mode Off and On

On the left the settings of Demo Bin 2 are locked; values can be read but not changed. The Collect PIN card at the top shows the PIN the collector types on the bin. On the right an operator has switched on Calibration mode, and the whole app turns teal with CALIBRATING in the header. The other bins are greyed out because only one bin is calibrated at a time. The app reads calibration mode back from the bin, so the dashboard and the bin’s screen turn teal at the same moment, whichever of the three started it.

Figure C23

Calibration Settings: Fill Range, Alert Limits and Air Sensor

Each value has minus and plus keys. Fill range sets the distances from the ultrasonic sensor to the bottom of an empty bin and to the full line; USE NOW copies the live reading, which the hint shows (29.8cm here). Alert limits sets the fill, temperature, humidity, air quality and tilt limits and the weight capacity. The air sensor group corrects the BME680: a reference thermometer reading sets the temperature offset, humidity and pressure have their own offsets, and RESET GAS BASELINE restarts the gas sensor's clean-air learning.

Figure C24

Calibration Settings: Tilt Sensor, Lid, Weight and Touchscreen

Tilt sensor (BMI160) shows how far the bin leans from upright and the board’s pitch and roll right now. The saved upright pitch and roll can be typed in or taken from the live reading with USE NOW, and the switch below takes the upright again every time the bin is powered up, which is on by default. Lid and lock holds the servo angles, the Collect PIN, how long the lid stays open after motion and how long it then stays closed (5s and 2s), the minimum buzzer time and the option to lock the lid when the bin is full. Weight calibration is done with TARE / ZERO on the empty bin, then CALIBRATE with a known weight inside (1000g here). Touchscreen starts the four-cross touch calibration on the bin or resets it.

Figure C25

Simulation Tab in the Android App

Each sensor of the selected bin can be set to MANUAL with its own value, as on the dashboard. APPLY sends the values to the bin over Wi-Fi and ALL TO AUTO ends the simulation. While any sensor is manual the whole app turns violet.

Figure C26

App Settings

The gear key opens the settings of the phone app itself. Sound and Vibration turn the key sounds and haptics on or off, with three vibration strengths and a sound test. Appearance follows the phone (Auto) or forces light or dark. The second screen shows the signed-in role, the operator PIN, the Connection and Bluetooth setup pages, and the background options: push alerts while the app is closed, and unrestricted battery so Android does not pause the app.

Figure C27

Connection Page

This page shows how the app reaches the bins, in order: local Wi-Fi is always tried first and read every second; Firebase is used when no bin answers on Wi-Fi and is read every 5 s; ThingSpeak is the last route, read every 15 s, with LOCK, UNLOCK and COLLECT sent through TalkBack. Over the clouds only those three commands are allowed. In this figure the demo options pretend that Wi-Fi and Firebase are down, so ThingSpeak is IN USE and the header chip turns blue. The second screen lists the route each bin is using and the Firebase account fields.

Figure C28

Bluetooth Setup Page and Bin Settings Sheet

Bluetooth setup gives a bin a new Wi-Fi network: PAIR A BIN, type the 6-digit code shown on the bin's screen, pick the network, enter its password and SEND WI-FI, then turn the bin's Bluetooth off. With a demo bin the app shows a stand-in for the bin's screen. The sheet on the right opens from a bin's SET key and holds its name, location, IP address, Firebase cloud name and ThingSpeak keys. The values shown are the demo bin's placeholders, not real addresses or keys.

Figure C29

Overview in List View, Light and Dark Themes

The same Overview in list view in the light theme (left) and the dark theme (right). Appearance in App settings picks the theme, or follows the phone. The banners, chips and tiles keep the same colors in both themes.

## Appendix D

Source Code

This appendix lists every source file of the Smart-Bin system, in full, from the main branch of the group's repository, https://github.com/GenReyes/Smart-Bin-Embedded-Systems. Part 1 is the firmware that runs on both ESP32 boards, with the Firebase security rules and the script that packs the web dashboard into the firmware. Part 2 is the web dashboard, one HTML file holding its markup, CSS and JavaScript, which the bin serves over Wi-Fi. Part 3 is the Kotlin code of the Android app, grouped into data (talking to the bins and the clouds), notify (sounds, vibration and notifications) and ui (the screens). Part 4 is the app's manifest and XML resources. Part 5 is the Gradle build files, the GitHub Actions workflow that builds the APK, and the two development tools: the sound generator and the mock server used to test the app without hardware.

No passwords or keys appear in the code. They are kept in secrets.h on the board and typed into the app or dashboard at run time; secrets.example.h in Part 1 is the layout of secrets.h with placeholders in place of the real values. dashboard_gz.h is not printed: embed_dashboard.py generates it from the Part 2 file, and it holds the same dashboard as 3,965 lines of compressed bytes. Sound, font, image and other binary files are not code and are not listed.

Part 1. Firmware (ESP32)

firmware/SmartBin/SmartBin.ino

1	// ============================================================================

2	// SmartBin.ino - entry point

3	// Starts every module in a fixed order, then hands control to FreeRTOS tasks.

4	// The Arduino loop() is not used: it deletes itself, so all work happens in

5	// the tasks created below. If the board resets in a loop, open the Serial

6	// Monitor at 115200 baud; the first line printed here shows the bin ID and

7	// firmware version, and a crash before it points to a module's init().

8	// ============================================================================

9	// Smart-Bin ESP32 firmware. Group 2, BET-CpET 4A-M, TUP Manila.

10	// Before uploading: copy secrets.example.h to secrets.h and fill it in,

11	// and set BIN_NUMBER in config.h (1 for Bin A, 2 for Bin B).

12

13	#include "config.h"

14	#include "state.h"

15	#include "sensors.h"

16	#include "control.h"

17	#include "buzzer.h"

18	#include "net.h"

19	#include "cloud.h"

20	#include "ui.h"

21

22	void setup() {

23	  Serial.begin(115200);

24	  delay(200);

25	  Serial.printf("\nSmart-Bin %s firmware v%s\n", BIN_ID, FW_VERSION);

26

27	  // Order matters: the state module creates the mutex and queues every other

28	  // module uses, so it must run first. The UI comes next so the screen shows

29	  // the boot page while the sensors are still being probed.

30	  stateInit();

31	  uiInit();

32	  buzzerInit();

33	  controlInit();

34	  sensorsInit();

35	  netInit();

36

37	  // Task layout: name, stack bytes, priority, core. Core 1 runs the time

38	  // critical work (control has the highest priority so the lid and alarms

39	  // react first); core 0 runs the slow network jobs next to the Wi-Fi stack.

40	  // A "stack canary" or "stack overflow" reset names the task whose stack

41	  // size here is too small.

42	  xTaskCreatePinnedToCore(uiTask, "ui", 8192, nullptr, 2, &hUi, 1);

43	  xTaskCreatePinnedToCore(cloudTask, "cloud", 12288, nullptr, 1, &hCloud, 0);

44	  xTaskCreatePinnedToCore(pushTask, "push", 4096, nullptr, 2, &hPush, 0);

45	  xTaskCreatePinnedToCore(controlTask, "control", 4096, nullptr, 4, &hControl, 1);

46	  xTaskCreatePinnedToCore(sensorTask, "sensor", 4096, nullptr, 3, &hSensor, 1);

47

48	  // Interrupts only wake a task; no work is done inside the ISR itself.

49	  attachInterrupt(digitalPinToInterrupt(PIN_PIR), isrPir, CHANGE);

50	  attachInterrupt(digitalPinToInterrupt(PIN_TOUCH_IRQ), isrTouch, FALLING);

51	#if USE_IMU_INT

52	  pinMode(PIN_IMU_INT, INPUT);

53	#endif

54

55	  // The local web server starts before Wi-Fi joins so it is ready the moment

56	  // an IP address is given (see onWifiEvent in net.cpp).

57	  startWebServer();

58	  wifiConnect();

59	}

60

61	// Not used. Deleting the loop task frees its stack for the FreeRTOS tasks.

62	void loop() { vTaskDelete(nullptr); }

firmware/SmartBin/config.h

1	// ============================================================================

2	// config.h - build-time settings for one bin

3	// Pins, timing constants and the Bluetooth UUIDs. Values that users may

4	// change at run time (thresholds, PIN, sounds) live in struct Cal in state.h

5	// instead and are saved in flash.

6	// ============================================================================

7	#pragma once

8	#include <Arduino.h>

9

10	// Which bin this board is: 1 = Bin A (bin01), 2 = Bin B (bin02).

11	// Change BIN_NUMBER before flashing the second board. Both bins running the

12	// same number will overwrite each other's data in Firebase and ThingSpeak.

13	#ifndef BIN_NUMBER

14	#define BIN_NUMBER 1

15	#endif

16

17	#define FW_VERSION "1.0.0"

18

19	// Pins, from the Implementation Plan v2 pin table.

20	// GPIO 34 to 39 are input only on the ESP32, which is why the input lines

21	// SPI MISO 35, touch IRQ 36 and IMU INT 34 sit there. The buzzer is on GPIO 4,

22	// not GPIO 12: GPIO 12 is a boot strapping pin, and with the buzzer holding it

23	// high the ESP32 would not boot.

24	constexpr int PIN_TRIG = 5;

25	constexpr int PIN_ECHO = 18;

26	constexpr int PIN_BUZZER = 4;

27	constexpr int PIN_SERVO = 13;

28	constexpr int PIN_HX_SCK = 14;

29	constexpr int PIN_HX_DT = 32;

30	constexpr int PIN_TOUCH_CS = 15;

31	constexpr int PIN_TFT_DC = 16;

32	constexpr int PIN_TFT_CS = 17;

33	constexpr int PIN_SPI_MOSI = 19;

34	constexpr int PIN_SPI_SCK = 23;

35	constexpr int PIN_SPI_MISO = 35;

36	constexpr int PIN_TOUCH_IRQ = 36;

37	constexpr int PIN_SDA = 21;

38	constexpr int PIN_SCL = 22;

39	constexpr int PIN_LED_R = 25;

40	constexpr int PIN_LED_G = 26;

41	constexpr int PIN_LED_B = 33;

42	constexpr int PIN_PIR = 27;

43	constexpr int PIN_IMU_INT = 34;     // optional BMI160 INT1 wire; set USE_IMU_INT to 1 once it is wired

44

45	// Hardware polarity switches. If the LED shows the wrong colors or the buzzer

46	// is silent while the Serial log says it should sound, flip these.

47	#define USE_IMU_INT 0

48	#define BUZZER_ACTIVE_LOW 0          // 1 for "low level trigger" modules

49	#define LED_ACTIVE_LOW 0             // 1 for a common-anode RGB LED

50

51	// Touch calibration (raw XPT2046 values at the screen edges). Print raw values

52	// with TOUCH_DEBUG 1, press the four corners, and copy the numbers here.

53	#define TOUCH_DEBUG 0

54	constexpr int TS_RAW_X0 = 3800, TS_RAW_X1 = 300;   // raw x at screen x = 0 and x = 239

55	constexpr int TS_RAW_Y0 = 3800, TS_RAW_Y1 = 300;   // raw y at screen y = 0 and y = 319

56	#define TS_SWAP_XY 0

57	constexpr uint8_t TFT_ROTATION = 2;                // 0 = portrait 240 x 320

58

59	// Timing (ESP32 Integration Guide, sections 3 and 6A).

60	// All times are in milliseconds unless the name ends in _S. FB_ = Firebase,

61	// TS_ = ThingSpeak channel writes, TB_ = ThingSpeak TalkBack polling,

62	// BT_ = Bluetooth Wi-Fi setup. TS_MIN_GAP_MS keeps writes above the 15 s

63	// limit of the free ThingSpeak plan; a write sent faster than that returns 0.

64	constexpr uint32_t FB_WATCHED_MS = 5000;

65	constexpr uint32_t FB_IDLE_MS = 60000;

66	constexpr uint32_t FB_CMD_WATCHED_MS = 2000;

67	constexpr uint32_t FB_CMD_IDLE_MS = 10000;

68	constexpr uint32_t FB_WATCHERS_MS = 30000;

69	constexpr uint32_t FB_USAGE_MS = 600000;

70	constexpr uint32_t TS_WRITE_MS = 30000;

71	constexpr uint32_t TS_MIN_GAP_MS = 15500;

72	constexpr uint32_t TS_HEARTBEAT_MS = 300000;

73	constexpr uint32_t TB_WATCHED_MS = 5000;

74	constexpr uint32_t TB_IDLE_MS = 15000;

75	constexpr uint32_t CMD_MAX_AGE_MS = 60000;

76	constexpr uint32_t TAMPER_COUNTDOWN_S = 10;

77	constexpr uint32_t TAMPER_ALARM_EVERY_MS = 2000;   // latched tamper siren repeats this often

78	constexpr uint32_t SCREEN_TIMEOUT_MS = 30000;

79	constexpr uint32_t BT_IDLE_OFF_MS = 300000;

80	constexpr uint32_t BT_CODE_MS = 60000;

81	constexpr uint32_t BT_LOCKOUT_MS = 60000;

82	// 10 GiB, the monthly download quota of the Firebase free plan. Used only to

83	// estimate usage; Firebase does not report it to the device.

84	constexpr uint64_t FB_MONTH_LIMIT = 10737418240ULL;

85

86	// 0 = always use the fast "someone is watching" pace. Set to 1 once the app writes

87	// /sb/watchers (Integration Guide 6A); the dashboard already does.

88	#define FB_REQUIRE_WATCHERS 0

89

90	// Bluetooth service used by the app and dashboard for Wi-Fi setup. These must

91	// match the UUIDs in the Android app (data/Ble.kt) and the web dashboard.

92	#define BT_SVC_UUID  "b1a70001-5c2e-4f8a-9d33-0a9c2b6f17c1"

93	#define BT_SSID_UUID "b1a70002-5c2e-4f8a-9d33-0a9c2b6f17c1"

94	#define BT_PASS_UUID "b1a70003-5c2e-4f8a-9d33-0a9c2b6f17c1"

95	#define BT_CTRL_UUID "b1a70004-5c2e-4f8a-9d33-0a9c2b6f17c1"

96	#define BT_STAT_UUID "b1a70005-5c2e-4f8a-9d33-0a9c2b6f17c1"

97	#define BT_SCAN_UUID "b1a70006-5c2e-4f8a-9d33-0a9c2b6f17c1"

98

99	// Names derived from BIN_NUMBER. BIN_HOST gives http://smartbin-01.local on

100	// networks that support mDNS.

101	#if BIN_NUMBER == 1

102	#define BIN_ID "bin01"

103	#define BIN_LABEL "BIN 01"

104	#define BIN_HOST "smartbin-01"

105	#define BIN_BLE_NAME "SmartBin-01"

106	#else

107	#define BIN_ID "bin02"

108	#define BIN_LABEL "BIN 02"

109	#define BIN_HOST "smartbin-02"

110	#define BIN_BLE_NAME "SmartBin-02"

111	#endif

firmware/SmartBin/secrets.example.h

1	// ============================================================================

2	// secrets.example.h - template for secrets.h

3	// The firmware includes "secrets.h", not this file. A build error saying

4	// secrets.h is missing means this copy step was skipped.

5	// ============================================================================

6	#pragma once

7

8	// Copy this file to secrets.h in the same folder and fill it in.

9	// secrets.h is git-ignored. Never commit real keys, passwords or the Collect PIN.

10

11	// Wi-Fi (2.4 GHz only). Can also be changed later from WI-FI SETUP on the touchscreen.

12	#define WIFI_SSID "your-wifi-name"

13	#define WIFI_PASSWORD "your-wifi-password"

14

15	// Firebase Realtime Database. Empty = Firebase off (local Wi-Fi only).

16	// See firmware/README.md section 4. The bin signs in with its own account,

17	// for example bin01@your-project.test.

18	#define FIREBASE_URL ""          // https://<project>-default-rtdb.asia-southeast1.firebasedatabase.app

19	#define FIREBASE_API_KEY ""      // AIza... (Project settings > Web API key)

20	#define FIREBASE_EMAIL ""

21	#define FIREBASE_PASSWORD ""

22

23	// ThingSpeak, one channel per bin. Empty = ThingSpeak off. See firmware/README.md section 5.

24	#define THINGSPEAK_SERVER "https://api.thingspeak.com"

25	#define THINGSPEAK_WRITE_KEY_BIN_A ""

26	#define THINGSPEAK_WRITE_KEY_BIN_B ""

27	#define THINGSPEAK_TALKBACK_ID_BIN_A ""

28	#define THINGSPEAK_TALKBACK_KEY_BIN_A ""

29	#define THINGSPEAK_TALKBACK_ID_BIN_B ""

30	#define THINGSPEAK_TALKBACK_KEY_BIN_B ""

31

32	// Default Collect PIN (4 digits). It can be changed later from the dashboard or app.

33	#define COLLECT_PIN "0000"

firmware/SmartBin/state.h

1	// ============================================================================

2	// state.h - shared data model

3	// Every task reads and writes the same BinState (st), calibration (cal) and

4	// simulation (sim) objects. Always wrap access in lockState()/unlockState(),

5	// or take a copy with snapshot()/calSnapshot()/simSnapshot(). Reading st

6	// without the lock can show half-updated values on the screen or dashboard.

7	// ============================================================================

8	#pragma once

9	#include <Arduino.h>

10	#include <ArduinoJson.h>

11	#include "config.h"

12

13	// The one status shown everywhere. Priority, highest first, is decided in

14	// controlTask: CALIBRATING > COLLECTING > TAMPER > LOCKED > OK.

15	enum BinStatus : uint8_t { ST_OK = 0, ST_LOCKED, ST_TAMPER, ST_COLLECTING, ST_CALIBRATING };

16	extern const char* const STATUS_NAMES[5];

17

18	// User settings, saved in flash as one binary blob ("cal"). Distances are in

19	// cm, weights in grams, temperatures in deg C, gas_th is the IAQ index limit,

20	// quiet_from/quiet_to are minutes after midnight (local time, UTC+8).

21	// Adding or reordering a field changes sizeof(Cal), and loadCal() then

22	// ignores the old saved blob and falls back to these defaults.

23	struct Cal {

24	  float d_empty = 30, d_full = 5, fill_th = 80, temp_th = 40, hum_th = 85, gas_th = 200, tilt_th = 30, weight_cap = 4500;

25	  uint8_t auto_lock = 0;

26	  int16_t servo_closed = 0, servo_open = 90;

27	  uint16_t collect_pin = 0;

28	  uint16_t pir_hold_s = 5, buzz_s = 2;

29	  float temp_off = -2, hum_off = 0, press_off = 0;

30	  uint8_t buzz_on = 1, buzz_vol = 2, buzz_ui = 1, buzz_lid = 0;

31	  uint16_t buzz_repeat_s = 30;

32	  uint8_t quiet_on = 0;

33	  uint16_t quiet_from = 1320, quiet_to = 360;

34	};

35

36	// Raw XPT2046 readings at the screen edges, set by the touch calibration.

37	struct TouchCal {

38	  uint8_t swap = TS_SWAP_XY;

39	  int16_t x0 = TS_RAW_X0, x1 = TS_RAW_X1, y0 = TS_RAW_Y0, y1 = TS_RAW_Y1;

40	};

41	extern TouchCal touchCal;

42	extern bool touchCalCustom;

43	void saveTouchCal();

44	void loadTouchCal();

45	void resetTouchCal();

46

47	// Simulation overrides for testing without real waste. mask bits:

48	// 1 fill, 2 weight, 4 gas, 8 temperature and humidity, 16 tilt (tamper),

49	// 32 motion, 64 lid. A set bit replaces that sensor's value in the view.

50	struct Sim {

51	  uint8_t mask = 0;

52	  float fill = 60, weight = 2000, gas = 120, temp = 28, humidity = 60;

53	  bool tamper = false, motion = false, lid = false;

54	};

55

56	// One set of sensor values. iaq_acc is the air-quality accuracy:

57	// 0 warming up, 1 low, 2 medium, 3 high.

58	struct Reading {

59	  float fill = 0, distance_cm = 0, weight_g = 0, temp = 0, temp_raw = 0, humidity = 0, pressure = 0, iaq = 0, tilt = 0, gas_k = 0;

60	  uint8_t iaq_acc = 0;

61	  bool motion = false;

62	};

63

64	struct LastCmd {

65	  char cmd[20] = "";

66	  uint64_t ts = 0;

67	  bool ok = false;

68	};

69

70	// The live state of the bin. raw holds sensor values, v holds what users see

71	// (raw with simulation applied by recomputeView()). The ok* flags are false

72	// while a sensor is not answering; they drive the "faults" list in the JSON.

73	struct BinState {

74	  Reading raw;            // what the sensors read

75	  Reading v;              // what everyone sees (raw with simulation applied)

76	  bool lidOpen = false;

77	  BinStatus status = ST_OK;

78	  bool locked = false, collecting = false, calibrating = false, autoLocked = false;

79	  uint8_t tamperPhase = 0;            // 0 none, 1 countdown, 2 latched

80	  uint32_t tamperStartMs = 0;

81	  bool okHC = false, okHX = false, okBME = false, okIMU = false;

82	  bool booting = true;

83	  bool wifi = false, cloudOk = false, tsOk = false, btOn = false;

84	  int rssi = 0;

85	  uint32_t lidCloseAtMs = 0;          // 0 = no countdown; lid holds while motion is high

86	  bool alarmFull = false, alarmEnv = false, buzzerOn = false;

87	  uint32_t buzzerUntilMs = 0;

88	  float fillRatePerHour = 0;

89	  uint8_t calSteps = 0;               // bit 0 tare, bit 1 empty distance, bit 2 full distance

90	  uint32_t doneAtMs = 0;              // when COLLECT_DONE ran, for the A5 screen

91	  LastCmd last;

92	  uint32_t version = 0;

93	  uint32_t calVersion = 0;

94	};

95

96	// Where a command came from. Results are written back only for Firebase

97	// commands (they carry a ref key); screen commands skip some beeps because

98	// the screen already gave feedback.

99	enum CmdSource : uint8_t { SRC_LOCAL = 0, SRC_SCREEN, SRC_FIREBASE, SRC_TALKBACK };

100

101	struct Command {

102	  char cmd[20];

103	  float grams;

104	  char sound[16];

105	  uint8_t src;

106	  char ref[28];

107	};

108

109	struct CmdResult {

110	  char cmd[20];

111	  bool ok;

112	  char why[40];

113	  uint8_t src;

114	  char ref[28];

115	  uint64_t ts;

116	};

117

118	struct AlertEv {

119	  char kind[10];

120	  char sev[10];

121	  char title[40];

122	  char meta[72];

123	};

124

125	// Task notification bits. A task waits on these with xTaskNotifyWait, so one

126	// notify wakes it immediately instead of waiting for its next timeout.

127	enum : uint32_t {

128	  EV_PIR = 1 << 0, EV_TOUCH = 1 << 1, EV_IMU = 1 << 2, EV_STATE = 1 << 3, EV_CMD = 1 << 4,

129	  EV_CAL = 1 << 5, EV_WIFI = 1 << 6, EV_ALERT = 1 << 7, EV_RESULT = 1 << 8, EV_BT = 1 << 9,

130	};

131

132	extern BinState st;

133	extern Cal cal;

134	extern Sim sim;

135	extern SemaphoreHandle_t stLock;

136	extern QueueHandle_t cmdQueue, resultQueue, alertQueue;

137	extern TaskHandle_t hControl, hSensor, hUi, hCloud, hPush;

138

139	// Network and cloud credentials. Loaded from flash, falling back to the

140	// values in secrets.h when nothing has been saved yet.

141	struct NetCfg {

142	  String ssid, pass;

143	  String fbUrl, fbKey, fbEmail, fbPass;

144	  String tsServer, tsWriteKey, tbId, tbKey;

145	};

146	extern NetCfg net;

147

148	// --- Lock, copy and publish ---

149	// publish() bumps st.version and wakes the UI, cloud and push tasks.

150	// Call it after any change that users should see.

151	void stateInit();

152	void lockState();

153	void unlockState();

154	BinState snapshot();

155	Cal calSnapshot();

156	Sim simSnapshot();

157	void publish(uint32_t extraBits = 0);

158	void notifyAll(uint32_t bits);

159

160	// --- Flash storage (Preferences namespace "smartbin") ---

161	void saveCal();

162	void loadCal();

163	void saveNet();

164	void loadNet();

165	uint8_t loadTheme();

166	void saveTheme(uint8_t dark);

167	void saveHx(int32_t offset, float scale);

168	bool loadHx(int32_t& offset, float& scale);

169	void saveGasBaseline(float g);

170	float loadGasBaseline();

171	void saveImuRef(float x, float y, float z);

172	bool loadImuRef(float& x, float& y, float& z);

173

174	// --- JSON for the local API, live events and Firebase ---

175	void calToJson(const Cal& c, JsonObject o);

176	bool calFromJson(JsonObjectConst o, Cal& c);

177	void statusToJson(const BinState& s, const Cal& c, const Sim& sm, JsonObject o, bool withCal, bool forCloud);

178	String statusJson(bool withCal = true, bool forCloud = false);

179	String localIp();

180

181	// --- Commands and alerts ---

182	// sendCommand() only queues; controlTask runs it. validateCommand() is also

183	// called by the web server first so it can answer 400/409 right away.

184	bool sendCommand(const char* cmd, uint8_t src, float grams = 0, const char* sound = nullptr, const char* ref = nullptr);

185	bool validateCommand(const char* cmd, const BinState& s, String& why);

186	void raiseAlert(const char* kind, const char* sev, const char* title, const char* meta);

187

188	// --- Time. nowMs() returns 0 until SNTP has set the clock. ---

189	uint64_t nowMs();

190	bool clockSet();

191	int localMinuteOfDay();

192	void monthKeyUtc(char* out, size_t n);

firmware/SmartBin/state.cpp

1	// ============================================================================

2	// state.cpp - shared state, flash storage, JSON and command plumbing

3	// ============================================================================

4	#include "state.h"

5	#include "sensors.h"

6	#include <Preferences.h>

7	#include <WiFi.h>

8	#include <sys/time.h>

9	#include "secrets.h"

10

11	const char* const STATUS_NAMES[5] = {"OK", "LOCKED", "TAMPER", "COLLECTING", "CALIBRATING"};

12

13	BinState st;

14	Cal cal;

15	Sim sim;

16	NetCfg net;

17	SemaphoreHandle_t stLock;

18	QueueHandle_t cmdQueue, resultQueue, alertQueue;

19	TaskHandle_t hControl = nullptr, hSensor = nullptr, hUi = nullptr, hCloud = nullptr, hPush = nullptr;

20

21	// Preferences (NVS flash) is not safe to use from two tasks at once, so every

22	// access goes through prefsBegin()/prefsEnd(), which hold prefsLock.

23	static Preferences prefs;

24	static SemaphoreHandle_t prefsLock;

25

26	// Queue depths are small on purpose: if a queue is full, sendCommand()

27	// fails and the caller reports "Busy" instead of blocking a task.

28	void stateInit() {

29	  stLock = xSemaphoreCreateMutex();

30	  prefsLock = xSemaphoreCreateMutex();

31	  cmdQueue = xQueueCreate(8, sizeof(Command));

32	  resultQueue = xQueueCreate(8, sizeof(CmdResult));

33	  alertQueue = xQueueCreate(8, sizeof(AlertEv));

34	  cal.collect_pin = (uint16_t)atoi(COLLECT_PIN);

35	  loadCal();

36	  loadTouchCal();

37	  loadNet();

38	}

39

40	void lockState() { xSemaphoreTake(stLock, portMAX_DELAY); }

41	void unlockState() { xSemaphoreGive(stLock); }

42

43	BinState snapshot() {

44	  lockState();

45	  BinState s = st;

46	  unlockState();

47	  return s;

48	}

49

50	Cal calSnapshot() {

51	  lockState();

52	  Cal c = cal;

53	  unlockState();

54	  return c;

55	}

56

57	Sim simSnapshot() {

58	  lockState();

59	  Sim s = sim;

60	  unlockState();

61	  return s;

62	}

63

64	// Wakes the three tasks that display or send state. A handle is null until

65	// its task has been created in setup().

66	void notifyAll(uint32_t bits) {

67	  TaskHandle_t hs[] = {hUi, hCloud, hPush};

68	  for (TaskHandle_t h : hs)

69	    if (h) xTaskNotify(h, bits, eSetBits);

70	}

71

72	void publish(uint32_t extraBits) {

73	  lockState();

74	  st.version++;

75	  unlockState();

76	  notifyAll(EV_STATE | extraBits);

77	}

78

79	static void prefsBegin(bool ro) {

80	  xSemaphoreTake(prefsLock, portMAX_DELAY);

81	  prefs.begin("smartbin", ro);

82	}

83	static void prefsEnd() {

84	  prefs.end();

85	  xSemaphoreGive(prefsLock);

86	}

87

88	// Calibration is stored as a raw struct. loadCal() only accepts a blob of the

89	// exact current size, so a firmware with a changed Cal struct starts from

90	// defaults instead of reading garbage.

91	void saveCal() {

92	  Cal c = calSnapshot();

93	  prefsBegin(false);

94	  prefs.putBytes("cal", &c, sizeof(c));

95	  prefsEnd();

96	}

97

98	void loadCal() {

99	  prefsBegin(true);

100	  if (prefs.getBytesLength("cal") == sizeof(Cal)) prefs.getBytes("cal", &cal, sizeof(Cal));

101	  prefsEnd();

102	}

103

104	TouchCal touchCal;

105	bool touchCalCustom = false;

106

107	void saveTouchCal() {

108	  prefsBegin(false);

109	  prefs.putBytes("touch", &touchCal, sizeof(touchCal));

110	  prefsEnd();

111	  touchCalCustom = true;

112	}

113

114	void loadTouchCal() {

115	  prefsBegin(true);

116	  if (prefs.getBytesLength("touch") == sizeof(TouchCal)) {

117	    prefs.getBytes("touch", &touchCal, sizeof(TouchCal));

118	    touchCalCustom = true;

119	  }

120	  prefsEnd();

121	}

122

123	void resetTouchCal() {

124	  touchCal = TouchCal();

125	  prefsBegin(false);

126	  prefs.remove("touch");

127	  prefsEnd();

128	  touchCalCustom = false;

129	}

130

131	// Wi-Fi and Firebase settings changed from the screen, Bluetooth setup or

132	// /api/cloud are kept here and survive a reflash of the same partition.

133	void saveNet() {

134	  prefsBegin(false);

135	  prefs.putString("ssid", net.ssid);

136	  prefs.putString("pass", net.pass);

137	  prefs.putString("fbUrl", net.fbUrl);

138	  prefs.putString("fbKey", net.fbKey);

139	  prefs.putString("fbEmail", net.fbEmail);

140	  prefs.putString("fbPass", net.fbPass);

141	  prefsEnd();

142	}

143

144	// ThingSpeak keys come only from secrets.h; each bin picks its own channel.

145	void loadNet() {

146	  prefsBegin(true);

147	  net.ssid = prefs.getString("ssid", WIFI_SSID);

148	  net.pass = prefs.getString("pass", WIFI_PASSWORD);

149	  net.fbUrl = prefs.getString("fbUrl", FIREBASE_URL);

150	  net.fbKey = prefs.getString("fbKey", FIREBASE_API_KEY);

151	  net.fbEmail = prefs.getString("fbEmail", FIREBASE_EMAIL);

152	  net.fbPass = prefs.getString("fbPass", FIREBASE_PASSWORD);

153	  prefsEnd();

154	  net.tsServer = THINGSPEAK_SERVER;

155	#if BIN_NUMBER == 1

156	  net.tsWriteKey = THINGSPEAK_WRITE_KEY_BIN_A;

157	  net.tbId = THINGSPEAK_TALKBACK_ID_BIN_A;

158	  net.tbKey = THINGSPEAK_TALKBACK_KEY_BIN_A;

159	#else

160	  net.tsWriteKey = THINGSPEAK_WRITE_KEY_BIN_B;

161	  net.tbId = THINGSPEAK_TALKBACK_ID_BIN_B;

162	  net.tbKey = THINGSPEAK_TALKBACK_KEY_BIN_B;

163	#endif

164	  if (net.tsWriteKey.startsWith("WRITE_KEY")) net.tsWriteKey = "";

165	}

166

167	uint8_t loadTheme() {

168	  prefsBegin(true);

169	  uint8_t t = prefs.getUChar("theme", 0);

170	  prefsEnd();

171	  return t;

172	}

173

174	void saveTheme(uint8_t dark) {

175	  prefsBegin(false);

176	  prefs.putUChar("theme", dark);

177	  prefsEnd();

178	}

179

180	void saveHx(int32_t offset, float scale) {

181	  prefsBegin(false);

182	  prefs.putInt("hxOff", offset);

183	  prefs.putFloat("hxScale", scale);

184	  prefsEnd();

185	}

186

187	// Load-cell offset and scale from the last TARE and CAL_WEIGHT. Returns false

188	// when the scale was never calibrated (weights will then be rough).

189	bool loadHx(int32_t& offset, float& scale) {

190	  prefsBegin(true);

191	  bool has = prefs.isKey("hxScale");

192	  offset = prefs.getInt("hxOff", 0);

193	  scale = prefs.getFloat("hxScale", 420.0f);

194	  prefsEnd();

195	  return has;

196	}

197

198	void saveGasBaseline(float g) {

199	  prefsBegin(false);

200	  prefs.putFloat("gasBase", g);

201	  prefsEnd();

202	}

203

204	float loadGasBaseline() {

205	  prefsBegin(true);

206	  float g = prefs.getFloat("gasBase", 0);

207	  prefsEnd();

208	  return g;

209	}

210

211	void saveImuRef(float x, float y, float z) {

212	  float v[3] = {x, y, z};

213	  prefsBegin(false);

214	  prefs.putBytes("imuRef", v, sizeof(v));

215	  prefsEnd();

216	}

217

218	bool loadImuRef(float& x, float& y, float& z) {

219	  float v[3];

220	  prefsBegin(true);

221	  bool ok = prefs.getBytesLength("imuRef") == sizeof(v) && prefs.getBytes("imuRef", v, sizeof(v)) == sizeof(v);

222	  prefsEnd();

223	  if (ok) { x = v[0]; y = v[1]; z = v[2]; }

224	  return ok;

225	}

226

227	// Round to 1 or 2 decimals so the JSON stays short and readable.

228	static double r1(float v) { return round(v * 10.0) / 10.0; }

229	static double r2(float v) { return round(v * 100.0) / 100.0; }

230

231	void calToJson(const Cal& c, JsonObject o) {

232	  o["d_empty"] = r1(c.d_empty);

233	  o["d_full"] = r1(c.d_full);

234	  o["fill_th"] = r1(c.fill_th);

235	  o["temp_th"] = r1(c.temp_th);

236	  o["hum_th"] = r1(c.hum_th);

237	  o["gas_th"] = r1(c.gas_th);

238	  o["tilt_th"] = r1(c.tilt_th);

239	  o["weight_cap"] = r1(c.weight_cap);

240	  o["auto_lock"] = c.auto_lock;

241	  o["servo_closed"] = c.servo_closed;

242	  o["servo_open"] = c.servo_open;

243	  o["collect_pin"] = c.collect_pin;

244	  o["pir_hold_s"] = c.pir_hold_s;

245	  o["buzz_s"] = c.buzz_s;

246	  o["temp_off"] = r1(c.temp_off);

247	  o["hum_off"] = r1(c.hum_off);

248	  o["press_off"] = r1(c.press_off);

249	  o["buzz_on"] = c.buzz_on;

250	  o["buzz_vol"] = c.buzz_vol;

251	  o["buzz_ui"] = c.buzz_ui;

252	  o["buzz_lid"] = c.buzz_lid;

253	  o["buzz_repeat_s"] = c.buzz_repeat_s;

254	  o["quiet_on"] = c.quiet_on;

255	  o["quiet_from"] = c.quiet_from;

256	  o["quiet_to"] = c.quiet_to;

257	}

258

259	// Reads one numeric key, accepts numbers or numeric strings, and clamps it to

260	// [lo, hi]. Out-of-range values are clamped, not rejected, so a typo cannot

261	// push the bin into an unusable setting.

262	template <typename T>

263	static bool take(JsonObjectConst o, const char* k, T& dst, float lo, float hi) {

264	  JsonVariantConst v = o[k];

265	  if (v.isNull() || !(v.is<float>() || v.is<int>() || v.is<const char*>())) return false;

266	  float f = v.is<const char*>() ? atof(v.as<const char*>()) : v.as<float>();

267	  if (isnan(f)) return false;

268	  f = constrain(f, lo, hi);

269	  dst = (T)f;

270	  return true;

271	}

272

273	// Applies any known calibration keys. Returns false when the request had none

274	// (the API then answers 400). The empty distance must stay larger than the

275	// full distance or the fill formula divides by zero or goes negative.

276	bool calFromJson(JsonObjectConst o, Cal& c) {

277	  bool any = false;

278	  any |= take(o, "d_empty", c.d_empty, 5, 200);

279	  any |= take(o, "d_full", c.d_full, 1, 100);

280	  any |= take(o, "fill_th", c.fill_th, 10, 100);

281	  any |= take(o, "temp_th", c.temp_th, 10, 85);

282	  any |= take(o, "hum_th", c.hum_th, 10, 100);

283	  any |= take(o, "gas_th", c.gas_th, 50, 500);

284	  any |= take(o, "tilt_th", c.tilt_th, 5, 90);

285	  any |= take(o, "weight_cap", c.weight_cap, 100, 50000);

286	  any |= take(o, "auto_lock", c.auto_lock, 0, 1);

287	  any |= take(o, "servo_closed", c.servo_closed, 0, 180);

288	  any |= take(o, "servo_open", c.servo_open, 0, 180);

289	  any |= take(o, "collect_pin", c.collect_pin, 0, 9999);

290	  any |= take(o, "pir_hold_s", c.pir_hold_s, 1, 120);

291	  any |= take(o, "buzz_s", c.buzz_s, 0, 60);

292	  any |= take(o, "temp_off", c.temp_off, -20, 20);

293	  any |= take(o, "hum_off", c.hum_off, -30, 30);

294	  any |= take(o, "press_off", c.press_off, -100, 100);

295	  any |= take(o, "buzz_on", c.buzz_on, 0, 1);

296	  any |= take(o, "buzz_vol", c.buzz_vol, 1, 3);

297	  any |= take(o, "buzz_ui", c.buzz_ui, 0, 1);

298	  any |= take(o, "buzz_lid", c.buzz_lid, 0, 1);

299	  any |= take(o, "buzz_repeat_s", c.buzz_repeat_s, 10, 300);

300	  any |= take(o, "quiet_on", c.quiet_on, 0, 1);

301	  any |= take(o, "quiet_from", c.quiet_from, 0, 1439);

302	  any |= take(o, "quiet_to", c.quiet_to, 0, 1439);

303	  if (c.d_full >= c.d_empty) c.d_full = c.d_empty - 1;

304	  return any;

305	}

306

307	String localIp() { return WiFi.isConnected() ? WiFi.localIP().toString() : String(""); }

308

309	// Builds the status object used by /api/status, the live event stream and

310	// Firebase. A sensor's values are left out while it is faulty (and not

311	// simulated), so the apps show "--" instead of stale numbers.

312	void statusToJson(const BinState& s, const Cal& c, const Sim& sm, JsonObject o, bool withCal, bool forCloud) {

313	  const Reading& v = s.v;

314	  bool hc = s.okHC || (sm.mask & 1), hx = s.okHX || (sm.mask & 2), bme = s.okBME || (sm.mask & 12) == 12;

315	  o["id"] = BIN_ID;

316	  o["name"] = BIN_LABEL;

317	  if (hc) {

318	    o["fill"] = r1(v.fill);

319	    o["distance_cm"] = r1(v.distance_cm);

320	  }

321	  if (hx) o["weight_g"] = (long)lround(v.weight_g);

322	  if (bme || (sm.mask & 8)) {

323	    o["temp"] = r2(v.temp);

324	    o["temp_raw"] = r2(v.temp_raw);

325	    o["humidity"] = r1(v.humidity);

326	  }

327	  if (s.okBME) o["pressure"] = r1(v.pressure);

328	  if (s.okBME) o["gas_k"] = r1(v.gas_k);

329	  if (s.okBME && v.iaq_acc == 0 && !(sm.mask & 4)) o["iaq_warm_s"] = airWarmLeft();

330	  if (s.okBME || (sm.mask & 4)) {

331	    o["iaq"] = (long)lround(v.iaq);

332	    o["iaq_acc"] = v.iaq_acc;

333	  }

334	  o["tilt"] = r1(v.tilt);

335	  o["motion"] = v.motion;

336	  o["lid"] = s.lidOpen ? "OPEN" : "CLOSED";

337	  o["status"] = STATUS_NAMES[s.status];

338	  o["sim_mask"] = sm.mask;

339	  if (s.tamperPhase) o["tamper_phase"] = s.tamperPhase == 1 ? "COUNTDOWN" : "LATCHED";

340	  JsonArray f = o["faults"].to<JsonArray>();

341	  if (!s.okHC) f.add("HC-SR04");

342	  if (!s.okHX) f.add("HX711");

343	  if (!s.okBME) f.add("BME680");

344	  if (!s.okIMU) f.add("BMI160");

345	  if (withCal) calToJson(c, o["cal"].to<JsonObject>());

346	  if (withCal) {

347	    JsonObject t = o["touch"].to<JsonObject>();

348	    t["swap"] = touchCal.swap;

349	    t["x0"] = touchCal.x0;

350	    t["x1"] = touchCal.x1;

351	    t["y0"] = touchCal.y0;

352	    t["y1"] = touchCal.y1;

353	    t["custom"] = touchCalCustom;

354	  }

355	  if (s.last.cmd[0]) {

356	    JsonObject lc = o["last_cmd"].to<JsonObject>();

357	    lc["cmd"] = s.last.cmd;

358	    lc["ts"] = s.last.ts;

359	    lc["ok"] = s.last.ok;

360	  }

361	  o["uptime_s"] = (uint32_t)(millis() / 1000);

362	  o["rssi"] = s.rssi;

363	  o["fw"] = FW_VERSION;

364	  // ".sv": "timestamp" asks Firebase to fill in its own server time.

365	  if (forCloud) {

366	    o["local_ip"] = localIp();

367	    o["ts"][".sv"] = "timestamp";

368	  }

369	}

370

371	String statusJson(bool withCal, bool forCloud) {

372	  lockState();

373	  BinState s = st;

374	  Cal c = cal;

375	  Sim sm = sim;

376	  unlockState();

377	  JsonDocument doc;

378	  statusToJson(s, c, sm, doc.to<JsonObject>(), withCal, forCloud);

379	  String out;

380	  serializeJson(doc, out);

381	  return out;

382	}

383

384	// Every command the bin accepts. The apps send these exact strings.

385	static const char* const KNOWN[] = {"LOCK", "UNLOCK", "CLEAR_TAMPER", "COLLECT", "COLLECT_DONE", "CALIBRATE_START",

386	                                    "CALIBRATE_END", "TARE", "CAL_WEIGHT", "RESET_GAS", "BUZZ_TEST",

387	                                    "TOUCH_CAL", "TOUCH_CAL_RESET"};

388

389	// Rejects commands that would conflict with the current state. The "why"

390	// text is shown to the user, so keep it short.

391	bool validateCommand(const char* cmd, const BinState& s, String& why) {

392	  bool known = false;

393	  for (const char* k : KNOWN)

394	    if (!strcmp(k, cmd)) known = true;

395	  if (!known) { why = "Unknown command"; return false; }

396	  if (!strcmp(cmd, "COLLECT")) {

397	    if (s.calibrating) { why = "Calibrating"; return false; }

398	    if (s.tamperPhase) { why = "TAMPER active"; return false; }

399	  }

400	  if (!strcmp(cmd, "LOCK") && s.collecting) { why = "Collection in progress"; return false; }

401	  if (!strcmp(cmd, "CALIBRATE_START") && s.collecting) { why = "Collection in progress"; return false; }

402	  if ((!strcmp(cmd, "TARE") || !strcmp(cmd, "CAL_WEIGHT")) && !s.okHX) { why = "Load cell not answering"; return false; }

403	  if (!strcmp(cmd, "RESET_GAS") && !s.okBME) { why = "BME680 not answering"; return false; }

404	  return true;

405	}

406

407	// Queues a command for controlTask and wakes it. Waits at most 50 ms for a

408	// free slot; false means the queue was full.

409	bool sendCommand(const char* cmd, uint8_t src, float grams, const char* sound, const char* ref) {

410	  Command c{};

411	  strlcpy(c.cmd, cmd, sizeof(c.cmd));

412	  c.grams = grams;

413	  c.src = src;

414	  if (sound) strlcpy(c.sound, sound, sizeof(c.sound));

415	  if (ref) strlcpy(c.ref, ref, sizeof(c.ref));

416	  if (xQueueSend(cmdQueue, &c, pdMS_TO_TICKS(50)) != pdTRUE) return false;

417	  if (hControl) xTaskNotify(hControl, EV_CMD, eSetBits);

418	  return true;

419	}

420

421	// Queues an alert for cloudTask, which logs it to Serial as "[alert] ..." and

422	// posts it to Firebase. If alerts never reach the app, check the Serial log

423	// for these lines first, then the Firebase rules.

424	void raiseAlert(const char* kind, const char* sev, const char* title, const char* meta) {

425	  AlertEv a{};

426	  strlcpy(a.kind, kind, sizeof(a.kind));

427	  strlcpy(a.sev, sev, sizeof(a.sev));

428	  strlcpy(a.title, title, sizeof(a.title));

429	  strlcpy(a.meta, meta, sizeof(a.meta));

430	  xQueueSend(alertQueue, &a, 0);

431	  if (hCloud) xTaskNotify(hCloud, EV_ALERT, eSetBits);

432	}

433

434	// The clock counts as set once it is past November 2023 (Unix 1700000000).

435	bool clockSet() { return time(nullptr) > 1700000000; }

436

437	// Philippine time is UTC+8 with no daylight saving, so a fixed offset is used.

438	int localMinuteOfDay() {

439	  if (!clockSet()) return -1;

440	  return (int)(((uint64_t)time(nullptr) + 8 * 3600ULL) / 60ULL % 1440ULL);

441	}

442

443	// Converts today's date to "YYYY-MM" without the C time library (civil-from-

444	// days algorithm). Used as the key for the monthly Firebase usage counter.

445	void monthKeyUtc(char* out, size_t n) {

446	  out[0] = 0;

447	  if (!clockSet()) return;

448	  int64_t z = (int64_t)(time(nullptr) / 86400) + 719468;

449	  int64_t era = z / 146097;

450	  int64_t doe = z - era * 146097;

451	  int64_t yoe = (doe - doe / 1460 + doe / 36524 - doe / 146096) / 365;

452	  int64_t doy = doe - (365 * yoe + yoe / 4 - yoe / 100);

453	  int64_t mp = (5 * doy + 2) / 153;

454	  int m = (int)(mp < 10 ? mp + 3 : mp - 9);

455	  int y = (int)(yoe + era * 400 + (m <= 2));

456	  snprintf(out, n, "%04d-%02d", y, m);

457	}

458

459	uint64_t nowMs() {

460	  if (!clockSet()) return 0;

461	  struct timeval tv;

462	  gettimeofday(&tv, nullptr);

463	  return (uint64_t)tv.tv_sec * 1000ULL + tv.tv_usec / 1000;

464	}

firmware/SmartBin/sensors.h

1	// ============================================================================

2	// sensors.h - sensor task and calibration requests

3	// The request* functions only set a flag; sensorTask does the actual work on

4	// its next pass, so they are safe to call from any task.

5	// ============================================================================

6	#pragma once

7	#include <Arduino.h>

8

9	void sensorsInit();

10	void sensorTask(void*);

11	void recomputeView();

12

13	void requestTare();

14	void requestCalWeight(float grams);

15	void requestGasReset();

16	void requestImuReference();

17	int airWarmLeft();

firmware/SmartBin/sensors.cpp

1	// ============================================================================

2	// sensors.cpp - reads every sensor and keeps st.raw up to date

3	//   HC-SR04 ultrasonic   -> distance and fill %   every 200 ms

4	//   BMI160 accelerometer -> tilt in degrees       every 200 ms (raw I2C)

5	//   HX711 load cell      -> weight in grams       whenever a sample is ready

6	//   BME680               -> temp, humidity, pressure, gas, IAQ  every 3 s

7	// A sensor that stops answering sets its ok* flag to false; controlTask turns

8	// that into a "Sensor not answering" alert and the screen shows "--".

9	// ============================================================================

10	#include "sensors.h"

11	#include "state.h"

12	#include "buzzer.h"

13	#include <Wire.h>

14	#include <Adafruit_BME680.h>

15	#include <HX711.h>

16

17	static Adafruit_BME680 bme(&Wire);

18	static uint8_t imuAddr = 0;

19

20	// --- BMI160 over raw I2C (no library) ---

21	// Register map from the BMI160 datasheet: 0x00 CHIP_ID (0xD1), 0x03 PMU

22	// status, 0x12 accel data, 0x40 ACC_CONF, 0x41 ACC_RANGE, 0x7E CMD.

23	static bool imuWrite(uint8_t reg, uint8_t val) {

24	  Wire.beginTransmission(imuAddr);

25	  Wire.write(reg);

26	  Wire.write(val);

27	  return Wire.endTransmission() == 0;

28	}

29

30	static bool imuRead(uint8_t reg, uint8_t* buf, uint8_t len) {

31	  Wire.beginTransmission(imuAddr);

32	  Wire.write(reg);

33	  if (Wire.endTransmission(false) != 0) return false;

34	  if (Wire.requestFrom(imuAddr, len) != len) return false;

35	  for (uint8_t i = 0; i < len; i++) buf[i] = Wire.read();

36	  return true;

37	}

38

39	// Finds the BMI160 at 0x68 or 0x69 (depends on the SDO pin), soft-resets it,

40	// sets 100 Hz / +-2 g and switches the accelerometer to normal mode. Returns

41	// false if the chip ID is wrong or the accelerometer never powers up; the

42	// boot screen then shows BMI160 FAIL. Check SDA 21 / SCL 22 wiring first.

43	static bool imuStart() {

44	  const uint8_t addrs[2] = {0x68, 0x69};

45	  uint8_t id = 0;

46	  for (uint8_t a : addrs) {

47	    imuAddr = a;

48	    if (imuRead(0x00, &id, 1) && id == 0xD1) break;

49	    id = 0;

50	  }

51	  if (id != 0xD1) return false;

52	  imuWrite(0x7E, 0xB6);

53	  delay(100);

54	  imuWrite(0x40, 0x28);

55	  imuWrite(0x41, 0x03);

56	  for (int i = 0; i < 3; i++) {

57	    imuWrite(0x7E, 0x11);

58	    delay(10);

59	    uint8_t pmu = 0;

60	    if (imuRead(0x03, &pmu, 1) && ((pmu >> 4) & 3) == 1) return true;

61	  }

62	  return false;

63	}

64

65	// Reads X, Y, Z acceleration as signed 16-bit values (little endian).

66	static bool imuAccel(int16_t a[3]) {

67	  uint8_t d[6];

68	  if (!imuRead(0x12, d, 6)) return false;

69	  a[0] = (int16_t)(d[1] << 8 | d[0]);

70	  a[1] = (int16_t)(d[3] << 8 | d[2]);

71	  a[2] = (int16_t)(d[5] << 8 | d[4]);

72	  return true;

73	}

74	static HX711 scale;

75

76	// Flags set by the request* functions and cleared here after the work is done.

77	static volatile bool reqTare = false, reqCalW = false, reqGas = false, reqImuRef = false;

78	static volatile float reqGrams = 0;

79

80	// Load cell: grams = (raw - hxOffset) / hxScale. 420 is a rough default for a

81	// 5 kg bar until CAL_WEIGHT is run with a known weight.

82	static int32_t hxOffset = 0;

83	static float hxScale = 420.0f;

84	static float imuRef[3] = {0, 0, 1};

85

86	// Highest gas resistance seen in clean air. IAQ is computed against it, so a

87	// bad baseline (e.g. warmed up next to garbage) makes the air look clean.

88	// RESET_GAS clears it and restarts the warm-up.

89	static float gasBaseline = 0;

90	static uint32_t gasStartMs = 0;

91	static bool baselineFromNvs = false;

92

93	void requestTare() { reqTare = true; }

94	void requestCalWeight(float grams) { reqGrams = grams; reqCalW = true; }

95	void requestGasReset() { reqGas = true; }

96	void requestImuReference() { reqImuRef = true; }

97

98	// Seconds left in the BME680 warm-up: 5 min on first start, 2 min when a

99	// baseline was saved earlier. The screen shows this countdown as "Air (warm)".

100	int airWarmLeft() {

101	  uint32_t burn = baselineFromNvs ? 120 : 300;

102	  uint32_t age = (millis() - gasStartMs) / 1000;

103	  return age < burn ? (int)(burn - age) : 0;

104	}

105

106	// Builds st.v from st.raw plus the simulation overrides. The caller must hold

107	// the state lock. With a fill override, the distance is worked backwards so

108	// the dashboard's distance and fill stay consistent.

109	void recomputeView() {

110	  Reading v = st.raw;

111	  const Sim& s = sim;

112	  const Cal& c = cal;

113	  if (s.mask & 1) {

114	    v.fill = constrain(s.fill, 0, 100);

115	    v.distance_cm = c.d_empty - v.fill / 100.0f * (c.d_empty - c.d_full);

116	  }

117	  if (s.mask & 2) v.weight_g = s.weight;

118	  if (s.mask & 4) {

119	    v.iaq = s.gas;

120	    v.iaq_acc = 3;

121	  }

122	  if (s.mask & 8) {

123	    v.temp = s.temp;

124	    v.temp_raw = s.temp - c.temp_off;

125	    v.humidity = s.humidity;

126	  }

127	  if (s.mask & 16) v.tilt = s.tamper ? max(c.tilt_th + 8.0f, 38.0f) : 0.0f;

128	  if (s.mask & 32) v.motion = s.motion;

129	  st.v = v;

130	}

131

132	// Probes each sensor once at boot. A sensor that fails here is retried by

133	// sensorTask, so a loose wire that reconnects comes back without a reset.

134	void sensorsInit() {

135	  Wire.begin(PIN_SDA, PIN_SCL);

136	  Wire.setClock(100000);

137

138	  pinMode(PIN_TRIG, OUTPUT);

139	  digitalWrite(PIN_TRIG, LOW);

140	  pinMode(PIN_ECHO, INPUT);

141

142	  // The BME680 address is 0x76 or 0x77 depending on its SDO pin; both are tried.

143	  // Heater 320 deg C for 150 ms is the Adafruit default for gas readings.

144	  bool bmeOk = bme.begin(0x76) || bme.begin(0x77);

145	  if (bmeOk) {

146	    bme.setTemperatureOversampling(BME680_OS_8X);

147	    bme.setHumidityOversampling(BME680_OS_2X);

148	    bme.setPressureOversampling(BME680_OS_4X);

149	    bme.setIIRFilterSize(BME680_FILTER_SIZE_3);

150	    bme.setGasHeater(320, 150);

151	  }

152

153	  bool imuOk = imuStart();

154	  float x, y, z;

155	  if (loadImuRef(x, y, z)) { imuRef[0] = x; imuRef[1] = y; imuRef[2] = z; }

156

157	  scale.begin(PIN_HX_DT, PIN_HX_SCK);

158	  loadHx(hxOffset, hxScale);

159	  if (fabsf(hxScale) < 0.001f) hxScale = 420.0f;

160

161	  gasBaseline = loadGasBaseline();

162	  baselineFromNvs = gasBaseline > 0;

163	  gasStartMs = millis();

164

165	  lockState();

166	  st.okBME = bmeOk;

167	  st.okIMU = imuOk;

168	  unlockState();

169	}

170

171	// Median of up to 5 values. Throws away single bad ultrasonic echoes.

172	static float median5(float* a, int n) {

173	  float b[5];

174	  for (int i = 0; i < n; i++) b[i] = a[i];

175	  for (int i = 1; i < n; i++)

176	    for (int j = i; j > 0 && b[j - 1] > b[j]; j--) { float t = b[j]; b[j] = b[j - 1]; b[j - 1] = t; }

177	  return b[n / 2];

178	}

179

180	// Estimated IAQ index (0 good .. 500 bad) from gas resistance and humidity.

181	// This is not Bosch's BSEC algorithm: 25% of the score comes from humidity

182	// distance to 40% RH, 75% from gas resistance compared with the clean-air

183	// baseline. Accuracy rises from 1 to 3 over the 30 minutes after warm-up. Returns NaN

184	// during warm-up, and the old IAQ value is kept.

185	static float iaqFrom(float gasOhm, float hum, uint8_t& acc) {

186	  uint32_t age = (millis() - gasStartMs) / 1000;

187	  uint32_t burn = baselineFromNvs ? 120 : 300;

188	  if (age < burn) {

189	    if (gasOhm > gasBaseline) gasBaseline = gasOhm;

190	    acc = 0;

191	    return NAN;

192	  }

193	  if (gasOhm > gasBaseline) gasBaseline = gasOhm;

194	  else gasBaseline = gasBaseline * 0.99995f + gasOhm * 0.00005f;

195	  acc = age < burn + 600 ? 1 : age < burn + 1800 ? 2 : 3;

196	  const float humBase = 40.0f, humWeight = 0.25f;

197	  float hOff = hum - humBase;

198	  float humScore = hOff > 0 ? (100.0f - humBase - hOff) / (100.0f - humBase) * humWeight * 100.0f

199	                            : (humBase + hOff) / humBase * humWeight * 100.0f;

200	  float gOff = gasBaseline - gasOhm;

201	  float gasScore = gOff > 0 ? (gasOhm / gasBaseline) * (100.0f - humWeight * 100.0f) : 100.0f - humWeight * 100.0f;

202	  float quality = constrain(humScore + gasScore, 0.0f, 100.0f);

203	  return (100.0f - quality) * 5.0f;

204	}

205

206	// Runs on core 1, priority 3. Wakes every 50 ms or on EV_IMU. Each block below

207	// has its own timer so one slow sensor does not hold up the others.

208	void sensorTask(void*) {

209	  float dist[5];

210	  int distN = 0, distI = 0, echoMiss = 0;

211	  float hxBuf[5];

212	  int hxN = 0, hxI = 0;

213	  int32_t tareAcc = 0;

214	  int64_t calAcc = 0;

215	  int tareN = 0, calN = 0;

216	  uint32_t lastTrig = 0, lastImu = 0, lastHxOk = millis(), lastEnv = 0, envReadyAt = 0, lastRate = 0, lastBaseSave = millis();

217	  bool envPending = false;

218	  int bmeFail = 0, imuFail = 0, sameCount = 0;

219	  int16_t lastA[3] = {0, 0, 0};

220	  float tiltF = 0;

221	  float rateBuf[31];

222	  int rateN = 0;

223	  uint8_t lastAcc = 0;

224	  bool firstEnv = false;

225	  Reading pub;

226	  bool pubFaults[4] = {false, false, false, false};

227	  uint32_t bootStart = millis();

228

229	  for (;;) {

230	    uint32_t ev = 0;

231	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(50));

232	    uint32_t now = millis();

233	    Cal c = calSnapshot();

234	    bool changed = false;

235

236	    // --- Ultrasonic fill level ---

237	    // 10 us trigger pulse, echo timeout 25 ms (about 4 m). us / 58 = cm.

238	    // Readings outside 2..400 cm are ignored. After 10 missed echoes in a row

239	    // the sensor is marked faulty. If fill stays at 0 or 100, check the

240	    // empty/full distances in the calibration, not the sensor.

241	    if (now - lastTrig >= 200) {

242	      lastTrig = now;

243	      digitalWrite(PIN_TRIG, HIGH);

244	      delayMicroseconds(10);

245	      digitalWrite(PIN_TRIG, LOW);

246	      unsigned long us = pulseIn(PIN_ECHO, HIGH, 25000);

247	      if (us > 0) {

248	        float cm = us / 58.0f;

249	        if (cm >= 2 && cm <= 400) {

250	          dist[distI] = cm;

251	          distI = (distI + 1) % 5;

252	          if (distN < 5) distN++;

253	          echoMiss = 0;

254	        }

255	      } else if (++echoMiss > 10) {

256	        distN = 0;

257	      }

258	      bool ok = distN >= 3;

259	      float d = ok ? median5(dist, distN) : 0;

260	      float fill = ok ? constrain((c.d_empty - d) / (c.d_empty - c.d_full) * 100.0f, 0.0f, 100.0f) : 0;

261	      lockState();

262	      if (st.okHC != ok) { st.okHC = ok; changed = true; }

263	      st.raw.distance_cm = d;

264	      st.raw.fill = fill;

265	      unlockState();

266	    }

267

268	    // --- Tilt from the accelerometer ---

269	    // Tilt is the angle between the current gravity vector and the reference

270	    // saved at calibration. 25 identical readings in a row (5 s) mean the

271	    // chip has frozen, so it is restarted. A 0.6/0.4 low-pass filter stops a

272	    // single bump from starting the tamper countdown.

273	    if ((ev & EV_IMU) || now - lastImu >= 200) {

274	      lastImu = now;

275	      int16_t a[3] = {0, 0, 0};

276	      bool ok = imuAccel(a) && (a[0] || a[1] || a[2]);

277	      if (ok && a[0] == lastA[0] && a[1] == lastA[1] && a[2] == lastA[2]) {

278	        if (++sameCount >= 25) {

279	          sameCount = 0;

280	          ok = false;

281	          imuFail = 10;

282	          imuStart();

283	        }

284	      } else {

285	        sameCount = 0;

286	      }

287	      if (ok) {

288	        memcpy(lastA, a, sizeof(lastA));

289	        imuFail = 0;

290	        float x = a[0], y = a[1], z = a[2];

291	        float m = sqrtf(x * x + y * y + z * z);

292	        if (m > 1) {

293	          x /= m; y /= m; z /= m;

294	          if (reqImuRef) {

295	            reqImuRef = false;

296	            imuRef[0] = x; imuRef[1] = y; imuRef[2] = z;

297	            saveImuRef(x, y, z);

298	          }

299	          float dot = constrain(x * imuRef[0] + y * imuRef[1] + z * imuRef[2], -1.0f, 1.0f);

300	          float deg = acosf(dot) * 57.29578f;

301	          tiltF = tiltF * 0.6f + deg * 0.4f;

302	        }

303	      } else if (++imuFail > 10) {

304	        tiltF = 0;

305	      }

306	      lockState();

307	      bool okNow = imuFail <= 10;

308	      if (st.okIMU != okNow) { st.okIMU = okNow; changed = true; }

309	      st.raw.tilt = tiltF;

310	      unlockState();

311	    }

312

313	    // --- Load cell ---

314	    // TARE and CAL_WEIGHT average 10 raw samples before saving. The weight is

315	    // a moving average of 5 samples; anything under 5 g counts as 0. No sample

316	    // for 1.5 s marks the HX711 faulty (check DT 32 / SCK 14 and the 5V supply).

317	    if (scale.is_ready()) {

318	      long raw = scale.read();

319	      lastHxOk = now;

320	      if (reqTare) {

321	        tareAcc += raw;

322	        if (++tareN >= 10) {

323	          hxOffset = tareAcc / tareN;

324	          tareAcc = 0; tareN = 0; reqTare = false; hxN = 0;

325	          saveHx(hxOffset, hxScale);

326	          buzz(T_CAL_STEP);

327	        }

328	      } else if (reqCalW) {

329	        calAcc += raw;

330	        if (++calN >= 10) {

331	          float avg = (float)calAcc / calN;

332	          if (reqGrams > 0 && fabsf(avg - hxOffset) > 10) hxScale = (avg - hxOffset) / reqGrams;

333	          calAcc = 0; calN = 0; reqCalW = false; hxN = 0;

334	          saveHx(hxOffset, hxScale);

335	          buzz(T_CAL_STEP);

336	        }

337	      }

338	      hxBuf[hxI] = (raw - hxOffset) / hxScale;

339	      hxI = (hxI + 1) % 5;

340	      if (hxN < 5) hxN++;

341	      float sum = 0;

342	      for (int i = 0; i < hxN; i++) sum += hxBuf[i];

343	      float g = sum / hxN;

344	      if (fabsf(g) < 5) g = 0;

345	      lockState();

346	      if (!st.okHX) { st.okHX = true; changed = true; }

347	      st.raw.weight_g = g;

348	      unlockState();

349	    } else if (now - lastHxOk > 1500) {

350	      lockState();

351	      if (st.okHX) { st.okHX = false; changed = true; }

352	      unlockState();

353	    }

354

355	    // --- Gas baseline reset (RESET_GAS command) ---

356	    if (reqGas) {

357	      reqGas = false;

358	      gasBaseline = 0;

359	      baselineFromNvs = false;

360	      gasStartMs = now;

361	      saveGasBaseline(0);

362	    }

363

364	    // --- BME680, non-blocking ---

365	    // beginReading() starts a measurement and returns when it will be ready,

366	    // so the task never sleeps through the 150 ms heater cycle. Three failed

367	    // starts in a row mark the sensor faulty and re-run bme.begin().

368	    if (!envPending && now - lastEnv >= 3000) {

369	      lastEnv = now;

370	      envReadyAt = bme.beginReading();

371	      envPending = envReadyAt != 0;

372	      if (!envPending && ++bmeFail >= 3) {

373	        lockState();

374	        if (st.okBME) { st.okBME = false; changed = true; }

375	        unlockState();

376	        if (bme.begin(0x76) || bme.begin(0x77)) bme.setGasHeater(320, 150);

377	      }

378	    }

379	    if (envPending && now >= envReadyAt) {

380	      envPending = false;

381	      if (bme.endReading()) {

382	        bmeFail = 0;

383	        firstEnv = true;

384	        uint8_t acc = 0;

385	        float hum = constrain(bme.humidity + c.hum_off, 0.0f, 100.0f);

386	        float iaq = bme.gas_resistance > 0 ? iaqFrom(bme.gas_resistance, hum, acc) : NAN;

387	        lockState();

388	        if (!st.okBME) { st.okBME = true; changed = true; }

389	        st.raw.temp_raw = bme.temperature;

390	        st.raw.temp = bme.temperature + c.temp_off;

391	        st.raw.humidity = hum;

392	        st.raw.pressure = bme.pressure / 100.0f + c.press_off;

393	        st.raw.gas_k = bme.gas_resistance / 1000.0f;

394	        if (!isnan(iaq)) st.raw.iaq = constrain(iaq, 0.0f, 500.0f);

395	        st.raw.iaq_acc = acc;

396	        unlockState();

397	        // First usable IAQ after warm-up: beep once and tell the apps.

398	        if (acc >= 1 && lastAcc == 0) {

399	          buzz(T_AIR_READY);

400	          char meta[72];

401	          snprintf(meta, sizeof(meta), "%s air quality readings have started", BIN_LABEL);

402	          raiseAlert("sensor", "success", "Air sensor ready", meta);

403	        }

404	        lastAcc = acc;

405	        // Save the baseline at most once an hour to spare the flash.

406	        if (acc >= 3 && now - lastBaseSave > 3600000UL) {

407	          lastBaseSave = now;

408	          saveGasBaseline(gasBaseline);

409	        }

410	      } else if (++bmeFail >= 3) {

411	        lockState();

412	        if (st.okBME) { st.okBME = false; changed = true; }

413	        unlockState();

414	      }

415	    }

416

417	    // --- Fill rate for the "Full in ~x h" estimate ---

418	    // One sample a minute, up to 30 minutes. A drop of more than 5% means the

419	    // bin was emptied, so the history restarts. Needs 5 samples (4 min).

420	    if (now - lastRate >= 60000 || lastRate == 0) {

421	      lastRate = now;

422	      lockState();

423	      float f = st.v.fill;

424	      bool ok = st.okHC || (sim.mask & 1);

425	      unlockState();

426	      if (ok) {

427	        if (rateN && f < rateBuf[rateN - 1] - 5) rateN = 0;

428	        if (rateN == 31) { memmove(rateBuf, rateBuf + 1, 30 * sizeof(float)); rateN = 30; }

429	        rateBuf[rateN++] = f;

430	        float rate = rateN >= 5 ? (rateBuf[rateN - 1] - rateBuf[0]) / ((rateN - 1) / 60.0f) : 0;

431	        lockState();

432	        st.fillRatePerHour = rate > 0.05f ? rate : 0;

433	        unlockState();

434	      }

435	    }

436

437	    lockState();

438	    // Leave the boot screen once the BME680 answered (or is known faulty) and

439	    // at least 3.5 s have passed.

440	    if (st.booting && (firstEnv || !st.okBME) && now - bootStart > 3500) {

441	      st.booting = false;

442	      changed = true;

443	    }

444	    recomputeView();

445	    const Reading& v = st.v;

446	    bool faults[4] = {st.okHC, st.okHX, st.okBME, st.okIMU};

447	    // Publish only on a real change (1% fill, 20 g, 0.2 C, ...). This keeps the

448	    // live dashboard and Firebase from being flooded by sensor noise.

449	    if (fabsf(v.fill - pub.fill) >= 1 || fabsf(v.weight_g - pub.weight_g) >= 20 || fabsf(v.temp - pub.temp) >= 0.2f ||

450	        fabsf(v.humidity - pub.humidity) >= 1 || fabsf(v.iaq - pub.iaq) >= 5 || fabsf(v.tilt - pub.tilt) >= 1 ||

451	        fabsf(v.pressure - pub.pressure) >= 0.5f || v.iaq_acc != pub.iaq_acc || memcmp(faults, pubFaults, sizeof(faults)))

452	      changed = true;

453	    if (changed) {

454	      pub = v;

455	      memcpy(pubFaults, faults, sizeof(faults));

456	    }

457	    unlockState();

458	    if (changed) {

459	      publish();

460	      if (hControl) xTaskNotify(hControl, EV_STATE, eSetBits);

461	    }

462	  }

463	}

firmware/SmartBin/control.h

1	// control.h - lid, LED, alarms and the command runner (controlTask).

2	#pragma once

3	#include <Arduino.h>

4

5	void controlInit();

6	void controlTask(void*);

7	void isrPir();

firmware/SmartBin/control.cpp

1	// ============================================================================

2	// control.cpp - decides what the bin does

3	// Runs every command from cmdQueue, works out the status, moves the lid,

4	// drives the RGB LED and raises the full / tamper / environment / sensor

5	// alarms. Priority 4 on core 1, the highest of all tasks, so the lid and

6	// alarms react even while the screen or cloud is busy.

7	// ============================================================================

8	#include "control.h"

9	#include "state.h"

10	#include "sensors.h"

11	#include "buzzer.h"

12	#include "ui.h"

13	#include <ESP32Servo.h>

14

15	static Servo lid;

16	static int lidAngle = -1;

17

18	// PIR interrupt. Only wakes controlTask; the pin itself is read in the task.

19	void IRAM_ATTR isrPir() {

20	  BaseType_t woke = pdFALSE;

21	  if (hControl) xTaskNotifyFromISR(hControl, EV_PIR, eSetBits, &woke);

22	  portYIELD_FROM_ISR(woke);

23	}

24

25	// LED colors (see updateLed): blue = Bluetooth setup, blinking red = tamper

26	// or full alarm, cyan = calibrating, green = collecting or OK, magenta =

27	// locked, yellow = 50% full or more.

28	static void led(bool r, bool g, bool b) {

29	  digitalWrite(PIN_LED_R, r ^ LED_ACTIVE_LOW);

30	  digitalWrite(PIN_LED_G, g ^ LED_ACTIVE_LOW);

31	  digitalWrite(PIN_LED_B, b ^ LED_ACTIVE_LOW);

32	}

33

34	// The servo uses LEDC timer 0 at 50 Hz with 500..2400 us pulses (SG90).

35	// The lid is driven closed at boot so a reset never leaves it open.

36	void controlInit() {

37	  pinMode(PIN_LED_R, OUTPUT);

38	  pinMode(PIN_LED_G, OUTPUT);

39	  pinMode(PIN_LED_B, OUTPUT);

40	  led(false, false, true);

41	  pinMode(PIN_PIR, INPUT);

42	  ESP32PWM::allocateTimer(0);

43	  lid.setPeriodHertz(50);

44	  lid.attach(PIN_SERVO, 500, 2400);

45	  Cal c = calSnapshot();

46	  lid.write(c.servo_closed);

47	  lidAngle = c.servo_closed;

48	}

49

50	static void setLid(int angle) {

51	  if (angle == lidAngle) return;

52	  lid.write(angle);

53	  lidAngle = angle;

54	}

55

56	// Records the outcome of a command. For commands from Firebase the result is

57	// queued so cloudTask can write it to /sb/results/<ref>, which is how the app

58	// knows its button press worked. A failed command from a remote source beeps.

59	static void finish(const Command& c, bool ok, const char* why) {

60	  lockState();

61	  strlcpy(st.last.cmd, c.cmd, sizeof(st.last.cmd));

62	  st.last.ts = nowMs();

63	  st.last.ok = ok;

64	  unlockState();

65	  if (c.src == SRC_FIREBASE && c.ref[0]) {

66	    CmdResult r{};

67	    strlcpy(r.cmd, c.cmd, sizeof(r.cmd));

68	    r.ok = ok;

69	    strlcpy(r.why, why ? why : "", sizeof(r.why));

70	    r.src = c.src;

71	    strlcpy(r.ref, c.ref, sizeof(r.ref));

72	    r.ts = nowMs();

73	    xQueueSend(resultQueue, &r, 0);

74	  }

75	  if (!ok && c.src != SRC_SCREEN) buzz(T_CMD_FAIL);

76	  publish(EV_RESULT);

77	}

78

79	// Runs one command. State changes happen under the lock; sounds and alerts

80	// come after it is released, because buzz() takes the same lock (the mutex

81	// is not recursive, so calling it while holding the lock would hang the task).

82	static void runCommand(const Command& c) {

83	  BinState s = snapshot();

84	  String why;

85	  if (!validateCommand(c.cmd, s, why)) {

86	    finish(c, false, why.c_str());

87	    return;

88	  }

89	  const char* k = c.cmd;

90	  char meta[72];

91	  lockState();

92	  if (!strcmp(k, "LOCK")) {

93	    st.locked = true;

94	    st.autoLocked = false;

95	  } else if (!strcmp(k, "UNLOCK")) {

96	    st.locked = false;

97	    st.autoLocked = false;

98	  } else if (!strcmp(k, "CLEAR_TAMPER")) {

99	    st.tamperPhase = 0;

100	  } else if (!strcmp(k, "COLLECT")) {

101	    st.collecting = true;

102	  // Collection finishing also releases an automatic (full-bin) lock, but not

103	  // a lock that a person set.

104	  } else if (!strcmp(k, "COLLECT_DONE")) {

105	    if (st.collecting) st.doneAtMs = millis();

106	    st.collecting = false;

107	    if (st.autoLocked) { st.locked = false; st.autoLocked = false; }

108	  } else if (!strcmp(k, "CALIBRATE_START")) {

109	    st.calibrating = true;

110	    st.calSteps = 0;

111	  } else if (!strcmp(k, "CALIBRATE_END")) {

112	    st.calibrating = false;

113	  } else if (!strcmp(k, "TARE")) {

114	    st.calSteps |= 1;

115	  }

116	  unlockState();

117

118	  if (!strcmp(k, "LOCK")) {

119	    buzz(T_LOCK);

120	    snprintf(meta, sizeof(meta), "%s locked", BIN_LABEL);

121	    raiseAlert("lock", "info", "Bin locked", meta);

122	  } else if (!strcmp(k, "UNLOCK")) {

123	    buzz(T_UNLOCK);

124	    snprintf(meta, sizeof(meta), "%s is open for use again", BIN_LABEL);

125	    raiseAlert("lock", "success", "Bin unlocked", meta);

126	  } else if (!strcmp(k, "CLEAR_TAMPER")) {

127	    buzz(T_TAMPER_SETTLED);

128	  } else if (!strcmp(k, "COLLECT")) {

129	    if (c.src != SRC_SCREEN) buzz(T_PIN_OK);

130	    snprintf(meta, sizeof(meta), "%s pickup started", BIN_LABEL);

131	    raiseAlert("pickup", "info", "Collection started", meta);

132	  } else if (!strcmp(k, "COLLECT_DONE")) {

133	    buzz(T_DONE);

134	    snprintf(meta, sizeof(meta), "%s is back to normal", BIN_LABEL);

135	    raiseAlert("pickup", "success", "Collection complete", meta);

136	  } else if (!strcmp(k, "CALIBRATE_START")) {

137	    requestImuReference();

138	    buzz(T_CAL_START);

139	    snprintf(meta, sizeof(meta), "%s is being calibrated", BIN_LABEL);

140	    raiseAlert("setup", "info", "Calibration started", meta);

141	  } else if (!strcmp(k, "CALIBRATE_END")) {

142	    buzz(c.src == SRC_SCREEN ? T_CAL_CANCEL : T_SAVED);

143	  } else if (!strcmp(k, "TARE")) {

144	    requestTare();

145	  } else if (!strcmp(k, "CAL_WEIGHT")) {

146	    requestCalWeight(c.grams > 0 ? c.grams : 500);

147	  } else if (!strcmp(k, "RESET_GAS")) {

148	    requestGasReset();

149	    buzz(T_CAL_STEP);

150	  } else if (!strcmp(k, "TOUCH_CAL")) {

151	    uiStartTouchCal();

152	  } else if (!strcmp(k, "TOUCH_CAL_RESET")) {

153	    uiResetTouchCal();

154	  } else if (!strcmp(k, "BUZZ_TEST")) {

155	    if (!buzzByName(c.sound[0] ? c.sound : "full")) {

156	      finish(c, false, "Unknown sound");

157	      return;

158	    }

159	  }

160	  finish(c, true, nullptr);

161	}

162

163	// Called every 100 ms. The first matching state wins.

164	static void updateLed(const BinState& s, const Cal& c, uint32_t now) {

165	  bool blink = (now / 500) % 2;

166	  if (s.btOn) led(false, false, true);

167	  else if (s.tamperPhase) led(blink, false, false);

168	  else if (s.calibrating) led(false, true, true);

169	  else if (s.collecting) led(false, true, false);

170	  else if (s.status == ST_LOCKED) led(true, false, true);

171	  else if (s.alarmFull) led(blink, false, false);

172	  else if (s.v.fill >= 50) led(true, true, false);

173	  else led(false, true, false);

174	}

175

176	// Wakes every 100 ms, on PIR change, on a new command or on a sensor update.

177	void controlTask(void*) {

178	  uint32_t lastTick = 0, lastWave = 0, lastFullBuzz = 0, lastEnvBuzz = 0, lastTamperTick = 0;

179	  bool prevFull = false, prevEnv = false, prevHC = true, prevHX = true, prevBME = true;

180	  bool prevMotion = false, firstPass = true;

181	  BinStatus prevStatus = ST_OK;

182	  uint8_t prevMask = 0;

183

184	  for (;;) {

185	    uint32_t ev = 0;

186	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(100));

187

188	    Command c;

189	    // Drain all waiting commands before evaluating the state.

190	    while (xQueueReceive(cmdQueue, &c, 0) == pdTRUE) runCommand(c);

191

192	    uint32_t now = millis();

193	    Cal cl = calSnapshot();

194	    bool pir = digitalRead(PIN_PIR);

195	    bool changed = false;

196	    char meta[72];

197

198	    lockState();

199	    st.raw.motion = pir;

200	    recomputeView();

201	    bool motion = st.v.motion;

202	    bool hcOk = st.okHC || (sim.mask & 1);

203	    // Alarms are ignored while collecting or calibrating, otherwise opening

204	    // the bin to empty it would set them off.

205	    bool fullNow = hcOk && st.v.fill >= cl.fill_th && !st.collecting && !st.calibrating;

206	    bool tiltBad = (st.okIMU || (sim.mask & 16)) && st.v.tilt > cl.tilt_th && !st.collecting && !st.calibrating;

207

208	    // --- Tamper state machine ---

209	    // 0 none -> 1 countdown (TAMPER_COUNTDOWN_S, beeps each second) -> 2 latched.

210	    // Going back under the tilt limit during the countdown cancels it. Once

211	    // latched it stays on until CLEAR_TAMPER, even if the bin is set upright.

212	    // The lock is released around buzz() because buzz() takes it itself.

213	    if (tiltBad && st.tamperPhase == 0) {

214	      st.tamperPhase = 1;

215	      st.tamperStartMs = now;

216	      changed = true;

217	    } else if (!tiltBad && st.tamperPhase == 1) {

218	      st.tamperPhase = 0;

219	      changed = true;

220	      unlockState();

221	      buzz(T_TAMPER_SETTLED);

222	      lockState();

223	    } else if (st.tamperPhase == 1 && now - st.tamperStartMs >= TAMPER_COUNTDOWN_S * 1000) {

224	      st.tamperPhase = 2;

225	      changed = true;

226	      snprintf(meta, sizeof(meta), "%s was moved or tilted (%.0f deg)", BIN_LABEL, st.v.tilt);

227	      unlockState();

228	      raiseAlert("tamper", "critical", "Tamper detected", meta);

229	      lockState();

230	    }

231

232	    // Optional auto-lock when full (setting "auto_lock"). Off by default.

233	    if (cl.auto_lock && fullNow && !st.locked && !st.collecting && !st.tamperPhase) {

234	      st.locked = true;

235	      st.autoLocked = true;

236	      changed = true;

237	      snprintf(meta, sizeof(meta), "%s locked itself at %.0f%%", BIN_LABEL, st.v.fill);

238	      unlockState();

239	      buzz(T_LOCK);

240	      raiseAlert("lock", "info", "Bin locked", meta);

241	      lockState();

242	    }

243

244	    // One status, highest priority first.

245	    BinStatus status = st.calibrating ? ST_CALIBRATING : st.collecting ? ST_COLLECTING

246	                     : st.tamperPhase ? ST_TAMPER : st.locked ? ST_LOCKED : ST_OK;

247	    if (status != st.status) { st.status = status; changed = true; }

248

249	    // --- Lid decision ---

250	    // Collecting: open. Locked, tamper or calibrating: closed. Lid simulated:

251	    // follow the simulation. Otherwise motion opens the lid, and it closes

252	    // pir_hold_s seconds after the last motion. If the lid opens and closes by

253	    // itself, check the PIR's own delay knob (it should be at minimum).

254	    bool wantOpen;

255	    if (status == ST_COLLECTING) {

256	      wantOpen = true;

257	      st.lidCloseAtMs = 0;

258	    } else if (status != ST_OK) {

259	      wantOpen = false;

260	      st.lidCloseAtMs = 0;

261	    } else if (sim.mask & 64) {

262	      wantOpen = sim.lid;

263	      st.lidCloseAtMs = 0;

264	    } else if (motion) {

265	      wantOpen = true;

266	      st.lidCloseAtMs = 0;

267	    } else if (st.lidOpen && st.lidCloseAtMs == 0) {

268	      st.lidCloseAtMs = now + cl.pir_hold_s * 1000UL;

269	      wantOpen = true;

270	    } else if (st.lidOpen && (int32_t)(now - st.lidCloseAtMs) < 0) {

271	      wantOpen = true;

272	    } else {

273	      wantOpen = false;

274	      st.lidCloseAtMs = 0;

275	    }

276	    bool lidChanged = wantOpen != st.lidOpen;

277	    if (lidChanged) { st.lidOpen = wantOpen; changed = true; }

278	    if (motion != prevMotion) changed = true;

279

280	    // Environment alarm: temperature or humidity over its limit, or IAQ over

281	    // gas_th once the air sensor has warmed up.

282	    st.alarmFull = fullNow && status == ST_OK;

283	    bool gasOk = st.okBME || (sim.mask & 4);

284	    bool envBad = !st.collecting && !st.calibrating &&

285	                  (((st.okBME || (sim.mask & 8)) && (st.v.temp >= cl.temp_th || st.v.humidity >= cl.hum_th)) ||

286	                   (gasOk && st.v.iaq_acc >= 1 && st.v.iaq >= cl.gas_th));

287	    st.alarmEnv = envBad;

288	    st.buzzerOn = buzzerBusy() && (st.alarmFull || st.tamperPhase || envBad);

289	    BinState s = st;

290	    unlockState();

291

292	    // Hardware is touched only after the lock is released.

293	    setLid(s.lidOpen ? cl.servo_open : cl.servo_closed);

294	    if (lidChanged) buzz(s.lidOpen ? T_LID_OPEN : T_LID_CLOSE);

295	    if (motion && !prevMotion && (s.status == ST_LOCKED || s.status == ST_TAMPER) && now - lastWave > 5000) {

296	      lastWave = now;

297	      buzz(T_LOCKED_WAVE);

298	    }

299	    prevMotion = motion;

300

301	    // Countdown beeps change pitch for the last 3 seconds; the latched siren

302	    // repeats every TAMPER_ALARM_EVERY_MS.

303	    if (s.tamperPhase == 1 && now - lastTamperTick >= 1000) {

304	      lastTamperTick = now;

305	      uint32_t left = TAMPER_COUNTDOWN_S - min<uint32_t>(TAMPER_COUNTDOWN_S, (now - s.tamperStartMs) / 1000);

306	      buzz(left <= 3 ? T_TAMPER_TICK_END : T_TAMPER_TICK);

307	      changed = true;

308	    }

309	    if (s.tamperPhase == 2 && now - lastTamperTick >= TAMPER_ALARM_EVERY_MS) {

310	      lastTamperTick = now;

311	      buzz(T_TAMPER_LATCHED);

312	    }

313

314	    // Full alarm: alert once on the rising edge, then repeat the beep every

315	    // buzz_repeat_s seconds. buzzerUntilMs drives the BUZZER row on screen.

316	    if (s.alarmFull && (!prevFull || now - lastFullBuzz >= cl.buzz_repeat_s * 1000UL)) {

317	      if (!prevFull) {

318	        snprintf(meta, sizeof(meta), "%s is at %.0f%%", BIN_LABEL, s.v.fill);

319	        raiseAlert("full", "critical", "Fill threshold passed", meta);

320	      }

321	      lastFullBuzz = now;

322	      buzz(T_FULL);

323	      lockState();

324	      st.buzzerUntilMs = now + max<uint32_t>(cl.buzz_s, 1) * 1000UL;

325	      unlockState();

326	    }

327	    prevFull = s.alarmFull;

328

329	    // Environment alarm repeats every 5 minutes while it lasts.

330	    if (envBad && (!prevEnv || now - lastEnvBuzz >= 300000)) {

331	      if (!prevEnv) {

332	        if (s.v.temp >= cl.temp_th) snprintf(meta, sizeof(meta), "%s at %.1f C, limit %.0f", BIN_LABEL, s.v.temp, cl.temp_th);

333	        else if (s.v.humidity >= cl.hum_th) snprintf(meta, sizeof(meta), "%s at %.0f%%, limit %.0f%%", BIN_LABEL, s.v.humidity, cl.hum_th);

334	        else snprintf(meta, sizeof(meta), "%s index %.0f, limit %.0f", BIN_LABEL, s.v.iaq, cl.gas_th);

335	        raiseAlert("sensor", "warning", "Reading over its limit", meta);

336	      }

337	      lastEnvBuzz = now;

338	      buzz(T_ENV_OVER);

339	    } else if (!envBad && prevEnv) {

340	      buzz(T_ENV_OK);

341	    }

342	    prevEnv = envBad;

343

344	    // Sensor lost / back alerts. The first pass after boot only records the

345	    // starting state and plays the boot sound (OK or FAULT).

346	    if (!firstPass && !s.booting) {

347	      const bool oks[3] = {s.okHC, s.okHX, s.okBME};

348	      bool* prevs[3] = {&prevHC, &prevHX, &prevBME};

349	      const char* names[3] = {"HC-SR04", "HX711", "BME680"};

350	      for (int i = 0; i < 3; i++) {

351	        if (oks[i] == *prevs[i]) continue;

352	        *prevs[i] = oks[i];

353	        buzz(oks[i] ? T_SENSOR_BACK : T_SENSOR_LOST);

354	        snprintf(meta, sizeof(meta), "%s: %s %s", BIN_LABEL, names[i], oks[i] ? "answers again" : "stopped answering");

355	        raiseAlert("sensor", oks[i] ? "success" : "warning", oks[i] ? "Sensor back" : "Sensor not answering", meta);

356	      }

357	    } else if (!s.booting) {

358	      prevHC = s.okHC; prevHX = s.okHX; prevBME = s.okBME;

359	      firstPass = false;

360	      buzz(s.okHC && s.okHX && s.okBME && s.okIMU ? T_BOOT_OK : T_BOOT_FAULT);

361	    }

362

363	    // Sound when simulation is switched on or off from the dashboard.

364	    Sim sm = simSnapshot();

365	    if ((sm.mask != 0) != (prevMask != 0)) buzz(sm.mask ? T_SIM_ON : T_SIM_OFF);

366	    prevMask = sm.mask;

367

368	    if (s.status != prevStatus) {

369	      prevStatus = s.status;

370	      changed = true;

371	    }

372	    if (now - lastTick >= 100) {

373	      lastTick = now;

374	      updateLed(s, cl, now);

375	    }

376	    if (changed) publish();

377	  }

378	}

firmware/SmartBin/buzzer.h

1	// buzzer.h - every sound the bin makes. The Tune order must match TUNES[]

2	// in buzzer.cpp (a static_assert checks the count).

3	#pragma once

4	#include <Arduino.h>

5

6	enum Tune : uint8_t {

7	  T_KEY, T_PIN_KEY, T_PIN_DEL, T_CANCEL, T_THEME_LIGHT, T_THEME_DARK, T_OPEN, T_PIN_OK, T_PIN_WRONG, T_DONE,

8	  T_BOOT_OK, T_BOOT_FAULT, T_LID_OPEN, T_LID_CLOSE, T_LOCKED_WAVE, T_LOCK, T_UNLOCK, T_CAL_START, T_CAL_STEP,

9	  T_SAVED, T_SIM_ON, T_SIM_OFF, T_CAL_CANCEL, T_AIR_READY, T_BACK_MAIN,

10	  T_FULL, T_TAMPER_TICK, T_TAMPER_TICK_END, T_TAMPER_LATCHED, T_ENV_OVER, T_SENSOR_LOST, T_TAMPER_SETTLED,

11	  T_ENV_OK, T_SENSOR_BACK,

12	  T_WIFI_UP, T_WIFI_LOST, T_CLOUD_DOWN, T_CLOUD_UP, T_CMD_FAIL,

13	  T_BT_WAIT, T_BT_CODE, T_BT_PAIRED, T_BT_WRONG, T_BT_STOPPED, T_BT_JOINED, T_BT_JOIN_FAIL, T_BT_OFF, T_BT_AUTO_OFF,

14	  T_COUNT

15	};

16

17	void buzzerInit();

18	void buzz(Tune t);

19	bool buzzByName(const char* name);

20	bool buzzerBusy();

firmware/SmartBin/buzzer.cpp

1	// ============================================================================

2	// buzzer.cpp - non-blocking sound player

3	// buzz() drops a tune ID into a one-slot queue; buzzerTask plays it with LEDC

4	// PWM on GPIO 4. Callers never wait for a sound to finish.

5	// ============================================================================

6	#include "buzzer.h"

7	#include "state.h"

8

9	// A note is a frequency in Hz and a length in ms. hz 0 is a rest. With hzEnd

10	// set the pitch slides from hz to hzEnd (used for the tamper siren).

11	// pri: higher can interrupt lower. kind decides which mute setting applies.

12	struct Note { uint16_t hz, ms, hzEnd; };

13	struct TuneDef { const char* name; const Note* n; uint8_t len; uint8_t pri; uint8_t kind; };

14	enum : uint8_t { K_UI = 0, K_LID, K_EVENT, K_CAL, K_ALARM };

15

16	// --- Tune data. Names match the sound names used by the apps and dashboard. ---

17	static const Note n_key[] = {{2400, 15}};

18	static const Note n_pin_key[] = {{2600, 12}};

19	static const Note n_pin_del[] = {{2000, 15}};

20	static const Note n_cancel[] = {{2300, 30}, {1800, 45}};

21	static const Note n_theme_light[] = {{2300, 35}, {2700, 50}};

22	static const Note n_theme_dark[] = {{2000, 35}, {1700, 50}};

23	static const Note n_open[] = {{1800, 30}, {2300, 40}};

24	static const Note n_pin_ok[] = {{1600, 60}, {1900, 60}, {2200, 60}, {1900, 60}, {2200, 120}};

25	static const Note n_pin_wrong[] = {{1700, 60}, {0, 30}, {1700, 60}, {0, 30}, {1500, 150}};

26	static const Note n_done[] = {{1800, 70}, {2100, 70}, {2400, 70}, {2800, 200}};

27	static const Note n_boot_ok[] = {{1600, 70}, {1900, 70}, {2200, 70}, {2800, 180}};

28	static const Note n_boot_fault[] = {{1600, 70}, {1900, 70}, {0, 40}, {1700, 100}, {0, 60}, {1500, 210}};

29	static const Note n_lid_open[] = {{1800, 25}, {2200, 30}};

30	static const Note n_lid_close[] = {{2200, 25}, {1800, 30}};

31	static const Note n_locked_wave[] = {{1600, 60}, {0, 40}, {1500, 120}};

32	static const Note n_lock[] = {{2000, 40}, {0, 20}, {1600, 120}};

33	static const Note n_unlock[] = {{1600, 40}, {0, 20}, {2000, 120}};

34	static const Note n_cal_start[] = {{2300, 40}, {0, 50}, {2300, 40}, {0, 50}, {2300, 40}, {0, 50}, {2700, 120}};

35	static const Note n_cal_step[] = {{2400, 40}};

36	static const Note n_saved[] = {{2200, 60}, {2600, 100}};

37	static const Note n_sim_on[] = {{1600, 60}, {1800, 60}, {2000, 60}, {2300, 140}};

38	static const Note n_sim_off[] = {{2300, 60}, {2000, 60}, {1800, 60}, {1600, 140}};

39	static const Note n_cal_cancel[] = {{2400, 40}, {0, 40}, {1900, 120}};

40	static const Note n_air_ready[] = {{2000, 40}, {2400, 70}};

41	static const Note n_back_main[] = {{2400, 30}, {2000, 50}};

42	static const Note n_full[] = {{2000, 180}, {0, 80}, {2000, 180}, {0, 80}, {2000, 180}, {0, 200}, {2600, 420}};

43	static const Note n_tamper_tick[] = {{2000, 60}};

44	static const Note n_tamper_tick_end[] = {{2400, 60}};

45	static const Note n_tamper[] = {{1600, 340, 2800}, {2800, 340, 1600}, {1600, 340, 2800}, {2800, 340, 1600}};

46	static const Note n_env_over[] = {{2400, 120}, {1900, 120}, {2400, 120}, {1900, 260}};

47	static const Note n_sensor_lost[] = {{1900, 100}, {0, 60}, {1500, 210}};

48	static const Note n_tamper_settled[] = {{1900, 60}, {2300, 60}, {2600, 120}};

49	static const Note n_env_ok[] = {{2600, 50}, {2200, 100}};

50	static const Note n_sensor_back[] = {{1500, 80}, {2000, 150}};

51	static const Note n_wifi_up[] = {{1800, 70}, {2200, 70}, {2700, 150}};

52	static const Note n_wifi_lost[] = {{2700, 90}, {2200, 90}, {1800, 190}};

53	static const Note n_cloud_down[] = {{2200, 60}, {0, 30}, {2400, 60}, {0, 30}, {1900, 130}};

54	static const Note n_cloud_up[] = {{1900, 60}, {0, 30}, {2400, 60}, {0, 30}, {2700, 120}};

55	static const Note n_cmd_fail[] = {{1900, 80}, {0, 30}, {1700, 80}, {0, 30}, {1500, 200}};

56	static const Note n_bt_wait[] = {{1800, 30}, {0, 30}, {1800, 30}, {2300, 60}};

57	static const Note n_bt_code[] = {{2400, 30}, {0, 60}, {2400, 30}};

58	static const Note n_bt_paired[] = {{2300, 40}, {2800, 40}, {2300, 40}, {2800, 140}};

59	static const Note n_bt_wrong[] = {{1800, 50}, {0, 25}, {1800, 50}, {1500, 140}};

60	static const Note n_bt_stopped[] = {{2200, 180}, {0, 50}, {1500, 320}};

61	static const Note n_bt_joined[] = {{1700, 60}, {2000, 60}, {2400, 60}, {2800, 180}};

62	static const Note n_bt_join_fail[] = {{1600, 70}, {0, 40}, {1500, 120}};

63	static const Note n_bt_off[] = {{2300, 50}, {1800, 50}, {0, 25}, {1500, 140}};

64	static const Note n_bt_auto_off[] = {{2300, 50}, {1800, 50}, {0, 25}, {1500, 60}, {0, 60}, {1500, 60}};

65

66	// Name, notes, note count, priority (0 low .. 5 alarm), kind.

67	static const TuneDef TUNES[] = {

68	  {"key", n_key, sizeof(n_key) / sizeof(Note), 0, K_UI},

69	  {"pin_key", n_pin_key, sizeof(n_pin_key) / sizeof(Note), 0, K_UI},

70	  {"pin_del", n_pin_del, sizeof(n_pin_del) / sizeof(Note), 0, K_UI},

71	  {"cancel", n_cancel, sizeof(n_cancel) / sizeof(Note), 0, K_UI},

72	  {"theme_light", n_theme_light, sizeof(n_theme_light) / sizeof(Note), 0, K_UI},

73	  {"theme_dark", n_theme_dark, sizeof(n_theme_dark) / sizeof(Note), 0, K_UI},

74	  {"open", n_open, sizeof(n_open) / sizeof(Note), 0, K_UI},

75	  {"pin_ok", n_pin_ok, sizeof(n_pin_ok) / sizeof(Note), 3, K_EVENT},

76	  {"pin_wrong", n_pin_wrong, sizeof(n_pin_wrong) / sizeof(Note), 3, K_EVENT},

77	  {"done", n_done, sizeof(n_done) / sizeof(Note), 3, K_EVENT},

78	  {"boot_ok", n_boot_ok, sizeof(n_boot_ok) / sizeof(Note), 1, K_EVENT},

79	  {"boot_fault", n_boot_fault, sizeof(n_boot_fault) / sizeof(Note), 4, K_EVENT},

80	  {"lid_open", n_lid_open, sizeof(n_lid_open) / sizeof(Note), 0, K_LID},

81	  {"lid_close", n_lid_close, sizeof(n_lid_close) / sizeof(Note), 0, K_LID},

82	  {"locked_wave", n_locked_wave, sizeof(n_locked_wave) / sizeof(Note), 2, K_EVENT},

83	  {"lock", n_lock, sizeof(n_lock) / sizeof(Note), 3, K_EVENT},

84	  {"unlock", n_unlock, sizeof(n_unlock) / sizeof(Note), 3, K_EVENT},

85	  {"cal_start", n_cal_start, sizeof(n_cal_start) / sizeof(Note), 2, K_CAL},

86	  {"cal_step", n_cal_step, sizeof(n_cal_step) / sizeof(Note), 1, K_CAL},

87	  {"saved", n_saved, sizeof(n_saved) / sizeof(Note), 2, K_CAL},

88	  {"sim_on", n_sim_on, sizeof(n_sim_on) / sizeof(Note), 2, K_EVENT},

89	  {"sim_off", n_sim_off, sizeof(n_sim_off) / sizeof(Note), 2, K_EVENT},

90	  {"cal_cancel", n_cal_cancel, sizeof(n_cal_cancel) / sizeof(Note), 2, K_CAL},

91	  {"air_ready", n_air_ready, sizeof(n_air_ready) / sizeof(Note), 1, K_EVENT},

92	  {"back_main", n_back_main, sizeof(n_back_main) / sizeof(Note), 1, K_EVENT},

93	  {"full", n_full, sizeof(n_full) / sizeof(Note), 5, K_ALARM},

94	  {"tamper_tick", n_tamper_tick, sizeof(n_tamper_tick) / sizeof(Note), 5, K_ALARM},

95	  {"tamper_tick_end", n_tamper_tick_end, sizeof(n_tamper_tick_end) / sizeof(Note), 5, K_ALARM},

96	  {"tamper", n_tamper, sizeof(n_tamper) / sizeof(Note), 5, K_ALARM},

97	  {"env_over", n_env_over, sizeof(n_env_over) / sizeof(Note), 4, K_ALARM},

98	  {"sensor_lost", n_sensor_lost, sizeof(n_sensor_lost) / sizeof(Note), 4, K_EVENT},

99	  {"tamper_settled", n_tamper_settled, sizeof(n_tamper_settled) / sizeof(Note), 4, K_EVENT},

100	  {"env_ok", n_env_ok, sizeof(n_env_ok) / sizeof(Note), 2, K_EVENT},

101	  {"sensor_back", n_sensor_back, sizeof(n_sensor_back) / sizeof(Note), 3, K_EVENT},

102	  {"wifi_up", n_wifi_up, sizeof(n_wifi_up) / sizeof(Note), 2, K_EVENT},

103	  {"wifi_lost", n_wifi_lost, sizeof(n_wifi_lost) / sizeof(Note), 3, K_EVENT},

104	  {"cloud_down", n_cloud_down, sizeof(n_cloud_down) / sizeof(Note), 2, K_EVENT},

105	  {"cloud_up", n_cloud_up, sizeof(n_cloud_up) / sizeof(Note), 2, K_EVENT},

106	  {"cmd_fail", n_cmd_fail, sizeof(n_cmd_fail) / sizeof(Note), 3, K_EVENT},

107	  {"bt_wait", n_bt_wait, sizeof(n_bt_wait) / sizeof(Note), 1, K_EVENT},

108	  {"bt_code", n_bt_code, sizeof(n_bt_code) / sizeof(Note), 2, K_EVENT},

109	  {"bt_paired", n_bt_paired, sizeof(n_bt_paired) / sizeof(Note), 3, K_EVENT},

110	  {"bt_wrong", n_bt_wrong, sizeof(n_bt_wrong) / sizeof(Note), 3, K_EVENT},

111	  {"bt_stopped", n_bt_stopped, sizeof(n_bt_stopped) / sizeof(Note), 4, K_EVENT},

112	  {"bt_joined", n_bt_joined, sizeof(n_bt_joined) / sizeof(Note), 3, K_EVENT},

113	  {"bt_join_fail", n_bt_join_fail, sizeof(n_bt_join_fail) / sizeof(Note), 3, K_EVENT},

114	  {"bt_off", n_bt_off, sizeof(n_bt_off) / sizeof(Note), 2, K_EVENT},

115	  {"bt_auto_off", n_bt_auto_off, sizeof(n_bt_auto_off) / sizeof(Note), 1, K_EVENT},

116	};

117

118	static_assert(sizeof(TUNES) / sizeof(TUNES[0]) == T_COUNT, "tune table and enum differ");

119

120	// playing holds the priority of the tune now sounding, 0xFF when silent.

121	static QueueHandle_t q;

122	static volatile uint8_t playing = 0xFF;

123	static constexpr uint8_t CH = 6, RES = 10;

124

125	// Volume 1/2/3 maps to PWM duty 100/256/512 of 1023 (50% is the loudest a

126	// passive buzzer gets).

127	static uint16_t duty() {

128	  uint8_t v = calSnapshot().buzz_vol;

129	  uint16_t d = v >= 3 ? 512 : v == 2 ? 256 : 100;

130	  return BUZZER_ACTIVE_LOW ? (1023 - d) : d;

131	}

132

133	static void silence() {

134	  ledcWriteTone(PIN_BUZZER, 0);

135	  ledcWrite(PIN_BUZZER, BUZZER_ACTIVE_LOW ? 1023 : 0);

136	}

137

138	static void tone(uint16_t hz) {

139	  ledcWriteTone(PIN_BUZZER, hz);

140	  ledcWrite(PIN_BUZZER, duty());

141	}

142

143	// Quiet hours, which may wrap past midnight (e.g. 22:00 to 06:00).

144	static bool quietNow(const Cal& c) {

145	  int m = localMinuteOfDay();

146	  if (!c.quiet_on || m < 0) return false;

147	  return c.quiet_from <= c.quiet_to ? (m >= c.quiet_from && m < c.quiet_to) : (m >= c.quiet_from || m < c.quiet_to);

148	}

149

150	// Plays one tune at a time. A new tune waiting in the queue cuts the current

151	// one short at the next note.

152	static void buzzerTask(void*) {

153	  uint8_t id;

154	  for (;;) {

155	    if (xQueueReceive(q, &id, portMAX_DELAY) != pdTRUE) continue;

156	    const TuneDef& t = TUNES[id];

157	    playing = t.pri;

158	    for (uint8_t i = 0; i < t.len; i++) {

159	      if (uxQueueMessagesWaiting(q)) break;

160	      const Note& n = t.n[i];

161	      if (!n.hz) {

162	        silence();

163	        vTaskDelay(pdMS_TO_TICKS(n.ms));

164	      } else if (n.hzEnd) {

165	        for (uint16_t s = 0; s < n.ms; s += 10) {

166	          tone(n.hz + (int32_t)(n.hzEnd - n.hz) * s / n.ms);

167	          vTaskDelay(pdMS_TO_TICKS(10));

168	        }

169	      } else {

170	        tone(n.hz);

171	        vTaskDelay(pdMS_TO_TICKS(n.ms));

172	      }

173	    }

174	    silence();

175	    playing = 0xFF;

176	  }

177	}

178

179	// 2 kHz carrier, 10-bit resolution, LEDC channel 6.

180	void buzzerInit() {

181	  ledcAttachChannel(PIN_BUZZER, 2000, RES, CH);

182	  silence();

183	  q = xQueueCreate(1, sizeof(uint8_t));

184	  xTaskCreatePinnedToCore(buzzerTask, "buzzer", 2048, nullptr, 2, nullptr, 1);

185	}

186

187	// Filters, in order: master switch, UI and lid sound switches, quiet hours

188	// (only priority-5 alarms play), alarms muted while collecting, only

189	// calibration and UI sounds while calibrating, and no interrupting a

190	// higher-priority tune. If a sound never plays, check these settings first.

191	void buzz(Tune id) {

192	  if (id >= T_COUNT || !q) return;

193	  const TuneDef& t = TUNES[id];

194	  Cal c = calSnapshot();

195	  BinState s = snapshot();

196	  if (!c.buzz_on) return;

197	  if (t.kind == K_UI && !c.buzz_ui) return;

198	  if (t.kind == K_LID && !c.buzz_lid) return;

199	  if (quietNow(c) && t.pri < 5) return;

200	  if (s.collecting && t.kind == K_ALARM) return;

201	  if (s.calibrating && t.kind != K_CAL && t.kind != K_UI) return;

202	  if (playing != 0xFF && t.pri < playing) return;

203	  uint8_t v = id;

204	  xQueueOverwrite(q, &v);

205	}

206

207	// Used by BUZZ_TEST from the apps. Skips all filters so the test always plays.

208	bool buzzByName(const char* name) {

209	  for (uint8_t i = 0; i < T_COUNT; i++)

210	    if (!strcmp(TUNES[i].name, name)) {

211	      uint8_t v = i;

212	      if (q) xQueueOverwrite(q, &v);

213	      return true;

214	    }

215	  return false;

216	}

217

218	bool buzzerBusy() { return playing != 0xFF; }

firmware/SmartBin/net.h

1	// net.h - Wi-Fi, the local web server (dashboard + REST API) and live events.

2	#pragma once

3	#include <Arduino.h>

4

5	void netInit();

6	void wifiConnect();

7	void startWebServer();

8	void pushTask(void*);

firmware/SmartBin/net.cpp

1	// ============================================================================

2	// net.cpp - Wi-Fi station, mDNS, SNTP and the local HTTP API

3	// Local API (port 80, CORS open so the dashboard file can be opened anywhere):

4	//   GET  /             the web dashboard (gzipped, from dashboard_gz.h)

5	//   GET  /api/status   full status JSON including calibration

6	//   GET  /api/events   Server-Sent Events: "status" on change, "ping" every 10 s

7	//   POST /api/command  {"cmd": "LOCK", "grams": 500, "sound": "full"}

8	//   POST /api/calibrate  any Cal keys, e.g. {"fill_th": 80}

9	//   POST /api/simulate {"mask": 3, "values": {"fill": 90}}

10	//   POST /api/cloud    {"url", "key", "email", "pass"} for Firebase

11	// Quick test from a laptop on the same network:

12	//   curl http://smartbin-01.local/api/status

13	// ============================================================================

14	#include "net.h"

15	#include "state.h"

16	#include "sensors.h"

17	#include "buzzer.h"

18	#include <WiFi.h>

19	#include <ESPmDNS.h>

20	#include <ESPAsyncWebServer.h>

21	#include <AsyncJson.h>

22	#include <esp_sntp.h>

23

24	// dashboard_gz.h is generated by tools/embed_dashboard.py. Without it the bin

25	// still builds and serves a small fallback page.

26	#if __has_include("dashboard_gz.h")

27	#include "dashboard_gz.h"

28	#define HAVE_DASHBOARD 1

29	#else

30	#define HAVE_DASHBOARD 0

31	#endif

32

33	static AsyncWebServer server(80);

34	static AsyncEventSource events("/api/events");

35	static bool everConnected = false;

36

37	// Wi-Fi callback, runs in the Wi-Fi event task. On GOT_IP it starts mDNS

38	// (smartbin-0x.local), starts SNTP time once, beeps and prints the address:

39	//   [wifi] 192.168.1.23  http://smartbin-01.local

40	// If that line never appears, the SSID or password is wrong or the network

41	// is 5 GHz only (the ESP32 supports 2.4 GHz only).

42	static void onWifiEvent(arduino_event_id_t event, arduino_event_info_t) {

43	  if (event == ARDUINO_EVENT_WIFI_STA_GOT_IP) {

44	    lockState();

45	    st.wifi = true;

46	    st.rssi = WiFi.RSSI();

47	    unlockState();

48	    MDNS.end();

49	    if (MDNS.begin(BIN_HOST)) MDNS.addService("http", "tcp", 80);

50	    if (!esp_sntp_enabled()) {

51	      esp_sntp_setoperatingmode(ESP_SNTP_OPMODE_POLL);

52	      esp_sntp_setservername(0, "pool.ntp.org");

53	      esp_sntp_setservername(1, "time.google.com");

54	      esp_sntp_init();

55	    }

56	    buzz(T_WIFI_UP);

57	    everConnected = true;

58	    Serial.printf("[wifi] %s  http://%s.local\n", WiFi.localIP().toString().c_str(), BIN_HOST);

59	    publish(EV_WIFI);

60	  } else if (event == ARDUINO_EVENT_WIFI_STA_DISCONNECTED) {

61	    bool was;

62	    lockState();

63	    was = st.wifi;

64	    st.wifi = false;

65	    unlockState();

66	    if (was) {

67	      buzz(T_WIFI_LOST);

68	      Serial.println("[wifi] lost");

69	      publish(EV_WIFI);

70	    }

71	  }

72	}

73

74	// persistent(false): the ESP32 SDK does not store credentials; this firmware

75	// keeps them in its own Preferences (see saveNet).

76	void netInit() {

77	  WiFi.persistent(false);

78	  WiFi.mode(WIFI_STA);

79	  WiFi.setHostname(BIN_HOST);

80	  WiFi.setAutoReconnect(true);

81	  WiFi.onEvent(onWifiEvent);

82	}

83

84	// Uses the saved network, or the one in secrets.h. Auto-reconnect handles drops.

85	void wifiConnect() {

86	  if (net.ssid.length() == 0 || net.ssid == "your-wifi-name") {

87	    Serial.println("[wifi] no network set; use WI-FI SETUP on the touchscreen");

88	    return;

89	  }

90	  WiFi.begin(net.ssid.c_str(), net.pass.c_str());

91	}

92

93	static void sendJson(AsyncWebServerRequest* r, int code, const String& body) {

94	  AsyncWebServerResponse* res = r->beginResponse(code, "application/json", body);

95	  res->addHeader("Cache-Control", "no-store");

96	  r->send(res);

97	}

98

99	// The status the bin will have after the command runs, returned at once so

100	// the dashboard can update its button without waiting for the next event.

101	static const char* expectStatus(const char* cmd, const BinState& s) {

102	  if (!strcmp(cmd, "LOCK")) return "LOCKED";

103	  if (!strcmp(cmd, "COLLECT")) return "COLLECTING";

104	  if (!strcmp(cmd, "CALIBRATE_START")) return "CALIBRATING";

105	  if (!strcmp(cmd, "UNLOCK") || !strcmp(cmd, "CLEAR_TAMPER") || !strcmp(cmd, "COLLECT_DONE") || !strcmp(cmd, "CALIBRATE_END"))

106	    return s.tamperPhase && strcmp(cmd, "CLEAR_TAMPER") ? "TAMPER" : (s.locked && strcmp(cmd, "UNLOCK") && !s.autoLocked) ? "LOCKED" : "OK";

107	  return STATUS_NAMES[s.status];

108	}

109

110	// POST /api/command. 400 = unknown command, 409 = not allowed right now (the

111	// body says why), 503 = command queue full.

112	static void handleCommand(AsyncWebServerRequest* r, JsonVariant& j) {

113	  const char* cmd = j["cmd"] | "";

114	  BinState s = snapshot();

115	  String why;

116	  JsonDocument out;

117	  out["cmd"] = cmd;

118	  if (!validateCommand(cmd, s, why)) {

119	    out["ok"] = false;

120	    out["why"] = why;

121	    String b;

122	    serializeJson(out, b);

123	    sendJson(r, why == "Unknown command" ? 400 : 409, b);

124	    return;

125	  }

126	  if (!sendCommand(cmd, SRC_LOCAL, j["grams"] | 0.0f, j["sound"] | (const char*)nullptr)) {

127	    sendJson(r, 503, "{\"ok\":false,\"why\":\"Busy, try again\"}");

128	    return;

129	  }

130	  out["ok"] = true;

131	  out["status"] = expectStatus(cmd, s);

132	  String b;

133	  serializeJson(out, b);

134	  sendJson(r, 200, b);

135	}

136

137	// POST /api/calibrate. Sending d_empty or d_full during calibration ticks

138	// the matching step on the bin's calibration screen.

139	static void handleCalibrate(AsyncWebServerRequest* r, JsonVariant& j) {

140	  if (!j.is<JsonObject>()) { sendJson(r, 400, "{\"ok\":false,\"why\":\"Send a JSON object\"}"); return; }

141	  JsonObjectConst o = j.as<JsonObjectConst>();

142	  lockState();

143	  Cal c = cal;

144	  bool any = calFromJson(o, c);

145	  if (any) {

146	    cal = c;

147	    if (!o["d_empty"].isNull()) st.calSteps |= 2;

148	    if (!o["d_full"].isNull()) st.calSteps |= 4;

149	    st.calVersion++;

150	    recomputeView();

151	  }

152	  bool calibrating = st.calibrating;

153	  unlockState();

154	  if (!any) { sendJson(r, 400, "{\"ok\":false,\"why\":\"No known calibration keys\"}"); return; }

155	  saveCal();

156	  buzz(calibrating ? T_CAL_STEP : T_SAVED);

157	  publish(EV_CAL);

158	  sendJson(r, 200, "{\"ok\":true}");

159	}

160

161	// POST /api/simulate. mask 0 turns simulation off. Values not sent keep their

162	// previous simulated value.

163	static void handleSimulate(AsyncWebServerRequest* r, JsonVariant& j) {

164	  lockState();

165	  sim.mask = (uint8_t)(j["mask"] | 0) & 0x7F;

166	  JsonObjectConst v = j["values"].as<JsonObjectConst>();

167	  if (!v.isNull()) {

168	    sim.fill = v["fill"] | sim.fill;

169	    sim.weight = v["weight"] | sim.weight;

170	    sim.gas = v["gas"] | sim.gas;

171	    sim.temp = v["temp"] | sim.temp;

172	    sim.humidity = v["humidity"] | sim.humidity;

173	    if (!v["tamper"].isNull()) sim.tamper = v["tamper"].as<bool>();

174	    if (!v["motion"].isNull()) sim.motion = v["motion"].as<bool>();

175	    if (!v["lid"].isNull()) sim.lid = v["lid"].as<bool>() || (v["lid"].is<const char*>() && !strcmp(v["lid"], "OPEN"));

176	  }

177	  recomputeView();

178	  unlockState();

179	  if (hControl) xTaskNotify(hControl, EV_STATE, eSetBits);

180	  publish();

181	  sendJson(r, 200, "{\"ok\":true}");

182	}

183

184	// POST /api/cloud. Saves Firebase settings and tells cloudTask to sign in

185	// again. An empty URL turns Firebase off.

186	static void handleCloud(AsyncWebServerRequest* r, JsonVariant& j) {

187	  String url = j["url"] | "", key = j["key"] | "", email = j["email"] | "", pass = j["pass"] | "";

188	  url.trim();

189	  while (url.endsWith("/")) url.remove(url.length() - 1);

190	  if (url.length() && !url.startsWith("https://") && !url.startsWith("http://")) {

191	    sendJson(r, 400, "{\"ok\":false,\"why\":\"The database URL starts with https://\"}");

192	    return;

193	  }

194	  net.fbUrl = url;

195	  net.fbKey = key;

196	  net.fbEmail = email;

197	  net.fbPass = pass;

198	  saveNet();

199	  if (hCloud) xTaskNotify(hCloud, EV_WIFI, eSetBits);

200	  sendJson(r, 200, "{\"ok\":true}");

201	}

202

203	// The extra CORS header allows the dashboard, opened from a public page,

204	// to call a bin on the private network (Chrome's Private Network Access).

205	void startWebServer() {

206	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Origin", "*");

207	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Methods", "GET, POST, OPTIONS");

208	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Headers", "Content-Type, X-SB-Op");

209	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Private-Network", "true");

210

211	  server.on("/api/status", HTTP_GET, [](AsyncWebServerRequest* r) { sendJson(r, 200, statusJson(true, false)); });

212

213	  auto add = [](const char* uri, ArJsonRequestHandlerFunction fn) {

214	    AsyncCallbackJsonWebHandler* h = new AsyncCallbackJsonWebHandler(uri, fn);

215	    h->setMethod(HTTP_POST);

216	    h->setMaxContentLength(1024);

217	    server.addHandler(h);

218	  };

219	  add("/api/command", handleCommand);

220	  add("/api/calibrate", handleCalibrate);

221	  add("/api/simulate", handleSimulate);

222	  add("/api/cloud", handleCloud);

223

224	  // A new live-event client gets the full status at once; 3000 is the

225	  // reconnect delay in ms suggested to the browser.

226	  events.onConnect([](AsyncEventSourceClient* c) { c->send(statusJson(true, false).c_str(), "status", millis(), 3000); });

227	  server.addHandler(&events);

228

229	  server.on("/", HTTP_GET, [](AsyncWebServerRequest* r) {

230	#if HAVE_DASHBOARD

231	    AsyncWebServerResponse* res = r->beginResponse(200, "text/html", DASHBOARD_GZ, DASHBOARD_GZ_LEN);

232	    res->addHeader("Content-Encoding", "gzip");

233	    r->send(res);

234	#else

235	    r->send(200, "text/html",

236	            "<!doctype html><meta name=viewport content='width=device-width'><title>" BIN_LABEL "</title>"

237	            "<body style='font-family:sans-serif;padding:24px'><h1>" BIN_LABEL "</h1>"

238	            "<p>The dashboard is not built into this firmware. Open <a href='/api/status'>/api/status</a>, or run "

239	            "tools/embed_dashboard.py and upload again.</p>");

240	#endif

241	  });

242

243	  server.onNotFound([](AsyncWebServerRequest* r) {

244	    if (r->method() == HTTP_OPTIONS) r->send(204);

245	    else sendJson(r, 404, "{\"ok\":false,\"why\":\"Not found\"}");

246	  });

247	  server.begin();

248	}

249

250	// Sends a "status" event to all live dashboards when st.version changes, at

251	// most every 250 ms, and a "ping" every 10 s so browsers notice a dead link.

252	// Also refreshes the Wi-Fi signal strength every 5 s.

253	void pushTask(void*) {

254	  uint32_t lastSent = 0, lastPush = 0, lastPing = 0, lastRssi = 0;

255	  for (;;) {

256	    uint32_t ev = 0;

257	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(250));

258	    uint32_t now = millis();

259	    if (now - lastRssi > 5000 && WiFi.isConnected()) {

260	      lastRssi = now;

261	      lockState();

262	      st.rssi = WiFi.RSSI();

263	      unlockState();

264	    }

265	    if (!events.count()) continue;

266	    lockState();

267	    uint32_t v = st.version;

268	    unlockState();

269	    if (v != lastSent && now - lastPush >= 250) {

270	      events.send(statusJson(true, false).c_str(), "status", now);

271	      lastSent = v;

272	      lastPush = now;

273	    }

274	    if (now - lastPing >= 10000) {

275	      lastPing = now;

276	      events.send("1", "ping", now);

277	    }

278	  }

279	}

firmware/SmartBin/cloud.h

1	// cloud.h - Firebase and ThingSpeak (runs as cloudTask on core 0).

2	#pragma once

3	#include <Arduino.h>

4

5	void cloudTask(void*);

6	bool cloudFirebaseOn();

firmware/SmartBin/cloud.cpp

1	// ============================================================================

2	// cloud.cpp - Firebase Realtime Database and ThingSpeak over HTTPS (REST)

3	// Firebase paths used (rules in firmware/firebase.rules.json):

4	//   sb/bins/<id>/status   status JSON (PUT on change, at least every 60 s)

5	//   sb/bins/<id>/cal      calibration (PUT when it changes)

6	//   sb/commands/<id>/<k>  commands from the app: read, then deleted

7	//   sb/results/<k>        result of each command, written back by the bin

8	//   sb/alerts/<id>        alert log, trimmed to the newest 50

9	//   sb/watchers           app/dashboard heartbeats (who is looking)

10	//   sb/usage              estimated monthly download, shared by both bins

11	// ThingSpeak fields: 1 fill %, 2 weight g, 3 temp C, 4 humidity %, 5 IAQ,

12	// 6 pressure hPa, 7 tilt deg, 8 flags (1 motion + 2 lid open + 4 x status).

13	// Serial log tags: [firebase], [thingspeak], [talkback], [alert].

14	// ============================================================================

15	#include "cloud.h"

16	#include "state.h"

17	#include "buzzer.h"

18	#include <WiFi.h>

19	#include <WiFiClientSecure.h>

20	#include <HTTPClient.h>

21	#include <vector>

22	#include <algorithm>

23

24	// One HTTP client per service, each with its own retry timer, so a slow

25	// ThingSpeak does not delay Firebase.

26	struct Svc {

27	  WiFiClientSecure tls;

28	  WiFiClient plain;

29	  HTTPClient http;

30	  uint32_t retryAt = 0;

31	  uint8_t fails = 0;

32	};

33

34	static Svc fb, ts, auth;

35	static String idToken;

36	static uint32_t tokenUntil = 0;

37	static bool fbOk = false;

38	static int watchers = 0;

39	static uint64_t monthBytes = 0;

40	static String monthKey;

41	static float usageFraction = 0;

42

43	bool cloudFirebaseOn() { return net.fbUrl.length() && net.fbKey.length() && net.fbEmail.length() && net.fbPass.length(); }

44

45	// One HTTP request with keep-alive. Returns the HTTP code, or a negative

46	// value for a connection error (no Wi-Fi, DNS, TLS handshake).

47	static int request(Svc& s, const String& url, const char* method, const String& body, String* out,

48	                   const char* ctype = "application/json") {

49	  Client& c = url.startsWith("https://") ? (Client&)s.tls : (Client&)s.plain;

50	  s.http.setReuse(true);

51	  s.http.setTimeout(8000);

52	  s.http.setConnectTimeout(6000);

53	  if (!s.http.begin((NetworkClient&)c, url)) return -1;

54	  if (body.length()) s.http.addHeader("Content-Type", ctype);

55	  int code = s.http.sendRequest(method, (uint8_t*)body.c_str(), body.length());

56	  if (out) *out = code > 0 ? s.http.getString() : String();

57	  else if (code > 0) s.http.getString();

58	  s.http.end();

59	  return code;

60	}

61

62	// After a failure, wait 5, 15, 30 then 60 s before the next try and close the

63	// connection so the next attempt starts fresh.

64	static void backoff(Svc& s) {

65	  static const uint16_t steps[] = {5, 15, 30, 60};

66	  s.retryAt = millis() + steps[min<uint8_t>(s.fails, 3)] * 1000UL;

67	  if (s.fails < 10) s.fails++;

68	  s.tls.stop();

69	  s.plain.stop();

70	}

71

72	static bool due(const Svc& s) { return (int32_t)(millis() - s.retryAt) >= 0; }

73

74	// Tracks whether Firebase is reachable; the change beeps, logs and shows on

75	// the Info screen as "Firebase OK" or "NOT SENDING".

76	static void setFbOk(bool ok) {

77	  if (ok == fbOk) return;

78	  fbOk = ok;

79	  lockState();

80	  st.cloudOk = ok;

81	  unlockState();

82	  buzz(ok ? T_CLOUD_UP : T_CLOUD_DOWN);

83	  Serial.printf("[firebase] %s\n", ok ? "reachable" : "cannot be reached");

84	  publish();

85	}

86

87	// URL-encodes keys and tokens for query strings.

88	static String enc(const String& s) {

89	  String o;

90	  const char* hex = "0123456789ABCDEF";

91	  for (size_t i = 0; i < s.length(); i++) {

92	    char ch = s[i];

93	    if (isalnum((unsigned char)ch) || ch == '-' || ch == '_' || ch == '.' || ch == '~') o += ch;

94	    else { o += '%'; o += hex[(ch >> 4) & 15]; o += hex[ch & 15]; }

95	  }

96	  return o;

97	}

98

99	// Email/password sign-in through Firebase Auth's REST API. The ID token lasts

100	// about an hour; it is renewed 5 minutes early. A "sign-in failed (400)" line

101	// usually means a wrong API key, email or password, or that Email/Password

102	// sign-in is not enabled in the Firebase console. The http:// branch is for

103	// the local mock server used in testing.

104	static bool signIn() {

105	  String url = net.fbUrl.startsWith("http://")

106	                   ? net.fbUrl + "/__auth/signInWithPassword?key=" + enc(net.fbKey)

107	                   : "https://identitytoolkit.googleapis.com/v1/accounts:signInWithPassword?key=" + enc(net.fbKey);

108	  JsonDocument d;

109	  d["email"] = net.fbEmail;

110	  d["password"] = net.fbPass;

111	  d["returnSecureToken"] = true;

112	  String body, resp;

113	  serializeJson(d, body);

114	  auth.http.setReuse(false);

115	  Client& c = url.startsWith("https://") ? (Client&)auth.tls : (Client&)auth.plain;

116	  auth.http.setTimeout(10000);

117	  if (!auth.http.begin((NetworkClient&)c, url)) return false;

118	  auth.http.addHeader("Content-Type", "application/json");

119	  int code = auth.http.POST(body);

120	  if (code > 0) resp = auth.http.getString();

121	  auth.http.end();

122	  auth.tls.stop();

123	  if (code != 200) {

124	    Serial.printf("[firebase] sign-in failed (%d) %s\n", code, resp.substring(0, 120).c_str());

125	    return false;

126	  }

127	  JsonDocument r;

128	  if (deserializeJson(r, resp)) return false;

129	  idToken = r["idToken"] | "";

130	  uint32_t secs = atoi(r["expiresIn"] | "3600");

131	  tokenUntil = millis() + (secs > 400 ? secs - 300 : secs) * 1000UL;

132	  return idToken.length() > 0;

133	}

134

135	static String fbUrl(const String& path, const String& query = "") {

136	  return net.fbUrl + "/" + path + ".json?auth=" + enc(idToken) + query;

137	}

138

139	// Every Firebase call goes through here: signs in when needed, retries once

140	// on 401 (expired token), backs off on network or 5xx errors. Writes are

141	// counted to estimate monthly download (each write is downloaded once per

142	// app or dashboard watching).

143	static int fbCall(const char* method, const String& path, const String& body, String* out, const String& query = "",

144	                  bool count = true) {

145	  if (!idToken.length() || (int32_t)(millis() - tokenUntil) >= 0) {

146	    if (!signIn()) {

147	      backoff(fb);

148	      setFbOk(false);

149	      return -1;

150	    }

151	  }

152	  int code = request(fb, fbUrl(path, query), method, body, out);

153	  if (code == 401) {

154	    idToken = "";

155	    code = request(fb, fbUrl(path, query), method, body, out);

156	  }

157	  if (code <= 0 || code >= 500) {

158	    backoff(fb);

159	    setFbOk(false);

160	    return code;

161	  }

162	  fb.fails = 0;

163	  setFbOk(code < 400);

164	  if (count && (!strcmp(method, "PUT") || !strcmp(method, "PATCH") || !strcmp(method, "POST")))

165	    monthBytes += (uint64_t)body.length() * max(1, watchers);

166	  return code;

167	}

168

169	static String currentMonth() {

170	  char b[12];

171	  monthKeyUtc(b, sizeof(b));

172	  return b;

173	}

174

175	// Restores this month's byte count after a reboot.

176	static void loadUsage() {

177	  String out;

178	  if (fbCall("GET", String("sb/usage/by_bin/") + BIN_ID, "", &out, "", false) != 200) return;

179	  JsonDocument d;

180	  if (deserializeJson(d, out) || !d.is<JsonObject>()) return;

181	  String m = d["month"] | "";

182	  monthKey = currentMonth();

183	  if (m == monthKey) monthBytes = d["bytes"] | (uint64_t)0;

184	}

185

186	// Saves this bin's count, then sums both bins and publishes the shared

187	// usage level (ok / warn 70% / near 90% / bad 100%) for the apps to show.

188	static void writeUsage() {

189	  String m = currentMonth();

190	  if (!m.length()) return;

191	  if (m != monthKey) { monthKey = m; monthBytes = 0; }

192	  JsonDocument me;

193	  me["month"] = m;

194	  me["bytes"] = monthBytes;

195	  String b;

196	  serializeJson(me, b);

197	  fbCall("PUT", String("sb/usage/by_bin/") + BIN_ID, b, nullptr, "", false);

198	  String out;

199	  if (fbCall("GET", "sb/usage/by_bin", "", &out, "", false) != 200) return;

200	  JsonDocument all;

201	  if (deserializeJson(all, out)) return;

202	  uint64_t total = 0;

203	  for (JsonPair p : all.as<JsonObject>())

204	    if (m == (p.value()["month"] | "")) total += p.value()["bytes"] | (uint64_t)0;

205	  usageFraction = (float)((double)total / (double)FB_MONTH_LIMIT);

206	  JsonDocument u;

207	  u["month"] = m;

208	  u["bytes_month"] = total;

209	  u["down_bytes"] = total;

210	  u["limit_bytes"] = FB_MONTH_LIMIT;

211	  u["fraction"] = round(usageFraction * 10000.0) / 10000.0;

212	  u["level"] = usageFraction >= 1 ? "bad" : usageFraction >= 0.9 ? "near" : usageFraction >= 0.7 ? "warn" : "ok";

213	  u["estimate"] = true;

214	  b = "";

215	  serializeJson(u, b);

216	  fbCall("PATCH", "sb/usage", b, nullptr, "", false);

217	}

218

219	// Counts apps/dashboards that sent a heartbeat in the last 30 s. Uses the

220	// "ts" index when the clock is set; falls back to reading the whole list.

221	static void readWatchers() {

222	  String out;

223	  int code = -1;

224	  if (clockSet()) {

225	    uint64_t since = nowMs() - 30000ULL;

226	    code = fbCall("GET", "sb/watchers", "", &out, "&orderBy=%22ts%22&startAt=" + String((unsigned long long)since), false);

227	  }

228	  if (code != 200) code = fbCall("GET", "sb/watchers", "", &out, "", false);

229	  if (code != 200) return;

230	  JsonDocument d;

231	  if (deserializeJson(d, out) || !d.is<JsonObject>()) { watchers = 0; return; }

232	  int n = 0;

233	  uint64_t now = nowMs();

234	  for (JsonPair p : d.as<JsonObject>()) {

235	    uint64_t t = p.value()["ts"] | (uint64_t)0;

236	    if (!now || (t && now - t <= 30000ULL)) n++;

237	  }

238	  watchers = n;

239	}

240

241	// Reads queued commands and deletes each one first, so a command never runs

242	// twice after a reboot. Only LOCK, UNLOCK and COLLECT are accepted from the

243	// cloud; calibration and simulation need the bin's own Wi-Fi. Commands older

244	// than CMD_MAX_AGE_MS are refused, so a lost phone cannot replay old taps.

245	static void pollCommands() {

246	  String out;

247	  if (fbCall("GET", String("sb/commands/") + BIN_ID, "", &out, "", false) != 200) return;

248	  if (out == "null" || out.length() < 3) return;

249	  JsonDocument d;

250	  if (deserializeJson(d, out) || !d.is<JsonObject>()) return;

251	  uint64_t now = nowMs();

252	  for (JsonPair p : d.as<JsonObject>()) {

253	    String key = p.key().c_str();

254	    const char* cmd = p.value()["cmd"] | "";

255	    uint64_t t = p.value()["ts"] | (uint64_t)0;

256	    fbCall("DELETE", String("sb/commands/") + BIN_ID + "/" + key, "", nullptr, "", false);

257	    bool stale = now && t && now > t && now - t > CMD_MAX_AGE_MS;

258	    bool allowed = !strcmp(cmd, "LOCK") || !strcmp(cmd, "UNLOCK") || !strcmp(cmd, "COLLECT");

259	    if (stale || !allowed) {

260	      JsonDocument r;

261	      r["ok"] = false;

262	      r["cmd"] = cmd;

263	      r["why"] = stale ? "Too old, ignored" : "Needs the bin's Wi-Fi";

264	      r["ts"][".sv"] = "timestamp";

265	      String b;

266	      serializeJson(r, b);

267	      fbCall("PUT", "sb/results/" + key, b, nullptr);

268	      continue;

269	    }

270	    Serial.printf("[firebase] command %s\n", cmd);

271	    if (!sendCommand(cmd, SRC_FIREBASE, 0, nullptr, key.c_str())) {

272	      JsonDocument r;

273	      r["ok"] = false;

274	      r["cmd"] = cmd;

275	      r["why"] = "Busy";

276	      String b;

277	      serializeJson(r, b);

278	      fbCall("PUT", "sb/results/" + key, b, nullptr);

279	    }

280	  }

281	}

282

283	// Writes queued command results to sb/results/<ref>.

284	static void flushResults() {

285	  CmdResult r;

286	  while (xQueueReceive(resultQueue, &r, 0) == pdTRUE) {

287	    if (!r.ref[0]) continue;

288	    JsonDocument d;

289	    d["ok"] = r.ok;

290	    d["cmd"] = r.cmd;

291	    if (r.why[0]) d["why"] = r.why;

292	    d["ts"][".sv"] = "timestamp";

293	    String b;

294	    serializeJson(d, b);

295	    fbCall("PUT", String("sb/results/") + r.ref, b, nullptr);

296	  }

297	}

298

299	// Posts queued alerts (POST makes a time-ordered key). Always logged to Serial.

300	static void flushAlerts(bool fbOn) {

301	  AlertEv a;

302	  while (xQueueReceive(alertQueue, &a, 0) == pdTRUE) {

303	    Serial.printf("[alert] %s: %s (%s)\n", a.title, a.meta, a.sev);

304	    if (!fbOn) continue;

305	    JsonDocument d;

306	    d["kind"] = a.kind;

307	    d["sev"] = a.sev;

308	    d["title"] = a.title;

309	    d["meta"] = a.meta;

310	    d["ts"][".sv"] = "timestamp";

311	    String b;

312	    serializeJson(d, b);

313	    fbCall("POST", String("sb/alerts/") + BIN_ID, b, nullptr);

314	  }

315	}

316

317	// Keeps the newest 50 alerts. Keys made by POST sort by time, so the lowest

318	// keys are the oldest.

319	static void trimAlerts() {

320	  String out;

321	  if (fbCall("GET", String("sb/alerts/") + BIN_ID, "", &out, "&shallow=true", false) != 200) return;

322	  JsonDocument d;

323	  if (deserializeJson(d, out) || !d.is<JsonObject>()) return;

324	  JsonObject o = d.as<JsonObject>();

325	  int n = o.size();

326	  if (n <= 50) return;

327	  std::vector<String> keys;

328	  for (JsonPair p : o) keys.push_back(p.key().c_str());

329	  std::sort(keys.begin(), keys.end());

330	  for (int i = 0; i < n - 50; i++) fbCall("DELETE", String("sb/alerts/") + BIN_ID + "/" + keys[i], "", nullptr, "", false);

331	}

332

333	// What was last sent to a service, used to skip uploads when nothing changed.

334	struct Sent {

335	  float fill = -100, weight = -1000, temp = -100, hum = -100, iaq = -100, tilt = -100;

336	  uint8_t status = 255, tamper = 255, acc = 255;

337	  bool lid = false, motion = false, full = false, env = false;

338	  uint8_t faults = 255;

339	};

340

341	static uint8_t faultBits(const BinState& s) { return (!s.okHC) | (!s.okHX << 1) | (!s.okBME << 2) | (!s.okIMU << 3); }

342

343	// A change in status, lid, tamper, alarms or faults: sent right away.

344	static bool importantChange(const Sent& a, const BinState& s) {

345	  return a.status != s.status || a.lid != s.lidOpen || a.tamper != s.tamperPhase || a.full != s.alarmFull ||

346	         a.env != s.alarmEnv || a.faults != faultBits(s);

347	}

348

349	// Sensor drift above the same steps used in sensorTask: sent on the slower pace.

350	static bool minorChange(const Sent& a, const BinState& s) {

351	  const Reading& v = s.v;

352	  return fabsf(a.fill - v.fill) >= 1 || fabsf(a.weight - v.weight_g) >= 20 || fabsf(a.temp - v.temp) >= 0.2f ||

353	         fabsf(a.hum - v.humidity) >= 1 || fabsf(a.iaq - v.iaq) >= 5 || fabsf(a.tilt - v.tilt) >= 1 ||

354	         a.motion != v.motion || a.acc != v.iaq_acc;

355	}

356

357	static void remember(Sent& a, const BinState& s) {

358	  a.fill = s.v.fill; a.weight = s.v.weight_g; a.temp = s.v.temp; a.hum = s.v.humidity; a.iaq = s.v.iaq;

359	  a.tilt = s.v.tilt; a.status = s.status; a.tamper = s.tamperPhase; a.acc = s.v.iaq_acc; a.lid = s.lidOpen;

360	  a.motion = s.v.motion; a.full = s.alarmFull; a.env = s.alarmEnv; a.faults = faultBits(s);

361	}

362

363	// One ThingSpeak channel update. The reply is the entry number; "0" means the

364	// write was refused (wrong key, or less than 15 s since the last write).

365	static bool tsWrite(const BinState& s) {

366	  const Reading& v = s.v;

367	  int flags = (v.motion ? 1 : 0) + (s.lidOpen ? 2 : 0) + 4 * s.status;

368	  String url = net.tsServer + "/update?api_key=" + enc(net.tsWriteKey) + "&field1=" + String(v.fill, 1) +

369	               "&field2=" + String(v.weight_g, 0) + "&field3=" + String(v.temp, 2) + "&field4=" + String(v.humidity, 1) +

370	               "&field5=" + String(v.iaq, 0) + "&field6=" + String(v.pressure, 1) + "&field7=" + String(v.tilt, 1) +

371	               "&field8=" + String(flags) + "&status=" + enc(localIp());

372	  String out;

373	  int code = request(ts, url, "GET", "", &out);

374	  out.trim();

375	  bool ok = code == 200 && out != "0";

376	  if (code <= 0 || code >= 500) backoff(ts);

377	  else ts.fails = 0;

378	  lockState();

379	  st.tsOk = ok;

380	  unlockState();

381	  if (!ok) Serial.printf("[thingspeak] write failed (%d) %s\n", code, out.c_str());

382	  return ok;

383	}

384

385	static Svc tb;

386

387	// ThingSpeak TalkBack: takes the next queued command (LOCK, UNLOCK or COLLECT).

388	static void talkBack() {

389	  String url = net.tsServer + "/talkbacks/" + enc(net.tbId) + "/commands/execute.json";

390	  String out;

391	  int code = request(tb, url, "POST", "api_key=" + enc(net.tbKey), &out, "application/x-www-form-urlencoded");

392	  if (code <= 0 || code >= 500) { backoff(tb); return; }

393	  tb.fails = 0;

394	  if (code != 200 || out.length() < 3) return;

395	  JsonDocument d;

396	  if (deserializeJson(d, out)) return;

397	  String cmd = d["command_string"] | "";

398	  cmd.trim();

399	  cmd.toUpperCase();

400	  if (cmd == "LOCK" || cmd == "UNLOCK" || cmd == "COLLECT") {

401	    Serial.printf("[talkback] command %s\n", cmd.c_str());

402	    sendCommand(cmd.c_str(), SRC_TALKBACK);

403	  }

404	}

405

406	// Main cloud loop, every 500 ms or when woken. Paused while Wi-Fi is down or

407	// Bluetooth setup is running (the radio is shared), in which case alerts are

408	// only printed to Serial.

409	void cloudTask(void*) {

410	  for (Svc* s : {&fb, &ts, &auth, &tb}) {

411	    // The server certificate is not checked (no CA bundle on the board). The

412	    // traffic is still encrypted.

413	    s->tls.setInsecure();

414	    s->tls.setHandshakeTimeout(12);

415	  }

416	  Sent fbSent, tsSent;

417	  uint32_t lastStatus = 0, lastCmd = 0, lastWatch = 0, lastUsage = 0, lastTrim = 0, lastTs = 0, lastTb = 0, lastCalVer = 0;

418	  bool usageLoaded = false, wasConnected = false;

419	  bool calSent = false;

420

421	  for (;;) {

422	    uint32_t ev = 0;

423	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(500));

424	    // New network or new Firebase settings: sign in again from scratch.

425	    if (ev & EV_WIFI) {

426	      idToken = "";

427	      fb.retryAt = 0;

428	      fb.fails = 0;

429	      calSent = false;

430	    }

431	    BinState s = snapshot();

432	    bool online = WiFi.isConnected() && !s.btOn;

433	    bool fbOn = cloudFirebaseOn();

434	    if (!online) {

435	      if (wasConnected) {

436	        for (Svc* x : {&fb, &ts, &tb}) { x->tls.stop(); x->plain.stop(); }

437	      }

438	      wasConnected = false;

439	      AlertEv a;

440	      while (xQueueReceive(alertQueue, &a, 0) == pdTRUE) Serial.printf("[alert] %s: %s\n", a.title, a.meta);

441	      continue;

442	    }

443	    wasConnected = true;

444	    uint32_t now = millis();

445

446	    // --- Firebase ---

447	    // Status pace: right away on an important change, every 5 s on minor

448	    // changes while someone watches (15 s above 90% of the free quota,

449	    // 60 s at 100%), and a 60 s heartbeat otherwise.

450	    if (fbOn && due(fb)) {

451	      if (!usageLoaded && clockSet()) {

452	        usageLoaded = true;

453	        loadUsage();

454	      }

455	      flushResults();

456	      flushAlerts(true);

457

458	      if (now - lastWatch >= FB_WATCHERS_MS || lastWatch == 0) {

459	        lastWatch = now;

460	        readWatchers();

461	      }

462	      bool watched = watchers > 0 || !FB_REQUIRE_WATCHERS;

463	      uint32_t every = !watched ? FB_IDLE_MS : usageFraction >= 0.9f ? 15000 : FB_WATCHED_MS;

464	      if (usageFraction >= 1.0f) every = FB_IDLE_MS;

465	      bool imp = importantChange(fbSent, s);

466	      bool minor = minorChange(fbSent, s);

467	      if (imp || (minor && now - lastStatus >= every) || now - lastStatus >= FB_IDLE_MS || lastStatus == 0) {

468	        String body = statusJson(false, true);

469	        if (fbCall("PUT", String("sb/bins/") + BIN_ID + "/status", body, nullptr) == 200) {

470	          remember(fbSent, s);

471	          lastStatus = now;

472	        }

473	      }

474	      if (!calSent || s.calVersion != lastCalVer) {

475	        JsonDocument d;

476	        calToJson(calSnapshot(), d.to<JsonObject>());

477	        String b;

478	        serializeJson(d, b);

479	        if (fbCall("PUT", String("sb/bins/") + BIN_ID + "/cal", b, nullptr) == 200) {

480	          calSent = true;

481	          lastCalVer = s.calVersion;

482	        }

483	      }

484	      if (now - lastCmd >= (watched ? FB_CMD_WATCHED_MS : FB_CMD_IDLE_MS)) {

485	        lastCmd = now;

486	        pollCommands();

487	      }

488	      if (now - lastUsage >= FB_USAGE_MS || lastUsage == 0) {

489	        lastUsage = now;

490	        writeUsage();

491	      }

492	      if (now - lastTrim >= 600000 || lastTrim == 0) {

493	        lastTrim = now;

494	        trimAlerts();

495	      }

496	    } else if (!fbOn) {

497	      flushAlerts(false);

498	      CmdResult r;

499	      while (xQueueReceive(resultQueue, &r, 0) == pdTRUE) {}

500	    }

501

502	    // --- ThingSpeak: every 30 s on change, 15.5 s on an alarm edge, and a

503	    // 5 minute heartbeat so the charts never show a gap. ---

504	    if (net.tsWriteKey.length() && due(ts)) {

505	      bool alertEdge = s.alarmFull != tsSent.full || s.tamperPhase != tsSent.tamper || s.status != tsSent.status;

506	      uint32_t since = now - lastTs;

507	      bool write = lastTs == 0 || (alertEdge && since >= TS_MIN_GAP_MS) ||

508	                   (since >= TS_WRITE_MS && (minorChange(tsSent, s) || importantChange(tsSent, s))) || since >= TS_HEARTBEAT_MS;

509	      if (write) {

510	        lastTs = now;

511	        if (tsWrite(s)) remember(tsSent, s);

512	      }

513	    }

514

515	    // --- TalkBack polling every 5 s (15 s when nobody watches) ---

516	    if (net.tbId.length() && net.tbKey.length() && due(tb)) {

517	      uint32_t every = (fbOn && watchers == 0 && FB_REQUIRE_WATCHERS) ? TB_IDLE_MS : TB_WATCHED_MS;

518	      if (now - lastTb >= every) {

519	        lastTb = now;

520	        talkBack();

521	      }

522	    }

523	  }

524	}

firmware/SmartBin/ble_setup.h

1	// ble_setup.h - Bluetooth LE Wi-Fi setup (started from WI-FI SETUP on the screen).

2	#pragma once

3	#include <Arduino.h>

4

5	enum BtPhase : uint8_t { BT_IDLE = 0, BT_WAIT, BT_CODE, BT_LOCKOUT, BT_PAIRED, BT_CONN, BT_OK, BT_FAIL, BT_OFF };

6

7	struct BtView {

8	  BtPhase phase;

9	  uint32_t code;

10	  uint32_t phaseMs;      // millis() when the phase began

11	  uint32_t offAtMs;      // millis() when Bluetooth switches itself off

12	  char ssid[33];

13	  char why[40];

14	  char ip[16];

15	  uint8_t netCount;

16	  char nets[6][33];

17	  int8_t rssi[6];

18	};

19

20	void bleStart();

21	void bleStop(bool fromRemote);

22	BtView bleView();

23	void bleRetry();

firmware/SmartBin/ble_setup.cpp

1	// ============================================================================

2	// ble_setup.cpp - give the bin a new Wi-Fi network over Bluetooth LE

3	// Flow: WI-FI SETUP + Collect PIN on the screen -> advertise "SmartBin-0x" ->

4	// the phone or laptop connects -> the bin shows a 6-digit passkey -> user

5	// types it -> paired -> app writes SSID and password, then 1 to CTRL -> the

6	// bin tries to join for up to 15 s and reports on STAT. Three wrong passkeys

7	// stop pairing for 60 s. Bluetooth turns itself off after 5 minutes idle.

8	// Characteristics (UUIDs in config.h): SSID w, PASS w, CTRL w (1 join, 2 stop),

9	// STAT read/notify JSON, SCAN read JSON of the 6 strongest networks.

10	// ============================================================================

11	#include "ble_setup.h"

12	#include "state.h"

13	#include "buzzer.h"

14	#include <WiFi.h>

15	#include <NimBLEDevice.h>

16

17	// view is read by the UI task and written by BLE callbacks, so vLock guards it.

18	static SemaphoreHandle_t vLock;

19	static BtView view;

20	static volatile bool running = false, stopReq = false, stopRemote = false, joinReq = false, retryReq = false;

21	static String newSsid, newPass;

22	static uint8_t wrongCodes = 0;

23	static volatile bool codeShown = false;

24	static NimBLEServer* server = nullptr;

25	static NimBLECharacteristic *chStat = nullptr, *chScan = nullptr;

26	static uint16_t connHandle = 0xFFFF;

27	static TaskHandle_t hBle = nullptr;

28

29	// Changes the setup phase and wakes the UI task to redraw the BT screen.

30	static void setPhase(BtPhase p) {

31	  xSemaphoreTake(vLock, portMAX_DELAY);

32	  view.phase = p;

33	  view.phaseMs = millis();

34	  xSemaphoreGive(vLock);

35	  if (hUi) xTaskNotify(hUi, EV_BT, eSetBits);

36	}

37

38	BtView bleView() {

39	  if (!vLock) { BtView v{}; return v; }

40	  xSemaphoreTake(vLock, portMAX_DELAY);

41	  BtView v = view;

42	  xSemaphoreGive(vLock);

43	  return v;

44	}

45

46	static void notifyStat(const String& json) {

47	  if (!chStat) return;

48	  chStat->setValue(json);

49	  chStat->notify();

50	}

51

52	// Connection and pairing callbacks from the NimBLE stack.

53	class ServerCb : public NimBLEServerCallbacks {

54	  void onConnect(NimBLEServer*, NimBLEConnInfo& info) override {

55	    connHandle = info.getConnHandle();

56	    codeShown = false;

57	    xSemaphoreTake(vLock, portMAX_DELAY);

58	    view.offAtMs = millis() + BT_IDLE_OFF_MS;

59	    xSemaphoreGive(vLock);

60	  }

61	  void onDisconnect(NimBLEServer*, NimBLEConnInfo&, int) override {

62	    connHandle = 0xFFFF;

63	    BtPhase p = bleView().phase;

64	    if (p == BT_CODE || p == BT_PAIRED) setPhase(BT_WAIT);

65	    if (p != BT_LOCKOUT && p != BT_OFF && running) NimBLEDevice::startAdvertising();

66	  }

67	  // Called when the phone asks to pair: pick a random 6-digit code, show it

68	  // on the screen and return it to the stack.

69	  uint32_t onPassKeyDisplay() override {

70	    uint32_t code = esp_random() % 1000000;

71	    xSemaphoreTake(vLock, portMAX_DELAY);

72	    view.code = code;

73	    xSemaphoreGive(vLock);

74	    codeShown = true;

75	    setPhase(BT_CODE);

76	    buzz(T_BT_CODE);

77	    return code;

78	  }

79	  void onAuthenticationComplete(NimBLEConnInfo& info) override {

80	    if (info.isEncrypted() && info.isAuthenticated()) {

81	      wrongCodes = 0;

82	      setPhase(BT_PAIRED);

83	      buzz(T_BT_PAIRED);

84	      return;

85	    }

86	    server->disconnect(info.getConnHandle());

87	    // Only a code that was shown and then typed wrong counts. A phone or laptop that

88	    // still remembers an old pairing fails here without any code, and that must not

89	    // use up the 3 tries.

90	    bool counted = codeShown;

91	    codeShown = false;

92	    if (!counted) {

93	      setPhase(BT_WAIT);

94	      return;

95	    }

96	    if (++wrongCodes >= 3) {

97	      NimBLEDevice::stopAdvertising();

98	      setPhase(BT_LOCKOUT);

99	      buzz(T_BT_STOPPED);

100	    } else {

101	      setPhase(BT_WAIT);

102	      buzz(T_BT_WRONG);

103	    }

104	  }

105	};

106

107	// Writes from the app. Every write pushes the auto-off time back 5 minutes.

108	class CharCb : public NimBLECharacteristicCallbacks {

109	  void onWrite(NimBLECharacteristic* c, NimBLEConnInfo&) override {

110	    std::string v = c->getValue();

111	    NimBLEUUID u = c->getUUID();

112	    if (u == NimBLEUUID(BT_SSID_UUID)) newSsid = String(v.c_str());

113	    else if (u == NimBLEUUID(BT_PASS_UUID)) newPass = String(v.c_str());

114	    else if (u == NimBLEUUID(BT_CTRL_UUID) && v.size()) {

115	      uint8_t b = (uint8_t)v[0];

116	      if (b == 1 || b == '1') joinReq = true;

117	      else if (b == 2 || b == '2') { stopRemote = true; stopReq = true; }

118	    }

119	    xSemaphoreTake(vLock, portMAX_DELAY);

120	    view.offAtMs = millis() + BT_IDLE_OFF_MS;

121	    xSemaphoreGive(vLock);

122	  }

123	  // The SCAN value is built fresh on each read from the last Wi-Fi scan.

124	  void onRead(NimBLECharacteristic* c, NimBLEConnInfo&) override {

125	    if (c != chScan) return;

126	    BtView v = bleView();

127	    JsonDocument d;

128	    JsonArray a = d.to<JsonArray>();

129	    for (int i = 0; i < v.netCount; i++) {

130	      JsonObject o = a.add<JsonObject>();

131	      o["ssid"] = v.nets[i];

132	      o["rssi"] = v.rssi[i];

133	    }

134	    String s;

135	    serializeJson(d, s);

136	    c->setValue(s);

137	  }

138	};

139

140	static ServerCb serverCb;

141	static CharCb charCb;

142

143	// Keeps the 6 strongest unique network names from the last async scan,

144	// strongest first.

145	static void collectScan() {

146	  int n = WiFi.scanComplete();

147	  if (n < 0) return;

148	  struct Net { String s; int r; };

149	  Net best[6];

150	  int k = 0;

151	  for (int i = 0; i < n; i++) {

152	    String s = WiFi.SSID(i);

153	    if (!s.length()) continue;

154	    bool dup = false;

155	    for (int j = 0; j < k; j++) if (best[j].s == s) dup = true;

156	    if (dup) continue;

157	    int r = WiFi.RSSI(i);

158	    if (k < 6) best[k++] = {s, r};

159	    else {

160	      int w = 0;

161	      for (int j = 1; j < 6; j++) if (best[j].r < best[w].r) w = j;

162	      if (r > best[w].r) best[w] = {s, r};

163	    }

164	  }

165	  for (int i = 0; i < k; i++)

166	    for (int j = i + 1; j < k; j++)

167	      if (best[j].r > best[i].r) { Net t = best[i]; best[i] = best[j]; best[j] = t; }

168	  xSemaphoreTake(vLock, portMAX_DELAY);

169	  view.netCount = k;

170	  for (int i = 0; i < k; i++) {

171	    strlcpy(view.nets[i], best[i].s.c_str(), sizeof(view.nets[i]));

172	    view.rssi[i] = best[i].r;

173	  }

174	  xSemaphoreGive(vLock);

175	  WiFi.scanDelete();

176	}

177

178	// Tries the new network for 15 s. On success it is saved and the IP is sent

179	// to the app; on failure the bin goes back to its old network. "Wrong

180	// password" and "Network not found" come from the ESP32's own status codes.

181	static void doJoin() {

182	  String ssid = newSsid, pass = newPass;

183	  xSemaphoreTake(vLock, portMAX_DELAY);

184	  strlcpy(view.ssid, ssid.c_str(), sizeof(view.ssid));

185	  xSemaphoreGive(vLock);

186	  setPhase(BT_CONN);

187	  notifyStat("{\"state\":\"CONNECTING\"}");

188	  String oldSsid = net.ssid, oldPass = net.pass;

189	  WiFi.disconnect(false, false);

190	  vTaskDelay(pdMS_TO_TICKS(200));

191	  WiFi.begin(ssid.c_str(), pass.c_str());

192	  uint32_t t0 = millis();

193	  wl_status_t s = WL_IDLE_STATUS;

194	  while (millis() - t0 < 15000) {

195	    s = WiFi.status();

196	    if (s == WL_CONNECTED || s == WL_CONNECT_FAILED || s == WL_NO_SSID_AVAIL) break;

197	    vTaskDelay(pdMS_TO_TICKS(250));

198	  }

199	  if (s == WL_CONNECTED) {

200	    net.ssid = ssid;

201	    net.pass = pass;

202	    saveNet();

203	    String ip = WiFi.localIP().toString();

204	    xSemaphoreTake(vLock, portMAX_DELAY);

205	    strlcpy(view.ip, ip.c_str(), sizeof(view.ip));

206	    view.offAtMs = millis() + BT_IDLE_OFF_MS;

207	    xSemaphoreGive(vLock);

208	    setPhase(BT_OK);

209	    buzz(T_BT_JOINED);

210	    notifyStat("{\"state\":\"CONNECTED\",\"ip\":\"" + ip + "\",\"ssid\":\"" + ssid + "\"}");

211	    return;

212	  }

213	  const char* why = s == WL_NO_SSID_AVAIL ? "Network not found" : s == WL_CONNECT_FAILED ? "Wrong password" : "Could not join";

214	  xSemaphoreTake(vLock, portMAX_DELAY);

215	  strlcpy(view.why, why, sizeof(view.why));

216	  xSemaphoreGive(vLock);

217	  setPhase(BT_FAIL);

218	  buzz(T_BT_JOIN_FAIL);

219	  notifyStat(String("{\"state\":\"FAILED\",\"why\":\"") + why + "\"}");

220	  WiFi.disconnect(false, false);

221	  if (oldSsid.length()) WiFi.begin(oldSsid.c_str(), oldPass.c_str());

222	}

223

224	// Stops advertising and releases the whole BLE stack (deinit(true) frees its

225	// memory). cloudTask resumes once st.btOn is false again.

226	static void shutdown(bool remote, bool automatic) {

227	  NimBLEDevice::stopAdvertising();

228	  NimBLEDevice::deinit(true);

229	  server = nullptr;

230	  chStat = chScan = nullptr;

231	  running = false;

232	  lockState();

233	  st.btOn = false;

234	  unlockState();

235	  publish();

236	  if (remote || automatic) {

237	    setPhase(BT_OFF);

238	    buzz(automatic ? T_BT_AUTO_OFF : T_BT_OFF);

239	  } else {

240	    setPhase(BT_IDLE);

241	  }

242	}

243

244	// Watches for stop, timeout, join and retry requests every 200 ms, refreshes

245	// the scan every 30 s, and deletes itself when Bluetooth turns off.

246	static void bleTask(void*) {

247	  uint32_t lastScan = 0;

248	  for (;;) {

249	    vTaskDelay(pdMS_TO_TICKS(200));

250	    uint32_t now = millis();

251	    if (stopReq) {

252	      bool remote = stopRemote;

253	      stopReq = false;

254	      stopRemote = false;

255	      shutdown(remote, false);

256	      break;

257	    }

258	    BtView v = bleView();

259	    if ((int32_t)(now - v.offAtMs) >= 0 && v.phase != BT_CONN) {

260	      shutdown(false, true);

261	      break;

262	    }

263	    if (joinReq) {

264	      joinReq = false;

265	      doJoin();

266	    }

267	    if (retryReq) {

268	      retryReq = false;

269	      setPhase(connHandle != 0xFFFF ? BT_PAIRED : BT_WAIT);

270	    }

271	    if (v.phase == BT_CODE && now - v.phaseMs >= BT_CODE_MS && connHandle != 0xFFFF) {

272	      server->disconnect(connHandle);

273	      setPhase(BT_WAIT);

274	    }

275	    if (v.phase == BT_LOCKOUT && now - v.phaseMs >= BT_LOCKOUT_MS) {

276	      wrongCodes = 0;

277	      setPhase(BT_WAIT);

278	      NimBLEDevice::startAdvertising();

279	      buzz(T_BT_WAIT);

280	    }

281	    collectScan();

282	    if (now - lastScan > 30000 && v.phase != BT_CONN && WiFi.scanComplete() != WIFI_SCAN_RUNNING) {

283	      lastScan = now;

284	      WiFi.scanNetworks(true);

285	    }

286	  }

287	  hBle = nullptr;

288	  vTaskDelete(nullptr);

289	}

290

291	// Security: bonding, MITM protection and secure connections, with the bin as

292	// a display-only device. Every characteristic needs an encrypted,

293	// authenticated link, so nothing can be written before the passkey step.

294	void bleStart() {

295	  if (!vLock) vLock = xSemaphoreCreateMutex();

296	  if (running) return;

297	  running = true;

298	  wrongCodes = 0;

299	  stopReq = false;

300	  stopRemote = false;

301	  joinReq = false;

302	  retryReq = false;

303	  xSemaphoreTake(vLock, portMAX_DELAY);

304	  memset(&view, 0, sizeof(view));

305	  view.offAtMs = millis() + BT_IDLE_OFF_MS;

306	  xSemaphoreGive(vLock);

307	  setPhase(BT_WAIT);

308	  lockState();

309	  st.btOn = true;

310	  unlockState();

311	  publish();

312

313	  NimBLEDevice::init(BIN_BLE_NAME);

314	  NimBLEDevice::setSecurityAuth(true, true, true);

315	  NimBLEDevice::setSecurityIOCap(BLE_HS_IO_DISPLAY_ONLY);

316	  server = NimBLEDevice::createServer();

317	  server->setCallbacks(&serverCb, false);

318	  NimBLEService* svc = server->createService(BT_SVC_UUID);

319	  const uint32_t W = NIMBLE_PROPERTY::WRITE | NIMBLE_PROPERTY::WRITE_ENC | NIMBLE_PROPERTY::WRITE_AUTHEN;

320	  const uint32_t R = NIMBLE_PROPERTY::READ | NIMBLE_PROPERTY::READ_ENC | NIMBLE_PROPERTY::READ_AUTHEN;

321	  svc->createCharacteristic(BT_SSID_UUID, W)->setCallbacks(&charCb);

322	  svc->createCharacteristic(BT_PASS_UUID, W)->setCallbacks(&charCb);

323	  svc->createCharacteristic(BT_CTRL_UUID, W)->setCallbacks(&charCb);

324	  chStat = svc->createCharacteristic(BT_STAT_UUID, R | NIMBLE_PROPERTY::NOTIFY);

325	  chStat->setValue("{\"state\":\"WAITING\"}");

326	  chScan = svc->createCharacteristic(BT_SCAN_UUID, R);

327	  chScan->setCallbacks(&charCb);

328	  server->start();

329	  NimBLEAdvertising* adv = NimBLEDevice::getAdvertising();

330	  adv->setName(BIN_BLE_NAME);

331	  adv->addServiceUUID(BT_SVC_UUID);

332	  adv->enableScanResponse(true);

333	  adv->start();

334	  WiFi.scanNetworks(true);

335	  buzz(T_BT_WAIT);

336	  xTaskCreatePinnedToCore(bleTask, "ble", 6144, nullptr, 1, &hBle, 0);

337	}

338

339	// Asks bleTask to shut Bluetooth down on its next pass.

340	void bleStop(bool fromRemote) {

341	  if (!running) return;

342	  stopRemote = fromRemote;

343	  stopReq = true;

344	}

345

346	void bleRetry() { retryReq = true; }

firmware/SmartBin/ui.h

1	// ui.h - 2.4-inch ILI9341 touchscreen (240 x 320, portrait) and its task.

2	#pragma once

3	#include <Arduino.h>

4

5	void uiInit();

6	void uiTask(void*);

7	void isrTouch();

8	void uiStartTouchCal();

9	void uiResetTouchCal();

firmware/SmartBin/ui.cpp

1	// ============================================================================

2	// ui.cpp - touchscreen user interface

3	// uiTask (core 1, priority 2) redraws about every 100 ms. Each screen is drawn

4	// by one draw*() function; pickScreen() decides which one is shown.

5	// To avoid flicker, nothing is redrawn unless it changed: every widget passes

6	// a short "signature" string to dirty(), and only a different signature

7	// triggers a redraw. If part of a screen does not update, its signature is

8	// probably missing the value that changed.

9	// Touches are matched against the hit boxes that buttons register while they

10	// are drawn (addHit), so a button that is not drawn cannot be pressed.

11	// ============================================================================

12	#include "ui.h"

13	#include "state.h"

14	#include "buzzer.h"

15	#include "ble_setup.h"

16	#include "sensors.h"

17	#include <SPI.h>

18	#include <WiFi.h>

19	#include <Adafruit_GFX.h>

20	#include <Adafruit_ILI9341.h>

21	#include <XPT2046_Touchscreen.h>

22	#include <Fonts/FreeSans9pt7b.h>

23	#include <Fonts/FreeSansBold9pt7b.h>

24	#include <Fonts/FreeSansBold12pt7b.h>

25	#include <Fonts/FreeSansBold18pt7b.h>

26	#include <Fonts/FreeSansBold24pt7b.h>

27

28	// The display and the touch controller share one SPI bus (separate CS pins).

29	static Adafruit_ILI9341 tft(&SPI, PIN_TFT_DC, PIN_TFT_CS, -1);

30	static XPT2046_Touchscreen touch(PIN_TOUCH_CS);

31

32	// Touch IRQ (GPIO 36, active low). Wakes uiTask so a press is read within

33	// 30 ms instead of waiting for the next 100 ms frame.

34	void IRAM_ATTR isrTouch() {

35	  BaseType_t woke = pdFALSE;

36	  if (hUi) xTaskNotifyFromISR(hUi, EV_TOUCH, eSetBits, &woke);

37	  portYIELD_FROM_ISR(woke);

38	}

39

40	// Converts a web color 0xRRGGBB to the display's 16-bit RGB565.

41	static constexpr uint16_t C(uint32_t rgb) {

42	  return (uint16_t)(((rgb >> 8) & 0xF800) | ((rgb >> 5) & 0x07E0) | ((rgb >> 3) & 0x001F));

43	}

44

45	// Color palette: background, surface, border, text, the five level colors

46	// (green, amber, red, purple, teal), blue, cyan, the text colors used on top

47	// of each, light tints, and dim/rule grays. Colors match the dashboard and app.

48	struct Pal {

49	  uint16_t bg, surf, bd, tx, tx2, g, a, r, p, t, bl, cy, on_g, on_a, on_r, on_p, on_t, on_bl, on_cy, tg, ta, tr, tp, tt, dim, rule;

50	};

51	static const Pal LIGHT = {C(0xF5F0EB), C(0xFFFFFF), C(0x1A1A1A), C(0x1A1A1A), C(0x6B635B), C(0x1E8E3E), C(0xD98A1F),

52	                          C(0xD93025), C(0x7A45B5), C(0x0D8A8A), C(0x1A62C4), C(0x0E7F9E), C(0xFFFFFF), C(0x1A1A1A),

53	                          C(0xFFFFFF), C(0xFFFFFF), C(0xFFFFFF), C(0xFFFFFF), C(0xFFFFFF), C(0xCADCC8), C(0xEFDCC2),

54	                          C(0xEFCAC3), C(0xDCCEE0), C(0xC7DCD8), C(0xB5AFA9), C(0xC6C2BE)};

55	static const Pal DARK = {C(0x2A2520), C(0x3A3530), C(0xE8E0D8), C(0xEFE7DC), C(0xA89E94), C(0x4FBF6E), C(0xE8A94A),

56	                         C(0xFF6B5E), C(0xB98BE8), C(0x3FC7C2), C(0x79AEF5), C(0x5FD0E8), C(0x13100D), C(0x13100D),

57	                         C(0x13100D), C(0x13100D), C(0x13100D), C(0x13100D), C(0x13100D), C(0x324731), C(0x544229),

58	                         C(0x59342E), C(0x493B4C), C(0x2F4944), C(0x6E6862), C(0x5E5852)};

59	static const Pal* P = &LIGHT;

60	static uint8_t darkTheme = 0;

61

62	// Level colors: LG green (OK), LA amber (filling), LR red (full/alarm),

63	// LP purple (locked), LT teal (system).

64	enum Lv : uint8_t { LG, LA, LR, LP, LT };

65	static uint16_t fillOf(Lv k) { const uint16_t v[] = {P->g, P->a, P->r, P->p, P->t}; return v[k]; }

66	static uint16_t onOf(Lv k) { const uint16_t v[] = {P->on_g, P->on_a, P->on_r, P->on_p, P->on_t}; return v[k]; }

67	static uint16_t tintOf(Lv k) { const uint16_t v[] = {P->tg, P->ta, P->tr, P->tp, P->tt}; return v[k]; }

68

69	// Fonts from smallest to largest. CAP[] is each font's cap height in pixels,

70	// used to center text vertically.

71	enum Ft : uint8_t { F_TINY = 0, F_SR, F_S, F_M, F_L, F_XL };

72	static const GFXfont* const FONTS[] = {nullptr, &FreeSans9pt7b, &FreeSansBold9pt7b, &FreeSansBold12pt7b,

73	                                       &FreeSansBold18pt7b, &FreeSansBold24pt7b};

74	static const uint8_t CAP[] = {7, 13, 13, 17, 25, 34};

75

76	static Ft fontFor(int px, bool regular = false) {

77	  if (px <= 12) return F_TINY;

78	  if (px <= 17) return regular ? F_SR : F_S;

79	  if (px <= 24) return F_M;

80	  if (px <= 36) return F_L;

81	  return F_XL;

82	}

83

84	static void useFont(Ft f, uint8_t mag) {

85	  tft.setFont(FONTS[f]);

86	  tft.setTextSize(mag);

87	  tft.setTextWrap(false);

88	}

89

90	// Width of a string in pixels for a given font.

91	static int textW(const char* s, Ft f, uint8_t mag = 1) {

92	  useFont(f, mag);

93	  int16_t x1, y1;

94	  uint16_t w, h;

95	  tft.getTextBounds(s, 0, 50, &x1, &y1, &w, &h);

96	  return f == F_TINY ? (int)strlen(s) * 6 * mag : (int)w + x1;

97	}

98

99	// Draws text in a box w pixels wide; px is the wanted text height. If the

100	// text does not fit it steps down to a smaller font. align: 'l', 'c', 'r'.

101	static void txt(int x, int y, int w, const char* s, int px, uint16_t col, char align = 'l', bool regular = false, uint8_t mag = 1) {

102	  Ft f = fontFor(px, regular);

103	  int tw = textW(s, f, mag);

104	  while (tw > w && f > F_TINY) {

105	    f = (Ft)(f - 1);

106	    if (f == F_SR && !regular) f = F_TINY;

107	    tw = textW(s, f, mag);

108	  }

109	  int cx = align == 'c' ? x + (w - tw) / 2 : align == 'r' ? x + w - tw : x;

110	  useFont(f, mag);

111	  tft.setTextColor(col);

112	  if (f == F_TINY) tft.setCursor(cx, y + (px - 7 * mag) / 2);

113	  else tft.setCursor(cx, y + (px + CAP[f] * mag) / 2);

114	  tft.print(s);

115	}

116

117	// Like txt() but cuts long text and adds "..", for Wi-Fi names.

118	static void txtFit(int x, int y, int w, const char* s, int px, uint16_t col, char align = 'l', bool regular = false) {

119	  char b[40];

120	  strlcpy(b, s, sizeof(b));

121	  Ft f = fontFor(px, regular);

122	  size_t n = strlen(b);

123	  if (textW(b, f) > w) {

124	    while (n > 2) {

125	      b[--n] = 0;

126	      char t[44];

127	      snprintf(t, sizeof(t), "%s..", b);

128	      if (textW(t, f) <= w) { strlcpy(b, t, sizeof(b)); break; }

129	    }

130	  }

131	  txt(x, y, w, b, px, col, align, regular);

132	}

133

134	// Word-wraps centered text over several lines lh pixels apart.

135	static void wrapTxt(int x, int y, int w, const char* s, int px, uint16_t col, int lh) {

136	  char line[64] = "", word[40];

137	  const char* p = s;

138	  int ly = y;

139	  while (*p) {

140	    int n = 0;

141	    while (*p == ' ') p++;

142	    while (*p && *p != ' ' && n < 39) word[n++] = *p++;

143	    word[n] = 0;

144	    if (!n) break;

145	    char test[104];

146	    snprintf(test, sizeof(test), "%s%s%s", line, line[0] ? " " : "", word);

147	    if (textW(test, fontFor(px, true)) > w && line[0]) {

148	      txt(x, ly, w, line, px, col, 'c', true);

149	      ly += lh;

150	      strlcpy(line, word, sizeof(line));

151	    } else {

152	      strlcpy(line, test, sizeof(line));

153	    }

154	  }

155	  if (line[0]) txt(x, ly, w, line, px, col, 'c', true);

156	}

157

158	// --- Drawing helpers in the app's neo-brutalist style: thick dark border

159	// and a hard offset shadow. ---

160	static void frame(int x, int y, int w, int h, uint16_t fill, int border, uint16_t bd) {

161	  tft.fillRect(x, y, w, h, bd);

162	  tft.fillRect(x + border, y + border, w - 2 * border, h - 2 * border, fill);

163	}

164

165	static void box(int x, int y, int w, int h, uint16_t fill, int border = 3, int shadow = 0) {

166	  if (shadow) tft.fillRect(x + shadow, y + shadow, w, h, P->bd);

167	  frame(x, y, w, h, fill, border, P->bd);

168	}

169

170	static void thick(int x0, int y0, int x1, int y1, int t, uint16_t c) {

171	  for (int dx = -(t / 2); dx <= t / 2; dx++)

172	    for (int dy = -(t / 2); dy <= t / 2; dy++) tft.drawLine(x0 + dx, y0 + dy, x1 + dx, y1 + dy, c);

173	}

174

175	static void arc(int cx, int cy, int r, int a0, int a1, int t, uint16_t c) {

176	  for (int a = a0; a <= a1; a += 4) {

177	    float rad = a * DEG_TO_RAD;

178	    tft.fillCircle(cx + cosf(rad) * r, cy + sinf(rad) * r, t / 2, c);

179	  }

180	}

181

182	// Line icons drawn with thick lines on a 24 x 24 grid scaled to size s.

183	enum Icon : uint8_t { I_BIN, I_LIDOPEN, I_SPEAKER, I_LOCK, I_TRI, I_CHECK, I_X, I_BT, I_TRUCK, I_SUN, I_MOON, I_RING, I_WIFI };

184

185	static void icon(Icon k, int x, int y, int s, uint16_t c, uint16_t bgc, int phase = 0) {

186	  float u = s / 24.0f;

187	  int t = max(2, (int)(s / 9));

188	  auto X = [&](float v) { return x + (int)(v * u); };

189	  auto Y = [&](float v) { return y + (int)(v * u); };

190	  switch (k) {

191	    case I_BIN:

192	      thick(X(5), Y(8), X(19), Y(8), t, c);

193	      thick(X(3), Y(8), X(21), Y(8), t, c);

194	      thick(X(5), Y(8), X(6.4), Y(20), t, c);

195	      thick(X(19), Y(8), X(17.6), Y(20), t, c);

196	      thick(X(6.4), Y(20), X(17.6), Y(20), t, c);

197	      thick(X(9), Y(4.5), X(15), Y(4.5), t, c);

198	      break;

199	    case I_LIDOPEN:

200	      thick(X(6), Y(11), X(18), Y(11), t, c);

201	      thick(X(6), Y(11), X(7.2), Y(20), t, c);

202	      thick(X(18), Y(11), X(16.8), Y(20), t, c);

203	      thick(X(7.2), Y(20), X(16.8), Y(20), t, c);

204	      thick(X(3.5), Y(8.5), X(16.5), Y(4.3), t, c);

205	      break;

206	    case I_SPEAKER:

207	      tft.fillTriangle(X(3), Y(12), X(12), Y(5.5), X(12), Y(18.5), c);

208	      tft.fillRect(X(3), Y(9.5), (int)(5 * u), (int)(5 * u) + 1, c);

209	      if (phase != 1) arc(X(13), Y(12), (int)(4 * u), -45, 45, t, c);

210	      if (phase != 2) arc(X(13), Y(12), (int)(8 * u), -50, 50, t, c);

211	      break;

212	    case I_LOCK:

213	      frame(X(4.5), Y(10.5), (int)(15 * u), (int)(10.5 * u), bgc, t, c);

214	      arc(X(12), Y(8.5), (int)(4 * u), 180, 360, t, c);

215	      thick(X(8), Y(8.5), X(8), Y(10.5), t, c);

216	      thick(X(16), Y(8.5), X(16), Y(10.5), t, c);

217	      thick(X(12), Y(14.5), X(12), Y(17.5), t, c);

218	      break;

219	    case I_TRI:

220	      thick(X(12), Y(3.2), X(22), Y(20.5), t, c);

221	      thick(X(22), Y(20.5), X(2), Y(20.5), t, c);

222	      thick(X(2), Y(20.5), X(12), Y(3.2), t, c);

223	      thick(X(12), Y(9.5), X(12), Y(14.5), t + 1, c);

224	      tft.fillCircle(X(12), Y(17.3), t, c);

225	      break;

226	    case I_CHECK:

227	      thick(X(4), Y(12.5), X(9), Y(17.5), t + 1, c);

228	      thick(X(9), Y(17.5), X(20), Y(6.5), t + 1, c);

229	      break;

230	    case I_X:

231	      thick(X(6), Y(6), X(18), Y(18), t + 1, c);

232	      thick(X(18), Y(6), X(6), Y(18), t + 1, c);

233	      break;

234	    case I_BT:

235	      thick(X(7), Y(7.5), X(17), Y(16.5), t, c);

236	      thick(X(17), Y(16.5), X(12), Y(21), t, c);

237	      thick(X(12), Y(21), X(12), Y(3), t, c);

238	      thick(X(12), Y(3), X(17), Y(7.5), t, c);

239	      thick(X(17), Y(7.5), X(7), Y(16.5), t, c);

240	      break;

241	    case I_TRUCK:

242	      frame(X(2), Y(7), (int)(11 * u), (int)(9 * u), bgc, max(2, t - 1), c);

243	      thick(X(13), Y(10), X(18), Y(10), t - 1, c);

244	      thick(X(18), Y(10), X(21.5), Y(13), t - 1, c);

245	      thick(X(21.5), Y(13), X(21.5), Y(16), t - 1, c);

246	      thick(X(13), Y(16), X(21.5), Y(16), t - 1, c);

247	      tft.fillCircle(X(6.5), Y(18.5), (int)(2.2 * u), c);

248	      tft.fillCircle(X(17), Y(18.5), (int)(2.2 * u), c);

249	      break;

250	    case I_SUN:

251	      tft.fillCircle(X(12), Y(12), (int)(4.5 * u), c);

252	      for (int a = 0; a < 360; a += 45) {

253	        float r = a * DEG_TO_RAD;

254	        thick(X(12 + cosf(r) * 7), Y(12 + sinf(r) * 7), X(12 + cosf(r) * 9.5f), Y(12 + sinf(r) * 9.5f), max(1, t - 1), c);

255	      }

256	      break;

257	    case I_MOON:

258	      tft.fillCircle(X(11), Y(12.5), (int)(8.5 * u), c);

259	      tft.fillCircle(X(15.5), Y(8.5), (int)(7 * u), bgc);

260	      break;

261	    case I_RING:

262	      for (int a = 0; a < 360; a += 40) arc(X(12), Y(12), (int)(9 * u), a + phase, a + phase + 22, t + 1, c);

263	      break;

264	    case I_WIFI:

265	      arc(X(12), Y(20), (int)(5 * u), 220, 320, t, c);

266	      arc(X(12), Y(20), (int)(10 * u), 220, 320, t, c);

267	      tft.fillCircle(X(12), Y(20), t, c);

268	      break;

269	  }

270	}

271

272	// Button IDs used by the touch handler (onPress). B_K0..B_K9 are PIN keys.

273	enum BtnId : uint8_t {

274	  B_NONE = 0, B_COLLECT, B_INFO, B_THEME, B_DONE, B_DONE_BACK, B_CAL_CANCEL, B_BACK, B_WIFI_SETUP, B_BT_CANCEL,

275	  B_BT_DONE, B_BT_RETRY, B_PIN_CANCEL, B_K0, B_K1, B_K2, B_K3, B_K4, B_K5, B_K6, B_K7, B_K8, B_K9, B_KDEL, B_KOK

276	};

277	struct Hit { int16_t x, y, w, h; uint8_t id; };

278	static Hit hits[24];

279	static uint8_t hitN = 0;

280	static void addHit(int x, int y, int w, int h, uint8_t id) {

281	  for (uint8_t i = 0; i < hitN; i++)

282	    if (hits[i].id == id && hits[i].x == x && hits[i].y == y) return;

283	  if (hitN < 24) hits[hitN++] = {(int16_t)x, (int16_t)y, (int16_t)w, (int16_t)h, id};

284	}

285

286	// A button: shadowed box, optional icon, label. If the label and icon do not

287	// fit, the icon shrinks and is dropped last.

288	static void btn(int x, int y, int w, int h, const char* label, uint16_t fill, uint16_t on, int px, int ic, int isz, uint8_t id,

289	                bool pressed = false) {

290	  int o = pressed ? 4 : 0;

291	  if (pressed) tft.fillRect(x, y, w + 4, h + 4, P->bg == fill ? P->surf : P->bg);

292	  box(x + o, y + o, w, h, fill, 3, pressed ? 0 : 4);

293	  int lw = textW(label, fontFor(px));

294	  int gap = 5;

295	  while (ic >= 0 && lw + isz + gap > w - 12 && isz > 14) {

296	    isz -= 2;

297	    gap = 3;

298	  }

299	  if (ic >= 0 && lw + isz + gap > w - 12) ic = -1;

300	  int tw = lw + (ic >= 0 ? isz + gap : 0);

301	  int sx = x + o + (w - tw) / 2;

302	  if (ic >= 0) {

303	    icon((Icon)ic, sx, y + o + (h - isz) / 2, isz, on, fill);

304	    sx += isz + gap;

305	  }

306	  txt(sx, y + o + (h - px) / 2, x + o + w - 6 - sx, label, px, on);

307	  if (id) addHit(x, y, w + 4, h + 4, id);

308	}

309

310	static void chip(int x, int y, int w, int h, const char* label, uint16_t fill, uint16_t on, int px = 12, bool sh = false, int border = 2) {

311	  if (sh) tft.fillRect(x + 3, y + 3, w, h, P->bd);

312	  frame(x, y, w, h, fill, border, P->bd);

313	  txt(x + 2, y + (h - px) / 2, w - 4, label, px, on, 'c');

314	}

315

316	// FNV-1a hash of a widget signature.

317	static uint32_t hashOf(const char* s) {

318	  uint32_t h = 2166136261u;

319	  while (*s) { h ^= (uint8_t)*s++; h *= 16777619u; }

320	  return h;

321	}

322

323	// wcache holds the last signature hash of each widget slot (id 0..39).

324	// fullDraw forces every widget to redraw, used after a screen is cleared.

325	static bool fullDraw = true;

326	static uint32_t wcache[40];

327	static bool dirty(uint8_t id, const char* sig) {

328	  uint32_t h = hashOf(sig);

329	  if (fullDraw || wcache[id] != h) { wcache[id] = h; return true; }

330	  return false;

331	}

332

333	// Screens and overlays. Overlays (Info, PIN, Bluetooth) sit above the normal

334	// screens and close by themselves after SCREEN_TIMEOUT_MS without a touch.

335	enum Screen : uint8_t { S_BOOT, S_CAL, S_MAIN, S_URGENT, S_PIN, S_COLLECT, S_DONE, S_TAMPER, S_LOCKED, S_INFO, S_BT, S_TOUCHCAL };

336	enum Overlay : uint8_t { O_NONE, O_INFO, O_PIN, O_BT };

337	static Screen cur = S_BOOT;

338	static Overlay overlay = O_NONE;

339	static uint32_t overlayMs = 0;

340	static bool pinForBt = false;

341	static char pinBuf[5] = "";

342	static uint32_t pinWrongUntil = 0;

343	static uint32_t doneShownAt = 0;

344	static uint32_t lastTouchMs = 0;

345	static uint8_t blink = 0, spin = 0;

346	static uint32_t lastAnim = 0;

347

348	static const char* lidText(bool open) { return open ? "LID: OPEN" : "LID: CLOSED"; }

349

350	// Top bar: bin name, Info and theme buttons, optional BACK button and the lid chip.

351	static void topBar(const BinState& s, bool icons, int lidMode, const char* extraBack = nullptr) {

352	  char sig[48];

353	  snprintf(sig, sizeof(sig), "%d%d%d", icons, lidMode, darkTheme);

354	  if (dirty(0, sig)) {

355	    tft.fillRect(0, 0, 240, 34, P->surf);

356	    tft.fillRect(0, 34, 240, 3, P->bd);

357	    int nw = icons ? 60 : lidMode >= 0 ? 130 : 150;

358	    txt(8, 9, nw, BIN_LABEL, 16, P->tx);

359	    if (icons) {

360	      tft.fillRect(76, 6, 26, 26, P->bd);

361	      frame(74, 4, 26, 26, P->bg, 2, P->bd);

362	      txt(74, 10, 26, "i", 14, P->tx, 'c');

363	      addHit(0, 0, 72, 34, B_INFO);

364	      addHit(72, 0, 30, 34, B_INFO);

365	      tft.fillRect(106, 6, 26, 26, P->bd);

366	      frame(104, 4, 26, 26, P->bg, 2, P->bd);

367	      icon(darkTheme ? I_MOON : I_SUN, 107, 7, 20, P->tx, P->bg);

368	      addHit(102, 0, 34, 34, B_THEME);

369	    }

370	    if (extraBack) {

371	      frame(168, 4, 64, 26, P->bg, 2, P->bd);

372	      txt(168, 11, 64, extraBack, 12, P->tx, 'c');

373	      addHit(160, 0, 80, 34, B_BACK);

374	    }

375	  }

376	  if (lidMode >= 0) {

377	    snprintf(sig, sizeof(sig), "lid%d%d", lidMode, darkTheme);

378	    if (dirty(1, sig)) {

379	      bool open = lidMode == 1;

380	      chip(142, 6, 90, 22, lidText(open), open ? P->a : P->bg, open ? P->on_a : P->tx);

381	    }

382	  }

383	}

384

385	// Status chips under the top bar: WI-FI/OFFLINE, MOTION, SIM, CAL.

386	static void drawChips(const BinState& s, const Sim& sm, uint16_t tint) {

387	  char sig[24];

388	  snprintf(sig, sizeof(sig), "%d%d%d%d%d", s.wifi, s.v.motion, sm.mask != 0, s.calibrating, darkTheme);

389	  if (!dirty(2, sig)) return;

390	  tft.fillRect(0, 40, 240, 24, tint);

391	  int wx;

392	  if (s.wifi) { chip(8, 42, 52, 20, "WI-FI", P->g, P->on_g); wx = 66; }

393	  else { chip(8, 42, 70, 20, "OFFLINE", P->r, P->on_r); wx = 84; }

394	  addHit(4, 38, wx - 4, 28, B_INFO);

395	  if (s.v.motion) chip(wx, 42, 62, 20, "MOTION", P->cy, P->on_cy);

396	  else chip(wx, 42, 62, 20, "MOTION", P->surf, P->dim);

397	  int sx = wx + 68;

398	  if (sm.mask) chip(sx, 42, 40, 20, "SIM", P->p, P->on_p);

399	  else chip(sx, 42, 40, 20, "SIM", P->surf, P->dim);

400	  if (s.calibrating) chip(sx + 46, 42, 40, 20, "CAL", P->t, P->on_t);

401	  else chip(sx + 46, 42, 40, 20, "CAL", P->surf, P->dim);

402	}

403

404	// Vertical fill bar with marks at 50% and at the fill threshold.

405	static void fillBar(int x, int y, int w, int h, float pct, uint16_t col, float th) {

406	  box(x, y, w, h, P->surf, 3, 0);

407	  int inner = h - 6;

408	  int fh = (int)roundf(inner * constrain(pct, 0.0f, 100.0f) / 100.0f);

409	  if (fh > 0) {

410	    tft.fillRect(x + 3, y + 3 + inner - fh, w - 6, fh, col);

411	    tft.fillRect(x + 3, y + 3 + inner - fh, w - 6, min(2, fh), P->bd);

412	  }

413	  for (float t : {50.0f, th}) {

414	    int ty = y + 3 + (int)roundf(inner * (1 - t / 100.0f));

415	    tft.fillRect(x, ty - 1, w, 2, P->bd);

416	  }

417	}

418

419	// One sensor row: label and value. Turns red when over its limit and shows

420	// "--" with a "!" when the sensor is faulty.

421	static void reading(uint8_t id, int y, const char* label, const char* value, bool alarm, bool fault, int dot, uint16_t rowBg) {

422	  char sig[64];

423	  snprintf(sig, sizeof(sig), "%s|%d%d%d%d", value, alarm, fault, dot, darkTheme);

424	  if (!dirty(id, sig)) return;

425	  tft.fillRect(52, y - 2, 188, 22, rowBg);

426	  if (fault) {

427	    txt(56, y + 2, 90, label, 13, P->tx2, 'l', true);

428	    frame(178, y + 1, 12, 15, rowBg, 2, P->tx2);

429	    txt(178, y + 2, 12, "!", 12, P->tx2, 'c');

430	    txt(196, y + 1, 36, "--", 15, P->tx2, 'r');

431	    return;

432	  }

433	  if (alarm) {

434	    tft.fillRect(57, y + 2, 180, 18, P->bd);

435	    frame(54, y - 1, 180, 18, P->r, 2, P->bd);

436	    txt(60, y + 2, 90, label, 13, P->on_r, 'l', true);

437	    txt(120, y + 1, 106, value, 15, P->on_r, 'r');

438	    return;

439	  }

440	  txt(56, y + 2, 90, label, 13, P->tx2, 'l', true);

441	  int vw = textW(value, fontFor(15));

442	  if (dot >= 0) tft.fillRect(232 - vw - 15, y + 5, 9, 9, fillOf((Lv)dot));

443	  txt(110, y + 1, 122, value, 15, P->tx, 'r');

444	}

445

446	// A row with a shrinking progress line (lid closing, buzzer time left).

447	static void timerRow(uint8_t id, int y, int ic, const char* label, float frac, uint16_t col, uint16_t rowBg) {

448	  char sig[48];

449	  snprintf(sig, sizeof(sig), "%s|%d|%d", label, (int)(frac * 50), darkTheme);

450	  if (!dirty(id, sig)) return;

451	  frame(8, y, 224, 20, P->surf, 2, P->bd);

452	  icon((Icon)ic, 11, y + 2, 14, P->tx, P->surf);

453	  txt(29, y + 3, 200, label, 12, P->tx);

454	  tft.fillRect(10, y + 15, 220, 3, P->bg);

455	  tft.fillRect(10, y + 15, (int)(220 * constrain(frac, 0.0f, 1.0f)), 3, col);

456	}

457

458	static void clearRow(uint8_t id, int y, int h, uint16_t bgc) {

459	  if (dirty(id, "none")) tft.fillRect(0, y, 240, h, bgc);

460	}

461

462	// Bottom row: bin status box and the COLLECT button.

463	static void bottomRow(const char* status, uint16_t fill, uint16_t on, int px) {

464	  char sig[40];

465	  snprintf(sig, sizeof(sig), "%s%d", status, darkTheme);

466	  if (!dirty(3, sig)) return;

467	  tft.fillRect(8 + 4, 274 + 4, 112, 36, P->bd);

468	  frame(8, 274, 112, 36, fill, 3, P->bd);

469	  txt(10, 274 + 11, 108, status, px, on, 'c');

470	  btn(128, 270, 104, 44, "COLLECT", P->surf, P->tx, 15, I_TRUCK, 20, B_COLLECT);

471	}

472

473	// "Full in ~x h" text from the fill rate. Hidden above 72 h or when unknown.

474	static void fmtTtf(const BinState& s, const Cal& c, char* out, size_t n) {

475	  out[0] = 0;

476	  if (s.fillRatePerHour <= 0 || s.v.fill >= c.fill_th) return;

477	  float h = (c.fill_th - s.v.fill) / s.fillRatePerHour;

478	  if (h > 72) return;

479	  if (h < 1) snprintf(out, n, "Full in ~%d min", max(1, (int)roundf(h * 60)));

480	  else snprintf(out, n, "Full in ~%.1f h", h);

481	}

482

483	static void lidRows(const BinState& s, const Cal& c, uint32_t now, int y, uint16_t rowBg, uint8_t id) {

484	  char label[40];

485	  if (s.lidOpen && s.v.motion && s.status == ST_OK) {

486	    timerRow(id, y, I_LIDOPEN, "LID HOLDING (motion)", 1, P->cy, rowBg);

487	  } else if (s.lidOpen && s.lidCloseAtMs) {

488	    float left = max<int32_t>(0, (int32_t)(s.lidCloseAtMs - now)) / 1000.0f;

489	    snprintf(label, sizeof(label), "LID CLOSES IN %.1f s", left);

490	    timerRow(id, y, I_LIDOPEN, label, left / max(1, (int)c.pir_hold_s), P->cy, rowBg);

491	  }

492	}

493

494	static bool lidRowActive(const BinState& s) { return s.lidOpen && (s.v.motion || s.lidCloseAtMs) && s.status == ST_OK; }

495	static bool buzzRowActive(const BinState& s, uint32_t now) { return s.buzzerOn || (int32_t)(s.buzzerUntilMs - now) > 0; }

496

497	static void buzzRow(const BinState& s, const Cal& c, uint32_t now, int y, uint16_t rowBg, uint8_t id) {

498	  char label[32];

499	  int32_t left = (int32_t)(s.buzzerUntilMs - now);

500	  if (left > 0) {

501	    snprintf(label, sizeof(label), "BUZZER %.1f s", left / 1000.0f);

502	    timerRow(id, y, I_SPEAKER, label, left / (max(1, (int)c.buzz_s) * 1000.0f), P->r, rowBg);

503	  } else {

504	    timerRow(id, y, I_SPEAKER, "BUZZER ON", 1, P->r, rowBg);

505	  }

506	}

507

508	// --- Main screen: fill level, weight, temperature, humidity, air ---

509	// The background tint follows the level: green under 50%, amber from 50%,

510	// red at the fill threshold.

511	static void drawMain(const BinState& s, const Cal& c, const Sim& sm, uint32_t now) {

512	  bool hcFault = !(s.okHC || (sm.mask & 1));

513	  int fill = (int)roundf(s.v.fill);

514	  Lv lvl = fill >= c.fill_th ? LR : fill >= 50 ? LA : LG;

515	  uint16_t tint = tintOf(lvl);

516	  char sig[64];

517	  snprintf(sig, sizeof(sig), "main%d%d", lvl, darkTheme);

518	  if (dirty(4, sig)) {

519	    fullDraw = true;

520	    tft.fillScreen(tint);

521	  }

522	  topBar(s, true, s.lidOpen ? 1 : 0);

523	  drawChips(s, sm, tint);

524

525	  snprintf(sig, sizeof(sig), "%d|%d|%d|%.0f", fill, hcFault, darkTheme, c.fill_th);

526	  if (dirty(5, sig)) {

527	    tft.fillRect(8, 66, 224, 78, tint);

528	    fillBar(10, 68, 36, 150, hcFault ? 0 : fill, fillOf(lvl), c.fill_th);

529	    txt(56, 68, 100, "FILL LEVEL", 12, P->tx2);

530	    if (hcFault) {

531	      chip(150, 64, 82, 20, "NO READING", P->surf, P->tx2, 11);

532	      txt(56, 82, 176, "--", 56, P->tx);

533	    } else {

534	      chip(172, 64, 60, 20, fill >= c.fill_th ? "FULL" : fill >= 50 ? "FILLING" : "OK", fillOf(lvl), onOf(lvl));

535	      char n[8];

536	      snprintf(n, sizeof(n), "%d", fill);

537	      txt(56, 82, 176, n, 56, P->tx);

538	      int nw = textW(n, F_XL);

539	      txt(56 + nw + 2, 98, 40, "%", 26, P->tx);

540	    }

541	  }

542

543	  char val[24];

544	  bool hx = s.okHX || (sm.mask & 2), bme = s.okBME || (sm.mask & 8), bmeGas = s.okBME || (sm.mask & 4), gasOk = bmeGas && s.v.iaq_acc > 0;

545	  snprintf(val, sizeof(val), "%.2f kg", s.v.weight_g / 1000.0f);

546	  reading(6, 146, "Weight", val, hx && s.v.weight_g >= c.weight_cap, !hx, -1, tint);

547	  snprintf(val, sizeof(val), "%.1f C", s.v.temp);

548	  reading(7, 164, "Temperature", val, bme && s.v.temp >= c.temp_th, !bme, -1, tint);

549	  snprintf(val, sizeof(val), "%d%%", (int)roundf(s.v.humidity));

550	  reading(8, 182, "Humidity", val, bme && s.v.humidity >= c.hum_th, !bme, -1, tint);

551	  float gp = s.v.iaq / max(1.0f, c.gas_th) * 100;

552	  Lv al = gp >= 80 ? LR : gp >= 50 ? LA : LG;

553	  snprintf(val, sizeof(val), "%s %d", gp >= 80 ? "Poor" : gp >= 50 ? "Moderate" : "Good", (int)roundf(s.v.iaq));

554	  // During BME680 warm-up the Air row shows the countdown and the raw gas

555	  // resistance in kOhm instead of an index.

556	  if (bmeGas && s.v.iaq_acc == 0) {

557	    int left = airWarmLeft();

558	    if (left > 0) snprintf(val, sizeof(val), "%d:%02d  %.0fk", left / 60, left % 60, s.v.gas_k);

559	    else if (s.v.gas_k > 0) snprintf(val, sizeof(val), "Starting %.0fk", s.v.gas_k);

560	    else strcpy(val, "No gas reading");

561	    reading(9, 200, "Air (warm)", val, false, false, -1, tint);

562	  }

563	  else reading(9, 200, "Air", val, gasOk && gp >= 100, !gasOk, al, tint);

564

565	  // Up to two timer rows (lid, buzzer); otherwise the "Full in" line.

566	  bool lr = lidRowActive(s), br = buzzRowActive(s, now);

567	  if (lr && br) {

568	    lidRows(s, c, now, 222, tint, 10);

569	    buzzRow(s, c, now, 244, tint, 11);

570	  } else if (lr || br) {

571	    if (lr) lidRows(s, c, now, 224, tint, 10);

572	    else buzzRow(s, c, now, 224, tint, 10);

573	    clearRow(11, 244, 22, tint);

574	  } else {

575	    char t[32];

576	    fmtTtf(s, c, t, sizeof(t));

577	    snprintf(sig, sizeof(sig), "ttf%s%d", hcFault ? "" : t, darkTheme);

578	    if (dirty(10, sig)) {

579	      tft.fillRect(0, 220, 240, 26, tint);

580	      if (!hcFault && t[0]) txt(8, 230, 224, t, 13, P->tx);

581	    }

582	    clearRow(11, 246, 22, tint);

583	  }

584	  if (hcFault) bottomRow("BIN: ?", P->surf, P->tx2, 14);

585	  else bottomRow(fill >= c.fill_th ? "BIN: FULL" : "BIN: OK", fillOf(lvl), onOf(lvl), 14);

586	}

587

588	// --- Urgent screen: shown when fill passes the threshold ---

589	// Blinking border and speaker icon every 500 ms.

590	static void drawUrgent(const BinState& s, const Cal& c, const Sim& sm, uint32_t now) {

591	  char sig[48];

592	  snprintf(sig, sizeof(sig), "urg%d", darkTheme);

593	  if (dirty(4, sig)) {

594	    fullDraw = true;

595	    tft.fillScreen(P->r);

596	  }

597	  topBar(s, true, s.lidOpen ? 1 : 0);

598	  if (fullDraw) {

599	    txt(12, 46, 170, "URGENT", 40, P->on_r);

600	    txt(12, 98, 200, "FILL LEVEL", 12, P->on_r);

601	    txt(12, 196, 216, "Bin is almost full.", 16, P->on_r, 'l', true);

602	    txt(12, 216, 216, "Collect it now!", 20, P->on_r);

603	  }

604	  snprintf(sig, sizeof(sig), "%d%d", (int)roundf(s.v.fill), sm.mask != 0);

605	  if (dirty(5, sig)) {

606	    tft.fillRect(10, 112, 176, 82, P->r);

607	    char n[8];

608	    snprintf(n, sizeof(n), "%d", (int)roundf(s.v.fill));

609	    txt(12, 112, 160, n, 80, P->on_r, 'l', false, 2);

610	    int nw = textW(n, F_XL, 2);

611	    txt(14 + nw, 146, 50, "%", 36, P->on_r);

612	    tft.fillRect(186, 94, 50, 26, P->r);

613	    if (sm.mask) chip(188, 96, 44, 20, "SIM", P->p, P->on_p);

614	  }

615	  snprintf(sig, sizeof(sig), "b%d", blink);

616	  if (dirty(12, sig)) {

617	    uint16_t bc = blink ? P->bd : P->r;

618	    for (int i = 0; i < 5; i++) tft.drawRect(i, 37 + i, 240 - 2 * i, 283 - 2 * i, bc);

619	    tft.fillRect(190, 46, 38, 38, P->r);

620	    if (blink) icon(I_SPEAKER, 190, 46, 38, P->on_r, P->r);

621	  }

622	  bool lr = lidRowActive(s), br = buzzRowActive(s, now);

623	  if (br) buzzRow(s, c, now, 244, P->r, 10);

624	  else if (lr) lidRows(s, c, now, 244, P->r, 10);

625	  else clearRow(10, 242, 24, P->r);

626	  bottomRow("BIN: FULL", P->surf, P->tx, 14);

627	}

628

629	// --- Collect PIN keypad. Also used to unlock WI-FI SETUP. ---

630	// A wrong PIN shows the red banner for 1.6 s.

631	static void drawPin(uint32_t now) {

632	  bool wrong = (int32_t)(pinWrongUntil - now) > 0;

633	  char sig[24];

634	  snprintf(sig, sizeof(sig), "pin%d%d", wrong, darkTheme);

635	  if (dirty(4, sig)) {

636	    fullDraw = true;

637	    tft.fillScreen(P->bg);

638	    tft.fillRect(0, 0, 240, 34, P->surf);

639	    tft.fillRect(0, 34, 240, 3, P->bd);

640	    frame(6, 4, 84, 26, P->r, 2, P->bd);

641	    icon(I_X, 13, 10, 13, P->on_r, P->r);

642	    txt(30, 11, 56, "CANCEL", 12, P->on_r);

643	    addHit(0, 0, 96, 36, B_PIN_CANCEL);

644	    txt(98, 9, 134, "COLLECT PIN", 16, P->tx, 'r');

645	    const char* keys[] = {"1", "2", "3", "4", "5", "6", "7", "8", "9", "<", "0", "OK"};

646	    const uint8_t ids[] = {B_K1, B_K2, B_K3, B_K4, B_K5, B_K6, B_K7, B_K8, B_K9, B_KDEL, B_K0, B_KOK};

647	    for (int i = 0; i < 12; i++) {

648	      int r = i / 3, cc = i % 3;

649	      bool ok = i == 11;

650	      btn(8 + cc * 76, 98 + r * 46, 68, 42, keys[i], ok ? P->g : P->surf, ok ? P->on_g : P->tx, 22, -1, 0, ids[i]);

651	    }

652	    if (wrong) {

653	      frame(8, 288, 224, 26, P->r, 3, P->bd);

654	      txt(8, 294, 224, "WRONG PIN. Try again.", 14, P->on_r, 'c');

655	      for (int i = 0; i < 5; i++) tft.drawRect(i, 37 + i, 240 - 2 * i, 283 - 2 * i, P->r);

656	    } else {

657	      txt(8, 296, 224, "4 digits, then OK", 13, P->tx2, 'c', true);

658	    }

659	  }

660	  int n = wrong ? 4 : strlen(pinBuf);

661	  snprintf(sig, sizeof(sig), "%d", n);

662	  if (dirty(5, sig)) {

663	    for (int i = 0; i < 4; i++) {

664	      box(20 + i * 52, 46, 44, 40, P->surf, 3, 3);

665	      if (i < n) tft.fillCircle(20 + i * 52 + 22, 66, 6, P->tx);

666	    }

667	  }

668	}

669

670	// --- Collecting screen: lid held open until DONE ---

671	static void drawCollect(uint32_t now) {

672	  char sig[24];

673	  snprintf(sig, sizeof(sig), "col%d", darkTheme);

674	  if (dirty(4, sig)) {

675	    fullDraw = true;

676	    tft.fillScreen(P->g);

677	    BinState s = snapshot();

678	    topBar(s, false, 1);

679	    icon(I_LIDOPEN, 88, 84, 64, P->on_g, P->g);

680	    const char* rows[] = {"Lid: open", "Motion: ignored", "Tamper: paused", "Alerts: muted"};

681	    for (int i = 0; i < 4; i++) {

682	      tft.fillRect(28, 172 + i * 22, 9, 9, P->on_g);

683	      txt(44, 164 + i * 22, 180, rows[i], 16, P->on_g, 'l', true);

684	    }

685	    btn(8, 262, 224, 48, "DONE", P->surf, P->tx, 22, I_CHECK, 24, B_DONE);

686	  }

687	  char dots[24];

688	  int d = (now / 300) % 4;

689	  snprintf(dots, sizeof(dots), "COLLECTING%.*s", d, "...");

690	  if (dirty(5, dots)) {

691	    tft.fillRect(0, 44, 240, 34, P->g);

692	    txt(8, 48, 224, dots, 26, P->on_g, 'c');

693	  }

694	}

695

696	// --- Collection complete: goes back to the main screen after 30 s ---

697	static void drawDone(uint32_t now) {

698	  char sig[24];

699	  snprintf(sig, sizeof(sig), "done%d", darkTheme);

700	  if (dirty(4, sig)) {

701	    fullDraw = true;

702	    tft.fillScreen(P->tg);

703	    BinState s = snapshot();

704	    topBar(s, false, 0);

705	    txt(8, 50, 224, "COLLECTION", 22, P->tx, 'c');

706	    txt(8, 76, 224, "COMPLETE", 22, P->tx, 'c');

707	    box(84, 108, 72, 72, P->g, 3, 4);

708	    icon(I_CHECK, 94, 118, 52, P->on_g, P->g);

709	    btn(8, 198, 224, 56, "DONE", P->g, P->on_g, 26, -1, 0, B_DONE_BACK);

710	  }

711	  int left = 30 - (int)((now - doneShownAt) / 1000);

712	  if (left < 0) left = 0;

713	  snprintf(sig, sizeof(sig), "%d", left);

714	  if (dirty(5, sig)) {

715	    char t[48];

716	    snprintf(t, sizeof(t), "Back to the main screen in %d s", left);

717	    tft.fillRect(0, 264, 240, 20, P->tg);

718	    txt(8, 266, 224, t, 13, P->tx, 'c', true);

719	    frame(8, 286, 224, 16, P->surf, 2, P->bd);

720	    tft.fillRect(10, 288, 220 * left / 30, 12, P->g);

721	  }

722	}

723

724	// --- Tamper screen: countdown, then latched until reset from the dashboard ---

725	static void drawTamper(const BinState& s, const Cal& c, uint32_t now) {

726	  bool latched = s.tamperPhase == 2;

727	  char sig[24];

728	  snprintf(sig, sizeof(sig), "tmp%d%d", latched, darkTheme);

729	  if (dirty(4, sig)) {

730	    fullDraw = true;

731	    tft.fillScreen(P->r);

732	    txt(8, 82, 224, latched ? "TAMPER LATCHED" : "TAMPER DETECTED", 22, P->on_r, 'c');

733	    txt(8, 112, 224, "Bin moved or tilted", 15, P->on_r, 'c', true);

734	    if (latched) {

735	      box(16, 160, 208, 84, P->surf, 3, 4);

736	      txt(16, 176, 208, "RESET ON", 22, P->tx, 'c');

737	      txt(16, 204, 208, "DASHBOARD", 22, P->tx, 'c');

738	      txt(8, 252, 224, "Stays on until it is cleared", 13, P->on_r, 'c', true);

739	    } else {

740	      txt(8, 236, 224, "SECONDS, CLEARS IF STABLE", 12, P->on_r, 'c');

741	    }

742	    tft.fillRect(53, 279, 140, 32, P->bd);

743	    frame(50, 276, 140, 32, P->surf, 3, P->bd);

744	    icon(I_SPEAKER, 70, 281, 22, P->tx, P->surf);

745	    txt(98, 284, 90, latched ? "ALARM ON" : "ALARM", 15, P->tx);

746	  }

747	  char t[24];

748	  snprintf(t, sizeof(t), "Tilt: %.0f deg", s.v.tilt);

749	  if (dirty(5, t)) {

750	    tft.fillRect(8, 130, 224, 20, P->r);

751	    txt(8, 132, 224, t, 15, P->on_r, 'c', true);

752	  }

753	  if (!latched) {

754	    int left = (int)TAMPER_COUNTDOWN_S - (int)((now - s.tamperStartMs) / 1000);

755	    if (left < 0) left = 0;

756	    snprintf(t, sizeof(t), "%d", left);

757	    if (dirty(6, t)) {

758	      tft.fillRect(8, 150, 224, 84, P->r);

759	      txt(8, 152, 224, t, 84, P->on_r, 'c', false, 2);

760	    }

761	    float frac = 1.0f - (now - s.tamperStartMs) / (TAMPER_COUNTDOWN_S * 1000.0f);

762	    snprintf(t, sizeof(t), "%d", (int)(frac * 52));

763	    if (dirty(7, t)) {

764	      frame(16, 254, 208, 14, P->surf, 2, P->bd);

765	      tft.fillRect(18, 256, (int)(204 * constrain(frac, 0.0f, 1.0f)), 10, P->bd);

766	    }

767	  }

768	  snprintf(t, sizeof(t), "%d", blink);

769	  if (dirty(12, t)) {

770	    tft.fillRect(88, 16, 64, 58, P->r);

771	    icon(I_TRI, 88 + (blink ? 2 : -2), 14, 60, P->on_r, P->r);

772	    uint16_t bc = blink ? P->bd : P->r;

773	    for (int i = 0; i < 8; i++) tft.drawRect(i, i, 240 - 2 * i, 320 - 2 * i, bc);

774	  }

775	}

776

777	// --- Locked screen: lid stays closed, unlock from the app or dashboard ---

778	static void drawLocked(const BinState& s) {

779	  char sig[24];

780	  snprintf(sig, sizeof(sig), "lock%d", darkTheme);

781	  if (dirty(4, sig)) {

782	    fullDraw = true;

783	    tft.fillScreen(P->p);

784	    icon(I_LOCK, 80, 46, 80, P->on_p, P->p);

785	    txt(8, 134, 224, "LOCKED", 40, P->on_p, 'c');

786	    txt(8, 184, 224, "Lid stays closed.", 15, P->on_p, 'c', true);

787	    txt(8, 204, 224, "Motion is ignored.", 15, P->on_p, 'c', true);

788	    frame(8, 228, 224, 34, P->surf, 3, P->bd);

789	    txt(8, 238, 224, "Unlock on the app or dashboard.", 13, P->tx, 'c');

790	  }

791	  topBar(s, true, 0);

792	  bottomRow("BIN: LOCKED", P->surf, P->tx, 13);

793	}

794

795	// --- Boot screen: progress bar and the result of each sensor check ---

796	// A FAIL row here matches a "faults" entry in /api/status.

797	static void drawBoot(const BinState& s, uint32_t now) {

798	  if (dirty(4, "boot")) {

799	    fullDraw = true;

800	    tft.fillScreen(P->t);

801	    box(16, 18, 208, 66, P->surf, 3, 4);

802	    icon(I_BIN, 28, 30, 38, P->tx, P->surf);

803	    txt(76, 30, 140, "SMART", 20, P->tx);

804	    txt(76, 54, 140, "BIN", 20, P->tx);

805	    frame(16, 100, 208, 18, P->surf, 3, P->bd);

806	  }

807	  tft.fillRect(19, 103, min(202, (int)(now / 20)), 12, P->bd);

808	  const char* names[] = {"HC-SR04", "BME680", "HX711", "BMI160"};

809	  bool ok[] = {s.okHC, s.okBME, s.okHX, s.okIMU};

810	  for (int i = 0; i < 4; i++) {

811	    char sig[24];

812	    bool done = now > 1500u + i * 500u;

813	    snprintf(sig, sizeof(sig), "%d%d%d", i, done, ok[i]);

814	    if (!dirty(5 + i, sig)) continue;

815	    int y = 136 + i * 26;

816	    tft.fillRect(16, y, 210, 22, P->t);

817	    txt(20, y + 3, 120, names[i], 14, P->on_t);

818	    if (!done) chip(168, y, 52, 18, "...", P->surf, P->tx);

819	    else if (ok[i]) chip(168, y, 52, 18, "OK", P->surf, P->tx);

820	    else chip(168, y, 52, 18, "FAIL", P->r, P->on_r);

821	  }

822	  char ip[40];

823	  snprintf(ip, sizeof(ip), "IP: %s", s.wifi ? localIp().c_str() : "joining Wi-Fi...");

824	  if (dirty(9, ip)) {

825	    box(16, 252, 208, 56, P->surf, 3, 4);

826	    txt(26, 261, 190, ip, 16, P->tx);

827	    txt(26, 284, 190, "Firmware v" FW_VERSION, 13, P->tx2, 'l', true);

828	  }

829	}

830

831	// --- Calibration screen: three steps ticked by TARE, d_empty and d_full ---

832	static void drawCal(const BinState& s, const Cal& c, uint32_t now) {

833	  if (dirty(4, darkTheme ? "cal1" : "cal0")) {

834	    fullDraw = true;

835	    tft.fillScreen(P->t);

836	    frame(8, 222, 224, 44, P->surf, 3, P->bd);

837	    txt(8, 228, 224, "Keep the bin empty and", 13, P->tx, 'c');

838	    txt(8, 245, 224, "do not touch it.", 13, P->tx, 'c');

839	    btn(60, 268, 120, 42, "CANCEL", P->r, P->on_r, 15, -1, 0, B_CAL_CANCEL);

840	  }

841	  topBar(s, false, 0);

842	  char sig[40];

843	  snprintf(sig, sizeof(sig), "CALIBRATING%.*s", (int)((now / 300) % 4), "...");

844	  if (dirty(5, sig)) {

845	    tft.fillRect(0, 44, 240, 34, P->t);

846	    txt(8, 48, 224, "CALIBRATING", 26, P->on_t, 'c');

847	  }

848	  snprintf(sig, sizeof(sig), "%d", spin);

849	  if (dirty(6, sig)) {

850	    tft.fillRect(90, 80, 60, 60, P->t);

851	    icon(I_RING, 92, 82, 56, P->on_t, P->t, spin * 15);

852	  }

853	  char rows[3][24];

854	  strcpy(rows[0], "Weight zero");

855	  snprintf(rows[1], 24, "Empty: %.0f cm", c.d_empty);

856	  snprintf(rows[2], 24, "Full: %.0f cm", c.d_full);

857	  for (int i = 0; i < 3; i++) {

858	    bool ok = s.calSteps & (1 << i);

859	    char rs[40];

860	    snprintf(rs, sizeof(rs), "%s%d", rows[i], ok);

861	    if (!dirty(7 + i, rs)) continue;

862	    int y = 150 + i * 24;

863	    tft.fillRect(16, y, 210, 22, P->t);

864	    txt(20, y + 4, 140, rows[i], 15, P->on_t, 'l', true);

865	    chip(168, y, 52, 20, ok ? "OK" : "...", P->surf, P->tx);

866	  }

867	}

868

869	static String uptimeText() {

870	  uint32_t m = millis() / 60000;

871	  char b[24];

872	  if (m < 60) snprintf(b, sizeof(b), "%lu m", (unsigned long)m);

873	  else snprintf(b, sizeof(b), "%lu h %lu m", (unsigned long)(m / 60), (unsigned long)(m % 60));

874	  return b;

875	}

876

877	extern bool cloudFirebaseOn();

878

879	// --- Info screen: IP address, Wi-Fi, signal, cloud, Bluetooth, firmware,

880	// uptime. First place to look when the bin does not show up online. ---

881	static void drawInfo(const BinState& s) {

882	  if (dirty(4, darkTheme ? "info1" : "info0")) {

883	    fullDraw = true;

884	    tft.fillScreen(P->bg);

885	    btn(8, 266, 164, 44, "WI-FI SETUP", P->bl, P->on_bl, 15, I_BT, 20, B_WIFI_SETUP);

886	    box(180, 266, 52, 44, P->surf, 3, 4);

887	    icon(darkTheme ? I_MOON : I_SUN, 194, 275, 24, P->tx, P->surf);

888	    addHit(176, 262, 64, 58, B_THEME);

889	  }

890	  topBar(s, false, -1, "BACK");

891	  String ip = s.wifi ? localIp() : String("Not connected");

892	  if (dirty(5, ip.c_str())) {

893	    box(8, 46, 224, 66, P->surf, 3, 4);

894	    txt(18, 54, 200, "IP ADDRESS", 12, P->tx2);

895	    txt(18, 72, 204, ip.c_str(), 27, P->tx);

896	  }

897	  char rows[6][2][28];

898	  strcpy(rows[0][0], "Wi-Fi");

899	  strlcpy(rows[0][1], s.wifi ? WiFi.SSID().c_str() : net.ssid.c_str(), 28);

900	  strcpy(rows[1][0], "Signal");

901	  if (s.wifi) snprintf(rows[1][1], 28, "%d dBm", s.rssi);

902	  else strcpy(rows[1][1], "--");

903	  strcpy(rows[2][0], "Cloud");

904	  strcpy(rows[2][1], !cloudFirebaseOn() ? "OFF" : s.cloudOk ? "Firebase OK" : "NOT SENDING");

905	  strcpy(rows[3][0], "Bluetooth");

906	  strcpy(rows[3][1], s.btOn ? "ON" : "OFF");

907	  strcpy(rows[4][0], "Firmware");

908	  strcpy(rows[4][1], "v" FW_VERSION);

909	  strcpy(rows[5][0], "Uptime");

910	  strlcpy(rows[5][1], uptimeText().c_str(), 28);

911	  for (int i = 0; i < 6; i++) {

912	    char sig[40];

913	    snprintf(sig, sizeof(sig), "%s", rows[i][1]);

914	    if (!dirty(6 + i, sig)) continue;

915	    int y = 124 + i * 23;

916	    tft.fillRect(8, y, 224, 20, P->bg);

917	    txt(10, y + 3, 90, rows[i][0], 13, P->tx2, 'l', true);

918	    txt(100, y + 2, 130, rows[i][1], 15, P->tx, 'r');

919	    tft.drawFastHLine(8, y + 20, 224, P->rule);

920	  }

921	}

922

923	static void btTop() {

924	  tft.fillRect(0, 0, 240, 34, P->bl);

925	  tft.fillRect(0, 34, 240, 3, P->bd);

926	  txt(8, 9, 170, "WI-FI SETUP", 16, P->on_bl);

927	  chip(190, 8, 42, 18, "BT", P->surf, P->tx);

928	}

929

930	static void mmss(char* b, size_t n, int32_t ms) {

931	  if (ms < 0) ms = 0;

932	  int s = ms / 1000;

933	  snprintf(b, n, "%d:%02d", s / 60, s % 60);

934	}

935

936	// --- Bluetooth Wi-Fi setup screens, one layout per phase (see ble_setup.cpp) ---

937	static void drawBt(uint32_t now) {

938	  BtView v = bleView();

939	  char sig[64];

940	  snprintf(sig, sizeof(sig), "bt%d%d%s%lu%d", v.phase, darkTheme, v.ip, (unsigned long)v.code, v.netCount);

941	  bool cancel = v.phase != BT_OK && v.phase != BT_FAIL && v.phase != BT_OFF;

942	  if (dirty(4, sig)) {

943	    fullDraw = true;

944	    tft.fillScreen(P->bg);

945	    btTop();

946	    if (cancel) btn(8, 266, 224, 44, "CANCEL", P->r, P->on_r, 15, -1, 0, B_BT_CANCEL);

947	    switch (v.phase) {

948	      case BT_IDLE:

949	      case BT_WAIT:

950	        txt(8, 120, 224, BIN_BLE_NAME, 22, P->tx, 'c');

951	        wrapTxt(16, 152, 208, "On the dashboard or app press PAIR A BIN and pick this name.", 13, P->tx2, 17);

952	        break;

953	      case BT_CODE: {

954	        txt(8, 46, 224, "PAIRING CODE", 14, P->tx2, 'c');

955	        box(8, 66, 224, 72, P->surf, 3, 4);

956	        char code[12];

957	        snprintf(code, sizeof(code), "%03lu %03lu", (unsigned long)(v.code / 1000), (unsigned long)(v.code % 1000));

958	        txt(8, 76, 224, code, 46, P->tx, 'c');

959	        wrapTxt(16, 146, 208, "Type it in the pairing window that pops up on the laptop or phone.", 13, P->tx2, 17);

960	        txt(8, 242, 224, "3 wrong codes stop pairing for 60 s.", 12, P->tx2, 'c');

961	        break;

962	      }

963	      case BT_LOCKOUT:

964	        box(8, 48, 224, 130, P->r, 3, 4);

965	        txt(16, 62, 208, "PAIRING STOPPED", 22, P->on_r, 'c');

966	        txt(16, 98, 208, "3 wrong codes", 20, P->on_r, 'c');

967	        wrapTxt(16, 192, 208, "Only someone who can see this screen can pair. The phone that tried was turned away.", 13, P->tx2, 17);

968	        break;

969	      case BT_PAIRED:

970	        box(96, 44, 48, 48, P->g, 3, 4);

971	        icon(I_CHECK, 102, 50, 36, P->on_g, P->g);

972	        txt(8, 98, 224, "PAIRED", 22, P->tx, 'c');

973	        wrapTxt(16, 126, 208, "Pick the network on the dashboard.", 13, P->tx2, 17);

974	        for (int i = 0; i < min<int>(v.netCount, 3); i++) {

975	          int y = 168 + i * 27;

976	          frame(8, y, 224, 23, P->surf, 2, P->bd);

977	          txtFit(14, y + 4, 156, v.nets[i], 13, P->tx);

978	          char r[12];

979	          snprintf(r, sizeof(r), "%d dBm", v.rssi[i]);

980	          txt(172, y + 4, 54, r, 12, P->tx2, 'r');

981	        }

982	        break;

983	      case BT_CONN:

984	        txt(8, 128, 224, "CONNECTING...", 22, P->tx, 'c');

985	        box(24, 166, 192, 34, P->surf, 3, 3);

986	        txtFit(32, 174, 176, v.ssid, 16, P->tx, 'c');

987	        txt(20, 214, 200, "Password received.", 13, P->tx2, 'c', true);

988	        break;

989	      case BT_OK:

990	        box(8, 48, 224, 140, P->g, 3, 4);

991	        txt(16, 58, 208, "CONNECTED", 22, P->on_g, 'c');

992	        txt(16, 88, 208, "IP ADDRESS", 12, P->on_g, 'c');

993	        txt(16, 102, 208, v.ip, 28, P->on_g, 'c');

994	        txtFit(16, 140, 208, v.ssid, 14, P->on_g, 'c', true);

995	        txt(16, 162, 208, "Type it in the dashboard.", 13, P->on_g, 'c', true);

996	        wrapTxt(16, 200, 208, "Bluetooth turns off by itself in 5 minutes.", 13, P->tx2, 17);

997	        btn(8, 266, 224, 44, "DONE", P->surf, P->tx, 18, I_CHECK, 20, B_BT_DONE);

998	        break;

999	      case BT_FAIL:

1000	        box(8, 48, 224, 126, P->r, 3, 4);

1001	        txt(16, 62, 208, "FAILED", 28, P->on_r, 'c');

1002	        txt(16, 102, 208, v.why, 20, P->on_r, 'c');

1003	        txt(16, 132, 208, "Check it and try again.", 14, P->on_r, 'c', true);

1004	        wrapTxt(16, 190, 208, "Other reasons: network not found, or a 5 GHz network.", 13, P->tx2, 17);

1005	        btn(8, 266, 108, 44, "RETRY", P->bl, P->on_bl, 16, -1, 0, B_BT_RETRY);

1006	        btn(124, 266, 108, 44, "CANCEL", P->surf, P->tx, 16, -1, 0, B_BT_CANCEL);

1007	        break;

1008	      case BT_OFF:

1009	        box(84, 60, 72, 72, P->surf, 3, 4);

1010	        icon(I_CHECK, 94, 70, 52, P->tx, P->surf);

1011	        txt(8, 150, 224, "BLUETOOTH OFF", 24, P->tx, 'c');

1012	        txt(20, 184, 200, "Setup finished. Wi-Fi stays on.", 14, P->tx2, 'c', true);

1013	        txt(8, 262, 224, "Back to the main screen", 12, P->tx2, 'c');

1014	        break;

1015	    }

1016	  }

1017	  // Parts of the BT screens that change every second (countdowns, spinner).

1018	  char t[32], mm[12];

1019	  switch (v.phase) {

1020	    case BT_IDLE:

1021	    case BT_WAIT:

1022	      snprintf(sig, sizeof(sig), "%d%d", blink, (int)((v.offAtMs - now) / 1000));

1023	      if (dirty(5, sig)) {

1024	        tft.fillRect(84, 44, 76, 76, P->bg);

1025	        int o = blink ? 0 : 2;

1026	        box(88 + o, 48 + o, 64 - 2 * o, 64 - 2 * o, P->surf, 3, 4);

1027	        icon(I_BT, 96 + o, 56 + o, 48 - 2 * o, P->bl, P->surf);

1028	        mmss(mm, sizeof(mm), (int32_t)(v.offAtMs - now));

1029	        snprintf(t, sizeof(t), "BT OFF IN %s", mm);

1030	        tft.fillRect(20, 220, 204, 36, P->bg);

1031	        box(24, 222, 192, 30, P->surf, 3, 3);

1032	        txt(32, 229, 176, t, 14, P->tx, 'c');

1033	      }

1034	      break;

1035	    case BT_CODE:

1036	      mmss(mm, sizeof(mm), (int32_t)(v.phaseMs + BT_CODE_MS - now));

1037	      if (dirty(5, mm)) {

1038	        snprintf(t, sizeof(t), "NEW CODE IN %s", mm);

1039	        tft.fillRect(20, 202, 204, 34, P->bg);

1040	        box(24, 204, 192, 28, P->surf, 3, 3);

1041	        txt(32, 210, 176, t, 14, P->tx, 'c');

1042	      }

1043	      break;

1044	    case BT_LOCKOUT:

1045	      mmss(mm, sizeof(mm), (int32_t)(v.phaseMs + BT_LOCKOUT_MS - now));

1046	      if (dirty(5, mm)) {

1047	        snprintf(t, sizeof(t), "Try again in %s", mm);

1048	        tft.fillRect(12, 128, 216, 40, P->r);

1049	        txt(8, 134, 224, t, 16, P->on_r, 'c', true);

1050	      }

1051	      break;

1052	    case BT_CONN:

1053	      snprintf(sig, sizeof(sig), "%d", spin);

1054	      if (dirty(5, sig)) {

1055	        tft.fillRect(86, 48, 68, 68, P->bg);

1056	        icon(I_RING, 88, 50, 64, P->bl, P->bg, spin * 15);

1057	      }

1058	      break;

1059	    case BT_OFF: {

1060	      int32_t left = (int32_t)(v.phaseMs + 1500 - now);

1061	      snprintf(sig, sizeof(sig), "%d", max<int32_t>(0, left / 100));

1062	      if (dirty(5, sig)) {

1063	        frame(60, 282, 120, 12, P->surf, 2, P->bd);

1064	        tft.fillRect(62, 284, 116 * max<int32_t>(0, left) / 1500, 8, P->bd);

1065	      }

1066	      if (left <= 0) overlay = O_NONE;

1067	      break;

1068	    }

1069	    default:

1070	      break;

1071	  }

1072	}

1073

1074	// --- Touch calibration ---

1075	// Four crosses at the corners. Each press is averaged over at least 3 samples.

1076	// Started by the TOUCH_CAL command or by holding the screen for 5 s.

1077	static volatile bool tcReq = false, tcResetReq = false;

1078	static bool tcActive = false, tcWaitUp = false;

1079	static uint8_t tcStep = 0;

1080	static uint32_t tcLastMs = 0, tcDoneMs = 0;

1081	static long tcSx = 0, tcSy = 0;

1082	static int tcN = 0;

1083	static int tcRaw[4][2];

1084	static const int TC_X[4] = {20, 219, 20, 219};

1085	static const int TC_Y[4] = {20, 20, 299, 299};

1086

1087	void uiStartTouchCal() {

1088	  tcReq = true;

1089	  if (hUi) xTaskNotify(hUi, EV_STATE, eSetBits);

1090	}

1091

1092	void uiResetTouchCal() {

1093	  tcResetReq = true;

1094	  if (hUi) xTaskNotify(hUi, EV_STATE, eSetBits);

1095	}

1096

1097	// Calibration screen: cross for the current step, or "TOUCH SAVED".

1098	static void drawTouchCal() {

1099	  char sig[16];

1100	  snprintf(sig, sizeof(sig), "tc%d%d", tcStep, darkTheme);

1101	  if (!dirty(4, sig)) return;

1102	  fullDraw = true;

1103	  tft.fillScreen(P->bg);

1104	  if (tcStep < 4) {

1105	    txt(8, 112, 224, "TOUCH CALIBRATION", 16, P->tx, 'c');

1106	    char b[32];

1107	    snprintf(b, sizeof(b), "Press the cross  %d of 4", tcStep + 1);

1108	    txt(8, 142, 224, b, 14, P->tx, 'c', true);

1109	    wrapTxt(20, 170, 200, "Press firmly on the center, then let go. Wait 30 s to cancel.", 13, P->tx2, 17);

1110	    int x = TC_X[tcStep], y = TC_Y[tcStep];

1111	    tft.drawFastHLine(x - 14, y, 29, P->r);

1112	    tft.drawFastVLine(x, y - 14, 29, P->r);

1113	    tft.drawCircle(x, y, 7, P->r);

1114	  } else {

1115	    txt(8, 120, 224, "TOUCH SAVED", 22, P->tx, 'c');

1116	    txt(8, 160, 224, "Buttons should line up now", 14, P->tx2, 'c', true);

1117	  }

1118	}

1119

1120	// Which screen to show, highest priority first: touch calibration, boot,

1121	// calibration, Bluetooth, Info/PIN overlays, collection complete, tamper,

1122	// collecting, locked, urgent, main.

1123	static Screen pickScreen(const BinState& s, const Cal& c, const Sim& sm, uint32_t now) {

1124	  if (tcActive) return S_TOUCHCAL;

1125	  if (s.booting) return S_BOOT;

1126	  if (s.calibrating) return S_CAL;

1127	  if (overlay == O_BT) return S_BT;

1128	  if ((overlay == O_INFO || overlay == O_PIN) && now - overlayMs > SCREEN_TIMEOUT_MS) {

1129	    overlay = O_NONE;

1130	    buzz(T_BACK_MAIN);

1131	  }

1132	  if (overlay == O_INFO) return S_INFO;

1133	  if (overlay == O_PIN) return S_PIN;

1134	  if (doneShownAt && !s.collecting) {

1135	    if (now - doneShownAt < 30000) return S_DONE;

1136	    doneShownAt = 0;

1137	    buzz(T_BACK_MAIN);

1138	  }

1139	  if (s.tamperPhase) return S_TAMPER;

1140	  if (s.collecting) return S_COLLECT;

1141	  if (s.status == ST_LOCKED) return S_LOCKED;

1142	  if ((s.okHC || (sm.mask & 1)) && s.v.fill >= c.fill_th) return S_URGENT;

1143	  return S_MAIN;

1144	}

1145

1146	// Clears the widget cache and hit boxes whenever the screen changes, then

1147	// draws the current screen.

1148	static void render(uint32_t now) {

1149	  BinState s = snapshot();

1150	  Cal c = calSnapshot();

1151	  Sim sm = simSnapshot();

1152	  if (s.doneAtMs && !doneShownAt && now - s.doneAtMs < 2000) doneShownAt = s.doneAtMs;

1153	  Screen want = pickScreen(s, c, sm, now);

1154	  if (want != cur) {

1155	    cur = want;

1156	    memset(wcache, 0, sizeof(wcache));

1157	    hitN = 0;

1158	  }

1159	  fullDraw = false;

1160	  switch (cur) {

1161	    case S_BOOT: drawBoot(s, now); break;

1162	    case S_CAL: drawCal(s, c, now); break;

1163	    case S_MAIN: drawMain(s, c, sm, now); break;

1164	    case S_URGENT: drawUrgent(s, c, sm, now); break;

1165	    case S_PIN: drawPin(now); break;

1166	    case S_COLLECT: drawCollect(now); break;

1167	    case S_DONE: drawDone(now); break;

1168	    case S_TAMPER: drawTamper(s, c, now); break;

1169	    case S_LOCKED: drawLocked(s); break;

1170	    case S_INFO: drawInfo(s); break;

1171	    case S_BT: drawBt(now); break;

1172	    case S_TOUCHCAL: drawTouchCal(); break;

1173	  }

1174	}

1175

1176	static void resetScreen() {

1177	  memset(wcache, 0, sizeof(wcache));

1178	  hitN = 0;

1179	}

1180

1181	static void openOverlay(Overlay o) {

1182	  overlay = o;

1183	  overlayMs = millis();

1184	  resetScreen();

1185	}

1186

1187	// What each button does. PIN digits are collected here; OK compares them with

1188	// the Collect PIN from the calibration settings.

1189	static void onPress(uint8_t id) {

1190	  BinState s = snapshot();

1191	  if (id >= B_K0 && id <= B_K9) {

1192	    if (strlen(pinBuf) < 4) {

1193	      size_t n = strlen(pinBuf);

1194	      pinBuf[n] = '0' + (id - B_K0);

1195	      pinBuf[n + 1] = 0;

1196	    }

1197	    pinWrongUntil = 0;

1198	    buzz(T_PIN_KEY);

1199	    overlayMs = millis();

1200	    return;

1201	  }

1202	  switch (id) {

1203	    case B_COLLECT:

1204	      pinForBt = false;

1205	      pinBuf[0] = 0;

1206	      buzz(T_OPEN);

1207	      openOverlay(O_PIN);

1208	      break;

1209	    case B_INFO:

1210	      buzz(T_OPEN);

1211	      openOverlay(O_INFO);

1212	      break;

1213	    case B_THEME:

1214	      darkTheme = !darkTheme;

1215	      P = darkTheme ? &DARK : &LIGHT;

1216	      saveTheme(darkTheme);

1217	      buzz(darkTheme ? T_THEME_DARK : T_THEME_LIGHT);

1218	      resetScreen();

1219	      break;

1220	    case B_BACK:

1221	    case B_PIN_CANCEL:

1222	      buzz(T_CANCEL);

1223	      pinBuf[0] = 0;

1224	      openOverlay(O_NONE);

1225	      break;

1226	    case B_KDEL: {

1227	      size_t n = strlen(pinBuf);

1228	      if (n) pinBuf[n - 1] = 0;

1229	      buzz(T_PIN_DEL);

1230	      overlayMs = millis();

1231	      break;

1232	    }

1233	    case B_KOK: {

1234	      char want[8];

1235	      snprintf(want, sizeof(want), "%04u", calSnapshot().collect_pin);

1236	      if (strlen(pinBuf) == 4 && !strcmp(pinBuf, want)) {

1237	        pinBuf[0] = 0;

1238	        if (pinForBt) {

1239	          buzz(T_OPEN);

1240	          openOverlay(O_BT);

1241	          bleStart();

1242	        } else {

1243	          buzz(T_PIN_OK);

1244	          openOverlay(O_NONE);

1245	          sendCommand("COLLECT", SRC_SCREEN);

1246	        }

1247	      } else {

1248	        pinBuf[0] = 0;

1249	        pinWrongUntil = millis() + 1600;

1250	        buzz(T_PIN_WRONG);

1251	        overlayMs = millis();

1252	      }

1253	      break;

1254	    }

1255	    case B_DONE:

1256	      sendCommand("COLLECT_DONE", SRC_SCREEN);

1257	      doneShownAt = millis();

1258	      break;

1259	    case B_DONE_BACK:

1260	      doneShownAt = 0;

1261	      buzz(T_BACK_MAIN);

1262	      resetScreen();

1263	      break;

1264	    case B_CAL_CANCEL:

1265	      sendCommand("CALIBRATE_END", SRC_SCREEN);

1266	      break;

1267	    case B_WIFI_SETUP:

1268	      pinForBt = true;

1269	      pinBuf[0] = 0;

1270	      buzz(T_OPEN);

1271	      openOverlay(O_PIN);

1272	      break;

1273	    case B_BT_CANCEL:

1274	      buzz(T_CANCEL);

1275	      bleStop(false);

1276	      openOverlay(O_NONE);

1277	      break;

1278	    case B_BT_DONE:

1279	      buzz(T_KEY);

1280	      openOverlay(O_NONE);

1281	      break;

1282	    case B_BT_RETRY:

1283	      buzz(T_KEY);

1284	      bleRetry();

1285	      resetScreen();

1286	      break;

1287	  }

1288	  (void)s;

1289	}

1290

1291	// Reads the touch controller and maps raw values to screen pixels using the

1292	// saved calibration. Pressure z > 200 counts as a press. Set TOUCH_DEBUG to 1

1293	// in config.h to print raw values when the touch is offset.

1294	static bool readTouch(int& x, int& y, int& rawX, int& rawY) {

1295	  if (!touch.touched()) return false;

1296	  TS_Point p = touch.getPoint();

1297	  rawX = p.x;

1298	  rawY = p.y;

1299	  int rx = p.x, ry = p.y;

1300	  if (touchCal.swap) { rx = p.y; ry = p.x; }

1301	#if TOUCH_DEBUG

1302	  Serial.printf("[touch] raw %d %d z %d\n", rx, ry, p.z);

1303	#endif

1304	  x = constrain(map(rx, touchCal.x0, touchCal.x1, 0, 239), 0, 239);

1305	  y = constrain(map(ry, touchCal.y0, touchCal.y1, 0, 319), 0, 319);

1306	  return p.z > 200;

1307	}

1308

1309	static void tcBegin(uint32_t now) {

1310	  tcActive = true;

1311	  tcWaitUp = true;

1312	  tcStep = 0;

1313	  tcN = 0;

1314	  tcSx = tcSy = 0;

1315	  tcLastMs = now;

1316	  overlay = O_NONE;

1317	  resetScreen();

1318	  buzz(T_CAL_START);

1319	}

1320

1321	static void tcEnd() {

1322	  tcActive = false;

1323	  resetScreen();

1324	}

1325

1326	// Works out the raw value at each screen edge from the four presses. If the

1327	// presses changed more along the other axis, the X and Y wires are swapped.

1328	// Presses less than 800 raw units apart are rejected (probably a missed press).

1329	static void tcFinish(uint32_t now) {

1330	  float dxx = (tcRaw[1][0] + tcRaw[3][0]) / 2.0f - (tcRaw[0][0] + tcRaw[2][0]) / 2.0f;

1331	  float dxy = (tcRaw[1][1] + tcRaw[3][1]) / 2.0f - (tcRaw[0][1] + tcRaw[2][1]) / 2.0f;

1332	  bool sw = fabsf(dxy) > fabsf(dxx);

1333	  float px[4], py[4];

1334	  for (int i = 0; i < 4; i++) {

1335	    px[i] = sw ? tcRaw[i][1] : tcRaw[i][0];

1336	    py[i] = sw ? tcRaw[i][0] : tcRaw[i][1];

1337	  }

1338	  float left = (px[0] + px[2]) / 2, right = (px[1] + px[3]) / 2;

1339	  float top = (py[0] + py[1]) / 2, bottom = (py[2] + py[3]) / 2;

1340	  if (fabsf(right - left) < 800 || fabsf(bottom - top) < 800) {

1341	    Serial.println("[touch] calibration rejected, presses too close together");

1342	    buzz(T_CAL_CANCEL);

1343	    raiseAlert("setup", "warning", "Touch calibration failed", "Presses were too close together. Try again.");

1344	    tcEnd();

1345	    return;

1346	  }

1347	  float kx = (right - left) / (TC_X[1] - TC_X[0]);

1348	  float ky = (bottom - top) / (TC_Y[2] - TC_Y[0]);

1349	  TouchCal t;

1350	  t.swap = sw;

1351	  t.x0 = lroundf(left - kx * TC_X[0]);

1352	  t.x1 = lroundf(right + kx * (239 - TC_X[1]));

1353	  t.y0 = lroundf(top - ky * TC_Y[0]);

1354	  t.y1 = lroundf(bottom + ky * (319 - TC_Y[2]));

1355	  touchCal = t;

1356	  saveTouchCal();

1357	  Serial.printf("[touch] saved swap %d x %d..%d y %d..%d\n", t.swap, t.x0, t.x1, t.y0, t.y1);

1358	  buzz(T_SAVED);

1359	  char meta[72];

1360	  snprintf(meta, sizeof(meta), "%s touchscreen calibrated", BIN_LABEL);

1361	  raiseAlert("setup", "success", "Touch calibrated", meta);

1362	  publish(EV_CAL);

1363	  tcStep = 4;

1364	  tcDoneMs = now;

1365	}

1366

1367	// Touch calibration input: wait for release, collect samples while pressed,

1368	// store the average on release. Gives up after 30 s with no press.

1369	static void tcHandle(bool pressed, int rx, int ry, uint32_t now) {

1370	  if (tcStep >= 4) {

1371	    if (now - tcDoneMs > 2500) tcEnd();

1372	    return;

1373	  }

1374	  if (tcWaitUp) {

1375	    if (!pressed) tcWaitUp = false;

1376	    tcLastMs = now;

1377	    return;

1378	  }

1379	  if (pressed) {

1380	    tcSx += rx;

1381	    tcSy += ry;

1382	    tcN++;

1383	    tcLastMs = now;

1384	    return;

1385	  }

1386	  if (tcN >= 3) {

1387	    tcRaw[tcStep][0] = tcSx / tcN;

1388	    tcRaw[tcStep][1] = tcSy / tcN;

1389	    tcStep++;

1390	    if (tcStep == 4) tcFinish(now);

1391	    else buzz(T_CAL_STEP);

1392	  }

1393	  tcSx = tcSy = 0;

1394	  tcN = 0;

1395	  if (tcActive && tcStep < 4 && now - tcLastMs > 30000) {

1396	    buzz(T_CAL_CANCEL);

1397	    tcEnd();

1398	  }

1399	}

1400

1401	// The touch CS is driven high before the display starts so the touch chip

1402	// stays off the bus. SPI at 40 MHz for the display.

1403	void uiInit() {

1404	  SPI.begin(PIN_SPI_SCK, PIN_SPI_MISO, PIN_SPI_MOSI);

1405	  pinMode(PIN_TOUCH_CS, OUTPUT);

1406	  digitalWrite(PIN_TOUCH_CS, HIGH);

1407	  tft.begin(40000000);

1408	  tft.setRotation(TFT_ROTATION);

1409	  touch.begin(SPI);

1410	  touch.setRotation(TFT_ROTATION);

1411	  darkTheme = loadTheme();

1412	  P = darkTheme ? &DARK : &LIGHT;

1413	  pinMode(PIN_TOUCH_IRQ, INPUT);

1414	  tft.fillScreen(P->t);

1415	}

1416

1417	// Polls touch every 30 ms while pressed, 100 ms otherwise. A press is accepted

1418	// at most every 220 ms (debounce). Holding anywhere for 5 s opens touch

1419	// calibration, a way out when the touch is too far off to hit any button.

1420	void uiTask(void*) {

1421	  bool down = false;

1422	  uint32_t lastDraw = 0, holdSince = 0;

1423	  for (;;) {

1424	    uint32_t ev = 0;

1425	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(down ? 30 : 100));

1426	    uint32_t now = millis();

1427	    if (now - lastAnim >= 500) {

1428	      lastAnim = now;

1429	      blink = !blink;

1430	    }

1431	    spin = (now / 120) % 24;

1432

1433	    if (tcReq) {

1434	      tcReq = false;

1435	      tcBegin(now);

1436	    }

1437	    if (tcResetReq) {

1438	      tcResetReq = false;

1439	      resetTouchCal();

1440	      buzz(T_SAVED);

1441	      publish(EV_CAL);

1442	    }

1443	    {

1444	      int x, y, rx = 0, ry = 0;

1445	      bool pressed = readTouch(x, y, rx, ry);

1446	      if (!pressed) holdSince = 0;

1447	      else if (!holdSince) holdSince = now;

1448	      if (tcActive) {

1449	        tcHandle(pressed, rx, ry, now);

1450	        down = pressed;

1451	        holdSince = 0;

1452	      } else if (pressed && holdSince && now - holdSince > 5000) {

1453	        holdSince = 0;

1454	        tcBegin(now);

1455	      } else if (pressed && !down && now - lastTouchMs > 220) {

1456	        down = true;

1457	        lastTouchMs = now;

1458	        // Last hit box first, so a box drawn on top wins.

1459	        for (int i = hitN - 1; i >= 0; i--) {

1460	          const Hit& h = hits[i];

1461	          if (x >= h.x && x < h.x + h.w && y >= h.y && y < h.y + h.h) {

1462	            if (h.id < B_K0 && h.id != B_KOK && h.id != B_KDEL) buzz(T_KEY);

1463	            onPress(h.id);

1464	            break;

1465	          }

1466	        }

1467	      } else if (!pressed) {

1468	        down = false;

1469	      }

1470	    }

1471	    if ((ev & ~EV_TOUCH) || now - lastDraw >= 100) {

1472	      lastDraw = now;

1473	      render(now);

1474	    }

1475	  }

1476	}

firmware/firebase.rules.json

1	{

2	  "rules": {

3	    "sb_users": {

4	      "$uid": {

5	        ".read": "auth != null && auth.uid === $uid"

6	      }

7	    },

8	    "sb": {

9	      ".read": "auth != null && (root.child('sb_users').child(auth.uid).exists() || auth.token.email.matches(/^bin.*@/))",

10	      "bins": {

11	        "$b": {

12	          ".write": "auth != null && auth.token.email.matches(/^bin.*@/)"

13	        }

14	      },

15	      "commands": {

16	        "$b": {

17	          "$c": {

18	            ".write": "auth != null && ((!data.exists() && root.child('sb_users').child(auth.uid).exists()) || (!newData.exists() && auth.token.email.matches(/^bin.*@/)))",

19	            ".validate": "!newData.exists() || (newData.hasChildren(['cmd', 'ts']) && newData.child('cmd').isString())"

20	          }

21	        }

22	      },

23	      "results": {

24	        "$r": {

25	          ".write": "auth != null"

26	        }

27	      },

28	      "alerts": {

29	        "$b": {

30	          ".write": "auth != null && auth.token.email.matches(/^bin.*@/)"

31	        }

32	      },

33	      "watchers": {

34	        ".indexOn": "ts",

35	        "$w": {

36	          ".write": "auth != null && root.child('sb_users').child(auth.uid).exists()"

37	        }

38	      },

39	      "usage": {

40	        ".write": "auth != null && auth.token.email.matches(/^bin.*@/)"

41	      }

42	    }

43	  }

44	}

firmware/tools/embed_dashboard.py

1	#!/usr/bin/env python3

2	"""Packs Web Dashboard/smartbin-dashboard.html into firmware/SmartBin/dashboard_gz.h

3	so each bin serves the dashboard at http://<bin-ip>/ . Run it again after changing the

4	dashboard, then upload the firmware.

5

6	    python firmware/tools/embed_dashboard.py

7	"""

8	import gzip

9	import pathlib

10

11	root = pathlib.Path(__file__).resolve().parents[2]

12	src = root / "Web Dashboard" / "smartbin-dashboard.html"

13	dst = root / "firmware" / "SmartBin" / "dashboard_gz.h"

14

15	data = gzip.compress(src.read_bytes(), compresslevel=9, mtime=0)

16	rows = [", ".join("0x%02x" % b for b in data[i:i + 20]) for i in range(0, len(data), 20)]

17	dst.write_text(

18	    "#pragma once\n// Generated by firmware/tools/embed_dashboard.py from Web Dashboard/smartbin-dashboard.html. Do not edit.\n"

19	    "#include <Arduino.h>\n"

20	    "const size_t DASHBOARD_GZ_LEN = %d;\n"

21	    "const uint8_t DASHBOARD_GZ[] PROGMEM = {\n  %s\n};\n" % (len(data), ",\n  ".join(rows)),

22	    encoding="ascii",

23	)

24	print("%s: %d bytes of HTML -> %d bytes gzipped" % (dst.name, src.stat().st_size, len(data)))

Part 2. Web Dashboard

Web Dashboard/smartbin-dashboard.html

1	<!DOCTYPE html>

2	<!--

3	============================================================================

4	smartbin-dashboard.html - Smart-Bin web dashboard (one self-contained file)

5	Served gzipped by the bin itself at http://<bin-ip>/ (see net.cpp and

6	tools/embed_dashboard.py) and can also be opened straight from disk.

7	Layout of this file:

8	  <style>   neumorphic + neo-brutalist theme, light and dark, mode tints

9	  <body>    start page (gate), PIN dialog, header, five tab sections

10	  <script>  state, transport (Wi-Fi, Firebase, ThingSpeak), alerts,

11	            rendering, charts, settings, simulation, demo tour, sounds,

12	            operator PIN and Bluetooth Wi-Fi setup

13	Debug tip: open the browser's developer tools (F12). Network shows each

14	/api/status, /api/events, Firebase and ThingSpeak request; Console shows

15	script errors. The dashboard keeps its bin list and settings in

16	localStorage (keys starting with "smartbin."); clearing site data resets it.

17	============================================================================

18	-->

19	<html lang="en" data-theme="light" data-mode="normal">

20	<head>

21	<meta charset="utf-8">

22	<meta name="viewport" content="width=device-width, initial-scale=1">

23	<title>Smart Bin Dashboard</title>

24	<style>

25	/* ---------- Theme tokens ----------

26	   Light values here; [data-theme="dark"] overrides them further down.

27	   --shL/--shD are the light and dark halves of every soft shadow, so

28	   changing them restyles every raised and sunk surface at once.

29	   [data-mode] (normal, sim, calib, ...) on <html> tints the page per mode. */

30	:root{

31	  --base:#E4DCD2; --tint:#E4DCD2;

32	  --shL:#FFFBF5; --shD:#BEB2A4;

33	  --ink:#2A241E; --ink2:#7C7265;

34	  --well:#DED5CA;

35	  --ok:#1E8E3E; --warn:#D98A1F; --bad:#D93025; --sim:#7A45B5; --cal:#0D8A8A;

36	  --accent:#1E8E3E; --accentSoft:rgba(30,142,62,.22); --onAccent:#FFFFFF;

37	  --o1:-6px -6px 12px var(--shL), 6px 6px 12px var(--shD);

38	  --o2:-11px -11px 22px var(--shL), 11px 11px 22px var(--shD);

39	  --i1:inset -5px -5px 10px var(--shL), inset 5px 5px 10px var(--shD);

40	  --i2:inset -8px -8px 16px var(--shL), inset 8px 8px 16px var(--shD);

41	  --r:20px;

42	}

43	html[data-theme="dark"]{

44	  --base:#2B2622; --tint:#2B2622;

45	  --shL:#3B342E; --shD:#1A1613;

46	  --ink:#EFE7DC; --ink2:#A2978B;

47	  --well:#262220;

48	  --ok:#4FBF6E; --warn:#E8A94A; --bad:#FF6B5E; --sim:#B98BE8; --cal:#3FC7C2;

49	  --accent:#4FBF6E; --accentSoft:rgba(79,191,110,.22); --onAccent:#13100D;

50	}

51	:root{--accentBar:var(--accent);--simSoft:rgba(122,69,181,.3);--calSoft:rgba(13,138,138,.3);

52	  --c-temp:#C2591B; --c-humid:#1F6FA8; --c-weight:#7A5A2E; --c-gas:#6B4FB8; --c-press:#B03A6F;

53	  --c-fill:#1E7A52; --c-bt:#1A62C4; --c-lid:#B07415; --c-motion:#0E7F9E; --c-link:#1E8E3E; --c-cloud:#D9631E; --c-ts:#1A62C4; --c-tamper:#D93025;

54	  --lv-cool:#1F6FA8; --lv-low:#1E8E3E; --lv-mid:#B88410; --lv-high:#D9641F; --lv-crit:#D93025}

55	html[data-theme="dark"]{--simSoft:rgba(185,139,232,.34);--calSoft:rgba(63,199,194,.32);

56	  --c-temp:#F0A368; --c-humid:#6CB8E8; --c-weight:#D6B98C; --c-gas:#B596F0; --c-press:#E58AB4;

57	  --c-fill:#4FC48D; --c-bt:#79AEF5; --c-lid:#E8B45C; --c-motion:#5FD0E8; --c-link:#4FBF6E; --c-cloud:#F29A5C; --c-ts:#79AEF5; --c-tamper:#FF6B5E;

58	  --lv-cool:#6CB8E8; --lv-low:#4FBF6E; --lv-mid:#E8C45A; --lv-high:#F0A368; --lv-crit:#FF6B5E}

59	html[data-mode="sim"]{--accent:var(--sim);--accentSoft:rgba(122,69,181,.22);--tint:#E2DAE1}

60	html[data-mode="calib"]{--accent:var(--cal);--accentSoft:rgba(13,138,138,.2);--tint:#DBE3E0}

61	html[data-mode="dual"]{--accent:#43679F;--accentSoft:rgba(67,103,159,.24);--tint:#DEDDE6;

62	  --accentBar:linear-gradient(90deg,var(--sim) 0%,#43679F 50%,var(--cal) 100%)}

63	html[data-mode="alert"]{--accent:var(--bad);--accentSoft:rgba(217,48,37,.2);--tint:#EADCD5}

64	html[data-theme="dark"][data-mode="sim"]{--accentSoft:rgba(185,139,232,.22);--tint:#2C2530}

65	html[data-theme="dark"][data-mode="calib"]{--accentSoft:rgba(63,199,194,.2);--tint:#222B2A}

66	html[data-theme="dark"][data-mode="dual"]{--accent:#7CA9D5;--accentSoft:rgba(124,169,213,.24);--tint:#262A33;

67	  --accentBar:linear-gradient(90deg,var(--sim) 0%,#7CA9D5 50%,var(--cal) 100%)}

68	html[data-theme="dark"][data-mode="alert"]{--accentSoft:rgba(255,107,94,.2);--tint:#302422}

69	html[data-mode="sim"] .ico-accent,html[data-mode="calib"] .ico-accent{color:var(--accent)}

70

71	*{box-sizing:border-box}

72	html,body{margin:0;padding:0}

73	body{background:var(--tint);color:var(--ink);transition:background .6s ease;

74	  font-family:Inter,"Segoe UI",system-ui,-apple-system,Arial,sans-serif;font-size:15px;line-height:1.45}

75	button,input,select{font:inherit;color:inherit}

76	button{cursor:pointer;background:none;border:0}

77	:focus-visible{outline:3px solid var(--accent);outline-offset:3px;border-radius:6px}

78	h2{font-size:12px;letter-spacing:1.4px;font-weight:800;text-transform:uppercase;color:var(--ink2);margin:0 0 14px}

79

80	.nm{background:var(--base);border-radius:var(--r);box-shadow:var(--o2)}

81	.nm-sm{background:var(--base);border-radius:14px;box-shadow:var(--o1)}

82	.nm-in{background:var(--well);border-radius:14px;box-shadow:var(--i1)}

83	.nm-deep{background:var(--well);border-radius:16px;box-shadow:var(--i2)}

84	.glow{box-shadow:var(--o2), 0 0 0 2px var(--accentSoft)}

85

86	.btn{background:var(--base);border-radius:14px;padding:11px 18px;font-weight:700;letter-spacing:.3px;

87	  box-shadow:var(--o1);transition:box-shadow .18s ease,transform .18s ease,color .2s}

88	.btn:hover{color:var(--accent)}

89	.btn:active,.btn[aria-pressed="true"]{box-shadow:var(--i1);transform:translateY(1px);color:var(--accent)}

90	.btn[disabled]{opacity:.4;cursor:not-allowed;box-shadow:var(--o1)}

91	.btn.sm{padding:7px 13px;font-size:13px;border-radius:11px}

92	.btn.accent{color:var(--onAccent);background:var(--accent);box-shadow:var(--o1)}

93	.btn.accent:hover{color:var(--onAccent);filter:brightness(1.06)}

94	.btn.danger{color:#fff;background:var(--bad);border:2px solid var(--ink);border-radius:6px;box-shadow:4px 4px 0 var(--ink)}

95	.btn.danger:active{box-shadow:1px 1px 0 var(--ink);transform:translate(3px,3px)}

96	.btn.wide{width:100%;text-align:center}

97	.iconbtn{width:38px;height:38px;border-radius:12px;background:var(--base);box-shadow:var(--o1);

98	  display:grid;place-items:center;color:var(--ink2);transition:box-shadow .18s,color .18s}

99	.iconbtn:hover{color:var(--accent)}

100	.iconbtn:active,.iconbtn[aria-expanded="true"]{box-shadow:var(--i1);color:var(--accent)}

101

102	input[type=text],input[type=number],input[type=password],select{background:var(--well);border:0;border-radius:12px;padding:10px 13px;width:100%;

103	  box-shadow:var(--i1);transition:box-shadow .2s}

104	input:focus{box-shadow:var(--i1), 0 0 0 2px var(--accentSoft);outline:none}

105	input[type=number]{font-variant-numeric:tabular-nums}

106	input[type=range]{accent-color:var(--accent);width:100%}

107	input[type=checkbox]{accent-color:var(--accent);transform:scale(1.3)}

108

109	.ico{width:24px;height:24px;display:block;overflow:visible}

110	.ico.lg{width:60px;height:60px;overflow:hidden}

111	.ico.xs{width:17px;height:17px}

112

113	@media(prefers-reduced-motion:no-preference){

114	  .wave{animation:drift 3.4s linear infinite}

115	  .vap{animation:rise 2.6s ease-in-out infinite}

116	  .vap.v2{animation-delay:.5s} .vap.v3{animation-delay:1s}

117	  .fast .vap{animation-duration:1.1s}

118	  .pulse{animation:breathe 2.8s ease-in-out infinite}

119	  .ring{animation:spin 2.4s linear infinite;transform-origin:12px 12px}

120	  .shake{animation:shake .45s ease-in-out infinite}

121	  .arcs>*{animation:sonar 1.6s ease-out infinite}

122	  .arcs>:nth-child(2){animation-delay:.25s} .arcs>:nth-child(3){animation-delay:.5s}

123	  .wifi.live>*{animation:sonar 2.2s ease-out infinite}

124	  .wifi.live>:nth-child(2){animation-delay:.3s} .wifi.live>:nth-child(3){animation-delay:.6s}

125	  .wheel{animation:spin 1.1s linear infinite}

126	  .glowdot{animation:breathe 2s ease-in-out infinite}

127	}

128	@keyframes drift{to{transform:translateX(24px)}}

129	@keyframes rise{0%{transform:translateY(2px);opacity:.25}50%{opacity:1}100%{transform:translateY(-4px);opacity:.2}}

130	@keyframes breathe{0%,100%{transform:scale(.82);opacity:.45}50%{transform:scale(1.12);opacity:1}}

131	@keyframes spin{to{transform:rotate(360deg)}}

132	@keyframes shake{0%,100%{transform:rotate(-6deg)}50%{transform:rotate(6deg)}}

133	@keyframes sonar{0%{opacity:0}30%{opacity:1}100%{opacity:0}}

134	.lidflap{transform-origin:6px 9px;transition:transform .45s cubic-bezier(.3,1.4,.5,1)}

135	.open .lidflap{transform:rotate(-38deg)}

136	.shackle{transition:transform .3s ease}

137	.locked .shackle{transform:translateY(0)} .unlocked .shackle{transform:translateY(-3px)}

138	.merc,.needle,.wavewrap{transition:all .6s ease}

139	.pulse{transform-origin:center}

140

141	header.top{position:sticky;top:0;z-index:40;background:var(--tint);padding:12px 16px;

142	  box-shadow:0 10px 18px -16px var(--shD);transition:background .6s ease}

143	header.top:after{content:"";display:block;height:3px;border-radius:3px;background:var(--accentBar);

144	  max-width:1280px;margin:12px auto -2px;opacity:.85;transition:background .4s}

145	.topin{max-width:1280px;margin:0 auto;display:flex;gap:12px;align-items:center;flex-wrap:wrap}

146	.brand{display:flex;align-items:center;gap:10px;font-weight:800;font-size:18px;letter-spacing:.6px}

147	.brand .ico{color:var(--accent)}

148	.spacer{flex:1}

149	.link{display:flex;align-items:center;gap:9px;font-size:13px;font-weight:700;padding:8px 15px;border-radius:999px;

150	  background:var(--base);box-shadow:var(--o1)}

151	.link .ico{color:var(--accent)}

152	#demoBtn{color:var(--warn);display:inline-flex;align-items:center;gap:6px}

153	#demoBtn .ico{width:15px;height:15px}

154	.btnets{display:flex;flex-wrap:wrap;gap:8px;margin-top:10px}

155	.btnets:empty{display:none}

156	.btnets .btn{font-size:12.5px;padding:8px 12px;display:flex;gap:8px;align-items:center}

157	.btnets .btn small{opacity:.65;font-weight:600}

158	.btnets .btn[aria-pressed="true"]{color:var(--accent)}

159	.modechip{display:flex;align-items:center;gap:8px;font-size:12px;font-weight:800;letter-spacing:.6px;

160	  padding:8px 14px;border-radius:999px;background:var(--accentBar);color:var(--onAccent);box-shadow:var(--o1)}

161

162	.wrap{max-width:1280px;margin:0 auto;padding:22px 16px 70px}

163	.tabs{display:flex;gap:10px;flex-wrap:wrap;margin-bottom:20px}

164	.tab{display:flex;align-items:center;gap:9px;padding:10px 17px;border-radius:14px;font-weight:700;

165	  background:var(--base);box-shadow:var(--o1);color:var(--ink2);transition:all .2s}

166	.tab:hover{color:var(--accent)}

167	.tab[aria-selected="true"]{box-shadow:var(--i1);color:var(--accent)}

168

169	.banner{display:flex;gap:12px;align-items:center;flex-wrap:wrap;padding:13px 17px;margin-bottom:14px;

170	  border:2px solid var(--ink);border-radius:8px;box-shadow:5px 5px 0 var(--ink);font-weight:700;color:#fff}

171	.banner .ico{color:#fff}

172	.banner.red{background:var(--bad)} .banner.purple{background:var(--sim)}

173	.banner.teal{background:var(--cal)}

174	.banner.green{background:var(--ok)}

175	.banner.amber{background:var(--warn);color:#241C10}

176	.banner.amber .ico{color:#241C10}

177	.banner small{font-weight:600;opacity:.92}

178

179	.grid{display:grid;gap:22px}

180	.bins{grid-template-columns:repeat(auto-fill,minmax(340px,1fr))}

181	.lower{grid-template-columns:1fr 1fr;margin-top:22px}

182	@media(max-width:860px){.lower{grid-template-columns:1fr}}

183

184	.card{padding:20px;position:relative;transition:box-shadow .4s}

185	.card.hot{box-shadow:var(--o2), 0 0 0 2px var(--accentSoft)}

186	.cardtop{display:flex;gap:16px;align-items:flex-start}

187	.hero{width:76px;height:76px;flex:none;border-radius:50%;display:grid;place-items:center;

188	  background:var(--well);box-shadow:var(--i1);overflow:hidden}

189	.who{flex:1;min-width:0}

190	.binname{font-size:19px;font-weight:800;letter-spacing:.3px;overflow:hidden;text-overflow:ellipsis;white-space:nowrap}

191	.binplace{font-size:13px;color:var(--ink2)}

192	.fillnum{font-size:42px;font-weight:800;line-height:1;font-variant-numeric:tabular-nums;color:var(--accent)}

193	.fillnum span{font-size:17px}

194	.chips{display:flex;gap:8px;flex-wrap:wrap;margin-top:10px}

195	.chip{display:flex;align-items:center;gap:6px;font-size:11.5px;font-weight:800;letter-spacing:.4px;

196	  padding:6px 11px;border-radius:999px;background:var(--base);box-shadow:var(--o1);color:var(--ink2)}

197	.chip.loud{color:#fff;box-shadow:3px 3px 0 var(--ink);border:2px solid var(--ink);border-radius:6px}

198	.bar{height:26px;margin:18px 0 7px;position:relative;overflow:hidden;border-radius:13px}

199	.bar i{position:absolute;inset:3px auto 3px 3px;border-radius:11px;transition:width .6s ease,background .4s}

200	.bar b{position:absolute;top:4px;bottom:4px;width:3px;border-radius:2px;background:var(--ink);opacity:.35}

201	.ticks{display:flex;justify-content:space-between;font-size:11px;color:var(--ink2);font-weight:700}

202	.faultnote{margin:6px 0 0;font-size:12px;font-weight:800;color:var(--bad)}

203	.cardfoot{display:flex;align-items:center;gap:10px;margin:10px 0 2px;font-size:11.5px;font-weight:700;color:var(--ink2)}

204	.cardfoot .h1{font-size:11px;font-weight:800}

205	.cardfoot svg{flex:1;height:22px;overflow:visible}

206	.cardfoot svg path{fill:none;stroke-width:2;stroke-linecap:round;stroke-linejoin:round;vector-effect:non-scaling-stroke}

207	.cardfoot [data-r="ago"]{white-space:nowrap}

208	.cardfoot [data-r="ago"].old{color:var(--bad)}

209	.sensors{display:grid;grid-template-columns:repeat(4,1fr);gap:11px;margin-top:17px}

210	@media(max-width:520px){.sensors{grid-template-columns:repeat(2,1fr)}}

211	.sensor{padding:12px 6px 10px;text-align:center;display:flex;flex-direction:column;align-items:center;gap:5px}

212	.sensor .ico{color:var(--ink2)}

213	.sensor .v{font-size:17px;font-weight:800;font-variant-numeric:tabular-nums;line-height:1}

214	.sensor .k{font-size:10.5px;font-weight:700;color:var(--ink2);letter-spacing:.3px}

215	.sensor.alarm{background:var(--bad);box-shadow:4px 4px 0 var(--ink);border:2px solid var(--ink);border-radius:10px;color:#fff}

216	.sensor.alarm .ico,.sensor.alarm .k{color:#fff}

217	.actions{display:flex;gap:11px;margin-top:18px;flex-wrap:wrap}

218	.actions .btn{flex:1;min-width:116px;display:flex;align-items:center;justify-content:center;gap:8px}

219	.editrow{margin-top:16px;padding:16px;display:grid;gap:11px;grid-template-columns:repeat(auto-fit,minmax(140px,1fr))}

220	.editrow .field{grid-column:span 1}

221

222	.addrow{display:flex;align-items:center;gap:14px;margin-top:16px;flex-wrap:wrap}

223	.addmini{display:inline-flex;align-items:center;gap:9px;padding:9px 16px;border-radius:999px;font-size:13.5px;

224	  font-weight:700;color:var(--ink2);background:var(--base);box-shadow:var(--o1)}

225	.addmini:hover{color:var(--accent)}

226	.addmini .ico{width:18px;height:18px}

227	.addnote{font-size:12.5px;color:var(--ink2)}

228

229	.panel{padding:22px}

230	.prio{display:flex;flex-direction:column;gap:11px}

231	.prio .row{display:flex;align-items:center;gap:13px;padding:13px 15px;border-radius:14px;background:var(--base);box-shadow:var(--o1)}

232	.prio .n{width:28px;height:28px;flex:none;border-radius:50%;display:grid;place-items:center;font-size:13px;font-weight:800;

233	  background:var(--well);box-shadow:var(--i1)}

234	.prio .nm2{flex:1;font-weight:700;min-width:0;overflow:hidden;text-overflow:ellipsis;white-space:nowrap}

235	.prio .pc{font-weight:800;font-variant-numeric:tabular-nums}

236	.tag{font-size:10.5px;font-weight:800;letter-spacing:.5px;padding:4px 9px;border-radius:6px;color:#fff;border:2px solid var(--ink)}

237	.ttf{display:flex;align-items:center;gap:12px;padding:12px 15px;border-radius:14px;background:var(--base);box-shadow:var(--o1);margin-bottom:11px}

238	.ttf .nm2{flex:1;font-weight:700}

239	.ttf b{font-variant-numeric:tabular-nums}

240

241	.chartbox{padding:22px;margin-bottom:22px}

242	.chartbox canvas{width:100%;height:240px;display:block;border-radius:14px;background:var(--well);box-shadow:var(--i1)}

243	.legend{display:flex;gap:18px;font-size:12px;font-weight:700;margin-top:14px;flex-wrap:wrap;color:var(--ink2)}

244	.legend i{display:inline-block;width:16px;height:4px;border-radius:2px;margin-right:7px;vertical-align:middle}

245	.range{display:flex;gap:8px}

246

247	.seg{display:flex;gap:9px;margin-bottom:18px;flex-wrap:wrap}

248	.fields{display:grid;grid-template-columns:repeat(auto-fit,minmax(190px,1fr));gap:15px}

249	.field label{display:block;font-size:11.5px;font-weight:800;letter-spacing:.4px;color:var(--ink2);margin-bottom:7px}

250	.hint{font-size:12.5px;color:var(--ink2);margin-top:10px}

251	.switch{display:flex;align-items:center;gap:11px;font-weight:700;font-size:13.5px}

252	.switch input{width:auto;box-shadow:none;margin:0}

253

254	fieldset{border:0;margin:0;padding:0;min-width:0;transition:opacity .35s ease,filter .35s ease}

255	fieldset[disabled] .sbody{opacity:.5;filter:saturate(.4)}

256	.calbar{display:flex;gap:18px;align-items:center;flex-wrap:wrap;padding:18px 20px;border-radius:16px;margin-top:4px}

257	.calbar .calico{width:44px;height:44px;flex:none;border-radius:50%;display:grid;place-items:center;

258	  background:var(--base);box-shadow:var(--o1);color:var(--ink2);transition:color .4s}

259	.calbar.on{box-shadow:var(--i1), 0 0 0 2px var(--accentSoft)}

260	.calbar.on .calico{color:var(--accent)}

261	.calbar .caltxt{flex:1;min-width:200px}

262	.calbar .caltxt b{display:block;font-size:15px}

263	.calbar .caltxt span{font-size:12.5px;color:var(--ink2)}

264	.switch.big{gap:12px;font-size:14px;padding:11px 16px;border-radius:14px;background:var(--base);box-shadow:var(--o1);cursor:pointer}

265	.switch.big input{transform:scale(1.45)}

266	.calbar.on .switch.big{color:var(--accent)}

267	.withbtn{display:flex;gap:9px;align-items:center}

268	.withbtn .btn{flex:none;white-space:nowrap}

269	.readout{padding:11px 14px;font-weight:800;border-radius:12px;font-variant-numeric:tabular-nums}

270	.steps{display:grid;gap:13px}

271	.step{display:flex;gap:15px;align-items:center;padding:15px;border-radius:16px;background:var(--base);box-shadow:var(--o1)}

272	.step.active{box-shadow:var(--o1), 0 0 0 2px var(--accentSoft)}

273	.step.done{opacity:.75}

274	.step .num{width:34px;height:34px;flex:none;border-radius:50%;display:grid;place-items:center;font-weight:800;

275	  background:var(--well);box-shadow:var(--i1);color:var(--ink2)}

276	.step.active .num,.step.done .num{background:var(--accent);color:var(--onAccent);box-shadow:var(--o1)}

277	.step .txt{flex:1;min-width:0}

278	.step .txt b{display:block;font-size:14.5px}

279	.step .txt span{font-size:12.5px;color:var(--ink2)}

280

281	table.sim{width:100%;border-collapse:separate;border-spacing:0 11px}

282	table.sim th{text-align:left;font-size:10.5px;letter-spacing:.8px;color:var(--ink2);padding:0 12px;text-transform:uppercase}

283	table.sim td{padding:13px 12px;background:var(--base);vertical-align:middle}

284	table.sim td:first-child{border-radius:14px 0 0 14px;font-weight:700}

285	table.sim td:last-child{border-radius:0 14px 14px 0}

286	table.sim tr{box-shadow:var(--o1)}

287	.simname{display:flex;align-items:center;gap:11px}

288	.modebtn{min-width:96px;text-align:center;display:flex;align-items:center;justify-content:center;gap:7px}

289	.modebtn[data-mode="MANUAL"]{background:var(--sim);color:#fff;box-shadow:var(--o1)}

290	.simval{display:flex;gap:11px;align-items:center}

291	.simval input[type=number]{width:102px}

292	.simval span{font-size:12px;color:var(--ink2);font-weight:700}

293

294	.alerts{display:flex;flex-direction:column;gap:12px}

295	.alert{display:flex;gap:14px;align-items:center;padding:14px 16px;border-radius:14px;background:var(--base);box-shadow:var(--o1)}

296	.alert .av{width:40px;height:40px;flex:none;border-radius:12px;display:grid;place-items:center;background:var(--well);box-shadow:var(--i1)}

297	.alert .ttl{font-weight:800;font-size:14px}

298	.alert .meta{font-size:12px;color:var(--ink2)}

299	.empty{padding:34px;text-align:center;color:var(--ink2);font-weight:700}

300

301	.gate{position:fixed;inset:0;background:var(--tint);display:flex;align-items:center;justify-content:center;z-index:100;padding:20px;overflow:auto}

302	.gatehead{display:flex;align-items:center;gap:18px;margin:0 0 18px}

303	.gatehead .sblogo{width:92px;height:92px;flex:none}

304	.gatehead h1{margin:0 0 4px}

305	.gatehead p{margin:0}

306	@media(max-width:480px){.gatehead{flex-direction:column;text-align:center}}

307	.gatedemo{display:flex;align-items:center;gap:16px;margin-top:22px;padding:16px 18px;border-radius:18px;background:var(--well);box-shadow:var(--i1);flex-wrap:wrap}

308	.gatedemo .btn{display:inline-flex;align-items:center;gap:8px;flex:none}

309	.gatedemo .btn[aria-pressed="true"]{color:var(--warn)}

310	.gatedemo p{flex:1;min-width:200px;margin:0;font-size:13px;color:var(--ink2)}

311	.sndgrp{margin:0 0 14px}

312	.sndgrp h3{margin:0 0 8px;font-size:12px;letter-spacing:1px;text-transform:uppercase;color:var(--ink2)}

313	.sndgrp div{display:flex;flex-wrap:wrap;gap:8px}

314	.sndgrp .btn{font-size:12.5px;padding:8px 12px;display:inline-flex;align-items:center;gap:7px}

315	.sndgrp .btn .ico{width:14px;height:14px}

316	.sndgrp .btn.playing{color:var(--accent)}

317	.gatecard{max-width:660px;width:100%;padding:34px}

318	.gatecard h1{margin:0 0 7px;font-size:27px;letter-spacing:.4px}

319	.gatecard p{margin:0 0 24px;color:var(--ink2)}

320	.roles{display:grid;grid-template-columns:1fr 1fr;gap:18px}

321	@media(max-width:580px){.roles{grid-template-columns:1fr}}

322	.role{padding:22px;text-align:left;border-radius:18px;background:var(--base);box-shadow:var(--o1);transition:box-shadow .2s}

323	.role:hover{box-shadow:var(--o1), 0 0 0 2px var(--accentSoft)}

324	.role .ico{color:var(--accent);margin-bottom:10px}

325	.role h3{margin:0 0 8px;font-size:16px}

326	.role ul{margin:0;padding-left:19px;font-size:13px;color:var(--ink2);line-height:1.65}

327	.card[data-bmode="sim"]:after,.card[data-bmode="dual"]:after,.card[data-bmode="cal"]:after{

328	  content:"";position:absolute;inset:-4px;border-radius:24px;pointer-events:none;border:3px solid var(--simSoft)}

329	.card[data-bmode="cal"]:after{border-color:var(--calSoft)}

330	.card[data-bmode="dual"]:before{content:"";position:absolute;inset:-9px;border-radius:29px;

331	  pointer-events:none;border:3px solid var(--calSoft)}

332

333	.chips{display:grid;grid-template-columns:repeat(auto-fit,minmax(62px,1fr));gap:9px;margin-top:13px}

334	.chip{--cc:var(--ink2);justify-content:center;padding:9px 7px;font-size:10.5px;border-radius:12px;

335	  min-width:0;letter-spacing:.3px}

336	.chip{white-space:normal;text-align:center;min-height:44px;line-height:1.2}

337	.chip>span{overflow:visible;white-space:normal;word-break:keep-all}

338	.chip .ico{width:15px;height:15px;flex:none}

339	.chip.on{color:var(--cc);box-shadow:var(--o1)}

340	.chip.on{box-shadow:var(--o1), 0 0 0 2px color-mix(in srgb,var(--cc) 30%,transparent)}

341	.chip.on .ico{color:var(--cc)}

342	.chip.off{opacity:.38;color:var(--ink2)}

343	.actions{display:grid;grid-template-columns:repeat(auto-fit,minmax(102px,1fr));gap:11px}

344	.actions .btn{--cc:var(--ink2);min-width:0;padding:11px 10px;font-size:13px}

345	.actions .btn span{overflow:hidden;text-overflow:ellipsis;white-space:nowrap}

346	.btn{--cc:var(--ink2)}

347	.btn.tint,.btn.tint .ico{color:var(--cc)}

348	.btn.tint{box-shadow:var(--o1)}

349	.btn.tint{box-shadow:var(--o1), 0 0 0 2px color-mix(in srgb,var(--cc) 30%,transparent)}

350	.btn.tint:hover{color:var(--cc)}

351	.btn.tint:hover{box-shadow:var(--o1), 0 0 0 4px color-mix(in srgb,var(--cc) 34%,transparent)}

352	.btn.solid{background:var(--cc);color:var(--onAccent);box-shadow:var(--o1)}

353	.btn.solid .ico{color:var(--onAccent)}

354	.btn.solid:hover{color:var(--onAccent);filter:brightness(1.07)}

355	.btn[disabled],.btn.tint[disabled],.btn.solid[disabled]{color:var(--ink2);background:var(--base);

356	  box-shadow:var(--o1);opacity:.4}

357	.btn[disabled] .ico{color:var(--ink2)}

358

359	.field label{display:flex;align-items:center;gap:7px;color:var(--lc,var(--ink2))}

360	.field label .ico{width:15px;height:15px;color:var(--lc,var(--ink2))}

361	.twoway{display:inline-flex;gap:5px;padding:5px;border-radius:13px;background:var(--well);box-shadow:var(--i1)}

362	.tw{display:flex;align-items:center;gap:7px;padding:8px 13px;border-radius:9px;font-size:12px;

363	  font-weight:700;color:var(--ink2);transition:background .25s,color .25s,box-shadow .25s}

364	.tw .ico{width:15px;height:15px}

365	.tw[aria-pressed="true"]{background:var(--cc);color:var(--onAccent);box-shadow:var(--o1)}

366	.tw[aria-pressed="true"] .ico{color:var(--onAccent)}

367	.tw[disabled]{opacity:.42;cursor:not-allowed}

368	.modebtn[data-mode="AUTO"]{--cc:var(--ok)}

369	.modebtn[data-mode="MANUAL"]{background:var(--sim);color:#fff;box-shadow:var(--o1)}

370	.modebtn[data-mode="MANUAL"] .ico{color:#fff}

371

372	.sensor{--cc:var(--ink2)}

373	.sensor[data-s="temp"]{--cc:var(--c-temp)}

374	.sensor[data-s="humid"]{--cc:var(--c-humid)}

375	.sensor[data-s="weight"]{--cc:var(--c-weight)}

376	.sensor[data-s="gas"]{--cc:var(--c-gas)}

377	.sensor .ico{color:var(--cc)}

378	.sensor .v{color:var(--lvc,var(--ink));transition:color .5s ease}

379	.sensor[data-lv="cool"]{--lvc:var(--lv-cool)}

380	.sensor[data-lv="low"]{--lvc:var(--lv-low)}

381	.sensor[data-lv="mid"]{--lvc:var(--lv-mid)}

382	.sensor[data-lv="high"]{--lvc:var(--lv-high)}

383	.sensor.alarm{--cc:#fff;--lvc:#fff}

384	.sensor .k{font-size:10px}

385

386	.sect{--cc:var(--ink2)}

387	.sect[data-sec="range"]{--cc:var(--cal)}

388	.sect[data-sec="limits"]{--cc:var(--bad)}

389	.sect[data-sec="lid"]{--cc:var(--sim)}

390	.sect[data-sec="weight"]{--cc:var(--c-weight)}

391	.sect[data-sec="air"]{--cc:var(--c-gas)}

392	.sect[data-sec="touch"]{--cc:var(--cal)}

393	.sect{margin-top:12px;border-radius:16px;background:var(--base);box-shadow:var(--o1)}

394	.dhd{margin:6px 0 8px;font-size:12px;letter-spacing:1px;text-transform:uppercase;color:var(--ink2)}

395	#demoBody .seg{margin-bottom:12px}

396	.shead{all:unset;box-sizing:border-box;width:100%;min-height:54px;padding:0 16px;display:flex;align-items:center;gap:10px;cursor:pointer;border-radius:16px}

397	.shead:focus-visible{outline:2px solid var(--cc);outline-offset:2px}

398	.shead:before{content:"";width:4px;height:15px;border-radius:2px;background:var(--cc);flex:none}

399	.shead .ico{width:18px;height:18px;color:var(--cc);flex:none}

400	.shead .stitle{font-size:12px;font-weight:800;letter-spacing:1.2px;text-transform:uppercase;color:var(--cc);white-space:nowrap}

401	.shead .ssum{flex:1;text-align:right;font-size:12.5px;font-weight:700;color:var(--ink2);white-space:nowrap;overflow:hidden;text-overflow:ellipsis}

402	.shead .schev{color:var(--ink2);transition:transform .25s}

403	.sect.open .schev{transform:rotate(180deg)}

404	.sect .sbody{display:none;padding:2px 16px 16px}

405	.sect.open .sbody{display:block}

406	.sect .sbody>.fields:first-child{margin-top:4px}

407	.sect input:focus{box-shadow:var(--i1), 0 0 0 2px var(--accentSoft)}

408	.sect input:focus{box-shadow:var(--i1), 0 0 0 2px color-mix(in srgb,var(--cc) 38%,transparent)}

409	.simname{--cc:var(--ink2)}

410	.simname .ico{color:var(--cc)}

411	table.sim tr[data-manual="1"] td:first-child{box-shadow:inset 4px 0 0 var(--cc)}

412

413	.ttf{border-left:5px solid var(--ink2)}

414	.ttf[data-lv="near"]{border-left-color:var(--bad)}

415	.ttf[data-lv="mid"]{border-left-color:var(--warn)}

416	.ttf[data-lv="far"]{border-left-color:var(--ok)}

417	.ttf .when{font-variant-numeric:tabular-nums;font-weight:800;margin-left:auto}

418	.ttf .lvtag{font-size:10px;font-weight:800;letter-spacing:.5px;padding:4px 10px;border-radius:20px;color:#fff;min-width:56px;text-align:center;

419	  box-shadow:-2px -2px 4px rgba(255,255,255,.18), 2px 2px 4px rgba(0,0,0,.22)}

420

421	.alert{border-left:5px solid var(--ink2)}

422	.alert[data-lv="warning"]{border-left-color:var(--warn)}

423	.alert[data-lv="info"]{border-left-color:var(--sim)}

424	.alert[data-lv="success"]{border-left-color:var(--ok)}

425	.alert[data-lv="critical"]{background:var(--bad);color:#fff;border:2px solid var(--ink);

426	  border-radius:8px;box-shadow:5px 5px 0 var(--ink)}

427	.alert[data-lv="critical"] .av{background:rgba(255,255,255,.17);box-shadow:none;color:#fff}

428	.alert[data-lv="critical"] .meta{color:rgba(255,255,255,.88)}

429	.alert .lvtag{margin-left:auto;font-size:10px;font-weight:800;letter-spacing:.6px;padding:4px 10px;

430	  border-radius:20px;color:#fff;white-space:nowrap;

431	  box-shadow:-2px -2px 4px rgba(255,255,255,.18), 2px 2px 4px rgba(0,0,0,.22)}

432	.alert[data-lv="critical"] .lvtag{background:rgba(0,0,0,.28)}

433

434	.addrow{justify-content:flex-end;margin:0 0 16px}

435	.addpick{display:flex;align-items:center;gap:4px;padding:4px;border-radius:999px;background:var(--well);box-shadow:var(--i1)}

436	.addpick button{width:28px;height:28px;border-radius:50%;font-weight:800;font-size:16px;line-height:1;

437	  color:var(--ink2);background:var(--base);box-shadow:var(--o1)}

438	.addpick button:hover{color:var(--accent)}

439	.addpick button[disabled]{opacity:.35;cursor:not-allowed}

440	.addpick span{min-width:22px;text-align:center;font-weight:800;font-variant-numeric:tabular-nums}

441

442	@media(max-width:600px){

443	  .wrap{padding:16px 12px 56px}

444	  header.top{padding:10px 12px}

445	  .brand{font-size:16px}

446	  .link,.modechip{font-size:11px;padding:7px 11px}

447	  .tabs{gap:7px}

448	  .tab{padding:9px 13px;font-size:13px;gap:7px}

449	  .card{padding:16px}

450	  .cardtop{gap:13px}

451	  .hero{width:62px;height:62px}

452	  .hero .ico.lg{width:46px;height:46px}

453	  .binname{font-size:17px}

454	  .fillnum{font-size:34px}

455	  .fillnum span{font-size:14px}

456	  .chips{grid-template-columns:repeat(auto-fit,minmax(58px,1fr));gap:8px}

457	  .sensors{grid-template-columns:repeat(2,1fr)}

458	  .actions{grid-template-columns:repeat(2,1fr)}

459	  .panel,.chartbox,.gatecard{padding:17px}

460	  .fields{grid-template-columns:1fr}

461	  .calbar{padding:15px;gap:13px}

462	  .switch.big{width:100%;justify-content:center}

463	  table.sim td{padding:11px 9px}

464	  .simval input[type=number]{width:84px}

465	  .addrow{justify-content:space-between}

466	  .prio .row,.ttf{padding:11px 13px;gap:10px}

467	  .alert{padding:12px 13px;gap:11px}

468	  .alert .lvtag{display:none}

469	}

470	@media(max-width:380px){

471	  .chips{grid-template-columns:repeat(auto-fit,minmax(58px,1fr))}

472	  .actions{grid-template-columns:1fr}

473	  .tab span{display:none}

474	  .tab{padding:10px 14px}

475	}

476

477	/* --- v3 additions --- */

478	body{background:var(--tint)}

479	body{background:

480	  radial-gradient(900px 520px at 12% -8%, color-mix(in srgb,var(--accent) 7%,transparent), transparent 70%),

481	  radial-gradient(760px 460px at 105% 4%, color-mix(in srgb,var(--accent) 5%,transparent), transparent 72%),

482	  var(--tint);background-attachment:fixed}

483	.nm,.panel,.card,.chartbox{background-image:linear-gradient(160deg,color-mix(in srgb,var(--shL) 42%,transparent),transparent 58%)}

484

485	.brand{cursor:default;position:relative;padding:4px 2px}

486	.brand .btxt{background:linear-gradient(95deg,var(--ink) 0%,var(--ink) 40%,var(--accent) 55%,var(--ink) 72%,var(--ink) 100%);

487	  background-size:280% 100%;background-position:100% 0;-webkit-background-clip:text;background-clip:text;color:transparent;

488	  transition:background-position 1.1s ease,letter-spacing .35s ease}

489	.brand:hover .btxt{background-position:0 0;letter-spacing:1.3px}

490	.brand .ico{transition:transform .5s cubic-bezier(.25,1.5,.45,1)}

491	.brand:hover .ico{transform:translateY(-2px) rotate(-8deg)}

492	.brand:hover .ico .lidflap{transform:rotate(-34deg)}

493

494	.sunmoon .sunside,.sunmoon .moonside{transition:opacity .45s ease,transform .5s cubic-bezier(.2,.9,.3,1.3)}

495	.sunmoon.is-light .sunside{opacity:.3;transform:scale(.82)}

496	.sunmoon.is-light .moonside{opacity:1;color:var(--accent)}

497	.sunmoon.is-dark .moonside{opacity:.3;transform:scale(.82)}

498	.sunmoon.is-dark .sunside{opacity:1;color:var(--accent)}

499	#theme{width:42px;height:42px}

500	#theme .ico{width:21px;height:21px;color:var(--accent)}

501

502	.statuspill{--cc:var(--warn);display:flex;align-items:center;gap:8px;padding:8px 15px;border-radius:999px;

503	  font-size:12px;font-weight:800;letter-spacing:.4px;color:var(--cc);background:var(--base);box-shadow:var(--o1)}

504	.statuspill{box-shadow:var(--o1), 0 0 0 2px color-mix(in srgb,var(--cc) 34%,transparent)}

505	.statuspill .ico{width:16px;height:16px;color:var(--cc)}

506	.statuspill[aria-expanded="true"]{box-shadow:var(--i1)}

507	.bannerpop{position:absolute;left:16px;right:16px;top:100%;margin:10px auto 0;max-width:1280px;z-index:60;

508	  display:flex;flex-direction:column;gap:10px;padding:14px;border-radius:18px;background:var(--base);

509	  box-shadow:var(--o2), 0 18px 40px -22px rgba(0,0,0,.5);max-height:62vh;overflow-y:auto}

510	.bannerpop .banner{margin:0}

511

512	.sechead{display:flex;align-items:center;gap:14px;flex-wrap:wrap;margin-bottom:14px;min-height:46px;position:relative;z-index:2;

513	  padding:6px 4px;margin-left:-4px;margin-right:-4px;border-radius:14px}

514	.panel .sechead{background:var(--base)}

515	section > .sechead{background:var(--tint)}

516	.sectitle,.ph{display:flex;align-items:center;gap:9px;margin:0;position:relative;z-index:1;

517	  font-size:12px;letter-spacing:1.4px;font-weight:800;text-transform:uppercase;color:var(--cc,var(--ink2))}

518	.sectitle .ico,.ph .ico{width:17px;height:17px;color:var(--cc,var(--ink2))}

519	.panel>.ph,.chartbox>.ph{margin:0 0 18px}

520	.sechead .sectitle,.sechead .ph{margin:0}

521	.sechead .addrow{margin:0 0 0 auto}

522	.alertctl{margin-left:auto;display:flex;gap:9px;align-items:center;flex-wrap:wrap}

523	.alertctl select{width:auto;padding:7px 11px;font-size:12.5px;font-weight:700}

524

525	.scrollpane{overflow:visible}

526	.alertpane{max-height:min(58vh,560px);overflow-y:auto;padding:26px;margin:-26px}

527	@media(min-width:900px) and (min-height:620px){

528	  #binsPane{max-height:calc(100vh - 430px);min-height:500px;overflow-y:auto;padding:26px;margin:-26px}

529	}

530	.scrollpane::-webkit-scrollbar,.bannerpop::-webkit-scrollbar{width:11px}

531	.scrollpane::-webkit-scrollbar-track,.bannerpop::-webkit-scrollbar-track{background:transparent}

532	.scrollpane::-webkit-scrollbar-thumb,.bannerpop::-webkit-scrollbar-thumb{

533	  background:var(--base);border:3px solid transparent;background-clip:content-box;border-radius:999px;

534	  box-shadow:inset 0 0 0 1px color-mix(in srgb,var(--ink2) 30%,transparent)}

535	.scrollpane{scrollbar-width:thin;scrollbar-color:var(--shD) transparent}

536

537	.tab{--cc:var(--ink2)}

538	.tab[aria-selected="true"]{color:var(--cc)}

539	.tab[aria-selected="true"]{box-shadow:var(--i1), 0 0 0 2px color-mix(in srgb,var(--cc) 26%,transparent)}

540	.tab:hover{color:var(--cc)}

541	.tab .ico{color:inherit}

542

543	.chip{font-size:clamp(8.5px,2.1vw,10.5px);padding:8px 5px;gap:5px}

544	.chip .ico{width:14px;height:14px}

545	.actions .btn{white-space:normal;min-height:52px;line-height:1.15;text-align:center}

546	.actions .btn span{white-space:normal;overflow:visible;font-size:clamp(10.5px,2.3vw,13px)}

547	.btn .sub{display:block;font-size:9.5px;font-weight:700;opacity:.72;letter-spacing:.3px;margin-top:2px}

548

549	.gearbtn{width:46px;height:46px;border-radius:14px;display:flex;flex-direction:column;align-items:center;

550	  justify-content:center;gap:1px;background:var(--base);box-shadow:var(--o1);color:var(--ink2)}

551	.gearbtn .ico{width:16px;height:16px}

552	.gearbtn em{font-style:normal;font-size:7.5px;font-weight:800;letter-spacing:.4px}

553	.gearbtn:hover,.gearbtn[aria-expanded="true"]{color:var(--accent)}

554	.gearbtn[aria-expanded="true"]{box-shadow:var(--i1)}

555

556	.binbtn{display:inline-flex;align-items:center;gap:8px}

557	.binbtn .ico{width:15px;height:15px;color:var(--cc,var(--ink2))}

558	.binbtn[data-sim="1"]{--cc:var(--sim)}

559	.binbtn[data-sim="1"]{box-shadow:var(--o1), 0 0 0 2px color-mix(in srgb,var(--sim) 32%,transparent)}

560	.binbtn[aria-pressed="true"]{color:var(--cc,var(--accent))}

561	.binbtn[aria-pressed="true"]{box-shadow:var(--i1), 0 0 0 2px color-mix(in srgb,var(--cc,var(--accent)) 30%,transparent)}

562

563	.twoway.multi{flex-wrap:wrap}

564	.tw .dot{width:10px;height:10px;border-radius:50%;display:block;box-shadow:inset 0 0 0 1px rgba(0,0,0,.18)}

565	table.sim th{display:table-cell}

566	table.sim th .thh{display:flex;align-items:center;gap:7px;color:var(--cc,var(--ink2))}

567	table.sim th .ico{width:14px;height:14px}

568	.alert .del{width:30px;height:30px;flex:none;border-radius:9px;display:grid;place-items:center;color:var(--ink2);

569	  background:var(--base);box-shadow:var(--o1);opacity:0;transition:opacity .25s,color .25s}

570	.alert:hover .del,.alert:focus-within .del{opacity:1}

571	.alert .del:hover{color:var(--bad)}

572	.alert[data-lv="critical"] .del{background:rgba(255,255,255,.18);color:#fff;box-shadow:none}

573	@media(hover:none){.alert .del{opacity:1}}

574

575	.chip{flex-direction:column;gap:4px;padding:9px 5px;min-height:62px;overflow:hidden;justify-content:center}

576	.chip>span{display:block;width:100%;overflow-wrap:break-word;word-break:normal;line-height:1.15}

577	.chip .ico{width:16px;height:16px}

578	@media(max-width:600px){.chip{min-height:58px}}

579

580	.btstate{margin-left:auto;display:flex;align-items:center;gap:8px;font-size:12px;font-weight:800;

581	  letter-spacing:.4px;padding:7px 14px;border-radius:999px;color:var(--ink2);background:var(--base);box-shadow:var(--o1)}

582	.btstate[data-on="1"]{color:var(--c-bt)}

583	.btstate[data-on="1"]{box-shadow:var(--o1), 0 0 0 2px color-mix(in srgb,var(--c-bt) 32%,transparent)}

584	.btsteps{display:grid;grid-template-columns:repeat(4,1fr);gap:10px}

585	@media(max-width:600px){.btsteps{grid-template-columns:repeat(2,1fr)}}

586	.btstep{display:flex;flex-direction:column;align-items:center;gap:7px;text-align:center;padding:14px 8px;

587	  border-radius:14px;background:var(--well);box-shadow:var(--i1);color:var(--ink2);transition:all .35s ease}

588	.btstep .ico{width:19px;height:19px}

589	.btstep b{font-size:11px;font-weight:800;letter-spacing:.4px;line-height:1.2}

590	.btstep.on{color:var(--c-bt);background:var(--base);box-shadow:var(--o1)}

591	.btstep.on{box-shadow:var(--o1), 0 0 0 2px color-mix(in srgb,var(--c-bt) 30%,transparent)}

592	.btstep.done{color:var(--ok)}

593	.btresult{margin-top:18px;padding:16px;border-radius:14px;background:var(--well);box-shadow:var(--i1);

594	  display:flex;align-items:center;gap:14px;flex-wrap:wrap}

595	.btresult .ip{font-weight:800;font-size:19px;font-variant-numeric:tabular-nums;color:var(--ok)}

596

597	.hide{display:none !important}

598

599	/* --- v6: collect PIN card for the collector --- */

600	/* Panel button rows are a flex row, not the card grid, so hiding the write

601	   buttons for a collector leaves RELOAD at its own width instead of stretched. */

602	.panel>.actions{display:flex;flex-wrap:wrap}

603	.panel>.actions .btn{flex:0 0 auto}

604

605	.pincard{display:flex;align-items:center;gap:16px;flex-wrap:wrap;padding:18px 20px;

606	  border-radius:16px;margin:0 0 20px}

607	.pincard .pinico{width:46px;height:46px;flex:none;border-radius:50%;display:grid;place-items:center;

608	  background:var(--base);box-shadow:var(--o1);color:var(--cal)}

609	.pincard .pinico .ico{width:22px;height:22px;display:block}

610	.pincard .pintxt{flex:1;min-width:200px}

611	.pincard .pintxt b{display:block;font-size:15px}

612	.pincard .pintxt span{font-size:12.5px;color:var(--ink2)}

613	.pinval{font-size:34px;font-weight:800;letter-spacing:7px;line-height:1.1;color:var(--cal);

614	  padding:2px 0 2px 10px;font-variant-numeric:tabular-nums}

615	.pinval.masked{letter-spacing:4px}

616	@media(max-width:600px){.pincard{padding:15px}.pinval{font-size:28px;letter-spacing:5px}}

617

618	/* --- v5: uniform status chips, always 3 across and 2 down --- */

619	.chips{display:grid;grid-template-columns:repeat(3,minmax(0,1fr));gap:7px;margin-top:14px}

620	.chip{flex-direction:row;align-items:center;justify-content:center;gap:5px;

621	  min-height:0;height:28px;padding:0 8px;border-radius:9px;

622	  font-size:10.5px;letter-spacing:.3px;line-height:1;white-space:nowrap;overflow:hidden}

623	.chip>span{display:inline;width:auto;white-space:nowrap;overflow-wrap:normal;line-height:1}

624	.chip .ico{width:12px;height:12px;flex:none}

625	@media(max-width:420px){.chip{padding:0 6px;gap:4px;font-size:10px}.chip .ico{width:11px;height:11px}}

626

627	/* --- v3: read / unread alerts --- */

628	.alert .rd{width:26px;height:26px;flex:none;border-radius:50%;display:grid;place-items:center;

629	  padding:0;background:none;box-shadow:none;color:var(--cc,var(--ink2))}

630	.alert .rd i{width:11px;height:11px;border-radius:50%;background:currentColor;

631	  box-shadow:0 0 0 3px color-mix(in srgb,currentColor 24%,transparent);transition:transform .25s ease,box-shadow .25s ease,background .25s ease}

632	.alert[data-read="1"] .rd i{background:transparent;box-shadow:inset 0 0 0 2px currentColor;opacity:.5}

633	.alert[data-read="1"]{opacity:.66}

634	.alert[data-read="1"] .ttl{font-weight:700}

635	.alert[data-lv="critical"] .rd{color:#fff}

636	.alert .rd:hover i{transform:scale(1.3)}

637	.unreadpip{font-style:normal;font-size:10px;font-weight:800;letter-spacing:.5px;padding:3px 8px;

638	  border-radius:5px;background:var(--c-tamper);color:#fff}

639	.popsec{font-size:10.5px;font-weight:800;letter-spacing:1.1px;text-transform:uppercase;

640	  color:var(--ink2);padding:4px 4px 0}

641	.popact{display:flex;gap:9px;flex-wrap:wrap;padding:0 0 4px}

642	.popact .btn{display:inline-flex;align-items:center;gap:8px}

643

644	/* --- motion layer --- */

645	.btn,.tab,.iconbtn,.role,.addmini,.chip,.prio .row,.ttf,.alert,.step,.card{position:relative}

646	.btn,.tab,.iconbtn,.role,.addmini{overflow:hidden}

647	.ripple{position:absolute;border-radius:50%;background:currentColor;opacity:.16;pointer-events:none;

648	  transform:scale(0);animation:ripple .62s cubic-bezier(.2,.8,.3,1) forwards}

649	.halo{opacity:0;transform-origin:12px 17.2px;transition:opacity .45s ease}

650	.sensor.alarm .halo{opacity:.38;animation:breathe 2.6s ease-in-out infinite}

651	.sensor.alarm .vap{animation-duration:1.15s}

652

653	@keyframes ripple{to{transform:scale(1);opacity:0}}

654	@keyframes riseIn{from{opacity:0;transform:translateY(22px) scale(.97)}to{opacity:1;transform:none}}

655	@keyframes slideDown{from{opacity:0;transform:translateY(-14px)}to{opacity:1;transform:none}}

656	@keyframes slideUp{from{opacity:0;transform:translateY(20px)}to{opacity:1;transform:none}}

657	@keyframes fadeIn{from{opacity:0;transform:translateY(8px)}to{opacity:1;transform:none}}

658	@keyframes expand{from{opacity:0;transform:scaleY(.8);transform-origin:top}to{opacity:1;transform:none}}

659	@keyframes pop{0%{transform:scale(1)}35%{transform:scale(1.14)}100%{transform:scale(1)}}

660	@keyframes haze{0%,100%{box-shadow:var(--o2), 0 0 0 2px var(--accentSoft)}

661	  50%{box-shadow:var(--o2), 0 0 0 7px var(--accentSoft)}}

662	@keyframes sheen{from{transform:translateX(-120%)}to{transform:translateX(320%)}}

663	@keyframes stepGlow{0%,100%{box-shadow:var(--o1), 0 0 0 2px var(--accentSoft)}

664	  50%{box-shadow:var(--o1), 0 0 0 6px var(--accentSoft)}}

665

666	@media(prefers-reduced-motion:no-preference){

667	  .card{transition:transform .38s cubic-bezier(.2,.8,.3,1),box-shadow .38s ease,background .6s ease}

668	  .card:hover{transform:translateY(-5px);

669	    box-shadow:-15px -15px 30px var(--shL), 15px 15px 30px var(--shD), 0 0 0 2px var(--accentSoft)}

670	  .card.urgent{animation:haze 2.6s ease-in-out infinite}

671	  .hero .ico{transition:transform .45s cubic-bezier(.2,.9,.3,1.3)}

672	  .card:hover .hero .ico{transform:scale(1.08) rotate(-3deg)}

673	  .card:hover .fillnum{transform:scale(1.04)}

674	  .fillnum{transition:transform .35s cubic-bezier(.2,.9,.3,1.3)}

675

676	  .sensor{transition:transform .26s cubic-bezier(.2,.8,.3,1),box-shadow .26s ease}

677	  .sensor.nm-in:hover{transform:translateY(-4px);box-shadow:var(--o1)}

678	  .sensor.nm-in:hover .ico{color:var(--accent)}

679	  .sensor .ico{transition:color .3s ease,transform .3s ease}

680	  .sensor.nm-in:hover .ico{transform:scale(1.12)}

681

682	  .btn .ico,.tab .ico,.iconbtn .ico,.chip .ico{transition:transform .32s cubic-bezier(.2,.9,.3,1.3)}

683	  .btn:hover .ico{transform:scale(1.18)}

684	  .btn.danger:hover .ico{transform:scale(1.18) rotate(-8deg)}

685	  .tab:hover .ico{transform:translateY(-3px)}

686	  .iconbtn:hover .ico{transform:rotate(60deg)}

687	  .chip:hover{transform:translateY(-2px)}

688	  .chip{transition:transform .22s ease}

689

690	  .addmini{transition:transform .3s ease,box-shadow .3s ease,color .25s}

691	  .addmini:hover{transform:translateY(-2px);box-shadow:var(--o2)}

692	  .addmini .ico{transition:transform .5s cubic-bezier(.25,1.5,.45,1)}

693	  .addmini:hover .ico{transform:rotate(90deg)}

694

695	  .prio .row,.ttf,.alert{transition:transform .28s cubic-bezier(.2,.8,.3,1),box-shadow .28s}

696	  .prio .row:hover,.ttf:hover,.alert:hover{transform:translateX(6px);box-shadow:var(--o2)}

697	  .prio .row:hover .n{background:var(--accent);color:var(--onAccent);box-shadow:var(--o1)}

698	  .prio .n{transition:background .3s,color .3s,box-shadow .3s}

699	  .role{transition:transform .32s cubic-bezier(.2,.8,.3,1),box-shadow .32s}

700	  .role:hover{transform:translateY(-6px)}

701	  .role:hover .ico{transform:scale(1.1) rotate(6deg)}

702	  .role .ico{transition:transform .45s cubic-bezier(.2,.9,.3,1.3)}

703

704	  .step{transition:transform .3s ease,box-shadow .3s ease}

705	  .step:hover{transform:translateX(5px)}

706	  .step.active{animation:stepGlow 2.8s ease-in-out infinite}

707	  .step .num{transition:background .35s,color .35s,transform .35s cubic-bezier(.2,.9,.3,1.4)}

708	  .step.done .num{transform:scale(1.06)}

709

710	  .enter{animation:riseIn .55s cubic-bezier(.2,.8,.3,1) both}

711	  .banner{animation:slideDown .45s cubic-bezier(.2,.8,.3,1) both}

712	  .toast{animation:slideUp .4s cubic-bezier(.2,.8,.3,1) both}

713	  section[data-view]:not(.hide){animation:fadeIn .38s ease both}

714	  .editrow:not(.hide){animation:expand .36s cubic-bezier(.2,.8,.3,1) both}

715	  .modechip.flash,.statuspill.flash{animation:pop .65s cubic-bezier(.2,.9,.3,1.3)}

716	  table.sim tr{transition:transform .26s ease,box-shadow .26s}

717	  table.sim tr:hover{transform:translateX(4px);box-shadow:var(--o2)}

718	  .link:hover,.modechip:hover{transform:translateY(-2px)}

719	  .link,.modechip{transition:transform .25s ease,background .4s ease}

720

721	  .bar i:after{content:"";position:absolute;inset:0;border-radius:11px;overflow:hidden}

722	  .urgent .bar i:before{content:"";position:absolute;top:0;bottom:0;width:38%;border-radius:11px;

723	    background:linear-gradient(90deg,transparent,rgba(255,255,255,.45),transparent);

724	    animation:sheen 2.1s ease-in-out infinite}

725	  .bar i{overflow:hidden}

726	}

727	.toast{position:fixed;left:16px;right:16px;bottom:18px;max-width:560px;margin:0 auto;z-index:90}

728

729	/* --- v9: soft button glows no longer collide with the hard banner shadows --- */

730	.bannerpop{padding:16px 24px 18px 16px;gap:12px}

731	.bannerpop .banner{margin:0 0 4px}

732	.popact{padding:12px 10px 12px 8px;gap:16px;margin-top:2px}

733	.popact .btn.tint{box-shadow:3px 3px 8px var(--shD),-3px -3px 8px var(--shL),

734	  0 0 0 2px color-mix(in srgb,var(--cc) 30%,transparent)}

735	.popact .btn.tint:hover{box-shadow:3px 3px 8px var(--shD),-3px -3px 8px var(--shL),

736	  0 0 0 4px color-mix(in srgb,var(--cc) 34%,transparent)}

737	.popact .btn.tint:active{box-shadow:var(--i1),0 0 0 2px color-mix(in srgb,var(--cc) 30%,transparent)}

738	/* The alert list used to clip its own glow into a white line along the top and left. */

739	.alertpane{padding:26px;margin:-26px}

740	.alertpane .alert:hover{box-shadow:var(--o1)}

741	.alert[data-lv="critical"]:hover{box-shadow:7px 7px 0 var(--ink)}

742	.alertctl .btn.tint,.sechead .btn.tint{box-shadow:3px 3px 8px var(--shD),-3px -3px 8px var(--shL),

743	  0 0 0 2px color-mix(in srgb,var(--cc) 30%,transparent)}

744	.alertctl .btn.tint:hover{box-shadow:3px 3px 8px var(--shD),-3px -3px 8px var(--shL),

745	  0 0 0 4px color-mix(in srgb,var(--cc) 34%,transparent)}

746	.alertctl .btn.tint:active{box-shadow:var(--i1),0 0 0 2px color-mix(in srgb,var(--cc) 30%,transparent)}

747	.alertctl .btn.tint[disabled]{box-shadow:3px 3px 8px var(--shD),-3px -3px 8px var(--shL)}

748	.legend .now{font-weight:800;color:var(--ink)}

749	@media(min-width:900px){.field.span2{grid-column:span 2}}

750	.withbtn input{min-width:0;flex:1}

751

752	/* scroll fade masks — soften the hard cutoff at scroll edges */

753	.alertpane,.scrollpane{-webkit-mask-image:linear-gradient(to bottom,transparent 0,#000 26px,#000 calc(100% - 26px),transparent 100%);

754	  mask-image:linear-gradient(to bottom,transparent 0,#000 26px,#000 calc(100% - 26px),transparent 100%)}

755	.alertpane.at-top,.scrollpane.at-top{-webkit-mask-image:linear-gradient(to bottom,#000 0,#000 calc(100% - 26px),transparent 100%);

756	  mask-image:linear-gradient(to bottom,#000 0,#000 calc(100% - 26px),transparent 100%)}

757	.alertpane.at-bot,.scrollpane.at-bot{-webkit-mask-image:linear-gradient(to bottom,transparent 0,#000 26px,#000 100%);

758	  mask-image:linear-gradient(to bottom,transparent 0,#000 26px,#000 100%)}

759	.alertpane.at-top.at-bot,.scrollpane.at-top.at-bot{-webkit-mask-image:none;mask-image:none}

760

761	/* ---- v10 additions ---- */

762	.ovtools{display:flex;align-items:center;gap:10px;margin-bottom:12px;flex-wrap:wrap;position:relative;z-index:2;

763	  background:var(--tint);padding:8px 4px;margin-left:-4px;margin-right:-4px;border-radius:14px}

764	.filterchips{display:flex;gap:6px;flex-wrap:wrap}

765	.fchip{position:relative;width:38px;height:38px;border-radius:12px;background:var(--base);box-shadow:var(--o1);

766	  cursor:pointer;border:0;color:var(--ink2);display:flex;align-items:center;justify-content:center;

767	  transition:box-shadow .18s,color .18s,transform .18s}

768	.fchip svg{width:18px;height:18px;pointer-events:none;transition:transform .25s cubic-bezier(.2,.9,.3,1.3),color .18s}

769	.fchip:hover{box-shadow:var(--o2);transform:translateY(-2px)}

770	.fchip:hover svg{transform:scale(1.15);color:var(--fc,var(--accent))}

771	.fchip:active{box-shadow:var(--i1);transform:translateY(0)}

772	.fchip.on{box-shadow:var(--i1);color:var(--fc,var(--accent));background:color-mix(in srgb,var(--fc,var(--accent)) 14%,var(--base))}

773	.fchip.on svg{color:var(--fc,var(--accent))}

774	.fbadge{position:absolute;top:-3px;right:-3px;min-width:16px;height:16px;font-size:10px;font-weight:800;

775	  line-height:16px;text-align:center;padding:0 4px;border-radius:8px;color:#fff;

776	  background:var(--fc,var(--accent));box-shadow:0 1px 3px rgba(0,0,0,.25);pointer-events:none;display:none}

777	#viewToggle{position:relative;overflow:hidden}

778	#viewToggle svg{pointer-events:none}

779	.vico{transition:opacity .2s ease,transform .25s cubic-bezier(.2,.9,.3,1.3);position:absolute}

780	.vico.hide{opacity:0;transform:rotate(90deg) scale(.5)}

781	.vico:not(.hide){opacity:1;transform:rotate(0) scale(1)}

782

783	/* Compact tile grid */

784	#bins.gv{display:grid;grid-template-columns:repeat(auto-fill,minmax(150px,1fr));gap:12px}

785	#bins.gv .card{display:none}

786	#bins.gv .tile{display:flex}

787	#bins:not(.gv) .tile{display:none}

788	.tile{flex-direction:column;align-items:center;gap:4px;padding:12px 10px 10px;border-radius:16px;

789	  background:var(--base);box-shadow:var(--o1);cursor:pointer;position:relative;

790	  transition:box-shadow .18s,transform .15s;min-width:0;text-align:center}

791	.tile:hover{box-shadow:var(--o2);transform:translateY(-2px)}

792	.tile[data-bm="sim"]{background:var(--simSoft)}

793	.tile[data-bm="cal"]{background:var(--calSoft)}

794	.tile[data-bm="dual"]{background:linear-gradient(135deg,var(--simSoft) 50%,var(--calSoft) 50%)}

795	.tile .tname{font-size:12px;font-weight:800;overflow:hidden;text-overflow:ellipsis;white-space:nowrap;max-width:120px}

796	.tile .tplace{font-size:10px;color:var(--ink2);font-weight:600;overflow:hidden;text-overflow:ellipsis;white-space:nowrap;max-width:120px;margin:-2px 0 2px}

797	.tile .tfill{font-size:18px;font-weight:800;letter-spacing:-.3px}

798	.tile .ico.lg{width:44px;height:44px;overflow:hidden}

799	.tile .tbar{width:80%;height:6px;border-radius:3px;background:var(--well);box-shadow:var(--i1);overflow:hidden;margin:2px 0}

800	.tile .tbar i{display:block;height:100%;border-radius:3px;transition:width .4s ease}

801	.tile .tstatus{font-size:9px;font-weight:800;letter-spacing:.4px;color:var(--ink2);padding:2px 8px;border-radius:10px;background:var(--well);box-shadow:inset -1px -1px 2px var(--shL),inset 1px 1px 2px var(--shD)}

802	.tile .tstatus.alert-status{color:var(--bad)}

803	.tile .twarn{position:absolute;top:6px;right:8px;width:22px;height:22px;display:flex;align-items:center;justify-content:center;

804	  font-size:12px;font-weight:900;color:#fff;background:var(--bad);border-radius:50%;

805	  box-shadow:0 0 6px var(--bad), 0 0 12px rgba(217,48,37,.4);border:2px solid var(--base)}

806	@media(prefers-reduced-motion:no-preference){

807	  .tile .twarn{animation:warnPulse 1.8s ease-in-out infinite}

808	}

809	@keyframes warnPulse{0%,100%{box-shadow:0 0 6px var(--bad),0 0 12px rgba(217,48,37,.4)}50%{box-shadow:0 0 10px var(--bad),0 0 20px rgba(217,48,37,.55),0 0 30px rgba(217,48,37,.2)}}

810

811	/* Tile detail overlay */

812	.tile-overlay{position:fixed;inset:0;z-index:300;display:flex;align-items:center;justify-content:center;

813	  background:rgba(0,0,0,.45);backdrop-filter:blur(3px)}

814	.tile-overlay.hide{display:none}

815	.tile-detail{position:relative;max-width:480px;width:calc(100vw - 32px);

816	  border-radius:var(--r);box-shadow:var(--o2);background:var(--base);padding:14px 14px 10px}

817	.tile-detail .card{box-shadow:none;border-radius:14px;overflow:hidden}

818	.tile-detail .card::before,.tile-detail .card::after{display:none!important}

819	.tile-detail .sensors{grid-template-columns:repeat(4,1fr)}

820	@media(max-width:480px){.tile-detail .sensors{grid-template-columns:repeat(2,1fr)}}

821	.tile-nav{display:flex;gap:8px;justify-content:center;padding:8px 0 4px}

822	.tile-close{position:absolute;top:-6px;right:10px;z-index:10;width:32px;height:32px;

823	  background:var(--base);border:0;border-radius:50%;box-shadow:var(--o1);cursor:pointer;

824	  color:var(--ink2);display:flex;align-items:center;justify-content:center;

825	  transition:box-shadow .18s,transform .2s cubic-bezier(.2,.9,.3,1.3),color .15s}

826	.tile-close svg{width:16px;height:16px;pointer-events:none;transition:transform .25s cubic-bezier(.2,.9,.3,1.3)}

827	.tile-close:hover{box-shadow:var(--i1);color:var(--bad);transform:rotate(90deg)}

828	.tile-close:active{transform:rotate(90deg) scale(.9)}

829	.tile-detail .cardtop{padding-right:42px}

830

831	/* Settings overlay (SET button) */

832	.set-overlay{position:fixed;inset:0;z-index:350;display:flex;align-items:center;justify-content:center;

833	  background:rgba(0,0,0,.45);backdrop-filter:blur(3px)}

834	.set-overlay.hide{display:none}

835	.set-panel{position:relative;max-width:400px;width:calc(100vw - 32px);

836	  border-radius:var(--r);box-shadow:var(--o2);background:var(--base);padding:20px 18px 16px}

837	.set-panel h3{margin:0 0 14px;font-size:15px;font-weight:800;display:flex;align-items:center;gap:8px}

838	.set-panel h3 .ico{width:18px;height:18px;color:var(--accent)}

839	.set-panel .set-fields{display:grid;gap:11px;grid-template-columns:1fr 1fr}

840	.set-panel .field{grid-column:span 1}

841	@media(max-width:400px){.set-panel .set-fields{grid-template-columns:1fr}}

842	.set-panel .set-actions{display:flex;gap:10px;margin-top:16px;justify-content:flex-end}

843	.set-close{position:absolute;top:10px;right:10px;width:32px;height:32px;

844	  background:var(--base);border:0;border-radius:50%;box-shadow:var(--o1);cursor:pointer;

845	  color:var(--ink2);display:flex;align-items:center;justify-content:center;

846	  transition:box-shadow .18s,transform .2s cubic-bezier(.2,.9,.3,1.3),color .15s}

847	.set-close svg{width:14px;height:14px;pointer-events:none}

848	.set-close:hover{box-shadow:var(--i1);color:var(--bad);transform:rotate(90deg)}

849

850	/* Collection priority panel */

851	.collectnext-panel{padding:20px;margin-top:22px}

852	.collecthead{display:flex;align-items:center;justify-content:space-between;margin-bottom:12px}

853	.collecthead h2{margin:0}

854	.crow{display:flex;align-items:center;gap:8px;padding:8px 10px;border-radius:12px;

855	  background:var(--well);margin-bottom:6px;cursor:pointer;transition:box-shadow .15s}

856	.crow:hover{box-shadow:var(--o1)}

857	.crow .cn{font-size:11px;font-weight:800;color:var(--ink2);min-width:18px}

858	.crow .cnm{flex:1;font-weight:600;font-size:13px;overflow:hidden;text-overflow:ellipsis;white-space:nowrap}

859	.crow .cpc{font-size:12px;font-variant-numeric:tabular-nums;min-width:34px;text-align:right}

860	.crow .cwhen{font-size:11px;color:var(--ink2);min-width:52px;text-align:right}

861	.crow .lvtag{font-size:10px;font-weight:800;letter-spacing:.5px;padding:5px 14px;border-radius:20px;color:#fff;

862	  min-width:56px;text-align:center;display:inline-block;

863	  box-shadow:-2px -2px 4px rgba(255,255,255,.18), 2px 2px 4px rgba(0,0,0,.22)}

864	.coll-more{display:none}

865	.coll-show .coll-more{display:flex}

866	#collectToggle{flex-shrink:0}

867

868	/* Alert filter */

869	.alertctl{flex-wrap:wrap;gap:6px}

870	#alertFilters{display:flex;gap:5px;flex-wrap:wrap}

871

872	/* Adaptive picker */

873	.pickwrap{position:relative;display:inline-block}

874	.picklist{position:absolute;top:calc(100% + 6px);left:0;z-index:200;min-width:220px;max-height:320px;

875	  overflow-y:auto;border-radius:14px;background:var(--base);box-shadow:var(--o2);padding:8px}

876	.picklist.hide{display:none}

877	.picklist input{margin-bottom:8px;font-size:13px}

878	.picklist button{display:flex;width:100%;align-items:center;gap:8px;padding:7px 10px;border-radius:10px;

879	  background:none;border:0;cursor:pointer;font-size:13px;font-weight:600;text-align:left;color:var(--ink)}

880	.picklist button:hover,.picklist button[aria-pressed="true"]{background:var(--accentSoft);color:var(--accent)}

881	.picklist button .bcc{width:9px;height:9px;border-radius:50%;flex-shrink:0}

882

883	/* ---- v11 additions ---- */

884	/* Alert history tools (from Garbage-Guard): kind symbols, date range, scoped actions */

885	#alertKinds{display:flex;gap:7px;flex-wrap:wrap}

886	#alertKinds .fchip,#filterChips .fchip{width:36px;height:36px}

887	#filterChips{gap:7px}

888	#alertRange{display:flex;gap:6px;flex-wrap:wrap}

889	.alertscope{margin:14px 2px 12px;font-size:13px;font-weight:800;color:var(--ink)}

890	.alertscope small{display:block;font-weight:600;color:var(--ink2);margin-top:2px}

891	.aconfirm{display:flex;align-items:center;gap:10px;flex-wrap:wrap;margin:0 0 14px;padding:12px 14px;border:2px solid var(--ink);border-radius:8px;box-shadow:4px 4px 0 var(--ink);background:var(--bad);color:#fff;font-weight:700;font-size:13px}

892	.aconfirm span{flex:1;min-width:200px}

893	.aconfirm .btn{background:var(--base);color:var(--ink)}

894	.aconfirm.hide{display:none}

895	/* Cloud backup panel */

896	.cloudrows{display:flex;flex-direction:column;gap:8px;margin-top:14px}

897	.cloudrow{display:flex;align-items:center;gap:10px;font-size:13px;font-weight:700;padding:9px 12px;border-radius:12px;background:var(--well,var(--base));box-shadow:var(--i1)}

898	.cloudrow .dot{width:10px;height:10px;border-radius:50%;background:var(--cc);box-shadow:0 0 0 3px color-mix(in srgb,var(--cc) 22%,transparent);flex:none}

899	.cloudrow .cn{flex:1;min-width:0}

900	.cloudrow .cs{color:var(--cc);white-space:nowrap;font-size:12px}

901	.cloudmsg{margin-top:14px;padding:10px 13px;border:2px solid var(--ink);border-radius:8px;box-shadow:3px 3px 0 var(--ink);color:#fff;font-weight:700;font-size:13px}

902	.cloudmsg.ok{background:var(--ok)}.cloudmsg.bad{background:var(--bad)}.cloudmsg.hide{display:none}

903	#cloudState[data-on="1"]{color:var(--c-cloud);box-shadow:var(--o1),0 0 0 2px color-mix(in srgb,var(--c-cloud) 32%,transparent)}

904	.usebar{height:12px;border-radius:6px;background:var(--well,var(--base));box-shadow:var(--i1);overflow:hidden;margin-top:14px}

905	.usebar i{display:block;height:100%;width:3.1%;border-radius:6px;background:var(--c-cloud)}

906

907	/* v12: operator PIN */

908	.pinwrap{position:fixed;inset:0;z-index:120;display:flex;align-items:center;justify-content:center;padding:16px;

909	  background:color-mix(in srgb,var(--tint) 72%,transparent);backdrop-filter:blur(6px)}

910	.pinbox{width:100%;max-width:360px;max-height:100%;overflow:auto;padding:26px 22px 20px;text-align:center}

911	.pinbox h3{margin:8px 0 6px;font-size:18px}

912	.pinbox p{margin:0;color:var(--ink2);font-size:13px}

913	.pinico{color:var(--accent);display:grid;place-items:center}

914	.pinico .ico{width:38px;height:38px}

915	.pindots{display:flex;justify-content:center;gap:10px;margin:16px 0 6px;min-height:14px}

916	.pindots i{width:14px;height:14px;border-radius:50%;background:var(--well);box-shadow:var(--i1)}

917	.pindots i.on{background:var(--accent);box-shadow:none}

918	.pinbox .pinmsg{min-height:18px;font-weight:700;font-size:12.5px;margin:4px 0 10px}

919	.pinbox .pinmsg.bad{color:var(--bad)}

920	.pinbox .pinmsg.ok{color:var(--ok)}

921	.pinpad{display:grid;grid-template-columns:repeat(3,1fr);gap:10px;margin-bottom:14px}

922	.pinpad .btn{padding:14px 0;font-size:20px;font-variant-numeric:tabular-nums}

923	.pinpad .btn.go{font-size:13px;letter-spacing:.6px;color:var(--accent)}

924	.pinbox.shake{animation:pinshake .32s}

925	@keyframes pinshake{20%,60%{transform:translateX(-7px)}40%,80%{transform:translateX(7px)}}

926	@media (prefers-reduced-motion: reduce){.pinbox.shake{animation:none}}

927	.binscreen{background:#14181C;border-radius:14px;padding:12px 10px;margin:0 0 14px;color:#fff;text-align:center}

928	.binscreen small{display:block;font-size:10px;letter-spacing:1px;color:#8A96A3;font-weight:700}

929	.binscreen .pc{display:block;color:#5FD0E8;font-size:11px;letter-spacing:1.8px;font-weight:800;margin-top:4px}

930	.binscreen b{display:block;font-size:34px;letter-spacing:5px;font-variant-numeric:tabular-nums;line-height:1.25}

931	.binscreen .exp{font-size:11.5px;letter-spacing:.2px;font-variant-numeric:tabular-nums}

932	.binscreen .exp.low{color:#FF8A7A}

933	/* Notices never block a key: taps go through, and any touch clears them. */

934	.toast{pointer-events:none}

935	#rolechip{display:inline-flex;align-items:center;gap:6px}

936	#rolechip .ico{width:15px;height:15px}

937

938	/* Fill chart note */

939	.chart-note{font-size:11px;color:var(--ink2);margin:4px 0 0;font-style:italic}

940

941	/* ---------- v2 logo: a bin with a fill wave, a controller chip and a Wi-Fi signal.

942	   It plays a short build-up (lid opens, the bin fills, the chip lights, the signal

943	   goes out) and then holds its still frame. */

944	.sblogo{overflow:visible;display:block}

945	.sblogo .sb-ln{stroke:var(--ink)}

946	.sblogo .sb-ac{stroke:var(--ok)}

947	.sblogo .sb-chp{stroke:var(--onBrand)}

948	.sblogo .sb-nd{fill:var(--onBrand)}

949	.sblogo .sb-led{fill:var(--ok)}

950	.sblogo .g0{stop-color:var(--ok)}

951	.sblogo .g1{stop-color:var(--brand2)}

952	:root{--onBrand:#FFFFFF;--brand2:#0D8A8A}

953	html[data-theme="dark"]{--onBrand:#13100D;--brand2:#3FC7C2}

954	.sblogo *{transform-box:view-box}

955	.sblogo .sb-lid{transform-origin:13px 22.5px}

956	.sblogo .sb-wave{animation:sbWave 3.2s linear infinite}

957	.sblogo .sb-wave.b{animation-duration:4.6s;animation-direction:reverse}

958	.sblogo .sb-sig path{animation:sbPing 3.6s ease-in-out infinite}

959	.sblogo .sb-sig .s2{animation-delay:.25s}

960	.sblogo .sb-sig .s3{animation-delay:.5s}

961	.sblogo .sb-led{animation:sbLed 2.4s ease-in-out infinite}

962	.sblogo.play .sb-lid{animation:sbLid 1.1s cubic-bezier(.3,.7,.3,1.2) both}

963	.sblogo.play .sb-fill{animation:sbFill 1.5s cubic-bezier(.2,.8,.2,1) .25s both}

964	.sblogo.play .sb-chip{transform-origin:32px 46.5px;animation:sbChip .6s cubic-bezier(.2,.9,.3,1.4) 1s both}

965	.sblogo.play .sb-ticks{animation:sbFade .4s ease 1.2s both}

966	.sblogo.play .sb-glow{transform-origin:32px 12px;animation:sbGlow 1.2s ease 1.3s both}

967	.sblogo.play .sb-sig path{animation:sbIn .45s ease both,sbPing 3.6s ease-in-out 2.4s infinite}

968	.sblogo.play .sb-sig .s1{animation-delay:1.35s,2.4s}

969	.sblogo.play .sb-sig .s2{animation-delay:1.5s,2.65s}

970	.sblogo.play .sb-sig .s3{animation-delay:1.65s,2.9s}

971	@keyframes sbWave{from{transform:translateX(0)}to{transform:translateX(-10px)}}

972	@keyframes sbPing{0%,70%,100%{opacity:1}35%{opacity:.35}}

973	@keyframes sbLed{0%,100%{opacity:1}50%{opacity:.35}}

974	@keyframes sbLid{0%{transform:none}35%{transform:translateY(-3px) rotate(-16deg)}70%{transform:translateY(-1px) rotate(-6deg)}100%{transform:none}}

975	@keyframes sbFill{from{transform:translateY(17px)}to{transform:none}}

976	@keyframes sbChip{from{opacity:0;transform:scale(.4)}to{opacity:1;transform:none}}

977	@keyframes sbFade{from{opacity:0}to{opacity:1}}

978	@keyframes sbGlow{0%{opacity:0;transform:scale(.3)}60%{opacity:1;transform:scale(1.25)}100%{opacity:1;transform:none}}

979	@keyframes sbIn{from{opacity:0;transform:translateY(3px)}to{opacity:1;transform:none}}

980	@media(prefers-reduced-motion:reduce){.sblogo *,.sblogo.play *{animation:none!important}}

981	#brand{cursor:pointer}

982	#brand .sblogo{width:40px;height:40px;margin:-8px 0}

983	</style>

984	</head>

985	<body>

986

987	<!-- Start page. Shown on every visit; picking a role hides it. Demo mode is

988	     switched on or off only here (setDemo). -->

989	<div class="gate" id="gate">

990	  <div class="gatecard nm">

991	    <div class="gatehead">

992	      <div id="gateLogo"></div>

993	      <div><h1>Smart-Bin Dashboard</h1>

994	      <p>Pick how you are using the system today. Operator asks for a PIN. Collector is the default.</p></div>

995	    </div>

996	    <div class="roles">

997	      <button class="role" data-role="operator">

998	        <svg class="ico lg" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="1.8" stroke-linecap="round"><circle cx="12" cy="12" r="3.2"/><path d="M12 2.6v3M12 18.4v3M2.6 12h3M18.4 12h3M5.3 5.3l2.1 2.1M16.6 16.6l2.1 2.1M18.7 5.3l-2.1 2.1M7.4 16.6l-2.1 2.1"/></svg>

999	        <h3>Admin / Operator</h3>

1000	        <ul><li>Charts and history</li><li>Lock, unlock, reset tamper</li><li>Calibration wizard</li><li>Simulation controls</li><li>Add and edit bins</li></ul>

1001	      </button>

1002	      <button class="role" data-role="collector">

1003	        <svg class="ico lg" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="1.8" stroke-linecap="round" stroke-linejoin="round"><path d="M2 7h11v9H2zM13 10h4.5l3.5 3.5V16h-8z"/><circle cx="6.5" cy="18.5" r="1.8"/><circle cx="16.5" cy="18.5" r="1.8"/></svg>

1004	        <h3>Collector</h3>

1005	        <ul><li>Which bin to empty first</li><li>Charts, alerts and the collect PIN</li><li>Start and finish a pickup</li><li>Lock and unlock a bin</li><li>Pair a bin to Wi-Fi over Bluetooth</li><li>Settings are read only</li></ul>

1006	      </button>

1007	    </div>

1008	    <div class="gatedemo">

1009	      <button class="btn" id="gateDemo" aria-pressed="false"></button>

1010	      <p id="gateDemoNote"></p>

1011	    </div>

1012	  </div>

1013	</div>

1014

1015	<!-- Operator PIN dialog. Also reused to show the Bluetooth pairing code. -->

1016	<div class="pinwrap hide" id="pinWrap" role="dialog" aria-modal="true" aria-labelledby="pinTitle">

1017	  <div class="pinbox nm" id="pinBox">

1018	    <div class="binscreen hide" id="pinScreen" aria-live="polite"></div>

1019	    <div class="pinico" id="pinIco"></div>

1020	    <h3 id="pinTitle"></h3>

1021	    <p id="pinText"></p>

1022	    <div class="pindots" id="pinDots"></div>

1023	    <p class="pinmsg" id="pinMsg"></p>

1024	    <div class="pinpad" id="pinPad"></div>

1025	    <button class="btn sm" id="pinCancel">CANCEL</button>

1026	  </div>

1027	</div>

1028

1029	<!-- Header: logo, mode chip, link chip, role, theme key and the banner pop-up. -->

1030	<header class="top">

1031	  <div class="topin">

1032	    <div class="brand" id="brand" title="Smart-Bin">

1033	      <span id="brandLogo"></span>

1034	      <span class="btxt">SMART-BIN</span>

1035	    </div>

1036	    <button class="statuspill hide" id="statusPill" aria-expanded="false">

1037	      <span id="pillIco"></span><span id="pillText">0</span>

1038	    </button>

1039	    <div class="spacer"></div>

1040	    <div class="modechip" id="modechip"><span id="modeico"></span><span id="modetext">NORMAL</span></div>

1041	    <div class="link" id="link"><span id="wifiico"></span><span id="linktext">Demo data</span></div>

1042	    <button class="btn sm hide" id="demoBtn" title="Demo mode is on. Press to go back to the start page, where demo is turned off.">DEMO</button>

1043	    <button class="btn sm" id="rolechip">COLLECTOR</button>

1044	    <button class="iconbtn" id="sound" title="Sounds on or off" aria-pressed="true"><span id="soundIco"></span></button>

1045	    <button class="iconbtn" id="theme" title="Switch light or dark"><span id="themeIco"></span></button>

1046	  </div>

1047	  <div class="bannerpop hide" id="bannerPop"></div>

1048	</header>

1049

1050	<div class="wrap">

1051	  <div class="tabs" role="tablist" id="tabs"></div>

1052

1053	  <!-- Overview tab: bin cards (or the tile grid in grid view with 3 or more

1054	         bins) and Collection priority. Cards are built by buildCards/renderCard. -->

1055	  <section data-view="overview">

1056	    <div class="sechead">

1057	      <h2 class="sectitle" style="--cc:var(--c-fill)" id="binsTitle"></h2>

1058	      <div class="addrow hide" id="addRow">

1059	        <span class="addnote" id="addNote"></span>

1060	        <div class="addpick">

1061	          <button id="addLess" aria-label="Fewer bins">&minus;</button>

1062	          <span id="addCount">1</span>

1063	          <button id="addMore" aria-label="More bins">+</button>

1064	        </div>

1065	        <button class="addmini" id="addTile"><svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2.4" stroke-linecap="round"><path d="M12 5v14M5 12h14"/></svg><span id="addLabel">Add a bin</span></button>

1066	      </div>

1067	    </div>

1068	    <div class="ovtools hide" id="ovTools">

1069	      <div class="filterchips" id="filterChips">

1070	        <button class="fchip on" data-filter="all" title="All bins" aria-label="All bins" style="--fc:var(--accent)" data-ico="layers"><span class="fbadge"></span></button>

1071	        <button class="fchip" data-filter="attn" title="Needs attention" aria-label="Needs attention" style="--fc:var(--c-tamper)" data-ico="tamper"><span class="fbadge"></span></button>

1072	        <button class="fchip" data-filter="sim" title="Simulation active" aria-label="Simulation active" style="--fc:var(--sim)" data-ico="flask"><span class="fbadge"></span></button>

1073	        <button class="fchip" data-filter="cal" title="Calibrating" aria-label="Calibrating" style="--fc:var(--cal)" data-ico="calib"><span class="fbadge"></span></button>

1074	      </div>

1075	      <button class="iconbtn" id="viewToggle" title="Switch to grid view" aria-label="Toggle view">

1076	        <svg class="ico xs vico" id="vicoGrid" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2.2"><rect x="3" y="3" width="7" height="7" rx="1.4"/><rect x="14" y="3" width="7" height="7" rx="1.4"/><rect x="3" y="14" width="7" height="7" rx="1.4"/><rect x="14" y="14" width="7" height="7" rx="1.4"/></svg>

1077	        <svg class="ico xs vico hide" id="vicoList" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2.2" stroke-linecap="round"><path d="M8 6h13M8 12h13M8 18h13"/><circle cx="4" cy="6" r="1" fill="currentColor"/><circle cx="4" cy="12" r="1" fill="currentColor"/><circle cx="4" cy="18" r="1" fill="currentColor"/></svg>

1078	      </button>

1079	    </div>

1080	    <div class="scrollpane" id="binsPane"><div class="grid bins" id="bins"></div></div>

1081	    <div class="panel nm collectnext-panel">

1082	      <div class="collecthead">

1083	        <h2 class="ph" style="--cc:var(--c-fill)" id="collectHead"></h2>

1084	        <button class="btn sm" id="collectToggle">SHOW ALL</button>

1085	      </div>

1086	      <div id="collectNext"></div>

1087	      <p class="hint" id="collectHint">Sorted by urgency. Fullest and fastest-filling bins first.</p>

1088	      <div id="prio" style="display:none"></div>

1089	      <div id="ttf" style="display:none"></div>

1090	    </div>

1091	  </section>

1092

1093	  <!-- Analytics tab: fill level and environment charts drawn on <canvas>. -->

1094	  <section data-view="charts" class="hide">

1095	    <div class="chartbox nm">

1096	      <div style="display:flex;justify-content:space-between;align-items:center;gap:14px;flex-wrap:wrap;margin-bottom:16px">

1097	        <h2 class="ph" style="--cc:var(--c-fill);margin:0" id="fillHead"></h2>

1098	        <div class="range" id="range">

1099	          <button class="btn sm" data-range="15">15 m</button>

1100	          <button class="btn sm" data-range="60" aria-pressed="true">1 h</button>

1101	          <button class="btn sm" data-range="360">6 h</button>

1102	          <button class="btn sm" data-range="1440">24 h</button>

1103	        </div>

1104	      </div>

1105	      <div class="seg" id="fillSeg"></div>

1106	      <canvas id="cFill"></canvas>

1107	      <div class="legend" id="legFill"></div>

1108	    </div>

1109	    <div class="chartbox nm">

1110	      <h2 class="ph" style="--cc:var(--c-temp)" id="envHead"></h2>

1111	      <div class="seg" id="envMode"></div>

1112	      <div class="seg" id="envSeg"></div>

1113	      <canvas id="cEnv"></canvas>

1114	      <div class="legend" id="legEnv"></div>

1115	      <p class="hint" id="envNote"></p>

1116	    </div>

1117	  </section>

1118

1119	  <!-- Settings tab: calibration of one bin, Bluetooth Wi-Fi setup, Firebase,

1120	         operator access, demo options and the sound test. Fields stay disabled

1121	         until an operator turns calibration on (refreshCalGate). -->

1122	  <section data-view="settings" class="hide">

1123	    <div class="panel nm">

1124	      <h2>Settings</h2>

1125	      <div class="seg" id="setSeg"></div>

1126

1127	      <div class="pincard nm-in hide" id="pinCard">

1128	        <span class="pinico" id="pinCardIco"></span>

1129	        <span class="pintxt">

1130	          <b>Collect PIN for <span id="pinName">this bin</span></b>

1131	          <span id="pinSub">Type this on the bin touchscreen when you tap COLLECT.</span>

1132	        </span>

1133	        <span class="pinval" id="pinVal">----</span>

1134	        <button class="btn sm" id="pinEye"></button>

1135	      </div>

1136

1137	      <div class="calbar nm-in hide" id="roNote">

1138	        <span class="calico" id="roIco"></span>

1139	        <span class="caltxt">

1140	          <b>These settings are read only</b>

1141	          <span>You can see every value and every bin, but only the operator can change them or run a calibration.</span>

1142	        </span>

1143	      </div>

1144

1145	      <div class="calbar nm-in" id="calBar">

1146	        <span class="calico" id="calIco"></span>

1147	        <span class="caltxt">

1148	          <b id="calTitle">Settings are locked</b>

1149	          <span id="calSub">Switch on calibration mode to change anything here. The bin shows its teal calibration screen while it is on.</span>

1150	        </span>

1151	        <label class="switch big" for="calToggle"><input type="checkbox" id="calToggle"><span>Calibration mode</span></label>

1152	      </div>

1153

1154	      <div class="actions" style="margin:16px 0 0"><button class="btn sm tint" id="sectAll" style="flex:0 0 auto;--cc:var(--accent)"></button></div>

1155	      <fieldset id="setFields" disabled>

1156	        <div class="sect" data-sec="range">

1157	        <h2 style="margin-top:24px">Fill range</h2>

1158	        <div class="fields">

1159	          <div class="field"><label for="s_dempty">Distance empty (cm)</label>

1160	            <div class="withbtn"><input id="s_dempty" type="number" min="1" max="400" step="0.5"><button class="btn sm" data-capture="s_dempty">USE NOW</button></div></div>

1161	          <div class="field"><label for="s_dfull">Distance full (cm)</label>

1162	            <div class="withbtn"><input id="s_dfull" type="number" min="1" max="400" step="0.5"><button class="btn sm" data-capture="s_dfull">USE NOW</button></div></div>

1163	        </div>

1164	        <p class="hint">Empty the bin and press USE NOW for the first box. Hold a board at your full line and press it for the second. Sensor reads <b id="s_dist">-- cm</b> right now.</p>

1165	        </div>

1166

1167	        <div class="sect" data-sec="limits">

1168	        <h2 style="margin-top:24px">Alert limits</h2>

1169	        <div class="fields">

1170	          <div class="field"><label for="s_fill">Fill threshold (%)</label><input id="s_fill" type="number" min="10" max="100"></div>

1171	          <div class="field"><label for="s_temp">Temperature threshold (&deg;C)</label><input id="s_temp" type="number" min="10" max="85"></div>

1172	          <div class="field"><label for="s_hum">Humidity threshold (%RH)</label><input id="s_hum" type="number" min="30" max="100"></div>

1173	          <div class="field"><label for="s_gas">Air quality limit (IAQ)</label><input id="s_gas" type="number" min="50" max="500"></div>

1174	          <div class="field"><label for="s_tilt">Tilt threshold (deg)</label><input id="s_tilt" type="number" min="5" max="90"></div>

1175	          <div class="field"><label for="s_wcap">Weight capacity (g)</label><input id="s_wcap" type="number" min="100" max="5000" step="50"></div>

1176	        </div>

1177

1178	        </div>

1179	        <div class="sect" data-sec="air">

1180	        <h2 style="margin-top:24px">Air sensor (BME680)</h2>

1181	        <div class="fields">

1182	          <div class="field"><label>Temperature raw / corrected</label><div class="readout nm-in" id="s_tlive">-- / -- &deg;C</div></div>

1183	          <div class="field span2"><label for="s_tref">Reference thermometer (&deg;C)</label>

1184	            <div class="withbtn"><input id="s_tref" type="number" min="-40" max="85" step="0.1" placeholder="e.g. 27.5"><button class="btn sm" id="btnRefT">SET OFFSET</button></div></div>

1185	          <div class="field"><label for="s_toff">Temperature offset (&deg;C)</label><input id="s_toff" type="number" min="-10" max="10" step="0.1"></div>

1186	          <div class="field"><label>Gas sensor state</label><div class="readout nm-in" id="s_iaq">--</div></div>

1187	          <div class="field"><label for="s_hoff">Humidity offset (%RH)</label><input id="s_hoff" type="number" min="-20" max="20" step="0.5"></div>

1188	          <div class="field"><label for="s_poff">Pressure offset (hPa)</label><input id="s_poff" type="number" min="-50" max="50" step="0.1"></div>

1189	          <div class="field span2" style="display:flex;align-items:flex-end"><button class="btn" id="btnGasReset">RESET GAS BASELINE</button></div>

1190	        </div>

1191	        <p class="hint">The BME680 warms itself, so it reads a little hot and its humidity a little low. Put a reference thermometer next to the bin for a few minutes, type its reading, press SET OFFSET, then SAVE SETTINGS. Humidity and pressure offsets are optional. The gas reading has no manual offset: it learns its own baseline in normal air, and RESET GAS BASELINE restarts that.</p>

1192	        </div>

1193	        <div class="sect" data-sec="lid">

1194	        <h2 style="margin-top:24px">Lid and lock</h2>

1195	        <div class="fields">

1196	          <div class="field"><label for="s_sclosed">Servo closed (deg)</label><input id="s_sclosed" type="number" min="0" max="180"></div>

1197	          <div class="field"><label for="s_sopen">Servo open (deg)</label><input id="s_sopen" type="number" min="0" max="180"></div>

1198	          <div class="field"><label for="s_pin">Collect PIN</label><input id="s_pin" type="number" min="0" max="9999"></div>

1199	          <div class="field"><label for="s_pirhold">Lid stays open (s)</label><input id="s_pirhold" type="number" min="1" max="60" step="1"></div>

1200	          <div class="field"><label for="s_buzz">Buzzer minimum (s)</label><input id="s_buzz" type="number" min="1" max="30" step="1"></div>

1201	          <div class="field" style="display:flex;align-items:flex-end"><label class="switch"><input id="s_autolock" type="checkbox">Lock the lid when the bin is full</label></div>

1202	        </div>

1203

1204	        </div>

1205	        <div class="sect" data-sec="weight">

1206	        <h2 style="margin-top:24px">Weight</h2>

1207	        <div class="fields">

1208	          <div class="field"><label>Reading now</label><div class="readout nm-in" id="s_live">0.00 kg</div></div>

1209	          <div class="field"><label for="s_known">Known weight (g)</label><input id="s_known" type="number" value="1000" min="10" max="5000"></div>

1210	          <div class="field" style="display:flex;align-items:flex-end;gap:10px">

1211	            <button class="btn" id="btnTare">TARE / ZERO</button>

1212	            <button class="btn" id="btnCalW">CALIBRATE</button>

1213	          </div>

1214	        </div>

1215	        </div>

1216	      </fieldset>

1217

1218	      <div class="sect" data-sec="touch">

1219	      <h2 style="margin-top:24px">Touchscreen</h2>

1220	      <div class="fields">

1221	        <div class="field span2"><label>Touch calibration</label><div class="readout nm-in" id="s_touch">--</div></div>

1222	        <div class="field span2" style="display:flex;align-items:flex-end;gap:10px">

1223	          <button class="btn" id="btnTouchCal">CALIBRATE TOUCH</button>

1224	          <button class="btn" id="btnTouchReset">RESET TOUCH</button>

1225	        </div>

1226	      </div>

1227	      <p class="hint">If presses land in the wrong place, press CALIBRATE TOUCH. The bin shows four crosses one after another: press each one firmly, then let go. The result is saved on the bin straight away. It can also be started on the bin itself by holding a finger anywhere on the screen for 5 seconds. Needs the bin&rsquo;s Wi-Fi.</p>

1228	      </div>

1229

1230	      <div class="actions" style="margin-top:26px">

1231	        <button class="btn" id="btnSave" style="flex:0 0 auto" disabled>SAVE SETTINGS</button>

1232	        <button class="btn" id="btnCancel" style="flex:0 0 auto" disabled>CANCEL CHANGES</button>

1233	        <button class="btn" id="btnReload" style="flex:0 0 auto">Reload from bin</button>

1234	        <button class="btn" id="btnExport" style="flex:0 0 auto">Export bin list</button>

1235	      </div>

1236	      <p class="hint" id="setMsg">Values are read-only until calibration mode is on. Saving sends them to the bin and they survive a restart.</p>

1237	    </div>

1238

1239	    <div class="panel nm" id="btPanel" style="margin-top:22px">

1240	      <div class="sechead">

1241	        <h2 class="ph" style="--cc:var(--c-bt)" id="btHead"></h2>

1242	        <span class="btstate" id="btState">Not paired</span>

1243	      </div>

1244	      <p class="hint" style="margin:-6px 0 8px">Gives a bin a new Wi-Fi name and password over Bluetooth. The bin keeps its current Wi-Fi until the new one works, and your laptop can stay on Wi-Fi too.</p>

1245	      <ol class="hint" style="margin:0 0 16px;padding-left:20px;line-height:1.6">

1246	        <li><b>On the bin:</b> tap <b>i</b> (top left), then <b>WI-FI SETUP</b>, and type the <b>Collect PIN</b> (the same 4 digits as for COLLECT).</li>

1247	        <li><b>Here:</b> press <b>PAIR A BIN</b> and pick <b>SmartBin-01</b> (or 02) in the list the browser shows.</li>

1248	        <li>The bin shows a <b>6-digit pairing code</b>. Windows or your phone opens its own pairing window: type the code <b>there</b>, not on this page.</li>

1249	        <li>Pick the network, type its password, press <b>SEND WI-FI</b>. The bin shows its new IP address, and so does this page.</li>

1250	        <li>Press <b>TURN BT OFF</b> when you are done.</li>

1251	      </ol>

1252	      <div class="btsteps" id="btSteps"></div>

1253	      <div class="fields" style="margin-top:18px">

1254	        <div class="field"><label for="bt_ssid">Wi-Fi name (2.4 GHz)</label>

1255	          <div class="withbtn"><input id="bt_ssid" type="text" list="bt_nets" spellcheck="false" placeholder="HomeWiFi-2G" disabled>

1256	          <button class="btn sm" id="btScan" disabled></button></div>

1257	          <datalist id="bt_nets"></datalist>

1258	          <div class="btnets" id="btNetList"></div></div>

1259	        <div class="field"><label for="bt_pass">Password</label>

1260	          <div class="withbtn"><input id="bt_pass" type="password" spellcheck="false" disabled>

1261	          <button class="btn sm" id="btEye" disabled></button></div></div>

1262	      </div>

1263	      <div class="actions" style="margin-top:20px">

1264	        <button class="btn" id="btPair"></button>

1265	        <button class="btn" id="btSend" disabled></button>

1266	        <button class="btn" id="btOff" disabled></button>

1267	      </div>

1268	      <div class="btresult hide" id="btResult"></div>

1269	      <p class="hint" id="btMsg"></p>

1270	      <p class="hint" id="btWhy"></p>

1271	      <p class="hint" id="btRoleNote"></p>

1272	    </div>

1273

1274	    <div class="panel nm" id="cloudPanel" style="margin-top:22px">

1275	      <div class="sechead">

1276	        <h2 class="ph" style="--cc:var(--c-cloud)" id="cloudHead"></h2>

1277	        <span class="btstate" id="cloudState">Not tested</span>

1278	      </div>

1279	      <p class="hint" style="margin:-6px 0 16px">The bins copy their readings to Firebase so the phone app keeps working away from this Wi-Fi. Local Wi-Fi always comes first: this dashboard and the app both use the bins’ own API whenever a bin answers, and the app falls back to Firebase only when none does.</p>

1280	      <div class="fields">

1281	        <div class="field span2"><label for="fb_url">Database URL</label><input id="fb_url" type="text" inputmode="url" spellcheck="false" placeholder="https://...firebasedatabase.app"></div>

1282	        <div class="field"><label for="fb_key">Web API key</label><input id="fb_key" type="text" spellcheck="false" placeholder="AIza..."></div>

1283	        <div class="field"><label for="fb_email">Account email</label><input id="fb_email" type="text" inputmode="email" spellcheck="false"></div>

1284	        <div class="field"><label for="fb_pass">Password</label>

1285	          <div class="withbtn"><input id="fb_pass" type="password" spellcheck="false"><button class="btn sm" id="fbEye"></button></div></div>

1286	      </div>

1287	      <div class="actions" style="margin-top:20px">

1288	        <button class="btn" id="fbTest" style="flex:0 0 auto"></button>

1289	        <button class="btn" id="fbSave" style="flex:0 0 auto"></button>

1290	        <button class="btn" id="fbRestore" style="flex:0 0 auto"></button>

1291	        <button class="btn" id="fbSend" style="flex:0 0 auto"></button>

1292	      </div>

1293	      <div class="fields" style="margin-top:16px">

1294	        <div class="field span2"><label for="ts_server">ThingSpeak server</label><input id="ts_server" type="text" inputmode="url" spellcheck="false" placeholder="https://api.thingspeak.com"></div>

1295	      </div>

1296	      <div class="cloudmsg hide" id="cloudMsg"></div>

1297	      <div class="cloudrows" id="cloudRows"></div>

1298	      <div class="usebar"><i id="useFill" style="width:0%"></i></div>

1299	      <p class="hint" id="cloudUse">Each bin writes every 5 s while a phone or this dashboard is watching over the cloud, every minute otherwise, and at once when an alert starts. LOCK, UNLOCK and COLLECT sent through the cloud reach the bin within a few seconds. The data meter appears once the bins report it.</p>

1300	      <p class="hint" id="cloudRoleNote"></p>

1301	    </div>

1302

1303	    <div class="panel nm" id="opPanel" style="margin-top:22px">

1304	      <div class="sechead">

1305	        <h2 class="ph" style="--cc:var(--sim)">Operator access</h2>

1306	        <span class="btstate" id="opState"></span>

1307	      </div>

1308	      <p class="hint" style="margin:-6px 0 16px">The dashboard opens on the start page every time, the same as the phone app. Operator needs the PIN, and drops back to collector after 2 min with this tab hidden. After 5 wrong PINs it waits 30 s, longer each time. Only an operator can change the PIN, and only by typing the current one first.</p>

1309	      <div class="actions">

1310	        <button class="btn" id="opChange" style="flex:0 0 auto">CHANGE OPERATOR PIN</button>

1311	        <button class="btn" id="opLockNow" style="flex:0 0 auto">BACK TO COLLECTOR</button>

1312	        <button class="btn" id="opStart" style="flex:0 0 auto">START PAGE</button>

1313	      </div>

1314	      <p class="hint" id="opNote"></p>

1315	    </div>

1316

1317	    <div class="panel nm hide" id="demoPanel" style="margin-top:22px">

1318	      <div class="sechead"><h2 class="ph" style="--cc:var(--sim)" id="demoHead"></h2><span class="btstate" id="demoState"></span></div>

1319	      <p class="hint" style="margin:-6px 0 14px">Act out what simulation cannot, on the demo bins only, the same as the app's demo options. Any choice here pauses the automatic tour for a minute. RESET puts it all back.</p>

1320	      <div id="demoBody"></div>

1321	    </div>

1322

1323	    <div class="panel nm" id="sndPanel" style="margin-top:22px">

1324	      <div class="sechead"><h2 class="ph" style="--cc:var(--accent)" id="sndHead"></h2></div>

1325	      <p class="hint" style="margin:-6px 0 16px">Press a key to hear that sound. The phone app has the same sounds under the same names, and vibrates with each one. This test plays even with sounds turned off.</p>

1326	      <div id="sndGroups"></div>

1327	    </div>

1328	  </section>

1329	  <!-- Simulation tab: built by buildSim from the SENSORS list. -->

1330	  <section data-view="sim" class="hide">

1331	    <div class="panel nm">

1332	      <h2>Simulation</h2>

1333	      <div class="seg" id="simSeg"></div>

1334	      <p class="hint" style="margin:-6px 0 16px">AUTO reads the real hardware and lets the firmware drive the outputs. MANUAL holds the value you set, one row at a time.</p>

1335	      <table class="sim"><thead><tr><th id="thSensor"></th><th style="width:132px" id="thMode"></th><th id="thValue"></th></tr></thead><tbody id="simBody"></tbody></table>

1336	      <div class="actions" style="margin-top:20px">

1337	        <button class="btn accent" id="btnSim" style="flex:0 0 auto">APPLY</button>

1338	        <button class="btn" id="btnSimOff" style="flex:0 0 auto">All back to AUTO</button>

1339	      </div>

1340	      <p class="hint" id="simMsg">The whole dashboard turns violet while any sensor is manual.</p>

1341	    </div>

1342	  </section>

1343

1344	  <!-- Alerts tab: alert history with kind and date filters. -->

1345	  <section data-view="alerts" class="hide">

1346	    <div class="panel nm">

1347	      <div class="sechead">

1348	        <h2 class="ph" style="--cc:var(--c-tamper)" id="alertHead"></h2>

1349	        <div class="alertctl">

1350	          <div class="filterchips" id="alertKinds"></div>

1351	          <div id="alertRange"></div>

1352	          <button class="btn sm" id="btnMarkAll" disabled></button>

1353	          <button class="btn sm" id="btnClear"></button>

1354	        </div>

1355	      </div>

1356	      <p class="alertscope" id="alertScope"></p>

1357	      <div class="aconfirm hide" id="alertConfirm"><span id="alertConfirmText"></span><button class="btn sm" id="alertNo">CANCEL</button><button class="btn sm" id="alertYes">DELETE</button></div>

1358	      <div class="scrollpane alertpane"><div class="alerts" id="alertList"></div></div>

1359	    </div>

1360	  </section>

1361	</div>

1362

1363	<script>

1364	// ============================================================================

1365	// Script. Everything runs in the page; there is no build step.

1366	// Main loop: setInterval(poll, 1000) near the end. poll() reads every bin by

1367	// the first route that answers and then calls render(). Most renderers keep a

1368	// "signature" string (cardSig, alertSig, banSig ...) and skip the DOM update

1369	// when it has not changed, which is what stops the page from flickering. If

1370	// something on screen does not update, its signature is the first suspect.

1371	// ============================================================================

1372	// Bin colors: evenly spread hues; after 12 bins the hue shifts by 37 degrees

1373	// and the lightness changes, so every bin stays distinct.

1374	const HUES=[153,287,47,207,100,260,20,180,313,73,233,127];

1375	function binColor(i){

1376	  const dark=document.documentElement.getAttribute('data-theme')==='dark';

1377	  const round=Math.floor(i/HUES.length);

1378	  const h=(HUES[i%HUES.length]+round*37)%360;

1379	  const s=50,l=dark?(round===0?64:round===1?55:72):(round===0?46:round===1?38:54);

1380	  return 'hsl('+h+','+s+'%,'+l+'%)';

1381	}

1382	// Default calibration, the same keys and values as the firmware's Cal struct.

1383	const DEF_CAL={d_empty:30,d_full:5,fill_th:80,temp_th:40,hum_th:85,gas_th:200,tilt_th:30,weight_cap:4500,auto_lock:0,servo_closed:0,servo_open:90,collect_pin:1234,pir_hold_s:5,buzz_s:2,temp_off:-2,hum_off:0,press_off:0};

1384	// Simulation rows. bit is the bit in the firmware's Sim mask (0 fill .. 6 lid).

1385	const SENSORS=[

1386	  {key:'fill',bit:0,label:'Fill level',icon:'fill',kind:'range',min:0,max:100,step:1,unit:'%'},

1387	  {key:'weight',bit:1,label:'Weight',icon:'weight',kind:'number',min:0,max:5000,step:10,unit:'g'},

1388	  {key:'gas',bit:2,label:'Air quality',icon:'gas',kind:'range',min:0,max:500,step:5,unit:'IAQ'},

1389	  {key:'temp',bit:3,label:'Temperature and humidity',icon:'temp',kind:'pair'},

1390	  {key:'tamper',bit:4,label:'Tamper',icon:'tamper',kind:'toggle'},

1391	  {key:'motion',bit:5,label:'Motion',icon:'motion',kind:'toggle'},

1392	  {key:'lid',bit:6,label:'Lid',icon:'lid',kind:'lid'}

1393	];

1394	const ENV_MODES=[

1395	  ['both','ALL','layers','var(--ink2)'],

1396	  ['temp','TEMPERATURE','temp','var(--c-temp)'],

1397	  ['humid','HUMIDITY','humid','var(--c-humid)'],

1398	  ['gas','AIR QUALITY','gas','var(--c-gas)'],

1399	  ['press','PRESSURE','press','var(--c-press)']

1400	];

1401	/* BME680 datasheet ranges. A single-reading chart runs to the top of its range.

1402	   Pressure is charted only: it has no tile on the bin card and no alert. */

1403	const SPEC={

1404	  temp:{min:0,max:85,ticks:[0,20,40,60,85],unit:'C',name:'Temperature',dec:1,col:'--c-temp'},

1405	  humid:{min:0,max:100,ticks:[0,25,50,75,100],unit:'%',name:'Humidity',dec:0,col:'--c-humid'},

1406	  press:{min:300,max:1100,ticks:[300,500,700,900,1100],unit:'hPa',name:'Pressure',dec:1,col:'--c-press'},

1407	  gas:{min:0,max:500,ticks:[0,100,200,300,400,500],unit:'IAQ',name:'Air quality',dec:0,col:'--c-gas'}

1408	};

1409	const TABS=[

1410	  {v:'overview',t:'Overview',i:'grid',c:'var(--c-fill)'},

1411	  {v:'charts',t:'Charts',i:'chart',c:'var(--c-humid)'},

1412	  {v:'settings',t:'Settings',i:'sliders',c:'var(--cal)'},

1413	  {v:'sim',t:'Simulation',i:'flask',c:'var(--sim)'},

1414	  {v:'alerts',t:'Alerts',i:'bell',c:'var(--c-tamper)'}

1415	];

1416

1417	/* ---------- icons ---------- */

1418	const S=(b,c)=>'<svg class="ico '+(c||'')+'" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round" stroke-linejoin="round">'+b+'</svg>';

1419	const ICONS={

1420	  grid:()=>S('<rect x="3" y="3" width="7.5" height="7.5" rx="1.6"/><rect x="13.5" y="3" width="7.5" height="7.5" rx="1.6"/><rect x="3" y="13.5" width="7.5" height="7.5" rx="1.6"/><rect x="13.5" y="13.5" width="7.5" height="7.5" rx="1.6"/>'),

1421	  chart:()=>S('<path d="M3 20h18"/><path d="M5 15l4.5-5 3.5 3L19 6"/>'),

1422	  sliders:()=>S('<path d="M4 7h10M18 7h2M4 17h4M12 17h8"/><circle cx="16" cy="7" r="2.2"/><circle cx="10" cy="17" r="2.2"/>'),

1423	  flask:()=>S('<path d="M9.5 3v6.2L4.6 18a2 2 0 0 0 1.7 3h11.4a2 2 0 0 0 1.7-3L14.5 9.2V3"/><path d="M8 3h8M7.4 15h9.2"/>'),

1424	  bell:()=>S('<path d="M18 9a6 6 0 1 0-12 0c0 6-2 7-2 7h16s-2-1-2-7"/><path d="M10.4 20.5a2 2 0 0 0 3.2 0"/>'),

1425	  plus:()=>S('<path d="M12 5v14M5 12h14"/>'),

1426	  gear:()=>S('<circle cx="12" cy="12" r="3"/><path d="M12 2.8v2.6M12 18.6v2.6M2.8 12h2.6M18.6 12h2.6M5.5 5.5l1.8 1.8M16.7 16.7l1.8 1.8M18.5 5.5l-1.8 1.8M7.3 16.7l-1.8 1.8"/>','xs'),

1427	  truck:()=>S('<path d="M2 7h11v9H2zM13 10h4.5l3.5 3.5V16h-8z"/><circle cx="6.5" cy="18.5" r="1.8"/><circle cx="16.5" cy="18.5" r="1.8"/>','xs'),

1428	  check:()=>S('<path d="M4 12.5l5 5L20 6.5"/>','xs'),

1429	  temp:()=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round">'+

1430	    '<circle class="halo" cx="12" cy="17.2" r="5.4" fill="currentColor" stroke="none"/>'+

1431	    '<path d="M14.2 14.4V5.2a2.2 2.2 0 1 0-4.4 0v9.2a4.6 4.6 0 1 0 4.4 0z"/>'+

1432	    '<rect class="merc" x="10.6" y="9" width="2.8" height="6" rx="1.4" fill="currentColor" stroke="none"/>'+

1433	    '<circle cx="12" cy="17.2" r="2.6" fill="currentColor" stroke="none"/></svg>',

1434	  humid:()=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linejoin="round">'+

1435	    '<path d="M12 3.2s6.2 6.6 6.2 10.3a6.2 6.2 0 1 1-12.4 0C5.8 9.8 12 3.2 12 3.2z"/>'+

1436	    '<circle class="pulse" cx="12" cy="14.4" r="3" fill="currentColor" stroke="none" opacity=".45"/></svg>',

1437	  gas:()=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round">'+

1438	    '<path class="vap" d="M6.5 17c0-2.6 2.6-2.6 2.6-5.2S6.5 9.2 6.5 6.6"/>'+

1439	    '<path class="vap v2" d="M12 18c0-2.6 2.6-2.6 2.6-5.2S12 10.2 12 7.6"/>'+

1440	    '<path class="vap v3" d="M17.5 17c0-2.6 2.6-2.6 2.6-5.2s-2.6-2.6-2.6-5.2"/></svg>',

1441	  cloud:()=>S('<path d="M7 18.6h10.4a4.2 4.2 0 0 0 .5-8.4 6 6 0 0 0-11.5-1A4.8 4.8 0 0 0 7 18.6z"/>'),

1442	  press:()=>S('<path d="M4.6 17.6a8.6 8.6 0 1 1 14.8 0"/><path d="M12 14l3.6-4.4"/><circle cx="12" cy="14.2" r="1.5" fill="currentColor" stroke="none"/><path d="M7.2 17.6h9.6"/>'),

1443	  weight:()=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round" stroke-linejoin="round">'+

1444	    '<path d="M4.6 20.5h14.8l-1.9-9.6H6.5z"/><path d="M9 10.9a3 3 0 1 1 6 0"/>'+

1445	    '<path class="needle" d="M12 17.6v-3.1" stroke-width="2.2" style="transform-origin:12px 17.6px"/></svg>',

1446	  lid:(open)=>'<svg class="ico '+(open?'open':'')+'" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round" stroke-linejoin="round">'+

1447	    '<path d="M6.6 9.5l1.1 11h8.6l1.1-11"/><g class="lidflap"><path d="M4.5 9.5h15"/><path d="M9.6 7.1h4.8"/></g></svg>',

1448	  motion:(on)=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round">'+

1449	    '<circle cx="5.5" cy="12" r="1.8" fill="currentColor" stroke="none"/>'+

1450	    '<g class="'+(on?'arcs':'')+'" opacity="'+(on?'1':'.4')+'"><path d="M10 7.5a7 7 0 0 1 0 9"/><path d="M13.6 5a11 11 0 0 1 0 14"/><path d="M17.2 2.6a15 15 0 0 1 0 18.8"/></g></svg>',

1451	  lock:(locked)=>'<svg class="ico '+(locked?'locked':'unlocked')+'" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round">'+

1452	    '<rect x="4.5" y="10.5" width="15" height="10.5" rx="2.4"/><path class="shackle" d="M8 10.5V7.4a4 4 0 0 1 8 0v3.1"/>'+

1453	    '<circle cx="12" cy="15.7" r="1.5" fill="currentColor" stroke="none"/></svg>',

1454	  tamper:()=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round" stroke-linejoin="round">'+

1455	    '<g class="shake" style="transform-origin:12px 13px"><path d="M12 3.4L22 20H2z"/><path d="M12 9.6v4.2M12 17h.01"/></g></svg>',

1456	  calib:()=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round">'+

1457	    '<circle class="ring" cx="12" cy="12" r="8.4" stroke-dasharray="7 6"/><circle cx="12" cy="12" r="2.4" fill="currentColor" stroke="none"/>'+

1458	    '<path d="M12 1.6v2.4M12 20v2.4M1.6 12h2.4M20 12h2.4"/></svg>',

1459	  sim:()=>ICONS.flask(),

1460	  fill:()=>S('<path d="M5.4 4.4h13.2l-1.3 15.2a2 2 0 0 1-2 1.8H8.7a2 2 0 0 1-2-1.8z"/><path d="M6.9 12.6c1.6-1.1 3.3-1.1 4.9 0s3.3 1.1 4.9 0"/>'),

1461	  bins:()=>S('<path d="M3.2 7.6h7l-.8 11.8H4zM13.8 7.6h7l-.8 11.8h-5.4z"/><path d="M2.4 7.6h8.6M13 7.6h8.6M5.4 5h2.6M16 5h2.6"/>'),

1462	  clock:()=>S('<circle cx="12" cy="12" r="8.4"/><path d="M12 7.2V12l3.2 2"/>'),

1463	  trash:()=>S('<path d="M4.4 6.8h15.2M9.6 6.8V4.6h4.8v2.2M6.8 6.8l1 12.6h8.4l1-12.6M10.2 10.4v5.4M13.8 10.4v5.4"/>'),

1464	  stopwatch:()=>S('<circle cx="12" cy="13.6" r="7.4"/><path d="M12 9.8v3.8l2.6 1.6M9.6 2.8h4.8M19.4 7.2l1.4-1.4"/>'),

1465	  palette:()=>S('<path d="M12 3.4a8.6 8.6 0 1 0 0 17.2c1.2 0 1.9-.8 1.9-1.8 0-1-.9-1.5-.9-2.4s.7-1.6 1.7-1.6h1.3a4.6 4.6 0 0 0 4.6-4.6c0-3.8-3.9-6.8-8.6-6.8z"/><circle cx="7.8" cy="11.6" r="1.1" fill="currentColor" stroke="none"/><circle cx="10.6" cy="7.8" r="1.1" fill="currentColor" stroke="none"/><circle cx="15" cy="8.4" r="1.1" fill="currentColor" stroke="none"/>'),

1466	  speaker:()=>S('<path d="M4.6 9.4h3.4l4.2-3.6v12.4L8 14.6H4.6z"/><path d="M15.6 9.4a3.6 3.6 0 0 1 0 5.2M18 6.8a7 7 0 0 1 0 10.4"/>'),

1467	  layers:()=>S('<path d="M12 3.4l8.6 4.6L12 12.6 3.4 8z"/><path d="M3.4 12.4L12 17l8.6-4.6M3.4 16.6L12 21.2l8.6-4.6"/>'),

1468	  rank:()=>S('<path d="M9.6 6.6h10.8M9.6 12h10.8M9.6 17.4h10.8"/><path d="M4.8 5v3.6M3.6 5h1.2M3.4 10.4h2.8M3.4 15.4h2.6v1.6h-2.6v1.8h2.6"/>'),

1469	  eye:()=>S('<path d="M2.4 12s3.6-6.4 9.6-6.4S21.6 12 21.6 12s-3.6 6.4-9.6 6.4S2.4 12 2.4 12z"/><circle cx="12" cy="12" r="2.8"/>'),

1470	  sunmoon:(dark)=>'<svg class="ico sunmoon '+(dark?"is-dark":"is-light")+'" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round">'+

1471	    '<g class="sunside"><circle cx="8.2" cy="12" r="3"/><path d="M8.2 5.4v1.6M8.2 17v1.6M3.4 7.2l1.2 1.2M11.8 15.6l1.2 1.2M3.4 16.8l1.2-1.2M1.2 12h1.6"/></g>'+

1472	    '<g class="moonside"><path d="M19.4 7.6a5.4 5.4 0 1 0 0 8.8 6.2 6.2 0 0 1 0-8.8z"/></g></svg>',

1473	  bluetooth:()=>S('<path d="M7.4 7.4l9.2 9.2L12 20.6V3.4l4.6 4L7.4 16.6"/>'),

1474	  radar:()=>S('<path d="M12 12l5.6-5.6"/><circle cx="12" cy="12" r="1.6" fill="currentColor" stroke="none"/><path d="M8.4 15.6a5.1 5.1 0 0 1 0-7.2M5.6 18.4a9 9 0 0 1 0-12.8"/><path d="M15.6 8.4a5.1 5.1 0 0 1 0 7.2"/>'),

1475	  key:()=>S('<circle cx="8.2" cy="12" r="3.6"/><path d="M11.8 12h9v3M17.6 12v2.6"/>'),

1476	  eyeoff:()=>S('<path d="M4 4l16 16"/><path d="M9.6 9.7a2.8 2.8 0 0 0 3.9 3.9"/><path d="M6.4 6.8C3.9 8.5 2.4 12 2.4 12s3.6 6.4 9.6 6.4c1.6 0 3-.4 4.2-1M18.3 15.1c1.9-1.6 3.3-3.1 3.3-3.1S18 5.6 12 5.6c-.8 0-1.5.1-2.2.3"/>'),

1477	  ruler:()=>S('<rect x="2.6" y="8" width="18.8" height="8" rx="1.8"/><path d="M7 8v3M11 8v4M15 8v3M19 8v4"/>'),

1478	  percent:()=>S('<path d="M18.5 5.5l-13 13"/><circle cx="7.6" cy="7.6" r="2.3"/><circle cx="16.4" cy="16.4" r="2.3"/>'),

1479	  save:()=>S('<path d="M4.5 4.5h11l4 4v11h-15z"/><path d="M8 4.5v5h6v-5M8 19.5v-5.5h8v5.5"/>'),

1480	  reload:()=>S('<path d="M20.5 12a8.5 8.5 0 1 1-2.6-6.1"/><path d="M20.8 4.4v4.8h-4.8"/>'),

1481	  download:()=>S('<path d="M12 3.6v11.2M7.6 10.6l4.4 4.4 4.4-4.4"/><path d="M4.4 19.4h15.2"/>'),

1482	  undo:()=>S('<path d="M3.6 12a8.4 8.4 0 1 0 2.6-6.1"/><path d="M3.3 4.4v4.8h4.8"/>'),

1483	  capture:()=>S('<circle cx="12" cy="12" r="7.6"/><circle cx="12" cy="12" r="1.8" fill="currentColor" stroke="none"/><path d="M12 1.8v2.8M12 19.4v2.8M1.8 12h2.8M19.4 12h2.8"/>'),

1484	  pin:()=>S('<rect x="3.4" y="4.6" width="17.2" height="14.8" rx="2.2"/><path d="M7.4 9.4h.01M12 9.4h.01M16.6 9.4h.01M7.4 14.6h9.2"/>'),

1485	  tilt:()=>S('<path d="M3.4 20.2h17.2"/><path d="M6.6 20.2L15 6.4"/><path d="M11.4 20.2a7.5 7.5 0 0 0-1.6-4.4"/>'),

1486	  servo:()=>S('<circle cx="12" cy="13.6" r="3.2"/><path d="M12 10.4V4.6"/><path d="M4.6 13.6a7.4 7.4 0 0 1 3.2-6.1M19.4 13.6a7.4 7.4 0 0 0-3.2-6.1"/>'),

1487	  cross:()=>S('<path d="M6.4 6.4l11.2 11.2M17.6 6.4L6.4 17.6"/>'),

1488	  power:()=>S('<path d="M12 3.4v8.2"/><path d="M6.8 6.6a7.6 7.6 0 1 0 10.4 0"/>'),

1489	  wifi:(live)=>'<svg class="ico xs wifi '+(live?'live':'')+'" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2.2" stroke-linecap="round">'+

1490	    '<path d="M8.5 15.2a5 5 0 0 1 7 0"/><path d="M5.5 11.8a9.4 9.4 0 0 1 13 0"/><path d="M2.4 8.4a14 14 0 0 1 19.2 0"/>'+

1491	    '<circle cx="12" cy="18.6" r="1.4" fill="currentColor" stroke="none" class="glowdot"/></svg>',

1492	  bin:(id,pct,col,prefix)=>{

1493	    const cid=(prefix||'clip')+'-'+id;

1494	    const y=(42-(pct/100)*30).toFixed(1);

1495	    return '<svg class="ico lg" viewBox="0 0 48 48" style="color:'+col+'">'+

1496	    '<defs><clipPath id="'+cid+'"><path d="M11.5 14h25l-2.6 26.2a2.4 2.4 0 0 1-2.4 2.1H16.5a2.4 2.4 0 0 1-2.4-2.1z"/></clipPath></defs>'+

1497	    '<g clip-path="url(#'+cid+')"><g class="wavewrap" transform="translate(0,'+y+')">'+

1498	    '<path class="wave" d="M-24 4 q6 -5 12 0 t12 0 t12 0 t12 0 t12 0 t12 0 v36 h-72z" fill="currentColor" opacity=".85"/>'+

1499	    '<path class="wave" d="M-24 7 q6 -5 12 0 t12 0 t12 0 t12 0 t12 0 t12 0 v36 h-72z" fill="currentColor" opacity=".35" style="animation-duration:5s"/>'+

1500	    '</g></g>'+

1501	    '<path d="M11.5 14h25l-2.6 26.2a2.4 2.4 0 0 1-2.4 2.1H16.5a2.4 2.4 0 0 1-2.4-2.1z" fill="none" stroke="currentColor" stroke-width="2.6"/>'+

1502	    '<path d="M8 13h32M19 9.4h10" fill="none" stroke="currentColor" stroke-width="2.6" stroke-linecap="round"/></svg>';

1503	  }

1504	};

1505	const icon=(n,a)=>(ICONS[n]?ICONS[n](a):'');

1506

1507	/* ---------- state ---------- */

1508	// The bin list is saved in localStorage, so it survives a reload. Only bins

1509	// with an address (IP, ThingSpeak channel or Firebase name) count as real.

1510	const store={

1511	  read(){try{return JSON.parse(localStorage.getItem('smartbin.bins')||'null')}catch(e){return null}},

1512	  write(v){try{localStorage.setItem('smartbin.bins',JSON.stringify(v))}catch(e){}}

1513	};

1514	function newBin(seed){

1515	  return {id:seed.id,name:seed.name,place:seed.place,ip:seed.ip||'',online:false,live:false,

1516	    cloud:seed.cloud||'',tsCh:seed.tsCh||'',tsKey:seed.tsKey||'',tbId:seed.tbId||'',tbKey:seed.tbKey||'',

1517	    route:null,seen:{local:0,fb:0,ts:0},lastAt:0,ipHint:'',

1518	    data:{fill:seed.fill||20,distance_cm:20,weight_g:800,temp:28,temp_raw:30,humidity:62,pressure:1008,gas:70,iaq_acc:3,tilt:1,lid:'CLOSED',status:'OK',motion:false},

1519	    cal:Object.assign({},DEF_CAL),

1520	    sim:{mask:0,values:{fill:60,weight:2000,gas:120,temp:28,humidity:60,tamper:0,motion:0,lid:0,rgb:1,buzzer:0}},

1521	    hist:[],prev:{}};

1522	}

1523	const saved=store.read();

1524	const DEMO_SEEDS=[

1525	  {id:'demo1',name:'Demo Bin 1',place:'Main Hall',fill:34},

1526	  {id:'demo2',name:'Demo Bin 2',place:'Cafeteria',fill:62},

1527	  {id:'demo3',name:'Demo Bin 3',place:'Library',fill:88}

1528	];

1529	const mkDemo=s=>Object.assign(newBin(s),{demo:true});

1530	const fbSaved=(()=>{try{const v=JSON.parse(localStorage.getItem('smartbin.fb')||'null');return !!(v&&v.url)}catch(e){return false}})();

1531	const isRealSeed=s=>!!(s&&(s.ip||(s.tsCh&&s.tsKey)||(s.cloud&&fbSaved)));

1532	const realStart=(saved||[]).filter(isRealSeed).map(newBin);

1533	let demoPref=null;try{demoPref=localStorage.getItem('smartbin.demo')}catch(e){}

1534	const demoStart=demoPref===null?realStart.length===0:demoPref==='1';

1535	// Everything the page draws comes from this object; render() reads it.

1536	const state={

1537	  role:'collector',view:'overview',rangeMin:60,alerts:[],calib:null,envBin:null,addCount:1,chartPick:[],envMode:'both',

1538	  ovView:'list',ovFilter:'all',collectShowAll:false,akind:'all',arange:'any',ovTile:null,

1539	  bins:demoStart?realStart.concat(DEMO_SEEDS.map(mkDemo)):realStart,

1540	  demo:demoStart,selSet:null,selSim:null

1541	};

1542	state.selSet=state.selSim=state.envBin=state.bins[0]?state.bins[0].id:null;

1543	var TOUR={i:0,next:0,pauseUntil:0,msg:'',busy:false};

1544

1545	const $=s=>document.querySelector(s);

1546	const $$=s=>Array.from(document.querySelectorAll(s));

1547	const byId=id=>state.bins.find(b=>b.id===id);

1548	const clamp=(v,a,b)=>Math.max(a,Math.min(b,v));

1549	const fmt=(v,d=1)=>Number.isFinite(v)?v.toFixed(d):'--';

1550	const colorOf=i=>binColor(i);

1551	function persist(){

1552	  state.bins.forEach(b=>{if(b.demo&&(b.ip||(b.tsCh&&b.tsKey)))b.demo=false;});

1553	  store.write(state.bins.filter(b=>!b.demo).map(b=>({id:b.id,name:b.name,place:b.place,ip:b.ip,fill:b.data.fill,

1554	  cloud:b.cloud,tsCh:b.tsCh,tsKey:b.tsKey,tbId:b.tbId,tbKey:b.tbKey})))}

1555

1556	function seedHistory(b,i){

1557	  const now=Date.now(),pts=[];

1558	  let f=Math.max(2,b.data.fill-24-i*8);

1559	  for(let m=1440;m>=0;m-=4){

1560	    f=Math.min(99,f+Math.random()*0.075);

1561	    pts.push({t:now-m*60000,fill:+f.toFixed(1),

1562	      temp:+(b.data.temp-2+Math.sin(m/90)*1.5+Math.random()*.4).toFixed(1),

1563	      humidity:+clamp(b.data.humidity+Math.sin(m/140)*7+Math.random()*1.4,30,95).toFixed(1),

1564	      pressure:+(1008+Math.sin(m/260)*4+Math.cos(m/90)*0.8+Math.random()*0.25).toFixed(1),

1565	      gas:Math.round(clamp(40+f*1.6+Math.sin(m/120)*9+Math.random()*6,10,500))});

1566	  }

1567	  b.hist=pts;

1568	}

1569

1570	const LABEL_ICONS={

1571	  s_dempty:['ruler','var(--cal)'],      s_dfull:['fill','var(--c-fill)'],

1572	  s_fill:['percent','var(--c-fill)'],   s_temp:['temp','var(--c-temp)'],   s_hum:['humid','var(--c-humid)'],

1573	  s_gas:['gas','var(--c-gas)'],         s_tilt:['tilt','var(--c-tamper)'],

1574	  s_wcap:['weight','var(--c-weight)'],  s_sclosed:['servo','var(--c-lid)'],

1575	  s_sopen:['servo','var(--c-lid)'],     s_pin:['pin','var(--sim)'],

1576	  s_known:['weight','var(--c-weight)'], s_pirhold:['stopwatch','var(--c-motion)'],

1577	  s_buzz:['speaker','var(--warn)'],

1578	  s_toff:['temp','var(--c-temp)'],      s_tref:['temp','var(--c-temp)'],

1579	  s_hoff:['humid','var(--c-humid)'],    s_poff:['press','var(--c-press)']

1580	};

1581	const BTN_STYLES={

1582	  btnSave:['save','tint','var(--ok)','SAVE SETTINGS'],

1583	  btnCancel:['undo','tint','var(--warn)','CANCEL CHANGES'],

1584	  btnReload:['reload','tint','var(--c-humid)','RELOAD FROM BIN'],

1585	  btnExport:['download','tint','var(--c-weight)','EXPORT BIN LIST'],

1586	  btnTare:['capture','tint','var(--c-weight)','TARE / ZERO'],

1587	  btnCalW:['weight','tint','var(--cal)','CALIBRATE'],

1588	  btnRefT:['capture','tint','var(--c-temp)','SET OFFSET'],

1589	  btnGasReset:['reload','tint','var(--c-gas)','RESET GAS BASELINE'],

1590	  btnSim:['flask','solid','var(--sim)','APPLY'],

1591	  btnSimOff:['reload','tint','var(--ok)','ALL BACK TO AUTO']

1592	};

1593	function decorateStatics(){

1594	  for(const id in LABEL_ICONS){

1595	    const inp=document.getElementById(id);if(!inp)continue;

1596	    const lab=inp.closest('.field')&&inp.closest('.field').querySelector('label');

1597	    if(!lab||lab.dataset.deco)continue;

1598	    lab.dataset.deco='1';

1599	    lab.style.setProperty('--lc',LABEL_ICONS[id][1]);

1600	    lab.insertAdjacentHTML('afterbegin',icon(LABEL_ICONS[id][0]));

1601	  }

1602	  for(const id in BTN_STYLES){

1603	    const el=document.getElementById(id);if(!el||el.dataset.deco)continue;

1604	    const [ic,cls,cc,txt]=BTN_STYLES[id];

1605	    el.dataset.deco='1';

1606	    el.classList.remove('accent');el.classList.add(cls);

1607	    el.style.setProperty('--cc',cc);

1608	    el.innerHTML=icon(ic)+'<span>'+txt+'</span>';

1609	    el.style.display='inline-flex';el.style.alignItems='center';el.style.gap='9px';

1610	  }

1611	  $$('[data-capture]').forEach(b=>{

1612	    if(b.dataset.deco)return;b.dataset.deco='1';

1613	    b.classList.add('tint');b.style.setProperty('--cc','var(--cal)');

1614	    b.innerHTML=icon('capture')+'<span>USE NOW</span>';

1615	    b.style.display='inline-flex';b.style.alignItems='center';b.style.gap='7px';

1616	  });

1617	  const al=document.getElementById('s_autolock');

1618	  if(al&&!al.parentElement.dataset.deco){

1619	    al.parentElement.dataset.deco='1';

1620	    al.parentElement.style.setProperty('--lc','var(--sim)');

1621	  }

1622	}

1623

1624	function paintHeadings(){

1625	  const H={binsTitle:['bins','BINS'],collectHead:['rank','COLLECTION PRIORITY'],alertHead:['clock','ALERT HISTORY'],

1626	    fillHead:['fill','FILL LEVEL'],envHead:['temp','ENVIRONMENT']};

1627	  for(const id in H){const el=document.getElementById(id);if(el)el.innerHTML=icon(H[id][0])+'<span>'+H[id][1]+'</span>';}

1628	  const bc=$('#btnClear');

1629	  if(bc&&!bc.dataset.deco){bc.dataset.deco='1';bc.classList.add('tint');

1630	    bc.style.setProperty('--cc','var(--c-tamper)');

1631	    bc.innerHTML=icon('trash')+'<span>DELETE</span>';

1632	    bc.style.display='inline-flex';bc.style.alignItems='center';bc.style.gap='8px';}

1633	  const bm=$('#btnMarkAll');

1634	  if(bm&&!bm.dataset.deco){bm.dataset.deco='1';bm.classList.add('tint');

1635	    bm.style.setProperty('--cc','var(--ok)');

1636	    bm.innerHTML=icon('check')+'<span>MARK READ</span>';

1637	    bm.style.display='inline-flex';bm.style.alignItems='center';bm.style.gap='8px';}

1638	  const TH={thSensor:['sliders','SENSOR','var(--c-humid)'],thMode:['power','MODE','var(--sim)'],

1639	    thValue:['capture','MANUAL VALUE','var(--cal)']};

1640	  for(const id in TH){const el=document.getElementById(id);

1641	    if(el)el.innerHTML='<span class="thh" style="--cc:'+TH[id][2]+'">'+icon(TH[id][0])+TH[id][1]+'</span>';}

1642	  $$('#filterChips .fchip[data-ico]').forEach(c=>{if(!c.querySelector('svg'))c.insertAdjacentHTML('afterbegin',icon(c.dataset.ico));});

1643	  const SEC={range:'ruler',limits:'bell',lid:'servo',weight:'weight',air:'gas',touch:'capture'};

1644	  /* Each group folds, closed at first like the app, so the one you need is a short scroll away.

1645	     The summary on the right says what is inside without opening it. */

1646	  $$('.sect').forEach(d=>{

1647	    const h=d.querySelector('h2');if(!h||h.dataset.deco)return;h.dataset.deco='1';

1648	    h.removeAttribute('style');

1649	    const body=document.createElement('div');body.className='sbody';

1650	    while(h.nextSibling)body.appendChild(h.nextSibling);

1651	    d.appendChild(body);

1652	    /* A div, not a button: the groups sit in a fieldset that is disabled until calibration is on. */

1653	    const btn=document.createElement('div');btn.setAttribute('role','button');btn.tabIndex=0;btn.className='shead';btn.setAttribute('aria-expanded','false');

1654	    btn.innerHTML=icon(SEC[d.dataset.sec]||'sliders')+'<span class="stitle">'+h.textContent+'</span><span class="ssum"></span>'+

1655	      '<svg class="ico schev" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2.2" stroke-linecap="round" stroke-linejoin="round"><path d="M6 9l6 6 6-6"/></svg>';

1656	    h.replaceWith(btn);

1657	  });

1658	  const subs={btnTare:'SETS 0 KG POINT',btnCalW:'SETS KG PER STEP'};

1659	  for(const id in subs){const el=document.getElementById(id);

1660	    if(el&&!el.querySelector('.sub'))el.querySelector('span').insertAdjacentHTML('beforeend','<em class="sub">'+subs[id]+'</em>');}

1661	  updateThemeIcon();

1662	}

1663	function updateThemeIcon(){

1664	  const dark=document.documentElement.getAttribute('data-theme')==='dark';

1665	  $('#themeIco').innerHTML=ICONS.sunmoon(dark);

1666	}

1667

1668	// Web Bluetooth UUIDs of the bin's setup service (BT_*_UUID in config.h).

1669	// Web Bluetooth works only in Chrome or Edge, and only on https:// or

1670	// localhost pages, so it is unavailable from a bin's http:// page (btWhy

1671	// below tells the user to use the phone app or a local http.server).

1672	const BT={

1673	  SVC:'b1a70001-5c2e-4f8a-9d33-0a9c2b6f17c1',

1674	  SSID:'b1a70002-5c2e-4f8a-9d33-0a9c2b6f17c1',

1675	  PASS:'b1a70003-5c2e-4f8a-9d33-0a9c2b6f17c1',

1676	  CTRL:'b1a70004-5c2e-4f8a-9d33-0a9c2b6f17c1',

1677	  STAT:'b1a70005-5c2e-4f8a-9d33-0a9c2b6f17c1',

1678	  SCAN:'b1a70006-5c2e-4f8a-9d33-0a9c2b6f17c1'

1679	};

1680	const BT_STEPS=[['bluetooth','PAIR + CODE'],['radar','PICK NETWORK'],['key','SEND'],['power','BLUETOOTH OFF']];

1681	const bt={dev:null,ch:{},step:0,demo:false,ip:'',name:'',busy:false,off:false,pollT:null,waitT:null};

1682	function btRender(){

1683	  $('#btSteps').innerHTML=BT_STEPS.map((s,i)=>

1684	    '<div class="btstep '+(bt.step>i?'done':bt.step===i?'on':'')+'">'+icon(bt.step>i?'check':s[0])+'<b>'+s[1]+'</b></div>').join('');

1685	  const paired=bt.step>0,op=state.role==='operator';

1686	  ['bt_ssid','bt_pass'].forEach(id=>{const e=$('#'+id);if(e)e.disabled=!paired});

1687	  ['btScan','btEye'].forEach(id=>{const e=$('#'+id);if(e)e.disabled=!paired});

1688	  $('#btSend').disabled=!paired;

1689	  $('#btOff').disabled=bt.step<3;

1690	  const note=$('#btRoleNote');

1691	  if(note)note.textContent='';

1692	  if($('#cloudHead')&&$('#cloudHead').innerHTML)cloudRender();

1693	  const st=$('#btState');

1694	  st.dataset.on=paired?'1':'0';

1695	  st.innerHTML=icon('bluetooth')+'<span>'+(bt.demo&&paired?'Paired (demo)':paired?'Paired':'Not paired')+'</span>';

1696	}

1697	/* ---------- cloud backup (Firebase), from Garbage-Guard: local Wi-Fi first, Firebase second ---------- */

1698	const FB_DEF={url:'',key:'',email:'',pass:''};

1699	const cloud={working:Object.assign({},FB_DEF),tested:false,msg:null,show:false};

1700	try{const v=JSON.parse(localStorage.getItem('smartbin.fb')||'null');if(v&&v.url){cloud.working=Object.assign({},FB_DEF,v);cloud.tested=true;}}catch(e){}

1701	function fbVals(){return {url:$('#fb_url').value.trim(),key:$('#fb_key').value.trim(),email:$('#fb_email').value.trim(),pass:$('#fb_pass').value}}

1702	function fbFill(v){$('#fb_url').value=v.url;$('#fb_key').value=v.key;$('#fb_email').value=v.email;$('#fb_pass').value=v.pass}

1703	function fbCheck(f){

1704	  const mock=/^http:\/\/[^/]+$/.test(f.url);

1705	  if(!mock&&!/^https:\/\/.+\.(firebasedatabase\.app|firebaseio\.com)$/.test(f.url))return 'The database URL is wrong. It starts with https:// and ends in .firebasedatabase.app, with no / at the end.';

1706	  if(!mock&&!/^AIza.{35}$/.test(f.key))return 'The Web API key is wrong. It starts with AIza and is 39 characters long.';

1707	  if(!/^[^@\s]+@[^@\s]+\.[^@\s]+$/.test(f.email)||f.pass.length<6)return 'Wrong email or password.';

1708	  return '';

1709	}

1710	function cloudRender(){

1711	  const op=state.role==='operator';

1712	  ['fb_url','fb_key','fb_email','fb_pass','fbSave','fbRestore','fbSend','ts_server','btnTouchCal','btnTouchReset'].forEach(id=>{const e=$('#'+id);if(e)e.disabled=!op});

1713	  const st=$('#cloudState');st.dataset.on=cloud.tested?'1':'0';

1714	  st.innerHTML=icon('cloud')+'<span>'+(cloud.tested?'Working':'Not tested')+'</span>';

1715	  const m=$('#cloudMsg');m.classList.toggle('hide',!cloud.msg);if(cloud.msg){m.className='cloudmsg '+cloud.msg[0];m.textContent=cloud.msg[1];}

1716	  $('#cloudRows').innerHTML=state.bins.slice(0,6).map(b=>{const s=cloudRowText(b);

1717	    return '<div class="cloudrow" style="--cc:'+s[0]+'"><span class="dot"></span><span class="cn">'+esc(b.name)+'</span><span class="cs">'+s[1]+'</span></div>'}).join('')+

1718	    (state.bins.length>6?'<div class="cloudrow" style="--cc:var(--ink2)"><span class="cn">and '+(state.bins.length-6)+' more</span></div>':'')+

1719	    (fbReady()?'<div class="cloudrow" style="--cc:var(--c-cloud)"><span class="dot"></span><span class="cn">This dashboard</span><span class="cs">'+(FB.watching?'tells the bins it is watching, so they write every 5 s':'reads the bins over Wi-Fi, so it is not asking the cloud')+'</span></div>':'');

1720	  $('#cloudRoleNote').textContent=op?'':'You are signed in as a collector. You can test the details, but only an operator can change or save them.';

1721	}

1722	function cloudDecorate(){

1723	  const defs={fbTest:['radar','tint','var(--c-cloud)','TEST'],fbSave:['save','solid','var(--c-cloud)','TEST AND SAVE'],fbRestore:['undo','tint','var(--ink2)','RESTORE WORKING'],fbEye:['eye','tint','var(--ink2)','SHOW']};

1724	  for(const id in defs){const el=$('#'+id);if(!el)continue;const [ic,cls,cc,tx]=defs[id];

1725	    el.classList.add(cls);el.style.setProperty('--cc',cc);el.innerHTML=icon(ic)+'<span>'+tx+'</span>';

1726	    el.style.display='inline-flex';el.style.alignItems='center';el.style.gap='8px';}

1727	  $('#cloudHead').innerHTML=icon('cloud')+'<span>CLOUD BACKUP (FIREBASE)</span>';

1728	  fbFill(cloud.working);cloudRender();

1729	  $('#fbEye').addEventListener('click',()=>{cloud.show=!cloud.show;$('#fb_pass').type=cloud.show?'text':'password';$('#fbEye').innerHTML=icon(cloud.show?'eyeoff':'eye')+'<span>'+(cloud.show?'HIDE':'SHOW')+'</span>';});

1730	  const test=async save=>{const v=fbVals(),e=fbCheck(v);

1731	    if(e){cloud.msg=['bad',e+(save?' Nothing was saved.':'')];cloudRender();return;}

1732	    cloud.msg=['ok','Signing in\u2026'];cloudRender();

1733	    try{

1734	      const n=await fbTest(v);

1735	      cloud.tested=true;

1736	      if(save){cloud.working=Object.assign({},v);fbUse(v);

1737	        try{localStorage.setItem('smartbin.fb',JSON.stringify(v))}catch(err){}

1738	        cloud.msg=['ok','Signed in and found '+n+' bin'+(n===1?'':'s')+' in the database. Saved on this computer. Press SEND TO BINS to give the bins this account too.'];

1739	        state.bins.forEach(b=>{if(!b.hist.length&&isLive(b))b.hist=[]});sfx('saved');}

1740	      else cloud.msg=['ok','Signed in as '+v.email+' and found '+n+' bin'+(n===1?'':'s')+'. Nothing was saved yet.'];

1741	    }catch(err){cloud.msg=['bad',(err.message||String(err))+(save?' Nothing was saved.':'')];sfx('denied');}

1742	    cloudRender();};

1743	  $('#fbTest').addEventListener('click',()=>test(false));

1744	  $('#fbSave').addEventListener('click',()=>test(true));

1745	  $('#fbRestore').addEventListener('click',()=>{fbFill(cloud.working);cloud.msg=['ok','Put back the last details that passed a test.'];cloudRender();});

1746	  const send=$('#fbSend');

1747	  if(send){send.classList.add('tint');send.style.setProperty('--cc','var(--c-cloud)');

1748	    send.innerHTML=icon('bins')+'<span>SEND TO BINS</span>';send.style.display='inline-flex';send.style.alignItems='center';send.style.gap='8px';

1749	    send.addEventListener('click',sendCloudToBins);}

1750	  const tsIn=$('#ts_server');

1751	  if(tsIn){tsIn.value=TS.server;tsIn.addEventListener('change',()=>{TS.server=(tsIn.value.trim()||'https://api.thingspeak.com').replace(/\/+$/,'');

1752	    tsIn.value=TS.server;try{localStorage.setItem('smartbin.ts',TS.server)}catch(e){}toast('ThingSpeak server set to '+TS.server+'.');});}

1753	}

1754	function btDecorate(){

1755	  const defs={btPair:['bluetooth','tint','var(--c-bt)','PAIR A BIN'],btSend:['key','solid','var(--c-bt)','SEND WI-FI'],

1756	    btOff:['power','tint','var(--warn)','TURN BT OFF'],btScan:['radar','tint','var(--c-bt)','SCAN'],

1757	    btEye:['eye','tint','var(--ink2)','SHOW']};

1758	  for(const id in defs){const el=$('#'+id);if(!el)continue;

1759	    const [ic,cls,cc,tx]=defs[id];

1760	    el.classList.add(cls);el.style.setProperty('--cc',cc);

1761	    el.innerHTML=icon(ic)+'<span>'+tx+'</span>';

1762	    el.style.display='inline-flex';el.style.alignItems='center';el.style.gap='8px';}

1763	  $('#btHead').innerHTML=icon('bluetooth')+'<span>BLUETOOTH SETUP</span>';

1764	  $('#btWhy').textContent=btSupported()

1765	    ?'Web Bluetooth is ready in this browser.'

1766	    :'This page cannot use Bluetooth: browsers allow it only on secure pages, and a bin serves the dashboard over plain http. Use the Smart-Bin phone app, or on a laptop run "python -m http.server" in the Web Dashboard folder and open http://localhost:8000 in Chrome or Edge. Demo bins still walk through the steps.';

1767	  btRender();

1768	}

1769	function btSupported(){return typeof navigator!=='undefined'&&!!navigator.bluetooth}

1770	const btWait=ms=>new Promise(r=>setTimeout(r,ms));

1771	const btDec=v=>new TextDecoder().decode(v);

1772	function btMsg(t){$('#btMsg').textContent=t}

1773	const btDevs=new WeakSet();

1774	/* Connect and find the five characteristics. A laptop or phone that remembers an old

1775	   pairing often drops the first try, so it tries 3 times. */

1776	// Bluetooth Wi-Fi setup: connect, read SCAN, write SSID and PASS, write 1 to

1777	// CTRL, then follow STAT until CONNECTED or FAILED (see ble_setup.cpp).

1778	async function btConnect(){

1779	  for(let i=0;i<3;i++){

1780	    try{

1781	      if(i)btMsg('The bin dropped the link, trying again ('+(i+1)+' of 3)...');

1782	      const srv=bt.dev.gatt.connected?bt.dev.gatt:await bt.dev.gatt.connect();

1783	      await btWait(400);

1784	      const svc=await srv.getPrimaryService(BT.SVC);

1785	      bt.ch={};

1786	      for(const k of ['SSID','PASS','CTRL','STAT','SCAN']){try{bt.ch[k]=await svc.getCharacteristic(BT[k]);}catch(e){}}

1787	      if(bt.ch.STAT&&bt.ch.SSID)return true;

1788	    }catch(e){try{bt.dev.gatt.disconnect()}catch(_){}}

1789	    await btWait(1200);

1790	  }

1791	  return false;

1792	}

1793	/* Every characteristic is encrypted, so the first read makes Windows or the phone ask for

1794	   the code the bin shows. That can take as long as the person needs to type it, so the

1795	   read is tried again while the code (60 s) is still good. */

1796	async function btSecure(){

1797	  const end=Date.now()+70000;let last=null;

1798	  while(Date.now()<end){

1799	    try{return btDec(await bt.ch.STAT.readValue());}

1800	    catch(e){last=e;

1801	      if(!bt.dev.gatt.connected&&!(await btConnect()))throw e;

1802	      await btWait(1500);}

1803	  }

1804	  throw last||new Error('No code was typed in time');

1805	}

1806	function btWatch(dev){

1807	  if(btDevs.has(dev))return;

1808	  btDevs.add(dev);

1809	  dev.addEventListener('gattserverdisconnected',btLost);

1810	}

1811	function btLost(){

1812	  if(bt.demo||bt.busy)return;

1813	  if(bt.step===2){btMsg('The Bluetooth link dropped while the bin joins the Wi-Fi. Reconnecting to read the result...');btRejoin();return;}

1814	  if(bt.step===1){bt.step=0;btRender();sfx('bin_lost');

1815	    btMsg('The bin closed the Bluetooth link. On the bin, open WI-FI SETUP again, then press PAIR A BIN.');}

1816	}

1817	async function btRejoin(){

1818	  if(await btConnect()){try{await bt.ch.STAT.startNotifications();bt.ch.STAT.addEventListener('characteristicvaluechanged',btOnNotify);}catch(e){}btReadStat();}

1819	}

1820	/* A notification is cut to 20 bytes on a small link, so a reply that does not parse is read in full. */

1821	function btOnNotify(e){

1822	  let j=null;try{j=JSON.parse(btDec(e.target.value))}catch(err){}

1823	  if(j)btStatus(j);else btReadStat();

1824	}

1825	async function btReadStat(){

1826	  if(!bt.ch.STAT)return;

1827	  try{btStatus(JSON.parse(btDec(await bt.ch.STAT.readValue())));}catch(e){}

1828	}

1829	function btStop(){clearInterval(bt.pollT);clearTimeout(bt.waitT);bt.pollT=bt.waitT=null;}

1830	async function btPair(){

1831	  const b=byId(state.selSet);

1832	  if(bt.busy)return;

1833	  btStop();

1834	  if(!btSupported()){

1835	    if(b&&b.demo){bt.demo=true;btCodeStart();return;}

1836	    sfx('denied');

1837	    btMsg(location.protocol==='http:'&&location.hostname!=='localhost'

1838	      ?'This page came from the bin over plain http, and browsers only allow Bluetooth on secure pages. Use the Smart-Bin phone app, or open the dashboard file on a laptop as the note above says. Demo bins can still walk through the steps.'

1839	      :'This browser has no Web Bluetooth. Use Chrome or Edge as the note above says, or the Smart-Bin phone app. Demo bins can still walk through the steps.');

1840	    return;

1841	  }

1842	  bt.demo=false;bt.busy=true;bt.step=0;bt.ip='';btRender();

1843	  $('#btResult').classList.add('hide');

1844	  try{

1845	    btMsg('Pick SmartBin-01 or SmartBin-02 in the browser list.');

1846	    bt.dev=await navigator.bluetooth.requestDevice({filters:[{services:[BT.SVC]},{namePrefix:'SmartBin'}],optionalServices:[BT.SVC]});

1847	    bt.name=bt.dev.name||'the bin';

1848	    btWatch(bt.dev);

1849	    if(!(await btConnect())){

1850	      sfx('bt_wrong');

1851	      btMsg(bt.name+' keeps dropping the Bluetooth link. Windows (or the phone) most likely remembers it from an earlier pairing. '+

1852	        'Open Settings > Bluetooth & devices, remove '+bt.name+', then press PAIR A BIN again. On the bin, cancel and open WI-FI SETUP again first.');

1853	      return;

1854	    }

1855	    btMsg('Connected to '+bt.name+'. The bin now shows a 6-digit code. Type it into the pairing window Windows or your phone opens, not on this page. The code lasts 60 s.');

1856	    await btSecure();

1857	    try{await bt.ch.STAT.startNotifications();bt.ch.STAT.addEventListener('characteristicvaluechanged',btOnNotify);}catch(e){}

1858	    sfx('bt_paired');

1859	    bt.step=1;btRender();

1860	    btMsg('Paired with '+bt.name+'. The link is encrypted now. Pick the Wi-Fi network next.');

1861	    bt.busy=false;

1862	    btScan();

1863	  }catch(e){

1864	    if(e&&e.name==='NotFoundError')btMsg('No bin was picked. Press PAIR A BIN when the bin shows WI-FI SETUP.');

1865	    else{

1866	      sfx('bt_wrong');

1867	      btMsg('Pairing did not finish ('+(e&&e.message||e)+'). If the code was wrong, the bin shows a new one; after 3 wrong codes it waits 60 s. '+

1868	        'If it keeps failing, remove '+(bt.name||'the bin')+' in Settings > Bluetooth & devices and press PAIR A BIN again.');

1869	    }

1870	    try{bt.dev&&bt.dev.gatt.disconnect()}catch(_){}

1871	    bt.step=0;btRender();

1872	  }finally{bt.busy=false;}

1873	}

1874	function btShowNets(nets){

1875	  const list=nets.map(n=>typeof n==='string'?{ssid:n}:n).filter(n=>n&&n.ssid);

1876	  $('#bt_nets').innerHTML=list.map(n=>'<option value="'+esc(n.ssid)+'">').join('');

1877	  const cur=$('#bt_ssid').value.trim();

1878	  $('#btNetList').innerHTML=list.map(n=>'<button type="button" class="btn sm" data-ssid="'+esc(n.ssid)+'" aria-pressed="'+(n.ssid===cur)+'">'+

1879	    icon('wifi',true)+esc(n.ssid)+(Number.isFinite(+n.rssi)&&n.rssi!==undefined?' <small>'+n.rssi+' dBm</small>':'')+'</button>').join('');

1880	}

1881	function btFakeNets(){

1882	  btShowNets([{ssid:'HomeWiFi-2G',rssi:-48},{ssid:'TUP-Manila',rssi:-61},{ssid:'Smart_Bin_Lab',rssi:-70},{ssid:'PLDTHOMEFIBR24',rssi:-79}]);

1883	}

1884	/* The bin scans when WI-FI SETUP opens. An empty list means it is still scanning, so try again after 5 s. */

1885	async function btScan(){

1886	  const tail=' Tap one to fill in the name, or type it yourself.';

1887	  sfx('tap');

1888	  if(bt.demo){btFakeNets();btMsg('Demo networks listed.'+tail);return;}

1889	  if(!bt.ch.SCAN){btMsg('This bin does not share a network list, so type the name yourself.');return;}

1890	  for(let i=0;i<3;i++){

1891	    try{

1892	      const nets=JSON.parse(btDec(await bt.ch.SCAN.readValue()));

1893	      if(nets.length){btShowNets(nets);btMsg('Found '+nets.length+' network'+(nets.length===1?'':'s')+' (strongest first). Only 2.4 GHz ones work.'+tail);return;}

1894	      btMsg('The bin is still scanning for networks. Trying again in 5 s...');

1895	    }catch(e){btMsg('Could not read the network list ('+(e.message||e)+'). Trying again in 5 s...');}

1896	    await btWait(5000);

1897	    if(bt.step!==1)return;

1898	  }

1899	  btMsg('The bin found no networks. Check that the router is on 2.4 GHz and close by, press SCAN again, or type the name yourself.');

1900	}

1901	function btStatus(j){

1902	  if(bt.step!==2)return;

1903	  const st=String(j.state||'').toUpperCase();

1904	  if(st==='CONNECTED'&&j.ip){btStop();bt.ip=j.ip;bt.step=3;btRender();btShowResult(j.ssid);}

1905	  else if(st==='FAILED'){btStop();bt.step=1;btRender();sfx('wifi_failed');

1906	    btMsg('The bin could not join: '+(j.why||'it gave no reason')+'. It went back to its old Wi-Fi. Check the name and password, then press SEND WI-FI again.');}

1907	  else if(st==='CONNECTING')btMsg('The bin is joining '+($('#bt_ssid').value.trim()||'the network')+'. This takes up to 15 s.');

1908	}

1909	async function btSend(){

1910	  const ssid=$('#bt_ssid').value.trim(),pass=$('#bt_pass').value;

1911	  if(!ssid){sfx('denied');btMsg('Type the Wi-Fi name first.');return;}

1912	  if(ssid.length>32||pass.length>63){sfx('denied');btMsg('The Wi-Fi name can be 32 characters at most and the password 63.');return;}

1913	  if(pass&&pass.length<8){sfx('denied');btMsg('A Wi-Fi password has at least 8 characters. Leave it empty only for an open network.');return;}

1914	  sfx('confirm');

1915	  bt.step=2;btRender();$('#btResult').classList.add('hide');

1916	  btMsg('Sending the Wi-Fi name and password and waiting for the bin to join...');

1917	  if(bt.demo){

1918	    const b=byId(state.selSet);

1919	    bt.waitT=setTimeout(()=>{

1920	      if(/fail|wrong/i.test(ssid)){btStatus({state:'FAILED',why:'Network not found'});return;}

1921	      btStatus({state:'CONNECTED',ip:'192.168.1.'+(101+Math.max(0,state.bins.indexOf(b))),ssid});},1800);

1922	    return;

1923	  }

1924	  try{

1925	    if(!bt.dev.gatt.connected&&!(await btConnect()))throw new Error('the bin is out of reach');

1926	    const enc=new TextEncoder();

1927	    await bt.ch.SSID.writeValue(enc.encode(ssid));

1928	    await bt.ch.PASS.writeValue(enc.encode(pass));

1929	    await bt.ch.CTRL.writeValue(Uint8Array.of(1));

1930	  }catch(e){sfx('cmd_fail');btMsg('Could not send: '+(e.message||e)+'. Press SEND WI-FI again.');bt.step=1;btRender();return;}

1931	  /* Notifications can be missed, so read the status every 2 s too. The bin gives up after 15 s. */

1932	  bt.pollT=setInterval(()=>{if(bt.step===2&&bt.dev.gatt.connected)btReadStat();},2000);

1933	  bt.waitT=setTimeout(async()=>{

1934	    if(bt.step!==2)return;

1935	    await btReadStat();

1936	    if(bt.step!==2)return;

1937	    btStop();bt.step=1;btRender();sfx('wifi_failed');

1938	    btMsg('No answer from the bin after 25 s. Check the touchscreen: it shows whether it joined. Then press SEND WI-FI again or PAIR A BIN.');

1939	  },25000);

1940	}

1941	function btShowResult(ssid){

1942	  const b=byId(state.selSet),real=b&&!b.demo&&!bt.demo;

1943	  sfx('wifi_joined');

1944	  const num=(/(\d+)\s*$/.exec(bt.name||'')||[])[1];

1945	  const label=bt.demo?(b?'USE FOR '+esc(b.name):'USE'):real?'USE FOR '+esc(b.name):'ADD AS BIN '+(num||'');

1946	  const r=$('#btResult');r.classList.remove('hide');

1947	  r.innerHTML=icon('wifi',true)+'<span>Joined '+esc(ssid||'the network')+'. Address: <span class="ip">'+esc(bt.ip)+'</span></span>'+

1948	    '<button class="btn sm solid" style="--cc:var(--ok)" id="btUse">'+icon('check')+'<span>'+label.trim()+'</span></button>';

1949	  btMsg('Press '+label.trim()+' to store this address, then TURN BT OFF.');

1950	}

1951	function btUse(){

1952	  if(!bt.ip)return;

1953	  let b=byId(state.selSet);

1954	  if(!bt.demo&&(!b||b.demo)){

1955	    if(state.role!=='operator'&&state.bins.some(x=>!x.demo)){toast('Pick the bin in Settings first, or ask an operator to add it.');sfx('denied');return;}

1956	    const num=(/(\d+)\s*$/.exec(bt.name||'')||[])[1]||String(state.bins.filter(x=>!x.demo).length+1).padStart(2,'0');

1957	    b=state.bins.find(x=>!x.demo&&x.ip===bt.ip);

1958	    if(!b){b=newBin({id:'bin'+num+'-'+Date.now().toString(36),name:'Bin '+num,place:'',ip:bt.ip,cloud:'bin'+num});state.bins.unshift(b);}

1959	    state.selSet=b.id;

1960	  }

1961	  if(b.demo){toast(b.name+' is a demo bin, so nothing was stored.');return;}

1962	  b.ip=bt.ip;relink(b);persist();cardSig='';render();loadSettings();sfx('saved');

1963	  toast(b.name+' will now be read at '+bt.ip+'.');

1964	}

1965	async function btOff(){

1966	  btStop();

1967	  bt.off=true;

1968	  if(!bt.demo&&bt.ch.CTRL){try{await bt.ch.CTRL.writeValue(Uint8Array.of(2));}catch(e){}}

1969	  try{if(bt.dev&&bt.dev.gatt&&bt.dev.gatt.connected)bt.dev.gatt.disconnect();}catch(e){}

1970	  bt.step=4;btRender();bt.off=false;

1971	  sfx('bt_off');

1972	  btMsg('Bluetooth is off on the bin. It stays on its Wi-Fi until someone opens WI-FI SETUP again.');

1973	}

1974	const reduce=()=>window.matchMedia('(prefers-reduced-motion: reduce)').matches;

1975	function setAddCount(n){

1976	  state.addCount=clamp(n,1,6);

1977	  const c=$('#addCount');if(!c)return;

1978	  c.textContent=state.addCount;

1979	  $('#addLabel').textContent=state.addCount===1?'Add a bin':'Add '+state.addCount+' bins';

1980	  $('#addLess').disabled=state.addCount<=1;

1981	  $('#addMore').disabled=state.addCount>=6;

1982	}

1983	function tweenPct(el,b,to){

1984	  const paint=v=>{el.innerHTML=Math.round(v)+'<span>%</span>'};

1985	  const from=Number.isFinite(b._shown)?b._shown:0;

1986	  if(reduce()||(el.dataset.init&&Math.abs(to-from)<0.6)){b._shown=to;paint(to);return;}

1987	  el.dataset.init='1';

1988	  cancelAnimationFrame(b._raf||0);

1989	  const t0=performance.now(),dur=520;

1990	  const run=now=>{

1991	    const k=Math.min(1,(now-t0)/dur),e=1-Math.pow(1-k,3),v=from+(to-from)*e;

1992	    b._shown=v;paint(v);

1993	    if(k<1)b._raf=requestAnimationFrame(run);

1994	  };

1995	  b._raf=requestAnimationFrame(run);

1996	}

1997	function levelColor(p,c){return p>=(c.fill_th||80)?'var(--bad)':p>=50?'var(--warn)':'var(--ok)'}

1998	function statusColor(st,p,c){

1999	  if(st==='CALIBRATING')return 'var(--cal)';

2000	  if(st==='TAMPER')return 'var(--bad)';

2001	  if(st==='LOCKED')return 'var(--sim)';

2002	  if(st==='COLLECTING')return 'var(--cal)';

2003	  return levelColor(p,c);

2004	}

2005

2006	/* ---------- transport ---------- */

2007	// The bin's base URL. An IP without http:// gets it added.

2008	function binBase(b){const ip=String(b.ip||'').trim().replace(/\/+$/,'');return /^https?:\/\//.test(ip)?ip:'http://'+ip}

2009	// Every local call has a 1.5 s timeout so a dead bin cannot stall the loop.

2010	// callRaw returns the HTTP status (for 400/409 reasons); call() throws on

2011	// any non-2xx reply.

2012	async function callRaw(b,path,body){

2013	  if(!b.ip)throw new Error('no ip');

2014	  const ctrl=new AbortController(),timer=setTimeout(()=>ctrl.abort(),1500);

2015	  try{

2016	    const res=await fetch(binBase(b)+path,{method:body?'POST':'GET',

2017	      headers:body?{'Content-Type':'application/json'}:undefined,

2018	      body:body?JSON.stringify(body):undefined,signal:ctrl.signal,cache:'no-store'});

2019	    let j=null;try{j=await res.json()}catch(e){}

2020	    return {ok:res.ok&&(!j||j.ok!==false),status:res.status,j};

2021	  }finally{clearTimeout(timer)}

2022	}

2023	async function call(b,path,body){

2024	  if(!b.ip)throw new Error('no ip');

2025	  const ctrl=new AbortController(),timer=setTimeout(()=>ctrl.abort(),1500);

2026	  try{

2027	    const res=await fetch(binBase(b)+path,{method:body?'POST':'GET',

2028	      headers:body?{'Content-Type':'application/json'}:undefined,

2029	      body:body?JSON.stringify(body):undefined,signal:ctrl.signal,cache:'no-store'});

2030	    if(!res.ok)throw new Error('HTTP '+res.status);

2031	    return await res.json();

2032	  }finally{clearTimeout(timer)}

2033	}

2034	// Copies a status JSON from any route into b.data with safe defaults, so the

2035	// renderers never see undefined values.

2036	function normalise(b,j){

2037	  const d=b.data,n=(a,f)=>Number.isFinite(+a)?+a:f;

2038	  d.fill=clamp(n(j.fill??j.fill_percentage,d.fill),0,100);

2039	  d.distance_cm=n(j.distance_cm??j.distance,d.distance_cm);

2040	  d.weight_g=n(j.weight_g??j.weight_grams??j.weight,d.weight_g);

2041	  d.temp=n(j.temp??j.temperature,d.temp);

2042	  d.temp_raw=n(j.temp_raw,d.temp-(b.cal.temp_off||0));

2043	  d.humidity=n(j.humidity,d.humidity);

2044	  d.pressure=n(j.pressure??j.pressure_hpa,d.pressure);

2045	  d.gas=n(j.iaq??j.gas??j.gas_level,d.gas);

2046	  d.iaq_acc=n(j.iaq_acc,d.iaq_acc);

2047	  d.gas_k=n(j.gas_k,d.gas_k);

2048	  d.warmEnd=Number.isFinite(+j.iaq_warm_s)&&j.iaq_warm_s!==null?Date.now()+j.iaq_warm_s*1000:null;

2049	  d.tilt=n(j.tilt??j.tilt_deg,d.tilt);

2050	  if(j.motion!==undefined||j.pir!==undefined)d.motion=!!(j.motion??j.pir);

2051	  if(j.lid!==undefined||j.lid_state!==undefined)d.lid=(j.lid??j.lid_state)===1||String(j.lid??j.lid_state).toUpperCase()==='OPEN'?'OPEN':'CLOSED';

2052	  const sm=['OK','LOCKED','TAMPER','COLLECTING','CALIBRATING'];

2053	  d.status=typeof j.status==='number'?(sm[j.status]||'OK'):String(j.status||j.bin_status||'OK').toUpperCase();

2054	  if(j.cal)Object.assign(b.cal,j.cal);

2055	  if(j.touch)b.touch=j.touch;

2056	  if(Number.isFinite(+j.sim_mask))b.sim.mask=+j.sim_mask;

2057	  d.faults=Array.isArray(j.faults)?j.faults:[];

2058	  d.tamper_phase=j.tamper_phase||'';

2059	  if(j.last_cmd)d.last_cmd=j.last_cmd;

2060	  if(j.fw)b.fw=j.fw;

2061	  if(Number.isFinite(+j.rssi))b.rssi=+j.rssi;

2062	}

2063	// --- Demo data: readings that drift like a real bin when no address is set ---

2064	function stepEnv(b){

2065	  const d=b.data,s=b.sim,on=bit=>(s.mask>>bit)&1;

2066	  /* BME680 demo: the air quality index drifts toward a level set by how full the bin is,

2067	     the raw temperature carries about 2 C of self-heating that the offset removes. */

2068	  d.gas=on(2)?+s.values.gas:Math.round(clamp(d.gas+((35+d.fill*2.2)-d.gas)*0.02+(Math.random()-0.5)*5,10,500));

2069	  if(on(3)){d.temp=+s.values.temp;d.temp_raw=d.temp-(b.cal.temp_off||0);d.humidity=+s.values.humidity;}

2070	  else{

2071	    d.temp_raw=clamp((d.temp_raw||30)+(Math.random()-0.5)*0.14+d.fill*0.0009,20,70);

2072	    d.temp=+(d.temp_raw+(b.cal.temp_off||0)).toFixed(2);

2073	    d.humidity=clamp(d.humidity+(Math.random()-0.5)*0.6,35,95);

2074	  }

2075	  d.pressure=+(clamp((d.pressure||1008)+(Math.random()-0.5)*0.06,985,1030)).toFixed(2);

2076	  d.iaq_acc=b._accT?Math.min(3,Math.floor((Date.now()-b._accT)/20000)):3;

2077	  d.warmEnd=b._accT&&d.iaq_acc===0?b._accT+20000:null;

2078	  if(d.iaq_acc>=3)b._accT=0;

2079	}

2080	const DEMO={speed:1,pick:null};

2081	const DEMO_SPEEDS=[[0,'Pause'],[1/60,'Real'],[1,'Normal'],[10,'Fast']];

2082	function demoInUse(){return DEMO.speed!==1||state.bins.some(b=>b.demo&&(b._demoLink||(b._hold&&Object.keys(b._hold).length)))}

2083	function demoPanelRender(){

2084	  const p=$('#demoPanel');if(!p)return;

2085	  const demos=state.bins.filter(b=>b.demo);

2086	  p.classList.toggle('hide',!state.demo||!demos.length);

2087	  if(!state.demo||!demos.length)return;

2088	  if(!demos.some(b=>b.id===DEMO.pick))DEMO.pick=demos[0].id;

2089	  const b=byId(DEMO.pick),link=b._demoLink||'';

2090	  const btn=(attr,val,label,ic,cc,on)=>'<button class="btn sm binbtn" style="--cc:'+cc+'" data-'+attr+'="'+val+'" aria-pressed="'+!!on+'">'+icon(ic)+'<span>'+label+'</span></button>';

2091	  const html='<h3 class="dhd">Fill speed</h3><div class="seg">'+DEMO_SPEEDS.map(x=>btn('dspeed',x[0],x[1].toUpperCase(),'stopwatch','var(--sim)',DEMO.speed===x[0])).join('')+'</div>'+

2092	    '<h3 class="dhd">Act out on a bin</h3><div class="seg">'+demos.map(x=>btn('dpick',x.id,esc(x.name),'fill','var(--sim)',x.id===DEMO.pick)).join('')+'</div>'+

2093	    '<div class="seg">'+btn('devent','full','FULL','fill','var(--bad)')+btn('devent','tamper','TAMPER','tamper','var(--c-tamper)')+

2094	      btn('devent','air','BAD AIR','gas','var(--c-gas)')+btn('devent','empty','EMPTY','truck','var(--cal)')+'</div>'+

2095	    '<h3 class="dhd">How '+esc(b.name)+' answers</h3><div class="seg">'+btn('dlink','','WI-FI','wifi','var(--c-link)',link==='')+

2096	      btn('dlink','fb','FIREBASE','cloud','var(--c-cloud)',link==='fb')+btn('dlink','ts','THINGSPEAK','chart','var(--c-ts)',link==='ts')+

2097	      btn('dlink','down','GO QUIET','power','var(--warn)',link==='down')+'</div>'+

2098	    '<div class="actions">'+btn('dreset','1','RESET','undo','var(--sim)')+'</div>';

2099	  const body=$('#demoBody');if(body.dataset.sig!==html){body.dataset.sig=html;body.innerHTML=html;}

2100	  $('#demoHead').innerHTML=icon('flask')+'<span>DEMO OPTIONS</span>';

2101	  $('#demoState').textContent=demoInUse()?'In use':'Not in use';

2102	}

2103	function demoAct(el){

2104	  const b=byId(DEMO.pick);tourPause();

2105	  if(el.dataset.dspeed!=null){DEMO.speed=+el.dataset.dspeed;sfx('choice');}

2106	  else if(el.dataset.dpick!=null){DEMO.pick=el.dataset.dpick;sfx('choice');}

2107	  else if(el.dataset.devent&&b){const d=b.data,c=b.cal,e=el.dataset.devent;

2108	    if(e==='full')d.fill=Math.min(99,c.fill_th+4);

2109	    if(e==='tamper')tourHold(b,{tilt:40});

2110	    if(e==='air'){tourHold(b,{gas:c.gas_th+45});setTimeout(()=>{tourDrop(b,['gas']);b.data.gas=60;},20000);}

2111	    if(e==='empty'){d.fill=0;d.weight_g=0;}

2112	    sfx('confirm');toast({full:b.name+' is now over its fill limit.',tamper:b.name+' was tipped. Reset it from its card.',

2113	      air:'Air quality spiked in '+b.name+'. It settles by itself.',empty:b.name+' is empty.'}[e]);}

2114	  else if(el.dataset.dlink!=null&&b){const v=el.dataset.dlink||null;

2115	    if(v==='down'&&b._demoLink!=='down')b._quietAt=Date.now();b._demoLink=v;sfx(v==='down'?'link_lost':v?'cloud':'link_up');}

2116	  else if(el.dataset.dreset){DEMO.speed=1;state.bins.forEach(x=>{if(x.demo){x._demoLink=null;x._hold={};}});sfx('confirm');toast('Demo options reset.');}

2117	  demoPanelRender();render();

2118	}

2119	function stepDemo(b){

2120	  const d=b.data,s=b.sim,on=bit=>(s.mask>>bit)&1,h=b._hold||{};

2121	  stepEnv(b);

2122	  if(h.gas!=null)d.gas=h.gas;

2123	  if(h.temp!=null){d.temp=h.temp;d.temp_raw=h.temp-(b.cal.temp_off||0);}

2124	  if(h.humidity!=null)d.humidity=h.humidity;

2125	  d.faults=h.faults||[];

2126	  if(d.status==='CALIBRATING'){d.lid='CLOSED';d.distance_cm=+(b.cal.d_empty-Math.random()*0.4).toFixed(1);return;}

2127	  if(on(0))d.fill=clamp(+s.values.fill,0,100);

2128	  else if(d.status!=='COLLECTING')d.fill=clamp(d.fill+(0.022+Math.random()*0.015)*DEMO.speed,0,100);

2129	  d.weight_g=on(1)?+s.values.weight:Math.round(clamp(d.fill/100*b.cal.weight_cap*0.92+Math.random()*25,0,5000));

2130	  d.tilt=h.tilt!=null?h.tilt:on(4)&&+s.values.tamper?38:clamp(Math.random()*2.2,0,90);

2131	  d.motion=on(5)?!!+s.values.motion:Math.random()<0.025;

2132	  d.distance_cm=+(b.cal.d_empty-(d.fill/100)*(b.cal.d_empty-b.cal.d_full)).toFixed(1);

2133	  if(d.status==='COLLECTING'){d.lid='OPEN';return;}

2134	  if(d.tilt>b.cal.tilt_th){d.status='TAMPER';d.lid='CLOSED';return;}

2135	  if(d.status==='TAMPER')d.status='OK';

2136	  if(d.status==='LOCKED'){d.lid='CLOSED';return;}

2137	  if(b.cal.auto_lock&&d.fill>=b.cal.fill_th){d.status='LOCKED';d.lid='CLOSED';return;}

2138	  d.lid=on(6)?(+s.values.lid?'OPEN':'CLOSED'):(d.motion?'OPEN':'CLOSED');

2139	}

2140	/* ---------- live links: the bin's own Wi-Fi first, then Firebase, then ThingSpeak ----------

2141	   ESP32 Integration Guide sections 4 to 7 and the Interrupt-Driven Design Guide section 5.

2142	   A bin with an IP address, a Firebase cloud name (once Firebase is saved) or a ThingSpeak

2143	   channel is live: it never shows made-up numbers, it shows the last real reading and

2144	   "not answering" instead. A bin with none of these keeps the demo data for class. */

2145	const ROUTE={local:['Wi-Fi','var(--c-link)','wifi'],fb:['Firebase','var(--c-cloud)','cloud'],ts:['ThingSpeak','var(--c-ts)','chart']};

2146	// A dashboard opened from an https:// page cannot call http:// bins (mixed

2147	// content), so local Wi-Fi is skipped there and only the clouds are used.

2148	const SECURE=location.protocol==='https:';

2149	const CLOUD_CMDS=['LOCK','UNLOCK','COLLECT'];

2150	const enc=encodeURIComponent;

2151	const CLIENT_ID=(function(){let v='';try{v=localStorage.getItem('smartbin.client')||''}catch(e){}

2152	  if(!v){v='dash-'+Math.random().toString(36).slice(2,10);try{localStorage.setItem('smartbin.client',v)}catch(e){}}return v})();

2153	const FB={cfg:null,token:'',until:0,uid:'',signing:null,watching:false,lastWatch:0,usageAt:0};

2154	const TS={server:'https://api.thingspeak.com'};

2155	try{const t=localStorage.getItem('smartbin.ts');if(t)TS.server=t}catch(e){}

2156	const links={};

2157	function L(b){return links[b.id]||(links[b.id]={es:null,esIp:'',esFails:0,esRetryAt:0,nextLocal:0,localFails:0,busyLocal:false,nextFb:0,busyFb:false,nextTs:0,busyTs:false})}

2158	function fbUse(v){FB.cfg=v&&v.url?Object.assign({},v):null;FB.token='';FB.until=0;}

2159	if(cloud.tested)fbUse(cloud.working);

2160	function fbReady(){return !!(FB.cfg&&FB.cfg.url&&FB.cfg.key&&FB.cfg.email&&FB.cfg.pass)}

2161	// A live bin never shows demo numbers. With no answer it shows its last real

2162	// reading and "not answering".

2163	function isLive(b){return !!((b.ip&&!SECURE)||(b.tsCh&&b.tsKey)||(fbReady()&&b.cloud))}

2164	function timeout(ms){return typeof AbortSignal!=='undefined'&&AbortSignal.timeout?AbortSignal.timeout(ms):undefined}

2165

2166	/* Firebase over plain REST, the same calls as the app's Cloud.kt. */

2167	function fbFriendly(m){

2168	  m=String(m||'');

2169	  if(/INVALID_LOGIN_CREDENTIALS|INVALID_PASSWORD|EMAIL_NOT_FOUND|INVALID_EMAIL/.test(m))return 'Wrong email or password.';

2170	  if(/API key not valid|API_KEY_INVALID/.test(m))return 'The Web API key is wrong. Copy it again from the Firebase console.';

2171	  if(/PASSWORD_LOGIN_DISABLED|OPERATION_NOT_ALLOWED/.test(m))return 'Email/Password sign-in is off in Firebase Authentication.';

2172	  if(/TOO_MANY_ATTEMPTS/.test(m))return 'Too many tries, wait a few minutes.';

2173	  if(/USER_DISABLED/.test(m))return 'This account is disabled.';

2174	  if(/Permission denied|HTTP 401/.test(m))return 'The account is not on the allowed list (sb_users), or the rules were not pasted.';

2175	  if(/HTTP 404/.test(m))return 'The database URL is wrong.';

2176	  if(/Failed to fetch|NetworkError|aborted|timed out|TimeoutError/i.test(m))return 'No internet, or the database URL is wrong.';

2177	  return m.length>90?m.slice(0,90):m;

2178	}

2179	async function fbSignIn(cfg){

2180	  const url=cfg.url.startsWith('http://')?cfg.url.replace(/\/+$/,'')+'/__auth/signInWithPassword?key='+enc(cfg.key)

2181	    :'https://identitytoolkit.googleapis.com/v1/accounts:signInWithPassword?key='+enc(cfg.key.trim());

2182	  let r,j={};

2183	  try{r=await fetch(url,{method:'POST',headers:{'Content-Type':'application/json'},

2184	    body:JSON.stringify({email:cfg.email.trim(),password:cfg.pass,returnSecureToken:true}),signal:timeout(10000)});

2185	    j=await r.json().catch(()=>({}));}

2186	  catch(e){throw new Error(fbFriendly(e.message||e))}

2187	  if(!r.ok)throw new Error(fbFriendly((j.error&&(j.error.message||j.error))||('HTTP '+r.status)));

2188	  return {token:j.idToken,uid:j.localId,until:Date.now()+Math.max(60,(+j.expiresIn||3600)-300)*1000};

2189	}

2190	async function fbToken(){

2191	  if(FB.token&&Date.now()<FB.until)return FB.token;

2192	  if(!FB.signing)FB.signing=fbSignIn(FB.cfg).then(t=>{FB.token=t.token;FB.uid=t.uid;FB.until=t.until;return t.token}).finally(()=>{FB.signing=null});

2193	  return FB.signing;

2194	}

2195	async function fbRest(method,path,body,query,cfg){

2196	  const c=cfg||FB.cfg;

2197	  if(!c)throw new Error('Firebase is not set up');

2198	  const tok=cfg?cfg._token:await fbToken();

2199	  let r,t='';

2200	  try{r=await fetch(c.url.replace(/\/+$/,'')+'/'+path+'.json?auth='+enc(tok)+(query||''),{method,

2201	    headers:body?{'Content-Type':'application/json'}:undefined,body:body?JSON.stringify(body):undefined,signal:timeout(8000),cache:'no-store'});

2202	    t=await r.text();}

2203	  catch(e){throw new Error(fbFriendly(e.message||e))}

2204	  if(r.status===401&&!cfg)FB.token='';

2205	  if(!r.ok){let m='HTTP '+r.status;try{const j=JSON.parse(t);m=(j.error&&(j.error.message||j.error))||m}catch(e){}throw new Error(fbFriendly(m))}

2206	  return t&&t.trim()!=='null'?JSON.parse(t):null;

2207	}

2208	async function fbTest(v){

2209	  const t=await fbSignIn(v),c=Object.assign({},v,{_token:t.token});

2210	  const me=await fbRest('GET','sb_users/'+enc(t.uid),null,'',c);

2211	  if(me===null)throw new Error('The account is not on the allowed list (sb_users).');

2212	  const bins=await fbRest('GET','sb/bins',null,'&shallow=true',c);

2213	  return bins&&typeof bins==='object'?Object.keys(bins).length:0;

2214	}

2215	async function sendCloudToBins(){

2216	  if(state.role!=='operator'){toast('Only an operator can send the cloud account to the bins.');return;}

2217	  const v=fbVals(),e=fbCheck(v);

2218	  if(e){toast(e);return;}

2219	  const targets=state.bins.filter(b=>b.ip&&!SECURE);

2220	  if(!targets.length){toast('No bin has an IP address yet, so there is nothing to send to.');return;}

2221	  let ok=0;const miss=[];

2222	  await Promise.all(targets.map(async b=>{try{const r=await callRaw(b,'/api/cloud',{url:v.url,key:v.key,email:v.email,pass:v.pass});

2223	    if(r.ok)ok++;else miss.push(b.name)}catch(err){miss.push(b.name)}}));

2224	  sfx(ok?'cmd_ok':'cmd_fail');

2225	  cloud.msg=[ok?'ok':'bad','Sent to '+ok+' of '+targets.length+' bins'+(miss.length?'. Not answering: '+miss.join(', '):'')+'. They sign in again within a few seconds.'];

2226	  cloudRender();

2227	}

2228	async function fbWatchTick(now){

2229	  if(!fbReady())return;

2230	  const want=!document.hidden&&state.bins.some(b=>isLive(b)&&b.cloud&&b.route!=='local');

2231	  if(want&&now-FB.lastWatch>=10000){

2232	    FB.lastWatch=now;

2233	    try{await fbRest('PUT','sb/watchers/'+CLIENT_ID,{ts:{'.sv':'timestamp'},device:(navigator.userAgentData&&navigator.userAgentData.platform)||navigator.platform||'Computer',

2234	      kind:'dashboard',mode:'foreground',bin:'all'});FB.watching=true;}catch(e){}

2235	  }else if(!want&&FB.watching)fbUnwatch();

2236	}

2237	function fbUnwatch(){

2238	  if(!FB.watching||!fbReady()||!FB.token)return;

2239	  FB.watching=false;FB.lastWatch=0;

2240	  try{fetch(FB.cfg.url.replace(/\/+$/,'')+'/sb/watchers/'+CLIENT_ID+'.json?auth='+enc(FB.token),{method:'DELETE',keepalive:true}).catch(()=>{})}catch(e){}

2241	}

2242	async function fbUsageTick(now){

2243	  if(!fbReady()||now-FB.usageAt<60000)return;

2244	  FB.usageAt=now;

2245	  try{

2246	    const u=await fbRest('GET','sb/usage');

2247	    if(!u)return;

2248	    const bytes=+(u.bytes_month??u.down_bytes)||0,lim=+u.limit_bytes||10737418240,frac=bytes/lim;

2249	    const gb=bytes/1073741824;

2250	    const fill=$('#useFill');if(fill)fill.style.width=Math.min(100,Math.max(0.5,frac*100)).toFixed(1)+'%';

2251	    const p=$('#cloudUse');

2252	    if(p)p.textContent='About '+(gb<0.1?(bytes/1048576).toFixed(0)+' MB':gb.toFixed(2)+' GB')+' of the 10 GB the free Firebase plan allows this month ('+

2253	      (frac*100).toFixed(1)+'%, an estimate from the bins; the exact figure is in the Firebase console). Bins write every 5 s while someone watches over the cloud, every minute otherwise, and at once when an alert starts.';

2254	  }catch(e){}

2255	}

2256

2257	/* ThingSpeak: field1 fill, 2 weight, 3 temperature, 4 humidity, 5 IAQ, 6 pressure, 7 tilt, 8 motion + 2 lid + 4 status. */

2258	function tsMap(o){

2259	  const f=n=>{const v=parseFloat(o['field'+n]);return Number.isFinite(v)?v:undefined};

2260	  const flags=(f(8)||0)|0;

2261	  return {fill:f(1),weight_g:f(2),temp:f(3),humidity:f(4),iaq:f(5),pressure:f(6),tilt:f(7),motion:!!(flags&1),lid:(flags&2)?'OPEN':'CLOSED',status:flags>>2};

2262	}

2263	async function tsHistory(b){

2264	  if(!b.tsCh||!b.tsKey||b._tsHist)return;

2265	  b._tsHist=true;

2266	  try{

2267	    const r=await fetch(TS.server+'/channels/'+enc(b.tsCh)+'/feeds.json?api_key='+enc(b.tsKey)+'&minutes=1440',{signal:timeout(15000),cache:'no-store'});

2268	    if(!r.ok)throw new Error('HTTP '+r.status);

2269	    const j=await r.json();

2270	    const pts=(j.feeds||[]).map(o=>{const m=tsMap(o);return {t:Date.parse(o.created_at),fill:m.fill,temp:m.temp,humidity:m.humidity,pressure:m.pressure,gas:m.iaq}})

2271	      .filter(p=>Number.isFinite(p.t)&&Number.isFinite(p.fill));

2272	    if(pts.length){const last=pts[pts.length-1].t;b.hist=pts.concat(b.hist.filter(p=>p.t>last));if(state.view==='charts')drawCharts();}

2273	  }catch(e){b._tsHist=false;}

2274	}

2275

2276	function onStatus(b,j,route){

2277	  normalise(b,j);

2278	  const now=Date.now();

2279	  b.seen[route]=now;b.lastAt=now;

2280	  if(j.local_ip&&!b.ip)b.ipHint=j.local_ip;

2281	}

2282	// Live updates over Server-Sent Events (/api/events). Two failed opens in a

2283	// row pause the stream for 60 s and fall back to polling /api/status.

2284	function openStream(b){

2285	  const l=L(b);

2286	  if(l.es||!b.ip||SECURE||typeof EventSource==='undefined'||document.hidden||Date.now()<l.esRetryAt)return;

2287	  let opened=false;

2288	  const es=new EventSource(binBase(b)+'/api/events');

2289	  l.es=es;l.esIp=b.ip;

2290	  es.onopen=()=>{opened=true;l.esFails=0;};

2291	  es.addEventListener('status',e=>{try{onStatus(b,JSON.parse(e.data),'local')}catch(x){}});

2292	  es.addEventListener('ping',()=>{b.seen.local=Date.now();});

2293	  es.onerror=()=>{

2294	    if(!opened||es.readyState===2){

2295	      l.esFails++;

2296	      if(l.esFails>=2){es.close();if(l.es===es)l.es=null;l.esRetryAt=Date.now()+60000;}

2297	    }

2298	  };

2299	}

2300	function closeStream(b){const l=L(b);if(l.es){l.es.close();l.es=null;}}

2301	function clearReadings(b){

2302	  Object.assign(b.data,{fill:0,distance_cm:0,weight_g:0,temp:0,temp_raw:0,humidity:0,pressure:0,gas:0,iaq_acc:0,tilt:0,lid:'CLOSED',status:'OK',motion:false});

2303	  b.hist=[];b.prev={};

2304	}

2305	function relink(b){

2306	  closeStream(b);

2307	  const l=L(b);

2308	  Object.assign(l,{esRetryAt:0,esFails:0,localFails:0,nextLocal:0,nextFb:0,nextTs:0});

2309	  b.route=null;b.seen={local:0,fb:0,ts:0};b._tsHist=false;

2310	  if(isLive(b)&&!b.lastAt)clearReadings(b);

2311	  if(b.tsCh&&b.tsKey)tsHistory(b);

2312	}

2313	// Poll intervals: local 0.9 s (2 s after a miss, 30 s after 3 misses),

2314	// Firebase 5 s (15 s after an error), ThingSpeak 15 s (30 s after an error).

2315	async function pollLocal(b,l){

2316	  if(l.busyLocal||Date.now()<l.nextLocal)return;

2317	  l.busyLocal=true;

2318	  try{onStatus(b,await call(b,'/api/status'),'local');l.localFails=0;l.nextLocal=Date.now()+900;}

2319	  catch(e){l.localFails++;l.nextLocal=Date.now()+(l.localFails<3?2000:30000);}

2320	  finally{l.busyLocal=false;}

2321	}

2322	async function pollFb(b,l){

2323	  if(l.busyFb||Date.now()<l.nextFb)return;

2324	  l.busyFb=true;l.nextFb=Date.now()+5000;

2325	  try{

2326	    const j=await fbRest('GET','sb/bins/'+enc(b.cloud)+'/status');

2327	    if(j&&typeof j==='object'){

2328	      // A Firebase status older than 90 s is ignored, as in the app.

2329	      if(!j.ts||Date.now()-j.ts<90000)onStatus(b,j,'fb');

2330	      else if(j.local_ip&&!b.ip)b.ipHint=j.local_ip;

2331	    }

2332	  }catch(e){l.nextFb=Date.now()+15000;}

2333	  finally{l.busyFb=false;}

2334	}

2335	async function pollTs(b,l){

2336	  if(l.busyTs||Date.now()<l.nextTs)return;

2337	  l.busyTs=true;l.nextTs=Date.now()+15000;

2338	  try{

2339	    const r=await fetch(TS.server+'/channels/'+enc(b.tsCh)+'/feeds/last.json?api_key='+enc(b.tsKey)+'&status=true',{signal:timeout(8000),cache:'no-store'});

2340	    const t=(await r.text()).trim();

2341	    if(r.ok&&t&&t!=='-1'&&t!=='null'){

2342	      const o=JSON.parse(t),at=Date.parse(o.created_at);

2343	      if(!Number.isFinite(at)||Date.now()-at<600000)onStatus(b,tsMap(o),'ts');

2344	      if(o.status&&o.status!=='null'&&!b.ip)b.ipHint=o.status;

2345	    }

2346	  }catch(e){l.nextTs=Date.now()+30000;}

2347	  finally{l.busyTs=false;}

2348	}

2349	// One round per second. Route choice: Wi-Fi if heard in the last 4 s (15 s

2350	// with an open stream), else Firebase if heard in 20 s, else ThingSpeak if

2351	// heard in 45 s, else the bin counts as lost and plays "bin_lost".

2352	async function poll(){

2353	  const now=Date.now();

2354	  let binSnd='';

2355	  for(const b of state.bins){

2356	    if(!isLive(b)){

2357	      closeStream(b);b.online=b.live=false;b.route=null;

2358	      if(b._demoLink!=='down'){stepDemo(b);pushHistory(b);}

2359	      watchAlerts(b);continue;

2360	    }

2361	    const l=L(b);

2362	    if(b.ip&&!SECURE){

2363	      if(l.es&&l.esIp!==b.ip)closeStream(b);

2364	      openStream(b);

2365	      const streaming=l.es&&l.es.readyState===1;

2366	      if(!document.hidden&&(!streaming||now-b.seen.local>12000))pollLocal(b,l);

2367	    }else closeStream(b);

2368	    const streamOpen=l.es&&l.es.readyState===1;

2369	    const localOk=now-b.seen.local<4000||(streamOpen&&now-b.seen.local<15000);

2370	    if(!localOk&&!document.hidden){

2371	      if(fbReady()&&b.cloud)pollFb(b,l);

2372	      if(now-b.seen.fb>=20000&&b.tsCh&&b.tsKey)pollTs(b,l);

2373	    }

2374	    const route=localOk?'local':now-b.seen.fb<20000?'fb':now-b.seen.ts<45000?'ts':null;

2375	    const was=b.live;

2376	    b.route=route;b.live=b.online=!!route;

2377	    if(b._linked&&was!==b.live)binSnd=b.live?'bin_back':'bin_lost';

2378	    if(b.live)b._linked=true;

2379	    if(b.live){pushHistory(b);watchAlerts(b);}

2380	  }

2381	  linkSound(binSnd);tourTick(now);

2382	  fbWatchTick(now);fbUsageTick(now);

2383	  if($('#cloudRows')&&state.view==='settings'&&now-(poll._cloudAt||0)>2000){poll._cloudAt=now;cloudRender();}

2384	  render();

2385	}

2386

2387	/* Commands. On the bin's Wi-Fi the 200 reply is the confirmation. Over Firebase or

2388	   ThingSpeak the dashboard waits for the bin's next status, as the app does (section 7). */

2389	const EXPECT={LOCK:s=>s==='LOCKED',UNLOCK:s=>s!=='LOCKED',COLLECT:s=>s==='COLLECTING'};

2390	function confirmCloud(b,cmd,pushId,ms,via){

2391	  const t0=Date.now();

2392	  const tick=async()=>{

2393	    const age=Date.now()-t0;

2394	    let res=null;

2395	    if(pushId&&fbReady()){try{res=await fbRest('GET','sb/results/'+enc(pushId))}catch(e){}}

2396	    if(res&&res.cmd===cmd){

2397	      if(res.ok){sfx('cmd_ok');toast(b.name+' confirmed '+cmd+' through '+via+'.');}

2398	      else{sfx('cmd_fail');toast(b.name+' refused '+cmd+(res.why?': '+res.why:'')+'.');}

2399	      fbRest('DELETE','sb/results/'+enc(pushId)).catch(()=>{});render();return;

2400	    }

2401	    if(age>1500&&b.live&&EXPECT[cmd](b.data.status)){sfx('cmd_ok');toast(b.name+' confirmed '+cmd+' through '+via+'.');render();return;}

2402	    if(age>=ms){sfx('cmd_fail');toast(b.name+' did not confirm '+cmd+' within '+Math.round(ms/1000)+' s.');return;}

2403	    setTimeout(tick,1000);

2404	  };

2405	  setTimeout(tick,1000);

2406	}

2407	// Sends a command by the best route. Only LOCK, UNLOCK and COLLECT may go

2408	// through the clouds; the rest need the bin's Wi-Fi.

2409	async function sendLive(b,cmd,extra){

2410	  const body=Object.assign({cmd},extra||{});

2411	  if(b.ip&&!SECURE&&(b.route==='local'||!b.route||!CLOUD_CMDS.includes(cmd))){

2412	    try{

2413	      const r=await callRaw(b,'/api/command',body);

2414	      if(r.ok){sfx('cmd_ok');if(r.j&&r.j.status)b.data.status=String(r.j.status).toUpperCase();L(b).nextLocal=0;return true;}

2415	      sfx('cmd_fail');toast(b.name+' refused '+cmd+(r.j&&r.j.why?': '+r.j.why:' (HTTP '+r.status+')')+'.');return false;

2416	    }catch(e){

2417	      if(!CLOUD_CMDS.includes(cmd)){sfx('cmd_fail');toast(b.name+' did not answer. '+cmd.replace(/_/g,' ')+' needs the bin’s Wi-Fi.');return false;}

2418	    }

2419	  }

2420	  if(!CLOUD_CMDS.includes(cmd)){sfx('cmd_fail');toast(cmd.replace(/_/g,' ')+' needs '+b.name+'’s Wi-Fi. Over the cloud only LOCK, UNLOCK and COLLECT work.');return false;}

2421	  if(fbReady()&&b.cloud){

2422	    try{

2423	      const r=await fbRest('POST','sb/commands/'+enc(b.cloud),{cmd,ts:{'.sv':'timestamp'},from:'dashboard'});

2424	      toast(cmd+' sent to '+b.name+' through Firebase. Waiting for the bin…');

2425	      confirmCloud(b,cmd,r&&r.name,8000,'Firebase');return true;

2426	    }catch(e){}

2427	  }

2428	  if(b.tbId&&b.tbKey){

2429	    try{

2430	      const r=await fetch(TS.server+'/talkbacks/'+enc(b.tbId)+'/commands.json',{method:'POST',

2431	        headers:{'Content-Type':'application/x-www-form-urlencoded'},body:'api_key='+enc(b.tbKey)+'&command_string='+enc(cmd),signal:timeout(10000)});

2432	      if(!r.ok)throw new Error('HTTP '+r.status);

2433	      toast(cmd+' queued for '+b.name+' on ThingSpeak TalkBack. The bin picks it up within 15 s.');

2434	      confirmCloud(b,cmd,null,45000,'ThingSpeak');return true;

2435	    }catch(e){}

2436	  }

2437	  sfx('cmd_fail');toast(b.name+' cannot be reached on any link, so '+cmd+' was not sent.');return false;

2438	}

2439

2440	function cloudRowText(b){

2441	  if(!isLive(b))return ['var(--warn)','demo bin, nothing sent'];

2442	  if(b.route==='local')return ['var(--c-link)','on Wi-Fi, last reading '+ago(b.seen.local)];

2443	  if(b.route==='fb')return ['var(--c-cloud)','through Firebase, last reading '+ago(b.seen.fb)];

2444	  if(b.route==='ts')return ['var(--c-ts)','through ThingSpeak, last reading '+ago(b.seen.ts)];

2445	  return ['var(--c-tamper)','not answering'+(b.lastAt?', last reading '+ago(b.lastAt):'')];

2446	}

2447	/* The link sound follows the whole dashboard, not each bin: Wi-Fi, a cloud, or nothing.

2448	   A single bin dropping or coming back plays its own sound only when the overall link stays. */

2449	function overallLink(){

2450	  const live=state.bins.filter(isLive);

2451	  if(!live.length)return '';

2452	  const r=live.map(b=>b.route);

2453	  return r.includes('local')?'local':r.includes('fb')?'fb':r.includes('ts')?'ts':'down';

2454	}

2455	function linkSound(binSnd){

2456	  const ov=overallLink(),was=linkSound.last;

2457	  linkSound.last=ov;

2458	  if(!inMain())return;

2459	  if(was!==undefined&&ov!==was&&ov)sfx(ov==='local'?'link_up':ov==='down'?'link_lost':'cloud');

2460	  else if(binSnd)sfx(binSnd);

2461	}

2462	function inMain(){const g=$('#gate');return !!g&&g.classList.contains('hide')}

2463	function linkSlot(b){

2464	  if(!isLive(b)&&b._demoLink==='down')return [icon('wifi',false),'OFFLINE',false,'var(--c-tamper)','Demo: this bin stopped answering'];

2465	  if(!isLive(b)&&b._demoLink){const r=ROUTE[b._demoLink];return [icon(r[2]),b._demoLink==='fb'?'CLOUD':'TS',true,r[1],'Demo: read through '+r[0]+' because its Wi-Fi is down'];}

2466	  if(!isLive(b))return [icon('wifi',false),'DEMO',true,'var(--warn)','Demo data, no IP address or cloud link set'];

2467	  if(!b.live)return [icon('wifi',false),'OFFLINE',false,'var(--c-tamper)','Bin is not answering'+(b.lastAt?'. Last reading '+ago(b.lastAt):'')];

2468	  const r=ROUTE[b.route];

2469	  return [b.route==='local'?icon('wifi',true):icon(r[2]),b.route==='local'?'WI-FI':b.route==='fb'?'CLOUD':'TS',true,r[1],'Reading this bin through '+r[0]];

2470	}

2471	// At start-up: if this page was served by a bin, that bin is added (or

2472	// matched by its cloud name) and read live without typing its address.

2473	async function liveStart(){

2474	  state.bins.forEach((b,i)=>{if(isLive(b))clearReadings(b);else if(!b.hist.length)seedHistory(b,i);if(b.tsCh&&b.tsKey)tsHistory(b);});

2475	  document.addEventListener('visibilitychange',()=>{if(document.hidden){state.bins.forEach(closeStream);fbUnwatch();}});

2476	  window.addEventListener('pagehide',fbUnwatch);

2477	  if(!SECURE&&location.protocol==='http:'&&location.host&&!/^(localhost|127\.|\[::1\])/.test(location.hostname)){

2478	    try{

2479	      const r=await fetch('/api/status',{signal:timeout(1500),cache:'no-store'});

2480	      if(r.ok){

2481	        const j=await r.json(),here=location.host;

2482	        if(j&&j.id&&!state.bins.some(b=>b.ip===here)){

2483	          let b=state.bins.find(x=>!x.demo&&x.cloud===j.id&&!x.ip)||state.bins.find(x=>!x.demo&&!x.ip&&!x.cloud);

2484	          if(!b){

2485	            b=newBin({id:'b'+Date.now().toString(36),name:String(j.name||j.id).replace(/^BIN\b/i,'Bin'),place:'',ip:here,cloud:j.id});

2486	            state.bins.unshift(b);

2487	          }

2488	          b.ip=here;if(!b.cloud)b.cloud=j.id;relink(b);persist();

2489	          if(!byId(state.selSet))state.selSet=state.selSim=state.envBin=b.id;

2490	          cardSig='';render();loadSettings();buildSim();

2491	          toast(b.name+' is the bin serving this page, so it is read live at '+here+'.');

2492	        }

2493	      }

2494	    }catch(e){}

2495	  }

2496	}

2497	function pushHistory(b){

2498	  const now=Date.now(),last=b.hist[b.hist.length-1];

2499	  if(last&&now-last.t<10000)return;

2500	  b.hist.push({t:now,fill:b.data.fill,temp:b.data.temp,humidity:b.data.humidity,pressure:b.data.pressure,gas:b.data.gas});

2501	  const cut=now-864e5;

2502	  while(b.hist.length&&b.hist[0].t<cut)b.hist.shift();

2503	}

2504

2505	/* ---------- alerts ---------- */

2506	let alertSeq=0,lastUnread=0;

2507	const SEV_RANK=['critical','warning','info','success'];

2508	const SEV_TONE={critical:['bell','var(--bad)','red'],warning:['bell','var(--warn)','amber'],

2509	  info:['bell','var(--sim)','purple'],success:['bell','var(--ok)','green']};

2510	const unreadCount=()=>state.alerts.reduce((n,a)=>n+(a.read?0:1),0);

2511	const KIND_BY_IC={fill:'full',temp:'sensor',humid:'sensor',gas:'sensor',tamper:'tamper',lock:'lock',truck:'pickup',calib:'setup'};

2512	// Adds one row to the alert history and plays its sound (not on the start page).

2513	function addAlert(sev,ic,title,meta,k){

2514	  const kind=k||KIND_BY_IC[ic]||'setup';

2515	  /* No alert sounds on the start page. A pickup alert right after the DONE key would

2516	     play over the key's own tune, so it only sounds when nothing else just played. */

2517	  if(inMain()){

2518	    if(kind==='pickup'){if(Date.now()-SND.at>500)sfx('alert_pickup');}

2519	    else sfx(kind==='full'?'alert_full':kind==='tamper'?'alert_tamper':kind==='sensor'?(sev==='success'?'alert_info':'alert_sensor'):(kind==='lock'&&ic==='lock')?'alert_lock':'alert_info');

2520	  }

2521	  state.alerts.unshift({id:++alertSeq,sev,ic,k:k||KIND_BY_IC[ic]||'setup',title,meta,t:Date.now(),read:false});

2522	  state.alerts=state.alerts.slice(0,120);

2523	}

2524	/* Demo history so the alert tools have something to act on. Only when no bin has an address. */

2525	function seedAlerts(){

2526	  if(state.bins.some(isLive))return;

2527	  const n=state.bins.map(b=>b.name),a=n[0]||'Bin 01',b2=n[1]||a,M=60000;

2528	  [[2,'critical','fill','Fill threshold passed',b2+' is at 81%'],[9,'warning','gas','Air quality warning',b2+' index 212, limit 200'],

2529	   [34,'critical','tamper','Tamper detected',a+' was moved or tilted'],[41,'warning','humid','Humidity high',b2+' at 88%, limit 85%'],

2530	   [60,'info','lock','Bin locked',a+' locked from the dashboard'],[62,'success','check','Bin unlocked',a+' is open for use again','lock'],

2531	   [180,'success','truck','Collection complete',b2+' is back to normal'],[300,'info','calib','Calibration started',a+' is being calibrated'],

2532	   [302,'success','check','Settings saved',a+': empty 30 cm, full 5 cm, alert at 80%','setup'],[1260,'critical','temp','Temperature high',a+' at 41.2 \u00B0C'],

2533	   [4320,'success','truck','Collection complete',a+' is back to normal'],[17280,'critical','fill','Fill threshold passed',a+' is at 83%']]

2534	  .forEach((s,i)=>state.alerts.push({id:++alertSeq,sev:s[1],ic:s[2],k:s[5]||KIND_BY_IC[s[2]]||'setup',title:s[3],meta:s[4],t:Date.now()-s[0]*M,read:i>=2}));

2535	  lastUnread=unreadCount();

2536	}

2537	function markAllRead(){

2538	  const n=unreadCount();

2539	  if(!n)return;

2540	  state.alerts.forEach(a=>{a.read=true});

2541	  alertSig='';banSig='';renderAlerts();renderBanners();

2542	  toast('Marked '+n+' alert'+(n>1?'s':'')+' as read.');

2543	}

2544	function toggleRead(id){

2545	  const a=state.alerts.find(x=>x.id===id);

2546	  if(!a)return;

2547	  a.read=!a.read;

2548	  alertSig='';banSig='';renderAlerts();renderBanners();

2549	}

2550	// Compares this reading with the previous one (b.prev) and raises an alert

2551	// only on a change: full, temperature, humidity, air, tamper, lock, unlock,

2552	// calibration, sensor lost or back, and air sensor ready.

2553	function watchAlerts(b){

2554	  const d=b.data,p=b.prev,c=b.cal;

2555	  const full=d.fill>=c.fill_th,hot=d.temp>=c.temp_th,humid=d.humidity>=c.hum_th,gassy=d.gas>=c.gas_th;

2556	  if(full&&!p.full)addAlert('critical','fill','Fill threshold passed',b.name+' is at '+Math.round(d.fill)+'%');

2557	  if(hot&&!p.hot)addAlert('critical','temp','Temperature high',b.name+' at '+fmt(d.temp)+' °C');

2558	  if(humid&&!p.humid)addAlert('warning','humid','Humidity high',b.name+' at '+Math.round(d.humidity)+'%, limit '+c.hum_th+'%');

2559	  if(gassy&&!p.gassy)addAlert('warning','gas','Air quality warning',b.name+' index '+Math.round(d.gas)+', limit '+c.gas_th);

2560	  if(d.status==='TAMPER'&&p.status!=='TAMPER')addAlert('critical','tamper','Tamper detected',b.name+' was moved or tilted');

2561	  if(d.status==='LOCKED'&&p.status!=='LOCKED')addAlert('info','lock','Bin locked',b.name+' locked from the dashboard');

2562	  if(d.status!=='LOCKED'&&p.status==='LOCKED')addAlert('success','check','Bin unlocked',b.name+' is open for use again','lock');

2563	  if(d.status==='CALIBRATING'&&p.status!=='CALIBRATING')addAlert('info','calib','Calibration started',b.name+' is being calibrated');

2564	  const fl=(d.faults||[]).join(', ');

2565	  if(fl&&fl!==p.faults)addAlert('warning','gear','Sensor not answering',b.name+': '+fl,'sensor');

2566	  if(!fl&&p.faults)addAlert('success','check','Sensor back',b.name+' reads every sensor again','sensor');

2567	  if(p.acc===0&&d.iaq_acc>0)addAlert('success','gas','Air sensor ready',b.name+' air quality readings are settled','sensor');

2568	  b.prev={full,hot,humid,gassy,status:d.status,faults:fl,acc:d.iaq_acc};

2569	}

2570

2571	/* ---------- commands ---------- */

2572	// Every command key ends here. Live bins go to sendLive; demo bins change

2573	// their own data so the demo behaves like the real thing.

2574	async function command(id,cmd,extra){

2575	  const b=byId(id);if(!b)return;

2576	  if(b.demo&&!TOUR.busy)tourPause();

2577	  sfx(CMD_SFX[cmd]||'confirm');

2578	  if(isLive(b)){await sendLive(b,cmd,extra);render();return;}

2579	  const d=b.data;

2580	  if(cmd==='LOCK'){d.status='LOCKED';d.lid='CLOSED';}

2581	  if(cmd==='UNLOCK')d.status='OK';

2582	  if(cmd==='CLEAR_TAMPER'){d.status='OK';d.tilt=0;tourDrop(b,['tilt']);}

2583	  if(cmd==='COLLECT'){d.status='COLLECTING';d.lid='OPEN';}

2584	  if(cmd==='COLLECT_DONE'){d.status='OK';d.lid='CLOSED';d.fill=0;d.weight_g=0;

2585	    addAlert('success','truck','Collection complete',b.name+' is back to normal');}

2586	  if(cmd==='TARE')d.weight_g=0;

2587	  if(cmd==='RESET_GAS'){d.iaq_acc=0;b._accT=Date.now();d.warmEnd=b._accT+20000;}

2588	  if(cmd==='CALIBRATE_START')d.status='CALIBRATING';

2589	  if(cmd==='CALIBRATE_END')d.status='OK';

2590	  render();

2591	}

2592	let toastEl=null,toastTimer=null;

2593	function toast(msg){

2594	  if(toastEl)toastEl.remove();

2595	  toastEl=document.createElement('div');

2596	  toastEl.className='banner amber toast';

2597	  toastEl.innerHTML=icon('bell')+'<span>'+msg+'</span>';

2598	  document.body.appendChild(toastEl);

2599	  clearTimeout(toastTimer);toastTimer=setTimeout(()=>{toastEl&&toastEl.remove();toastEl=null},4500);

2600	}

2601

2602	/* ---------- mode ---------- */

2603	function currentMode(){

2604	  const cal=!!state.calib,sim=state.bins.some(b=>b.sim.mask>0);

2605	  if(cal&&sim)return 'dual';

2606	  if(cal)return 'calib';

2607	  if(sim)return 'sim';

2608	  if(state.bins.some(b=>b.data.status==='TAMPER'||b.data.fill>=b.cal.fill_th))return 'alert';

2609	  return 'normal';

2610	}

2611	function binMode(b){

2612	  const cal=!!(state.calib&&state.calib.bin===b.id),sim=b.sim.mask>0;

2613	  return cal&&sim?'dual':cal?'cal':sim?'sim':'none';

2614	}

2615	let lastMode='';

2616	/* --- overview filter & view --- */

2617	function applyOvFilter(){

2618	  const f=state.ovFilter;

2619	  const counts={all:state.bins.length,attn:0,sim:0,cal:0};

2620	  state.bins.forEach(b=>{

2621	    const card=$('#card-'+b.id);

2622	    const tile=$('#tile-'+b.id);

2623	    let show=true;

2624	    if(f==='attn')show=b.data.fill>=b.cal.fill_th||b.data.status!=='OK';

2625	    else if(f==='sim')show=b.sim.mask>0;

2626	    else if(f==='cal')show=b.data.status==='CALIBRATING';

2627	    if(card)card.classList.toggle('hide',!show);

2628	    if(tile)tile.classList.toggle('hide',!show);

2629	    if(b.data.fill>=b.cal.fill_th||b.data.status!=='OK')counts.attn++;

2630	    if(b.sim.mask>0)counts.sim++;

2631	    if(b.data.status==='CALIBRATING')counts.cal++;

2632	  });

2633	  $$('#filterChips .fchip').forEach(c=>{

2634	    const k=c.dataset.filter;

2635	    c.classList.toggle('on',k===f);

2636	    const n=counts[k]||0;

2637	    const badge=c.querySelector('.fbadge');

2638	    if(badge){

2639	      if(k!=='all'&&n>0){badge.textContent=n;badge.style.display='';}

2640	      else{badge.style.display='none';}

2641	    }

2642	  });

2643	}

2644	function applyOvView(){

2645	  const gv=state.ovView==='grid'&&state.bins.length>=3;

2646	  const host=$('#bins');

2647	  if(host)host.classList.toggle('gv',gv);

2648	  const btn=$('#viewToggle');

2649	  if(btn){

2650	    btn.title=gv?'Switch to list view':'Switch to grid view';

2651	    const gi=$('#vicoGrid'),li=$('#vicoList');

2652	    if(gi&&li){gi.classList.toggle('hide',gv);li.classList.toggle('hide',!gv);}

2653	  }

2654	  const tp=$('#ovTools');

2655	  if(tp)tp.classList.toggle('hide',state.bins.length<3);

2656	}

2657	function updateTiles(){

2658	  state.bins.forEach((b,i)=>{

2659	    const t=$('#tile-'+b.id);if(!t)return;

2660	    const pct=Math.round(b.data.fill),c=b.cal;

2661	    const full=pct>=c.fill_th,tamper=b.data.status==='TAMPER';

2662	    const bm=binMode(b);

2663	    t.dataset.bm=bm;

2664	    t.style.outline=(full||tamper)?'2.5px solid var(--bad)':'';

2665	    t.style.outlineOffset='-2px';

2666	    const fn=t.querySelector('.tfill');if(fn)fn.textContent=pct+'%';

2667	    let warn=t.querySelector('.twarn');

2668	    const hasWarn=b.data.temp>=c.temp_th||b.data.humidity>=c.hum_th||b.data.gas>=c.gas_th;

2669	    if(hasWarn&&!warn){warn=document.createElement('span');warn.className='twarn';warn.textContent='!';t.appendChild(warn);}

2670	    if(warn)warn.style.display=hasWarn?'':'none';

2671	    const bar=t.querySelector('.tbar i');

2672	    if(bar){bar.style.width=clamp(pct,0,100)+'%';bar.style.background=full?'var(--bad)':pct>c.fill_th*0.7?'var(--warn)':colorOf(i);}

2673	    const st=t.querySelector('.tstatus');

2674	    if(st){const txt=tamper?'TAMPER':full?'FULL':b.data.status==='CALIBRATING'?'CALIB':b.data.lid==='OPEN'?'LID OPEN':'OK';

2675	      st.textContent=txt;st.classList.toggle('alert-status',tamper||full);}

2676	    const svg=t.querySelector('svg.ico.lg');

2677	    if(svg){svg.style.color=colorOf(i);

2678	      const wrap=svg.querySelector('.wavewrap');

2679	      if(wrap)wrap.setAttribute('transform','translate(0,'+(42-(clamp(pct,0,100)/100)*30).toFixed(1)+')');}

2680	  });

2681	}

2682	/* tile overlay */

2683	function showTileOverlay(bid){

2684	  state.ovTile=bid;

2685	  let ov=$('#tileOverlay');

2686	  if(!ov){

2687	    ov=document.createElement('div');ov.id='tileOverlay';ov.className='tile-overlay hide';

2688	    ov.innerHTML='<div class="tile-detail" id="tileDetail"><button class="tile-close" id="tileClose"><svg viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2.5" stroke-linecap="round"><path d="M6 6l12 12M18 6L6 18"/></svg></button><div id="tileCard"></div>'+

2689	      '<div class="tile-nav"><button class="btn sm" id="tilePrev">&#8592; Prev</button><button class="btn sm" id="tileNext">Next &#8594;</button></div></div>';

2690	    document.body.appendChild(ov);

2691	    ov.addEventListener('click',e=>{if(e.target===ov)hideTileOverlay();});

2692	    $('#tileClose').addEventListener('click',hideTileOverlay);

2693	    $('#tilePrev').addEventListener('click',()=>stepTile(-1));

2694	    $('#tileNext').addEventListener('click',()=>stepTile(1));

2695	    document.addEventListener('keydown',e=>{if(state.ovTile){if(e.key==='Escape')hideTileOverlay();else if(e.key==='ArrowLeft')stepTile(-1);else if(e.key==='ArrowRight')stepTile(1);}});

2696	  }

2697	  /* fill content first, then show - prevents late appearance */

2698	  fillTileDetail(bid);

2699	  ov.classList.remove('hide');

2700	}

2701	function hideTileOverlay(){state.ovTile=null;const ov=$('#tileOverlay');if(ov)ov.classList.add('hide');}

2702	function stepTile(dir){

2703	  const visible=state.bins.filter(b=>!$('#card-'+b.id).classList.contains('hide'));

2704	  const idx=visible.findIndex(b=>b.id===state.ovTile);

2705	  if(idx<0)return;

2706	  const next=visible[(idx+dir+visible.length)%visible.length];

2707	  if(next)fillTileDetail(next.id);

2708	}

2709	let ovDetailSig='';

2710	function fillTileDetail(bid){

2711	  state.ovTile=bid;

2712	  const tc=$('#tileCard');if(!tc)return;

2713	  const card=$('#card-'+bid);

2714	  if(!card)return;

2715	  /* sig-check: build a stable fingerprint from data, not innerHTML (which shifts every renderCard) */

2716	  const b=byId(bid);

2717	  const bsig=b?[b.data.fill,b.data.status,b.data.locked,b.data.temp,b.data.gas,b.data.humidity,b.data.pressure,

2718	    b.data.iaq_acc,b.sim.mask,b.name,b.place,state.role,

2719	    state.calib?state.calib.bin:''].join('|'):'';

2720	  const sig=bid+'|'+bsig;

2721	  if(sig===ovDetailSig&&tc.firstElementChild)return;

2722	  ovDetailSig=sig;

2723	  tc.innerHTML=card.outerHTML;

2724	  const clone=tc.firstElementChild;

2725	  if(clone){

2726	    clone.removeAttribute('id');/* avoid duplicate IDs */

2727	    clone.setAttribute('data-bin',bid);/* keep bin reference for event delegation */

2728	    clone.classList.remove('hot');

2729	    clone.style.cssText='';/* clear any inline sim/cal border styles */

2730	    /* rename clipPath IDs so they don't collide with the original card */

2731	    clone.querySelectorAll('clipPath[id]').forEach(cp=>{

2732	      const oldId=cp.id,newId='ov-'+oldId;

2733	      cp.id=newId;

2734	      const g=cp.closest('svg')?.querySelector('[clip-path="url(#'+oldId+')"]');

2735	      if(g)g.setAttribute('clip-path','url(#'+newId+')');

2736	    });

2737	    /* gear/SET button stays enabled in overlay */

2738	    clone.querySelectorAll('[data-r="edit"]').forEach(r=>r.classList.add('hide'));

2739	  }

2740	}

2741

2742	/* --- live-update overlay card in place (no re-clone) --- */

2743	function renderOverlayCard(){

2744	  const tc=$('#tileCard');if(!tc)return;

2745	  const clone=tc.firstElementChild;if(!clone)return;

2746	  const bid=state.ovTile;

2747	  const b=byId(bid);if(!b)return;

2748	  const d=b.data,c=b.cal,bi=state.bins.indexOf(b);

2749	  const bcol=bi>=0?colorOf(bi):statusColor(d.status,d.fill,c);

2750	  const q=r=>clone.querySelector('[data-r="'+r+'"]');

2751

2752	  /* wave position + color */

2753	  const hero=q('hero');

2754	  if(hero){

2755	    const kind=d.status==='CALIBRATING'?'cal':'bin';

2756	    if(kind==='bin'){

2757	      const svg=hero.querySelector('svg'),wrap=hero.querySelector('.wavewrap');

2758	      if(svg)svg.style.color=bcol;

2759	      if(wrap)wrap.setAttribute('transform','translate(0,'+(42-(clamp(d.fill,0,100)/100)*30).toFixed(1)+')');

2760	    }

2761	  }

2762

2763	  /* fill percentage */

2764	  const pctEl=q('pct');

2765	  if(pctEl){const v=Math.round(d.fill);pctEl.innerHTML=v+'<span>%</span>';}

2766

2767	  /* bar */

2768	  const bar=q('bar');

2769	  if(bar){bar.style.width='calc('+clamp(d.fill,0,100)+'% - 6px)';bar.style.background=levelColor(d.fill,c);}

2770

2771	  /* sensors */

2772	  const vals={

2773	    temp:[fmt(d.temp,1)+'°',d.temp>=c.temp_th,d.temp<22?'cool':(d.temp/c.temp_th>=1?'crit':d.temp/c.temp_th>=.8?'high':d.temp/c.temp_th>=.5?'mid':'low')],

2774	    humid:[Math.round(d.humidity)+'%',d.humidity>=c.hum_th,d.humidity>=c.hum_th?'crit':d.humidity>=c.hum_th-10?'high':d.humidity>=60?'mid':d.humidity<=35?'cool':'low'],

2775	    weight:[(d.weight_g/1000).toFixed(2)+'kg',d.weight_g>=c.weight_cap,(()=>{const f=d.weight_g/(c.weight_cap||4500);return f>=1?'crit':f>=.8?'high':f>=.5?'mid':'low';})()],

2776	    gas:[String(Math.round(d.gas)),d.gas>=c.gas_th,(()=>{const f=d.gas/(c.gas_th||200);return f>=1?'crit':f>=.8?'high':f>=.5?'mid':'low';})()]

2777	  };

2778	  clone.querySelectorAll('[data-s]').forEach(el=>{

2779	    const v=vals[el.dataset.s];if(!v)return;

2780	    const t=el.querySelector('.v');if(t&&t.textContent!==v[0])t.textContent=v[0];

2781	    el.dataset.lv=v[2];

2782	    el.classList.toggle('alarm',v[1]);el.classList.toggle('nm-in',!v[1]);

2783	  });

2784

2785	  /* status chips - use same sig-check as renderCard */

2786	  const simOn=b.sim.mask>0,calOn=binMode(b)==='cal'||binMode(b)==='dual';

2787	  const tilting=d.tamper_phase==='COUNTDOWN'&&d.status!=='TAMPER'&&d.status!=='CALIBRATING';

2788	  const csig=[d.lid,d.status,d.motion,simOn,calOn,b.live,b.route,isLive(b),b._demoLink,tilting].join('|');

2789	  const ch=q('chips');

2790	  if(ch&&ch.dataset.sig!==csig){

2791	    ch.dataset.sig=csig;

2792	    const stKey=d.status==='CALIBRATING'?'OK':d.status;

2793	    const stIco=stKey==='TAMPER'?icon('tamper'):stKey==='LOCKED'?icon('lock',true)

2794	      :stKey==='COLLECTING'?icon('truck'):icon('check');

2795	    const stCol={OK:'var(--ok)',LOCKED:'var(--sim)',TAMPER:'var(--c-tamper)',

2796	      COLLECTING:'var(--cal)'}[stKey]||'var(--ink2)';

2797	    const st=STATUS_SHORT[stKey]||[stKey,stKey];

2798	    ch.innerHTML=[

2799	      slot(icon('lid',d.lid==='OPEN'),d.lid==='OPEN'?'OPEN':'CLOSED',d.lid==='OPEN','var(--c-lid)',

2800	        d.lid==='OPEN'?'Lid is open':'Lid is closed'),

2801	      tilting?slot(icon('tamper'),'TILTED',true,'var(--warn)','Tipped over. Tamper alert if it stays tipped'):slot(stIco,st[0],true,stCol,st[1]),

2802	      slot.apply(null,linkSlot(b)),

2803	      slot(icon('flask'),'SIM',simOn,'var(--sim)',simOn?'Simulation is on':'Simulation is off'),

2804	      slot(icon('calib'),'CAL',calOn,'var(--cal)',calOn?'Calibration mode is on':'Calibration mode is off'),

2805	      slot(icon('motion',d.motion),'MOTION',d.motion,'var(--c-motion)',

2806	        d.motion?'Motion detected':'No motion')

2807	    ].join('');

2808	  }

2809

2810	  /* action buttons */

2811	  const op=state.role==='operator',busy=d.status==='CALIBRATING';

2812	  const locked=d.status==='LOCKED',tamper=d.status==='TAMPER',collecting=d.status==='COLLECTING';

2813	  const want=[];

2814	  want.push(locked

2815	    ?['UNLOCK','UNLOCK','solid','lock',!busy,'var(--sim)']

2816	    :['LOCK','LOCK','tint','lock',!(busy||tamper),'var(--sim)']);

2817	  if(op)want.push(['CLEAR_TAMPER','RESET TAMPER',tamper?'danger':'tint','tamper',tamper,'var(--c-tamper)']);

2818	  want.push(collecting

2819	    ?['COLLECT_DONE','DONE','solid','check',true,'var(--cal)']

2820	    :['COLLECT','COLLECT','tint','truck',!busy,'var(--c-link)']);

2821	  const asig=want.map(w=>w.join('|')).join(',');

2822	  const act=q('actions');

2823	  if(act&&act.dataset.sig!==asig){

2824	    act.dataset.sig=asig;

2825	    act.innerHTML=want.map(w=>{

2826	      const tip=w[0]==='CLEAR_TAMPER'&&!tamper?' title="Nothing to reset on this bin"':'';

2827	      return '<button class="btn '+w[2]+'" style="--cc:'+w[5]+'" data-cmd="'+w[0]+

2828	        '" data-bin="'+bid+'"'+(w[4]?'':' disabled')+tip+'>'+icon(w[3],w[0]==='UNLOCK')+

2829	        '<span>'+w[1]+'</span></button>';

2830	    }).join('');

2831	  }

2832	}

2833

2834	/* --- settings overlay (SET button) --- */

2835	function openSetOverlay(bid){

2836	  const b=byId(bid);if(!b)return;

2837	  let ov=$('#setOverlay');

2838	  if(!ov){

2839	    ov=document.createElement('div');ov.id='setOverlay';ov.className='set-overlay hide';

2840	    ov.innerHTML='<div class="set-panel nm" id="setPanel">'+

2841	      '<button class="set-close" id="setClose"><svg viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2.5" stroke-linecap="round"><path d="M6 6l12 12M18 6L6 18"/></svg></button>'+

2842	      '<h3>'+icon('gear')+' <span id="setTitle"></span></h3>'+

2843	      '<div class="set-fields">'+

2844	        '<div class="field"><label>Name</label><input type="text" id="setName"></div>'+

2845	        '<div class="field"><label>Location</label><input type="text" id="setPlace"></div>'+

2846	        '<div class="field"><label>IP address</label><input type="text" id="setIP" placeholder="192.168.1.101"></div>'+

2847	        '<div class="field"><label>Firebase cloud name</label><input type="text" id="setCloud" placeholder="bin01" spellcheck="false"></div>'+

2848	        '<div class="field"><label>ThingSpeak channel ID</label><input type="text" id="setTsCh" inputmode="numeric" placeholder="1234567"></div>'+

2849	        '<div class="field"><label>ThingSpeak read key</label><input type="text" id="setTsKey" spellcheck="false"></div>'+

2850	        '<div class="field"><label>TalkBack ID</label><input type="text" id="setTbId" inputmode="numeric"></div>'+

2851	        '<div class="field"><label>TalkBack API key</label><input type="text" id="setTbKey" spellcheck="false"></div>'+

2852	      '</div>'+

2853	      '<div class="set-actions">'+

2854	        '<button class="btn sm danger" id="setRemove">Remove</button>'+

2855	        '<button class="btn sm accent" id="setSave">Save</button>'+

2856	      '</div></div>';

2857	    document.body.appendChild(ov);

2858	    ov.addEventListener('click',e=>{if(e.target===ov)closeSetOverlay();});

2859	    $('#setClose').addEventListener('click',closeSetOverlay);

2860	    $('#setSave').addEventListener('click',()=>{const bid=ov.dataset.bin;if(bid)saveFromSetOverlay(bid);});

2861	    $('#setRemove').addEventListener('click',()=>{const bid=ov.dataset.bin;if(bid)removeFromSetOverlay(bid);});

2862	    document.addEventListener('keydown',e=>{if(e.key==='Escape'&&ov&&!ov.classList.contains('hide'))closeSetOverlay();});

2863	  }

2864	  ov.dataset.bin=bid;

2865	  $('#setTitle').textContent=b.name+' Settings';

2866	  $('#setName').value=b.name;

2867	  $('#setPlace').value=b.place;

2868	  $('#setIP').value=b.ip;

2869	  $('#setIP').placeholder=b.ipHint||'192.168.1.101';

2870	  $('#setCloud').value=b.cloud||'';$('#setTsCh').value=b.tsCh||'';$('#setTsKey').value=b.tsKey||'';

2871	  $('#setTbId').value=b.tbId||'';$('#setTbKey').value=b.tbKey||'';

2872	  ov.classList.remove('hide');

2873	}

2874	function closeSetOverlay(){

2875	  const ov=$('#setOverlay');if(ov)ov.classList.add('hide');

2876	}

2877	function saveFromSetOverlay(bid){

2878	  const b=byId(bid);if(!b)return;

2879	  const ov=$('#setOverlay');

2880	  if(ov&&!ov.classList.contains('hide')&&ov.dataset.bin===bid){

2881	    b.name=$('#setName').value.trim()||b.name;

2882	    b.place=$('#setPlace').value.trim();

2883	    b.ip=$('#setIP').value.trim();

2884	    b.cloud=$('#setCloud').value.trim();b.tsCh=$('#setTsCh').value.trim();b.tsKey=$('#setTsKey').value.trim();

2885	    b.tbId=$('#setTbId').value.trim();b.tbKey=$('#setTbKey').value.trim();

2886	    relink(b);

2887	  }else{

2888	    const row=$('#card-'+bid+' [data-r="edit"]');

2889	    if(row){b.name=row.querySelector('[data-k="name"]')?.value.trim()||b.name;

2890	      b.place=row.querySelector('[data-k="place"]')?.value.trim()||'';

2891	      b.ip=row.querySelector('[data-k="ip"]')?.value.trim()||'';relink(b);}

2892	  }

2893	  persist();cardSig='';render();loadSettings();buildSim();toast(b.name+' saved.');

2894	  closeSetOverlay();

2895	}

2896	function removeFromSetOverlay(bid){

2897	  const i=state.bins.findIndex(b=>b.id===bid);

2898	  if(i>-1){const gone=state.bins.splice(i,1)[0];

2899	    if(state.selSet===gone.id)state.selSet=state.bins[0]&&state.bins[0].id;

2900	    if(state.selSim===gone.id)state.selSim=state.selSet;

2901	    if(state.calib&&state.calib.bin===gone.id)state.calib=null;

2902	    if(state.envBin===gone.id)state.envBin=state.selSet;

2903	    persist();cardSig='';render();loadSettings();buildSim();toast(gone.name+' removed.');

2904	    closeSetOverlay();hideTileOverlay();

2905	  }

2906	}

2907

2908	function applyMode(){

2909	  const m=currentMode();

2910	  if(m===lastMode)return;

2911	  lastMode=m;

2912	  document.documentElement.setAttribute('data-mode',m);

2913	  const map={normal:['check','NORMAL'],sim:['flask','SIMULATION'],calib:['calib','CALIBRATING'],

2914	    dual:['calib','SIM + CALIBRATION'],alert:['bell','ALERT']};

2915	  $('#modeico').innerHTML=icon(map[m][0]);

2916	  $('#modetext').textContent=map[m][1];

2917	  const chip=$('#modechip');

2918	  chip.classList.remove('flash');void chip.offsetWidth;chip.classList.add('flash');

2919	}

2920

2921	/* ---------- build ---------- */

2922	function buildTabs(){

2923	  $('#tabs').innerHTML=TABS.map(t=>'<button class="tab" role="tab" style="--cc:'+t.c+'" data-view="'+t.v+

2924	    '" aria-selected="'+(t.v===state.view)+'">'+icon(t.i)+'<span>'+t.t+'</span></button>').join('');

2925	}

2926	function makeTile(b,i){

2927	  const d=b.data,c=b.cal;

2928	  const pct=Math.round(d.fill);

2929	  const full=pct>=c.fill_th,tamper=d.status==='TAMPER';

2930	  const bm=binMode(b);

2931	  const warn=(d.temp>=c.temp_th||d.humidity>=c.hum_th||d.gas>=c.gas_th)?'<span class="twarn">!</span>':'';

2932	  const barCol=full?'var(--bad)':pct>c.fill_th*0.7?'var(--warn)':colorOf(i);

2933	  const statusTxt=tamper?'TAMPER':full?'FULL':d.status==='CALIBRATING'?'CALIB':d.lid==='OPEN'?'LID OPEN':'OK';

2934	  const statusCls=tamper||full?'tstatus alert-status':'tstatus';

2935	  return '<div class="tile" id="tile-'+b.id+'" data-bm="'+bm+'" data-tid="'+b.id+'"'+((full||tamper)?' style="outline:2.5px solid var(--bad);outline-offset:-2px"':'')+'>'+

2936	    warn+

2937	    ICONS.bin(b.id,pct,colorOf(i),'tclip')+

2938	    '<span class="tfill" style="color:'+colorOf(i)+'">'+pct+'%</span>'+

2939	    '<span class="tname">'+esc(b.name)+'</span>'+

2940	    '<span class="tplace">'+esc(b.place||'')+'</span>'+

2941	    '<div class="tbar"><i style="width:'+clamp(pct,0,100)+'%;background:'+barCol+'"></i></div>'+

2942	    '<span class="'+statusCls+'">'+statusTxt+'</span>'+

2943	    '</div>';

2944	}

2945	function buildCards(){

2946	  const op=state.role==='operator';

2947	  $('#bins').innerHTML=state.bins.length?state.bins.map((b,i)=>cardHTML(b)+makeTile(b,i)).join('')

2948	    :'<div class="nm" style="grid-column:1/-1;padding:28px 24px;border-radius:24px"><h2 style="margin:0 0 8px">No bins linked yet</h2>'+

2949	     '<p class="hint" style="margin:0">Open this page from the bin itself (type its IP address in the browser, for example http://192.168.0.112) and it links automatically. '+

2950	     'Or switch to operator, add a bin and type its IP address. To look around with made-up bins, open the START PAGE (Settings) and turn DEMO on.</p></div>';

2951	  $('#addRow').classList.toggle('hide',!op);

2952	  $('#addNote').textContent=state.bins.length?'':'No bins yet.';

2953	  const bt=$('#binsTitle');

2954	  if(bt)bt.innerHTML=icon('bins')+'<span>BINS &middot; '+state.bins.length+'</span>';

2955	  if(reduce())return;

2956	  Array.from($('#bins').children).forEach((el,i)=>{

2957	    el.style.animationDelay=(i*75)+'ms';el.classList.add('enter');

2958	  });

2959	  applyOvFilter();

2960	  applyOvView();

2961	}

2962	function cardHTML(b){

2963	  return '<article class="card nm" id="card-'+b.id+'" data-bin="'+b.id+'">'+

2964	    '<div class="cardtop">'+

2965	      '<div class="hero" data-r="hero"></div>'+

2966	      '<div class="who"><div class="binname">'+esc(b.name)+'</div><div class="binplace">'+esc(b.place)+'</div></div>'+

2967	      '<div style="display:flex;flex-direction:column;align-items:flex-end;gap:10px">'+

2968	        '<div class="fillnum" data-r="pct">0<span>%</span></div>'+

2969	        (state.role==='operator'?'<button class="gearbtn" data-edit="'+b.id+'" aria-expanded="false" title="Bin settings">'+icon('gear')+'<em>SET</em></button>':'')+

2970	      '</div>'+

2971	    '</div>'+

2972	    '<div class="chips" data-r="chips"></div>'+

2973	    '<div class="bar nm-deep"><i data-r="bar"></i><b style="left:50%"></b><b data-r="th"></b></div>'+

2974	    '<div class="ticks"><span>0</span><span>50</span><span data-r="tl">80</span><span>100</span></div>'+

2975	    '<p class="faultnote hide" data-r="fault">Fill sensor not answering. The bar shows the last reading.</p>'+

2976	    '<div class="cardfoot"><span class="h1">1 h</span><svg data-r="spark" viewBox="0 0 100 22" preserveAspectRatio="none"><path/></svg><span data-r="ago"></span></div>'+

2977	    '<div class="sensors" data-r="sensors"></div>'+

2978	    '<div class="actions" data-r="actions"></div>'+

2979	    '<div class="editrow nm-in hide" data-r="edit">'+

2980	      '<div class="field"><label>Name</label><input type="text" data-k="name" value="'+esc(b.name)+'"></div>'+

2981	      '<div class="field"><label>Location</label><input type="text" data-k="place" value="'+esc(b.place)+'"></div>'+

2982	      '<div class="field"><label>IP address</label><input type="text" data-k="ip" value="'+esc(b.ip)+'" placeholder="192.168.1.101"></div>'+

2983	      '<div class="field" style="display:flex;align-items:flex-end;gap:10px">'+

2984	        '<button class="btn sm accent" data-save="'+b.id+'">Save</button>'+

2985	        '<button class="btn sm danger" data-remove="'+b.id+'">Remove</button></div>'+

2986	    '</div></article>';

2987	}

2988	/* The last hour of fill as a small line, and how long ago the last reading came, as in the app. */

2989	function cardFoot(b,svg,agoEl,col){

2990	  const now=Date.now(),H=3600000,pts=(b.hist||[]).filter(h=>now-h.t<=H);

2991	  const d=pts.length>1?pts.map((h,i)=>(i?'L':'M')+((h.t-(now-H))/H*100).toFixed(2)+' '+(22-clamp(h.fill,0,100)/100*22).toFixed(2)).join(''):'';

2992	  const path=svg.firstChild;if(path.getAttribute('d')!==d)path.setAttribute('d',d);path.style.stroke=col;

2993	  const at=isLive(b)?b.lastAt:(b._demoLink==='down'?b._quietAt:now);

2994	  const s=at?Math.round((now-at)/1000):null;

2995	  agoEl.textContent=s==null?'No reading yet':s<5?'Updated just now':'Updated '+ago(at);

2996	  agoEl.classList.toggle('old',isLive(b)?!b.live:b._demoLink==='down');

2997	}

2998	function esc(s){return String(s).replace(/[&<>"]/g,c=>({'&':'&amp;','<':'&lt;','>':'&gt;','"':'&quot;'}[c]))}

2999

3000	/* ---------- render ---------- */

3001	let cardSig='';

3002	// Redraws whatever changed: cards, tiles, banners, priority, alerts, link chip.

3003	function render(){

3004	  const sig=state.bins.map(b=>b.id).join(',')+'|'+state.role;

3005	  if(sig!==cardSig){cardSig=sig;buildCards();}

3006	  state.bins.forEach(renderCard);

3007	  applyMode();renderCollectNext();renderBanners();renderAlerts();renderLink();

3008	  if(state.view==='charts')drawCharts();

3009	  if(state.view==='settings'){updateSettingsLive();demoPanelRender();}

3010	  /* live-update overlay clone in place (no re-clone, no flicker) */

3011	  if(state.ovTile){const ov=$('#tileOverlay');if(ov&&!ov.classList.contains('hide'))renderOverlayCard();}

3012

3013	}

3014	// One bin card. Each part (wave, bar, sensor tiles, chips, keys) is updated in

3015	// place instead of rebuilding the card, so animations are not restarted.

3016	function renderCard(b){

3017	  const root=$('#card-'+b.id);if(!root)return;

3018	  const d=b.data,c=b.cal,bi=state.bins.indexOf(b);

3019	  const bcol=bi>=0?colorOf(bi):statusColor(d.status,d.fill,c);

3020	  const q=r=>root.querySelector('[data-r="'+r+'"]');

3021	  root.classList.toggle('hot',d.status!=='OK'||d.fill>=c.fill_th);

3022

3023	  root.classList.toggle('urgent',d.status==='TAMPER'||d.fill>=c.fill_th);

3024

3025	  const hero=q('hero'),kind=d.status==='CALIBRATING'?'cal':'bin';

3026	  if(hero.dataset.kind!==kind){

3027	    hero.dataset.kind=kind;

3028	    hero.innerHTML=kind==='cal'?'<span style="color:var(--cal)">'+ICONS.calib()+'</span>':ICONS.bin(b.id,d.fill,bcol);

3029	  }

3030	  if(kind==='bin'){

3031	    const svg=hero.querySelector('svg'),wrap=hero.querySelector('.wavewrap');

3032	    if(svg)svg.style.color=bcol;

3033	    if(wrap)wrap.setAttribute('transform','translate(0,'+(42-(clamp(d.fill,0,100)/100)*30).toFixed(1)+')');

3034	  }

3035

3036	  tweenPct(q('pct'),b,d.fill);

3037	  q('fault').classList.toggle('hide',!(d.faults||[]).includes('HC-SR04'));

3038	  cardFoot(b,q('spark'),q('ago'),levelColor(d.fill,c));

3039	  const bar=q('bar');bar.style.width='calc('+clamp(d.fill,0,100)+'% - 6px)';bar.style.background=levelColor(d.fill,c);

3040	  q('th').style.left=c.fill_th+'%';q('tl').textContent=c.fill_th;

3041

3042	  const bm=binMode(b);

3043	  root.dataset.bmode=bm;

3044	  const simOn=b.sim.mask>0,calOn=bm==='cal'||bm==='dual';

3045	  const tilting=d.tamper_phase==='COUNTDOWN'&&d.status!=='TAMPER'&&d.status!=='CALIBRATING';

3046	  const csig=[d.lid,d.status,d.motion,simOn,calOn,b.live,b.route,isLive(b),b._demoLink,tilting].join('|');

3047	  const ch=q('chips');

3048	  if(ch.dataset.sig!==csig){

3049	    ch.dataset.sig=csig;

3050	    /* CALIBRATING is shown by the CAL chip only, the same way SIM shows simulation.

3051	       The status chip keeps reporting the underlying bin state. */

3052	    const stKey=d.status==='CALIBRATING'?'OK':d.status;

3053	    const stIco=stKey==='TAMPER'?icon('tamper'):stKey==='LOCKED'?icon('lock',true)

3054	      :stKey==='COLLECTING'?icon('truck'):icon('check');

3055	    const stCol={OK:'var(--ok)',LOCKED:'var(--sim)',TAMPER:'var(--c-tamper)',

3056	      COLLECTING:'var(--cal)'}[stKey]||'var(--ink2)';

3057	    const linkOk=b.live||!b.ip;

3058	    const st=STATUS_SHORT[stKey]||[stKey,stKey];

3059	    ch.innerHTML=[

3060	      slot(icon('lid',d.lid==='OPEN'),d.lid==='OPEN'?'OPEN':'CLOSED',d.lid==='OPEN','var(--c-lid)',

3061	        d.lid==='OPEN'?'Lid is open':'Lid is closed'),

3062	      tilting?slot(icon('tamper'),'TILTED',true,'var(--warn)','Tipped over. Tamper alert if it stays tipped'):slot(stIco,st[0],true,stCol,st[1]),

3063	      slot.apply(null,linkSlot(b)),

3064	      slot(icon('flask'),'SIM',simOn,'var(--sim)',simOn?'Simulation is on':'Simulation is off'),

3065	      slot(icon('calib'),'CAL',calOn,'var(--cal)',calOn?'Calibration mode is on':'Calibration mode is off'),

3066	      slot(icon('motion',d.motion),'MOTION',d.motion,'var(--c-motion)',

3067	        d.motion?'Motion detected':'No motion')

3068	    ].join('');

3069	  }

3070

3071	  const host=q('sensors');

3072	  if(!host.dataset.built){

3073	    host.dataset.built='1';

3074	    host.innerHTML=[['temp','Temperature'],['humid','Humidity'],['weight','Weight'],['gas','Air quality']]

3075	      .map(s=>'<div class="sensor nm-in" data-s="'+s[0]+'">'+icon(s[0])+'<span class="v">--</span><span class="k">'+s[1]+'</span></div>').join('');

3076	  }

3077	  const band=(f)=>f>=1?'crit':f>=.8?'high':f>=.5?'mid':'low';

3078	  const vals={

3079	    temp:[fmt(d.temp,1)+'°',d.temp>=c.temp_th,d.temp<22?'cool':band(d.temp/c.temp_th)],

3080	    humid:[Math.round(d.humidity)+'%',d.humidity>=c.hum_th,d.humidity>=c.hum_th?'crit':d.humidity>=c.hum_th-10?'high':d.humidity>=60?'mid':d.humidity<=35?'cool':'low'],

3081	    weight:[(d.weight_g/1000).toFixed(2)+'kg',d.weight_g>=c.weight_cap,band(d.weight_g/(c.weight_cap||4500))],

3082	    gas:[String(Math.round(d.gas)),d.gas>=c.gas_th,band(d.gas/(c.gas_th||200))]

3083	  };

3084	  const warming=(isLive(b)||b.demo)&&d.iaq_acc===0&&!(b.sim.mask&4);

3085	  if(warming){

3086	    const l=d.warmEnd?Math.max(0,Math.round((d.warmEnd-Date.now())/1000)):null;

3087	    vals.gas=[l?Math.floor(l/60)+':'+String(l%60).padStart(2,'0'):'...',false,'low'];

3088	  }

3089	  /* A sensor that stops answering shows -- and says so, instead of an old number. */

3090	  const F=d.faults||[],dead={temp:F.includes('BME680'),humid:F.includes('BME680'),gas:F.includes('BME680'),weight:F.includes('HX711')};

3091	  const lbl={temp:'Temperature',humid:'Humidity',weight:'Weight',gas:warming?'Air warming up':'Air quality'};

3092	  for(const k in dead)if(dead[k]){vals[k]=['--',true,'crit'];lbl[k]='Not answering';}

3093	  host.querySelectorAll('[data-s]').forEach(el=>{

3094	    const kk=el.querySelector('.k');if(kk&&lbl[el.dataset.s]&&kk.textContent!==lbl[el.dataset.s])kk.textContent=lbl[el.dataset.s];

3095	  });

3096	  host.querySelectorAll('[data-s]').forEach(el=>{

3097	    const v=vals[el.dataset.s];if(!v)return;

3098	    const t=el.querySelector('.v');if(t.textContent!==v[0])t.textContent=v[0];

3099	    el.dataset.lv=v[2];

3100	    el.classList.toggle('alarm',v[1]);el.classList.toggle('nm-in',!v[1]);

3101	  });

3102	  tuneSensorIcons(root,d,c);

3103	  updateTiles();

3104

3105	  const op=state.role==='operator',busy=d.status==='CALIBRATING';

3106	  const locked=d.status==='LOCKED',tamper=d.status==='TAMPER',collecting=d.status==='COLLECTING';

3107	  const want=[];

3108	  /* LOCK and UNLOCK travel together: an action you can do but cannot undo

3109	     strands a collector who locked a bin with no operator around. RESET TAMPER

3110	     is operator only and is left out of the collector's card entirely, since a

3111	     control they can never use is just noise. */

3112	  want.push(locked

3113	    ?['UNLOCK','UNLOCK','solid','lock',!busy,'var(--sim)']

3114	    :['LOCK','LOCK','tint','lock',!(busy||tamper),'var(--sim)']);

3115	  if(op)want.push(['CLEAR_TAMPER','RESET TAMPER',tamper?'danger':'tint','tamper',tamper,'var(--c-tamper)']);

3116	  want.push(collecting

3117	    ?['COLLECT_DONE','DONE','solid','check',true,'var(--cal)']

3118	    :['COLLECT','COLLECT','tint','truck',!busy,'var(--c-link)']);

3119	  const asig=want.map(w=>w.join('|')).join(',');

3120	  const act=q('actions');

3121	  if(act.dataset.sig!==asig){

3122	    act.dataset.sig=asig;

3123	    act.innerHTML=want.map(w=>{

3124	      const tip=w[0]==='CLEAR_TAMPER'&&!tamper?' title="Nothing to reset on this bin"':'';

3125	      return '<button class="btn '+w[2]+'" style="--cc:'+w[5]+'" data-cmd="'+w[0]+

3126	        '" data-bin="'+b.id+'"'+(w[4]?'':' disabled')+tip+'>'+icon(w[3],w[0]==='UNLOCK')+

3127	        '<span>'+w[1]+'</span></button>';

3128	    }).join('');

3129	  }

3130	}

3131	/* Every chip is one grid cell of the same size, so the label has to fit on one

3132	   line even in the narrowest card (about 94px per cell with 3 bins at 1100px).

3133	   The long words are shortened and the full wording moves to the tooltip. */

3134	const STATUS_SHORT={OK:['OK','Bin is OK'],FULL:['FULL','Bin is full'],

3135	  LOCKED:['LOCKED','Lid is locked'],TAMPER:['TAMPER','Bin was moved or tilted'],

3136	  COLLECTING:['PICKUP','Collection in progress']};

3137	function slot(ic,txt,active,cc,tip){

3138	  return '<span class="chip '+(active?'on':'off')+'" style="--cc:'+(cc||'var(--ink2)')+'"'+

3139	    (tip?' title="'+tip+'"':'')+'>'+ic+'<span>'+txt+'</span></span>';

3140	}

3141	function chip(ic,txt,bg){

3142	  return '<span class="chip'+(bg?' loud':'')+'"'+(bg?' style="background:'+bg+'"':'')+'>'+ic+'<span>'+txt+'</span></span>';

3143	}

3144	function tuneSensorIcons(root,d,c){

3145	  const merc=root.querySelector('.merc');

3146	  if(merc){const f=clamp((d.temp-10)/40,0.05,1),h=(6*f+1.4).toFixed(1);merc.setAttribute('height',h);merc.setAttribute('y',(15.4-h).toFixed(1));}

3147	  const nd=root.querySelector('.needle');

3148	  if(nd)nd.style.transform='rotate('+(clamp(d.weight_g/(c.weight_cap||4500),0,1.1)*150-75).toFixed(1)+'deg)';

3149	}

3150

3151	// Fill rate in %/h over the last 15 minutes of history (needs 3 points);

3152	// drives "time to full" and Collection priority.

3153	function fillRate(b){

3154	  const h=b.hist;if(h.length<3)return 0;

3155	  const last=h[h.length-1];let i=h.length-1;

3156	  while(i>0&&last.t-h[i].t<9e5)i--;

3157	  const span=(last.t-h[i].t)/36e5;

3158	  return span<=0.02?0:(last.fill-h[i].fill)/span;

3159	}

3160	/* renderPriority / renderTTF merged into renderCollectNext */

3161	let collectSig='';

3162	function ttfText(b){

3163	  const r=fillRate(b),left=b.cal.fill_th-b.data.fill;

3164	  if(left<=0)return {txt:'Full now',tc:'var(--bad)',tag:'NOW'};

3165	  if(r<=0.05)return {txt:'Not filling',tc:'var(--ink2)',tag:'IDLE'};

3166	  const h=left/r;

3167	  const txt=h<1?'~'+Math.round(h*60)+' min':'~'+h.toFixed(1)+' h';

3168	  if(h<2)return {txt,tc:'var(--bad)',tag:'SOON'};

3169	  if(h<8)return {txt,tc:'var(--warn)',tag:'TODAY'};

3170	  return {txt,tc:'var(--ok)',tag:'LATER'};

3171	}

3172	function collectUrgency(b){

3173	  /* lower = more urgent */

3174	  const p=b.data.fill,th=b.cal.fill_th;

3175	  if(p>=th)return -1000+p;

3176	  const r=fillRate(b);

3177	  if(r>0.05)return (th-p)/r;   /* hours to full */

3178	  return 9999-p;

3179	}

3180	function renderCollectNext(){

3181	  if(!state.bins.length){

3182	    const h=$('#collectNext');if(h&&collectSig!=='empty'){h.innerHTML='<p class="hint">No bins to collect from yet.</p>';collectSig='empty';}

3183	    const t=$('#collectToggle');if(t)t.classList.add('hide');

3184	    return;

3185	  }

3186	  const rows=state.bins.slice().sort((a,b2)=>collectUrgency(a)-collectUrgency(b2));

3187	  const sig=rows.map(b=>b.id+Math.round(b.data.fill)+b.cal.fill_th).join(',')+'|'+state.collectShowAll;

3188	  if(sig===collectSig)return;

3189	  collectSig=sig;

3190	  const N=3;

3191	  const tog=$('#collectToggle');

3192	  if(tog){

3193	    if(rows.length<=N){tog.classList.add('hide');}

3194	    else{tog.classList.remove('hide');tog.textContent=state.collectShowAll?'SHOW LESS':'SHOW ALL';}

3195	  }

3196	  const host=$('#collectNext');if(!host)return;

3197	  host.className=state.collectShowAll?'coll-show':'';

3198	  host.innerHTML=rows.map((b,i)=>{

3199	    const p=Math.round(b.data.fill),th=b.cal.fill_th;

3200	    const {txt,tc,tag}=ttfText(b);

3201	    const extra=i>=N?'coll-more':'';

3202	    return '<div class="crow '+extra+'" data-pb="'+b.id+'" title="Go to '+esc(b.name)+'">'+

3203	      '<span class="cn">'+(i+1)+'</span>'+

3204	      '<span style="width:9px;height:9px;border-radius:50%;background:'+tc+';flex-shrink:0"></span>'+

3205	      '<span class="cnm">'+esc(b.name)+'</span>'+

3206	      '<span class="cpc">'+p+'%</span>'+

3207	      '<span class="cwhen" style="color:'+tc+'">'+txt+'</span>'+

3208	      '<span class="lvtag" style="background:'+tc+'">'+tag+'</span></div>';

3209	  }).join('')||'<div class="empty">No bins yet.</div>';

3210	}

3211	let banSig='';

3212	function renderBanners(){

3213	  const out=[];

3214	  const cal=state.bins.filter(b=>b.data.status==='CALIBRATING');

3215	  const sim=state.bins.filter(b=>b.sim.mask>0);

3216	  const tam=state.bins.filter(b=>b.data.status==='TAMPER');

3217	  const full=state.bins.filter(b=>b.data.fill>=b.cal.fill_th);

3218	  if(cal.length)out.push(ban('teal','calib','CALIBRATING: '+cal.map(b=>b.name).join(', '),'Keep the bin empty and do not touch it.'));

3219	  if(tam.length)out.push(ban('red','tamper','TAMPER: '+tam.map(b=>b.name).join(', '),'Check the bin before resetting the alert.'));

3220	  if(full.length)out.push(ban('red','truck','COLLECT NOW: '+full.map(b=>b.name+' at '+Math.round(b.data.fill)+'%').join(', '),''));

3221	  if(sim.length)out.push(ban('purple','flask','SIMULATION MODE','Some readings are set by hand, not by the sensors.'));

3222	  if(!state.bins.length)out.push(ban('amber','wifi','No bins linked yet','Add a bin and type its IP address, or turn DEMO on at the start page to look around with made-up bins.'));

3223	  else if(state.demo)out.push(ban('amber','demo','Demo mode','Bins marked DEMO show made-up readings. Press DEMO in the top bar to go back to the start page and turn it off.'));

3224	  if(state.demo&&TOUR.msg)out.unshift(ban('purple','demo','Demo tour: '+TOUR.msg,'The tour runs by itself. Press a key on a demo bin and it waits a minute for you.'));

3225	  const flt=state.bins.filter(b=>(b.data.faults||[]).length);

3226	  if(flt.length)out.push(ban('amber','gear','SENSOR NOT ANSWERING: '+flt.map(b=>b.name+' ('+b.data.faults.join(', ')+')').join(', '),'Its readings show -- until the sensor answers again. Check the wiring.'));

3227	  if(state.bins.some(isLive)){const down=state.bins.filter(b=>isLive(b)&&!b.live);

3228	    if(down.length)out.push(ban('red','wifi',down.length===1?down[0].name+' is not answering':down.length+' bins are not answering','Readings shown are the last ones received. Check that the bin is on and on the same Wi-Fi, or set up Firebase or ThingSpeak.'));}

3229

3230	  const unread=unreadCount(),pending=state.alerts.filter(a=>!a.read);

3231	  let html='';

3232	  if(unread){

3233	    html+='<div class="popsec">Unread alerts</div>'+

3234	      pending.slice(0,6).map(a=>ban((SEV_TONE[a.sev]||SEV_TONE.info)[2],a.ic,a.title,a.meta+' · '+clock(a.t))).join('')+

3235	      (unread>6?'<div class="popsec">and '+(unread-6)+' more in the Alerts tab</div>':'')+

3236	      '<div class="popact"><button class="btn sm tint" style="--cc:var(--ok)" id="popRead">'+icon('check')+

3237	      '<span>MARK ALL READ</span></button><button class="btn sm tint" style="--cc:var(--c-tamper)" id="popOpen">'+

3238	      icon('bell')+'<span>OPEN ALERTS TAB</span></button></div>';

3239	  }

3240	  if(out.length)html+='<div class="popsec">Right now</div>'+out.join('');

3241	  if(html!==banSig){

3242	    banSig=html;

3243	    $('#bannerPop').innerHTML=html||'<div class="empty">Nothing needs attention.</div>';

3244	  }

3245

3246	  const pill=$('#statusPill'),n=out.length;

3247	  pill.classList.toggle('hide',!(unread||n));

3248	  if(!(unread||n)){

3249	    $('#bannerPop').classList.add('hide');

3250	    pill.setAttribute('aria-expanded','false');

3251	    lastUnread=0;return;

3252	  }

3253	  let pIco,pCol,pTxt;

3254	  if(unread){

3255	    const tone=SEV_TONE[SEV_RANK.find(s=>pending.some(a=>a.sev===s))]||SEV_TONE.info;

3256	    pIco=tone[0];pCol=tone[1];pTxt=unread+' unread';

3257	  }else{

3258	    const w=cal.length?['calib','var(--cal)']:tam.length?['tamper','var(--c-tamper)']

3259	      :full.length?['truck','var(--c-tamper)']:sim.length?['flask','var(--sim)']:['wifi','var(--warn)'];

3260	    pIco=w[0];pCol=w[1];pTxt=n+' active';

3261	  }

3262	  pill.style.setProperty('--cc',pCol);

3263	  $('#pillIco').innerHTML=icon(pIco);

3264	  $('#pillText').textContent=pTxt;

3265	  pill.title=unread?'Unread alerts. Click to see them.':'Live conditions. Click to see them.';

3266	  if(unread>lastUnread&&!reduce()){

3267	    pill.classList.remove('flash');void pill.offsetWidth;pill.classList.add('flash');

3268	  }

3269	  lastUnread=unread;

3270	}

3271	function ban(cls,ic,t,s){return '<div class="banner '+cls+'">'+icon(ic)+'<span>'+t+'</span>'+(s?'<small>'+s+'</small>':'')+'</div>'}

3272	let alertSig='';

3273	/* History tools from Garbage-Guard: one bar of symbols filters by kind, a second bar by date,

3274	   and MARK READ and DELETE act on exactly the alerts shown. */

3275	const AKINDS=[['all','layers','All','var(--accent)'],['full','fill','Full bins','var(--bad)'],['sensor','temp','Sensor readings','var(--c-temp)'],['tamper','tamper','Tamper','var(--c-tamper)'],

3276	  ['lock','lock','Lock and unlock','var(--sim)'],['pickup','truck','Pickups','var(--c-link)'],['setup','calib','Calibration and settings','var(--cal)']];

3277	const AKNAME={all:['alert','alerts'],full:['full-bin alert','full-bin alerts'],sensor:['sensor alert','sensor alerts'],tamper:['tamper alert','tamper alerts'],

3278	  lock:['lock event','lock events'],pickup:['pickup','pickups'],setup:['setup event','setup events']};

3279	const ARANGE=[['today','Today'],['7','7 days'],['30','30 days'],['any','Any']];

3280	function rangeStart(){const r=state.arange,d=new Date();if(r==='today'){d.setHours(0,0,0,0);return +d}if(r==='any')return 0;return Date.now()-(+r)*86400000}

3281	function shownAlerts(){const s=rangeStart();return state.alerts.filter(a=>(state.akind==='all'||a.k===state.akind)&&a.t>=s)}

3282	function scopeTxt(n){const nm=AKNAME[state.akind]||AKNAME.all,w=n===1?nm[0]:nm[1],r={today:' from today','7':' from the last 7 days','30':' from the last 30 days',any:''}[state.arange];

3283	  return (state.akind==='all'&&state.arange==='any'?'all '+n+' '+w:n+' '+w)+r}

3284	function renderAlertTools(){

3285	  const kh=AKINDS.map(k=>'<button class="fchip'+(state.akind===k[0]?' on':'')+'" data-akind="'+k[0]+'" title="'+k[2]+'" aria-label="'+k[2]+'" style="--fc:'+k[3]+'">'+icon(k[1])+'</button>').join('');

3286	  if($('#alertKinds').dataset.sig!==kh){$('#alertKinds').dataset.sig=kh;$('#alertKinds').innerHTML=kh;}

3287	  const rh=ARANGE.map(r=>'<button class="btn sm binbtn" data-arange="'+r[0]+'" aria-pressed="'+(state.arange===r[0])+'" style="--cc:var(--c-tamper)">'+r[1]+'</button>').join('');

3288	  if($('#alertRange').dataset.sig!==rh){$('#alertRange').dataset.sig=rh;$('#alertRange').innerHTML=rh;}

3289	}

3290	function markShown(){

3291	  const s=shownAlerts().filter(a=>!a.read);if(!s.length)return;

3292	  s.forEach(a=>{a.read=true});alertSig='';banSig='';renderAlerts();renderBanners();

3293	  toast('Marked '+s.length+' alert'+(s.length>1?'s':'')+' as read.');

3294	}

3295	function askDelete(){

3296	  const n=shownAlerts().length;if(!n)return;

3297	  $('#alertConfirmText').textContent='Delete '+scopeTxt(n)+'? This cannot be undone.';

3298	  $('#alertYes').innerHTML=icon('trash')+'<span>DELETE '+n+'</span>';

3299	  $('#alertConfirm').classList.remove('hide');

3300	}

3301	function doDelete(){

3302	  const s=shownAlerts(),ids=new Set(s.map(a=>a.id)),t=scopeTxt(s.length);

3303	  state.alerts=state.alerts.filter(a=>!ids.has(a.id));

3304	  $('#alertConfirm').classList.add('hide');alertSig='';banSig='';renderAlerts();renderBanners();

3305	  toast('Deleted '+t+'.');

3306	}

3307	function renderAlerts(){

3308	  const list=$('#alertList'),unread=unreadCount(),shown=shownAlerts(),unShown=shown.filter(a=>!a.read).length;

3309	  renderAlertTools();

3310	  const mk=$('#btnMarkAll');if(mk)mk.disabled=!unShown;

3311	  const dl=$('#btnClear');if(dl)dl.disabled=!shown.length;

3312	  const sc=$('#alertScope'),kn=(AKINDS.find(k=>k[0]===state.akind)||AKINDS[0])[2];

3313	  const sh=state.alerts.length?(shown.length?'Showing '+scopeTxt(shown.length)+'<small>'+kn+(unShown?', '+unShown+' unread':', all read')+'. MARK READ and DELETE act on exactly these.</small>':'No alerts match.<small>Change the kind or the dates.</small>'):'';

3314	  if(sc&&sc.dataset.sig!==sh){sc.dataset.sig=sh;sc.innerHTML=sh;}

3315	  const hd=$('#alertHead');

3316	  if(hd){

3317	    const hh=icon('clock')+'<span>ALERT HISTORY</span>'+(unread?'<em class="unreadpip">'+unread+' UNREAD</em>':'');

3318	    if(hd.dataset.sig!==hh){hd.dataset.sig=hh;hd.innerHTML=hh;}

3319	  }

3320	  if(!state.alerts.length){

3321	    if(alertSig!=='@empty'){alertSig='@empty';

3322	      list.innerHTML='<div class="empty">No alerts yet. This fills up as bins reach their limits.</div>';}

3323	    return;

3324	  }

3325	  const sv={critical:['var(--bad)','CRITICAL'],warning:['var(--warn)','WARNING'],

3326	    info:['var(--sim)','INFO'],success:['var(--ok)','RESOLVED']};

3327	  const visible=shown;

3328	  const html=visible.map(a=>{

3329	    const s=sv[a.sev]||['var(--ink2)','NOTE'];

3330	    const lbl=a.read?'Mark as unread':'Mark as read';

3331	    return '<div class="alert" data-lv="'+a.sev+'" data-read="'+(a.read?1:0)+'" style="--cc:'+s[0]+'">'+

3332	      '<button class="rd" data-read-toggle="'+a.id+'" title="'+lbl+'" aria-label="'+lbl+'"><i></i></button>'+

3333	      '<span class="av" style="color:'+(a.sev==='critical'?'#fff':s[0])+'">'+icon(a.ic)+

3334	      '</span><span style="flex:1;min-width:0"><span class="ttl">'+a.title+'</span><div class="meta">'+a.meta+' &middot; '+ago(a.t)+'</div></span>'+

3335	      '<span class="lvtag" style="'+(a.sev==='critical'?'':'background:'+s[0])+'">'+s[1]+'</span>'+

3336	      '<button class="del" data-del="'+a.id+'" title="Remove this alert">'+icon('cross')+'</button></div>';

3337	  }).join('')||(visible.length===0&&state.alerts.length?'<div class="empty">No alerts match. Change the kind or the dates.</div>':'');

3338	  if(html===alertSig)return;

3339	  alertSig=html;list.innerHTML=html;

3340	}

3341	function ago(t){const s=Math.round((Date.now()-t)/1000);return s<60?s+' s ago':s<3600?Math.round(s/60)+' min ago':s<172800?Math.round(s/3600)+' h ago':Math.round(s/86400)+' days ago'}

3342	function renderLink(){

3343	  const used={local:0,fb:0,ts:0};let live=0,cfg=0;

3344	  state.bins.forEach(b=>{if(isLive(b))cfg++;if(b.live){live++;if(b.route)used[b.route]++;}});

3345	  const names=Object.keys(used).filter(k=>used[k]);

3346	  $('#wifiico').innerHTML=names.length===1&&names[0]!=='local'?icon(ROUTE[names[0]][2]):icon('wifi',live>0);

3347	  const link=$('#link');

3348	  link.style.setProperty('--lc',names.length?ROUTE[names[0]][1]:'var(--ink2)');

3349	  link.style.color=names.length?ROUTE[names[0]][1]:'';

3350	  link.title=names.map(k=>ROUTE[k][0]+': '+used[k]+' bin'+(used[k]>1?'s':'')).join(', ');

3351	  $('#linktext').textContent=!state.bins.length?'No bins':!cfg?'Demo data':!live?'Not connected':

3352	    names.length===3?'All 3 links':names.map(k=>ROUTE[k][0]).join(' + ')+(live<cfg?' ('+live+' of '+cfg+')':'');

3353	}

3354

3355	/* ---------- charts ---------- */

3356	// Charts are drawn by hand on <canvas> (no chart library), see line().

3357	function drawCharts(){

3358	  const since=Date.now()-state.rangeMin*60000;

3359	  const picked=state.bins.filter(b=>state.chartPick.includes(b.id));

3360	  let show=picked.length?picked:state.bins;

3361	  /* adaptive fill selector — sig check prevents rebuild from closing an open dropdown */

3362	  const fillSegHtml=adaptiveSeg('fillSeg','pick',state.chartPick,true);

3363	  if($('#fillSeg').dataset.sig!==fillSegHtml){$('#fillSeg').dataset.sig=fillSegHtml;$('#fillSeg').innerHTML=fillSegHtml;}

3364	  /* cap ALL BINS display at 5 fullest */

3365	  let capped=false;

3366	  if(!state.chartPick.length&&state.bins.length>5){

3367	    show=state.bins.slice().sort((a,b2)=>b2.data.fill-a.data.fill).slice(0,5);

3368	    capped=true;

3369	  }

3370	  const series=show.map(b=>({color:colorOf(state.bins.indexOf(b)),dash:dashOf(state.bins.indexOf(b)),

3371	    mark:b.cal.fill_th,pts:b.hist.filter(p=>p.t>=since).map(p=>({t:p.t,v:p.fill}))}));

3372	  const uniqTh=[...new Set(show.map(x=>x.cal.fill_th))];

3373	  const fillMarks=[];

3374	  show.forEach(x=>{

3375	    const txt=(uniqTh.length>1?x.name.toUpperCase()+' LIMIT ':'ALERT LIMIT ')+x.cal.fill_th+'%';

3376	    if(!fillMarks.some(m=>m.text===txt))

3377	      fillMarks.push({v:x.cal.fill_th,color:getVar('--bad'),text:txt,dash:[7,6]});

3378	  });

3379	  line($('#cFill'),series,0,100,fillMarks);

3380	  $('#legFill').innerHTML=show.map(b=>{const i=state.bins.indexOf(b);

3381	    return '<span><i style="background:'+colorOf(i)+';'+(dashOf(i)?'opacity:.95;background:repeating-linear-gradient(90deg,'+colorOf(i)+' 0 5px,transparent 5px 9px)':'')+'"></i>'+esc(b.name)+'</span>';

3382	  }).join('')+'<span><i style="background:repeating-linear-gradient(90deg,var(--bad) 0 6px,transparent 6px 11px)"></i>Threshold</span>';

3383	  let cn=$('#fillCappedNote');if(!cn){cn=document.createElement('p');cn.id='fillCappedNote';cn.className='chart-note';$('#legFill').parentElement.appendChild(cn);}

3384	  cn.textContent=capped?'Showing 5 fullest of '+state.bins.length+' bins. Pick a bin above to see others.':'';

3385

3386	  const b=byId(state.envBin)||state.bins[0];

3387	  const SHORT=['ALL','TEMPERATURE','HUMIDITY','AIR QUALITY','PRESSURE'];

3388	  const modeHtml=ENV_MODES.map((m,mi)=>'<button class="btn sm binbtn" style="--cc:'+m[3]+'" data-envmode="'+m[0]+

3389	    '" aria-pressed="'+(state.envMode===m[0])+'">'+icon(m[2])+'<span>'+SHORT[mi]+'</span></button>').join('');

3390	  if($('#envMode').dataset.sig!==modeHtml){$('#envMode').dataset.sig=modeHtml;$('#envMode').innerHTML=modeHtml;}

3391	  const segHtml=adaptiveSeg('envSeg','env',[b&&b.id],false);

3392	  if($('#envSeg').dataset.sig!==segHtml){$('#envSeg').dataset.sig=segHtml;$('#envSeg').innerHTML=segHtml;}

3393	  if(!b)return;

3394	  drawEnv(b,since);

3395	}

3396	const peak=pts=>pts.reduce((m,p)=>Math.max(m,p.v),0);

3397	function niceMax(v){

3398	  const steps=[5,10,15,20,25,50,100,150,200,250,500,1000,1250,2000,2500,5000];

3399	  for(const s of steps)if(s*4>=v)return s*4;

3400	  return Math.ceil(v/4000)*4000;

3401	}

3402	/* One reading at a time is drawn in its real unit, on the BME680 datasheet range (see SPEC).

3403	   ALL would need several y-scales, which cannot be read honestly, so BOTH draws temperature,

3404	   humidity and air quality as a share of their own limit (one dashed line at 100% is the limit

3405	   for all three). Pressure has no limit, so it only has its own tab. */

3406	const HKEY={temp:'temp',humid:'humidity',press:'pressure',gas:'gas'};

3407	function drawEnv(b,since){

3408	  const h=b.hist.filter(p=>p.t>=since),d=b.data;

3409	  const tTh=b.cal.temp_th||40,hTh=b.cal.hum_th||85,gTh=b.cal.gas_th||200;

3410	  const tc=getVar('--c-temp'),gc=getVar('--c-gas'),hc=getVar('--c-humid'),bad=getVar('--bad');

3411	  const dashed=(c,a,b2)=>'repeating-linear-gradient(90deg,'+c+' 0 '+a+'px,transparent '+a+'px '+b2+'px)';

3412	  let series,marks,min=0,max,ticks,note,leg='';

3413	  if(SPEC[state.envMode]){

3414	    const k=state.envMode,sp=SPEC[k],col=getVar(sp.col),key=HKEY[k];

3415	    const pts=h.filter(p=>Number.isFinite(p[key])).map(p=>({t:p.t,v:p[key]}));

3416	    const lim=k==='temp'?tTh:k==='humid'?hTh:k==='gas'?gTh:null;

3417	    min=sp.min;max=sp.max;ticks=sp.ticks;

3418	    series=[{color:col,dash:0,mark:lim,pts:pts}];

3419	    marks=lim==null?[]:[{v:lim,color:bad,text:'ALERT LIMIT '+lim+' '+sp.unit,dash:[7,6]}];

3420	    leg='<span><i style="background:'+col+'"></i>'+sp.name+', now <span class="now">'+fmt(d[key],sp.dec)+' '+sp.unit+'</span></span>'+

3421	      (lim==null?'':'<span><i style="background:'+dashed(bad,6,11)+'"></i>Alert limit '+lim+' '+sp.unit+'</span>')+

3422	      '<span>Scale '+sp.min+' to '+sp.max+' '+sp.unit+'</span>';

3423	    note={

3424	      temp:'Temperature of '+b.name+' in degrees C, after the offset from Settings. The scale runs to 85 C, the most the BME680 can read. The dashed line is its alert limit.',

3425	      humid:'Relative humidity of '+b.name+', 0 to 100 percent, the full BME680 range. The dashed line is its alert limit.',

3426	      press:'Air pressure around '+b.name+' in hPa, on the BME680 range of 300 to 1100 hPa, so normal weather changes look small. Pressure is charted here only. It has no tile on the bin card and raises no alert.',

3427	      gas:'Air quality index of '+b.name+' from the BME680 gas sensor, 0 to 500, lower is cleaner. The dashed line is its alert limit.'

3428	    }[k];

3429	  }else{

3430	    const tp=h.map(p=>({t:p.t,v:p.temp/tTh*100})),gp=h.map(p=>({t:p.t,v:p.gas/gTh*100}));

3431	    const hp=h.filter(p=>Number.isFinite(p.humidity)).map(p=>({t:p.t,v:p.humidity/hTh*100}));

3432	    max=200;ticks=[0,50,100,150,200]; /* fixed, so the 100% limit always sits in the middle */

3433	    series=[{color:tc,dash:0,mark:100,pts:tp},{color:gc,dash:1,mark:100,pts:gp},{color:hc,dash:2,mark:100,pts:hp}];

3434	    marks=[{v:100,color:bad,text:'ALERT LIMIT (100%)',dash:[7,6]}];

3435	    note='Temperature, humidity and air quality are each drawn as a share of their own alert limit, so the single dashed line at 100% is the limit for all three. The scale runs to 200%, which keeps that line in the middle. '+

3436	      'Pick one reading to see real units. Pressure has its own tab, last.';

3437	    leg='<span><i style="background:'+tc+'"></i>Temperature, limit '+tTh+' C</span>'+

3438	      '<span><i style="background:'+dashed(hc,2.5,6.5)+'"></i>Humidity, limit '+hTh+' %RH</span>'+

3439	      '<span><i style="background:'+dashed(gc,5,9)+'"></i>Air quality, limit '+gTh+' IAQ</span>'+

3440	      '<span><i style="background:'+dashed(bad,6,11)+'"></i>Alert limit (100%)</span>';

3441	  }

3442	  const lg=$('#legEnv');

3443	  if(lg.dataset.sig!==leg){lg.dataset.sig=leg;lg.innerHTML=leg;}

3444	  lg.classList.toggle('hide',!leg);

3445	  if($('#envNote').textContent!==note)$('#envNote').textContent=note;

3446	  line($('#cEnv'),series,min,max,marks,ticks);

3447	}

3448	function dashOf(i){return [0,1,2,3][i%4]}

3449	function dashPat(d){return d===1?[7,5]:d===2?[2.5,4]:d===3?[11,4,2.5,4]:[]}

3450	let chartT=1,chartRaf=0;

3451	function revealCharts(){

3452	  if(reduce()){chartT=1;drawCharts();return;}

3453	  cancelAnimationFrame(chartRaf);

3454	  const t0=performance.now(),dur=700;

3455	  const run=now=>{

3456	    const k=Math.min(1,(now-t0)/dur);

3457	    chartT=1-Math.pow(1-k,3);

3458	    drawCharts();

3459	    if(k<1)chartRaf=requestAnimationFrame(run);else chartT=1;

3460	  };

3461	  chartRaf=requestAnimationFrame(run);

3462	}

3463	function getVar(n){return getComputedStyle(document.documentElement).getPropertyValue(n).trim()}

3464	function hex2rgb(h){

3465	  h=(h||'').trim().replace('#','');

3466	  if(h.length===3)h=h.split('').map(c=>c+c).join('');

3467	  const n=parseInt(h||'888888',16);

3468	  return [(n>>16)&255,(n>>8)&255,n&255];

3469	}

3470	function rgba(h,a){const c=hex2rgb(h);return 'rgba('+c[0]+','+c[1]+','+c[2]+','+a+')'}

3471	function mixHex(a,b,t){

3472	  const A=hex2rgb(a),B=hex2rgb(b);

3473	  const m=i=>Math.round(A[i]+(B[i]-A[i])*t).toString(16).padStart(2,'0');

3474	  return '#'+m(0)+m(1)+m(2);

3475	}

3476	function line(cv,series,min,max,marks,ticks){

3477	  if(!cv)return;

3478	  const dpr=window.devicePixelRatio||1,w=cv.clientWidth,h=cv.clientHeight;

3479	  cv.width=w*dpr;cv.height=h*dpr;

3480	  const g=cv.getContext('2d');g.setTransform(dpr,0,0,dpr,0,0);g.clearRect(0,0,w,h);

3481	  const ink2=getVar('--ink2'),acc=getVar('--accent');

3482	  const L=40,R=14,T=16,B=24,pw=w-L-R,ph=h-T-B,y=v=>T+ph-((v-min)/(max-min))*ph;

3483	  g.font='11px Inter, system-ui, sans-serif';g.strokeStyle=ink2;g.lineWidth=1;

3484	  const tk=ticks&&ticks.length?ticks:[0,1,2,3,4].map(i=>min+(max-min)*i/4);

3485	  tk.forEach(v=>{g.globalAlpha=.25;g.beginPath();g.moveTo(L,y(v));g.lineTo(w-R,y(v));g.stroke();});

3486	  g.globalAlpha=1;g.fillStyle=ink2;g.textAlign='right';g.textBaseline='middle';

3487	  tk.forEach(v=>{g.fillText(Math.round(v),L-8,y(v));});

3488	  const all=series.flatMap(s=>s.pts);

3489	  if(all.length<2){g.textAlign='center';g.fillText('Collecting data',w/2,T+ph/2);return;}

3490	  const t0=Math.min(...all.map(p=>p.t)),t1=Math.max(...all.map(p=>p.t));

3491	  const x=t=>L+(t1===t0?pw:(t-t0)/(t1-t0)*pw);

3492	  drawMarks(g,marks||[],L,R,w,T,ph,y);

3493	  const hot=getVar('--bad')||'#D93025';

3494	  const baseCol=getVar('--base')||'#E4DCD2';

3495	  series.forEach(s=>{

3496	    if(s.pts.length<2)return;

3497	    const pts=chartT>=1?s.pts:s.pts.slice(0,Math.max(2,Math.ceil(s.pts.length*chartT)));

3498	    const warm=mixHex(s.color,hot,.82);

3499	    const sMark=s.mark;

3500	    let stroke=s.color,fill;

3501	    if(sMark!=null&&sMark>min&&sMark<max){

3502	      const p=clamp((max-sMark)/(max-min),0,1);

3503	      stroke=g.createLinearGradient(0,T,0,T+ph);

3504	      stroke.addColorStop(0,warm);stroke.addColorStop(Math.max(0,p-0.001),warm);

3505	      stroke.addColorStop(p,s.color);stroke.addColorStop(1,s.color);

3506	      fill=g.createLinearGradient(0,T,0,T+ph);

3507	      fill.addColorStop(0,rgba(warm,.42));fill.addColorStop(Math.max(0,p-0.001),rgba(warm,.26));

3508	      fill.addColorStop(p,rgba(s.color,.30));fill.addColorStop(1,rgba(s.color,0));

3509	    }else{

3510	      fill=g.createLinearGradient(0,T,0,T+ph);

3511	      fill.addColorStop(0,rgba(s.color,.34));fill.addColorStop(1,rgba(s.color,0));

3512	    }

3513	    g.beginPath();

3514	    pts.forEach((p,i)=>{const X=x(p.t),Y=y(clamp(p.v,min,max));i?g.lineTo(X,Y):g.moveTo(X,Y);});

3515	    g.lineTo(x(pts[pts.length-1].t),T+ph);g.lineTo(x(pts[0].t),T+ph);g.closePath();

3516	    g.fillStyle=fill;g.fill();

3517	    const trace=()=>{g.beginPath();

3518	      pts.forEach((p,i)=>{const X=x(p.t),Y=y(clamp(p.v,min,max));i?g.lineTo(X,Y):g.moveTo(X,Y);});};

3519	    g.lineJoin='round';g.lineCap='round';

3520	    g.setLineDash([]);trace();

3521	    g.strokeStyle=rgba(baseCol,.85);g.lineWidth=6.2;g.stroke();

3522	    g.setLineDash(dashPat(s.dash||0));trace();

3523	    g.strokeStyle=stroke;g.lineWidth=2.9;g.stroke();

3524	    g.setLineDash([]);

3525	    const last=pts[pts.length-1],ly=clamp(last.v,min,max);

3526	    const dotCol=(sMark!=null&&last.v>=sMark)?warm:s.color;

3527	    g.fillStyle=rgba(dotCol,.25);g.beginPath();g.arc(x(last.t),y(ly),8,0,7);g.fill();

3528	    g.fillStyle=dotCol;g.beginPath();g.arc(x(last.t),y(ly),3.8+(chartT<1?2.5*(1-chartT):0),0,7);g.fill();

3529	  });

3530	  g.fillStyle=ink2;g.textBaseline='top';g.textAlign='left';g.fillText(clock(t0),L,T+ph+8);

3531	  g.textAlign='right';g.fillStyle=acc;g.fillText('now',w-R,T+ph+8);

3532	}

3533	function roundRect(g,x,y,w,h,r){

3534	  g.beginPath();g.moveTo(x+r,y);g.arcTo(x+w,y,x+w,y+h,r);g.arcTo(x+w,y+h,x,y+h,r);

3535	  g.arcTo(x,y+h,x,y,r);g.arcTo(x,y,x+w,y,r);g.closePath();

3536	}

3537	/* A dashed line on its own does not say which reading it belongs to, so every

3538	   threshold carries a small label chip pinned to the left end of its line. */

3539	function drawMarks(g,marks,L,R,w,T,ph,y){

3540	  const surface=getVar('--well')||'#DED5CA',used=[];

3541	  marks.forEach(m=>{

3542	    if(m.v==null)return;

3543	    const yy=y(m.v);

3544	    if(yy<T-1||yy>T+ph+1)return;

3545	    g.save();

3546	    g.setLineDash(m.dash&&m.dash.length?m.dash:[7,6]);

3547	    g.strokeStyle=m.color;g.lineWidth=1.8;g.globalAlpha=.9;

3548	    g.beginPath();g.moveTo(L,yy);g.lineTo(w-R,yy);g.stroke();

3549	    g.restore();

3550	    if(!m.text)return;

3551	    g.save();

3552	    g.font='700 10px Inter, system-ui, -apple-system, sans-serif';

3553	    const bw=g.measureText(m.text).width+13,bh=16,bx=L+9;

3554	    let by=yy-bh-4;

3555	    if(by<T+1)by=yy+4;

3556	    let guard=0;

3557	    while(used.some(u=>Math.abs(u-by)<bh+3)&&guard++<8)by+=bh+4;

3558	    if(by+bh>T+ph-1)by=Math.max(T+1,yy-bh-4);

3559	    used.push(by);

3560	    roundRect(g,bx,by,bw,bh,5);

3561	    g.fillStyle=surface;g.globalAlpha=.94;g.fill();

3562	    g.globalAlpha=1;g.strokeStyle=m.color;g.lineWidth=1.2;g.stroke();

3563	    g.fillStyle=m.color;g.textAlign='left';g.textBaseline='middle';

3564	    g.fillText(m.text,bx+6.5,by+bh/2+.5);

3565	    g.restore();

3566	  });

3567	}

3568	function clock(t){const d=new Date(t);return String(d.getHours()).padStart(2,'0')+':'+String(d.getMinutes()).padStart(2,'0')}

3569

3570	/* ---------- bin selectors ---------- */

3571	const SEG_THRESH=8;

3572	function segHTML(attr,sel){

3573	  return state.bins.map(b=>'<button class="btn sm binbtn" data-'+attr+'="'+b.id+'" data-sim="'+(b.sim.mask?1:0)+

3574	    '" aria-pressed="'+(b.id===sel)+'">'+icon('fill')+'<span>'+esc(b.name)+'</span></button>').join('');

3575	}

3576	/* adaptiveSeg: for fill and env pickers in charts */

3577	function adaptiveSeg(hostId,attr,selIds,withAll){

3578	  if(state.bins.length<=SEG_THRESH){

3579	    /* button strip */

3580	    const allBtn=withAll?'<button class="btn sm binbtn" data-'+attr+'="ALL" aria-pressed="'+(selIds.length===0||!selIds[0])+'">'+icon('layers')+'<span>ALL BINS</span></button>':'';

3581	    return allBtn+state.bins.map((b,i)=>'<button class="btn sm binbtn" style="--cc:'+colorOf(i)+'" data-'+attr+'="'+b.id+'" data-sim="'+(b.sim.mask?1:0)+'" aria-pressed="'+selIds.includes(b.id)+'">'+icon('fill')+'<span>'+esc(b.name)+'</span></button>').join('');

3582	  }

3583	  /* picker button + floating list */

3584	  const cur=state.bins.find(b=>selIds.includes(b.id));

3585	  const label=withAll&&!cur?'ALL BINS':cur?esc(cur.name):'PICK BIN';

3586	  const searchRow=state.bins.length>=9?'<input type="text" placeholder="Search bins…" id="'+hostId+'Search" aria-label="Search bins">':'';

3587	  const items=(withAll?'<button data-'+attr+'="ALL" aria-pressed="'+(selIds.length===0||!selIds[0])+'">'+icon('layers')+'<span>ALL BINS</span></button>':'')+

3588	    state.bins.map((b,i)=>'<button data-'+attr+'="'+b.id+'" data-sim="'+(b.sim.mask?1:0)+'" aria-pressed="'+selIds.includes(b.id)+'"><span class="bcc" style="background:'+colorOf(i)+'"></span>'+esc(b.name)+'</button>').join('');

3589	  return '<div class="pickwrap" id="'+hostId+'Wrap"><button class="btn sm binbtn" id="'+hostId+'Btn">'+icon('fill')+'<span>'+label+'</span></button>'+

3590	    '<div class="picklist hide" id="'+hostId+'List">'+searchRow+items+'</div></div>';

3591	}

3592	/* ---------- settings ---------- */

3593	const MAP={s_dempty:'d_empty',s_dfull:'d_full',s_fill:'fill_th',s_temp:'temp_th',s_hum:'hum_th',s_gas:'gas_th',s_tilt:'tilt_th',

3594	  s_wcap:'weight_cap',s_sclosed:'servo_closed',s_sopen:'servo_open',s_pin:'collect_pin',

3595	  s_pirhold:'pir_hold_s',s_buzz:'buzz_s',s_toff:'temp_off',s_hoff:'hum_off',s_poff:'press_off'};

3596	// MAP links each settings input id to its Cal key.

3597	function loadSettings(){

3598	  const seg=adaptiveSeg('setSeg','set',[state.selSet],false);

3599	  if($('#setSeg').dataset.sig!==seg){$('#setSeg').dataset.sig=seg;$('#setSeg').innerHTML=seg;}

3600	  const b=byId(state.selSet);if(!b)return;

3601	  for(const k in MAP)$('#'+k).value=b.cal[MAP[k]];

3602	  $('#s_autolock').checked=!!b.cal.auto_lock;

3603	  sectSummaries(b);

3604	  sectAllRender();

3605	  refreshCalGate();

3606	}

3607	function allSectsOpen(){return $$('.sect').every(d=>d.classList.contains('open'))}

3608	function sectAllRender(){const b=$('#sectAll');if(!b)return;const o=allSectsOpen();

3609	  b.innerHTML=icon('layers')+'<span>'+(o?'HIDE ALL':'SHOW ALL')+'</span>';b.style.display='inline-flex';b.style.alignItems='center';b.style.gap='8px';}

3610	function sectSummaries(b){

3611	  const c=b.cal,f=(v,d)=>Number(v).toFixed(d),d=b.data||{};

3612	  const sum={range:f(c.d_empty,1)+' to '+f(c.d_full,1)+' cm',

3613	    limits:f(c.fill_th,0)+'% \u00B7 '+f(c.temp_th,0)+'\u00B0C \u00B7 '+f(c.hum_th,0)+'%RH \u00B7 '+f(c.gas_th,0)+' IAQ',

3614	    air:'offset '+f(c.temp_off,1)+'\u00B0C',

3615	    lid:'PIN \u2022\u2022\u2022\u2022 \u00B7 '+f(c.pir_hold_s,0)+' s'+(c.auto_lock?' \u00B7 auto-lock':''),

3616	    weight:Number.isFinite(d.weight_g)?(d.weight_g/1000).toFixed(2)+' kg now':'',

3617	    touch:b.touch&&b.touch.custom!=null?(b.touch.custom?'calibrated':'default values'):''};

3618	  $$('.sect').forEach(x=>{const e=x.querySelector('.ssum');if(e)e.textContent=sum[x.dataset.sec]||'';});

3619	}

3620	function calibrating(){const b=byId(state.selSet);return !!(b&&state.calib&&state.calib.bin===b.id)}

3621	function formDirty(){

3622	  const b=byId(state.selSet);if(!b)return false;

3623	  for(const k in MAP)if(+$('#'+k).value!==+b.cal[MAP[k]])return true;

3624	  return ($('#s_autolock').checked?1:0)!==(b.cal.auto_lock?1:0);

3625	}

3626	function refreshDirty(){

3627	  const on=state.role!=='collector'&&calibrating(),dirty=on&&formDirty();

3628	  const save=$('#btnSave'),cancel=$('#btnCancel');

3629	  save.disabled=!dirty;cancel.disabled=!dirty;

3630	  save.classList.toggle('solid',dirty);save.classList.toggle('tint',!dirty);

3631	}

3632	/* The collector sees every setting and every bin but edits nothing, so the

3633	   calibration switch is replaced by a read-only notice and the write buttons

3634	   go away entirely rather than sitting there greyed out. */

3635	function renderPin(){

3636	  const card=$('#pinCard'),b=byId(state.selSet),show=!!b;

3637	  card.classList.toggle('hide',!show);

3638	  if(!show)return;

3639	  if(card.dataset.binid!==b.id){card.dataset.binid=b.id;card.dataset.masked='1';}

3640	  const masked=card.dataset.masked!=='0';

3641	  $('#pinCardIco').innerHTML=icon('pin');

3642	  $('#pinName').textContent=b.name;

3643	  $('#pinSub').textContent=state.role==='collector'?'Type this on the bin touchscreen when you tap COLLECT.':

3644	    'Collectors type this on the bin touchscreen for COLLECT and WI-FI SETUP. Change it under Lid and lock.';

3645	  $('#pinVal').textContent=masked?'\u2022\u2022\u2022\u2022':String(b.cal.collect_pin);

3646	  $('#pinVal').classList.toggle('masked',masked);

3647	  const eye=$('#pinEye');

3648	  eye.classList.add('tint');eye.style.setProperty('--cc','var(--cal)');

3649	  eye.style.display='inline-flex';eye.style.alignItems='center';eye.style.gap='8px';

3650	  eye.innerHTML=icon(masked?'eye':'eyeoff')+'<span>'+(masked?'SHOW':'HIDE')+'</span>';

3651	}

3652	function refreshCalGate(){

3653	  const col=state.role==='collector';

3654	  const on=!col&&calibrating(),b=byId(state.selSet);

3655	  $('#setFields').disabled=!on;

3656	  $('#calBar').classList.toggle('hide',col);

3657	  $('#roNote').classList.toggle('hide',!col);

3658	  $('#roIco').innerHTML=icon('eye');

3659	  ['btnSave','btnCancel','btnExport'].forEach(id=>$('#'+id).classList.toggle('hide',col));

3660	  renderPin();

3661	  refreshDirty();

3662	  $('#calToggle').checked=on;

3663	  $('#calBar').classList.toggle('on',on);

3664	  $('#calIco').innerHTML=icon(on?'calib':'lock',true);

3665	  $('#calTitle').textContent=on?'Calibration mode is on'+(b?' for '+b.name:''):'Settings are locked';

3666	  $('#calSub').textContent=on

3667	    ?'Change what you need, then press SAVE SETTINGS. Switching this off without saving leaves the bin as it was.'

3668	    :'Switch on calibration mode to change anything here. The bin shows its teal calibration screen while it is on.';

3669	  $$('#setSeg .btn').forEach(x=>{x.disabled=on});

3670	  if(col)$('#setMsg').textContent='Read only. Pick any bin above to see its PIN and the limits it is running on.';

3671	  updateSettingsLive();

3672	}

3673	function updateSettingsLive(){

3674	  const b=byId(state.selSet);if(!b)return;

3675	  const live=$('#s_live'),dist=$('#s_dist');

3676	  if(live)live.textContent=(b.data.weight_g/1000).toFixed(2)+' kg';

3677	  if(dist)dist.textContent=fmt(b.data.distance_cm,1)+' cm';

3678	  const tc=$('#s_touch');

3679	  if(tc){

3680	    const t=b.touch;

3681	    tc.textContent=!isLive(b)?'Demo bin, no touchscreen'

3682	      :!t?'Waiting for the bin'

3683	      :(t.custom?'Calibrated on the bin':'Default values')+' \u00B7 X '+t.x0+'\u2013'+t.x1+' \u00B7 Y '+t.y0+'\u2013'+t.y1+(t.swap?' \u00B7 axes swapped':'');

3684	  }

3685	  const tl=$('#s_tlive'),iq=$('#s_iaq');

3686	  if(tl)tl.textContent=fmt(b.data.temp_raw,1)+' / '+fmt(b.data.temp,1)+' \u00B0C';

3687	  if(iq){

3688	    const st=['Stabilizing','Low accuracy','Medium accuracy','Calibrated'][clamp(Math.round(b.data.iaq_acc),0,3)];

3689	    const l=b.data.warmEnd?Math.max(0,Math.round((b.data.warmEnd-Date.now())/1000)):null;

3690	    iq.textContent=isLive(b)&&b.data.iaq_acc===0

3691	      ?'Warming up'+(l?', '+Math.floor(l/60)+':'+String(l%60).padStart(2,'0')+' left':', almost ready')+(Number.isFinite(b.data.gas_k)?' \u00B7 gas '+Math.round(b.data.gas_k)+' k\u03A9':'')

3692	      :st+', IAQ '+Math.round(b.data.gas);

3693	  }

3694	}

3695	// POST /api/calibrate with the changed keys. Needs the bin's Wi-Fi.

3696	async function saveSettings(){

3697	  const b=byId(state.selSet);if(!b)return;

3698	  const c={};for(const k in MAP)c[MAP[k]]=+$('#'+k).value;

3699	  c.auto_lock=$('#s_autolock').checked?1:0;

3700	  if(c.d_full>=c.d_empty){$('#setMsg').textContent='Distance full must be smaller than distance empty.';return;}

3701	  if(Math.abs(c.temp_off)>10||Math.abs(c.hum_off)>20||Math.abs(c.press_off)>50){

3702	    $('#setMsg').textContent='An offset that large means the sensor is faulty or wired wrong. Temperature up to 10 C, humidity up to 20 %RH, pressure up to 50 hPa.';return;}

3703	  if(c.temp_th>85||c.hum_th>100||c.gas_th>500){$('#setMsg').textContent='The BME680 reads up to 85 C, 100 %RH and an air quality index of 500, so a limit above that can never trigger.';return;}

3704	  if(c.hum_th<30){$('#setMsg').textContent='A humidity limit under 30 %RH would sound in normal indoor air. Use 30 or more.';return;}

3705	  Object.assign(b.cal,c);

3706	  let msg='Saved to '+b.name+'.';

3707	  if(b.ip){try{const r=await callRaw(b,'/api/calibrate',c);if(!r.ok)msg=b.name+' refused the values'+(r.j&&r.j.why?': '+r.j.why:'')+'.';}catch(e){msg=b.name+' did not answer, so the change is only on this screen. Settings need the bin\u2019s Wi-Fi.';}}

3708	  else msg='Saved on this screen. Add '+b.name+'’s IP address to send it to the bin.';

3709	  addAlert('success','check','Settings saved',b.name+': empty '+c.d_empty+' cm, full '+c.d_full+' cm, alert at '+c.fill_th+'%','setup');

3710	  $('#setMsg').textContent=msg;render();

3711	}

3712	/* ---------- simulation ---------- */

3713	function buildSim(){

3714	  const simSegHtml=adaptiveSeg('simSeg','sim',[state.selSim],false);

3715	  if($('#simSeg').dataset.sig!==simSegHtml){$('#simSeg').dataset.sig=simSegHtml;$('#simSeg').innerHTML=simSegHtml;}

3716	  const b=byId(state.selSim);if(!b){$('#simBody').innerHTML='';return;}

3717	  $('#simBody').innerHTML=SENSORS.map(s=>{

3718	    const manual=(b.sim.mask>>s.bit)&1;

3719	    const arg=s.key==='lid'?!!b.sim.values.lid:s.key==='motion'?!!b.sim.values.motion:false;

3720	    const cc={fill:'var(--c-fill)',weight:'var(--c-weight)',gas:'var(--c-gas)',temp:'var(--c-temp)',

3721	      tamper:'var(--c-tamper)',motion:'var(--c-motion)',lid:'var(--c-lid)'}[s.key]||'var(--ink2)';

3722	    return '<tr data-manual="'+(manual?1:0)+'" style="--cc:'+cc+'"><td><span class="simname" style="--cc:'+cc+'">'+icon(s.icon,arg)+esc(s.label)+'</span></td>'+

3723	      '<td><button class="btn sm modebtn" data-mode="'+(manual?'MANUAL':'AUTO')+'" data-bit="'+s.bit+'">'+

3724	      icon(manual?'flask':'check')+'<span>'+(manual?'MANUAL':'AUTO')+'</span></button></td>'+

3725	      '<td>'+simInput(s,b,manual)+'</td></tr>';

3726	  }).join('');

3727	}

3728	function simInput(s,b,manual){

3729	  const v=b.sim.values,dis=manual?'':'disabled';

3730	  if(s.kind==='range')return '<div class="simval"><input type="range" min="'+s.min+'" max="'+s.max+'" step="'+s.step+'" value="'+v[s.key]+'" data-sv="'+s.key+'" '+dis+'><input type="number" min="'+s.min+'" max="'+s.max+'" step="'+s.step+'" value="'+v[s.key]+'" data-sv="'+s.key+'" '+dis+'><span>'+(s.unit||'')+'</span></div>';

3731	  if(s.kind==='number')return '<div class="simval"><input type="number" min="'+s.min+'" max="'+s.max+'" step="'+s.step+'" value="'+v[s.key]+'" data-sv="'+s.key+'" '+dis+'><span>g</span></div>';

3732	  if(s.kind==='pair')return '<div class="simval"><input type="number" value="'+v.temp+'" step="0.1" data-sv="temp" '+dis+'><span>&deg;C</span><input type="number" value="'+v.humidity+'" step="1" data-sv="humidity" '+dis+'><span>%</span></div>';

3733	  if(s.kind==='rgb')return multiway('rgb',v.rgb,[[0,'OFF','var(--ink2)'],[1,'GREEN','var(--ok)'],

3734	    [2,'AMBER','var(--warn)'],[3,'RED','var(--bad)'],[4,'WHITE','var(--ink)']],manual);

3735	  if(s.kind==='buzzer')return multiway('buzzer',v.buzzer,[[0,'SILENT','var(--ink2)'],[1,'LID BEEP','var(--c-link)'],

3736	    [2,'WARNING','var(--warn)'],[3,'TAMPER','var(--bad)']],manual);

3737	  if(s.kind==='lid')return twoway('lid',v.lid,[['0','CLOSED','lock'],['1','OPEN','lid']],manual,'var(--c-lid)');

3738	  const cc=s.key==='tamper'?'var(--c-tamper)':'var(--c-motion)';

3739	  return twoway(s.key,v[s.key],[['0','OFF','cross'],['1','TRIGGERED',s.key==='tamper'?'tamper':'motion']],manual,cc);

3740	}

3741	function multiway(key,val,opts,enabled){

3742	  return '<div class="twoway multi" data-sv="'+key+'" data-val="'+(val||0)+'">'+

3743	    opts.map(o=>'<button class="tw" style="--cc:'+o[2]+'" data-v="'+o[0]+'" aria-pressed="'+

3744	      (Number(val||0)===o[0])+'"'+(enabled?'':' disabled')+'><i class="dot" style="background:'+o[2]+'"></i><span>'+o[1]+'</span></button>').join('')+'</div>';

3745	}

3746	function twoway(key,val,opts,enabled,cc){

3747	  return '<div class="twoway" data-sv="'+key+'" data-val="'+(val?1:0)+'" style="--cc:'+cc+'">'+

3748	    opts.map(o=>'<button class="tw" data-v="'+o[0]+'" aria-pressed="'+(String(val?1:0)===o[0])+'"'+

3749	      (enabled?'':' disabled')+'>'+icon(o[2],o[0]==='1')+'<span>'+o[1]+'</span></button>').join('')+'</div>';

3750	}

3751	// POST /api/simulate with the mask and values. Needs the bin's Wi-Fi.

3752	async function applySim(){

3753	  const b=byId(state.selSim);if(!b)return;

3754	  $$('#simBody [data-sv]').forEach(el=>{

3755	    b.sim.values[el.dataset.sv]=el.classList.contains('twoway')?+el.dataset.val:+el.value;

3756	  });

3757	  let msg=b.sim.mask?'Simulation applied to '+b.name+'.':b.name+' is reading real sensors again.';

3758	  if(b.ip){try{const r=await callRaw(b,'/api/simulate',{mask:b.sim.mask,values:b.sim.values});if(!r.ok)throw new Error('refused');}

3759	    catch(e){msg=b.name+' did not answer, so this only changes the screen.';}}

3760	  $('#simMsg').textContent=msg;render();

3761	}

3762

3763	/* ---------- views ---------- */

3764	function setRole(r){

3765	  if(state.role==='operator'&&r==='collector'&&opBooted)sfx('op_out');

3766	  if(r==='collector'&&state.calib){

3767	    const b=byId(state.calib.bin);state.calib=null;

3768	    if(b)command(b.id,'CALIBRATE_END');

3769	  }

3770	  state.role=r;cardSig='';

3771	  $('#rolechip').innerHTML=icon('lock',r!=='operator')+'<span>'+(r==='operator'?'OPERATOR':'COLLECTOR')+'</span>';

3772	  opRender();

3773	  const hidden=r==='collector'?['sim']:[];

3774	  $$('.tab').forEach(t=>t.classList.toggle('hide',hidden.includes(t.dataset.view)));

3775	  if(hidden.includes(state.view))setView('overview');

3776	  else{render();if(state.view==='settings')loadSettings();}

3777	}

3778	function setView(v){

3779	  state.view=v;

3780	  $$('.tab').forEach(t=>t.setAttribute('aria-selected',String(t.dataset.view===v)));

3781	  $$('section[data-view]').forEach(s=>s.classList.toggle('hide',s.dataset.view!==v));

3782	  if(v==='charts')revealCharts();

3783	  if(v==='settings')loadSettings();

3784	  if(v==='sim')buildSim();

3785	}

3786

3787	/* ---------- events ---------- */

3788	document.addEventListener('click',async e=>{

3789	  const t=e.target;

3790	  const role=t.closest('[data-role]');

3791	  if(role){

3792	    if(role.dataset.role==='operator'){askOperator(true);return;}

3793	    setRole('collector');leaveGate();return;

3794	  }

3795	  const tab=t.closest('.tab'); if(tab){setView(tab.dataset.view);return;}

3796	  const cmd=t.closest('[data-cmd]'); if(cmd){command(cmd.dataset.bin,cmd.dataset.cmd);return;}

3797	  const rg=t.closest('[data-range]');

3798	  if(rg){state.rangeMin=+rg.dataset.range;$$('#range .btn').forEach(x=>x.setAttribute('aria-pressed',String(x===rg)));revealCharts();return;}

3799	  const pick=t.closest('[data-pick]');

3800	  if(pick){

3801	    const id=pick.dataset.pick;

3802	    if(id==='ALL')state.chartPick=[];

3803	    else{const i=state.chartPick.indexOf(id);i>-1?state.chartPick.splice(i,1):state.chartPick.push(id);}

3804	    revealCharts();return;

3805	  }

3806	  const emd=t.closest('[data-envmode]');

3807	  if(emd){state.envMode=emd.dataset.envmode;revealCharts();return;}

3808	  const env=t.closest('[data-env]'); if(env){state.envBin=env.dataset.env;revealCharts();return;}

3809	  const sset=t.closest('[data-set]'); if(sset){state.selSet=sset.dataset.set;loadSettings();return;}

3810	  const cap=t.closest('[data-capture]');

3811	  if(cap&&!cap.disabled){

3812	    const b=byId(state.selSet);if(!b)return;

3813	    $('#'+cap.dataset.capture).value=fmt(b.data.distance_cm,1);

3814	    $('#setMsg').textContent='Captured '+fmt(b.data.distance_cm,1)+' cm from the sensor. Press SAVE SETTINGS to keep it.';

3815	    return;

3816	  }

3817	  const ssim=t.closest('[data-sim]'); if(ssim){state.selSim=ssim.dataset.sim;buildSim();return;}

3818	  const tw=t.closest('.tw');

3819	  if(tw&&!tw.disabled){

3820	    const box=tw.closest('.twoway');

3821	    box.dataset.val=tw.dataset.v;

3822	    box.querySelectorAll('.tw').forEach(x=>x.setAttribute('aria-pressed',String(x===tw)));

3823	    const b=byId(state.selSim);if(b)b.sim.values[box.dataset.sv]=+tw.dataset.v;

3824	    return;

3825	  }

3826	  if(t.closest('#btUse')){btUse();return;}

3827	  const snd=t.closest('[data-snd]');

3828	  if(snd){SND.armed=true;sfx(snd.dataset.snd,true);

3829	    snd.classList.add('playing');setTimeout(()=>snd.classList.remove('playing'),600);return;}

3830	  const rdt=t.closest('[data-read-toggle]');

3831	  if(rdt){toggleRead(+rdt.dataset.readToggle);return;}

3832	  if(t.closest('#pinEye')){

3833	    const c=$('#pinCard');c.dataset.masked=c.dataset.masked==='0'?'1':'0';renderPin();return;

3834	  }

3835	  if(t.closest('#popRead')){markAllRead();return;}

3836	  if(t.closest('#popOpen')){

3837	    $('#bannerPop').classList.add('hide');

3838	    $('#statusPill').setAttribute('aria-expanded','false');

3839	    setView('alerts');return;

3840	  }

3841	  const del=t.closest('[data-del]');

3842	  if(del){state.alerts=state.alerts.filter(a=>a.id!==+del.dataset.del);alertSig='';banSig='';renderAlerts();renderBanners();return;}

3843	  const pill=t.closest('#statusPill');

3844	  if(pill){const pop=$('#bannerPop'),show=pop.classList.contains('hide');

3845	    pop.classList.toggle('hide',!show);pill.setAttribute('aria-expanded',String(show));return;}

3846	  if(!t.closest('#bannerPop')&&!t.closest('#statusPill')){

3847	    const pop=$('#bannerPop');

3848	    if(pop&&!pop.classList.contains('hide')){pop.classList.add('hide');$('#statusPill').setAttribute('aria-expanded','false');}

3849	  }

3850	  const mb=t.closest('.modebtn');

3851	  if(mb){const b=byId(state.selSim);b.sim.mask^=(1<<+mb.dataset.bit);buildSim();applyMode();return;}

3852	  const ed=t.closest('[data-edit]');

3853	  if(ed){

3854	    const inOverlay=!!t.closest('#tileOverlay');

3855	    if(inOverlay){openSetOverlay(ed.dataset.edit);return;}

3856	    /* normal view: inline toggle */

3857	    const row=$('#card-'+ed.dataset.edit+' [data-r="edit"]');

3858	    if(row){const show=row.classList.contains('hide');

3859	      row.classList.toggle('hide',!show);ed.setAttribute('aria-expanded',String(show));}

3860	    return;

3861	  }

3862	  const sv=t.closest('[data-save]');

3863	  if(sv){

3864	    const inOverlay=!!t.closest('#setOverlay');

3865	    if(inOverlay){saveFromSetOverlay(sv.dataset.save);return;}

3866	    /* normal view inline save */

3867	    const b=byId(sv.dataset.save),row=$('#card-'+b.id+' [data-r="edit"]');

3868	    if(b&&row){b.name=row.querySelector('[data-k="name"]').value.trim()||b.name;

3869	      b.place=row.querySelector('[data-k="place"]').value.trim();

3870	      b.ip=row.querySelector('[data-k="ip"]').value.trim();relink(b);

3871	      persist();cardSig='';render();loadSettings();buildSim();toast(b.name+' saved.');}

3872	    return;

3873	  }

3874	  const rm=t.closest('[data-remove]');

3875	  if(rm){

3876	    const inOverlay=!!t.closest('#setOverlay');

3877	    if(inOverlay){removeFromSetOverlay(rm.dataset.remove);return;}

3878	    /* normal view inline remove */

3879	    const i=state.bins.findIndex(b=>b.id===rm.dataset.remove);

3880	    if(i>-1){const gone=state.bins.splice(i,1)[0];

3881	      if(state.selSet===gone.id)state.selSet=state.bins[0]&&state.bins[0].id;

3882	      if(state.selSim===gone.id)state.selSim=state.selSet;

3883	      if(state.calib&&state.calib.bin===gone.id)state.calib=null;

3884	      if(state.envBin===gone.id)state.envBin=state.selSet;

3885	      persist();cardSig='';render();loadSettings();buildSim();toast(gone.name+' removed.');}

3886	    return;

3887	  }

3888	  const less=t.closest('#addLess'),more=t.closest('#addMore');

3889	  if(less||more){setAddCount(state.addCount+(more?1:-1));return;}

3890	  if(t.closest('#addTile')){

3891	    let first=null;

3892	    for(let k=0;k<state.addCount;k++){

3893	      const n=state.bins.length+1;

3894	      const b=newBin({id:'b'+(Date.now()+k).toString(36),name:'Bin '+String(n).padStart(2,'0'),place:'New location',ip:'',fill:5});

3895	      seedHistory(b,n);state.bins.push(b);

3896	      if(!first)first=b;

3897	    }

3898	    persist();cardSig='';render();

3899	    if(first){

3900	      if(state.addCount===1){

3901	        if(state.ovView==='grid'&&state.bins.length>=3){openSetOverlay(first.id);}

3902	        else{const row=$('#card-'+first.id+' [data-r="edit"]');if(row)row.classList.remove('hide');}

3903	      }

3904	      else toast('Added '+state.addCount+' bins. Open each one with the gear to set its name and IP.');

3905	      $('#card-'+first.id).scrollIntoView({behavior:'smooth',block:'center'});

3906	    }

3907	    setAddCount(1);return;}

3908	});

3909	let demoStash=[];

3910	function demoBtnRender(){

3911	  const d=$('#demoBtn');

3912	  if(d){d.classList.toggle('hide',!state.demo);d.innerHTML=icon('demo')+'<span>DEMO</span>';}

3913	  const g=$('#gateDemo');

3914	  if(g){g.setAttribute('aria-pressed',state.demo?'true':'false');g.innerHTML=icon('demo')+'<span>'+(state.demo?'DEMO ON':'DEMO OFF')+'</span>';}

3915	  const n=$('#gateDemoNote');

3916	  if(n)n.textContent=state.demo

3917	    ?'Three made-up bins sit next to your real ones and run through every alert by themselves: full, pickup, tamper, lock, bad air, heat, humidity, a sensor fault, warm-up and a lost link.'

3918	    :'Turn DEMO on to add three made-up bins that show every alert by themselves. Demo can only be turned on or off here, on the start page.';

3919	}

3920	// Demo on: real bins are put aside (demoStash) and demo bins take their

3921	// place. Demo off: the real bins come back.

3922	function setDemo(on){

3923	  state.demo=on;

3924	  try{localStorage.setItem('smartbin.demo',on?'1':'0')}catch(e){}

3925	  if(on){

3926	    const extra=demoStash.length?demoStash:DEMO_SEEDS.map(mkDemo);

3927	    extra.forEach((b,i)=>{if(!b.hist.length)seedHistory(b,i)});

3928	    state.bins=state.bins.filter(b=>!b.demo).concat(extra);

3929	    demoStash=[];

3930	    TOUR.i=0;TOUR.next=0;TOUR.pauseUntil=0;

3931	  }else{

3932	    demoStash=state.bins.filter(b=>b.demo);

3933	    demoStash.forEach(tourClear);

3934	    state.bins=state.bins.filter(b=>!b.demo);

3935	    TOUR.msg='';

3936	  }

3937	  const first=state.bins[0]?state.bins[0].id:null;

3938	  if(!byId(state.selSet))state.selSet=first;

3939	  if(!byId(state.selSim))state.selSim=state.selSet;

3940	  if(!byId(state.envBin))state.envBin=state.selSet;

3941	  state.chartPick=state.chartPick.filter(id=>byId(id));

3942	  if(state.calib&&!byId(state.calib.bin))state.calib=null;

3943	  demoBtnRender();cardSig='';banSig='';render();loadSettings();buildSim();

3944	}

3945	/* The start page opens on every visit. Demo is turned on or off only here. */

3946	function showGate(){

3947	  if(state.role==='operator')setRole('collector');

3948	  if(!$('#pinWrap').classList.contains('hide'))pinClose();

3949	  demoBtnRender();

3950	  $('#gate').classList.remove('hide');

3951	  sfx('back');

3952	  playLogo('#gateLogo .sblogo');

3953	}

3954	$('#gateDemo').addEventListener('click',()=>{

3955	  SND.armed=true;

3956	  setDemo(!state.demo);

3957	  sfx(state.demo?'demo':'switch_off');

3958	});

3959	$('#demoBtn').addEventListener('click',showGate);

3960	$('#opStart').addEventListener('click',showGate);

3961	demoBtnRender();

3962

3963	/* ---------- demo tour ----------

3964	   With DEMO on and nobody pressing anything on a demo bin, the demo bins act out every

3965	   outcome in turn, with the same alerts, banners and sounds as a real bin. A key pressed

3966	   on a demo bin pauses the tour for a minute. */

3967	function tourPause(){TOUR.pauseUntil=Date.now()+60000;TOUR.msg='';banSig='';}

3968	function tourClear(b){delete b._hold;b._demoLink=null;}

3969	function tourBins(){const d=state.bins.filter(b=>b.demo);return [d[0],d[1%d.length],d[2%d.length]];}

3970	function tourHold(b,o){b._hold=Object.assign({},b._hold||{},o);}

3971	function tourDrop(b,k){if(b._hold){k.forEach(x=>delete b._hold[x]);}}

3972	function tourCmd(b,c){TOUR.busy=true;try{command(b.id,c)}finally{TOUR.busy=false}}

3973	// Demo tour: a script of situations (full, tamper, lost link ...) that plays

3974	// out on the demo bins while nobody is touching the page.

3975	const TOUR_STEPS=[

3976	  ([a])=>{a.data.fill=Math.min(99,a.cal.fill_th+4);return a.name+' fills past its '+a.cal.fill_th+'% limit.';},

3977	  ([a])=>{if(a.data.status==='LOCKED')tourCmd(a,'UNLOCK');tourCmd(a,'COLLECT');return 'A collector starts the pickup at '+a.name+'. The lid opens.';},

3978	  ([a])=>{tourCmd(a,'COLLECT_DONE');return a.name+' is emptied and back to normal.';},

3979	  ([,b])=>{tourHold(b,{tilt:40});return b.name+' is tipped over. Tamper alert.';},

3980	  ([,b])=>{tourDrop(b,['tilt']);b.data.tilt=1;tourCmd(b,'CLEAR_TAMPER');return 'The tamper alert on '+b.name+' is reset.';},

3981	  ([,,c])=>{tourCmd(c,'LOCK');return c.name+' is locked from the dashboard.';},

3982	  ([,,c])=>{tourCmd(c,'UNLOCK');return c.name+' is unlocked again.';},

3983	  ([,,c])=>{tourHold(c,{gas:c.cal.gas_th+45});return 'The air in '+c.name+' goes bad.';},

3984	  ([,b,c])=>{tourDrop(c,['gas']);c.data.gas=60;tourHold(b,{temp:+c.cal.temp_th+3.5});return b.name+' gets too hot.';},

3985	  ([,b])=>{tourDrop(b,['temp']);b.data.temp=b.data.temp_raw=29;tourHold(b,{humidity:+b.cal.hum_th+5});return b.name+' gets too humid.';},

3986	  ([a,b])=>{tourDrop(b,['humidity']);b.data.humidity=62;tourHold(a,{faults:['HX711']});return 'The weight sensor of '+a.name+' stops answering.';},

3987	  ([a])=>{tourDrop(a,['faults']);return 'The weight sensor of '+a.name+' answers again.';},

3988	  ([,,c])=>{tourCmd(c,'RESET_GAS');return 'The air sensor of '+c.name+' warms up. The reading is held until it is settled.';},

3989	  ([,b])=>{b._demoLink='fb';sfx('cloud');return b.name+' loses Wi-Fi and is read through Firebase.';},

3990	  ([,b])=>{b._demoLink='ts';sfx('cloud');return 'Firebase is down too, so '+b.name+' is read through ThingSpeak.';},

3991	  ([,b])=>{b._demoLink='down';sfx('bin_lost');return b.name+' stops answering on every link.';},

3992	  ([,b])=>{b._demoLink=null;sfx('link_up');return b.name+' is back on Wi-Fi.';},

3993	  ([,b])=>{tourCmd(b,'CALIBRATE_START');return b.name+' is being calibrated.';},

3994	  ([,b])=>{tourCmd(b,'CALIBRATE_END');return 'Calibration of '+b.name+' is done. The tour starts again.';}

3995	];

3996	function tourTick(now){

3997	  if(!state.demo||!inMain()||document.hidden||!state.bins.some(b=>b.demo)){TOUR.next=Math.max(TOUR.next,now+6000);return;}

3998	  if(now<TOUR.pauseUntil||now<TOUR.next)return;

3999	  if(state.calib&&byId(state.calib.bin)&&byId(state.calib.bin).demo){TOUR.next=now+10000;return;}

4000	  TOUR.next=now+10000;

4001	  const step=TOUR_STEPS[TOUR.i%TOUR_STEPS.length];

4002	  TOUR.i=(TOUR.i+1)%TOUR_STEPS.length;

4003	  try{TOUR.msg=step(tourBins())||'';}catch(e){TOUR.msg='';}

4004	  banSig='';

4005	}

4006	$('#rolechip').addEventListener('click',()=>{

4007	  if(state.role==='operator'){setRole('collector');toast('Back to collector. Operator needs the PIN again.');}

4008	  else askOperator(false);

4009	  if(typeof btRender==='function')btRender();

4010	});

4011	$('#theme').addEventListener('click',()=>{

4012	  const d=document.documentElement.getAttribute('data-theme')==='dark';

4013	  document.documentElement.setAttribute('data-theme',d?'light':'dark');

4014	  sfx(d?'theme_light':'theme_dark');

4015	  updateThemeIcon();

4016	  if(state.view==='charts')drawCharts();

4017	});

4018	$('#btnSave').addEventListener('click',saveSettings);

4019	$('#btnReload').addEventListener('click',async()=>{

4020	  const b=byId(state.selSet);if(!b)return;

4021	  if(!b.ip){$('#setMsg').textContent='Add an IP address for '+b.name+' first.';return;}

4022	  try{const j=await call(b,'/api/status');if(j.cal)Object.assign(b.cal,j.cal);loadSettings();

4023	    $('#setMsg').textContent='Loaded the values stored in '+b.name+'.';}

4024	  catch(e){$('#setMsg').textContent=b.name+' did not answer.';}

4025	});

4026	$('#btnExport').addEventListener('click',()=>{

4027	  const cfg=JSON.stringify(state.bins.map(b=>({id:b.id,name:b.name,place:b.place,ip:b.ip,cloud:b.cloud,ts_channel:b.tsCh,talkback_id:b.tbId})),null,2);

4028	  const url=URL.createObjectURL(new Blob([cfg],{type:'application/json'}));

4029	  const a=document.createElement('a');a.href=url;a.download='smartbin-bins.json';a.click();

4030	  setTimeout(()=>URL.revokeObjectURL(url),2000);

4031	});

4032	$('#btPair').addEventListener('click',btPair);

4033	$('#btScan').addEventListener('click',btScan);

4034	$('#btNetList').addEventListener('click',e=>{

4035	  const b=e.target.closest('[data-ssid]');if(!b||b.disabled)return;

4036	  const i=$('#bt_ssid');i.value=b.dataset.ssid;i.dispatchEvent(new Event('input',{bubbles:true}));

4037	  $$('#btNetList [data-ssid]').forEach(x=>x.setAttribute('aria-pressed',x===b?'true':'false'));

4038	  sfx('pin_key');$('#bt_pass').focus();

4039	  $('#btMsg').textContent=b.dataset.ssid+' chosen. Type its password, then press SEND WI-FI.';

4040	});

4041	$('#btSend').addEventListener('click',btSend);

4042	$('#btOff').addEventListener('click',btOff);

4043	$('#btEye').addEventListener('click',()=>{

4044	  const i=$('#bt_pass'),show=i.type==='password';

4045	  i.type=show?'text':'password';

4046	  $('#btEye').innerHTML=icon(show?'eyeoff':'eye')+'<span>'+(show?'HIDE':'SHOW')+'</span>';

4047	});

4048	$('#btnMarkAll').addEventListener('click',markShown);

4049	$('#btnClear').addEventListener('click',askDelete);

4050	$('#alertYes').addEventListener('click',doDelete);

4051	$('#alertNo').addEventListener('click',()=>$('#alertConfirm').classList.add('hide'));

4052	$('#setFields').addEventListener('input',refreshDirty);

4053	$('#btnCancel').addEventListener('click',()=>{

4054	  const b=byId(state.selSet);if(!b)return;

4055	  loadSettings();

4056	  $('#setMsg').textContent='Changes discarded. The fields show what is stored on '+b.name+' again.';

4057	});

4058	$('#btnTare').addEventListener('click',()=>{

4059	  const b=byId(state.selSet);if(!b)return;

4060	  command(b.id,'TARE');

4061	  $('#setMsg').textContent='Scale zeroed on '+b.name+'. The reading should sit at 0.00 kg.';

4062	});

4063	$('#btnCalW').addEventListener('click',()=>{

4064	  const b=byId(state.selSet);if(!b)return;

4065	  const g=+$('#s_known').value;

4066	  if(!(g>0)){$('#setMsg').textContent='Type the weight you put in the bin, in grams.';return;}

4067	  command(b.id,'CAL_WEIGHT',{grams:g});

4068	  $('#setMsg').textContent='Load cell calibrated with '+g+' g. Take the weight off and check it returns to 0.00 kg.';

4069	});

4070	$('#btnRefT').addEventListener('click',()=>{

4071	  const b=byId(state.selSet);if(!b)return;

4072	  const ref=parseFloat($('#s_tref').value);

4073	  if(!Number.isFinite(ref)){$('#setMsg').textContent='Type the reading of your reference thermometer first.';return;}

4074	  const raw=Number.isFinite(b.data.temp_raw)?b.data.temp_raw:b.data.temp-(b.cal.temp_off||0);

4075	  const off=+(ref-raw).toFixed(1),o=$('#s_toff');

4076	  o.value=off;o.dispatchEvent(new Event('input',{bubbles:true}));

4077	  $('#setMsg').textContent='Offset set to '+off+' C (reference '+ref+' minus raw '+fmt(raw,1)+'). Press SAVE SETTINGS to keep it.';

4078	});

4079	$('#btnTouchCal').addEventListener('click',()=>{

4080	  const b=byId(state.selSet);if(!b)return;

4081	  if(!isLive(b)){$('#setMsg').textContent=b.name+' is a demo bin, so there is no touchscreen to calibrate.';return;}

4082	  command(b.id,'TOUCH_CAL');

4083	  $('#setMsg').textContent=b.name+' is showing the touch calibration screen. Press each of the 4 crosses firmly, then let go. It saves by itself.';

4084	});

4085	$('#btnTouchReset').addEventListener('click',()=>{

4086	  const b=byId(state.selSet);if(!b)return;

4087	  if(!isLive(b)){$('#setMsg').textContent=b.name+' is a demo bin, so there is no touchscreen to reset.';return;}

4088	  command(b.id,'TOUCH_CAL_RESET');

4089	  $('#setMsg').textContent='Touch calibration on '+b.name+' is back to the firmware defaults.';

4090	});

4091	$('#btnGasReset').addEventListener('click',()=>{

4092	  const b=byId(state.selSet);if(!b)return;

4093	  command(b.id,'RESET_GAS');

4094	  $('#setMsg').textContent='Gas baseline cleared on '+b.name+'. The air quality index needs time in normal air before it is fully accurate again.';

4095	});

4096	$('#calToggle').addEventListener('change',e=>{

4097	  const b=byId(state.selSet);

4098	  if(!b){e.target.checked=false;return;}

4099	  if(e.target.checked){

4100	    state.calib={bin:b.id};

4101	    command(b.id,'CALIBRATE_START');

4102	    $('#setMsg').textContent=b.name+' is in calibration mode. Its screen and the dashboard are teal until you switch this off.';

4103	  }else{

4104	    state.calib=null;

4105	    command(b.id,'CALIBRATE_END');

4106	    $('#setMsg').textContent='Calibration mode off. '+b.name+' is back to normal and the fields show what is stored on the bin.';

4107	  }

4108	  loadSettings();render();

4109	});

4110	$('#btnSim').addEventListener('click',applySim);

4111	$('#btnSimOff').addEventListener('click',()=>{const b=byId(state.selSim);if(b)b.sim.mask=0;buildSim();applySim();});

4112	$('#simBody').addEventListener('input',e=>{

4113	  const el=e.target.closest('[data-sv]');if(!el||el.classList.contains('twoway'))return;

4114	  $$('#simBody [data-sv="'+el.dataset.sv+'"]').forEach(x=>{if(x!==el&&!x.classList.contains('twoway'))x.value=el.value});

4115	  const b=byId(state.selSim);if(b)b.sim.values[el.dataset.sv]=+el.value;

4116	});

4117	document.addEventListener('pointerdown',e=>{

4118	  const host=e.target.closest('.btn,.tab,.iconbtn,.role,.addmini');

4119	  if(!host||host.disabled||reduce())return;

4120	  const r=host.getBoundingClientRect(),size=Math.max(r.width,r.height)*1.7;

4121	  const sp=document.createElement('span');

4122	  sp.className='ripple';

4123	  sp.style.width=sp.style.height=size+'px';

4124	  sp.style.left=(e.clientX-r.left-size/2)+'px';

4125	  sp.style.top=(e.clientY-r.top-size/2)+'px';

4126	  host.appendChild(sp);

4127	  setTimeout(()=>sp.remove(),640);

4128	});

4129	window.addEventListener('resize',()=>{if(state.view==='charts')drawCharts()});

4130	/* Number keys open the tabs in their order, M turns sounds on or off. Not while typing. */

4131	document.addEventListener('keydown',e=>{

4132	  if(e.ctrlKey||e.metaKey||e.altKey||!inMain())return;

4133	  const t=e.target;if(t&&(t.matches('input,textarea,select')||t.isContentEditable))return;

4134	  if(document.querySelector('.modal:not(.hide),.pinwrap:not(.hide)'))return;

4135	  if(/^[1-9]$/.test(e.key)){const tabs=$$('.tab').filter(x=>x.offsetParent!==null),tb=tabs[+e.key-1];

4136	    if(tb){e.preventDefault();tb.click();}return;}

4137	  if(e.key==='m'||e.key==='M'){e.preventDefault();$('#sound').click();}

4138	});

4139	document.addEventListener('keydown',e=>{const h=e.target.closest&&e.target.closest('.shead');if(h&&(e.key==='Enter'||e.key===' ')){e.preventDefault();h.click();}});

4140

4141	buildTabs();setAddCount(1);decorateStatics();paintHeadings();btDecorate();cloudDecorate();seedAlerts();setRole('collector');setView('overview');render();liveStart();

4142

4143	/* overview filter chips */

4144	document.addEventListener('click',e=>{

4145	  const sh=e.target.closest('.shead');

4146	  if(sh){const d=sh.closest('.sect'),o=d.classList.toggle('open');sh.setAttribute('aria-expanded',o);sfx(o?'open':'back');sectAllRender();return;}

4147	  const da=e.target.closest('#demoBody [data-dspeed],#demoBody [data-dpick],#demoBody [data-devent],#demoBody [data-dlink],#demoBody [data-dreset]');

4148	  if(da){demoAct(da);return;}

4149	  if(e.target.closest('#sectAll')){const open=!allSectsOpen();

4150	    $$('.sect').forEach(d=>{d.classList.toggle('open',open);const h=d.querySelector('.shead');if(h)h.setAttribute('aria-expanded',open);});

4151	    sfx(open?'open':'back');sectAllRender();return;}

4152	  const fc=e.target.closest('#filterChips .fchip');

4153	  if(fc){state.ovFilter=fc.dataset.filter;applyOvFilter();return;}

4154	  /* view toggle */

4155	  if(e.target.closest('#viewToggle')){state.ovView=state.ovView==='grid'?'list':'grid';applyOvView();return;}

4156	  /* tile click */

4157	  const tl=e.target.closest('.tile[data-tid]');

4158	  if(tl&&state.ovView==='grid'){showTileOverlay(tl.dataset.tid);return;}

4159	  /* collection priority row click */

4160	  const cr=e.target.closest('.crow[data-pb]');

4161	  if(cr){setView('overview');const card=$('#card-'+cr.dataset.pb);if(card)card.scrollIntoView({behavior:'smooth',block:'center'});return;}

4162	  /* collect toggle */

4163	  if(e.target.id==='collectToggle'){state.collectShowAll=!state.collectShowAll;collectSig='';renderCollectNext();return;}

4164	  /* alert filter chips */

4165	  const ak=e.target.closest('#alertKinds [data-akind]');

4166	  if(ak){state.akind=ak.dataset.akind;$('#alertConfirm').classList.add('hide');alertSig='';renderAlerts();return;}

4167	  const ar=e.target.closest('#alertRange [data-arange]');

4168	  if(ar){state.arange=ar.dataset.arange;$('#alertConfirm').classList.add('hide');alertSig='';renderAlerts();return;}

4169	  /* adaptive picker toggle — only match buttons inside a .pickwrap */

4170	  const pb=e.target.closest('.pickwrap > [id$="Btn"]');

4171	  if(pb){const listId=pb.id.replace('Btn','List');const lst=document.getElementById(listId);if(lst){lst.classList.toggle('hide');return;}}

4172	  /* close any open picker when clicking elsewhere */

4173	  if(!e.target.closest('.pickwrap')){$$('.picklist').forEach(l=>l.classList.add('hide'));}

4174	});

4175	/* picker search */

4176	document.addEventListener('input',e=>{

4177	  if(e.target.id&&e.target.id.endsWith('Search')){

4178	    const q=e.target.value.toLowerCase();

4179	    const list=e.target.closest('.picklist');if(!list)return;

4180	    list.querySelectorAll('button').forEach(b=>{b.style.display=q&&!b.textContent.toLowerCase().includes(q)?'none':'';});

4181	  }

4182	});

4183	/* scroll-edge detection for fade masks */

4184	function tagScrollEdge(el){

4185	  if(!el)return;

4186	  el.classList.toggle('at-top',el.scrollTop<=2);

4187	  el.classList.toggle('at-bot',el.scrollTop+el.clientHeight>=el.scrollHeight-2);

4188	}

4189	document.addEventListener('scroll',e=>{

4190	  if(e.target.classList&&(e.target.classList.contains('alertpane')||e.target.classList.contains('scrollpane')||e.target.id==='binsPane'))tagScrollEdge(e.target);

4191	},true);

4192	/* initial tag for static panes */

4193	new MutationObserver(()=>{tagScrollEdge($('.alertpane'));tagScrollEdge($('#binsPane'));}).observe(document.body,{childList:true,subtree:true});

4194	setTimeout(()=>{tagScrollEdge($('.alertpane'));tagScrollEdge($('#binsPane'));},200);

4195

4196	// The main loop starts here.

4197	setInterval(poll,1000);

4198

4199	/* ---------- v12: sounds ----------

4200	   The same tunes as the phone app (Android App/tools/make_sounds.py), played

4201	   live with Web Audio. Rising pitch means forward, on or OK; falling means

4202	   back, off or no. A laptop has no vibration motor, so the dashboard has

4203	   sound only. Browsers keep audio silent until the first click or key. */

4204	const SOUNDS={

4205	  tap:['ui',[[1320,22]]],choice:['ui',[[1180,16],[1480,18]]],

4206	  switch_on:['ui',[[880,40],[1320,55]]],switch_off:['ui',[[1320,40],[880,55]]],

4207	  step_up:['ui',[[1400,24]]],step_down:['ui',[[1100,24]]],tab:['ui',[[990,26],[0,10],[1245,30]]],

4208	  open:['ui',[[740,35],[990,50]]],back:['ui',[[990,35],[740,50]]],

4209	  confirm:['ui',[[880,50],[1175,50],[1760,90]]],danger:['ui',[[440,90],[0,30],[440,90]]],

4210	  denied:['ui',[[392,70],[0,40],[330,120]]],

4211	  theme_light:['ui',[[1175,45],[1568,75]]],theme_dark:['ui',[[784,45],[587,75]]],

4212	  saved:['event',[[1047,60],[1319,100]]],

4213	  lock:['event',[[660,40],[0,20],[440,120]]],unlock:['event',[[440,40],[0,20],[660,120]]],

4214	  pickup:['event',[[523,70],[659,70],[784,70],[659,70],[784,150]]],

4215	  done:['event',[[784,80],[1047,80],[1319,80],[1568,230]]],

4216	  calib:['event',[[988,40],[0,50],[988,40],[0,50],[988,40],[0,50],[1319,130]]],

4217	  sim:['event',[[523,60],[622,60],[740,60],[880,140]]],

4218	  link_up:['event',[[659,70],[880,70],[1319,150]]],

4219	  link_lost:['event',[[880,90],[659,90],[440,190]]],

4220	  cloud:['event',[[880,60],[0,30],[1047,60],[0,30],[784,130]]],

4221	  logo:['ui',[[784,70],[988,70],[1175,70],[1568,170]]],

4222	  boot:['event',[[523,80],[659,80],[784,80],[1047,220]]],

4223	  demo:['event',[[659,60],[784,60],[988,60],[784,60],[1319,160]]],

4224	  cmd_ok:['event',[[1319,50],[0,25],[1760,110]]],

4225	  cmd_fail:['event',[[659,80],[0,30],[523,80],[0,30],[392,200]]],

4226	  bt_paired:['event',[[1175,40],[1568,40],[1175,40],[1976,140]]],

4227	  bt_wrong:['event',[[466,50],[0,25],[466,50],[370,140]]],

4228	  bt_blocked:['event',[[622,180],[0,50],[311,320]]],

4229	  wifi_joined:['event',[[880,60],[1175,60],[1480,60],[1760,180]]],

4230	  wifi_failed:['event',[[740,60],[0,25],[554,60],[0,25],[440,90],[370,180]]],

4231	  bt_off:['event',[[1568,35],[1175,35],[784,35],[523,150]]],

4232	  pin_set:['event',[[1047,50],[1397,50],[1047,50],[1397,120]]],

4233	  bin_lost:['event',[[740,100],[0,60],[554,210]]],bin_back:['event',[[554,80],[740,150]]],

4234	  pin_key:['ui',[[1660,14]]],

4235	  pin_wrong:['event',[[349,60],[0,30],[349,60],[0,30],[262,160]]],

4236	  pin_lockout:['event',[[523,260,260],[0,60],[196,300]]],

4237	  op_in:['event',[[587,50],[0,25],[880,50],[1175,60],[1760,160]]],

4238	  op_out:['event',[[1175,50],[880,50],[0,25],[587,140]]],

4239	  alert_full:['alert',[[880,180],[0,80],[880,180],[0,80],[880,180],[0,200],[1175,420]]],

4240	  alert_tamper:['alert',[[700,1400,340],[1400,700,340],[700,1400,340],[1400,700,340]]],

4241	  alert_sensor:['alert',[[1047,120],[784,120],[1047,120],[784,260]]],

4242	  alert_lock:['alert',[[660,70],[0,30],[440,220]]],

4243	  alert_pickup:['alert',[[523,90],[659,90],[784,90],[1047,260]]],

4244	  alert_info:['alert',[[988,90],[1319,170]]]

4245	};

4246	/* The sound test list, the same groups and names as the phone app's Sound test. */

4247	const SND_GROUPS=[

4248	  ['Alerts',[['alert_full','Bin full','fill',null,'var(--bad)'],['alert_tamper','Tamper','tamper',null,'var(--c-tamper)'],['alert_sensor','Sensor limit','temp',null,'var(--c-temp)'],['alert_lock','Bin locked','lock',true,'var(--sim)'],['alert_pickup','Pickup alert','truck',null,'var(--cal)'],['alert_info','Info','bell',null,'var(--c-bt)']]],

4249	  ['Bins and links',[['link_up','Wi-Fi link','wifi',true,'var(--c-link)'],['cloud','Cloud link','cloud',null,'var(--c-cloud)'],['link_lost','Link lost','wifi',false,'var(--bad)'],['bin_lost','Bin quiet','radar',null,'var(--warn)'],['bin_back','Bin back','bins',null,'var(--ok)'],['cmd_ok','Command OK','check',null,'var(--ok)'],['cmd_fail','Command failed','cross',null,'var(--bad)']]],

4250	  ['Lid and pickup',[['lock','Lock','lock',true,'var(--sim)'],['unlock','Unlock','lock',false,'var(--c-lid)'],['pickup','Pickup start','truck',null,'var(--cal)'],['done','Pickup finished','check',null,'var(--c-fill)']]],

4251	  ['Setup',[['calib','Calibrate','calib',null,'var(--cal)'],['sim','Simulation','flask',null,'var(--sim)'],['saved','Saved','save',null,'var(--ok)'],['denied','Not allowed','cross',null,'var(--warn)']]],

4252	  ['Bluetooth Wi-Fi setup',[['bt_paired','Paired','bluetooth',null,'var(--c-bt)'],['bt_wrong','Wrong code','key',null,'var(--warn)'],['bt_blocked','Pairing stopped','lock',true,'var(--bad)'],['wifi_joined','Wi-Fi joined','wifi',true,'var(--ok)'],['wifi_failed','Wi-Fi failed','wifi',false,'var(--c-tamper)'],['bt_off','Bluetooth off','power',null,'var(--c-motion)']]],

4253	  ['Operator PIN',[['op_in','Operator in','sliders',null,'var(--accent)'],['op_out','Operator out','undo',null,'var(--c-weight)'],['pin_key','PIN key','pin',null,'var(--c-humid)'],['pin_wrong','Wrong PIN','key',null,'var(--bad)'],['pin_lockout','PIN lockout','stopwatch',null,'var(--c-tamper)'],['pin_set','PIN saved','save',null,'var(--c-gas)']]],

4254	  ['App',[['boot','Start','power',null,'var(--ok)'],['logo','Logo','bins',null,'var(--c-gas)'],['demo','Demo on','flask',null,'var(--warn)'],['theme_light','Light theme','sunmoon',false,'var(--c-lid)'],['theme_dark','Dark theme','sunmoon',true,'var(--c-humid)']]],

4255	  ['Keys',[['tap','Tap','capture',null,'var(--c-humid)'],['choice','Choice','sliders',null,'var(--c-gas)'],['tab','Tab','layers',null,'var(--c-motion)'],['open','Open','eye',null,'var(--c-fill)'],['back','Back','undo',null,'var(--c-weight)'],['confirm','Confirm','check',null,'var(--ok)'],['danger','Danger','trash',null,'var(--bad)'],['switch_on','Switch on','power',null,'var(--c-link)'],['switch_off','Switch off','power',null,'var(--ink2)'],['step_up','Step up','plus',null,'var(--c-temp)'],['step_down','Step down','percent',null,'var(--c-press)']]]

4256	];

4257	const SND_LEVEL={ui:.5,event:.72,alert:.85},SND_TIMBRE={ui:[.18,0],event:[.22,.05],alert:[.35,.22]};

4258	const CMD_SFX={LOCK:'lock',UNLOCK:'unlock',COLLECT:'pickup',COLLECT_DONE:'done',CALIBRATE_START:'calib',CALIBRATE_END:'switch_off',CLEAR_TAMPER:'confirm',TARE:'confirm',CAL_WEIGHT:'confirm',RESET_GAS:'confirm',TOUCH_CAL:'calib',TOUCH_CAL_RESET:'confirm'};

4259	var SND={on:true,ctx:null,armed:false,at:0};

4260	try{SND.on=localStorage.getItem('smartbin.sound')!=='0'}catch(e){}

4261	function sfx(name,force){

4262	  const s=SOUNDS[name];

4263	  if(!(SND.on||force)||!SND.armed||!s)return;

4264	  SND.at=Date.now();

4265	  try{SND.ctx=SND.ctx||new (window.AudioContext||window.webkitAudioContext)();}catch(e){return}

4266	  const ctx=SND.ctx;if(ctx.state==='suspended')ctx.resume();

4267	  const [h2,h3]=SND_TIMBRE[s[0]],lvl=SND_LEVEL[s[0]]*0.32,norm=1+h2+h3;

4268	  let t=ctx.currentTime+0.01;

4269	  s[1].forEach(n=>{

4270	    const sweep=n.length===3,dur=(sweep?n[2]:n[1])/1000;

4271	    if(!sweep&&n[0]===0){t+=dur;return;}

4272	    const f0=n[0],f1=sweep?n[1]:n[0],g=ctx.createGain();

4273	    g.connect(ctx.destination);

4274	    g.gain.setValueAtTime(0,t);

4275	    g.gain.linearRampToValueAtTime(lvl,t+0.004);

4276	    g.gain.exponentialRampToValueAtTime(0.0006,t+dur);

4277	    [[1,1],[2,h2],[3,h3]].forEach(([m,a])=>{

4278	      if(!a)return;

4279	      const o=ctx.createOscillator(),og=ctx.createGain();

4280	      og.gain.value=a/norm;

4281	      o.frequency.setValueAtTime(f0*m,t);

4282	      if(f1!==f0)o.frequency.linearRampToValueAtTime(f1*m,t+dur);

4283	      o.connect(og);og.connect(g);o.start(t);o.stop(t+dur+0.02);

4284	    });

4285	    t+=dur;

4286	  });

4287	}

4288	const SPEAKER=on=>'<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round" stroke-linejoin="round"><path d="M4 9.5h3.5L12 5.5v13l-4.5-4H4z"/>'+

4289	  (on?'<path d="M15.5 9.2a4 4 0 0 1 0 5.6M18.2 6.6a7.6 7.6 0 0 1 0 10.8"/>':'<path d="M16 9.5l5 5M21 9.5l-5 5"/>')+'</svg>';

4290	function soundRender(){

4291	  $('#soundIco').innerHTML=SPEAKER(SND.on);

4292	  $('#sound').setAttribute('aria-pressed',String(SND.on));

4293	}

4294	$('#sound').addEventListener('click',()=>{

4295	  SND.armed=true;SND.on=!SND.on;

4296	  try{localStorage.setItem('smartbin.sound',SND.on?'1':'0')}catch(e){}

4297	  soundRender();sfx('switch_on');

4298	  toast(SND.on?'Sounds on.':'Sounds off.');

4299	});

4300	/* A short sound for every key, chosen by what the key does. Commands, theme,

4301	   sound and the PIN pad play their own. */

4302	const ID_SFX={btnSim:'sim',btnSimOff:'switch_off',btnSave:'saved',btnExport:'saved',fbSave:'saved',

4303	  btnClear:'danger',alertYes:'danger',alertNo:'back',btnCancel:'back',btnMarkAll:'confirm',popRead:'confirm',

4304	  addMore:'step_up',addLess:'step_down',statusPill:'open',viewToggle:'choice'};

4305	document.addEventListener('pointerdown',()=>{SND.armed=true},{capture:true,once:true});

4306	document.addEventListener('pointerdown',()=>{if(toastEl){toastEl.remove();toastEl=null;clearTimeout(toastTimer);}},true);

4307	document.addEventListener('keydown',()=>{SND.armed=true},{capture:true,once:true});

4308	document.addEventListener('click',e=>{

4309	  SND.armed=true;

4310	  const h=e.target.closest('button,.tile,.crow,.fchip');

4311	  if(!h||h.disabled)return;

4312	  if(h.closest('[data-cmd],#pinWrap,[data-snd]')||['theme','sound','rolechip','gateDemo','brand','btPair','btSend','btOff','btScan'].includes(h.id))return;

4313	  sfx(ID_SFX[h.id]||(h.classList.contains('tab')?'tab':h.classList.contains('role')?'choice':

4314	    h.closest('[data-del],[data-remove]')?'danger':h.classList.contains('accent')?'confirm':'tap'));

4315	},true);

4316	document.addEventListener('change',e=>{

4317	  if(e.target.matches('input[type=checkbox]')&&e.target.id!=='calToggle')sfx(e.target.checked?'switch_on':'switch_off');

4318	},true);

4319

4320	/* ---------- v12: operator PIN ----------

4321	   The same rules as the phone app: collector is the default, operator needs a

4322	   4 to 8 digit PIN, only a salted hash is kept, 5 wrong tries start a 30 s

4323	   wait that doubles each time, and operator drops to collector after 2 min

4324	   with the tab hidden. SHA-256 is written out here because browsers hide

4325	   WebCrypto on plain-http pages, which is how a bin serves this dashboard. */

4326	var opBooted=false;

4327	// Plain JavaScript SHA-256, used for the stretched PIN hash (see above).

4328	function sha256(bytes){

4329	  const K=[0x428a2f98,0x71374491,0xb5c0fbcf,0xe9b5dba5,0x3956c25b,0x59f111f1,0x923f82a4,0xab1c5ed5,0xd807aa98,0x12835b01,0x243185be,0x550c7dc3,0x72be5d74,0x80deb1fe,0x9bdc06a7,0xc19bf174,

4330	    0xe49b69c1,0xefbe4786,0x0fc19dc6,0x240ca1cc,0x2de92c6f,0x4a7484aa,0x5cb0a9dc,0x76f988da,0x983e5152,0xa831c66d,0xb00327c8,0xbf597fc7,0xc6e00bf3,0xd5a79147,0x06ca6351,0x14292967,

4331	    0x27b70a85,0x2e1b2138,0x4d2c6dfc,0x53380d13,0x650a7354,0x766a0abb,0x81c2c92e,0x92722c85,0xa2bfe8a1,0xa81a664b,0xc24b8b70,0xc76c51a3,0xd192e819,0xd6990624,0xf40e3585,0x106aa070,

4332	    0x19a4c116,0x1e376c08,0x2748774c,0x34b0bcb5,0x391c0cb3,0x4ed8aa4a,0x5b9cca4f,0x682e6ff3,0x748f82ee,0x78a5636f,0x84c87814,0x8cc70208,0x90befffa,0xa4506ceb,0xbef9a3f7,0xc67178f2];

4333	  const H=[0x6a09e667,0xbb67ae85,0x3c6ef372,0xa54ff53a,0x510e527f,0x9b05688c,0x1f83d9ab,0x5be0cd19];

4334	  const len=bytes.length,padded=new Uint8Array(((len+9+63)>>6)<<6);

4335	  padded.set(bytes);padded[len]=0x80;

4336	  const dv=new DataView(padded.buffer);

4337	  dv.setUint32(padded.length-4,len*8);dv.setUint32(padded.length-8,Math.floor(len/0x20000000));

4338	  const W=new Uint32Array(64),rotr=(x,n)=>(x>>>n)|(x<<(32-n));

4339	  for(let o=0;o<padded.length;o+=64){

4340	    for(let i=0;i<16;i++)W[i]=dv.getUint32(o+i*4);

4341	    for(let i=16;i<64;i++){

4342	      const s0=rotr(W[i-15],7)^rotr(W[i-15],18)^(W[i-15]>>>3),s1=rotr(W[i-2],17)^rotr(W[i-2],19)^(W[i-2]>>>10);

4343	      W[i]=(W[i-16]+s0+W[i-7]+s1)>>>0;

4344	    }

4345	    let [a,b,c,d,e,f,g,h]=H;

4346	    for(let i=0;i<64;i++){

4347	      const t1=(h+(rotr(e,6)^rotr(e,11)^rotr(e,25))+((e&f)^(~e&g))+K[i]+W[i])>>>0;

4348	      const t2=((rotr(a,2)^rotr(a,13)^rotr(a,22))+((a&b)^(a&c)^(b&c)))>>>0;

4349	      h=g;g=f;f=e;e=(d+t1)>>>0;d=c;c=b;b=a;a=(t1+t2)>>>0;

4350	    }

4351	    H[0]=(H[0]+a)>>>0;H[1]=(H[1]+b)>>>0;H[2]=(H[2]+c)>>>0;H[3]=(H[3]+d)>>>0;

4352	    H[4]=(H[4]+e)>>>0;H[5]=(H[5]+f)>>>0;H[6]=(H[6]+g)>>>0;H[7]=(H[7]+h)>>>0;

4353	  }

4354	  const out=new Uint8Array(32),ov=new DataView(out.buffer);

4355	  H.forEach((x,i)=>ov.setUint32(i*4,x));

4356	  return out;

4357	}

4358	const hex=u=>Array.from(u,x=>x.toString(16).padStart(2,'0')).join('');

4359	var OPLOCK={

4360	  MIN:4,MAX:8,FREE:5,AWAY:120000,

4361	  get(k){try{return localStorage.getItem('smartbin.op.'+k)}catch(e){return null}},

4362	  put(k,v){try{localStorage.setItem('smartbin.op.'+k,v)}catch(e){}},

4363	  has(){return !!this.get('hash')},

4364	  waitLeft(){return Math.max(0,(+this.get('until')||0)-Date.now())},

4365	  triesLeft(){return Math.max(0,this.FREE-(+this.get('misses')||0))},

4366	  valid(p){return /^\d+$/.test(p)&&p.length>=this.MIN&&p.length<=this.MAX},

4367	  hash(pin,salt){

4368	    /* Stretched: 20 000 rounds, so guessing every PIN offline is slow. */

4369	    const enc=new TextEncoder(),pb=enc.encode(salt+':'+pin);

4370	    let h=sha256(pb);

4371	    for(let i=0;i<20000;i++){const m=new Uint8Array(h.length+pb.length);m.set(h);m.set(pb,h.length);h=sha256(m);}

4372	    return hex(h);

4373	  },

4374	  set(pin){

4375	    const salt=hex(crypto.getRandomValues(new Uint8Array(16)));

4376	    this.put('salt',salt);this.put('hash',this.hash(pin,salt));this.put('misses','0');this.put('until','0');

4377	  },

4378	  check(pin){

4379	    if(!this.has()||this.waitLeft()>0)return false;

4380	    if(this.hash(pin,this.get('salt')||'')===this.get('hash')){this.put('misses','0');return true;}

4381	    const m=(+this.get('misses')||0)+1;this.put('misses',String(m));

4382	    if(m>=this.FREE)this.put('until',String(Date.now()+30000*Math.pow(2,Math.min(m-this.FREE,6))));

4383	    return false;

4384	  }

4385	};

4386	const pinUI={mode:null,first:false,pin:'',pinNew:''};

4387	const KEY_SVG='<svg class="ico" viewBox="0 0 24 24" fill="none" stroke="currentColor" stroke-width="2" stroke-linecap="round" stroke-linejoin="round"><circle cx="8.2" cy="12" r="3.6"/><path d="M11.8 12h9v3M17.6 12v2.6"/></svg>';

4388	function leaveGate(){

4389	  $('#gate').classList.add('hide');

4390	  TOUR.next=Date.now()+5000;

4391	  setTimeout(()=>{sfx('boot');playLogo('#brandLogo .sblogo');},180);

4392	}

4393	function askOperator(first){

4394	  pinUI.first=first;pinUI.pin='';pinUI.pinNew='';

4395	  pinOpen(OPLOCK.has()?'unlock':'set',null);

4396	}

4397	function pinOpen(mode,msg){

4398	  pinUI.mode=mode;pinUI.pin='';

4399	  $('#pinWrap').classList.remove('hide');

4400	  pinMsg(msg);pinRender();

4401	}

4402	function pinClose(){$('#pinWrap').classList.add('hide');$('#pinScreen').classList.add('hide');clearInterval(btc.timer);pinUI.mode=null;pinUI.pin='';pinUI.pinNew='';}

4403	/* Bluetooth pairing code: new and random each time, 60 s to live, 3 wrong codes stop pairing for 60 s. */

4404	var btc={code:'',at:0,fails:0,until:0,timer:null,LEN:6,LIFE:60000};

4405	function btNewCode(){btc.code=String(crypto.getRandomValues(new Uint32Array(1))[0]%1000000).padStart(6,'0');btc.at=Date.now();}

4406	function btCodeStart(){

4407	  const w=btc.until-Date.now(),b=byId(state.selSet);

4408	  if(w>0){sfx('denied');$('#btMsg').textContent=(b?b.name:'The bin')+' stopped pairing after 3 wrong codes. Try again in '+Math.ceil(w/1000)+' s.';return;}

4409	  btc.fails=0;btNewCode();pinUI.pinNew='';

4410	  pinOpen('bt',null);

4411	  $('#pinScreen').classList.remove('hide');

4412	  clearInterval(btc.timer);btc.timer=setInterval(btScreen,250);btScreen();

4413	}

4414	function btScreen(){

4415	  if(pinUI.mode!=='bt'){clearInterval(btc.timer);return;}

4416	  let left=Math.ceil((btc.LIFE-(Date.now()-btc.at))/1000);

4417	  if(left<=0){btNewCode();left=60;}

4418	  const b=byId(state.selSet);

4419	  $('#pinScreen').innerHTML='<small>STAND-IN FOR '+esc((b?b.name:'the bin').toUpperCase())+'\u2019S SCREEN</small><span class="pc">PAIRING CODE</span><b>'+

4420	    btc.code.slice(0,3)+' '+btc.code.slice(3)+'</b><small class="exp'+(left<=10?' low':'')+'">Expires in '+left+' s</small>';

4421	}

4422	function btCodeCheck(){

4423	  const b=byId(state.selSet),n=b?b.name:'the bin';

4424	  if(Date.now()-btc.at>btc.LIFE){btNewCode();pinUI.pin='';sfx('bt_wrong');pinMsg('That code expired. The bin shows a new one.');btScreen();return;}

4425	  if(pinUI.pin===btc.code){

4426	    pinClose();sfx('bt_paired');

4427	    bt.step=1;btRender();btFakeNets();

4428	    $('#btMsg').textContent='Paired with '+n+' (demo). The link is encrypted now. Pick the Wi-Fi network next. A name with "fail" in it shows what a failed join looks like.';

4429	    return;

4430	  }

4431	  pinUI.pin='';btc.fails++;

4432	  if(btc.fails>=3){

4433	    pinClose();btc.until=Date.now()+60000;sfx('bt_blocked');

4434	    $('#btMsg').textContent='3 wrong codes. '+n+' stops pairing for 60 s.';

4435	    return;

4436	  }

4437	  sfx('bt_wrong');pinShake();

4438	  const left=3-btc.fails;pinMsg('Wrong code. '+left+' '+(left===1?'try':'tries')+' left.');

4439	}

4440	function pinMsg(m,ok){const el=$('#pinMsg');el.textContent=m||'';el.className='pinmsg '+(m?(ok?'ok':'bad'):'');}

4441	function pinShake(){const b=$('#pinBox');b.classList.remove('shake');void b.offsetWidth;b.classList.add('shake');}

4442	function pinRender(){

4443	  const m=pinUI.mode,again=!!pinUI.pinNew;

4444	  if(!m)return;

4445	  const T={unlock:['Operator PIN','Collector is the default. Enter the operator PIN to calibrate, simulate and edit bins.','UNLOCK'],

4446	    set:[pinUI.change?'New operator PIN':'Create the operator PIN',again?'Type it once more to be sure.':

4447	      OPLOCK.MIN+' to '+OPLOCK.MAX+' digits. Anyone without it stays a collector. Only an operator can change it later.',again?'SAVE':'NEXT'],

4448	    change:['Change operator PIN','Enter the current PIN first.','NEXT'],

4449	    bt:['Type the pairing code','Demo: this box stands in for the pairing window Windows or your phone opens. Type the 6 digits the bin shows.','PAIR']}[m];

4450	  $('#pinIco').innerHTML=m==='set'?KEY_SVG:m==='bt'?icon('bluetooth'):icon('lock',true);

4451	  $('#pinTitle').textContent=T[0];$('#pinText').textContent=T[1];

4452	  const lo=m==='bt'?btc.LEN:OPLOCK.MIN,n=Math.max(lo,pinUI.pin.length);

4453	  $('#pinDots').innerHTML=Array.from({length:n},(_,i)=>'<i class="'+(i<pinUI.pin.length?'on':'')+'"></i>').join('');

4454	  $('#pinPad').innerHTML=['1','2','3','4','5','6','7','8','9'].map(d=>'<button class="btn" data-pk="'+d+'">'+d+'</button>').join('')+

4455	    '<button class="btn" data-pk="back" aria-label="Delete"'+(pinUI.pin?'':' disabled')+'>&#9003;</button>'+

4456	    '<button class="btn" data-pk="0">0</button>'+

4457	    '<button class="btn go" data-pk="go"'+(pinUI.pin.length>=lo?'':' disabled')+'>'+T[2]+'</button>';

4458	}

4459	function pinKey(k){

4460	  const bt6=pinUI.mode==='bt',lo=bt6?btc.LEN:OPLOCK.MIN,hi=bt6?btc.LEN:OPLOCK.MAX;

4461	  if(k==='back'){if(pinUI.pin){pinUI.pin=pinUI.pin.slice(0,-1);sfx('step_down');}}

4462	  else if(k==='go'){if(pinUI.pin.length>=lo){sfx('confirm');pinGo();return;}}

4463	  else if(pinUI.pin.length<hi){pinUI.pin+=k;sfx('pin_key');}

4464	  pinRender();

4465	}

4466	function pinMiss(){

4467	  pinUI.pin='';

4468	  const left=OPLOCK.waitLeft();

4469	  sfx(left>0?'pin_lockout':'pin_wrong');pinShake();

4470	  pinMsg(left>0?'Wrong PIN. Locked for '+Math.ceil(left/1000)+' s.':'Wrong PIN. '+OPLOCK.triesLeft()+' tries before a wait.');

4471	}

4472	function pinGo(){

4473	  if(pinUI.mode==='bt'){btCodeCheck();if(pinUI.mode)pinRender();return;}

4474	  const wait=OPLOCK.waitLeft();

4475	  if((pinUI.mode==='unlock'||pinUI.mode==='change')&&wait>0){

4476	    pinUI.pin='';pinMsg('Too many wrong PINs. Try again in '+Math.ceil(wait/1000)+' s.');pinRender();return;

4477	  }

4478	  if(pinUI.mode==='unlock'){

4479	    if(OPLOCK.check(pinUI.pin)){

4480	      const first=pinUI.first;pinClose();sfx('op_in');

4481	      setRole('operator');if(first)leaveGate();else toast('Signed in as operator.');

4482	      if(typeof btRender==='function')btRender();

4483	    }else pinMiss();

4484	  }else if(pinUI.mode==='change'){

4485	    if(OPLOCK.check(pinUI.pin)){pinUI.change=true;pinOpen('set',null);}else pinMiss();

4486	  }else{

4487	    if(!OPLOCK.valid(pinUI.pin)){pinUI.pin='';pinMsg('Use '+OPLOCK.MIN+' to '+OPLOCK.MAX+' digits.');}

4488	    else if(!pinUI.pinNew){pinUI.pinNew=pinUI.pin;pinUI.pin='';pinMsg('Type the same PIN again.',true);}

4489	    else if(pinUI.pin!==pinUI.pinNew){pinUI.pin='';pinUI.pinNew='';sfx('pin_wrong');pinShake();pinMsg('The two PINs differ. Start again.');}

4490	    else{

4491	      OPLOCK.set(pinUI.pin);

4492	      const change=pinUI.change,first=pinUI.first;pinUI.change=false;pinClose();

4493	      if(change){sfx('pin_set');toast('Operator PIN changed.');}

4494	      else{sfx('op_in');setRole('operator');if(first)leaveGate();else toast('Operator PIN saved. Signed in as operator.');}

4495	      opRender();

4496	    }

4497	  }

4498	  pinRender();

4499	}

4500	function opRender(){

4501	  const op=state.role==='operator',el=$('#opState');if(!el||!OPLOCK)return;

4502	  el.textContent=op?'Operator':'Collector';

4503	  $('#opChange').classList.toggle('hide',!op||!OPLOCK.has());

4504	  $('#opLockNow').classList.toggle('hide',!op);

4505	  $('#opNote').textContent=op?'':'You are a collector. Press COLLECTOR in the header and enter the operator PIN to unlock operator tools.';

4506	}

4507	$('#pinPad').addEventListener('click',e=>{const k=e.target.closest('[data-pk]');if(k&&!k.disabled)pinKey(k.dataset.pk);});

4508	$('#pinCancel').addEventListener('click',()=>{sfx('back');pinClose();});

4509	$('#pinWrap').addEventListener('click',e=>{if(e.target.id==='pinWrap'){sfx('back');pinClose();}});

4510	document.addEventListener('keydown',e=>{

4511	  if(!pinUI.mode)return;

4512	  if(/^\d$/.test(e.key))pinKey(e.key);

4513	  else if(e.key==='Backspace')pinKey('back');

4514	  else if(e.key==='Enter')pinKey('go');

4515	  else if(e.key==='Escape'){sfx('back');pinClose();}

4516	  else return;

4517	  e.preventDefault();

4518	});

4519	$('#opChange').addEventListener('click',()=>{pinUI.pinNew='';pinUI.change=false;pinOpen('change',null);});

4520	$('#opLockNow').addEventListener('click',()=>{setRole('collector');toast('Back to collector. Operator needs the PIN again.');});

4521	let opAway=0;

4522	document.addEventListener('visibilitychange',()=>{

4523	  if(document.hidden){opAway=Date.now();return;}

4524	  if(opAway&&state.role==='operator'&&Date.now()-opAway>OPLOCK.AWAY){setRole('collector');toast('Operator locked after 2 min away. Collector mode.');}

4525	  opAway=0;

4526	});

4527	opBooted=true;soundRender();opRender();

4528

4529	/* ---------- v2 logo and sound test ---------- */

4530	let logoSeq=0;

4531	function sbLogo(){

4532	  const n=++logoSeq,body='M17.5 26h29l-2.7 27.6a3.2 3.2 0 0 1-3.2 2.9H23.4a3.2 3.2 0 0 1-3.2-2.9z';

4533	  return '<svg class="sblogo" viewBox="0 0 64 64" fill="none" stroke-linecap="round" stroke-linejoin="round" role="img" aria-label="Smart-Bin">'+

4534	    '<defs><linearGradient id="sbF'+n+'" x1="0" y1="0" x2="0" y2="1"><stop class="g0" offset="0"/><stop class="g1" offset="1"/></linearGradient>'+

4535	    '<linearGradient id="sbG'+n+'" x1="0" y1="0" x2="1" y2="0"><stop class="g0" offset="0" stop-opacity=".05"/><stop class="g0" offset=".5" stop-opacity=".16"/><stop class="g0" offset="1" stop-opacity=".05"/></linearGradient>'+

4536	    '<radialGradient id="sbR'+n+'"><stop class="g0" offset="0" stop-opacity=".45"/><stop class="g0" offset="1" stop-opacity="0"/></radialGradient>'+

4537	    '<clipPath id="sbC'+n+'"><path d="'+body+'"/></clipPath></defs>'+

4538	    '<circle class="sb-glow" cx="32" cy="12" r="12" fill="url(#sbR'+n+')"/>'+

4539	    '<g class="sb-sig"><path class="sb-ac s1" d="M27.8 12.6a6 6 0 0 1 8.4 0" stroke-width="2.6"/>'+

4540	    '<path class="sb-ac s2" d="M24.2 8.9a11 11 0 0 1 15.6 0" stroke-width="2.6" stroke-opacity=".7"/>'+

4541	    '<path class="sb-ac s3" d="M20.6 5.2a16 16 0 0 1 22.8 0" stroke-width="2.6" stroke-opacity=".4"/></g>'+

4542	    '<path d="'+body+'" fill="url(#sbG'+n+')"/>'+

4543	    '<g clip-path="url(#sbC'+n+')"><g class="sb-fill">'+

4544	    '<path class="sb-wave b" d="M0 37q5-3.6 10 0t10 0t10 0t10 0t10 0t10 0t10 0v30H0z" fill="url(#sbF'+n+')" opacity=".35"/>'+

4545	    '<path class="sb-wave" d="M-5 39q5-3.6 10 0t10 0t10 0t10 0t10 0t10 0t10 0v30H-5z" fill="url(#sbF'+n+')"/></g></g>'+

4546	    '<g class="sb-ticks"><path class="sb-ac" d="M22 30.5h3.4M22 34h2.2" stroke-width="1.8" stroke-opacity=".8"/></g>'+

4547	    '<g class="sb-chip"><rect class="sb-chp" x="27.5" y="42" width="9" height="9" rx="1.8" stroke-width="2"/>'+

4548	    '<path class="sb-chp" d="M32 42v-3.2M32 51v2.5M27.5 46.5h-4M36.5 46.5h4" stroke-width="1.8"/>'+

4549	    '<circle class="sb-nd" cx="32" cy="37.6" r="1.5"/><circle class="sb-nd" cx="22.6" cy="46.5" r="1.5"/><circle class="sb-nd" cx="41.4" cy="46.5" r="1.5"/></g>'+

4550	    '<path class="sb-ln" d="'+body+'" stroke-width="3"/>'+

4551	    '<g class="sb-lid"><path class="sb-ln" d="M13 22.5h38" stroke-width="3.4"/><path class="sb-ln" d="M27 22.5v-3.4h10v3.4" stroke-width="3"/>'+

4552	    '<circle class="sb-led" cx="45.5" cy="22.5" r="1.3"/></g></svg>';

4553	}

4554	function playLogo(sel){

4555	  const el=$(sel);if(!el)return;

4556	  el.classList.remove('play');void el.getBoundingClientRect();el.classList.add('play');

4557	}

4558	$('#gateLogo').innerHTML=sbLogo();

4559	$('#brandLogo').innerHTML=sbLogo();

4560	playLogo('#gateLogo .sblogo');

4561	$('#brand').addEventListener('click',()=>{SND.armed=true;sfx('logo');playLogo('#brandLogo .sblogo');});

4562	$('#sndHead').innerHTML=icon('speaker')+'<span>SOUND TEST</span>';

4563	$('#sndGroups').innerHTML=SND_GROUPS.map(g=>'<div class="sndgrp"><h3>'+g[0]+'</h3><div>'+

4564	  g[1].map(x=>'<button type="button" class="btn sm tint" style="--cc:'+x[4]+'" data-snd="'+x[0]+'">'+(x[3]===null?icon(x[2]):ICONS[x[2]](x[3]))+'<span>'+x[1]+'</span></button>').join('')+

4565	  '</div></div>').join('');

4566	</script>

4567	</body>

4568	</html>

Part 3. Android App (Kotlin)

Android App/app/src/main/java/smartbin/MainActivity.kt

1	// ============================================================================

2	// MainActivity.kt - the app's only activity

3	// Hosts the Compose UI (SmartBinUi), plays the opening sound, routes taps on

4	// notifications to the Overview, and starts the background WatchService when

5	// the app leaves the screen with push alerts allowed. Lifecycle problems

6	// (alerts not arriving in the background, bins read too slowly) usually start

7	// in onStart/onStop below.

8	// ============================================================================

9	package smartbin

10

11	import android.content.ComponentName

12	import android.content.Intent

13	import android.content.pm.ActivityInfo

14	import android.content.pm.PackageManager

15	import android.content.res.Configuration

16	import android.os.Bundle

17	import androidx.activity.ComponentActivity

18	import androidx.activity.compose.setContent

19	import androidx.activity.enableEdgeToEdge

20	import androidx.activity.viewModels

21	import smartbin.data.Hub

22	import smartbin.notify.Alerts

23	import smartbin.notify.Haptics

24	import smartbin.notify.Sounds

25	import smartbin.notify.WatchService

26	import smartbin.ui.AppViewModel

27	import smartbin.ui.SmartBinUi

28	import smartbin.ui.ThemeMode

29

30	class MainActivity : ComponentActivity() {

31

32	    private val vm: AppViewModel by viewModels()

33

34	    override fun onCreate(savedInstanceState: Bundle?) {

35	        super.onCreate(savedInstanceState)

36	        enableEdgeToEdge()

37	        if (android.os.Build.VERSION.SDK_INT >= 29) window.isNavigationBarContrastEnforced = false

38	        setContent { SmartBinUi(vm, ::setOrientation) }

39	        // The opening chime and its rising buzz. Not on a rotation.

40	        if (savedInstanceState == null) {

41	            Sounds.init(this)

42	            Sounds.boot()

43	            Haptics.init(this)

44	            Haptics.play(Haptics.Buzz.BOOT)

45	        }

46	        handle(intent)

47	    }

48

49	    override fun onNewIntent(intent: Intent) {

50	        super.onNewIntent(intent)

51	        handle(intent)

52	    }

53

54	    /** A tap on a notification opens the Overview. */

55	    private fun handle(intent: Intent?) {

56	        if (intent?.getStringExtra(EXTRA_OPEN) != null) vm.openFromNotification()

57	    }

58

59	    /**

60	     * Phones stay upright; the fullscreen chart is the one view drawn sideways.

61	     * Tablets may be held either way.

62	     */

63	    private fun setOrientation(landscape: Boolean) {

64	        val phone = resources.configuration.smallestScreenWidthDp < 600

65	        requestedOrientation = when {

66	            landscape -> ActivityInfo.SCREEN_ORIENTATION_SENSOR_LANDSCAPE

67	            phone -> ActivityInfo.SCREEN_ORIENTATION_PORTRAIT

68	            else -> ActivityInfo.SCREEN_ORIENTATION_UNSPECIFIED

69	        }

70	    }

71

72	    override fun onStart() {

73	        super.onStart()

74	        Hub.foreground = true

75	        vm.foregrounded()

76	        WatchService.stop(this)

77	    }

78

79	    override fun onResume() {

80	        super.onResume()

81	        // Picks up a change made in Android's own notification settings.

82	        vm.refreshSystemState()

83	    }

84

85	    override fun onStop() {

86	        Hub.foreground = false

87	        if (!isChangingConfigurations) vm.backgrounded()

88	        // With push alerts on, the bins keep being read while the app is in the

89	        // background, but only once the user is inside the main screens.

90	        if (Hub.inMain && Alerts.allowed(this) && !isChangingConfigurations) WatchService.start(this)

91	        syncIcon()

92	        super.onStop()

93	    }

94

95	    /**

96	     * Makes the home-screen logo match the theme the app was left in: the dark

97	     * logo for dark, the light logo for light. It runs as the app leaves the

98	     * screen, because some launchers redraw the icon then, and a change made

99	     * while the app is open can close it.

100	     */

101	    private fun syncIcon() {

102	        val dark = when (vm.theme) {

103	            ThemeMode.DARK -> true

104	            ThemeMode.LIGHT -> false

105	            ThemeMode.AUTO -> (resources.configuration.uiMode and Configuration.UI_MODE_NIGHT_MASK) == Configuration.UI_MODE_NIGHT_YES

106	        }

107	        fun switch(name: String, on: Boolean) {

108	            val entry = ComponentName(packageName, "smartbin.$name")

109	            val want = if (on) PackageManager.COMPONENT_ENABLED_STATE_ENABLED else PackageManager.COMPONENT_ENABLED_STATE_DISABLED

110	            if (packageManager.getComponentEnabledSetting(entry) != want) {

111	                packageManager.setComponentEnabledSetting(entry, want, PackageManager.DONT_KILL_APP)

112	            }

113	        }

114	        runCatching {

115	            // The new entry goes on before the old one goes off, so there is always one.

116	            switch(if (dark) "LauncherDark" else "LauncherLight", true)

117	            switch(if (dark) "LauncherLight" else "LauncherDark", false)

118	        }

119	    }

120

121	    companion object {

122	        const val EXTRA_OPEN = "open"

123	    }

124	}

Android App/app/src/main/java/smartbin/SmartBinApp.kt

1	// ============================================================================

2	// SmartBinApp.kt - Application class, created once per process

3	// Starts Hub (the reading loop) and turns every HubEvent into a sound,

4	// vibration or Android notification. If a bin event happens but nothing is

5	// heard or posted, check Hub.inMain and Hub.foreground first: nothing plays

6	// on the start page, and notifications are only posted while off screen.

7	// ============================================================================

8	package smartbin

9

10	import android.app.Application

11	import kotlinx.coroutines.CoroutineScope

12	import kotlinx.coroutines.Dispatchers

13	import kotlinx.coroutines.SupervisorJob

14	import kotlinx.coroutines.launch

15	import smartbin.data.Hub

16	import smartbin.data.HubEvent

17	import smartbin.data.Link

18	import smartbin.notify.Alerts

19	import smartbin.notify.Feel

20	import smartbin.notify.Haptics

21	import smartbin.notify.Sfx

22	import smartbin.notify.Sounds

23

24	/**

25	 * Starts the reading loop once per process and turns its events into sound,

26	 * vibration or a notification. On screen an alert plays the app's own sound

27	 * and pattern; off screen it becomes an Android notification, which carries

28	 * the same sound through its channel.

29	 */

30	class SmartBinApp : Application() {

31	    private val scope = CoroutineScope(SupervisorJob() + Dispatchers.Main.immediate)

32

33	    override fun onCreate() {

34	        super.onCreate()

35	        Hub.init(this)

36	        Alerts.ensureChannels(this)

37	        Sounds.init(this)

38	        Haptics.init(this)

39	        val store = Hub.store()

40	        Feel.sounds = store.bool(KEY_SOUNDS, true)

41	        Feel.haptics = store.bool(KEY_HAPTICS, true)

42	        Haptics.strength = runCatching { Haptics.Strength.valueOf(store.string(KEY_STRENGTH, "NORMAL")) }.getOrDefault(Haptics.Strength.NORMAL)

43

44	        scope.launch {

45	            Hub.events.collect { e ->

46	                // Nothing sounds or posts on the start page: only once the user is in the main screens.

47	                if (!Hub.inMain) return@collect

48	                val onScreen = Hub.foreground

49	                when (e) {

50	                    is HubEvent.NewAlert ->

51	                        if (onScreen) Feel.event(Alerts.sfxFor(e.type))

52	                        else if (!e.row.demo) Alerts.post(this@SmartBinApp, e.row, e.type, e.binId)

53	                    is HubEvent.LinkChanged -> if (onScreen) when (e.to) {

54	                        Link.LIVE -> Feel.event(Sfx.LINK_UP)

55	                        Link.CLOUD, Link.THINGSPEAK -> Feel.event(Sfx.CLOUD)

56	                        Link.OFFLINE -> Feel.event(Sfx.LINK_LOST)

57	                        Link.DEMO -> {}

58	                    }

59	                    is HubEvent.BinLost -> if (onScreen) Feel.event(Sfx.BIN_LOST)

60	                    is HubEvent.BinBack -> if (onScreen) Feel.event(Sfx.BIN_BACK)

61	                    is HubEvent.Sound -> if (onScreen) runCatching { Sfx.valueOf(e.name) }.getOrNull()?.let(Feel::event)

62	                    // The screens tell the user; nothing to post.

63	                    is HubEvent.CommandResult -> {}

64	                }

65	            }

66	        }

67	    }

68

69	    companion object {

70	        const val KEY_SOUNDS = "ui_sounds"

71	        const val KEY_HAPTICS = "ui_haptics"

72	        const val KEY_STRENGTH = "haptic_strength"

73	        const val KEY_AWAKE = "keep_awake"

74	        const val KEY_THEME = "theme"

75	        const val KEY_ROLE = "role"

76	        const val KEY_ASKED = "asked_alerts"

77	    }

78	}

Android App/app/src/main/java/smartbin/data/Models.kt

1	// ============================================================================

2	// data/Models.kt - plain data types shared by the whole app

3	// Reading mirrors the firmware's /api/status JSON; Bin is one bin with its

4	// settings and latest reading; HubState is everything the screens draw. The

5	// field names and the BinStatus order must stay in step with the firmware

6	// (state.h) and the web dashboard.

7	// ============================================================================

8	package smartbin.data

9

10	/** How the app is reaching the bins right now. Local Wi-Fi first, Firebase second, ThingSpeak third. */

11	enum class Link { DEMO, LIVE, CLOUD, THINGSPEAK, OFFLINE }

12

13	/** Where one bin's latest reading came from. */

14	enum class Source { DEMO, LIVE, CLOUD, THINGSPEAK, STALE }

15

16	/** The three ways to the bins, and which of them answer right now. */

17	data class Routes(val local: Boolean = false, val firebase: Boolean = false, val thingspeak: Boolean = false) {

18	    val any get() = local || firebase || thingspeak

19	    val count get() = listOf(local, firebase, thingspeak).count { it }

20	}

21

22	enum class BinStatus { OK, LOCKED, TAMPER, COLLECTING, CALIBRATING }

23

24	enum class Role { OPERATOR, COLLECTOR }

25

26	/** The app-wide colour mode, the same names as the dashboard. */

27	enum class Mode { NORMAL, SIM, CALIB, DUAL, ALERT }

28

29	enum class Severity { CRITICAL, WARNING, INFO, SUCCESS }

30

31	/** The kinds the alert filter bar knows. */

32	enum class AlertKind(val label: String, val one: String, val many: String) {

33	    ALL("All", "alert", "alerts"),

34	    FULL("Full bins", "full-bin alert", "full-bin alerts"),

35	    SENSOR("Sensor readings", "sensor alert", "sensor alerts"),

36	    TAMPER("Tamper", "tamper alert", "tamper alerts"),

37	    LOCK("Lock and unlock", "lock event", "lock events"),

38	    PICKUP("Pickups", "pickup", "pickups"),

39	    SETUP("Calibration and settings", "setup event", "setup events"),

40	}

41

42	/** One reading from a bin, the fields of GET /api/status. */

43	data class Reading(

44	    val fill: Double = 0.0,

45	    val distance: Double = 30.0,

46	    val weight: Double = 0.0,

47	    val temp: Double = 28.0,

48	    val tempRaw: Double = 30.0,

49	    val hum: Double = 60.0,

50	    val press: Double = 1008.4,

51	    val gas: Double = 60.0,

52	    val iaqAcc: Int = 3,

53	    val tilt: Double = 0.0,

54	    val motion: Boolean = false,

55	    val lidOpen: Boolean = false,

56	    val status: BinStatus = BinStatus.OK,

57	    /** When the phone received it. */

58	    val at: Long = 0L,

59	    /** Sensors that stopped answering, as the firmware names them: HC-SR04, HX711, BME680, BMI160. */

60	    val faults: List<String> = emptyList(),

61	    /** Seconds left in the air sensor's warm-up, sent only while [iaqAcc] is 0. */

62	    val iaqWarmS: Int? = null,

63	    /** The raw gas resistance in kOhm, shown while the air sensor warms up. */

64	    val gasK: Double? = null,

65	    /** COUNTDOWN (clears by itself) or LATCHED (needs RESET TAMPER), while the bin is in TAMPER. */

66	    val tamperPhase: String = "",

67	) {

68	    fun fault(sensor: String) = sensor in faults

69	    /** The air score is not settled yet. */

70	    val warming get() = iaqAcc == 0 && !fault("BME680")

71	}

72

73	/**

74	 * The calibration values of one bin, kept as named numbers so they map one to

75	 * one onto the JSON of POST /api/calibrate and onto the Settings steppers.

76	 */

77	typealias Cal = Map<String, Double>

78

79	// Used until a bin sends its own calibration. Same keys as the firmware's Cal.

80	val DEFAULT_CAL: Cal = mapOf(

81	    "d_empty" to 30.0, "d_full" to 5.0, "fill_th" to 80.0, "temp_th" to 40.0, "hum_th" to 85.0,

82	    "gas_th" to 200.0, "tilt_th" to 30.0, "weight_cap" to 4500.0, "auto_lock" to 0.0,

83	    "servo_closed" to 0.0, "servo_open" to 90.0, "collect_pin" to 1234.0, "pir_hold_s" to 5.0,

84	    "buzz_s" to 2.0, "temp_off" to -2.0, "hum_off" to 0.0, "press_off" to 0.0,

85	)

86

87	fun Cal.v(key: String): Double = this[key] ?: DEFAULT_CAL[key] ?: 0.0

88

89	/** Simulation: one bit per sensor that is held by hand, and the values it is held at. */

90	data class Sim(

91	    val mask: Int = 0,

92	    val values: Map<String, Double> = DEFAULT_SIM,

93	) {

94	    fun on(bit: Int) = (mask shr bit) and 1 == 1

95	}

96

97	// Starting values for the simulation sliders. Bit order of Sim.mask: 0 fill,

98	// 1 weight, 2 gas, 3 temperature and humidity, 4 tamper, 5 motion, 6 lid.

99	val DEFAULT_SIM: Map<String, Double> = mapOf(

100	    "fill" to 60.0, "weight" to 2000.0, "gas" to 120.0, "temp" to 28.0, "humidity" to 60.0,

101	    "tamper" to 0.0, "motion" to 0.0, "lid" to 0.0,

102	)

103

104	// One bin in the list. Real bins are saved by Store; demo bins are not.

105	data class Bin(

106	    /** Stable id inside the app. */

107	    val id: String,

108	    /** The bin's node in Firebase, /sb/bins/<cloudId>. */

109	    val cloudId: String,

110	    val name: String,

111	    val place: String,

112	    val ip: String = "",

113	    val cal: Cal = DEFAULT_CAL,

114	    val sim: Sim = Sim(),

115	    val reading: Reading = Reading(),

116	    val source: Source = Source.DEMO,

117	    /** Last time a real reading arrived, over Wi-Fi or the cloud. */

118	    val lastSeen: Long = 0L,

119	    /** Demo only: how fast it fills, in percent per hour. */

120	    val rate: Double = 0.5,

121	    /** ThingSpeak: the bin's channel and its read key, and the TalkBack queue it takes commands from. */

122	    val tsChannel: String = "",

123	    val tsKey: String = "",

124	    val tbId: String = "",

125	    val tbKey: String = "",

126	    /** One of the made-up bins that demo mode adds. They are never saved. */

127	    val isDemo: Boolean = false,

128	    /** From /api/status: firmware version, Wi-Fi signal in dBm, seconds since start, touch calibrated on the bin. */

129	    val fw: String = "",

130	    val rssi: Int? = null,

131	    val uptimeS: Long? = null,

132	    val touchCustom: Boolean? = null,

133	) {

134	    val onThingSpeak get() = tsChannel.isNotBlank()

135	    /** A made-up demo bin. Real bins are read over Wi-Fi, Firebase or ThingSpeak, and show "not answering" instead. */

136	    val demo get() = isDemo

137	    val stale get() = source == Source.STALE

138	    val calibrating get() = reading.status == BinStatus.CALIBRATING

139	    val simulating get() = sim.mask > 0

140	    val full get() = reading.fill >= cal.v("fill_th")

141	    val sensorOver get() = reading.temp >= cal.v("temp_th") || reading.hum >= cal.v("hum_th") ||

142	        reading.gas >= cal.v("gas_th") || reading.weight >= cal.v("weight_cap")

143	    val urgent get() = reading.status == BinStatus.TAMPER || full

144	    val needsAction get() = urgent || sensorOver

145	}

146

147	/** One point of history for the charts. */

148	data class Sample(val t: Long, val fill: Float, val temp: Float, val hum: Float, val press: Float, val gas: Float)

149

150	// One row of the Alerts page. id grows by one per alert (Hub.alertSeq).

151	data class AlertRow(

152	    val id: Long,

153	    val sev: Severity,

154	    /** Name of the icon, as in the mockup: fill, gas, tamper, humid, lock, check, truck, calib, temp. */

155	    val icon: String,

156	    val kind: AlertKind,

157	    val title: String,

158	    val meta: String,

159	    val at: Long,

160	    val read: Boolean = false,

161	    /** Raised by a demo bin: shown while demo mode is on, never saved. */

162	    val demo: Boolean = false,

163	)

164

165	/** The ThingSpeak server. The real one unless the mock server is used. */

166	const val THINGSPEAK_URL = "https://api.thingspeak.com"

167

168	data class FirebaseConfig(val url: String = "", val key: String = "", val email: String = "", val password: String = "") {

169	    val filled get() = url.isNotBlank() && key.isNotBlank() && email.isNotBlank() && password.isNotBlank()

170	}

171

172	/**

173	 * A command sent through Firebase or ThingSpeak that the bin has not confirmed

174	 * yet. Its key stays locked until the bin's next reading shows it was done,

175	 * or until [deadline] passes.

176	 */

177	data class Pending(val cmd: String, val via: Source, val at: Long, val deadline: Long)

178

179	/** Everything the screens draw, in one value. */

180	data class HubState(

181	    val bins: List<Bin> = emptyList(),

182	    val link: Link = Link.DEMO,

183	    val alerts: List<AlertRow> = emptyList(),

184	    /** Calibration mode, on one bin at a time. */

185	    val calibBin: String? = null,

186	    /** Bytes the cloud reported for this month, when it reports them. */

187	    val cloudBytes: Long? = null,

188	    val loading: Boolean = true,

189	    /** Firebase and ThingSpeak answered within the last minute. Null when they are not set up. */

190	    val fbUp: Boolean? = null,

191	    val tsUp: Boolean? = null,

192	    /** ThingSpeak messages a day for all channels, estimated from how fast their entry numbers rise. */

193	    val tsPerDay: Double? = null,

194	    /** Demo options: links to pretend are up, so the link colours can be shown with no hardware. */

195	    val demoLinks: Routes = Routes(),

196	    /** Demo options: pretend cloud commands are never confirmed, so their keys time out. */

197	    val demoNoConfirm: Boolean = false,

198	    /** Cloud commands waiting for the bin to confirm them, by bin id. */

199	    val pending: Map<String, Pending> = emptyMap(),

200	    /** Demo mode, turned on or off only on the start page. It adds the demo bins. */

201	    val demoOn: Boolean = false,

202	    /** What the demo tour is acting out right now, for its banner. */

203	    val tourMsg: String? = null,

204	) {

205	    /** Every bin runs on demo data. The demo options can still make it look like any link. */

206	    val demo get() = bins.isNotEmpty() && bins.all { it.demo }

207	    val routes: Routes

208	        get() = if (demo) demoLinks

209	        else Routes(local = bins.any { it.source == Source.LIVE }, firebase = fbUp == true, thingspeak = tsUp == true)

210	    val mode: Mode

211	        get() {

212	            val cal = calibBin != null

213	            val sim = bins.any { it.simulating }

214	            return when {

215	                cal && sim -> Mode.DUAL

216	                cal -> Mode.CALIB

217	                sim -> Mode.SIM

218	                bins.any { it.urgent } -> Mode.ALERT

219	                else -> Mode.NORMAL

220	            }

221	        }

222	    val unread get() = alerts.count { !it.read }

223	}

Android App/app/src/main/java/smartbin/data/Store.kt

1	// ============================================================================

2	// data/Store.kt - everything saved on the phone (SharedPreferences "smartbin")

3	// Clearing the app's storage in Android settings resets bins, alerts and the

4	// Firebase account. The Firebase password is AES-GCM encrypted with a key in

5	// the Android Keystore; if the Keystore key is lost, open() returns "" and the

6	// user is asked to type the password again.

7	// ============================================================================

8	package smartbin.data

9

10	import android.content.Context

11	import android.content.SharedPreferences

12	import android.security.keystore.KeyGenParameterSpec

13	import android.security.keystore.KeyProperties

14	import android.util.Base64

15	import org.json.JSONArray

16	import org.json.JSONObject

17	import smartbin.BuildConfig

18	import java.security.KeyStore

19	import javax.crypto.Cipher

20	import javax.crypto.KeyGenerator

21	import javax.crypto.SecretKey

22	import javax.crypto.spec.GCMParameterSpec

23

24	/**

25	 * What the app remembers between runs: the bin list, alert history, the

26	 * user's choices, and the Firebase account. The Firebase password is sealed

27	 * with a key in the Android Keystore, so it is never on disk in the clear.

28	 */

29	class Store(context: Context) {

30	    private val prefs: SharedPreferences = context.applicationContext.getSharedPreferences("smartbin", Context.MODE_PRIVATE)

31

32	    // ---------- small values ----------

33

34	    fun bool(key: String, def: Boolean) = prefs.getBoolean(key, def)

35	    fun putBool(key: String, v: Boolean) = prefs.edit().putBoolean(key, v).apply()

36	    fun string(key: String, def: String) = prefs.getString(key, def) ?: def

37	    fun putString(key: String, v: String) = prefs.edit().putString(key, v).apply()

38

39	    // ---------- bins ----------

40

41	    // Reads the saved bin list. Only settings and the last fill, weight and

42	    // gas are saved; every other reading comes fresh from the bins.

43	    fun bins(): List<Bin>? {

44	        val raw = prefs.getString("bins", null) ?: return null

45	        return runCatching {

46	            val a = JSONArray(raw)

47	            (0 until a.length()).map { i ->

48	                val o = a.getJSONObject(i)

49	                val cal = DEFAULT_CAL.toMutableMap()

50	                o.optJSONObject("cal")?.let { c -> c.keys().forEach { k -> cal[k] = c.optDouble(k, cal[k] ?: 0.0) } }

51	                Bin(

52	                    id = o.getString("id"),

53	                    cloudId = o.optString("cloudId", "bin%02d".format(i + 1)),

54	                    name = o.optString("name", "Bin %02d".format(i + 1)),

55	                    place = o.optString("place", ""),

56	                    ip = o.optString("ip", ""),

57	                    cal = cal,

58	                    rate = o.optDouble("rate", 0.5),

59	                    tsChannel = o.optString("tsChannel", ""),

60	                    tsKey = o.optString("tsKey", ""),

61	                    tbId = o.optString("tbId", ""),

62	                    tbKey = o.optString("tbKey", ""),

63	                    reading = Reading(fill = o.optDouble("fill", 0.0), weight = o.optDouble("weight", 0.0), gas = o.optDouble("gas", 60.0)),

64	                )

65	            }

66	        }.getOrNull()

67	    }

68

69	    fun saveBins(bins: List<Bin>) {

70	        val a = JSONArray()

71	        bins.forEach { b ->

72	            val cal = JSONObject()

73	            b.cal.forEach { (k, v) -> cal.put(k, v) }

74	            a.put(

75	                JSONObject().put("id", b.id).put("cloudId", b.cloudId).put("name", b.name).put("place", b.place)

76	                    .put("ip", b.ip).put("cal", cal).put("rate", b.rate)

77	                    .put("tsChannel", b.tsChannel).put("tsKey", b.tsKey).put("tbId", b.tbId).put("tbKey", b.tbKey)

78	                    .put("fill", b.reading.fill).put("weight", b.reading.weight).put("gas", b.reading.gas)

79	            )

80	        }

81	        prefs.edit().putString("bins", a.toString()).apply()

82	    }

83

84	    // ---------- alerts ----------

85

86	    // Alert history, newest first, at most 200 rows (trimmed by Hub.add).

87	    fun alerts(): List<AlertRow>? {

88	        val raw = prefs.getString("alerts", null) ?: return null

89	        return runCatching {

90	            val a = JSONArray(raw)

91	            (0 until a.length()).map { i ->

92	                val o = a.getJSONObject(i)

93	                AlertRow(

94	                    id = o.getLong("id"),

95	                    sev = Severity.valueOf(o.getString("sev")),

96	                    icon = o.getString("icon"),

97	                    kind = AlertKind.valueOf(o.getString("kind")),

98	                    title = o.getString("title"),

99	                    meta = o.getString("meta"),

100	                    at = o.getLong("at"),

101	                    read = o.optBoolean("read"),

102	                )

103	            }

104	        }.getOrNull()

105	    }

106

107	    fun saveAlerts(rows: List<AlertRow>) {

108	        val a = JSONArray()

109	        rows.forEach { r ->

110	            a.put(

111	                JSONObject().put("id", r.id).put("sev", r.sev.name).put("icon", r.icon).put("kind", r.kind.name)

112	                    .put("title", r.title).put("meta", r.meta).put("at", r.at).put("read", r.read)

113	            )

114	        }

115	        prefs.edit().putString("alerts", a.toString()).apply()

116	    }

117

118	    // ---------- Firebase account ----------

119

120	    /** The team's account from local.properties at build time. Empty when none was given. */

121	    fun firebaseDefaults() = FirebaseConfig(BuildConfig.FB_URL, BuildConfig.FB_KEY, BuildConfig.FB_EMAIL, BuildConfig.FB_PASSWORD)

122

123	    /** The saved account, or the built-in one when nothing was saved. */

124	    fun firebase(): FirebaseConfig = readFb("saved") ?: firebaseDefaults()

125

126	    /** The last account that passed a test. */

127	    fun firebaseWorking(): FirebaseConfig? = readFb("good") ?: firebaseDefaults().takeIf { it.filled }

128

129	    fun saveFirebase(cfg: FirebaseConfig, passedTest: Boolean) {

130	        writeFb("saved", cfg)

131	        if (passedTest) writeFb("good", cfg)

132	    }

133

134	    private fun readFb(slot: String): FirebaseConfig? {

135	        val url = prefs.getString("${slot}_url", null) ?: return null

136	        return FirebaseConfig(

137	            url = url,

138	            key = prefs.getString("${slot}_key", "").orEmpty(),

139	            email = prefs.getString("${slot}_email", "").orEmpty(),

140	            password = open(prefs.getString("${slot}_pw", "").orEmpty()),

141	        )

142	    }

143

144	    private fun writeFb(slot: String, cfg: FirebaseConfig) {

145	        prefs.edit()

146	            .putString("${slot}_url", cfg.url.trim())

147	            .putString("${slot}_key", cfg.key.trim())

148	            .putString("${slot}_email", cfg.email.trim())

149	            .putString("${slot}_pw", seal(cfg.password))

150	            .apply()

151	    }

152

153	    private fun key(): SecretKey {

154	        val store = KeyStore.getInstance("AndroidKeyStore").apply { load(null) }

155	        (store.getKey(KEY_ALIAS, null) as? SecretKey)?.let { return it }

156	        val maker = KeyGenerator.getInstance(KeyProperties.KEY_ALGORITHM_AES, "AndroidKeyStore")

157	        maker.init(

158	            KeyGenParameterSpec.Builder(KEY_ALIAS, KeyProperties.PURPOSE_ENCRYPT or KeyProperties.PURPOSE_DECRYPT)

159	                .setBlockModes(KeyProperties.BLOCK_MODE_GCM)

160	                .setEncryptionPaddings(KeyProperties.ENCRYPTION_PADDING_NONE)

161	                .build()

162	        )

163	        return maker.generateKey()

164	    }

165

166	    private fun seal(plain: String): String = runCatching {

167	        if (plain.isEmpty()) return ""

168	        val cipher = Cipher.getInstance("AES/GCM/NoPadding").apply { init(Cipher.ENCRYPT_MODE, key()) }

169	        val out = cipher.doFinal(plain.toByteArray())

170	        Base64.encodeToString(cipher.iv, Base64.NO_WRAP) + ":" + Base64.encodeToString(out, Base64.NO_WRAP)

171	    }.getOrDefault("")

172

173	    private fun open(sealed: String): String = runCatching {

174	        val parts = sealed.split(':')

175	        if (parts.size != 2) return ""

176	        val cipher = Cipher.getInstance("AES/GCM/NoPadding")

177	        cipher.init(Cipher.DECRYPT_MODE, key(), GCMParameterSpec(128, Base64.decode(parts[0], Base64.NO_WRAP)))

178	        String(cipher.doFinal(Base64.decode(parts[1], Base64.NO_WRAP)))

179	    }.getOrDefault("")

180

181	    private companion object {

182	        const val KEY_ALIAS = "smartbin_cloud_key"

183	    }

184	}

Android App/app/src/main/java/smartbin/data/LocalApi.kt

1	// ============================================================================

2	// data/LocalApi.kt - HTTP client for the bin's own API (net.cpp in the firmware)

3	// Debug tip: open http://<bin-ip>/api/status in the phone's browser. If that

4	// page loads but the app shows the bin as not answering, the IP saved in the

5	// bin's settings is wrong; if it does not load, the phone is not on the

6	// bins' Wi-Fi (or the router isolates clients).

7	// ============================================================================

8	package smartbin.data

9

10	import kotlinx.coroutines.Dispatchers

11	import kotlinx.coroutines.withContext

12	import org.json.JSONObject

13	import java.io.IOException

14	import java.net.HttpURLConnection

15	import java.net.URL

16

17	/**

18	 * The bin's own REST API over the same Wi-Fi (implementation plan, section

19	 * "Local API"). Every call has a 1.5 s timeout, the same as the dashboard, so

20	 * a bin that is off is noticed quickly and the app can try the cloud.

21	 *

22	 *   GET  /api/status     every reading, the bin status, its calibration and sim mask

23	 *   POST /api/command    {"cmd": "LOCK"} and the other commands

24	 *   POST /api/calibrate  the calibration values, stored in EEPROM

25	 *   POST /api/simulate   {"mask": 5, "values": {...}}

26	 *   POST /api/cloud      {"url","key","email","pass"}: the Firebase account the bin signs in with

27	 *

28	 * A command the bin refuses comes back as HTTP 409 {"ok":false,"why":"..."},

29	 * thrown as [Refused] so the reason reaches the user and the cloud is not tried.

30	 */

31	object LocalApi {

32	    private const val TIMEOUT_MS = 1500

33

34	    /** The parts of /api/status the app uses. */

35	    class Status(

36	        val reading: Reading, val cal: Cal?, val simMask: Int?,

37	        val fw: String = "", val rssi: Int? = null, val uptimeS: Long? = null, val touchCustom: Boolean? = null,

38	    )

39

40	    /** The bin answered and said no, with its reason. */

41	    class Refused(val why: String) : IOException(why)

42

43	    suspend fun status(ip: String): Status = parse(JSONObject(call(ip, "/api/status", null)))

44

45	    suspend fun command(ip: String, cmd: String, extra: Map<String, Any> = emptyMap()) {

46	        val o = JSONObject().put("cmd", cmd)

47	        extra.forEach { (k, v) -> o.put(k, v) }

48	        call(ip, "/api/command", o.toString())

49	    }

50

51	    suspend fun calibrate(ip: String, cal: Cal) {

52	        val o = JSONObject()

53	        cal.forEach { (k, v) -> o.put(k, if (v == Math.floor(v)) v.toLong() else v) }

54	        call(ip, "/api/calibrate", o.toString())

55	    }

56

57	    /** Gives the bin the Firebase account it signs in with. */

58	    suspend fun sendCloud(ip: String, cfg: FirebaseConfig) {

59	        val o = JSONObject().put("url", cfg.url.trim()).put("key", cfg.key.trim()).put("email", cfg.email.trim()).put("pass", cfg.password)

60	        call(ip, "/api/cloud", o.toString())

61	    }

62

63	    suspend fun simulate(ip: String, sim: Sim) {

64	        val values = JSONObject()

65	        sim.values.forEach { (k, v) -> values.put(k, v) }

66	        call(ip, "/api/simulate", JSONObject().put("mask", sim.mask).put("values", values).toString())

67	    }

68

69	    /** An address the user typed, with or without http:// and a port. */

70	    fun base(ip: String): String {

71	        val t = ip.trim().trimEnd('/')

72	        return if (t.startsWith("http://") || t.startsWith("https://")) t else "http://$t"

73	    }

74

75	    // All calls run on Dispatchers.IO. 400 and 409 become Refused with the

76	    // bin's own reason; any other non-2xx code is a plain IOException.

77	    private suspend fun call(ip: String, path: String, body: String?): String = withContext(Dispatchers.IO) {

78	        if (ip.isBlank()) throw IOException("no address")

79	        val conn = URL(base(ip) + path).openConnection() as HttpURLConnection

80	        conn.connectTimeout = TIMEOUT_MS

81	        conn.readTimeout = TIMEOUT_MS

82	        conn.useCaches = false

83	        try {

84	            if (body != null) {

85	                val bytes = body.toByteArray()

86	                conn.requestMethod = "POST"

87	                conn.doOutput = true

88	                conn.setRequestProperty("Content-Type", "application/json")

89	                conn.setFixedLengthStreamingMode(bytes.size)

90	                conn.outputStream.use { it.write(bytes) }

91	            }

92	            val code = conn.responseCode

93	            if (code == 409 || code == 400) {

94	                val why = runCatching { JSONObject(conn.errorStream.bufferedReader().use { it.readText() }).optString("why") }.getOrNull()

95	                throw Refused(why?.takeIf { it.isNotBlank() } ?: "The bin refused it")

96	            }

97	            if (code !in 200..299) throw IOException("HTTP $code")

98	            conn.inputStream.bufferedReader().use { it.readText() }.ifBlank { "{}" }

99	        } finally {

100	            conn.disconnect()

101	        }

102	    }

103

104	    /**

105	     * Reads the status JSON. Field names follow the plan; the older names the

106	     * dashboard also accepts (fill_percentage, weight_grams and so on) work too.

107	     */

108	    fun parse(j: JSONObject): Status {

109	        fun num(vararg keys: String, def: Double): Double {

110	            for (k in keys) if (j.has(k) && !j.isNull(k)) {

111	                val d = j.optDouble(k)

112	                if (!d.isNaN()) return d

113	            }

114	            return def

115	        }

116	        val temp = num("temp", "temperature", def = 28.0)

117	        val lidRaw = j.opt("lid") ?: j.opt("lid_state")

118	        val lidOpen = lidRaw == 1 || lidRaw.toString().equals("OPEN", true) || lidRaw == true

119	        val statusRaw = j.opt("status") ?: j.opt("bin_status")

120	        val names = BinStatus.entries

121	        val status = when (statusRaw) {

122	            is Number -> names.getOrElse(statusRaw.toInt()) { BinStatus.OK }

123	            null -> BinStatus.OK

124	            else -> names.firstOrNull { it.name.equals(statusRaw.toString(), true) } ?: BinStatus.OK

125	        }

126	        val reading = Reading(

127	            fill = num("fill", "fill_percentage", def = 0.0).coerceIn(0.0, 100.0),

128	            distance = num("distance_cm", "distance", def = 30.0),

129	            weight = num("weight_g", "weight_grams", "weight", def = 0.0),

130	            temp = temp,

131	            tempRaw = num("temp_raw", def = temp),

132	            hum = num("humidity", def = 60.0),

133	            press = num("pressure", "pressure_hpa", def = 1008.0),

134	            gas = num("iaq", "gas", "gas_level", def = 60.0),

135	            iaqAcc = num("iaq_acc", def = 3.0).toInt(),

136	            tilt = num("tilt", "tilt_deg", def = 0.0),

137	            motion = j.optBoolean("motion", j.optInt("pir", 0) == 1),

138	            lidOpen = lidOpen,

139	            status = status,

140	            at = System.currentTimeMillis(),

141	            faults = j.optJSONArray("faults")?.let { a -> (0 until a.length()).map { a.optString(it) }.filter { it.isNotBlank() } } ?: emptyList(),

142	            iaqWarmS = if (j.has("iaq_warm_s") && !j.isNull("iaq_warm_s")) j.optInt("iaq_warm_s") else null,

143	            gasK = if (j.has("gas_k") && !j.isNull("gas_k")) j.optDouble("gas_k").takeUnless { it.isNaN() } else null,

144	            tamperPhase = j.optString("tamper_phase", ""),

145	        )

146	        val cal = j.optJSONObject("cal")?.let { c ->

147	            val m = DEFAULT_CAL.toMutableMap()

148	            c.keys().forEach { k -> c.optDouble(k).takeUnless { it.isNaN() }?.let { m[k] = it } }

149	            m.toMap()

150	        }

151	        val mask = if (j.has("sim_mask")) j.optInt("sim_mask") else null

152	        return Status(

153	            reading, cal, mask,

154	            fw = j.optString("fw", ""),

155	            rssi = if (j.has("rssi") && !j.isNull("rssi")) j.optInt("rssi") else null,

156	            uptimeS = if (j.has("uptime_s") && !j.isNull("uptime_s")) j.optLong("uptime_s") else null,

157	            touchCustom = j.optJSONObject("touch")?.let { t -> if (t.has("custom")) t.optBoolean("custom") else null },

158	        )

159	    }

160	}

Android App/app/src/main/java/smartbin/data/Cloud.kt

1	// ============================================================================

2	// data/Cloud.kt - Firebase Realtime Database client over REST

3	// Debug tip: Settings > Connection > TEST runs test() below and reports the

4	// same plain-language errors as friendly(). "Permission denied" almost always

5	// means the account is missing from /sb_users or the rules were not published.

6	// ============================================================================

7	package smartbin.data

8

9	import kotlinx.coroutines.CancellationException

10	import kotlinx.coroutines.Dispatchers

11	import kotlinx.coroutines.sync.Mutex

12	import kotlinx.coroutines.sync.withLock

13	import kotlinx.coroutines.withContext

14	import org.json.JSONObject

15	import java.io.IOException

16	import java.net.HttpURLConnection

17	import java.net.URL

18	import java.net.URLEncoder

19

20	/**

21	 * Firebase Realtime Database over plain REST, the way Garbage-Guard does it:

22	 * no SDK, so the URL, key and account can be typed in at run time.

23	 *

24	 * Data layout (the firmware writes the same paths):

25	 *   /sb/bins/<cloudId>/status    the /api/status JSON plus "ts" (ms) and "local_ip"

26	 *   /sb/commands/<cloudId>       the app pushes {"cmd": "LOCK", "ts": ...}; the bin runs and removes it

27	 *   /sb/results/<push key>       the bin's answer to that command: {"ok", "cmd", "why", "ts"}

28	 *   /sb/watchers/<id>            written every 10 s while the app reads bins over Firebase, so they send faster

29	 *   /sb/alerts/<cloudId>         the bin's own alert list, newest 50

30	 *   /sb/usage/bytes_month        optional, the month's download total for the meter

31	 *   /sb_users/<uid>              the accounts allowed in; Test reads this

32	 *

33	 * Only LOCK, UNLOCK and COLLECT go over the cloud. Calibration and

34	 * simulation need live readings, so they stay on the bins' Wi-Fi.

35	 */

36	class Cloud(private var config: FirebaseConfig) {

37	    private val signIn = Mutex()

38	    private var token: String? = null

39	    private var tokenFor: FirebaseConfig? = null

40	    private var tokenExpires = 0L

41

42	    fun use(cfg: FirebaseConfig) {

43	        config = cfg

44	    }

45

46	    val ready get() = config.filled

47

48	    class CloudStatus(val status: LocalApi.Status, val ts: Long, val localIp: String?)

49

50	    suspend fun status(cloudId: String): CloudStatus? {

51	        val raw = rest("GET", "sb/bins/$cloudId/status") ?: return null

52	        val o = JSONObject(raw)

53	        return CloudStatus(LocalApi.parse(o), o.optLong("ts", 0L), o.optString("local_ip").takeIf { it.isNotBlank() })

54	    }

55

56	    /** Pushes a command and returns its push key, under which the bin writes its answer. */

57	    suspend fun command(cloudId: String, cmd: String): String? {

58	        require(cmd in CLOUD_COMMANDS) { "$cmd needs the bins' Wi-Fi" }

59	        val body = JSONObject().put("cmd", cmd).put("ts", System.currentTimeMillis()).put("from", "app")

60	        val raw = rest("POST", "sb/commands/$cloudId", body.toString()) ?: return null

61	        return runCatching { JSONObject(raw).optString("name") }.getOrNull()?.takeIf { it.isNotBlank() }

62	    }

63

64	    /** The bin's answer to a command, or null while it has not answered. */

65	    suspend fun result(key: String): JSONObject? = rest("GET", "sb/results/$key")?.let { runCatching { JSONObject(it) }.getOrNull() }

66

67	    suspend fun dropResult(key: String) {

68	        runCatching { rest("DELETE", "sb/results/$key") }

69	    }

70

71	    /** Tells the bins someone is watching, so they write every 5 s instead of every minute. */

72	    suspend fun watch(id: String, device: String) {

73	        val body = JSONObject()

74	            .put("ts", JSONObject().put(".sv", "timestamp"))

75	            .put("kind", "app").put("device", device).put("mode", "foreground").put("bin", "all")

76	        rest("PUT", "sb/watchers/$id", body.toString())

77	    }

78

79	    suspend fun unwatch(id: String) {

80	        runCatching { rest("DELETE", "sb/watchers/$id") }

81	    }

82

83	    /** The alerts a bin wrote to the cloud, oldest first. */

84	    suspend fun alerts(cloudId: String): List<JSONObject> {

85	        val raw = rest("GET", "sb/alerts/$cloudId") ?: return emptyList()

86	        val o = runCatching { JSONObject(raw) }.getOrNull() ?: return emptyList()

87	        return o.keys().asSequence().mapNotNull { o.optJSONObject(it) }.sortedBy { it.optLong("ts") }.toList()

88	    }

89

90	    suspend fun usageBytes(): Long? = rest("GET", "sb/usage/bytes_month")?.trim()?.toLongOrNull()

91

92	    /** True when Firebase answers with the saved account, for the link chip. */

93	    suspend fun ping(): Boolean = runCatching { rest("GET", "sb/usage/bytes_month"); true }.getOrDefault(false)

94

95	    /** Signs in with [cfg] and reads its line in /sb_users, the same check as the dashboard's TEST. */

96	    suspend fun test(cfg: FirebaseConfig): Result<Int> = try {

97	        val (idToken, uid) = signInWith(cfg)

98	        call(cfg.url, "GET", "sb_users/$uid", idToken)

99	            ?: throw IOException("The account is not on the allowed list (sb_users).")

100	        val bins = call(cfg.url, "GET", "sb/bins", idToken, query = "&shallow=true")

101	        Result.success(bins?.let { runCatching { JSONObject(it).length() }.getOrDefault(0) } ?: 0)

102	    } catch (e: CancellationException) {

103	        throw e

104	    } catch (e: Exception) {

105	        Result.failure(IOException(friendly(e.message.orEmpty())))

106	    }

107

108	    private suspend fun signInWith(cfg: FirebaseConfig): Pair<String, String> {

109	        val body = JSONObject().put("email", cfg.email.trim()).put("password", cfg.password).put("returnSecureToken", true)

110	        val o = JSONObject(http("POST", authUrl(cfg), body.toString()))

111	        tokenExpires = System.currentTimeMillis() + (o.optString("expiresIn", "3600").toLongOrNull() ?: 3600L) * 1000

112	        return o.getString("idToken") to o.optString("localId")

113	    }

114

115	    // The Mutex makes sure only one sign-in runs at a time when several bins

116	    // are read in parallel.

117	    /** A token for the saved settings, renewed when under five minutes remain or the settings changed. */

118	    private suspend fun auth(): String = signIn.withLock {

119	        val cfg = config

120	        val current = token

121	        if (current != null && tokenFor == cfg && System.currentTimeMillis() < tokenExpires - 300_000) return@withLock current

122	        val (fresh, _) = signInWith(cfg)

123	        token = fresh

124	        tokenFor = cfg

125	        fresh

126	    }

127

128	    private suspend fun rest(method: String, path: String, body: String? = null): String? {

129	        if (!ready) return null

130	        return call(config.url, method, path, auth(), body)

131	    }

132

133	    private suspend fun call(db: String, method: String, path: String, idToken: String, body: String? = null, query: String = ""): String? {

134	        val text = http(method, "${db.trim().trimEnd('/')}/$path.json?auth=${enc(idToken)}$query", body)

135	        return text.takeIf { it.isNotBlank() && it.trim() != "null" }

136	    }

137

138	    private suspend fun http(method: String, url: String, body: String?): String = withContext(Dispatchers.IO) {

139	        val conn = URL(url).openConnection() as HttpURLConnection

140	        conn.requestMethod = method

141	        conn.connectTimeout = 8000

142	        conn.readTimeout = 10000

143	        try {

144	            if (body != null) {

145	                val bytes = body.toByteArray()

146	                conn.doOutput = true

147	                conn.setRequestProperty("Content-Type", "application/json")

148	                conn.setFixedLengthStreamingMode(bytes.size)

149	                conn.outputStream.use { it.write(bytes) }

150	            }

151	            val code = conn.responseCode

152	            val stream = if (code in 200..299) conn.inputStream else conn.errorStream

153	            val text = stream?.bufferedReader()?.use { it.readText() }.orEmpty()

154	            if (code !in 200..299) throw IOException(errorOf(text, code))

155	            text

156	        } catch (e: IOException) {

157	            conn.disconnect()

158	            throw e

159	        }

160	    }

161

162	    /** Firebase reports errors as {"error": "text"} or {"error": {"message": "CODE"}}. */

163	    private fun errorOf(text: String, code: Int): String {

164	        val o = runCatching { JSONObject(text) }.getOrNull()

165	        val inner = o?.optJSONObject("error")?.optString("message").orEmpty()

166	        val flat = if (o != null && o.optJSONObject("error") == null) o.optString("error") else ""

167	        return inner.ifEmpty { flat }.ifEmpty { "HTTP $code" }

168	    }

169

170	    /** A plain-http database address can only be the mock server, so sign-in goes there too. */

171	    private fun authUrl(cfg: FirebaseConfig): String =

172	        if (cfg.url.startsWith("http://")) "${cfg.url.trim().trimEnd('/')}/__auth/signInWithPassword?key=${enc(cfg.key)}"

173	        else "https://identitytoolkit.googleapis.com/v1/accounts:signInWithPassword?key=${enc(cfg.key.trim())}"

174

175	    private fun enc(s: String) = URLEncoder.encode(s, "UTF-8")

176

177	    companion object {

178	        val CLOUD_COMMANDS = setOf("LOCK", "UNLOCK", "COLLECT")

179

180	        /** Firebase's error codes in plain words. */

181	        fun friendly(message: String): String = when {

182	            "INVALID_LOGIN_CREDENTIALS" in message || "INVALID_PASSWORD" in message || "EMAIL_NOT_FOUND" in message ||

183	                "INVALID_EMAIL" in message -> "Wrong email or password."

184	            "API key not valid" in message || "API_KEY_INVALID" in message -> "The Web API key is wrong. Copy it again from the Firebase console."

185	            "PASSWORD_LOGIN_DISABLED" in message || "OPERATION_NOT_ALLOWED" in message ->

186	                "Email/Password sign-in is off in Firebase Authentication."

187	            "TOO_MANY_ATTEMPTS_TRY_LATER" in message -> "Too many tries, wait a few minutes."

188	            "USER_DISABLED" in message -> "This account is disabled."

189	            "Permission denied" in message || "HTTP 401" in message ->

190	                "The account is not on the allowed list (sb_users), or the rules were not pasted."

191	            "HTTP 404" in message -> "The database URL is wrong."

192	            "sb_users" in message -> message

193	            "Unable to resolve host" in message || "failed to connect" in message || "timeout" in message.lowercase() ->

194	                "No internet, or the database URL is wrong."

195	            else -> "Could not sign in. ${message.take(80)}"

196	        }

197

198	        /** The same checks as the dashboard. Null means every field looks right. */

199	        fun problemWith(cfg: FirebaseConfig): String? {

200	            val url = cfg.url.trim()

201	            val local = url.startsWith("http://")

202	            return when {

203	                !url.startsWith("https://") && !local -> "The database URL is wrong. It starts with https:// and ends in .firebasedatabase.app."

204	                url.endsWith("/") -> "Remove the / at the end of the database URL."

205	                !local && !(url.endsWith(".firebasedatabase.app") || url.endsWith(".firebaseio.com")) ->

206	                    "The database URL is wrong. It starts with https:// and ends in .firebasedatabase.app."

207	                !local && (!cfg.key.trim().startsWith("AIza") || cfg.key.trim().length != 39) ->

208	                    "The Web API key is wrong. Copy it again from the Firebase console."

209	                !Regex("""^[^@\s]+@[^@\s]+\.[^@\s]+$""").matches(cfg.email.trim()) || cfg.password.length < 6 ->

210	                    "Wrong email or password."

211	                else -> null

212	            }

213	        }

214	    }

215	}

Android App/app/src/main/java/smartbin/data/ThingSpeak.kt

1	// ============================================================================

2	// data/ThingSpeak.kt - ThingSpeak channel reader and TalkBack sender

3	// Debug tip: open https://api.thingspeak.com/channels/<id>/feeds/last.json?api_key=<read key>

4	// in a browser. "-1" means the channel ID or key is wrong; an old created_at

5	// means the bin has stopped writing (see the [thingspeak] lines on its Serial log).

6	// ============================================================================

7	package smartbin.data

8

9	import kotlinx.coroutines.Dispatchers

10	import kotlinx.coroutines.withContext

11	import org.json.JSONObject

12	import java.io.IOException

13	import java.net.HttpURLConnection

14	import java.net.URL

15	import java.net.URLEncoder

16	import java.text.SimpleDateFormat

17	import java.util.Locale

18	import java.util.TimeZone

19

20	/**

21	 * ThingSpeak, the third way to the bins. Each bin has its own channel and

22	 * writes to it at most every 15 s (the free licence's limit). The app reads

23	 * the newest entry with the channel's read key, and sends LOCK, UNLOCK and

24	 * COLLECT through the bin's TalkBack queue, which the bin polls.

25	 *

26	 * Channel fields (the firmware writes the same ones):

27	 *   field1 fill %         field5 air quality (IAQ)

28	 *   field2 weight g       field6 pressure hPa

29	 *   field3 temperature C  field7 tilt deg

30	 *   field4 humidity %     field8 motion + 2 * lid open + 4 * status

31	 *   status (text)         the bin's local IP address

32	 * Status numbers: 0 OK, 1 LOCKED, 2 TAMPER, 3 COLLECTING, 4 CALIBRATING.

33	 */

34	class ThingSpeak(private var server: String = THINGSPEAK_URL) {

35

36	    fun use(url: String) {

37	        server = url.trim().trimEnd('/').ifBlank { THINGSPEAK_URL }

38	    }

39

40	    val url get() = server

41

42	    /** [entryId] goes up by one with each write, which is how the app estimates the message count. */

43	    class Entry(val reading: Reading, val ts: Long, val localIp: String?, val entryId: Long)

44

45	    /** The newest entry of a bin's channel, or null when the channel has none yet. */

46	    suspend fun last(channel: String, key: String): Entry? {

47	        val q = "api_key=${enc(key.trim())}&status=true"

48	        val text = http("GET", "$server/channels/${enc(channel.trim())}/feeds/last.json?$q", null)

49	        if (text.isBlank() || text.trim() == "-1" || text.trim() == "null") return null

50	        val o = JSONObject(text)

51	        fun f(n: Int, def: Double) = o.optString("field$n").toDoubleOrNull() ?: def

52	        val flags = f(8, 0.0).toInt()

53	        val status = BinStatus.entries.getOrElse(flags shr 2) { BinStatus.OK }

54	        val fill = f(1, 0.0)

55	        val r = Reading(

56	            fill = fill, weight = f(2, 0.0), temp = f(3, 28.0), tempRaw = f(3, 28.0), hum = f(4, 60.0),

57	            gas = f(5, 60.0), press = f(6, 1008.4), tilt = f(7, 0.0),

58	            motion = flags and 1 == 1, lidOpen = flags and 2 == 2, status = status,

59	            at = System.currentTimeMillis(),

60	        )

61	        return Entry(r, parseTime(o.optString("created_at")), o.optString("status").takeIf { it.isNotBlank() && it != "null" }, o.optLong("entry_id", 0L))

62	    }

63

64	    /** Puts a command on the bin's TalkBack queue. Only LOCK, UNLOCK and COLLECT. */

65	    suspend fun command(tbId: String, tbKey: String, cmd: String) {

66	        require(cmd in Cloud.CLOUD_COMMANDS) { "$cmd needs the bins' Wi-Fi" }

67	        val body = "api_key=${enc(tbKey.trim())}&command_string=${enc(cmd)}"

68	        http("POST", "$server/talkbacks/${enc(tbId.trim())}/commands.json", body)

69	    }

70

71	    private suspend fun http(method: String, url: String, form: String?): String = withContext(Dispatchers.IO) {

72	        val conn = URL(url).openConnection() as HttpURLConnection

73	        conn.requestMethod = method

74	        conn.connectTimeout = 8000

75	        conn.readTimeout = 10000

76	        try {

77	            if (form != null) {

78	                val bytes = form.toByteArray()

79	                conn.doOutput = true

80	                conn.setRequestProperty("Content-Type", "application/x-www-form-urlencoded")

81	                conn.setFixedLengthStreamingMode(bytes.size)

82	                conn.outputStream.use { it.write(bytes) }

83	            }

84	            val code = conn.responseCode

85	            val stream = if (code in 200..299) conn.inputStream else conn.errorStream

86	            val text = stream?.bufferedReader()?.use { it.readText() }.orEmpty()

87	            if (code !in 200..299) throw IOException(if (code == 400 || code == 401) "The channel ID or key is wrong." else "HTTP $code")

88	            text

89	        } catch (e: IOException) {

90	            conn.disconnect()

91	            throw e

92	        }

93	    }

94

95	    private fun parseTime(s: String): Long = runCatching {

96	        SimpleDateFormat("yyyy-MM-dd'T'HH:mm:ssX", Locale.US).apply { timeZone = TimeZone.getTimeZone("UTC") }.parse(s)!!.time

97	    }.getOrDefault(0L)

98

99	    private fun enc(s: String) = URLEncoder.encode(s, "UTF-8")

100	}

Android App/app/src/main/java/smartbin/data/Ble.kt

1	// ============================================================================

2	// data/Ble.kt - Bluetooth LE client for the bin's Wi-Fi setup (ble_setup.cpp)

3	// Android's GATT API allows one operation at a time, so every read, write and

4	// descriptor write goes through call(), which waits for its callback before

5	// the next one starts. "Bluetooth error 5, 15 or 137" (GattError.auth) means

6	// the link was not secured with the passkey; StaleBond means the phone still

7	// remembers an old pairing. For both, forget the bin in the phone's Bluetooth

8	// settings (forget() below tries this) and pair again.

9	// ============================================================================

10	package smartbin.data

11

12	import android.Manifest

13	import android.annotation.SuppressLint

14	import android.bluetooth.BluetoothAdapter

15	import android.bluetooth.BluetoothDevice

16	import android.bluetooth.BluetoothGatt

17	import android.bluetooth.BluetoothGattCallback

18	import android.bluetooth.BluetoothGattCharacteristic

19	import android.bluetooth.BluetoothGattDescriptor

20	import android.bluetooth.BluetoothManager

21	import android.bluetooth.BluetoothProfile

22	import android.bluetooth.le.ScanCallback

23	import android.bluetooth.le.ScanResult

24	import android.bluetooth.le.ScanSettings

25	import android.content.BroadcastReceiver

26	import android.content.Context

27	import android.content.Intent

28	import android.content.IntentFilter

29	import android.content.pm.PackageManager

30	import android.os.Build

31	import android.os.ParcelUuid

32	import kotlinx.coroutines.CompletableDeferred

33	import kotlinx.coroutines.delay

34	import kotlinx.coroutines.flow.MutableStateFlow

35	import kotlinx.coroutines.flow.first

36	import kotlinx.coroutines.sync.Mutex

37	import kotlinx.coroutines.sync.withLock

38	import kotlinx.coroutines.withTimeout

39	import kotlinx.coroutines.withTimeoutOrNull

40	import org.json.JSONArray

41	import org.json.JSONObject

42	import java.io.IOException

43	import java.util.UUID

44

45	/**

46	 * Wi-Fi setup over Bluetooth with the bin's own GATT service, as the firmware

47	 * (ble_setup.cpp) and Gen's INTEGRATION.md describe it:

48	 *

49	 *   b1a70001 service     advertised by SmartBin-01 and SmartBin-02

50	 *   b1a70002 SSID        write, the network name

51	 *   b1a70003 PASS        write, the password

52	 *   b1a70004 CTRL        write one byte: 1 joins with SSID and PASS, 2 turns Bluetooth off

53	 *   b1a70005 STAT        read and notify, {"state":"WAITING|CONNECTING|CONNECTED|FAILED", ...}

54	 *   b1a70006 SCAN        read, up to 6 networks, strongest first

55	 *

56	 * Every characteristic needs an encrypted, passkey-authenticated link, so the

57	 * phone bonds first and Android shows its own box for the 6-digit code the bin

58	 * shows. One GATT call runs at a time; each waits for its callback.

59	 */

60	@SuppressLint("MissingPermission")

61	class BinBle(context: Context) {

62	    private val app = context.applicationContext

63	    private val manager = app.getSystemService(BluetoothManager::class.java)

64	    private val adapter: BluetoothAdapter? get() = manager?.adapter

65

66	    class Found(val name: String, val address: String, val rssi: Int, val device: BluetoothDevice)

67	    class Net(val ssid: String, val rssi: Int?)

68

69	    /** A link that drops at once: the phone or the bin still remembers an old pairing. */

70	    class StaleBond : IOException("The bin dropped the link")

71

72	    private var gatt: BluetoothGatt? = null

73	    private var chars = HashMap<UUID, BluetoothGattCharacteristic>()

74	    private val ops = Mutex()

75	    private var op: CompletableDeferred<ByteArray>? = null

76	    private val linkUp = MutableStateFlow(false)

77	    private val bond = MutableStateFlow(BluetoothDevice.BOND_NONE)

78	    private var receiver: BroadcastReceiver? = null

79

80	    /** Each STAT message the bin sends, whole. */

81	    var onStat: ((JSONObject) -> Unit)? = null

82	    /** The link dropped while connected. */

83	    var onDrop: (() -> Unit)? = null

84

85	    val device: BluetoothDevice? get() = gatt?.device

86	    val connected get() = linkUp.value

87

88	    // ---------- permissions and the adapter ----------

89

90	    // Android 12+ needs BLUETOOTH_SCAN and BLUETOOTH_CONNECT; older versions

91	    // need location for scanning.

92	    fun permissions(): Array<String> =

93	        if (Build.VERSION.SDK_INT >= 31) arrayOf(Manifest.permission.BLUETOOTH_SCAN, Manifest.permission.BLUETOOTH_CONNECT)

94	        else arrayOf(Manifest.permission.ACCESS_FINE_LOCATION)

95

96	    fun allowed() = permissions().all { app.checkSelfPermission(it) == PackageManager.PERMISSION_GRANTED }

97

98	    val present get() = adapter != null

99	    val on get() = adapter?.isEnabled == true

100

101	    // ---------- scan ----------

102

103	    /** Lists bins nearby for [millis]: by the service in the advert, or by a name that starts with SmartBin. */

104	    suspend fun scan(millis: Long, onFound: (Found) -> Unit) {

105	        val scanner = adapter?.bluetoothLeScanner ?: throw IOException("Bluetooth is off")

106	        val seen = HashSet<String>()

107	        val svc = ParcelUuid(SVC)

108	        val cb = object : ScanCallback() {

109	            override fun onScanResult(callbackType: Int, result: ScanResult) {

110	                val d = result.device

111	                val name = result.scanRecord?.deviceName ?: runCatching { d.name }.getOrNull() ?: ""

112	                val ours = result.scanRecord?.serviceUuids?.contains(svc) == true || name.startsWith("SmartBin", ignoreCase = true)

113	                if (ours && seen.add(d.address)) onFound(Found(name.ifBlank { "SmartBin" }, d.address, result.rssi, d))

114	            }

115	        }

116	        val settings = ScanSettings.Builder().setScanMode(ScanSettings.SCAN_MODE_LOW_LATENCY).build()

117	        scanner.startScan(null, settings, cb)

118	        try {

119	            delay(millis)

120	        } finally {

121	            runCatching { scanner.stopScan(cb) }

122	        }

123	    }

124

125	    // ---------- connect, pair, read and write ----------

126

127	    private val callback = object : BluetoothGattCallback() {

128	        // GATT callbacks. Each one finishes the waiting call() through done().

129	        override fun onConnectionStateChange(g: BluetoothGatt, status: Int, newState: Int) {

130	            if (newState == BluetoothProfile.STATE_CONNECTED && status == BluetoothGatt.GATT_SUCCESS) {

131	                linkUp.value = true

132	            } else if (newState == BluetoothProfile.STATE_DISCONNECTED) {

133	                val was = linkUp.value

134	                linkUp.value = false

135	                op?.completeExceptionally(IOException("The Bluetooth link dropped"))

136	                runCatching { g.close() }

137	                if (gatt === g) gatt = null

138	                if (was) onDrop?.invoke()

139	            }

140	        }

141

142	        override fun onServicesDiscovered(g: BluetoothGatt, status: Int) = done(status, ByteArray(0))

143

144	        override fun onMtuChanged(g: BluetoothGatt, mtu: Int, status: Int) = done(BluetoothGatt.GATT_SUCCESS, ByteArray(0))

145

146	        @Deprecated("Android 12 and older")

147	        override fun onCharacteristicRead(g: BluetoothGatt, ch: BluetoothGattCharacteristic, status: Int) {

148	            @Suppress("DEPRECATION") done(status, ch.value ?: ByteArray(0))

149	        }

150

151	        override fun onCharacteristicRead(g: BluetoothGatt, ch: BluetoothGattCharacteristic, value: ByteArray, status: Int) = done(status, value)

152

153	        override fun onCharacteristicWrite(g: BluetoothGatt, ch: BluetoothGattCharacteristic, status: Int) = done(status, ByteArray(0))

154

155	        override fun onDescriptorWrite(g: BluetoothGatt, d: BluetoothGattDescriptor, status: Int) = done(status, ByteArray(0))

156

157	        @Deprecated("Android 12 and older")

158	        override fun onCharacteristicChanged(g: BluetoothGatt, ch: BluetoothGattCharacteristic) {

159	            @Suppress("DEPRECATION") changed(ch, ch.value ?: ByteArray(0))

160	        }

161

162	        override fun onCharacteristicChanged(g: BluetoothGatt, ch: BluetoothGattCharacteristic, value: ByteArray) = changed(ch, value)

163	    }

164

165	    private fun done(status: Int, value: ByteArray) {

166	        val d = op ?: return

167	        if (status == BluetoothGatt.GATT_SUCCESS) d.complete(value)

168	        else d.completeExceptionally(GattError(status))

169	    }

170

171	    /** A notification is cut to 20 bytes on a small link; one that does not parse is read in full. */

172	    private fun changed(ch: BluetoothGattCharacteristic, value: ByteArray) {

173	        if (ch.uuid != STAT) return

174	        val j = runCatching { JSONObject(String(value)) }.getOrNull()

175	        if (j != null) onStat?.invoke(j) else needFullStat = true

176	    }

177

178	    /** Set when a cut-off STAT notification came in; the waiting loop reads STAT then. */

179	    @Volatile var needFullStat = false

180

181	    class GattError(val status: Int) : IOException("Bluetooth error $status") {

182	        /** 5, 15 and 137: the link is not encrypted with a code the bin accepts. */

183	        val auth get() = status == 5 || status == 15 || status == 137

184	    }

185

186	    /** Runs one GATT call and waits for its answer. */

187	    private suspend fun call(timeout: Long = 8_000, start: (BluetoothGatt) -> Boolean): ByteArray = ops.withLock {

188	        val g = gatt ?: throw IOException("Not connected")

189	        val d = CompletableDeferred<ByteArray>()

190	        op = d

191	        try {

192	            if (!start(g)) throw IOException("Bluetooth is busy")

193	            withTimeout(timeout) { d.await() }

194	        } finally {

195	            op = null

196	        }

197	    }

198

199	    /**

200	     * Connects to the bin and finds its characteristics. A phone that remembers

201	     * an old pairing often gets dropped at once, so it tries 3 times before

202	     * saying so with [StaleBond].

203	     */

204	    suspend fun connect(device: BluetoothDevice) {

205	        close()

206	        watchBond(device)

207	        repeat(3) { i ->

208	            val ok = runCatching {

209	                val g = if (Build.VERSION.SDK_INT >= 26) device.connectGatt(app, false, callback, BluetoothDevice.TRANSPORT_LE)

210	                else device.connectGatt(app, false, callback)

211	                gatt = g

212	                withTimeout(12_000) { linkUp.first { it } }

213	                delay(400)

214	                // A larger packet size, so a whole status message fits in one notification.

215	                runCatching { call(3_000) { it.requestMtu(247) } }

216	                call(10_000) { it.discoverServices() }

217	                val svc = gatt?.getService(SVC) ?: throw IOException("This is not a Smart-Bin")

218	                chars = HashMap(svc.characteristics.associateBy { it.uuid })

219	                if (STAT !in chars || SSID !in chars) throw IOException("This bin has no Wi-Fi setup")

220	            }.isSuccess

221	            if (ok) return

222	            close()

223	            if (i < 2) delay(1200)

224	        }

225	        throw StaleBond()

226	    }

227

228	    /**

229	     * Pairs, so Android asks for the bin's 6-digit code in its own box. Waits as

230	     * long as the code lasts on the bin (60 s), then reads STAT to prove the

231	     * link is encrypted. Returns the STAT message.

232	     */

233	    suspend fun secure(): JSONObject {

234	        val d = gatt?.device ?: throw IOException("Not connected")

235	        if (d.bondState != BluetoothDevice.BOND_BONDED) {

236	            if (d.bondState != BluetoothDevice.BOND_BONDING && !d.createBond()) throw IOException("Android would not start pairing")

237	            val end = withTimeoutOrNull(75_000) { bond.first { it == BluetoothDevice.BOND_BONDED || it == BOND_FAILED } }

238	            if (end != BluetoothDevice.BOND_BONDED) throw IOException("No code was typed, or it was wrong")

239	            delay(600)

240	        }

241	        var last: Exception? = null

242	        repeat(3) {

243	            try {

244	                return readStat()

245	            } catch (e: GattError) {

246	                last = e

247	                if (e.auth) throw StaleBond()

248	                delay(1500)

249	            } catch (e: Exception) {

250	                last = e

251	                delay(1500)

252	            }

253	        }

254	        throw last ?: IOException("The bin did not answer")

255	    }

256

257	    // STAT holds the bin's setup state as JSON: WAITING, CONNECTING,

258	    // CONNECTED (with ip and ssid) or FAILED (with why).

259	    suspend fun readStat(): JSONObject = JSONObject(String(read(STAT)))

260

261	    /** Turns on STAT notifications. */

262	    suspend fun listen() {

263	        val ch = chars[STAT] ?: return

264	        val g = gatt ?: return

265	        g.setCharacteristicNotification(ch, true)

266	        val cccd = ch.getDescriptor(CCCD) ?: return

267	        val v = BluetoothGattDescriptor.ENABLE_NOTIFICATION_VALUE

268	        call {

269	            if (Build.VERSION.SDK_INT >= 33) it.writeDescriptor(cccd, v) == 0

270	            else {

271	                @Suppress("DEPRECATION") cccd.value = v

272	                @Suppress("DEPRECATION") it.writeDescriptor(cccd)

273	            }

274	        }

275	    }

276

277	    /** The networks the bin found, strongest first. Empty while it is still scanning. */

278	    suspend fun networks(): List<Net> {

279	        if (SCAN !in chars) return emptyList()

280	        val a = JSONArray(String(read(SCAN)))

281	        return (0 until a.length()).mapNotNull { i ->

282	            when (val o = a.opt(i)) {

283	                is JSONObject -> o.optString("ssid").takeIf { it.isNotBlank() }?.let { Net(it, if (o.has("rssi")) o.optInt("rssi") else null) }

284	                is String -> o.takeIf { it.isNotBlank() }?.let { Net(it, null) }

285	                else -> null

286	            }

287	        }

288	    }

289

290	    /** Hands over the network and asks the bin to join it. The answer comes on STAT. */

291	    suspend fun join(ssid: String, pass: String) {

292	        write(SSID, ssid.toByteArray())

293	        write(PASS, pass.toByteArray())

294	        write(CTRL, byteArrayOf(1))

295	    }

296

297	    /** Turns Bluetooth off on the bin; it stays on its Wi-Fi. */

298	    suspend fun off() {

299	        runCatching { write(CTRL, byteArrayOf(2)) }

300	        close()

301	    }

302

303	    private suspend fun read(uuid: UUID): ByteArray {

304	        val ch = chars[uuid] ?: throw IOException("Missing $uuid")

305	        return call { it.readCharacteristic(ch) }

306	    }

307

308	    private suspend fun write(uuid: UUID, value: ByteArray) {

309	        val ch = chars[uuid] ?: throw IOException("Missing $uuid")

310	        val type = BluetoothGattCharacteristic.WRITE_TYPE_DEFAULT

311	        call {

312	            if (Build.VERSION.SDK_INT >= 33) it.writeCharacteristic(ch, value, type) == 0

313	            else {

314	                @Suppress("DEPRECATION") ch.value = value

315	                ch.writeType = type

316	                @Suppress("DEPRECATION") it.writeCharacteristic(ch)

317	            }

318	        }

319	    }

320

321	    fun close() {

322	        val g = gatt

323	        gatt = null

324	        linkUp.value = false

325	        runCatching { g?.disconnect() }

326	        runCatching { g?.close() }

327	    }

328

329	    /** Ends everything, when the page or the app goes away. */

330	    fun release() {

331	        close()

332	        receiver?.let { runCatching { app.unregisterReceiver(it) } }

333	        receiver = null

334	    }

335

336	    // Follows the pairing (bond) state so a refused passkey is noticed.

337	    private fun watchBond(device: BluetoothDevice) {

338	        bond.value = device.bondState

339	        if (receiver != null) return

340	        val r = object : BroadcastReceiver() {

341	            override fun onReceive(c: Context, i: Intent) {

342	                val d: BluetoothDevice? = if (Build.VERSION.SDK_INT >= 33) i.getParcelableExtra(BluetoothDevice.EXTRA_DEVICE, BluetoothDevice::class.java)

343	                else @Suppress("DEPRECATION") i.getParcelableExtra(BluetoothDevice.EXTRA_DEVICE)

344	                if (d == null || d.address != gatt?.device?.address) return

345	                val now = i.getIntExtra(BluetoothDevice.EXTRA_BOND_STATE, BluetoothDevice.BOND_NONE)

346	                val before = i.getIntExtra(BluetoothDevice.EXTRA_PREVIOUS_BOND_STATE, BluetoothDevice.BOND_NONE)

347	                bond.value = if (now == BluetoothDevice.BOND_NONE && before == BluetoothDevice.BOND_BONDING) BOND_FAILED else now

348	            }

349	        }

350	        val filter = IntentFilter(BluetoothDevice.ACTION_BOND_STATE_CHANGED)

351	        if (Build.VERSION.SDK_INT >= 33) app.registerReceiver(r, filter, Context.RECEIVER_EXPORTED)

352	        else app.registerReceiver(r, filter)

353	        receiver = r

354	    }

355

356	    /**

357	     * Makes Android forget the bin, for a pairing the bin no longer knows. There

358	     * is no public call for it, so this uses the hidden one most phones keep;

359	     * the caller opens Android's Bluetooth settings when it fails.

360	     */

361	    fun forget(device: BluetoothDevice): Boolean = runCatching {

362	        device.javaClass.getMethod("removeBond").invoke(device) as? Boolean ?: false

363	    }.getOrDefault(false)

364

365	    companion object {

366	        // UUIDs b1a70001..6, the same as BT_*_UUID in the firmware's config.h.

367	        private fun u(n: String) = UUID.fromString("b1a7000$n-5c2e-4f8a-9d33-0a9c2b6f17c1")

368	        val SVC: UUID = u("1")

369	        val SSID: UUID = u("2")

370	        val PASS: UUID = u("3")

371	        val CTRL: UUID = u("4")

372	        val STAT: UUID = u("5")

373	        val SCAN: UUID = u("6")

374	        val CCCD: UUID = UUID.fromString("00002902-0000-1000-8000-00805f9b34fb")

375	        private const val BOND_FAILED = -1

376	    }

377	}

Android App/app/src/main/java/smartbin/data/OperatorLock.kt

1	// ============================================================================

2	// data/OperatorLock.kt - PIN that guards the Operator role on this phone

3	// This is the app's own PIN, separate from the bin's Collect PIN.

4	// ============================================================================

5	package smartbin.data

6

7	import android.util.Base64

8	import java.security.MessageDigest

9	import java.security.SecureRandom

10	import javax.crypto.SecretKeyFactory

11	import javax.crypto.spec.PBEKeySpec

12

13	/**

14	 * The operator PIN. Only a salted PBKDF2 hash is kept, never the PIN. Wrong

15	 * tries are counted on disk, so closing the app does not reset them: after 5

16	 * the lock waits 30 s, and the wait doubles with every miss after that.

17	 */

18	class OperatorLock(private val store: Store) {

19	    val hasPin get() = store.string(KEY_HASH, "").isNotEmpty()

20

21	    /** Milliseconds until another try is allowed, 0 when one is. */

22	    fun waitLeft(now: Long = System.currentTimeMillis()) = (store.string(KEY_UNTIL, "0").toLongOrNull() ?: 0L).minus(now).coerceAtLeast(0L)

23

24	    fun set(pin: String) {

25	        val salt = ByteArray(16).also { SecureRandom().nextBytes(it) }

26	        store.putString(KEY_SALT, b64(salt))

27	        store.putString(KEY_HASH, b64(hash(pin, salt)))

28	        store.putString(KEY_MISSES, "0")

29	        store.putString(KEY_UNTIL, "0")

30	    }

31

32	    /** True when the PIN matches. A miss counts toward the wait. */

33	    fun check(pin: String): Boolean {

34	        if (!hasPin || waitLeft() > 0) return false

35	        val salt = Base64.decode(store.string(KEY_SALT, ""), Base64.NO_WRAP)

36	        val want = Base64.decode(store.string(KEY_HASH, ""), Base64.NO_WRAP)

37	        if (MessageDigest.isEqual(hash(pin, salt), want)) {

38	            store.putString(KEY_MISSES, "0")

39	            return true

40	        }

41	        val misses = (store.string(KEY_MISSES, "0").toIntOrNull() ?: 0) + 1

42	        store.putString(KEY_MISSES, misses.toString())

43	        if (misses >= FREE_TRIES) {

44	            val wait = 30_000L shl (misses - FREE_TRIES).coerceAtMost(6)

45	            store.putString(KEY_UNTIL, (System.currentTimeMillis() + wait).toString())

46	        }

47	        return false

48	    }

49

50	    /** Tries left before the lock starts waiting. */

51	    fun triesLeft() = (FREE_TRIES - (store.string(KEY_MISSES, "0").toIntOrNull() ?: 0)).coerceAtLeast(0)

52

53	    private fun hash(pin: String, salt: ByteArray): ByteArray =

54	        SecretKeyFactory.getInstance("PBKDF2WithHmacSHA256")

55	            .generateSecret(PBEKeySpec(pin.toCharArray(), salt, 20_000, 256)).encoded

56

57	    private fun b64(b: ByteArray) = Base64.encodeToString(b, Base64.NO_WRAP)

58

59	    companion object {

60	        const val MIN = 4

61	        const val MAX = 8

62	        private const val FREE_TRIES = 5

63	        private const val KEY_HASH = "op_pin_hash"

64	        private const val KEY_SALT = "op_pin_salt"

65	        private const val KEY_MISSES = "op_pin_misses"

66	        private const val KEY_UNTIL = "op_pin_until"

67

68	        fun valid(pin: String) = pin.length in MIN..MAX && pin.all { it.isDigit() }

69	    }

70	}

Android App/app/src/main/java/smartbin/data/Demo.kt

1	// ============================================================================

2	// data/Demo.kt - made-up bins for demonstrations without hardware

3	// Demo bins exist only while DEMO is switched on at the start page and are

4	// never saved. step() moves their readings a little every tick so the charts

5	// and alerts behave like real bins.

6	// ============================================================================

7	package smartbin.data

8

9	import kotlin.math.floor

10	import kotlin.math.roundToInt

11	import kotlin.random.Random

12

13	/**

14	 * Invented readings for the demo bins that DEMO on the start page adds, so the

15	 * whole app works with no hardware. The numbers move the way the dashboard's demo

16	 * moves them, and stay inside what the HC-SR04, HX711 and BME680 can report.

17	 * One demo minute fills a bin as much as one real hour would.

18	 */

19	object Demo {

20	    /** How many times faster than real life the demo bins fill. Demo options can change it; 60 is normal. */

21	    const val NORMAL_SPEED = 60.0

22	    var speed = NORMAL_SPEED

23

24	    /** The three demo bins, named as on the dashboard. */

25	    fun bins(): List<Bin> = listOf(

26	        bin(0, "Demo Bin 1", "Main Hall", fill = 34.0, weight = 1430.0, temp = 28.0, hum = 62.0, gas = 72.0, rate = 0.38, id = "demo1"),

27	        bin(1, "Demo Bin 2", "Cafeteria", fill = 62.0, weight = 2570.0, temp = 28.1, hum = 63.0, gas = 118.0, rate = 3.1, id = "demo2"),

28	        bin(2, "Demo Bin 3", "Library", fill = 88.0, weight = 3650.0, temp = 28.4, hum = 61.0, gas = 96.0, rate = 0.9, id = "demo3"),

29	    ).map { it.copy(isDemo = true, cloudId = "demo%02d".format(it.id.last().digitToInt())) }

30

31	    private val PLACES = listOf("Main Hall", "Cafeteria", "Library", "Gym", "Lobby", "Parking", "Admin", "Lab A", "Lab B", "Clinic", "Chapel", "Field")

32

33	    /** A new real bin from the Add sheet. It shows "not answering" until it gets an address or a cloud link. */

34	    fun newBin(index: Int, id: String): Bin =

35	        bin(index, "Bin %02d".format(index + 1), "New location", fill = 0.0, weight = 0.0, temp = 27.6, hum = 58.0, gas = 40.0, rate = 0.0, id = id)

36	            .copy(source = Source.STALE)

37

38	    fun placeFor(index: Int) = PLACES[index % PLACES.size]

39

40	    // Builds one bin with a starting reading and fill rate. Also used by

41	    // newBin() for real bins added from the Add sheet.

42	    private fun bin(

43	        i: Int, name: String, place: String, fill: Double, weight: Double, temp: Double, hum: Double, gas: Double, rate: Double,

44	        id: String = "b${i + 1}",

45	    ): Bin {

46	        val cal = DEFAULT_CAL

47	        return Bin(

48	            id = id, cloudId = "bin%02d".format(i + 1), name = name, place = place, cal = cal, rate = rate,

49	            reading = Reading(

50	                fill = fill, weight = weight, temp = temp, tempRaw = temp - cal.v("temp_off"), hum = hum, press = 1008.4, gas = gas,

51	                distance = distanceFor(fill, cal), at = System.currentTimeMillis(),

52	            ),

53	        )

54	    }

55

56	    // Inverse of the firmware's fill formula, so demo distance matches fill.

57	    fun distanceFor(fill: Double, cal: Cal) = cal.v("d_empty") - fill / 100.0 * (cal.v("d_empty") - cal.v("d_full"))

58

59	    /** One second of the demo for one bin. [calibrating] is the app's calibration switch. */

60	    fun step(b: Bin, calibrating: Boolean, now: Long): Reading {

61	        val r = b.reading

62	        val s = b.sim

63	        val c = b.cal

64	        val sv = s.values

65	        // BME680: the air quality index drifts toward a level set by how full the bin is;

66	        // the raw temperature carries about 2 C of self-heating that the offset removes.

67	        val gas = if (s.on(2)) sv["gas"] ?: r.gas else (r.gas + ((35 + r.fill * 2.2) - r.gas) * 0.02 + (Random.nextDouble() - 0.5) * 5).coerceIn(10.0, 500.0)

68	        val temp: Double

69	        val tempRaw: Double

70	        val hum: Double

71	        if (s.on(3)) {

72	            temp = sv["temp"] ?: r.temp

73	            tempRaw = temp - c.v("temp_off")

74	            hum = sv["humidity"] ?: r.hum

75	        } else {

76	            tempRaw = (r.tempRaw + (Random.nextDouble() - 0.5) * 0.08).coerceIn(24.0, 34.0)

77	            temp = tempRaw + c.v("temp_off")

78	            hum = (r.hum + (Random.nextDouble() - 0.5) * 0.6).coerceIn(35.0, 95.0)

79	        }

80	        val press = (r.press + (Random.nextDouble() - 0.5) * 0.06).coerceIn(985.0, 1030.0)

81	        // After RESET_GAS the air sensor warms up for 20 demo seconds, counting down as the real one does.

82	        val warm = if (r.iaqAcc == 0) ((r.iaqWarmS ?: 20) - 1).coerceAtLeast(0) else null

83	        val iaqAcc = when {

84	            r.iaqAcc == 0 -> if (warm == 0) 1 else 0

85	            r.iaqAcc < 3 && Random.nextDouble() < 0.05 -> r.iaqAcc + 1

86	            else -> r.iaqAcc

87	        }

88	        val base = r.copy(

89	            gas = gas, temp = temp, tempRaw = tempRaw, hum = hum, press = press, iaqAcc = iaqAcc, at = now,

90	            iaqWarmS = if (iaqAcc == 0) warm else null, gasK = if (iaqAcc == 0) 140.0 + Random.nextDouble() * 6 else null,

91	        )

92

93	        if (calibrating) {

94	            return base.copy(status = BinStatus.CALIBRATING, lidOpen = false, distance = c.v("d_empty") - Random.nextDouble() * 0.4)

95	        }

96	        val leaving = r.status == BinStatus.CALIBRATING

97	        var status = if (leaving) BinStatus.OK else r.status

98	        val fill = when {

99	            s.on(0) -> (sv["fill"] ?: r.fill).coerceIn(0.0, 100.0)

100	            status == BinStatus.COLLECTING -> r.fill

101	            else -> (r.fill + b.rate * speed / 3600.0 * (0.7 + Random.nextDouble() * 0.6)).coerceIn(0.0, 100.0)

102	        }

103	        val weight = if (s.on(1)) sv["weight"] ?: r.weight

104	        else (fill / 100 * c.v("weight_cap") * 0.92 + Random.nextDouble() * 25).coerceIn(0.0, 5000.0).roundToInt().toDouble()

105	        val tilt = if (s.on(4) && (sv["tamper"] ?: 0.0) > 0) 38.0 else Random.nextDouble() * 2.2

106	        val motion = if (s.on(5)) (sv["motion"] ?: 0.0) > 0 else Random.nextDouble() < 0.025

107	        val distance = floor(distanceFor(fill, c) * 10) / 10

108	        var lid: Boolean

109	        when {

110	            status == BinStatus.COLLECTING -> lid = true

111	            tilt > c.v("tilt_th") -> { status = BinStatus.TAMPER; lid = false }

112	            status == BinStatus.TAMPER -> { lid = false }

113	            status == BinStatus.LOCKED -> lid = false

114	            c.v("auto_lock") > 0 && fill >= c.v("fill_th") -> { status = BinStatus.LOCKED; lid = false }

115	            else -> lid = if (s.on(6)) (sv["lid"] ?: 0.0) > 0 else motion

116	        }

117	        return base.copy(fill = fill, weight = weight, tilt = tilt, motion = motion, distance = distance, lidOpen = lid, status = status)

118	    }

119

120	    /** A day of made-up history so the charts have something to draw in demo mode. */

121	    fun history(b: Bin, now: Long): List<Sample> {

122	        val n = 24 * 60

123	        val rnd = Random(b.id.hashCode())

124	        val out = ArrayList<Sample>(n)

125	        var fill = b.reading.fill

126	        var temp = b.reading.temp

127	        var hum = b.reading.hum

128	        var press = b.reading.press

129	        var gas = b.reading.gas

130	        for (k in 0 until n) {

131	            out.add(Sample(now - k * 60_000L, fill.toFloat(), temp.toFloat(), hum.toFloat(), press.toFloat(), gas.toFloat()))

132	            // Backwards in time: the bin was emptier before, and was emptied every few hours.

133	            fill -= b.rate / 60.0 + (rnd.nextDouble() - 0.5) * 0.25

134	            if (fill < 2) fill = 60 + rnd.nextDouble() * 25

135	            temp = (temp + (rnd.nextDouble() - 0.5) * 0.3).coerceIn(22.0, 36.0)

136	            hum = (hum + (rnd.nextDouble() - 0.5) * 1.1).coerceIn(35.0, 95.0)

137	            press = (press + (rnd.nextDouble() - 0.5) * 0.18).coerceIn(990.0, 1025.0)

138	            gas = (gas + (rnd.nextDouble() - 0.5) * 7 - (fill - 30) * 0.004).coerceIn(15.0, 400.0)

139	        }

140	        return out.asReversed()

141	    }

142

143	    /** The mockup's example history for the demo bins, so the alert tools have something to act on. */

144	    fun alerts(bins: List<Bin>, now: Long): List<AlertRow> {

145	        val a = bins.getOrNull(0)?.name ?: "Bin 01"

146	        val b = bins.getOrNull(1)?.name ?: a

147	        val m = 60_000L

148	        val rows = listOf(

149	            Triple(2, Severity.CRITICAL, listOf("fill", "Fill threshold passed", "$b is at 81%")),

150	            Triple(9, Severity.WARNING, listOf("gas", "Air quality warning", "$b index 212, limit 200")),

151	            Triple(34, Severity.CRITICAL, listOf("tamper", "Tamper detected", "$a was moved or tilted")),

152	            Triple(41, Severity.WARNING, listOf("humid", "Humidity high", "$b at 88%, limit 85%")),

153	            Triple(60, Severity.INFO, listOf("lock", "Bin locked", "$a locked from the app")),

154	            Triple(62, Severity.SUCCESS, listOf("unlock", "Bin unlocked", "$a is open for use again")),

155	            Triple(180, Severity.SUCCESS, listOf("truck", "Collection complete", "$b is back to normal")),

156	            Triple(300, Severity.INFO, listOf("calib", "Calibration started", "$a is being calibrated")),

157	            Triple(302, Severity.SUCCESS, listOf("save", "Settings saved", "$a: empty 30 cm, full 5 cm, alert at 80%")),

158	            Triple(1260, Severity.CRITICAL, listOf("temp", "Temperature high", "$a at 41.2 °C")),

159	            Triple(4320, Severity.SUCCESS, listOf("truck", "Collection complete", "$a is back to normal")),

160	            Triple(17280, Severity.CRITICAL, listOf("fill", "Fill threshold passed", "$a is at 83%")),

161	        )

162	        return rows.mapIndexed { i, (mins, sev, x) ->

163	            val icon = x[0]

164	            val shown = when (icon) { "unlock" -> "check"; "save" -> "check"; else -> icon }

165	            AlertRow(

166	                id = (i + 1).toLong(), sev = sev, icon = shown, kind = kindFor(icon), title = x[1], meta = x[2],

167	                at = now - mins * m, read = i >= 3, demo = true,

168	            )

169	        }

170	    }

171

172	    // Maps an alert icon name to the filter group on the Alerts page.

173	    fun kindFor(icon: String): AlertKind = when (icon) {

174	        "fill" -> AlertKind.FULL

175	        "temp", "humid", "gas" -> AlertKind.SENSOR

176	        "tamper" -> AlertKind.TAMPER

177	        "lock", "unlock" -> AlertKind.LOCK

178	        "truck" -> AlertKind.PICKUP

179	        else -> AlertKind.SETUP

180	    }

181	}

Android App/app/src/main/java/smartbin/data/Hub.kt

1	// ============================================================================

2	// data/Hub.kt - the app's data engine

3	// tick() runs once a second (every 5 s in the background): it polls every bin

4	// in parallel, merges the answers into HubState, settles cloud commands,

5	// raises alerts and samples history for the charts. Screens never call the

6	// network themselves; they read Hub.state and call the functions below.

7	// Debug tip: a bin's Source (LIVE, CLOUD, THINGSPEAK, STALE) shows which route

8	// answered last; the link chip on screen shows the same thing.

9	// ============================================================================

10	package smartbin.data

11

12	import android.content.Context

13	import android.net.ConnectivityManager

14	import android.net.Network

15	import kotlinx.coroutines.CancellationException

16	import kotlinx.coroutines.CoroutineScope

17	import kotlinx.coroutines.Dispatchers

18	import kotlinx.coroutines.Job

19	import kotlinx.coroutines.SupervisorJob

20	import kotlinx.coroutines.async

21	import kotlinx.coroutines.awaitAll

22	import kotlinx.coroutines.coroutineScope

23	import kotlinx.coroutines.delay

24	import kotlinx.coroutines.flow.MutableSharedFlow

25	import kotlinx.coroutines.flow.MutableStateFlow

26	import kotlinx.coroutines.flow.SharedFlow

27	import kotlinx.coroutines.flow.StateFlow

28	import kotlinx.coroutines.isActive

29	import kotlinx.coroutines.launch

30	import kotlin.math.max

31

32	/** The kinds of alert, one notification channel and one sound each. */

33	enum class AlertType { FULL, TAMPER, SENSOR, LOCK, PICKUP, INFO }

34

35	// One-off events for sounds and notifications (collected in SmartBinApp).

36	sealed interface HubEvent {

37	    data class NewAlert(val row: AlertRow, val type: AlertType, val binId: String) : HubEvent

38	    data class LinkChanged(val from: Link, val to: Link) : HubEvent

39	    data class BinLost(val name: String) : HubEvent

40	    data class BinBack(val name: String) : HubEvent

41	    /** A cloud command was confirmed by the bin, refused ([why]), or ran out of time. */

42	    data class CommandResult(val binId: String, val name: String, val cmd: String, val ok: Boolean, val why: String? = null) : HubEvent

43	    /** A sound for something the demo tour did, named as in the Sfx list (LOCK, PICKUP, ...). */

44	    data class Sound(val name: String) : HubEvent

45	}

46

47	/**

48	 * The one place that talks to the bins. It reads every bin once a second

49	 * (every five while the app is closed), local Wi-Fi first and Firebase second,

50	 * keeps a day of history for the charts, turns changes into alerts, and runs

51	 * the commands. Screens only read [state] and call these functions.

52	 *

53	 * Everything runs on the main thread; only the network calls leave it.

54	 */

55	object Hub {

56	    private val scope = CoroutineScope(SupervisorJob() + Dispatchers.Main.immediate)

57	    private lateinit var store: Store

58	    private var loop: Job? = null

59	    private var started = false

60

61	    private val _state = MutableStateFlow(HubState())

62	    val state: StateFlow<HubState> = _state

63

64	    private val _history = MutableStateFlow<Map<String, List<Sample>>>(emptyMap())

65	    val history: StateFlow<Map<String, List<Sample>>> = _history

66

67	    private val _events = MutableSharedFlow<HubEvent>(extraBufferCapacity = 32)

68	    val events: SharedFlow<HubEvent> = _events

69

70	    lateinit var cloud: Cloud

71	        private set

72	    lateinit var ts: ThingSpeak

73	        private set

74

75	    /** True while the app is on screen. Off screen the bins are read less often. */

76	    @Volatile var foreground = true

77

78	    /**

79	     * True once the user is past the start page and the role picker in this

80	     * run of the app. Alerts reach the notification shade only then, and only

81	     * while the app is in the background: never from the start page, and

82	     * never after the app is swiped away or closed.

83	     */

84	    @Volatile var inMain = false

85

86	    private val nextLocal = HashMap<String, Long>()

87	    private val localFails = HashMap<String, Int>()

88	    private val nextCloud = HashMap<String, Long>()

89	    private val nextTs = HashMap<String, Long>()

90	    private var fbOk = 0L

91	    private var tsOk = 0L

92	    private var lastCheck = 0L

93	    /** Per channel: the first and the latest (time, entry number) seen, for the message estimate. */

94	    private val tsSeen = HashMap<String, Pair<Pair<Long, Long>, Pair<Long, Long>>>()

95	    /** Demo options: bins pretending to have gone quiet. */

96	    private val demoQuiet = HashSet<String>()

97	    /** Demo options: when each pretend cloud command is carried out, by bin. */

98	    private val demoAnswer = HashMap<String, Pair<Long, String>>()

99	    private val simDirty = HashSet<String>()

100	    private val prev = HashMap<String, Flags>()

101	    private var lastSample = 0L

102	    private var lastUsage = 0L

103	    private var alertSeq = 0L

104

105	    // The alert-worthy facts of a bin from the last tick. watch() compares the

106	    // new Flags with these to raise an alert only when something changes.

107	    private data class Flags(

108	        val full: Boolean, val hot: Boolean, val humid: Boolean, val gassy: Boolean, val status: BinStatus, val source: Source,

109	        val faults: String, val acc: Int,

110	    )

111

112	    /** Demo tour: readings held on a demo bin while the tour acts something out. */

113	    private val holds = HashMap<String, (Reading) -> Reading>()

114	    private var tourStep = 0

115	    private var tourNext = 0L

116	    private var tourPauseUntil = 0L

117

118	    /** Firebase watchers: this phone's id, and when it last told the bins it is watching. */

119	    private var watcherId = ""

120	    private var watchedAt = 0L

121	    private var watching = false

122	    /** Firebase alerts written by the bins while the app was away, read once per bin per run. */

123	    private val alertsRead = HashSet<String>()

124	    private var alertsCheck = 0L

125	    private var startedAt = 0L

126

127	    // Runs once per process. Loads saved bins (marked STALE until they answer),

128	    // listens for network changes, and starts the polling loop.

129	    fun init(context: Context) {

130	        if (started) return

131	        started = true

132	        store = Store(context)

133	        cloud = Cloud(store.firebase())

134	        ts = ThingSpeak().apply { use(store.string(KEY_TS_URL, THINGSPEAK_URL)) }

135	        val now = System.currentTimeMillis()

136	        startedAt = now

137	        watcherId = store.string(KEY_WATCHER, "").ifBlank { "app-" + java.util.UUID.randomUUID().toString().take(10) }.also { store.putString(KEY_WATCHER, it) }

138	        var bins = store.bins() ?: emptyList()

139	        var alerts = store.alerts() ?: emptyList()

140	        // Version 1 kept its demo bins in the saved list, as bins with no address. Version 2 makes

141	        // demo bins only when DEMO is on at the start page, so those old ones go, once.

142	        if (!store.bool(KEY_V2, false)) {

143	            // b1 and b2 were the demo bins; a bin with no address, channel or cloud account was demo data too.

144	            val real = bins.filter { it.ip.isNotBlank() || it.onThingSpeak || (cloud.ready && it.id !in setOf("b1", "b2")) }

145	            if (real.isEmpty()) alerts = emptyList()

146	            bins = real

147	            store.saveBins(bins)

148	            store.saveAlerts(alerts)

149	            store.putBool(KEY_V2, true)

150	        }

151	        bins = bins.map { it.copy(source = Source.STALE) }

152	        alertSeq = alerts.maxOfOrNull { it.id } ?: 0L

153	        _state.value = HubState(bins = bins, alerts = alerts, link = linkOf(bins), loading = true)

154	        _history.value = emptyMap()

155

156	        // A new network (joining the bins' Wi-Fi, or leaving it) means try Wi-Fi again at once.

157	        runCatching {

158	            val cm = context.getSystemService(ConnectivityManager::class.java)

159	            cm?.registerDefaultNetworkCallback(object : ConnectivityManager.NetworkCallback() {

160	                override fun onAvailable(network: Network) {

161	                    scope.launch { nextLocal.clear(); localFails.clear(); nextCloud.clear(); nextTs.clear(); lastCheck = 0L }

162	                }

163	            })

164	        }

165	        loop = scope.launch {

166	            while (isActive) {

167	                val began = System.currentTimeMillis()

168	                try {

169	                    tick()

170	                } catch (e: CancellationException) {

171	                    throw e

172	                } catch (_: Exception) {

173	                    // One bad round never stops the loop.

174	                }

175	                val period = if (foreground) 1000L else 5000L

176	                delay(max(200L, period - (System.currentTimeMillis() - began)))

177	            }

178	        }

179	    }

180

181	    // ---------- the loop ----------

182

183	    // One round: poll all bins at once, merge, then derived work. The merge

184	    // keeps edits the user made while the network calls were in flight.

185	    private suspend fun tick() {

186	        val now = System.currentTimeMillis()

187	        val before = _state.value

188	        val polled = coroutineScope { before.bins.map { b -> async { poll(b, now) } }.awaitAll() }

189	        // Bins may have been added, edited or removed while the calls were out: keep those edits.

190	        val byId = polled.associateBy { it.id }

191	        val current = _state.value

192	        val merged = current.bins.map { b ->

193	            val p = byId[b.id] ?: return@map b

194	            val was = before.bins.firstOrNull { it.id == b.id } ?: return@map b

195	            // An edit made while the calls were out wins over the bin's answer.

196	            if (b.ip != was.ip) return@map b

197	            b.copy(

198	                reading = p.reading, source = p.source, lastSeen = p.lastSeen,

199	                cal = if (p.cal !== was.cal && b.cal === was.cal) p.cal else b.cal,

200	                ip = p.ip,

201	                sim = if (b.id in simDirty) b.sim else p.sim,

202	            )

203	        }

204	        val link = linkOf(merged)

205	        if (link != current.link && !current.loading) _events.tryEmit(HubEvent.LinkChanged(current.link, link))

206	        checkRoutes(merged, now)

207	        val pending = settle(_state.value.pending, merged, now)

208	        _state.value = current.copy(

209	            pending = pending,

210	            bins = merged, link = link, loading = false,

211	            fbUp = if (cloud.ready) now - fbOk < 60_000 else null,

212	            tsUp = if (merged.any { it.onThingSpeak }) now - tsOk < 60_000 else null,

213	            tsPerDay = tsPerDay(merged),

214	        )

215	        if (merged.any { it.ip != (current.bins.firstOrNull { c -> c.id == it.id }?.ip) }) persist(merged)

216	        watch(merged, now)

217	        tour(now)

218	        watchers(merged, now)

219	        cloudAlerts(merged, now)

220	        if (now - lastSample >= 10_000) {

221	            lastSample = now

222	            sample(merged, now)

223	        }

224	        if (current.fbUp == true && now - lastUsage > 60_000) {

225	            lastUsage = now

226	            runCatching { cloud.usageBytes() }.getOrNull()?.let { _state.value = _state.value.copy(cloudBytes = it) }

227	        }

228	    }

229

230	    // Reads one bin by the first route that answers: demo data, local Wi-Fi,

231	    // Firebase, then ThingSpeak. Cloud readings older than 90 s are ignored so

232	    // a dead bin is not shown as live.

233	    private suspend fun poll(b: Bin, now: Long): Bin {

234	        if (b.demo) {

235	            // Demo options can make a bin go quiet; it keeps its last reading until it comes back.

236	            if (b.id in demoQuiet) {

237	                demoAnswer.remove(b.id)

238	                return b.copy(source = Source.STALE)

239	            }

240	            // A pretend cloud command is carried out a few seconds after it was sent, as a real bin would.

241	            var from = b

242	            demoAnswer[b.id]?.let { (at, cmd) ->

243	                if (now >= at) {

244	                    demoAnswer.remove(b.id)

245	                    from = b.copy(reading = effect(b.reading, cmd))

246	                }

247	            }

248	            var r = Demo.step(from, _state.value.calibBin == b.id, now)

249	            holds[b.id]?.let { r = it(r) }

250	            return from.copy(reading = r, source = demoSource(_state.value.demoLinks), lastSeen = now)

251	        }

252	        // Local Wi-Fi first: every second while it answers. After a miss it is tried

253	        // again in 2 s, and after three misses in a row every 30 s, as on the dashboard.

254	        if (b.ip.isNotBlank() && now >= (nextLocal[b.id] ?: 0L)) {

255	            try {

256	                val s = LocalApi.status(b.ip)

257	                nextLocal.remove(b.id)

258	                localFails.remove(b.id)

259	                return b.copy(

260	                    reading = s.reading, source = Source.LIVE, lastSeen = now,

261	                    cal = s.cal ?: b.cal,

262	                    sim = if (s.simMask != null && b.id !in simDirty) b.sim.copy(mask = s.simMask) else b.sim,

263	                    fw = s.fw.ifBlank { b.fw }, rssi = s.rssi ?: b.rssi, uptimeS = s.uptimeS ?: b.uptimeS, touchCustom = s.touchCustom ?: b.touchCustom,

264	                )

265	            } catch (e: CancellationException) {

266	                throw e

267	            } catch (_: Exception) {

268	                val fails = (localFails[b.id] ?: 0) + 1

269	                localFails[b.id] = fails

270	                nextLocal[b.id] = now + if (fails < 3) 2_000 else 30_000

271	            }

272	        }

273	        // Then Firebase, every 5 s, when an account is set up.

274	        if (cloud.ready) {

275	            if (now >= (nextCloud[b.id] ?: 0L)) {

276	                nextCloud[b.id] = now + 5_000

277	                try {

278	                    val c = cloud.status(b.cloudId)

279	                    fbOk = now

280	                    val age = if (c != null && c.ts > 0) now - c.ts else 0L

281	                    if (c != null && age < 90_000) {

282	                        return b.copy(

283	                            reading = c.status.reading, source = Source.CLOUD, lastSeen = now,

284	                            cal = c.status.cal ?: b.cal,

285	                            // Each bin writes its own address, so a new one is picked up without typing it.

286	                            ip = c.localIp ?: b.ip,

287	                            fw = c.status.fw.ifBlank { b.fw }, rssi = c.status.rssi ?: b.rssi, uptimeS = c.status.uptimeS ?: b.uptimeS,

288	                        )

289	                    }

290	                } catch (e: CancellationException) {

291	                    throw e

292	                } catch (_: Exception) {

293	                }

294	            } else if (b.source == Source.CLOUD && now - b.lastSeen < 90_000) {

295	                return b

296	            }

297	        }

298	        // Then ThingSpeak, every 15 s, the fastest its free licence lets a bin write.

299	        if (b.onThingSpeak) {

300	            if (now >= (nextTs[b.id] ?: 0L)) {

301	                nextTs[b.id] = now + 15_000

302	                try {

303	                    val e = ts.last(b.tsChannel, b.tsKey)

304	                    tsOk = now

305	                    e?.let { noteEntry(b.tsChannel, it.entryId, now) }

306	                    val age = if (e != null && e.ts > 0) now - e.ts else 0L

307	                    if (e != null && age < 90_000) {

308	                        return b.copy(reading = e.reading, source = Source.THINGSPEAK, lastSeen = now, ip = e.localIp ?: b.ip)

309	                    }

310	                } catch (e: CancellationException) {

311	                    throw e

312	                } catch (_: Exception) {

313	                }

314	            } else if (b.source == Source.THINGSPEAK && now - b.lastSeen < 90_000) {

315	                return b

316	            }

317	        }

318	        // A bin is called lost only after 90 s with no reading at all; until then its last one stays.

319	        if ((b.source == Source.LIVE || b.source == Source.CLOUD || b.source == Source.THINGSPEAK) && now - b.lastSeen < 90_000) return b

320	        return b.copy(source = Source.STALE)

321	    }

322

323	    // The overall link shown in the header: the best route any bin is using.

324	    private fun linkOf(bins: List<Bin>, pretend: Routes = _state.value.demoLinks): Link = when {

325	        bins.isEmpty() -> Link.OFFLINE

326	        bins.all { it.demo } -> when (demoSource(pretend)) {

327	            Source.LIVE -> Link.LIVE

328	            Source.CLOUD -> Link.CLOUD

329	            Source.THINGSPEAK -> Link.THINGSPEAK

330	            else -> Link.DEMO

331	        }

332	        bins.any { it.source == Source.LIVE } -> Link.LIVE

333	        bins.any { it.source == Source.CLOUD } -> Link.CLOUD

334	        bins.any { it.source == Source.THINGSPEAK } -> Link.THINGSPEAK

335	        else -> Link.OFFLINE

336	    }

337

338	    /**

339	     * Firebase and ThingSpeak are only read for data when Wi-Fi fails, so every

340	     * 30 s the one not in use gets a small call, and the link chip can show each

341	     * route that answers.

342	     */

343	    private suspend fun checkRoutes(bins: List<Bin>, now: Long) {

344	        if (now - lastCheck < 30_000) return

345	        lastCheck = now

346	        if (cloud.ready && now - fbOk > 30_000 && cloud.ping()) fbOk = now

347	        // Every channel is read here too, so the message estimate covers all of them.

348	        bins.filter { it.onThingSpeak && it.source != Source.THINGSPEAK }.distinctBy { it.tsChannel }.forEach { t ->

349	            runCatching { ts.last(t.tsChannel, t.tsKey) }.onSuccess { e ->

350	                tsOk = now

351	                e?.let { noteEntry(t.tsChannel, it.entryId, now) }

352	            }

353	        }

354	    }

355

356	    /** Drops each cloud command the bin has now carried out, or that waited too long. */

357	    private fun settle(pending: Map<String, Pending>, bins: List<Bin>, now: Long): Map<String, Pending> {

358	        if (pending.isEmpty()) return pending

359	        val out = HashMap<String, Pending>()

360	        pending.forEach { (id, p) ->

361	            val b = bins.firstOrNull { it.id == id } ?: return@forEach

362	            val fresh = b.lastSeen > p.at

363	            val done = fresh && when (p.cmd) {

364	                "LOCK" -> b.reading.status == BinStatus.LOCKED

365	                "UNLOCK" -> b.reading.status != BinStatus.LOCKED

366	                "COLLECT" -> b.reading.status == BinStatus.COLLECTING

367	                else -> true

368	            }

369	            when {

370	                done -> _events.tryEmit(HubEvent.CommandResult(id, b.name, p.cmd, true))

371	                now > p.deadline -> _events.tryEmit(HubEvent.CommandResult(id, b.name, p.cmd, false))

372	                else -> out[id] = p

373	            }

374	        }

375	        return out

376	    }

377

378	    // Remembers the first and latest ThingSpeak entry number seen per channel,

379	    // for the messages-per-day estimate.

380	    private fun noteEntry(channel: String, entry: Long, now: Long) {

381	        if (entry <= 0) return

382	        val seen = tsSeen[channel]

383	        tsSeen[channel] = if (seen == null || entry < seen.second.second) (now to entry) to (now to entry) else seen.first to (now to entry)

384	    }

385

386	    /** Sums each channel's writes per day, once it has been watched for two minutes. */

387	    private fun tsPerDay(bins: List<Bin>): Double? {

388	        val channels = bins.filter { it.onThingSpeak }.map { it.tsChannel }.toSet()

389	        var total = 0.0

390	        var any = false

391	        channels.forEach { ch ->

392	            val (a, b) = tsSeen[ch] ?: return@forEach

393	            val ms = b.first - a.first

394	            if (ms < 120_000) return@forEach

395	            any = true

396	            total += (b.second - a.second).toDouble() / ms * 86_400_000.0

397	        }

398	        return if (any) total else null

399	    }

400

401	    // Every 10 s, adds one history point per bin and drops points older than

402	    // 24 h. The charts on the Analytics page draw these lists.

403	    private fun sample(bins: List<Bin>, now: Long) {

404	        val cut = now - 24 * 3600_000L

405	        val h = _history.value.toMutableMap()

406	        bins.forEach { b ->

407	            if (b.source == Source.STALE) return@forEach

408	            val r = b.reading

409	            val list = (h[b.id] ?: emptyList()).dropWhile { it.t < cut } +

410	                Sample(now, r.fill.toFloat(), r.temp.toFloat(), r.hum.toFloat(), r.press.toFloat(), r.gas.toFloat())

411	            h[b.id] = list

412	        }

413	        h.keys.retainAll(bins.map { it.id }.toSet())

414	        _history.value = h

415	    }

416

417	    /** Turns changes into alerts. The first reading of a bin only sets the starting point. */

418	    private fun watch(bins: List<Bin>, now: Long) {

419	        bins.forEach { b ->

420	            val r = b.reading

421	            val c = b.cal

422	            val f = Flags(

423	                full = r.fill >= c.v("fill_th") && !r.fault("HC-SR04"), hot = r.temp >= c.v("temp_th") && !r.fault("BME680"),

424	                humid = r.hum >= c.v("hum_th") && !r.fault("BME680"), gassy = r.gas >= c.v("gas_th") && !r.warming && !r.fault("BME680"),

425	                status = r.status, source = b.source, faults = r.faults.joinToString(", "), acc = r.iaqAcc,

426	            )

427	            val p = prev[b.id]

428	            prev[b.id] = f

429	            if (p == null) return@forEach

430	            val n = b.name

431	            if (f.full && !p.full) add(b, Severity.CRITICAL, "fill", "Fill threshold passed", "$n is at ${r.fill.toInt()}%", AlertType.FULL)

432	            if (f.hot && !p.hot) add(b, Severity.CRITICAL, "temp", "Temperature high", "$n at ${"%.1f".format(r.temp)} °C", AlertType.SENSOR)

433	            if (f.humid && !p.humid) add(b, Severity.WARNING, "humid", "Humidity high", "$n at ${r.hum.toInt()}%, limit ${c.v("hum_th").toInt()}%", AlertType.SENSOR)

434	            if (f.gassy && !p.gassy) add(b, Severity.WARNING, "gas", "Air quality warning", "$n index ${r.gas.toInt()}, limit ${c.v("gas_th").toInt()}", AlertType.SENSOR)

435	            if (f.status != p.status) when (f.status) {

436	                BinStatus.TAMPER -> add(b, Severity.CRITICAL, "tamper", "Tamper detected", "$n was moved or tilted", AlertType.TAMPER)

437	                BinStatus.LOCKED -> add(b, Severity.INFO, "lock", "Bin locked", "$n locked from the app", AlertType.LOCK)

438	                BinStatus.CALIBRATING -> add(b, Severity.INFO, "calib", "Calibration started", "$n is being calibrated", AlertType.INFO, kind = AlertKind.SETUP)

439	                BinStatus.COLLECTING -> {}

440	                BinStatus.OK -> when (p.status) {

441	                    BinStatus.LOCKED -> add(b, Severity.SUCCESS, "check", "Bin unlocked", "$n is open for use again", AlertType.LOCK, kind = AlertKind.LOCK)

442	                    BinStatus.COLLECTING -> add(b, Severity.SUCCESS, "truck", "Collection complete", "$n is back to normal", AlertType.PICKUP)

443	                    else -> {}

444	                }

445	            }

446	            if (f.faults.isNotEmpty() && f.faults != p.faults) add(b, Severity.WARNING, "gear", "Sensor not answering", "$n: ${f.faults}", AlertType.SENSOR)

447	            if (f.faults.isEmpty() && p.faults.isNotEmpty()) add(b, Severity.SUCCESS, "check", "Sensor back", "$n reads every sensor again", AlertType.INFO, kind = AlertKind.SENSOR)

448	            if (p.acc == 0 && f.acc > 0) add(b, Severity.SUCCESS, "gas", "Air sensor ready", "$n air quality readings are settled", AlertType.INFO, kind = AlertKind.SENSOR)

449	            run {

450	                if (f.source == Source.STALE && p.source != Source.STALE) _events.tryEmit(HubEvent.BinLost(n))

451	                if (f.source != Source.STALE && p.source == Source.STALE) _events.tryEmit(HubEvent.BinBack(n))

452	            }

453	        }

454	        prev.keys.retainAll(bins.map { it.id }.toSet())

455	    }

456

457	    // Adds one alert row, keeps the newest 200, saves them and emits NewAlert.

458	    private fun add(b: Bin, sev: Severity, icon: String, title: String, meta: String, type: AlertType, kind: AlertKind = Demo.kindFor(icon)) {

459	        val row = AlertRow(++alertSeq, sev, icon, kind, title, meta, System.currentTimeMillis(), demo = b.demo)

460	        val list = (listOf(row) + _state.value.alerts).take(200)

461	        _state.value = _state.value.copy(alerts = list)

462	        saveAlerts(list)

463	        _events.tryEmit(HubEvent.NewAlert(row, type, b.id))

464	    }

465

466	    // ---------- commands ----------

467

468	    fun bin(id: String) = _state.value.bins.firstOrNull { it.id == id }

469

470	    // Changes one bin in the state, optionally saving the list.

471	    private fun update(id: String, persist: Boolean = false, f: (Bin) -> Bin) {

472	        val s = _state.value

473	        val bins = s.bins.map { if (it.id == id) f(it) else it }

474	        _state.value = s.copy(bins = bins, link = linkOf(bins))

475	        if (persist) persist(bins)

476	    }

477

478	    /** Only real bins and their alerts are saved. Demo ones live while demo mode is on. */

479	    private fun persist(bins: List<Bin>) = store.saveBins(bins.filterNot { it.demo })

480	    private fun saveAlerts(list: List<AlertRow>) = store.saveAlerts(list.filterNot { it.demo })

481

482	    /**

483	     * Sends one command. Over Wi-Fi every command works; over the cloud only

484	     * LOCK, UNLOCK and COLLECT. Returns null when it went through, or what to

485	     * tell the user when it did not. The card changes at once either way, the

486	     * same as the dashboard, and the next reading shows what the bin really did.

487	     */

488	    suspend fun command(id: String, cmd: String, extra: Map<String, Any> = emptyMap()): String? {

489	        val b = bin(id) ?: return null

490	        _state.value.pending[id]?.let { return "Waiting for ${b.name} to confirm ${it.cmd}." }

491	        var problem: String? = null

492	        // Set when the command went through Firebase or ThingSpeak, so its key waits for the bin.

493	        var via: Source? = null

494	        // Demo bins act out the route the demo options pretend to use.

495	        if (b.demo) when (b.source) {

496	            Source.STALE -> return "${b.name} is not answering."

497	            Source.CLOUD, Source.THINGSPEAK -> {

498	                if (cmd !in Cloud.CLOUD_COMMANDS) return "That needs the bins' Wi-Fi."

499	                val now = System.currentTimeMillis()

500	                _state.value = _state.value.copy(pending = _state.value.pending + (id to Pending(cmd, b.source, now, now + DEMO_TIMEOUT)))

501	                if (!_state.value.demoNoConfirm) demoAnswer[id] = (now + DEMO_CONFIRM) to cmd

502	                return null

503	            }

504	            else -> {}

505	        } else when (b.source) {

506	            Source.DEMO -> {}

507	            Source.LIVE -> try {

508	                LocalApi.command(b.ip, cmd, extra)

509	            } catch (e: CancellationException) {

510	                throw e

511	            } catch (e: LocalApi.Refused) {

512	                // The bin answered and said no: say why, and do not try the cloud.

513	                return "${b.name}: ${e.why}."

514	            } catch (_: Exception) {

515	                problem = if (cmd in Cloud.CLOUD_COMMANDS && (cloud.ready || b.tbId.isNotBlank())) {

516	                    val viaFb = cloud.ready && runCatching { cloud.command(b.cloudId, cmd).also { k -> k?.let { confirm(id, b.name, cmd, it) } } }.isSuccess

517	                    val viaTs = !viaFb && b.tbId.isNotBlank() && runCatching { ts.command(b.tbId, b.tbKey, cmd) }.isSuccess

518	                    if (viaFb) via = Source.CLOUD

519	                    if (viaTs) via = Source.THINGSPEAK

520	                    if (viaFb || viaTs) null else "${b.name} did not answer on Wi-Fi or the cloud."

521	                } else "${b.name} did not answer. Check its power and Wi-Fi."

522	            }

523	            Source.CLOUD -> problem = if (cmd !in Cloud.CLOUD_COMMANDS) "That needs the bins' Wi-Fi."

524	            else runCatching { cloud.command(b.cloudId, cmd).also { k -> k?.let { confirm(id, b.name, cmd, it) } } }

525	                .exceptionOrNull()?.let { "The cloud did not take it. ${it.message.orEmpty()}" }

526	                .also { if (it == null) via = Source.CLOUD }

527	            Source.THINGSPEAK -> problem = when {

528	                cmd !in Cloud.CLOUD_COMMANDS -> "That needs the bins' Wi-Fi."

529	                b.tbId.isBlank() -> "Add ${b.name}'s TalkBack ID and key in SET to send commands through ThingSpeak."

530	                else -> runCatching { ts.command(b.tbId, b.tbKey, cmd) }.exceptionOrNull()?.let { "ThingSpeak did not take it. ${it.message.orEmpty()}" }

531	                    .also { if (it == null) via = Source.THINGSPEAK }

532	            }

533	            Source.STALE -> problem = "${b.name} is not answering."

534	        }

535	        if (problem != null && b.source != Source.LIVE) return problem

536	        // Over the cloud the card does not change until the bin confirms; the key stays locked meanwhile.

537	        via?.let { v ->

538	            if (problem != null) return problem

539	            val now = System.currentTimeMillis()

540	            val wait = if (v == Source.THINGSPEAK) 120_000L else 60_000L

541	            _state.value = _state.value.copy(pending = _state.value.pending + (id to Pending(cmd, v, now, now + wait)))

542	            return null

543	        }

544	        update(id) { x ->

545	            x.copy(

546	                reading = effect(x.reading, cmd),

547	                sim = if (cmd == "CLEAR_TAMPER") x.sim.copy(values = x.sim.values + ("tamper" to 0.0)) else x.sim,

548	            )

549	        }

550	        return problem

551	    }

552

553	    /**

554	     * The bin writes its answer to a Firebase command under the push key. It is

555	     * read every second for 8 s; with no answer the key keeps waiting for the

556	     * bin's status instead, as before.

557	     */

558	    private fun confirm(id: String, name: String, cmd: String, key: String) {

559	        scope.launch {

560	            repeat(8) {

561	                delay(1000)

562	                val r = runCatching { cloud.result(key) }.getOrNull()

563	                if (r != null && (r.optString("cmd").isBlank() || r.optString("cmd") == cmd)) {

564	                    cloud.dropResult(key)

565	                    if (_state.value.pending[id]?.cmd != cmd) return@launch

566	                    _state.value = _state.value.copy(pending = _state.value.pending - id)

567	                    val ok = r.optBoolean("ok", false)

568	                    _events.tryEmit(HubEvent.CommandResult(id, name, cmd, ok, r.optString("why").takeIf { !ok && it.isNotBlank() }))

569	                    if (ok) update(id) { it.copy(reading = effect(it.reading, cmd)) }

570	                    return@launch

571	                }

572	            }

573	        }

574	    }

575

576	    /** What a command does to a bin's reading, as the card shows it. */

577	    private fun effect(r: Reading, cmd: String): Reading = when (cmd) {

578	        "LOCK" -> r.copy(status = BinStatus.LOCKED, lidOpen = false)

579	        "UNLOCK" -> r.copy(status = BinStatus.OK)

580	        "CLEAR_TAMPER" -> r.copy(status = BinStatus.OK, tilt = 0.0)

581	        "COLLECT" -> r.copy(status = BinStatus.COLLECTING, lidOpen = true)

582	        "COLLECT_DONE" -> r.copy(status = BinStatus.OK, lidOpen = false, fill = 0.0, weight = 0.0, gas = minOf(r.gas, 48.0))

583	        "TARE" -> r.copy(weight = 0.0)

584	        "RESET_GAS" -> r.copy(iaqAcc = 0, iaqWarmS = 20)

585	        else -> r

586	    }

587

588	    /** Calibration mode, on one bin at a time. The bin shows its teal calibration screen while it is on. */

589	    suspend fun setCalibration(id: String?) {

590	        val old = _state.value.calibBin

591	        if (old == id) return

592	        _state.value = _state.value.copy(calibBin = id)

593	        old?.let { o -> bin(o)?.takeIf { it.source == Source.LIVE && !it.demo }?.let { runCatching { LocalApi.command(it.ip, "CALIBRATE_END") } } }

594	        old?.let { o -> update(o) { b -> if (b.reading.status == BinStatus.CALIBRATING) b.copy(reading = b.reading.copy(status = BinStatus.OK)) else b } }

595	        id?.let { n -> bin(n)?.takeIf { it.source == Source.LIVE && !it.demo }?.let { runCatching { LocalApi.command(it.ip, "CALIBRATE_START") } } }

596	        id?.let { n -> update(n) { b -> b.copy(reading = b.reading.copy(status = BinStatus.CALIBRATING)) } }

597	    }

598

599	    /** Stores new calibration values on the bin (EEPROM over Wi-Fi) and in the app. */

600	    suspend fun saveCal(id: String, cal: Cal): String? {

601	        val b = bin(id) ?: return null

602	        if (b.source == Source.LIVE && !b.demo) {

603	            try {

604	                LocalApi.calibrate(b.ip, cal)

605	            } catch (e: CancellationException) {

606	                throw e

607	            } catch (_: Exception) {

608	                return "${b.name} did not answer, so nothing was saved."

609	            }

610	        }

611	        update(id, persist = true) { it.copy(cal = cal) }

612	        bin(id)?.let {

613	            add(it, Severity.SUCCESS, "check", "Settings saved",

614	                "${it.name}: empty ${fmt(cal.v("d_empty"))} cm, full ${fmt(cal.v("d_full"))} cm, alert at ${cal.v("fill_th").toInt()}%",

615	                AlertType.INFO, kind = AlertKind.SETUP)

616	        }

617	        return null

618	    }

619

620	    /** Re-reads the values a bin is running on. */

621	    suspend fun reload(id: String): String? {

622	        val b = bin(id) ?: return null

623	        if (b.ip.isBlank()) return null

624	        return try {

625	            val s = LocalApi.status(b.ip)

626	            update(id, persist = true) { it.copy(cal = s.cal ?: it.cal, reading = s.reading, source = Source.LIVE, lastSeen = System.currentTimeMillis()) }

627	            null

628	        } catch (e: CancellationException) {

629	            throw e

630	        } catch (_: Exception) {

631	            "${b.name} did not answer on Wi-Fi."

632	        }

633	    }

634

635	    /** Changes the simulation rows on screen. [apply] sends them to the bin. */

636	    fun editSim(id: String, sim: Sim) {

637	        simDirty.add(id)

638	        update(id) { it.copy(sim = sim) }

639	    }

640

641	    // Sends the simulation rows to the bin. Simulation only works over local

642	    // Wi-Fi because the bin needs to reply with the simulated values at once.

643	    suspend fun applySim(id: String): String? {

644	        val b = bin(id) ?: return null

645	        if (b.source == Source.LIVE && !b.demo) {

646	            try {

647	                LocalApi.simulate(b.ip, b.sim)

648	            } catch (e: CancellationException) {

649	                throw e

650	            } catch (_: Exception) {

651	                return "${b.name} did not answer. Simulation needs the bins' Wi-Fi."

652	            }

653	        }

654	        simDirty.remove(id)

655	        return null

656	    }

657

658	    // ---------- bins ----------

659

660	    // Adds n empty bins numbered after the existing real ones (bin03, bin04...).

661	    fun addBins(n: Int) {

662	        val s = _state.value

663	        val now = System.currentTimeMillis()

664	        val added = (0 until n).map { k ->

665	            val index = s.bins.count { !it.demo } + k

666	            Demo.newBin(index, "b$now-$k").copy(cloudId = "bin%02d".format(index + 1))

667	        }

668	        val bins = s.bins + added

669	        _state.value = s.copy(bins = bins, link = linkOf(bins))

670	        _history.value = _history.value + added.associate { it.id to emptyList() }

671	        persist(bins)

672	    }

673

674	    /**

675	     * A bin that just joined Wi-Fi over Bluetooth and is not in the list yet.

676	     * Named after its Bluetooth name (SmartBin-01 is Bin 01, cloud name bin01).

677	     */

678	    fun addRealBin(number: String, ip: String): Bin {

679	        val s = _state.value

680	        s.bins.firstOrNull { !it.demo && it.ip == ip }?.let { return it }

681	        val index = s.bins.count { !it.demo }

682	        val num = number.ifBlank { "%02d".format(index + 1) }

683	        val b = Demo.newBin(index, "b${System.currentTimeMillis()}").copy(name = "Bin $num", place = "", cloudId = "bin$num", ip = ip)

684	        val bins = listOf(b) + s.bins

685	        _state.value = s.copy(bins = bins, link = linkOf(bins))

686	        _history.value = _history.value + (b.id to emptyList())

687	        persist(bins)

688	        return b

689	    }

690

691	    /** Sends the saved Firebase account to every real bin on Wi-Fi. Returns how many took it and how many were tried. */

692	    suspend fun sendCloudToBins(): Pair<Int, Int> {

693	        val cfg = store.firebase()

694	        val bins = _state.value.bins.filter { !it.demo && it.ip.isNotBlank() }

695	        var ok = 0

696	        bins.forEach { b -> if (runCatching { LocalApi.sendCloud(b.ip, cfg) }.isSuccess) ok++ }

697	        return ok to bins.size

698	    }

699

700	    // Saves a bin's settings. A new IP or ThingSpeak channel marks it STALE

701	    // and resets its retry timers so the new address is tried at once.

702	    fun editBin(

703	        id: String, name: String, place: String, ip: String, cloudId: String,

704	        tsChannel: String? = null, tsKey: String? = null, tbId: String? = null, tbKey: String? = null,

705	    ) {

706	        val clean = ip.trim()

707	        update(id, persist = true) {

708	            val next = it.copy(

709	                name = name.trim().ifBlank { it.name }, place = place.trim(), ip = clean,

710	                cloudId = cloudId.trim().ifBlank { it.cloudId },

711	                tsChannel = tsChannel?.trim() ?: it.tsChannel, tsKey = tsKey?.trim() ?: it.tsKey,

712	                tbId = tbId?.trim() ?: it.tbId, tbKey = tbKey?.trim() ?: it.tbKey,

713	            )

714	            val moved = next.ip != it.ip || next.tsChannel != it.tsChannel

715	            next.copy(source = if (moved) (if (next.demo) Source.DEMO else Source.STALE) else it.source)

716	        }

717	        nextLocal.remove(id)

718	        nextCloud.remove(id)

719	        nextTs.remove(id)

720	        demoQuiet.remove(id)

721	    }

722

723	    fun removeBin(id: String) {

724	        val s = _state.value

725	        val bins = s.bins.filterNot { it.id == id }

726	        _state.value = s.copy(bins = bins, link = linkOf(bins), calibBin = s.calibBin.takeIf { it != id })

727	        persist(bins)

728	    }

729

730	    // ---------- alerts ----------

731

732	    // Alert list edits from the Alerts page.

733	    fun markRead(ids: Set<Long>, read: Boolean = true) = editAlerts { list -> list.map { if (it.id in ids) it.copy(read = read) else it } }

734

735	    fun deleteAlerts(ids: Set<Long>) = editAlerts { list -> list.filterNot { it.id in ids } }

736

737	    private fun editAlerts(f: (List<AlertRow>) -> List<AlertRow>) {

738	        val list = f(_state.value.alerts)

739	        _state.value = _state.value.copy(alerts = list)

740	        saveAlerts(list)

741	    }

742

743	    // ---------- demo options ----------

744

745	    /** Demo options change only bins with no address. Left alone, the demo runs as normal. */

746	    val demoInUse get() = Demo.speed != Demo.NORMAL_SPEED || _state.value.demoLinks.any || _state.value.demoNoConfirm || demoQuiet.isNotEmpty()

747

748	    // Demo options: how fast the demo bins fill.

749	    fun demoSpeed(x: Double) {

750	        Demo.speed = x

751	        _state.value = _state.value.copy()

752	    }

753

754	    /**

755	     * Pretend links. The first one on, in the app's own order (Wi-Fi, Firebase,

756	     * ThingSpeak), is the one the demo bins answer on, so the cards, banners,

757	     * link sounds and command waits behave as they would on that route.

758	     */

759	    fun demoLinks(r: Routes) {

760	        val s = _state.value

761	        val via = demoSource(r)

762	        val bins = s.bins.map { if (it.demo && it.source != Source.STALE) it.copy(source = via) else it }

763	        val link = linkOf(bins, r)

764	        if (link != s.link) _events.tryEmit(HubEvent.LinkChanged(s.link, link))

765	        _state.value = s.copy(bins = bins, demoLinks = r, link = link)

766	    }

767

768	    /** Pretend cloud commands time out instead of being confirmed. */

769	    fun setDemoNoConfirm(no: Boolean) {

770	        if (no) demoAnswer.clear()

771	        _state.value = _state.value.copy(demoNoConfirm = no)

772	    }

773

774	    private fun demoSource(r: Routes) = when {

775	        r.local -> Source.LIVE

776	        r.firebase -> Source.CLOUD

777	        r.thingspeak -> Source.THINGSPEAK

778	        else -> Source.DEMO

779	    }

780

781	    // Demo options: make a demo bin stop answering, to show the "lost" state.

782	    fun demoQuiet(id: String) = id in demoQuiet

783

784	    fun setDemoQuiet(id: String, quiet: Boolean) {

785	        if (quiet) demoQuiet.add(id) else demoQuiet.remove(id)

786	        if (quiet) update(id) { it.copy(lastSeen = System.currentTimeMillis()) }

787	        if (!quiet) update(id) { it.copy(source = demoSource(_state.value.demoLinks)) }

788	    }

789

790	    // Demo options: jump a demo bin straight into one of these situations.

791	    enum class DemoEvent { FULL, TAMPER, AIR, EMPTY }

792

793	    fun demoEvent(id: String, e: DemoEvent) {

794	        update(id) { b ->

795	            val r = b.reading

796	            val c = b.cal

797	            b.copy(

798	                reading = when (e) {

799	                    DemoEvent.FULL -> r.copy(fill = minOf(100.0, c.v("fill_th") + 3), distance = Demo.distanceFor(c.v("fill_th") + 3, c))

800	                    DemoEvent.TAMPER -> r.copy(status = BinStatus.TAMPER, tilt = 38.0, lidOpen = false)

801	                    DemoEvent.AIR -> r.copy(gas = c.v("gas_th") + 60)

802	                    DemoEvent.EMPTY -> r.copy(fill = 0.0, weight = 0.0, gas = minOf(r.gas, 48.0), status = BinStatus.OK, distance = c.v("d_empty"))

803	                },

804	            )

805	        }

806	    }

807

808	    /** Puts every demo option back, so the demo runs as it does on a fresh start. */

809	    fun resetDemo() {

810	        Demo.speed = Demo.NORMAL_SPEED

811	        demoQuiet.clear()

812	        demoAnswer.clear()

813	        val now = System.currentTimeMillis()

814	        val start = Demo.bins().associateBy { it.id }

815	        val s = _state.value

816	        val bins = s.bins.map { b -> if (!b.demo) b else start[b.id]?.let { o -> b.copy(reading = o.reading, rate = o.rate, source = Source.DEMO) } ?: b.copy(source = Source.DEMO) }

817	        val link = linkOf(bins, Routes())

818	        if (link != s.link) _events.tryEmit(HubEvent.LinkChanged(s.link, link))

819	        _state.value = s.copy(bins = bins, link = link, demoLinks = Routes(), demoNoConfirm = false, pending = s.pending.filterKeys { k -> bins.none { it.id == k && it.demo } })

820	        _history.value = _history.value + bins.filter { it.demo }.associate { it.id to Demo.history(it, now) }

821	    }

822

823	    // ---------- demo mode and the demo tour ----------

824

825	    /**

826	     * Demo mode, from the start page only. On adds the three demo bins with a

827	     * day of made-up history and alerts; off takes them away with everything

828	     * the demo options changed.

829	     */

830	    fun setDemo(on: Boolean) {

831	        val s = _state.value

832	        if (on == s.demoOn) return

833	        val now = System.currentTimeMillis()

834	        holds.clear()

835	        tourStep = 0

836	        tourNext = now + 6_000

837	        tourPauseUntil = 0L

838	        if (on) {

839	            val demo = Demo.bins()

840	            val alerts = (Demo.alerts(demo, now).map { it.copy(id = ++alertSeq) } + s.alerts).sortedByDescending { it.at }

841	            val bins = s.bins.filterNot { it.demo } + demo

842	            _state.value = s.copy(bins = bins, alerts = alerts, demoOn = true, link = linkOf(bins), tourMsg = null)

843	            _history.value = _history.value + demo.associate { it.id to Demo.history(it, now) }

844	        } else {

845	            Demo.speed = Demo.NORMAL_SPEED

846	            demoQuiet.clear()

847	            demoAnswer.clear()

848	            val gone = s.bins.filter { it.demo }.map { it.id }.toSet()

849	            val bins = s.bins.filterNot { it.demo }

850	            _state.value = s.copy(

851	                bins = bins, alerts = s.alerts.filterNot { it.demo }, demoOn = false, link = linkOf(bins, Routes()),

852	                demoLinks = Routes(), demoNoConfirm = false, tourMsg = null,

853	                pending = s.pending - gone, calibBin = s.calibBin.takeIf { it !in gone },

854	            )

855	            _history.value = _history.value - gone

856	            gone.forEach { prev.remove(it) }

857	        }

858	    }

859

860	    /** A key pressed on a demo bin, or a demo option changed: the tour waits a minute. */

861	    fun pauseTour() {

862	        tourPauseUntil = System.currentTimeMillis() + 60_000

863	        if (_state.value.tourMsg != null) _state.value = _state.value.copy(tourMsg = null)

864	    }

865

866	    private fun hold(id: String, f: ((Reading) -> Reading)?) {

867	        if (f == null) holds.remove(id) else holds[id] = f

868	    }

869

870	    /** A command acted out by the tour: the card changes and its sound plays, as if a key was pressed. */

871	    private fun tourCmd(b: Bin, cmd: String) {

872	        update(b.id) { it.copy(reading = effect(it.reading, cmd)) }

873	        TOUR_SFX[cmd]?.let { _events.tryEmit(HubEvent.Sound(it)) }

874	    }

875

876	    /**

877	     * With demo mode on and nobody pressing anything on a demo bin, the demo

878	     * bins act out every outcome in turn, about every 10 s: full, pickup,

879	     * tamper, lock, bad air, heat, humidity, a sensor fault, warm-up, the

880	     * links changing, a bin going quiet, and calibration.

881	     */

882	    private fun tour(now: Long) {

883	        val s = _state.value

884	        val demo = s.bins.filter { it.demo }

885	        if (!s.demoOn || demo.isEmpty() || !inMain || !foreground) {

886	            tourNext = maxOf(tourNext, now + 6_000)

887	            return

888	        }

889	        if (now < tourPauseUntil || now < tourNext) return

890	        tourNext = now + 10_000

891	        val a = demo[0]

892	        val b = demo[1 % demo.size]

893	        val c = demo[2 % demo.size]

894	        val step = tourStep

895	        tourStep = (tourStep + 1) % TOUR_STEPS

896	        val msg: String = when (step) {

897	            0 -> { demoEvent(a.id, DemoEvent.FULL); "${a.name} fills past its ${a.cal.v("fill_th").toInt()}% limit." }

898	            1 -> { if (bin(a.id)?.reading?.status == BinStatus.LOCKED) tourCmd(a, "UNLOCK"); tourCmd(a, "COLLECT"); "A collector starts the pickup at ${a.name}. The lid opens." }

899	            2 -> { tourCmd(a, "COLLECT_DONE"); "${a.name} is emptied and back to normal." }

900	            3 -> { hold(b.id) { r -> r.copy(tilt = 40.0, status = BinStatus.TAMPER, lidOpen = false, tamperPhase = "LATCHED") }; "${b.name} is tipped over. Tamper alert." }

901	            4 -> { hold(b.id, null); tourCmd(b, "CLEAR_TAMPER"); "The tamper alert on ${b.name} is reset." }

902	            5 -> { tourCmd(c, "LOCK"); "${c.name} is locked from the app." }

903	            6 -> { tourCmd(c, "UNLOCK"); "${c.name} is unlocked again." }

904	            7 -> { val g = c.cal.v("gas_th") + 45; hold(c.id) { r -> r.copy(gas = g) }; "The air in ${c.name} goes bad." }

905	            8 -> {

906	                hold(c.id, null); update(c.id) { it.copy(reading = it.reading.copy(gas = 60.0)) }

907	                val t = b.cal.v("temp_th") + 3.5

908	                hold(b.id) { r -> r.copy(temp = t, tempRaw = t - b.cal.v("temp_off")) }

909	                "${b.name} gets too hot."

910	            }

911	            9 -> {

912	                val h = b.cal.v("hum_th") + 5

913	                hold(b.id) { r -> r.copy(hum = h) }

914	                update(b.id) { it.copy(reading = it.reading.copy(temp = 29.0, tempRaw = 29.0 - it.cal.v("temp_off"))) }

915	                "${b.name} gets too humid."

916	            }

917	            10 -> {

918	                hold(b.id, null); update(b.id) { it.copy(reading = it.reading.copy(hum = 62.0)) }

919	                hold(a.id) { r -> r.copy(faults = listOf("HX711"), weight = 0.0) }

920	                "The weight sensor of ${a.name} stops answering."

921	            }

922	            11 -> { hold(a.id, null); "The weight sensor of ${a.name} answers again." }

923	            12 -> { tourCmd(c, "RESET_GAS"); "The air sensor of ${c.name} warms up. Its score is held until it settles." }

924	            13 -> { demoLinks(Routes(firebase = true)); "${b.name} and the others lose Wi-Fi, so they are read through Firebase." }

925	            14 -> { demoLinks(Routes(thingspeak = true)); "Firebase is down too, so the bins are read through ThingSpeak." }

926	            15 -> { setDemoQuiet(b.id, true); "${b.name} stops answering on every link." }

927	            16 -> { setDemoQuiet(b.id, false); demoLinks(Routes(local = true)); "${b.name} is back, and the bins answer on Wi-Fi again." }

928	            17 -> { demoLinks(Routes()); scope.launch { setCalibration(b.id) }; TOUR_SFX["CALIBRATE_START"]?.let { _events.tryEmit(HubEvent.Sound(it)) }; "${b.name} is being calibrated." }

929	            else -> { scope.launch { setCalibration(null) }; "Calibration of ${b.name} is done. The tour starts again." }

930	        }

931	        _state.value = _state.value.copy(tourMsg = msg)

932	    }

933

934	    // ---------- Firebase watchers and alerts ----------

935

936	    /**

937	     * While the app is open and reads a bin through Firebase, it tells the bins

938	     * every 10 s, so they write every 5 s instead of every minute. Going to

939	     * the background takes the note away.

940	     */

941	    private suspend fun watchers(bins: List<Bin>, now: Long) {

942	        if (!cloud.ready) return

943	        val want = foreground && bins.any { !it.demo && it.source == Source.CLOUD }

944	        if (want && now - watchedAt >= 10_000) {

945	            watchedAt = now

946	            if (runCatching { cloud.watch(watcherId, android.os.Build.MODEL ?: "phone") }.isSuccess) watching = true

947	        } else if (!want && watching) {

948	            watching = false

949	            watchedAt = 0L

950	            cloud.unwatch(watcherId)

951	        }

952	    }

953

954	    /**

955	     * The alerts each bin wrote to Firebase while the app was closed, read once

956	     * per bin after the app starts. Alerts while the app is open come from the

957	     * readings themselves, so they are not read twice.

958	     */

959	    private suspend fun cloudAlerts(bins: List<Bin>, now: Long) {

960	        if (!cloud.ready || !foreground || now - alertsCheck < 30_000) return

961	        alertsCheck = now

962	        bins.filter { !it.demo && it.cloudId.isNotBlank() && it.cloudId !in alertsRead }.forEach { b ->

963	            val list = runCatching { cloud.alerts(b.cloudId) }.getOrNull() ?: return@forEach

964	            alertsRead.add(b.cloudId)

965	            val seenKey = "fb_alerts_" + b.cloudId

966	            val seen = store.string(seenKey, "0").toLongOrNull() ?: 0L

967	            val fresh = list.filter { it.optLong("ts") in (seen + 1) until startedAt }

968	            if (fresh.isNotEmpty()) {

969	                val rows = fresh.map { o ->

970	                    val kind = when (o.optString("kind")) {

971	                        "full" -> AlertKind.FULL; "sensor" -> AlertKind.SENSOR; "tamper" -> AlertKind.TAMPER

972	                        "lock" -> AlertKind.LOCK; "pickup" -> AlertKind.PICKUP; else -> AlertKind.SETUP

973	                    }

974	                    val sev = when (o.optString("sev")) {

975	                        "critical" -> Severity.CRITICAL; "warning" -> Severity.WARNING; "success" -> Severity.SUCCESS; else -> Severity.INFO

976	                    }

977	                    val icon = when (kind) {

978	                        AlertKind.FULL -> "fill"; AlertKind.SENSOR -> "gas"; AlertKind.TAMPER -> "tamper"

979	                        AlertKind.LOCK -> "lock"; AlertKind.PICKUP -> "truck"; else -> "calib"

980	                    }

981	                    AlertRow(++alertSeq, sev, icon, kind, o.optString("title", "Bin alert"), o.optString("meta", b.name), o.optLong("ts"))

982	                }

983	                val all = (rows + _state.value.alerts).sortedByDescending { it.at }.take(200)

984	                _state.value = _state.value.copy(alerts = all)

985	                saveAlerts(all)

986	            }

987	            list.maxOfOrNull { it.optLong("ts") }?.let { store.putString(seenKey, it.toString()) }

988	        }

989	    }

990

991	    // ---------- ThingSpeak ----------

992

993	    // ---------- ThingSpeak and Firebase settings ----------

994	    fun thingSpeakUrl() = ts.url

995

996	    fun saveThingSpeak(url: String) {

997	        ts.use(url)

998	        store.putString(KEY_TS_URL, ts.url)

999	        nextTs.clear()

1000	        lastCheck = 0L

1001	    }

1002

1003	    /** Reads each bin's channel once. Returns how many answered and the first problem. */

1004	    suspend fun testThingSpeak(): Pair<Int, String?> {

1005	        val bins = _state.value.bins.filter { it.onThingSpeak }

1006	        if (bins.isEmpty()) return 0 to "No bin has a ThingSpeak channel yet. Add one in its SET sheet."

1007	        var ok = 0

1008	        var problem: String? = null

1009	        bins.forEach { b ->

1010	            try {

1011	                ts.last(b.tsChannel, b.tsKey)

1012	                ok++

1013	            } catch (e: CancellationException) {

1014	                throw e

1015	            } catch (e: Exception) {

1016	                if (problem == null) problem = "${b.name}: ${e.message ?: "no answer"}"

1017	            }

1018	        }

1019	        if (ok > 0) tsOk = System.currentTimeMillis()

1020	        return ok to problem

1021	    }

1022

1023	    // ---------- Firebase ----------

1024

1025	    fun firebase() = store.firebase()

1026	    fun firebaseWorking() = store.firebaseWorking()

1027

1028	    fun saveFirebase(cfg: FirebaseConfig) {

1029	        store.saveFirebase(cfg, passedTest = true)

1030	        cloud.use(cfg)

1031	        nextCloud.clear()

1032	    }

1033

1034	    /** Estimated fill speed in percent per hour, from the last hour of history; demo bins use their own rate. */

1035	    fun ratePerHour(b: Bin): Double {

1036	        if (b.demo) return b.rate

1037	        val h = _history.value[b.id] ?: return 0.0

1038	        val now = System.currentTimeMillis()

1039	        val recent = h.filter { it.t >= now - 3600_000L }

1040	        if (recent.size < 6) return 0.0

1041	        val hours = (recent.last().t - recent.first().t) / 3600_000.0

1042	        if (hours <= 0.05) return 0.0

1043	        return ((recent.last().fill - recent.first().fill) / hours).coerceAtLeast(0.0)

1044	    }

1045

1046	    private const val KEY_TS_URL = "ts_url"

1047	    private const val KEY_V2 = "v2_bins"

1048	    private const val KEY_WATCHER = "watcher_id"

1049	    private const val TOUR_STEPS = 19

1050	    /** The sound each tour command plays, the same as its key in the app. */

1051	    private val TOUR_SFX = mapOf(

1052	        "LOCK" to "LOCK", "UNLOCK" to "UNLOCK", "COLLECT" to "PICKUP", "COLLECT_DONE" to "DONE",

1053	        "CLEAR_TAMPER" to "CONFIRM", "RESET_GAS" to "CONFIRM", "CALIBRATE_START" to "CALIB",

1054	    )

1055	    /** Pretend cloud commands: confirmed after 4 s, or given up after 8 s, as in Garbage-Guard's demo. */

1056	    private const val DEMO_CONFIRM = 4_000L

1057	    private const val DEMO_TIMEOUT = 8_000L

1058

1059	    private fun fmt(v: Double) = if (v == Math.floor(v)) v.toInt().toString() else "%.1f".format(v)

1060

1061	    fun store() = store

1062	}

Android App/app/src/main/java/smartbin/notify/Sounds.kt

1	// ============================================================================

2	// notify/Sounds.kt - in-app sound effects

3	// Every Sfx entry is a WAV in res/raw. SoundPool loads them all at start-up

4	// and can mix up to 6 at once. A sound asked for before it has loaded is

5	// dropped (except the boot chime), so a missing first sound is not a bug.

6	// ============================================================================

7	package smartbin.notify

8

9	import android.content.Context

10	import android.media.AudioAttributes

11	import android.media.SoundPool

12	import android.os.SystemClock

13	import smartbin.R

14

15	/**

16	 * The app's own sound for each kind of action, built by tools/make_sounds.py.

17	 * Rising pitch means forward, on, open or OK; falling means back, off, close

18	 * or no. Alerts that must sound while the app is closed go through the

19	 * notification channels in [Alerts], because only Android can play those.

20	 */

21	enum class Sfx(val res: Int, val volume: Float = 0.6f) {

22	    TAP(R.raw.sb_tap),

23	    CHOICE(R.raw.sb_choice),

24	    SWITCH_ON(R.raw.sb_switch_on),

25	    SWITCH_OFF(R.raw.sb_switch_off),

26	    STEP_UP(R.raw.sb_step_up),

27	    STEP_DOWN(R.raw.sb_step_down),

28	    TAB(R.raw.sb_tab),

29	    OPEN(R.raw.sb_open),

30	    BACK(R.raw.sb_back),

31	    CONFIRM(R.raw.sb_confirm),

32	    DANGER(R.raw.sb_danger),

33	    DENIED(R.raw.sb_denied),

34	    LOGO(R.raw.sb_logo),

35	    THEME_LIGHT(R.raw.sb_theme_light),

36	    THEME_DARK(R.raw.sb_theme_dark),

37

38	    BOOT(R.raw.sb_boot, 0.8f),

39	    SAVED(R.raw.sb_saved, 0.7f),

40	    LINK_UP(R.raw.sb_link_up, 0.7f),

41	    LINK_LOST(R.raw.sb_link_lost, 0.7f),

42	    CLOUD(R.raw.sb_cloud, 0.7f),

43	    LOCK(R.raw.sb_lock, 0.7f),

44	    UNLOCK(R.raw.sb_unlock, 0.7f),

45	    PICKUP(R.raw.sb_pickup, 0.7f),

46	    DONE(R.raw.sb_done, 0.75f),

47	    CALIB(R.raw.sb_calib, 0.7f),

48	    SIM(R.raw.sb_sim, 0.7f),

49	    BIN_LOST(R.raw.sb_bin_lost, 0.7f),

50	    BIN_BACK(R.raw.sb_bin_back, 0.7f),

51	    PIN_KEY(R.raw.sb_pin_key, 0.5f),

52	    PIN_WRONG(R.raw.sb_pin_wrong, 0.7f),

53	    PIN_LOCKOUT(R.raw.sb_pin_lockout, 0.75f),

54	    OP_IN(R.raw.sb_op_in, 0.7f),

55	    OP_OUT(R.raw.sb_op_out, 0.7f),

56	    CMD_OK(R.raw.sb_cmd_ok, 0.7f),

57	    CMD_FAIL(R.raw.sb_cmd_fail, 0.7f),

58	    BT_PAIRED(R.raw.sb_bt_paired, 0.7f),

59	    BT_WRONG(R.raw.sb_bt_wrong, 0.7f),

60	    BT_BLOCKED(R.raw.sb_bt_blocked, 0.75f),

61	    WIFI_JOINED(R.raw.sb_wifi_joined, 0.7f),

62	    PIN_SET(R.raw.sb_pin_set, 0.7f),

63	    WIFI_FAILED(R.raw.sb_wifi_failed, 0.7f),

64	    BT_OFF(R.raw.sb_bt_off, 0.7f),

65	    DEMO(R.raw.sb_demo, 0.7f),

66

67	    ALERT_FULL(R.raw.sb_alert_full, 0.8f),

68	    ALERT_TAMPER(R.raw.sb_alert_tamper, 0.8f),

69	    ALERT_SENSOR(R.raw.sb_alert_sensor, 0.8f),

70	    ALERT_LOCK(R.raw.sb_alert_lock, 0.8f),

71	    ALERT_PICKUP(R.raw.sb_alert_pickup, 0.8f),

72	    ALERT_INFO(R.raw.sb_alert_info, 0.8f),

73	}

74

75	object Sounds {

76	    private var pool: SoundPool? = null

77	    private val loaded = HashMap<Sfx, Int>()

78	    private val ready = HashSet<Int>()

79	    private var bootWanted = false

80	    @Volatile private var last = 0L

81

82	    fun init(context: Context) {

83	        if (pool != null) return

84	        val app = context.applicationContext

85	        val made = SoundPool.Builder()

86	            .setMaxStreams(6)

87	            .setAudioAttributes(

88	                AudioAttributes.Builder()

89	                    .setUsage(AudioAttributes.USAGE_ASSISTANCE_SONIFICATION)

90	                    .setContentType(AudioAttributes.CONTENT_TYPE_SONIFICATION)

91	                    .build()

92	            )

93	            .build()

94	        pool = made

95	        made.setOnLoadCompleteListener { p, sample, status ->

96	            if (status != 0) return@setOnLoadCompleteListener

97	            ready.add(sample)

98	            // The opening chime is asked for before the sounds have loaded; it plays as soon as it can.

99	            if (bootWanted && sample == loaded[Sfx.BOOT]) {

100	                bootWanted = false

101	                if (Feel.sounds) p.play(sample, Sfx.BOOT.volume, Sfx.BOOT.volume, 1, 0, 1f)

102	            }

103	        }

104	        Sfx.entries.forEach { loaded[it] = made.load(app, it.res, 1) }

105	    }

106

107	    fun play(sfx: Sfx, force: Boolean = false) {

108	        if (!Feel.sounds && !force) return

109	        val id = loaded[sfx] ?: return

110	        if (id !in ready) return

111	        last = SystemClock.uptimeMillis()

112	        pool?.play(id, sfx.volume, sfx.volume, 1, 0, 1f)

113	    }

114

115	    /** For sounds that follow from another action: skipped when a key has just made its own sound. */

116	    fun playIfQuiet(sfx: Sfx) {

117	        if (SystemClock.uptimeMillis() - last > 250) play(sfx)

118	    }

119

120	    /** The opening chime. Waits for the sound to load if it has not yet. */

121	    fun boot() {

122	        val id = loaded[Sfx.BOOT]

123	        if (id != null && id in ready) play(Sfx.BOOT) else bootWanted = true

124	    }

125	}

Android App/app/src/main/java/smartbin/notify/Haptics.kt

1	// ============================================================================

2	// notify/Haptics.kt - custom vibration patterns

3	// Strength scales every amplitude: LIGHT 0.45, NORMAL 0.75, STRONG 1.0.

4	// Vibration off, or a phone with no motor, skips the pattern silently.

5	// ============================================================================

6	package smartbin.notify

7

8	import android.content.Context

9	import android.os.Build

10	import android.os.VibrationEffect

11	import android.os.Vibrator

12	import android.os.VibratorManager

13	import kotlin.math.roundToInt

14

15	/**

16	 * The app's own vibration patterns, for moments that deserve more than a

17	 * plain click: a full bin, tamper, a lock, a pickup, the link dropping. Each

18	 * has its own rhythm, so the phone can be told apart in a pocket. Plain keys

19	 * keep Android's touch clicks in [Feel]; these patterns follow the Vibration

20	 * switch and the strength chosen in Settings.

21	 */

22	object Haptics {

23	    enum class Strength(val label: String, val scale: Float) {

24	        LIGHT("Light", 0.45f), NORMAL("Normal", 0.75f), STRONG("Strong", 1f),

25	    }

26

27	    @Volatile var strength = Strength.NORMAL

28

29	    /** One pattern, as (milliseconds, amplitude) steps. Amplitude runs 1 to 255; 0 is a pause. */

30	    enum class Buzz(vararg val steps: Pair<Int, Int>) {

31	        // Alerts: long and hard. Routine events: short and soft.

32	        FULL(300 to 255, 120 to 0, 300 to 255, 120 to 0, 300 to 255),

33	        TAMPER(90 to 255, 60 to 0, 90 to 255, 60 to 0, 90 to 255, 60 to 0, 400 to 255),

34	        SENSOR(150 to 200, 100 to 0, 300 to 200),

35	        LOCK(60 to 200, 50 to 0, 160 to 150),

36	        UNLOCK(160 to 150, 50 to 0, 60 to 200),

37	        PICKUP(40 to 120, 60 to 0, 40 to 160, 60 to 0, 120 to 220),

38	        DONE(30 to 120, 40 to 0, 30 to 160, 40 to 0, 30 to 200, 40 to 0, 140 to 255),

39	        INFO(40 to 120, 80 to 0, 60 to 170),

40

41	        // In the app only.

42	        DANGER(60 to 255, 70 to 0, 90 to 255),

43	        LOGO(40 to 80, 40 to 0, 40 to 150, 40 to 0, 70 to 255),

44	        BOOT(30 to 40, 30 to 80, 30 to 120, 30 to 160, 40 to 200),

45	        SAVED(20 to 170, 60 to 0, 20 to 170),

46	        DENIED(50 to 200, 50 to 0, 50 to 200),

47	        CALIB(20 to 140, 80 to 0, 20 to 140, 80 to 0, 20 to 140, 80 to 0, 80 to 220),

48	        SIM(60 to 90, 40 to 130, 40 to 170, 60 to 210),

49	        LINK_LOST(120 to 220, 100 to 150, 120 to 80, 140 to 40),

50	        LINK_UP(40 to 120, 60 to 0, 70 to 230),

51	        CLOUD(50 to 140, 60 to 0, 50 to 140, 60 to 0, 120 to 90),

52	        BIN_LOST(80 to 150, 80 to 60),

53	        BIN_BACK(40 to 70, 50 to 0, 50 to 160),

54	        PIN_WRONG(70 to 230, 50 to 0, 70 to 230, 50 to 0, 140 to 160),

55	        PIN_LOCKOUT(400 to 255, 120 to 0, 250 to 120),

56	        OP_IN(30 to 100, 40 to 0, 30 to 160, 40 to 0, 110 to 240),

57	        OP_OUT(110 to 200, 40 to 0, 30 to 90),

58	        CMD_OK(25 to 140, 40 to 0, 70 to 210),

59	        CMD_FAIL(90 to 200, 50 to 0, 90 to 160, 50 to 0, 160 to 90),

60	        BT_PAIRED(30 to 110, 40 to 0, 30 to 110, 40 to 0, 100 to 230),

61	        BT_WRONG(40 to 220, 40 to 0, 40 to 220, 40 to 0, 40 to 220),

62	        BT_BLOCKED(300 to 230, 150 to 0, 300 to 120),

63	        WIFI_JOINED(40 to 120, 30 to 0, 40 to 170, 30 to 0, 120 to 240),

64	        PIN_SET(20 to 150, 50 to 0, 20 to 150, 50 to 0, 60 to 200),

65	        WIFI_FAILED(120 to 220, 60 to 0, 120 to 160, 60 to 0, 200 to 90),

66	        BT_OFF(40 to 160, 40 to 110, 40 to 70, 120 to 40),

67	        DEMO(30 to 100, 40 to 0, 30 to 140, 40 to 0, 30 to 180, 40 to 0, 90 to 230),

68	        ;

69

70	        /** The same rhythm as a notification channel wants it: a pause first, then buzz and pause in turn. */

71	        fun channelPattern(): LongArray {

72	            val out = mutableListOf(0L)

73	            var on = false

74	            steps.forEach { (ms, amp) ->

75	                if ((amp > 0) == on) out[out.lastIndex] = out.last() + ms

76	                else { out.add(ms.toLong()); on = amp > 0 }

77	            }

78	            return out.toLongArray()

79	        }

80	    }

81

82	    private var vibrator: Vibrator? = null

83

84	    fun init(context: Context) {

85	        if (vibrator != null) return

86	        val app = context.applicationContext

87	        vibrator = runCatching {

88	            if (Build.VERSION.SDK_INT >= 31) app.getSystemService(VibratorManager::class.java)?.defaultVibrator

89	            else @Suppress("DEPRECATION") (app.getSystemService(Context.VIBRATOR_SERVICE) as? Vibrator)

90	        }.getOrNull()

91	    }

92

93	    /** Plays one pattern, unless vibration is switched off in Settings. */

94	    fun play(buzz: Buzz) {

95	        if (!Feel.haptics) return

96	        val v = vibrator ?: return

97	        if (!v.hasVibrator()) return

98	        val scale = strength.scale

99	        val timings = LongArray(buzz.steps.size) { buzz.steps[it].first.toLong() }

100	        val amps = IntArray(buzz.steps.size) {

101	            val a = buzz.steps[it].second

102	            if (a == 0) 0 else (a * scale).roundToInt().coerceIn(1, 255)

103	        }

104	        try {

105	            v.vibrate(VibrationEffect.createWaveform(timings, amps, -1))

106	        } catch (_: Exception) {

107	            // No motor: the sound still plays.

108	        }

109	    }

110

111	    /** The pattern that goes with a sound, for the sounds that have one. */

112	    fun forSfx(sfx: Sfx): Buzz? = when (sfx) {

113	        Sfx.DANGER -> Buzz.DANGER

114	        Sfx.LOGO -> Buzz.LOGO

115	        Sfx.DENIED -> Buzz.DENIED

116	        Sfx.SAVED -> Buzz.SAVED

117	        Sfx.BOOT -> Buzz.BOOT

118	        Sfx.LINK_LOST -> Buzz.LINK_LOST

119	        Sfx.LINK_UP -> Buzz.LINK_UP

120	        Sfx.CLOUD -> Buzz.CLOUD

121	        Sfx.LOCK -> Buzz.LOCK

122	        Sfx.UNLOCK -> Buzz.UNLOCK

123	        Sfx.PICKUP -> Buzz.PICKUP

124	        Sfx.DONE -> Buzz.DONE

125	        Sfx.CALIB -> Buzz.CALIB

126	        Sfx.SIM -> Buzz.SIM

127	        Sfx.BIN_LOST -> Buzz.BIN_LOST

128	        Sfx.BIN_BACK -> Buzz.BIN_BACK

129	        Sfx.PIN_WRONG -> Buzz.PIN_WRONG

130	        Sfx.PIN_LOCKOUT -> Buzz.PIN_LOCKOUT

131	        Sfx.OP_IN -> Buzz.OP_IN

132	        Sfx.OP_OUT -> Buzz.OP_OUT

133	        Sfx.CMD_OK -> Buzz.CMD_OK

134	        Sfx.CMD_FAIL -> Buzz.CMD_FAIL

135	        Sfx.BT_PAIRED -> Buzz.BT_PAIRED

136	        Sfx.BT_WRONG -> Buzz.BT_WRONG

137	        Sfx.BT_BLOCKED -> Buzz.BT_BLOCKED

138	        Sfx.WIFI_JOINED -> Buzz.WIFI_JOINED

139	        Sfx.PIN_SET -> Buzz.PIN_SET

140	        Sfx.WIFI_FAILED -> Buzz.WIFI_FAILED

141	        Sfx.BT_OFF -> Buzz.BT_OFF

142	        Sfx.DEMO -> Buzz.DEMO

143	        Sfx.ALERT_FULL -> Buzz.FULL

144	        Sfx.ALERT_TAMPER -> Buzz.TAMPER

145	        Sfx.ALERT_SENSOR -> Buzz.SENSOR

146	        Sfx.ALERT_LOCK -> Buzz.LOCK

147	        Sfx.ALERT_PICKUP -> Buzz.DONE

148	        Sfx.ALERT_INFO -> Buzz.INFO

149	        else -> null

150	    }

151	}

Android App/app/src/main/java/smartbin/notify/Feel.kt

1	// ============================================================================

2	// notify/Feel.kt - touch feedback (key sounds and haptic clicks)

3	// sounds and haptics are the Sound and Vibration switches on the App settings

4	// page. Sound off also mutes in-app event sounds, but not notification sounds,

5	// which follow the phone's own notification volume.

6	// ============================================================================

7	package smartbin.notify

8

9	import android.os.Build

10	import android.view.HapticFeedbackConstants

11	import android.view.View

12

13	/**

14	 * How the app feels under the finger: a small vibration and a soft sound on

15	 * keys and switches. Both can be turned off in Settings. Alert sounds are

16	 * separate; those belong to Android's notification settings.

17	 */

18	object Feel {

19	    @Volatile var sounds = true

20	    @Volatile var haptics = true

21

22	    /** Any control: its own sound and a vibration that fits it. */

23	    fun play(view: View, sfx: Sfx) {

24	        // Events have patterns of their own; plain keys use Android's touch clicks.

25	        val buzz = Haptics.forSfx(sfx)

26	        if (buzz != null) Haptics.play(buzz)

27	        else if (haptics) view.performHapticFeedback(hapticFor(sfx))

28	        Sounds.play(sfx)

29	    }

30

31	    /** A sound and its vibration with no key involved, such as a bin going full. */

32	    fun event(sfx: Sfx) {

33	        Haptics.forSfx(sfx)?.let(Haptics::play)

34	        Sounds.play(sfx)

35	    }

36

37	    private fun hapticFor(sfx: Sfx): Int {

38	        val v = Build.VERSION.SDK_INT

39	        val key = HapticFeedbackConstants.VIRTUAL_KEY

40	        return when (sfx) {

41	            Sfx.CONFIRM -> if (v >= 30) HapticFeedbackConstants.CONFIRM else key

42	            Sfx.SWITCH_ON -> if (v >= 34) HapticFeedbackConstants.TOGGLE_ON else HapticFeedbackConstants.KEYBOARD_TAP

43	            Sfx.SWITCH_OFF -> if (v >= 34) HapticFeedbackConstants.TOGGLE_OFF else HapticFeedbackConstants.KEYBOARD_TAP

44	            Sfx.STEP_UP, Sfx.STEP_DOWN, Sfx.CHOICE -> if (v >= 30) HapticFeedbackConstants.SEGMENT_TICK else HapticFeedbackConstants.KEYBOARD_TAP

45	            Sfx.BACK -> HapticFeedbackConstants.CLOCK_TICK

46	            Sfx.PIN_KEY, Sfx.TAB, Sfx.THEME_LIGHT, Sfx.THEME_DARK, Sfx.OPEN -> HapticFeedbackConstants.KEYBOARD_TAP

47	            else -> key

48	        }

49	    }

50

51	    /** Something weighty, such as a long hold being recognised. */

52	    fun thump(view: View) {

53	        if (haptics) view.performHapticFeedback(HapticFeedbackConstants.LONG_PRESS)

54	    }

55	}

Android App/app/src/main/java/smartbin/notify/Alerts.kt

1	// ============================================================================

2	// notify/Alerts.kt - Android notification channels and posting

3	// Channel ids end in _v1 because Android never lets an app change a channel's

4	// sound after it is created. To ship a new alert sound, bump the id (_v2);

5	// otherwise phones that already have the channel keep the old sound.

6	// Debug tip: if an alert shows in the app but no notification appears, check

7	// Android Settings > Apps > SmartBin > Notifications for that channel first.

8	// ============================================================================

9	package smartbin.notify

10

11	import android.annotation.SuppressLint

12	import android.app.NotificationChannel

13	import android.app.NotificationManager

14	import android.app.PendingIntent

15	import android.content.Context

16	import android.content.Intent

17	import android.media.AudioAttributes

18	import android.net.Uri

19	import android.os.Build

20	import androidx.core.app.NotificationCompat

21	import androidx.core.app.NotificationManagerCompat

22	import smartbin.MainActivity

23	import smartbin.R

24	import smartbin.data.AlertRow

25	import smartbin.data.AlertType

26

27	/**

28	 * Android notifications for alerts, each kind with its own sound and its own

29	 * vibration. On Android a notification's sound belongs to its channel, so

30	 * every kind has a channel. That is also what lets the right sound play while

31	 * the app is closed, and lets each kind be changed or silenced in Android's

32	 * settings.

33	 */

34	object Alerts {

35	    const val CHANNEL_WATCH = "sb_watch"

36	    const val ACTION_COLLECT = "smartbin.START_PICKUP"

37	    const val EXTRA_BIN = "bin"

38	    const val EXTRA_NOTIFICATION = "notification"

39

40	    private class Kind(val channel: String, val name: String, val about: String, val sfx: Sfx, val importance: Int, val buzz: Haptics.Buzz)

41

42	    // One channel per alert type: id, name shown in Android settings, its

43	    // description, the in-app sound, importance (HIGH makes a heads-up

44	    // banner) and the vibration rhythm.

45	    private val kinds = mapOf(

46	        AlertType.FULL to Kind("sb_full_v1", "Bin full", "A bin passed its fill limit. Has a Start pickup button.", Sfx.ALERT_FULL, NotificationManager.IMPORTANCE_HIGH, Haptics.Buzz.FULL),

47	        AlertType.TAMPER to Kind("sb_tamper_v1", "Tamper", "A bin was moved or tilted.", Sfx.ALERT_TAMPER, NotificationManager.IMPORTANCE_HIGH, Haptics.Buzz.TAMPER),

48	        AlertType.SENSOR to Kind("sb_sensor_v1", "Sensor over its limit", "Temperature, humidity or air quality passed its limit.", Sfx.ALERT_SENSOR, NotificationManager.IMPORTANCE_HIGH, Haptics.Buzz.SENSOR),

49	        AlertType.LOCK to Kind("sb_lock_v1", "Lock and unlock", "A bin was locked or unlocked.", Sfx.ALERT_LOCK, NotificationManager.IMPORTANCE_DEFAULT, Haptics.Buzz.LOCK),

50	        AlertType.PICKUP to Kind("sb_pickup_v1", "Pickups", "A bin was emptied.", Sfx.ALERT_PICKUP, NotificationManager.IMPORTANCE_DEFAULT, Haptics.Buzz.DONE),

51	        AlertType.INFO to Kind("sb_info_v1", "Calibration and settings", "A bin was calibrated or its settings were saved.", Sfx.ALERT_INFO, NotificationManager.IMPORTANCE_LOW, Haptics.Buzz.INFO),

52	    )

53

54	    /** The sound the app plays for a kind of alert while it is on screen. */

55	    fun sfxFor(type: AlertType): Sfx = kinds.getValue(type).sfx

56

57	    fun ensureChannels(context: Context) {

58	        if (Build.VERSION.SDK_INT < 26) return

59	        val manager = context.getSystemService(NotificationManager::class.java) ?: return

60	        val audio = AudioAttributes.Builder()

61	            .setUsage(AudioAttributes.USAGE_NOTIFICATION)

62	            .setContentType(AudioAttributes.CONTENT_TYPE_SONIFICATION)

63	            .build()

64	        kinds.values.forEach { k ->

65	            manager.createNotificationChannel(

66	                NotificationChannel(k.channel, k.name, k.importance).apply {

67	                    description = k.about

68	                    setSound(Uri.parse("android.resource://${context.packageName}/${k.sfx.res}"), audio)

69	                    enableVibration(true)

70	                    vibrationPattern = k.buzz.channelPattern()

71	                }

72	            )

73	        }

74	        manager.createNotificationChannel(

75	            NotificationChannel(CHANNEL_WATCH, "Watching the bins", NotificationManager.IMPORTANCE_MIN).apply {

76	                description = "Keeps reading the bins so alerts arrive while the app is closed."

77	                setShowBadge(false)

78	            }

79	        )

80	    }

81

82	    fun allowed(context: Context) = NotificationManagerCompat.from(context).areNotificationsEnabled()

83

84	    // Posts one alert. The alert row id doubles as the notification id, so a

85	    // later alert never replaces an earlier one. FULL alerts also get a

86	    // "Start pickup" action handled by ActionReceiver.

87	    @SuppressLint("MissingPermission")

88	    fun post(context: Context, row: AlertRow, type: AlertType, binId: String) {

89	        if (!allowed(context)) return

90	        val k = kinds.getValue(type)

91	        val id = (row.id % Int.MAX_VALUE).toInt()

92	        val open = PendingIntent.getActivity(

93	            context, id,

94	            Intent(context, MainActivity::class.java).addFlags(Intent.FLAG_ACTIVITY_SINGLE_TOP or Intent.FLAG_ACTIVITY_CLEAR_TOP)

95	                .putExtra(MainActivity.EXTRA_OPEN, "overview"),

96	            PendingIntent.FLAG_UPDATE_CURRENT or PendingIntent.FLAG_IMMUTABLE,

97	        )

98	        val n = NotificationCompat.Builder(context, k.channel)

99	            .setSmallIcon(R.drawable.ic_stat_bin)

100	            .setContentTitle(row.title)

101	            .setContentText(row.meta)

102	            .setStyle(NotificationCompat.BigTextStyle().bigText(row.meta))

103	            .setContentIntent(open)

104	            .setAutoCancel(true)

105	            .setWhen(row.at)

106	            .setShowWhen(true)

107	            .setPriority(if (k.importance >= NotificationManager.IMPORTANCE_HIGH) NotificationCompat.PRIORITY_HIGH else NotificationCompat.PRIORITY_DEFAULT)

108	            .setCategory(NotificationCompat.CATEGORY_ALARM.takeIf { type == AlertType.TAMPER } ?: NotificationCompat.CATEGORY_STATUS)

109	            // Phones older than Android 8 have no channels, so the sound goes on the notification itself.

110	            .setSound(Uri.parse("android.resource://${context.packageName}/${k.sfx.res}"))

111	            .setVibrate(k.buzz.channelPattern())

112	        if (type == AlertType.FULL) {

113	            val pickup = PendingIntent.getBroadcast(

114	                context, id,

115	                Intent(context, ActionReceiver::class.java).setAction(ACTION_COLLECT).putExtra(EXTRA_BIN, binId).putExtra(EXTRA_NOTIFICATION, id),

116	                PendingIntent.FLAG_UPDATE_CURRENT or PendingIntent.FLAG_IMMUTABLE,

117	            )

118	            n.addAction(0, "Start pickup", pickup)

119	        }

120	        n.addAction(0, "Open", open)

121	        runCatching { NotificationManagerCompat.from(context).notify(id, n.build()) }

122	    }

123

124	    /** The quiet notification a foreground service must show. */

125	    fun watching(context: Context) = NotificationCompat.Builder(context, CHANNEL_WATCH)

126	        .setSmallIcon(R.drawable.ic_stat_bin)

127	        .setContentTitle("Watching your bins")

128	        .setContentText("Alerts arrive while the app is closed.")

129	        .setOngoing(true)

130	        .setPriority(NotificationCompat.PRIORITY_MIN)

131	        .setContentIntent(

132	            PendingIntent.getActivity(

133	                context, 0, Intent(context, MainActivity::class.java),

134	                PendingIntent.FLAG_UPDATE_CURRENT or PendingIntent.FLAG_IMMUTABLE,

135	            )

136	        )

137	        .build()

138	}

Android App/app/src/main/java/smartbin/notify/WatchService.kt

1	// ============================================================================

2	// notify/WatchService.kt - foreground service that keeps Hub polling

3	// Shows the quiet "watching" notification (id 7001, channel sb_watch). Without

4	// it, Android may stop the process a few minutes after the app leaves the

5	// screen and background alerts stop. Battery savers on some phones still kill

6	// it; setting the app's battery use to "Unrestricted" fixes that.

7	// ============================================================================

8	package smartbin.notify

9

10	import android.app.Service

11	import android.content.Context

12	import android.content.Intent

13	import android.content.pm.ServiceInfo

14	import android.os.Build

15	import android.os.IBinder

16	import androidx.core.content.ContextCompat

17	import smartbin.data.Hub

18

19	/**

20	 * Keeps the app's process alive while it is in the background, so [Hub] goes

21	 * on reading the bins every few seconds and alerts still arrive. It runs only

22	 * while push alerts are on, the app is off screen, and the user had reached

23	 * the main screens. Swiping the app away ends it, and with it the alerts.

24	 */

25	class WatchService : Service() {

26	    override fun onBind(intent: Intent?): IBinder? = null

27

28	    override fun onStartCommand(intent: Intent?, flags: Int, startId: Int): Int {

29	        // Android restarting a killed app's service is not the user opening it.

30	        if (!Hub.inMain) {

31	            stopSelf()

32	            return START_NOT_STICKY

33	        }

34	        Hub.init(this)

35	        Alerts.ensureChannels(this)

36	        val note = Alerts.watching(this)

37	        try {

38	            if (Build.VERSION.SDK_INT >= 29) startForeground(ID, note, ServiceInfo.FOREGROUND_SERVICE_TYPE_CONNECTED_DEVICE)

39	            else startForeground(ID, note)

40	        } catch (_: Exception) {

41	            // Android refused a background start; alerts then arrive only while the app is open.

42	            stopSelf()

43	        }

44	        return START_NOT_STICKY

45	    }

46

47	    /** Swiped away from recent apps: the app counts as closed, so no more alerts. */

48	    override fun onTaskRemoved(rootIntent: Intent?) {

49	        Hub.inMain = false

50	        stopSelf()

51	        super.onTaskRemoved(rootIntent)

52	    }

53

54	    companion object {

55	        private const val ID = 7001

56

57	        fun start(context: Context) {

58	            runCatching { ContextCompat.startForegroundService(context, Intent(context, WatchService::class.java)) }

59	        }

60

61	        fun stop(context: Context) {

62	            runCatching { context.stopService(Intent(context, WatchService::class.java)) }

63	        }

64	    }

65	}

Android App/app/src/main/java/smartbin/notify/ActionReceiver.kt

1	// ============================================================================

2	// notify/ActionReceiver.kt - handles the "Start pickup" button on a notification

3	// Sends COLLECT through Hub.command (local Wi-Fi first, then Firebase or

4	// ThingSpeak TalkBack if the bin is off the Wi-Fi) without opening

5	// the app, then shows a Toast with the result. goAsync() keeps the receiver

6	// alive while the network call runs. If the Toast says the bin did not answer,

7	// the same command from inside the app will fail the same way.

8	// ============================================================================

9	package smartbin.notify

10

11	import android.content.BroadcastReceiver

12	import android.content.Context

13	import android.content.Intent

14	import android.widget.Toast

15	import androidx.core.app.NotificationManagerCompat

16	import kotlinx.coroutines.CoroutineScope

17	import kotlinx.coroutines.Dispatchers

18	import kotlinx.coroutines.launch

19	import smartbin.data.Hub

20

21	/** "Start pickup" on a full-bin notification: puts the bin in pickup mode without opening the app. */

22	class ActionReceiver : BroadcastReceiver() {

23	    override fun onReceive(context: Context, intent: Intent) {

24	        if (intent.action != Alerts.ACTION_COLLECT) return

25	        val bin = intent.getStringExtra(Alerts.EXTRA_BIN) ?: return

26	        val note = intent.getIntExtra(Alerts.EXTRA_NOTIFICATION, 0)

27	        val pending = goAsync()

28	        Hub.init(context)

29	        CoroutineScope(Dispatchers.Main).launch {

30	            try {

31	                val problem = Hub.command(bin, "COLLECT")

32	                val name = Hub.bin(bin)?.name ?: "The bin"

33	                Toast.makeText(context, problem ?: "Pickup started on $name.", Toast.LENGTH_SHORT).show()

34	                if (problem == null) {

35	                    NotificationManagerCompat.from(context).cancel(note)

36	                    Feel.event(Sfx.PICKUP)

37	                }

38	            } finally {

39	                pending.finish()

40	            }

41	        }

42	    }

43	}

Android App/app/src/main/java/smartbin/ui/Theme.kt

1	// ============================================================================

2	// ui/Theme.kt - colors, fonts and the light and dark palettes

3	// SbColors holds every color token, named after the mockup's CSS variables.

4	// The palette also changes with the system mode (NORMAL, SIM, CALIB, DUAL for

5	// both, ALERT), which tints the page so an operator can tell modes apart.

6	// ============================================================================

7	package smartbin.ui

8

9	import androidx.compose.animation.animateColorAsState

10	import androidx.compose.animation.core.tween

11	import androidx.compose.runtime.Composable

12	import androidx.compose.runtime.Immutable

13	import androidx.compose.runtime.getValue

14	import androidx.compose.runtime.staticCompositionLocalOf

15	import androidx.compose.ui.graphics.Brush

16	import androidx.compose.ui.graphics.Color

17	import androidx.compose.ui.text.font.Font

18	import androidx.compose.ui.text.font.FontFamily

19	import androidx.compose.ui.text.font.FontWeight

20	import smartbin.R

21	import smartbin.data.Mode

22

23	enum class ThemeMode { AUTO, LIGHT, DARK }

24

25	/** ThingSpeak's colour on the link chip and in Connection: blue, next to Wi-Fi green and Firebase orange. */

26	val SbColors.ts: Color get() = humid

27

28	/**

29	 * The design tokens of the mockup, the .ph[data-th] and .ph[data-mode]

30	 * blocks of smartbin-app-mockup v3.html. The accent and the page tint follow

31	 * the app mode: green for normal, violet for simulation, teal for

32	 * calibration, blue for both, red for an alert.

33	 */

34	@Immutable

35	data class SbColors(

36	    val isDark: Boolean,

37	    val base: Color,

38	    val tint: Color,

39	    val shL: Color,

40	    val shD: Color,

41	    val ink: Color,

42	    val ink2: Color,

43	    val well: Color,

44	    val ok: Color,

45	    val warn: Color,

46	    val bad: Color,

47	    val sim: Color,

48	    val cal: Color,

49	    val accent: Color,

50	    val accentSoft: Color,

51	    val onAccent: Color,

52	    val simSoft: Color,

53	    val calSoft: Color,

54	    val temp: Color,

55	    val humid: Color,

56	    val weight: Color,

57	    val gas: Color,

58	    val press: Color,

59	    val fill: Color,

60	    val bt: Color,

61	    val lid: Color,

62	    val motion: Color,

63	    val link: Color,

64	    val cloud: Color,

65	    val tamper: Color,

66	    val lvCool: Color,

67	    val lvLow: Color,

68	    val lvMid: Color,

69	    val lvHigh: Color,

70	    val lvCrit: Color,

71	    /** The accent bar and mode chip: one colour, or violet to blue to teal in SIM + CAL. */

72	    val bar: List<Color>,

73	) {

74	    val barBrush: Brush get() = if (bar.size == 1) Brush.linearGradient(listOf(bar[0], bar[0])) else Brush.horizontalGradient(bar)

75	    /** Text on the brutal coloured banners: white on light, near black on dark. */

76	    val onBanner: Color get() = if (isDark) Color(0xFF13100D) else Color.White

77	}

78

79	private val Light = SbColors(

80	    isDark = false,

81	    base = Color(0xFFE4DCD2), tint = Color(0xFFE4DCD2), shL = Color(0xFFFFFBF5), shD = Color(0xFFBEB2A4),

82	    ink = Color(0xFF2A241E), ink2 = Color(0xFF7C7265), well = Color(0xFFDED5CA),

83	    ok = Color(0xFF1E8E3E), warn = Color(0xFFD98A1F), bad = Color(0xFFD93025), sim = Color(0xFF7A45B5), cal = Color(0xFF0D8A8A),

84	    accent = Color(0xFF1E8E3E), accentSoft = Color(0x381E8E3E), onAccent = Color.White,

85	    simSoft = Color(0x4D7A45B5), calSoft = Color(0x4D0D8A8A),

86	    temp = Color(0xFFC2591B), humid = Color(0xFF1F6FA8), weight = Color(0xFF7A5A2E), gas = Color(0xFF6B4FB8), press = Color(0xFFB03A6F),

87	    fill = Color(0xFF1E7A52), bt = Color(0xFF1A62C4), lid = Color(0xFFB07415), motion = Color(0xFF0E7F9E), link = Color(0xFF1E8E3E),

88	    cloud = Color(0xFFD9631E), tamper = Color(0xFFD93025),

89	    lvCool = Color(0xFF1F6FA8), lvLow = Color(0xFF1E8E3E), lvMid = Color(0xFFB88410), lvHigh = Color(0xFFD9641F), lvCrit = Color(0xFFD93025),

90	    bar = listOf(Color(0xFF1E8E3E)),

91	)

92

93	private val Dark = SbColors(

94	    isDark = true,

95	    base = Color(0xFF2B2622), tint = Color(0xFF2B2622), shL = Color(0xFF3B342E), shD = Color(0xFF1A1613),

96	    ink = Color(0xFFEFE7DC), ink2 = Color(0xFFA2978B), well = Color(0xFF262220),

97	    ok = Color(0xFF4FBF6E), warn = Color(0xFFE8A94A), bad = Color(0xFFFF6B5E), sim = Color(0xFFB98BE8), cal = Color(0xFF3FC7C2),

98	    accent = Color(0xFF4FBF6E), accentSoft = Color(0x384FBF6E), onAccent = Color(0xFF13100D),

99	    simSoft = Color(0x57B98BE8), calSoft = Color(0x523FC7C2),

100	    temp = Color(0xFFF0A368), humid = Color(0xFF6CB8E8), weight = Color(0xFFD6B98C), gas = Color(0xFFB596F0), press = Color(0xFFE58AB4),

101	    fill = Color(0xFF4FC48D), bt = Color(0xFF79AEF5), lid = Color(0xFFE8B45C), motion = Color(0xFF5FD0E8), link = Color(0xFF4FBF6E),

102	    cloud = Color(0xFFF29A5C), tamper = Color(0xFFFF6B5E),

103	    lvCool = Color(0xFF6CB8E8), lvLow = Color(0xFF4FBF6E), lvMid = Color(0xFFE8C45A), lvHigh = Color(0xFFF0A368), lvCrit = Color(0xFFFF6B5E),

104	    bar = listOf(Color(0xFF4FBF6E)),

105	)

106

107	/** The tokens for one theme and one app mode. */

108	fun sbColors(dark: Boolean, mode: Mode): SbColors {

109	    val c = if (dark) Dark else Light

110	    return when (mode) {

111	        Mode.NORMAL -> c

112	        Mode.SIM -> c.copy(

113	            accent = c.sim, bar = listOf(c.sim),

114	            accentSoft = if (dark) Color(0x38B98BE8) else Color(0x387A45B5),

115	            tint = if (dark) Color(0xFF2C2530) else Color(0xFFE2DAE1),

116	        )

117	        Mode.CALIB -> c.copy(

118	            accent = c.cal, bar = listOf(c.cal),

119	            accentSoft = if (dark) Color(0x333FC7C2) else Color(0x330D8A8A),

120	            tint = if (dark) Color(0xFF222B2A) else Color(0xFFDBE3E0),

121	        )

122	        Mode.DUAL -> {

123	            val mid = if (dark) Color(0xFF7CA9D5) else Color(0xFF43679F)

124	            c.copy(

125	                accent = mid, bar = listOf(c.sim, mid, c.cal),

126	                accentSoft = if (dark) Color(0x3D7CA9D5) else Color(0x3D43679F),

127	                tint = if (dark) Color(0xFF262A33) else Color(0xFFDEDDE6),

128	            )

129	        }

130	        Mode.ALERT -> c.copy(

131	            accent = c.bad, bar = listOf(c.bad),

132	            accentSoft = if (dark) Color(0x33FF6B5E) else Color(0x33D93025),

133	            tint = if (dark) Color(0xFF302422) else Color(0xFFEADCD5),

134	        )

135	    }

136	}

137

138	/** Colours that change with the mode ease across, as the mockup's background does in 0.5 s. */

139	@Composable

140	fun animated(c: SbColors): SbColors {

141	    val spec = tween<Color>(500)

142	    val accent by animateColorAsState(c.accent, spec, label = "accent")

143	    val soft by animateColorAsState(c.accentSoft, spec, label = "soft")

144	    val tint by animateColorAsState(c.tint, spec, label = "tint")

145	    return c.copy(accent = accent, accentSoft = soft, tint = tint)

146	}

147

148	val LocalSb = staticCompositionLocalOf { Light }

149

150	object Sb {

151	    val c: SbColors

152	        @Composable get() = LocalSb.current

153	}

154

155	val Inter = FontFamily(

156	    Font(R.font.inter_regular, FontWeight.Normal),

157	    Font(R.font.inter_regular, FontWeight.Medium),

158	    Font(R.font.inter_semibold, FontWeight.SemiBold),

159	    Font(R.font.inter_bold, FontWeight.Bold),

160	    Font(R.font.inter_extrabold, FontWeight.ExtraBold),

161	    Font(R.font.inter_extrabold, FontWeight.Black),

162	)

163

164	/**

165	 * Each bin has its own colour. Twelve hues spread round the wheel, red left

166	 * out because red means a limit, ordered so neighbours differ. Bins 13 to 24

167	 * reuse them one step darker and 25 to 36 one step lighter, the same formula

168	 * as the mockup.

169	 */

170	private val HUES = intArrayOf(153, 287, 47, 207, 100, 260, 20, 180, 313, 73, 233, 127)

171	private val ODD = intArrayOf(0, 1, 0, 1, 0, 1, 1, 0, 0, 1, 1, 1)

172

173	fun binColor(i: Int, dark: Boolean): Color {

174	    val t = (i / 12) % 3

175	    val r = i / 36

176	    val h = (HUES[i % 12] + r * 11) % 360

177	    val odd = ODD[i % 12] == 1

178	    val l = (if (dark) intArrayOf(64, 78, 50)[t] else intArrayOf(40, 27, 54)[t]) + if (odd) (if (dark) -9 else 8) else 0

179	    val s = if (dark) intArrayOf(70, 60, 72)[t] else intArrayOf(66, 62, 72)[t]

180	    return Color.hsl(h.toFloat(), s / 100f, l / 100f)

181	}

182

183	/** color-mix(in srgb, x p%, transparent) */

184	fun Color.soft(p: Float) = copy(alpha = alpha * p)

185

186	/** color-mix(in srgb, a p%, b) */

187	fun mix(a: Color, p: Float, b: Color) = Color(

188	    red = a.red * p + b.red * (1 - p),

189	    green = a.green * p + b.green * (1 - p),

190	    blue = a.blue * p + b.blue * (1 - p),

191	    alpha = 1f,

192	)

Android App/app/src/main/java/smartbin/ui/Neu.kt

1	// ============================================================================

2	// ui/Neu.kt - the neumorphic and neo-brutalist surfaces

3	// Modifiers that draw the soft raised and sunk shadows (raised, sunk, nm,

4	// bump, well) and the hard offset block (brutal). Shadow sizes match the

5	// mockup's --o1/--o2/--i1/--i2 values so the app and dashboard look alike.

6	// ============================================================================

7	package smartbin.ui

8

9	import android.graphics.BlurMaskFilter

10	import androidx.compose.foundation.background

11	import androidx.compose.foundation.border

12	import androidx.compose.foundation.shape.RoundedCornerShape

13	import androidx.compose.ui.Modifier

14	import androidx.compose.ui.draw.clip

15	import androidx.compose.ui.draw.drawBehind

16	import androidx.compose.ui.draw.drawWithCache

17	import androidx.compose.ui.geometry.CornerRadius

18	import androidx.compose.ui.geometry.Offset

19	import androidx.compose.ui.geometry.RoundRect

20	import androidx.compose.ui.geometry.Size

21	import androidx.compose.ui.graphics.Brush

22	import androidx.compose.ui.graphics.Color

23	import androidx.compose.ui.graphics.Paint

24	import androidx.compose.ui.graphics.Path

25	import androidx.compose.ui.graphics.drawscope.Stroke

26	import androidx.compose.ui.graphics.drawscope.clipPath

27	import androidx.compose.ui.graphics.drawscope.drawIntoCanvas

28	import androidx.compose.ui.graphics.nativeCanvas

29	import androidx.compose.ui.graphics.toArgb

30	import androidx.compose.ui.unit.Dp

31	import androidx.compose.ui.unit.dp

32

33	/*

34	 * The mockup's two shadow systems. The soft pair (light above left, dark

35	 * below right) is the Neumorphism depth: --o1 and --o2 raise a surface,

36	 * --i1 and --i2 sink it. The hard offset block with a 2 dp ink edge is the

37	 * Neo-Brutalism part, used for banners, alarms and danger keys.

38	 */

39

40	/** --o1 and --o2: offset and blur in px. */

41	enum class Out(val off: Dp, val blur: Dp) { O1(4.dp, 9.dp), O2(7.dp, 15.dp), POP(3.dp, 8.dp) }

42

43	/** --i1 and --i2 */

44	enum class In(val off: Dp, val blur: Dp) { I1(4.dp, 8.dp), I2(6.dp, 12.dp) }

45

46	// A CSS blur of b px is a Gaussian with sigma b / 2. BlurMaskFilter takes a

47	// radius where sigma is about 0.577 * radius, so the factor is close to 0.85.

48	private const val BLUR_K = 0.85f

49

50	private fun blurPaint(color: Color, blurPx: Float, stroke: Float = 0f) =

51	    Paint().asFrameworkPaint().apply {

52	        isAntiAlias = true

53	        this.color = color.toArgb()

54	        if (stroke > 0f) {

55	            style = android.graphics.Paint.Style.STROKE

56	            strokeWidth = stroke

57	        }

58	        if (blurPx > 0f) maskFilter = BlurMaskFilter(blurPx, BlurMaskFilter.Blur.NORMAL)

59	    }

60

61	/**

62	 * Raised: the soft pair, plus an optional ring (box-shadow 0 0 0 Npx) drawn

63	 * outside the edge. Put it before the clip and the background.

64	 */

65	fun Modifier.raised(c: SbColors, radius: Dp, out: Out = Out.O1, ring: Color? = null, ringWidth: Dp = 2.dp): Modifier = drawWithCache {

66	    val r = radius.toPx().coerceAtMost(size.minDimension / 2f)

67	    val off = out.off.toPx()

68	    val light = blurPaint(c.shL, out.blur.toPx() * BLUR_K)

69	    val dark = blurPaint(c.shD, out.blur.toPx() * BLUR_K)

70	    val rw = ringWidth.toPx()

71	    onDrawBehind {

72	        drawIntoCanvas { canvas ->

73	            val n = canvas.nativeCanvas

74	            n.drawRoundRect(-off, -off, size.width - off, size.height - off, r, r, light)

75	            n.drawRoundRect(off, off, size.width + off, size.height + off, r, r, dark)

76	        }

77	        if (ring != null && ring.alpha > 0f) {

78	            drawRoundRect(

79	                ring, topLeft = Offset(-rw / 2, -rw / 2), size = Size(size.width + rw, size.height + rw),

80	                cornerRadius = CornerRadius(r + rw / 2, r + rw / 2), style = Stroke(rw),

81	            )

82	        }

83	    }

84	}

85

86	/** Sunk: the inner soft pair. Put it after the background. */

87	fun Modifier.sunk(c: SbColors, radius: Dp, inset: In = In.I1): Modifier = drawWithCache {

88	    val r = radius.toPx().coerceAtMost(size.minDimension / 2f)

89	    val off = inset.off.toPx()

90	    val clip = Path().apply { addRoundRect(RoundRect(0f, 0f, size.width, size.height, CornerRadius(r, r))) }

91	    val dark = blurPaint(c.shD, inset.blur.toPx() * BLUR_K, stroke = off * 2)

92	    val light = blurPaint(c.shL, inset.blur.toPx() * BLUR_K, stroke = off * 2)

93	    onDrawBehind {

94	        clipPath(clip) {

95	            drawIntoCanvas { canvas ->

96	                val n = canvas.nativeCanvas

97	                n.drawRoundRect(off, off, size.width + off, size.height + off, r, r, dark)

98	                n.drawRoundRect(-off, -off, size.width - off, size.height - off, r, r, light)

99	            }

100	        }

101	    }

102	}

103

104	/** A ring outside the edge with nothing else, for a surface that is already sunk. */

105	fun Modifier.ring(color: Color, radius: Dp, width: Dp = 2.dp): Modifier = drawBehind {

106	    val rw = width.toPx()

107	    val r = radius.toPx().coerceAtMost(size.minDimension / 2f)

108	    drawRoundRect(

109	        color, topLeft = Offset(-rw / 2, -rw / 2), size = Size(size.width + rw, size.height + rw),

110	        cornerRadius = CornerRadius(r + rw / 2, r + rw / 2), style = Stroke(rw),

111	    )

112	}

113

114	/** The brutal block: a hard offset shadow and a 2 dp ink edge. */

115	fun Modifier.brutal(c: SbColors, radius: Dp, fill: Color, off: Dp = 4.dp, edge: Color = c.ink): Modifier {

116	    val shape = RoundedCornerShape(radius)

117	    return this

118	        .drawBehind {

119	            val r = radius.toPx()

120	            drawRoundRect(edge, topLeft = Offset(off.toPx(), off.toPx()), size = size, cornerRadius = CornerRadius(r, r))

121	        }

122	        .clip(shape)

123	        .background(fill)

124	        .border(2.dp, edge, shape)

125	}

126

127	/** .nm: a raised card, a shade lighter toward the light. */

128	fun Modifier.nm(c: SbColors, radius: Dp = 18.dp, out: Out = Out.O2, ring: Color? = null, ringWidth: Dp = 2.dp): Modifier {

129	    val shape = RoundedCornerShape(radius)

130	    return this

131	        .raised(c, radius, out, ring, ringWidth)

132	        .clip(shape)

133	        .background(c.base)

134	        .background(

135	            Brush.linearGradient(

136	                0f to c.shL.copy(alpha = 0.42f), 0.58f to Color.Transparent,

137	                start = Offset(0f, 0f), end = Offset(Float.POSITIVE_INFINITY, Float.POSITIVE_INFINITY),

138	            )

139	        )

140	}

141

142	/** A flat raised surface: --o1 on the page colour. */

143	fun Modifier.bump(c: SbColors, radius: Dp, out: Out = Out.O1, ring: Color? = null, fill: Color = c.base): Modifier =

144	    this.raised(c, radius, out, ring).clip(RoundedCornerShape(radius)).background(fill)

145

146	/** A well: the page sunk in, for inputs, steppers and readouts. */

147	fun Modifier.well(c: SbColors, radius: Dp, inset: In = In.I1, fill: Color = c.well): Modifier =

148	    this.clip(RoundedCornerShape(radius)).background(fill).sunk(c, radius, inset)

149

150	/** The page: two soft blooms of the accent over the mode tint. */

151	fun Modifier.page(c: SbColors): Modifier = drawBehind {

152	    drawRect(c.tint)

153	    drawRect(

154	        Brush.radialGradient(

155	            0f to c.accent.copy(alpha = 0.09f), 0.7f to Color.Transparent,

156	            center = Offset(size.width * 0.10f, -size.height * 0.06f), radius = size.width * 1.3f,

157	        )

158	    )

159	    drawRect(

160	        Brush.radialGradient(

161	            0f to c.accent.copy(alpha = 0.06f), 0.72f to Color.Transparent,

162	            center = Offset(size.width * 1.08f, size.height * 0.06f), radius = size.width * 1.1f,

163	        )

164	    )

165	}

Android App/app/src/main/java/smartbin/ui/Icons.kt

1	// ============================================================================

2	// ui/Icons.kt - line icons and small animations

3	// svg() turns a tiny SVG string (path, circle, rect, line) into a Compose

4	// ImageVector, so each icon is written as the same SVG markup a web page uses.

5	// Ic holds every icon by name; shake, spin and breathe are the animations.

6	// ============================================================================

7	package smartbin.ui

8

9	import androidx.compose.animation.core.LinearEasing

10	import androidx.compose.animation.core.RepeatMode

11	import androidx.compose.animation.core.animateFloat

12	import androidx.compose.animation.core.animateFloatAsState

13	import androidx.compose.animation.core.infiniteRepeatable

14	import androidx.compose.animation.core.rememberInfiniteTransition

15	import androidx.compose.animation.core.tween

16	import androidx.compose.foundation.Canvas

17	import androidx.compose.foundation.layout.size

18	import androidx.compose.material3.Icon

19	import androidx.compose.material3.LocalContentColor

20	import androidx.compose.runtime.Composable

21	import androidx.compose.runtime.getValue

22	import androidx.compose.ui.Modifier

23	import androidx.compose.ui.geometry.Offset

24	import androidx.compose.ui.graphics.Color

25	import androidx.compose.ui.graphics.SolidColor

26	import androidx.compose.ui.graphics.StrokeCap

27	import androidx.compose.ui.graphics.StrokeJoin

28	import androidx.compose.ui.graphics.drawscope.Stroke

29	import androidx.compose.ui.graphics.drawscope.clipPath

30	import androidx.compose.ui.graphics.drawscope.scale

31	import androidx.compose.ui.graphics.drawscope.translate

32	import androidx.compose.ui.graphics.graphicsLayer

33	import androidx.compose.ui.graphics.vector.ImageVector

34	import androidx.compose.ui.graphics.vector.PathParser

35	import androidx.compose.ui.graphics.vector.addPathNodes

36	import androidx.compose.ui.unit.Dp

37	import androidx.compose.ui.unit.dp

38	import kotlin.math.PI

39	import kotlin.math.cos

40	import kotlin.math.sin

41

42	/*

43	 * The icon set of the mockup (copied there from dashboard v9), kept as the

44	 * same SVG markup and turned into vectors by a small reader. Only what the

45	 * mockup uses is understood: path, circle and rect, fill="currentColor" for

46	 * a solid shape, opacity, stroke-width, and groups that turn or move.

47	 */

48

49	private val TAG = Regex("""<(/?)(\w+)([^>]*?)/?>""")

50	private val ATTR = Regex("""([\w-]+)="([^"]*)"""")

51

52	private fun circleD(cx: Float, cy: Float, r: Float) = "M${cx - r} ${cy}a$r $r 0 1 0 ${2 * r} 0a$r $r 0 1 0 ${-2 * r} 0z"

53

54	private fun rectD(x: Float, y: Float, w: Float, h: Float, rx: Float): String {

55	    if (rx <= 0f) return "M$x ${y}h${w}v${h}h${-w}z"

56	    val r = minOf(rx, w / 2, h / 2)

57	    val iw = w - 2 * r

58	    val ih = h - 2 * r

59	    return "M${x + r} ${y}h${iw}a$r $r 0 0 1 $r ${r}v${ih}a$r $r 0 0 1 ${-r} ${r}h${-iw}a$r $r 0 0 1 ${-r} ${-r}v${-ih}a$r $r 0 0 1 $r ${-r}z"

60	}

61

62	fun svg(name: String, body: String, sw: Float = 2f): ImageVector {

63	    val b = ImageVector.Builder(name, 24.dp, 24.dp, 24f, 24f)

64	    val ink = SolidColor(Color.Black)

65	    val alphas = ArrayList<Float>().apply { add(1f) }

66	    for (m in TAG.findAll(body)) {

67	        val closing = m.groupValues[1] == "/"

68	        val tag = m.groupValues[2]

69	        val a = ATTR.findAll(m.groupValues[3]).associate { it.groupValues[1] to it.groupValues[2] }

70	        fun f(k: String, d: Float = 0f) = a[k]?.toFloatOrNull() ?: d

71	        if (tag == "g") {

72	            if (closing) {

73	                b.clearGroup()

74	                if (alphas.size > 1) alphas.removeAt(alphas.lastIndex)

75	            } else {

76	                b.addGroup(

77	                    rotate = f("rot"), pivotX = f("px"), pivotY = f("py"),

78	                    translationX = f("tx"), translationY = f("ty"),

79	                )

80	                alphas.add(alphas.last() * f("opacity", 1f))

81	            }

82	            continue

83	        }

84	        if (closing) continue

85	        val d = when (tag) {

86	            "path" -> a["d"] ?: continue

87	            "circle" -> circleD(f("cx"), f("cy"), f("r"))

88	            "rect" -> rectD(f("x"), f("y"), f("width"), f("height"), f("rx"))

89	            else -> continue

90	        }

91	        val alpha = alphas.last() * f("opacity", 1f)

92	        if (a["fill"] == "currentColor") {

93	            b.addPath(addPathNodes(d), fill = ink, fillAlpha = alpha)

94	        } else {

95	            b.addPath(

96	                addPathNodes(d), fill = null, stroke = ink, strokeAlpha = alpha, strokeLineWidth = f("stroke-width", sw),

97	                strokeLineCap = StrokeCap.Round, strokeLineJoin = StrokeJoin.Round,

98	            )

99	        }

100	    }

101	    return b.build()

102	}

103

104	/** The calibration ring is dashed 7 on, 6 off; vectors have no dashes, so the dashes are arcs. */

105	private fun dashedRing(cx: Float, cy: Float, r: Float, on: Float, off: Float): String {

106	    val circ = 2 * PI.toFloat() * r

107	    val n = (circ / (on + off)).toInt().coerceAtLeast(1)

108	    val step = 360f / n

109	    val sweep = step * on / (on + off)

110	    val sb = StringBuilder()

111	    for (k in 0 until n) {

112	        val a0 = Math.toRadians((k * step - 90).toDouble())

113	        val a1 = Math.toRadians((k * step + sweep - 90).toDouble())

114	        val x0 = cx + r * cos(a0).toFloat()

115	        val y0 = cy + r * sin(a0).toFloat()

116	        val x1 = cx + r * cos(a1).toFloat()

117	        val y1 = cy + r * sin(a1).toFloat()

118	        sb.append("M$x0 ${y0}A$r $r 0 0 1 $x1 $y1")

119	    }

120	    return sb.toString()

121	}

122

123	object Ic {

124	    private val cache = HashMap<String, ImageVector>()

125	    private fun k(key: String, body: () -> String): ImageVector = cache.getOrPut(key) { svg(key, body()) }

126

127	    val grid get() = k("grid") { """<rect x="3" y="3" width="7.5" height="7.5" rx="1.6"/><rect x="13.5" y="3" width="7.5" height="7.5" rx="1.6"/><rect x="3" y="13.5" width="7.5" height="7.5" rx="1.6"/><rect x="13.5" y="13.5" width="7.5" height="7.5" rx="1.6"/>""" }

128	    val chart get() = k("chart") { """<path d="M3 20h18"/><path d="M5 15l4.5-5 3.5 3L19 6"/>""" }

129	    val sliders get() = k("sliders") { """<path d="M4 7h10M18 7h2M4 17h4M12 17h8"/><circle cx="16" cy="7" r="2.2"/><circle cx="10" cy="17" r="2.2"/>""" }

130	    val flask get() = k("flask") { """<path d="M9.5 3v6.2L4.6 18a2 2 0 0 0 1.7 3h11.4a2 2 0 0 0 1.7-3L14.5 9.2V3"/><path d="M8 3h8M7.4 15h9.2"/>""" }

131	    val bell get() = k("bell") { """<path d="M18 9a6 6 0 1 0-12 0c0 6-2 7-2 7h16s-2-1-2-7"/><path d="M10.4 20.5a2 2 0 0 0 3.2 0"/>""" }

132	    val plus get() = k("plus") { """<path d="M12 5v14M5 12h14"/>""" }

133	    val gear get() = k("gear") { """<circle cx="12" cy="12" r="3"/><path d="M12 2.8v2.6M12 18.6v2.6M2.8 12h2.6M18.6 12h2.6M5.5 5.5l1.8 1.8M16.7 16.7l1.8 1.8M18.5 5.5l-1.8 1.8M7.3 16.7l-1.8 1.8"/>""" }

134	    val truck get() = k("truck") { """<path d="M2 7h11v9H2zM13 10h4.5l3.5 3.5V16h-8z"/><circle cx="6.5" cy="18.5" r="1.8"/><circle cx="16.5" cy="18.5" r="1.8"/>""" }

135	    val check get() = k("check") { """<path d="M4 12.5l5 5L20 6.5"/>""" }

136	    val humid get() = k("humid") { """<path d="M12 3.2s6.2 6.6 6.2 10.3a6.2 6.2 0 1 1-12.4 0C5.8 9.8 12 3.2 12 3.2z"/><circle cx="12" cy="14.4" r="3" fill="currentColor" opacity=".45"/>""" }

137	    val gas get() = k("gas") { """<path d="M6.5 17c0-2.6 2.6-2.6 2.6-5.2S6.5 9.2 6.5 6.6"/><path d="M12 18c0-2.6 2.6-2.6 2.6-5.2S12 10.2 12 7.6"/><path d="M17.5 17c0-2.6 2.6-2.6 2.6-5.2s-2.6-2.6-2.6-5.2"/>""" }

138	    val press get() = k("press") { """<path d="M4.6 17.6a8.6 8.6 0 1 1 14.8 0"/><path d="M12 14l3.6-4.4"/><circle cx="12" cy="14.2" r="1.5" fill="currentColor"/><path d="M7.2 17.6h9.6"/>""" }

139	    val tamper get() = k("tamper") { """<path d="M12 3.4L22 20H2z"/><path d="M12 9.6v4.2M12 17h.01"/>""" }

140	    val calib get() = k("calib") { """<path d="${dashedRing(12f, 12f, 8.4f, 7f, 6f)}"/><circle cx="12" cy="12" r="2.4" fill="currentColor"/><path d="M12 1.6v2.4M12 20v2.4M1.6 12h2.4M20 12h2.4"/>""" }

141	    val fill get() = k("fill") { """<path d="M5.4 4.4h13.2l-1.3 15.2a2 2 0 0 1-2 1.8H8.7a2 2 0 0 1-2-1.8z"/><path d="M6.9 12.6c1.6-1.1 3.3-1.1 4.9 0s3.3 1.1 4.9 0"/>""" }

142	    val bins get() = k("bins") { """<path d="M3.2 7.6h7l-.8 11.8H4zM13.8 7.6h7l-.8 11.8h-5.4z"/><path d="M2.4 7.6h8.6M13 7.6h8.6M5.4 5h2.6M16 5h2.6"/>""" }

143	    val clock get() = k("clock") { """<circle cx="12" cy="12" r="8.4"/><path d="M12 7.2V12l3.2 2"/>""" }

144	    val trash get() = k("trash") { """<path d="M4.4 6.8h15.2M9.6 6.8V4.6h4.8v2.2M6.8 6.8l1 12.6h8.4l1-12.6M10.2 10.4v5.4M13.8 10.4v5.4"/>""" }

145	    val stopwatch get() = k("stopwatch") { """<circle cx="12" cy="13.6" r="7.4"/><path d="M12 9.8v3.8l2.6 1.6M9.6 2.8h4.8M19.4 7.2l1.4-1.4"/>""" }

146	    val layers get() = k("layers") { """<path d="M12 3.4l8.6 4.6L12 12.6 3.4 8z"/><path d="M3.4 12.4L12 17l8.6-4.6M3.4 16.6L12 21.2l8.6-4.6"/>""" }

147	    val rank get() = k("rank") { """<path d="M9.6 6.6h10.8M9.6 12h10.8M9.6 17.4h10.8"/><path d="M4.8 5v3.6M3.6 5h1.2M3.4 10.4h2.8M3.4 15.4h2.6v1.6h-2.6v1.8h2.6"/>""" }

148	    val eye get() = k("eye") { """<path d="M2.4 12s3.6-6.4 9.6-6.4S21.6 12 21.6 12s-3.6 6.4-9.6 6.4S2.4 12 2.4 12z"/><circle cx="12" cy="12" r="2.8"/>""" }

149	    val eyeoff get() = k("eyeoff") { """<path d="M4 4l16 16"/><path d="M9.6 9.7a2.8 2.8 0 0 0 3.9 3.9"/><path d="M6.4 6.8C3.9 8.5 2.4 12 2.4 12s3.6 6.4 9.6 6.4c1.6 0 3-.4 4.2-1M18.3 15.1c1.9-1.6 3.3-3.1 3.3-3.1S18 5.6 12 5.6c-.8 0-1.5.1-2.2.3"/>""" }

150	    val bluetooth get() = k("bluetooth") { """<path d="M7.4 7.4l9.2 9.2L12 20.6V3.4l4.6 4L7.4 16.6"/>""" }

151	    val radar get() = k("radar") { """<path d="M12 12l5.6-5.6"/><circle cx="12" cy="12" r="1.6" fill="currentColor"/><path d="M8.4 15.6a5.1 5.1 0 0 1 0-7.2M5.6 18.4a9 9 0 0 1 0-12.8"/><path d="M15.6 8.4a5.1 5.1 0 0 1 0 7.2"/>""" }

152	    val key get() = k("key") { """<circle cx="8.2" cy="12" r="3.6"/><path d="M11.8 12h9v3M17.6 12v2.6"/>""" }

153	    val ruler get() = k("ruler") { """<rect x="2.6" y="8" width="18.8" height="8" rx="1.8"/><path d="M7 8v3M11 8v4M15 8v3M19 8v4"/>""" }

154	    val percent get() = k("percent") { """<path d="M18.5 5.5l-13 13"/><circle cx="7.6" cy="7.6" r="2.3"/><circle cx="16.4" cy="16.4" r="2.3"/>""" }

155	    val save get() = k("save") { """<path d="M4.5 4.5h11l4 4v11h-15z"/><path d="M8 4.5v5h6v-5M8 19.5v-5.5h8v5.5"/>""" }

156	    val reload get() = k("reload") { """<path d="M20.5 12a8.5 8.5 0 1 1-2.6-6.1"/><path d="M20.8 4.4v4.8h-4.8"/>""" }

157	    val download get() = k("download") { """<path d="M12 3.6v11.2M7.6 10.6l4.4 4.4 4.4-4.4"/><path d="M4.4 19.4h15.2"/>""" }

158	    val undo get() = k("undo") { """<path d="M3.6 12a8.4 8.4 0 1 0 2.6-6.1"/><path d="M3.3 4.4v4.8h4.8"/>""" }

159	    val capture get() = k("capture") { """<circle cx="12" cy="12" r="7.6"/><circle cx="12" cy="12" r="1.8" fill="currentColor"/><path d="M12 1.8v2.8M12 19.4v2.8M1.8 12h2.8M19.4 12h2.8"/>""" }

160	    val pin get() = k("pin") { """<rect x="3.4" y="4.6" width="17.2" height="14.8" rx="2.2"/><path d="M7.4 9.4h.01M12 9.4h.01M16.6 9.4h.01M7.4 14.6h9.2"/>""" }

161	    val tilt get() = k("tilt") { """<path d="M3.4 20.2h17.2"/><path d="M6.6 20.2L15 6.4"/><path d="M11.4 20.2a7.5 7.5 0 0 0-1.6-4.4"/>""" }

162	    val servo get() = k("servo") { """<circle cx="12" cy="13.6" r="3.2"/><path d="M12 10.4V4.6"/><path d="M4.6 13.6a7.4 7.4 0 0 1 3.2-6.1M19.4 13.6a7.4 7.4 0 0 0-3.2-6.1"/>""" }

163	    val cross get() = k("cross") { """<path d="M6.4 6.4l11.2 11.2M17.6 6.4L6.4 17.6"/>""" }

164	    val power get() = k("power") { """<path d="M12 3.4v8.2"/><path d="M6.8 6.6a7.6 7.6 0 1 0 10.4 0"/>""" }

165	    val chev get() = k("chev") { """<path d="M6 9l6 6 6-6"/>""" }

166	    val cloud get() = k("cloud") { """<path d="M7 18.6h10.4a4.2 4.2 0 0 0 .5-8.4 6 6 0 0 0-11.5-1A4.8 4.8 0 0 0 7 18.6z"/>""" }

167	    val back get() = k("back") { """<path d="M15 5l-7 7 7 7"/>""" }

168	    val expand get() = k("expand") { """<path d="M9.5 3.5H3.5v6M14.5 3.5h6v6M20.5 14.5v6h-6M3.5 14.5v6h6"/>""" }

169	    val shrink get() = k("shrink") { """<path d="M3.5 9.5h6v-6M20.5 9.5h-6v-6M14.5 20.5v-6h6M9.5 20.5v-6h-6"/>""" }

170	    val user get() = k("user") { """<circle cx="12" cy="8.2" r="3.8"/><path d="M4.6 20.4a7.4 7.4 0 0 1 14.8 0"/>""" }

171	    val palette get() = k("palette") { """<path d="M12 3.4a8.6 8.6 0 1 0 0 17.2c1.2 0 1.9-.8 1.9-1.8 0-1-.9-1.5-.9-2.4s.7-1.6 1.7-1.6h1.3a4.6 4.6 0 0 0 4.6-4.6c0-3.8-3.9-6.8-8.6-6.8z"/><circle cx="7.8" cy="11.6" r="1.1" fill="currentColor"/><circle cx="10.6" cy="7.8" r="1.1" fill="currentColor"/><circle cx="15" cy="8.4" r="1.1" fill="currentColor"/>""" }

172	    val speaker get() = k("speaker") { """<path d="M4 9.5h3.6L12.4 5v14l-4.8-4.5H4z"/><path d="M15.6 9a4.2 4.2 0 0 1 0 6M18.4 6.4a8 8 0 0 1 0 11.2"/>""" }

173	    val vibrate get() = k("vibrate") { """<rect x="7.4" y="3.6" width="9.2" height="16.8" rx="2"/><path d="M3.6 8.4v7.2M20.4 8.4v7.2M11 17.2h2"/>""" }

174	    val sun get() = k("sun") { """<circle cx="12" cy="12" r="3.8"/><path d="M12 2.6v2.4M12 19v2.4M2.6 12h2.4M19 12h2.4M5.4 5.4l1.7 1.7M16.9 16.9l1.7 1.7M18.6 5.4l-1.7 1.7M7.1 16.9l-1.7 1.7"/>""" }

175

176	    /** The thermometer fills with the temperature, 10 to 50 C. */

177	    fun temp(t: Double?): ImageVector {

178	        val f = (((t ?: 28.0) - 10) / 40).coerceIn(0.05, 1.0)

179	        val q = (f * 20).toInt() / 20.0

180	        return k("temp$q") {

181	            val h = 6 * q + 1.4

182	            val y = 15.4 - h

183	            """<path d="M14.2 14.4V5.2a2.2 2.2 0 1 0-4.4 0v9.2a4.6 4.6 0 1 0 4.4 0z"/><rect x="10.6" y="$y" width="2.8" height="$h" rx="1.4" fill="currentColor"/><circle cx="12" cy="17.2" r="2.6" fill="currentColor"/>"""

184	        }

185	    }

186

187	    /** The scale needle swings with the load, a share of capacity. */

188	    fun weight(f: Double?): ImageVector {

189	        val deg = (((f ?: 0.3).coerceIn(0.0, 1.1) * 150 - 75) / 5).toInt() * 5

190	        return k("weight$deg") {

191	            """<path d="M4.6 20.5h14.8l-1.9-9.6H6.5z"/><path d="M9 10.9a3 3 0 1 1 6 0"/><g rot="$deg" px="12" py="17.6"><path d="M12 17.6v-3.1" stroke-width="2.2"/></g>"""

192	        }

193	    }

194

195	    fun lid(open: Boolean) = k("lid$open") {

196	        """<path d="M6.6 9.5l1.1 11h8.6l1.1-11"/><g rot="${if (open) -38 else 0}" px="6" py="9"><path d="M4.5 9.5h15"/><path d="M9.6 7.1h4.8"/></g>"""

197	    }

198

199	    fun motion(on: Boolean) = k("motion$on") {

200	        """<circle cx="5.5" cy="12" r="1.8" fill="currentColor"/><g opacity="${if (on) 1 else 0.4}"><path d="M10 7.5a7 7 0 0 1 0 9"/><path d="M13.6 5a11 11 0 0 1 0 14"/><path d="M17.2 2.6a15 15 0 0 1 0 18.8"/></g>"""

201	    }

202

203	    fun lock(locked: Boolean) = k("lock$locked") {

204	        """<rect x="4.5" y="10.5" width="15" height="10.5" rx="2.4"/><g ty="${if (locked) 0 else -3}"><path d="M8 10.5V7.4a4 4 0 0 1 8 0v3.1"/></g><circle cx="12" cy="15.7" r="1.5" fill="currentColor"/>"""

205	    }

206

207	    fun sunmoon(dark: Boolean) = k("sunmoon$dark") {

208	        val sun = if (dark) 1.0 else 0.3

209	        val moon = if (dark) 0.3 else 1.0

210	        """<g opacity="$sun"><circle cx="8.2" cy="12" r="3"/><path d="M8.2 5.4v1.6M8.2 17v1.6M3.4 7.2l1.2 1.2M11.8 15.6l1.2 1.2M3.4 16.8l1.2-1.2M1.2 12h1.6"/></g><g opacity="$moon"><path d="M19.4 7.6a5.4 5.4 0 1 0 0 8.8 6.2 6.2 0 0 1 0-8.8z"/></g>"""

211	    }

212

213	    fun wifi(live: Boolean) = k("wifi$live") {

214	        """<path d="M8.5 15.2a5 5 0 0 1 7 0" stroke-width="2.2"/><path d="M5.5 11.8a9.4 9.4 0 0 1 13 0" stroke-width="2.2"/><path d="M2.4 8.4a14 14 0 0 1 19.2 0" stroke-width="2.2"/><circle cx="12" cy="18.6" r="1.4" fill="currentColor"/>"""

215	    }

216

217	    /** The icon names alert rows carry. */

218	    fun named(n: String): ImageVector = when (n) {

219	        "fill" -> fill; "gas" -> gas; "tamper" -> tamper; "humid" -> humid; "lock" -> lock(true)

220	        "check" -> check; "truck" -> truck; "calib" -> calib; "temp" -> temp(41.0); "save" -> save; "gear" -> gear

221	        else -> bell

222	    }

223	}

224

225	/** An icon from [Ic] in the colour of the text around it, or [tint]. */

226	@Composable

227	fun Ico(icon: ImageVector, size: Dp = 21.dp, tint: Color = LocalContentColor.current, modifier: Modifier = Modifier) {

228	    Icon(icon, contentDescription = null, modifier = modifier.size(size), tint = tint)

229	}

230

231	/** @keyframes shake: the tamper triangle rocks while it is shown. */

232	@Composable

233	fun Modifier.shake(on: Boolean = true): Modifier {

234	    if (!on) return this

235	    val t = rememberInfiniteTransition(label = "shake")

236	    val r by t.animateFloat(-6f, 6f, infiniteRepeatable(tween(225), RepeatMode.Reverse), label = "shake")

237	    return graphicsLayer { rotationZ = r }

238	}

239

240	@Composable

241	fun Modifier.spin(on: Boolean = true): Modifier {

242	    if (!on) return this

243	    val t = rememberInfiniteTransition(label = "spin")

244	    val r by t.animateFloat(0f, 360f, infiniteRepeatable(tween(1100, easing = LinearEasing)), label = "spin")

245	    return graphicsLayer { rotationZ = r }

246	}

247

248	/** @keyframes breathe, 0.82 to 1.12 in scale and 0.45 to 1 in opacity. */

249	@Composable

250	fun Modifier.breathe(on: Boolean = true, millis: Int = 2800): Modifier {

251	    if (!on) return this

252	    val t = rememberInfiniteTransition(label = "breathe")

253	    val f by t.animateFloat(0f, 1f, infiniteRepeatable(tween(millis / 2), RepeatMode.Reverse), label = "breathe")

254	    return graphicsLayer {

255	        val s = 0.82f + 0.30f * f

256	        scaleX = s

257	        scaleY = s

258	        alpha = 0.45f + 0.55f * f

259	    }

260	}

261

262	private val BIN_BODY = PathParser().parsePathString("M11.5 14h25l-2.6 26.2a2.4 2.4 0 0 1-2.4 2.1H16.5a2.4 2.4 0 0 1-2.4-2.1z").toPath()

263	private val BIN_LID = PathParser().parsePathString("M8 13h32M19 9.4h10").toPath()

264	private val WAVE_A = PathParser().parsePathString("M-24 4 q6 -5 12 0 t12 0 t12 0 t12 0 t12 0 t12 0 v36 h-72z").toPath()

265	private val WAVE_B = PathParser().parsePathString("M-24 7 q6 -5 12 0 t12 0 t12 0 t12 0 t12 0 t12 0 v36 h-72z").toPath()

266

267	/**

268	 * The bin with its fill level as a moving wave, the hero of every bin card

269	 * and tile. The waves drift sideways the whole time and rise or fall to a

270	 * new level over 0.6 s.

271	 */

272	@Composable

273	fun BinIcon(pct: Double, color: Color, size: Dp = 46.dp, modifier: Modifier = Modifier) {

274	    val level by animateFloatAsState(pct.coerceIn(0.0, 100.0).toFloat(), tween(600), label = "level")

275	    val t = rememberInfiniteTransition(label = "wave")

276	    val a by t.animateFloat(0f, 24f, infiniteRepeatable(tween(3400, easing = LinearEasing)), label = "a")

277	    val b by t.animateFloat(0f, 24f, infiniteRepeatable(tween(5000, easing = LinearEasing)), label = "b")

278	    Canvas(modifier.size(size)) {

279	        scale(this.size.width / 48f, pivot = Offset.Zero) {

280	            val y = 42f - level / 100f * 30f

281	            clipPath(BIN_BODY) {

282	                translate(a, y) { drawPath(WAVE_A, color, alpha = 0.85f) }

283	                translate(b, y) { drawPath(WAVE_B, color, alpha = 0.35f) }

284	            }

285	            drawPath(BIN_BODY, color, style = Stroke(2.6f, join = StrokeJoin.Round))

286	            drawPath(BIN_LID, color, style = Stroke(2.6f, cap = StrokeCap.Round))

287	        }

288	    }

289	}

Android App/app/src/main/java/smartbin/ui/Logo.kt

1	// ============================================================================

2	// ui/Logo.kt - the animated Smart-Bin logo on the start page and header

3	// The bin, waves, signal arcs and chip are vector paths on a 64 x 64 grid.

4	// play counts up to replay the build-up animation.

5	// ============================================================================

6	package smartbin.ui

7

8	import androidx.compose.animation.core.Animatable

9	import androidx.compose.animation.core.CubicBezierEasing

10	import androidx.compose.animation.core.LinearEasing

11	import androidx.compose.animation.core.RepeatMode

12	import androidx.compose.animation.core.animateFloat

13	import androidx.compose.animation.core.infiniteRepeatable

14	import androidx.compose.animation.core.rememberInfiniteTransition

15	import androidx.compose.animation.core.tween

16	import androidx.compose.foundation.Canvas

17	import androidx.compose.foundation.layout.size

18	import androidx.compose.runtime.Composable

19	import androidx.compose.runtime.LaunchedEffect

20	import androidx.compose.runtime.getValue

21	import androidx.compose.runtime.remember

22	import androidx.compose.ui.Modifier

23	import androidx.compose.ui.geometry.Offset

24	import androidx.compose.ui.graphics.Brush

25	import androidx.compose.ui.graphics.Color

26	import androidx.compose.ui.graphics.StrokeCap

27	import androidx.compose.ui.graphics.StrokeJoin

28	import androidx.compose.ui.graphics.drawscope.DrawScope

29	import androidx.compose.ui.graphics.drawscope.Stroke

30	import androidx.compose.ui.graphics.drawscope.clipPath

31	import androidx.compose.ui.graphics.drawscope.rotate

32	import androidx.compose.ui.graphics.drawscope.scale

33	import androidx.compose.ui.graphics.drawscope.translate

34	import androidx.compose.ui.graphics.vector.PathParser

35	import androidx.compose.ui.unit.Dp

36	import androidx.compose.ui.unit.dp

37	import kotlinx.coroutines.coroutineScope

38	import kotlinx.coroutines.launch

39

40	/*

41	 * The v2 Smart-Bin logo, the same drawing as the dashboard's: a bin with a fill

42	 * wave, a controller chip on its front, a status light on the lid and a Wi-Fi

43	 * signal above it. It plays a short build-up (the lid lifts, the bin fills, the

44	 * chip lights, the signal goes out) and then holds its still frame, which is

45	 * also the home-screen icon. Drawn on a 64 by 64 grid.

46	 */

47	private fun p(d: String) = PathParser().parsePathString(d).toPath()

48

49	private val L_BODY = p("M17.5 26h29l-2.7 27.6a3.2 3.2 0 0 1-3.2 2.9H23.4a3.2 3.2 0 0 1-3.2-2.9z")

50	private val L_WAVE_BACK = p("M0 37q5-3.6 10 0t10 0t10 0t10 0t10 0t10 0t10 0v30H0z")

51	private val L_WAVE_FRONT = p("M-5 39q5-3.6 10 0t10 0t10 0t10 0t10 0t10 0t10 0v30H-5z")

52	private val L_SIG = listOf(

53	    p("M27.8 12.6a6 6 0 0 1 8.4 0"),

54	    p("M24.2 8.9a11 11 0 0 1 15.6 0"),

55	    p("M20.6 5.2a16 16 0 0 1 22.8 0"),

56	)

57	private val L_TICKS = p("M22 30.5h3.4M22 34h2.2")

58	private val L_PINS = p("M32 42v-3.2M32 51v2.5M27.5 46.5h-4M36.5 46.5h4")

59	private val L_LID = p("M13 22.5h38")

60	private val L_HANDLE = p("M27 22.5v-3.4h10v3.4")

61	private val SPRING = CubicBezierEasing(0.2f, 0.9f, 0.3f, 1.4f)

62	private val SOFT = CubicBezierEasing(0.2f, 0.8f, 0.2f, 1f)

63

64	/**

65	 * The logo at [size]. Each new value of [play] plays the build-up again, as a

66	 * tap on it does; 0 shows the still frame at once.

67	 */

68	@Composable

69	fun SmartBinLogo(size: Dp, play: Int = 0, modifier: Modifier = Modifier) {

70	    val c = Sb.c

71	    val ok = c.ok

72	    val teal = c.cal

73	    val ink = c.ink

74	    val onBrand = if (c.isDark) Color(0xFF13100D) else Color.White

75

76	    // 0 to 1 for each part of the build-up. The still frame is every part at 1.

77	    val lid = remember { Animatable(1f) }

78	    val fill = remember { Animatable(1f) }

79	    val chip = remember { Animatable(1f) }

80	    val sig = remember { Animatable(1f) }

81	    LaunchedEffect(play) {

82	        if (play == 0) return@LaunchedEffect

83	        lid.snapTo(0f); fill.snapTo(0f); chip.snapTo(0f); sig.snapTo(0f)

84	        coroutineScope {

85	            launch { lid.animateTo(1f, tween(1100)) }

86	            launch { kotlinx.coroutines.delay(250); fill.animateTo(1f, tween(1500, easing = SOFT)) }

87	            launch { kotlinx.coroutines.delay(1000); chip.animateTo(1f, tween(600, easing = SPRING)) }

88	            launch { kotlinx.coroutines.delay(1300); sig.animateTo(1f, tween(800)) }

89	        }

90	    }

91	    val t = rememberInfiniteTransition(label = "logo")

92	    val wave by t.animateFloat(0f, -10f, infiniteRepeatable(tween(3200, easing = LinearEasing)), label = "wave")

93	    val wave2 by t.animateFloat(-10f, 0f, infiniteRepeatable(tween(4600, easing = LinearEasing)), label = "wave2")

94	    val ping by t.animateFloat(0f, 1f, infiniteRepeatable(tween(3600, easing = LinearEasing)), label = "ping")

95	    val led by t.animateFloat(0.35f, 1f, infiniteRepeatable(tween(1200), RepeatMode.Reverse), label = "led")

96

97	    Canvas(modifier.size(size)) {

98	        scale(this.size.width / 64f, pivot = Offset.Zero) {

99	            drawLogo(

100	                ink, ok, teal, onBrand,

101	                lid = lid.value, fill = fill.value, chip = chip.value, sig = sig.value,

102	                wave = wave, wave2 = wave2, ping = ping, led = led,

103	            )

104	        }

105	    }

106	}

107

108	private fun DrawScope.drawLogo(

109	    ink: Color, ok: Color, teal: Color, onBrand: Color,

110	    lid: Float, fill: Float, chip: Float, sig: Float,

111	    wave: Float, wave2: Float, ping: Float, led: Float,

112	) {

113	    // The glow behind the signal grows in with it.

114	    if (sig > 0f) {

115	        drawCircle(

116	            Brush.radialGradient(listOf(ok.copy(alpha = 0.45f * sig), ok.copy(alpha = 0f)), center = Offset(32f, 12f), radius = 12f * (0.3f + 0.7f * sig)),

117	            radius = 12f, center = Offset(32f, 12f),

118	        )

119	    }

120	    // Three arcs go out one after another, then pulse gently.

121	    val base = floatArrayOf(1f, 0.7f, 0.4f)

122	    L_SIG.forEachIndexed { i, path ->

123	        val appear = ((sig * 3f) - i).coerceIn(0f, 1f)

124	        val phase = (ping - i * 0.07f + 1f) % 1f

125	        val pulse = 1f - 0.65f * kotlin.math.sin(phase * Math.PI.toFloat()).let { it * it } * 0.6f

126	        if (appear > 0f) {

127	            translate(0f, 3f * (1f - appear)) {

128	                drawPath(path, ok, alpha = base[i] * appear * pulse, style = Stroke(2.6f, cap = StrokeCap.Round))

129	            }

130	        }

131	    }

132	    // A soft glass tint inside the bin.

133	    drawPath(

134	        L_BODY,

135	        Brush.horizontalGradient(listOf(ok.copy(alpha = 0.05f), ok.copy(alpha = 0.16f), ok.copy(alpha = 0.05f)), startX = 17.5f, endX = 46.5f),

136	    )

137	    // The fill rises from the bottom, its two waves drifting the whole time.

138	    val gradient = Brush.verticalGradient(listOf(ok, teal), startY = 34f, endY = 57f)

139	    clipPath(L_BODY) {

140	        translate(wave2, 17f * (1f - fill)) { drawPath(L_WAVE_BACK, gradient, alpha = 0.35f) }

141	        translate(wave, 17f * (1f - fill)) { drawPath(L_WAVE_FRONT, gradient) }

142	    }

143	    drawPath(L_TICKS, ok, alpha = 0.8f * chip, style = Stroke(1.8f, cap = StrokeCap.Round))

144	    // The controller chip pops in.

145	    if (chip > 0f) {

146	        scale(chip, pivot = Offset(32f, 46.5f)) {

147	            val a = chip.coerceIn(0f, 1f)

148	            drawRoundRect(onBrand, topLeft = Offset(27.5f, 42f), size = androidx.compose.ui.geometry.Size(9f, 9f),

149	                cornerRadius = androidx.compose.ui.geometry.CornerRadius(1.8f), alpha = a, style = Stroke(2f))

150	            drawPath(L_PINS, onBrand, alpha = a, style = Stroke(1.8f, cap = StrokeCap.Round))

151	            listOf(Offset(32f, 37.6f), Offset(22.6f, 46.5f), Offset(41.4f, 46.5f)).forEach { drawCircle(onBrand, 1.5f, it, alpha = a) }

152	        }

153	    }

154	    drawPath(L_BODY, ink, style = Stroke(3f, join = StrokeJoin.Round))

155	    // The lid lifts on its hinge at the left and settles back.

156	    val swing = when {

157	        lid < 0.35f -> lid / 0.35f

158	        lid < 0.7f -> 1f - (lid - 0.35f) / 0.35f * 0.62f

159	        else -> 0.38f * (1f - (lid - 0.7f) / 0.3f)

160	    }

161	    translate(0f, -3f * swing) {

162	        rotate(-16f * swing, pivot = Offset(13f, 22.5f)) {

163	            drawPath(L_LID, ink, style = Stroke(3.4f, cap = StrokeCap.Round))

164	            drawPath(L_HANDLE, ink, style = Stroke(3f, cap = StrokeCap.Round, join = StrokeJoin.Round))

165	            drawCircle(ok, 1.3f, Offset(45.5f, 22.5f), alpha = led)

166	        }

167	    }

168	}

Android App/app/src/main/java/smartbin/ui/Kit.kt

1	// ============================================================================

2	// ui/Kit.kt - shared building blocks for every page

3	// Text (T, Hint, Sh), keys (Btn, IconKey, Toggle, Stepper), inputs, banners,

4	// cards, sheets and the toast. Each one names the mockup CSS class it copies

5	// (.hint, .tgl, .sheet ...). Sizes use em() on a 14 sp base, like the mockup.

6	// ============================================================================

7	package smartbin.ui

8

9	import androidx.activity.compose.BackHandler

10	import androidx.compose.animation.core.Animatable

11	import androidx.compose.animation.core.RepeatMode

12	import androidx.compose.animation.core.animateFloat

13	import androidx.compose.animation.core.animateFloatAsState

14	import androidx.compose.animation.core.infiniteRepeatable

15	import androidx.compose.animation.core.rememberInfiniteTransition

16	import androidx.compose.animation.core.tween

17	import androidx.compose.foundation.background

18	import androidx.compose.foundation.clickable

19	import androidx.compose.foundation.interaction.MutableInteractionSource

20	import androidx.compose.foundation.interaction.collectIsPressedAsState

21	import androidx.compose.foundation.layout.Arrangement

22	import androidx.compose.foundation.layout.Box

23	import androidx.compose.foundation.layout.BoxScope

24	import androidx.compose.foundation.layout.Column

25	import androidx.compose.foundation.layout.ColumnScope

26	import androidx.compose.foundation.layout.PaddingValues

27	import androidx.compose.foundation.layout.Row

28	import androidx.compose.foundation.layout.RowScope

29	import androidx.compose.foundation.layout.Spacer

30	import androidx.compose.foundation.layout.defaultMinSize

31	import androidx.compose.foundation.layout.fillMaxSize

32	import androidx.compose.foundation.layout.fillMaxWidth

33	import androidx.compose.foundation.layout.height

34	import androidx.compose.foundation.layout.heightIn

35	import androidx.compose.foundation.layout.imePadding

36	import androidx.compose.foundation.layout.navigationBarsPadding

37	import androidx.compose.foundation.layout.offset

38	import androidx.compose.foundation.layout.padding

39	import androidx.compose.foundation.layout.size

40	import androidx.compose.foundation.layout.width

41	import androidx.compose.foundation.layout.widthIn

42	import androidx.compose.foundation.rememberScrollState

43	import androidx.compose.foundation.shape.CircleShape

44	import androidx.compose.foundation.shape.RoundedCornerShape

45	import androidx.compose.foundation.text.BasicTextField

46	import androidx.compose.foundation.text.KeyboardOptions

47	import androidx.compose.foundation.verticalScroll

48	import androidx.compose.material3.LocalContentColor

49	import androidx.compose.material3.Text

50	import androidx.compose.runtime.Composable

51	import androidx.compose.runtime.CompositionLocalProvider

52	import androidx.compose.runtime.LaunchedEffect

53	import androidx.compose.runtime.getValue

54	import androidx.compose.runtime.mutableFloatStateOf

55	import androidx.compose.runtime.remember

56	import androidx.compose.runtime.setValue

57	import androidx.compose.ui.Alignment

58	import androidx.compose.ui.Modifier

59	import androidx.compose.ui.draw.alpha

60	import androidx.compose.ui.draw.clip

61	import androidx.compose.ui.draw.drawBehind

62	import androidx.compose.ui.geometry.CornerRadius

63	import androidx.compose.ui.geometry.Offset

64	import androidx.compose.ui.graphics.Color

65	import androidx.compose.ui.graphics.SolidColor

66	import androidx.compose.ui.graphics.graphicsLayer

67	import androidx.compose.ui.graphics.vector.ImageVector

68	import androidx.compose.ui.platform.LocalView

69	import androidx.compose.ui.text.AnnotatedString

70	import androidx.compose.ui.text.SpanStyle

71	import androidx.compose.ui.text.TextStyle

72	import androidx.compose.ui.text.buildAnnotatedString

73	import androidx.compose.ui.text.font.FontWeight

74	import androidx.compose.ui.text.input.KeyboardType

75	import androidx.compose.ui.text.input.PasswordVisualTransformation

76	import androidx.compose.ui.text.input.VisualTransformation

77	import androidx.compose.ui.text.style.TextAlign

78	import androidx.compose.ui.text.style.TextOverflow

79	import androidx.compose.ui.text.withStyle

80	import androidx.compose.ui.unit.Dp

81	import androidx.compose.ui.unit.IntOffset

82	import androidx.compose.ui.unit.TextUnit

83	import androidx.compose.ui.unit.dp

84	import androidx.compose.ui.unit.sp

85	import androidx.compose.ui.zIndex

86	import smartbin.notify.Feel

87	import smartbin.notify.Sfx

88	import smartbin.notify.Sounds

89

90	/** The mockup sizes text in em on a 14 px base. */

91	fun em(x: Float): TextUnit = (14f * x).sp

92

93	/** Text in the mockup's type scale. **bold** marks the parts .hint b makes darker. */

94	@Composable

95	fun T(

96	    text: String,

97	    modifier: Modifier = Modifier,

98	    size: Float = 1f,

99	    weight: FontWeight = FontWeight.ExtraBold,

100	    color: Color = LocalContentColor.current,

101	    align: TextAlign? = null,

102	    maxLines: Int = Int.MAX_VALUE,

103	    tnum: Boolean = false,

104	    spacing: Float = 0f,

105	    lineHeight: Float = 1.35f,

106	    bold: Color? = null,

107	    /** Steps the size down until it fits one line, the mockup's container-query labels. */

108	    fit: Boolean = false,

109	) {

110	    var scale by remember(text.length, size) { mutableFloatStateOf(1f) }

111	    val shown = if (fit) size * scale else size

112	    val body: AnnotatedString = if ("**" in text) marked(text, bold ?: color) else AnnotatedString(text)

113	    Text(

114	        text = body,

115	        modifier = modifier,

116	        color = color,

117	        textAlign = align,

118	        maxLines = if (fit) 1 else maxLines,

119	        softWrap = !fit && maxLines != 1,

120	        overflow = TextOverflow.Ellipsis,

121	        onTextLayout = { if (fit && it.didOverflowWidth && scale > 0.6f) scale = (scale * 0.92f).coerceAtLeast(0.6f) },

122	        style = TextStyle(

123	            fontFamily = Inter,

124	            fontSize = em(shown),

125	            fontWeight = weight,

126	            lineHeight = em(shown * lineHeight),

127	            letterSpacing = (spacing * 14f * shown).sp,

128	            fontFeatureSettings = if (tnum) "tnum" else null,

129	        ),

130	    )

131	}

132

133	private fun marked(text: String, bold: Color): AnnotatedString = buildAnnotatedString {

134	    text.split("**").forEachIndexed { i, part ->

135	        if (i % 2 == 1) withStyle(SpanStyle(color = bold, fontWeight = FontWeight.ExtraBold)) { append(part) } else append(part)

136	    }

137	}

138

139	/** .hint */

140	@Composable

141	fun Hint(text: String, modifier: Modifier = Modifier, align: TextAlign? = null) {

142	    val c = Sb.c

143	    T(text, modifier, size = 0.8f, weight = FontWeight.Medium, color = c.ink2, bold = c.ink, align = align, lineHeight = 1.45f)

144	}

145

146	/** .sh, the small uppercase section heading. */

147	@Composable

148	fun Sh(icon: ImageVector, text: String, cc: Color = Sb.c.ink2, modifier: Modifier = Modifier) {

149	    Row(modifier, verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(8.dp)) {

150	        Ico(icon, 16.dp, cc)

151	        T(text.uppercase(), size = 0.76f, spacing = 0.1f, color = cc, maxLines = 1)

152	    }

153	}

154

155	/** A press with no ripple and the key's own sound and tick. */

156	@Composable

157	fun Modifier.press(enabled: Boolean = true, sound: Sfx = Sfx.TAP, src: MutableInteractionSource? = null, onClick: () -> Unit): Modifier {

158	    val view = LocalView.current

159	    return clickable(interactionSource = src ?: remember { MutableInteractionSource() }, indication = null, enabled = enabled) {

160	        Feel.play(view, sound)

161	        onClick()

162	    }

163	}

164

165	/** Swallows taps, so a sheet does not pass them to the page behind it. */

166	@Composable

167	fun Modifier.swallow(): Modifier = clickable(interactionSource = remember { MutableInteractionSource() }, indication = null) {}

168

169	enum class Look { PLAIN, TINT, SOLID, ACCENT, DANGER }

170

171	/**

172	 * .btn and its kinds. PLAIN is a raised key on the page colour; TINT adds a

173	 * ring and colours the label; SOLID and ACCENT fill it; DANGER is the brutal

174	 * red key. Pressed, a key sinks into the page. [stack] puts the icon above

175	 * the label, as on the bin card's three action keys.

176	 */

177	@Composable

178	fun Btn(

179	    text: String?,

180	    onClick: () -> Unit,

181	    modifier: Modifier = Modifier,

182	    icon: ImageVector? = null,

183	    look: Look = Look.PLAIN,

184	    cc: Color = Sb.c.ink2,

185	    small: Boolean = false,

186	    enabled: Boolean = true,

187	    stack: Boolean = false,

188	    sound: Sfx = Sfx.TAP,

189	    /** For a key that stays pushed in, such as a picked bin. */

190	    pressedIn: Boolean = false,

191	    iconMod: Modifier = Modifier,

192	    textSize: Float? = null,

193	) {

194	    val c = Sb.c

195	    val src = remember { MutableInteractionSource() }

196	    val pressed by src.collectIsPressedAsState()

197	    val down = (pressed && enabled) || pressedIn

198	    val radius = if (look == Look.DANGER) 7.dp else if (small) 11.dp else 13.dp

199	    val shape = RoundedCornerShape(radius)

200	    val kind = if (enabled) look else Look.PLAIN

201	    val fg = when {

202	        !enabled -> c.ink2

203	        kind == Look.SOLID || kind == Look.ACCENT -> c.onAccent

204	        kind == Look.DANGER -> Color.White

205	        kind == Look.TINT || pressedIn -> cc

206	        else -> c.ink

207	    }

208	    val iconTint = when {

209	        !enabled -> c.ink2

210	        kind == Look.PLAIN && !pressedIn -> cc.takeIf { it != c.ink2 } ?: c.ink

211	        else -> fg

212	    }

213	    val fill = when (kind) {

214	        Look.SOLID -> cc

215	        Look.ACCENT -> c.accent

216	        Look.DANGER -> c.bad

217	        else -> c.base

218	    }

219	    val ring = if (enabled && (kind == Look.TINT || pressedIn)) cc.soft(0.3f) else null

220	    val surface = if (kind == Look.DANGER) {

221	        Modifier

222	            .offset { if (down) IntOffset(3.dp.roundToPx(), 3.dp.roundToPx()) else IntOffset.Zero }

223	            .brutal(c, radius, fill, if (down) 1.dp else 4.dp)

224	    } else if (down) {

225	        Modifier

226	            .offset { IntOffset(0, if (pressed) 1.dp.roundToPx() else 0) }

227	            .then(if (ring != null) Modifier.ring(ring, radius) else Modifier)

228	            .clip(shape).background(fill).sunk(c, radius)

229	    } else {

230	        Modifier.raised(c, radius, Out.O1, ring).clip(shape).background(fill)

231	    }

232	    val minH = if (stack) 56.dp else if (small) 38.dp else 44.dp

233	    val size = textSize ?: if (stack) 0.76f else if (small) 0.8f else 0.88f

234	    val content: @Composable () -> Unit = {

235	        CompositionLocalProvider(LocalContentColor provides fg) {

236	            if (icon != null) Ico(icon, if (stack) 22.dp else if (small) 15.dp else 18.dp, iconTint, iconMod)

237	            if (text != null) T(text, size = size, weight = FontWeight.Bold, spacing = 0.02f, maxLines = if (stack) 2 else 1, align = TextAlign.Center, lineHeight = 1.1f)

238	        }

239	    }

240	    val base = modifier

241	        .alpha(if (enabled) 1f else 0.4f)

242	        .then(surface)

243	        .press(enabled, sound, src, onClick)

244	        .defaultMinSize(minHeight = minH)

245	    if (stack) {

246	        Column(

247	            base.padding(horizontal = 4.dp, vertical = 6.dp),

248	            horizontalAlignment = Alignment.CenterHorizontally,

249	            verticalArrangement = Arrangement.spacedBy(3.dp, Alignment.CenterVertically),

250	        ) { content() }

251	    } else {

252	        Row(

253	            base.padding(horizontal = if (small) 12.dp else 14.dp, vertical = if (small) 5.dp else 8.dp),

254	            horizontalArrangement = Arrangement.spacedBy(7.dp, Alignment.CenterHorizontally),

255	            verticalAlignment = Alignment.CenterVertically,

256	        ) { content() }

257	    }

258	}

259

260	/** A square key with one icon: .iconbtn, .cardbtn, .gearbtn. */

261	@Composable

262	fun IconKey(

263	    icon: ImageVector,

264	    onClick: () -> Unit,

265	    modifier: Modifier = Modifier,

266	    tint: Color = Sb.c.accent,

267	    size: Dp = 44.dp,

268	    iconSize: Dp = 21.dp,

269	    radius: Dp = 13.dp,

270	    enabled: Boolean = true,

271	    sound: Sfx = Sfx.TAP,

272	    label: String? = null,

273	    iconMod: Modifier = Modifier,

274	) {

275	    val c = Sb.c

276	    val src = remember { MutableInteractionSource() }

277	    val pressed by src.collectIsPressedAsState()

278	    val shape = RoundedCornerShape(radius)

279	    Column(

280	        modifier

281	            .size(size)

282	            .alpha(if (enabled) 1f else 0.35f)

283	            .then(if (pressed) Modifier.clip(shape).background(c.base).sunk(c, radius) else Modifier.bump(c, radius))

284	            .press(enabled, sound, src, onClick),

285	        horizontalAlignment = Alignment.CenterHorizontally,

286	        verticalArrangement = Arrangement.Center,

287	    ) {

288	        val t = if (pressed && label != null) c.accent else tint

289	        Ico(icon, iconSize, t, iconMod)

290	        if (label != null) T(label, size = 8f / 14f, spacing = 0.05f, color = t, maxLines = 1)

291	    }

292	}

293

294	/** .tag, the small white-on-colour label. */

295	@Composable

296	fun Tag(text: String, bg: Color, modifier: Modifier = Modifier, edge: Boolean = true) {

297	    val c = Sb.c

298	    val shape = RoundedCornerShape(6.dp)

299	    Box(

300	        modifier

301	            .clip(shape)

302	            .background(bg)

303	            .then(if (edge) Modifier.drawBehind { drawRoundRect(c.ink, cornerRadius = CornerRadius(6.dp.toPx()), style = androidx.compose.ui.graphics.drawscope.Stroke(2.dp.toPx())) } else Modifier)

304	            .padding(horizontal = 9.dp, vertical = 4.dp),

305	    ) { T(text, size = 0.7f, spacing = 0.05f, color = Color.White, maxLines = 1) }

306	}

307

308	/** .tgl, the switch. */

309	@Composable

310	fun Toggle(on: Boolean, onChange: (Boolean) -> Unit, enabled: Boolean = true) {

311	    val c = Sb.c

312	    val x by animateFloatAsState(if (on) 1f else 0f, tween(220), label = "tgl")

313	    Box(

314	        Modifier

315	            .size(54.dp, 32.dp)

316	            .alpha(if (enabled) 1f else 0.42f)

317	            .clip(RoundedCornerShape(16.dp))

318	            .background(if (on) c.accent else c.well)

319	            .sunk(c, 16.dp)

320	            .press(enabled, if (on) Sfx.SWITCH_OFF else Sfx.SWITCH_ON) { onChange(!on) },

321	    ) {

322	        Box(

323	            Modifier

324	                .padding(4.dp)

325	                .offset { IntOffset((22.dp.toPx() * x).toInt(), 0) }

326	                .size(24.dp)

327	                .bump(c, 12.dp),

328	        )

329	    }

330	}

331

332	/** .twoway: a sunk tray of choices. Use [Tw] inside. */

333	@Composable

334	fun TwoWay(modifier: Modifier = Modifier, full: Boolean = false, content: @Composable RowScope.() -> Unit) {

335	    val c = Sb.c

336	    Row(

337	        (if (full) modifier.fillMaxWidth() else modifier).well(c, 13.dp).padding(5.dp),

338	        horizontalArrangement = Arrangement.spacedBy(5.dp),

339	        verticalAlignment = Alignment.CenterVertically,

340	        content = content,

341	    )

342	}

343

344	/** .tw, one choice in a [TwoWay]. */

345	@Composable

346	fun RowScope.Tw(

347	    on: Boolean,

348	    onClick: () -> Unit,

349	    label: String? = null,

350	    icon: ImageVector? = null,

351	    cc: Color = Sb.c.accent,

352	    enabled: Boolean = true,

353	    grow: Float? = 1f,

354	    stack: Boolean = false,

355	    minHeight: Dp = 38.dp,

356	    textSize: Float = 0.78f,

357	) {

358	    val c = Sb.c

359	    val shape = RoundedCornerShape(9.dp)

360	    val fg = if (on) c.onAccent else c.ink2

361	    val m = Modifier

362	        .then(if (grow != null) Modifier.weight(grow) else Modifier)

363	        .defaultMinSize(minHeight = minHeight)

364	        .alpha(if (enabled) 1f else 0.42f)

365	        .then(if (on) Modifier.bump(c, 9.dp, fill = cc) else Modifier.clip(shape))

366	        .press(enabled && !on, Sfx.CHOICE, onClick = onClick)

367	        .padding(horizontal = if (stack) 2.dp else 9.dp, vertical = if (stack) 5.dp else 0.dp)

368	    val inner: @Composable () -> Unit = {

369	        if (icon != null) Ico(icon, if (stack) 26.dp else 17.dp, fg)

370	        if (label != null) T(label, size = textSize, weight = FontWeight.Bold, color = fg, fit = true, align = TextAlign.Center)

371	    }

372	    if (stack) Column(m, horizontalAlignment = Alignment.CenterHorizontally, verticalArrangement = Arrangement.spacedBy(2.dp, Alignment.CenterVertically)) { inner() }

373	    else Row(m, horizontalArrangement = Arrangement.spacedBy(6.dp, Alignment.CenterHorizontally), verticalAlignment = Alignment.CenterVertically) { inner() }

374	}

375

376	/** .chip on the bin card. */

377	@Composable

378	fun Chip(icon: ImageVector, text: String, on: Boolean, cc: Color, modifier: Modifier = Modifier, iconMod: Modifier = Modifier) {

379	    val c = Sb.c

380	    Row(

381	        modifier

382	            .height(30.dp)

383	            .alpha(if (on) 1f else 0.38f)

384	            .bump(c, 9.dp, ring = if (on) cc.soft(0.3f) else null)

385	            .padding(horizontal = 6.dp),

386	        horizontalArrangement = Arrangement.spacedBy(5.dp, Alignment.CenterHorizontally),

387	        verticalAlignment = Alignment.CenterVertically,

388	    ) {

389	        val t = if (on) cc else c.ink2

390	        Ico(icon, 13.dp, t, iconMod)

391	        T(text, size = 0.72f, spacing = 0.02f, color = t, maxLines = 1)

392	    }

393	}

394

395	/** .field: a label line over its control. */

396	@Composable

397	fun Field(label: String, icon: ImageVector? = null, cc: Color = Sb.c.ink2, content: @Composable ColumnScope.() -> Unit) {

398	    Column(verticalArrangement = Arrangement.spacedBy(7.dp)) {

399	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(6.dp)) {

400	            if (icon != null) Ico(icon, 15.dp, cc)

401	            T(label, size = 0.76f, spacing = 0.03f, color = cc)

402	        }

403	        content()

404	    }

405	}

406

407	/** .well with − and + keys: the stepper. */

408	@Composable

409	fun Stepper(value: String, unit: String, onStep: (Int) -> Unit, modifier: Modifier = Modifier, enabled: Boolean = true, cc: Color = Sb.c.accent) {

410	    val c = Sb.c

411	    Row(

412	        modifier.fillMaxWidth().well(c, 13.dp).padding(4.dp),

413	        verticalAlignment = Alignment.CenterVertically,

414	        horizontalArrangement = Arrangement.spacedBy(6.dp),

415	    ) {

416	        StepKey("−", enabled, Sfx.STEP_DOWN, cc) { onStep(-1) }

417	        T(value, Modifier.weight(1f), size = 1.25f, tnum = true, align = TextAlign.Center, maxLines = 1)

418	        T(unit, Modifier.padding(end = 2.dp), size = 0.76f, color = c.ink2, maxLines = 1)

419	        StepKey("+", enabled, Sfx.STEP_UP, cc) { onStep(1) }

420	    }

421	}

422

423	@Composable

424	private fun StepKey(label: String, enabled: Boolean, sound: Sfx, cc: Color, onClick: () -> Unit) {

425	    val c = Sb.c

426	    val src = remember { MutableInteractionSource() }

427	    val pressed by src.collectIsPressedAsState()

428	    Box(

429	        Modifier

430	            .size(44.dp)

431	            .then(if (pressed) Modifier.clip(RoundedCornerShape(11.dp)).background(c.base).sunk(c, 11.dp) else Modifier.bump(c, 11.dp))

432	            .press(enabled, sound, src, onClick),

433	        contentAlignment = Alignment.Center,

434	    ) { T(label, size = 1.3f, color = if (pressed) cc else c.ink2, lineHeight = 1f) }

435	}

436

437	/** .readout, a value with no control. */

438	@Composable

439	fun Readout(text: String) {

440	    val c = Sb.c

441	    Box(Modifier.fillMaxWidth().well(c, 12.dp).padding(horizontal = 14.dp, vertical = 11.dp)) { T(text, tnum = true) }

442	}

443

444	/** .inp, a text field sunk into the page. */

445	@Composable

446	fun Inp(

447	    value: String,

448	    onChange: (String) -> Unit,

449	    modifier: Modifier = Modifier,

450	    placeholder: String = "",

451	    password: Boolean = false,

452	    keyboard: KeyboardType = KeyboardType.Text,

453	    enabled: Boolean = true,

454	) {

455	    val c = Sb.c

456	    val style = TextStyle(fontFamily = Inter, fontSize = em(1.02f), fontWeight = FontWeight.Bold, color = c.ink)

457	    BasicTextField(

458	        value = value,

459	        onValueChange = onChange,

460	        enabled = enabled,

461	        singleLine = true,

462	        textStyle = style,

463	        cursorBrush = SolidColor(c.accent),

464	        keyboardOptions = KeyboardOptions(keyboardType = if (password) KeyboardType.Password else keyboard, autoCorrectEnabled = false),

465	        visualTransformation = if (password) PasswordVisualTransformation() else VisualTransformation.None,

466	        modifier = modifier

467	            .fillMaxWidth()

468	            .heightIn(min = 50.dp)

469	            .alpha(if (enabled) 1f else 0.5f)

470	            .well(c, 12.dp),

471	        decorationBox = { inner ->

472	            Box(Modifier.padding(horizontal = 13.dp, vertical = 14.dp), contentAlignment = Alignment.CenterStart) {

473	                if (value.isEmpty() && placeholder.isNotEmpty()) T(placeholder, size = 1.02f, weight = FontWeight.Bold, color = c.ink2.soft(0.7f), maxLines = 1)

474	                inner()

475	            }

476	        },

477	    )

478	}

479

480	enum class Tone { RED, PURPLE, TEAL, GREEN, AMBER, ORANGE, BLUE }

481

482	/** .banner: the brutal message block. */

483	@Composable

484	fun Banner(tone: Tone, icon: ImageVector, title: String, small: String? = null, modifier: Modifier = Modifier, iconMod: Modifier = Modifier) {

485	    val c = Sb.c

486	    val bg = when (tone) {

487	        Tone.RED -> c.bad; Tone.PURPLE -> c.sim; Tone.TEAL -> c.cal; Tone.GREEN -> c.ok; Tone.AMBER -> c.warn; Tone.ORANGE -> c.cloud; Tone.BLUE -> c.ts

488	    }

489	    val fg = if (c.isDark || tone == Tone.AMBER) (if (c.isDark) Color(0xFF13100D) else Color(0xFF241C10)) else Color.White

490	    Row(

491	        modifier.fillMaxWidth().brutal(c, 8.dp, bg).padding(horizontal = 13.dp, vertical = 11.dp),

492	        horizontalArrangement = Arrangement.spacedBy(10.dp),

493	    ) {

494	        Ico(icon, 20.dp, fg, iconMod)

495	        Column(Modifier.weight(1f)) {

496	            T(title, size = 0.88f, weight = FontWeight.Bold, color = fg)

497	            if (!small.isNullOrBlank()) T(small, Modifier.padding(top = 2.dp).alpha(0.92f), size = 0.79f, weight = FontWeight.SemiBold, color = fg)

498	        }

499	    }

500	}

501

502	/** .pcard.nm: the plain raised card. */

503	@Composable

504	fun Card(modifier: Modifier = Modifier, padding: Dp = 14.dp, gap: Dp = 12.dp, content: @Composable ColumnScope.() -> Unit) {

505	    val c = Sb.c

506	    Column(modifier.fillMaxWidth().nm(c).padding(padding), verticalArrangement = Arrangement.spacedBy(gap), content = content)

507	}

508

509	/** .lrow: a row of the App section. */

510	@Composable

511	fun LRow(

512	    icon: ImageVector,

513	    title: String,

514	    sub: String? = null,

515	    cc: Color = Sb.c.ink2,

516	    onClick: (() -> Unit)? = null,

517	    chevron: Boolean = onClick != null,

518	    sound: Sfx = Sfx.OPEN,

519	    iconMod: Modifier = Modifier,

520	    trailing: @Composable (RowScope.() -> Unit)? = null,

521	) {

522	    val c = Sb.c

523	    val src = remember { MutableInteractionSource() }

524	    val pressed by src.collectIsPressedAsState()

525	    Row(

526	        Modifier

527	            .fillMaxWidth()

528	            .defaultMinSize(minHeight = 58.dp)

529	            .then(if (pressed) Modifier.clip(RoundedCornerShape(14.dp)).background(c.base).sunk(c, 14.dp) else Modifier.bump(c, 14.dp))

530	            .then(if (onClick != null) Modifier.press(true, sound, src, onClick) else Modifier)

531	            .padding(horizontal = 13.dp, vertical = 9.dp),

532	        verticalAlignment = Alignment.CenterVertically,

533	        horizontalArrangement = Arrangement.spacedBy(12.dp),

534	    ) {

535	        Ico(icon, 21.dp, cc, iconMod)

536	        Column(Modifier.weight(1f)) {

537	            T(title, weight = FontWeight.Bold)

538	            if (sub != null) T(sub, Modifier.padding(top = 1.dp), size = 0.8f, weight = FontWeight.SemiBold, color = c.ink2)

539	        }

540	        trailing?.invoke(this)

541	        if (chevron) Ico(Ic.chev, 18.dp, c.ink2, Modifier.graphicsLayer { rotationZ = -90f })

542	    }

543	}

544

545	/** .opt: one line of a pick list, with its radio. */

546	@Composable

547	fun Opt(on: Boolean, onClick: () -> Unit, modifier: Modifier = Modifier, content: @Composable RowScope.() -> Unit) {

548	    val c = Sb.c

549	    Row(

550	        modifier

551	            .fillMaxWidth()

552	            .defaultMinSize(minHeight = 50.dp)

553	            .then(if (on) Modifier.clip(RoundedCornerShape(13.dp)).background(c.base).sunk(c, 13.dp) else Modifier.bump(c, 13.dp))

554	            .press(true, Sfx.CHOICE, onClick = onClick)

555	            .padding(horizontal = 12.dp, vertical = 6.dp),

556	        verticalAlignment = Alignment.CenterVertically,

557	        horizontalArrangement = Arrangement.spacedBy(12.dp),

558	    ) {

559	        Box(Modifier.size(22.dp).well(c, 11.dp), contentAlignment = Alignment.Center) {

560	            if (on) Box(Modifier.size(10.dp).clip(CircleShape).background(c.accent))

561	        }

562	        CompositionLocalProvider(LocalContentColor provides if (on) c.accent else c.ink) { content() }

563	    }

564	}

565

566	/** .scrim and .sheet: slides up from the bottom; a tap outside or Back closes it. */

567	@Composable

568	fun Sheet(onDismiss: () -> Unit, content: @Composable ColumnScope.() -> Unit) {

569	    val c = Sb.c

570	    val slide = remember { Animatable(1f) }

571	    val fade = remember { Animatable(0f) }

572	    LaunchedEffect(Unit) { fade.animateTo(1f, tween(180)) }

573	    LaunchedEffect(Unit) { slide.animateTo(0f, tween(240)) }

574	    LaunchedEffect(Unit) { Sounds.playIfQuiet(Sfx.OPEN) }

575	    val view = LocalView.current

576	    BackHandler { Feel.play(view, Sfx.BACK); onDismiss() }

577	    androidx.compose.foundation.layout.BoxWithConstraints(Modifier.fillMaxSize().zIndex(40f)) {

578	        val maxH = maxHeight * 0.82f

579	        Box(

580	            Modifier

581	                .fillMaxSize()

582	                .alpha(fade.value)

583	                .background(Color(0x800A0806))

584	                .press(true, Sfx.BACK, onClick = onDismiss),

585	        )

586	        val shape = RoundedCornerShape(topStart = 24.dp, topEnd = 24.dp)

587	        Column(

588	            Modifier

589	                .align(Alignment.BottomCenter)

590	                .widthIn(max = 520.dp)

591	                .fillMaxWidth()

592	                .heightIn(max = maxH)

593	                .graphicsLayer { translationY = slide.value * size.height }

594	                .drawBehind {

595	                    drawRoundRect(Color(0x47000000), topLeft = Offset(0f, -6.dp.toPx()), size = size, cornerRadius = CornerRadius(24.dp.toPx()))

596	                }

597	                .clip(shape)

598	                .background(c.base)

599	                .swallow()

600	                .verticalScroll(rememberScrollState())

601	                .navigationBarsPadding()

602	                .imePadding()

603	                .padding(start = 16.dp, end = 16.dp, top = 8.dp, bottom = 26.dp),

604	            verticalArrangement = Arrangement.spacedBy(12.dp),

605	        ) {

606	            Box(Modifier.align(Alignment.CenterHorizontally).size(42.dp, 5.dp).clip(CircleShape).background(c.ink2.soft(0.4f)))

607	            CompositionLocalProvider(LocalContentColor provides c.ink) { content() }

608	        }

609	    }

610	}

611

612	@Composable

613	fun SheetTitle(icon: ImageVector?, text: String, center: Boolean = false, trailing: (@Composable RowScope.() -> Unit)? = null) {

614	    val c = Sb.c

615	    Row(

616	        if (center) Modifier.fillMaxWidth() else Modifier.fillMaxWidth(),

617	        verticalAlignment = Alignment.CenterVertically,

618	        horizontalArrangement = Arrangement.spacedBy(8.dp, if (center) Alignment.CenterHorizontally else Alignment.Start),

619	    ) {

620	        if (icon != null) Ico(icon, 21.dp, c.accent)

621	        T(text, size = 1.1f, align = if (center) TextAlign.Center else null)

622	        if (trailing != null) {

623	            Spacer(Modifier.weight(1f))

624	            trailing()

625	        }

626	    }

627	}

628

629	@Composable

630	fun SheetText(text: String, center: Boolean = false) {

631	    T(text, if (center) Modifier.fillMaxWidth() else Modifier, size = 0.9f, weight = FontWeight.Medium, align = if (center) TextAlign.Center else null)

632	}

633

634	/** .acts: two keys sharing a row. */

635	@Composable

636	fun Acts(content: @Composable RowScope.() -> Unit) {

637	    Row(Modifier.fillMaxWidth(), horizontalArrangement = Arrangement.spacedBy(10.dp), content = content)

638	}

639

640	/** .toast: an amber banner that rises over the bottom of the page. */

641	@Composable

642	fun BoxScope.Toast(message: String, bottom: Dp) {

643	    val rise = remember(message) { Animatable(0f) }

644	    LaunchedEffect(message) { rise.animateTo(1f, tween(250)) }

645	    Box(

646	        Modifier

647	            .align(Alignment.BottomCenter)

648	            .zIndex(60f)

649	            .navigationBarsPadding()

650	            .padding(start = 14.dp, end = 14.dp, bottom = bottom)

651	            .widthIn(max = 520.dp)

652	            .graphicsLayer {

653	                alpha = rise.value

654	                translationY = (1f - rise.value) * 8.dp.toPx()

655	            },

656	    ) { Banner(Tone.AMBER, Ic.bell, message) }

657	}

658

659	/** .sk: a breathing block while the first reading comes in. */

660	@Composable

661	fun Skeleton(height: Dp, modifier: Modifier = Modifier, radius: Dp = 18.dp) {

662	    val c = Sb.c

663	    val t = rememberInfiniteTransition(label = "sk")

664	    val a by t.animateFloat(0.55f, 1f, infiniteRepeatable(tween(600), RepeatMode.Reverse), label = "sk")

665	    Box(modifier.fillMaxWidth().height(height).alpha(a).well(c, radius))

666	}

667

668	/** The page's scrolling column, .scroll. */

669	@Composable

670	fun ScrollPage(modifier: Modifier = Modifier, state: androidx.compose.foundation.ScrollState = rememberScrollState(), bottomExtra: Dp = 0.dp, content: @Composable ColumnScope.() -> Unit) {

671	    Column(

672	        modifier

673	            .fillMaxSize()

674	            .verticalScroll(state)

675	            .padding(PaddingValues(start = 14.dp, end = 14.dp, top = 16.dp, bottom = 20.dp + bottomExtra)),

676	        verticalArrangement = Arrangement.spacedBy(18.dp),

677	        content = content,

678	    )

679	}

680

681	/** .backbar: the back key and a title, for the pages under Settings. */

682	@Composable

683	fun BackBar(title: String, onBack: () -> Unit, trailing: @Composable RowScope.() -> Unit = {}) {

684	    Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(10.dp)) {

685	        IconKey(Ic.back, onBack, sound = Sfx.BACK)

686	        T(title, Modifier.weight(1f), size = 1.12f)

687	        trailing()

688	    }

689	}

690

691	/** .btstate: the small status pill in a back bar. */

692	@Composable

693	fun StatePill(icon: ImageVector, text: String, cc: Color, on: Boolean) {

694	    val c = Sb.c

695	    Row(

696	        Modifier.height(34.dp).bump(c, 17.dp, ring = if (on) cc.soft(0.32f) else null).padding(horizontal = 12.dp),

697	        verticalAlignment = Alignment.CenterVertically,

698	        horizontalArrangement = Arrangement.spacedBy(7.dp),

699	    ) {

700	        val t = if (on) cc else c.ink2

701	        Ico(icon, 18.dp, t)

702	        T(text.uppercase(), size = 0.76f, spacing = 0.03f, color = t, maxLines = 1)

703	    }

704	}

705

706	/** A swatch for chart legends: solid, or dashed like the line it stands for. */

707	@Composable

708	fun Swatch(color: Color, dash: Int = 0) {

709	    androidx.compose.foundation.Canvas(Modifier.size(16.dp, 4.dp)) {

710	        val h = size.height

711	        if (dash == 0) {

712	            drawRoundRect(color, cornerRadius = CornerRadius(h / 2))

713	        } else {

714	            val on = (if (dash == 1) 5f else 2.5f) * density

715	            val gap = (if (dash == 1) 4f else 4f) * density

716	            var x = 0f

717	            while (x < size.width) {

718	                drawRect(color, topLeft = Offset(x, 0f), size = androidx.compose.ui.geometry.Size(minOf(on, size.width - x), h))

719	                x += on + gap

720	            }

721	        }

722	    }

723	}

724

725	@Composable

726	fun Gap(h: Dp) = Spacer(Modifier.height(h))

727

728	@Composable

729	fun RowScope.Grow() = Spacer(Modifier.weight(1f))

730

731	@Composable

732	fun HGap(w: Dp) = Spacer(Modifier.width(w))

Android App/app/src/main/java/smartbin/ui/AppViewModel.kt

1	// ============================================================================

2	// ui/AppViewModel.kt - screen state and user actions

3	// Holds what the screens need beyond Hub.state: the current page, role, PIN

4	// entry, sheet contents, chart choices, the calibration edit copy and the

5	// Bluetooth setup steps. Each button calls one function here; the function

6	// calls Hub (or BinBle) and reports back with say(), which shows the toast.

7	// Debug tip: almost every failure reaches the user as a toast string built in

8	// this file, so searching for the toast text finds the code path.

9	// ============================================================================

10	package smartbin.ui

11

12	import android.app.Application

13	import android.content.ContentValues

14	import android.content.Intent

15	import android.net.Uri

16	import android.os.Build

17	import android.os.PowerManager

18	import android.provider.Settings

19	import android.provider.MediaStore

20	import androidx.compose.runtime.getValue

21	import androidx.compose.runtime.mutableIntStateOf

22	import androidx.compose.runtime.mutableLongStateOf

23	import androidx.compose.runtime.mutableStateMapOf

24	import androidx.compose.runtime.mutableStateOf

25	import androidx.compose.runtime.setValue

26	import androidx.lifecycle.AndroidViewModel

27	import androidx.lifecycle.viewModelScope

28	import kotlinx.coroutines.Job

29	import kotlinx.coroutines.delay

30	import kotlinx.coroutines.launch

31	import org.json.JSONArray

32	import org.json.JSONObject

33	import smartbin.SmartBinApp

34	import smartbin.data.AlertKind

35	import smartbin.data.AlertRow

36	import smartbin.data.Bin

37	import smartbin.data.BinBle

38	import smartbin.data.Cal

39	import smartbin.data.Cloud

40	import smartbin.data.DEFAULT_SIM

41	import smartbin.data.FirebaseConfig

42	import smartbin.data.Demo

43	import smartbin.data.Hub

44	import smartbin.data.OperatorLock

45	import smartbin.data.HubEvent

46	import smartbin.data.Routes

47	import smartbin.data.Role

48	import smartbin.data.Source

49	import smartbin.data.v

50	import smartbin.notify.Alerts

51	import smartbin.notify.Feel

52	import smartbin.notify.Haptics

53	import smartbin.notify.Sfx

54	import java.util.Calendar

55

56	/** The five tabs, then the pages opened from the gear key: app settings, Bluetooth setup and Connection. */

57	enum class Screen { OVERVIEW, CHARTS, SETTINGS, SIM, ALERTS, APP, BT, NET }

58

59	enum class BinView { AUTO, GRID, CARDS }

60	enum class BinFilter { ALL, ACTION, MODES }

61

62	/** Which list a bin picker sheet chooses for. */

63	enum class PickFor(val title: String) { CHART("Fill level chart"), ENV("Environment chart"), SET("Settings"), SIM("Simulation") }

64

65	enum class EnvMode { ALL, TEMP, HUMID, GAS, PRESS }

66

67	enum class AlertRange(val label: String) { TODAY("Today"), WEEK("7 days"), MONTH("30 days"), ANY("Any") }

68

69	enum class Perm { ON, OFF, BLOCKED }

70

71	sealed interface Overlay {

72	    data object Attention : Overlay

73	    data class BinEdit(val id: String) : Overlay

74	    data class BinDetail(val id: String) : Overlay

75	    data class Pick(val forWhat: PickFor) : Overlay

76	    data object AddBins : Overlay

77	    data object ClearAlerts : Overlay

78	    data object Wifi : Overlay

79	    data object NotifPre : Overlay

80	    data class PinUnlock(val first: Boolean) : Overlay

81	    data class PinSet(val first: Boolean, val change: Boolean) : Overlay

82	    data object PinChange : Overlay

83	    data class BtCode(val id: String) : Overlay

84	    /** Bins found over Bluetooth, to pick the one to set up. */

85	    data object BtDevices : Overlay

86	}

87

88	/**

89	 * What the screens show besides the bins themselves: the tab, the open sheet,

90	 * the role, the theme, the choices on each page and the edits not saved yet.

91	 * The bins, their readings and the alerts live in [Hub].

92	 */

93	/** A cloud command's answer: confirmed ([ok]) or timed out, and when. */

94	data class Settled(val cmd: String, val ok: Boolean, val at: Long)

95

96	/** How long a key flashes after its cloud command is answered. */

97	const val SETTLE_MILLIS = 1100L

98

99	// Survives screen rotation; recreated when the process restarts.

100	class AppViewModel(app: Application) : AndroidViewModel(app) {

101	    private val store = Hub.store()

102

103	    /** Cloud commands just confirmed or timed out, by bin, while their key flashes. */

104	    val settled = mutableStateMapOf<String, Settled>()

105

106	    init {

107	        // A cloud command's key unlocks when the bin confirms it, or when it waited too long.

108	        viewModelScope.launch {

109	            Hub.events.collect { e ->

110	                if (e is HubEvent.CommandResult) {

111	                    // The key flashes green or red for a moment, then lets go.

112	                    val s = Settled(e.cmd, e.ok, System.currentTimeMillis())

113	                    settled[e.binId] = s

114	                    launch {

115	                        delay(SETTLE_MILLIS)

116	                        if (settled[e.binId] == s) settled.remove(e.binId)

117	                    }

118	                    if (e.ok) {

119	                        Feel.event(Sfx.CMD_OK)

120	                        say("${e.name} confirmed ${e.cmd}.")

121	                    } else {

122	                        Feel.event(Sfx.CMD_FAIL)

123	                        say(e.why?.let { "${e.name} refused ${e.cmd}: $it." } ?: "${e.name} did not confirm ${e.cmd}. Check the bin, then try again.")

124	                    }

125	                }

126	            }

127	        }

128	    }

129

130	    // ---------- whole app ----------

131

132	    // Every start opens on the start page with no role picked. Operator is never

133	    // remembered, and a return after a while away drops it to collector.

134	    var role by mutableStateOf<Role?>(null)

135	        private set

136	    val lock = OperatorLock(store)

137	    val operator get() = role != Role.COLLECTOR

138	    var screen by mutableStateOf(Screen.OVERVIEW)

139	    /** The tab the gear key was pressed on, so back returns there. */

140	    private var beforeApp = Screen.OVERVIEW

141	    /**

142	     * False on the start page. A new view model means the app was closed or

143	     * Android ended it, so the start page shows again. Coming back from the

144	     * background keeps this one, and the page the user left.

145	     */

146	    var started by mutableStateOf(false)

147

148	    /** Back to the start page, where demo mode is turned on or off. Operator needs the PIN again after. */

149	    fun toStart() {

150	        if (role == Role.OPERATOR) Feel.event(Sfx.OP_OUT)

151	        if (editing) viewModelScope.launch { Hub.setCalibration(null) }

152	        edit = null

153	        dirty = false

154	        overlay = null

155	        fullscreen = null

156	        screen = Screen.OVERVIEW

157	        role = null

158	        started = false

159	        syncInMain()

160	    }

161

162	    /** The DEMO key on the start page. */

163	    fun toggleDemo() {

164	        val on = !Hub.state.value.demoOn

165	        Hub.setDemo(on)

166	        demoSpeed = Demo.speed

167	        demoTick++

168	        Feel.event(if (on) Sfx.DEMO else Sfx.SWITCH_OFF)

169	    }

170

171	    /** Tells [Hub] whether the user is inside the main screens, which is when alerts may be posted. */

172	    fun syncInMain() {

173	        Hub.inMain = started && role != null

174	    }

175

176	    override fun onCleared() {

177	        // The app was closed for good (back out of it, or the activity finished).

178	        Hub.inMain = false

179	        ble.release()

180	        super.onCleared()

181	    }

182	    var overlay by mutableStateOf<Overlay?>(null)

183	    var theme by mutableStateOf(runCatching { ThemeMode.valueOf(store.string(SmartBinApp.KEY_THEME, "AUTO")) }.getOrDefault(ThemeMode.AUTO))

184	        private set

185	    var toast by mutableStateOf<String?>(null)

186	        private set

187	    var toastSeq by mutableIntStateOf(0)

188	        private set

189	    private var toastJob: Job? = null

190

191	    fun say(message: String) {

192	        toast = message

193	        toastSeq++

194	        toastJob?.cancel()

195	        toastJob = viewModelScope.launch {

196	            delay(2200)

197	            toast = null

198	        }

199	    }

200

201	    /** Any touch on the screen clears the notice, so it never sits over a key. */

202	    fun hideToast() {

203	        if (toast == null) return

204	        toastJob?.cancel()

205	        toast = null

206	    }

207

208	    fun go(s: Screen) {

209	        overlay = null

210	        fullscreen = null

211	        if (s == Screen.APP && screen.ordinal < Screen.APP.ordinal) beforeApp = screen

212	        screen = if (s == Screen.SIM && !operator) Screen.OVERVIEW else s

213	    }

214

215	    /** Back from a page: Bluetooth and Connection go to the gear page, the gear page to its tab, a tab to Overview. */

216	    fun back() = when (screen) {

217	        Screen.BT, Screen.NET -> go(Screen.APP)

218	        Screen.APP -> go(beforeApp)

219	        else -> go(Screen.OVERVIEW)

220	    }

221

222	    fun toggleTheme(nowDark: Boolean) = setThemeMode(if (nowDark) ThemeMode.LIGHT else ThemeMode.DARK)

223

224	    fun setThemeMode(m: ThemeMode) {

225	        theme = m

226	        store.putString(SmartBinApp.KEY_THEME, m.name)

227	    }

228

229	    /** Asking for operator opens the PIN sheet. Collector needs nothing. */

230	    fun pickRole(r: Role, first: Boolean = false) {

231	        if (r == role && !first) return

232	        if (r == Role.OPERATOR) {

233	            opPin = ""; pinNew = ""; pinMsg = null

234	            overlay = if (lock.hasPin) Overlay.PinUnlock(first) else Overlay.PinSet(first, change = false)

235	            return

236	        }

237	        setRole(r, first)

238	    }

239

240	    private fun setRole(r: Role, first: Boolean) {

241	        val had = role != null

242	        if (role == Role.OPERATOR && r == Role.COLLECTOR) Feel.event(Sfx.OP_OUT)

243	        role = r

244	        started = true

245	        syncInMain()

246	        // Only "a role was picked" is kept; the next start is always collector.

247	        store.putString(SmartBinApp.KEY_ROLE, Role.COLLECTOR.name)

248	        if (r == Role.COLLECTOR) {

249	            if (editing) viewModelScope.launch { Hub.setCalibration(null) }

250	            edit = null

251	            dirty = false

252	            if (screen == Screen.SIM) screen = Screen.OVERVIEW

253	        }

254	        if (first || !had) {

255	            screen = Screen.OVERVIEW

256	            if (notifications != Perm.ON && !store.bool(SmartBinApp.KEY_ASKED, false)) overlay = Overlay.NotifPre

257	        } else {

258	            say("Signed in as ${if (r == Role.OPERATOR) "operator" else "collector"}.")

259	        }

260	    }

261

262	    // ---------- operator PIN ----------

263

264	    /** The digits typed so far, the first entry of a new PIN, and the line under the dots. */

265	    var opPin by mutableStateOf("")

266	    var pinNew by mutableStateOf("")

267	    var pinMsg by mutableStateOf<Pair<Boolean, String>?>(null)

268	    /** Set when the app went to the background, to drop operator after AUTO_LOCK. */

269	    private var awaySince = 0L

270

271	    fun pinKey(d: Char) {

272	        if (opPin.length < OperatorLock.MAX) opPin += d

273	    }

274

275	    fun pinBack() {

276	        opPin = opPin.dropLast(1)

277	    }

278

279	    fun pinUnlock(first: Boolean) {

280	        val wait = lock.waitLeft()

281	        if (wait > 0) {

282	            pinMsg = false to "Too many wrong PINs. Try again in ${(wait + 999) / 1000} s."

283	            return

284	        }

285	        if (lock.check(opPin)) {

286	            overlay = null

287	            opPin = ""

288	            Feel.event(Sfx.OP_IN)

289	            setRole(Role.OPERATOR, first)

290	        } else {

291	            opPin = ""

292	            val left = lock.waitLeft()

293	            Feel.event(if (left > 0) Sfx.PIN_LOCKOUT else Sfx.PIN_WRONG)

294	            pinMsg = false to if (left > 0) "Wrong PIN. Locked for ${(left + 999) / 1000} s." else "Wrong PIN. ${lock.triesLeft()} tries before a wait."

295	        }

296	    }

297

298	    /** Making or changing the PIN: type it, then type it again. */

299	    fun pinSave(first: Boolean, change: Boolean) {

300	        if (!OperatorLock.valid(opPin)) {

301	            pinMsg = false to "Use ${OperatorLock.MIN} to ${OperatorLock.MAX} digits."

302	            return

303	        }

304	        if (pinNew.isEmpty()) {

305	            pinNew = opPin

306	            opPin = ""

307	            pinMsg = true to "Type the same PIN again."

308	            return

309	        }

310	        if (opPin != pinNew) {

311	            opPin = ""; pinNew = ""

312	            Feel.event(Sfx.PIN_WRONG)

313	            pinMsg = false to "The two PINs differ. Start again."

314	            return

315	        }

316	        lock.set(opPin)

317	        opPin = ""; pinNew = ""; pinMsg = null

318	        overlay = null

319	        Feel.event(if (change) Sfx.PIN_SET else Sfx.OP_IN)

320	        if (change) say("Operator PIN changed.") else setRole(Role.OPERATOR, first)

321	    }

322

323	    /** Changing the PIN asks for the old one first. */

324	    fun changePin() {

325	        opPin = ""; pinNew = ""; pinMsg = null

326	        overlay = Overlay.PinChange

327	    }

328

329	    fun pinChangeCheck() {

330	        val wait = lock.waitLeft()

331	        if (wait > 0) {

332	            pinMsg = false to "Too many wrong PINs. Try again in ${(wait + 999) / 1000} s."

333	            return

334	        }

335	        if (!lock.check(opPin)) {

336	            opPin = ""

337	            val left = lock.waitLeft()

338	            Feel.event(if (left > 0) Sfx.PIN_LOCKOUT else Sfx.PIN_WRONG)

339	            pinMsg = false to if (left > 0) "Wrong PIN. Locked for ${(left + 999) / 1000} s." else "Wrong PIN. ${lock.triesLeft()} tries before a wait."

340	            return

341	        }

342	        opPin = ""; pinMsg = null

343	        overlay = Overlay.PinSet(first = false, change = true)

344	    }

345

346	    fun backgrounded() {

347	        awaySince = System.currentTimeMillis()

348	    }

349

350	    /** Back after more than AUTO_LOCK away, operator drops to collector. */

351	    fun foregrounded() {

352	        val away = awaySince

353	        awaySince = 0L

354	        if (away > 0 && role == Role.OPERATOR && System.currentTimeMillis() - away > AUTO_LOCK) {

355	            setRole(Role.COLLECTOR, first = false)

356	            say("Operator locked after 2 min away. Collector mode.")

357	        }

358	    }

359

360	    fun openFromNotification() {

361	        if (role == null) return

362	        started = true

363	        overlay = null

364	        fullscreen = null

365	        screen = Screen.OVERVIEW

366	    }

367

368	    // ---------- sound and feel, from Garbage-Guard ----------

369

370	    var sounds by mutableStateOf(Feel.sounds)

371	        private set

372	    var haptics by mutableStateOf(Feel.haptics)

373	        private set

374	    var strength by mutableStateOf(Haptics.strength)

375	        private set

376	    var keepAwake by mutableStateOf(store.bool(SmartBinApp.KEY_AWAKE, false))

377	        private set

378

379	    fun toggleSounds() {

380	        sounds = !sounds

381	        Feel.sounds = sounds

382	        store.putBool(SmartBinApp.KEY_SOUNDS, sounds)

383	    }

384

385	    fun toggleHaptics() {

386	        haptics = !haptics

387	        Feel.haptics = haptics

388	        store.putBool(SmartBinApp.KEY_HAPTICS, haptics)

389	        if (haptics) Haptics.play(Haptics.Buzz.SAVED)

390	    }

391

392	    fun chooseStrength(s: Haptics.Strength) {

393	        strength = s

394	        Haptics.strength = s

395	        store.putString(SmartBinApp.KEY_STRENGTH, s.name)

396	        Haptics.play(Haptics.Buzz.SAVED)

397	    }

398

399	    fun toggleAwake() {

400	        keepAwake = !keepAwake

401	        store.putBool(SmartBinApp.KEY_AWAKE, keepAwake)

402	    }

403

404	    // ---------- battery ----------

405

406	    /** True when Android lets the app run in the background with no battery limits. */

407	    var unrestricted by mutableStateOf(batteryFree())

408	        private set

409

410	    private fun batteryFree(): Boolean {

411	        val ctx = getApplication<Application>()

412	        return ctx.getSystemService(PowerManager::class.java)?.isIgnoringBatteryOptimizations(ctx.packageName) == true

413	    }

414

415	    /** Asks Android for unrestricted battery, or opens the list when the phone has no such prompt. */

416	    fun askUnrestricted() {

417	        val ctx = getApplication<Application>()

418	        val ask = Intent(Settings.ACTION_REQUEST_IGNORE_BATTERY_OPTIMIZATIONS, Uri.parse("package:${ctx.packageName}"))

419	        val list = Intent(Settings.ACTION_IGNORE_BATTERY_OPTIMIZATION_SETTINGS)

420	        val ok = runCatching { ctx.startActivity(ask.addFlags(Intent.FLAG_ACTIVITY_NEW_TASK)) }.isSuccess ||

421	            runCatching { ctx.startActivity(list.addFlags(Intent.FLAG_ACTIVITY_NEW_TASK)) }.isSuccess

422	        if (!ok) say("This phone has no battery setting for apps.")

423	    }

424

425	    /** Opens the app's own page in Android settings, where some phones keep the battery choice. */

426	    fun openAppInfo() {

427	        val ctx = getApplication<Application>()

428	        runCatching {

429	            ctx.startActivity(Intent(Settings.ACTION_APPLICATION_DETAILS_SETTINGS, Uri.parse("package:${ctx.packageName}")).addFlags(Intent.FLAG_ACTIVITY_NEW_TASK))

430	        }

431	    }

432

433	    // ---------- push alerts ----------

434

435	    var notifications by mutableStateOf(if (Alerts.allowed(app)) Perm.ON else Perm.OFF)

436	        private set

437

438	    fun refreshSystemState() {

439	        unrestricted = batteryFree()

440	        val on = Alerts.allowed(getApplication())

441	        notifications = when {

442	            on -> Perm.ON

443	            store.bool(SmartBinApp.KEY_ASKED, false) -> Perm.BLOCKED

444	            else -> Perm.OFF

445	        }

446	    }

447

448	    fun permissionAnswered(granted: Boolean) {

449	        store.putBool(SmartBinApp.KEY_ASKED, true)

450	        notifications = if (granted) Perm.ON else Perm.BLOCKED

451	        say(if (granted) "Push alerts are on." else "Push alerts are off. You can change this in Settings.")

452	    }

453

454	    fun notNow() {

455	        store.putBool(SmartBinApp.KEY_ASKED, true)

456	        overlay = null

457	    }

458

459	    // ---------- overview ----------

460

461	    var binView by mutableStateOf(BinView.AUTO)

462	    var binFilter by mutableStateOf(BinFilter.ALL)

463	    var prioAll by mutableStateOf(false)

464	    var addN by mutableIntStateOf(1)

465

466	    // LOCK, UNLOCK, COLLECT, COLLECT_DONE and CLEAR_TAMPER from the bin card.

467	    // Over local Wi-Fi the result is immediate. Over Firebase or ThingSpeak the

468	    // command waits in hub.pending until the bin confirms it or times out; the

469	    // key then flashes green or red for SETTLE_MILLIS (see init above).

470	    fun command(id: String, cmd: String) {

471	        viewModelScope.launch {

472	            val b = Hub.bin(id) ?: return@launch

473	            if (b.demo) Hub.pauseTour()

474	            val problem = Hub.command(id, cmd)

475	            if (problem != null) {

476	                Feel.event(Sfx.DENIED)

477	                say(problem)

478	                return@launch

479	            }

480	            val n = b.name

481	            val msg = when (cmd) {

482	                "LOCK" -> "$n is locked."

483	                "UNLOCK" -> "$n is unlocked."

484	                "CLEAR_TAMPER" -> "Tamper reset on $n."

485	                "COLLECT" -> "Pickup started on $n. The lid is open."

486	                "COLLECT_DONE" -> "$n is empty and back to normal."

487	                else -> "Sent to $n."

488	            }

489	            val waiting = Hub.state.value.pending[id]

490	            if (waiting != null) {

491	                val via = if (waiting.via == Source.THINGSPEAK) "ThingSpeak" else "Firebase"

492	                Feel.event(Sfx.CLOUD)

493	                say("${waiting.cmd} sent to $n via $via. Waiting for the bin.")

494	            } else say(msg)

495	        }

496	    }

497

498	    // Bin edit sheet

499	    var formName by mutableStateOf("")

500	    var formPlace by mutableStateOf("")

501	    var formIp by mutableStateOf("")

502	    var formCloud by mutableStateOf("")

503	    var formTsCh by mutableStateOf("")

504	    var formTsKey by mutableStateOf("")

505	    var formTbId by mutableStateOf("")

506	    var formTbKey by mutableStateOf("")

507

508	    fun openBinEdit(b: Bin) {

509	        formName = b.name

510	        formPlace = b.place

511	        formIp = b.ip

512	        formCloud = b.cloudId

513	        formTsCh = b.tsChannel

514	        formTsKey = b.tsKey

515	        formTbId = b.tbId

516	        formTbKey = b.tbKey

517	        overlay = Overlay.BinEdit(b.id)

518	    }

519

520	    fun saveBinEdit(id: String) {

521	        Hub.editBin(id, formName, formPlace, formIp, formCloud, formTsCh, formTsKey, formTbId, formTbKey)

522	        overlay = null

523	        say(

524	            "Saved ${Hub.bin(id)?.name ?: "the bin"}." + when {

525	                formIp.isNotBlank() -> " Reading it over Wi-Fi now."

526	                formTsCh.isNotBlank() -> " Reading it from ThingSpeak now."

527	                else -> ""

528	            }

529	        )

530	    }

531

532	    fun removeBin(id: String) {

533	        val name = Hub.bin(id)?.name ?: "The bin"

534	        if (calibBin() == id) {

535	            edit = null

536	            dirty = false

537	        }

538	        Hub.removeBin(id)

539	        overlay = null

540	        say("$name removed.")

541	    }

542

543	    fun addBins() {

544	        val n = addN

545	        Hub.addBins(n)

546	        overlay = null

547	        say("Added $n bin${if (n > 1) "s" else ""}. Tap SET to give each one an address.")

548	    }

549

550	    // ---------- charts ----------

551

552	    /** null means all bins. */

553	    var chartBin by mutableStateOf<String?>(null)

554	    var envBin by mutableStateOf<String?>(null)

555	    var envMode by mutableStateOf(EnvMode.ALL)

556	    var rangeMin by mutableIntStateOf(60)

557	    /** "fill" or "env" while the sideways chart is open. */

558	    var fullscreen by mutableStateOf<String?>(null)

559

560	    // ---------- settings ----------

561

562	    var selSet by mutableStateOf<String?>(null)

563	    var selSim by mutableStateOf<String?>(null)

564	    var edit by mutableStateOf<Cal?>(null)

565	        private set

566	    var dirty by mutableStateOf(false)

567	        private set

568	    /** Settings groups that are open. All start closed, so the one needed is a short scroll away. */

569	    var open by mutableStateOf(emptySet<String>())

570	    var pinShown by mutableStateOf(false)

571	    var tref by mutableStateOf(27.5)

572	    var known by mutableStateOf(1000.0)

573

574	    private fun calibBin() = Hub.state.value.calibBin

575	    val editing get() = edit != null && calibBin() != null

576

577	    fun binFor(id: String?, bins: List<Bin>): Bin? = bins.firstOrNull { it.id == id } ?: bins.firstOrNull()

578

579	    fun toggleSection(k: String) {

580	        open = if (k in open) open - k else open + k

581	    }

582

583	    // Calibration mode on one bin at a time. edit is a local copy of the bin's

584	    // Cal; nothing reaches the bin until save(). Turning calibration off drops

585	    // unsaved edits.

586	    fun setCalibration(on: Boolean, b: Bin) {

587	        viewModelScope.launch {

588	            if (on) {

589	                Hub.setCalibration(b.id)

590	                edit = b.cal

591	                dirty = false

592	                Feel.event(Sfx.CALIB)

593	                say("Calibration mode on for ${b.name}.")

594	            } else {

595	                val lost = dirty

596	                Hub.setCalibration(null)

597	                edit = null

598	                dirty = false

599	                say(if (lost) "Calibration off. Unsaved changes were dropped." else "Calibration off. ${b.name} is back to normal.")

600	            }

601	        }

602	    }

603

604	    fun step(key: String, dir: Int) {

605	        val f = FIELDS.getValue(key)

606	        when (key) {

607	            "tref" -> tref = round2((tref + f.step * dir).coerceIn(f.min, f.max))

608	            "known" -> known = round2((known + f.step * dir).coerceIn(f.min, f.max))

609	            else -> {

610	                val e = edit ?: return

611	                edit = e + (key to round2((e.v(key) + f.step * dir).coerceIn(f.min, f.max)))

612	                dirty = true

613	            }

614	        }

615	    }

616

617	    fun setPin(text: String) {

618	        val e = edit ?: return

619	        val digits = text.filter { it.isDigit() }.take(4)

620	        edit = e + ("collect_pin" to (digits.toIntOrNull() ?: 0).toDouble())

621	        dirty = true

622	    }

623

624	    fun toggleAutoLock() {

625	        val e = edit ?: return

626	        edit = e + ("auto_lock" to if (e.v("auto_lock") > 0) 0.0 else 1.0)

627	        dirty = true

628	    }

629

630	    fun useNow(key: String, b: Bin) {

631	        val e = edit ?: return

632	        var v = Math.round(b.reading.distance * 10) / 10.0

633	        if (key == "d_full") v = minOf(v, e.v("d_empty") - 1)

634	        edit = e + (key to v)

635	        dirty = true

636	        say("Took ${fmt(v, 1)} cm from the sensor.")

637	    }

638

639	    fun setOffset(b: Bin) {

640	        val e = edit ?: return

641	        val off = Math.round((tref - b.reading.tempRaw).coerceIn(-10.0, 10.0) * 10) / 10.0

642	        edit = e + ("temp_off" to off)

643	        dirty = true

644	        say("Offset set to ${fmt(off, 1)} C. Press SAVE to keep it.")

645	    }

646

647	    fun binCommand(b: Bin, cmd: String, extra: Map<String, Any>, ok: String) {

648	        viewModelScope.launch {

649	            if (b.demo) Hub.pauseTour()

650	            val problem = Hub.command(b.id, cmd, extra)

651	            if (problem != null) Feel.event(Sfx.DENIED)

652	            say(problem ?: ok)

653	        }

654	    }

655

656	    /** The bin's touchscreen: four crosses to press on the bin, or back to the firmware's values. Wi-Fi only. */

657	    fun touchCal(b: Bin, reset: Boolean) {

658	        if (!b.demo && b.source != Source.LIVE) {

659	            Feel.event(Sfx.DENIED)

660	            say("Touch calibration needs ${b.name} on the same Wi-Fi as the phone.")

661	            return

662	        }

663	        Feel.event(if (reset) Sfx.CONFIRM else Sfx.CALIB)

664	        binCommand(

665	            b, if (reset) "TOUCH_CAL_RESET" else "TOUCH_CAL", emptyMap(),

666	            if (reset) "${b.name}'s touchscreen is back to the default values."

667	            else "${b.name} shows 4 crosses. Press the centre of each one on the bin.",

668	        )

669	    }

670

671	    /** Gives every bin on Wi-Fi the saved Firebase account, so they write to the same database. */

672	    fun sendCloudToBins() {

673	        if (!operator) {

674	            Feel.event(Sfx.DENIED)

675	            say("Only an operator can send the cloud account to the bins.")

676	            return

677	        }

678	        viewModelScope.launch {

679	            val (ok, tried) = Hub.sendCloudToBins()

680	            when {

681	                tried == 0 -> { Feel.event(Sfx.DENIED); say("No bin is on Wi-Fi with an address, so nothing was sent.") }

682	                ok == tried -> { Feel.event(Sfx.SAVED); say("Sent the cloud account to $ok bin${if (ok == 1) "" else "s"}.") }

683	                else -> { Feel.event(Sfx.CMD_FAIL); say("$ok of $tried bins took the cloud account. Check the others are on.") }

684	            }

685	        }

686	    }

687

688	    // Sends the edited calibration to the bin. The full distance must be

689	    // shorter than the empty distance, the same check the firmware makes.

690	    fun save(b: Bin) {

691	        val e = edit ?: return

692	        if (e.v("d_full") >= e.v("d_empty")) {

693	            Feel.event(Sfx.DENIED)

694	            say("Distance full must be smaller than distance empty.")

695	            return

696	        }

697	        viewModelScope.launch {

698	            val problem = Hub.saveCal(b.id, e)

699	            if (problem != null) {

700	                Feel.event(Sfx.DENIED)

701	                say(problem)

702	            } else {

703	                dirty = false

704	                Feel.event(Sfx.SAVED)

705	                say("Saved to ${b.name}.")

706	            }

707	        }

708	    }

709

710	    fun cancel(b: Bin) {

711	        edit = b.cal

712	        dirty = false

713	        say("Changes dropped.")

714	    }

715

716	    fun reload(b: Bin) {

717	        viewModelScope.launch {

718	            if (b.ip.isBlank()) {

719	                say(if (b.demo) "${b.name} is a demo bin." else "Give ${b.name} an address first.")

720	                return@launch

721	            }

722	            val problem = Hub.reload(b.id)

723	            if (problem == null && editing && calibBin() == b.id) {

724	                edit = Hub.bin(b.id)?.cal

725	                dirty = false

726	            }

727	            say(problem ?: "Read the latest values from ${b.name}.")

728	        }

729	    }

730

731	    /** Writes the bins' names, places and addresses to Downloads, or offers to share them on older phones. */

732	    fun exportBins(bins: List<Bin>) {

733	        val a = JSONArray()

734	        bins.forEach { b -> a.put(JSONObject().put("name", b.name).put("place", b.place).put("ip", b.ip).put("cloud_id", b.cloudId)) }

735	        val text = a.toString(2)

736	        val ctx = getApplication<Application>()

737	        if (Build.VERSION.SDK_INT >= 29) {

738	            val ok = runCatching {

739	                val values = ContentValues().apply {

740	                    put(MediaStore.Downloads.DISPLAY_NAME, "smartbin-bins.json")

741	                    put(MediaStore.Downloads.MIME_TYPE, "application/json")

742	                }

743	                val uri = ctx.contentResolver.insert(MediaStore.Downloads.EXTERNAL_CONTENT_URI, values) ?: error("no file")

744	                ctx.contentResolver.openOutputStream(uri)?.use { it.write(text.toByteArray()) }

745	            }.isSuccess

746	            say(if (ok) "Bin list saved to Downloads." else "The bin list could not be saved.")

747	        } else {

748	            val send = Intent(Intent.ACTION_SEND).setType("application/json").putExtra(Intent.EXTRA_TEXT, text)

749	            runCatching { ctx.startActivity(Intent.createChooser(send, "Export bin list").addFlags(Intent.FLAG_ACTIVITY_NEW_TASK)) }

750	        }

751	    }

752

753	    // ---------- Bluetooth Wi-Fi setup ----------

754

755	    /** 0 not paired, 1 paired, 2 joining, 3 joined, 4 Bluetooth off on the bin. */

756	    var bt by mutableIntStateOf(0)

757	    var ssid by mutableStateOf("")

758	    var pass by mutableStateOf("")

759	    var showPass by mutableStateOf(false)

760	    /** What is happening now, under the keys. */

761	    var btNote by mutableStateOf<String?>(null)

762	    var btBusy by mutableStateOf(false)

763	    /** The bins found nearby, and whether the search still runs. */

764	    val btFound = androidx.compose.runtime.mutableStateListOf<BinBle.Found>()

765	    var btScanning by mutableStateOf(false)

766	    /** The networks the bin can see. */

767	    var btNets by mutableStateOf<List<BinBle.Net>>(emptyList())

768	    /** The address and network the bin joined. */

769	    var btIp by mutableStateOf("")

770	    var btJoined by mutableStateOf("")

771	    /** The bin being set up: its Bluetooth name, and whether it is the demo stand-in. */

772	    var btName by mutableStateOf("")

773	    var btDemo by mutableStateOf(false)

774	    /** Set when the phone and the bin disagree about an old pairing. */

775	    var btStale by mutableStateOf(false)

776

777	    private val ble by lazy { BinBle(getApplication()) }

778	    private var btJob: Job? = null

779	    private var btWait: Job? = null

780

781	    /**

782	     * Pairing a demo bin uses a stand-in for its screen: a fresh random code

783	     * each time, gone after 60 s, and 3 wrong codes stop pairing for 60 s, the

784	     * same rules the bin's firmware keeps.

785	     */

786	    var btCode by mutableStateOf("")

787	        private set

788	    var btCodeAt by mutableLongStateOf(0L)

789	        private set

790	    var btEntry by mutableStateOf("")

791	    var btMsg by mutableStateOf<Pair<Boolean, String>?>(null)

792	    private var btFails = 0

793	    private var btWaitUntil = 0L

794

795	    fun btPermissions() = ble.permissions()

796	    fun btAllowed() = ble.allowed()

797

798	    private fun btReset() {

799	        btWait?.cancel()

800	        bt = 0

801	        btIp = ""

802	        btJoined = ""

803	        btNets = emptyList()

804	        btStale = false

805	    }

806

807	    /** PAIR A BIN. A demo bin walks through the stand-in; anything else looks for real bins nearby. */

808	    fun btPair(b: Bin?) {

809	        btReset()

810	        if (b != null && b.demo) {

811	            val wait = btWaitUntil - System.currentTimeMillis()

812	            if (wait > 0) {

813	                Feel.event(Sfx.DENIED)

814	                say("${b.name} stopped pairing after 3 wrong codes. Try again in ${(wait + 999) / 1000} s.")

815	                return

816	            }

817	            btDemo = true

818	            btName = b.name

819	            btFails = 0

820	            btEntry = ""

821	            btMsg = null

822	            btNewCode()

823	            overlay = Overlay.BtCode(b.id)

824	            return

825	        }

826	        btDemo = false

827	        when {

828	            !ble.present -> { Feel.event(Sfx.DENIED); btNote = "This phone has no Bluetooth." }

829	            !ble.allowed() -> { Feel.event(Sfx.DENIED); btNote = "Allow Nearby devices (Bluetooth) for Smart-Bin, then press PAIR A BIN again." }

830	            !ble.on -> { Feel.event(Sfx.DENIED); btNote = "Turn Bluetooth on, then press PAIR A BIN again." }

831	            else -> btSearch()

832	        }

833	    }

834

835	    /** Looks for bins for 8 s and lists them in a sheet. */

836	    fun btSearch() {

837	        btFound.clear()

838	        btScanning = true

839	        overlay = Overlay.BtDevices

840	        btNote = "Looking for bins nearby. On the bin, tap i, then WI-FI SETUP, and type the Collect PIN."

841	        btJob?.cancel()

842	        btJob = viewModelScope.launch {

843	            try {

844	                ble.scan(8_000) { f -> if (btFound.none { it.address == f.address }) btFound.add(f) }

845	            } catch (e: Exception) {

846	                if (e is kotlinx.coroutines.CancellationException) throw e

847	                btNote = e.message ?: "Could not look for bins."

848	            } finally {

849	                btScanning = false

850	            }

851	            if (btFound.isEmpty()) btNote = "No bin found. Check that the bin shows WI-FI SETUP and stands close by" +

852	                (if (Build.VERSION.SDK_INT < 31) ", and that Location is on." else ".")

853	        }

854	    }

855

856	    /** A bin picked in the sheet: connect, pair with the code Android asks for, then read its networks. */

857	    fun btConnect(f: BinBle.Found) {

858	        overlay = null

859	        btJob?.cancel()

860	        btName = f.name

861	        btBusy = true

862	        btNote = "Connecting to ${f.name}..."

863	        btJob = viewModelScope.launch {

864	            try {

865	                ble.onDrop = { viewModelScope.launch { btDropped() } }

866	                ble.onStat = { j -> viewModelScope.launch { btStatus(j) } }

867	                ble.connect(f.device)

868	                btNote = "${f.name} shows a 6-digit code now. Type it in the box Android opens. The code lasts 60 s."

869	                ble.secure()

870	                ble.listen()

871	                Feel.event(Sfx.BT_PAIRED)

872	                bt = 1

873	                btNote = "Paired with ${f.name}. The link is encrypted. Pick the network next."

874	                btLoadNets()

875	            } catch (e: kotlinx.coroutines.CancellationException) {

876	                throw e

877	            } catch (e: BinBle.StaleBond) {

878	                ble.close()

879	                btStale = true

880	                Feel.event(Sfx.BT_WRONG)

881	                btNote = "${f.name} keeps dropping the link. The phone most likely remembers it from an earlier pairing. Press FORGET AND PAIR AGAIN, then on the bin open WI-FI SETUP again."

882	            } catch (e: Exception) {

883	                ble.close()

884	                Feel.event(Sfx.BT_WRONG)

885	                btNote = "Pairing did not finish (${e.message}). If the code was wrong, the bin shows a new one; after 3 wrong codes it waits 60 s."

886	            } finally {

887	                btBusy = false

888	            }

889	        }

890	    }

891

892	    /** Makes the phone forget the bin, then searches again. Opens Android's Bluetooth settings if that is not allowed. */

893	    fun btForget() {

894	        val d = ble.device ?: btFound.firstOrNull { it.name == btName }?.device

895	        btStale = false

896	        if (d != null && ble.forget(d)) {

897	            Feel.event(Sfx.CONFIRM)

898	            say("The phone forgot $btName. Pair it again.")

899	            btSearch()

900	        } else {

901	            say("Remove $btName in Android's Bluetooth settings, then press PAIR A BIN.")

902	            runCatching {

903	                getApplication<Application>().startActivity(Intent(Settings.ACTION_BLUETOOTH_SETTINGS).addFlags(Intent.FLAG_ACTIVITY_NEW_TASK))

904	            }

905	        }

906	    }

907

908	    /** The bin scans when WI-FI SETUP opens; an empty list means it is not done, so it asks again after 5 s. */

909	    private suspend fun btLoadNets() {

910	        repeat(3) { k ->

911	            val nets = runCatching { ble.networks() }.getOrDefault(emptyList())

912	            if (nets.isNotEmpty()) {

913	                btNets = nets

914	                if (ssid.isBlank()) ssid = nets.first().ssid

915	                return

916	            }

917	            if (k < 2) {

918	                btNote = "The bin is still looking for networks. Asking again in 5 s. You can also type the name."

919	                delay(5_000)

920	                if (bt != 1) return

921	            }

922	        }

923	        btNote = "The bin found no networks. Check the router is on 2.4 GHz and close by, press SCAN, or type the name."

924	    }

925

926	    fun btScanNets() {

927	        if (btDemo) {

928	            overlay = Overlay.Wifi

929	            return

930	        }

931	        viewModelScope.launch {

932	            btBusy = true

933	            btLoadNets()

934	            btBusy = false

935	            if (btNets.isNotEmpty()) overlay = Overlay.Wifi

936	        }

937	    }

938

939	    fun btNewCode() {

940	        btCode = "%06d".format(java.security.SecureRandom().nextInt(1_000_000))

941	        btCodeAt = System.currentTimeMillis()

942	    }

943

944	    fun btKey(d: Char) {

945	        if (btEntry.length < BT_CODE_LEN) btEntry += d

946	    }

947

948	    fun btBack() {

949	        btEntry = btEntry.dropLast(1)

950	    }

951

952	    fun btCheck(b: Bin) {

953	        if (System.currentTimeMillis() - btCodeAt > BT_CODE_LIFE) {

954	            btNewCode()

955	            btEntry = ""

956	            Feel.event(Sfx.BT_WRONG)

957	            btMsg = false to "That code expired. The bin shows a new one."

958	            return

959	        }

960	        if (btEntry == btCode) {

961	            overlay = null

962	            btEntry = ""

963	            btCode = ""

964	            bt = 1

965	            btNets = DEMO_NETS

966	            Feel.event(Sfx.BT_PAIRED)

967	            btNote = "Paired with ${b.name} (demo). Pick a network. A name with \"fail\" in it shows what a failed join looks like."

968	            return

969	        }

970	        btEntry = ""

971	        btFails++

972	        if (btFails >= 3) {

973	            overlay = null

974	            btCode = ""

975	            btWaitUntil = System.currentTimeMillis() + 60_000L

976	            Feel.event(Sfx.BT_BLOCKED)

977	            say("3 wrong codes. ${b.name} stops pairing for 60 s.")

978	        } else {

979	            val left = 3 - btFails

980	            Feel.event(Sfx.BT_WRONG)

981	            btMsg = false to "Wrong code. $left ${if (left == 1) "try" else "tries"} left."

982	        }

983	    }

984

985	    fun btCancel() {

986	        overlay = null

987	        btEntry = ""

988	        btCode = ""

989	    }

990

991	    /** SEND WI-FI: the bin tries the network for up to 15 s and keeps its old one if it fails. */

992	    fun btSend() {

993	        val name = ssid.trim()

994	        when {

995	            name.isBlank() -> { Feel.event(Sfx.DENIED); say("Pick or type the Wi-Fi name first."); return }

996	            name.length > 32 || pass.length > 63 -> { Feel.event(Sfx.DENIED); say("The name can be 32 characters at most and the password 63."); return }

997	            pass.isNotEmpty() && pass.length < 8 -> { Feel.event(Sfx.DENIED); say("A Wi-Fi password has at least 8 characters. Leave it empty only for an open network."); return }

998	        }

999	        bt = 2

1000	        btNote = "Sending the network to $btName and waiting for it to join..."

1001	        btWait?.cancel()

1002	        if (btDemo) {

1003	            btWait = viewModelScope.launch {

1004	                delay(1800)

1005	                if (name.contains("fail", true)) btStatus(JSONObject().put("state", "FAILED").put("why", "Network not found"))

1006	                else btStatus(JSONObject().put("state", "CONNECTED").put("ip", "192.168.1.${101 + (Hub.state.value.bins.indexOfFirst { it.id == selSet }.coerceAtLeast(0))}").put("ssid", name))

1007	            }

1008	            return

1009	        }

1010	        btWait = viewModelScope.launch {

1011	            try {

1012	                ble.join(name, pass)

1013	            } catch (e: kotlinx.coroutines.CancellationException) {

1014	                throw e

1015	            } catch (e: Exception) {

1016	                bt = 1

1017	                Feel.event(Sfx.CMD_FAIL)

1018	                btNote = "Could not send (${e.message}). Press SEND WI-FI again."

1019	                return@launch

1020	            }

1021	            // Notifications can be missed or cut short, so the status is read every 2 s too.

1022	            val end = System.currentTimeMillis() + 25_000

1023	            while (bt == 2 && System.currentTimeMillis() < end) {

1024	                delay(2_000)

1025	                if (bt == 2 && ble.connected) runCatching { ble.readStat() }.getOrNull()?.let { btStatus(it) }

1026	            }

1027	            if (bt == 2) {

1028	                bt = 1

1029	                Feel.event(Sfx.WIFI_FAILED)

1030	                btNote = "No answer from the bin after 25 s. Its touchscreen shows whether it joined. Press SEND WI-FI again, or PAIR A BIN."

1031	            }

1032	        }

1033	    }

1034

1035	    /** One STAT message from the bin. */

1036	    private fun btStatus(j: JSONObject) {

1037	        if (bt != 2) return

1038	        when (j.optString("state").uppercase()) {

1039	            "CONNECTING" -> btNote = "$btName is joining ${ssid.trim()}. This takes up to 15 s."

1040	            "CONNECTED" -> {

1041	                val ip = j.optString("ip")

1042	                if (ip.isBlank()) return

1043	                btWait?.cancel()

1044	                btIp = ip

1045	                btJoined = j.optString("ssid").ifBlank { ssid.trim() }

1046	                bt = 3

1047	                Feel.event(Sfx.WIFI_JOINED)

1048	                btNote = "Press ${btUseLabel()} to keep this address, then TURN BT OFF."

1049	            }

1050	            "FAILED" -> {

1051	                btWait?.cancel()

1052	                bt = 1

1053	                Feel.event(Sfx.WIFI_FAILED)

1054	                btNote = "$btName could not join: ${j.optString("why").ifBlank { "no reason given" }}. It went back to its old Wi-Fi. Check the name and password, then SEND WI-FI again."

1055	            }

1056	        }

1057	    }

1058

1059	    /** The link dropped. While joining, the answer is read again after reconnecting. */

1060	    private suspend fun btDropped() {

1061	        when (bt) {

1062	            1 -> {

1063	                bt = 0

1064	                Feel.event(Sfx.BIN_LOST)

1065	                btNote = "$btName closed the Bluetooth link. On the bin, open WI-FI SETUP again, then PAIR A BIN."

1066	            }

1067	            2 -> {

1068	                btNote = "The link dropped while $btName joins. Reconnecting to read the result..."

1069	                val d = btFound.firstOrNull { it.name == btName }?.device ?: return

1070	                runCatching {

1071	                    ble.connect(d)

1072	                    ble.listen()

1073	                    btStatus(ble.readStat())

1074	                }

1075	            }

1076	        }

1077	    }

1078

1079	    /** The bin being set up is the picked one if it is real; otherwise it becomes a new bin. */

1080	    private fun btTarget(): Bin? = Hub.bin(selSet ?: "")?.takeIf { !it.demo }

1081

1082	    fun btUseLabel(): String = when {

1083	        btDemo -> "USE"

1084	        btTarget() != null -> "USE FOR ${btTarget()!!.name.uppercase()}"

1085	        else -> "ADD AS BIN ${btNumber()}".trim()

1086	    }

1087

1088	    private fun btNumber() = Regex("(\\d+)\\s*$").find(btName)?.groupValues?.get(1) ?: ""

1089

1090	    fun btUse() {

1091	        if (btIp.isBlank()) return

1092	        if (btDemo) {

1093	            Feel.event(Sfx.SAVED)

1094	            say("$btName is a demo bin, so nothing was stored.")

1095	            return

1096	        }

1097	        val t = btTarget()

1098	        if (t != null) {

1099	            Hub.editBin(t.id, t.name, t.place, btIp, t.cloudId)

1100	            say("${t.name} will now be read at $btIp.")

1101	        } else {

1102	            val b = Hub.addRealBin(btNumber().padStart(2, '0').takeIf { btNumber().isNotEmpty() } ?: "", btIp)

1103	            selSet = b.id

1104	            say("Added ${b.name} at $btIp.")

1105	        }

1106	        Feel.event(Sfx.SAVED)

1107	    }

1108

1109	    fun btOff() {

1110	        btWait?.cancel()

1111	        viewModelScope.launch {

1112	            if (!btDemo) ble.off()

1113	            bt = 4

1114	            Feel.event(Sfx.BT_OFF)

1115	            btNote = "Bluetooth is off on $btName. It stays on its Wi-Fi until someone opens WI-FI SETUP again."

1116	        }

1117	    }

1118

1119	    // ---------- simulation ----------

1120

1121	    fun simToggle(b: Bin, bit: Int) {

1122	        val s = b.sim

1123	        Hub.editSim(b.id, s.copy(mask = s.mask xor (1 shl bit)))

1124	    }

1125

1126	    fun simValue(b: Bin, key: String, v: Double) {

1127	        val s = b.sim

1128	        Hub.editSim(b.id, s.copy(values = s.values + (key to v)))

1129	    }

1130

1131	    fun simStep(b: Bin, key: String, dir: Int) {

1132	        val f = SIM_STEP.getValue(key)

1133	        val v = (b.sim.values[key] ?: DEFAULT_SIM[key] ?: 0.0) + f.step * dir

1134	        simValue(b, key, Math.round(v.coerceIn(f.min, f.max) * 10) / 10.0)

1135	    }

1136

1137	    fun applySim(b: Bin) {

1138	        viewModelScope.launch {

1139	            val problem = Hub.applySim(b.id)

1140	            if (problem != null) {

1141	                Feel.event(Sfx.DENIED)

1142	                say(problem)

1143	            } else {

1144	                if (b.sim.mask > 0) Feel.event(Sfx.SIM)

1145	                say(if (b.sim.mask > 0) "Simulation applied to ${b.name}." else "${b.name} is reading real sensors again.")

1146	            }

1147	        }

1148	    }

1149

1150	    fun simAllAuto(b: Bin) {

1151	        Hub.editSim(b.id, b.sim.copy(mask = 0))

1152	        viewModelScope.launch {

1153	            Hub.applySim(b.id)

1154	            say("All rows back to AUTO.")

1155	        }

1156	    }

1157

1158	    // ---------- alerts ----------

1159

1160	    var akind by mutableStateOf(AlertKind.ALL)

1161	    var arange by mutableStateOf(AlertRange.ANY)

1162	    var alertShown by mutableIntStateOf(6)

1163

1164	    fun shown(all: List<AlertRow>): List<AlertRow> {

1165	        val now = System.currentTimeMillis()

1166	        val since = when (arange) {

1167	            AlertRange.TODAY -> Calendar.getInstance().apply {

1168	                set(Calendar.HOUR_OF_DAY, 0); set(Calendar.MINUTE, 0); set(Calendar.SECOND, 0); set(Calendar.MILLISECOND, 0)

1169	            }.timeInMillis

1170	            AlertRange.WEEK -> now - 7 * DAY

1171	            AlertRange.MONTH -> now - 30 * DAY

1172	            AlertRange.ANY -> Long.MIN_VALUE

1173	        }

1174	        return all.filter { (akind == AlertKind.ALL || it.kind == akind) && it.at >= since }

1175	    }

1176

1177	    fun scope(n: Int): String {

1178	        val w = if (n == 1) akind.one else akind.many

1179	        val r = when (arange) {

1180	            AlertRange.TODAY -> " from today"; AlertRange.WEEK -> " from the last 7 days"

1181	            AlertRange.MONTH -> " from the last 30 days"; AlertRange.ANY -> ""

1182	        }

1183	        return (if (akind == AlertKind.ALL && arange == AlertRange.ANY) "all $n $w" else "$n $w") + r

1184	    }

1185

1186	    fun markShown(all: List<AlertRow>) {

1187	        val s = shown(all)

1188	        Hub.markRead(s.map { it.id }.toSet())

1189	        say("Marked ${scope(s.size)} as read.")

1190	    }

1191

1192	    fun markAll() {

1193	        Hub.markRead(Hub.state.value.alerts.map { it.id }.toSet())

1194	        overlay = null

1195	        say("All alerts marked read.")

1196	    }

1197

1198	    fun deleteShown(all: List<AlertRow>) {

1199	        val s = shown(all)

1200	        val t = scope(s.size)

1201	        Hub.deleteAlerts(s.map { it.id }.toSet())

1202	        overlay = null

1203	        Feel.event(Sfx.DANGER)

1204	        say("Deleted $t.")

1205	    }

1206

1207	    // ---------- demo options ----------

1208

1209	    /** Demo fill speed as a multiple of real life. 60 is the normal demo. */

1210	    var demoSpeed by mutableStateOf(Demo.speed)

1211	        private set

1212	    /** The pretend links live in [Hub], because the demo tour changes them too. */

1213	    val demoLinks: Routes get() = Hub.state.value.demoLinks

1214

1215	    fun changeDemoSpeed(x: Double) {

1216	        Hub.pauseTour()

1217	        demoSpeed = x

1218	        Hub.demoSpeed(x)

1219	    }

1220

1221	    fun toggleDemoLink(which: Int) {

1222	        Hub.pauseTour()

1223	        val r = demoLinks

1224	        val next = when (which) {

1225	            0 -> r.copy(local = !r.local)

1226	            1 -> r.copy(firebase = !r.firebase)

1227	            else -> r.copy(thingspeak = !r.thingspeak)

1228	        }

1229	        Hub.demoLinks(next)

1230	        val main = when {

1231	            next.local -> "Demo bins now answer on Wi-Fi."

1232	            next.firebase -> "Demo bins now answer through Firebase. LOCK, UNLOCK and COLLECT wait for the bin."

1233	            next.thingspeak -> "Demo bins now answer through ThingSpeak. LOCK, UNLOCK and COLLECT wait for the bin."

1234	            else -> "Demo bins show plain demo data again."

1235	        }

1236	        say(main)

1237	    }

1238

1239	    /** Pretend cloud commands: false confirms them after 4 s, true lets them time out after 8 s. */

1240	    fun setDemoNoConfirm(no: Boolean) {

1241	        Hub.pauseTour()

1242	        Hub.setDemoNoConfirm(no)

1243	        say(if (no) "Cloud commands on demo bins will not be confirmed. Their keys give up after 8 s." else "Cloud commands on demo bins are confirmed after 4 s.")

1244	    }

1245

1246	    fun demoEvent(b: Bin, e: Hub.DemoEvent) {

1247	        if (!b.demo) {

1248	            Feel.event(Sfx.DENIED)

1249	            say("${b.name} is a real bin. Demo options only change demo bins.")

1250	            return

1251	        }

1252	        Hub.pauseTour()

1253	        Hub.demoEvent(b.id, e)

1254	        say(

1255	            when (e) {

1256	                Hub.DemoEvent.FULL -> "${b.name} is now over its fill limit."

1257	                Hub.DemoEvent.TAMPER -> "${b.name} was tipped. Reset it from its card."

1258	                Hub.DemoEvent.AIR -> "Air quality spiked in ${b.name}. It settles by itself."

1259	                Hub.DemoEvent.EMPTY -> "${b.name} is empty."

1260	            }

1261	        )

1262	    }

1263

1264	    fun toggleDemoQuiet(b: Bin) {

1265	        if (!b.demo) {

1266	            Feel.event(Sfx.DENIED)

1267	            say("${b.name} is a real bin. Demo options only change demo bins.")

1268	            return

1269	        }

1270	        Hub.pauseTour()

1271	        val quiet = !Hub.demoQuiet(b.id)

1272	        Hub.setDemoQuiet(b.id, quiet)

1273	        demoTick++

1274	        say(if (quiet) "${b.name} stops answering. It shows as lost after 90 s." else "${b.name} answers again.")

1275	    }

1276

1277	    /** Bumped when a demo choice kept in [Hub] changes, so the page redraws. */

1278	    var demoTick by mutableIntStateOf(0)

1279	        private set

1280

1281	    fun resetDemo() {

1282	        Hub.resetDemo()

1283	        demoSpeed = Demo.speed

1284	        demoTick++

1285	        Feel.event(Sfx.SAVED)

1286	        say("Demo options are back to normal.")

1287	    }

1288

1289	    // ---------- ThingSpeak ----------

1290

1291	    var tsUrl by mutableStateOf(Hub.thingSpeakUrl())

1292	    var tsMsg by mutableStateOf<Pair<Boolean, String>?>(null)

1293	    var tsBusy by mutableStateOf(false)

1294

1295	    fun tsTest() {

1296	        val url = tsUrl.trim()

1297	        if (!url.startsWith("https://") && !url.startsWith("http://")) {

1298	            Feel.event(Sfx.DENIED)

1299	            tsMsg = false to "The server address starts with https://."

1300	            return

1301	        }

1302	        Hub.saveThingSpeak(url)

1303	        tsBusy = true

1304	        viewModelScope.launch {

1305	            val (ok, problem) = Hub.testThingSpeak()

1306	            tsBusy = false

1307	            if (ok > 0 && problem == null) {

1308	                Feel.event(Sfx.CLOUD)

1309	                tsMsg = true to "$ok channel${if (ok == 1) "" else "s"} answered. Saved."

1310	            } else {

1311	                Feel.event(Sfx.DENIED)

1312	                tsMsg = false to (if (ok > 0) "$ok answered. " else "") + (problem ?: "No channel answered.")

1313	            }

1314	        }

1315	    }

1316

1317	    // ---------- Firebase ----------

1318

1319	    var fb by mutableStateOf(Hub.firebase())

1320	    var fbShow by mutableStateOf(false)

1321	    /** (passed, message) */

1322	    var fbMsg by mutableStateOf<Pair<Boolean, String>?>(null)

1323	    var fbBusy by mutableStateOf(false)

1324

1325	    fun fbTest(save: Boolean) {

1326	        val cfg = fb

1327	        Cloud.problemWith(cfg)?.let {

1328	            Feel.event(Sfx.DENIED)

1329	            fbMsg = false to (it + if (save) " Nothing was saved." else "")

1330	            return

1331	        }

1332	        fbBusy = true

1333	        viewModelScope.launch {

1334	            val r = Hub.cloud.test(cfg)

1335	            fbBusy = false

1336	            r.onSuccess { n ->

1337	                if (save) Hub.saveFirebase(cfg)

1338	                Feel.event(if (save) Sfx.SAVED else Sfx.CLOUD)

1339	                fbMsg = true to "Signed in as ${cfg.email.trim()}. $n bin${if (n == 1) " is" else "s are"} in the database." + if (save) " Saved as the working set." else ""

1340	            }.onFailure {

1341	                Feel.event(Sfx.DENIED)

1342	                fbMsg = false to (it.message.orEmpty() + if (save) " Nothing was saved." else "")

1343	            }

1344	        }

1345	    }

1346

1347	    fun fbRestore() {

1348	        val w = Hub.firebaseWorking()

1349	        if (w == null) {

1350	            fbMsg = false to "No details have passed a test on this phone yet."

1351	            return

1352	        }

1353	        fb = w

1354	        Hub.saveFirebase(w)

1355	        fbMsg = true to "Put back the last details that passed a test."

1356	    }

1357

1358	    companion object {

1359	        private const val DAY = 24 * 3600_000L

1360	        private const val AUTO_LOCK = 2 * 60_000L

1361	        const val BT_CODE_LEN = 6

1362	        const val BT_CODE_LIFE = 60_000L

1363	        val DEMO_NETS = listOf(BinBle.Net("HomeWiFi-2G", -48), BinBle.Net("TUP-Manila", -61), BinBle.Net("Smart_Bin_Lab", -70), BinBle.Net("PLDTHOMEFIBR24", -79))

1364	        private fun round2(v: Double) = Math.round(v * 100) / 100.0

1365	    }

1366	}

1367

1368	/** One setting stepper: the mockup's FIELDS table. */

1369	class FieldSpec(val label: String, val icon: () -> androidx.compose.ui.graphics.vector.ImageVector, val unit: String, val step: Double, val min: Double, val max: Double, val dec: Int)

1370

1371	val FIELDS: Map<String, FieldSpec> = mapOf(

1372	    "d_empty" to FieldSpec("Distance empty", { Ic.ruler }, "cm", 0.5, 1.0, 400.0, 1),

1373	    "d_full" to FieldSpec("Distance full", { Ic.ruler }, "cm", 0.5, 1.0, 400.0, 1),

1374	    "fill_th" to FieldSpec("Fill threshold", { Ic.percent }, "%", 1.0, 10.0, 100.0, 0),

1375	    "temp_th" to FieldSpec("Temperature threshold", { Ic.temp(40.0) }, "°C", 1.0, 10.0, 85.0, 0),

1376	    "hum_th" to FieldSpec("Humidity threshold", { Ic.humid }, "%RH", 1.0, 30.0, 100.0, 0),

1377	    "gas_th" to FieldSpec("Air quality limit", { Ic.gas }, "IAQ", 5.0, 50.0, 500.0, 0),

1378	    "tilt_th" to FieldSpec("Tilt threshold", { Ic.tilt }, "deg", 1.0, 5.0, 90.0, 0),

1379	    "weight_cap" to FieldSpec("Weight capacity", { Ic.weight(0.6) }, "g", 50.0, 100.0, 5000.0, 0),

1380	    "temp_off" to FieldSpec("Temperature offset", { Ic.temp(28.0) }, "°C", 0.1, -10.0, 10.0, 1),

1381	    "hum_off" to FieldSpec("Humidity offset", { Ic.humid }, "%RH", 0.5, -20.0, 20.0, 1),

1382	    "press_off" to FieldSpec("Pressure offset", { Ic.press }, "hPa", 0.1, -50.0, 50.0, 1),

1383	    "servo_closed" to FieldSpec("Servo closed", { Ic.servo }, "deg", 1.0, 0.0, 180.0, 0),

1384	    "servo_open" to FieldSpec("Servo open", { Ic.servo }, "deg", 1.0, 0.0, 180.0, 0),

1385	    "pir_hold_s" to FieldSpec("Lid stays open", { Ic.clock }, "s", 1.0, 1.0, 60.0, 0),

1386	    "buzz_s" to FieldSpec("Buzzer minimum", { Ic.bell }, "s", 1.0, 1.0, 30.0, 0),

1387	    "tref" to FieldSpec("Reference thermometer", { Ic.temp(28.0) }, "°C", 0.1, -40.0, 85.0, 1),

1388	    "known" to FieldSpec("Known weight", { Ic.weight(0.3) }, "g", 50.0, 10.0, 5000.0, 0),

1389	)

1390

1391	class SimStep(val unit: String, val step: Double, val min: Double, val max: Double)

1392

1393	val SIM_STEP = mapOf(

1394	    "weight" to SimStep("g", 100.0, 0.0, 5000.0),

1395	    "temp" to SimStep("°C", 0.5, -40.0, 85.0),

1396	    "humidity" to SimStep("%", 1.0, 0.0, 100.0),

1397	)

1398

1399	fun fmt(v: Double, dec: Int): String = if (dec == 0) Math.round(v).toString() else "%.${dec}f".format(v)

1400

1401	/** "2 min ago", as the alert rows say it. */

1402	fun ago(at: Long, now: Long = System.currentTimeMillis()): String {

1403	    val m = ((now - at) / 60_000L).coerceAtLeast(0)

1404	    return when {

1405	        m < 1 -> "now"

1406	        m < 60 -> "$m min ago"

1407	        m < 24 * 60 -> "${m / 60} h ago"

1408	        m < 48 * 60 -> "1 day ago"

1409	        else -> "${m / (24 * 60)} days ago"

1410	    }

1411	}

Android App/app/src/main/java/smartbin/ui/SmartBinUi.kt

1	// ============================================================================

2	// ui/SmartBinUi.kt - the root composable

3	// Draws the start page (Gate) until a role is picked, then the header, the

4	// current page, the bottom tabs, the overlays and the toast. Every page reads

5	// Hub.state through collectAsState, so a screen that does not update usually

6	// means Hub did not change the value, not that the UI missed it.

7	// The layout is drawn for a 384 dp wide phone and scaled (0.8x to 1.6x), and

8	// the phone's font size is clamped to 0.9..1.1 so labels are not cut off.

9	// ============================================================================

10	package smartbin.ui

11

12	import android.Manifest

13	import android.content.Intent

14	import android.os.Build

15	import android.provider.Settings

16	import androidx.activity.ComponentActivity

17	import androidx.activity.SystemBarStyle

18	import androidx.activity.compose.BackHandler

19	import androidx.activity.compose.rememberLauncherForActivityResult

20	import androidx.activity.enableEdgeToEdge

21	import androidx.activity.result.contract.ActivityResultContracts

22	import androidx.compose.animation.AnimatedContent

23	import androidx.compose.animation.core.tween

24	import androidx.compose.animation.fadeIn

25	import androidx.compose.animation.fadeOut

26	import androidx.compose.animation.slideInHorizontally

27	import androidx.compose.animation.slideOutHorizontally

28	import androidx.compose.animation.togetherWith

29	import androidx.compose.foundation.background

30	import androidx.compose.foundation.interaction.MutableInteractionSource

31	import androidx.compose.foundation.interaction.collectIsPressedAsState

32	import androidx.compose.foundation.isSystemInDarkTheme

33	import androidx.compose.foundation.layout.Arrangement

34	import androidx.compose.foundation.layout.Box

35	import androidx.compose.foundation.layout.Column

36	import androidx.compose.foundation.layout.Row

37	import androidx.compose.foundation.layout.RowScope

38	import androidx.compose.foundation.layout.Spacer

39	import androidx.compose.foundation.layout.defaultMinSize

40	import androidx.compose.foundation.layout.fillMaxHeight

41	import androidx.compose.foundation.layout.fillMaxSize

42	import androidx.compose.foundation.layout.fillMaxWidth

43	import androidx.compose.foundation.layout.height

44	import androidx.compose.foundation.layout.navigationBarsPadding

45	import androidx.compose.foundation.layout.offset

46	import androidx.compose.foundation.layout.padding

47	import androidx.compose.foundation.layout.size

48	import androidx.compose.foundation.layout.statusBarsPadding

49	import androidx.compose.foundation.layout.widthIn

50	import androidx.compose.foundation.shape.RoundedCornerShape

51	import androidx.compose.material3.LocalContentColor

52	import androidx.compose.runtime.Composable

53	import androidx.compose.runtime.CompositionLocalProvider

54	import androidx.compose.runtime.LaunchedEffect

55	import androidx.compose.foundation.gestures.awaitEachGesture

56	import androidx.compose.foundation.gestures.awaitFirstDown

57	import androidx.compose.ui.input.pointer.PointerEventPass

58	import androidx.compose.ui.input.pointer.pointerInput

59	import androidx.compose.runtime.collectAsState

60	import androidx.compose.runtime.getValue

61	import androidx.compose.runtime.setValue

62	import androidx.compose.runtime.remember

63	import androidx.compose.ui.Alignment

64	import androidx.compose.ui.Modifier

65	import androidx.compose.ui.draw.clip

66	import androidx.compose.ui.draw.drawBehind

67	import androidx.compose.ui.geometry.Offset

68	import androidx.compose.ui.graphics.Color

69	import androidx.compose.ui.graphics.vector.ImageVector

70	import androidx.compose.ui.platform.LocalConfiguration

71	import androidx.compose.ui.platform.LocalContext

72	import androidx.compose.ui.platform.LocalDensity

73	import androidx.compose.ui.platform.LocalView

74	import androidx.compose.ui.text.font.FontWeight

75	import androidx.compose.ui.unit.Density

76	import androidx.compose.ui.unit.IntOffset

77	import androidx.compose.ui.unit.dp

78	import androidx.compose.ui.zIndex

79	import smartbin.data.Bin

80	import smartbin.data.BinStatus

81	import smartbin.data.Hub

82	import smartbin.data.HubState

83	import smartbin.data.Link

84	import smartbin.data.Mode

85	import smartbin.data.Role

86	import smartbin.data.Severity

87	import smartbin.data.v

88	import smartbin.notify.Feel

89	import smartbin.notify.Sfx

90

91	/** The whole app: the gate on first launch, then header, page, tabs and sheets, as the mockup's phone frame. */

92	@Composable

93	fun SmartBinUi(vm: AppViewModel, setLandscape: (Boolean) -> Unit) {

94	    val hub by Hub.state.collectAsState()

95	    val dark = when (vm.theme) {

96	        ThemeMode.AUTO -> isSystemInDarkTheme()

97	        ThemeMode.LIGHT -> false

98	        ThemeMode.DARK -> true

99	    }

100	    val mode = if (vm.role == null || !vm.started) Mode.NORMAL else hub.mode

101	    LaunchedEffect(vm.started, vm.role) { vm.syncInMain() }

102	    val c = animated(sbColors(dark, mode))

103	    val context = LocalContext.current

104

105	    LaunchedEffect(dark) {

106	        val clear = android.graphics.Color.TRANSPARENT

107	        val style = if (dark) SystemBarStyle.dark(clear) else SystemBarStyle.light(clear, clear)

108	        (context as? ComponentActivity)?.enableEdgeToEdge(statusBarStyle = style, navigationBarStyle = style)

109	    }

110	    LaunchedEffect(vm.fullscreen) { setLandscape(vm.fullscreen != null) }

111	    val root = LocalView.current

112	    LaunchedEffect(vm.keepAwake) { root.keepScreenOn = vm.keepAwake }

113

114	    val askAlerts = rememberLauncherForActivityResult(ActivityResultContracts.RequestPermission()) { vm.permissionAnswered(it) }

115	    val allowAlerts: () -> Unit = {

116	        vm.overlay = null

117	        if (Build.VERSION.SDK_INT >= 33) askAlerts.launch(Manifest.permission.POST_NOTIFICATIONS)

118	        else openAlertSettings(context)

119	    }

120

121	    // The mockup is drawn for a phone 384 dp wide. Other phones scale it, so

122	    // sizes, spacing and text keep their proportions, and the phone's own font

123	    // size is held to a small range so no label is cut off.

124	    val cfg = LocalConfiguration.current

125	    val base = LocalDensity.current

126	    val land = vm.fullscreen != null

127	    val byWidth = cfg.smallestScreenWidthDp / 384f

128	    val fit = (if (land) byWidth else minOf(byWidth, cfg.screenHeightDp / 700f)).coerceIn(0.8f, 1.6f)

129	    val scaled = remember(base.density, base.fontScale, fit) { Density(base.density * fit, base.fontScale.coerceIn(0.9f, 1.1f)) }

130

131	    CompositionLocalProvider(LocalSb provides c, LocalContentColor provides c.ink, LocalDensity provides scaled) {

132	        Box(

133	            Modifier.fillMaxSize().page(c).pointerInput(Unit) {

134	                // Watches every touch before the keys get it, without taking it:

135	                // the notice goes away and the key under the finger still works.

136	                awaitEachGesture {

137	                    awaitFirstDown(requireUnconsumed = false, pass = PointerEventPass.Initial)

138	                    vm.hideToast()

139	                }

140	            },

141	        ) {

142	            when {

143	                vm.role == null || !vm.started -> Column(Modifier.fillMaxSize().statusBarsPadding().navigationBarsPadding()) { Gate(vm, hub) }

144	                land -> FullChart(vm, hub)

145	                else -> Column(

146	                    Modifier.align(Alignment.TopCenter).fillMaxHeight().widthIn(max = 520.dp).fillMaxWidth().statusBarsPadding(),

147	                ) {

148	                    Header(vm, hub, dark)

149	                    Box(Modifier.weight(1f)) {

150	                        if (hub.loading) Loading()

151	                        else AnimatedContent(

152	                            targetState = vm.screen,

153	                            transitionSpec = {

154	                                val dir = if (targetState.ordinal > initialState.ordinal) 1 else -1

155	                                (slideInHorizontally(tween(220)) { dir * it / 8 } + fadeIn(tween(220))) togetherWith

156	                                    (slideOutHorizontally(tween(160)) { -dir * it / 8 } + fadeOut(tween(110)))

157	                            },

158	                            label = "tab",

159	                        ) { s ->

160	                            when (s) {

161	                                Screen.OVERVIEW -> Overview(vm, hub)

162	                                Screen.CHARTS -> Charts(vm, hub)

163	                                Screen.SETTINGS -> SettingsPage(vm, hub)

164	                                Screen.SIM -> if (vm.operator) Simulation(vm, hub) else Overview(vm, hub)

165	                                Screen.ALERTS -> AlertsPage(vm, hub)

166	                                Screen.APP -> AppPage(vm, hub)

167	                                Screen.BT -> Bluetooth(vm, hub)

168	                                Screen.NET -> Connection(vm, hub)

169	                            }

170	                        }

171	                    }

172	                    BottomNav(vm, hub)

173	                }

174	            }

175	            // Back from a page under Settings, or from a tab, goes one step toward Overview.

176	            val view = LocalView.current

177	            BackHandler(enabled = vm.overlay == null && vm.role != null && vm.started && (land || vm.screen != Screen.OVERVIEW)) {

178	                Feel.play(view, Sfx.BACK)

179	                if (land) vm.fullscreen = null else vm.back()

180	            }

181	            Overlays(vm, hub, allowAlerts)

182	            vm.toast?.let { Toast(it, bottom = if (land || vm.role == null || !vm.started) 24.dp else 92.dp) }

183	        }

184	    }

185	}

186

187	fun openAlertSettings(context: android.content.Context) {

188	    runCatching {

189	        context.startActivity(

190	            Intent(Settings.ACTION_APP_NOTIFICATION_SETTINGS)

191	                .putExtra(Settings.EXTRA_APP_PACKAGE, context.packageName)

192	                .addFlags(Intent.FLAG_ACTIVITY_NEW_TASK)

193	        )

194	    }

195	}

196

197	@Composable

198	private fun Loading() {

199	    ScrollPage {

200	        Skeleton(40.dp, Modifier.fillMaxWidth(0.5f), radius = 12.dp)

201	        Skeleton(430.dp)

202	        Skeleton(180.dp)

203	    }

204	}

205

206	// ---------- header ----------

207

208	private val MODES = mapOf(

209	    Mode.NORMAL to "NORMAL", Mode.SIM to "SIMULATION", Mode.CALIB to "CALIBRATING", Mode.DUAL to "SIM + CAL", Mode.ALERT to "ALERT",

210	)

211

212	private fun modeIcon(m: Mode): ImageVector = when (m) {

213	    Mode.NORMAL -> Ic.check; Mode.SIM -> Ic.flask; Mode.CALIB, Mode.DUAL -> Ic.calib; Mode.ALERT -> Ic.bell

214	}

215

216	@Composable

217	private fun Header(vm: AppViewModel, hub: HubState, dark: Boolean) {

218	    val c = Sb.c

219	    val view = LocalView.current

220	    // Brand, mode chip, theme key

221	    Row(

222	        Modifier.fillMaxWidth().padding(horizontal = 14.dp, vertical = 4.dp),

223	        verticalAlignment = Alignment.CenterVertically,

224	        horizontalArrangement = Arrangement.spacedBy(8.dp),

225	    ) {

226	        // A tap on the logo plays its chime and its build-up again.

227	        var logoPlay by remember { androidx.compose.runtime.mutableIntStateOf(1) }

228	        Row(

229	            Modifier.press(true, Sfx.LOGO) { logoPlay++ },

230	            verticalAlignment = Alignment.CenterVertically,

231	            horizontalArrangement = Arrangement.spacedBy(6.dp),

232	        ) {

233	            SmartBinLogo(30.dp, logoPlay)

234	            T("SMART-BIN", size = 1.12f, spacing = 0.04f, maxLines = 1)

235	        }

236	        Spacer(Modifier.weight(1f))

237	        // DEMO shows only while demo mode is on. It goes back to the start page, where demo is turned off.

238	        if (hub.demoOn) {

239	            Box(

240	                Modifier.defaultMinSize(minHeight = 36.dp).bump(c, 11.dp).press(true, Sfx.BACK) { vm.toStart() }.padding(horizontal = 10.dp),

241	                contentAlignment = Alignment.Center,

242	            ) { T("DEMO", size = 0.74f, spacing = 0.04f, color = c.warn, maxLines = 1) }

243	        }

244	        val m = hub.mode

245	        Row(

246	            Modifier

247	                .raised(c, 999.dp)

248	                .clip(RoundedCornerShape(999.dp))

249	                .background(c.barBrush)

250	                .padding(horizontal = 12.dp, vertical = 8.dp),

251	            verticalAlignment = Alignment.CenterVertically,

252	            horizontalArrangement = Arrangement.spacedBy(6.dp),

253	        ) {

254	            Ico(modeIcon(m), 15.dp, c.onAccent)

255	            T(MODES.getValue(m), size = 0.74f, spacing = 0.04f, color = c.onAccent, maxLines = 1)

256	        }

257	        IconKey(Ic.sunmoon(dark), {

258	            vm.toggleTheme(dark)

259	        }, size = 40.dp, iconSize = 19.dp, sound = if (dark) Sfx.THEME_LIGHT else Sfx.THEME_DARK)

260	        // The gear opens the app's own settings: sound, vibration, battery, links and demo options.

261	        val onApp = vm.screen == Screen.APP || vm.screen == Screen.NET || vm.screen == Screen.BT

262	        IconKey(Ic.gear, { if (onApp) vm.back() else vm.go(Screen.APP) }, size = 40.dp, iconSize = 19.dp, tint = if (onApp) c.accent else c.ink2, sound = if (onApp) Sfx.BACK else Sfx.OPEN)

263	    }

264	    // The live strip: what needs attention, the link and the role

265	    Row(

266	        Modifier.fillMaxWidth().padding(horizontal = 14.dp, vertical = 6.dp),

267	        verticalAlignment = Alignment.CenterVertically,

268	        horizontalArrangement = Arrangement.spacedBy(8.dp),

269	    ) {

270	        Row(Modifier.weight(1f), verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(8.dp)) {

271	            StatusPill(vm, hub)

272	            LinkChip(hub, Modifier.weight(1f, fill = false)) { vm.go(Screen.NET) }

273	        }

274	        val src = remember { MutableInteractionSource() }

275	        val pressed by src.collectIsPressedAsState()

276	        Box(

277	            Modifier

278	                .defaultMinSize(minHeight = 36.dp)

279	                .then(if (pressed) Modifier.clip(RoundedCornerShape(11.dp)).background(c.base).sunk(c, 11.dp) else Modifier.bump(c, 11.dp))

280	                .press(true, Sfx.CHOICE, src) { vm.pickRole(if (vm.operator) Role.COLLECTOR else Role.OPERATOR) }

281	                .padding(horizontal = 13.dp),

282	            contentAlignment = Alignment.Center,

283	        ) { T(if (vm.operator) "OPERATOR" else "COLLECTOR", size = 0.76f, spacing = 0.04f, maxLines = 1) }

284	    }

285	    Box(Modifier.padding(start = 14.dp, end = 14.dp, top = 4.dp).fillMaxWidth().height(3.dp).clip(RoundedCornerShape(3.dp)).background(c.barBrush))

286	}

287

288	/** One active condition: the banner kind, its icon, title and line under it. */

289	class Live(val tone: Tone, val icon: ImageVector, val title: String, val small: String? = null, val pill: Pair<ImageVector, Color>)

290

291	// Builds the attention banners (calibration, tamper, full, simulation, lost

292	// bins, sensor faults) shown in the header pill and the Attention sheet.

293	/** The conditions the mockup shows as banners, in its order. */

294	@Composable

295	fun liveBanners(hub: HubState): List<Live> {

296	    val c = Sb.c

297	    val b = hub.bins

298	    val out = ArrayList<Live>()

299	    val cal = b.filter { it.calibrating }

300	    val tam = b.filter { it.reading.status == BinStatus.TAMPER }

301	    val full = b.filter { it.full }

302	    val sim = b.filter { it.simulating }

303	    val lost = b.filter { it.stale }

304	    if (cal.isNotEmpty()) out += Live(Tone.TEAL, Ic.calib, "CALIBRATING: " + cal.joinToString { it.name }, "Keep the bin empty and do not touch it.", Ic.calib to c.cal)

305	    if (tam.isNotEmpty()) out += Live(Tone.RED, Ic.tamper, "TAMPER: " + tam.joinToString { it.name }, "Check the bin before resetting the alert.", Ic.tamper to c.tamper)

306	    if (full.isNotEmpty()) out += Live(Tone.RED, Ic.truck, "COLLECT NOW: " + full.joinToString { "${it.name} at ${Math.round(it.reading.fill)}%" }, null, Ic.truck to c.tamper)

307	    if (sim.isNotEmpty()) out += Live(Tone.PURPLE, Ic.flask, "SIMULATION MODE", "Some readings are set by hand, not by the sensors.", Ic.flask to c.sim)

308	    // Pretend links from the demo options say so, so nobody mistakes them for real ones.

309	    hub.tourMsg?.let { out += Live(Tone.PURPLE, Ic.flask, "Demo tour: $it", "The tour runs by itself. Press a key on a demo bin and it waits a minute for you.", Ic.flask to c.sim) }

310	    val broken = b.filter { it.reading.faults.isNotEmpty() }

311	    if (broken.isNotEmpty()) out += Live(

312	        Tone.AMBER, Ic.gear, "SENSOR NOT ANSWERING: " + broken.joinToString { "${it.name} (${it.reading.faults.joinToString()})" },

313	        "Its readings show -- until the sensor answers again. Check the wiring.", Ic.gear to c.warn,

314	    )

315	    if (b.isEmpty()) {

316	        out += Live(Tone.AMBER, Ic.wifi(false), "No bins linked yet", "Pair a bin over Bluetooth in App settings, or add one with its IP address as operator. Turn DEMO on at the start page to look around.", Ic.wifi(false) to c.warn)

317	        return out

318	    }

319	    if (hub.demoOn) out += Live(Tone.AMBER, Ic.flask, "Demo mode", "Bins named Demo show made-up readings. Press DEMO at the top to go back to the start page and turn it off.", Ic.flask to c.warn)

320	    val pretend = if (hub.demo) "Pretend link from the demo options. " else ""

321	    when (hub.link) {

322	        Link.DEMO -> {}

323	        Link.OFFLINE -> out += Live(Tone.RED, Ic.wifi(false), "Bins are not answering", "No bin answers on Wi-Fi and the cloud cannot be reached. Check the phone has internet.", Ic.wifi(false) to c.tamper)

324	        Link.CLOUD -> out += Live(

325	            Tone.ORANGE, Ic.cloud, "Cloud mode",

326	            pretend + "No bin answers on this Wi-Fi, so readings come through Firebase every 5 s. LOCK, UNLOCK and COLLECT still work. Calibration and simulation need the bins’ Wi-Fi.",

327	            Ic.cloud to c.cloud,

328	        )

329	        Link.THINGSPEAK -> out += Live(

330	            Tone.BLUE, Ic.chart, "ThingSpeak mode",

331	            pretend + "Wi-Fi and Firebase do not answer, so readings come from ThingSpeak every 15 s. LOCK, UNLOCK and COLLECT go through TalkBack.",

332	            Ic.chart to c.ts,

333	        )

334	        Link.LIVE -> {}

335	    }

336	    if (lost.isNotEmpty() && hub.link != Link.OFFLINE) {

337	        out += Live(Tone.RED, Ic.wifi(false), "Not answering: " + lost.joinToString { it.name }, "No reading for more than 90 s. Check its power and Wi-Fi.", Ic.wifi(false) to c.tamper)

338	    }

339	    return out

340	}

341

342	@Composable

343	private fun StatusPill(vm: AppViewModel, hub: HubState) {

344	    val c = Sb.c

345	    val live = liveBanners(hub)

346	    val unread = hub.alerts.filter { !it.read }

347	    if (unread.isEmpty() && live.isEmpty()) return

348	    val (icon, cc, text) = if (unread.isNotEmpty()) {

349	        val worst = Severity.entries.first { s -> unread.any { it.sev == s } }

350	        Triple(Ic.bell, sevColor(worst, c), "${unread.size} unread")

351	    } else {

352	        val p = live.first().pill

353	        Triple(p.first, p.second, "${live.size} active")

354	    }

355	    val src = remember { MutableInteractionSource() }

356	    val pressed by src.collectIsPressedAsState()

357	    Row(

358	        Modifier

359	            .defaultMinSize(minHeight = 36.dp)

360	            .then(if (pressed) Modifier.ring(cc.soft(0.34f), 18.dp).clip(RoundedCornerShape(18.dp)).background(c.base).sunk(c, 18.dp) else Modifier.bump(c, 18.dp, ring = cc.soft(0.34f)))

361	            .press(true, Sfx.OPEN, src) { vm.overlay = Overlay.Attention }

362	            .padding(horizontal = 13.dp),

363	        verticalAlignment = Alignment.CenterVertically,

364	        horizontalArrangement = Arrangement.spacedBy(7.dp),

365	    ) {

366	        Ico(icon, 18.dp, cc)

367	        T(text, size = 0.8f, spacing = 0.03f, color = cc, maxLines = 1)

368	    }

369	}

370

371	fun sevColor(s: Severity, c: SbColors) = when (s) {

372	    Severity.CRITICAL -> c.bad; Severity.WARNING -> c.warn; Severity.INFO -> c.sim; Severity.SUCCESS -> c.ok

373	}

374

375	fun sevTone(s: Severity) = when (s) {

376	    Severity.CRITICAL -> Tone.RED; Severity.WARNING -> Tone.AMBER; Severity.INFO -> Tone.PURPLE; Severity.SUCCESS -> Tone.GREEN

377	}

378

379	/** One way to the bins, as the link chip and Connection draw it. */

380	class RouteLook(val name: String, val icon: ImageVector, val color: Color)

381

382	/** The routes that answer, in their order: Wi-Fi green, Firebase orange, ThingSpeak blue. */

383	fun routeLooks(r: smartbin.data.Routes, c: SbColors): List<RouteLook> = buildList {

384	    if (r.local) add(RouteLook("Wi-Fi", Ic.wifi(true), c.link))

385	    if (r.firebase) add(RouteLook("Firebase", Ic.cloud, c.cloud))

386	    if (r.thingspeak) add(RouteLook("ThingSpeak", Ic.chart, c.ts))

387	}

388

389	// The header chip: which route is reaching the bins. Tapping it opens the

390	// Connection page.

391	/**

392	 * Which links answer. Each route has its own colour and icon, so every mix

393	 * looks different: one icon per route that is up, a ring in the colour of the

394	 * one the readings come from, and a strip of all their colours underneath.

395	 */

396	@Composable

397	private fun LinkChip(hub: HubState, modifier: Modifier = Modifier, onClick: () -> Unit) {

398	    val c = Sb.c

399	    val link = hub.link

400	    val parts = routeLooks(hub.routes, c)

401	    val main = when (link) {

402	        Link.LIVE -> c.link; Link.CLOUD -> c.cloud; Link.THINGSPEAK -> c.ts

403	        Link.OFFLINE -> c.bad; Link.DEMO -> parts.firstOrNull()?.color ?: c.warn

404	    }

405	    val text = when {

406	        parts.size == 3 -> "All 3 links"

407	        parts.isNotEmpty() -> parts.joinToString(" + ") { it.name }

408	        link == Link.OFFLINE -> "Not connected"

409	        link == Link.DEMO -> "Demo data"

410	        else -> "Connecting"

411	    }

412	    val src = remember { MutableInteractionSource() }

413	    val pressed by src.collectIsPressedAsState()

414	    Row(

415	        modifier

416	            .defaultMinSize(minHeight = 36.dp)

417	            .then(if (pressed) Modifier.clip(RoundedCornerShape(18.dp)).background(c.base).sunk(c, 18.dp) else Modifier.bump(c, 18.dp, ring = if (parts.isNotEmpty()) main.soft(0.34f) else null))

418	            .press(true, Sfx.OPEN, src, onClick)

419	            .drawBehind {

420	                // The colour strip: one segment per route that answers.

421	                if (parts.size > 1) {

422	                    val h = 3.dp.toPx()

423	                    val w = (size.width - 26.dp.toPx()) / parts.size

424	                    parts.forEachIndexed { k, p ->

425	                        drawRoundRect(

426	                            p.color, topLeft = Offset(13.dp.toPx() + k * w, size.height - h - 3.dp.toPx()),

427	                            size = androidx.compose.ui.geometry.Size(w - 2.dp.toPx(), h),

428	                            cornerRadius = androidx.compose.ui.geometry.CornerRadius(h / 2),

429	                        )

430	                    }

431	                }

432	            }

433	            .padding(horizontal = 13.dp),

434	        verticalAlignment = Alignment.CenterVertically,

435	        horizontalArrangement = Arrangement.spacedBy(5.dp),

436	    ) {

437	        if (parts.isEmpty()) Ico(Ic.wifi(false), 17.dp, if (link == Link.OFFLINE) c.bad else c.warn)

438	        parts.forEach { p -> Ico(p.icon, 16.dp, p.color, Modifier.breathe(p.color == main, 2200)) }

439	        T(text, Modifier.padding(start = 2.dp), size = 0.8f, weight = FontWeight.Bold, color = if (parts.size == 1) main else c.ink, fit = true)

440	    }

441	}

442

443	// ---------- bottom nav ----------

444

445	private class TabDef(val screen: Screen, val label: String, val icon: ImageVector, val cc: (SbColors) -> Color)

446

447	private val TABS = listOf(

448	    TabDef(Screen.OVERVIEW, "Overview", Ic.grid) { it.fill },

449	    TabDef(Screen.CHARTS, "Charts", Ic.chart) { it.humid },

450	    TabDef(Screen.SETTINGS, "Settings", Ic.sliders) { it.cal },

451	    TabDef(Screen.SIM, "Simulation", Ic.flask) { it.sim },

452	    TabDef(Screen.ALERTS, "Alerts", Ic.bell) { it.tamper },

453	)

454

455	// Five tabs. A red badge on Alerts counts unread alert rows.

456	@Composable

457	private fun BottomNav(vm: AppViewModel, hub: HubState) {

458	    val c = Sb.c

459	    // The gear pages belong to no tab.

460	    val current = vm.screen.takeIf { it.ordinal < Screen.APP.ordinal }

461	    val unread = hub.unread

462	    Row(

463	        Modifier

464	            .fillMaxWidth()

465	            .drawBehind {

466	                drawRect(

467	                    androidx.compose.ui.graphics.Brush.verticalGradient(listOf(Color.Transparent, c.shD.soft(0.6f))),

468	                    topLeft = Offset(0f, -14.dp.toPx()), size = androidx.compose.ui.geometry.Size(size.width, 14.dp.toPx()),

469	                )

470	            }

471	            .background(c.base)

472	            .navigationBarsPadding()

473	            .padding(start = 8.dp, end = 8.dp, top = 8.dp, bottom = 4.dp),

474	        horizontalArrangement = Arrangement.spacedBy(5.dp),

475	    ) {

476	        TABS.filter { it.screen != Screen.SIM || vm.operator }.forEach { t ->

477	            NavItem(t.icon, t.label, current == t.screen, t.cc(c), if (t.screen == Screen.ALERTS) unread else 0) {

478	                vm.go(t.screen)

479	            }

480	        }

481	    }

482	}

483

484	@Composable

485	private fun RowScope.NavItem(icon: ImageVector, label: String, selected: Boolean, cc: Color, badge: Int, onClick: () -> Unit) {

486	    val c = Sb.c

487	    Box(Modifier.weight(1f)) {

488	        Column(

489	            Modifier

490	                .fillMaxWidth()

491	                .defaultMinSize(minHeight = 56.dp)

492	                .then(if (selected) Modifier.ring(cc.soft(0.26f), 14.dp).clip(RoundedCornerShape(14.dp)).background(c.base).sunk(c, 14.dp) else Modifier.clip(RoundedCornerShape(14.dp)))

493	                .press(!selected, Sfx.TAB, onClick = onClick),

494	            horizontalAlignment = Alignment.CenterHorizontally,

495	            verticalArrangement = Arrangement.spacedBy(3.dp, Alignment.CenterVertically),

496	        ) {

497	            val t = if (selected) cc else c.ink2

498	            Ico(icon, 22.dp, t)

499	            T(label, size = 9.8f / 14f, weight = FontWeight.Bold, color = t, maxLines = 1)

500	        }

501	        if (badge > 0) {

502	            Box(

503	                Modifier

504	                    .align(Alignment.TopCenter)

505	                    .offset { IntOffset(15.dp.roundToPx(), 3.dp.roundToPx()) }

506	                    .defaultMinSize(minWidth = 20.dp)

507	                    .height(20.dp)

508	                    .brutal(c, 6.dp, c.tamper, off = 2.dp)

509	                    .padding(horizontal = 4.dp),

510	                contentAlignment = Alignment.Center,

511	            ) { T(if (badge > 99) "99+" else badge.toString(), size = 11f / 14f, color = Color.White, maxLines = 1, lineHeight = 1f) }

512	        }

513	    }

514	}

515

516	// ---------- start page ----------

517

518	// The start page: role cards (Collector, Operator), the DEMO switch and the

519	// state of each route. Demo mode can only be turned on or off here.

520	/**

521	 * The start page, on every start of the app and from App settings: the logo,

522	 * the two roles, demo mode, and how the bins answer. Nothing sounds or posts

523	 * from here; alerts start once a role is picked.

524	 */

525	@Composable

526	private fun Gate(vm: AppViewModel, hub: HubState) {

527	    val c = Sb.c

528	    var play by remember { androidx.compose.runtime.mutableIntStateOf(1) }

529	    ScrollPage {

530	        Column(Modifier.fillMaxWidth().padding(top = 14.dp, bottom = 4.dp), horizontalAlignment = Alignment.CenterHorizontally, verticalArrangement = Arrangement.spacedBy(10.dp)) {

531	            Box(Modifier.size(124.dp).well(c, 62.dp).press(true, Sfx.LOGO) { play++ }, contentAlignment = Alignment.Center) {

532	                SmartBinLogo(96.dp, play)

533	            }

534	            T("SMART-BIN", size = 1.65f, spacing = 0.04f)

535	            Hint("Pick how you are using the system today. Operator asks for a PIN. Collector is the default.", Modifier.widthIn(max = 290.dp), align = androidx.compose.ui.text.style.TextAlign.Center)

536	        }

537	        RoleCard(Ic.sliders, "Admin / Operator", listOf("Charts and history", "Lock, unlock, reset tamper", "Calibration", "Simulation controls", "Add and edit bins")) {

538	            vm.pickRole(Role.OPERATOR, first = true)

539	        }

540	        RoleCard(Ic.truck, "Collector", listOf("Which bin to empty first", "Charts, alerts and the collect PIN", "Start and finish a pickup", "Lock and unlock a bin", "Pair a bin to Wi-Fi over Bluetooth", "Settings are read only")) {

541	            vm.pickRole(Role.COLLECTOR, first = true)

542	        }

543	        // Demo mode is turned on or off only here.

544	        Row(

545	            Modifier.fillMaxWidth().well(c, 18.dp).padding(14.dp),

546	            verticalAlignment = Alignment.CenterVertically,

547	            horizontalArrangement = Arrangement.spacedBy(12.dp),

548	        ) {

549	            Btn(

550	                if (hub.demoOn) "DEMO ON" else "DEMO OFF", { vm.toggleDemo() }, icon = Ic.flask, look = Look.TINT,

551	                cc = if (hub.demoOn) c.warn else c.ink2, small = true, pressedIn = hub.demoOn, sound = Sfx.TAP,

552	            )

553	            T(

554	                if (hub.demoOn) "Three made-up bins sit next to your real ones and act out every alert by themselves."

555	                else "Turn DEMO on to add three made-up bins that show every alert by themselves.",

556	                Modifier.weight(1f), size = 0.8f, weight = FontWeight.SemiBold, color = c.ink2,

557	            )

558	        }

559	        val real = hub.bins.filter { !it.demo }

560	        if (real.isNotEmpty()) {

561	            Column(Modifier.fillMaxWidth().well(c, 16.dp).padding(14.dp), verticalArrangement = Arrangement.spacedBy(10.dp)) {

562	                val anyIp = real.any { it.ip.isNotBlank() }

563	                StartLine(

564	                    Ic.wifi(true), "Local Wi-Fi", c.link,

565	                    when { real.any { it.source == smartbin.data.Source.LIVE } -> true; !anyIp -> null; else -> false },

566	                    if (anyIp) "Trying" else "No address",

567	                )

568	                StartLine(Ic.cloud, "Firebase", c.cloud, hub.fbUp, if (hub.fbUp == null) "Not set up" else "Trying")

569	                StartLine(Ic.chart, "ThingSpeak", c.ts, hub.tsUp, if (hub.tsUp == null) "Not set up" else "Trying")

570	            }

571	        }

572	    }

573	}

574

575	@Composable

576	private fun StartLine(icon: ImageVector, name: String, cc: Color, up: Boolean?, waiting: String) {

577	    val c = Sb.c

578	    Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(10.dp)) {

579	        Ico(icon, 19.dp, if (up == true) cc else c.ink2, Modifier.breathe(up == false, 1200))

580	        T(name, Modifier.weight(1f), size = 0.9f, weight = FontWeight.Bold)

581	        when (up) {

582	            true -> Tag("CONNECTED", cc)

583	            false -> T(waiting, size = 0.78f, weight = FontWeight.Bold, color = c.ink2)

584	            null -> T(waiting, size = 0.78f, weight = FontWeight.SemiBold, color = c.ink2)

585	        }

586	    }

587	}

588

589	@Composable

590	private fun RoleCard(icon: ImageVector, title: String, points: List<String>, onClick: () -> Unit) {

591	    val c = Sb.c

592	    val src = remember { MutableInteractionSource() }

593	    val pressed by src.collectIsPressedAsState()

594	    Row(

595	        Modifier

596	            .fillMaxWidth()

597	            .then(if (pressed) Modifier.ring(c.accentSoft, 18.dp).clip(RoundedCornerShape(18.dp)).background(c.base).sunk(c, 18.dp) else Modifier.bump(c, 18.dp))

598	            .press(true, Sfx.CONFIRM, src, onClick)

599	            .padding(16.dp),

600	        horizontalArrangement = Arrangement.spacedBy(13.dp),

601	    ) {

602	        Ico(icon, 34.dp, c.accent)

603	        Column(verticalArrangement = Arrangement.spacedBy(6.dp)) {

604	            T(title, size = 1.05f)

605	            Column {

606	                points.forEach { p -> T("•  $p", size = 0.84f, weight = FontWeight.Medium, color = c.ink2, lineHeight = 1.6f) }

607	            }

608	        }

609	    }

610	}

611

612	/** The bin's colour on the card and in the tiles: the status colour, or how full it is. */

613	fun statusColor(b: Bin, c: SbColors): Color = when (b.reading.status) {

614	    BinStatus.CALIBRATING, BinStatus.COLLECTING -> c.cal

615	    BinStatus.TAMPER -> c.bad

616	    BinStatus.LOCKED -> c.sim

617	    BinStatus.OK -> levelColor(b, c)

618	}

619

620	fun levelColor(b: Bin, c: SbColors): Color {

621	    val p = b.reading.fill

622	    return when {

623	        p >= b.cal.v("fill_th") -> c.bad

624	        p >= 50 -> c.warn

625	        else -> c.ok

626	    }

627	}

628

629	/** A short line for a stale bin: when its last reading came. */

630	fun lastSeen(b: Bin): String = if (b.lastSeen <= 0) "No reading yet" else

631	    "Last reading " + java.text.SimpleDateFormat("H:mm", java.util.Locale.getDefault()).format(java.util.Date(b.lastSeen))

Android App/app/src/main/java/smartbin/ui/Overview.kt

1	// ============================================================================

2	// ui/Overview.kt - the first tab: bin cards, the tile grid and collection priority

3	// Cards show fill, sensor tiles, status chips and the command keys for one

4	// bin. In the AUTO view, more than 2 bins switch to a compact tile grid. A key

5	// that is waiting for a cloud command shows a progress line along its edge

6	// (WaitKey) until the bin answers.

7	// ============================================================================

8	package smartbin.ui

9

10	import androidx.compose.runtime.setValue

11	import androidx.compose.runtime.collectAsState

12	import androidx.compose.foundation.Canvas

13	import androidx.compose.animation.core.Animatable

14	import androidx.compose.animation.core.LinearEasing

15	import androidx.compose.animation.core.RepeatMode

16	import androidx.compose.animation.core.animateFloat

17	import androidx.compose.animation.core.animateFloatAsState

18	import androidx.compose.animation.core.infiniteRepeatable

19	import androidx.compose.animation.core.rememberInfiniteTransition

20	import androidx.compose.animation.core.tween

21	import androidx.compose.foundation.background

22	import androidx.compose.foundation.horizontalScroll

23	import androidx.compose.foundation.interaction.MutableInteractionSource

24	import androidx.compose.foundation.interaction.collectIsPressedAsState

25	import androidx.compose.foundation.layout.Arrangement

26	import androidx.compose.foundation.layout.Box

27	import androidx.compose.foundation.layout.BoxWithConstraints

28	import androidx.compose.foundation.layout.Column

29	import androidx.compose.foundation.layout.Row

30	import androidx.compose.foundation.layout.Spacer

31	import androidx.compose.foundation.layout.defaultMinSize

32	import androidx.compose.foundation.layout.fillMaxWidth

33	import androidx.compose.foundation.layout.height

34	import androidx.compose.foundation.layout.offset

35	import androidx.compose.foundation.layout.padding

36	import androidx.compose.foundation.layout.size

37	import androidx.compose.foundation.layout.width

38	import androidx.compose.foundation.rememberScrollState

39	import androidx.compose.foundation.shape.RoundedCornerShape

40	import androidx.compose.runtime.Composable

41	import androidx.compose.runtime.LaunchedEffect

42	import androidx.compose.runtime.State

43	import androidx.compose.runtime.getValue

44	import androidx.compose.runtime.mutableLongStateOf

45	import androidx.compose.runtime.remember

46	import androidx.compose.runtime.withFrameMillis

47	import androidx.compose.ui.Alignment

48	import androidx.compose.ui.Modifier

49	import androidx.compose.ui.draw.alpha

50	import androidx.compose.ui.draw.clip

51	import androidx.compose.ui.draw.drawBehind

52	import androidx.compose.ui.draw.drawWithContent

53	import androidx.compose.ui.geometry.CornerRadius

54	import androidx.compose.ui.geometry.Offset

55	import androidx.compose.ui.geometry.RoundRect

56	import androidx.compose.ui.geometry.Size

57	import androidx.compose.ui.graphics.Brush

58	import androidx.compose.ui.graphics.Color

59	import androidx.compose.ui.graphics.Path

60	import androidx.compose.ui.graphics.PathMeasure

61	import androidx.compose.ui.graphics.StrokeCap

62	import androidx.compose.ui.graphics.drawscope.DrawScope

63	import androidx.compose.ui.graphics.drawscope.Stroke

64	import androidx.compose.ui.graphics.vector.ImageVector

65	import androidx.compose.ui.text.font.FontWeight

66	import androidx.compose.ui.text.style.TextAlign

67	import androidx.compose.ui.unit.IntOffset

68	import androidx.compose.ui.unit.dp

69	import smartbin.data.Bin

70	import smartbin.data.BinStatus

71	import smartbin.data.Hub

72	import smartbin.data.HubState

73	import smartbin.data.Source

74	import smartbin.data.v

75	import smartbin.notify.Sfx

76

77	@Composable

78	fun Overview(vm: AppViewModel, hub: HubState) {

79	    val c = Sb.c

80	    val bins = hub.bins

81	    val n = bins.size

82	    val grid = vm.binView == BinView.GRID || (vm.binView == BinView.AUTO && n > 2)

83	    val list = bins.filter {

84	        when (vm.binFilter) {

85	            BinFilter.ALL -> true

86	            BinFilter.ACTION -> it.needsAction

87	            BinFilter.MODES -> it.simulating || it.calibrating

88	        }

89	    }

90	    ScrollPage {

91	        liveBanners(hub).forEach { Banner(it.tone, it.icon, it.title, it.small, iconMod = Modifier.shake(it.icon === Ic.tamper)) }

92	        val head: @Composable () -> Unit = {

93	            Row(Modifier.fillMaxWidth().defaultMinSize(minHeight = 40.dp), verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(10.dp)) {

94	                Sh(Ic.bins, "Bins · $n", c.fill)

95	                Spacer(Modifier.weight(1f))

96	                if (n > 0) TwoWay {

97	                    Tw(grid, { vm.binView = BinView.GRID }, icon = Ic.grid, grow = null, minHeight = 36.dp)

98	                    Tw(!grid, { vm.binView = BinView.CARDS }, icon = Ic.layers, grow = null, minHeight = 36.dp)

99	                }

100	                if (vm.operator) AddMini { vm.addN = 1; vm.overlay = Overlay.AddBins }

101	            }

102	        }

103	        if (n == 0) {

104	            head()

105	            Card { Empty(Ic.bins, "No bins yet. Tap Add.") }

106	            return@ScrollPage

107	        }

108	        if (grid) {

109	            CollectNext(vm, bins)

110	            head()

111	            if (n > 2) Row(Modifier.fillMaxWidth().horizontalScroll(rememberScrollState()).padding(vertical = 2.dp), horizontalArrangement = Arrangement.spacedBy(9.dp)) {

112	                FilterKey(Ic.layers, "All $n", vm.binFilter == BinFilter.ALL) { vm.binFilter = BinFilter.ALL }

113	                FilterKey(Ic.tamper, "Needs action ${bins.count { it.needsAction }}", vm.binFilter == BinFilter.ACTION, Sb.c.tamper) { vm.binFilter = BinFilter.ACTION }

114	                FilterKey(Ic.flask, "Sim / Cal ${bins.count { it.simulating || it.calibrating }}", vm.binFilter == BinFilter.MODES, Sb.c.sim) { vm.binFilter = BinFilter.MODES }

115	            }

116	            if (list.isEmpty()) Card { Empty(Ic.check, "No bins match this filter.") }

117	            else TileGrid(list) { b -> Tile(b) { vm.overlay = Overlay.BinDetail(b.id) } }

118	            TileLegend()

119	        } else {

120	            head()

121	            bins.forEach { BinCard(vm, it, hub) }

122	            CollectNext(vm, bins)

123	        }

124	    }

125	}

126

127	@Composable

128	private fun AddMini(onClick: () -> Unit) {

129	    val c = Sb.c

130	    val src = remember { MutableInteractionSource() }

131	    val pressed by src.collectIsPressedAsState()

132	    Row(

133	        Modifier

134	            .height(40.dp)

135	            .then(if (pressed) Modifier.clip(RoundedCornerShape(20.dp)).background(c.base).sunk(c, 20.dp) else Modifier.bump(c, 20.dp))

136	            .press(true, Sfx.OPEN, src, onClick)

137	            .padding(horizontal = 14.dp),

138	        verticalAlignment = Alignment.CenterVertically,

139	        horizontalArrangement = Arrangement.spacedBy(7.dp),

140	    ) {

141	        val t = if (pressed) c.accent else c.ink2

142	        Ico(Ic.plus, 17.dp, t)

143	        T("Add", size = 0.86f, weight = FontWeight.Bold, color = t)

144	    }

145	}

146

147	/** .btn.sm.binbtn: a key that stays pushed in while it is the chosen one. */

148	@Composable

149	fun FilterKey(icon: ImageVector?, text: String, on: Boolean, cc: Color = Sb.c.accent, simRing: Boolean = false, enabled: Boolean = true, onClick: () -> Unit) {

150	    val c = Sb.c

151	    Box(if (simRing && !on) Modifier.ring(c.simSoft, 11.dp) else Modifier) {

152	        Btn(

153	            text, onClick, icon = icon, small = true, pressedIn = on, cc = if (on) cc else c.ink2,

154	            sound = Sfx.CHOICE, enabled = enabled,

155	        )

156	    }

157	}

158

159	@Composable

160	fun Empty(icon: ImageVector, text: String) {

161	    val c = Sb.c

162	    Column(Modifier.fillMaxWidth().padding(vertical = 30.dp, horizontal = 14.dp), horizontalAlignment = Alignment.CenterHorizontally, verticalArrangement = Arrangement.spacedBy(10.dp)) {

163	        Ico(icon, 34.dp, c.ink2.soft(0.55f))

164	        T(text, size = 0.92f, weight = FontWeight.Bold, color = c.ink2, align = TextAlign.Center)

165	    }

166	}

167

168	// ---------- the bin card ----------

169

170	private fun band(f: Double) = when { f >= 1 -> 4; f >= .8 -> 3; f >= .5 -> 2; else -> 1 }

171

172	private fun lvColor(lv: Int, c: SbColors) = when (lv) { 0 -> c.lvCool; 1 -> c.lvLow; 2 -> c.lvMid; 3 -> c.lvHigh; else -> c.lvCrit }

173

174	@Composable

175	fun BinCard(vm: AppViewModel, b: Bin, hub: HubState, inSheet: Boolean = false) {

176	    val c = Sb.c

177	    val r = b.reading

178	    val cal = b.cal

179	    val op = vm.operator

180	    val fillTh = cal.v("fill_th")

181	    val calOn = b.calibrating

182	    val simOn = b.simulating

183	    val hot = r.status != BinStatus.OK || r.fill >= fillTh

184	    val urgent = b.urgent

185	    val stale = b.stale

186	    val haze = if (urgent && !inSheet) {

187	        val t = rememberInfiniteTransition(label = "haze")

188	        t.animateFloat(2f, 7f, infiniteRepeatable(tween(1300), RepeatMode.Reverse), label = "haze").value

189	    } else 2f

190	    val outline = when {

191	        calOn && simOn -> listOf(c.simSoft to 4.dp, c.calSoft to 9.dp)

192	        calOn -> listOf(c.calSoft to 4.dp)

193	        simOn -> listOf(c.simSoft to 4.dp)

194	        else -> emptyList()

195	    }

196	    Column(

197	        Modifier

198	            .fillMaxWidth()

199	            .drawBehind {

200	                outline.forEach { (col, out) ->

201	                    val o = out.toPx()

202	                    drawRoundRect(

203	                        col, topLeft = Offset(-o + 1.5.dp.toPx(), -o + 1.5.dp.toPx()),

204	                        size = Size(size.width + 2 * o - 3.dp.toPx(), size.height + 2 * o - 3.dp.toPx()),

205	                        cornerRadius = CornerRadius(18.dp.toPx() + o), style = Stroke(3.dp.toPx()),

206	                    )

207	                }

208	            }

209	            .nm(c, out = if (inSheet) Out.O1 else Out.O2, ring = if (hot && !inSheet) c.accentSoft else null, ringWidth = haze.dp)

210	            .padding(14.dp),

211	    ) {

212	        // Top: hero, name, fill

213	        Row(horizontalArrangement = Arrangement.spacedBy(12.dp)) {

214	            Box(Modifier.size(62.dp).well(c, 31.dp), contentAlignment = Alignment.Center) {

215	                if (calOn) Ico(Ic.calib, 46.dp, c.cal, Modifier.spin())

216	                else BinIcon(r.fill, statusColor(b, c), 46.dp)

217	            }

218	            Column(Modifier.weight(1f).padding(top = 4.dp)) {

219	                T(b.name, size = 1.2f, spacing = 0.02f, maxLines = 1)

220	                T(b.place, size = 0.86f, weight = FontWeight.Medium, color = c.ink2, maxLines = 1)

221	                if (stale) Row(Modifier.padding(top = 3.dp), verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(5.dp)) {

222	                    Ico(Ic.clock, 13.dp, c.bad)

223	                    T(lastSeen(b), size = 0.74f, color = c.bad, maxLines = 1)

224	                }

225	            }

226	            Column(horizontalAlignment = Alignment.End, verticalArrangement = Arrangement.spacedBy(8.dp)) {

227	                Row(Modifier.alpha(if (stale) 0.42f else 1f), verticalAlignment = Alignment.Bottom) {

228	                    T(Math.round(r.fill).toString(), size = 2.45f, color = c.accent, tnum = true, lineHeight = 1f)

229	                    T("%", Modifier.padding(bottom = 3.dp), size = 2.45f * 0.42f, color = c.accent, lineHeight = 1f)

230	                }

231	                if (op) IconKey(Ic.gear, { vm.openBinEdit(b) }, tint = c.ink2, iconSize = 16.dp, label = "SET", sound = Sfx.OPEN)

232	            }

233	        }

234	        // Six chips

235	        val live = b.source == Source.LIVE

236	        val cloud = b.source == Source.CLOUD

237	        val tsp = b.source == Source.THINGSPEAK

238	        val stKey = if (calOn) BinStatus.OK else r.status

239	        val stIcon = when (stKey) {

240	            BinStatus.TAMPER -> Ic.tamper; BinStatus.LOCKED -> Ic.lock(true); BinStatus.COLLECTING -> Ic.truck; else -> Ic.check

241	        }

242	        val stColor = when (stKey) { BinStatus.LOCKED -> c.sim; BinStatus.TAMPER -> c.tamper; BinStatus.COLLECTING -> c.cal; else -> c.ok }

243	        // While the bin is tipped but not yet latched, the firmware counts down before calling it tamper.

244	        val tilting = !calOn && r.tamperPhase == "COUNTDOWN" && stKey != BinStatus.TAMPER

245	        val stText = if (tilting) "TILTED" else when (stKey) { BinStatus.LOCKED -> "LOCKED"; BinStatus.TAMPER -> "TAMPER"; BinStatus.COLLECTING -> "PICKUP"; else -> "OK" }

246	        Column(Modifier.padding(top = 13.dp), verticalArrangement = Arrangement.spacedBy(7.dp)) {

247	            Row(horizontalArrangement = Arrangement.spacedBy(7.dp)) {

248	                Chip(Ic.lid(r.lidOpen), if (r.lidOpen) "OPEN" else "CLOSED", r.lidOpen, c.lid, Modifier.weight(1f))

249	                Chip(if (tilting) Ic.tamper else stIcon, stText, true, if (tilting) c.warn else stColor, Modifier.weight(1f), Modifier.shake(stKey == BinStatus.TAMPER || tilting))

250	                Chip(

251	                    if (cloud) Ic.cloud else if (tsp) Ic.chart else Ic.wifi(live),

252	                    when { live -> "WI-FI"; cloud -> "CLOUD"; tsp -> "TSPEAK"; stale -> "OFFLINE"; else -> "DEMO" }, true,

253	                    when { live -> c.link; cloud -> c.cloud; tsp -> c.ts; stale -> c.tamper; else -> c.warn }, Modifier.weight(1f),

254	                )

255	            }

256	            Row(horizontalArrangement = Arrangement.spacedBy(7.dp)) {

257	                Chip(Ic.flask, "SIM", simOn, c.sim, Modifier.weight(1f))

258	                Chip(Ic.calib, "CAL", calOn, c.cal, Modifier.weight(1f))

259	                Chip(Ic.motion(r.motion), "MOTION", r.motion, c.motion, Modifier.weight(1f))

260	            }

261	        }

262	        // The fill bar with ticks at 50 and the limit

263	        val shown by animateFloatAsState(r.fill.coerceIn(0.0, 100.0).toFloat(), tween(600), label = "bar")

264	        val barColor = levelColor(b, c)

265	        Column(Modifier.alpha(if (stale) 0.42f else 1f)) {

266	            Box(

267	                Modifier

268	                    .padding(top = 16.dp, bottom = 6.dp)

269	                    .fillMaxWidth()

270	                    .height(24.dp)

271	                    .well(c, 12.dp, In.I2)

272	                    .drawBehind {

273	                        val p = 3.dp.toPx()

274	                        val w = (size.width * shown / 100f - 2 * p).coerceAtLeast(0f)

275	                        if (w > 0) drawRoundRect(barColor, topLeft = Offset(p, p), size = Size(w, size.height - 2 * p), cornerRadius = CornerRadius(10.dp.toPx()))

276	                        listOf(50.0, fillTh).forEach { at ->

277	                            val x = (size.width * at / 100).toFloat()

278	                            drawRoundRect(

279	                                c.ink.soft(0.35f), topLeft = Offset(x - 1.5.dp.toPx(), 4.dp.toPx()),

280	                                size = Size(3.dp.toPx(), size.height - 8.dp.toPx()), cornerRadius = CornerRadius(2.dp.toPx()),

281	                            )

282	                        }

283	                    },

284	            )

285	            Ticks(listOf(0.0, 50.0, fillTh, 100.0))

286	            if (r.fault("HC-SR04")) T("Fill sensor not answering. The bar shows the last reading.", Modifier.padding(top = 4.dp), size = 0.74f, weight = FontWeight.Bold, color = c.bad)

287	            CardFoot(b, barColor)

288	        }

289	        // Four sensor tiles

290	        Row(Modifier.padding(top = 12.dp).alpha(if (stale) 0.42f else 1f), horizontalArrangement = Arrangement.spacedBy(8.dp)) {

291	            val tTh = cal.v("temp_th")

292	            val hTh = cal.v("hum_th")

293	            val wCap = cal.v("weight_cap")

294	            val gTh = cal.v("gas_th")

295	            // A sensor that stops answering shows -- and says so, instead of an old number.

296	            val bme = r.fault("BME680")

297	            val hx = r.fault("HX711")

298	            val warm = r.warming && (b.sim.mask and 4) == 0

299	            if (bme) SensorTile(Ic.temp(r.temp), c.temp, "--", "Not answering", true, 3, Modifier.weight(1f))

300	            else SensorTile(Ic.temp(r.temp), c.temp, "${fmt(r.temp, 1)}°", "Temperature", r.temp >= tTh, if (r.temp < 22) 0 else band(r.temp / tTh), Modifier.weight(1f))

301	            if (bme) SensorTile(Ic.humid, c.humid, "--", "Not answering", true, 3, Modifier.weight(1f))

302	            else SensorTile(Ic.humid, c.humid, "${Math.round(r.hum)}%", "Humidity", r.hum >= hTh,

303	                when { r.hum >= hTh - 10 -> 3; r.hum >= 60 -> 2; r.hum <= 35 -> 0; else -> 1 }, Modifier.weight(1f))

304	            if (hx) SensorTile(Ic.weight(0.0), c.weight, "--", "Not answering", true, 3, Modifier.weight(1f))

305	            else SensorTile(Ic.weight(r.weight / wCap), c.weight, "%.2fkg".format(r.weight / 1000), "Weight", r.weight >= wCap, band(r.weight / wCap), Modifier.weight(1f))

306	            when {

307	                bme -> SensorTile(Ic.gas, c.gas, "--", "Not answering", true, 3, Modifier.weight(1f))

308	                warm -> SensorTile(Ic.gas, c.gas, r.iaqWarmS?.takeIf { it > 0 }?.let { "${it / 60}:%02d".format(it % 60) } ?: "...", "Air warming up", false, 0, Modifier.weight(1f))

309	                else -> SensorTile(Ic.gas, c.gas, Math.round(r.gas).toString(), "Air quality", r.gas >= gTh, band(r.gas / gTh), Modifier.weight(1f))

310	            }

311	        }

312	        // LOCK or UNLOCK, RESET TAMPER (operator), COLLECT or DONE

313	        // A command sent through Firebase or ThingSpeak holds every key until the bin confirms it.

314	        val wait = hub.pending[b.id]

315	        val busy = calOn || stale || wait != null

316	        val locked = r.status == BinStatus.LOCKED

317	        val tamper = r.status == BinStatus.TAMPER

318	        val coll = r.status == BinStatus.COLLECTING

319	        // Once the bin answers, the key that sent the command flashes green (confirmed) or red (timed out).

320	        val done = vm.settled[b.id]

321	        val fade = settleFade(done)

322	        fun flashFor(vararg cmds: String) = if (done != null && done.cmd in cmds) Modifier.settleFlash(done.ok, fade, c) else Modifier

323	        Row(Modifier.padding(top = 14.dp), horizontalArrangement = Arrangement.spacedBy(9.dp)) {

324	            val m = Modifier.weight(1f)

325	            val lm = m.then(flashFor("LOCK", "UNLOCK"))

326	            val cm = m.then(flashFor("COLLECT", "COLLECT_DONE"))

327	            if (wait != null && (wait.cmd == "LOCK" || wait.cmd == "UNLOCK")) WaitKey(wait, if (wait.cmd == "LOCK") "LOCKING" else "UNLOCKING", c.sim, m)

328	            else if (locked) Btn("UNLOCK", { vm.command(b.id, "UNLOCK") }, lm, Ic.lock(false), Look.SOLID, c.sim, enabled = !busy, stack = true, sound = Sfx.UNLOCK)

329	            else Btn("LOCK", { vm.command(b.id, "LOCK") }, lm, Ic.lock(true), Look.TINT, c.sim, enabled = !busy && !tamper, stack = true, sound = Sfx.LOCK)

330	            if (op) Btn(

331	                "RESET TAMPER", { vm.command(b.id, "CLEAR_TAMPER") }, m.then(flashFor("CLEAR_TAMPER")), Ic.tamper, if (tamper) Look.DANGER else Look.TINT, c.tamper,

332	                enabled = tamper && !stale && wait == null, stack = true, sound = Sfx.CONFIRM,

333	            )

334	            if (wait != null && wait.cmd == "COLLECT") WaitKey(wait, "STARTING", c.link, m)

335	            else if (coll) Btn("DONE", { vm.command(b.id, "COLLECT_DONE") }, cm, Ic.check, Look.SOLID, c.cal, enabled = !stale && wait == null, stack = true, sound = Sfx.DONE)

336	            else Btn("COLLECT", { vm.command(b.id, "COLLECT") }, cm, Ic.truck, Look.TINT, c.link, enabled = !busy, stack = true, sound = Sfx.PICKUP)

337	        }

338	        if (wait != null) {

339	            val via = if (wait.via == Source.THINGSPEAK) "ThingSpeak" else "Firebase"

340	            T(

341	                "Sent through $via. The keys stay locked until ${b.name} confirms.", Modifier.padding(top = 8.dp).fillMaxWidth(),

342	                size = 0.74f, weight = FontWeight.Bold, color = if (wait.via == Source.THINGSPEAK) c.ts else c.cloud, align = TextAlign.Center,

343	            )

344	        }

345	    }

346	}

347

348	/** Corner radius of a stacked key, for the lines drawn round it. */

349	private val KEY_RADIUS = 13.dp

350

351	/**

352	 * A key held down while a cloud command waits for the bin: pushed in under

353	 * a light veil, its icon turning, and a line round its edge running down to

354	 * the deadline, moved every frame.

355	 */

356	@Composable

357	private fun WaitKey(p: smartbin.data.Pending, label: String, cc: Color, modifier: Modifier) {

358	    val c = Sb.c

359	    val clock = remember { mutableLongStateOf(System.currentTimeMillis()) }

360	    LaunchedEffect(p) {

361	        while (true) withFrameMillis { clock.longValue = System.currentTimeMillis() }

362	    }

363	    Btn(

364	        label, {}, modifier.drawWithContent {

365	            drawContent()

366	            val left = ((p.deadline - clock.longValue).toFloat() / (p.deadline - p.at).coerceAtLeast(1)).coerceIn(0f, 1f)

367	            drawRoundRect(c.base.soft(0.35f), cornerRadius = CornerRadius(KEY_RADIUS.toPx()))

368	            edgeLine(1f, cc.soft(0.22f))

369	            edgeLine(left, cc)

370	        },

371	        Ic.reload, Look.TINT, cc, stack = true, pressedIn = true, iconMod = Modifier.spin(true), sound = Sfx.DENIED,

372	    )

373	}

374

375	/** The part [part] (0 to 1) of a line round a key's edge, from its top-left corner. */

376	private fun DrawScope.edgeLine(part: Float, color: Color) {

377	    if (part <= 0f) return

378	    val w = 3.dp.toPx()

379	    val r = (KEY_RADIUS.toPx() - w / 2f).coerceAtLeast(0f)

380	    val whole = Path().apply { addRoundRect(RoundRect(w / 2f, w / 2f, size.width - w / 2f, size.height - w / 2f, CornerRadius(r, r))) }

381	    val measure = PathMeasure().apply { setPath(whole, false) }

382	    val piece = Path()

383	    measure.getSegment(0f, measure.length * part, piece, true)

384	    drawPath(piece, color, style = Stroke(width = w, cap = StrokeCap.Round))

385	}

386

387	/** Runs from 1 to 0 over [SETTLE_MILLIS] each time a new answer arrives. */

388	@Composable

389	private fun settleFade(s: Settled?): State<Float> {

390	    val fade = remember(s) { Animatable(if (s == null) 0f else 1f) }

391	    LaunchedEffect(s) {

392	        if (s != null) fade.animateTo(0f, tween(SETTLE_MILLIS.toInt(), easing = LinearEasing))

393	    }

394	    return fade.asState()

395	}

396

397	/** A green or red wash and edge over a key, fading out as [fade] runs down. */

398	private fun Modifier.settleFlash(ok: Boolean, fade: State<Float>, c: SbColors): Modifier = drawWithContent {

399	    drawContent()

400	    val f = fade.value

401	    if (f <= 0f) return@drawWithContent

402	    val tone = if (ok) c.ok else c.bad

403	    drawRoundRect(tone.soft(0.28f * f), cornerRadius = CornerRadius(KEY_RADIUS.toPx()))

404	    edgeLine(1f, tone.soft(f))

405	}

406

407	/** .ticks: 0, 50, the limit and 100 under the bar, each centred on its mark. */

408	@Composable

409	private fun Ticks(at: List<Double>) {

410	    val c = Sb.c

411	    BoxWithConstraints(Modifier.fillMaxWidth().height(18.dp)) {

412	        val w = maxWidth

413	        at.forEachIndexed { i, v ->

414	            val label = Math.round(v).toString()

415	            val x = w * (v / 100).toFloat()

416	            val align = when (i) { 0 -> 0f; at.lastIndex -> 1f; else -> 0.5f }

417	            Box(Modifier.offset { IntOffset((x.toPx() - 30.dp.toPx() * align * 2).toInt(), 0) }.width(60.dp)) {

418	                T(

419	                    label, Modifier.fillMaxWidth(), size = 0.76f, weight = FontWeight.Bold, color = c.ink2, maxLines = 1,

420	                    align = when (i) { 0 -> TextAlign.Start; at.lastIndex -> TextAlign.End; else -> TextAlign.Center },

421	                )

422	            }

423	        }

424	    }

425	}

426

427	@Composable

428	private fun SensorTile(icon: ImageVector, cc: Color, value: String, label: String, alarm: Boolean, level: Int, modifier: Modifier) {

429	    val c = Sb.c

430	    val surface = if (alarm) Modifier.brutal(c, 10.dp, c.bad, off = 3.dp) else Modifier.well(c, 13.dp)

431	    Column(

432	        modifier.then(surface).padding(top = 10.dp, bottom = 8.dp, start = 2.dp, end = 2.dp),

433	        horizontalAlignment = Alignment.CenterHorizontally,

434	        verticalArrangement = Arrangement.spacedBy(4.dp),

435	    ) {

436	        Box(contentAlignment = Alignment.Center) {

437	            if (alarm) Box(Modifier.size(21.dp).breathe(true, 2600).clip(RoundedCornerShape(50)).background(Color.White.soft(0.38f)))

438	            Ico(icon, 21.dp, if (alarm) Color.White else cc)

439	        }

440	        T(value, size = 1.04f, color = if (alarm) Color.White else lvColor(level, c), tnum = true, maxLines = 1, lineHeight = 1.05f, fit = true)

441	        T(label, size = 0.66f, weight = FontWeight.Bold, color = if (alarm) Color.White else c.ink2, fit = true)

442	    }

443	}

444

445	// ---------- collection priority ----------

446

447	// Sort key for Collection priority. Full bins sort first (negative values),

448	// then by hours left to the fill threshold, then bins that are not filling.

449	// The fill rate comes from Hub.ratePerHour: the change over the last hour,

450	// needing at least 6 history points, so a new bin shows "Not filling" at first.

451	/** Lower is more urgent: a full bin, then hours to full, then bins that are not filling. */

452	private fun urgency(b: Bin): Double {

453	    val p = b.reading.fill

454	    val th = b.cal.v("fill_th")

455	    if (p >= th) return -1000 + p

456	    val rate = if (b.reading.status == BinStatus.COLLECTING) 0.0 else Hub.ratePerHour(b)

457	    return if (rate > 0.05) (th - p) / rate else 9999 - p

458	}

459

460	/** The last hour of fill as a small line, and how long ago the last reading came. */

461	@Composable

462	private fun CardFoot(b: Bin, cc: Color) {

463	    val c = Sb.c

464	    val history by Hub.history.collectAsState()

465	    var now by remember { mutableLongStateOf(System.currentTimeMillis()) }

466	    LaunchedEffect(Unit) {

467	        while (true) {

468	            kotlinx.coroutines.delay(1000)

469	            now = System.currentTimeMillis()

470	        }

471	    }

472	    val hour = 3_600_000L

473	    val pts = history[b.id].orEmpty().filter { now - it.t <= hour }

474	    Row(

475	        Modifier.fillMaxWidth().padding(top = 8.dp),

476	        verticalAlignment = Alignment.CenterVertically,

477	        horizontalArrangement = Arrangement.spacedBy(10.dp),

478	    ) {

479	        T("1 h", size = 0.68f, weight = FontWeight.Bold, color = c.ink2)

480	        Canvas(Modifier.weight(1f).height(22.dp)) {

481	            if (pts.size >= 2) {

482	                val line = Path()

483	                pts.forEachIndexed { k, s ->

484	                    val x = ((s.t - (now - hour)).toFloat() / hour * size.width).coerceIn(0f, size.width)

485	                    val y = size.height - (s.fill.coerceIn(0f, 100f) / 100f) * size.height

486	                    if (k == 0) line.moveTo(x, y) else line.lineTo(x, y)

487	                }

488	                drawPath(line, cc, style = Stroke(2.dp.toPx(), cap = StrokeCap.Round))

489	            }

490	        }

491	        val age = (now - b.lastSeen) / 1000

492	        T(

493	            when {

494	                b.lastSeen <= 0 -> "No reading yet"

495	                age < 5 -> "Updated just now"

496	                age < 60 -> "Updated $age s ago"

497	                age < 3600 -> "Updated ${age / 60} min ago"

498	                else -> "Updated ${age / 3600} h ago"

499	            },

500	            size = 0.7f, weight = FontWeight.SemiBold, color = if (b.stale) c.bad else c.ink2, maxLines = 1,

501	        )

502	    }

503	}

504

505	// The tags follow from this: under 2 h SOON, under 8 h TODAY, longer LATER,

506	// no rate IDLE, full NOW. The dashboard uses the same limits.

507	/** Time to full from how fast the bin has been filling. Returns the text and how near it is: 0 near, 1 mid, 2 far, 3 none. */

508	fun etaOf(b: Bin): Pair<String, Int> {

509	    val left = b.cal.v("fill_th") - b.reading.fill

510	    val rate = if (b.reading.status == BinStatus.COLLECTING) 0.0 else Hub.ratePerHour(b)

511	    if (left <= 0) return "Full now" to 0

512	    if (rate <= 0.05) return "Not filling" to 3

513	    val h = left / rate

514	    val t = if (h < 1) "~${Math.round(h * 60)} min" else "~${"%.1f".format(h)} h"

515	    return "$t to full" to when { h < 2 -> 0; h < 8 -> 1; else -> 2 }

516	}

517

518	@Composable

519	private fun CollectNext(vm: AppViewModel, bins: List<Bin>) {

520	    val c = Sb.c

521	    // The same order and tags as the dashboard: full bins, then the soonest to fill.

522	    val rows = bins.sortedBy { urgency(it) }

523	    val show = if (vm.prioAll) rows else rows.take(3)

524	    Card {

525	        Sh(Ic.rank, "Collection priority", c.fill)

526	        Column(verticalArrangement = Arrangement.spacedBy(10.dp)) {

527	            show.forEachIndexed { k, b ->

528	                val p = Math.round(b.reading.fill)

529	                val th = b.cal.v("fill_th")

530	                val (eta, near) = etaOf(b)

531	                val (tag, tagColor) = when {

532	                    p >= th -> "NOW" to c.bad

533	                    near == 3 -> "IDLE" to c.ink2

534	                    near == 0 -> "SOON" to c.bad

535	                    near == 1 -> "TODAY" to c.warn

536	                    else -> "LATER" to c.ok

537	                }

538	                val edge = when (near) { 0 -> c.bad; 1 -> c.warn; 2 -> c.ok; else -> c.ink2 }

539	                val src = remember { MutableInteractionSource() }

540	                val pressed by src.collectIsPressedAsState()

541	                Row(

542	                    Modifier

543	                        .fillMaxWidth()

544	                        .then(if (pressed) Modifier.clip(RoundedCornerShape(14.dp)).background(c.base).sunk(c, 14.dp) else Modifier.bump(c, 14.dp))

545	                        .drawBehind { drawRect(edge, size = Size(5.dp.toPx(), size.height)) }

546	                        .press(true, Sfx.OPEN, src) { vm.overlay = Overlay.BinDetail(b.id) }

547	                        .padding(start = 16.dp, end = 13.dp, top = 11.dp, bottom = 11.dp),

548	                    verticalAlignment = Alignment.CenterVertically,

549	                    horizontalArrangement = Arrangement.spacedBy(11.dp),

550	                ) {

551	                    Box(Modifier.size(28.dp).well(c, 14.dp), contentAlignment = Alignment.Center) { T("${k + 1}", size = 0.9f) }

552	                    Column(Modifier.weight(1f)) {

553	                        T(b.name, weight = FontWeight.Bold, maxLines = 1)

554	                        Row(Modifier.padding(top = 2.dp), verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(4.dp)) {

555	                            Ico(Ic.stopwatch, 12.dp, c.ink2)

556	                            T(eta, size = 0.76f, weight = FontWeight.SemiBold, color = c.ink2, maxLines = 1)

557	                        }

558	                    }

559	                    T("$p%", tnum = true)

560	                    Tag(tag, tagColor)

561	                }

562	            }

563	        }

564	        if (rows.size > 3) Btn(

565	            if (vm.prioAll) "Show top 3" else "Show all ${rows.size} bins", { vm.prioAll = !vm.prioAll },

566	            Modifier.fillMaxWidth(), if (vm.prioAll) Ic.chev else Ic.rank, small = true,

567	        )

568	        Hint("Full bins first, then the ones that will fill soonest. Time to full comes from how fast each bin filled over the last hour. Tap a row to open that bin.")

569	    }

570	}

571

572	// ---------- many bins: the grid ----------

573

574	@Composable

575	private fun TileGrid(bins: List<Bin>, tile: @Composable (Bin) -> Unit) {

576	    val cols = 3

577	    Column(verticalArrangement = Arrangement.spacedBy(12.dp)) {

578	        bins.chunked(cols).forEach { row ->

579	            Row(horizontalArrangement = Arrangement.spacedBy(12.dp)) {

580	                row.forEach { b -> Box(Modifier.weight(1f)) { tile(b) } }

581	                repeat(cols - row.size) { Spacer(Modifier.weight(1f)) }

582	            }

583	        }

584	    }

585	}

586

587	@Composable

588	private fun Tile(b: Bin, onClick: () -> Unit) {

589	    val c = Sb.c

590	    val r = b.reading

591	    val calOn = b.calibrating

592	    val simOn = b.simulating

593	    val lc = levelColor(b, c)

594	    val stale = b.stale

595	    val src = remember { MutableInteractionSource() }

596	    val pressed by src.collectIsPressedAsState()

597	    val ringW = if (b.urgent) {

598	        val t = rememberInfiniteTransition(label = "thaze")

599	        t.animateFloat(2.5f, 5f, infiniteRepeatable(tween(1100), RepeatMode.Reverse), label = "thaze").value

600	    } else 0f

601	    val fill: Brush = when {

602	        calOn && simOn -> Brush.linearGradient(

603	            0f to mix(c.sim, 0.22f, c.base), 0.5f to mix(c.sim, 0.22f, c.base), 0.5f to mix(c.cal, 0.22f, c.base), 1f to mix(c.cal, 0.22f, c.base),

604	        )

605	        calOn -> Brush.linearGradient(listOf(mix(c.cal, 0.18f, c.base), mix(c.cal, 0.18f, c.base)))

606	        simOn -> Brush.linearGradient(listOf(mix(c.sim, 0.18f, c.base), mix(c.sim, 0.18f, c.base)))

607	        else -> Brush.linearGradient(listOf(c.base, c.base))

608	    }

609	    Box(Modifier.fillMaxWidth().alpha(if (stale) 0.5f else 1f)) {

610	        Column(

611	            Modifier

612	                .fillMaxWidth()

613	                .then(if (pressed) Modifier else Modifier.raised(c, 16.dp, ring = if (b.urgent) c.bad.soft(1f - (ringW - 2.5f) / 2.5f * 0.55f) else null, ringWidth = ringW.dp))

614	                .clip(RoundedCornerShape(16.dp))

615	                .background(fill)

616	                .then(if (pressed) Modifier.sunk(c, 16.dp) else Modifier)

617	                .press(true, Sfx.OPEN, src, onClick)

618	                .padding(start = 6.dp, end = 6.dp, top = 10.dp, bottom = 9.dp),

619	            horizontalAlignment = Alignment.CenterHorizontally,

620	            verticalArrangement = Arrangement.spacedBy(3.dp),

621	        ) {

622	            BinIcon(r.fill, lc, 42.dp)

623	            T(b.name, size = 0.84f, maxLines = 1)

624	            T("${Math.round(r.fill)}%", size = 1.22f, color = lc, tnum = true, lineHeight = 1.05f)

625	            Row(Modifier.height(15.dp), horizontalArrangement = Arrangement.spacedBy(4.dp, Alignment.CenterHorizontally)) {

626	                if (r.status == BinStatus.TAMPER) Ico(Ic.tamper, 15.dp, c.tamper, Modifier.shake())

627	                if (r.status == BinStatus.LOCKED) Ico(Ic.lock(true), 15.dp, c.sim)

628	                if (r.status == BinStatus.COLLECTING) Ico(Ic.truck, 15.dp, c.cal)

629	                if (simOn) Ico(Ic.flask, 15.dp, c.sim)

630	                if (calOn) Ico(Ic.calib, 15.dp, c.cal)

631	                if (r.lidOpen && r.status != BinStatus.COLLECTING) Ico(Ic.lid(true), 15.dp, c.lid)

632	                if (stale) Ico(Ic.wifi(false), 15.dp, c.tamper)

633	            }

634	        }

635	        if (b.sensorOver) Adot(Modifier.align(Alignment.TopEnd).padding(6.dp))

636	    }

637	}

638

639	/** .adot: the brutal "!" for a sensor over its limit. */

640	@Composable

641	fun Adot(modifier: Modifier = Modifier, small: Boolean = false) {

642	    val c = Sb.c

643	    Box(modifier.size(if (small) 16.dp else 19.dp).brutal(c, 5.dp, c.bad, off = 2.dp), contentAlignment = Alignment.Center) {

644	        T("!", size = (if (small) 10f else 11f) / 14f, weight = FontWeight.Black, color = Color.White, lineHeight = 1f)

645	    }

646	}

647

648	@OptIn(androidx.compose.foundation.layout.ExperimentalLayoutApi::class)

649	@Composable

650	private fun TileLegend() {

651	    val c = Sb.c

652	    @Composable

653	    fun key(fill: Brush, text: String, ring: Color? = null) {

654	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(5.dp)) {

655	            Box(Modifier.size(14.dp).then(if (ring != null) Modifier.ring(ring, 4.dp) else Modifier).clip(RoundedCornerShape(4.dp)).background(fill))

656	            T(text, size = 0.72f, weight = FontWeight.Bold, color = c.ink2)

657	        }

658	    }

659	    val solid = { col: Color -> Brush.linearGradient(listOf(col, col)) }

660	    androidx.compose.foundation.layout.FlowRow(

661	        horizontalArrangement = Arrangement.spacedBy(13.dp),

662	        verticalArrangement = Arrangement.spacedBy(7.dp),

663	    ) {

664	        key(solid(mix(c.sim, 0.3f, c.base)), "Simulation")

665	        key(solid(mix(c.cal, 0.3f, c.base)), "Calibration")

666	        key(

667	            Brush.linearGradient(0f to mix(c.sim, 0.34f, c.base), 0.5f to mix(c.sim, 0.34f, c.base), 0.5f to mix(c.cal, 0.34f, c.base), 1f to mix(c.cal, 0.34f, c.base)),

668	            "Both",

669	        )

670	        key(solid(c.base), "Full or tamper", ring = c.bad)

671	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(5.dp)) {

672	            Adot(small = true)

673	            T("Sensor over limit", size = 0.72f, weight = FontWeight.Bold, color = c.ink2)

674	        }

675	    }

676	}

Android App/app/src/main/java/smartbin/ui/Charts.kt

1	// ============================================================================

2	// ui/Charts.kt - the Analytics tab: fill level and environment line charts

3	// Data comes from Hub.history (one sample every 10 s, kept for 24 h). Each

4	// series is cut to the chosen time range and averaged into at most 60 points

5	// so long ranges stay fast to draw. "Collecting readings for this chart"

6	// means fewer than 2 points fall inside the range yet.

7	// ============================================================================

8	package smartbin.ui

9

10	import androidx.compose.foundation.Canvas

11	import androidx.compose.foundation.background

12	import androidx.compose.foundation.horizontalScroll

13	import androidx.compose.foundation.interaction.MutableInteractionSource

14	import androidx.compose.foundation.interaction.collectIsPressedAsState

15	import androidx.compose.foundation.layout.Arrangement

16	import androidx.compose.foundation.layout.Box

17	import androidx.compose.foundation.layout.Column

18	import androidx.compose.foundation.layout.ExperimentalLayoutApi

19	import androidx.compose.foundation.layout.FlowRow

20	import androidx.compose.foundation.layout.Row

21	import androidx.compose.foundation.layout.defaultMinSize

22	import androidx.compose.foundation.layout.fillMaxSize

23	import androidx.compose.foundation.layout.fillMaxWidth

24	import androidx.compose.foundation.layout.height

25	import androidx.compose.foundation.layout.navigationBarsPadding

26	import androidx.compose.foundation.layout.padding

27	import androidx.compose.foundation.layout.size

28	import androidx.compose.foundation.layout.statusBarsPadding

29	import androidx.compose.foundation.layout.widthIn

30	import androidx.compose.foundation.rememberScrollState

31	import androidx.compose.foundation.shape.RoundedCornerShape

32	import androidx.compose.runtime.Composable

33	import androidx.compose.runtime.collectAsState

34	import androidx.compose.runtime.getValue

35	import androidx.compose.runtime.remember

36	import androidx.compose.ui.Alignment

37	import androidx.compose.ui.Modifier

38	import androidx.compose.ui.draw.clip

39	import androidx.compose.ui.geometry.CornerRadius

40	import androidx.compose.ui.geometry.Offset

41	import androidx.compose.ui.geometry.Size

42	import androidx.compose.ui.graphics.Brush

43	import androidx.compose.ui.graphics.Color

44	import androidx.compose.ui.graphics.Path

45	import androidx.compose.ui.graphics.PathEffect

46	import androidx.compose.ui.graphics.StrokeCap

47	import androidx.compose.ui.graphics.StrokeJoin

48	import androidx.compose.ui.graphics.drawscope.DrawScope

49	import androidx.compose.ui.graphics.drawscope.Stroke

50	import androidx.compose.ui.graphics.graphicsLayer

51	import androidx.compose.ui.text.TextMeasurer

52	import androidx.compose.ui.text.TextStyle

53	import androidx.compose.ui.text.drawText

54	import androidx.compose.ui.text.font.FontWeight

55	import androidx.compose.ui.text.rememberTextMeasurer

56	import androidx.compose.ui.unit.Dp

57	import androidx.compose.ui.unit.dp

58	import androidx.compose.ui.unit.sp

59	import smartbin.data.Bin

60	import smartbin.data.Hub

61	import smartbin.data.HubState

62	import smartbin.data.Sample

63	import smartbin.data.v

64	import smartbin.notify.Sfx

65	import java.text.SimpleDateFormat

66	import java.util.Date

67	import java.util.Locale

68

69	class Series(val color: Color, val pts: List<Pair<Long, Float>>, val dash: Int = 0, val area: Boolean = false)

70	class Mark(val v: Float, val text: String)

71

72	/** One chart: its scale, lines, limit lines, legend and the line of text under it. */

73	class ChartSpec(

74	    val min: Float, val max: Float, val ticks: List<Float>, val series: List<Series>, val marks: List<Mark>,

75	    val legend: List<Triple<Color, Int, String>>, val note: String? = null,

76	)

77

78	private const val POINTS = 60

79

80	/** History of one bin over the window, averaged into at most 60 points so the line stays smooth. */

81	private fun window(h: List<Sample>, from: Long, to: Long, pick: (Sample) -> Float): List<Pair<Long, Float>> {

82	    val inside = h.filter { it.t in from..to }

83	    if (inside.size <= POINTS) return inside.map { it.t to pick(it) }

84	    val span = (to - from).toDouble() / POINTS

85	    return inside.groupBy { ((it.t - from) / span).toInt().coerceIn(0, POINTS - 1) }

86	        .toSortedMap()

87	        .map { (_, g) -> g.map { it.t }.average().toLong() to g.map(pick).average().toFloat() }

88	}

89

90	private val TEMP = Triple(0f, 85f, listOf(0f, 20f, 40f, 60f, 85f))

91	private val HUM = Triple(0f, 100f, listOf(0f, 25f, 50f, 75f, 100f))

92	private val PRESS = Triple(300f, 1100f, listOf(300f, 500f, 700f, 900f, 1100f))

93	private val GAS = Triple(0f, 500f, listOf(0f, 100f, 200f, 300f, 400f, 500f))

94

95	@Composable

96	fun fillSpec(vm: AppViewModel, hub: HubState, history: Map<String, List<Sample>>): ChartSpec {

97	    val c = Sb.c

98	    val now = System.currentTimeMillis()

99	    val from = now - vm.rangeMin * 60_000L

100	    val bins = hub.bins

101	    val all = vm.chartBin == null || bins.none { it.id == vm.chartBin }

102	    val ids = if (all) bins.sortedByDescending { it.reading.fill }.take(5) else bins.filter { it.id == vm.chartBin }

103	    val th = ids.firstOrNull()?.cal?.v("fill_th")?.toFloat() ?: 80f

104	    val series = ids.mapIndexed { k, b ->

105	        Series(binColor(bins.indexOf(b), c.isDark), window(history[b.id].orEmpty(), from, now) { it.fill }, if (all) k % 3 else 0, k == 0)

106	    }

107	    val legend = ids.mapIndexed { k, b -> Triple(binColor(bins.indexOf(b), c.isDark), if (all) k % 3 else 0, b.name) } +

108	        Triple(c.bad, 1, "Threshold")

109	    val note = if (all && bins.size > 5) "The 5 fullest of ${bins.size}. Pick a bin to see any other." else null

110	    return ChartSpec(0f, 100f, listOf(0f, 25f, 50f, 75f, 100f), series, listOf(Mark(th, "ALERT LIMIT ${th.toInt()}%")), legend, note)

111	}

112

113	@Composable

114	fun envSpec(vm: AppViewModel, hub: HubState, history: Map<String, List<Sample>>): ChartSpec? {

115	    val c = Sb.c

116	    val b = vm.binFor(vm.envBin, hub.bins) ?: return null

117	    val now = System.currentTimeMillis()

118	    val from = now - vm.rangeMin * 60_000L

119	    val h = history[b.id].orEmpty()

120	    val tTh = b.cal.v("temp_th").toFloat()

121	    val hTh = b.cal.v("hum_th").toFloat()

122	    val gTh = b.cal.v("gas_th").toFloat()

123	    val r = b.reading

124	    fun one(scale: Triple<Float, Float, List<Float>>, col: Color, name: String, unit: String, cur: String, lim: Float?, pick: (Sample) -> Float, note: String): ChartSpec {

125	        val legend = mutableListOf(Triple(col, 0, "$name, now **$cur $unit**"))

126	        if (lim != null) legend += Triple(c.bad, 1, "Alert limit ${lim.toInt()} $unit")

127	        legend += Triple(Color.Transparent, -1, "Scale ${scale.first.toInt()} to ${scale.second.toInt()} $unit")

128	        return ChartSpec(

129	            scale.first, scale.second, scale.third, listOf(Series(col, window(h, from, now, pick), area = true)),

130	            if (lim == null) emptyList() else listOf(Mark(lim, "ALERT LIMIT ${lim.toInt()} $unit")), legend, note,

131	        )

132	    }

133	    return when (vm.envMode) {

134	        EnvMode.TEMP -> one(TEMP, c.temp, "Temperature", "C", fmt(r.temp, 1), tTh, { it.temp },

135	            "Temperature of ${b.name} in degrees C, after the offset from Settings. The scale runs to 85 C, the most the BME680 can read.")

136	        EnvMode.HUMID -> one(HUM, c.humid, "Humidity", "%", fmt(r.hum, 0), hTh, { it.hum },

137	            "Relative humidity of ${b.name}, 0 to 100 percent, the full BME680 range. The dashed line is its alert limit, set in Settings.")

138	        EnvMode.GAS -> one(GAS, c.gas, "Air quality", "IAQ", fmt(r.gas, 0), gTh, { it.gas },

139	            "Air quality index of ${b.name} from the BME680, 0 to 500, lower is cleaner. The dashed line is its alert limit.")

140	        EnvMode.PRESS -> one(PRESS, c.press, "Pressure", "hPa", fmt(r.press, 1), null, { it.press },

141	            "Air pressure around ${b.name} on the BME680 range of 300 to 1100 hPa, so normal weather looks flat. Charted here only, with no tile and no alert.")

142	        EnvMode.ALL -> ChartSpec(

143	            0f, 200f, listOf(0f, 50f, 100f, 150f, 200f),

144	            listOf(

145	                Series(c.temp, window(h, from, now) { it.temp / tTh * 100 }, area = true),

146	                Series(c.gas, window(h, from, now) { it.gas / gTh * 100 }, dash = 1),

147	                Series(c.humid, window(h, from, now) { it.hum / hTh * 100 }, dash = 2),

148	            ),

149	            listOf(Mark(100f, "ALERT LIMIT (100%)")),

150	            listOf(

151	                Triple(c.temp, 0, "Temperature, limit ${tTh.toInt()} C"),

152	                Triple(c.humid, 2, "Humidity, limit ${hTh.toInt()} %RH"),

153	                Triple(c.gas, 1, "Air quality, limit ${gTh.toInt()} IAQ"),

154	                Triple(c.bad, 1, "Alert limit (100%)"),

155	            ),

156	            "Temperature, humidity and air quality are each drawn as a share of their own limit, so the dashed line at 100% is the limit for all three. The scale runs to 200%, which keeps that line in the middle. Pick one reading for real units.",

157	        )

158	    }

159	}

160

161	/**

162	 * The mockup's lineChart(): grid lines with their values, the lines with a

163	 * soft fill under the first, a dot at the newest point, the dashed alert

164	 * limit with its label, and the start time and "now" along the bottom.

165	 */

166	@Composable

167	fun LineChart(spec: ChartSpec, from: Long, modifier: Modifier = Modifier, height: Dp = 190.dp) {

168	    val c = Sb.c

169	    val measurer = rememberTextMeasurer()

170	    val start = remember(from / 60_000) { SimpleDateFormat("HH:mm", Locale.getDefault()).format(Date(from)) }

171	    Box(modifier.fillMaxWidth().then(if (height > 0.dp) Modifier.height(height) else Modifier.fillMaxSize()).well(c, 14.dp)) {

172	        Canvas(Modifier.fillMaxSize()) { chart(spec, from, start, measurer, c) }

173	        if (spec.series.all { it.pts.size < 2 }) {

174	            Box(Modifier.fillMaxSize(), contentAlignment = Alignment.Center) {

175	                T("Collecting readings for this chart", size = 0.8f, weight = FontWeight.Bold, color = c.ink2)

176	            }

177	        }

178	    }

179	}

180

181	private fun DrawScope.chart(spec: ChartSpec, from: Long, start: String, tm: TextMeasurer, c: SbColors) {

182	    val to = System.currentTimeMillis()

183	    val l = 32.dp.toPx()

184	    val r = 12.dp.toPx()

185	    val t = 12.dp.toPx()

186	    val b = 20.dp.toPx()

187	    val pw = size.width - l - r

188	    val ph = size.height - t - b

189	    fun y(v: Float) = t + ph - ((v.coerceIn(spec.min, spec.max) - spec.min) / (spec.max - spec.min)) * ph

190	    fun x(time: Long) = l + ((time - from).toFloat() / (to - from).toFloat()).coerceIn(0f, 1f) * pw

191	    val label = TextStyle(fontFamily = Inter, fontSize = 9.5.sp, fontWeight = FontWeight.Bold, color = c.ink2)

192	    spec.ticks.forEach { v ->

193	        val yy = y(v)

194	        drawLine(c.ink2.soft(0.22f), Offset(l, yy), Offset(size.width - r, yy), 1.dp.toPx())

195	        val m = tm.measure(Math.round(v).toString(), label)

196	        drawText(m, topLeft = Offset(l - 7.dp.toPx() - m.size.width, yy - m.size.height / 2f))

197	    }

198	    val dashes = listOf(null, floatArrayOf(7f, 5f), floatArrayOf(2.5f, 4.5f))

199	    spec.series.forEach { s ->

200	        if (s.pts.size < 2) return@forEach

201	        val line = Path()

202	        s.pts.forEachIndexed { k, (time, v) -> if (k == 0) line.moveTo(x(time), y(v)) else line.lineTo(x(time), y(v)) }

203	        if (s.area) {

204	            val area = Path().apply {

205	                addPath(line)

206	                lineTo(x(s.pts.last().first), t + ph)

207	                lineTo(x(s.pts.first().first), t + ph)

208	                close()

209	            }

210	            drawPath(area, Brush.verticalGradient(listOf(s.color.soft(0.24f), s.color.copy(alpha = 0f)), startY = t, endY = t + ph))

211	        }

212	        val d = dashes[s.dash.coerceIn(0, 2)]

213	        drawPath(

214	            line, s.color,

215	            style = Stroke(

216	                2.4.dp.toPx(), cap = StrokeCap.Round, join = StrokeJoin.Round,

217	                pathEffect = d?.let { PathEffect.dashPathEffect(floatArrayOf(it[0].dp.toPx(), it[1].dp.toPx())) },

218	            ),

219	        )

220	        val end = Offset(x(s.pts.last().first), y(s.pts.last().second))

221	        drawCircle(s.color.soft(0.22f), 7.5.dp.toPx(), end)

222	        drawCircle(s.color, 3.8.dp.toPx(), end)

223	    }

224	    val markStyle = TextStyle(fontFamily = Inter, fontSize = 8.6.sp, fontWeight = FontWeight.ExtraBold, color = c.bad)

225	    spec.marks.forEach { m ->

226	        val my = y(m.v)

227	        drawLine(c.bad, Offset(l, my), Offset(size.width - r, my), 1.6.dp.toPx(), pathEffect = PathEffect.dashPathEffect(floatArrayOf(7.dp.toPx(), 6.dp.toPx())))

228	        val txt = tm.measure(m.text, markStyle)

229	        val pad = 7.dp.toPx()

230	        val box = Size(txt.size.width + pad * 2, 14.dp.toPx())

231	        val tl = Offset(l + 8.dp.toPx(), my - 17.dp.toPx())

232	        drawRoundRect(c.well, tl, box, CornerRadius(4.dp.toPx()))

233	        drawRoundRect(c.bad, tl, box, CornerRadius(4.dp.toPx()), style = Stroke(1.2.dp.toPx()))

234	        drawText(txt, topLeft = Offset(tl.x + pad, tl.y + (box.height - txt.size.height) / 2f))

235	    }

236	    val bottom = TextStyle(fontFamily = Inter, fontSize = 9.sp, fontWeight = FontWeight.Bold, color = c.ink2)

237	    val st = tm.measure(start, bottom)

238	    drawText(st, topLeft = Offset(l, size.height - st.size.height - 2.dp.toPx()))

239	    val nw = tm.measure("now", bottom.copy(color = c.ok))

240	    drawText(nw, topLeft = Offset(size.width - r - nw.size.width, size.height - nw.size.height - 2.dp.toPx()))

241	}

242

243	@OptIn(ExperimentalLayoutApi::class)

244	@Composable

245	fun Legend(items: List<Triple<Color, Int, String>>, modifier: Modifier = Modifier) {

246	    val c = Sb.c

247	    FlowRow(modifier, horizontalArrangement = Arrangement.spacedBy(14.dp), verticalArrangement = Arrangement.spacedBy(6.dp)) {

248	        items.forEach { (col, dash, text) ->

249	            Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(6.dp)) {

250	                if (dash >= 0) Swatch(col, dash)

251	                T(text, size = 0.76f, weight = FontWeight.Bold, color = c.ink2, bold = c.ink)

252	            }

253	        }

254	    }

255	}

256

257	@Composable

258	fun RangeSeg(vm: AppViewModel) {

259	    val c = Sb.c

260	    TwoWay(full = true) {

261	        listOf(15 to "15 m", 60 to "1 h", 360 to "6 h", 1440 to "24 h").forEach { (m, label) ->

262	            Tw(vm.rangeMin == m, { vm.rangeMin = m }, label, cc = c.fill)

263	        }

264	    }

265	}

266

267	@Composable

268	fun EnvSeg(vm: AppViewModel) {

269	    val c = Sb.c

270	    TwoWay(full = true) {

271	        listOf(

272	            Triple(EnvMode.ALL, "ALL", Ic.layers to c.ink),

273	            Triple(EnvMode.TEMP, "TEMPERATURE", Ic.temp(30.0) to c.temp),

274	            Triple(EnvMode.HUMID, "HUMIDITY", Ic.humid to c.humid),

275	            Triple(EnvMode.GAS, "AIR QUALITY", Ic.gas to c.gas),

276	            Triple(EnvMode.PRESS, "PRESSURE", Ic.press to c.press),

277	        ).forEach { (m, label, ic) ->

278	            Tw(

279	                vm.envMode == m, { vm.envMode = m }, label, ic.first, ic.second,

280	                grow = if (m == EnvMode.ALL) 0.75f else 1f, stack = true, minHeight = 52.dp, textSize = 0.62f,

281	            )

282	        }

283	    }

284	}

285

286	/**

287	 * The bin selector: one row of keys while the bins fit, otherwise a picker

288	 * that opens a sheet. [allowAll] adds ALL BINS for the fill chart.

289	 */

290	@Composable

291	fun BinSeg(vm: AppViewModel, hub: HubState, forWhat: PickFor, selected: String?, allowAll: Boolean, locked: Boolean = false, onPick: (String?) -> Unit) {

292	    val c = Sb.c

293	    val bins = hub.bins

294	    val sel = if (allowAll) selected?.takeIf { id -> bins.any { it.id == id } } else (vm.binFor(selected, bins)?.id)

295	    if (bins.size <= (if (allowAll) 3 else 4)) {

296	        Row(Modifier.fillMaxWidth().horizontalScroll(rememberScrollState()).padding(vertical = 4.dp, horizontal = 2.dp), horizontalArrangement = Arrangement.spacedBy(9.dp)) {

297	            if (allowAll) FilterKey(Ic.layers, "ALL BINS", sel == null) { onPick(null) }

298	            bins.forEachIndexed { i, b ->

299	                FilterKey(Ic.fill, b.name, sel == b.id, binColor(i, c.isDark), simRing = b.simulating, enabled = !locked || sel == b.id) { onPick(b.id) }

300	            }

301	        }

302	        return

303	    }

304	    val b = bins.firstOrNull { it.id == sel }

305	    val i = bins.indexOf(b)

306	    val src = remember { MutableInteractionSource() }

307	    val pressed by src.collectIsPressedAsState()

308	    Row(

309	        Modifier

310	            .fillMaxWidth()

311	            .defaultMinSize(minHeight = 54.dp)

312	            .graphicsLayer { alpha = if (locked) 0.6f else 1f }

313	            .then(if (pressed) Modifier.clip(RoundedCornerShape(14.dp)).background(c.base).sunk(c, 14.dp) else Modifier.bump(c, 14.dp, ring = c.accentSoft))

314	            .press(!locked, Sfx.OPEN, src) { vm.overlay = Overlay.Pick(forWhat) }

315	            .padding(start = 8.dp, end = 12.dp, top = 6.dp, bottom = 6.dp),

316	        verticalAlignment = Alignment.CenterVertically,

317	        horizontalArrangement = Arrangement.spacedBy(10.dp),

318	    ) {

319	        if (b != null) BinIcon(b.reading.fill, binColor(i, c.isDark), 34.dp)

320	        else Box(Modifier.size(34.dp), contentAlignment = Alignment.Center) { Ico(Ic.layers, 21.dp, c.ink2) }

321	        Column(Modifier.weight(1f)) {

322	            T(b?.name ?: "All bins", size = 0.95f, maxLines = 1)

323	            T(

324	                if (b != null) "${b.place} · ${Math.round(b.reading.fill)}%" + modeTxt(b) else if (bins.size > 5) "The 5 fullest of ${bins.size} are drawn" else "${bins.size} bins",

325	                size = 0.76f, weight = FontWeight.SemiBold, color = c.ink2, maxLines = 1,

326	            )

327	        }

328	        T((if (b != null) "${i + 1} of ${bins.size}" else "") + if (locked) "\nlocked" else "", size = 0.72f, color = c.ink2, align = androidx.compose.ui.text.style.TextAlign.End)

329	        Ico(Ic.chev, 18.dp, c.ink2)

330	    }

331	}

332

333	fun modeTxt(b: Bin): String {

334	    val cal = b.calibrating

335	    val sim = b.simulating

336	    return when { cal && sim -> " · sim + cal"; cal -> " · calibrating"; sim -> " · simulation"; else -> "" }

337	}

338

339	@Composable

340	fun Charts(vm: AppViewModel, hub: HubState) {

341	    val c = Sb.c

342	    val history by Hub.history.collectAsState()

343	    val from = System.currentTimeMillis() - vm.rangeMin * 60_000L

344	    ScrollPage {

345	        Card {

346	            Row(verticalAlignment = Alignment.CenterVertically) {

347	                Sh(Ic.fill, "Fill level", c.fill, Modifier.weight(1f))

348	                IconKey(Ic.expand, { vm.fullscreen = "fill" }, tint = c.ink2, size = 40.dp, iconSize = 18.dp, radius = 12.dp, sound = Sfx.OPEN)

349	            }

350	            RangeSeg(vm)

351	            BinSeg(vm, hub, PickFor.CHART, vm.chartBin, allowAll = true) { vm.chartBin = it }

352	            val f = fillSpec(vm, hub, history)

353	            LineChart(f, from)

354	            Legend(f.legend)

355	            f.note?.let { Hint(it) }

356	        }

357	        val e = envSpec(vm, hub, history)

358	        if (e != null) Card {

359	            Row(verticalAlignment = Alignment.CenterVertically) {

360	                Sh(Ic.temp(30.0), "Environment", c.temp, Modifier.weight(1f))

361	                IconKey(Ic.expand, { vm.fullscreen = "env" }, tint = c.ink2, size = 40.dp, iconSize = 18.dp, radius = 12.dp, sound = Sfx.OPEN)

362	            }

363	            EnvSeg(vm)

364	            BinSeg(vm, hub, PickFor.ENV, vm.envBin, allowAll = false) { vm.envBin = it }

365	            LineChart(e, from)

366	            Legend(e.legend)

367	            e.note?.let { Hint(it) }

368	        }

369	    }

370	}

371

372	/** The sideways chart: a bar with the title, its picker and controls, then the chart over the whole width. */

373	@Composable

374	fun FullChart(vm: AppViewModel, hub: HubState) {

375	    val c = Sb.c

376	    val history by Hub.history.collectAsState()

377	    val env = vm.fullscreen == "env"

378	    val from = System.currentTimeMillis() - vm.rangeMin * 60_000L

379	    val spec = if (env) envSpec(vm, hub, history) else fillSpec(vm, hub, history)

380	    Column(

381	        Modifier.fillMaxSize().statusBarsPadding().navigationBarsPadding().padding(horizontal = 12.dp, vertical = 8.dp),

382	        verticalArrangement = Arrangement.spacedBy(8.dp),

383	    ) {

384	        Row(

385	            Modifier.fillMaxWidth().bump(c, 14.dp).padding(start = 6.dp, end = 6.dp, top = 5.dp, bottom = 5.dp),

386	            verticalAlignment = Alignment.CenterVertically,

387	            horizontalArrangement = Arrangement.spacedBy(10.dp),

388	        ) {

389	            val name = if (env) vm.binFor(vm.envBin, hub.bins)?.name.orEmpty() else hub.bins.firstOrNull { it.id == vm.chartBin }?.name ?: "All bins"

390	            Row(

391	                Modifier.defaultMinSize(minHeight = 44.dp).clip(RoundedCornerShape(12.dp))

392	                    .press(true, Sfx.OPEN) { vm.overlay = Overlay.Pick(if (env) PickFor.ENV else PickFor.CHART) }.padding(horizontal = 8.dp),

393	                verticalAlignment = Alignment.CenterVertically,

394	                horizontalArrangement = Arrangement.spacedBy(10.dp),

395	            ) {

396	                Sh(if (env) Ic.temp(30.0) else Ic.fill, if (env) "Environment" else "Fill level", if (env) c.temp else c.fill)

397	                T(name, size = 0.86f, maxLines = 1)

398	                Ico(Ic.chev, 16.dp, c.ink2)

399	            }

400	            Box(Modifier.weight(1f)) { if (env) EnvSeg(vm) else RangeSeg(vm) }

401	            IconKey(Ic.shrink, { vm.fullscreen = null }, tint = c.ink2, size = 40.dp, iconSize = 18.dp, radius = 12.dp, sound = Sfx.BACK)

402	        }

403	        if (spec != null) {

404	            Box(Modifier.weight(1f).fillMaxWidth(), contentAlignment = Alignment.Center) {

405	                LineChart(spec, from, Modifier.widthIn(max = 1400.dp), height = 0.dp)

406	            }

407	            Legend(spec.legend, Modifier.padding(horizontal = 6.dp))

408	        }

409	    }

410	}

Android App/app/src/main/java/smartbin/ui/Settings.kt

1	// ============================================================================

2	// ui/Settings.kt - the Settings tab, plus the Connection and Bluetooth pages

3	// Settings edits one bin's calibration (Cal keys such as d_empty, fill_th,

4	// temp_off). Fields are read-only until an operator turns calibration on,

5	// and the Save bar appears only when something changed.

6	// Connection sets up the Firebase account and ThingSpeak keys and shows which

7	// of the three routes is answering. Bluetooth walks through the Wi-Fi setup

8	// of a bin (pair, pick network, send, confirm).

9	// ============================================================================

10	package smartbin.ui

11

12	import androidx.activity.compose.rememberLauncherForActivityResult

13	import androidx.activity.result.contract.ActivityResultContracts

14	import androidx.compose.animation.AnimatedVisibility

15	import androidx.compose.animation.core.animateFloatAsState

16	import androidx.compose.foundation.background

17	import androidx.compose.foundation.layout.Arrangement

18	import androidx.compose.foundation.layout.Box

19	import androidx.compose.foundation.layout.Column

20	import androidx.compose.foundation.layout.Row

21	import androidx.compose.foundation.layout.defaultMinSize

22	import androidx.compose.foundation.layout.fillMaxSize

23	import androidx.compose.foundation.layout.fillMaxWidth

24	import androidx.compose.foundation.layout.height

25	import androidx.compose.foundation.layout.padding

26	import androidx.compose.foundation.layout.size

27	import androidx.compose.foundation.layout.widthIn

28	import androidx.compose.foundation.rememberScrollState

29	import androidx.compose.foundation.shape.RoundedCornerShape

30	import androidx.compose.runtime.Composable

31	import androidx.compose.runtime.getValue

32	import androidx.compose.ui.Alignment

33	import androidx.compose.ui.Modifier

34	import androidx.compose.ui.draw.alpha

35	import androidx.compose.ui.draw.clip

36	import androidx.compose.ui.draw.drawBehind

37	import androidx.compose.ui.geometry.CornerRadius

38	import androidx.compose.ui.geometry.Offset

39	import androidx.compose.ui.geometry.Size

40	import androidx.compose.ui.graphics.Color

41	import androidx.compose.ui.graphics.graphicsLayer

42	import androidx.compose.ui.graphics.vector.ImageVector

43	import androidx.compose.ui.text.font.FontWeight

44	import androidx.compose.ui.text.input.KeyboardType

45	import androidx.compose.ui.text.style.TextAlign

46	import androidx.compose.ui.unit.dp

47	import smartbin.data.Bin

48	import smartbin.data.Cal

49	import smartbin.data.HubState

50	import smartbin.data.Link

51	import smartbin.data.Source

52	import smartbin.data.v

53	import smartbin.notify.Sfx

54

55	@Composable

56	fun SettingsPage(vm: AppViewModel, hub: HubState) {

57	    val c = Sb.c

58	    val b = vm.binFor(vm.selSet, hub.bins)

59	    val scroll = rememberScrollState()

60	    val calHere = b != null && hub.calibBin == b.id

61	    val on = vm.operator && calHere && vm.edit != null

62	    val showSave = on && vm.dirty

63	    Box(Modifier.fillMaxSize()) {

64	        ScrollPage(state = scroll, bottomExtra = if (showSave) 72.dp else 0.dp) {

65	            Sh(Ic.sliders, "Settings", c.cal)

66	            if (b == null) {

67	                Card { Empty(Ic.bins, "No bins yet.") }

68	            } else {

69	                // Calibration runs on one bin at a time, so the picker is held while it is on.

70	                BinSeg(vm, hub, PickFor.SET, vm.selSet, allowAll = false, locked = hub.calibBin != null) {

71	                    vm.selSet = it

72	                    vm.pinShown = false

73	                }

74	                // The Collect PIN sits at the top for both roles, as on the dashboard.

75	                PinCard(vm, b)

76	                if (vm.operator) CalBar(vm, b, on, hub) else ReadOnlyNote()

77	                val v: Cal = if (on) vm.edit!! else b.cal

78	                val keys = SECTION_KEYS + if (vm.operator) listOf("touch") else emptyList()

79	                val allOpen = keys.all { it in vm.open }

80	                Btn(

81	                    if (allOpen) "HIDE ALL" else "SHOW ALL", { vm.open = if (allOpen) emptySet() else keys.toSet() },

82	                    Modifier.fillMaxWidth(), Ic.layers, Look.TINT, small = true, sound = if (allOpen) Sfx.BACK else Sfx.OPEN,

83	                )

84	                Sections(vm, b, v, on)

85	            }

86	            if (b != null) {

87	                Row(Modifier.padding(top = 4.dp)) { Sh(Ic.reload, "Bin tools") }

88	                LRow(Ic.reload, "Reload from bin", "Read the values ${b.name} is running on", onClick = { vm.reload(b) }, chevron = false, sound = Sfx.TAP)

89	            }

90	            // The app's own settings live behind the gear key in the header.

91	            LRow(Ic.gear, "App settings", "Sound, vibration, battery, links and demo options. Also the gear at the top right.", onClick = { vm.go(Screen.APP) })

92	        }

93	        if (showSave) {

94	            SaveBar(

95	                Modifier.align(Alignment.BottomCenter).padding(horizontal = 14.dp, vertical = 8.dp),

96	                onCancel = { vm.cancel(b) }, onSave = { vm.save(b) },

97	            )

98	        }

99	    }

100	}

101

102	@Composable

103	private fun SaveBar(modifier: Modifier, onCancel: () -> Unit, onSave: () -> Unit) {

104	    val c = Sb.c

105	    Row(

106	        modifier.widthIn(max = 520.dp).fillMaxWidth().bump(c, 16.dp, Out.O2, ring = c.accentSoft).padding(10.dp),

107	        verticalAlignment = Alignment.CenterVertically,

108	        horizontalArrangement = Arrangement.spacedBy(8.dp),

109	    ) {

110	        Row(Modifier.weight(1f), verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(6.dp)) {

111	            Ico(Ic.save, 18.dp, c.accent)

112	            T("Unsaved changes", size = 0.8f, color = c.accent, maxLines = 1)

113	        }

114	        Btn("CANCEL", onCancel, small = true, sound = Sfx.BACK)

115	        Btn("SAVE", onSave, icon = Ic.save, look = Look.ACCENT, small = true, sound = Sfx.CONFIRM)

116	    }

117	}

118

119	@Composable

120	private fun CalBar(vm: AppViewModel, b: Bin, on: Boolean, hub: HubState) {

121	    val c = Sb.c

122	    val cloud = b.source == Source.CLOUD

123	    val elsewhere = hub.calibBin != null && hub.calibBin != b.id

124	    Column(

125	        Modifier.fillMaxWidth().then(if (on) Modifier.ring(c.accentSoft, 16.dp) else Modifier).well(c, 16.dp).padding(14.dp),

126	        verticalArrangement = Arrangement.spacedBy(12.dp),

127	    ) {

128	        Row(horizontalArrangement = Arrangement.spacedBy(12.dp), verticalAlignment = Alignment.CenterVertically) {

129	            Box(Modifier.size(42.dp).bump(c, 21.dp), contentAlignment = Alignment.Center) {

130	                Ico(if (on) Ic.calib else if (cloud) Ic.cloud else Ic.lock(true), 21.dp, if (on) c.accent else c.ink2, Modifier.spin(on))

131	            }

132	            Column(Modifier.weight(1f)) {

133	                T(if (on) "Calibration mode is on for ${b.name}" else "Settings are locked", size = 0.98f)

134	                T(

135	                    when {

136	                        on -> "Change what you need, then SAVE. Switching off without saving leaves the bin as it was."

137	                        cloud -> "Calibration needs live readings, so it only runs on the bins’ Wi-Fi. The app is on cloud now. Every value can still be read."

138	                        elsewhere -> "Calibration is on for another bin. Switch it off there first."

139	                        else -> "Switch on calibration mode to change anything. The bin shows its teal calibration screen while it is on."

140	                    },

141	                    Modifier.padding(top = 2.dp), size = 0.8f, weight = FontWeight.Medium, color = c.ink2,

142	                )

143	            }

144	        }

145	        Row(

146	            Modifier.fillMaxWidth().bump(c, 14.dp).padding(horizontal = 12.dp, vertical = 10.dp),

147	            verticalAlignment = Alignment.CenterVertically,

148	            horizontalArrangement = Arrangement.spacedBy(12.dp),

149	        ) {

150	            T("Calibration mode", Modifier.weight(1f), size = 0.9f, weight = FontWeight.Bold, color = if (on) c.accent else c.ink)

151	            Toggle(on, { vm.setCalibration(it, b) }, enabled = on || (!cloud && !elsewhere && b.source != Source.STALE))

152	        }

153	    }

154	}

155

156	@Composable

157	private fun PinCard(vm: AppViewModel, b: Bin) {

158	    val c = Sb.c

159	    Column(

160	        Modifier.fillMaxWidth().well(c, 16.dp).padding(14.dp),

161	        verticalArrangement = Arrangement.spacedBy(10.dp),

162	    ) {

163	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(12.dp)) {

164	            Box(Modifier.size(44.dp).bump(c, 22.dp), contentAlignment = Alignment.Center) { Ico(Ic.pin, 21.dp, c.cal) }

165	            Column(Modifier.weight(1f)) {

166	                T("Collect PIN for ${b.name}", size = 0.98f)

167	                T(

168	                    if (vm.operator) "Collectors type this on the bin for COLLECT and WI-FI SETUP. Change it under Lid and lock."

169	                    else "Type this on the bin touchscreen when you tap COLLECT.",

170	                    size = 0.8f, weight = FontWeight.Medium, color = c.ink2,

171	                )

172	            }

173	        }

174	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(12.dp)) {

175	            T(

176	                if (vm.pinShown) "%04d".format(b.cal.v("collect_pin").toInt()) else "••••", Modifier.weight(1f).padding(start = 4.dp),

177	                size = 2f, color = c.cal, tnum = true, spacing = if (vm.pinShown) 0.21f else 0.14f, lineHeight = 1.1f,

178	            )

179	            Btn(if (vm.pinShown) "HIDE" else "SHOW", { vm.pinShown = !vm.pinShown }, icon = if (vm.pinShown) Ic.eyeoff else Ic.eye, look = Look.TINT, cc = c.cal, small = true, sound = Sfx.CHOICE)

180	        }

181	    }

182	}

183

184	@Composable

185	private fun ReadOnlyNote() {

186	    val c = Sb.c

187	    Row(

188	        Modifier.fillMaxWidth().well(c, 16.dp).padding(14.dp),

189	        verticalAlignment = Alignment.CenterVertically,

190	        horizontalArrangement = Arrangement.spacedBy(12.dp),

191	    ) {

192	        Box(Modifier.size(42.dp).bump(c, 21.dp), contentAlignment = Alignment.Center) { Ico(Ic.eye, 21.dp, c.ink2) }

193	        Column(Modifier.weight(1f)) {

194	            T("These settings are read only", size = 0.98f)

195	            T("You can see every value, but only the operator can change them or run a calibration.", size = 0.8f, weight = FontWeight.Medium, color = c.ink2)

196	        }

197	    }

198	}

199

200	/** The fold-out groups, in order. Touchscreen is added for operators. */

201	private val SECTION_KEYS = listOf("range", "limits", "air", "lid", "weight")

202

203	/** .sect: a fold-out group of settings with its summary on the right. */

204	@Composable

205	private fun Sect(vm: AppViewModel, key: String, title: String, summary: String, cc: Color, on: Boolean, body: @Composable () -> Unit) {

206	    val c = Sb.c

207	    val open = key in vm.open

208	    val turn by animateFloatAsState(if (open) 180f else 0f, label = "chev")

209	    Column(Modifier.fillMaxWidth().bump(c, 16.dp)) {

210	        Row(

211	            Modifier.fillMaxWidth().defaultMinSize(minHeight = 54.dp).press(true, if (open) Sfx.BACK else Sfx.OPEN) { vm.toggleSection(key) }.padding(horizontal = 14.dp),

212	            verticalAlignment = Alignment.CenterVertically,

213	            horizontalArrangement = Arrangement.spacedBy(10.dp),

214	        ) {

215	            Box(Modifier.size(4.dp, 15.dp).clip(RoundedCornerShape(2.dp)).background(cc))

216	            T(title.uppercase(), size = 0.76f, spacing = 0.09f, color = cc, maxLines = 1)

217	            T(summary, Modifier.weight(1f), size = 0.76f, weight = FontWeight.Bold, color = c.ink2, maxLines = 1, align = TextAlign.End)

218	            Ico(Ic.chev, 18.dp, c.ink2, Modifier.graphicsLayer { rotationZ = turn })

219	        }

220	        AnimatedVisibility(open) {

221	            Column(

222	                Modifier.fillMaxWidth().padding(start = 14.dp, end = 14.dp, top = 2.dp, bottom = 16.dp).alpha(if (on) 1f else 0.5f),

223	                verticalArrangement = Arrangement.spacedBy(14.dp),

224	            ) { body() }

225	        }

226	    }

227	}

228

229	@Composable

230	private fun Fld(vm: AppViewModel, key: String, value: Double, on: Boolean, cc: Color, extra: (@Composable () -> Unit)? = null) {

231	    val f = FIELDS.getValue(key)

232	    Field("${f.label} (${f.unit})", f.icon(), cc) {

233	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(8.dp)) {

234	            Stepper(fmt(value, f.dec), f.unit, { vm.step(key, it) }, Modifier.weight(1f), enabled = on, cc = cc)

235	            extra?.invoke()

236	        }

237	    }

238	}

239

240	// The folding groups: fill range, alert limits, air sensor, lid and weight.

241	// USE NOW copies the bin's live reading into the field (for d_empty, d_full).

242	@Composable

243	private fun Sections(vm: AppViewModel, b: Bin, v: Cal, on: Boolean) {

244	    val c = Sb.c

245	    val r = b.reading

246	    Sect(vm, "range", "Fill range", "${fmt(v.v("d_empty"), 1)} to ${fmt(v.v("d_full"), 1)} cm", c.cal, on) {

247	        Fld(vm, "d_empty", v.v("d_empty"), on, c.cal) { Btn("USE NOW", { vm.useNow("d_empty", b) }, look = Look.TINT, cc = c.cal, small = true, enabled = on, sound = Sfx.CONFIRM) }

248	        Fld(vm, "d_full", v.v("d_full"), on, c.cal) { Btn("USE NOW", { vm.useNow("d_full", b) }, look = Look.TINT, cc = c.cal, small = true, enabled = on, sound = Sfx.CONFIRM) }

249	        Hint("Empty the bin and press USE NOW for the first one. Hold a board at your full line and press it for the second. Sensor reads **${fmt(r.distance, 1)} cm** right now.")

250	    }

251	    Sect(

252	        vm, "limits", "Alert limits",

253	        "${fmt(v.v("fill_th"), 0)}% · ${fmt(v.v("temp_th"), 0)}°C · ${fmt(v.v("hum_th"), 0)}%RH · ${fmt(v.v("gas_th"), 0)} IAQ", c.bad, on,

254	    ) {

255	        listOf("fill_th", "temp_th", "hum_th", "gas_th", "tilt_th", "weight_cap").forEach { Fld(vm, it, v.v(it), on, c.bad) }

256	        Hint("A reading at or above its threshold turns its tile red, raises an alert and sounds the bin buzzer. Humidity uses 85 %RH by default, the same limit as the bin touchscreen.")

257	    }

258	    Sect(vm, "air", "Air sensor (BME680)", "offset ${fmt(v.v("temp_off"), 1)}°C", c.gas, on) {

259	        Field("Temperature raw / corrected", Ic.temp(r.temp), c.gas) { Readout("${fmt(r.tempRaw, 1)} / ${fmt(r.tempRaw + v.v("temp_off"), 1)} °C") }

260	        Fld(vm, "tref", vm.tref, on, c.gas) { Btn("SET OFFSET", { vm.setOffset(b) }, look = Look.TINT, cc = c.gas, small = true, enabled = on, sound = Sfx.CONFIRM) }

261	        Fld(vm, "temp_off", v.v("temp_off"), on, c.gas)

262	        val acc = listOf("Stabilizing", "Low accuracy", "Medium accuracy", "Calibrated")[r.iaqAcc.coerceIn(0, 3)]

263	        Field("Gas sensor state", Ic.gas, c.gas) {

264	            Readout(

265	                when {

266	                    r.fault("BME680") -> "Not answering"

267	                    r.warming -> "Warming up" + (r.iaqWarmS?.let { " (${it / 60}:%02d)".format(it % 60) } ?: "") + (r.gasK?.let { ", gas ${Math.round(it)}kΩ" } ?: "")

268	                    else -> "$acc, IAQ ${Math.round(r.gas)}"

269	                },

270	            )

271	        }

272	        Fld(vm, "hum_off", v.v("hum_off"), on, c.gas)

273	        Fld(vm, "press_off", v.v("press_off"), on, c.gas)

274	        Btn(

275	            "RESET GAS BASELINE", { vm.binCommand(b, "RESET_GAS", emptyMap(), "Gas baseline cleared. It needs time in normal air to be accurate again.") },

276	            Modifier.fillMaxWidth(), Ic.reload, Look.TINT, c.gas, enabled = on, sound = Sfx.CONFIRM,

277	        )

278	        Hint("The BME680 warms itself, so it reads a little hot and its humidity a little low. Put a reference thermometer next to the bin for a few minutes, enter its reading, press SET OFFSET, then SAVE. The gas reading learns its own baseline in normal air, and RESET GAS BASELINE restarts that.")

279	    }

280	    Sect(vm, "lid", "Lid and lock", "PIN •••• · ${fmt(v.v("pir_hold_s"), 0)} s", c.sim, on) {

281	        Fld(vm, "servo_closed", v.v("servo_closed"), on, c.sim)

282	        Fld(vm, "servo_open", v.v("servo_open"), on, c.sim)

283	        Field("Collect PIN", Ic.pin, c.sim) {

284	            Inp(

285	                if (vm.operator) v.v("collect_pin").toInt().takeIf { it > 0 }?.toString().orEmpty() else "••••", vm::setPin,

286	                keyboard = KeyboardType.NumberPassword, enabled = on,

287	            )

288	        }

289	        Fld(vm, "pir_hold_s", v.v("pir_hold_s"), on, c.sim)

290	        Fld(vm, "buzz_s", v.v("buzz_s"), on, c.sim)

291	        Row(

292	            Modifier.fillMaxWidth().bump(c, 14.dp).padding(horizontal = 12.dp, vertical = 10.dp),

293	            verticalAlignment = Alignment.CenterVertically,

294	            horizontalArrangement = Arrangement.spacedBy(12.dp),

295	        ) {

296	            T("Lock the lid when the bin is full", Modifier.weight(1f), size = 0.9f, weight = FontWeight.Bold)

297	            Toggle(v.v("auto_lock") > 0, { vm.toggleAutoLock() }, enabled = on)

298	        }

299	    }

300	    Sect(vm, "weight", "Weight", "%.2f kg now".format(r.weight / 1000), c.weight, on) {

301	        Field("Reading now", Ic.weight(r.weight / v.v("weight_cap")), c.weight) { Readout("%.2f kg".format(r.weight / 1000)) }

302	        Fld(vm, "known", vm.known, on, c.weight)

303	        Acts {

304	            Btn("TARE / ZERO", { vm.binCommand(b, "TARE", emptyMap(), "Scale zeroed on ${b.name}.") }, Modifier.weight(1f), Ic.undo, Look.TINT, c.weight, enabled = on, sound = Sfx.CONFIRM)

305	            Btn(

306	                "CALIBRATE", { vm.binCommand(b, "CAL_WEIGHT", mapOf("grams" to vm.known), "Calibrated with ${fmt(vm.known, 0)} g.") },

307	                Modifier.weight(1f), Ic.capture, Look.TINT, c.weight, enabled = on, sound = Sfx.CONFIRM,

308	            )

309	        }

310	        Hint("Tare with the bin empty. Then put the known weight in and press CALIBRATE.")

311	    }

312	    if (vm.operator) Sect(

313	        vm, "touch", "Touchscreen",

314	        when (b.touchCustom) { true -> "calibrated"; false -> "default values"; null -> "not reported" }, c.accent, true,

315	    ) {

316	        Acts {

317	            Btn("CALIBRATE TOUCH", { vm.touchCal(b, false) }, Modifier.weight(1f), Ic.capture, Look.TINT, c.accent, sound = Sfx.TAP)

318	            Btn("RESET", { vm.touchCal(b, true) }, Modifier.weight(1f), Ic.undo, Look.TINT, c.accent, sound = Sfx.TAP)

319	        }

320	        Hint("If taps on the bin land in the wrong place, press CALIBRATE TOUCH and press the centre of each of the 4 crosses the bin shows. RESET goes back to the firmware's own values. Needs the bin on the same Wi-Fi as the phone.")

321	    }

322	}

323

324	// ---------- connection ----------

325

326	@Composable

327	fun Connection(vm: AppViewModel, hub: HubState) {

328	    val c = Sb.c

329	    val link = hub.link

330	    val (stateText, stateCc) = when (link) {

331	        Link.LIVE -> "Local Wi-Fi" to c.link; Link.CLOUD -> "Firebase" to c.cloud; Link.THINGSPEAK -> "ThingSpeak" to c.ts

332	        Link.OFFLINE -> "Not connected" to c.bad; Link.DEMO -> "Demo data" to c.warn

333	    }

334	    val r = hub.routes

335	    val demo = hub.demo

336	    ScrollPage {

337	        BackBar("Connection", { vm.back() }) {

338	            StatePill(when (link) { Link.CLOUD -> Ic.cloud; Link.THINGSPEAK -> Ic.chart; else -> Ic.wifi(link == Link.LIVE) }, stateText, stateCc, true)

339	        }

340	        Card {

341	            Sh(Ic.layers, "How the app reaches the bins", c.link)

342	            Route(1, Ic.wifi(true), c.link, link == Link.LIVE, if (demo) r.local else r.local.takeIf { hub.bins.any { it.ip.isNotBlank() } }, "Local Wi-Fi", "Always tried first. Readings every second and every control.")

343	            Route(2, Ic.cloud, c.cloud, link == Link.CLOUD, if (demo) r.firebase else hub.fbUp, "Firebase", "When no bin answers on Wi-Fi. Readings every 5 s. LOCK, UNLOCK and COLLECT only.")

344	            Route(3, Ic.chart, c.ts, link == Link.THINGSPEAK, if (demo) r.thingspeak else hub.tsUp, "ThingSpeak", "When Wi-Fi and Firebase do not answer. Readings every 15 s. LOCK, UNLOCK and COLLECT through TalkBack.")

345	            if (demo && r.any) Hint("These are pretend links from the demo options, to show the colours.")

346	            Column(verticalArrangement = Arrangement.spacedBy(7.dp)) {

347	                hub.bins.take(4).forEach { b ->

348	                    val (cc, txt) = when (b.source) {

349	                        Source.DEMO -> c.warn to "demo data"

350	                        Source.STALE -> c.bad to "not answering"

351	                        Source.LIVE -> c.link to "answers on Wi-Fi"

352	                        Source.CLOUD -> c.cloud to "via Firebase"

353	                        Source.THINGSPEAK -> c.ts to "via ThingSpeak"

354	                    }

355	                    Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(9.dp)) {

356	                        Box(Modifier.size(10.dp).ring(cc.soft(0.22f), 5.dp, 3.dp).clip(RoundedCornerShape(5.dp)).background(cc))

357	                        Row(Modifier.weight(1f), verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(6.dp)) {

358	                            T(b.name, size = 0.84f, weight = FontWeight.Bold, maxLines = 1)

359	                            T(b.ip.ifBlank { "no address" }, size = 0.84f, weight = FontWeight.SemiBold, color = c.ink2, maxLines = 1)

360	                        }

361	                        T(txt, size = 0.72f, weight = FontWeight.Bold, color = cc, maxLines = 1)

362	                    }

363	                    if (!b.isDemo && (b.fw.isNotBlank() || b.rssi != null)) {

364	                        val up = b.uptimeS?.let { u -> if (u >= 86_400) "up ${u / 86_400} d" else if (u >= 3600) "up ${u / 3600} h" else "up ${u / 60} min" }

365	                        T(

366	                            listOfNotNull(b.fw.takeIf { it.isNotBlank() }?.let { "firmware $it" }, b.rssi?.let { "signal $it dBm" }, up).joinToString(" · "),

367	                            Modifier.padding(start = 19.dp), size = 0.72f, weight = FontWeight.SemiBold, color = c.ink2, maxLines = 1,

368	                        )

369	                    }

370	                }

371	                if (hub.bins.size > 4) T("and ${hub.bins.size - 4} more", size = 0.84f, weight = FontWeight.SemiBold, color = c.ink2)

372	            }

373	            Hint("The app calls every bin address with a 1.5 s timeout when it opens, when the phone changes network, and every 30 s while it is on cloud, and goes back to Wi-Fi by itself. Each bin also writes its own address to the cloud, so a new address is picked up without typing it.")

374	        }

375	        Card {

376	            Sh(Ic.cloud, "Firebase account", c.cloud)

377	            Field("Database URL", Ic.cloud) { Inp(vm.fb.url, { vm.fb = vm.fb.copy(url = it.trim()) }, placeholder = "https://...firebasedatabase.app", keyboard = KeyboardType.Uri) }

378	            Field("Web API key", Ic.key) { Inp(vm.fb.key, { vm.fb = vm.fb.copy(key = it.trim()) }, placeholder = "AIza...") }

379	            Field("Email", Ic.user) { Inp(vm.fb.email, { vm.fb = vm.fb.copy(email = it.trim()) }, placeholder = "name@example.com", keyboard = KeyboardType.Email) }

380	            Field("Password", Ic.key) {

381	                Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(8.dp)) {

382	                    Inp(vm.fb.password, { vm.fb = vm.fb.copy(password = it) }, Modifier.weight(1f), password = !vm.fbShow)

383	                    Btn(if (vm.fbShow) "HIDE" else "SHOW", { vm.fbShow = !vm.fbShow }, icon = if (vm.fbShow) Ic.eyeoff else Ic.eye, look = Look.TINT, small = true, sound = Sfx.CHOICE)

384	                }

385	            }

386	            vm.fbMsg?.let { (ok, msg) ->

387	                Row(

388	                    Modifier.fillMaxWidth().brutal(c, 12.dp, if (ok) c.ok else c.bad, off = 3.dp).padding(horizontal = 12.dp, vertical = 10.dp),

389	                    horizontalArrangement = Arrangement.spacedBy(8.dp),

390	                ) {

391	                    Ico(if (ok) Ic.check else Ic.tamper, 18.dp, c.onBanner)

392	                    T(msg, size = 0.82f, weight = FontWeight.Bold, color = c.onBanner)

393	                }

394	            }

395	            Acts {

396	                Btn(if (vm.fbBusy) "TESTING" else "TEST", { vm.fbTest(false) }, Modifier.weight(1f), Ic.radar, Look.TINT, c.cloud, enabled = !vm.fbBusy)

397	                Btn("TEST AND SAVE", { vm.fbTest(true) }, Modifier.weight(1f), Ic.save, Look.SOLID, c.cloud, enabled = !vm.fbBusy)

398	            }

399	            Btn("RESTORE WORKING", { vm.fbRestore() }, Modifier.fillMaxWidth(), Ic.undo, Look.TINT)

400	            if (vm.operator) {

401	                Btn("SEND TO BINS", { vm.sendCloudToBins() }, Modifier.fillMaxWidth(), Ic.bins, Look.TINT, c.cloud, sound = Sfx.TAP)

402	                Hint("SEND TO BINS gives every bin on Wi-Fi this account, so they write to the same database. Save it with TEST AND SAVE first.")

403	            }

404	            Hint("TEST signs in and reads the list of bins. TEST AND SAVE keeps the details only when the test passes. RESTORE WORKING puts back the last details that passed. The password is stored encrypted on this phone.")

405	        }

406	        Card {

407	            Sh(Ic.chart, "ThingSpeak", c.ts)

408	            Field("Server", Ic.chart, c.ts) { Inp(vm.tsUrl, { vm.tsUrl = it.trim() }, placeholder = "https://api.thingspeak.com", keyboard = KeyboardType.Uri) }

409	            vm.tsMsg?.let { (ok, msg) ->

410	                Row(

411	                    Modifier.fillMaxWidth().brutal(c, 12.dp, if (ok) c.ok else c.bad, off = 3.dp).padding(horizontal = 12.dp, vertical = 10.dp),

412	                    horizontalArrangement = Arrangement.spacedBy(8.dp),

413	                ) {

414	                    Ico(if (ok) Ic.check else Ic.tamper, 18.dp, c.onBanner)

415	                    T(msg, size = 0.82f, weight = FontWeight.Bold, color = c.onBanner)

416	                }

417	            }

418	            Btn(if (vm.tsBusy) "TESTING" else "TEST AND SAVE", { vm.tsTest() }, Modifier.fillMaxWidth(), Ic.radar, Look.SOLID, c.ts, enabled = !vm.tsBusy)

419	            // ThingSpeak has no call that reports the message count, so the app estimates it.

420	            val perDay = hub.tsPerDay

421	            val year = (perDay ?: 0.0) * 365

422	            val yearShare = (year / 3_000_000.0).coerceIn(0.0, 1.0)

423	            Box(

424	                Modifier.fillMaxWidth().height(14.dp).well(c, 7.dp).drawBehind {

425	                    if (yearShare > 0) drawRoundRect(

426	                        if (year > 3_000_000) c.bad else c.ts,

427	                        size = Size((size.width * yearShare).toFloat().coerceAtLeast(size.height), size.height), cornerRadius = CornerRadius(7.dp.toPx()),

428	                    )

429	                },

430	            )

431	            Hint(

432	                if (perDay == null) "No estimate yet. Once a channel has been read for two minutes, the app counts how fast its entries rise and works out the year. The free licence allows 3 million messages a year. The exact count is on your ThingSpeak account page."

433	                else "About **${"%,d".format(Math.round(perDay))}** messages a day, so about **${"%,d".format(Math.round(year))}** a year of the 3,000,000 the free licence allows" +

434	                    (if (year > 3_000_000) ". That is over the limit, so make the bins write less often." else ".") + " This is an estimate. The exact count is on your ThingSpeak account page.",

435	            )

436	            Hint(

437	                "Each bin needs its own channel. Put the channel ID and read API key in the bin's SET sheet, and its TalkBack ID and key for LOCK, UNLOCK and COLLECT. " +

438	                    "The write API key stays in the bin's firmware. Fields: **1** fill %, **2** weight g, **3** temperature, **4** humidity, **5** air quality, **6** pressure, **7** tilt, " +

439	                    "**8** motion + 2 × lid + 4 × status, and the status text holds the bin's IP address. The free licence allows a write every 15 s per channel and 3 million a year in all.",

440	            )

441	        }

442	        Card {

443	            Sh(Ic.download, "Data use this month", c.cloud)

444	            val bytes = hub.cloudBytes

445	            val limit = 10L * 1024 * 1024 * 1024

446	            val share = ((bytes ?: 0L).toDouble() / limit).coerceIn(0.0, 1.0)

447	            Box(

448	                Modifier.fillMaxWidth().height(14.dp).well(c, 7.dp).drawBehind {

449	                    if (share > 0) drawRoundRect(c.cloud, size = Size((size.width * share).toFloat().coerceAtLeast(size.height), size.height), cornerRadius = CornerRadius(7.dp.toPx()))

450	                },

451	            )

452	            Hint(

453	                if (bytes == null) "No figure yet. The bins report how much they sent once the app has used the cloud. The free Firebase plan allows 10 GB each month."

454	                else "**${bytes / (1024 * 1024)} MB** of the 10 GB the free Firebase plan allows each month. The bins write every 5 s while a phone watches over cloud and every minute otherwise, and at once when an alert starts."

455	            )

456	        }

457	    }

458	}

459

460	@Composable

461	private fun Route(n: Int, icon: ImageVector, cc: Color, on: Boolean, up: Boolean?, title: String, sub: String) {

462	    val c = Sb.c

463	    Row(

464	        Modifier.fillMaxWidth().then(if (on) Modifier.bump(c, 14.dp, ring = cc.soft(0.34f)) else Modifier.well(c, 14.dp)).padding(horizontal = 12.dp, vertical = 11.dp),

465	        verticalAlignment = Alignment.CenterVertically,

466	        horizontalArrangement = Arrangement.spacedBy(11.dp),

467	    ) {

468	        Box(Modifier.size(26.dp).bump(c, 13.dp), contentAlignment = Alignment.Center) { T("$n", size = 0.8f, color = cc) }

469	        Ico(icon, 21.dp, cc)

470	        Column(Modifier.weight(1f)) {

471	            T(title, size = 0.92f)

472	            T(sub, Modifier.padding(top = 2.dp), size = 0.77f, weight = FontWeight.SemiBold, color = c.ink2)

473	        }

474	        when {

475	            on -> Tag("IN USE", cc)

476	            up == true -> Tag("ANSWERS", cc.soft(0.85f))

477	            up == false -> T("NO ANSWER", size = 0.66f, spacing = 0.05f, color = c.bad, maxLines = 1)

478	            else -> T("NOT SET UP", size = 0.66f, spacing = 0.05f, color = c.ink2, maxLines = 1)

479	        }

480	    }

481	}

482

483	// ---------- Bluetooth setup ----------

484

485	@Composable

486	fun Bluetooth(vm: AppViewModel, hub: HubState) {

487	    val c = Sb.c

488	    val b = vm.binFor(vm.selSet, hub.bins)

489	    val paired = vm.bt in 1..3

490	    val name = vm.btName.ifBlank { b?.takeIf { it.isDemo }?.name ?: "the bin" }

491	    // Android asks for Nearby devices (or Location before Android 12) the first time.

492	    val ask = rememberLauncherForActivityResult(ActivityResultContracts.RequestMultiplePermissions()) { vm.btPair(b) }

493	    val pair = { if (b?.isDemo != true && !vm.btAllowed()) ask.launch(vm.btPermissions()) else vm.btPair(b) }

494	    ScrollPage {

495	        BackBar("Bluetooth setup", { vm.back() }) {

496	            StatePill(Ic.bluetooth, if (vm.bt >= 4) "Off" else if (paired) "Paired" else "Not paired", c.bt, paired)

497	        }

498	        Card {

499	            Hint("Hand the Wi-Fi name and password to a bin over Bluetooth, read back the address it gets, then switch its Bluetooth off so only Wi-Fi stays on.")

500	            Row(horizontalArrangement = Arrangement.spacedBy(8.dp)) {

501	                listOf(Ic.bluetooth to "PAIR + CODE", Ic.radar to "PICK NETWORK", Ic.key to "SEND", Ic.power to "BT OFF").forEachIndexed { k, (ic, label) ->

502	                    val done = vm.bt > k

503	                    val now = vm.bt == k

504	                    val cc = when { done -> c.ok; now -> c.bt; else -> c.ink2 }

505	                    Column(

506	                        Modifier.weight(1f).then(if (now) Modifier.bump(c, 13.dp, ring = c.bt.soft(0.3f)) else Modifier.well(c, 13.dp)).padding(vertical = 11.dp, horizontal = 4.dp),

507	                        horizontalAlignment = Alignment.CenterHorizontally,

508	                        verticalArrangement = Arrangement.spacedBy(6.dp),

509	                    ) {

510	                        Ico(if (done) Ic.check else ic, 19.dp, cc)

511	                        T(label, size = 0.64f, color = cc, align = TextAlign.Center, lineHeight = 1.2f)

512	                    }

513	                }

514	            }

515	            vm.btNote?.let { note ->

516	                Row(

517	                    Modifier.fillMaxWidth().well(c, 12.dp).padding(horizontal = 12.dp, vertical = 10.dp),

518	                    verticalAlignment = Alignment.CenterVertically,

519	                    horizontalArrangement = Arrangement.spacedBy(9.dp),

520	                ) {

521	                    Ico(if (vm.btBusy || vm.bt == 2) Ic.reload else Ic.bluetooth, 18.dp, c.bt, Modifier.spin(vm.btBusy || vm.bt == 2))

522	                    T(note, Modifier.weight(1f), size = 0.82f, weight = FontWeight.SemiBold)

523	                }

524	            }

525	            val canEdit = vm.bt == 1

526	            Field("Wi-Fi name (2.4 GHz)", Ic.wifi(false)) {

527	                Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(8.dp)) {

528	                    Inp(vm.ssid, { vm.ssid = it }, Modifier.weight(1f), placeholder = "HomeWiFi-2G", enabled = canEdit)

529	                    Btn("SCAN", { vm.btScanNets() }, icon = Ic.radar, look = Look.TINT, cc = c.bt, small = true, enabled = canEdit && !vm.btBusy, sound = Sfx.OPEN)

530	                }

531	            }

532	            Field("Password", Ic.key) {

533	                Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(8.dp)) {

534	                    Inp(vm.pass, { vm.pass = it }, Modifier.weight(1f), password = !vm.showPass, enabled = canEdit)

535	                    Btn(if (vm.showPass) "HIDE" else "SHOW", { vm.showPass = !vm.showPass }, icon = if (vm.showPass) Ic.eyeoff else Ic.eye, look = Look.TINT, small = true, sound = Sfx.CHOICE)

536	                }

537	            }

538	            if (vm.bt >= 3 && vm.btIp.isNotBlank()) {

539	                Column(Modifier.fillMaxWidth().well(c, 14.dp).padding(14.dp), verticalArrangement = Arrangement.spacedBy(12.dp)) {

540	                    Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(12.dp)) {

541	                        Ico(Ic.wifi(true), 21.dp, c.ok)

542	                        Column(Modifier.weight(1f)) {

543	                            T("$name joined ${vm.btJoined}", size = 0.86f, weight = FontWeight.Bold)

544	                            T(vm.btIp, size = 1.3f, color = c.ok, tnum = true)

545	                        }

546	                    }

547	                    Btn(vm.btUseLabel(), { vm.btUse() }, Modifier.fillMaxWidth(), Ic.check, Look.SOLID, c.ok, small = true, sound = Sfx.TAP)

548	                }

549	            }

550	            if (vm.btStale) {

551	                Btn("FORGET AND PAIR AGAIN", { vm.btForget() }, Modifier.fillMaxWidth(), Ic.undo, Look.TINT, c.warn, sound = Sfx.TAP)

552	            }

553	            Btn(

554	                if (vm.btBusy && vm.bt == 0) "PAIRING" else "PAIR A BIN", { pair() }, Modifier.fillMaxWidth(), Ic.bluetooth, Look.TINT, c.bt,

555	                enabled = (vm.bt == 0 || vm.bt >= 4) && !vm.btBusy, sound = Sfx.OPEN,

556	            )

557	            Btn(

558	                "SEND WI-FI", { vm.btSend() }, Modifier.fillMaxWidth(), if (vm.bt == 2) Ic.reload else Ic.key, Look.SOLID, c.bt,

559	                enabled = vm.bt == 1, iconMod = Modifier.spin(vm.bt == 2), sound = Sfx.CONFIRM,

560	            )

561	            Btn("TURN BT OFF", { vm.btOff() }, Modifier.fillMaxWidth(), Ic.power, Look.TINT, c.warn, enabled = vm.bt in 1..3, sound = Sfx.TAP)

562	            Hint(

563	                when (vm.bt) {

564	                    0 -> "On the bin, tap i, then WI-FI SETUP, and type the Collect PIN. Then press PAIR A BIN, pick the bin, and type the 6-digit code its screen shows."

565	                    1 -> "Pick the 2.4 GHz network the bin should join, then SEND WI-FI."

566	                    2 -> "The bin tries the network for up to 15 s. If it fails it goes back to its old Wi-Fi."

567	                    3 -> "Press ${vm.btUseLabel()} to keep this address, then TURN BT OFF."

568	                    else -> "Bluetooth is off on the bin. It stays on Wi-Fi until someone opens WI-FI SETUP on it again."

569	                }

570	            )

571	            if (b?.isDemo == true) Hint("${b.name} is a demo bin, so pairing uses a stand-in code here. Pick a real bin, or none, to set up a real one.")

572	            Hint("The code keeps strangers from pairing and encrypts the link, so the Wi-Fi password cannot be overheard. It is new each time, lasts 60 s, and 3 wrong codes stop pairing for 60 s.")

573	        }

574	    }

575	}

Android App/app/src/main/java/smartbin/ui/Pages.kt

1	// ============================================================================

2	// ui/Pages.kt - the Simulation and Alerts tabs

3	// Simulation needs the bin on local Wi-Fi: each MANUAL row sets one bit of the

4	// firmware's Sim mask and its value, and APPLY sends them in one POST to

5	// /api/simulate. Alerts lists Hub's alert history with filters by kind and

6	// time range.

7	// ============================================================================

8	package smartbin.ui

9

10	import androidx.compose.foundation.background

11	import androidx.compose.foundation.layout.Arrangement

12	import androidx.compose.foundation.layout.Box

13	import androidx.compose.foundation.layout.Column

14	import androidx.compose.foundation.layout.Row

15	import androidx.compose.foundation.layout.defaultMinSize

16	import androidx.compose.foundation.layout.fillMaxSize

17	import androidx.compose.foundation.layout.fillMaxWidth

18	import androidx.compose.foundation.layout.height

19	import androidx.compose.foundation.layout.padding

20	import androidx.compose.foundation.layout.size

21	import androidx.compose.foundation.layout.widthIn

22	import androidx.compose.foundation.shape.CircleShape

23	import androidx.compose.foundation.shape.RoundedCornerShape

24	import androidx.compose.material3.Slider

25	import androidx.compose.material3.SliderDefaults

26	import androidx.compose.runtime.Composable

27	import androidx.compose.ui.Alignment

28	import androidx.compose.ui.Modifier

29	import androidx.compose.ui.draw.alpha

30	import androidx.compose.ui.draw.clip

31	import androidx.compose.ui.draw.drawBehind

32	import androidx.compose.ui.geometry.Size

33	import androidx.compose.ui.graphics.Color

34	import androidx.compose.ui.graphics.vector.ImageVector

35	import androidx.compose.ui.text.font.FontWeight

36	import androidx.compose.ui.text.style.TextAlign

37	import androidx.compose.ui.unit.dp

38	import smartbin.data.AlertKind

39	import smartbin.data.AlertRow

40	import smartbin.data.Bin

41	import smartbin.data.DEFAULT_SIM

42	import smartbin.data.Hub

43	import smartbin.data.HubState

44	import smartbin.data.Severity

45	import smartbin.data.Source

46	import smartbin.notify.Sfx

47

48	// ---------- simulation ----------

49

50	private enum class SimKind { RANGE, NUM, PAIR, TOGGLE, LID }

51

52	private class SimRowDef(val key: String, val bit: Int, val label: String, val kind: SimKind, val min: Float = 0f, val max: Float = 100f, val step: Float = 1f, val unit: String = "")

53

54	private val SENSORS = listOf(

55	    SimRowDef("fill", 0, "Fill level", SimKind.RANGE, 0f, 100f, 1f, "%"),

56	    SimRowDef("weight", 1, "Weight", SimKind.NUM),

57	    SimRowDef("gas", 2, "Air quality", SimKind.RANGE, 0f, 500f, 5f, "IAQ"),

58	    SimRowDef("temp", 3, "Temperature and humidity", SimKind.PAIR),

59	    SimRowDef("tamper", 4, "Tamper", SimKind.TOGGLE),

60	    SimRowDef("motion", 5, "Motion", SimKind.TOGGLE),

61	    SimRowDef("lid", 6, "Lid", SimKind.LID),

62	)

63

64	@Composable

65	fun Simulation(vm: AppViewModel, hub: HubState) {

66	    val c = Sb.c

67	    val b = vm.binFor(vm.selSim, hub.bins)

68	    Box(Modifier.fillMaxSize()) {

69	        ScrollPage(bottomExtra = 80.dp) {

70	            Sh(Ic.flask, "Simulation", c.sim)

71	            if (b == null) {

72	                Card { Empty(Ic.bins, "No bins yet.") }

73	                return@ScrollPage

74	            }

75	            BinSeg(vm, hub, PickFor.SIM, vm.selSim, allowAll = false) { vm.selSim = it }

76	            Hint("AUTO reads the real hardware and lets the firmware drive the outputs. MANUAL holds the value you set, one row at a time.")

77	            if (b.source == Source.CLOUD || b.source == Source.THINGSPEAK) Row(

78	                Modifier.fillMaxWidth().well(c, 16.dp).padding(14.dp),

79	                verticalAlignment = Alignment.CenterVertically,

80	                horizontalArrangement = Arrangement.spacedBy(12.dp),

81	            ) {

82	                Box(Modifier.size(42.dp).bump(c, 21.dp), contentAlignment = Alignment.Center) { Ico(Ic.cloud, 21.dp, c.cloud) }

83	                Column(Modifier.weight(1f)) {

84	                    T("Simulation needs the bins’ Wi-Fi", size = 0.98f)

85	                    T("The app is on cloud now, so APPLY rests until a bin answers on Wi-Fi again.", size = 0.8f, weight = FontWeight.Medium, color = c.ink2)

86	                }

87	            }

88	            SENSORS.forEach { SimRow(vm, b, it) }

89	            Hint("The whole app turns violet while any sensor is manual.", Modifier.fillMaxWidth(), align = TextAlign.Center)

90	        }

91	        if (b != null) Row(

92	            Modifier.align(Alignment.BottomCenter).padding(horizontal = 14.dp, vertical = 8.dp).widthIn(max = 520.dp).fillMaxWidth()

93	                .bump(c, 16.dp, Out.O2).padding(10.dp),

94	            horizontalArrangement = Arrangement.spacedBy(8.dp),

95	        ) {

96	            Btn("APPLY", { vm.applySim(b) }, Modifier.weight(1f), Ic.flask, Look.ACCENT, enabled = b.source != Source.CLOUD && b.source != Source.THINGSPEAK, sound = Sfx.CONFIRM)

97	            Btn("ALL TO AUTO", { vm.simAllAuto(b) }, Modifier.weight(1f), Ic.reload, Look.TINT, c.ok, enabled = b.sim.mask > 0, sound = Sfx.SWITCH_OFF)

98	        }

99	    }

100	}

101

102	@Composable

103	private fun SimRow(vm: AppViewModel, b: Bin, s: SimRowDef) {

104	    val c = Sb.c

105	    val cc = when (s.key) {

106	        "fill" -> c.fill; "weight" -> c.weight; "gas" -> c.gas; "temp" -> c.temp; "tamper" -> c.tamper; "motion" -> c.motion; else -> c.lid

107	    }

108	    val values = b.sim.values

109	    fun value(k: String) = values[k] ?: DEFAULT_SIM[k] ?: 0.0

110	    val manual = b.sim.on(s.bit)

111	    val icon: ImageVector = when (s.key) {

112	        "fill" -> Ic.fill; "weight" -> Ic.weight(value("weight") / 4500); "gas" -> Ic.gas; "temp" -> Ic.temp(value("temp"))

113	        "tamper" -> Ic.tamper; "motion" -> Ic.motion(value("motion") > 0); else -> Ic.lid(value("lid") > 0)

114	    }

115	    Column(

116	        Modifier

117	            .fillMaxWidth()

118	            .bump(c, 16.dp)

119	            .drawBehind { if (manual) drawRect(cc, size = Size(4.dp.toPx(), size.height)) }

120	            .padding(horizontal = 13.dp, vertical = 12.dp),

121	        verticalArrangement = Arrangement.spacedBy(10.dp),

122	    ) {

123	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(10.dp)) {

124	            Ico(icon, 21.dp, cc)

125	            T(s.label, Modifier.weight(1f), weight = FontWeight.Bold)

126	            Btn(

127	                if (manual) "MANUAL" else "AUTO", { vm.simToggle(b, s.bit) }, Modifier.widthIn(min = 104.dp),

128	                if (manual) Ic.flask else Ic.check, if (manual) Look.SOLID else Look.TINT, if (manual) c.sim else c.ok,

129	                small = true, sound = if (manual) Sfx.SWITCH_OFF else Sfx.SWITCH_ON,

130	            )

131	        }

132	        val on = manual

133	        Box(Modifier.alpha(if (on) 1f else 0.45f)) {

134	            when (s.kind) {

135	                SimKind.RANGE -> Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(10.dp)) {

136	                    Slider(

137	                        value = value(s.key).toFloat().coerceIn(s.min, s.max),

138	                        onValueChange = { vm.simValue(b, s.key, (Math.round(it / s.step) * s.step).toDouble()) },

139	                        valueRange = s.min..s.max,

140	                        enabled = on,

141	                        modifier = Modifier.weight(1f).height(28.dp),

142	                        colors = SliderDefaults.colors(

143	                            thumbColor = c.sim, activeTrackColor = c.sim, inactiveTrackColor = c.well,

144	                            disabledThumbColor = c.ink2, disabledActiveTrackColor = c.ink2, disabledInactiveTrackColor = c.well,

145	                        ),

146	                    )

147	                    Row(Modifier.defaultMinSize(minWidth = 66.dp).well(c, 11.dp).padding(horizontal = 10.dp, vertical = 8.dp), verticalAlignment = Alignment.Bottom) {

148	                        T(Math.round(value(s.key)).toString(), tnum = true)

149	                        T(" ${s.unit}", size = 0.7f, color = c.ink2)

150	                    }

151	                }

152	                SimKind.NUM -> SimStepper(vm, b, "weight", value("weight"), on)

153	                SimKind.PAIR -> Column(verticalArrangement = Arrangement.spacedBy(10.dp)) {

154	                    SimStepper(vm, b, "temp", value("temp"), on)

155	                    SimStepper(vm, b, "humidity", value("humidity"), on)

156	                }

157	                SimKind.TOGGLE -> TwoWay(full = true) {

158	                    Tw(value(s.key) <= 0, { vm.simValue(b, s.key, 0.0) }, "OFF", Ic.cross, cc, enabled = on)

159	                    Tw(value(s.key) > 0, { vm.simValue(b, s.key, 1.0) }, "TRIGGERED", if (s.key == "motion") Ic.motion(true) else Ic.tamper, cc, enabled = on)

160	                }

161	                SimKind.LID -> TwoWay(full = true) {

162	                    Tw(value("lid") <= 0, { vm.simValue(b, "lid", 0.0) }, "CLOSED", Ic.lock(true), c.lid, enabled = on)

163	                    Tw(value("lid") > 0, { vm.simValue(b, "lid", 1.0) }, "OPEN", Ic.lid(true), c.lid, enabled = on)

164	                }

165	            }

166	        }

167	    }

168	}

169

170	@Composable

171	private fun SimStepper(vm: AppViewModel, b: Bin, key: String, v: Double, on: Boolean) {

172	    val f = SIM_STEP.getValue(key)

173	    Stepper(fmt(v, if (key == "temp") 1 else 0), f.unit, { vm.simStep(b, key, it) }, enabled = on, cc = Sb.c.sim)

174	}

175

176	// ---------- alerts ----------

177

178	private val KIND_ICONS: List<Pair<AlertKind, ImageVector>>

179	    get() = listOf(

180	        AlertKind.ALL to Ic.layers, AlertKind.FULL to Ic.fill, AlertKind.SENSOR to Ic.temp(30.0), AlertKind.TAMPER to Ic.tamper,

181	        AlertKind.LOCK to Ic.lock(true), AlertKind.PICKUP to Ic.truck, AlertKind.SETUP to Ic.calib,

182	    )

183

184	@Composable

185	fun AlertsPage(vm: AppViewModel, hub: HubState) {

186	    val c = Sb.c

187	    val every = hub.alerts

188	    val unread = hub.unread

189	    val filtered = vm.shown(every)

190	    val unShown = filtered.count { !it.read }

191	    val list = filtered.take(vm.alertShown)

192	    ScrollPage {

193	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(10.dp)) {

194	            Sh(Ic.clock, "Alert history", c.tamper, Modifier.weight(1f))

195	            if (unread > 0) Box(Modifier.clip(RoundedCornerShape(5.dp)).background(c.tamper).padding(horizontal = 8.dp, vertical = 4.dp)) {

196	                T("$unread UNREAD", size = 0.68f, spacing = 0.05f, color = Color.White)

197	            }

198	        }

199	        TwoWay(full = true) {

200	            KIND_ICONS.forEach { (k, ic) ->

201	                Tw(vm.akind == k, { vm.akind = k; vm.alertShown = 6 }, icon = ic, cc = if (k == AlertKind.ALL) c.ink else c.tamper, minHeight = 40.dp)

202	            }

203	        }

204	        TwoWay(full = true) {

205	            AlertRange.entries.forEach { r -> Tw(vm.arange == r, { vm.arange = r; vm.alertShown = 6 }, r.label, cc = c.tamper, minHeight = 36.dp) }

206	        }

207	        Column {

208	            T(if (filtered.isNotEmpty()) "Showing ${vm.scope(filtered.size)}" else "No alerts match.", size = 0.82f)

209	            T(

210	                if (filtered.isNotEmpty()) vm.akind.label + if (unShown > 0) ", $unShown unread" else ", all read" else "Change the kind or the dates.",

211	                Modifier.padding(top = 2.dp), size = 0.82f, weight = FontWeight.SemiBold, color = c.ink2,

212	            )

213	        }

214	        Acts {

215	            Btn("MARK READ", { vm.markShown(every) }, Modifier.weight(1f), Ic.check, Look.TINT, c.ok, small = true, enabled = unShown > 0, sound = Sfx.CONFIRM)

216	            Btn("DELETE", { vm.overlay = Overlay.ClearAlerts }, Modifier.weight(1f), Ic.trash, Look.TINT, c.tamper, small = true, enabled = filtered.isNotEmpty(), sound = Sfx.OPEN)

217	        }

218	        if (list.isEmpty()) {

219	            Card { Empty(Ic.bell, if (every.isNotEmpty()) "No alerts match. Change the kind or the dates." else "No alerts yet. This fills up as bins reach their limits.") }

220	        } else {

221	            Column(verticalArrangement = Arrangement.spacedBy(11.dp)) { list.forEach { AlertLine(it) } }

222	        }

223	        if (filtered.size > list.size) Btn("Show older (${filtered.size - list.size} more)", { vm.alertShown += 6 }, Modifier.fillMaxWidth(), Ic.clock)

224	        if (list.isNotEmpty()) Hint("MARK READ and DELETE act on exactly the alerts shown. Tap the dot to mark one read or unread. The cross removes one alert.", Modifier.fillMaxWidth(), align = TextAlign.Center)

225	    }

226	}

227

228	@Composable

229	private fun AlertLine(a: AlertRow) {

230	    val c = Sb.c

231	    val cc = sevColor(a.sev, c)

232	    val critical = a.sev == Severity.CRITICAL

233	    val surface = if (critical) Modifier.brutal(c, 8.dp, c.bad)

234	    else Modifier.bump(c, 14.dp).drawBehind { drawRect(cc, size = Size(5.dp.toPx(), size.height)) }

235	    val fg = if (critical) Color.White else c.ink

236	    val sub = if (critical) Color.White.soft(0.88f) else c.ink2

237	    Row(

238	        Modifier.fillMaxWidth().alpha(if (a.read) 0.66f else 1f).then(surface).padding(start = 8.dp, end = 10.dp, top = 11.dp, bottom = 11.dp),

239	        verticalAlignment = Alignment.CenterVertically,

240	        horizontalArrangement = Arrangement.spacedBy(10.dp),

241	    ) {

242	        // The dot: full while unread, a ring once read. A tap flips it.

243	        Box(

244	            Modifier.size(30.dp, 44.dp).press(true, Sfx.CHOICE) { Hub.markRead(setOf(a.id), !a.read) },

245	            contentAlignment = Alignment.Center,

246	        ) {

247	            val dot = if (critical) Color.White else cc

248	            if (a.read) Box(Modifier.size(11.dp).alpha(0.5f).ring(dot, 5.5.dp, 2.dp).padding(2.dp))

249	            else Box(Modifier.size(11.dp).ring(dot.soft(0.24f), 5.5.dp, 3.dp).clip(CircleShape).background(dot))

250	        }

251	        Box(

252	            Modifier.size(38.dp).then(if (critical) Modifier.clip(RoundedCornerShape(11.dp)).background(Color.White.soft(0.17f)) else Modifier.well(c, 11.dp)),

253	            contentAlignment = Alignment.Center,

254	        ) { Ico(Ic.named(a.icon), 20.dp, if (critical) Color.White else cc) }

255	        Column(Modifier.weight(1f)) {

256	            T(a.title, size = 0.95f, weight = if (a.read) FontWeight.Bold else FontWeight.ExtraBold, color = fg)

257	            T("${a.meta} · ${ago(a.at)}", Modifier.padding(top = 2.dp), size = 0.78f, weight = FontWeight.Medium, color = sub)

258	        }

259	        Box(

260	            Modifier.size(34.dp).then(if (critical) Modifier.clip(RoundedCornerShape(10.dp)).background(Color.White.soft(0.18f)) else Modifier.bump(c, 10.dp))

261	                .press(true, Sfx.DANGER) { Hub.deleteAlerts(setOf(a.id)) },

262	            contentAlignment = Alignment.Center,

263	        ) { Ico(Ic.cross, 15.dp, if (critical) Color.White else c.ink2) }

264	    }

265	}

Android App/app/src/main/java/smartbin/ui/Overlays.kt

1	// ============================================================================

2	// ui/Overlays.kt - sheets and pop-ups drawn over the current page

3	// vm.overlay picks one: bin edit, bin detail, add bins, clear alerts, bin

4	// picker, Wi-Fi help, operator PIN pads, Bluetooth device list and pairing

5	// code, the Attention list and the push-alert prompt. Only one shows at a time; a

6	// tap outside or Back closes it (vm.overlay = null).

7	// ============================================================================

8	package smartbin.ui

9

10	import smartbin.notify.Feel

11	import androidx.compose.foundation.gestures.detectHorizontalDragGestures

12	import androidx.compose.ui.input.pointer.pointerInput

13	import androidx.compose.foundation.layout.Arrangement

14	import androidx.compose.foundation.layout.Box

15	import androidx.compose.foundation.layout.Column

16	import androidx.compose.foundation.layout.Row

17	import androidx.compose.foundation.layout.fillMaxWidth

18	import androidx.compose.foundation.layout.height

19	import androidx.compose.foundation.layout.padding

20	import androidx.compose.foundation.layout.size

21	import androidx.compose.foundation.shape.RoundedCornerShape

22	import androidx.compose.foundation.background

23	import androidx.compose.runtime.Composable

24	import androidx.compose.runtime.LaunchedEffect

25	import androidx.compose.runtime.getValue

26	import androidx.compose.runtime.mutableStateOf

27	import androidx.compose.runtime.remember

28	import androidx.compose.runtime.setValue

29	import androidx.compose.ui.Alignment

30	import androidx.compose.ui.Modifier

31	import androidx.compose.ui.draw.clip

32	import androidx.compose.ui.graphics.graphicsLayer

33	import androidx.compose.ui.text.font.FontWeight

34	import androidx.compose.ui.text.input.KeyboardType

35	import androidx.compose.ui.unit.dp

36	import smartbin.data.HubState

37	import smartbin.data.OperatorLock

38	import smartbin.notify.Sfx

39

40	/** Every sheet, drawn over the page. Only one is open at a time. */

41	@Composable

42	fun Overlays(vm: AppViewModel, hub: HubState, allowAlerts: () -> Unit) {

43	    val o = vm.overlay ?: return

44	    val close = { vm.overlay = null }

45	    when (o) {

46	        Overlay.Attention -> Sheet(close) { Attention(vm, hub) }

47	        is Overlay.BinEdit -> {

48	            val b = hub.bins.firstOrNull { it.id == o.id } ?: return Gone(vm)

49	            Sheet(close) {

50	                SheetTitle(Ic.gear, "${b.name} settings")

51	                Field("Name") { Inp(vm.formName, { vm.formName = it }) }

52	                Field("Location") { Inp(vm.formPlace, { vm.formPlace = it }) }

53	                Field("IP address") { Inp(vm.formIp, { vm.formIp = it.trim() }, placeholder = "192.168.1.101", keyboard = KeyboardType.Uri) }

54	                Field("Cloud name (Firebase)") { Inp(vm.formCloud, { vm.formCloud = it.trim() }, placeholder = "bin01") }

55	                Field("ThingSpeak channel ID", Ic.chart, Sb.c.ts) { Inp(vm.formTsCh, { vm.formTsCh = it.trim() }, placeholder = "2345678", keyboard = KeyboardType.Number) }

56	                Field("ThingSpeak read API key", Ic.key, Sb.c.ts) { Inp(vm.formTsKey, { vm.formTsKey = it.trim() }, placeholder = "Only for a private channel") }

57	                Field("TalkBack ID and API key", Ic.key, Sb.c.ts) {

58	                    Row(horizontalArrangement = Arrangement.spacedBy(8.dp)) {

59	                        Inp(vm.formTbId, { vm.formTbId = it.trim() }, Modifier.weight(0.42f), placeholder = "ID", keyboard = KeyboardType.Number)

60	                        Inp(vm.formTbKey, { vm.formTbKey = it.trim() }, Modifier.weight(0.58f), placeholder = "API key")

61	                    }

62	                }

63	                Hint("With an IP address the app reads this bin live over Wi-Fi. With none of the three it shows demo data. Bluetooth setup can fill the address in. The cloud name is the bin's node in Firebase, tried when Wi-Fi does not answer. The ThingSpeak channel is tried after that, and TalkBack carries LOCK, UNLOCK and COLLECT to the bin.")

64	                Acts {

65	                    Btn("Remove", { vm.removeBin(b.id) }, Modifier.weight(1f), Ic.trash, Look.DANGER, sound = Sfx.DANGER)

66	                    Btn("Save", { vm.saveBinEdit(b.id) }, Modifier.weight(1f), Ic.save, Look.ACCENT, sound = Sfx.SAVED)

67	                }

68	            }

69	        }

70	        is Overlay.BinDetail -> {

71	            val bins = hub.bins

72	            val i = bins.indexOfFirst { it.id == o.id }

73	            if (i < 0) return Gone(vm)

74	            val b = bins[i]

75	            Sheet(close) {

76	                Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(8.dp)) {

77	                    IconKey(Ic.back, { vm.overlay = Overlay.BinDetail(bins[i - 1].id) }, tint = muted(), enabled = i > 0, sound = Sfx.STEP_DOWN)

78	                    Column(Modifier.weight(1f), horizontalAlignment = Alignment.CenterHorizontally) {

79	                        T(b.name, size = 1.02f, maxLines = 1)

80	                        T("${i + 1} of ${bins.size} · swipe or use the arrows", size = 0.74f, weight = FontWeight.Bold, color = Sb.c.ink2, maxLines = 1)

81	                    }

82	                    IconKey(

83	                        Ic.back, { vm.overlay = Overlay.BinDetail(bins[i + 1].id) }, tint = muted(), enabled = i < bins.size - 1, sound = Sfx.STEP_UP,

84	                        iconMod = Modifier.rotate180(),

85	                    )

86	                    IconKey(Ic.cross, close, tint = muted(), sound = Sfx.BACK)

87	                }

88	                // A swipe across the card moves to the next or previous bin, like the arrows.

89	                var drag by remember { mutableStateOf(0f) }

90	                Box(

91	                    Modifier.pointerInput(i, bins.size) {

92	                        detectHorizontalDragGestures(

93	                            onDragStart = { drag = 0f },

94	                            onDragEnd = {

95	                                val far = 80.dp.toPx()

96	                                when {

97	                                    drag < -far && i < bins.size - 1 -> { Feel.event(Sfx.STEP_UP); vm.overlay = Overlay.BinDetail(bins[i + 1].id) }

98	                                    drag > far && i > 0 -> { Feel.event(Sfx.STEP_DOWN); vm.overlay = Overlay.BinDetail(bins[i - 1].id) }

99	                                }

100	                                drag = 0f

101	                            },

102	                        ) { _, dx -> drag += dx }

103	                    },

104	                ) { BinCard(vm, b, hub, inSheet = true) }

105	            }

106	        }

107	        is Overlay.Pick -> Sheet(close) { Picker(vm, hub, o.forWhat) }

108	        Overlay.AddBins -> Sheet(close) {

109	            SheetTitle(Ic.plus, "Add bins")

110	            SheetText("How many bins are you adding? Each one gets a card with demo data until you give it an address.")

111	            val c = Sb.c

112	            Row(

113	                Modifier.align(Alignment.CenterHorizontally).well(c, 999.dp).padding(5.dp),

114	                verticalAlignment = Alignment.CenterVertically,

115	                horizontalArrangement = Arrangement.spacedBy(8.dp),

116	            ) {

117	                RoundKey("−", vm.addN > 1, Sfx.STEP_DOWN) { vm.addN-- }

118	                T("${vm.addN}", Modifier.size(40.dp, 30.dp), size = 1.4f, tnum = true, align = androidx.compose.ui.text.style.TextAlign.Center)

119	                RoundKey("+", vm.addN < 3, Sfx.STEP_UP) { vm.addN++ }

120	            }

121	            Acts {

122	                Btn("Cancel", close, Modifier.weight(1f), sound = Sfx.BACK)

123	                Btn("Add ${vm.addN} bin${if (vm.addN > 1) "s" else ""}", { vm.addBins() }, Modifier.weight(1f), Ic.plus, Look.ACCENT, sound = Sfx.CONFIRM)

124	            }

125	        }

126	        Overlay.ClearAlerts -> Sheet(close) {

127	            val n = vm.shown(hub.alerts).size

128	            SheetTitle(Ic.trash, "Delete alerts")

129	            SheetText("Delete ${vm.scope(n)}? This cannot be undone.")

130	            Acts {

131	                Btn("Cancel", close, Modifier.weight(1f), sound = Sfx.BACK)

132	                Btn("Delete $n", { vm.deleteShown(hub.alerts) }, Modifier.weight(1f), Ic.trash, Look.DANGER, sound = Sfx.DANGER)

133	            }

134	        }

135	        Overlay.Wifi -> Sheet(close) {

136	            SheetTitle(Ic.radar, "Networks the bin can see")

137	            Hint("Only 2.4 GHz networks are listed. The bin cannot join 5 GHz.")

138	            val nets = vm.btNets.ifEmpty { if (vm.btDemo) AppViewModel.DEMO_NETS else emptyList() }

139	            if (nets.isEmpty()) Hint("The bin has not listed any networks yet. Close this and press SCAN again, or type the name.")

140	            nets.forEach { n ->

141	                Opt(vm.ssid == n.ssid, { vm.ssid = n.ssid; vm.overlay = null }) {

142	                    T(n.ssid, Modifier.weight(1f), weight = FontWeight.Bold, maxLines = 1)

143	                    T(

144	                        n.rssi?.let { r -> (if (r >= -55) "Strong" else if (r >= -67) "Good" else if (r >= -75) "Fair" else "Weak") + " · $r dBm" } ?: "",

145	                        size = 0.8f, weight = FontWeight.Bold, color = Sb.c.ink2,

146	                    )

147	                }

148	            }

149	        }

150	        Overlay.BtDevices -> Sheet(close) {

151	            SheetTitle(Ic.bluetooth, "Bins nearby")

152	            Hint(if (vm.btScanning) "Looking for bins with WI-FI SETUP open..." else vm.btNote ?: "Pick the bin to pair with.")

153	            vm.btFound.forEach { f ->

154	                Opt(false, { vm.btConnect(f) }) {

155	                    Ico(Ic.bluetooth, 19.dp, Sb.c.bt)

156	                    T(f.name, Modifier.weight(1f), weight = FontWeight.Bold, maxLines = 1)

157	                    T("${f.rssi} dBm", size = 0.8f, weight = FontWeight.Bold, color = Sb.c.ink2)

158	                }

159	            }

160	            Acts {

161	                Btn("Cancel", close, Modifier.weight(1f), sound = Sfx.BACK)

162	                Btn(

163	                    if (vm.btScanning) "SEARCHING" else "SEARCH AGAIN", { vm.btSearch() }, Modifier.weight(1f), Ic.radar, Look.TINT, Sb.c.bt,

164	                    enabled = !vm.btScanning, iconMod = Modifier.spin(vm.btScanning),

165	                )

166	            }

167	        }

168	        is Overlay.PinUnlock -> Sheet(close) {

169	            OpPad(vm, Ic.lock(true), "Operator PIN", "Collector is the default. Enter the operator PIN to calibrate, simulate and edit bins.", "UNLOCK") { vm.pinUnlock(o.first) }

170	        }

171	        is Overlay.PinSet -> Sheet(close) {

172	            val again = vm.pinNew.isNotEmpty()

173	            OpPad(

174	                vm, Ic.key, if (o.change) "New operator PIN" else "Create the operator PIN",

175	                if (again) "Type it once more to be sure."

176	                else "${OperatorLock.MIN} to ${OperatorLock.MAX} digits. Anyone without it stays a collector. Only an operator can change it later.",

177	                if (again) "SAVE" else "NEXT",

178	            ) { vm.pinSave(o.first, o.change) }

179	        }

180	        Overlay.PinChange -> Sheet(close) {

181	            OpPad(vm, Ic.lock(true), "Change operator PIN", "Enter the current PIN first.", "NEXT") { vm.pinChangeCheck() }

182	        }

183	        is Overlay.BtCode -> {

184	            val b = hub.bins.firstOrNull { it.id == o.id } ?: return Gone(vm)

185	            Sheet({ vm.btCancel() }) {

186	                BinScreenCode(vm, b.name)

187	                PinPad(

188	                    Ic.bluetooth, "Type the pairing code", "On ${b.name}, open WI-FI SETUP, then type the 6 digits it shows. The code changes every time.",

189	                    "PAIR", vm.btEntry, AppViewModel.BT_CODE_LEN, AppViewModel.BT_CODE_LEN, vm.btMsg,

190	                    { vm.btKey(it) }, { vm.btBack() }, { vm.btCheck(b) },

191	                )

192	            }

193	        }

194	        Overlay.NotifPre -> Sheet({ vm.notNow() }) {

195	            val c = Sb.c

196	            Box(Modifier.fillMaxWidth(), contentAlignment = Alignment.Center) { Ico(Ic.bell, 40.dp, c.tamper) }

197	            SheetTitle(null, "Get alerts when the app is closed", center = true)

198	            SheetText("Smart-Bin tells you when a bin is full, tilted, too hot, too humid or its air turns bad, with a Start pickup button on the alert.", center = true)

199	            Acts {

200	                Btn("Not now", { vm.notNow() }, Modifier.weight(1f), sound = Sfx.BACK)

201	                Btn("Allow alerts", allowAlerts, Modifier.weight(1f), look = Look.ACCENT, sound = Sfx.CONFIRM)

202	            }

203	        }

204	    }

205	}

206

207	@Composable

208	private fun muted() = Sb.c.ink2

209

210	private fun Modifier.rotate180(): Modifier = this.graphicsLayer { rotationZ = 180f }

211

212	/** A sheet whose bin has gone, removed elsewhere: it closes itself. */

213	@Composable

214	private fun Gone(vm: AppViewModel) {

215	    LaunchedEffect(Unit) { vm.overlay = null }

216	}

217

218	@Composable

219	private fun RoundKey(label: String, enabled: Boolean, sound: Sfx, onClick: () -> Unit) {

220	    val c = Sb.c

221	    Box(

222	        Modifier.size(44.dp).then(if (enabled) Modifier else Modifier.padding(0.dp)).bump(c, 22.dp)

223	            .press(enabled, sound, onClick = onClick),

224	        contentAlignment = Alignment.Center,

225	    ) { T(label, size = 1.3f, color = if (enabled) c.ink2 else c.ink2.soft(0.35f), lineHeight = 1f) }

226	}

227

228	/** The operator PIN sheet. */

229	@Composable

230	private fun OpPad(vm: AppViewModel, icon: androidx.compose.ui.graphics.vector.ImageVector, title: String, text: String, go: String, onGo: () -> Unit) =

231	    PinPad(icon, title, text, go, vm.opPin, OperatorLock.MIN, OperatorLock.MAX, vm.pinMsg, { vm.pinKey(it) }, { vm.pinBack() }, onGo)

232

233	/** A code sheet: dots for the digits, a line for what went wrong, and a keypad. */

234	@Composable

235	private fun PinPad(

236	    icon: androidx.compose.ui.graphics.vector.ImageVector, title: String, text: String, go: String,

237	    value: String, min: Int, max: Int, msg: Pair<Boolean, String>?,

238	    onKey: (Char) -> Unit, onBack: () -> Unit, onGo: () -> Unit,

239	) {

240	    val c = Sb.c

241	    Box(Modifier.fillMaxWidth(), contentAlignment = Alignment.Center) { Ico(icon, 38.dp, c.accent) }

242	    SheetTitle(null, title, center = true)

243	    SheetText(text, center = true)

244	    Row(Modifier.fillMaxWidth().padding(vertical = 6.dp), horizontalArrangement = Arrangement.spacedBy(10.dp, Alignment.CenterHorizontally)) {

245	        repeat(maxOf(min, value.length)) { i ->

246	            Box(Modifier.size(14.dp).clip(RoundedCornerShape(7.dp)).background(if (i < value.length) c.accent else c.ink2.soft(0.25f)))

247	        }

248	    }

249	    msg?.let { (ok, m) -> T(m, Modifier.fillMaxWidth(), size = 0.82f, weight = FontWeight.Bold, color = if (ok) c.ok else c.bad, align = androidx.compose.ui.text.style.TextAlign.Center) }

250	    val rows = listOf("123", "456", "789")

251	    Column(Modifier.fillMaxWidth().padding(top = 4.dp), verticalArrangement = Arrangement.spacedBy(10.dp)) {

252	        rows.forEach { r ->

253	            Row(Modifier.fillMaxWidth(), horizontalArrangement = Arrangement.spacedBy(10.dp)) {

254	                r.forEach { d -> PinKey(d.toString(), Modifier.weight(1f), value.length < max) { onKey(d) } }

255	            }

256	        }

257	        Row(Modifier.fillMaxWidth(), horizontalArrangement = Arrangement.spacedBy(10.dp)) {

258	            PinKey("⌫", Modifier.weight(1f), value.isNotEmpty(), Sfx.STEP_DOWN, onClick = onBack)

259	            PinKey("0", Modifier.weight(1f), value.length < max) { onKey('0') }

260	            PinKey(go, Modifier.weight(1f), value.length >= min, Sfx.CONFIRM, accent = true, onClick = onGo)

261	        }

262	    }

263	}

264

265	/**

266	 * The stand-in for the bin's touchscreen while it pairs: the dark ILI9341

267	 * panel with the code in big digits and how long it lasts. A real bin shows

268	 * this on its own screen, never on the phone.

269	 */

270	@Composable

271	private fun BinScreenCode(vm: AppViewModel, name: String) {

272	    var left by remember { mutableStateOf(60) }

273	    LaunchedEffect(vm.btCodeAt) {

274	        while (true) {

275	            left = ((AppViewModel.BT_CODE_LIFE - (System.currentTimeMillis() - vm.btCodeAt)) / 1000).toInt().coerceAtLeast(0)

276	            if (left == 0) {

277	                vm.btNewCode()

278	                break

279	            }

280	            kotlinx.coroutines.delay(250)

281	        }

282	    }

283	    val dim = androidx.compose.ui.graphics.Color(0xFF8A96A3)

284	    Column(

285	        Modifier.fillMaxWidth().clip(RoundedCornerShape(14.dp)).background(androidx.compose.ui.graphics.Color(0xFF14181C)).padding(horizontal = 14.dp, vertical = 12.dp),

286	        horizontalAlignment = Alignment.CenterHorizontally,

287	        verticalArrangement = Arrangement.spacedBy(4.dp),

288	    ) {

289	        T("STAND-IN FOR ${name.uppercase()}'S SCREEN", size = 0.6f, spacing = 0.08f, color = dim)

290	        T("PAIRING CODE", size = 0.72f, spacing = 0.12f, color = androidx.compose.ui.graphics.Color(0xFF5FD0E8))

291	        T(vm.btCode.chunked(3).joinToString(" "), size = 2.1f, spacing = 0.08f, color = androidx.compose.ui.graphics.Color.White, tnum = true, maxLines = 1)

292	        T("Expires in $left s", size = 0.72f, weight = FontWeight.Bold, color = if (left <= 10) androidx.compose.ui.graphics.Color(0xFFFF8A7A) else dim, tnum = true)

293	    }

294	}

295

296	@Composable

297	private fun PinKey(label: String, modifier: Modifier, enabled: Boolean = true, sound: Sfx = Sfx.PIN_KEY, accent: Boolean = false, onClick: () -> Unit) {

298	    val c = Sb.c

299	    Box(

300	        modifier.height(54.dp).bump(c, 14.dp).press(enabled, sound, onClick = onClick),

301	        contentAlignment = Alignment.Center,

302	    ) {

303	        val ink = when { !enabled -> c.ink2.soft(0.35f); accent -> c.accent; else -> c.ink }

304	        T(label, size = if (label.length > 1 && label != "⌫") 0.86f else 1.35f, spacing = if (label.length > 1) 0.05f else 0f, color = ink, tnum = true, maxLines = 1)

305	    }

306	}

307

308	/** The Needs attention sheet: unread alerts first, then what is going on right now. */

309	@Composable

310	private fun Attention(vm: AppViewModel, hub: HubState) {

311	    val c = Sb.c

312	    SheetTitle(Ic.bell, "Needs attention")

313	    val pending = hub.alerts.filter { !it.read }

314	    val live = liveBanners(hub)

315	    if (pending.isNotEmpty()) {

316	        PopSec("Unread alerts")

317	        pending.take(4).forEach { a -> Banner(sevTone(a.sev), Ic.named(a.icon), a.title, "${a.meta} · ${ago(a.at)}") }

318	        if (pending.size > 4) PopSec("and ${pending.size - 4} more in the Alerts tab")

319	        Row(Modifier.padding(horizontal = 4.dp, vertical = 6.dp), horizontalArrangement = Arrangement.spacedBy(12.dp)) {

320	            Btn("MARK ALL READ", { vm.markAll() }, Modifier.weight(1f), Ic.check, Look.TINT, c.ok, small = true, sound = Sfx.CONFIRM)

321	            Btn("OPEN ALERTS", { vm.go(Screen.ALERTS) }, Modifier.weight(1f), Ic.bell, Look.TINT, c.tamper, small = true, sound = Sfx.OPEN)

322	        }

323	    }

324	    if (live.isNotEmpty()) {

325	        PopSec("Right now")

326	        live.forEach { Banner(it.tone, it.icon, it.title, it.small) }

327	    }

328	    if (pending.isEmpty() && live.isEmpty()) Empty(Ic.check, "Nothing needs attention.")

329	}

330

331	@Composable

332	private fun PopSec(text: String) {

333	    T(text.uppercase(), Modifier.padding(start = 2.dp, top = 4.dp), size = 0.7f, spacing = 0.08f, color = Sb.c.ink2)

334	}

335

336	/** Pick a bin, with a search box when there are many. */

337	@Composable

338	private fun Picker(vm: AppViewModel, hub: HubState, forWhat: PickFor) {

339	    val c = Sb.c

340	    val bins = hub.bins

341	    var q by remember { mutableStateOf("") }

342	    val sel: String? = when (forWhat) {

343	        PickFor.CHART -> vm.chartBin

344	        PickFor.ENV -> vm.binFor(vm.envBin, bins)?.id

345	        PickFor.SET -> vm.binFor(vm.selSet, bins)?.id

346	        PickFor.SIM -> vm.binFor(vm.selSim, bins)?.id

347	    }

348	    val choose = { id: String? ->

349	        when (forWhat) {

350	            PickFor.CHART -> vm.chartBin = id

351	            PickFor.ENV -> vm.envBin = id

352	            PickFor.SET -> { vm.selSet = id; vm.pinShown = false }

353	            PickFor.SIM -> vm.selSim = id

354	        }

355	        vm.overlay = null

356	    }

357	    SheetTitle(Ic.bins, "Pick a bin") { T(forWhat.title, size = 0.68f, weight = FontWeight.Bold, color = c.ink2) }

358	    if (bins.size > 8) Inp(q, { q = it }, placeholder = "Search by name or place")

359	    val shown = bins.withIndex().filter { (_, b) -> q.isBlank() || "${b.name} ${b.place}".contains(q.trim(), ignoreCase = true) }

360	    Column(verticalArrangement = Arrangement.spacedBy(9.dp)) {

361	        if (forWhat == PickFor.CHART && q.isBlank()) Opt(sel == null || bins.none { it.id == sel }, { choose(null) }) {

362	            Box(Modifier.size(34.dp), contentAlignment = Alignment.Center) { Ico(Ic.layers, 21.dp, c.ink2) }

363	            Column(Modifier.weight(1f)) {

364	                T("All bins", size = 0.95f)

365	                T(if (bins.size > 5) "The 5 fullest are drawn" else "Every bin on one chart", size = 0.76f, weight = FontWeight.SemiBold, color = c.ink2)

366	            }

367	        }

368	        shown.forEach { (i, b) ->

369	            Opt(sel == b.id, { choose(b.id) }) {

370	                BinIcon(b.reading.fill, binColor(i, c.isDark), 34.dp)

371	                Column(Modifier.weight(1f)) {

372	                    T(b.name, size = 0.95f, maxLines = 1)

373	                    T(b.place, size = 0.76f, weight = FontWeight.SemiBold, color = c.ink2, maxLines = 1)

374	                }

375	                if (b.simulating) MTag("SIM", c.sim)

376	                if (b.calibrating) MTag("CAL", c.cal)

377	                T("${Math.round(b.reading.fill)}%", size = 0.9f, color = levelColor(b, c))

378	            }

379	        }

380	        if (shown.isEmpty()) Empty(Ic.bins, "No bin matches.")

381	    }

382	}

383

384	@Composable

385	private fun MTag(text: String, bg: androidx.compose.ui.graphics.Color) {

386	    Box(Modifier.clip(RoundedCornerShape(5.dp)).background(bg).padding(horizontal = 6.dp, vertical = 3.dp)) {

387	        T(text, size = 0.6f, spacing = 0.04f, color = androidx.compose.ui.graphics.Color.White)

388	    }

389	}

Android App/app/src/main/java/smartbin/ui/AppPage.kt

1	// ============================================================================

2	// ui/AppPage.kt - App settings (the gear key)

3	// Sound and vibration switches, vibration strength, keep screen on, battery

4	// and push-alert permissions, the sound test and the demo options. Nothing

5	// here talks to the bins.

6	// ============================================================================

7	package smartbin.ui

8

9	import androidx.compose.foundation.background

10	import androidx.compose.foundation.interaction.MutableInteractionSource

11	import androidx.compose.foundation.interaction.collectIsPressedAsState

12	import androidx.compose.foundation.layout.Arrangement

13	import androidx.compose.foundation.layout.Box

14	import androidx.compose.foundation.layout.Column

15	import androidx.compose.foundation.layout.Row

16	import androidx.compose.foundation.layout.RowScope

17	import androidx.compose.foundation.layout.fillMaxSize

18	import androidx.compose.foundation.layout.fillMaxWidth

19	import androidx.compose.foundation.layout.padding

20	import androidx.compose.foundation.layout.size

21	import androidx.compose.foundation.layout.widthIn

22	import androidx.compose.foundation.shape.RoundedCornerShape

23	import androidx.compose.runtime.Composable

24	import androidx.compose.runtime.LaunchedEffect

25	import androidx.compose.runtime.getValue

26	import androidx.compose.runtime.mutableStateOf

27	import androidx.compose.runtime.remember

28	import androidx.compose.runtime.setValue

29	import androidx.compose.ui.Alignment

30	import androidx.compose.ui.Modifier

31	import androidx.compose.ui.draw.clip

32	import androidx.compose.ui.graphics.Color

33	import androidx.compose.ui.graphics.graphicsLayer

34	import androidx.compose.ui.graphics.vector.ImageVector

35	import androidx.compose.ui.platform.LocalContext

36	import androidx.compose.ui.text.font.FontWeight

37	import androidx.compose.ui.text.style.TextAlign

38	import androidx.compose.ui.unit.dp

39	import kotlinx.coroutines.delay

40	import smartbin.BuildConfig

41	import smartbin.data.AlertType

42	import smartbin.data.Hub

43	import smartbin.data.HubState

44	import smartbin.data.Link

45	import smartbin.data.Role

46	import smartbin.data.Source

47	import smartbin.notify.Alerts

48	import smartbin.notify.Feel

49	import smartbin.notify.Haptics

50	import smartbin.notify.Sfx

51	import smartbin.notify.Sounds

52

53	/**

54	 * The gear page: everything about the app itself, kept apart from the bins'

55	 * own settings on the Settings tab. Sound and vibration come first, then the

56	 * look, the links, running in the background, and the demo options.

57	 */

58	@Composable

59	fun AppPage(vm: AppViewModel, hub: HubState) {

60	    val c = Sb.c

61	    val context = LocalContext.current

62	    ScrollPage {

63	        BackBar("App settings", { vm.back() })

64

65	        // Sound and vibration, the two keys the gear is mostly opened for.

66	        Row(horizontalArrangement = Arrangement.spacedBy(10.dp)) {

67	            BigSwitch(Ic.speaker, "Sound", vm.sounds, c.accent) { vm.toggleSounds() }

68	            BigSwitch(Ic.vibrate, "Vibration", vm.haptics, c.accent) { vm.toggleHaptics() }

69	        }

70	        Column(

71	            Modifier.fillMaxWidth().bump(c, 14.dp).padding(horizontal = 13.dp, vertical = 11.dp),

72	            verticalArrangement = Arrangement.spacedBy(9.dp),

73	        ) {

74	            T("Vibration strength", weight = FontWeight.Bold)

75	            TwoWay(full = true) {

76	                Haptics.Strength.entries.forEach { s ->

77	                    Tw(vm.strength == s, { vm.chooseStrength(s) }, s.label, enabled = vm.haptics)

78	                }

79	            }

80	        }

81	        SoundTest()

82	        LRow(Ic.sun, "Keep screen on", "Stops the phone dimming while the app is open", chevron = false) {

83	            Toggle(vm.keepAwake, { vm.toggleAwake() })

84	        }

85

86	        Row(Modifier.padding(top = 4.dp)) { Sh(Ic.palette, "Look and role") }

87	        LRow(Ic.palette, "Appearance", chevron = false) {

88	            TwoWay {

89	                ThemeMode.entries.forEach { m ->

90	                    Tw(vm.theme == m, { vm.setThemeMode(m) }, m.name.lowercase().replaceFirstChar { it.uppercase() }, grow = null)

91	                }

92	            }

93	        }

94	        LRow(Ic.user, "Signed in as", chevron = false) {

95	            TwoWay {

96	                Tw(vm.operator, { vm.pickRole(Role.OPERATOR) }, "Operator", grow = null)

97	                Tw(!vm.operator, { vm.pickRole(Role.COLLECTOR) }, "Collector", grow = null)

98	            }

99	        }

100	        if (vm.operator) LRow(Ic.key, "Change operator PIN", "Asks for the current PIN first", onClick = { vm.changePin() })

101	        Hint("The app opens on the start page each time, where you pick a role. Operator needs the PIN, and drops back to collector after 2 min in the background. After 5 wrong PINs it waits 30 s, longer each time.")

102

103	        Row(Modifier.padding(top = 4.dp)) { Sh(Ic.layers, "Links to the bins") }

104	        val looks = routeLooks(hub.routes, c)

105	        LRow(

106	            looks.firstOrNull()?.icon ?: Ic.wifi(false), "Connection",

107	            when {

108	                looks.isNotEmpty() -> looks.joinToString(" + ") { it.name } + if (hub.demo) " (demo)" else " answering"

109	                hub.link == Link.DEMO -> "Demo bins only. Pair a bin over Bluetooth to go live."

110	                else -> "Nothing answers. Check Wi-Fi and the cloud accounts."

111	            },

112	            looks.firstOrNull()?.color ?: if (hub.link == Link.OFFLINE) c.bad else c.warn, onClick = { vm.go(Screen.NET) },

113	        )

114	        LRow(

115	            Ic.bluetooth, "Bluetooth setup",

116	            when { vm.bt in 1..3 -> "Paired, step ${vm.bt + 1} of 4"; vm.bt >= 4 -> "Done, Bluetooth is off"; else -> "Send Wi-Fi to a bin" },

117	            c.bt, onClick = { vm.go(Screen.BT) },

118	        )

119

120	        Row(Modifier.padding(top = 4.dp)) { Sh(Ic.bell, "Alerts in the background") }

121	        LRow(

122	            Ic.bell, "Push alerts",

123	            when (vm.notifications) {

124	                Perm.ON -> "On. Alerts arrive with the app closed."

125	                Perm.BLOCKED -> "Blocked in Android settings."

126	                Perm.OFF -> "Not turned on yet."

127	            },

128	            c.tamper, chevron = false,

129	        ) {

130	            when (vm.notifications) {

131	                Perm.ON -> Tag("ON", c.ok)

132	                Perm.OFF -> Btn("TURN ON", { vm.overlay = Overlay.NotifPre }, look = Look.TINT, cc = c.tamper, small = true)

133	                Perm.BLOCKED -> Btn("OPEN", { openAlertSettings(context) }, look = Look.TINT, cc = c.tamper, small = true, sound = Sfx.OPEN)

134	            }

135	        }

136	        LRow(

137	            Ic.power, "Unrestricted battery",

138	            if (vm.unrestricted) "On. Android will not pause the app to save battery."

139	            else "Optimized. Android may pause the app after a while in the background.",

140	            c.warn, chevron = false,

141	        ) {

142	            if (vm.unrestricted) Tag("ON", c.ok)

143	            else Btn("ALLOW", { vm.askUnrestricted() }, look = Look.TINT, cc = c.warn, small = true, sound = Sfx.OPEN)

144	        }

145	        LRow(Ic.bell, "Android alert settings", "Change or silence each kind of alert while the app is closed", onClick = { openAlertSettings(context) })

146	        Hint(

147	            "With push alerts on, the app keeps reading the bins after you close it, every 5 s, and a quiet **Watching bins** notice shows while it does. " +

148	                "Unrestricted battery stops Android from pausing that. Some phones also keep their own list in the app's page of Android settings.",

149	        )

150	        if (!vm.unrestricted) Btn("OPEN APP INFO", { vm.openAppInfo() }, Modifier.fillMaxWidth(), Ic.gear, Look.TINT, sound = Sfx.OPEN)

151

152	        Row(Modifier.padding(top = 4.dp)) { Sh(Ic.flask, "Demo", c.sim) }

153	        LRow(

154	            Ic.undo, "Start page",

155	            if (hub.demoOn) "Demo is on. Turn it off or on from the start page." else "Pick a role again, or turn on the demo",

156	            c.sim, onClick = { vm.toStart() },

157	        )

158

159	        if (vm.operator) {

160	            Row(Modifier.padding(top = 4.dp)) { Sh(Ic.download, "Bin list") }

161	            LRow(Ic.download, "Export bin list", "Names, places and addresses as a file", onClick = { vm.exportBins(hub.bins) }, chevron = false, sound = Sfx.SAVED)

162	        }

163

164	        if (hub.demoOn) DemoOptions(vm, hub)

165	        Hint("Smart-Bin app ${BuildConfig.VERSION_NAME}, matches dashboard v2 and firmware ${hub.bins.firstOrNull { !it.isDemo && it.fw.isNotBlank() }?.fw ?: "v2"}", Modifier.fillMaxWidth(), align = TextAlign.Center)

166	    }

167	}

168

169	/** A large on and off key for the two things people change most. */

170	@Composable

171	private fun RowScope.BigSwitch(icon: ImageVector, label: String, on: Boolean, cc: Color, onClick: () -> Unit) {

172	    val c = Sb.c

173	    val src = remember { MutableInteractionSource() }

174	    val pressed by src.collectIsPressedAsState()

175	    Column(

176	        Modifier

177	            .weight(1f)

178	            .then(

179	                if (on || pressed) Modifier.ring(cc.soft(0.3f), 16.dp).clip(RoundedCornerShape(16.dp)).background(c.base).sunk(c, 16.dp)

180	                else Modifier.bump(c, 16.dp)

181	            )

182	            .press(true, if (on) Sfx.SWITCH_OFF else Sfx.SWITCH_ON, src, onClick)

183	            .padding(vertical = 14.dp),

184	        horizontalAlignment = Alignment.CenterHorizontally,

185	        verticalArrangement = Arrangement.spacedBy(6.dp),

186	    ) {

187	        Ico(icon, 28.dp, if (on) cc else c.ink2)

188	        T(label, size = 0.9f, color = if (on) c.ink else c.ink2)

189	        T(if (on) "ON" else "OFF", size = 0.72f, spacing = 0.08f, color = if (on) cc else c.ink2)

190	    }

191	}

192

193	/** One key of the sound test: the sound, its name, and an icon and colour of its own. */

194	private class Snd(val sfx: Sfx, val label: String, val icon: ImageVector, val color: (SbColors) -> Color)

195

196	/**

197	 * The sound test, the same groups, names, icons and colours as the dashboard's.

198	 * Each key plays its sound and vibration even with sound off, so people can learn them.

199	 */

200	private val SOUND_GROUPS: List<Pair<String, List<Snd>>> = listOf(

201	    "Alerts" to listOf(

202	        Snd(Sfx.ALERT_FULL, "Bin full", Ic.fill) { it.bad },

203	        Snd(Sfx.ALERT_TAMPER, "Tamper", Ic.tamper) { it.tamper },

204	        Snd(Sfx.ALERT_SENSOR, "Sensor limit", Ic.temp(41.0)) { it.temp },

205	        Snd(Sfx.ALERT_LOCK, "Bin locked", Ic.lock(true)) { it.sim },

206	        Snd(Sfx.ALERT_PICKUP, "Pickup alert", Ic.truck) { it.cal },

207	        Snd(Sfx.ALERT_INFO, "Info", Ic.bell) { it.bt },

208	    ),

209	    "Bins and links" to listOf(

210	        Snd(Sfx.LINK_UP, "Wi-Fi link", Ic.wifi(true)) { it.link },

211	        Snd(Sfx.CLOUD, "Cloud link", Ic.cloud) { it.cloud },

212	        Snd(Sfx.LINK_LOST, "Link lost", Ic.wifi(false)) { it.bad },

213	        Snd(Sfx.BIN_LOST, "Bin quiet", Ic.radar) { it.warn },

214	        Snd(Sfx.BIN_BACK, "Bin back", Ic.bins) { it.ok },

215	        Snd(Sfx.CMD_OK, "Command OK", Ic.check) { it.ok },

216	        Snd(Sfx.CMD_FAIL, "Command failed", Ic.cross) { it.bad },

217	    ),

218	    "Lid and pickup" to listOf(

219	        Snd(Sfx.LOCK, "Lock", Ic.lock(true)) { it.sim },

220	        Snd(Sfx.UNLOCK, "Unlock", Ic.lock(false)) { it.lid },

221	        Snd(Sfx.PICKUP, "Pickup start", Ic.truck) { it.cal },

222	        Snd(Sfx.DONE, "Pickup finished", Ic.check) { it.fill },

223	    ),

224	    "Setup" to listOf(

225	        Snd(Sfx.CALIB, "Calibrate", Ic.calib) { it.cal },

226	        Snd(Sfx.SIM, "Simulation", Ic.flask) { it.sim },

227	        Snd(Sfx.SAVED, "Saved", Ic.save) { it.ok },

228	        Snd(Sfx.DENIED, "Not allowed", Ic.cross) { it.warn },

229	    ),

230	    "Bluetooth Wi-Fi setup" to listOf(

231	        Snd(Sfx.BT_PAIRED, "Paired", Ic.bluetooth) { it.bt },

232	        Snd(Sfx.BT_WRONG, "Wrong code", Ic.key) { it.warn },

233	        Snd(Sfx.BT_BLOCKED, "Pairing stopped", Ic.lock(true)) { it.bad },

234	        Snd(Sfx.WIFI_JOINED, "Wi-Fi joined", Ic.wifi(true)) { it.ok },

235	        Snd(Sfx.WIFI_FAILED, "Wi-Fi failed", Ic.wifi(false)) { it.tamper },

236	        Snd(Sfx.BT_OFF, "Bluetooth off", Ic.power) { it.motion },

237	    ),

238	    "Operator PIN" to listOf(

239	        Snd(Sfx.OP_IN, "Operator in", Ic.sliders) { it.accent },

240	        Snd(Sfx.OP_OUT, "Operator out", Ic.undo) { it.weight },

241	        Snd(Sfx.PIN_KEY, "PIN key", Ic.pin) { it.humid },

242	        Snd(Sfx.PIN_WRONG, "Wrong PIN", Ic.key) { it.bad },

243	        Snd(Sfx.PIN_LOCKOUT, "PIN lockout", Ic.stopwatch) { it.tamper },

244	        Snd(Sfx.PIN_SET, "PIN saved", Ic.save) { it.gas },

245	    ),

246	    "App" to listOf(

247	        Snd(Sfx.BOOT, "Start", Ic.power) { it.ok },

248	        Snd(Sfx.LOGO, "Logo", Ic.bins) { it.gas },

249	        Snd(Sfx.DEMO, "Demo on", Ic.flask) { it.warn },

250	        Snd(Sfx.THEME_LIGHT, "Light theme", Ic.sunmoon(false)) { it.lid },

251	        Snd(Sfx.THEME_DARK, "Dark theme", Ic.sunmoon(true)) { it.humid },

252	    ),

253	    "Keys" to listOf(

254	        Snd(Sfx.TAP, "Tap", Ic.capture) { it.humid },

255	        Snd(Sfx.CHOICE, "Choice", Ic.sliders) { it.gas },

256	        Snd(Sfx.TAB, "Tab", Ic.layers) { it.motion },

257	        Snd(Sfx.OPEN, "Open", Ic.eye) { it.fill },

258	        Snd(Sfx.BACK, "Back", Ic.undo) { it.weight },

259	        Snd(Sfx.CONFIRM, "Confirm", Ic.check) { it.ok },

260	        Snd(Sfx.DANGER, "Danger", Ic.trash) { it.bad },

261	        Snd(Sfx.SWITCH_ON, "Switch on", Ic.power) { it.link },

262	        Snd(Sfx.SWITCH_OFF, "Switch off", Ic.power) { it.ink2 },

263	        Snd(Sfx.STEP_UP, "Step up", Ic.plus) { it.temp },

264	        Snd(Sfx.STEP_DOWN, "Step down", Ic.percent) { it.press },

265	    ),

266	)

267

268	@Composable

269	private fun SoundTest() {

270	    val c = Sb.c

271	    var open by remember { mutableStateOf(false) }

272	    Column(

273	        Modifier.fillMaxWidth().bump(c, 14.dp).padding(horizontal = 13.dp, vertical = 11.dp),

274	        verticalArrangement = Arrangement.spacedBy(9.dp),

275	    ) {

276	        Row(verticalAlignment = Alignment.CenterVertically) {

277	            T("Sound test", Modifier.weight(1f), weight = FontWeight.Bold)

278	            Btn(if (open) "HIDE" else "SHOW", { open = !open }, icon = Ic.speaker, look = Look.TINT, small = true, sound = Sfx.OPEN)

279	        }

280	        T("Every sound the app and dashboard make, in the same groups. Each one plays with its vibration, even with sound off, so you can learn them.", size = 0.8f, weight = FontWeight.SemiBold, color = c.ink2)

281	        if (open) SOUND_GROUPS.forEach { (group, sounds) ->

282	            T(group, size = 0.78f, spacing = 0.06f, weight = FontWeight.Bold, color = c.ink2)

283	            sounds.chunked(3).forEach { row ->

284	                Row(horizontalArrangement = Arrangement.spacedBy(8.dp)) {

285	                    row.forEach { k ->

286	                        // The key itself plays the sound and its buzz; with sound off, play it anyway.

287	                        Btn(k.label, { if (!Feel.sounds) Sounds.play(k.sfx, force = true) }, Modifier.weight(1f), k.icon, Look.TINT, k.color(c), small = true, sound = k.sfx, textSize = 0.7f)

288	                    }

289	                    repeat(3 - row.size) { Box(Modifier.weight(1f)) }

290	                }

291	            }

292	        }

293	    }

294	}

295

296	/**

297	 * Demo options act out what simulation cannot: a faster or paused demo, links

298	 * coming and going, a bin going quiet, and one-tap events. They change only

299	 * bins with no address, are forgotten when the app closes, and while none is

300	 * used the demo runs exactly as before.

301	 */

302	@Composable

303	private fun DemoOptions(vm: AppViewModel, hub: HubState) {

304	    val c = Sb.c

305	    @Suppress("UNUSED_VARIABLE") val tick = vm.demoTick

306	    Row(Modifier.padding(top = 4.dp)) { Sh(Ic.flask, "Demo options", c.sim) }

307	    val inUse = Hub.demoInUse

308	    Card {

309	        Row(verticalAlignment = Alignment.CenterVertically, horizontalArrangement = Arrangement.spacedBy(10.dp)) {

310	            Ico(Ic.flask, 20.dp, if (inUse) c.sim else c.ink2)

311	            T(if (inUse) "Demo options are in use" else "Not in use. The demo runs as normal.", Modifier.weight(1f), size = 0.9f, weight = FontWeight.Bold)

312	            if (inUse) Btn("RESET", { vm.resetDemo() }, icon = Ic.undo, look = Look.TINT, cc = c.sim, small = true, sound = Sfx.CONFIRM)

313	        }

314

315	        Field("Fill speed", Ic.stopwatch, c.sim) {

316	            TwoWay(full = true) {

317	                listOf(0.0 to "Pause", 1.0 to "Real", 60.0 to "Normal", 600.0 to "Fast").forEach { (x, label) ->

318	                    Tw(vm.demoSpeed == x, { vm.changeDemoSpeed(x) }, label, cc = c.sim)

319	                }

320	            }

321	        }

322	        Hint("Normal fills a bin in one demo minute as much as in one real hour. Fast is ten times that, for showing a bin fill up in class.")

323

324	        Field("Pretend these links answer", Ic.layers, c.sim) {

325	            Row(horizontalArrangement = Arrangement.spacedBy(8.dp)) {

326	                val r = hub.demoLinks

327	                Btn("WI-FI", { vm.toggleDemoLink(0) }, Modifier.weight(1f), Ic.wifi(true), Look.TINT, c.link, small = true, pressedIn = r.local, sound = Sfx.CHOICE)

328	                Btn("FIREBASE", { vm.toggleDemoLink(1) }, Modifier.weight(1f), Ic.cloud, Look.TINT, c.cloud, small = true, pressedIn = r.firebase, sound = Sfx.CHOICE, textSize = 0.7f)

329	                Btn("TSPEAK", { vm.toggleDemoLink(2) }, Modifier.weight(1f), Ic.chart, Look.TINT, c.ts, small = true, pressedIn = r.thingspeak, sound = Sfx.CHOICE, textSize = 0.7f)

330	            }

331	        }

332	        Hint("The first one on, in the app's order (Wi-Fi, then Firebase, then ThingSpeak), is the one the demo bins answer on. The cards, banners and link chip change to match, in each link's colour: Wi-Fi green, Firebase orange, ThingSpeak blue.")

333

334	        Field("Cloud commands", Ic.cloud, c.sim) {

335	            TwoWay(full = true) {

336	                Tw(!hub.demoNoConfirm, { vm.setDemoNoConfirm(false) }, "Confirm in 4 s", cc = c.ok)

337	                Tw(hub.demoNoConfirm, { vm.setDemoNoConfirm(true) }, "Time out at 8 s", cc = c.bad)

338	            }

339	        }

340	        Hint("With Firebase or ThingSpeak on, LOCK, UNLOCK and COLLECT stay locked while the bin is asked. The bin either confirms after 4 s and the key flashes green, or never answers and the key flashes red at 8 s.")

341

342	        Field("Act out on a bin", Ic.bins, c.sim) {

343	            BinSeg(vm, hub, PickFor.SET, vm.selSet, allowAll = false) { vm.selSet = it }

344	        }

345	        val b = vm.binFor(vm.selSet, hub.bins)

346	        if (b != null) {

347	            Row(horizontalArrangement = Arrangement.spacedBy(8.dp)) {

348	                Btn("FULL", { vm.demoEvent(b, Hub.DemoEvent.FULL) }, Modifier.weight(1f), Ic.fill, Look.TINT, c.bad, small = true, sound = Sfx.CONFIRM)

349	                Btn("TAMPER", { vm.demoEvent(b, Hub.DemoEvent.TAMPER) }, Modifier.weight(1f), Ic.tamper, Look.TINT, c.tamper, small = true, sound = Sfx.CONFIRM)

350	            }

351	            Row(horizontalArrangement = Arrangement.spacedBy(8.dp)) {

352	                Btn("BAD AIR", { vm.demoEvent(b, Hub.DemoEvent.AIR) }, Modifier.weight(1f), Ic.gas, Look.TINT, c.gas, small = true, sound = Sfx.CONFIRM)

353	                Btn("EMPTY", { vm.demoEvent(b, Hub.DemoEvent.EMPTY) }, Modifier.weight(1f), Ic.truck, Look.TINT, c.cal, small = true, sound = Sfx.CONFIRM)

354	            }

355	            val quiet = Hub.demoQuiet(b.id)

356	            Btn(

357	                if (quiet) "ANSWER AGAIN" else "GO QUIET", { vm.toggleDemoQuiet(b) }, Modifier.fillMaxWidth(),

358	                Ic.wifi(quiet), Look.TINT, if (quiet) c.ok else c.warn, small = true, sound = if (quiet) Sfx.LINK_UP else Sfx.LINK_LOST,

359	            )

360	        }

361	        Hint("Simulation holds sensor values by hand. These act out events instead: a bin filling past its limit, being tipped, a burst of bad air, a pickup, or a bin that stops answering. They change only the demo bins and are forgotten when the app closes.")

362	    }

363	}

Part 4. Android App (XML resources)

Android App/app/src/main/AndroidManifest.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!--

3	============================================================================

4	AndroidManifest.xml - what the app is and what it may do

5	Lists the permissions, the single activity (MainActivity), the two launcher

6	aliases (light and dark logo), the background WatchService and the

7	notification action receiver. A missing permission here fails silently at

8	runtime: for example, without POST_NOTIFICATIONS no alert is ever shown on

9	Android 13 and newer.

10	============================================================================

11	-->

12	<manifest xmlns:android="http://schemas.android.com/apk/res/android"

13	    xmlns:tools="http://schemas.android.com/tools">

14

15	    <!-- Network: the bins' local API, Firebase and ThingSpeak. -->

16	    <uses-permission android:name="android.permission.INTERNET" />

17	    <uses-permission android:name="android.permission.ACCESS_NETWORK_STATE" />

18	    <uses-permission android:name="android.permission.POST_NOTIFICATIONS" />

19	    <!-- The app's own vibration patterns for alerts, warnings and the link to the bins. -->

20	    <uses-permission android:name="android.permission.VIBRATE" />

21	    <!-- Keeps reading the bins for alerts while the app is closed. -->

22	    <uses-permission android:name="android.permission.FOREGROUND_SERVICE" />

23	    <uses-permission android:name="android.permission.FOREGROUND_SERVICE_CONNECTED_DEVICE" />

24	    <uses-permission android:name="android.permission.CHANGE_NETWORK_STATE" />

25	    <!-- The Unrestricted battery option in App settings asks for this. -->

26	    <uses-permission android:name="android.permission.REQUEST_IGNORE_BATTERY_OPTIMIZATIONS" />

27	    <!-- Wi-Fi setup over Bluetooth: find the bin, pair with its code, hand over the network. -->

28	    <uses-permission android:name="android.permission.BLUETOOTH" android:maxSdkVersion="30" />

29	    <uses-permission android:name="android.permission.BLUETOOTH_ADMIN" android:maxSdkVersion="30" />

30	    <uses-permission android:name="android.permission.ACCESS_FINE_LOCATION" android:maxSdkVersion="30" />

31	    <uses-permission android:name="android.permission.BLUETOOTH_SCAN" android:usesPermissionFlags="neverForLocation" tools:targetApi="s" />

32	    <uses-permission android:name="android.permission.BLUETOOTH_CONNECT" />

33	    <uses-feature android:name="android.hardware.bluetooth_le" android:required="false" />

34

35	    <!-- usesCleartextTraffic="true" below: the bins answer on plain http://

36	         inside the local network, so Android must allow non-TLS traffic.

37	         Without it every local call fails and the app falls back to the clouds.

38	         allowBackup="false" and the two rules files keep the saved settings

39	         (with the sealed Firebase password) out of backups. -->

40	    <application

41	        android:name=".SmartBinApp"

42	        android:allowBackup="false"

43	        android:usesCleartextTraffic="true"

44	        android:dataExtractionRules="@xml/data_extraction_rules"

45	        android:fullBackupContent="@xml/backup_rules"

46	        android:icon="@mipmap/ic_launcher"

47	        android:label="@string/app_name"

48	        android:roundIcon="@mipmap/ic_launcher_round"

49	        android:supportsRtl="true"

50	        android:theme="@style/Theme.SmartBin">

51	        <!-- configChanges: rotation, screen size and dark-mode changes do not

52	             recreate the activity; Compose redraws for the new size or theme. -->

53	        <activity

54	            android:name=".MainActivity"

55	            android:exported="true"

56	            android:configChanges="orientation|screenSize|screenLayout|smallestScreenSize|keyboardHidden|uiMode"

57	            android:label="@string/app_name"

58	            android:launchMode="singleTop"

59	            android:windowSoftInputMode="adjustResize" />

60	        <!-- The home-screen entry. There are two, one per logo, and only one is

61	             switched on at a time: the light logo or the dark one, following the

62	             theme the app was last left in. MainActivity switches them. -->

63	        <activity-alias

64	            android:name=".LauncherLight"

65	            android:enabled="true"

66	            android:exported="true"

67	            android:icon="@mipmap/ic_launcher"

68	            android:roundIcon="@mipmap/ic_launcher_round"

69	            android:label="@string/app_name"

70	            android:targetActivity=".MainActivity">

71	            <intent-filter>

72	                <action android:name="android.intent.action.MAIN" />

73	                <category android:name="android.intent.category.LAUNCHER" />

74	            </intent-filter>

75	        </activity-alias>

76	        <activity-alias

77	            android:name=".LauncherDark"

78	            android:enabled="false"

79	            android:exported="true"

80	            android:icon="@mipmap/ic_launcher_dark"

81	            android:roundIcon="@mipmap/ic_launcher_dark_round"

82	            android:label="@string/app_name"

83	            android:targetActivity=".MainActivity">

84	            <intent-filter>

85	                <action android:name="android.intent.action.MAIN" />

86	                <category android:name="android.intent.category.LAUNCHER" />

87	            </intent-filter>

88	        </activity-alias>

89	        <!-- stopWithTask: swiping the app away from recent apps stops the

90	             service, so alerts end when the user closes the app. -->

91	        <service

92	            android:name=".notify.WatchService"

93	            android:exported="false"

94	            android:stopWithTask="true"

95	            android:foregroundServiceType="connectedDevice" />

96	        <receiver

97	            android:name=".notify.ActionReceiver"

98	            android:exported="false" />

99	    </application>

100

101	</manifest>

Android App/app/src/main/res/values/colors.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Colors Android itself needs before Compose starts: icon backgrounds and

3	     the splash background. The app's own palette lives in ui/Theme.kt. -->

4	<resources>

5	    <!-- The page colours of the mockup's light and dark themes. -->

6	    <color name="ic_launcher_bg_light">#FFE4DCD2</color>

7	    <color name="ic_launcher_bg_dark">#FF2B2622</color>

8	    <color name="splash_light">#FFE4DCD2</color>

9	</resources>

Android App/app/src/main/res/values/strings.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- The app name under the launcher icon. All other text is in the Kotlin code. -->

3	<resources>

4	    <string name="app_name">Smart-Bin</string>

5	</resources>

Android App/app/src/main/res/values/themes.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Window theme shown for the instant before Compose draws: no action bar,

3	     and the mockup's light page color so the start has no white flash. -->

4	<resources>

5	    <style name="Theme.SmartBin" parent="android:Theme.Material.Light.NoActionBar">

6	        <item name="android:windowBackground">@color/splash_light</item>

7	    </style>

8	</resources>

Android App/app/src/main/res/xml/backup_rules.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Nothing is backed up: the Firebase password is sealed with a key that never leaves this phone. -->

3	<full-backup-content>

4	    <exclude domain="sharedpref" path="." />

5	</full-backup-content>

Android App/app/src/main/res/xml/data_extraction_rules.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Android 12+ version of backup_rules.xml: the saved settings

3	     (SharedPreferences) are left out of cloud backup and phone-to-phone

4	     transfer, for the same reason. -->

5	<data-extraction-rules>

6	    <cloud-backup>

7	        <exclude domain="sharedpref" path="." />

8	    </cloud-backup>

9	    <device-transfer>

10	        <exclude domain="sharedpref" path="." />

11	    </device-transfer>

12	</data-extraction-rules>

Android App/app/src/main/res/mipmap-anydpi-v26/ic_launcher.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Adaptive launcher icon, light theme: background color, foreground logo and

3	     the one-color version Android 13+ uses for themed icons. -->

4	<adaptive-icon xmlns:android="http://schemas.android.com/apk/res/android">

5	    <background android:drawable="@color/ic_launcher_bg_light" />

6	    <foreground android:drawable="@drawable/ic_launcher_fg_light" />

7	    <monochrome android:drawable="@drawable/ic_launcher_mono" />

8	</adaptive-icon>

Android App/app/src/main/res/mipmap-anydpi-v26/ic_launcher_round.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Same icon for launchers that ask for a round one. -->

3	<adaptive-icon xmlns:android="http://schemas.android.com/apk/res/android">

4	    <background android:drawable="@color/ic_launcher_bg_light" />

5	    <foreground android:drawable="@drawable/ic_launcher_fg_light" />

6	    <monochrome android:drawable="@drawable/ic_launcher_mono" />

7	</adaptive-icon>

Android App/app/src/main/res/mipmap-anydpi-v26/ic_launcher_dark.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Adaptive launcher icon, dark theme (used by the LauncherDark alias). -->

3	<adaptive-icon xmlns:android="http://schemas.android.com/apk/res/android">

4	    <background android:drawable="@color/ic_launcher_bg_dark" />

5	    <foreground android:drawable="@drawable/ic_launcher_fg_dark" />

6	    <monochrome android:drawable="@drawable/ic_launcher_mono" />

7	</adaptive-icon>

Android App/app/src/main/res/mipmap-anydpi-v26/ic_launcher_dark_round.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Dark icon for launchers that ask for a round one. -->

3	<adaptive-icon xmlns:android="http://schemas.android.com/apk/res/android">

4	    <background android:drawable="@color/ic_launcher_bg_dark" />

5	    <foreground android:drawable="@drawable/ic_launcher_fg_dark" />

6	    <monochrome android:drawable="@drawable/ic_launcher_mono" />

7	</adaptive-icon>

Android App/app/src/main/res/drawable/ic_launcher_fg_light.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Home-screen logo, light: the still frame of the app's v2 logo (ui/Logo.kt), a bin with a

3	     fill wave, a controller chip, a status light on the lid and a Wi-Fi signal. -->

4	<vector xmlns:android="http://schemas.android.com/apk/res/android"

5	    xmlns:aapt="http://schemas.android.com/aapt"

6	    android:width="108dp"

7	    android:height="108dp"

8	    android:viewportWidth="108"

9	    android:viewportHeight="108">

10	    <group android:scaleX="1.1" android:scaleY="1.1" android:translateX="18.8" android:translateY="21">

11	        <path android:pathData="M32,0a12,12 0,1 1,0 24a12,12 0,1 1,0 -24z">

12	            <aapt:attr name="android:fillColor">

13	                <gradient android:type="radial" android:centerX="32" android:centerY="12" android:gradientRadius="12"

14	                    android:startColor="#731E8E3E" android:endColor="#001E8E3E" />

15	            </aapt:attr>

16	        </path>

17	        <path android:pathData="M27.8,12.6a6,6 0,0 1,8.4 0" android:strokeColor="#FF1E8E3E" android:strokeWidth="2.6" android:strokeLineCap="round" />

18	        <path android:pathData="M24.2,8.9a11,11 0,0 1,15.6 0" android:strokeColor="#B31E8E3E" android:strokeWidth="2.6" android:strokeLineCap="round" />

19	        <path android:pathData="M20.6,5.2a16,16 0,0 1,22.8 0" android:strokeColor="#661E8E3E" android:strokeWidth="2.6" android:strokeLineCap="round" />

20	        <path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z">

21	            <aapt:attr name="android:fillColor">

22	                <gradient android:type="linear" android:startX="17.5" android:startY="0" android:endX="46.5" android:endY="0"

23	                    android:startColor="#0D1E8E3E" android:centerColor="#291E8E3E" android:endColor="#0D1E8E3E" />

24	            </aapt:attr>

25	        </path>

26	        <group>

27	            <clip-path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z" />

28	            <path android:pathData="M0,37q5,-3.6 10,0t10,0t10,0t10,0t10,0t10,0t10,0v30H0z" android:fillAlpha="0.35">

29	                <aapt:attr name="android:fillColor">

30	                    <gradient android:type="linear" android:startX="0" android:startY="34" android:endX="0" android:endY="57"

31	                        android:startColor="#FF1E8E3E" android:endColor="#FF0D8A8A" />

32	                </aapt:attr>

33	            </path>

34	            <path android:pathData="M-5,39q5,-3.6 10,0t10,0t10,0t10,0t10,0t10,0t10,0v30H-5z">

35	                <aapt:attr name="android:fillColor">

36	                    <gradient android:type="linear" android:startX="0" android:startY="34" android:endX="0" android:endY="57"

37	                        android:startColor="#FF1E8E3E" android:endColor="#FF0D8A8A" />

38	                </aapt:attr>

39	            </path>

40	        </group>

41	        <path android:pathData="M22,30.5h3.4M22,34h2.2" android:strokeColor="#CC1E8E3E" android:strokeWidth="1.8" android:strokeLineCap="round" />

42	        <path android:pathData="M29.3,42h5.4a1.8,1.8 0,0 1,1.8 1.8v5.4a1.8,1.8 0,0 1,-1.8 1.8h-5.4a1.8,1.8 0,0 1,-1.8 -1.8v-5.4a1.8,1.8 0,0 1,1.8 -1.8z"

43	            android:strokeColor="#FFFFFFFF" android:strokeWidth="2" />

44	        <path android:pathData="M32,42v-3.2M32,51v2.5M27.5,46.5h-4M36.5,46.5h4" android:strokeColor="#FFFFFFFF" android:strokeWidth="1.8" android:strokeLineCap="round" />

45	        <path android:pathData="M30.5,37.6a1.5,1.5 0,1 0,3 0a1.5,1.5 0,1 0,-3 0zM21.1,46.5a1.5,1.5 0,1 0,3 0a1.5,1.5 0,1 0,-3 0zM39.9,46.5a1.5,1.5 0,1 0,3 0a1.5,1.5 0,1 0,-3 0z"

46	            android:fillColor="#FFFFFFFF" />

47	        <path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z" android:strokeColor="#FF2A241E" android:strokeWidth="3" android:strokeLineJoin="round" />

48	        <path android:pathData="M13,22.5h38" android:strokeColor="#FF2A241E" android:strokeWidth="3.4" android:strokeLineCap="round" />

49	        <path android:pathData="M27,22.5v-3.4h10v3.4" android:strokeColor="#FF2A241E" android:strokeWidth="3" android:strokeLineCap="round" android:strokeLineJoin="round" />

50	        <path android:pathData="M44.2,22.5a1.3,1.3 0,1 0,2.6 0a1.3,1.3 0,1 0,-2.6 0z" android:fillColor="#FF1E8E3E" />

51	    </group>

52	</vector>

Android App/app/src/main/res/drawable/ic_launcher_fg_dark.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Home-screen logo, dark: the still frame of the app's v2 logo (ui/Logo.kt), a bin with a

3	     fill wave, a controller chip, a status light on the lid and a Wi-Fi signal. -->

4	<vector xmlns:android="http://schemas.android.com/apk/res/android"

5	    xmlns:aapt="http://schemas.android.com/aapt"

6	    android:width="108dp"

7	    android:height="108dp"

8	    android:viewportWidth="108"

9	    android:viewportHeight="108">

10	    <group android:scaleX="1.1" android:scaleY="1.1" android:translateX="18.8" android:translateY="21">

11	        <path android:pathData="M32,0a12,12 0,1 1,0 24a12,12 0,1 1,0 -24z">

12	            <aapt:attr name="android:fillColor">

13	                <gradient android:type="radial" android:centerX="32" android:centerY="12" android:gradientRadius="12"

14	                    android:startColor="#734FBF6E" android:endColor="#004FBF6E" />

15	            </aapt:attr>

16	        </path>

17	        <path android:pathData="M27.8,12.6a6,6 0,0 1,8.4 0" android:strokeColor="#FF4FBF6E" android:strokeWidth="2.6" android:strokeLineCap="round" />

18	        <path android:pathData="M24.2,8.9a11,11 0,0 1,15.6 0" android:strokeColor="#B34FBF6E" android:strokeWidth="2.6" android:strokeLineCap="round" />

19	        <path android:pathData="M20.6,5.2a16,16 0,0 1,22.8 0" android:strokeColor="#664FBF6E" android:strokeWidth="2.6" android:strokeLineCap="round" />

20	        <path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z">

21	            <aapt:attr name="android:fillColor">

22	                <gradient android:type="linear" android:startX="17.5" android:startY="0" android:endX="46.5" android:endY="0"

23	                    android:startColor="#0D4FBF6E" android:centerColor="#294FBF6E" android:endColor="#0D4FBF6E" />

24	            </aapt:attr>

25	        </path>

26	        <group>

27	            <clip-path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z" />

28	            <path android:pathData="M0,37q5,-3.6 10,0t10,0t10,0t10,0t10,0t10,0t10,0v30H0z" android:fillAlpha="0.35">

29	                <aapt:attr name="android:fillColor">

30	                    <gradient android:type="linear" android:startX="0" android:startY="34" android:endX="0" android:endY="57"

31	                        android:startColor="#FF4FBF6E" android:endColor="#FF3FC7C2" />

32	                </aapt:attr>

33	            </path>

34	            <path android:pathData="M-5,39q5,-3.6 10,0t10,0t10,0t10,0t10,0t10,0t10,0v30H-5z">

35	                <aapt:attr name="android:fillColor">

36	                    <gradient android:type="linear" android:startX="0" android:startY="34" android:endX="0" android:endY="57"

37	                        android:startColor="#FF4FBF6E" android:endColor="#FF3FC7C2" />

38	                </aapt:attr>

39	            </path>

40	        </group>

41	        <path android:pathData="M22,30.5h3.4M22,34h2.2" android:strokeColor="#CC4FBF6E" android:strokeWidth="1.8" android:strokeLineCap="round" />

42	        <path android:pathData="M29.3,42h5.4a1.8,1.8 0,0 1,1.8 1.8v5.4a1.8,1.8 0,0 1,-1.8 1.8h-5.4a1.8,1.8 0,0 1,-1.8 -1.8v-5.4a1.8,1.8 0,0 1,1.8 -1.8z"

43	            android:strokeColor="#FF13100D" android:strokeWidth="2" />

44	        <path android:pathData="M32,42v-3.2M32,51v2.5M27.5,46.5h-4M36.5,46.5h4" android:strokeColor="#FF13100D" android:strokeWidth="1.8" android:strokeLineCap="round" />

45	        <path android:pathData="M30.5,37.6a1.5,1.5 0,1 0,3 0a1.5,1.5 0,1 0,-3 0zM21.1,46.5a1.5,1.5 0,1 0,3 0a1.5,1.5 0,1 0,-3 0zM39.9,46.5a1.5,1.5 0,1 0,3 0a1.5,1.5 0,1 0,-3 0z"

46	            android:fillColor="#FF13100D" />

47	        <path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z" android:strokeColor="#FFEFE7DC" android:strokeWidth="3" android:strokeLineJoin="round" />

48	        <path android:pathData="M13,22.5h38" android:strokeColor="#FFEFE7DC" android:strokeWidth="3.4" android:strokeLineCap="round" />

49	        <path android:pathData="M27,22.5v-3.4h10v3.4" android:strokeColor="#FFEFE7DC" android:strokeWidth="3" android:strokeLineCap="round" android:strokeLineJoin="round" />

50	        <path android:pathData="M44.2,22.5a1.3,1.3 0,1 0,2.6 0a1.3,1.3 0,1 0,-2.6 0z" android:fillColor="#FF4FBF6E" />

51	    </group>

52	</vector>

Android App/app/src/main/res/drawable/ic_launcher_mono.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Themed-icon version of the logo: Android tints it, so it is one colour with the chip cut out. -->

3	<vector xmlns:android="http://schemas.android.com/apk/res/android"

4	    android:width="108dp"

5	    android:height="108dp"

6	    android:viewportWidth="108"

7	    android:viewportHeight="108">

8	    <group android:scaleX="1.1" android:scaleY="1.1" android:translateX="18.8" android:translateY="21">

9	        <path android:pathData="M27.8,12.6a6,6 0,0 1,8.4 0M24.2,8.9a11,11 0,0 1,15.6 0M20.6,5.2a16,16 0,0 1,22.8 0" android:strokeColor="#FF000000" android:strokeWidth="2.6" android:strokeLineCap="round" />

10	        <path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z" android:strokeColor="#FF000000" android:strokeWidth="3" android:strokeLineJoin="round" />

11	        <group>

12	            <clip-path android:pathData="M17.5,26h29l-2.7,27.6a3.2,3.2 0,0 1,-3.2 2.9H23.4a3.2,3.2 0,0 1,-3.2 -2.9z" />

13	            <path android:pathData="M-5,39q5,-3.6 10,0t10,0t10,0t10,0t10,0t10,0t10,0v30H-5z" android:fillColor="#FF000000" android:fillAlpha="0.8" />

14	        </group>

15	        <path android:pathData="M13,22.5h38" android:strokeColor="#FF000000" android:strokeWidth="3.4" android:strokeLineCap="round" />

16	        <path android:pathData="M27,22.5v-3.4h10v3.4" android:strokeColor="#FF000000" android:strokeWidth="3" android:strokeLineCap="round" android:strokeLineJoin="round" />

17	    </group>

18	</vector>

Android App/app/src/main/res/drawable/ic_stat_bin.xml

1	<?xml version="1.0" encoding="utf-8"?>

2	<!-- Status bar icon: the bin with its fill line, one colour as Android requires. -->

3	<vector xmlns:android="http://schemas.android.com/apk/res/android"

4	    android:width="24dp"

5	    android:height="24dp"

6	    android:viewportWidth="24"

7	    android:viewportHeight="24">

8	    <path

9	        android:pathData="M6.6,9.5l1.1,11h8.6l1.1,-11"

10	        android:strokeColor="#FFFFFFFF"

11	        android:strokeWidth="2"

12	        android:strokeLineCap="round"

13	        android:strokeLineJoin="round" />

14	    <path

15	        android:pathData="M4.5,9.5h15M9.6,7.1h4.8"

16	        android:strokeColor="#FFFFFFFF"

17	        android:strokeWidth="2"

18	        android:strokeLineCap="round" />

19	    <path

20	        android:pathData="M8,15.2c1.3,-0.9 2.7,-0.9 4,0s2.7,0.9 4,0"

21	        android:strokeColor="#FFFFFFFF"

22	        android:strokeWidth="1.8"

23	        android:strokeLineCap="round" />

24	</vector>

Part 5. Build and Test Tools

Android App/settings.gradle.kts

1	// ============================================================================

2	// settings.gradle.kts - names the Gradle project ("SmartBin") and its one

3	// module (:app), and says where plugins and libraries are downloaded from.

4	// ============================================================================

5	pluginManagement {

6	    repositories {

7	        google {

8	            content {

9	                includeGroupByRegex("com\\.android.*")

10	                includeGroupByRegex("com\\.google.*")

11	                includeGroupByRegex("androidx.*")

12	            }

13	        }

14	        mavenCentral()

15	        gradlePluginPortal()

16	    }

17	}

18	plugins {

19	    id("org.gradle.toolchains.foojay-resolver-convention") version "1.0.0"

20	}

21	dependencyResolutionManagement {

22	    repositoriesMode.set(RepositoriesMode.FAIL_ON_PROJECT_REPOS)

23	    repositories {

24	        google()

25	        mavenCentral()

26	    }

27	}

28

29	rootProject.name = "SmartBin"

30	include(":app")

Android App/build.gradle.kts

1	// Top-level build file where you can add configuration options common to all sub-projects/modules.

2	// Root build file: declares the Android and Compose plugins once (apply false)

3	// so the :app module can use them without repeating versions.

4	plugins {

5	    alias(libs.plugins.android.application) apply false

6	    alias(libs.plugins.kotlin.compose) apply false

7	}

Android App/app/build.gradle.kts

1	// ============================================================================

2	// app/build.gradle.kts - how the APK is built

3	// applicationId edu.tup.smartbin, Android 8.0 (API 26) and newer. Bump

4	// versionCode and versionName for every release; the GitHub Actions workflow

5	// names the APK after versionName ("Smart-Bin v2.2.apk").

6	// Build from the "Android App" folder with:  ./gradlew :app:assembleDebug

7	// ============================================================================

8	import java.util.Properties

9

10	plugins {

11	    alias(libs.plugins.android.application)

12	    alias(libs.plugins.kotlin.compose)

13	}

14

15	android {

16	    // The code lives in the short package "smartbin" so the source folders stay shallow.

17	    namespace = "smartbin"

18	    compileSdk {

19	        version = release(37)

20	    }

21

22	    defaultConfig {

23	        applicationId = "edu.tup.smartbin"

24	        minSdk = 26

25	        targetSdk = 37

26	        versionCode = 6

27	        versionName = "2.2"

28

29	        // The team's Firebase account, read from local.properties (git-ignored) so the

30	        // password never reaches the public repository. Missing values become empty.

31	        val local = Properties().apply {

32	            rootProject.file("local.properties").takeIf { it.exists() }?.inputStream()?.use(::load)

33	        }

34	        fun prop(key: String) = "\"" + (local.getProperty(key) ?: "").replace("\\", "\\\\").replace("\"", "\\\"") + "\""

35	        buildConfigField("String", "FB_URL", prop("sb.fb.url"))

36	        buildConfigField("String", "FB_KEY", prop("sb.fb.key"))

37	        buildConfigField("String", "FB_EMAIL", prop("sb.fb.email"))

38	        buildConfigField("String", "FB_PASSWORD", prop("sb.fb.password"))

39	    }

40

41	    buildTypes {

42	        release {

43	            optimization {

44	                enable = true

45	                packageScope = setOf("androidx.**", "kotlin.**", "kotlinx.**")

46	            }

47	        }

48	    }

49	    compileOptions {

50	        sourceCompatibility = JavaVersion.VERSION_11

51	        targetCompatibility = JavaVersion.VERSION_11

52	    }

53	    buildFeatures {

54	        compose = true

55	        buildConfig = true

56	    }

57	}

58

59	dependencies {

60	    implementation(platform(libs.androidx.compose.bom))

61	    implementation(libs.androidx.activity.compose)

62	    implementation(libs.androidx.compose.material3)

63	    implementation(libs.androidx.compose.ui)

64	    implementation(libs.androidx.compose.ui.graphics)

65	    implementation(libs.androidx.compose.ui.tooling.preview)

66	    implementation(libs.androidx.core.ktx)

67	    implementation(libs.androidx.lifecycle.runtime.ktx)

68	    implementation("androidx.lifecycle:lifecycle-viewmodel-compose:2.8.7")

69	    debugImplementation(libs.androidx.compose.ui.tooling)

70	}

Android App/gradle.properties

1	# ----------------------------------------------------------------------------

2	# gradle.properties - Gradle settings shared by every build of the app.

3	# If a build runs out of memory, raise -Xmx below.

4	# ----------------------------------------------------------------------------

5	# Project-wide Gradle settings.

6	# IDE (e.g. Android Studio) users:

7	# Gradle settings configured through the IDE *will override*

8	# any settings specified in this file.

9	# For more details on how to configure your build environment visit

10	# http://www.gradle.org/docs/current/userguide/build_environment.html

11	# Specifies the JVM arguments used for the daemon process.

12	# The setting is particularly useful for tweaking memory settings.

13	org.gradle.jvmargs=-Xmx2048m -Dfile.encoding=UTF-8

14	# When configured, Gradle will run in incubating parallel mode.

15	# This option should only be used with decoupled projects. For more details, visit

16	# https://developer.android.com/r/tools/gradle-multi-project-decoupled-projects

17	# org.gradle.parallel=true

18	# When enabled, the Configuration Cache allows Gradle to skip the configuration

19	# phase entirely if nothing that affects the build configuration (such as build scripts)

20	# has changed. Additionally, Gradle applies performance optimizations to task execution.

21	org.gradle.configuration-cache=true

22	# Kotlin code style for this project: "official" or "obsolete":

23	kotlin.code.style=official

Android App/gradle/libs.versions.toml

1	# ----------------------------------------------------------------------------

2	# libs.versions.toml - Gradle version catalog: one place for the version of

3	# every library and plugin. build.gradle.kts refers to them as libs.<name>.

4	# ----------------------------------------------------------------------------

5	[versions]

6	agp = "9.4.1"

7	coreKtx = "1.10.1"

8	junit = "4.13.2"

9	junitVersion = "1.1.5"

10	espressoCore = "3.5.1"

11	lifecycleRuntimeKtx = "2.6.1"

12	activityCompose = "1.8.0"

13	kotlin = "2.2.10"

14	composeBom = "2026.02.01"

15

16	[libraries]

17	androidx-core-ktx = { group = "androidx.core", name = "core-ktx", version.ref = "coreKtx" }

18	junit = { group = "junit", name = "junit", version.ref = "junit" }

19	androidx-junit = { group = "androidx.test.ext", name = "junit", version.ref = "junitVersion" }

20	androidx-espresso-core = { group = "androidx.test.espresso", name = "espresso-core", version.ref = "espressoCore" }

21	androidx-lifecycle-runtime-ktx = { group = "androidx.lifecycle", name = "lifecycle-runtime-ktx", version.ref = "lifecycleRuntimeKtx" }

22	androidx-activity-compose = { group = "androidx.activity", name = "activity-compose", version.ref = "activityCompose" }

23	androidx-compose-bom = { group = "androidx.compose", name = "compose-bom", version.ref = "composeBom" }

24	androidx-compose-ui = { group = "androidx.compose.ui", name = "ui" }

25	androidx-compose-ui-graphics = { group = "androidx.compose.ui", name = "ui-graphics" }

26	androidx-compose-ui-tooling = { group = "androidx.compose.ui", name = "ui-tooling" }

27	androidx-compose-ui-tooling-preview = { group = "androidx.compose.ui", name = "ui-tooling-preview" }

28	androidx-compose-ui-test-manifest = { group = "androidx.compose.ui", name = "ui-test-manifest" }

29	androidx-compose-ui-test-junit4 = { group = "androidx.compose.ui", name = "ui-test-junit4" }

30	androidx-compose-material3 = { group = "androidx.compose.material3", name = "material3" }

31

32	[plugins]

33	android-application = { id = "com.android.application", version.ref = "agp" }

34	kotlin-compose = { id = "org.jetbrains.kotlin.plugin.compose", version.ref = "kotlin" }

Android App/gradle/wrapper/gradle-wrapper.properties

1	# gradle-wrapper.properties - the Gradle version gradlew downloads (9.6.0) and

2	# the checksum it verifies before running it.

3	#Sat Oct 10 09:56:41 PST 2026

4	distributionBase=GRADLE_USER_HOME

5	distributionPath=wrapper/dists

6	distributionSha256Sum=bbaeb2fef8710818cf0e261201dab964c572f92b942812df0c3620d62a529a01

7	distributionUrl=https\://services.gradle.org/distributions/gradle-9.6.0-bin.zip

8	networkTimeout=10000

9	validateDistributionUrl=true

10	zipStoreBase=GRADLE_USER_HOME

11	zipStorePath=wrapper/dists

Android App/gradle/gradle-daemon-jvm.properties

1	# gradle-daemon-jvm.properties - the Java version (25) Gradle runs on, with a

2	# download link per operating system. Generated; do not edit by hand.

3	#This file is generated by updateDaemonJvm

4	toolchainUrl.FREE_BSD.AARCH64=https\://api.foojay.io/disco/v3.0/ids/cf726b4a1c84b50457225f9bba6d7650/redirect

5	toolchainUrl.FREE_BSD.X86_64=https\://api.foojay.io/disco/v3.0/ids/fa1e318c287360478e3c83a9a3ef1007/redirect

6	toolchainUrl.LINUX.AARCH64=https\://api.foojay.io/disco/v3.0/ids/cf726b4a1c84b50457225f9bba6d7650/redirect

7	toolchainUrl.LINUX.X86_64=https\://api.foojay.io/disco/v3.0/ids/fa1e318c287360478e3c83a9a3ef1007/redirect

8	toolchainUrl.MAC_OS.AARCH64=https\://api.foojay.io/disco/v3.0/ids/c2dd35c9d0aaf0ba6ad0791320f99dfc/redirect

9	toolchainUrl.MAC_OS.X86_64=https\://api.foojay.io/disco/v3.0/ids/e5810bd7fd1f8a586644409d395a7e55/redirect

10	toolchainUrl.UNIX.AARCH64=https\://api.foojay.io/disco/v3.0/ids/cf726b4a1c84b50457225f9bba6d7650/redirect

11	toolchainUrl.UNIX.X86_64=https\://api.foojay.io/disco/v3.0/ids/fa1e318c287360478e3c83a9a3ef1007/redirect

12	toolchainUrl.WINDOWS.AARCH64=https\://api.foojay.io/disco/v3.0/ids/7b3c4877c0749019e6805bb61e421497/redirect

13	toolchainUrl.WINDOWS.X86_64=https\://api.foojay.io/disco/v3.0/ids/d76df094a9cbbabd3b08251f9e61444a/redirect

14	toolchainVersion=25

Android App/gradlew

1	#!/bin/sh

2

3	#

4	# gradlew - Gradle wrapper start script for Linux and macOS, generated by Gradle

5	# (Apache License 2.0). It downloads the Gradle version named in

6	# gradle-wrapper.properties on first use, then runs it.

7	# Copyright © 2015 the original authors.

8	#

9	# Licensed under the Apache License, Version 2.0 (the "License");

10	# you may not use this file except in compliance with the License.

11	# You may obtain a copy of the License at

12	#

13	#      https://www.apache.org/licenses/LICENSE-2.0

14	#

15	# Unless required by applicable law or agreed to in writing, software

16	# distributed under the License is distributed on an "AS IS" BASIS,

17	# WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

18	# See the License for the specific language governing permissions and

19	# limitations under the License.

20	#

21	# SPDX-License-Identifier: Apache-2.0

22	#

23

24	##############################################################################

25	#

26	#   Gradle start up script for POSIX generated by Gradle.

27	#

28	#   Important for running:

29	#

30	#   (1) You need a POSIX-compliant shell to run this script. If your /bin/sh is

31	#       noncompliant, but you have some other compliant shell such as ksh or

32	#       bash, then to run this script, type that shell name before the whole

33	#       command line, like:

34	#

35	#           ksh Gradle

36	#

37	#       Busybox and similar reduced shells will NOT work, because this script

38	#       requires all of these POSIX shell features:

39	#         * functions;

40	#         * expansions «$var», «${var}», «${var:-default}», «${var+SET}»,

41	#           «${var#prefix}», «${var%suffix}», and «$( cmd )»;

42	#         * compound commands having a testable exit status, especially «case»;

43	#         * various built-in commands including «command», «set», and «ulimit».

44	#

45	#   Important for patching:

46	#

47	#   (2) This script targets any POSIX shell, so it avoids extensions provided

48	#       by Bash, Ksh, etc; in particular arrays are avoided.

49	#

50	#       The "traditional" practice of packing multiple parameters into a

51	#       space-separated string is a well documented source of bugs and security

52	#       problems, so this is (mostly) avoided, by progressively accumulating

53	#       options in "$@", and eventually passing that to Java.

54	#

55	#       Where the inherited environment variables (DEFAULT_JVM_OPTS, JAVA_OPTS,

56	#       and GRADLE_OPTS) rely on word-splitting, this is performed explicitly;

57	#       see the in-line comments for details.

58	#

59	#       There are tweaks for specific operating systems such as AIX, CygWin,

60	#       Darwin, MinGW, and NonStop.

61	#

62	#   (3) This script is generated from the Groovy template

63	#       https://github.com/gradle/gradle/blob/HEAD/platforms/jvm/plugins-application/src/main/resources/org/gradle/api/internal/plugins/unixStartScript.txt

64	#       within the Gradle project.

65	#

66	#       You can find Gradle at https://github.com/gradle/gradle/.

67	#

68	##############################################################################

69

70	# Attempt to set APP_HOME

71

72	# Resolve links: $0 may be a link

73	app_path=$0

74

75	# Need this for daisy-chained symlinks.

76	while

77	    APP_HOME=${app_path%"${app_path##*/}"}  # leaves a trailing /; empty if no leading path

78	    [ -h "$app_path" ]

79	do

80	    ls=$( ls -ld "$app_path" )

81	    link=${ls#*' -> '}

82	    case $link in             #(

83	      /*)   app_path=$link ;; #(

84	      *)    app_path=$APP_HOME$link ;;

85	    esac

86	done

87

88	# This is normally unused

89	# shellcheck disable=SC2034

90	APP_BASE_NAME=${0##*/}

91	# Discard cd standard output in case $CDPATH is set (https://github.com/gradle/gradle/issues/25036)

92	APP_HOME=$( cd -P "${APP_HOME:-./}" > /dev/null && printf '%s\n' "$PWD" ) || exit

93

94	# Use the maximum available, or set MAX_FD != -1 to use that value.

95	MAX_FD=maximum

96

97	warn () {

98	    echo "$*"

99	} >&2

100

101	die () {

102	    echo

103	    echo "$*"

104	    echo

105	    exit 1

106	} >&2

107

108	# OS specific support (must be 'true' or 'false').

109	cygwin=false

110	msys=false

111	darwin=false

112	nonstop=false

113	case "$( uname )" in                #(

114	  CYGWIN* )         cygwin=true  ;; #(

115	  Darwin* )         darwin=true  ;; #(

116	  MSYS* | MINGW* )  msys=true    ;; #(

117	  NONSTOP* )        nonstop=true ;;

118	esac

119

120	CLASSPATH="\\\"\\\""

121

122

123	# Determine the Java command to use to start the JVM.

124	if [ -n "$JAVA_HOME" ] ; then

125	    if [ -x "$JAVA_HOME/jre/sh/java" ] ; then

126	        # IBM's JDK on AIX uses strange locations for the executables

127	        JAVACMD=$JAVA_HOME/jre/sh/java

128	    else

129	        JAVACMD=$JAVA_HOME/bin/java

130	    fi

131	    if [ ! -x "$JAVACMD" ] ; then

132	        die "ERROR: JAVA_HOME is set to an invalid directory: $JAVA_HOME

133

134	Please set the JAVA_HOME variable in your environment to match the

135	location of your Java installation."

136	    fi

137	else

138	    JAVACMD=java

139	    if ! command -v java >/dev/null 2>&1

140	    then

141	        die "ERROR: JAVA_HOME is not set and no 'java' command could be found in your PATH.

142

143	Please set the JAVA_HOME variable in your environment to match the

144	location of your Java installation."

145	    fi

146	fi

147

148	# Increase the maximum file descriptors if we can.

149	if ! "$cygwin" && ! "$darwin" && ! "$nonstop" ; then

150	    case $MAX_FD in #(

151	      max*)

152	        # In POSIX sh, ulimit -H is undefined. That's why the result is checked to see if it worked.

153	        # shellcheck disable=SC2039,SC3045

154	        MAX_FD=$( ulimit -H -n ) ||

155	            warn "Could not query maximum file descriptor limit"

156	    esac

157	    case $MAX_FD in  #(

158	      '' | soft) :;; #(

159	      *)

160	        # In POSIX sh, ulimit -n is undefined. That's why the result is checked to see if it worked.

161	        # shellcheck disable=SC2039,SC3045

162	        ulimit -n "$MAX_FD" ||

163	            warn "Could not set maximum file descriptor limit to $MAX_FD"

164	    esac

165	fi

166

167	# Collect all arguments for the java command, stacking in reverse order:

168	#   * args from the command line

169	#   * the main class name

170	#   * -classpath

171	#   * -D...appname settings

172	#   * --module-path (only if needed)

173	#   * DEFAULT_JVM_OPTS, JAVA_OPTS, and GRADLE_OPTS environment variables.

174

175	# For Cygwin or MSYS, switch paths to Windows format before running java

176	if "$cygwin" || "$msys" ; then

177	    APP_HOME=$( cygpath --path --mixed "$APP_HOME" )

178	    CLASSPATH=$( cygpath --path --mixed "$CLASSPATH" )

179

180	    JAVACMD=$( cygpath --unix "$JAVACMD" )

181

182	    # Now convert the arguments - kludge to limit ourselves to /bin/sh

183	    for arg do

184	        if

185	            case $arg in                                #(

186	              -*)   false ;;                            # don't mess with options #(

187	              /?*)  t=${arg#/} t=/${t%%/*}              # looks like a POSIX filepath

188	                    [ -e "$t" ] ;;                      #(

189	              *)    false ;;

190	            esac

191	        then

192	            arg=$( cygpath --path --ignore --mixed "$arg" )

193	        fi

194	        # Roll the args list around exactly as many times as the number of

195	        # args, so each arg winds up back in the position where it started, but

196	        # possibly modified.

197	        #

198	        # NB: a `for` loop captures its iteration list before it begins, so

199	        # changing the positional parameters here affects neither the number of

200	        # iterations, nor the values presented in `arg`.

201	        shift                   # remove old arg

202	        set -- "$@" "$arg"      # push replacement arg

203	    done

204	fi

205

206

207	# Add default JVM options here. You can also use JAVA_OPTS and GRADLE_OPTS to pass JVM options to this script.

208	DEFAULT_JVM_OPTS='"-Xmx64m" "-Xms64m"'

209

210	# Collect all arguments for the java command:

211	#   * DEFAULT_JVM_OPTS, JAVA_OPTS, and optsEnvironmentVar are not allowed to contain shell fragments,

212	#     and any embedded shellness will be escaped.

213	#   * For example: A user cannot expect ${Hostname} to be expanded, as it is an environment variable and will be

214	#     treated as '${Hostname}' itself on the command line.

215

216	set -- \

217	        "-Dorg.gradle.appname=$APP_BASE_NAME" \

218	        -classpath "$CLASSPATH" \

219	        -jar "$APP_HOME/gradle/wrapper/gradle-wrapper.jar" \

220	        "$@"

221

222	# Stop when "xargs" is not available.

223	if ! command -v xargs >/dev/null 2>&1

224	then

225	    die "xargs is not available"

226	fi

227

228	# Use "xargs" to parse quoted args.

229	#

230	# With -n1 it outputs one arg per line, with the quotes and backslashes removed.

231	#

232	# In Bash we could simply go:

233	#

234	#   readarray ARGS < <( xargs -n1 <<<"$var" ) &&

235	#   set -- "${ARGS[@]}" "$@"

236	#

237	# but POSIX shell has neither arrays nor command substitution, so instead we

238	# post-process each arg (as a line of input to sed) to backslash-escape any

239	# character that might be a shell metacharacter, then use eval to reverse

240	# that process (while maintaining the separation between arguments), and wrap

241	# the whole thing up as a single "set" statement.

242	#

243	# This will of course break if any of these variables contains a newline or

244	# an unmatched quote.

245	#

246

247	eval "set -- $(

248	        printf '%s\n' "$DEFAULT_JVM_OPTS $JAVA_OPTS $GRADLE_OPTS" |

249	        xargs -n1 |

250	        sed ' s~[^-[:alnum:]+,./:=@_]~\\&~g; ' |

251	        tr '\n' ' '

252	    )" '"$@"'

253

254	exec "$JAVACMD" "$@"

Android App/gradlew.bat

1	@rem gradlew.bat - the same Gradle wrapper start script for Windows, generated by Gradle.

2	@rem

3	@rem Copyright 2015 the original author or authors.

4	@rem

5	@rem Licensed under the Apache License, Version 2.0 (the "License");

6	@rem you may not use this file except in compliance with the License.

7	@rem You may obtain a copy of the License at

8	@rem

9	@rem      https://www.apache.org/licenses/LICENSE-2.0

10	@rem

11	@rem Unless required by applicable law or agreed to in writing, software

12	@rem distributed under the License is distributed on an "AS IS" BASIS,

13	@rem WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

14	@rem See the License for the specific language governing permissions and

15	@rem limitations under the License.

16	@rem

17	@rem SPDX-License-Identifier: Apache-2.0

18	@rem

19

20	@if "%DEBUG%"=="" @echo off

21	@rem ##########################################################################

22	@rem

23	@rem  Gradle startup script for Windows

24	@rem

25	@rem ##########################################################################

26

27	@rem Set local scope for the variables with windows NT shell

28	if "%OS%"=="Windows_NT" setlocal

29

30	set DIRNAME=%~dp0

31	if "%DIRNAME%"=="" set DIRNAME=.

32	@rem This is normally unused

33	set APP_BASE_NAME=%~n0

34	set APP_HOME=%DIRNAME%

35

36	@rem Resolve any "." and ".." in APP_HOME to make it shorter.

37	for %%i in ("%APP_HOME%") do set APP_HOME=%%~fi

38

39	@rem Add default JVM options here. You can also use JAVA_OPTS and GRADLE_OPTS to pass JVM options to this script.

40	set DEFAULT_JVM_OPTS="-Xmx64m" "-Xms64m"

41

42	@rem Find java.exe

43	if defined JAVA_HOME goto findJavaFromJavaHome

44

45	set JAVA_EXE=java.exe

46	%JAVA_EXE% -version >NUL 2>&1

47	if %ERRORLEVEL% equ 0 goto execute

48

49	echo. 1>&2

50	echo ERROR: JAVA_HOME is not set and no 'java' command could be found in your PATH. 1>&2

51	echo. 1>&2

52	echo Please set the JAVA_HOME variable in your environment to match the 1>&2

53	echo location of your Java installation. 1>&2

54

55	goto fail

56

57	:findJavaFromJavaHome

58	set JAVA_HOME=%JAVA_HOME:"=%

59	set JAVA_EXE=%JAVA_HOME%/bin/java.exe

60

61	if exist "%JAVA_EXE%" goto execute

62

63	echo. 1>&2

64	echo ERROR: JAVA_HOME is set to an invalid directory: %JAVA_HOME% 1>&2

65	echo. 1>&2

66	echo Please set the JAVA_HOME variable in your environment to match the 1>&2

67	echo location of your Java installation. 1>&2

68

69	goto fail

70

71	:execute

72	@rem Setup the command line

73

74	set CLASSPATH=

75

76

77	@rem Execute Gradle

78	"%JAVA_EXE%" %DEFAULT_JVM_OPTS% %JAVA_OPTS% %GRADLE_OPTS% "-Dorg.gradle.appname=%APP_BASE_NAME%" -classpath "%CLASSPATH%" -jar "%APP_HOME%\gradle\wrapper\gradle-wrapper.jar" %*

79

80	:end

81	@rem End local scope for the variables with windows NT shell

82	if %ERRORLEVEL% equ 0 goto mainEnd

83

84	:fail

85	rem Set variable GRADLE_EXIT_CONSOLE if you need the _script_ return code instead of

86	rem the _cmd.exe /c_ return code!

87	set EXIT_CODE=%ERRORLEVEL%

88	if %EXIT_CODE% equ 0 set EXIT_CODE=1

89	if not ""=="%GRADLE_EXIT_CONSOLE%" exit %EXIT_CODE%

90	exit /b %EXIT_CODE%

91

92	:mainEnd

93	if "%OS%"=="Windows_NT" endlocal

94

95	:omega

.github/workflows/android-apk.yml

1	# Builds the Android app and puts the debug APK in "Android App/Smart-Bin vX.Y.apk",

2	# replacing the older one, so the APK can be made without a laptop.

3	# The Firebase account is never baked in: local.properties holds only sdk.dir here.

4	# Debug tip: if the APK is not updated after a push, open the Actions tab on

5	# GitHub; a failed "Build" step shows the Gradle error in its log.

6	name: Android APK

7

8	on:

9	  workflow_dispatch:

10	  push:

11	    branches: ['claude/**']

12	    paths: ['Android App/app/**', 'Android App/*.kts', 'Android App/gradle/**', '.github/workflows/android-apk.yml']

13

14	permissions:

15	  contents: write

16

17	concurrency:

18	  group: apk-${{ github.ref }}

19	  cancel-in-progress: true

20

21	jobs:

22	  build:

23	    runs-on: ubuntu-latest

24	    defaults:

25	      run:

26	        working-directory: Android App

27	    steps:

28	      - uses: actions/checkout@v4

29	      - uses: actions/setup-java@v4

30	        with:

31	          distribution: temurin

32	          java-version: '21'

33	      # The runner image already has the Android SDK; AGP fetches any missing platform.

34	      - name: Point Gradle at the SDK

35	        run: |

36	          sdk="${ANDROID_HOME:-/usr/local/lib/android/sdk}"

37	          yes | "$sdk/cmdline-tools/latest/bin/sdkmanager" --licenses > /dev/null || true

38	          echo "sdk.dir=$sdk" > local.properties

39	      - name: Build

40	        run: |

41	          chmod +x gradlew

42	          ./gradlew :app:assembleDebug --no-configuration-cache --stacktrace

43	      - name: Put the APK in the repo

44	        run: |

45	          ver=$(sed -n 's/.*versionName = "\(.*\)".*/\1/p' app/build.gradle.kts | head -1)

46	          name="Smart-Bin v$ver.apk"

47	          git rm -q --ignore-unmatch Smart-Bin\ v*.apk

48	          cp app/build/outputs/apk/debug/app-debug.apk "$name"

49	          git add -f "$name"

50	          git config user.name "github-actions[bot]"

51	          git config user.email "41898282+github-actions[bot]@users.noreply.github.com"

52	          if git diff --cached --quiet; then echo "No change"; exit 0; fi

53	          git commit -q -m "Build $name"

54	          git push origin HEAD:${GITHUB_REF_NAME}

55	      - uses: actions/upload-artifact@v4

56	        with:

57	          name: smart-bin-apk

58	          path: Android App/app/build/outputs/apk/debug/app-debug.apk

Android App/tools/make_sounds.py

1	# ============================================================================

2	# make_sounds.py - generates every WAV in app/src/main/res/raw (development tool)

3	# Each sound is a list of notes rendered as sine waves with a 2nd and 3rd

4	# harmonic (the "timbre"), a short attack and release, and an exponential

5	# fade. Output is 22,050 Hz, 16-bit mono. Rerun it after editing SOUNDS, then

6	# rebuild the app so the new files are packed into the APK.

7	# ============================================================================

8	"""Builds the Smart-Bin app's sounds into app/src/main/res/raw.

9

10	Every action in the app has its own short tune, and every kind of alert has

11	a longer one, so the phone can be told apart by ear. The rules follow the

12	Garbage-Guard app: rising pitch means forward, on, open or OK; falling pitch

13	means back, off, close or no. Button sounds are quiet, events louder, and

14	alerts loudest with a buzzier tone.

15

16	The SOUNDS table is plain data, so the ESP32 buzzer can later play the same

17	tunes for the same actions and the bin and the phone will sound alike.

18

19	Run from anywhere after changing the table:

20	    python3 "Android App/tools/make_sounds.py"

21	"""

22	import math

23	import struct

24	import wave

25	from pathlib import Path

26

27	OUT = Path(__file__).resolve().parents[1] / "app" / "src" / "main" / "res" / "raw"

28	RATE = 22050

29	ATTACK = 0.004    # seconds

30	RELEASE = 0.014   # seconds

31

32	# name: (group, notes). A note is (hz, ms), (start_hz, end_hz, ms) for a sweep, or (0, ms) for a rest.

33	SOUNDS = {

34	    # Keys and switches.

35	    "tap":         ("ui", [(1320, 22)]),

36	    "choice":      ("ui", [(1180, 16), (1480, 18)]),

37	    "switch_on":   ("ui", [(880, 40), (1320, 55)]),

38	    "switch_off":  ("ui", [(1320, 40), (880, 55)]),

39	    "step_up":     ("ui", [(1400, 24)]),

40	    "step_down":   ("ui", [(1100, 24)]),

41	    "tab":         ("ui", [(990, 26), (0, 10), (1245, 30)]),

42	    "open":        ("ui", [(740, 35), (990, 50)]),

43	    "back":        ("ui", [(990, 35), (740, 50)]),

44	    "confirm":     ("ui", [(880, 50), (1175, 50), (1760, 90)]),

45	    "danger":      ("ui", [(440, 90), (0, 30), (440, 90)]),

46	    "denied":      ("ui", [(392, 70), (0, 40), (330, 120)]),

47	    "logo":        ("ui", [(784, 70), (988, 70), (1175, 70), (1568, 170)]),

48	    "theme_light": ("ui", [(1175, 45), (1568, 75)]),

49	    "theme_dark":  ("ui", [(784, 45), (587, 75)]),

50	    # Things that happen to a bin or to the link.

51	    "boot":        ("event", [(523, 80), (659, 80), (784, 80), (1047, 220)]),

52	    "saved":       ("event", [(1047, 60), (1319, 100)]),

53	    "link_up":     ("event", [(659, 70), (880, 70), (1319, 150)]),

54	    "link_lost":   ("event", [(880, 90), (659, 90), (440, 190)]),

55	    "cloud":       ("event", [(880, 60), (0, 30), (1047, 60), (0, 30), (784, 130)]),

56	    "lock":        ("event", [(660, 40), (0, 20), (440, 120)]),

57	    "unlock":      ("event", [(440, 40), (0, 20), (660, 120)]),

58	    "pickup":      ("event", [(523, 70), (659, 70), (784, 70), (659, 70), (784, 150)]),

59	    "done":        ("event", [(784, 80), (1047, 80), (1319, 80), (1568, 230)]),

60	    "calib":       ("event", [(988, 40), (0, 50), (988, 40), (0, 50), (988, 40), (0, 50), (1319, 130)]),

61	    "sim":         ("event", [(523, 60), (622, 60), (740, 60), (880, 140)]),

62	    "bin_lost":    ("event", [(740, 100), (0, 60), (554, 210)]),

63	    "bin_back":    ("event", [(554, 80), (740, 150)]),

64	    # The operator PIN: a dry tick per digit, a key-turn into operator, and a

65	    # falling bolt back to collector. A wrong PIN buzzes low; a lockout drops further.

66	    "pin_key":     ("ui", [(1660, 14)]),

67	    "pin_wrong":   ("event", [(349, 60), (0, 30), (349, 60), (0, 30), (262, 160)]),

68	    "pin_lockout": ("event", [(523, 260, 260), (0, 60), (196, 300)]),

69	    "op_in":       ("event", [(587, 50), (0, 25), (880, 50), (1175, 60), (1760, 160)]),

70	    "op_out":      ("event", [(1175, 50), (880, 50), (0, 25), (587, 140)]),

71	    # A bin confirming a command or not, Bluetooth pairing, the bin joining

72	    # Wi-Fi, and a new operator PIN: each its own, so none borrows another's.

73	    "cmd_ok":      ("event", [(1319, 50), (0, 25), (1760, 110)]),

74	    "cmd_fail":    ("event", [(659, 80), (0, 30), (523, 80), (0, 30), (392, 200)]),

75	    "bt_paired":   ("event", [(1175, 40), (1568, 40), (1175, 40), (1976, 140)]),

76	    "bt_wrong":    ("event", [(466, 50), (0, 25), (466, 50), (370, 140)]),

77	    "bt_blocked":  ("event", [(622, 180), (0, 50), (311, 320)]),

78	    "wifi_joined": ("event", [(880, 60), (1175, 60), (1480, 60), (1760, 180)]),

79	    "pin_set":     ("event", [(1047, 50), (1397, 50), (1047, 50), (1397, 120)]),

80	    # v2: the bin could not join the Wi-Fi, Bluetooth switched off on the bin,

81	    # and demo mode turned on at the start page.

82	    "wifi_failed": ("event", [(740, 60), (0, 25), (554, 60), (0, 25), (440, 90), (370, 180)]),

83	    "bt_off":      ("event", [(1568, 35), (1175, 35), (784, 35), (523, 150)]),

84	    "demo":        ("event", [(659, 60), (784, 60), (988, 60), (784, 60), (1319, 160)]),

85	    # Notification sounds, one per kind of alert. They also play inside the app.

86	    "alert_full":   ("alert", [(880, 180), (0, 80), (880, 180), (0, 80), (880, 180), (0, 200), (1175, 420)]),

87	    "alert_tamper": ("alert", [(700, 1400, 340), (1400, 700, 340), (700, 1400, 340), (1400, 700, 340)]),

88	    "alert_sensor": ("alert", [(1047, 120), (784, 120), (1047, 120), (784, 260)]),

89	    "alert_lock":   ("alert", [(660, 70), (0, 30), (440, 220)]),

90	    "alert_pickup": ("alert", [(523, 90), (659, 90), (784, 90), (1047, 260)]),

91	    "alert_info":   ("alert", [(988, 90), (1319, 170)]),

92	}

93

94	LEVEL = {"ui": 0.5, "event": 0.72, "alert": 0.85}

95	# How much of the 2nd and 3rd harmonic each group carries. Alerts buzz a little, keys stay soft.

96	TIMBRE = {"ui": (0.18, 0.0), "event": (0.22, 0.05), "alert": (0.35, 0.22)}

97

98

99	# One note or sweep (hz0 to hz1) as a list of samples between -1 and 1.

100	def note(hz0, hz1, secs, level, timbre):

101	    n = int(RATE * secs)

102	    h2, h3 = timbre

103	    norm = 1 + h2 + h3

104	    out = []

105	    phase = 0.0

106	    for i in range(n):

107	        t = i / RATE

108	        env = min(1.0, t / ATTACK) * min(1.0, (secs - t) / RELEASE)

109	        env *= math.exp(-1.6 * t / max(secs, 0.05))

110	        hz = hz0 + (hz1 - hz0) * (i / max(n - 1, 1))

111	        phase += 2 * math.pi * hz / RATE

112	        s = math.sin(phase) + h2 * math.sin(2 * phase) + h3 * math.sin(3 * phase)

113	        out.append(level * env * s / norm)

114	    return out

115

116

117	# A whole tune: notes, sweeps and rests (hz 0) joined, plus 12 ms of silence.

118	def render(group, notes):

119	    level, timbre = LEVEL[group], TIMBRE[group]

120	    samples = []

121	    for n in notes:

122	        if len(n) == 3:

123	            samples += note(n[0], n[1], n[2] / 1000.0, level, timbre)

124	        elif n[0] == 0:

125	            samples += [0.0] * int(RATE * n[1] / 1000.0)

126	        else:

127	            samples += note(n[0], n[0], n[1] / 1000.0, level, timbre)

128	    return samples + [0.0] * int(RATE * 0.012)

129

130

131	def write(path, samples):

132	    with wave.open(str(path), "wb") as w:

133	        w.setnchannels(1)

134	        w.setsampwidth(2)

135	        w.setframerate(RATE)

136	        w.writeframes(b"".join(struct.pack("<h", int(max(-1, min(1, s)) * 32767)) for s in samples))

137

138

139	def main():

140	    OUT.mkdir(parents=True, exist_ok=True)

141	    for name, (group, notes) in SOUNDS.items():

142	        write(OUT / f"sb_{name}.wav", render(group, notes))

143	    print(f"wrote {len(SOUNDS)} sounds to {OUT}")

144

145

146	if __name__ == "__main__":

147	    main()

Android App/tools/smartbin_mock_server.py

1	#!/usr/bin/env python3

2	# ============================================================================

3	# smartbin_mock_server.py - fake bins, Firebase and ThingSpeak for testing

4	# (development tool, not used in the finished system)

5	# Lets the app and dashboard be tested with no ESP32. Each fake bin follows

6	# the same rules as the firmware (fill rises, tilt over tilt_th means TAMPER,

7	# simulation mask bits override readings), so most app bugs can be reproduced

8	# here first.

9	# ============================================================================

10	"""

11	Stand-in Smart-Bin bins for testing the Android app with no hardware.

12

13	Each bin answers the same local REST API the ESP32 firmware will serve:

14

15	    GET  /api/status     readings, status, calibration and simulation mask

16	    POST /api/command    {"cmd": "LOCK"} and the other commands

17	    POST /api/calibrate  calibration values

18	    POST /api/simulate   {"mask": 5, "values": {...}}

19

20	Optionally it also plays a small Firebase Realtime Database on another port,

21	with the same paths the app uses for cloud mode:

22

23	    /sb/bins/<id>/status.json    GET   the bin's last status, ts and local_ip

24	    /sb/commands/<id>.json       POST  LOCK, UNLOCK or COLLECT

25	    /sb/usage/bytes_month.json   GET

26	    /sb_users/<uid>.json         GET   the allowed-user check

27	    /__auth/signInWithPassword   POST  sign-in, any email and a password of 6+ characters

28

29	and a small ThingSpeak on a third port, with --ts PORT:

30

31	    /channels/<id>/feeds/last.json   GET   the newest entry, fields 1 to 8 and the status text

32	    /talkbacks/<id>/commands.json    POST  command_string=LOCK, UNLOCK or COLLECT

33	    Bin 1 is channel 1001 and TalkBack 2001, bin 2 is 1002 and 2002, and so on. Any key works.

34

35	Plain Python 3, no packages. Run it on a computer on the same Wi-Fi as the phone:

36

37	    python smartbin_mock_server.py                 two bins on ports 8081 and 8082

38	    python smartbin_mock_server.py --bins 3        three bins, 8081 to 8083

39	    python smartbin_mock_server.py --cloud 8090    also the fake Firebase on 8090

40	    python smartbin_mock_server.py --ts 8095       also the fake ThingSpeak on 8095

41

42	In the app, open a bin with SET and type <computer IP>:8081 as its address.

43	For cloud mode, use http://<computer IP>:8090 as the database URL, any API

44	key, any email and a 6+ character password, then stop the bins with

45	--no-local so only the cloud answers. For ThingSpeak, put http://<computer IP>:8095

46	as the server on the Connection page, and channel 1001, TalkBack 2001 (any keys)

47	in bin 1's SET sheet.

48	"""

49

50	import argparse

51	import json

52	import random

53	import socket

54	import threading

55	import time

56	from http.server import BaseHTTPRequestHandler, ThreadingHTTPServer

57	from urllib.parse import parse_qs, urlparse

58

59	DEFAULT_CAL = {

60	    "d_empty": 30, "d_full": 5, "fill_th": 80, "temp_th": 40, "hum_th": 85, "gas_th": 200,

61	    "tilt_th": 30, "weight_cap": 4500, "auto_lock": 0, "servo_closed": 0, "servo_open": 90,

62	    "collect_pin": 1234, "pir_hold_s": 5, "buzz_s": 2, "temp_off": -2, "hum_off": 0, "press_off": 0,

63	}

64	DEFAULT_SIM = {"fill": 60, "weight": 2000, "gas": 120, "temp": 28, "humidity": 60, "tamper": 0, "motion": 0, "lid": 0}

65

66

67	# One fake bin with its own port, readings, calibration and simulation.

68	class Bin:

69	    """One bin, stepped once a second the same way the app's demo mode steps it."""

70

71	    def __init__(self, index, port):

72	        self.id = "bin%02d" % (index + 1)

73	        self.port = port

74	        self.lock = threading.Lock()

75	        self.cal = dict(DEFAULT_CAL)

76	        self.sim_mask = 0

77	        self.sim = dict(DEFAULT_SIM)

78	        self.fill = [34.0, 62.0, 18.0][index % 3]

79	        self.rate = [0.38, 3.1, 0.9][index % 3] * 60  # percent per hour, sped up 60 times

80	        self.temp_raw = 30.0

81	        self.hum = 62.0

82	        self.press = 1008.4

83	        self.gas = 60 + self.fill

84	        self.iaq_acc = 3

85	        self.tilt = 0.0

86	        self.motion = False

87	        self.lid_open = False

88	        self.status = "OK"

89	        self.calibrating = False

90

91	    def on(self, bit):

92	        return (self.sim_mask >> bit) & 1 == 1

93

94	    # Called about once a second: moves the readings and works out the

95	    # status the same way control.cpp does.

96	    def step(self):

97	        with self.lock:

98	            c, s = self.cal, self.sim

99	            if self.calibrating:

100	                self.status = "CALIBRATING"

101	                self.lid_open = False

102	                return

103	            if self.status == "CALIBRATING":

104	                self.status = "OK"

105	            if self.on(0):

106	                self.fill = float(s["fill"])

107	            elif self.status != "COLLECTING":

108	                self.fill = min(100.0, self.fill + self.rate / 3600 * random.uniform(0.7, 1.3))

109	            if self.on(2):

110	                self.gas = float(s["gas"])

111	            else:

112	                self.gas += ((35 + self.fill * 2.2) - self.gas) * 0.02 + random.uniform(-2.5, 2.5)

113	            if not self.on(3):

114	                self.temp_raw = min(34, max(24, self.temp_raw + random.uniform(-0.04, 0.04)))

115	                self.hum = min(95, max(35, self.hum + random.uniform(-0.3, 0.3)))

116	            self.press += random.uniform(-0.03, 0.03)

117	            self.tilt = 38.0 if self.on(4) and s["tamper"] else random.uniform(0, 2.2)

118	            self.motion = bool(s["motion"]) if self.on(5) else random.random() < 0.025

119	            if self.status == "COLLECTING":

120	                self.lid_open = True

121	            elif self.tilt > c["tilt_th"]:

122	                self.status, self.lid_open = "TAMPER", False

123	            elif self.status in ("TAMPER", "LOCKED"):

124	                self.lid_open = False

125	            elif c["auto_lock"] and self.fill >= c["fill_th"]:

126	                self.status, self.lid_open = "LOCKED", False

127	            else:

128	                self.lid_open = bool(s["lid"]) if self.on(6) else self.motion

129

130	    def status_json(self):

131	        with self.lock:

132	            c, s = self.cal, self.sim

133	            temp = float(s["temp"]) if self.on(3) else self.temp_raw + c["temp_off"]

134	            hum = float(s["humidity"]) if self.on(3) else self.hum

135	            weight = float(s["weight"]) if self.on(1) else round(self.fill / 100 * c["weight_cap"] * 0.92 + random.uniform(0, 25))

136	            distance = c["d_empty"] - self.fill / 100 * (c["d_empty"] - c["d_full"])

137	            return {

138	                "fill": round(self.fill, 1), "distance_cm": round(distance, 1), "weight_g": weight,

139	                "temp": round(temp, 2), "temp_raw": round(temp - c["temp_off"], 2), "humidity": round(hum, 1),

140	                "pressure": round(self.press, 1), "iaq": round(self.gas), "iaq_acc": self.iaq_acc,

141	                "tilt": round(self.tilt, 1), "motion": self.motion, "lid": "OPEN" if self.lid_open else "CLOSED",

142	                "status": self.status, "cal": c, "sim_mask": self.sim_mask,

143	            }

144

145	    def command(self, cmd, body):

146	        with self.lock:

147	            if cmd == "LOCK":

148	                self.status, self.lid_open = "LOCKED", False

149	            elif cmd == "UNLOCK":

150	                self.status = "OK"

151	            elif cmd == "CLEAR_TAMPER":

152	                self.status, self.tilt = "OK", 0.0

153	                self.sim["tamper"] = 0

154	            elif cmd == "COLLECT":

155	                self.status, self.lid_open = "COLLECTING", True

156	            elif cmd == "COLLECT_DONE":

157	                self.status, self.fill, self.lid_open = "OK", 0.0, False

158	                self.gas = min(self.gas, 48)

159	            elif cmd == "CALIBRATE_START":

160	                self.calibrating = True

161	            elif cmd == "CALIBRATE_END":

162	                self.calibrating = False

163	            elif cmd == "RESET_GAS":

164	                self.iaq_acc = 0

165	            elif cmd in ("TARE", "CAL_WEIGHT"):

166	                pass

167	            else:

168	                return False

169	            print("  %s: %s" % (self.id, cmd), flush=True)

170	            return True

171

172

173	def local_handler(b):

174	    class H(BaseHTTPRequestHandler):

175	        def log_message(self, *a):

176	            pass

177

178	        def send(self, code, obj):

179	            data = json.dumps(obj).encode()

180	            self.send_response(code)

181	            self.send_header("Content-Type", "application/json")

182	            self.send_header("Content-Length", str(len(data)))

183	            self.send_header("Access-Control-Allow-Origin", "*")

184	            self.send_header("Access-Control-Allow-Methods", "GET, POST, PUT, PATCH, DELETE, OPTIONS")

185	            self.send_header("Access-Control-Allow-Headers", "Content-Type, X-SB-Op")

186	            self.end_headers()

187	            self.wfile.write(data)

188

189	        def body(self):

190	            n = int(self.headers.get("Content-Length") or 0)

191	            try:

192	                return json.loads(self.rfile.read(n) or b"{}")

193	            except ValueError:

194	                return {}

195

196

197	        def do_OPTIONS(self):

198	            self.send_response(204)

199	            self.send_header("Access-Control-Allow-Origin", "*")

200	            self.send_header("Access-Control-Allow-Methods", "GET, POST, PUT, PATCH, DELETE, OPTIONS")

201	            self.send_header("Access-Control-Allow-Headers", "Content-Type, X-SB-Op")

202	            self.send_header("Content-Length", "0")

203	            self.end_headers()

204

205	        def do_GET(self):

206	            if urlparse(self.path).path == "/api/status":

207	                self.send(200, b.status_json())

208	            else:

209	                self.send(404, {"error": "not found"})

210

211	        def do_POST(self):

212	            p = urlparse(self.path).path

213	            j = self.body()

214	            if p == "/api/command":

215	                ok = b.command(str(j.get("cmd", "")), j)

216	                self.send(200 if ok else 400, {"ok": ok})

217	            elif p == "/api/calibrate":

218	                with b.lock:

219	                    for k, v in j.items():

220	                        if k in b.cal:

221	                            b.cal[k] = v

222	                print("  %s: calibration saved" % b.id, flush=True)

223	                self.send(200, {"ok": True})

224	            elif p == "/api/simulate":

225	                with b.lock:

226	                    b.sim_mask = int(j.get("mask", 0))

227	                    b.sim.update(j.get("values", {}))

228	                print("  %s: simulation mask %d" % (b.id, b.sim_mask), flush=True)

229	                self.send(200, {"ok": True})

230	            else:

231	                self.send(404, {"error": "not found"})

232

233	    return H

234

235

236	# Fake Firebase: an in-memory JSON tree with GET, PUT, PATCH, DELETE and POST.

237	def cloud_handler(bins, ip):

238	    by_id = {b.id: b for b in bins}

239	    sent = {"bytes": 0}

240	    tree = {}

241

242	    def tree_get(parts):

243	        node = tree

244	        for k in parts:

245	            if not isinstance(node, dict) or k not in node:

246	                return None

247	            node = node[k]

248	        return node

249

250	    def tree_set(parts, value, merge=False):

251	        node = tree

252	        for k in parts[:-1]:

253	            node = node.setdefault(k, {})

254	        if value is None:

255	            node.pop(parts[-1], None)

256	        elif merge and isinstance(node.get(parts[-1]), dict) and isinstance(value, dict):

257	            node[parts[-1]].update(value)

258	        else:

259	            node[parts[-1]] = value

260

261	    class H(BaseHTTPRequestHandler):

262	        def log_message(self, *a):

263	            pass

264

265	        def send(self, code, obj):

266	            data = json.dumps(obj).encode()

267	            sent["bytes"] += len(data)

268	            self.send_response(code)

269	            self.send_header("Content-Type", "application/json")

270	            self.send_header("Content-Length", str(len(data)))

271	            self.send_header("Access-Control-Allow-Origin", "*")

272	            self.send_header("Access-Control-Allow-Methods", "GET, POST, PUT, PATCH, DELETE, OPTIONS")

273	            self.send_header("Access-Control-Allow-Headers", "Content-Type, X-SB-Op")

274	            self.end_headers()

275	            self.wfile.write(data)

276

277	        def body(self):

278	            n = int(self.headers.get("Content-Length") or 0)

279	            try:

280	                return json.loads(self.rfile.read(n) or b"{}")

281	            except ValueError:

282	                return {}

283

284	        def do_GET(self):

285	            u = urlparse(self.path)

286	            parts = u.path.strip("/").removesuffix(".json").split("/")

287	            if parts[:2] == ["sb", "bins"] and len(parts) == 2:

288	                self.send(200, {k: True for k in by_id})

289	            elif parts[:2] == ["sb", "bins"] and len(parts) == 4 and parts[3] == "status" and parts[2] in by_id:

290	                b = by_id[parts[2]]

291	                s = b.status_json()

292	                s["ts"] = int(time.time() * 1000)

293	                s["local_ip"] = "%s:%d" % (ip, b.port)

294	                self.send(200, s)

295	            elif parts == ["sb", "usage", "bytes_month"]:

296	                self.send(200, sent["bytes"])

297	            elif parts == ["sb", "usage"]:

298	                self.send(200, {"bytes_month": sent["bytes"], "limit_bytes": 10737418240})

299	            elif parts[0] == "sb_users":

300	                self.send(200, {"role": "collector"})

301	            else:

302	                self.send(200, tree_get(parts))

303

304	        def do_OPTIONS(self):

305	            self.send_response(204)

306	            self.send_header("Access-Control-Allow-Origin", "*")

307	            self.send_header("Access-Control-Allow-Methods", "GET, POST, PUT, PATCH, DELETE, OPTIONS")

308	            self.send_header("Access-Control-Allow-Headers", "Content-Type, X-SB-Op")

309	            self.send_header("Content-Length", "0")

310	            self.end_headers()

311

312	        def do_PUT(self, merge=False):

313	            parts = urlparse(self.path).path.strip("/").removesuffix(".json").split("/")

314	            j = self.body()

315	            if isinstance(j, dict) and isinstance(j.get("ts"), dict):

316	                j["ts"] = int(time.time() * 1000)

317	            tree_set(parts, j, merge)

318	            self.send(200, j)

319

320	        def do_PATCH(self):

321	            self.do_PUT(merge=True)

322

323	        def do_DELETE(self):

324	            parts = urlparse(self.path).path.strip("/").removesuffix(".json").split("/")

325	            tree_set(parts, None)

326	            self.send(200, None)

327

328	        def do_POST(self):

329	            u = urlparse(self.path)

330	            if u.path.startswith("/__auth/signInWithPassword"):

331	                j = self.body()

332	                if "@" not in str(j.get("email", "")) or len(str(j.get("password", ""))) < 6:

333	                    self.send(400, {"error": {"message": "INVALID_LOGIN_CREDENTIALS"}})

334	                else:

335	                    self.send(200, {"idToken": "mock-token", "localId": "mock-user", "expiresIn": "3600"})

336	                return

337	            parts = u.path.strip("/").removesuffix(".json").split("/")

338	            if parts[:2] == ["sb", "commands"] and len(parts) == 3 and parts[2] in by_id:

339	                cmd = str(self.body().get("cmd", ""))

340	                ok = cmd in ("LOCK", "UNLOCK", "COLLECT") and by_id[parts[2]].command(cmd, {})

341	                self.send(200 if ok else 400, {"name": "-mock%d" % int(time.time())} if ok else {"error": "Permission denied"})

342	            else:

343	                self.send(404, {"error": "not found"})

344

345	    return H

346

347

348	STATUS_CODES = ["OK", "LOCKED", "TAMPER", "COLLECTING", "CALIBRATING"]

349

350

351	# Fake ThingSpeak: last feed per channel and a TalkBack command queue.

352	def ts_handler(bins, ip):

353	    """ThingSpeak's read and TalkBack calls, laid out as the app reads them."""

354

355	    class H(BaseHTTPRequestHandler):

356	        def log_message(self, *a):

357	            pass

358

359	        def send(self, code, obj):

360	            data = json.dumps(obj).encode()

361	            self.send_response(code)

362	            self.send_header("Content-Type", "application/json")

363	            self.send_header("Content-Length", str(len(data)))

364	            self.send_header("Access-Control-Allow-Origin", "*")

365	            self.send_header("Access-Control-Allow-Methods", "GET, POST, PUT, PATCH, DELETE, OPTIONS")

366	            self.send_header("Access-Control-Allow-Headers", "Content-Type, X-SB-Op")

367	            self.end_headers()

368	            self.wfile.write(data)

369

370	        def bin_for(self, n, base):

371	            k = n - base

372	            return bins[k] if 0 <= k < len(bins) else None

373

374	        def do_GET(self):

375	            parts = urlparse(self.path).path.strip("/").split("/")

376	            if len(parts) == 4 and parts[0] == "channels" and parts[2:] == ["feeds", "last.json"] and parts[1].isdigit():

377	                b = self.bin_for(int(parts[1]), 1001)

378	                if b is None:

379	                    self.send(404, -1)

380	                    return

381	                s = b.status_json()

382	                code = STATUS_CODES.index(s["status"]) if s["status"] in STATUS_CODES else 0

383	                flags = (1 if s["motion"] else 0) + (2 if s["lid"] == "OPEN" else 0) + 4 * code

384	                self.send(200, {

385	                    "created_at": time.strftime("%Y-%m-%dT%H:%M:%SZ", time.gmtime()), "entry_id": int(time.time()),

386	                    "field1": str(s["fill"]), "field2": str(s["weight_g"]), "field3": str(s["temp"]), "field4": str(s["humidity"]),

387	                    "field5": str(s["iaq"]), "field6": str(s["pressure"]), "field7": str(s["tilt"]), "field8": str(flags),

388	                    "status": "%s:%d" % (ip, b.port),

389	                })

390	            elif len(parts) == 3 and parts[0] == "channels" and parts[2] == "feeds.json" and parts[1].isdigit():

391	                b = self.bin_for(int(parts[1]), 1001)

392	                if b is None:

393	                    self.send(404, -1)

394	                    return

395	                s = b.status_json()

396	                now = time.time()

397	                feeds = []

398	                for i in range(48, -1, -1):

399	                    f = max(0.0, s["fill"] - i * 0.6)

400	                    feeds.append({"created_at": time.strftime("%Y-%m-%dT%H:%M:%SZ", time.gmtime(now - i * 1800)), "entry_id": 1000 - i,

401	                                  "field1": str(round(f, 1)), "field2": str(round(f * 40)), "field3": str(s["temp"]), "field4": str(s["humidity"]),

402	                                  "field5": str(s["iaq"]), "field6": str(s["pressure"]), "field7": "0", "field8": "0"})

403	                self.send(200, {"channel": {"id": int(parts[1])}, "feeds": feeds})

404	            else:

405	                self.send(404, -1)

406

407	        def do_OPTIONS(self):

408	            self.send_response(204)

409	            self.send_header("Access-Control-Allow-Origin", "*")

410	            self.send_header("Access-Control-Allow-Methods", "GET, POST, PUT, PATCH, DELETE, OPTIONS")

411	            self.send_header("Access-Control-Allow-Headers", "Content-Type, X-SB-Op")

412	            self.send_header("Content-Length", "0")

413	            self.end_headers()

414

415	        def do_POST(self):

416	            parts = urlparse(self.path).path.strip("/").split("/")

417	            n = int(self.headers.get("Content-Length") or 0)

418	            form = parse_qs((self.rfile.read(n) or b"").decode())

419	            if len(parts) == 3 and parts[0] == "talkbacks" and parts[2] == "commands.json" and parts[1].isdigit():

420	                b = self.bin_for(int(parts[1]), 2001)

421	                cmd = (form.get("command_string") or [""])[0]

422	                if b is not None and cmd in ("LOCK", "UNLOCK", "COLLECT") and b.command(cmd, {}):

423	                    self.send(200, {"id": int(time.time()), "command_string": cmd, "position": 1})

424	                else:

425	                    self.send(400, {"status": "400", "error": "bad command"})

426	            else:

427	                self.send(404, -1)

428

429	    return H

430

431

432	def my_ip():

433	    s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

434	    try:

435	        s.connect(("10.255.255.255", 1))

436	        return s.getsockname()[0]

437	    except OSError:

438	        return "127.0.0.1"

439	    finally:

440	        s.close()

441

442

443	def main():

444	    ap = argparse.ArgumentParser(description="Stand-in Smart-Bin bins for the Android app.")

445	    ap.add_argument("--bins", type=int, default=2, help="how many bins (default 2)")

446	    ap.add_argument("--port", type=int, default=8081, help="port of the first bin (default 8081)")

447	    ap.add_argument("--cloud", type=int, default=0, help="also serve a fake Firebase on this port")

448	    ap.add_argument("--ts", type=int, default=0, help="also serve a fake ThingSpeak on this port")

449	    ap.add_argument("--no-local", action="store_true", help="do not answer on the bin ports, to test cloud mode")

450	    a = ap.parse_args()

451

452	    ip = my_ip()

453	    bins = [Bin(i, a.port + i) for i in range(max(1, a.bins))]

454	    servers = []

455	    if not a.no_local:

456	        for b in bins:

457	            servers.append(ThreadingHTTPServer(("0.0.0.0", b.port), local_handler(b)))

458	            print("%s answers at %s:%d" % (b.id, ip, b.port))

459	    if a.cloud:

460	        servers.append(ThreadingHTTPServer(("0.0.0.0", a.cloud), cloud_handler(bins, ip)))

461	        print("Fake Firebase at http://%s:%d" % (ip, a.cloud))

462	    if a.ts:

463	        servers.append(ThreadingHTTPServer(("0.0.0.0", a.ts), ts_handler(bins, ip)))

464	        print("Fake ThingSpeak at http://%s:%d (channels %s)" % (ip, a.ts, ", ".join(str(1001 + i) for i in range(len(bins)))))

465	    for s in servers:

466	        threading.Thread(target=s.serve_forever, daemon=True).start()

467	    print("Press Ctrl+C to stop.", flush=True)

468	    try:

469	        while True:

470	            for b in bins:

471	                b.step()

472	            time.sleep(1)

473	    except KeyboardInterrupt:

474	        for s in servers:

475	            s.shutdown()

476

477

478	if __name__ == "__main__":

479	    main()