SMART-BIN: AN ESP32-BASED IoT WASTE BIN MONITORING SYSTEM WITH FILL, WEIGHT, AIR-QUALITY AND TAMPER DETECTION

FOR PRIORITY-BASED COLLECTION

WASTE MANAGEMENT AND CLEANLINESS

PROBLEM: Overflowing Garbage Bins and Inefficient Waste Collection Routes

Technological University of the Philippines - Manila

Bachelor of Engineering Technology major in Computer Engineering Technology

Embedded Systems: Final Project

BET-CPET 4A-M, Group 2

Submitted By:

Brian Jay Celeste

Genesis Walter D. Reyes II

Jade C. Gunayan

King Perth G. Pepito

Rodel S. Banogbanog

Submitted To:

Prof. Jonel R. Macalisang

November, 2026

TABLE OF CONTENTS

TITLE PAGE	i

TABLE OF CONTENTS	ii

LIST OF FIGURES	iii

LIST OF TABLES	vi

I. INTRODUCTION	1

1.1 Introduction and Problem	1

1.2 Sustainable Development Goal (SDG) Addressed	2

1.3 Review of Related Literature	3

1.4 Objectives	54

1.5 Conceptual Framework	55

1.6 Target Beneficiaries	59

II. PROTOTYPE DESIGN AND DEVELOPMENT	60

2.1 Prototype Design	60

2.2 Block Diagram	61

2.3 System Flowchart	63

2.4 Schematic Design	69

2.5 Component Materials	72

III. TESTING, RESULTS, AND CONCLUSION	83

3.1 Test Cases	83

3.2 Results and Discussion	83

3.3 Conclusion	83

3.4 Recommendations	83

REFERENCES	84

APPENDICES	94

Appendix A: Gantt Chart	95

Appendix B: Bill of Materials of the Smart-Bin Prototype	96

Appendix C: User Interface Screens	97

Appendix D: Source Code	98

LIST OF FIGURES

Figure 1.	Sustainable Development Goal 11: Sustainable Cities and Communities	2

Figure 2.	Patents Reviewed by Priority Year and Region	6

Figure 3.	Remote Status Units Linked to a Central Unit	7

Figure 4.	Compacting Ram and Chambers of the Solar Compactor	8

Figure 5.	Path of a Full Message From the Compactor Monitor to the Service Provider	9

Figure 6.	Trash Can With a Motor-Driven Lid	10

Figure 7.	Container With a Bag Between Capacitive Plates	11

Figure 8.	Waste Collection System of Containers, Server and Vehicles	12

Figure 9.	Refuse Container With Scale, Display and Lights	13

Figure 10.	Sensor Device Mounted Under the Lid Behind a Thermal Barrier	14

Figure 11.	Gauge Mounted Under the Lid of a Container	15

Figure 12.	Smart Garbage Bin With a Lid-Mounted Sensor Device	16

Figure 13.	Pay-by-Weight Food Waste Processor With a Load-Cell Hopper	17

Figure 14.	Infrared Beam Across a Container	18

Figure 15.	Circuit of the Induction Flip Lid With a Settable Sensing Distance	19

Figure 16.	Overall Smart Waste Management System	20

Figure 17.	Garbage Cans and the Monitoring Center	21

Figure 18.	Waste Measurement System With Weighing Devices and Reports	22

Figure 19.	Block Diagram of the Capacity-Detecting Bin	23

Figure 20.	Electrical Block Diagram of the Bin Intelligent Controller	24

Figure 21.	Automatic Alerting System for a Trash Can	25

Figure 22.	Network of Waste Bins, Cloud Server and Vehicles	26

Figure 23.	Block Diagram of the Smart Waste Management System	29

Figure 24.	Circuit Setup of the LoRa Bin Unit	30

Figure 25.	Proposed System Architecture for Hospital Waste Monitoring	32

Figure 26.	Proposed Network Architecture for Waste Collection in Bukavu	34

Figure 27.	Prototype of the Smart Dustbin	35

Figure 28.	Block Diagram of the Arduino-Based Smart Dustbin	37

Figure 29.	Data Communication Structure of the Smart Waste Management System	38

Figure 30.	Prototype of the SmartTrashBin	39

Figure 31.	Data Flow of the Garbage Monitoring System	40

Figure 32.	Power Chain of the ESP32-CAM Node	44

Figure 33.	SWOT Analysis of Smart-Bin	49

Figure 34.	Features Found in the 30 Related Studies	52

Figure 35.	Input-Process-Output Model of Smart-Bin	55

Figure 36.	System Architecture of Smart-Bin	57

Figure 37.	Block Diagram of the Smart-Bin System	61

Figure 38.	System Flowchart of the Smart-Bin	63

Figure 39.	Operator Flowchart of the Smart-Bin Apps	65

Figure 40.	Collector Flowchart of the Smart-Bin Apps	67

Figure 41.	Schematic Diagram, Sheet 1: Microcontroller	69

Figure 42.	Schematic Diagram, Sheet 2: Actuators and Outputs	70

Figure 43.	Schematic Diagram, Sheet 3: Sensors and Inputs	71

Figure 44.	ESP32 DevKit V1 (38-Pin, ESP32-WROOM-32D)	72

Figure 45.	HC-SR04 Ultrasonic Distance Sensor	73

Figure 46.	HC-SR501 PIR Motion Sensor	74

Figure 47.	BME680 Environmental Sensor	75

Figure 48.	BMI160 Six-Axis Inertial Measurement Unit	76

Figure 49.	HX711 Load Cell Amplifier and 5kg Load Cell	77

Figure 50.	SG90 Micro Servo Motor	78

Figure 51.	2.4-in. TFT Touchscreen (ILI9341 With XPT2046 Touch Controller)	79

Figure 52.	KY-016 RGB LED Module	80

Figure 53.	Passive Buzzer Module (MH-FMD)	81

Figure 54.	18650-Battery Shields for Bin A (4-Cell) and Bin B (2-Cell)	82

LIST OF TABLES

Table 1.	Related Patents, by Priority Year	4

Table 2.	Related Studies From 2023 to 2026	27

Table 3.	Commercial and Student Systems as Described by Their Own Pages	47

Table 4.	Technical Basis of the Main Design Choices	48

Table 5.	The Four Closest Patents Compared With Smart-Bin	51

Table 6.	The Research Gap by Capability	53

# I. INTRODUCTION

## 1.1 Introduction and Problem

An effective waste management plays a crucial role in creating and sustaining healthy, clean and prosperous communities. With the rising amount of waste that is being generated, it has become a problem for the local government and commercial waste collectors in the Philippines. The Philippines is estimated to be producing a total of 60.76 thousand tons of garbage every day in 2025, according to the Philippine Statistics Authority (PSA, 2026). Despite the significant actions taken by the City of Metro Manila, its garbage is projected to reach 9,498 MT/D in 2020, making it the country's waste capital (World Bank, 2022). This was projected to rise to 10,368 MT per day by 2025 (World Bank, 2022). Hence, a high priority of improving waste management and collection practices in the country. Most garbage collection systems are conducted on a fixed schedule, causing bins to overflow when they are picked up or not to be filled to capacity when collected. Additionally, the smell, pollution levels, temperature, tilting of garbage bins, and unauthorized access should also be monitored as they are important indicators of environmental conditions and the efficiency of garbage management.

The researchers propose the Smart-Bin which is an Internet of Things (IoT) based bin monitoring system based on the ESP32 microcontroller with the aim of improving garbage collection performance. Sensors will be used by Smart-Bin to determine the level of fullness, weight, temperature, humidity, and air quality. It'll also be tracking bins that tilt irregularly, and any unauthorized bin openings, through a web dashboard and mobile application. Smart-Bin includes a touchscreen at the bin for employees to view and acknowledge the collection instructions, as well as to set up PIN codes to select who can access the bin, and to make collection setting changes. The project will include the development of two Smart-Bin prototypes built at the table size that will feature the ESP32 boards, local Wi-Fi, ThingSpeak, web dashboard, Android app, and screen at the bin. Smart-Bin will arrange the list of collection priorities based on fill levels and how long they will take to fill up, with the aim of optimizing garbage collection and fuel consumption rates.

## 1.2 Sustainable Development Goal (SDG) Addressed

Figure 1

Sustainable Development Goal 11: Sustainable Cities and Communities

The project Smart-Bin is connected to one of the Sustainable Development Goals (SDGs) of the United Nations (Agenda 2030), namely SDG 11: Sustainable Cities and Communities. Smart-Bin is specifically targeting the environmental impact of cities, per capita, with particular attention to air quality and municipal and other waste management (United Nations Department of Economic and Social Affairs [UN DESA], n.d., Target 11.6).

Smart-Bin can contribute to achieve the target of Sustainable Development Goal 11. It is achieved by monitoring and control of waste state with help of sensors installed in smart-bins and collecting information to make timely decision to ensure waste management collection process. As part of the project, a dashboard and application are developed and send a notification to the operator regarding of the bins state (levels, weight, temperatures, humidity, air quality around bins).

This could be helpful when it comes to optimizing the collection process as priority could be given to those bins that contained more trash and should be emptied before the trash overflows. Moreover, it can also be employed to avoid the effects on the environment, e.g., when the temperature or air quality (sign of rot) is not acceptable. This can help to streamline the collection process by prioritizing bins and reducing vehicle and fuel usage.

## 1.3 Review of Related Literature

This review covers three kinds of sources. Patents show what has already been claimed, in any year. Studies from 2023 onward show what researchers have built lately, and a handful of commercial products show what is already on sale. The patents were found and read on Google Patents which also provided the legal status listed in Table 1. Every research study was followed up with its DOI or its publisher page and each element in the reference list was verified with the research. Each patent and each study has its own entry, and figure (where present) from the source that can be reused, and each one has a comparison with Smart-Bin. The synthesis of those comparisons is made in the synthesis, the SWOT analysis and the research gap at the end of the section.

### Prior Art

Review of 20 patents. They are in effect through 2023 and their owners are in the United States, Europe, China and Korea. They are listed in priority year order in Table 1, and their coverage and differences from Smart-Bin are summarized. Google classifies these labels as assumptions and not legal conclusions, it's not until someone checks with the patent office whether their status is Active or Expired, that they should rely on it. Figure 2 puts the twenty into a time line. Most of the activity is recent, as five are priority years before 2012 and fifteen are priority years in 2012 or after.

Each of the patents is discussed individually below, in the order of the table. The numbers for each entry are the first images of the patent released as published on Google Patents, and the text below in each are what the patent claims and how it relates to Smart-Bin. The four closest patents are Kurani (2023), Sait et al. (2025), Shahabdeen (2016) and Yan and Chen (2021) and the synthesis in the end of this section makes a side-to-side comparison of the features of the two Smart-Bins.

Table 1

Related Patents, by Priority Year

| No. | Patent | Priority year | Status | What it covers | How Smart-Bin differs |
| --- | --- | --- | --- | --- | --- |
| 1 | US 5,016,197 (Neumann & Allison, 1991) | 1986 | Expired | Remote status units judge how full compactor containers are from hydraulic pressure or weight, and report over a telephone line to a central unit that calls the hauler. | A phone-line system for hydraulic compactors. It has no gas, tilt, lid, display, wireless link or phone app. |
| 2 | US 7,124,680 (Poss et al., 2006) | 2003 | Expired | A public trash compactor powered by a solar panel and a battery, with a removable bin, so it is emptied less often than an ordinary bin. | It compacts rather than senses. It has no weight, air-quality or tilt sensing, touchscreen, PIN mode or priority list. |
| 3 | US 7,145,450 (Brown, 2006) | 2004 | Expired | A processor reads the pressure on a compactor ram, flags the unit as full after a preset time and emails a work order to a service provider. | Fullness is inferred from pressure only. It has no fill level, weight, gas, lid, local display or app. |
| 4 | US 7,781,995 (Yang & Sandor, 2010) | 2005 | Expired | A trash can whose electric drive opens the lid when a sensor detects an object near the lower part of the can. | The trigger is a foot-level sensor. There is no fill, weight, air-quality, tilt, wireless or collection feature. |
| 5 | EP 2 516 974 B1 (Di Giugno et al., 2016) | 2009 | Not in force | A container that holds a bag has capacitive plates that give several fill levels. Its status goes wirelessly to stations that plan the emptying routes. | The sensing is capacitive around a bag. There is no weight, gas, tilt, powered lid, PIN mode or phone app. |
| 6 | WO 2014/079586 A1 (Kekalainen & Engstrom, 2014) | 2012 | Ceased | Containers with sensors report their status over a wireless network to a server, which plans the collection vehicles and can run bidding for the jobs. | It is a fleet-level planning platform with sensors only. There is no weight, tamper detection, powered lid, PIN mode or on-bin display. |
| 7 | US 9,347,821 B1 (Mullaney, 2016) | 2012 | Expired | The refuse container has a scale on it that indicates the weight of refuse in the container and alerts to a maximum weight. | It is capable to sense weight. Fill level, air quality, tamper alarm, PIR lid, PIN mode and dashboard are not present. |
| 8 | US 9,927,278 B2 (Kekalainen & Engstrom, 2018) | 2013 | Expired | A sensor device fixed under the container lid, behind a thermal barrier, measures the amount of waste and the conditions inside and sends them to a remote location. | Its novelty is the heat-insulated lid mounting of a cellular sensor module. There is no weight, tamper alarm, powered lid, PIN mode or priority list. |
| 9 | US 9,930,429 B2 (Kekäläinen, 2018) | 2014 | Active | A gauge compares the fill rate with limits set by a server and contacts the server only when a limit is crossed, which saves battery. | It is a battery-saving reporting scheme. No motorized lid, tamper alarm, PIN mode, touchscreen or phone app is described. |
| 10 | US 2016/0176630 A1 (Shahabdeen, 2016) | 2014 | Abandoned | A bin carries a lid-mounted range finder or a weight sensor at the base, with optional hydrogen sulfide, ammonia and methane sensing, and a controller that sends the data wirelessly. | It has no tamper detection, powered lid, lock, on-bin display, PIN mode or priority list. Temperature and humidity are not mentioned. |
| 11 | KR 10-1581944 B1 (Shin, 2016) | 2015 | Active | The hopper of a pay-by-weight food waste processor sits on a load cell and rotates to drop the waste into a crushing tank. | It is a weighing and payment machine. There is no fill sensing, gas sensing, tilt sensing, wireless reporting or priority list. |
| 12 | US 11,054,545 B2 (Devitt & Walsh, 2021) | 2016 | Active | Two low-power ways to detect a full container: an infrared beam across the container, or the resting tilt angle of the lid. | The output is only full or not full. There is no weight, gas, temperature, powered lid, PIN mode or tamper alarm. |
| 13 | EP 3 620 405 A1 (Wang, Chen, et al., 2020) | 2017 | Withdrawn | An induction flip-lid can whose infrared sensing distance the user can set, built into a single microcomputer chip. | It only opens the lid. There is no fill, weight, gas, tilt, display or wireless link. |
| 14 | US 11,702,280 B2 (Kurani, 2023) | 2018 | Active | 12 sensors, including those measuring fill level, weight, air quality, temperature, humidity and pressure, along with an accelerometer, are installed on the bin and communicate with a cloud server, which generates statistics and works out collection routes, also notifying mobile apps. | Focuses on litter, biohazard and pathogen sensing, with a sterilizer. There was no PIR servo lid, no remote lock or no PIN collect mode or latched tamper alarm. |
| 15 | CN 109324552 B (Yan & Chen, 2021) | 2018 | Active | Each can has weight, infrared height, odor and smoke sensors and a positioning unit. A monitoring center decides which cans to visit and builds the truck route. | It plans city-wide truck routes with GPS. No powered lid, tamper detection, PIN mode, on-bin display or phone alert was found. |
| 16 | US 11,788,877 B2 (Balachandran & Talavera, 2023) | 2018 | Active | Load cells under a platform weigh the waste of two or more bins, and cameras with image analysis identify the contents for waste audit reports. | It is a waste audit and reporting service, not a single bin with fill, tilt, lid and lock control, a PIN collect mode and a collection priority list. |
| 17 | CN 210504152 U (Yang et al., 2020) | 2019 | Expired | A bin with an infrared induction module, an ultrasonic ranging module and a programming module lets users see how much capacity is left. | It is a stand-alone local device. There is no weight, gas, tilt, lid actuator, wireless link or phone app. |
| 18 | CN 111559590 A (Wang, Zhang, et al., 2020) | 2020 | Withdrawn | A controller for top or side installation measures the distance to the garbage and uploads it to a supervisory platform, which has an early-warning unit. | It comprises of one laser ToF sensor. No weight, gas, temperature, tilt, lid control or touchscreen collect mode or priority list. |
| 19 | US 12,125,361 B2 (Keller et al., 2024) | 2021 | Active | Two calibration setpoints enable a processor to convert distance readouts to a fill percentage and signal a user device when it exceeds a threshold, which varies depending on the can owner. | It uses a single laser time-of-flight sensor. There is no weight, gas, temperature, tilt, lid control, touchscreen collect mode or priority list. |
| 20 | US 12,450,575 B1 (Sait et al., 2025) | 2023 | Active | Each bin has methane, fill level and temperature/humidity sensors that provide data to neural networks which calculate the time required to reach the methane and fill limits; a scheduler creates routes considering the vehicle emissions. | It revolves around the sensing of methane by air quality. There is no load cell, tilt alarm, PIR lid, remote lock or PIN collect mode. |

Note. Status is as reported by Google Patents in October 2026. The differences are those found in the patent text read for this review.

Figure 2

Patents Reviewed by Priority Year and Region

Note. Numbers match Table 1. The four patents in green are the closest to Smart-Bin.

Automated Trash Management System (Neumann & Allison, 1991)

Figure 3

Remote Status Units Linked to a Central Unit

Note. From Automated trash management system (U.S. Patent No. 5,016,197), by R. H. Neumann and C. B. Allison, 1991, U.S. Patent and Trademark Office.

Figure 3 shows the layout of the system that Neumann and Allison (1991) patented for MGM Services. Each remote status unit serves three containers, each with its own compactor, and all units are linked by a communications line, typically a telephone line, to a central unit that keeps a database. The hauler sends a truck only to the containers that report full. For compactors, fullness is inferred from the hydraulic pressure during the packing stroke, from the work done per stroke or from the current drawn by the pump motor, and for any container a measured weight can be compared with a preset full weight.

Calling for a pickup only when a container is full is the idea behind the priority tags of Smart-Bin, and the load cell is a signal that both systems can use. The differences come from the era. The data travel by modem, a person calls the hauler, and no alert or route is generated. The patent describes no gas, temperature or tilt sensing, no lid, no display on the container and no phone app. Smart-Bin also measures the fill directly with an ultrasonic sensor and does not infer it from the work of a compactor.

Solar Powered Compaction Apparatus (Poss et al., 2006)

Figure 4

Compacting Ram and Chambers of the Solar Compactor

Note. From Solar powered compaction apparatus (U.S. Patent No. 7,124,680), by J. Poss, J. Satwicz, B. Richmond, and M. Taylor, 2006, U.S. Patent and Trademark Office.

Poss et al. (2006) of BigBelly Solar patented a public compactor that a photovoltaic array powers, so that it can stand where there is no mains power but many people pass. Figure 4 shows the compacting ram with its chain drive and guide track and the compaction chambers. One of the roof panels charges a battery, which causes a compaction cycle to be skipped if the battery has low charge. The trash is compacted, which means that it will not need to be emptied as frequently as a regular trash bin. Fullness is sensed by a photo-eye, by the ram motor current (the stall time) and by pressure sensors, and a wireless transmitter reports to sanitation staff.

The patent attacks the same public-space problem as Smart-Bin, a bin that overflows and is costly to visit, but it answers with compaction and solar power instead of sensing. Scales are named only as an alternative fullness sensor, and the only display is a set of status lamps. No weight or air sensing, tilt alarm, touchscreen, PIN mode or phone app is described. One remark does bear on the project: the patent says that real-time fill data could feed routing and scheduling software, which is the use that Smart-Bin makes of its fill ranking.

Compactor Service and Monitoring System (Brown, 2006)

Figure 5

Path of a Full Message From the Compactor Monitor to the Service Provider

Note. From Compactor service and monitoring system (U.S. Patent No. 7,145,450), by W. J. Brown, 2006, U.S. Patent and Trademark Office.

Brown (2006), an individual inventor, patented a monitor that is added to an existing compactor. A pressure sensor on the hydraulic system of the ram measures the pressure during each cycle, and when the pressure stays at or above a preset value for a preset time the unit is flagged as full. Figure 5, which is Figure 2 of the patent, shows the path of a message. The monitor at the compactor reaches a cell site, an internet gateway and a server that runs the message monitoring software, which creates a work order, emails it to the service provider (a hauler or maintenance company) and supplies a tracking web link.

Like Smart-Bin, the system detects a full container by itself and notifies the people who must empty it. Fullness is inferred from pressure, though, and the patent has no level or weight sensor, since the tonnage is typed in by hand after a pickup. Alerts go out as email work orders, with fax as an option, and not as phone notifications, and the only local indication is a set of LEDs. Overdue work orders are flagged for staff, which is a rough counterpart of the priority tags of Smart-Bin, except that Smart-Bin bases its tags on the measured fill rate.

Trash Can With Power Operated Lid (Yang & Sandor, 2010)

Figure 6

Trash Can With a Motor-Driven Lid

Note. From Trash can with power operated lid (U.S. Patent No. 7,781,995), by F. Yang and J. Sandor, 2010, U.S. Patent and Trademark Office.

Yang and Sandor (2010) patented the power-operated lid of Simplehuman. Figure 6 shows the can with its lid open. There's a recess in front that houses the sensor, two link rods on the back wall connect a motor-operated lifting bar to the lid, and the circuit board, batteries and motor gear all reside in the base. If the sensor senses an object in the proximity of the bottom of the can, the electric drive opens the lid and the lid remains open for a predetermined period (2 seconds, for example). Claim 1 relates to the motor for the door. The controller on each command verifies the supply voltage, regulates the motor power to maintain the speed of the door within a certain range, stops the motor by reversing it for a short time, and if the corresponding motor is running for too long, the motor stops and a fault is output.

Smart-Bin also opens its lid by motor when a sensor notices a user, so that nobody has to touch the lid, but it uses a PIR sensor and a servo and not a foot-level light beam. Everything else in the patent concerns lid mechanics. It has no fill, weight, air-quality, tilt, lock, display or wireless feature, and nothing on the collection side. The patent shows that a hands-free lid is old, so the contribution of Smart-Bin cannot be the lid itself. It lies in the sensing and reporting behind the lid.

Measuring Device and Monitoring System for Containers (Di Giugno et al., 2016)

Figure 7

Container With a Bag Between Capacitive Plates

Note. From Measuring device, container means and system for monitoring and managing container means (European Patent No. 2516974), by M. Di Giugno, F. Giovanelli, S. Tiraboschi, and P. Villa, 2016, European Patent Office.

Di Giugno et al. (2016), from Cefriel and the Milanese environmental services company AMSA, describe a container whose casing holds a bag. Figure 7, which is Figure 2 of the patent, shows the container: the casing, the bag that the lid element holds in place and the plates at the sides of the cavity. Claim 1 places capacitive plates on opposite sides of the cavity, spaced along its length, so that the change in electrical capacity caused by the waste gives several progressive fill levels, with the bag lying between the plates. Status goes by GPRS or short-range wireless to an internet server, and stations plan the paths of the vehicles.

What matches Smart-Bin is the chain and not the sensor: measure the fill inside the bin, send it to a back end with a user interface, and use it to plan the collection of full bins. The sensing is different. Capacitive plates around a bag give levels in steps, while Smart-Bin measures distance by ultrasound and converts it to a percentage with two calibrated distances. It has no gas, temperature, tilt or weight sensing, no powered lid or lock, no PIN collect mode and no phone app.

Smart Waste Collection System and Method (Kekalainen & Engstrom, 2014)

Figure 8

Waste Collection System of Containers, Server and Vehicles

Note. From Smart waste collection system and method (International Patent Application Publication No. WO 2014/079586 A1), by F. Kekalainen and J. Engstrom, 2014, World Intellectual Property Organization.

Figure 8 shows the system in the application that Kekalainen and Engstrom (2014) filed for Enevo. The status is transmitted via a wireless network (GSM/3G) to a central server, which maintains the information in a database and arranges collection for vehicles on the way to collection or recycling facilities. The sensors are located in a rotating lid to measure fill level, gasses (methane, CO2 and CO type) and temperature. Weight is not measured. The server forecasts container fill times and routes them using a Monte Carlo simulation based on GPS data of the vehicles.The application also describes a marketplace in which haulers bid for collection jobs.

This is an early statement of the idea that fill data should decide which bins are collected first, and Smart-Bin carries it out on a much smaller scale with its priority tags. The patent works at fleet level, with bidding, simulated models and vehicle tracking, and the bin is only a sensor. It has no weight sensing, tilt or tamper alarm, powered lid, PIN mode or display on the bin, and humidity and odor are not measured.

Refuse Container With Weight Indicator and Danger Alerting (Mullaney, 2016)

Figure 9

Refuse Container With Scale, Display and Lights

Note. From Refuse container with weight indicator and danger alerting (U.S. Patent No. 9,347,821), by G. Mullaney, 2016, U.S. Patent and Trademark Office.

Mullaney (2016), an individual inventor, patented a refuse container that shows how heavy its contents are. Figure 9, which is Figure 1A of the patent, shows a container with a floor scale, a controller, a display and four lights. The lights track the percentage of the maximum weight, and all of them light, with one flashing, once the limit is exceeded. The maximum weight can be set by location (a city or ZIP code lookup), by the user or by default, and an optional beep is mentioned. Claim 1 adds a memory that stores a history of readings, a controller that sends that history to a service provider, and a lock that locks the lid once the refuse reaches a predetermined weight. An inversion sensor releases the lock for emptying.

The container weighs its contents, shows the weight and warnings on the bin, locks the lid under a condition and can report to a remote party, and Smart-Bin does all four. The lock is the point of difference. It closes at a weight limit to stop overloading, where the lock of Smart-Bin is controlled remotely or by a PIN. The patent does not measure fill level or air quality, has no tamper alarm, PIR lid or collection priority list, and uses the mobile app only for configuration.

Sensor Device for Smart Waste Collection Systems (Kekalainen & Engstrom, 2018)

Figure 10

Sensor Device Mounted Under the Lid Behind a Thermal Barrier

Note. From Sensor device for smart waste collection systems and method (U.S. Patent No. 9,927,278), by F. Kekalainen and J. Engstrom, 2018, U.S. Patent and Trademark Office.

Kekalainen and Engstrom (2018) patented the sensor module itself and not the system around it. Figure 10 shows a container with a pivoting lid and the sensor device under the lid, held off its surface by spacers that leave an air gap. The spacers and a heat-reflecting metal foil layer form a thermal barrier between the lid and the electronics. The module has an ultrasonic sensor for the amount of waste and sensors for temperature, humidity and a hydrocarbon or methane gas that indicates decomposition. The data is sent constantly, periodically or when a threshold is reached via a cellular modem (GSM or 3G), and the fasteners can be designed to avoid unauthorized removing.

This is the nearest match to the sensing side of Smart-Bin. Ultrasonic fill, temperature, humidity and gas are all on the bin, and the data go wirelessly to a remote location. The novelty lies in the insulated lid mounting and not in the sensors. The data support collection scheduling, but the device does not compute routes, and no weight sensor, powered lid, PIN mode or priority list is described. The anti-removal fasteners are a mechanical measure, where Smart-Bin senses the tilt and sounds an alarm.

Monitoring and Communicating the Fill Rate of a Container (Kekäläinen, 2018)

Figure 11

Gauge Mounted Under the Lid of a Container

Note. From Method and system for monitoring and communicating fill rate of container (U.S. Patent No. 9,930,429), by F. Kekäläinen, 2018, U.S. Patent and Trademark Office.

Kekäläinen (2018), also for Enevo, is about how often the gauge talks and not about what it measures. Figure 11 shows a garbage container with a hinged lid and the gauge attached under the lid. The model of how the gauge is supposed to function is sent to the gauge by a server that includes the maximum and minimum fill-rate limits and a communication schedule. The gauge maintains the microcontroller and transceiver in sleep mode, sensing the fill rate and waking only if the fill rate is at, or exceeds, a limit, transmitting the fill rate measurement and receiving an updated model. A lower number of transmissions equals a longer battery life.

Smart-Bin takes the opposite path over the same issue. It reports on a fixed schedule, with a write to ThingSpeak every 30s. A bin that is left outside for weeks could be threshold driven. The parts read the patent describes no motorized lid, tamper alarm, PIN mode, touchscreen or phone app. Its user device is a laptop or desktop.

Smart Garbage Bin (Shahabdeen, 2016)

Figure 12

Smart Garbage Bin With a Lid-Mounted Sensor Device

Note. From Smart garbage bin (U.S. Patent Application Publication No. 2016/0176630 A1), by J. A. Shahabdeen, 2016, U.S. Patent and Trademark Office.

Shahabdeen (2016) filed this application for Zan Compute. Figure 12 shows the bin with a battery-powered wireless sensor device on its lid. The sensor is a reflective infrared range finder (a Sharp GP2Y0A41SK0F) aimed at the waste, and the description adds MQ-136, MQ-137 and MQ-4 gas sensors for hydrogen sulfide, ammonia and methane. A weight sensor at the base is the alternative way to measure how much has been deposited. A local controller sends the data wirelessly to a remote controller (MQTT in claim 17), and claim 13 sends an alert when a sensed value passes a threshold. The application says that an ultrasonic sensor was a possible alternative to the infrared range finder but judged it too power-hungry.

It's the closest to Smart-Bin in hardware: the distance to the trash from under the lid, the weight under it, gas sensing, and wireless reporting. It does not provide tamper detection, powered lid, lock, display, Pin mode, priority list and does not talk about temperature and humidity. The lid only carries the sensor. Google Patents lists the application as abandoned.

Input Device for a Pay-by-Weight Food Waste Processor (Shin, 2016)

Figure 13

Pay-by-Weight Food Waste Processor With a Load-Cell Hopper

Note. From Inputting device for quantity and weight loss of food garbage container (Korean Patent No. 10-1581944), by H. Shin, 2016, Korean Intellectual Property Office.

Shin (2016), for Concoftech, patented the waste input device of a pay-by-weight food waste processor. Figure 13 is a perspective view of part of the machine, with some parts omitted and separated. The waste is dropped into a hopper which sits on a load cell, the user pays electronically on the display, and the hopper then rotates approximately 170-180 degrees and releases the waste into a crushing and stirring tank. The guide member with rollers allows hopper to be turned within the space of the lid without snagging, maintaining the machine compactness and convenient loading height. Drawings show that payment data is sent to a primary server not shown in the drawing.

The load cell, a powered lid and a display for the user are shared with Smart-Bin. It is not for the same purpose. The Korean device crushes the garbage and charges according to its weight, while Smart-Bin weighs to verify the garbage level. There is no fill-level, gas, tilt or tamper sensing, no wireless or cloud reporting in the text read, no collection priority list and no mobile alerts in the patent. The reason for earlier deodorant sprayers and sterilizing lamps is that the odor is only in the background.

Techniques for Optimally Sensing Full Containers (Devitt & Walsh, 2021)

Figure 14

Infrared Beam Across a Container

Note. From Techniques for optimally sensing full containers (U.S. Patent No. 11,054,545), by S. Devitt and B. Walsh, 2021, U.S. Patent and Trademark Office.

Devitt and Walsh (2021), for Oneplus Systems, give two ways to detect a full container while keeping very low the current drawn by battery-powered wireless circuitry. Figure 14, which combines Figures 1A and 1B of the patent, shows the first: an infrared emitter and receiver mounted on the side walls, so that the line between them marks the full level. The emitter transmits a wake-up signal and coded pulses, the receiver waits in sleep mode until the wake-up signal is received, and at the end of a specified period of time, if the coded signals are not received, the container is considered full. The second method checks the resting tilt angle of lid in three axes since a lid that overfull waste holds up is set at a different angle. The data from fullness is sent to a cloud server.

The patent only gives full or not full, with Smart-Bin providing a calibrated percentage and a time to full. No weight, gas, temp, powered lid, PIN mode/tamper alarm is displayed in parts read.  The tilt here is the angle of the lid, used as a sensing element, and it is not a tamper check, so it should not be confused with the BMI160 tilt alarm of Smart-Bin, which watches the whole bin.

Trash Can With a Sensing Flip Lid (Wang, Chen, et al., 2020)

Figure 15

Circuit of the Induction Flip Lid With a Settable Sensing Distance

Note. From Trash can having sensing flip lid having settable sensing distance and control method therefor (European Patent No. 3 620 405 A1), 2020, European Patent Office, X. Wang et al. (sensing flip lid).

A clamshell trash can with an infrared sensing distance set by the user is described by Wang, Chen and Lin (2020) of Fujian Nashida Electronic. The circuit diagram is shown in figure 15. The major components are an infrared transmitter and receiver underneath an induction window, a single microcomputer chip for the amplifier, analog-to-digital converter and distance registers, and a drive circuit to operate the motor and reducer. Only when the reading is above the standard distance value, the motor will flip the cover open and when the user selects a distance by pressing a setting button, it will be stored and used for comparison later.

This is the closest equivalent of the servo-lid of Smart-Bin that is triggered by a PIR. A hand near the induction window opens the lid and how near is up to the user. As no fill or weight, gas, tilt, display or wireless link or monitoring is involved in the project, nothing else is addressed in the patent. It was listed as a published application (kind code A1) on Google Patents and withdrawn in 2021, meaning that it was not granted.

Smart Waste Bin Sensor Device and Methods (Kurani, 2023)

Figure 16

Overall Smart Waste Management System

Note. From Smart waste bin sensor device and methods for waste management system (U.S. Patent No. 11,702,280), by H. B. Kurani, 2023, U.S. Patent and Trademark Office.

Kurani (2023) is the broadest patent of the twenty on sensing. Figure 16 gives the whole system for monitoring bins continuously to optimize collection and reduce littering. The bin is equipped with a battery powered smart sensor device carrying approximately a dozen sensors, including fill level, temperature, humidity/pressure, air quality, smoke, gas, motion, a pathogen biosensor and sterilizer, and an accelerometer. Readings are transmitted via a wireless network to a cloud server which creates real-time bin statistics and optimizes collection routes. The data is displayed in a bin app and a facility app, and biohazard and biosafety alerts are sent to nearby phones. The accelerometer detects when the bin has been tilted, and resets the fill level to zero when the bin tips over.

It matches Smart-Bin on sensing, cloud, apps and routes: fill, gas, temperature and tilt data go to a cloud server with mobile alerts and an optional touchscreen on the device. Its centre of gravity lies elsewhere, in litter, biohazard and pathogen sensing with a sterilizer. The parts read show no PIR-actuated servo lid, remote lock, PIN collect mode or tamper alarm with a latch, and the tilt serves to reset the fill level and does not sound an alarm.

Smart City Garbage Management System (Yan & Chen, 2021)

Figure 17

Garbage Cans and the Monitoring Center

Note. From Smart city garbage management system (Chinese Patent No. CN 109324552 B), by S. Yan and J. Chen, 2021, China National Intellectual Property Administration. The labels in the original are in Chinese.

Yan and Chen (2021), for Shanghai Shuncom Smart Technology, patented a city-wide system. Figure 17 is the schematic of the garbage cans and the monitoring center. Each can has a positioning unit, a controller and sensors for weight, infrared height, odor and smoke, and the controller uploads the readings to the monitoring center. The center decides which cans need inspection (weight, height or odor above a threshold, or any smoke), generates the route of the truck. When a special-condition signal arrives, a nearby truck is rescheduled by priority, in the order fire, fault, pollution and full bin.

This patent is the closest in purpose. Weight, height and odor decide which cans are serviced first, as the priority tags of Smart-Bin do. It goes further by building routes for trucks with GPS, and Smart-Bin stops at ranking the bins. It also includes smoke sensing, which Smart-Bin does not have. No powered lid, tamper detection, PIN mode, on-bin display or phone alert appears in the text read.

Apparatus and Method for Waste Monitoring and Analysis (Balachandran & Talavera, 2023)

Figure 18

Waste Measurement System With Weighing Devices and Reports

Note. From Apparatus and method for waste monitoring and analysis (U.S. Patent No. 11,788,877), by N. Balachandran and O. Talavera, 2023, U.S. Patent and Trademark Office.

Balachandran and Talavera (2023), for Zabble, cover waste audits. Figure 18 gives an overview of the measurement system: a series of weighing devices record the weight of the waste, the data go to networked storage, and the stored data are turned into visual reports for an operator. Claim 1, though, is about images: a method that acquires images of the contents of at least two containers of a waste generator, identifies characteristics of the contents and generates a report by computer. The disclosure adds load cells under a platform for two or more bins and gas sensors for compost bins.

The shared ground with Smart-Bin is remote monitoring of bin contents, with cloud upload and dashboards, and gas sensing in one special case. The aim is different. This patent is an audit and reporting service that tells a waste generator what is being thrown away, while Smart-Bin is a single bin that works out when it needs emptying. The claims do not cover a bin with fill, tilt, lid and lock control, a PIN collect mode or a collection priority list.

Capacity Inspection Garbage Bin With Infrared Induction (Yang et al., 2020)

Figure 19

Block Diagram of the Capacity-Detecting Bin

Note. From Capacity automated inspection garbage bin with infrared induction function (Chinese Patent No. CN 210504152 U), by S. Yang, D. Liao, R. Li, and Y. Yu, 2020, China National Intellectual Property Administration. The labels in the original are in Chinese.

Yang et al. (2020), for Sichuan Grid Environmental Protection Technology, is a utility model for a bin that shows how much capacity it has left. Figure 19 is a block diagram of the inside of the bin. The body holds an infrared induction module, an ultrasonic ranging module, a programming module and a power module. The ultrasonic module is connected to the infrared and programming modules, and the power module supplies all three. The ultrasonic module measures the remaining capacity and two nixie-tube digits show it on the bin, while the infrared module detects a nearby user to wake the system. Google Patents lists the rights as terminated for non-payment of fees.

This utility model matches the local side of Smart-Bin: an ultrasonic ranging module, a wake-up by a nearby user, and the capacity shown on the bin itself, where Smart-Bin shows it as a percentage on its touchscreen. Little else carries over. The device is stand-alone, with no weight, gas, tilt, lid actuator, wireless link, server, route planning or phone app.

Garbage Bin Intelligent Controller and Supervisory System (Wang, Zhang, et al., 2020)

Figure 20

Electrical Block Diagram of the Bin Intelligent Controller

Note. From Garbage bin intelligent control ware and garbage bin supervisory systems (Chinese Patent No. CN111559590A), by X. Wang, Y. Zhang, and Z. Wang, 2020, China National Intellectual Property Administration. The labels in the original are in Chinese.

Wang, Zhang, and Wang (2020), for Weihai Jingxun Changtong Electronic Technology, published an application for a bin controller that can be installed on top of the bin or at its side. Figure 20 is its electrical block diagram, with a wireless communication module, a control module and a distance sensor in one housing. In top mode the controller works out the height of the garbage, in side mode it reports the distance to the opposite wall, and either value is uploaded to monitor whether the bin is full. In the description, an ultrasonic module measures the distance, an inclination sensor raises a tip-over or movement alarm, the data go by NB-IoT and alerts reach the phones of workers.

The overlap with Smart-Bin is strong on the alert side: a distance sensor for fill, an inclination sensor for tip-over alarms, wireless upload and alerts on a phone. The patent has no weight or gas sensing, powered lid, lock, PIN or touchscreen collect mode, priority list or time to full, and in the parts read the inclination alarm has no countdown or latch, which Smart-Bin adds. Google Patents lists the application as withdrawn.

Measuring the Fill Level of a Trash Can Using a Sensor (Keller et al., 2024)

Figure 21

Automatic Alerting System for a Trash Can

Note. From System and method for measuring the fill level of a trash can using a sensor (U.S. Patent No. 12,125,361), by J. R. Keller, M. O. Redmond, and S. A. Mirza, 2024, U.S. Patent and Trademark Office.

Keller et al. (2024), for 7-Eleven, patented a way to turn a distance reading into a fill percentage. Figure 21 shows the automatic alerting system: a sensor above the container, a gateway and network, and a computer system with its modules and a user device. In claim 1 a sensor above the can holds a laser diode that sends a pulse toward the bottom and measures the time of flight. A computer converts the distance to a percentage of waste with two calibration setpoints, picks a threshold for the current time period from stored information about the owner of the can, and alerts a user device when the threshold is exceeded. Alerts can be email or text on a smartphone.

The two setpoint calibration is identical to that of Smart-Bin: the empty distance (default: 30cm) and the full distance (default: 5cm) are programmed from the interface, and the fill percentage is programmed to the distance between. A threshold-based alert reaches a phone in both. Everything else in the patent is narrower: a single laser sensor, no weight, gas, tilt or tamper sensing, no lid control or lock, no touchscreen collect mode and no priority list.

Waste Bin Collection and Monitoring Method Using Bin Sensors (Sait et al., 2025)

Figure 22

Network of Waste Bins, Cloud Server and Vehicles

Note. From Waste bin collection and monitoring method using waste bin sensors (U.S. Patent No. 12,450,575), by S. Sait, F. M. Aliyu, S. Bukar, B. S. Maina, and D. U. Lawal, 2025, U.S. Patent and Trademark Office.

Sait et al. (2025), from King Fahd University of Petroleum and Minerals, is the newest patent in the set. Figure 22 is a network diagram of the waste bins, the cloud server, the cloud network that connects them and the waste vehicles. Claim 1 describes a method to monitor and control the CO2 and CH4 emissions of a collection system. The methane, humidity, waste level, location and temperature are sensed under every separate bin lid and transmitted to a cloud server. One processor calculates the time in which the methane limit will be reached and a second processor calculates the time in which the fill limit will be reached, and a third processor, based on the CO2 and CH4 readings of the vehicles, prepares the collection schedule and routes. Two neural networks make the time estimates, and a mobile app sets parameters and shows data.

It is the nearest to Smart-Bin in using a time to full, in sensing fill, gas, temperature and humidity inside the bin, and in sending them to a cloud server with a display and a mobile app. The estimate is made differently. Neural networks work from methane and fill data, while Smart-Bin uses the fill rate of the last 15 minutes with no trained model. The patent is centered on methane prediction and routing based on emissions. It has no load cell, tilt or tamper alarm, PIR lid, remote lock or PIN collect mode, and weight and tilt appear only in a list of possible sensors.

### Related Literature and Studies

Thirty studies were reviewed between 2023 to 2026. Each of them is listed in Table 2 with its hardware, the main reported result and what it is lacking in comparison with Smart-Bin. If a figure is derived from training data or is a synthetic data set, this is indicated in the table as these figures are not representative of the actual system in use. Each study is then taken in turn under one of four headings, according to what it puts the most weight on: level-sensing bins with lids and alerts, bins that add weight, gas or access control, collection routes and priority, and cameras, sorting and system design. A figure is shown wherever the source is open access and has a drawing or photograph that helps, with the licence of the figure in the note under it. The other studies are text only, because their publishers do not allow the figures to be reused.

Table 2

Related Studies From 2023 to 2026

| Study | Hardware and platform | Reported result | What it lacks |
| --- | --- | --- | --- |
| Saad et al. (2023) | NodeMCU ESP8266 and Arduino Nano, HC-SR04, HX711 load cell, GPS, SG90 lid, WhatsApp through Twilio | Lid opens within 50cm. Full at 3cm of free space or 4,000g. Height error 0.4%, weight error 0.15%, message in 2.61s. | Dashboard, air quality, tamper detection, cloud history, roles |
| Abdullahi et al. (2024) | Arduino with HC-SR04 and SIM800L GSM, servo lid, cloud server | Lid opens only when the bin is not full. Full at 20cm. No performance figures are given. | Weight, air quality, tamper detection, dashboard, app |
| Alnuaimi et al. (2024) | Simulation of smart-bin collection as a travelling salesman problem (AnyLogic) | Shorter travel distance and lower emissions than static routes. No figures are given. | Bin hardware, alerts, app |
| Ayyanagoudar et al. (2024) | Ultrasonic and weight sensors, web page for each bin level | Real-time level tracking for each bin on a web page. No measured results are given. | Air quality, tamper detection, app, alerts |
| Azmi et al. (2024) | Arduino Uno with LoRa, HC-SR04, DHT11, GPS, ThingSpeak dashboard | Ultrasonic accuracy 89.9% at half full, humidity 96.9%, temperature 94.4%. Alerts are dashboard indicators only. | Weight, app, push alerts, on-bin display |
| Enache et al. (2024) | ESP32-CAM nodes with solar cell, ESP-MESH, server image processing | Fill ratio estimated from hourly bin photographs. Relative error 11.43% to 30.00% on five bins. | Weight, air quality, tamper detection, app |
| González et al. (2024) | ESP32 and ESP32-CAM, ultrasonic fill, simulated weight, Oracle APEX dashboard, App Inventor app | Camera classifies waste; weight is simulated with potentiometers. No accuracy figures. | Real weight sensing, air quality, tamper detection, priority list |
| Kuzhin et al. (2024) | IoT sensors with route analytics; no hardware described | The results are 20% fewer overfilled bins, 15% fewer collection trips, 25% shorter trips, and 10% less fuel. | Any bin hardware; setting and sample are not given |
| Mehta and Rathour (2024) | Ultrasonic, temperature, weight and tilt sensors and data software. | 6-month pilot: approximately 50% improvement in collection efficiency, 25% reduction in cost, and 83.3% reduction in overflows. | Detail on site and method; air quality, app, on-bin display |
| Muthukumar et al. (2024) | Camera-based AI segregation bin with a staff app for fill level and temperature and humidity | Classifier test accuracy 92.3% (training accuracy 71.8%). The scalability claim is not tested. | Weight, air quality, tamper detection, collection priority; main aim is segregation |
| Rathnayake et al. (2024) | Three ESP32 bins with ultrasonic and methane sensors, Node-RED cloud, machine learning | 4-month pilot: fill accuracy 95%, methane detection 92%. Route simulation 15% more path-efficient. | Weight, tilt, app, on-bin display |
| Salleh et al. (2024) | Using NodeMCU ESP8266, ultrasonic sensor and ThingSpeak dashboards. | Live container levels support need-based collection scheduling. Proteus simulation and PCB layout are included. | Weight, air quality, tamper detection, app |
| Shaharil and Po’ad (2024) | Arduino, ultrasonic lid sensor, IR fill sensor, load cell, MQ-2, LCD, Blynk | Weight errors of −5g to +10g on four items. The Blynk app shows a full-bin notification. No numbers for the IR or gas sensors. | Web dashboard, collection priority, tamper detection |
| Thangam et al. (2024) | Use of ESP8266, ultrasonic and soil-moisture sensors | Analytics are meant to let authorities adjust routes. No results are reported. | Weight, air quality, tamper detection, app |
| Aasish et al. (2025) | Dual-core ESP32 on FreeRTOS, ultrasonic, DHT sensor, RFID bin tags, LCD or OLED, cloud server | Splits sensing and cloud communication across tasks. No measured results. | Weight, tamper detection, app, test data |
| Alnanih et al. (2025) | Arduino Uno and ESP8266 with GSM, HC-SR04, flame sensor, GPS, LCD, Firebase, web app, Flutter app | 6-month, 50-bin campus pilot: 99.5% uptime, fill error within ±3%. Smoke Test fire alarm in 5s. | Weight, humidity, tamper detection, time to full, UI calibration |
| Alourani et al. (2025) | Arduino Nano and ESP8266, ultrasonic sensor, Firebase, Android app, VGG-19 classifier | App alerts collectors at a fill threshold. Classifier 99.7% training accuracy. | Weight, air quality, tamper detection, priority list |
| Banciu et al. (2025) | GPS and RFID bin locations; Nearest Neighbor, OR-Tools and genetic algorithm | Three real routes with 735 locations: up to 52% shorter distance, over 14kg less CO₂. | Any bin sensing; the study is about routing |
| Gajula et al. (2025) | ESP32, HC-SR04, Moisture sensor, Servos and Arduino IoT Cloud. | Separates dry and wet waste by moisture. No figures are given. | Fill-level role of the ultrasonic sensor, weight, tamper detection, app |
| Gupta et al. (2025) | Microcontroller with ultrasonic sensor and Wi-Fi, cloud platform | 30-day, 50-bin pilot: 58.9% less distance, 59.2% less fuel, 95% fewer overflows, 99.8% uptime. | Weight, air quality, tamper detection, app detail |
| Ishaq et al. (2025) | Ultrasonic sensors in bins at ten hospitals, cloud monitoring station | Alerts led to pickups 3 to 16 minutes later. Biomedical waste about 43.6% to 52.5% of hospital waste. | Weight, air quality, tamper detection, app (proposed only) |
| Karthi et al. (2025) | ESP32, RFID reader, ultrasonic sensor, metal detector, MQ135, Blynk | 95% fill accuracy, 92% RFID reliability, 40% less manual intervention. | Weight, tamper detection, priority list, web dashboard |
| Kumar et al. (2025) | Using ESP32, ultrasonic sensors, Ant Colony Optimization, machine learning, and map. | Prioritizes filled bins and builds depot-to-depot routes. No numbers are reported. | Weight, air quality, tamper detection, app, alerts |
| Maulida et al. (2025) | ESP32, HC-SR04, MQ-2, HX711, SG90 lid, Firebase, Android app | Level accuracy about 95%, weight about 93%, 92% of spoiled-food cases flagged, usability score 86 (15 officers). | Tamper detection, collection priority, web dashboard, roles |
| Mehta et al. (2025) | ESP8266 with ultrasonic sensor, MQ4, MQ136, MQ137 and GPS, Firebase and React Native Application | Gas sensors 96% to 98% accurate against a reference. Field test of 28-bin samples cut trucks from 3 to 2. The 100-bin results are synthetic. | Weight, tilt, on-bin display; some tables conflict |
| Sadikin et al. (2025) | ESP32 with ultrasonic, infrared and GPS, Firebase, Kotlin Android app | Ranking by distance, vehicle capacity and fill level (greedy). 69 respondents: 92.37% effectiveness, 89.92% satisfaction.. | Weight, air quality, tamper detection, on-bin display |
| Shahibollah et al. (2025) | ESP32, ultrasonic sensors, servo lid, tri-colour LED, Blynk | Tested by simulation and on hardware. No figures are given. | Weight, air quality, tamper detection, priority list |
| Yogesh et al. (2025) | ESP32, HX711 load cell, ultrasonic sensor, Firebase, App Inventor app, LCD | Weight and fill percentage with full-bin alerts. No measured results are reported. | Air quality, tamper detection, priority list, test data |
| Awotayo and Dodo (2026) | ESP32, two HC-SR04, MG90S servo lid, LEDs and buzzer, Blynk email | Fill zones at 40% and 60%; email when full. No accuracy or response-time results. | Weight, air quality, tamper detection, dashboard, app |
| Mugaruka et al. (2026) | LoRaWAN-style network of ultrasonic sensors in seven 120-L bins | 12-week pilot: 57% of sensors at or below −107dBm; mass and route times are estimates. | Weight, air quality, tamper detection, app |

Note. Results are as reported by the authors. Lacks means that the feature is not described in the study.

A. Level-Sensing Bins With Lids and Alerts

Smart Bin With Automatic Lid Control (Abdullahi et al., 2024)

Figure 23

Block Diagram of the Smart Waste Management System

Note. From “Development of a smart waste management system with automatic bin lid control for smart city environment,” by A. Abdullahi, A. Mohammed, M. U. Bonet, A. El-Suleiman, R. B. Ahmad, and T. D. Chollom, 2024, EAI Endorsed Transactions on Smart Cities, 7(3). CC BY-NC-SA 4.0.

Abdullahi et al. (2024), of the Air Force Institute of Technology in Kaduna, Nigeria, built a bin whose lid opens only when the bin is not full. Figure 23 is their block diagram. The fill level is measured by an ultrasonic sensor (HC-SR04) and an Arduino, which also detects the approaching user, and the lid is opened by a servo motor that rotates 0-90 degrees. The thresholds are 20cm for a full bin, and 12cm for a user. Status connects to the cloud server via a SIM800L GSM module where data is stored and full bin notifications are sent to the collection team.

There are no figures for performance and it is inconsistent regarding the board as one figure shows Uno and the other shows Nano. There's a phone block in the diagram, but the app is never mentioned. Smart-Bin includes the ultrasonic fill sensor, a servo lid and weight, air sensing, tilt, a dashboard, and a phone app, all of which this study lacks.

Smart Waste Management System Using LoRa (Azmi et al., 2024)

Figure 24

Circuit Setup of the LoRa Bin Unit

Note. From “Smart waste management system using LoRa technology,” by A. Z. Azmi, N. M. Sahar, and M. T. Islam, 2024, E3S Web of Conferences, 599, Article 04007. CC BY 4.0.

Azmi et al. (2024) installed a LoRa connection between the bin and the receiver. The setup for their circuit is shown in Figure 24. The ultrasonic fill distance, DHT11 temperature and humidity and GPS position are transmitted through a bin-side Arduino Uno with LoRa shield to a second Arduino Uno with LoRa receiver, and then relayed to a NodeMCU which updates a ThingSpeak dashboard with indicators when thresholds are reached. The tests indicated an ultrasonic accuracy of 89.9% at half full, a humidity accuracy of approximately 96.9% and a temperature accuracy of approximately 94.4%.

The authors report that the ultrasonic sensor was not successful in a narrow, shallow bin, and the DHT11 temperature sensor error exceeded the sensor's specified error. They state that they have a LoRa range of several kilometers, but they did not measure it, and the abstract mentions air quality monitoring, but does not go into any detail about it in the body of the paper. Also, Smart-Bin senses temperature and humidity (BME680 used instead of DHT11) and uses ThingSpeak. The study has no weight sensing, push alerts, phone app or display on the bin.

Ultrasonic Trash Can Surveillance With ThingSpeak Dashboards (Salleh et al., 2024)

Salleh et al. (2024) developed a prototype for waste collection based on demand at a low cost. An ultrasonic distance sensor mounted in each container is read by a NodeMCU ESP8266 module which uploads the fill level to ThingSpeak so that live dashboards indicate the status of each container and enable container status-based scheduling. Also included is a Proteus simulation and a PCB layout. No results are reported, and the authors view this work as a scheduling tool, rather than a validated route planning method.

Smart-Bin also employ ThingSpeak for charts and analysis, but only as a second cloud besides Firebase, as optional. It provides the weight, air quality, tamper detection and a phone app, things this study doesn't.

ESP8266 Waste Monitor With Moisture Sensing (Thangam et al., 2024)

Thangam et al. (2024) of Amrita Vishwa Vidyapeetham, Bangalore, have developed an ESP8266 monitor that utilizes ultrasonic fill sensors, soil-moisture sensors to assess the waste composition. The real-time data is supposed to enable the authorities to modify routes and reduce trips and gas. The route claim is a stated aim but not a measured result, as no quantitative results are reported in the abstract.

Smart-Bin senses are filled by the same type of ultrasonic sensor with the addition of weight, air quality, tamper detection and a phone app. It makes no attempt to judge the composition of the waste.

Smart Bins for Hospital Waste in Kaduna (Ishaq et al., 2025)

Figure 25

Proposed System Architecture for Hospital Waste Monitoring

Note. From “Smart waste bin monitoring using IoT for sustainable biomedical waste management,” by A. Ishaq, S. J. Mohammad, A. D. Bello, S. A. Wada, A. Adebayo, and Z. T. Jagun, 2025, Environmental Science and Pollution Research, 32(32), 19434–19449. CC BY 4.0.

Ishaq et al. (2025) put ultrasonic fill-level sensors in bins at ten hospitals in Kaduna, Nigeria. Figure 25 is the proposed system architecture, from the hospital ward to the smart bins, an internet link, a laptop at the monitoring station and the collection truck. The bins connect wirelessly through the routers of the hospitals to a central cloud monitoring station, which alerts waste collectors when a bin is full. The authors also measured waste generation. Biomedical waste made up about 43.6% to 52.5% of hospital waste, at an average of 69.14kg per day and 1.52kg per bed per day, and they analysed the data with principal component analysis.

Alerts led to pickups 3 to 16 minutes later. The phone app appears only in the proposed system and is not described as built, and no microcontroller is named for the bins of the authors. The study shows what level sensing is worth in a place where overflow carries a health risk. Smart-Bin senses more than level, but it has not been tested in any such site.

IoT-Enabled Smart Dustbin for Real-Time Monitoring (Shahibollah et al., 2025)

Shahibollah et al. (2025), from Universiti Teknologi MARA in Terengganu, built an ESP32 dustbin with ultrasonic sensors for fill level and for proximity, a servo-driven lid and a tri-colour LED for local status. Data go over Wi-Fi to the Blynk platform, which alerts the maintenance staff. The authors tested the bin by simulation and with hardware, but the abstract gives no ranges or accuracy figures and describes the result as prototype function and not as municipal-scale reliability.

Smart-Bin uses the same ESP32 family and a servo lid. Its touchscreen takes the place of the LED, and it adds weight, air quality, tamper detection and a priority list.

IoT Smart Bin for Real-Time Fill Level Monitoring (Awotayo & Dodo, 2026)

Awotayo and Dodo (2026) of Baze University, Abuja designed and implemented an ESP32-based bin equipped with two HC-SR04 sensors. One of these is facing outwards and will open a servo lid (MG90S 0 to 90 degrees) when someone is in range of 30cm. The other indicates the level of fill, with a green LED when the fill level is between 0 and 39%, a yellow LED when the fill level is between 40 and 59%, and a red LED and buzzer when the fill level is between 60 and 100%. An e-mail is sent by Blynk when the bin is full. Two 3.7V Li-ion cells (7.4V, 3500mAh) power the unit. There are no accuracy or response-time results reported.

The local feedback is LEDs and a buzzer and not a screen. Smart-Bin also includes a touch screen, weight detection, air quality, tamper detection, dashboard and a phone app, none of which were identified in this study.

LoRaWAN Network for Waste Collection in Bukavu (Mugaruka et al., 2026)

Figure 26

Proposed Network Architecture for Waste Collection in Bukavu

Note. From “Design of an IoT sensor-based network architecture for waste collection in Bukavu city (DRC),” by O. M. Mugaruka, J. R. Kala, J. Ndikumagenge, R. van Schadewijk, and R. Nibitanga, 2026, Discover Internet of Things, 6(1), Article 168. CC BY-NC-ND 4.0.

Mugaruka et al. (2026) designed a LoRaWAN-style network for waste collection in Bukavu, in the Democratic Republic of the Congo. Figure 26 shows the proposed network. EM310-UDL ultrasonic sensors on the bins talk over LoRa to a gateway, the gateway reaches a cloud server by Ethernet, Wi-Fi or 4G, and users view the data on a control station and on mobile phones. The authors tested it in a 12-week pilot with seven 120-litre prototype bins in residential, dense and peri-urban areas.

The results concern the network more than the bins. Of the sensors, 57% had a signal at or below −107dBm, and the mass collected (an estimated 641kg over 14 days in one area) and the route times are estimates from sensor data and not direct measurements. The authors also report an energy autonomy above 90% for most units after 11 months, a period that the paper does not clearly tie to the 12-week pilot. Smart-Bin does not use LoRa at present, and the study is a reminder of the opportunity listed in the SWOT analysis: LoRa for places without Wi-Fi.

B. Bins That Add Weight, Gas or Access Control

IoT-Based Smart Dustbin Prototype (Saad et al., 2023)

Figure 27

Prototype of the Smart Dustbin

Note. From “IoT-based smart dustbin prototype,” by A. M. Saad, B. O. Jul, A. Basalamah, and S. Sayuti, 2023, PROtek: Jurnal Ilmiah Teknik Elektro, 10(2), 120–125. CC BY-NC-SA 4.0.

Saad et al. (2023) built the smart dustbin shown in Figure 27. An Arduino Nano with an ultrasonic sensor opens an SG90 servo lid to anyone who is within 50cm, while a NodeMCU ESP8266 reads the waste height with an HC-SR04, the weight with an HX711 load cell and the position with a NEO-6M GPS. Once the bin is full (either by level (3cm free space) or weight (4,000g), the NodeMCU will send a WhatsApp message via Twilio with a Google Maps link.

The tests reported a height error of 0.4%, a weight error of 0.15% and a message delay of 2.61s. This is the nearest study to the pairing of fill and weight with a servo lid in Smart-Bin. Its lid is triggered by an ultrasonic sensor, where Smart-Bin uses a PIR sensor. The study has no dashboard, air quality sensing, tamper detection, cloud history or roles.

Smart Bin With Real-Time Garbage Level Tracking (Ayyanagoudar et al., 2024)

Ayyanagoudar et al. (2024) propose a smart bin that uses weight sensors and ultrasonic transducers to track the fill level in real time and shows the level of each bin on a web page. The abstract also states that it can classify trash based on its weight, which is used for planning transportation. The abstract does not mention any microcontroller or cloud platform and does not provide any results of measurements.

Smart-Bin also correlates weight to an ultrasonic level. It includes air sensing, tamper detection and a mobile phone app, which are not included in this study.

Efficiency of IoT-Enabled Smart Bins (Mehta & Rathour, 2024)

Mehta and Rathour (2024) present the description of IoT smart bins, which are equipped with ultrasonic, temperature, weight and tilt sensors and software to collect and process the data. They claim a 6-month pilot in one city where collection efficiency has improved by approximately 50%, operating costs have decreased by approximately 25% and there have been approximately 83.3% fewer overflow incidents, while also offering reduced emissions through route optimization and increased satisfaction of citizens. Abstract makes no mention of the location or nature of these measures or of the number of samples collected, these figures are merely the authors' claims.

This is the only study to mention a tilt sensor but does not indicate a tamper alarm. Smart-Bin does exactly that, a 10 second (latched) countdown followed by a siren that latches.

Dual-Sensing Bins for Fill Level and Methane (Rathnayake et al., 2024)

Rathnayake et al. (2024) developed the dual sensing bins which combined the ultrasonic fill sensor with methane sensor for detection of spoilage. The ESP32 modules transmit the data via Wi-Fi to a cloud platform where the data is processed by the Node-RED software and ML algorithms predict the waste and optimize routes. Three bins were deployed in a four month pilot in Dehiwala, Sri Lanka and reported a 95% fill-level accuracy; methane-detection accuracy was 92% and simulated routes were 15% more path-efficient than fixed routes. There is no definition of the accuracies in the abstract and 15% is obtained from a simulation.

The ESP32 and the ultrasonic level are shared between Smart-Bin. It is not a methane reading, but rather an air-quality estimate as it is a gas channel. This study doesn't include any weight sensing, tilt alarm, phone app or on bin display.

Arduino-Based Smart Dustbin (Shaharil & Po’ad, 2024)

Figure 28

Block Diagram of the Arduino-Based Smart Dustbin

Note. From “Arduino-based smart dustbin with ultrasonic sensor for efficient waste management,” by M. H. F. Shaharil and F. A. Po’ad, 2024, Evolution in Electrical and Electronic Engineering, 5(2), 92–97. CC BY-NC-SA 4.0.

Shaharil and Po’ad (2024) built a household-scale bin and show its block diagram in Figure 28. The ultrasonic sensor opens up the servo lid if a person is within 30cm, the infrared sensor detects the level of fullness of the bin, the weight is displayed on the LCD using a load cell, and finally, a buzzer alerts when air quality is poor with the help of an MQ-2 sensor. The Blynk app shows a notification when the bin is full.

The testing is small: 15 distance trials and four weighed items with errors from −5g to +10g, and no numbers for the infrared or gas sensors. The bin is the closest study to the local side of Smart-Bin, with a lid, weight, gas and a display on the bin. It has no web dashboard, no collection priority and no tamper detection.

IoT-Driven System With Fire Detection and Role-Based Apps (Alnanih et al., 2025)

Figure 29

Data Communication Structure of the Smart Waste Management System

Note. From “Advancing sustainability through an IoT-driven smart waste management system with software engineering integration,” by R. Alnanih, L. Elrefaei, and A. Al-Ahwal, 2025, Sustainability, 17(21), Article 9803. CC BY 4.0.

Alnanih et al. (2025) combine fill and fire sensing in one bin unit. Figure 29 shows their data communication structure. An HC-SR04 is used to measure the fill level, a flame sensor detects fire, smoke and gas and an Arduino Uno with an ESP8266 and GSM receives the sensor data and an LCD, buzzer are added. Firebase and Firestore are used to launch alerts, coordinating collection routes from a central command center. A desktop web application is used by administrators and a Flutter Android application for workers based on their roles.

6-months pilot on campus with 50 bins achieved 99.5% uptime with fill error within ±3%, and a fire alert within 5s (smoke test). This is one of the longest field pilots of the studies which constructed their own bins. It's similar to Smart-Bin, it has Firebase, a web app, a phone app and different views for different users. There is no weight, tilt or lid feature and nothing is calibrated from the interface in the hardware list.

ESP32 Smart Trash Bin With Firebase (Maulida et al., 2025)

Figure 30

Prototype of the SmartTrashBin

Note. From “Design of IoT-based smart trash bin monitoring using ESP32 and Firebase,” by A. Maulida, C. S. Fayzalmi, F. A. C. Anaam, D. Ardiansyah, M. A. Gunawan, and N. Ridwan, 2025, Jurnal Komputer, Teknologi Informasi dan Sistem Informasi (JUKTISI), 4(2), 420–428. CC BY 4.0.

The SmartTrashBin in Figure 30 was created by Maulida et al. (2025) from Politeknik Negeri Semarang. The hand detection & fill level is read using HC-SR04, while MQ-2 is used to detect the odour and the load cell (HX711) detects the weight that is then fed to ESP32, which controls the opening of the lid using a servo (SG90). Data is stored in Firebase Realtime Database and sent to an Android application that includes notifications.

They claim a level accuracy of approximately 95%, a weight accuracy of approximately 93% (+/- 320g on 20kg), 92% of cases of spoiled food identified and 98% of notification reliability and a System Usability Scale score of 86 from 15 sanitation officers. The numbers require carefulness as the paper mentions both of Android Studio and Flutter and also provides two notification thresholds. The closest of the studies to Smart-Bin is the following: ESP32, ultrasonic sensor, load cell, servo lid, firebase and an android app. No tamper detection, collection priority or web dashboard/roles.

Real-Time Sensing and Methane Mapping (Mehta et al., 2025)

Figure 31

Data Flow of the Garbage Monitoring System

Note. From “Smart bins, smarter cities: IoT-driven waste collection with real-time sensing and methane mapping,” by H. Mehta, V. Deriya, D. Sankhe, P. Kanani, G. Pandya, and V. Shelke, 2025, Mathematical Modelling of Engineering Problems, 12(6), 2110–2122. CC BY 4.0.

Mehta et al. (2025) show the data flow of their system in Figure 31. Each bin has the ESP8266 equipped with an ultrasonic fill sensor, MQ4, MQ136 and MQ137 gas sensors, GPS and uploads data to Firebase every 10s. A React Native application displays a bin map, a methane and odor heatmap and truck routes that are ranked based on a priority score derived from fill and gas.

Gas sensors accuracy was 96% - 98% with reference. There are a number of trucks that are different – field samples of 28 bins in Malad v. 100 bins in the results (which use synthetic data), and some tables in the paper clash. The study is like Smart-Bin, which is based on Firebase, a mobile application and implements role-based views and priority-driven collection. No weight sensing and no tilt sensors and no display on the bin.

ESP32 Smart Dustbin With Weight and Level Monitoring (Yogesh et al., 2025)

Yogesh et al. (2025) created an ESP32 dustbin that includes an HX711 load cell and an ultrasonic sensor, which is called “Eco Bin”. Weights and fill percentage are sent to Firebase Realtime Database and a phone app, designed with MIT App Inventor, for full-bin alerts, and a 16×2 I2C LCD displays the alerts on the bin. However, since the paper doesn't report any actual results, it is here as a hardware precedent, not as a performance metric.

It's almost the same as in Smart-Bin, which is ESP32, load cell, ultrasonic sensor, Firebase, Android app and local display. Smart-Bin brings in air quality, tamper detection, a priority list and a web dashboard.

RFID Access Control and Blynk Integration (Karthi et al., 2025)

Karthi et al. (2025) developed ESP32 dustbin with servo lid which will open only for the authorized RFID user. A fill level sensor (ultrasonic sensor), a metal detector, and an air-quality sensor (MQ135) are used to perform measurement checks, with the data sent to the Blynk platform. Abstractly, the authors state 95% fill level accuracy, 92% RFID authentication reliability, 40% less manual work and 35% more efficient collection.

It is the only one study, which can open the bin, that is found. On the collecting side, however, Smart-Bin provides control via a PIN collect mode and remote lock, plus weight, tamper detection and a priority list. The web dashboard is not available for this study.

C. Collection Routes and Priority

Smart-Bin Collection as a Travelling Salesman Problem (Alnuaimi et al., 2024)

The paper by Alnuaimi et al. (2024) from University of Sharjah, UAE, considers the problem of municipal waste collection as a travelling salesman problem (TSP). The fill level of the bins is reported and the route is planned using the Nearest Neighbor heuristic within an AnyLogic simulation. The abstract claims that there are savings in travel distance and carbon emissions when compared to traditional static routes, but it does not provide figures and there is no description of hardware. The work is not a field deployment, but rather a simulation.

The results of this study is where the Smart-Bin ends. It is a list of bins sorted by fill and sorted by time to fill; and a planner such as this one might use that list as input.

Waste Management Optimization Through IoT and Analytics (Kuzhin et al., 2024)

According to Kuzhin et al. (2024), a case study of IoT sensor real-time data and analytics for route optimization has resulted in a 20% reduction in overfilled bins, a 15% reduction in collection frequency, a 25% reduction in the distance traveled, a 10% reduction in fuel consumption, and a 15% increase in recyclables collected. The abstract does not identify a location, sample size, hardware, time frame, and fails to specify whether the values are from a field experiment; the evidence is weak.

The study shows the size of the gain that people expect from collecting by priority. It gives no design that Smart-Bin could borrow.

Route Optimization With GPS and RFID Bin Locations (Banciu et al., 2025)

Banciu et al. (2025), from Lucian Blaga University of Sibiu, collect bin locations with GPS and RFID, build distance matrices and compare the Nearest Neighbor heuristic, Google OR-Tools and a genetic algorithm on three real routes with 735 locations. The genetic algorithm did best, with up to 52% shorter travel distance and more than 14kg less CO₂. RFID here only records the location of a bin when it is emptied, and the study senses no fill level.

It sits entirely on the routing side. Smart-Bin supplies the part that this study takes for granted, which is knowing which bins are worth a visit now.

IoT E-Tracking System for Waste Management (Gupta et al., 2025)

Gupta et al. (2025) propose an IoT system in which bins carry an ultrasonic fill sensor, a microcontroller (Arduino-based, going by the figure captions) and a Wi-Fi module that sends data to an unnamed cloud platform for on-demand routes. They say that in a 30 day trial with 50 bins they achieved 58.9% less distance, 59.2% less fuel, 95% less overflows and 99.8% uptime..

These are some of the highest figures in the entire review, and no pilot dates are mentioned, and no method of routing nor protocol for accuracy tests, so as a result this is what the authors report, but not an independent result. Only level sensing. Smart-Bin builds upon the same level reading with weight, air, tilt and user interface.

Ant Colony Route Optimization With Machine Learning (Kumar et al., 2025)

In Kumar et al. (2025), a system is designed where ultrasonic sensors are mounted on ESP32 boards to send fill level data to a central database. On one hand, Ant Colony Optimization is used to plan routes from the depot to the depot, only stopping at the bins that are filled; on the other hand, machine learning is used to forecast waste from past data; and the collection teams are provided with a Folium map as an interface. The abstract does not provide any actual figures, so there is no actual saving in fuel or cost that can be quoted from the abstract.

Smart-Bin also has the ESP32, the ultrasonic fill sensing and the concept of only going to bins that need it. It sets that priority from the fill and the time to full and not from a learned model. Weight, air quality and tamper detection and phone app not included in this study.

TrashTrack Collection Ranking (Sadikin et al., 2025)

Telkom University (Bandung) Researcher, Sadikin et al., (2025) developed an Android application using Kotlin and Jetpack Compose called TrashTrack. ESP32 bins are equipped with ultrasonic, infrared and GPS sensors and upload the collected data to a Firebase database; a modified constraint-based greedy algorithm is used to rank the collection based on the distance of the bin, vehicle capacity and fill level. The results of the usability test with 69 respondents showed that 92.37% were effective, 92.29% were useful and 89.92% were satisfied.

Smart-Bin is powered by the same ESP32, Firebase and a kotlin android app. It ranks by fill and time to full (no dist or vehicle capacity – it doesn't plan roads). Weight, Air Quality, Tamper Detection and Display on bin are not included in this study.

D. Sorting and System Design

Camera-Based Fill Level Detection (Enache et al., 2024)

Figure 32

Power Chain of the ESP32-CAM Node

Note. From “Intelligent trash level detection for sustainable urban waste management,” by B.-A. Enache, C.-K. Banica, and A. G. Bogdan, 2024, The Scientific Bulletin of Electrical Engineering Faculty, 24(1), 56–59. CC BY-NC-ND 4.0.

Enache et al. (2024), from the University Politehnica of Bucharest, estimate the fill level from photographs. Figure 32 shows the power chain of one node. A 5V solar cell feeds a TP4056 charger, which charges a 3.7V LiPo battery of 850mAh, and a step-up converter brings the voltage back to 5V for the ESP32-CAM. The battery was sized for 24 hours of operation. The fill ratio is estimated by a Python script with OpenCV and an SQL database, each node takes one image per hour and sends it over an ESP-MESH network to a server.

On five test bins the relative error of the estimated fill ratio ran from 11.43% to 30.00%, and the authors name the angle of the camera, the light, the kind of waste and the placement of the bag as causes. The figure matters to Smart-Bin because it shows what a stand-alone node needs between the source and the board: a charger, a battery and a converter. Smart-Bin builds the same chain with its battery shields and 18650 cells. It is not camera based, it is based on distance and weight and this study has no weight, air quality, tamper detection, or phone app.

AI-Based Waste Segregation With Monitoring (Muthukumar et al., 2024)

At M. Kumarasamy College of Engineering in Karur, Muthukumar et al. (2024) developed a bin fitted with a camera that can identify plastic, glass, metal and paper using AI and place them in separate compartments. A mobile application displays the level of fill, plus the temperature and humidity levels, and notifies cleaning personnel and allows for one team to use the application to claim each task. The number of images used is about 1500 images per class and the paper reports 71.8% training accuracy and 92.3% test accuracy, but does not explain the difference between these. The microcontroller and the cloud service are not named.

Segregation is outside the scope of Smart-Bin. The claim-a-task feature of the app echoes the collector role in Smart-Bin, and the study has no weight, air quality, tamper detection or collection priority.

Smart Trash Bins With IoT and AI (González et al., 2024)

González et al. (2024) constructed a bin made of ESP32 and ESP32-CAM. This includes ultrasonic sensors that measure fill level, an AI that detects waste from images from the camera, and potentiometers that replace the weight sensor. Events are captured by readings, which are transmitted over Wi-Fi to a central server and displayed on Oracle APEX dashboards; and an MIT App Inventor app is used to manually record the status. The abstract does not contain any accuracy figures.

The weight is simulated and the study indicates a dashboard design without any actual weighing. Smart-Bin offers a real load cell, and always offers a simulation as an option for testing, all set from the interface. This study has no air quality sensing, tamper detection or priority list.

Smart Waste Classification With IoT and Deep Learning (Alourani et al., 2025)

Alourani et al. (2025) propose iSSWMs, a framework that joins an IoT smart bin to a deep-learning classifier. The bin has an ultrasonic level sensor on an Arduino Nano and an ESP8266 that sends its status to Firebase, and an Android app alerts the collectors at a threshold. A VGG-19 classifier sorts plastic, glass and metal and reached 99.7% accuracy on about 1,530 TrashNet images over 100 epochs. That figure is training accuracy, and the authors report no field test.

Smart-Bin shares the ultrasonic level, Firebase and an Android app with alerts for collectors. Image classification is outside its scope, and this study has no weight, air quality, tamper detection or priority list.

ESP32 Smart Bin That Separates Dry and Wet Waste (Gajula et al., 2025)

Gajula et al. (2025) developed an ESP32 dust bin which detects the wetness of the waste deposited and separates dry and wet waste with the help of SG90 servo and MG90 servo. The HC-SR04 ultrasonic sensor and the moisture sensor provide information to the Arduino IoT Cloud that triggers the separation mechanism. Abstract does not provide any numbers that were measured.

It's not about monitoring the bin, it's about sorting, and shares only the ESP32, a servo and the cloud link with Smart-Bin. No weight measurement, tamper detection or phone application.

FreeRTOS-Based Smart Waste Management (Aasish et al., 2025)

Aasish et. al (2025) presents a system on a dual core ESP32 based on FreeRTOS. It has an ultrasonic sensor to track the level of the bin, as well as a DHT11/DHT22 to measure temperature and humidity, it has an optional LCD or OLED display and can upload to a cloud server via Wi-Fi for alerts and dashboards and dynamic collection scheduling using RFID modules to identify each bin. The basic concept is to separate the sensor acquisition and actuation tasks, from the cloud communication tasks, so that a slow upload does not block a reading. No actual results are stated and the RFID is not used for access control, it is merely used to identify bins.

The task split is a design to consider for the Smart-Bin firmware, as it is also based on the ESP32 family. The study has no weight sensing, tamper detection or phone app.

Commercial Systems

Commercial systems are available. Bigbelly's CLEAN software provides the data from the bins on a console and uploads it to a collector app (Bigbelly, n.d.). According to its website, Ecube Labs offers ultrasonic fill sensors and a cloud platform that provides fire event notification and route optimization, and claims to predict bin status up to 24 hours in advance (Ecube Labs, n.d.). Sensoneo's platform displays a map of each bin's fill and predicted full time, and plans routes from them (Sensoneo, n.d.), Nordsense's driver app with dynamic routing (Nordsense, n.d.), and Evreka's sensor enhances temperature and motion to flag fires (Evreka, 2020). Compology monitors containers with cameras (Compology, n.d.), and Bin-e is a segregating bin with a touch-free lid (Greenwalt, 2020). In the Philippines, students of the University of the Cordilleras built the Baraki bins for the DOST-TAPI Regional Invention Contest, which open automatically and report to an information center (Asido, 2024). Table 3 summarizes what the vendor pages state.

Table 3

Commercial and Student Systems as Described by Their Own Pages

| System | Sensing described | Who sees the data | Collection priority or route |
| --- | --- | --- | --- |
| Bigbelly | Fill level, collection events and GPS; sensor type not named | Management console and collector app | Not stated |
| Ecube Labs | Ultrasonic fill level, battery level and fire events | Cloud dashboard | Route optimization; predicts status up to 24h ahead |
| Sensoneo | Ultrasonic or radar fill level | Dashboard with a map and predicted full time | Semi-automated route planning |
| Nordsense | Page covers routing, not sensors | Driver app | Dynamic routing |
| Evreka | Ultrasonic fill level, temperature, motion and location | Web platform with SMS and email alarms | Route optimization |
| Compology | Camera: fullness, location and motion | Web dashboard | Not described |
| Bin-e | Fill level, motion for the flap and waste type | Mobile app | Not covered |
| Baraki | Automatic sensors; types not stated | Information center | Not stated |

Note. Sources: Bigbelly (n.d.), Ecube Labs (n.d.), Sensoneo (n.d.), Nordsense (n.d.), Evreka (2020), Compology (n.d.), Greenwalt (2020) and Asido (2024).

All of them sell via quotation: none of these pages mentions weight, gas or tilt sensing, or a price. That doesn't mean that the products don't have these characteristics, just that vendor pages are concise and designed to sell. What it shows is that a small system with those features at about ₱4,900 for two bins (Appendix B) occupies a different place from these services. Smart-Bin does not compete with them on scale, and it cannot, but it can show what a bin that senses more costs to build.

### Technical Merit

Seven design choices in Smart-Bin decide how much its readings can be trusted. Table 4 gives the reason for each. Each one is also a place where a measured result could differ from the real condition of the bin, which is what the tests of the project will have to check.

Table 4

Technical Basis of the Main Design Choices

| Design choice | What Smart-Bin does | Why it is sound |
| --- | --- | --- |
| BME680 for the air check | One I2C chip gives temperature, humidity, pressure and a gas resistance reading, from which the firmware estimates an air-quality index. | In Table 2, the studies that sense gas use MQ-series modules, which only provide a reading for the gas. Bosch Sensortec (n.d.) states that the BME680 uses 0.09 to 12mA during its different modes. The index is an estimate and needs about 5 minutes to settle. |
| Median distance reading is calculated from five distance readings. | The firmware makes five measurements using the HC-SR04 and uses the middle measurement. | When the echo bounces off an edge or soft material, a single reading from the ultrasonic can be spiked. One or two of such readings will be ignored without delaying the update, this is the median. |
| Two-point calibration | The full distance (5cm by default) and empty distance (30cm by default) are defined from the interface and the fill percentage is the distance between them. | The two setpoints are the same as in Keller et al., 2024. Other heights of bins do not require new firmware. |
| Weight as a second check | The HX711 and a 5kg load cell weigh the contents next to the fill percentage. | Level and weight are also combined in Saad et al. (2023) and Maulida et al. (2025). A loose pile in a bin is light but a full bin is heavy. |
| Tilt with a countdown | If the tilt is over 30° a countdown of 10s begins. When the bin is steady again the alarm will clear, or if not, the alarm will latch and need to be reset by an operator. | Countdown will not allow for a bump or moving bin to cause a false alarm and an overturned bin or carried away bin will still sound. |
| Local and cloud together | The bin responds on its local WiFi within ~1s, and to Firebase and ThingSpeak. | Fast control and calibration with local path, history, remote commands and alerts when phone is away with cloud path. The data is uploaded every 30s, which is half the frequency ThingSpeak can accommodate on its free plan of 1 write every 15s |
| Calibration and simulation from the interface | Distances, limits can be changed, without re-flashing, any sensor can be set to a test value. | None of the 30 studies describes this. It lets a test be repeated at a chosen reading and keeps demonstrations safe. |

Note. Defaults are those set in the firmware and can be changed in the settings.

### SWOT Analysis

The strengths, weaknesses, opportunities and threats of Smart-Bin are presented in Figure 33. The strengths and weaknesses are related to the construct itself, while the opportunities and threats are related to the literature mentioned above and to the situation in the Philippines. The count of studies that sense tilt or offer role-based views comes from this review. RA 9003 puts segregation and collection at the barangay level and leaves the collection of non-recyclable and special waste to the municipality or city (Ecological Solid Waste Management Act of 2000, 2001), and the World Bank (2022) puts the share of Metro Manila waste that is collected at about 60%.

Figure 33

SWOT Analysis of Smart-Bin

The weaknesses set the limits of what the tests of this project can claim. The tests demonstrate that it works on a bench, but not that the prototype can last on the street for 18 hours in Bin A and 9 hours in Bin B and that it is a table-size prototype that has never been outside. The opportunities are on the other side, the weaknesses: pilots on campus or in a barangay hall, LoRa for where there is no Wi-Fi, solar power and road routing.

### Synthesis of the Review

The patents, studies and products reviewed above converge on four points. First, the problem is a documented one. The Ecological Solid Waste Management Act of 2000 (2001) leaves the collection of non-recyclable and special waste to the municipality or city, and the World Bank (2022) puts the share of Metro Manila waste that is collected at about 60%. A bin that overflows between collections is therefore an expected condition of the system that these sources set out to monitor.

Second, sensing the fill level is a solved problem. It is measured by 27 of 30 studies, often using an HC-SR04: Patents have done it with hydraulic pressure (Neumann & Allison, 1991), capacitive plates (Di Giugno et al., 2016), an infrared beam (Devitt & Walsh, 2021) and an ultrasonic sensor (Kekalainen & Engstrom, 2018) and a laser (Keller et al., 2024). The commercial services in Table 3 sell the same measurement with a cloud console on top.

Third, the other sensors exist, but they are added one or two at a time. Saad et al. (2023) and Maulida et al. (2025) add weight, Mehta et al. (2025) add gas, Alnanih et al. (2025) add a flame sensor and Azmi et al. (2024) add temperature and humidity. There is only one reference by Mehta and Rathour (2024) who mention a tilt sensor but do not mention any alarm. In the patents, Mullaney (2016) weights contents, Kurani (2023) mentions around a dozen sensors, but one is based on litter and pathogens, not the user of the bin.

Fourthly, the work is directed more into the route than into the bin. Twelve of the 30 remaining studies report on routes or priority of collection, and have the biggest improvements: up to 52% shorter distance in Banciu et al. (2025) and 58.9% in a pilot of Gupta et al. (2025) with no date or method provided. The bins in these studies stay simple, ten studies have a phone app, two give different users different views, and none of them describes calibration or simulation from the interface of the bin.

None of the sources brings these pieces together in one bin. The studies that weigh the waste have no tilt alarm, the one that lists a tilt sensor describes no alarm, the ones with phone apps and roles have no weight, and the route studies take the readings of the bin for granted. Table 5 sets the four closest patents against Smart-Bin, feature by feature, and Figure 34 counts the features found in the studies. None of the twenty patents, as far as their text was read, combines a latched tamper alarm, a PIN collect mode on a touchscreen, a remote lock and sensor simulation and calibration from the interface with fill, weight and air sensing in one bin. A search of this size cannot prove that no such patent exists, so this is a statement about what was found and not about what exists.

Table 5

The Four Closest Patents Compared With Smart-Bin

| Feature | Shahabdeen (2016) | Yan and Chen (2021) | Kurani (2023) | Sait et al. (2025) | Smart-Bin |
| --- | --- | --- | --- | --- | --- |
| Fill level | Yes (range finder) | Yes (infrared height) | Yes | Yes | Yes |
| Weight | Yes (or fill level) | Yes | Yes | Not found | Yes |
| Gas or odor | Yes | Yes | Yes | Yes (methane) | Yes |
| Temperature and humidity | Not found | Not found | Yes | Yes | Yes |
| Tilt or tamper alarm | Not found | Not found | Accelerometer listed | Not found | Yes |
| Motorized lid | Not found | Not found | Not found | Not found | Yes |
| Remote lock or PIN collect | Not found | Not found | Not found | Not found | Yes |
| On-bin display | Not found | Not found | Not found | Not found | Yes |
| Phone alert | Not found | Not found | Yes | Not found | Yes |
| Priority or route | Not found | Yes (truck route) | Yes (route) | Yes (route) | Yes (priority) |

Note. Counts are derived from the hardware and the function described in each study (Table 2).

Figure 34

Features Found in the 30 Related Studies

Note. Counts come from the hardware and function each study describes (Table 2).

### Research Gap

There are 30 studies; 27 sense fill level. None has all of weight, gas, temperature, humidity and tilt in one bin; 6 add weight, 6 add gas, 4 sense temperature/humidity and 1 senses tilt. Two studies are providing role-oriented perspectives, one of which is using an RFID-based access control mechanism. None indicates simulation / calibration from the interface. The bin indicates its fill level, and the focus is on routes and fleet software in the patents and commercial systems. None of the four most closely related patents have all ten of the features listed in Table 5.

So, it's not a sensor, it's a gap. These factors: fill, weight, air and tilt sensing, alerts to the operator, collector and person at the bin, collection priority from the fill and time to fill, interfaces to allow bin calibration and testing without need to re-flash, are all combined in one single low-cost bin. The gap by capability and the specific objective in Section 1.4 which addresses it is provided in Table 6.

Table 6

The Research Gap by Capability

| Capability | Related studies | Patents and products | Smart-Bin | Objective |
| --- | --- | --- | --- | --- |
| Sensing breadth on one board | 27 fill, 6 weight, 6 gas, 4 temperature or humidity, 1 tilt; none has all | Kurani (2023) lists about a dozen sensors; the others are narrower | Fill, weight, temperature, humidity, pressure, air quality and tilt on one ESP32 | SO1 |
| Tamper detection | One study lists a tilt sensor | No latched tamper alarm found in the four closest patents | Tilt beyond 30° starts a 10s countdown, then a latched siren until RESET TAMPER | SO1 |
| Alerts to each kind of user | 10 studies have a phone app and 7 a web dashboard | Phone alerts in Kurani (2023) | Buzzer, LED and touchscreen on the bin, web dashboard, Android app and push alerts | SO2 |
| Roles and access | 2 studies have role-based views; 1 uses RFID | None found | Operator and collector views, remote lock, PIN collect mode | SO2, SO3 |
| Collection priority | 12 studies are about routes or priority, mostly with GPS and algorithms | Route planning in the closest patents | Fullest first, with a time to full from the recent fill rate; no road routing | SO3 |
| Testing and demonstration | No study describes simulation or calibration from the interface | None found | Per-sensor simulation and calibration from the dashboard and app | SO3 |

## 1.4 Objectives

### General Objective

To design and develop Smart-Bin, an IoT based Waste Bin Monitoring System which will monitor the fill level, weight and air quality of the waste bin, detect tampering and give real time monitoring and alerts to the concerned entities via Web Based Dashboard, Android App and On-Bin Touchscreen.

### Specific Objectives

- Sensor Monitoring and Detection

To develop 2 Smart-Bin units, which include sensors to measure fill level, weight, temperature, humidity and air quality, as well as to detect bin tilting and tampering and open the lid automatically when hand movement is detected.

- Real-Time Monitoring and Alerts

To develop a web dashboard and an Android application to display real-time data from the sensors, bin status, retrieve data from Thingspeak/Firebase and notify the operators and collectors when the bin is full, poor air quality, high temperature, tampered sensor reading and when the bin is not online.

- Bin Control and Collection Prioritization

To implement Collection Prioritization, based on bin fullness and estimated time to fullness, remote bin locking and unlocking, on-bin collection functions (via PIN), sensor simulation and system interface sensor calibration. This will lead to the prioritization of collections and will also help with efficient collection routes and fuel gas efficiency.

## 1.5 Conceptual Framework

The framework of Smart-Bin is shown in two parts. The first one which is below describes the process of a reading going from the sensor to the person who is going to respond to it and it is known as the Input-Process-Output (IPO) model. The second, under IoT Level, shows how the parts are deployed and connected. Both refer to the same system and both result in the three specific objectives listed in Section 1.4 above.

### Input-Process-Output Model

Figure 35

Input-Process-Output Model of Smart-Bin

The input column has seven items. The distance to the trash is measured by the HC-SR04 sensor, the weight is measured by the HX711 sensor (which includes a 5kg load cell), the BME680 sensor provides temperature, humidity, pressure and an estimate of the air quality, the BMI160 sensor gives the tilt, and the PIR sensor detects a hand. The final two inputs are from the people: commands input on the touchscreen, in the dashboard or in the app, and the settings, which contain the calibration and simulation values. All of these inputs are related to Specific Objective 1, and commands and settings are related to Specific Objective 3.

The process column is how the ESP32 firmware and the cloud use them. The firmware takes the median of five readings, converts the distance to a fill percentage from the calibration limits, compares the readings with their thresholds, runs the tamper countdown and applies the lock. Local REST API provides response in ~1s and status is stored in Firebase while the commands are stored in thing speak where the history is stored. The apps rank the bins by fullness, by time until they are full and give the operator or collector views. Specific Objectives 2 and 3 are addressed in this column.

Live status on the touch screen, the dashboard and the app, alerts for a full bin, bad air, heat, tamper, or bin being offline, Collection Priority list, live status for the lid, lock, and PIN collect on the bin, history charts, and remote control, are all things people see and do, which is the output column. The result is reduced overfilling bins and timely collection, which is the problem stated in Section 1.1. Feedback closes the loop; the results of the test and the operator's calibration will improve the limits and calibration values used by the firmware next.

### IoT Level

Figure 36

System Architecture of Smart-Bin

The connections between the parts of Smart-Bin are shown in Figure 36. The readings are sent from the cloud to the two apps at the top: the web dashboard (primarily for the operator) and the Android app (primarily for the garbage collector). Both apps can also reach a bin directly over the local Wi-Fi in about 1s for any command, calibration, or simulation.

There are two end nodes at the bottom: Trash bin A and Trash bin B. All are equipped with the same ESP32 and six input sensors (HC-SR04, PIR sensor, BME680, BMI160, load cell, touch panel) and four output actuators (servo, RGB LED, buzzer, TFT). Bin A is powered by 4x 18650 cells, Bin B by 2x.

Each bin sends its sensor data up to two cloud services and receives the LOCK, UNLOCK and COLLECT commands from them. Firebase is the primary cloud and stores the live status, ThingSpeak stores the history and charts and relays commands via TalkBack.

Smart-Bin fits IoT Level 6 of the deployment templates in Bahga and Madisetti (2014). A Level 6 system has several independent end nodes that sense or actuate and send their data to the cloud, where it is stored and shown by a cloud-based application. An analytics component works on that data and keeps its results in the cloud as well, and a centralized controller knows the status of every end node and sends control commands back to them. Trash bin A and Trash bin B are the end nodes in Figure 36. Both have their own ESP32, they read their own sensors and update their own status to Firebase and ThingSpeak, they don't depend on each other and there is no gateway between them and the cloud.

Cloud side is Firebase for live status, and ThingSpeak for stored history. ThingSpeak keeps the readings of every bin and plots and analyses them over time, and the apps use the same data to rank the bins by fill level and estimated time to fill in the Collection Priority list. The web dashboard and the Android app take the role of the centralized controller. They show the state of both bins side by side and send LOCK, UNLOCK and COLLECT back through the cloud, and calibration and simulation commands go to a bin directly over the local Wi-Fi.

## 1.6 Target Beneficiaries

- Waste Collectors and Utility Staff

Smart-Bin will assist waste collectors and utility workers by notifying them which bins are fullest and need to be emptied first. It could also prevent visits to bins that are not full and offer PIN access to the bin during collection.

- Facility Managers and Operators

The web dashboard can be used by facility managers or barangay hall, school, office or mall operators to monitor the status of the bins, make assessment of the data and set thresholds for the sensors, apart from the messages they receive on overflowing bins, poor air quality, high temperature, and even possible tampering.

- Barangay and Local Government Unit (LGU) Sanitation Offices

The sanitation offices of barangays and LGUs can utilize Smart-Bin's data readings to aid in garbage collection scheduling and management. The percentage of fullness and time to be full will help them optimize their collection routes and minimize the amount of fuel they need to use by making fewer unnecessary collections.

- Filipino Citizens

How about if the environment is cleaner, trash bins are not overflowed and there is increased awareness about waste among Filipino citizens who are residents, employees or students of schools, offices or other places? The hands-free lid would also improve the disposal of waste; making it easier and sanitary.

# II. PROTOTYPE DESIGN AND DEVELOPMENT

## 2.1 Prototype Design

[Describe and show the physical design of the Smart-Bin prototype here.]

## 2.2 Block Diagram

The block diagram of one Smart-Bin unit and the network around it is shown in Figure 37. It has two separate ESP32 microcontrollers, one in each of the two bins, both bins reporting to the same cloud services and user apps.

Figure 37

Block Diagram of the Smart-Bin System

The power supply is located in the top portion of the diagram, consisting of an 18650-battery shield in a closed compartment providing the circuit a regulated 5V. Bin A is powered by 4x 18650 cells and Bin B by 2x. The 5V is fed to the ESP32-WROOM-32D which supplies the 3.3V needed by the sensors, display and buzzer; the servo, HC-SR04 and PIR sensor operate directly from the 5V. ESP32 is a 240MHz dual-core Wi-Fi enabled micro-controller. It has firmware written in C++ and uses that to determine the fill percentage, indoor air quality (IAQ) estimate and tamper state, as well as controlling the lid and alarms and providing a local REST API.

The inputs are on the left. The HC-SR04 Ultrasonic sensor on GPIO 5 and 18 can be used to detect the distance between the lid and the garbage bin, and output the fill level information, the HC-SR501 PIR sensor on GPIO 27 can be used to sense whether a person approaches the garbage bin. Two sensors, BME680 (address 0x76) for temperature, humidity and air quality and BMI160 (address 0x68) for motion and tilt for tamper detection share this I2C bus. Using a 5kg load cell, it is connected to GPIO 14 and 32 via an amplifier module named HX711 to measure the weight of the trash. On the SPI, the touch panel XPT2046 accepts the COLLECT and PIN inputs.

The outputs are to the right. The lid is opened and closed by the SG90 servo on GPIO 13 (PWM). The fill level is indicated by an RGB LED (GPIO 25, 26 and 33) in green, amber or red. Beeps, warnings and the tamper siren come from a passive buzzer on GPIO 4. The fill level and the bin status are displayed on the 2.4 in. ILI9341 TFT display (also SPI).

Three paths of data exit the bin via Wi-Fi (802.11 b/g/n) as shown in the bottom half of the image. The local REST API (/api/status, /api/command, /api/calibrate and /api/simulate) responds in ~1s and provides full control of the bin. The status is sent to the Firebase Realtime Database every 5 to 60s and to ThingSpeak every 15 to 30s, as well as the LOCK, UNLOCK and COLLECT commands (the latter via ThingSpeak's TalkBack feature). Data goes up and commands come down to two user apps. The web dashboard runs in a browser on the same Wi-Fi and gives the operator a live view, settings and simulation, while the Android app gives operators and collectors alerts, the COLLECT command and the Collection Priority list.

## 2.3 System Flowchart

Figure 38

System Flowchart of the Smart-Bin

Note. Figure 38 shows the main program loop that runs on each bin. After start-up, seven parts run side by side on every pass of the loop, and their results meet at one update step.

Upon powering on the bin, the ESP32 starts, checks its sensors and loads its saved settings, with a boot screen and a beep. It then examines if a Wi-Fi network is saved. If not, the user opens WI-FI SETUP on the touchscreen and enters the collect PIN, and the app or dashboard pairs with the bin over Bluetooth. The 6-digit code on the bin's screen is typed into the pairing box (a device paired before skips it, and three wrong codes stop pairing for 60s). The bin returns a list of all the Wi-Fi networks within its range, and the user selects one and enters its password, after which the bin beeps to join. This loop is split into 7 parallel sections (1-7 on chart).

The lid part (1) reads the PIR sensor. If the bin is free (not locked, tampered with, collecting or being calibrated), it beeps to open the lid. Otherwise, bin lid remains closed and a beep is heard when motion occurs with the lid closed. The lid beeps and closes 5 seconds after the movement is stopped. The fill part (2) reads the weight from the HX711, and takes the median of 5 HC-SR04 readings for the fill percentage. At the default setting of 80%, the LED flashes red and an alert message is sent every 30s until the bin is collected (or locked) when the fill is at the limit. When set to auto-lock, the bin itself locks as well. If lower than the limit the LED is green or yellow. The air part (3) compares the measured temperature, humidity and estimated air quality of the BME680 with their limits (40C, 85% and an index of 200). The air quality is an approximation based on the gas reading and should be warmed up for approximately 5 minutes. When over a limit the buzzer sounds and an alert is sent every 5 minutes, whilst a short beep indicates when the reading is back under.

The BMI160 tilt angle is monitored by the tamper part (4). If the tilt exceeds 30°, a countdown of 10 seconds begins with a "tick" every second. The alarm automatically resets if the bin is steady once more. Until an operator sends RESET TAMPER, the alarm latches when it remains tilted: the siren sounds every 2s, the LED is red, the lid remains closed and an alarm message is generated. The screen part (5) is used for handling the COLLECT touch command. Once collect PIN is entered lid will open and alarms will be muted until DONE is pressed and the lid is closed and the fill level is reset, or if the collect PIN is wrong, an error beep is provided. Finger on screen for 5s to start touch calibration. The command’s part (6) receives the following commands from the app or dashboard: LOCK, UNLOCK, COLLECT, calibration (CAL) and simulation (SIM). Bin beeps if an unsafe one is attempted such as COLLECT during tamper alarm and CAL and SIM are Wi-Fi only. The network part (7) beeps once when the Wi-Fi is lost and once when it returns.

All seven paths then lead to an UPDATE step. The two arrows between this step and each cloud reflect that the reading and the alerts are sent to Firebase and the ThingSpeak, and the LOCK, UNLOCK and COLLECT commands are returned to it. This new status is then reflected on the screen, on the LED and in the apps and the process is repeated.

Figure 39

Operator Flowchart of the Smart-Bin Apps

Note. As seen in Figure 39, an operator can do the following actions in the Web Dashboard or in the Android App

.

In both apps, the collector mode is the default, and the operator enters his 4- to 8-digit PIN. A wrong PIN requires a delay between attempts, with 30s after 5 failed attempts and the delay for subsequent failed attempts doubling. The app puts itself in operator mode with the appropriate PIN. The operator can then add or edit a bin, giving it a name, a place, the IP address and cloud name for Firebase and ThingSpeak, and the bin is saved in the app.

There are five tasks branching out from there. Everything that a collector can do is Collector (1), as shown in Figure 40: alerts, COLLECT and DONE, LOCK and UNLOCK, charts and history, and setting up the Wi-Fi over Bluetooth. Tamper (2) is for a bin with an alarm that latched. The operator checks the bin and stands it up and then sends a RESET TAMPER to clear the alarm. Calibrate (3) requires bin to be on same Wi-Fi. The operator sets new limits (fill range, alert limits, BME680 offsets, lid angles, weight tare and more) and these limits are stored in the flash memory of the bin. Calibration is disabled if the bin is not connected to Wi-Fi. The touch calibration (4) can be initiated from the dashboard or app or by holding a finger on the bin's screen for 5s. Operator presses four crosses sequentially and result is stored on the bin (if pressed too close, it is a fail and the operator retries).

Simulate (5) keeps the value of any sensor for demonstration purposes. A SIMULATION banner will appear on the screen while it is on until the operator returns everything to AUTO. The sound, vibration and theme are set by Settings (6) and the operator can change the PIN in the settings after entering the PIN. The result of whichever task is used then iterates through the same 2-way UPDATE process with both the Firebase and the ThingSpeak services and the app will display the new status, plus the charts and alerts.

Figure 40

Collector Flowchart of the Smart-Bin Apps

Note. As seen in Figure 40, the collector's job in the web dashboard or in the Android app.

The apps are opened to collector mode which is the default mode and no pin is required. The collector uses Bluetooth to set up the bin if not already connected to Wi-Fi. The 6 digit code displayed on the screen of the bin is entered into the pairing box of the phone (code will be valid for 60s, 3 failed attempts will lock pairing for 60s, a device that has been paired before will not require a code). The app then lists the Wi-Fi networks the bin can see, and the collector picks one and enters its password. After the bin joins the network, there are four tasks available.

Alerts (1) are issued for a full bin, tampering, bad air, lock and pickup. Each has its own sound and vibration and they come even when the app is not in use. The primary task is collect (2). The Collection priority list is a ranking of the bins, with the first bin in the list being the one with the most capacity, and the time to fill each bin is shown. The collector taps COLLECT in the app or on the screen, and enters the collect PIN at the bin. Lid opens and alarms stop. The collector then empties the bin and taps DONE (no PIN required) and the lid will then close and the fill level will reset. A bin refuses COLLECT, with a beep, while a tamper alarm or a calibration is on.

Lock (3) allows the collector to lock or unlock a bin, while resetting a tamper alarm, calibration and simulation stay with the operator. View (4) displays the charts, alert history, and status and collect PIN in read-only mode. All tasks are completed with the same step in Firebase and ThingSpeak and the app updates the status.

## 2.4 Schematic Design

The circuit of each unit is drawn as one schematic split into three sheets: the microcontroller, the actuators and outputs, and the sensors and inputs. The order of the sheets is: Figures 41, 42, 43. Instead of long wires between sheets, each connection to the ESP32 has a net label (IO25, etc.) and a bunch of pins with the same net label are grouped together.

Figure 41

Schematic Diagram, Sheet 1: Microcontroller

U1 is the ESP32-WROOM-32D on a 38-pin board. The 4x or 2x 18650-battery shield, BT1, provides the 3.3V rail for the sensors, the display and the buzzer (pin 1) and a regulated 5V to pin 19. C1 and C2 (100nF and 10µF) smooth the 3.3V rail, and C3 and C4 (100nF and 10µF) do the same at the battery input. The ground pins share one GND net. Pins which are not configured in the design (such as EN, TXD, RXD, GPIO0, GPIO2) are identified as "no-connect". According to the note on the sheet, the battery's 5V should not be used while code is being uploaded; rather, it should only be used after the code is uploaded.

Figure 42

Schematic Diagram, Sheet 2: Actuators and Outputs

LED1 is a common cathode RGB LED KY-016. Its red, green and blue legs have 1kΩ resistors connected to IO25, IO26 and IO33. BZ1 is a buzzer module (MH-FMD) that operates on 3.3V and is controlled by IO4. The SG90 micro servo (M1) operates on the 5V power supply, and is controlled by IO13 via PWM, with C5 (100nF) and C6 (1000µF) placed across the power supply to filter current spikes as the servo moves.

U2 is the 2.4 in. ILI9341 touchscreen (240 × 320, SPI). It runs on 3.3V, with C7 (100nF) across its supply. The display and its touch controller share the SPI clock and data lines on IO19, IO23 and IO35, and each has its own chip select: IO17 for the display and IO15 for touch. DC goes to IO16 and the touch interrupt T_IRQ to IO36. R1 (10kΩ) pulls T_IRQ to 3.3V, the line remains high until a touch pulls it down. The RESET and the backlight pin (LED) are connected to 3.3V and the SD card pins are not connected.

Figure 43

Schematic Diagram, Sheet 3: Sensors and Inputs

U3, the HC-SR501 PIR sensor, runs on 5V and sends its output to IO27, with C8 (100nF) across its supply. The HC-SR04 (U4) operates on 5V also with the C9 (100nF) from supply and TRIG on IO5. It outputs a 5V pulse to its ECHO pin, which is then divided by R2 (1kΩ) and R3 (2.2kΩ) to produce ~3.4V before reaching IO18, and is thus very close to the 3.3V level that the esp32 input expects for it can tolerate up to 3.6V.

U5 (BME680) and U6 (BMI160) both run on 3.3V, with C10 and C11 (100nF each) across their supplies, and share the I2C bus, with SCL on IO22 and SDA on IO21. Tying the BME680's SDO pin to ground sets its address to 0x76, and tying the BMI160's SA0 pin to ground sets its address to 0x68, so the two do not clash. M2, the 5kg load cell, connects its red, black, white and green wires to the E+, E−, A− and A+ inputs of U7, the HX711. The HX711 runs on 3.3V, with C12 (10µF) and C13 (100nF) across its supply, and sends its data on IO32 with its clock on IO14.

## 2.5 Component Materials

This section describes the main electronic parts used in each Smart-Bin unit. Both bins use the same parts except for the battery shield. Figures 44 to 54 show each part, followed by what it does and how the system uses it.

Figure 44

ESP32 DevKit V1 (38-Pin, ESP32-WROOM-32D)

Note. Image from Sigmanortec (n.d.).

The ESP32 DevKit V1 is a development board built around the ESP32-WROOM-32D module. The module has a dual-core processor that runs at up to 240MHz, with Wi-Fi and Bluetooth on the same chip. The 38-pin board provides an additional USB connector for uploading code and power, a voltage regulator for 3.3V power and pin headers for breadboarding.

There is one board for each bin and each board governs all of the contents of that bin. It reads all six inputs (the HC-SR04, PIR sensor, BME680, BMI160, load cell and touchscreen), calculates the fill percentage, air quality reading and tamper state, and controls the servo, LED, buzzer and display. Providing the local REST API over Wi-Fi, it receives the status update command from the dashboard and the Android application and sends status update to Firebase and ThingSpeak.

Figure 45

HC-SR04 Ultrasonic Distance Sensor

Note. Image from SparkFun Electronics (n.d.).

The HC-SR04 is a sound-based distance sensor. It outputs a 40kHz ultrasonic burst when a short pulse is received at its TRIG pin and will keep its ECHO pin high until the echo is received. The distance is determined by the width of that pulse which varies in the range of 2cm to 4m. The sensor runs on 5V.

In Smart-Bin it is installed under the lid and points downward, and thus measures the distance between the lid and the top of the trash. The firmware converts that distance into the fill percentage displayed on the LED, touch screen and the apps and once the fill level reaches the fill limit (80% by default), the full-bin alert is activated. TRIG is wired to IO5, and ECHO is wired to IO18 through a divider of R2 (1kΩ) and R3 (2.2kΩ), which brings the 5V pulse down to about 3.4V, close to the 3.3V input level of the ESP32.

Figure 46

HC-SR501 PIR Motion Sensor

Note. Image from Circuitrocks (n.d.-c).

The HC-SR501 is a passive infrared (PIR) sensor. It detects the modulation of infrared light as a warm body passes in front of its white dome lens and its output pin will be "high" when motion is detected. The sensitivity and output high time is determined by two trimmers on the board.

It is used by Smart-Bin to determine when someone approaches the bin. When the output on IO27 is high and the bin is available for opening, the bin beeps and the servo raise the lid and the user never has to touch it. The lid will close 5 seconds (user definable length) after movement ceases. During COLLECT mode motion sensing is suppressed and the lid remains open while the collector is operating.

Figure 47

BME680 Environmental Sensor

Note. Image from Robu.in (n.d.).

The BME680 is a single chip sensor for measuring temperature, humidity, VOC (volatile organic gases) and air pressure. Based on the gas reading, an estimated indoor air quality (IAQ) rating can be obtained. It is powered by 3.3V and communicates via I2C.

In Smart-Bin it monitors the air within the bin. When the temperature, humidity or estimated air quality goes over its limit, the firmware sounds an alarm and sends an alert, even if the bin isn't full yet, because a high temperature may mean a fire or hot material. It is located at I2C address 0x76 (SDO tied to ground).

The purpose of this is using the combination of temperature, humidity, and gas reading  (IAQ) in order to determine by the collector or operator how much rotting trash is in the Smart-Bin for smell-sensitive areas. The IAQ value is an estimate from the gas reading, and it needs about 5 minutes after power-up to settle.

Figure 48

BMI160 Six-Axis Inertial Measurement Unit

Note. Image from ElectroPeak (n.d.).

BMI160 is a 3-axis accelerometer and 3-axis gyroscopes integrated onto one chip. It is an I2C acceleration and rotation sensor that operates on 3.3V.

It is used by Smart-Bin for tamper detection. If the bin is tilted, the readings exceed the range limits and the firmware sets the Tamper alarm, activates the buzzer and prevents the lid opening. It is on the same I2C bus as the BME680 with the address 0x68 by shorting out SA0 to ground. The tamper check is paused during COLLECT mode.

Figure 49

HX711 Load Cell Amplifier and 5kg Load Cell

Note. The HX711 board and the 5kg load cell are shown on a display base. Image from ElectronicsComp.com (n.d.).

A load cell is a metal bar with strain gauges designed so that as the load is applied, the bar's resistance varies slightly. The bar here is a 5kg model, and has a very small voltage output, which is amplified and converted in the HX711 board by a 24 bit analog to digital converter. The result is read by the ESP32 using two pins: DT (IO32) and SCK (IO14).

The two together measure the garbage. Weight is a second check on the fill level: wet waste can be heavy before the bin is full, and a bin full of light waste can be tall without being heavy. The weight is displayed on the dashboard and the app, and stored to the cloud with every measurement.

Figure 50

SG90 Micro Servo Motor

Note. Image from The Pi Hut (n.d.).

The SG90 is a small hobby servo which will rotate its output arm to a specific angle (roughly 180 degrees) depending on the width of a PWM pulse. Operates on approximately 5V and can be attached directly to the bin.

Servo is used to open and close the bin lid. It is managed by ESP32, it will open when it is detected people, or when the collector taps COLLECT, it will close after 5s (user settable) when people are not detected or the collector taps DONE. When the bin is locked, full or flagged for tampering, the firmware keeps the servo in the closed position.

Figure 51

2.4-in. TFT Touchscreen (ILI9341 With XPT2046 Touch Controller)

Note. Front, back, and angled views of the module. Image from LCDWIKI (n.d.).

This module is a 2.4 in., 240 × 320 color display driven by the ILI9341 controller over SPI. The same SPI bus also has a separate XPT2046 controller for the resistive touch layer on top. There is also an SD card slot on the board, which isn't used by Smart-Bin.

The bin's local screen is the display. It indicates the fill level, sensor readings and the bin status (LOCKED and TAMPER). The touch layer provides the collector with the buttons COLLECT, PIN and DONE, making it possible to empty a bin without the app.

Figure 52

KY-016 RGB LED Module

Note. Image from Circuitrocks (n.d.-d).

The KY-016 is a small board with a common-cathode RGB LED, a small package containing the red, green and blue chips. When the three colors are combined, other colors are created such as amber (red + green). A 1kΩ resistor is used for each colour to control the current.

The fill level is indicated on the LED, green if there is room in the bin, amber when it is filling up and red when the bin is full or a warning is triggered. It is driven from IO25, IO26 and IO33 and turns green again after the collector taps DONE.

Figure 53

Passive Buzzer Module (MH-FMD)

Note. Image from QuartzComponents (n.d.).

Passive buzzer: No built-in oscillator so it only buzzes if it is pulsed by the controller with a changing signal. This will allow the firmware to produce various tones and patterns, rather than just a single beep.

It is used by Smart-Bin for sound feedback from IO4. It beeps when lid is opened, alerts user if the BME680 senses high temp or humidity or poor air quality, and activates the siren if BMI160 senses tampering. It remains quiet while in COLLECT mode.

Figure 54

18650-Battery Shields for Bin A (4-Cell) and Bin B (2-Cell)

Note. Left: four-cell shield used in Bin A (Circuitrocks, n.d.-b). Right: two-cell shield used in Bin B (Circuitrocks, n.d.-a).

The 18650-battery shield comprises lithium-ion 18650 cells which will provide the electricity needed and convert its voltage to a regulated 5V output. It also features a charging USB port for the cells. Bin A are equipped with a four-cell V9 shield (5V/3A), while Bin B are equipped with a two-cell shield.

The shield supplies power to the entire bin, without the need for a wall outlet, which allows a bin to be set anywhere that Wi-Fi is available. It is placed in a closed compartment and connected to the 5V of the ESP32 which provides 3.3V for the sensors. As mentioned in the schematic, the battery should not be connected during code upload via USB.

# III. TESTING, RESULTS, AND CONCLUSION

## 3.1 Test Cases

[Introduce the test cases here. Each specific objective is tested below.]

### Specific Objective 1: Sensor Monitoring and Detection

[Insert test cases TC1.1 to TC1.6 here.]

### Specific Objective 2: Real-Time Monitoring and Alerts

[Insert test cases TC2.1 to TC2.6 here.]

### Specific Objective 3: Bin Control and Collection Prioritization

[Insert test cases TC3.1 to TC3.6 here.]

## 3.2 Results and Discussion

[Present and discuss the test results here.]

## 3.3 Conclusion

[Write the conclusion here.]

## 3.4 Recommendations

[Write the recommendations here.]

# REFERENCES

Aasish, V. G., Mari Shankar, C., & Shanmugasundaram, M. (2025). FreeRTOS based smart waste management system. In 2025 Innovations in Power and Advanced Computing Technologies (i-PACT) (pp. 1–7). IEEE. https://doi.org/10.1109/i-PACT65952.2025.11307820

Abdullahi, A., Mohammed, A., Bonet, M. U., El-Suleiman, A., Ahmad, R. B., & Chollom, T. D. (2024). Development of a smart waste management system with automatic bin lid control for smart city environment. EAI Endorsed Transactions on Smart Cities, 7(3). https://doi.org/10.4108/eetsc.4385

Alnanih, R., Elrefaei, L., & Al-Ahwal, A. (2025). Advancing sustainability through an IoT-driven smart waste management system with software engineering integration. Sustainability, 17(21), Article 9803. https://doi.org/10.3390/su17219803

Alnuaimi, A. N., Aydin, R., & Al-Shihabi, S. (2024). Optimizing municipal solid waste collection using smart bins: A case study in United Arab Emirates. In 2024 IEEE International Conference on Technology Management, Operations and Decisions (ICTMOD) (pp. 1–6). IEEE. https://doi.org/10.1109/ICTMOD63116.2024.10878234

Alourani, A., Ashraf, M. U., & Aloraini, M. (2025). Smart waste management and classification system using advanced IoT and AI technologies. PeerJ Computer Science, 11, Article e2777. https://doi.org/10.7717/peerj-cs.2777

Asido, D. (2024, January 19). Baguio students develop smart garbage bins. The Post. https://thepost.net.ph/news/campus/baguio-students-develop-smart-garbage-bins/

Awotayo, M. M., & Dodo, U. A. (2026). Design and implementation of an IoT-based smart bin for real-time fill level monitoring. Journal of Science Technology and Education, 14(1), 344–351. https://www.researchgate.net/publication/400748751

Ayyanagoudar, A., Mohankar, A., Gharte, J., Pasnsare, P., Ainapure, B., & Appasani, B. (2024). A smart bin with real-time monitoring and garbage level tracking using IoT. In 2024 11th International Conference on Reliability, Infocom Technologies and Optimization (Trends and Future Directions) (ICRITO) (pp. 1–7). IEEE. https://doi.org/10.1109/ICRITO61523.2024.10522263

Azmi, A. Z., Sahar, N. M., & Islam, M. T. (2024). Smart waste management system using LoRa technology. E3S Web of Conferences, 599, Article 04007. https://doi.org/10.1051/e3sconf/202459904007

Bahga, A., & Madisetti, V. (2014). Internet of things: A hands-on approach.

Balachandran, N., & Talavera, O. (2023). Apparatus and method for waste monitoring and analysis (U.S. Patent No. 11,788,877). U.S. Patent and Trademark Office. https://patents.google.com/patent/US11788877B2

Banciu, C., Florea, A., & Popa, M. (2025). IoT-enabled waste collection: Route optimization and environmental impact reduction using smart technologies. In 2025 29th International Conference on System Theory, Control and Computing (ICSTCC) (pp. 782–788). IEEE. https://doi.org/10.1109/ICSTCC66753.2025.11240462

Bigbelly. (n.d.). CLEAN software. Retrieved October 19, 2026, from https://bigbelly.com/products/clean-software

Bosch Sensortec. (n.d.). BME680 gas sensor (Document No. BST-BME680-DS001) [Data sheet]. Retrieved October 19, 2026, from https://www.bosch-sensortec.com/media/boschsensortec/downloads/datasheets/bst-bme680-ds001.pdf

Brown, W. J. (2006). Compactor service and monitoring system (U.S. Patent No. 7,145,450). U.S. Patent and Trademark Office. https://patents.google.com/patent/US7145450B2

Circuitrocks. (n.d.-a). 2 channel 18650-battery holder protection board. Retrieved October 15, 2026, from https://circuit.rocks/products/2-channel-18650-battery-holder-protection-board

Circuitrocks. (n.d.-b). 4 channel 18650-battery holder protection board. Retrieved October 15, 2026, from https://circuit.rocks/products/4-channel-18650-battery-holder-protection-board

Circuitrocks. (n.d.-c). HC-SR501 PIR motion sensor module. Retrieved October 15, 2026, from https://circuit.rocks/products/pir-motion-sensor-module-digital

Circuitrocks. (n.d.-d). KY-016 RGB 3 color full-color LED module. Retrieved October 15, 2026, from https://circuit.rocks/products/ky-016-rgb-3-color-full-color-led-module

Compology. (n.d.). Image-based container monitoring built for waste. Retrieved October 19, 2026, from https://compology.com/

Devitt, S., & Walsh, B. (2021). Techniques for optimally sensing full containers (U.S. Patent No. 11,054,545). U.S. Patent and Trademark Office. https://patents.google.com/patent/US11054545B2

Di Giugno, M., Giovanelli, F., Tiraboschi, S., & Villa, P. (2016). Measuring device, container means and system for monitoring and managing container means (European Patent No. 2516974). European Patent Office. https://patents.google.com/patent/EP2516974B1

Ecological Solid Waste Management Act of 2000, Republic Act No. 9003 (2001). https://lawphil.net/statutes/repacts/ra2001/ra_9003_2001.html

Ecube Labs. (n.d.). Waste analytics platform: CleanCityNetworks (CCN). Retrieved October 19, 2026, from https://www.ecubelabs.com/waste-analytics-platform/

ElectronicsComp.com. (n.d.). 5 kg load cell with HX711 module shell and 4P DuPont wire kit. Retrieved October 15, 2026, from https://www.electronicscomp.com/5-kg-load-cell-with-hx711-module-shell-and-4p-dupont-wire-kit

ElectroPeak. (n.d.). GY-BMI160 module 6DOF 6 axis angular velocity. Retrieved October 15, 2026, from https://electropeak.com/gy-bmi160-module-6dof-6-axis-angular-velocity

Enache, B.-A., Banica, C.-K., & Bogdan, A. G. (2024). Intelligent trash level detection for sustainable urban waste management. The Scientific Bulletin of Electrical Engineering Faculty, 24(1), 56–59. https://doi.org/10.2478/sbeef-2024-0010

Evreka. (2020). Fill level sensor. Retrieved October 19, 2026, from https://evreka.co/solutions/fill-level-sensor/

Gajula, R., Karanam, M., Vakkapatla, D. C., Endrapu, K. R., Raju, V. V. R., Reddy, S. B., Aljanabi, S. O., Koli, S., & Mahajan, S. (2025). Sustainable waste management solution: ESP32-powered smart bin. AIP Conference Proceedings, 3263(1), Article 140006. https://doi.org/10.1063/5.0261359

González, G. E., Escobar, E. R., Carvajal, I. I., & Bento, A. C. (2024). Smart trash bins: Integration of IoT with AI for efficient waste management. In 2024 IEEE International Humanitarian Technologies Conference (IHTC) (pp. 1–6). IEEE. https://doi.org/10.1109/IHTC61819.2024.10855147

Greenwalt, M. (2020, June 2). Bin-e creates contactless smart bin. Waste360. https://www.waste360.com/recycling/bin-e-creates-contactless-smart-bin

Gupta, S., Gautam, R., & Muneer, A. (2025). IoT-based e-tracking system for waste management. In 2025 12th International Conference on Reliability, Infocom Technologies and Optimization (Trends and Future Directions) (ICRITO) (pp. 1–6). IEEE. https://doi.org/10.1109/ICRITO66076.2025.11241930

Ishaq, A., Mohammad, S. J., Bello, A. D., Wada, S. A., Adebayo, A., & Jagun, Z. T. (2025). Smart waste bin monitoring using IoT for sustainable biomedical waste management. Environmental Science and Pollution Research, 32(32), 19434–19449. https://doi.org/10.1007/s11356-023-30240-1

Karthi, S. P., Aravind, A., Dharineesh, V., Dhayanidhi, J. V., & Dinesh Kumar, J. R. (2025). IoT enabled smart dustbin with RFID-based access control and Blynk integration. In 2025 IEEE First International Conference on Innovations in Engineering and Next-Generation Technologies for Sustainability (ICINVENTS). IEEE. https://doi.org/10.1109/ICINVENTS64613.2025.11402326

Kekäläinen, F. (2018). Method and system for monitoring and communicating fill rate of container (U.S. Patent No. 9,930,429). U.S. Patent and Trademark Office. https://patents.google.com/patent/US9930429B2

Kekalainen, F., & Engstrom, J. (2014). Smart waste collection system and method (International Patent Application Publication No. WO 2014/079586 A1). World Intellectual Property Organization. https://patents.google.com/patent/WO2014079586A1

Kekalainen, F., & Engstrom, J. (2018). Sensor device for smart waste collection systems and method (U.S. Patent No. 9,927,278). U.S. Patent and Trademark Office. https://patents.google.com/patent/US9927278B2

Keller, J. R., Redmond, M. O., & Mirza, S. A. (2024). System and method for measuring the fill level of a trash can using a sensor (U.S. Patent No. 12,125,361). U.S. Patent and Trademark Office. https://patents.google.com/patent/US12125361B2

Kumar, K. R., Pavan, O. S., Azeez, S. A., Suresh, C. V., Sree, K. R., & Reddy, P. U. (2025). IoT enabled route optimization for smart waste management using machine learning. i-manager's Journal on IoT and Smart Automation, 3(2), 11-23. https://doi.org/10.26634/jiot.3.2.21684

Kurani, H. B. (2023). Smart waste bin sensor device and methods for waste management system (U.S. Patent No. 11,702,280). U.S. Patent and Trademark Office. https://patents.google.com/patent/US11702280B2

Kuzhin, M. F., Joshi, A., Mittal, V., Khatkar, M., & Guven, U. (2024). Optimizing waste management through IoT and analytics: A case study using the Waste Management Optimization Test. BIO Web of Conferences, 86, Article 01090. https://doi.org/10.1051/bioconf/20248601090

LCDWIKI. (n.d.). 2.4inch SPI module ILI9341 SKU:MSP2402. Retrieved October 15, 2026, from https://www.lcdwiki.com/2.4inch_SPI_Module_ILI9341_SKU:MSP2402

Leones, J. R. (2025, March 3). Philippines ministerial statement [Paper presentation]. High-Level 12th Regional 3R and Circular Economy Forum in Asia and the Pacific, Jaipur, India. https://uncrd.un.org/sites/uncrd.un.org/files/12th3r_ps1-philippines.pdf

Maulida, A., Fayzalmi, C. S., Anaam, F. A. C., Ardiansyah, D., Gunawan, M. A., & Ridwan, N. (2025). Design of IoT-based smart trash bin monitoring using ESP32 and Firebase. Jurnal Komputer, Teknologi Informasi dan Sistem Informasi (JUKTISI), 4(2), 420-428. https://doi.org/10.62712/juktisi.v4i2.420

Mehta, H., Deriya, V., Sankhe, D., Kanani, P., Pandya, G., & Shelke, V. (2025). Smart bins, smarter cities: IoT-driven waste collection with real-time sensing and methane mapping. Mathematical Modelling of Engineering Problems, 12(6), 2110–2122. https://doi.org/10.18280/mmep.120626

Mehta, S., & Rathour, A. (2024). Innovative waste management: The efficiency of IoT-enabled smart bins. In 2024 7th International Conference on Circuit Power and Computing Technologies (ICCPCT) (pp. 467–471). IEEE. https://doi.org/10.1109/ICCPCT61902.2024.10672653

Mugaruka, O. M., Kala, J. R., Ndikumagenge, J., van Schadewijk, R., & Nibitanga, R. (2026). Design of an IoT sensor-based network architecture for waste collection in Bukavu city (DRC). Discover Internet of Things, 6(1), Article 168. https://doi.org/10.1007/s43926-026-00479-4

Mullaney, G. (2016). Refuse container with weight indicator and danger alerting (U.S. Patent No. 9,347,821). U.S. Patent and Trademark Office. https://patents.google.com/patent/US9347821B1

Muthukumar, K., Siddarth, T., Udhayakumar, K., & Saratha, M. (2024). Smart bin: AI-based waste segregation with monitoring system. International Journal of Innovative Research in Technology, 11(6), 1293–1298. https://ijirt.org/Article?manuscript=169606

Neumann, R. H., & Allison, C. B. (1991). Automated trash management system (U.S. Patent No. 5,016,197). U.S. Patent and Trademark Office. https://patents.google.com/patent/US5016197A

Nordsense. (n.d.). Intelligent routing. Retrieved October 19, 2026, from https://nordsense.com/intelligent-routing/

Philippine Statistics Authority. (2026). Highlights of the Compendium of Philippine Environment Statistics 2016–2025, Component 3: Residuals. https://psa.gov.ph/statistics/environment-statistics/highlights/component-3-residuals

Poss, J., Satwicz, J., Richmond, B., & Taylor, M. (2006). Solar powered compaction apparatus (U.S. Patent No. 7,124,680). U.S. Patent and Trademark Office. https://patents.google.com/patent/US7124680B2

QuartzComponents. (n.d.). Passive buzzer module with PCB. Retrieved October 15, 2026, from https://quartzcomponents.com/products/passive-buzzer-module

Rathnayake, R. M. P. M. D., Chanaka, T. M. M., Dabare, P. T. R., & Hewagamage, K. P. (2024). IoT-enabled dual-sensing smart waste management system: Enhancing urban cleanliness and sustainability in smart cities. In 2024 8th SLAAI International Conference on Artificial Intelligence (SLAAI-ICAI). IEEE. https://doi.org/10.1109/SLAAI-ICAI63667.2024.10844960

Robu.in. (n.d.). BME680 digital temperature humidity pressure sensor CJMCU-680 high altitude sensor module development board. Retrieved October 15, 2026, from https://robu.in/product/bme680-digital-temperature-humidity-pressure-sensor-cjmcu-680-high-altitude-sensor-module-development-board/

Saad, A. M., Jul, B. O., Basalamah, A., & Sayuti, S. (2023). IoT-based smart dustbin prototype. PROtek: Jurnal Ilmiah Teknik Elektro, 10(2), 120–125. https://doi.org/10.33387/protk.v10i2.6035

Sadikin, F. R., Hikmawati, E., & Aulia, S. (2025). TrashTrack: A smart waste monitoring system with constraint-based greedy algorithm for optimized collection routes. In 2025 IEEE International Conference on Artificial Intelligence for Learning and Optimization (ICoAILO) (pp. 262–268). IEEE. https://doi.org/10.1109/ICoAILO66760.2025.11155998

Sait, S., Aliyu, F. M., Bukar, S., Maina, B. S., & Lawal, D. U. (2025). Waste bin collection and monitoring method using waste bin sensors (U.S. Patent No. 12,450,575). U.S. Patent and Trademark Office. https://patents.google.com/patent/US12450575B1

Salleh, A., Mohamad, N. R., & Hashim, N. M. Z. (2024). Optimizing waste collection efficiency by NodeMCU enabled ultrasonic trash can surveillance and ThingSpeak dashboards. International Journal of Latest Technology in Engineering, Management & Applied Science, 12(12), 19-26. https://doi.org/10.51583/ijltemas.2023.121202

Sensoneo. (n.d.). Waste management system. Retrieved October 19, 2026, from https://www.sensoneo.com/products/waste-management-system/

Shahabdeen, J. A. (2016). Smart garbage bin (U.S. Patent Application Publication No. 2016/0176630 A1). U.S. Patent and Trademark Office. https://patents.google.com/patent/US20160176630A1

Shaharil, M. H. F., & Po’ad, F. A. (2024). Arduino-based smart dustbin with ultrasonic sensor for efficient waste management. Evolution in Electrical and Electronic Engineering, 5(2), 92–97. https://periodical.uthm.edu.my/index.php/eeee/article/view/17147

Shahibollah, R. A., Fadzal, N., Fadzal, A. N., Omar, S., Jaafar, A. N., & Ahmad, N. (2025). IoT-enabled smart dustbin for real-time waste monitoring and scalable hygiene management. In 2025 IEEE 16th Control and System Graduate Research Colloquium (ICSGRC) (pp. 75–80). IEEE. https://doi.org/10.1109/ICSGRC65918.2025.11159745

Shin, H. (2016). Inputting device for quantity and weight loss of food garbage container (Korean Patent No. 10-1581944). Korean Intellectual Property Office. https://patents.google.com/patent/KR101581944B1

Sigmanortec. (n.d.). ESP32-DevKitC ESP32-WROOM-32D WiFi BT 38P development board. Retrieved October 15, 2026, from https://sigmanortec.ro/en/esp32-devkitc-development-board-esp32-wroom-32d-38p

SparkFun Electronics. (n.d.). Ultrasonic distance sensor - 5V (HC-SR04). Retrieved October 15, 2026, from https://www.sparkfun.com/ultrasonic-distance-sensor-hc-sr04.html

Thangam, S., Gurupriya, M., Yadav, C. S. K., Harshith, M., & Kumar, M. S. (2024). A smart waste management system using ESP8266. In 2024 4th International Conference on Sustainable Expert Systems (ICSES) (pp. 1–5). IEEE. https://doi.org/10.1109/ICSES63445.2024.10762967

The Pi Hut. (n.d.). TowerPro servo motor - SG90 analog. Retrieved October 15, 2026, from https://thepihut.com/products/towerpro-servo-motor-sg90-analog

United Nations Department of Economic and Social Affairs. (n.d.). Goal 11: Make cities and human settlements inclusive, safe, resilient and sustainable. https://sdgs.un.org/goals/goal11

Wang, X., Chen, J., & Lin, Z. (2020). Trash can having sensing flip lid having settable sensing distance and control method therefor (European Patent No. 3 620 405 A1). European Patent Office. https://patents.google.com/patent/EP3620405A1

Wang, X., Zhang, Y., & Wang, Z. (2020). Garbage bin intelligent control ware and garbage bin supervisory systems (Chinese Patent No. CN111559590A). China National Intellectual Property Administration. https://patents.google.com/patent/CN111559590A

World Bank. (2022). An assessment of municipal solid waste plans, collection, recycling and disposal of Metro Manila (East Asia and Pacific Region: Marine Plastics Series). https://documents1.worldbank.org/curated/en/099125007222250938/pdf/P170994059e8e308c0a3ec02e0c4f57778b.pdf

Yan, S., & Chen, J. (2021). Smart city garbage management system (Chinese Patent No. CN 109324552 B). China National Intellectual Property Administration. https://patents.google.com/patent/CN109324552B

Yang, F., & Sandor, J. (2010). Trash can with power operated lid (U.S. Patent No. 7,781,995). U.S. Patent and Trademark Office. https://patents.google.com/patent/US7781995B2

Yang, S., Liao, D., Li, R., & Yu, Y. (2020). Capacity automated inspection garbage bin with infrared induction function (Chinese Patent No. CN 210504152 U). China National Intellectual Property Administration. https://patents.google.com/patent/CN210504152U

Yogesh, H. M., Shivalingappa, Suman, M., Vinodh, B., & Malini, V. L. (2025). IoT-enhanced smart dustbin with real-time weight and level monitoring using ESP32. International Journal of Advanced Research in Science, Communication and Technology, 5(10), 227-231. https://doi.org/10.48175/ijarsct-28841

# APPENDICES

## Appendix A

Gantt Chart

## Appendix B

Bill of Materials of the Smart-Bin Prototype

| Component | Qty | Unit price | Amount |
| --- | --- | --- | --- |
| ESP32 DevKit V1 (38-pin, WROOM-32D) | 2 | ₱250.00 | ₱500.00 |
| HC-SR04 ultrasonic sensor | 2 | ₱45.00 | ₱90.00 |
| HC-SR501 PIR motion sensor | 2 | ₱55.00 | ₱110.00 |
| SG90 micro servo motor (180°) | 2 | ₱65.00 | ₱130.00 |
| 2.4-in. TFT touch display (ILI9341 + XPT2046) | 2 | ₱280.00 | ₱560.00 |
| BMI160 six-axis inertial sensor module | 2 | ₱80.00 | ₱160.00 |
| BME680 environmental sensor | 2 | ₱400.00 | ₱800.00 |
| HX711 load cell amplifier board | 2 | ₱60.00 | ₱120.00 |
| 5kg straight bar load cell | 2 | ₱120.00 | ₱240.00 |
| KY-016 RGB LED module (common cathode) | 2 | ₱8.00 | ₱16.00 |
| MH-FMD passive buzzer module | 2 | ₱12.00 | ₱24.00 |
| 18650 battery shield, 4-cell (Bin A) | 1 | ₱310.00 | ₱310.00 |
| 18650 battery shield, 2-cell (Bin B) | 1 | ₱206.00 | ₱206.00 |
| 18650 Li-ion cell, 3.7V 2500mAh | 6 | ₱203.00 | ₱1,218.00 |
| Ceramic capacitor 100nF (pack of 10) | 2 | ₱75.00 | ₱150.00 |
| Electrolytic capacitor 10µF (pack of 10) | 1 | ₱39.00 | ₱39.00 |
| Electrolytic capacitor 1000µF (pack of 10) | 1 | ₱165.00 | ₱165.00 |
| Resistor 1kΩ (pack of 10) | 1 | ₱19.00 | ₱19.00 |
| Resistor 2.2kΩ (pack of 10) | 1 | ₱19.00 | ₱19.00 |
| Resistor 10kΩ (pack of 10) | 1 | ₱19.00 | ₱19.00 |
| Total |  |  | ₱4,895.00 |

Note. Quantities apply to both bins. The prices from the ESP32 to the buzzer are the market prices of the group as of their check in October 2026.

## Appendix C

User Interface Screens

[Include images of the touchscreen, Web dashboard and Android app here. Number them Figure C1, Figure C2 and so on, with the bold label and the italic title above each image.]

## Appendix D

Source Code

This appendix lists the firmware that runs on both Smart-Bin ESP32 boards. It is the firmware folder of the main branch of the group's repository, https://github.com/GenReyes/Smart-Bin-Embedded-Systems, as of October 24, 2026 (commit 1984c11), with comments added to explain what each part does and where to look when debugging. SmartBin.ino starts the modules, and each pair of .h and .cpp files is one module: shared state, sensors, control of the lid and alarms, buzzer, local Wi-Fi API, cloud (Firebase and ThingSpeak), Bluetooth Wi-Fi setup, and the touchscreen. The last two listings are the Firebase security rules and the script that packs the web dashboard into the firmware.

Two files are not listed. secrets.h holds the Wi-Fi password and the cloud keys and is kept out of the repository; secrets.example.h shows its layout. dashboard_gz.h is generated by embed_dashboard.py and holds the compressed web dashboard as a byte array. The Android app and the web dashboard are in the same repository.

firmware/SmartBin/SmartBin.ino

1	// ============================================================================

2	// SmartBin.ino - entry point

3	// Starts every module in a fixed order, then hands control to FreeRTOS tasks.

4	// The Arduino loop() is not used: it deletes itself, so all work happens in

5	// the tasks created below. If the board resets in a loop, open the Serial

6	// Monitor at 115200 baud; the first line printed here shows the bin ID and

7	// firmware version, and a crash before it points to a module's init().

8	// ============================================================================

9	// Smart-Bin ESP32 firmware. Group 2, BET-CpET 4A-M, TUP Manila.

10	// Before uploading: copy secrets.example.h to secrets.h and fill it in,

11	// and set BIN_NUMBER in config.h (1 for Bin A, 2 for Bin B).

12

13	#include "config.h"

14	#include "state.h"

15	#include "sensors.h"

16	#include "control.h"

17	#include "buzzer.h"

18	#include "net.h"

19	#include "cloud.h"

20	#include "ui.h"

21

22	void setup() {

23	  Serial.begin(115200);

24	  delay(200);

25	  Serial.printf("\nSmart-Bin %s firmware v%s\n", BIN_ID, FW_VERSION);

26

27	  // Order matters: the state module creates the mutex and queues every other

28	  // module uses, so it must run first. The UI comes next so the screen shows

29	  // the boot page while the sensors are still being probed.

30	  stateInit();

31	  uiInit();

32	  buzzerInit();

33	  controlInit();

34	  sensorsInit();

35	  netInit();

36

37	  // Task layout: name, stack bytes, priority, core. Core 1 runs the time

38	  // critical work (control has the highest priority so the lid and alarms

39	  // react first); core 0 runs the slow network jobs next to the Wi-Fi stack.

40	  // A "stack canary" or "stack overflow" reset names the task whose stack

41	  // size here is too small.

42	  xTaskCreatePinnedToCore(uiTask, "ui", 8192, nullptr, 2, &hUi, 1);

43	  xTaskCreatePinnedToCore(cloudTask, "cloud", 12288, nullptr, 1, &hCloud, 0);

44	  xTaskCreatePinnedToCore(pushTask, "push", 4096, nullptr, 2, &hPush, 0);

45	  xTaskCreatePinnedToCore(controlTask, "control", 4096, nullptr, 4, &hControl, 1);

46	  xTaskCreatePinnedToCore(sensorTask, "sensor", 4096, nullptr, 3, &hSensor, 1);

47

48	  // Interrupts only wake a task; no work is done inside the ISR itself.

49	  attachInterrupt(digitalPinToInterrupt(PIN_PIR), isrPir, CHANGE);

50	  attachInterrupt(digitalPinToInterrupt(PIN_TOUCH_IRQ), isrTouch, FALLING);

51	#if USE_IMU_INT

52	  pinMode(PIN_IMU_INT, INPUT);

53	#endif

54

55	  // The local web server starts before Wi-Fi joins so it is ready the moment

56	  // an IP address is given (see onWifiEvent in net.cpp).

57	  startWebServer();

58	  wifiConnect();

59	}

60

61	// Not used. Deleting the loop task frees its stack for the FreeRTOS tasks.

62	void loop() { vTaskDelete(nullptr); }

firmware/SmartBin/config.h

1	// ============================================================================

2	// config.h - build-time settings for one bin

3	// Pins, timing constants and the Bluetooth UUIDs. Values that users may

4	// change at run time (thresholds, PIN, sounds) live in struct Cal in state.h

5	// instead and are saved in flash.

6	// ============================================================================

7	#pragma once

8	#include <Arduino.h>

9

10	// Which bin this board is: 1 = Bin A (bin01), 2 = Bin B (bin02).

11	// Change BIN_NUMBER before flashing the second board. Both bins running the

12	// same number will overwrite each other's data in Firebase and ThingSpeak.

13	#ifndef BIN_NUMBER

14	#define BIN_NUMBER 1

15	#endif

16

17	#define FW_VERSION "1.0.0"

18

19	// Pins, from the Implementation Plan v2 pin table.

20	// GPIO 34 to 39 are input only on the ESP32, which is why the input lines

21	// SPI MISO 35, touch IRQ 36 and IMU INT 34 sit there. The buzzer is on GPIO 4,

22	// not GPIO 12: GPIO 12 is a boot strapping pin, and with the buzzer holding it

23	// high the ESP32 would not boot.

24	constexpr int PIN_TRIG = 5;

25	constexpr int PIN_ECHO = 18;

26	constexpr int PIN_BUZZER = 4;

27	constexpr int PIN_SERVO = 13;

28	constexpr int PIN_HX_SCK = 14;

29	constexpr int PIN_HX_DT = 32;

30	constexpr int PIN_TOUCH_CS = 15;

31	constexpr int PIN_TFT_DC = 16;

32	constexpr int PIN_TFT_CS = 17;

33	constexpr int PIN_SPI_MOSI = 19;

34	constexpr int PIN_SPI_SCK = 23;

35	constexpr int PIN_SPI_MISO = 35;

36	constexpr int PIN_TOUCH_IRQ = 36;

37	constexpr int PIN_SDA = 21;

38	constexpr int PIN_SCL = 22;

39	constexpr int PIN_LED_R = 25;

40	constexpr int PIN_LED_G = 26;

41	constexpr int PIN_LED_B = 33;

42	constexpr int PIN_PIR = 27;

43	constexpr int PIN_IMU_INT = 34;     // optional BMI160 INT1 wire; set USE_IMU_INT to 1 once it is wired

44

45	// Hardware polarity switches. If the LED shows the wrong colors or the buzzer

46	// is silent while the Serial log says it should sound, flip these.

47	#define USE_IMU_INT 0

48	#define BUZZER_ACTIVE_LOW 0          // 1 for "low level trigger" modules

49	#define LED_ACTIVE_LOW 0             // 1 for a common-anode RGB LED

50

51	// Touch calibration (raw XPT2046 values at the screen edges). Print raw values

52	// with TOUCH_DEBUG 1, press the four corners, and copy the numbers here.

53	#define TOUCH_DEBUG 0

54	constexpr int TS_RAW_X0 = 3800, TS_RAW_X1 = 300;   // raw x at screen x = 0 and x = 239

55	constexpr int TS_RAW_Y0 = 3800, TS_RAW_Y1 = 300;   // raw y at screen y = 0 and y = 319

56	#define TS_SWAP_XY 0

57	constexpr uint8_t TFT_ROTATION = 2;                // 0 = portrait 240 x 320

58

59	// Timing (ESP32 Integration Guide, sections 3 and 6A).

60	// All times are in milliseconds unless the name ends in _S. FB_ = Firebase,

61	// TS_ = ThingSpeak channel writes, TB_ = ThingSpeak TalkBack polling,

62	// BT_ = Bluetooth Wi-Fi setup. TS_MIN_GAP_MS keeps writes above the 15 s

63	// limit of the free ThingSpeak plan; a write sent faster than that returns 0.

64	constexpr uint32_t FB_WATCHED_MS = 5000;

65	constexpr uint32_t FB_IDLE_MS = 60000;

66	constexpr uint32_t FB_CMD_WATCHED_MS = 2000;

67	constexpr uint32_t FB_CMD_IDLE_MS = 10000;

68	constexpr uint32_t FB_WATCHERS_MS = 30000;

69	constexpr uint32_t FB_USAGE_MS = 600000;

70	constexpr uint32_t TS_WRITE_MS = 30000;

71	constexpr uint32_t TS_MIN_GAP_MS = 15500;

72	constexpr uint32_t TS_HEARTBEAT_MS = 300000;

73	constexpr uint32_t TB_WATCHED_MS = 5000;

74	constexpr uint32_t TB_IDLE_MS = 15000;

75	constexpr uint32_t CMD_MAX_AGE_MS = 60000;

76	constexpr uint32_t TAMPER_COUNTDOWN_S = 10;

77	constexpr uint32_t TAMPER_ALARM_EVERY_MS = 2000;   // latched tamper siren repeats this often

78	constexpr uint32_t SCREEN_TIMEOUT_MS = 30000;

79	constexpr uint32_t BT_IDLE_OFF_MS = 300000;

80	constexpr uint32_t BT_CODE_MS = 60000;

81	constexpr uint32_t BT_LOCKOUT_MS = 60000;

82	// 10 GiB, the monthly download quota of the Firebase free plan. Used only to

83	// estimate usage; Firebase does not report it to the device.

84	constexpr uint64_t FB_MONTH_LIMIT = 10737418240ULL;

85

86	// 0 = always use the fast "someone is watching" pace. Set to 1 once the app writes

87	// /sb/watchers (Integration Guide 6A); the dashboard already does.

88	#define FB_REQUIRE_WATCHERS 0

89

90	// Bluetooth service used by the app and dashboard for Wi-Fi setup. These must

91	// match the UUIDs in the Android app (data/Ble.kt) and the web dashboard.

92	#define BT_SVC_UUID  "b1a70001-5c2e-4f8a-9d33-0a9c2b6f17c1"

93	#define BT_SSID_UUID "b1a70002-5c2e-4f8a-9d33-0a9c2b6f17c1"

94	#define BT_PASS_UUID "b1a70003-5c2e-4f8a-9d33-0a9c2b6f17c1"

95	#define BT_CTRL_UUID "b1a70004-5c2e-4f8a-9d33-0a9c2b6f17c1"

96	#define BT_STAT_UUID "b1a70005-5c2e-4f8a-9d33-0a9c2b6f17c1"

97	#define BT_SCAN_UUID "b1a70006-5c2e-4f8a-9d33-0a9c2b6f17c1"

98

99	// Names derived from BIN_NUMBER. BIN_HOST gives http://smartbin-01.local on

100	// networks that support mDNS.

101	#if BIN_NUMBER == 1

102	#define BIN_ID "bin01"

103	#define BIN_LABEL "BIN 01"

104	#define BIN_HOST "smartbin-01"

105	#define BIN_BLE_NAME "SmartBin-01"

106	#else

107	#define BIN_ID "bin02"

108	#define BIN_LABEL "BIN 02"

109	#define BIN_HOST "smartbin-02"

110	#define BIN_BLE_NAME "SmartBin-02"

111	#endif

firmware/SmartBin/secrets.example.h

1	// ============================================================================

2	// secrets.example.h - template for secrets.h

3	// The firmware includes "secrets.h", not this file. A build error saying

4	// secrets.h is missing means this copy step was skipped.

5	// ============================================================================

6	#pragma once

7

8	// Copy this file to secrets.h in the same folder and fill it in.

9	// secrets.h is git-ignored. Never commit real keys, passwords or the Collect PIN.

10

11	// Wi-Fi (2.4 GHz only). Can also be changed later from WI-FI SETUP on the touchscreen.

12	#define WIFI_SSID "your-wifi-name"

13	#define WIFI_PASSWORD "your-wifi-password"

14

15	// Firebase Realtime Database. Empty = Firebase off (local Wi-Fi only).

16	// See firmware/README.md section 4. The bin signs in with its own account,

17	// for example bin01@your-project.test.

18	#define FIREBASE_URL ""          // https://<project>-default-rtdb.asia-southeast1.firebasedatabase.app

19	#define FIREBASE_API_KEY ""      // AIza... (Project settings > Web API key)

20	#define FIREBASE_EMAIL ""

21	#define FIREBASE_PASSWORD ""

22

23	// ThingSpeak, one channel per bin. Empty = ThingSpeak off. See firmware/README.md section 5.

24	#define THINGSPEAK_SERVER "https://api.thingspeak.com"

25	#define THINGSPEAK_WRITE_KEY_BIN_A ""

26	#define THINGSPEAK_WRITE_KEY_BIN_B ""

27	#define THINGSPEAK_TALKBACK_ID_BIN_A ""

28	#define THINGSPEAK_TALKBACK_KEY_BIN_A ""

29	#define THINGSPEAK_TALKBACK_ID_BIN_B ""

30	#define THINGSPEAK_TALKBACK_KEY_BIN_B ""

31

32	// Default Collect PIN (4 digits). It can be changed later from the dashboard or app.

33	#define COLLECT_PIN "0000"

firmware/SmartBin/state.h

1	// ============================================================================

2	// state.h - shared data model

3	// Every task reads and writes the same BinState (st), calibration (cal) and

4	// simulation (sim) objects. Always wrap access in lockState()/unlockState(),

5	// or take a copy with snapshot()/calSnapshot()/simSnapshot(). Reading st

6	// without the lock can show half-updated values on the screen or dashboard.

7	// ============================================================================

8	#pragma once

9	#include <Arduino.h>

10	#include <ArduinoJson.h>

11	#include "config.h"

12

13	// The one status shown everywhere. Priority, highest first, is decided in

14	// controlTask: CALIBRATING > COLLECTING > TAMPER > LOCKED > OK.

15	enum BinStatus : uint8_t { ST_OK = 0, ST_LOCKED, ST_TAMPER, ST_COLLECTING, ST_CALIBRATING };

16	extern const char* const STATUS_NAMES[5];

17

18	// User settings, saved in flash as one binary blob ("cal"). Distances are in

19	// cm, weights in grams, temperatures in deg C, gas_th is the IAQ index limit,

20	// quiet_from/quiet_to are minutes after midnight (local time, UTC+8).

21	// Adding or reordering a field changes sizeof(Cal), and loadCal() then

22	// ignores the old saved blob and falls back to these defaults.

23	struct Cal {

24	  float d_empty = 30, d_full = 5, fill_th = 80, temp_th = 40, hum_th = 85, gas_th = 200, tilt_th = 30, weight_cap = 4500;

25	  uint8_t auto_lock = 0;

26	  int16_t servo_closed = 0, servo_open = 90;

27	  uint16_t collect_pin = 0;

28	  uint16_t pir_hold_s = 5, buzz_s = 2;

29	  float temp_off = -2, hum_off = 0, press_off = 0;

30	  uint8_t buzz_on = 1, buzz_vol = 2, buzz_ui = 1, buzz_lid = 0;

31	  uint16_t buzz_repeat_s = 30;

32	  uint8_t quiet_on = 0;

33	  uint16_t quiet_from = 1320, quiet_to = 360;

34	};

35

36	// Raw XPT2046 readings at the screen edges, set by the touch calibration.

37	struct TouchCal {

38	  uint8_t swap = TS_SWAP_XY;

39	  int16_t x0 = TS_RAW_X0, x1 = TS_RAW_X1, y0 = TS_RAW_Y0, y1 = TS_RAW_Y1;

40	};

41	extern TouchCal touchCal;

42	extern bool touchCalCustom;

43	void saveTouchCal();

44	void loadTouchCal();

45	void resetTouchCal();

46

47	// Simulation overrides for testing without real waste. mask bits:

48	// 1 fill, 2 weight, 4 gas, 8 temperature and humidity, 16 tilt (tamper),

49	// 32 motion, 64 lid. A set bit replaces that sensor's value in the view.

50	struct Sim {

51	  uint8_t mask = 0;

52	  float fill = 60, weight = 2000, gas = 120, temp = 28, humidity = 60;

53	  bool tamper = false, motion = false, lid = false;

54	};

55

56	// One set of sensor values. iaq_acc is the air-quality accuracy:

57	// 0 warming up, 1 low, 2 medium, 3 high.

58	struct Reading {

59	  float fill = 0, distance_cm = 0, weight_g = 0, temp = 0, temp_raw = 0, humidity = 0, pressure = 0, iaq = 0, tilt = 0, gas_k = 0;

60	  uint8_t iaq_acc = 0;

61	  bool motion = false;

62	};

63

64	struct LastCmd {

65	  char cmd[20] = "";

66	  uint64_t ts = 0;

67	  bool ok = false;

68	};

69

70	// The live state of the bin. raw holds sensor values, v holds what users see

71	// (raw with simulation applied by recomputeView()). The ok* flags are false

72	// while a sensor is not answering; they drive the "faults" list in the JSON.

73	struct BinState {

74	  Reading raw;            // what the sensors read

75	  Reading v;              // what everyone sees (raw with simulation applied)

76	  bool lidOpen = false;

77	  BinStatus status = ST_OK;

78	  bool locked = false, collecting = false, calibrating = false, autoLocked = false;

79	  uint8_t tamperPhase = 0;            // 0 none, 1 countdown, 2 latched

80	  uint32_t tamperStartMs = 0;

81	  bool okHC = false, okHX = false, okBME = false, okIMU = false;

82	  bool booting = true;

83	  bool wifi = false, cloudOk = false, tsOk = false, btOn = false;

84	  int rssi = 0;

85	  uint32_t lidCloseAtMs = 0;          // 0 = no countdown; lid holds while motion is high

86	  bool alarmFull = false, alarmEnv = false, buzzerOn = false;

87	  uint32_t buzzerUntilMs = 0;

88	  float fillRatePerHour = 0;

89	  uint8_t calSteps = 0;               // bit 0 tare, bit 1 empty distance, bit 2 full distance

90	  uint32_t doneAtMs = 0;              // when COLLECT_DONE ran, for the A5 screen

91	  LastCmd last;

92	  uint32_t version = 0;

93	  uint32_t calVersion = 0;

94	};

95

96	// Where a command came from. Results are written back only for Firebase

97	// commands (they carry a ref key); screen commands skip some beeps because

98	// the screen already gave feedback.

99	enum CmdSource : uint8_t { SRC_LOCAL = 0, SRC_SCREEN, SRC_FIREBASE, SRC_TALKBACK };

100

101	struct Command {

102	  char cmd[20];

103	  float grams;

104	  char sound[16];

105	  uint8_t src;

106	  char ref[28];

107	};

108

109	struct CmdResult {

110	  char cmd[20];

111	  bool ok;

112	  char why[40];

113	  uint8_t src;

114	  char ref[28];

115	  uint64_t ts;

116	};

117

118	struct AlertEv {

119	  char kind[10];

120	  char sev[10];

121	  char title[40];

122	  char meta[72];

123	};

124

125	// Task notification bits. A task waits on these with xTaskNotifyWait, so one

126	// notify wakes it immediately instead of waiting for its next timeout.

127	enum : uint32_t {

128	  EV_PIR = 1 << 0, EV_TOUCH = 1 << 1, EV_IMU = 1 << 2, EV_STATE = 1 << 3, EV_CMD = 1 << 4,

129	  EV_CAL = 1 << 5, EV_WIFI = 1 << 6, EV_ALERT = 1 << 7, EV_RESULT = 1 << 8, EV_BT = 1 << 9,

130	};

131

132	extern BinState st;

133	extern Cal cal;

134	extern Sim sim;

135	extern SemaphoreHandle_t stLock;

136	extern QueueHandle_t cmdQueue, resultQueue, alertQueue;

137	extern TaskHandle_t hControl, hSensor, hUi, hCloud, hPush;

138

139	// Network and cloud credentials. Loaded from flash, falling back to the

140	// values in secrets.h when nothing has been saved yet.

141	struct NetCfg {

142	  String ssid, pass;

143	  String fbUrl, fbKey, fbEmail, fbPass;

144	  String tsServer, tsWriteKey, tbId, tbKey;

145	};

146	extern NetCfg net;

147

148	// --- Lock, copy and publish ---

149	// publish() bumps st.version and wakes the UI, cloud and push tasks.

150	// Call it after any change that users should see.

151	void stateInit();

152	void lockState();

153	void unlockState();

154	BinState snapshot();

155	Cal calSnapshot();

156	Sim simSnapshot();

157	void publish(uint32_t extraBits = 0);

158	void notifyAll(uint32_t bits);

159

160	// --- Flash storage (Preferences namespace "smartbin") ---

161	void saveCal();

162	void loadCal();

163	void saveNet();

164	void loadNet();

165	uint8_t loadTheme();

166	void saveTheme(uint8_t dark);

167	void saveHx(int32_t offset, float scale);

168	bool loadHx(int32_t& offset, float& scale);

169	void saveGasBaseline(float g);

170	float loadGasBaseline();

171	void saveImuRef(float x, float y, float z);

172	bool loadImuRef(float& x, float& y, float& z);

173

174	// --- JSON for the local API, live events and Firebase ---

175	void calToJson(const Cal& c, JsonObject o);

176	bool calFromJson(JsonObjectConst o, Cal& c);

177	void statusToJson(const BinState& s, const Cal& c, const Sim& sm, JsonObject o, bool withCal, bool forCloud);

178	String statusJson(bool withCal = true, bool forCloud = false);

179	String localIp();

180

181	// --- Commands and alerts ---

182	// sendCommand() only queues; controlTask runs it. validateCommand() is also

183	// called by the web server first so it can answer 400/409 right away.

184	bool sendCommand(const char* cmd, uint8_t src, float grams = 0, const char* sound = nullptr, const char* ref = nullptr);

185	bool validateCommand(const char* cmd, const BinState& s, String& why);

186	void raiseAlert(const char* kind, const char* sev, const char* title, const char* meta);

187

188	// --- Time. nowMs() returns 0 until SNTP has set the clock. ---

189	uint64_t nowMs();

190	bool clockSet();

191	int localMinuteOfDay();

192	void monthKeyUtc(char* out, size_t n);

firmware/SmartBin/state.cpp

1	// ============================================================================

2	// state.cpp - shared state, flash storage, JSON and command plumbing

3	// ============================================================================

4	#include "state.h"

5	#include "sensors.h"

6	#include <Preferences.h>

7	#include <WiFi.h>

8	#include <sys/time.h>

9	#include "secrets.h"

10

11	const char* const STATUS_NAMES[5] = {"OK", "LOCKED", "TAMPER", "COLLECTING", "CALIBRATING"};

12

13	BinState st;

14	Cal cal;

15	Sim sim;

16	NetCfg net;

17	SemaphoreHandle_t stLock;

18	QueueHandle_t cmdQueue, resultQueue, alertQueue;

19	TaskHandle_t hControl = nullptr, hSensor = nullptr, hUi = nullptr, hCloud = nullptr, hPush = nullptr;

20

21	// Preferences (NVS flash) is not safe to use from two tasks at once, so every

22	// access goes through prefsBegin()/prefsEnd(), which hold prefsLock.

23	static Preferences prefs;

24	static SemaphoreHandle_t prefsLock;

25

26	// Queue depths are small on purpose: if a queue is full, sendCommand()

27	// fails and the caller reports "Busy" instead of blocking a task.

28	void stateInit() {

29	  stLock = xSemaphoreCreateMutex();

30	  prefsLock = xSemaphoreCreateMutex();

31	  cmdQueue = xQueueCreate(8, sizeof(Command));

32	  resultQueue = xQueueCreate(8, sizeof(CmdResult));

33	  alertQueue = xQueueCreate(8, sizeof(AlertEv));

34	  cal.collect_pin = (uint16_t)atoi(COLLECT_PIN);

35	  loadCal();

36	  loadTouchCal();

37	  loadNet();

38	}

39

40	void lockState() { xSemaphoreTake(stLock, portMAX_DELAY); }

41	void unlockState() { xSemaphoreGive(stLock); }

42

43	BinState snapshot() {

44	  lockState();

45	  BinState s = st;

46	  unlockState();

47	  return s;

48	}

49

50	Cal calSnapshot() {

51	  lockState();

52	  Cal c = cal;

53	  unlockState();

54	  return c;

55	}

56

57	Sim simSnapshot() {

58	  lockState();

59	  Sim s = sim;

60	  unlockState();

61	  return s;

62	}

63

64	// Wakes the three tasks that display or send state. A handle is null until

65	// its task has been created in setup().

66	void notifyAll(uint32_t bits) {

67	  TaskHandle_t hs[] = {hUi, hCloud, hPush};

68	  for (TaskHandle_t h : hs)

69	    if (h) xTaskNotify(h, bits, eSetBits);

70	}

71

72	void publish(uint32_t extraBits) {

73	  lockState();

74	  st.version++;

75	  unlockState();

76	  notifyAll(EV_STATE | extraBits);

77	}

78

79	static void prefsBegin(bool ro) {

80	  xSemaphoreTake(prefsLock, portMAX_DELAY);

81	  prefs.begin("smartbin", ro);

82	}

83	static void prefsEnd() {

84	  prefs.end();

85	  xSemaphoreGive(prefsLock);

86	}

87

88	// Calibration is stored as a raw struct. loadCal() only accepts a blob of the

89	// exact current size, so a firmware with a changed Cal struct starts from

90	// defaults instead of reading garbage.

91	void saveCal() {

92	  Cal c = calSnapshot();

93	  prefsBegin(false);

94	  prefs.putBytes("cal", &c, sizeof(c));

95	  prefsEnd();

96	}

97

98	void loadCal() {

99	  prefsBegin(true);

100	  if (prefs.getBytesLength("cal") == sizeof(Cal)) prefs.getBytes("cal", &cal, sizeof(Cal));

101	  prefsEnd();

102	}

103

104	TouchCal touchCal;

105	bool touchCalCustom = false;

106

107	void saveTouchCal() {

108	  prefsBegin(false);

109	  prefs.putBytes("touch", &touchCal, sizeof(touchCal));

110	  prefsEnd();

111	  touchCalCustom = true;

112	}

113

114	void loadTouchCal() {

115	  prefsBegin(true);

116	  if (prefs.getBytesLength("touch") == sizeof(TouchCal)) {

117	    prefs.getBytes("touch", &touchCal, sizeof(TouchCal));

118	    touchCalCustom = true;

119	  }

120	  prefsEnd();

121	}

122

123	void resetTouchCal() {

124	  touchCal = TouchCal();

125	  prefsBegin(false);

126	  prefs.remove("touch");

127	  prefsEnd();

128	  touchCalCustom = false;

129	}

130

131	// Wi-Fi and Firebase settings changed from the screen, Bluetooth setup or

132	// /api/cloud are kept here and survive a reflash of the same partition.

133	void saveNet() {

134	  prefsBegin(false);

135	  prefs.putString("ssid", net.ssid);

136	  prefs.putString("pass", net.pass);

137	  prefs.putString("fbUrl", net.fbUrl);

138	  prefs.putString("fbKey", net.fbKey);

139	  prefs.putString("fbEmail", net.fbEmail);

140	  prefs.putString("fbPass", net.fbPass);

141	  prefsEnd();

142	}

143

144	// ThingSpeak keys come only from secrets.h; each bin picks its own channel.

145	void loadNet() {

146	  prefsBegin(true);

147	  net.ssid = prefs.getString("ssid", WIFI_SSID);

148	  net.pass = prefs.getString("pass", WIFI_PASSWORD);

149	  net.fbUrl = prefs.getString("fbUrl", FIREBASE_URL);

150	  net.fbKey = prefs.getString("fbKey", FIREBASE_API_KEY);

151	  net.fbEmail = prefs.getString("fbEmail", FIREBASE_EMAIL);

152	  net.fbPass = prefs.getString("fbPass", FIREBASE_PASSWORD);

153	  prefsEnd();

154	  net.tsServer = THINGSPEAK_SERVER;

155	#if BIN_NUMBER == 1

156	  net.tsWriteKey = THINGSPEAK_WRITE_KEY_BIN_A;

157	  net.tbId = THINGSPEAK_TALKBACK_ID_BIN_A;

158	  net.tbKey = THINGSPEAK_TALKBACK_KEY_BIN_A;

159	#else

160	  net.tsWriteKey = THINGSPEAK_WRITE_KEY_BIN_B;

161	  net.tbId = THINGSPEAK_TALKBACK_ID_BIN_B;

162	  net.tbKey = THINGSPEAK_TALKBACK_KEY_BIN_B;

163	#endif

164	  if (net.tsWriteKey.startsWith("WRITE_KEY")) net.tsWriteKey = "";

165	}

166

167	uint8_t loadTheme() {

168	  prefsBegin(true);

169	  uint8_t t = prefs.getUChar("theme", 0);

170	  prefsEnd();

171	  return t;

172	}

173

174	void saveTheme(uint8_t dark) {

175	  prefsBegin(false);

176	  prefs.putUChar("theme", dark);

177	  prefsEnd();

178	}

179

180	void saveHx(int32_t offset, float scale) {

181	  prefsBegin(false);

182	  prefs.putInt("hxOff", offset);

183	  prefs.putFloat("hxScale", scale);

184	  prefsEnd();

185	}

186

187	// Load-cell offset and scale from the last TARE and CAL_WEIGHT. Returns false

188	// when the scale was never calibrated (weights will then be rough).

189	bool loadHx(int32_t& offset, float& scale) {

190	  prefsBegin(true);

191	  bool has = prefs.isKey("hxScale");

192	  offset = prefs.getInt("hxOff", 0);

193	  scale = prefs.getFloat("hxScale", 420.0f);

194	  prefsEnd();

195	  return has;

196	}

197

198	void saveGasBaseline(float g) {

199	  prefsBegin(false);

200	  prefs.putFloat("gasBase", g);

201	  prefsEnd();

202	}

203

204	float loadGasBaseline() {

205	  prefsBegin(true);

206	  float g = prefs.getFloat("gasBase", 0);

207	  prefsEnd();

208	  return g;

209	}

210

211	void saveImuRef(float x, float y, float z) {

212	  float v[3] = {x, y, z};

213	  prefsBegin(false);

214	  prefs.putBytes("imuRef", v, sizeof(v));

215	  prefsEnd();

216	}

217

218	bool loadImuRef(float& x, float& y, float& z) {

219	  float v[3];

220	  prefsBegin(true);

221	  bool ok = prefs.getBytesLength("imuRef") == sizeof(v) && prefs.getBytes("imuRef", v, sizeof(v)) == sizeof(v);

222	  prefsEnd();

223	  if (ok) { x = v[0]; y = v[1]; z = v[2]; }

224	  return ok;

225	}

226

227	// Round to 1 or 2 decimals so the JSON stays short and readable.

228	static double r1(float v) { return round(v * 10.0) / 10.0; }

229	static double r2(float v) { return round(v * 100.0) / 100.0; }

230

231	void calToJson(const Cal& c, JsonObject o) {

232	  o["d_empty"] = r1(c.d_empty);

233	  o["d_full"] = r1(c.d_full);

234	  o["fill_th"] = r1(c.fill_th);

235	  o["temp_th"] = r1(c.temp_th);

236	  o["hum_th"] = r1(c.hum_th);

237	  o["gas_th"] = r1(c.gas_th);

238	  o["tilt_th"] = r1(c.tilt_th);

239	  o["weight_cap"] = r1(c.weight_cap);

240	  o["auto_lock"] = c.auto_lock;

241	  o["servo_closed"] = c.servo_closed;

242	  o["servo_open"] = c.servo_open;

243	  o["collect_pin"] = c.collect_pin;

244	  o["pir_hold_s"] = c.pir_hold_s;

245	  o["buzz_s"] = c.buzz_s;

246	  o["temp_off"] = r1(c.temp_off);

247	  o["hum_off"] = r1(c.hum_off);

248	  o["press_off"] = r1(c.press_off);

249	  o["buzz_on"] = c.buzz_on;

250	  o["buzz_vol"] = c.buzz_vol;

251	  o["buzz_ui"] = c.buzz_ui;

252	  o["buzz_lid"] = c.buzz_lid;

253	  o["buzz_repeat_s"] = c.buzz_repeat_s;

254	  o["quiet_on"] = c.quiet_on;

255	  o["quiet_from"] = c.quiet_from;

256	  o["quiet_to"] = c.quiet_to;

257	}

258

259	// Reads one numeric key, accepts numbers or numeric strings, and clamps it to

260	// [lo, hi]. Out-of-range values are clamped, not rejected, so a typo cannot

261	// push the bin into an unusable setting.

262	template <typename T>

263	static bool take(JsonObjectConst o, const char* k, T& dst, float lo, float hi) {

264	  JsonVariantConst v = o[k];

265	  if (v.isNull() || !(v.is<float>() || v.is<int>() || v.is<const char*>())) return false;

266	  float f = v.is<const char*>() ? atof(v.as<const char*>()) : v.as<float>();

267	  if (isnan(f)) return false;

268	  f = constrain(f, lo, hi);

269	  dst = (T)f;

270	  return true;

271	}

272

273	// Applies any known calibration keys. Returns false when the request had none

274	// (the API then answers 400). The empty distance must stay larger than the

275	// full distance or the fill formula divides by zero or goes negative.

276	bool calFromJson(JsonObjectConst o, Cal& c) {

277	  bool any = false;

278	  any |= take(o, "d_empty", c.d_empty, 5, 200);

279	  any |= take(o, "d_full", c.d_full, 1, 100);

280	  any |= take(o, "fill_th", c.fill_th, 10, 100);

281	  any |= take(o, "temp_th", c.temp_th, 10, 85);

282	  any |= take(o, "hum_th", c.hum_th, 10, 100);

283	  any |= take(o, "gas_th", c.gas_th, 50, 500);

284	  any |= take(o, "tilt_th", c.tilt_th, 5, 90);

285	  any |= take(o, "weight_cap", c.weight_cap, 100, 50000);

286	  any |= take(o, "auto_lock", c.auto_lock, 0, 1);

287	  any |= take(o, "servo_closed", c.servo_closed, 0, 180);

288	  any |= take(o, "servo_open", c.servo_open, 0, 180);

289	  any |= take(o, "collect_pin", c.collect_pin, 0, 9999);

290	  any |= take(o, "pir_hold_s", c.pir_hold_s, 1, 120);

291	  any |= take(o, "buzz_s", c.buzz_s, 0, 60);

292	  any |= take(o, "temp_off", c.temp_off, -20, 20);

293	  any |= take(o, "hum_off", c.hum_off, -30, 30);

294	  any |= take(o, "press_off", c.press_off, -100, 100);

295	  any |= take(o, "buzz_on", c.buzz_on, 0, 1);

296	  any |= take(o, "buzz_vol", c.buzz_vol, 1, 3);

297	  any |= take(o, "buzz_ui", c.buzz_ui, 0, 1);

298	  any |= take(o, "buzz_lid", c.buzz_lid, 0, 1);

299	  any |= take(o, "buzz_repeat_s", c.buzz_repeat_s, 10, 300);

300	  any |= take(o, "quiet_on", c.quiet_on, 0, 1);

301	  any |= take(o, "quiet_from", c.quiet_from, 0, 1439);

302	  any |= take(o, "quiet_to", c.quiet_to, 0, 1439);

303	  if (c.d_full >= c.d_empty) c.d_full = c.d_empty - 1;

304	  return any;

305	}

306

307	String localIp() { return WiFi.isConnected() ? WiFi.localIP().toString() : String(""); }

308

309	// Builds the status object used by /api/status, the live event stream and

310	// Firebase. A sensor's values are left out while it is faulty (and not

311	// simulated), so the apps show "--" instead of stale numbers.

312	void statusToJson(const BinState& s, const Cal& c, const Sim& sm, JsonObject o, bool withCal, bool forCloud) {

313	  const Reading& v = s.v;

314	  bool hc = s.okHC || (sm.mask & 1), hx = s.okHX || (sm.mask & 2), bme = s.okBME || (sm.mask & 12) == 12;

315	  o["id"] = BIN_ID;

316	  o["name"] = BIN_LABEL;

317	  if (hc) {

318	    o["fill"] = r1(v.fill);

319	    o["distance_cm"] = r1(v.distance_cm);

320	  }

321	  if (hx) o["weight_g"] = (long)lround(v.weight_g);

322	  if (bme || (sm.mask & 8)) {

323	    o["temp"] = r2(v.temp);

324	    o["temp_raw"] = r2(v.temp_raw);

325	    o["humidity"] = r1(v.humidity);

326	  }

327	  if (s.okBME) o["pressure"] = r1(v.pressure);

328	  if (s.okBME) o["gas_k"] = r1(v.gas_k);

329	  if (s.okBME && v.iaq_acc == 0 && !(sm.mask & 4)) o["iaq_warm_s"] = airWarmLeft();

330	  if (s.okBME || (sm.mask & 4)) {

331	    o["iaq"] = (long)lround(v.iaq);

332	    o["iaq_acc"] = v.iaq_acc;

333	  }

334	  o["tilt"] = r1(v.tilt);

335	  o["motion"] = v.motion;

336	  o["lid"] = s.lidOpen ? "OPEN" : "CLOSED";

337	  o["status"] = STATUS_NAMES[s.status];

338	  o["sim_mask"] = sm.mask;

339	  if (s.tamperPhase) o["tamper_phase"] = s.tamperPhase == 1 ? "COUNTDOWN" : "LATCHED";

340	  JsonArray f = o["faults"].to<JsonArray>();

341	  if (!s.okHC) f.add("HC-SR04");

342	  if (!s.okHX) f.add("HX711");

343	  if (!s.okBME) f.add("BME680");

344	  if (!s.okIMU) f.add("BMI160");

345	  if (withCal) calToJson(c, o["cal"].to<JsonObject>());

346	  if (withCal) {

347	    JsonObject t = o["touch"].to<JsonObject>();

348	    t["swap"] = touchCal.swap;

349	    t["x0"] = touchCal.x0;

350	    t["x1"] = touchCal.x1;

351	    t["y0"] = touchCal.y0;

352	    t["y1"] = touchCal.y1;

353	    t["custom"] = touchCalCustom;

354	  }

355	  if (s.last.cmd[0]) {

356	    JsonObject lc = o["last_cmd"].to<JsonObject>();

357	    lc["cmd"] = s.last.cmd;

358	    lc["ts"] = s.last.ts;

359	    lc["ok"] = s.last.ok;

360	  }

361	  o["uptime_s"] = (uint32_t)(millis() / 1000);

362	  o["rssi"] = s.rssi;

363	  o["fw"] = FW_VERSION;

364	  // ".sv": "timestamp" asks Firebase to fill in its own server time.

365	  if (forCloud) {

366	    o["local_ip"] = localIp();

367	    o["ts"][".sv"] = "timestamp";

368	  }

369	}

370

371	String statusJson(bool withCal, bool forCloud) {

372	  lockState();

373	  BinState s = st;

374	  Cal c = cal;

375	  Sim sm = sim;

376	  unlockState();

377	  JsonDocument doc;

378	  statusToJson(s, c, sm, doc.to<JsonObject>(), withCal, forCloud);

379	  String out;

380	  serializeJson(doc, out);

381	  return out;

382	}

383

384	// Every command the bin accepts. The apps send these exact strings.

385	static const char* const KNOWN[] = {"LOCK", "UNLOCK", "CLEAR_TAMPER", "COLLECT", "COLLECT_DONE", "CALIBRATE_START",

386	                                    "CALIBRATE_END", "TARE", "CAL_WEIGHT", "RESET_GAS", "BUZZ_TEST",

387	                                    "TOUCH_CAL", "TOUCH_CAL_RESET"};

388

389	// Rejects commands that would conflict with the current state. The "why"

390	// text is shown to the user, so keep it short.

391	bool validateCommand(const char* cmd, const BinState& s, String& why) {

392	  bool known = false;

393	  for (const char* k : KNOWN)

394	    if (!strcmp(k, cmd)) known = true;

395	  if (!known) { why = "Unknown command"; return false; }

396	  if (!strcmp(cmd, "COLLECT")) {

397	    if (s.calibrating) { why = "Calibrating"; return false; }

398	    if (s.tamperPhase) { why = "TAMPER active"; return false; }

399	  }

400	  if (!strcmp(cmd, "LOCK") && s.collecting) { why = "Collection in progress"; return false; }

401	  if (!strcmp(cmd, "CALIBRATE_START") && s.collecting) { why = "Collection in progress"; return false; }

402	  if ((!strcmp(cmd, "TARE") || !strcmp(cmd, "CAL_WEIGHT")) && !s.okHX) { why = "Load cell not answering"; return false; }

403	  if (!strcmp(cmd, "RESET_GAS") && !s.okBME) { why = "BME680 not answering"; return false; }

404	  return true;

405	}

406

407	// Queues a command for controlTask and wakes it. Waits at most 50 ms for a

408	// free slot; false means the queue was full.

409	bool sendCommand(const char* cmd, uint8_t src, float grams, const char* sound, const char* ref) {

410	  Command c{};

411	  strlcpy(c.cmd, cmd, sizeof(c.cmd));

412	  c.grams = grams;

413	  c.src = src;

414	  if (sound) strlcpy(c.sound, sound, sizeof(c.sound));

415	  if (ref) strlcpy(c.ref, ref, sizeof(c.ref));

416	  if (xQueueSend(cmdQueue, &c, pdMS_TO_TICKS(50)) != pdTRUE) return false;

417	  if (hControl) xTaskNotify(hControl, EV_CMD, eSetBits);

418	  return true;

419	}

420

421	// Queues an alert for cloudTask, which logs it to Serial as "[alert] ..." and

422	// posts it to Firebase. If alerts never reach the app, check the Serial log

423	// for these lines first, then the Firebase rules.

424	void raiseAlert(const char* kind, const char* sev, const char* title, const char* meta) {

425	  AlertEv a{};

426	  strlcpy(a.kind, kind, sizeof(a.kind));

427	  strlcpy(a.sev, sev, sizeof(a.sev));

428	  strlcpy(a.title, title, sizeof(a.title));

429	  strlcpy(a.meta, meta, sizeof(a.meta));

430	  xQueueSend(alertQueue, &a, 0);

431	  if (hCloud) xTaskNotify(hCloud, EV_ALERT, eSetBits);

432	}

433

434	// The clock counts as set once it is past November 2023 (Unix 1700000000).

435	bool clockSet() { return time(nullptr) > 1700000000; }

436

437	// Philippine time is UTC+8 with no daylight saving, so a fixed offset is used.

438	int localMinuteOfDay() {

439	  if (!clockSet()) return -1;

440	  return (int)(((uint64_t)time(nullptr) + 8 * 3600ULL) / 60ULL % 1440ULL);

441	}

442

443	// Converts today's date to "YYYY-MM" without the C time library (civil-from-

444	// days algorithm). Used as the key for the monthly Firebase usage counter.

445	void monthKeyUtc(char* out, size_t n) {

446	  out[0] = 0;

447	  if (!clockSet()) return;

448	  int64_t z = (int64_t)(time(nullptr) / 86400) + 719468;

449	  int64_t era = z / 146097;

450	  int64_t doe = z - era * 146097;

451	  int64_t yoe = (doe - doe / 1460 + doe / 36524 - doe / 146096) / 365;

452	  int64_t doy = doe - (365 * yoe + yoe / 4 - yoe / 100);

453	  int64_t mp = (5 * doy + 2) / 153;

454	  int m = (int)(mp < 10 ? mp + 3 : mp - 9);

455	  int y = (int)(yoe + era * 400 + (m <= 2));

456	  snprintf(out, n, "%04d-%02d", y, m);

457	}

458

459	uint64_t nowMs() {

460	  if (!clockSet()) return 0;

461	  struct timeval tv;

462	  gettimeofday(&tv, nullptr);

463	  return (uint64_t)tv.tv_sec * 1000ULL + tv.tv_usec / 1000;

464	}

firmware/SmartBin/sensors.h

1	// ============================================================================

2	// sensors.h - sensor task and calibration requests

3	// The request* functions only set a flag; sensorTask does the actual work on

4	// its next pass, so they are safe to call from any task.

5	// ============================================================================

6	#pragma once

7	#include <Arduino.h>

8

9	void sensorsInit();

10	void sensorTask(void*);

11	void recomputeView();

12

13	void requestTare();

14	void requestCalWeight(float grams);

15	void requestGasReset();

16	void requestImuReference();

17	int airWarmLeft();

firmware/SmartBin/sensors.cpp

1	// ============================================================================

2	// sensors.cpp - reads every sensor and keeps st.raw up to date

3	//   HC-SR04 ultrasonic   -> distance and fill %   every 200 ms

4	//   BMI160 accelerometer -> tilt in degrees       every 200 ms (raw I2C)

5	//   HX711 load cell      -> weight in grams       whenever a sample is ready

6	//   BME680               -> temp, humidity, pressure, gas, IAQ  every 3 s

7	// A sensor that stops answering sets its ok* flag to false; controlTask turns

8	// that into a "Sensor not answering" alert and the screen shows "--".

9	// ============================================================================

10	#include "sensors.h"

11	#include "state.h"

12	#include "buzzer.h"

13	#include <Wire.h>

14	#include <Adafruit_BME680.h>

15	#include <HX711.h>

16

17	static Adafruit_BME680 bme(&Wire);

18	static uint8_t imuAddr = 0;

19

20	// --- BMI160 over raw I2C (no library) ---

21	// Register map from the BMI160 datasheet: 0x00 CHIP_ID (0xD1), 0x03 PMU

22	// status, 0x12 accel data, 0x40 ACC_CONF, 0x41 ACC_RANGE, 0x7E CMD.

23	static bool imuWrite(uint8_t reg, uint8_t val) {

24	  Wire.beginTransmission(imuAddr);

25	  Wire.write(reg);

26	  Wire.write(val);

27	  return Wire.endTransmission() == 0;

28	}

29

30	static bool imuRead(uint8_t reg, uint8_t* buf, uint8_t len) {

31	  Wire.beginTransmission(imuAddr);

32	  Wire.write(reg);

33	  if (Wire.endTransmission(false) != 0) return false;

34	  if (Wire.requestFrom(imuAddr, len) != len) return false;

35	  for (uint8_t i = 0; i < len; i++) buf[i] = Wire.read();

36	  return true;

37	}

38

39	// Finds the BMI160 at 0x68 or 0x69 (depends on the SDO pin), soft-resets it,

40	// sets 100 Hz / +-2 g and switches the accelerometer to normal mode. Returns

41	// false if the chip ID is wrong or the accelerometer never powers up; the

42	// boot screen then shows BMI160 FAIL. Check SDA 21 / SCL 22 wiring first.

43	static bool imuStart() {

44	  const uint8_t addrs[2] = {0x68, 0x69};

45	  uint8_t id = 0;

46	  for (uint8_t a : addrs) {

47	    imuAddr = a;

48	    if (imuRead(0x00, &id, 1) && id == 0xD1) break;

49	    id = 0;

50	  }

51	  if (id != 0xD1) return false;

52	  imuWrite(0x7E, 0xB6);

53	  delay(100);

54	  imuWrite(0x40, 0x28);

55	  imuWrite(0x41, 0x03);

56	  for (int i = 0; i < 3; i++) {

57	    imuWrite(0x7E, 0x11);

58	    delay(10);

59	    uint8_t pmu = 0;

60	    if (imuRead(0x03, &pmu, 1) && ((pmu >> 4) & 3) == 1) return true;

61	  }

62	  return false;

63	}

64

65	// Reads X, Y, Z acceleration as signed 16-bit values (little endian).

66	static bool imuAccel(int16_t a[3]) {

67	  uint8_t d[6];

68	  if (!imuRead(0x12, d, 6)) return false;

69	  a[0] = (int16_t)(d[1] << 8 | d[0]);

70	  a[1] = (int16_t)(d[3] << 8 | d[2]);

71	  a[2] = (int16_t)(d[5] << 8 | d[4]);

72	  return true;

73	}

74	static HX711 scale;

75

76	// Flags set by the request* functions and cleared here after the work is done.

77	static volatile bool reqTare = false, reqCalW = false, reqGas = false, reqImuRef = false;

78	static volatile float reqGrams = 0;

79

80	// Load cell: grams = (raw - hxOffset) / hxScale. 420 is a rough default for a

81	// 5 kg bar until CAL_WEIGHT is run with a known weight.

82	static int32_t hxOffset = 0;

83	static float hxScale = 420.0f;

84	static float imuRef[3] = {0, 0, 1};

85

86	// Highest gas resistance seen in clean air. IAQ is computed against it, so a

87	// bad baseline (e.g. warmed up next to garbage) makes the air look clean.

88	// RESET_GAS clears it and restarts the warm-up.

89	static float gasBaseline = 0;

90	static uint32_t gasStartMs = 0;

91	static bool baselineFromNvs = false;

92

93	void requestTare() { reqTare = true; }

94	void requestCalWeight(float grams) { reqGrams = grams; reqCalW = true; }

95	void requestGasReset() { reqGas = true; }

96	void requestImuReference() { reqImuRef = true; }

97

98	// Seconds left in the BME680 warm-up: 5 min on first start, 2 min when a

99	// baseline was saved earlier. The screen shows this countdown as "Air (warm)".

100	int airWarmLeft() {

101	  uint32_t burn = baselineFromNvs ? 120 : 300;

102	  uint32_t age = (millis() - gasStartMs) / 1000;

103	  return age < burn ? (int)(burn - age) : 0;

104	}

105

106	// Builds st.v from st.raw plus the simulation overrides. The caller must hold

107	// the state lock. With a fill override, the distance is worked backwards so

108	// the dashboard's distance and fill stay consistent.

109	void recomputeView() {

110	  Reading v = st.raw;

111	  const Sim& s = sim;

112	  const Cal& c = cal;

113	  if (s.mask & 1) {

114	    v.fill = constrain(s.fill, 0, 100);

115	    v.distance_cm = c.d_empty - v.fill / 100.0f * (c.d_empty - c.d_full);

116	  }

117	  if (s.mask & 2) v.weight_g = s.weight;

118	  if (s.mask & 4) {

119	    v.iaq = s.gas;

120	    v.iaq_acc = 3;

121	  }

122	  if (s.mask & 8) {

123	    v.temp = s.temp;

124	    v.temp_raw = s.temp - c.temp_off;

125	    v.humidity = s.humidity;

126	  }

127	  if (s.mask & 16) v.tilt = s.tamper ? max(c.tilt_th + 8.0f, 38.0f) : 0.0f;

128	  if (s.mask & 32) v.motion = s.motion;

129	  st.v = v;

130	}

131

132	// Probes each sensor once at boot. A sensor that fails here is retried by

133	// sensorTask, so a loose wire that reconnects comes back without a reset.

134	void sensorsInit() {

135	  Wire.begin(PIN_SDA, PIN_SCL);

136	  Wire.setClock(100000);

137

138	  pinMode(PIN_TRIG, OUTPUT);

139	  digitalWrite(PIN_TRIG, LOW);

140	  pinMode(PIN_ECHO, INPUT);

141

142	  // The BME680 address is 0x76 or 0x77 depending on its SDO pin; both are tried.

143	  // Heater 320 deg C for 150 ms is the Adafruit default for gas readings.

144	  bool bmeOk = bme.begin(0x76) || bme.begin(0x77);

145	  if (bmeOk) {

146	    bme.setTemperatureOversampling(BME680_OS_8X);

147	    bme.setHumidityOversampling(BME680_OS_2X);

148	    bme.setPressureOversampling(BME680_OS_4X);

149	    bme.setIIRFilterSize(BME680_FILTER_SIZE_3);

150	    bme.setGasHeater(320, 150);

151	  }

152

153	  bool imuOk = imuStart();

154	  float x, y, z;

155	  if (loadImuRef(x, y, z)) { imuRef[0] = x; imuRef[1] = y; imuRef[2] = z; }

156

157	  scale.begin(PIN_HX_DT, PIN_HX_SCK);

158	  loadHx(hxOffset, hxScale);

159	  if (fabsf(hxScale) < 0.001f) hxScale = 420.0f;

160

161	  gasBaseline = loadGasBaseline();

162	  baselineFromNvs = gasBaseline > 0;

163	  gasStartMs = millis();

164

165	  lockState();

166	  st.okBME = bmeOk;

167	  st.okIMU = imuOk;

168	  unlockState();

169	}

170

171	// Median of up to 5 values. Throws away single bad ultrasonic echoes.

172	static float median5(float* a, int n) {

173	  float b[5];

174	  for (int i = 0; i < n; i++) b[i] = a[i];

175	  for (int i = 1; i < n; i++)

176	    for (int j = i; j > 0 && b[j - 1] > b[j]; j--) { float t = b[j]; b[j] = b[j - 1]; b[j - 1] = t; }

177	  return b[n / 2];

178	}

179

180	// Estimated IAQ index (0 good .. 500 bad) from gas resistance and humidity.

181	// This is not Bosch's BSEC algorithm: 25% of the score comes from humidity

182	// distance to 40% RH, 75% from gas resistance compared with the clean-air

183	// baseline. Accuracy rises from 1 to 3 over the 30 minutes after warm-up. Returns NaN

184	// during warm-up, and the old IAQ value is kept.

185	static float iaqFrom(float gasOhm, float hum, uint8_t& acc) {

186	  uint32_t age = (millis() - gasStartMs) / 1000;

187	  uint32_t burn = baselineFromNvs ? 120 : 300;

188	  if (age < burn) {

189	    if (gasOhm > gasBaseline) gasBaseline = gasOhm;

190	    acc = 0;

191	    return NAN;

192	  }

193	  if (gasOhm > gasBaseline) gasBaseline = gasOhm;

194	  else gasBaseline = gasBaseline * 0.99995f + gasOhm * 0.00005f;

195	  acc = age < burn + 600 ? 1 : age < burn + 1800 ? 2 : 3;

196	  const float humBase = 40.0f, humWeight = 0.25f;

197	  float hOff = hum - humBase;

198	  float humScore = hOff > 0 ? (100.0f - humBase - hOff) / (100.0f - humBase) * humWeight * 100.0f

199	                            : (humBase + hOff) / humBase * humWeight * 100.0f;

200	  float gOff = gasBaseline - gasOhm;

201	  float gasScore = gOff > 0 ? (gasOhm / gasBaseline) * (100.0f - humWeight * 100.0f) : 100.0f - humWeight * 100.0f;

202	  float quality = constrain(humScore + gasScore, 0.0f, 100.0f);

203	  return (100.0f - quality) * 5.0f;

204	}

205

206	// Runs on core 1, priority 3. Wakes every 50 ms or on EV_IMU. Each block below

207	// has its own timer so one slow sensor does not hold up the others.

208	void sensorTask(void*) {

209	  float dist[5];

210	  int distN = 0, distI = 0, echoMiss = 0;

211	  float hxBuf[5];

212	  int hxN = 0, hxI = 0;

213	  int32_t tareAcc = 0;

214	  int64_t calAcc = 0;

215	  int tareN = 0, calN = 0;

216	  uint32_t lastTrig = 0, lastImu = 0, lastHxOk = millis(), lastEnv = 0, envReadyAt = 0, lastRate = 0, lastBaseSave = millis();

217	  bool envPending = false;

218	  int bmeFail = 0, imuFail = 0, sameCount = 0;

219	  int16_t lastA[3] = {0, 0, 0};

220	  float tiltF = 0;

221	  float rateBuf[31];

222	  int rateN = 0;

223	  uint8_t lastAcc = 0;

224	  bool firstEnv = false;

225	  Reading pub;

226	  bool pubFaults[4] = {false, false, false, false};

227	  uint32_t bootStart = millis();

228

229	  for (;;) {

230	    uint32_t ev = 0;

231	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(50));

232	    uint32_t now = millis();

233	    Cal c = calSnapshot();

234	    bool changed = false;

235

236	    // --- Ultrasonic fill level ---

237	    // 10 us trigger pulse, echo timeout 25 ms (about 4 m). us / 58 = cm.

238	    // Readings outside 2..400 cm are ignored. After 10 missed echoes in a row

239	    // the sensor is marked faulty. If fill stays at 0 or 100, check the

240	    // empty/full distances in the calibration, not the sensor.

241	    if (now - lastTrig >= 200) {

242	      lastTrig = now;

243	      digitalWrite(PIN_TRIG, HIGH);

244	      delayMicroseconds(10);

245	      digitalWrite(PIN_TRIG, LOW);

246	      unsigned long us = pulseIn(PIN_ECHO, HIGH, 25000);

247	      if (us > 0) {

248	        float cm = us / 58.0f;

249	        if (cm >= 2 && cm <= 400) {

250	          dist[distI] = cm;

251	          distI = (distI + 1) % 5;

252	          if (distN < 5) distN++;

253	          echoMiss = 0;

254	        }

255	      } else if (++echoMiss > 10) {

256	        distN = 0;

257	      }

258	      bool ok = distN >= 3;

259	      float d = ok ? median5(dist, distN) : 0;

260	      float fill = ok ? constrain((c.d_empty - d) / (c.d_empty - c.d_full) * 100.0f, 0.0f, 100.0f) : 0;

261	      lockState();

262	      if (st.okHC != ok) { st.okHC = ok; changed = true; }

263	      st.raw.distance_cm = d;

264	      st.raw.fill = fill;

265	      unlockState();

266	    }

267

268	    // --- Tilt from the accelerometer ---

269	    // Tilt is the angle between the current gravity vector and the reference

270	    // saved at calibration. 25 identical readings in a row (5 s) mean the

271	    // chip has frozen, so it is restarted. A 0.6/0.4 low-pass filter stops a

272	    // single bump from starting the tamper countdown.

273	    if ((ev & EV_IMU) || now - lastImu >= 200) {

274	      lastImu = now;

275	      int16_t a[3] = {0, 0, 0};

276	      bool ok = imuAccel(a) && (a[0] || a[1] || a[2]);

277	      if (ok && a[0] == lastA[0] && a[1] == lastA[1] && a[2] == lastA[2]) {

278	        if (++sameCount >= 25) {

279	          sameCount = 0;

280	          ok = false;

281	          imuFail = 10;

282	          imuStart();

283	        }

284	      } else {

285	        sameCount = 0;

286	      }

287	      if (ok) {

288	        memcpy(lastA, a, sizeof(lastA));

289	        imuFail = 0;

290	        float x = a[0], y = a[1], z = a[2];

291	        float m = sqrtf(x * x + y * y + z * z);

292	        if (m > 1) {

293	          x /= m; y /= m; z /= m;

294	          if (reqImuRef) {

295	            reqImuRef = false;

296	            imuRef[0] = x; imuRef[1] = y; imuRef[2] = z;

297	            saveImuRef(x, y, z);

298	          }

299	          float dot = constrain(x * imuRef[0] + y * imuRef[1] + z * imuRef[2], -1.0f, 1.0f);

300	          float deg = acosf(dot) * 57.29578f;

301	          tiltF = tiltF * 0.6f + deg * 0.4f;

302	        }

303	      } else if (++imuFail > 10) {

304	        tiltF = 0;

305	      }

306	      lockState();

307	      bool okNow = imuFail <= 10;

308	      if (st.okIMU != okNow) { st.okIMU = okNow; changed = true; }

309	      st.raw.tilt = tiltF;

310	      unlockState();

311	    }

312

313	    // --- Load cell ---

314	    // TARE and CAL_WEIGHT average 10 raw samples before saving. The weight is

315	    // a moving average of 5 samples; anything under 5 g counts as 0. No sample

316	    // for 1.5 s marks the HX711 faulty (check DT 32 / SCK 14 and the 5V supply).

317	    if (scale.is_ready()) {

318	      long raw = scale.read();

319	      lastHxOk = now;

320	      if (reqTare) {

321	        tareAcc += raw;

322	        if (++tareN >= 10) {

323	          hxOffset = tareAcc / tareN;

324	          tareAcc = 0; tareN = 0; reqTare = false; hxN = 0;

325	          saveHx(hxOffset, hxScale);

326	          buzz(T_CAL_STEP);

327	        }

328	      } else if (reqCalW) {

329	        calAcc += raw;

330	        if (++calN >= 10) {

331	          float avg = (float)calAcc / calN;

332	          if (reqGrams > 0 && fabsf(avg - hxOffset) > 10) hxScale = (avg - hxOffset) / reqGrams;

333	          calAcc = 0; calN = 0; reqCalW = false; hxN = 0;

334	          saveHx(hxOffset, hxScale);

335	          buzz(T_CAL_STEP);

336	        }

337	      }

338	      hxBuf[hxI] = (raw - hxOffset) / hxScale;

339	      hxI = (hxI + 1) % 5;

340	      if (hxN < 5) hxN++;

341	      float sum = 0;

342	      for (int i = 0; i < hxN; i++) sum += hxBuf[i];

343	      float g = sum / hxN;

344	      if (fabsf(g) < 5) g = 0;

345	      lockState();

346	      if (!st.okHX) { st.okHX = true; changed = true; }

347	      st.raw.weight_g = g;

348	      unlockState();

349	    } else if (now - lastHxOk > 1500) {

350	      lockState();

351	      if (st.okHX) { st.okHX = false; changed = true; }

352	      unlockState();

353	    }

354

355	    // --- Gas baseline reset (RESET_GAS command) ---

356	    if (reqGas) {

357	      reqGas = false;

358	      gasBaseline = 0;

359	      baselineFromNvs = false;

360	      gasStartMs = now;

361	      saveGasBaseline(0);

362	    }

363

364	    // --- BME680, non-blocking ---

365	    // beginReading() starts a measurement and returns when it will be ready,

366	    // so the task never sleeps through the 150 ms heater cycle. Three failed

367	    // starts in a row mark the sensor faulty and re-run bme.begin().

368	    if (!envPending && now - lastEnv >= 3000) {

369	      lastEnv = now;

370	      envReadyAt = bme.beginReading();

371	      envPending = envReadyAt != 0;

372	      if (!envPending && ++bmeFail >= 3) {

373	        lockState();

374	        if (st.okBME) { st.okBME = false; changed = true; }

375	        unlockState();

376	        if (bme.begin(0x76) || bme.begin(0x77)) bme.setGasHeater(320, 150);

377	      }

378	    }

379	    if (envPending && now >= envReadyAt) {

380	      envPending = false;

381	      if (bme.endReading()) {

382	        bmeFail = 0;

383	        firstEnv = true;

384	        uint8_t acc = 0;

385	        float hum = constrain(bme.humidity + c.hum_off, 0.0f, 100.0f);

386	        float iaq = bme.gas_resistance > 0 ? iaqFrom(bme.gas_resistance, hum, acc) : NAN;

387	        lockState();

388	        if (!st.okBME) { st.okBME = true; changed = true; }

389	        st.raw.temp_raw = bme.temperature;

390	        st.raw.temp = bme.temperature + c.temp_off;

391	        st.raw.humidity = hum;

392	        st.raw.pressure = bme.pressure / 100.0f + c.press_off;

393	        st.raw.gas_k = bme.gas_resistance / 1000.0f;

394	        if (!isnan(iaq)) st.raw.iaq = constrain(iaq, 0.0f, 500.0f);

395	        st.raw.iaq_acc = acc;

396	        unlockState();

397	        // First usable IAQ after warm-up: beep once and tell the apps.

398	        if (acc >= 1 && lastAcc == 0) {

399	          buzz(T_AIR_READY);

400	          char meta[72];

401	          snprintf(meta, sizeof(meta), "%s air quality readings have started", BIN_LABEL);

402	          raiseAlert("sensor", "success", "Air sensor ready", meta);

403	        }

404	        lastAcc = acc;

405	        // Save the baseline at most once an hour to spare the flash.

406	        if (acc >= 3 && now - lastBaseSave > 3600000UL) {

407	          lastBaseSave = now;

408	          saveGasBaseline(gasBaseline);

409	        }

410	      } else if (++bmeFail >= 3) {

411	        lockState();

412	        if (st.okBME) { st.okBME = false; changed = true; }

413	        unlockState();

414	      }

415	    }

416

417	    // --- Fill rate for the "Full in ~x h" estimate ---

418	    // One sample a minute, up to 30 minutes. A drop of more than 5% means the

419	    // bin was emptied, so the history restarts. Needs 5 samples (4 min).

420	    if (now - lastRate >= 60000 || lastRate == 0) {

421	      lastRate = now;

422	      lockState();

423	      float f = st.v.fill;

424	      bool ok = st.okHC || (sim.mask & 1);

425	      unlockState();

426	      if (ok) {

427	        if (rateN && f < rateBuf[rateN - 1] - 5) rateN = 0;

428	        if (rateN == 31) { memmove(rateBuf, rateBuf + 1, 30 * sizeof(float)); rateN = 30; }

429	        rateBuf[rateN++] = f;

430	        float rate = rateN >= 5 ? (rateBuf[rateN - 1] - rateBuf[0]) / ((rateN - 1) / 60.0f) : 0;

431	        lockState();

432	        st.fillRatePerHour = rate > 0.05f ? rate : 0;

433	        unlockState();

434	      }

435	    }

436

437	    lockState();

438	    // Leave the boot screen once the BME680 answered (or is known faulty) and

439	    // at least 3.5 s have passed.

440	    if (st.booting && (firstEnv || !st.okBME) && now - bootStart > 3500) {

441	      st.booting = false;

442	      changed = true;

443	    }

444	    recomputeView();

445	    const Reading& v = st.v;

446	    bool faults[4] = {st.okHC, st.okHX, st.okBME, st.okIMU};

447	    // Publish only on a real change (1% fill, 20 g, 0.2 C, ...). This keeps the

448	    // live dashboard and Firebase from being flooded by sensor noise.

449	    if (fabsf(v.fill - pub.fill) >= 1 || fabsf(v.weight_g - pub.weight_g) >= 20 || fabsf(v.temp - pub.temp) >= 0.2f ||

450	        fabsf(v.humidity - pub.humidity) >= 1 || fabsf(v.iaq - pub.iaq) >= 5 || fabsf(v.tilt - pub.tilt) >= 1 ||

451	        fabsf(v.pressure - pub.pressure) >= 0.5f || v.iaq_acc != pub.iaq_acc || memcmp(faults, pubFaults, sizeof(faults)))

452	      changed = true;

453	    if (changed) {

454	      pub = v;

455	      memcpy(pubFaults, faults, sizeof(faults));

456	    }

457	    unlockState();

458	    if (changed) {

459	      publish();

460	      if (hControl) xTaskNotify(hControl, EV_STATE, eSetBits);

461	    }

462	  }

463	}

firmware/SmartBin/control.h

1	// control.h - lid, LED, alarms and the command runner (controlTask).

2	#pragma once

3	#include <Arduino.h>

4

5	void controlInit();

6	void controlTask(void*);

7	void isrPir();

firmware/SmartBin/control.cpp

1	// ============================================================================

2	// control.cpp - decides what the bin does

3	// Runs every command from cmdQueue, works out the status, moves the lid,

4	// drives the RGB LED and raises the full / tamper / environment / sensor

5	// alarms. Priority 4 on core 1, the highest of all tasks, so the lid and

6	// alarms react even while the screen or cloud is busy.

7	// ============================================================================

8	#include "control.h"

9	#include "state.h"

10	#include "sensors.h"

11	#include "buzzer.h"

12	#include "ui.h"

13	#include <ESP32Servo.h>

14

15	static Servo lid;

16	static int lidAngle = -1;

17

18	// PIR interrupt. Only wakes controlTask; the pin itself is read in the task.

19	void IRAM_ATTR isrPir() {

20	  BaseType_t woke = pdFALSE;

21	  if (hControl) xTaskNotifyFromISR(hControl, EV_PIR, eSetBits, &woke);

22	  portYIELD_FROM_ISR(woke);

23	}

24

25	// LED colors (see updateLed): blue = Bluetooth setup, blinking red = tamper

26	// or full alarm, cyan = calibrating, green = collecting or OK, magenta =

27	// locked, yellow = 50% full or more.

28	static void led(bool r, bool g, bool b) {

29	  digitalWrite(PIN_LED_R, r ^ LED_ACTIVE_LOW);

30	  digitalWrite(PIN_LED_G, g ^ LED_ACTIVE_LOW);

31	  digitalWrite(PIN_LED_B, b ^ LED_ACTIVE_LOW);

32	}

33

34	// The servo uses LEDC timer 0 at 50 Hz with 500..2400 us pulses (SG90).

35	// The lid is driven closed at boot so a reset never leaves it open.

36	void controlInit() {

37	  pinMode(PIN_LED_R, OUTPUT);

38	  pinMode(PIN_LED_G, OUTPUT);

39	  pinMode(PIN_LED_B, OUTPUT);

40	  led(false, false, true);

41	  pinMode(PIN_PIR, INPUT);

42	  ESP32PWM::allocateTimer(0);

43	  lid.setPeriodHertz(50);

44	  lid.attach(PIN_SERVO, 500, 2400);

45	  Cal c = calSnapshot();

46	  lid.write(c.servo_closed);

47	  lidAngle = c.servo_closed;

48	}

49

50	static void setLid(int angle) {

51	  if (angle == lidAngle) return;

52	  lid.write(angle);

53	  lidAngle = angle;

54	}

55

56	// Records the outcome of a command. For commands from Firebase the result is

57	// queued so cloudTask can write it to /sb/results/<ref>, which is how the app

58	// knows its button press worked. A failed command from a remote source beeps.

59	static void finish(const Command& c, bool ok, const char* why) {

60	  lockState();

61	  strlcpy(st.last.cmd, c.cmd, sizeof(st.last.cmd));

62	  st.last.ts = nowMs();

63	  st.last.ok = ok;

64	  unlockState();

65	  if (c.src == SRC_FIREBASE && c.ref[0]) {

66	    CmdResult r{};

67	    strlcpy(r.cmd, c.cmd, sizeof(r.cmd));

68	    r.ok = ok;

69	    strlcpy(r.why, why ? why : "", sizeof(r.why));

70	    r.src = c.src;

71	    strlcpy(r.ref, c.ref, sizeof(r.ref));

72	    r.ts = nowMs();

73	    xQueueSend(resultQueue, &r, 0);

74	  }

75	  if (!ok && c.src != SRC_SCREEN) buzz(T_CMD_FAIL);

76	  publish(EV_RESULT);

77	}

78

79	// Runs one command. State changes happen under the lock; sounds and alerts

80	// come after it is released, because buzz() takes the same lock (the mutex

81	// is not recursive, so calling it while holding the lock would hang the task).

82	static void runCommand(const Command& c) {

83	  BinState s = snapshot();

84	  String why;

85	  if (!validateCommand(c.cmd, s, why)) {

86	    finish(c, false, why.c_str());

87	    return;

88	  }

89	  const char* k = c.cmd;

90	  char meta[72];

91	  lockState();

92	  if (!strcmp(k, "LOCK")) {

93	    st.locked = true;

94	    st.autoLocked = false;

95	  } else if (!strcmp(k, "UNLOCK")) {

96	    st.locked = false;

97	    st.autoLocked = false;

98	  } else if (!strcmp(k, "CLEAR_TAMPER")) {

99	    st.tamperPhase = 0;

100	  } else if (!strcmp(k, "COLLECT")) {

101	    st.collecting = true;

102	  // Collection finishing also releases an automatic (full-bin) lock, but not

103	  // a lock that a person set.

104	  } else if (!strcmp(k, "COLLECT_DONE")) {

105	    if (st.collecting) st.doneAtMs = millis();

106	    st.collecting = false;

107	    if (st.autoLocked) { st.locked = false; st.autoLocked = false; }

108	  } else if (!strcmp(k, "CALIBRATE_START")) {

109	    st.calibrating = true;

110	    st.calSteps = 0;

111	  } else if (!strcmp(k, "CALIBRATE_END")) {

112	    st.calibrating = false;

113	  } else if (!strcmp(k, "TARE")) {

114	    st.calSteps |= 1;

115	  }

116	  unlockState();

117

118	  if (!strcmp(k, "LOCK")) {

119	    buzz(T_LOCK);

120	    snprintf(meta, sizeof(meta), "%s locked", BIN_LABEL);

121	    raiseAlert("lock", "info", "Bin locked", meta);

122	  } else if (!strcmp(k, "UNLOCK")) {

123	    buzz(T_UNLOCK);

124	    snprintf(meta, sizeof(meta), "%s is open for use again", BIN_LABEL);

125	    raiseAlert("lock", "success", "Bin unlocked", meta);

126	  } else if (!strcmp(k, "CLEAR_TAMPER")) {

127	    buzz(T_TAMPER_SETTLED);

128	  } else if (!strcmp(k, "COLLECT")) {

129	    if (c.src != SRC_SCREEN) buzz(T_PIN_OK);

130	    snprintf(meta, sizeof(meta), "%s pickup started", BIN_LABEL);

131	    raiseAlert("pickup", "info", "Collection started", meta);

132	  } else if (!strcmp(k, "COLLECT_DONE")) {

133	    buzz(T_DONE);

134	    snprintf(meta, sizeof(meta), "%s is back to normal", BIN_LABEL);

135	    raiseAlert("pickup", "success", "Collection complete", meta);

136	  } else if (!strcmp(k, "CALIBRATE_START")) {

137	    requestImuReference();

138	    buzz(T_CAL_START);

139	    snprintf(meta, sizeof(meta), "%s is being calibrated", BIN_LABEL);

140	    raiseAlert("setup", "info", "Calibration started", meta);

141	  } else if (!strcmp(k, "CALIBRATE_END")) {

142	    buzz(c.src == SRC_SCREEN ? T_CAL_CANCEL : T_SAVED);

143	  } else if (!strcmp(k, "TARE")) {

144	    requestTare();

145	  } else if (!strcmp(k, "CAL_WEIGHT")) {

146	    requestCalWeight(c.grams > 0 ? c.grams : 500);

147	  } else if (!strcmp(k, "RESET_GAS")) {

148	    requestGasReset();

149	    buzz(T_CAL_STEP);

150	  } else if (!strcmp(k, "TOUCH_CAL")) {

151	    uiStartTouchCal();

152	  } else if (!strcmp(k, "TOUCH_CAL_RESET")) {

153	    uiResetTouchCal();

154	  } else if (!strcmp(k, "BUZZ_TEST")) {

155	    if (!buzzByName(c.sound[0] ? c.sound : "full")) {

156	      finish(c, false, "Unknown sound");

157	      return;

158	    }

159	  }

160	  finish(c, true, nullptr);

161	}

162

163	// Called every 100 ms. The first matching state wins.

164	static void updateLed(const BinState& s, const Cal& c, uint32_t now) {

165	  bool blink = (now / 500) % 2;

166	  if (s.btOn) led(false, false, true);

167	  else if (s.tamperPhase) led(blink, false, false);

168	  else if (s.calibrating) led(false, true, true);

169	  else if (s.collecting) led(false, true, false);

170	  else if (s.status == ST_LOCKED) led(true, false, true);

171	  else if (s.alarmFull) led(blink, false, false);

172	  else if (s.v.fill >= 50) led(true, true, false);

173	  else led(false, true, false);

174	}

175

176	// Wakes every 100 ms, on PIR change, on a new command or on a sensor update.

177	void controlTask(void*) {

178	  uint32_t lastTick = 0, lastWave = 0, lastFullBuzz = 0, lastEnvBuzz = 0, lastTamperTick = 0;

179	  bool prevFull = false, prevEnv = false, prevHC = true, prevHX = true, prevBME = true;

180	  bool prevMotion = false, firstPass = true;

181	  BinStatus prevStatus = ST_OK;

182	  uint8_t prevMask = 0;

183

184	  for (;;) {

185	    uint32_t ev = 0;

186	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(100));

187

188	    Command c;

189	    // Drain all waiting commands before evaluating the state.

190	    while (xQueueReceive(cmdQueue, &c, 0) == pdTRUE) runCommand(c);

191

192	    uint32_t now = millis();

193	    Cal cl = calSnapshot();

194	    bool pir = digitalRead(PIN_PIR);

195	    bool changed = false;

196	    char meta[72];

197

198	    lockState();

199	    st.raw.motion = pir;

200	    recomputeView();

201	    bool motion = st.v.motion;

202	    bool hcOk = st.okHC || (sim.mask & 1);

203	    // Alarms are ignored while collecting or calibrating, otherwise opening

204	    // the bin to empty it would set them off.

205	    bool fullNow = hcOk && st.v.fill >= cl.fill_th && !st.collecting && !st.calibrating;

206	    bool tiltBad = (st.okIMU || (sim.mask & 16)) && st.v.tilt > cl.tilt_th && !st.collecting && !st.calibrating;

207

208	    // --- Tamper state machine ---

209	    // 0 none -> 1 countdown (TAMPER_COUNTDOWN_S, beeps each second) -> 2 latched.

210	    // Going back under the tilt limit during the countdown cancels it. Once

211	    // latched it stays on until CLEAR_TAMPER, even if the bin is set upright.

212	    // The lock is released around buzz() because buzz() takes it itself.

213	    if (tiltBad && st.tamperPhase == 0) {

214	      st.tamperPhase = 1;

215	      st.tamperStartMs = now;

216	      changed = true;

217	    } else if (!tiltBad && st.tamperPhase == 1) {

218	      st.tamperPhase = 0;

219	      changed = true;

220	      unlockState();

221	      buzz(T_TAMPER_SETTLED);

222	      lockState();

223	    } else if (st.tamperPhase == 1 && now - st.tamperStartMs >= TAMPER_COUNTDOWN_S * 1000) {

224	      st.tamperPhase = 2;

225	      changed = true;

226	      snprintf(meta, sizeof(meta), "%s was moved or tilted (%.0f deg)", BIN_LABEL, st.v.tilt);

227	      unlockState();

228	      raiseAlert("tamper", "critical", "Tamper detected", meta);

229	      lockState();

230	    }

231

232	    // Optional auto-lock when full (setting "auto_lock"). Off by default.

233	    if (cl.auto_lock && fullNow && !st.locked && !st.collecting && !st.tamperPhase) {

234	      st.locked = true;

235	      st.autoLocked = true;

236	      changed = true;

237	      snprintf(meta, sizeof(meta), "%s locked itself at %.0f%%", BIN_LABEL, st.v.fill);

238	      unlockState();

239	      buzz(T_LOCK);

240	      raiseAlert("lock", "info", "Bin locked", meta);

241	      lockState();

242	    }

243

244	    // One status, highest priority first.

245	    BinStatus status = st.calibrating ? ST_CALIBRATING : st.collecting ? ST_COLLECTING

246	                     : st.tamperPhase ? ST_TAMPER : st.locked ? ST_LOCKED : ST_OK;

247	    if (status != st.status) { st.status = status; changed = true; }

248

249	    // --- Lid decision ---

250	    // Collecting: open. Locked, tamper or calibrating: closed. Lid simulated:

251	    // follow the simulation. Otherwise motion opens the lid, and it closes

252	    // pir_hold_s seconds after the last motion. If the lid opens and closes by

253	    // itself, check the PIR's own delay knob (it should be at minimum).

254	    bool wantOpen;

255	    if (status == ST_COLLECTING) {

256	      wantOpen = true;

257	      st.lidCloseAtMs = 0;

258	    } else if (status != ST_OK) {

259	      wantOpen = false;

260	      st.lidCloseAtMs = 0;

261	    } else if (sim.mask & 64) {

262	      wantOpen = sim.lid;

263	      st.lidCloseAtMs = 0;

264	    } else if (motion) {

265	      wantOpen = true;

266	      st.lidCloseAtMs = 0;

267	    } else if (st.lidOpen && st.lidCloseAtMs == 0) {

268	      st.lidCloseAtMs = now + cl.pir_hold_s * 1000UL;

269	      wantOpen = true;

270	    } else if (st.lidOpen && (int32_t)(now - st.lidCloseAtMs) < 0) {

271	      wantOpen = true;

272	    } else {

273	      wantOpen = false;

274	      st.lidCloseAtMs = 0;

275	    }

276	    bool lidChanged = wantOpen != st.lidOpen;

277	    if (lidChanged) { st.lidOpen = wantOpen; changed = true; }

278	    if (motion != prevMotion) changed = true;

279

280	    // Environment alarm: temperature or humidity over its limit, or IAQ over

281	    // gas_th once the air sensor has warmed up.

282	    st.alarmFull = fullNow && status == ST_OK;

283	    bool gasOk = st.okBME || (sim.mask & 4);

284	    bool envBad = !st.collecting && !st.calibrating &&

285	                  (((st.okBME || (sim.mask & 8)) && (st.v.temp >= cl.temp_th || st.v.humidity >= cl.hum_th)) ||

286	                   (gasOk && st.v.iaq_acc >= 1 && st.v.iaq >= cl.gas_th));

287	    st.alarmEnv = envBad;

288	    st.buzzerOn = buzzerBusy() && (st.alarmFull || st.tamperPhase || envBad);

289	    BinState s = st;

290	    unlockState();

291

292	    // Hardware is touched only after the lock is released.

293	    setLid(s.lidOpen ? cl.servo_open : cl.servo_closed);

294	    if (lidChanged) buzz(s.lidOpen ? T_LID_OPEN : T_LID_CLOSE);

295	    if (motion && !prevMotion && (s.status == ST_LOCKED || s.status == ST_TAMPER) && now - lastWave > 5000) {

296	      lastWave = now;

297	      buzz(T_LOCKED_WAVE);

298	    }

299	    prevMotion = motion;

300

301	    // Countdown beeps change pitch for the last 3 seconds; the latched siren

302	    // repeats every TAMPER_ALARM_EVERY_MS.

303	    if (s.tamperPhase == 1 && now - lastTamperTick >= 1000) {

304	      lastTamperTick = now;

305	      uint32_t left = TAMPER_COUNTDOWN_S - min<uint32_t>(TAMPER_COUNTDOWN_S, (now - s.tamperStartMs) / 1000);

306	      buzz(left <= 3 ? T_TAMPER_TICK_END : T_TAMPER_TICK);

307	      changed = true;

308	    }

309	    if (s.tamperPhase == 2 && now - lastTamperTick >= TAMPER_ALARM_EVERY_MS) {

310	      lastTamperTick = now;

311	      buzz(T_TAMPER_LATCHED);

312	    }

313

314	    // Full alarm: alert once on the rising edge, then repeat the beep every

315	    // buzz_repeat_s seconds. buzzerUntilMs drives the BUZZER row on screen.

316	    if (s.alarmFull && (!prevFull || now - lastFullBuzz >= cl.buzz_repeat_s * 1000UL)) {

317	      if (!prevFull) {

318	        snprintf(meta, sizeof(meta), "%s is at %.0f%%", BIN_LABEL, s.v.fill);

319	        raiseAlert("full", "critical", "Fill threshold passed", meta);

320	      }

321	      lastFullBuzz = now;

322	      buzz(T_FULL);

323	      lockState();

324	      st.buzzerUntilMs = now + max<uint32_t>(cl.buzz_s, 1) * 1000UL;

325	      unlockState();

326	    }

327	    prevFull = s.alarmFull;

328

329	    // Environment alarm repeats every 5 minutes while it lasts.

330	    if (envBad && (!prevEnv || now - lastEnvBuzz >= 300000)) {

331	      if (!prevEnv) {

332	        if (s.v.temp >= cl.temp_th) snprintf(meta, sizeof(meta), "%s at %.1f C, limit %.0f", BIN_LABEL, s.v.temp, cl.temp_th);

333	        else if (s.v.humidity >= cl.hum_th) snprintf(meta, sizeof(meta), "%s at %.0f%%, limit %.0f%%", BIN_LABEL, s.v.humidity, cl.hum_th);

334	        else snprintf(meta, sizeof(meta), "%s index %.0f, limit %.0f", BIN_LABEL, s.v.iaq, cl.gas_th);

335	        raiseAlert("sensor", "warning", "Reading over its limit", meta);

336	      }

337	      lastEnvBuzz = now;

338	      buzz(T_ENV_OVER);

339	    } else if (!envBad && prevEnv) {

340	      buzz(T_ENV_OK);

341	    }

342	    prevEnv = envBad;

343

344	    // Sensor lost / back alerts. The first pass after boot only records the

345	    // starting state and plays the boot sound (OK or FAULT).

346	    if (!firstPass && !s.booting) {

347	      const bool oks[3] = {s.okHC, s.okHX, s.okBME};

348	      bool* prevs[3] = {&prevHC, &prevHX, &prevBME};

349	      const char* names[3] = {"HC-SR04", "HX711", "BME680"};

350	      for (int i = 0; i < 3; i++) {

351	        if (oks[i] == *prevs[i]) continue;

352	        *prevs[i] = oks[i];

353	        buzz(oks[i] ? T_SENSOR_BACK : T_SENSOR_LOST);

354	        snprintf(meta, sizeof(meta), "%s: %s %s", BIN_LABEL, names[i], oks[i] ? "answers again" : "stopped answering");

355	        raiseAlert("sensor", oks[i] ? "success" : "warning", oks[i] ? "Sensor back" : "Sensor not answering", meta);

356	      }

357	    } else if (!s.booting) {

358	      prevHC = s.okHC; prevHX = s.okHX; prevBME = s.okBME;

359	      firstPass = false;

360	      buzz(s.okHC && s.okHX && s.okBME && s.okIMU ? T_BOOT_OK : T_BOOT_FAULT);

361	    }

362

363	    // Sound when simulation is switched on or off from the dashboard.

364	    Sim sm = simSnapshot();

365	    if ((sm.mask != 0) != (prevMask != 0)) buzz(sm.mask ? T_SIM_ON : T_SIM_OFF);

366	    prevMask = sm.mask;

367

368	    if (s.status != prevStatus) {

369	      prevStatus = s.status;

370	      changed = true;

371	    }

372	    if (now - lastTick >= 100) {

373	      lastTick = now;

374	      updateLed(s, cl, now);

375	    }

376	    if (changed) publish();

377	  }

378	}

firmware/SmartBin/buzzer.h

1	// buzzer.h - every sound the bin makes. The Tune order must match TUNES[]

2	// in buzzer.cpp (a static_assert checks the count).

3	#pragma once

4	#include <Arduino.h>

5

6	enum Tune : uint8_t {

7	  T_KEY, T_PIN_KEY, T_PIN_DEL, T_CANCEL, T_THEME_LIGHT, T_THEME_DARK, T_OPEN, T_PIN_OK, T_PIN_WRONG, T_DONE,

8	  T_BOOT_OK, T_BOOT_FAULT, T_LID_OPEN, T_LID_CLOSE, T_LOCKED_WAVE, T_LOCK, T_UNLOCK, T_CAL_START, T_CAL_STEP,

9	  T_SAVED, T_SIM_ON, T_SIM_OFF, T_CAL_CANCEL, T_AIR_READY, T_BACK_MAIN,

10	  T_FULL, T_TAMPER_TICK, T_TAMPER_TICK_END, T_TAMPER_LATCHED, T_ENV_OVER, T_SENSOR_LOST, T_TAMPER_SETTLED,

11	  T_ENV_OK, T_SENSOR_BACK,

12	  T_WIFI_UP, T_WIFI_LOST, T_CLOUD_DOWN, T_CLOUD_UP, T_CMD_FAIL,

13	  T_BT_WAIT, T_BT_CODE, T_BT_PAIRED, T_BT_WRONG, T_BT_STOPPED, T_BT_JOINED, T_BT_JOIN_FAIL, T_BT_OFF, T_BT_AUTO_OFF,

14	  T_COUNT

15	};

16

17	void buzzerInit();

18	void buzz(Tune t);

19	bool buzzByName(const char* name);

20	bool buzzerBusy();

firmware/SmartBin/buzzer.cpp

1	// ============================================================================

2	// buzzer.cpp - non-blocking sound player

3	// buzz() drops a tune ID into a one-slot queue; buzzerTask plays it with LEDC

4	// PWM on GPIO 4. Callers never wait for a sound to finish.

5	// ============================================================================

6	#include "buzzer.h"

7	#include "state.h"

8

9	// A note is a frequency in Hz and a length in ms. hz 0 is a rest. With hzEnd

10	// set the pitch slides from hz to hzEnd (used for the tamper siren).

11	// pri: higher can interrupt lower. kind decides which mute setting applies.

12	struct Note { uint16_t hz, ms, hzEnd; };

13	struct TuneDef { const char* name; const Note* n; uint8_t len; uint8_t pri; uint8_t kind; };

14	enum : uint8_t { K_UI = 0, K_LID, K_EVENT, K_CAL, K_ALARM };

15

16	// --- Tune data. Names match the sound names used by the apps and dashboard. ---

17	static const Note n_key[] = {{2400, 15}};

18	static const Note n_pin_key[] = {{2600, 12}};

19	static const Note n_pin_del[] = {{2000, 15}};

20	static const Note n_cancel[] = {{2300, 30}, {1800, 45}};

21	static const Note n_theme_light[] = {{2300, 35}, {2700, 50}};

22	static const Note n_theme_dark[] = {{2000, 35}, {1700, 50}};

23	static const Note n_open[] = {{1800, 30}, {2300, 40}};

24	static const Note n_pin_ok[] = {{1600, 60}, {1900, 60}, {2200, 60}, {1900, 60}, {2200, 120}};

25	static const Note n_pin_wrong[] = {{1700, 60}, {0, 30}, {1700, 60}, {0, 30}, {1500, 150}};

26	static const Note n_done[] = {{1800, 70}, {2100, 70}, {2400, 70}, {2800, 200}};

27	static const Note n_boot_ok[] = {{1600, 70}, {1900, 70}, {2200, 70}, {2800, 180}};

28	static const Note n_boot_fault[] = {{1600, 70}, {1900, 70}, {0, 40}, {1700, 100}, {0, 60}, {1500, 210}};

29	static const Note n_lid_open[] = {{1800, 25}, {2200, 30}};

30	static const Note n_lid_close[] = {{2200, 25}, {1800, 30}};

31	static const Note n_locked_wave[] = {{1600, 60}, {0, 40}, {1500, 120}};

32	static const Note n_lock[] = {{2000, 40}, {0, 20}, {1600, 120}};

33	static const Note n_unlock[] = {{1600, 40}, {0, 20}, {2000, 120}};

34	static const Note n_cal_start[] = {{2300, 40}, {0, 50}, {2300, 40}, {0, 50}, {2300, 40}, {0, 50}, {2700, 120}};

35	static const Note n_cal_step[] = {{2400, 40}};

36	static const Note n_saved[] = {{2200, 60}, {2600, 100}};

37	static const Note n_sim_on[] = {{1600, 60}, {1800, 60}, {2000, 60}, {2300, 140}};

38	static const Note n_sim_off[] = {{2300, 60}, {2000, 60}, {1800, 60}, {1600, 140}};

39	static const Note n_cal_cancel[] = {{2400, 40}, {0, 40}, {1900, 120}};

40	static const Note n_air_ready[] = {{2000, 40}, {2400, 70}};

41	static const Note n_back_main[] = {{2400, 30}, {2000, 50}};

42	static const Note n_full[] = {{2000, 180}, {0, 80}, {2000, 180}, {0, 80}, {2000, 180}, {0, 200}, {2600, 420}};

43	static const Note n_tamper_tick[] = {{2000, 60}};

44	static const Note n_tamper_tick_end[] = {{2400, 60}};

45	static const Note n_tamper[] = {{1600, 340, 2800}, {2800, 340, 1600}, {1600, 340, 2800}, {2800, 340, 1600}};

46	static const Note n_env_over[] = {{2400, 120}, {1900, 120}, {2400, 120}, {1900, 260}};

47	static const Note n_sensor_lost[] = {{1900, 100}, {0, 60}, {1500, 210}};

48	static const Note n_tamper_settled[] = {{1900, 60}, {2300, 60}, {2600, 120}};

49	static const Note n_env_ok[] = {{2600, 50}, {2200, 100}};

50	static const Note n_sensor_back[] = {{1500, 80}, {2000, 150}};

51	static const Note n_wifi_up[] = {{1800, 70}, {2200, 70}, {2700, 150}};

52	static const Note n_wifi_lost[] = {{2700, 90}, {2200, 90}, {1800, 190}};

53	static const Note n_cloud_down[] = {{2200, 60}, {0, 30}, {2400, 60}, {0, 30}, {1900, 130}};

54	static const Note n_cloud_up[] = {{1900, 60}, {0, 30}, {2400, 60}, {0, 30}, {2700, 120}};

55	static const Note n_cmd_fail[] = {{1900, 80}, {0, 30}, {1700, 80}, {0, 30}, {1500, 200}};

56	static const Note n_bt_wait[] = {{1800, 30}, {0, 30}, {1800, 30}, {2300, 60}};

57	static const Note n_bt_code[] = {{2400, 30}, {0, 60}, {2400, 30}};

58	static const Note n_bt_paired[] = {{2300, 40}, {2800, 40}, {2300, 40}, {2800, 140}};

59	static const Note n_bt_wrong[] = {{1800, 50}, {0, 25}, {1800, 50}, {1500, 140}};

60	static const Note n_bt_stopped[] = {{2200, 180}, {0, 50}, {1500, 320}};

61	static const Note n_bt_joined[] = {{1700, 60}, {2000, 60}, {2400, 60}, {2800, 180}};

62	static const Note n_bt_join_fail[] = {{1600, 70}, {0, 40}, {1500, 120}};

63	static const Note n_bt_off[] = {{2300, 50}, {1800, 50}, {0, 25}, {1500, 140}};

64	static const Note n_bt_auto_off[] = {{2300, 50}, {1800, 50}, {0, 25}, {1500, 60}, {0, 60}, {1500, 60}};

65

66	// Name, notes, note count, priority (0 low .. 5 alarm), kind.

67	static const TuneDef TUNES[] = {

68	  {"key", n_key, sizeof(n_key) / sizeof(Note), 0, K_UI},

69	  {"pin_key", n_pin_key, sizeof(n_pin_key) / sizeof(Note), 0, K_UI},

70	  {"pin_del", n_pin_del, sizeof(n_pin_del) / sizeof(Note), 0, K_UI},

71	  {"cancel", n_cancel, sizeof(n_cancel) / sizeof(Note), 0, K_UI},

72	  {"theme_light", n_theme_light, sizeof(n_theme_light) / sizeof(Note), 0, K_UI},

73	  {"theme_dark", n_theme_dark, sizeof(n_theme_dark) / sizeof(Note), 0, K_UI},

74	  {"open", n_open, sizeof(n_open) / sizeof(Note), 0, K_UI},

75	  {"pin_ok", n_pin_ok, sizeof(n_pin_ok) / sizeof(Note), 3, K_EVENT},

76	  {"pin_wrong", n_pin_wrong, sizeof(n_pin_wrong) / sizeof(Note), 3, K_EVENT},

77	  {"done", n_done, sizeof(n_done) / sizeof(Note), 3, K_EVENT},

78	  {"boot_ok", n_boot_ok, sizeof(n_boot_ok) / sizeof(Note), 1, K_EVENT},

79	  {"boot_fault", n_boot_fault, sizeof(n_boot_fault) / sizeof(Note), 4, K_EVENT},

80	  {"lid_open", n_lid_open, sizeof(n_lid_open) / sizeof(Note), 0, K_LID},

81	  {"lid_close", n_lid_close, sizeof(n_lid_close) / sizeof(Note), 0, K_LID},

82	  {"locked_wave", n_locked_wave, sizeof(n_locked_wave) / sizeof(Note), 2, K_EVENT},

83	  {"lock", n_lock, sizeof(n_lock) / sizeof(Note), 3, K_EVENT},

84	  {"unlock", n_unlock, sizeof(n_unlock) / sizeof(Note), 3, K_EVENT},

85	  {"cal_start", n_cal_start, sizeof(n_cal_start) / sizeof(Note), 2, K_CAL},

86	  {"cal_step", n_cal_step, sizeof(n_cal_step) / sizeof(Note), 1, K_CAL},

87	  {"saved", n_saved, sizeof(n_saved) / sizeof(Note), 2, K_CAL},

88	  {"sim_on", n_sim_on, sizeof(n_sim_on) / sizeof(Note), 2, K_EVENT},

89	  {"sim_off", n_sim_off, sizeof(n_sim_off) / sizeof(Note), 2, K_EVENT},

90	  {"cal_cancel", n_cal_cancel, sizeof(n_cal_cancel) / sizeof(Note), 2, K_CAL},

91	  {"air_ready", n_air_ready, sizeof(n_air_ready) / sizeof(Note), 1, K_EVENT},

92	  {"back_main", n_back_main, sizeof(n_back_main) / sizeof(Note), 1, K_EVENT},

93	  {"full", n_full, sizeof(n_full) / sizeof(Note), 5, K_ALARM},

94	  {"tamper_tick", n_tamper_tick, sizeof(n_tamper_tick) / sizeof(Note), 5, K_ALARM},

95	  {"tamper_tick_end", n_tamper_tick_end, sizeof(n_tamper_tick_end) / sizeof(Note), 5, K_ALARM},

96	  {"tamper", n_tamper, sizeof(n_tamper) / sizeof(Note), 5, K_ALARM},

97	  {"env_over", n_env_over, sizeof(n_env_over) / sizeof(Note), 4, K_ALARM},

98	  {"sensor_lost", n_sensor_lost, sizeof(n_sensor_lost) / sizeof(Note), 4, K_EVENT},

99	  {"tamper_settled", n_tamper_settled, sizeof(n_tamper_settled) / sizeof(Note), 4, K_EVENT},

100	  {"env_ok", n_env_ok, sizeof(n_env_ok) / sizeof(Note), 2, K_EVENT},

101	  {"sensor_back", n_sensor_back, sizeof(n_sensor_back) / sizeof(Note), 3, K_EVENT},

102	  {"wifi_up", n_wifi_up, sizeof(n_wifi_up) / sizeof(Note), 2, K_EVENT},

103	  {"wifi_lost", n_wifi_lost, sizeof(n_wifi_lost) / sizeof(Note), 3, K_EVENT},

104	  {"cloud_down", n_cloud_down, sizeof(n_cloud_down) / sizeof(Note), 2, K_EVENT},

105	  {"cloud_up", n_cloud_up, sizeof(n_cloud_up) / sizeof(Note), 2, K_EVENT},

106	  {"cmd_fail", n_cmd_fail, sizeof(n_cmd_fail) / sizeof(Note), 3, K_EVENT},

107	  {"bt_wait", n_bt_wait, sizeof(n_bt_wait) / sizeof(Note), 1, K_EVENT},

108	  {"bt_code", n_bt_code, sizeof(n_bt_code) / sizeof(Note), 2, K_EVENT},

109	  {"bt_paired", n_bt_paired, sizeof(n_bt_paired) / sizeof(Note), 3, K_EVENT},

110	  {"bt_wrong", n_bt_wrong, sizeof(n_bt_wrong) / sizeof(Note), 3, K_EVENT},

111	  {"bt_stopped", n_bt_stopped, sizeof(n_bt_stopped) / sizeof(Note), 4, K_EVENT},

112	  {"bt_joined", n_bt_joined, sizeof(n_bt_joined) / sizeof(Note), 3, K_EVENT},

113	  {"bt_join_fail", n_bt_join_fail, sizeof(n_bt_join_fail) / sizeof(Note), 3, K_EVENT},

114	  {"bt_off", n_bt_off, sizeof(n_bt_off) / sizeof(Note), 2, K_EVENT},

115	  {"bt_auto_off", n_bt_auto_off, sizeof(n_bt_auto_off) / sizeof(Note), 1, K_EVENT},

116	};

117

118	static_assert(sizeof(TUNES) / sizeof(TUNES[0]) == T_COUNT, "tune table and enum differ");

119

120	// playing holds the priority of the tune now sounding, 0xFF when silent.

121	static QueueHandle_t q;

122	static volatile uint8_t playing = 0xFF;

123	static constexpr uint8_t CH = 6, RES = 10;

124

125	// Volume 1/2/3 maps to PWM duty 100/256/512 of 1023 (50% is the loudest a

126	// passive buzzer gets).

127	static uint16_t duty() {

128	  uint8_t v = calSnapshot().buzz_vol;

129	  uint16_t d = v >= 3 ? 512 : v == 2 ? 256 : 100;

130	  return BUZZER_ACTIVE_LOW ? (1023 - d) : d;

131	}

132

133	static void silence() {

134	  ledcWriteTone(PIN_BUZZER, 0);

135	  ledcWrite(PIN_BUZZER, BUZZER_ACTIVE_LOW ? 1023 : 0);

136	}

137

138	static void tone(uint16_t hz) {

139	  ledcWriteTone(PIN_BUZZER, hz);

140	  ledcWrite(PIN_BUZZER, duty());

141	}

142

143	// Quiet hours, which may wrap past midnight (e.g. 22:00 to 06:00).

144	static bool quietNow(const Cal& c) {

145	  int m = localMinuteOfDay();

146	  if (!c.quiet_on || m < 0) return false;

147	  return c.quiet_from <= c.quiet_to ? (m >= c.quiet_from && m < c.quiet_to) : (m >= c.quiet_from || m < c.quiet_to);

148	}

149

150	// Plays one tune at a time. A new tune waiting in the queue cuts the current

151	// one short at the next note.

152	static void buzzerTask(void*) {

153	  uint8_t id;

154	  for (;;) {

155	    if (xQueueReceive(q, &id, portMAX_DELAY) != pdTRUE) continue;

156	    const TuneDef& t = TUNES[id];

157	    playing = t.pri;

158	    for (uint8_t i = 0; i < t.len; i++) {

159	      if (uxQueueMessagesWaiting(q)) break;

160	      const Note& n = t.n[i];

161	      if (!n.hz) {

162	        silence();

163	        vTaskDelay(pdMS_TO_TICKS(n.ms));

164	      } else if (n.hzEnd) {

165	        for (uint16_t s = 0; s < n.ms; s += 10) {

166	          tone(n.hz + (int32_t)(n.hzEnd - n.hz) * s / n.ms);

167	          vTaskDelay(pdMS_TO_TICKS(10));

168	        }

169	      } else {

170	        tone(n.hz);

171	        vTaskDelay(pdMS_TO_TICKS(n.ms));

172	      }

173	    }

174	    silence();

175	    playing = 0xFF;

176	  }

177	}

178

179	// 2 kHz carrier, 10-bit resolution, LEDC channel 6.

180	void buzzerInit() {

181	  ledcAttachChannel(PIN_BUZZER, 2000, RES, CH);

182	  silence();

183	  q = xQueueCreate(1, sizeof(uint8_t));

184	  xTaskCreatePinnedToCore(buzzerTask, "buzzer", 2048, nullptr, 2, nullptr, 1);

185	}

186

187	// Filters, in order: master switch, UI and lid sound switches, quiet hours

188	// (only priority-5 alarms play), alarms muted while collecting, only

189	// calibration and UI sounds while calibrating, and no interrupting a

190	// higher-priority tune. If a sound never plays, check these settings first.

191	void buzz(Tune id) {

192	  if (id >= T_COUNT || !q) return;

193	  const TuneDef& t = TUNES[id];

194	  Cal c = calSnapshot();

195	  BinState s = snapshot();

196	  if (!c.buzz_on) return;

197	  if (t.kind == K_UI && !c.buzz_ui) return;

198	  if (t.kind == K_LID && !c.buzz_lid) return;

199	  if (quietNow(c) && t.pri < 5) return;

200	  if (s.collecting && t.kind == K_ALARM) return;

201	  if (s.calibrating && t.kind != K_CAL && t.kind != K_UI) return;

202	  if (playing != 0xFF && t.pri < playing) return;

203	  uint8_t v = id;

204	  xQueueOverwrite(q, &v);

205	}

206

207	// Used by BUZZ_TEST from the apps. Skips all filters so the test always plays.

208	bool buzzByName(const char* name) {

209	  for (uint8_t i = 0; i < T_COUNT; i++)

210	    if (!strcmp(TUNES[i].name, name)) {

211	      uint8_t v = i;

212	      if (q) xQueueOverwrite(q, &v);

213	      return true;

214	    }

215	  return false;

216	}

217

218	bool buzzerBusy() { return playing != 0xFF; }

firmware/SmartBin/net.h

1	// net.h - Wi-Fi, the local web server (dashboard + REST API) and live events.

2	#pragma once

3	#include <Arduino.h>

4

5	void netInit();

6	void wifiConnect();

7	void startWebServer();

8	void pushTask(void*);

firmware/SmartBin/net.cpp

1	// ============================================================================

2	// net.cpp - Wi-Fi station, mDNS, SNTP and the local HTTP API

3	// Local API (port 80, CORS open so the dashboard file can be opened anywhere):

4	//   GET  /             the web dashboard (gzipped, from dashboard_gz.h)

5	//   GET  /api/status   full status JSON including calibration

6	//   GET  /api/events   Server-Sent Events: "status" on change, "ping" every 10 s

7	//   POST /api/command  {"cmd": "LOCK", "grams": 500, "sound": "full"}

8	//   POST /api/calibrate  any Cal keys, e.g. {"fill_th": 80}

9	//   POST /api/simulate {"mask": 3, "values": {"fill": 90}}

10	//   POST /api/cloud    {"url", "key", "email", "pass"} for Firebase

11	// Quick test from a laptop on the same network:

12	//   curl http://smartbin-01.local/api/status

13	// ============================================================================

14	#include "net.h"

15	#include "state.h"

16	#include "sensors.h"

17	#include "buzzer.h"

18	#include <WiFi.h>

19	#include <ESPmDNS.h>

20	#include <ESPAsyncWebServer.h>

21	#include <AsyncJson.h>

22	#include <esp_sntp.h>

23

24	// dashboard_gz.h is generated by tools/embed_dashboard.py. Without it the bin

25	// still builds and serves a small fallback page.

26	#if __has_include("dashboard_gz.h")

27	#include "dashboard_gz.h"

28	#define HAVE_DASHBOARD 1

29	#else

30	#define HAVE_DASHBOARD 0

31	#endif

32

33	static AsyncWebServer server(80);

34	static AsyncEventSource events("/api/events");

35	static bool everConnected = false;

36

37	// Wi-Fi callback, runs in the Wi-Fi event task. On GOT_IP it starts mDNS

38	// (smartbin-0x.local), starts SNTP time once, beeps and prints the address:

39	//   [wifi] 192.168.1.23  http://smartbin-01.local

40	// If that line never appears, the SSID or password is wrong or the network

41	// is 5 GHz only (the ESP32 supports 2.4 GHz only).

42	static void onWifiEvent(arduino_event_id_t event, arduino_event_info_t) {

43	  if (event == ARDUINO_EVENT_WIFI_STA_GOT_IP) {

44	    lockState();

45	    st.wifi = true;

46	    st.rssi = WiFi.RSSI();

47	    unlockState();

48	    MDNS.end();

49	    if (MDNS.begin(BIN_HOST)) MDNS.addService("http", "tcp", 80);

50	    if (!esp_sntp_enabled()) {

51	      esp_sntp_setoperatingmode(ESP_SNTP_OPMODE_POLL);

52	      esp_sntp_setservername(0, "pool.ntp.org");

53	      esp_sntp_setservername(1, "time.google.com");

54	      esp_sntp_init();

55	    }

56	    buzz(T_WIFI_UP);

57	    everConnected = true;

58	    Serial.printf("[wifi] %s  http://%s.local\n", WiFi.localIP().toString().c_str(), BIN_HOST);

59	    publish(EV_WIFI);

60	  } else if (event == ARDUINO_EVENT_WIFI_STA_DISCONNECTED) {

61	    bool was;

62	    lockState();

63	    was = st.wifi;

64	    st.wifi = false;

65	    unlockState();

66	    if (was) {

67	      buzz(T_WIFI_LOST);

68	      Serial.println("[wifi] lost");

69	      publish(EV_WIFI);

70	    }

71	  }

72	}

73

74	// persistent(false): the ESP32 SDK does not store credentials; this firmware

75	// keeps them in its own Preferences (see saveNet).

76	void netInit() {

77	  WiFi.persistent(false);

78	  WiFi.mode(WIFI_STA);

79	  WiFi.setHostname(BIN_HOST);

80	  WiFi.setAutoReconnect(true);

81	  WiFi.onEvent(onWifiEvent);

82	}

83

84	// Uses the saved network, or the one in secrets.h. Auto-reconnect handles drops.

85	void wifiConnect() {

86	  if (net.ssid.length() == 0 || net.ssid == "your-wifi-name") {

87	    Serial.println("[wifi] no network set; use WI-FI SETUP on the touchscreen");

88	    return;

89	  }

90	  WiFi.begin(net.ssid.c_str(), net.pass.c_str());

91	}

92

93	static void sendJson(AsyncWebServerRequest* r, int code, const String& body) {

94	  AsyncWebServerResponse* res = r->beginResponse(code, "application/json", body);

95	  res->addHeader("Cache-Control", "no-store");

96	  r->send(res);

97	}

98

99	// The status the bin will have after the command runs, returned at once so

100	// the dashboard can update its button without waiting for the next event.

101	static const char* expectStatus(const char* cmd, const BinState& s) {

102	  if (!strcmp(cmd, "LOCK")) return "LOCKED";

103	  if (!strcmp(cmd, "COLLECT")) return "COLLECTING";

104	  if (!strcmp(cmd, "CALIBRATE_START")) return "CALIBRATING";

105	  if (!strcmp(cmd, "UNLOCK") || !strcmp(cmd, "CLEAR_TAMPER") || !strcmp(cmd, "COLLECT_DONE") || !strcmp(cmd, "CALIBRATE_END"))

106	    return s.tamperPhase && strcmp(cmd, "CLEAR_TAMPER") ? "TAMPER" : (s.locked && strcmp(cmd, "UNLOCK") && !s.autoLocked) ? "LOCKED" : "OK";

107	  return STATUS_NAMES[s.status];

108	}

109

110	// POST /api/command. 400 = unknown command, 409 = not allowed right now (the

111	// body says why), 503 = command queue full.

112	static void handleCommand(AsyncWebServerRequest* r, JsonVariant& j) {

113	  const char* cmd = j["cmd"] | "";

114	  BinState s = snapshot();

115	  String why;

116	  JsonDocument out;

117	  out["cmd"] = cmd;

118	  if (!validateCommand(cmd, s, why)) {

119	    out["ok"] = false;

120	    out["why"] = why;

121	    String b;

122	    serializeJson(out, b);

123	    sendJson(r, why == "Unknown command" ? 400 : 409, b);

124	    return;

125	  }

126	  if (!sendCommand(cmd, SRC_LOCAL, j["grams"] | 0.0f, j["sound"] | (const char*)nullptr)) {

127	    sendJson(r, 503, "{\"ok\":false,\"why\":\"Busy, try again\"}");

128	    return;

129	  }

130	  out["ok"] = true;

131	  out["status"] = expectStatus(cmd, s);

132	  String b;

133	  serializeJson(out, b);

134	  sendJson(r, 200, b);

135	}

136

137	// POST /api/calibrate. Sending d_empty or d_full during calibration ticks

138	// the matching step on the bin's calibration screen.

139	static void handleCalibrate(AsyncWebServerRequest* r, JsonVariant& j) {

140	  if (!j.is<JsonObject>()) { sendJson(r, 400, "{\"ok\":false,\"why\":\"Send a JSON object\"}"); return; }

141	  JsonObjectConst o = j.as<JsonObjectConst>();

142	  lockState();

143	  Cal c = cal;

144	  bool any = calFromJson(o, c);

145	  if (any) {

146	    cal = c;

147	    if (!o["d_empty"].isNull()) st.calSteps |= 2;

148	    if (!o["d_full"].isNull()) st.calSteps |= 4;

149	    st.calVersion++;

150	    recomputeView();

151	  }

152	  bool calibrating = st.calibrating;

153	  unlockState();

154	  if (!any) { sendJson(r, 400, "{\"ok\":false,\"why\":\"No known calibration keys\"}"); return; }

155	  saveCal();

156	  buzz(calibrating ? T_CAL_STEP : T_SAVED);

157	  publish(EV_CAL);

158	  sendJson(r, 200, "{\"ok\":true}");

159	}

160

161	// POST /api/simulate. mask 0 turns simulation off. Values not sent keep their

162	// previous simulated value.

163	static void handleSimulate(AsyncWebServerRequest* r, JsonVariant& j) {

164	  lockState();

165	  sim.mask = (uint8_t)(j["mask"] | 0) & 0x7F;

166	  JsonObjectConst v = j["values"].as<JsonObjectConst>();

167	  if (!v.isNull()) {

168	    sim.fill = v["fill"] | sim.fill;

169	    sim.weight = v["weight"] | sim.weight;

170	    sim.gas = v["gas"] | sim.gas;

171	    sim.temp = v["temp"] | sim.temp;

172	    sim.humidity = v["humidity"] | sim.humidity;

173	    if (!v["tamper"].isNull()) sim.tamper = v["tamper"].as<bool>();

174	    if (!v["motion"].isNull()) sim.motion = v["motion"].as<bool>();

175	    if (!v["lid"].isNull()) sim.lid = v["lid"].as<bool>() || (v["lid"].is<const char*>() && !strcmp(v["lid"], "OPEN"));

176	  }

177	  recomputeView();

178	  unlockState();

179	  if (hControl) xTaskNotify(hControl, EV_STATE, eSetBits);

180	  publish();

181	  sendJson(r, 200, "{\"ok\":true}");

182	}

183

184	// POST /api/cloud. Saves Firebase settings and tells cloudTask to sign in

185	// again. An empty URL turns Firebase off.

186	static void handleCloud(AsyncWebServerRequest* r, JsonVariant& j) {

187	  String url = j["url"] | "", key = j["key"] | "", email = j["email"] | "", pass = j["pass"] | "";

188	  url.trim();

189	  while (url.endsWith("/")) url.remove(url.length() - 1);

190	  if (url.length() && !url.startsWith("https://") && !url.startsWith("http://")) {

191	    sendJson(r, 400, "{\"ok\":false,\"why\":\"The database URL starts with https://\"}");

192	    return;

193	  }

194	  net.fbUrl = url;

195	  net.fbKey = key;

196	  net.fbEmail = email;

197	  net.fbPass = pass;

198	  saveNet();

199	  if (hCloud) xTaskNotify(hCloud, EV_WIFI, eSetBits);

200	  sendJson(r, 200, "{\"ok\":true}");

201	}

202

203	// The extra CORS header allows the dashboard, opened from a public page,

204	// to call a bin on the private network (Chrome's Private Network Access).

205	void startWebServer() {

206	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Origin", "*");

207	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Methods", "GET, POST, OPTIONS");

208	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Headers", "Content-Type, X-SB-Op");

209	  DefaultHeaders::Instance().addHeader("Access-Control-Allow-Private-Network", "true");

210

211	  server.on("/api/status", HTTP_GET, [](AsyncWebServerRequest* r) { sendJson(r, 200, statusJson(true, false)); });

212

213	  auto add = [](const char* uri, ArJsonRequestHandlerFunction fn) {

214	    AsyncCallbackJsonWebHandler* h = new AsyncCallbackJsonWebHandler(uri, fn);

215	    h->setMethod(HTTP_POST);

216	    h->setMaxContentLength(1024);

217	    server.addHandler(h);

218	  };

219	  add("/api/command", handleCommand);

220	  add("/api/calibrate", handleCalibrate);

221	  add("/api/simulate", handleSimulate);

222	  add("/api/cloud", handleCloud);

223

224	  // A new live-event client gets the full status at once; 3000 is the

225	  // reconnect delay in ms suggested to the browser.

226	  events.onConnect([](AsyncEventSourceClient* c) { c->send(statusJson(true, false).c_str(), "status", millis(), 3000); });

227	  server.addHandler(&events);

228

229	  server.on("/", HTTP_GET, [](AsyncWebServerRequest* r) {

230	#if HAVE_DASHBOARD

231	    AsyncWebServerResponse* res = r->beginResponse(200, "text/html", DASHBOARD_GZ, DASHBOARD_GZ_LEN);

232	    res->addHeader("Content-Encoding", "gzip");

233	    r->send(res);

234	#else

235	    r->send(200, "text/html",

236	            "<!doctype html><meta name=viewport content='width=device-width'><title>" BIN_LABEL "</title>"

237	            "<body style='font-family:sans-serif;padding:24px'><h1>" BIN_LABEL "</h1>"

238	            "<p>The dashboard is not built into this firmware. Open <a href='/api/status'>/api/status</a>, or run "

239	            "tools/embed_dashboard.py and upload again.</p>");

240	#endif

241	  });

242

243	  server.onNotFound([](AsyncWebServerRequest* r) {

244	    if (r->method() == HTTP_OPTIONS) r->send(204);

245	    else sendJson(r, 404, "{\"ok\":false,\"why\":\"Not found\"}");

246	  });

247	  server.begin();

248	}

249

250	// Sends a "status" event to all live dashboards when st.version changes, at

251	// most every 250 ms, and a "ping" every 10 s so browsers notice a dead link.

252	// Also refreshes the Wi-Fi signal strength every 5 s.

253	void pushTask(void*) {

254	  uint32_t lastSent = 0, lastPush = 0, lastPing = 0, lastRssi = 0;

255	  for (;;) {

256	    uint32_t ev = 0;

257	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(250));

258	    uint32_t now = millis();

259	    if (now - lastRssi > 5000 && WiFi.isConnected()) {

260	      lastRssi = now;

261	      lockState();

262	      st.rssi = WiFi.RSSI();

263	      unlockState();

264	    }

265	    if (!events.count()) continue;

266	    lockState();

267	    uint32_t v = st.version;

268	    unlockState();

269	    if (v != lastSent && now - lastPush >= 250) {

270	      events.send(statusJson(true, false).c_str(), "status", now);

271	      lastSent = v;

272	      lastPush = now;

273	    }

274	    if (now - lastPing >= 10000) {

275	      lastPing = now;

276	      events.send("1", "ping", now);

277	    }

278	  }

279	}

firmware/SmartBin/cloud.h

1	// cloud.h - Firebase and ThingSpeak (runs as cloudTask on core 0).

2	#pragma once

3	#include <Arduino.h>

4

5	void cloudTask(void*);

6	bool cloudFirebaseOn();

firmware/SmartBin/cloud.cpp

1	// ============================================================================

2	// cloud.cpp - Firebase Realtime Database and ThingSpeak over HTTPS (REST)

3	// Firebase paths used (rules in firmware/firebase.rules.json):

4	//   sb/bins/<id>/status   status JSON (PUT on change, at least every 60 s)

5	//   sb/bins/<id>/cal      calibration (PUT when it changes)

6	//   sb/commands/<id>/<k>  commands from the app: read, then deleted

7	//   sb/results/<k>        result of each command, written back by the bin

8	//   sb/alerts/<id>        alert log, trimmed to the newest 50

9	//   sb/watchers           app/dashboard heartbeats (who is looking)

10	//   sb/usage              estimated monthly download, shared by both bins

11	// ThingSpeak fields: 1 fill %, 2 weight g, 3 temp C, 4 humidity %, 5 IAQ,

12	// 6 pressure hPa, 7 tilt deg, 8 flags (1 motion + 2 lid open + 4 x status).

13	// Serial log tags: [firebase], [thingspeak], [talkback], [alert].

14	// ============================================================================

15	#include "cloud.h"

16	#include "state.h"

17	#include "buzzer.h"

18	#include <WiFi.h>

19	#include <WiFiClientSecure.h>

20	#include <HTTPClient.h>

21	#include <vector>

22	#include <algorithm>

23

24	// One HTTP client per service, each with its own retry timer, so a slow

25	// ThingSpeak does not delay Firebase.

26	struct Svc {

27	  WiFiClientSecure tls;

28	  WiFiClient plain;

29	  HTTPClient http;

30	  uint32_t retryAt = 0;

31	  uint8_t fails = 0;

32	};

33

34	static Svc fb, ts, auth;

35	static String idToken;

36	static uint32_t tokenUntil = 0;

37	static bool fbOk = false;

38	static int watchers = 0;

39	static uint64_t monthBytes = 0;

40	static String monthKey;

41	static float usageFraction = 0;

42

43	bool cloudFirebaseOn() { return net.fbUrl.length() && net.fbKey.length() && net.fbEmail.length() && net.fbPass.length(); }

44

45	// One HTTP request with keep-alive. Returns the HTTP code, or a negative

46	// value for a connection error (no Wi-Fi, DNS, TLS handshake).

47	static int request(Svc& s, const String& url, const char* method, const String& body, String* out,

48	                   const char* ctype = "application/json") {

49	  Client& c = url.startsWith("https://") ? (Client&)s.tls : (Client&)s.plain;

50	  s.http.setReuse(true);

51	  s.http.setTimeout(8000);

52	  s.http.setConnectTimeout(6000);

53	  if (!s.http.begin((NetworkClient&)c, url)) return -1;

54	  if (body.length()) s.http.addHeader("Content-Type", ctype);

55	  int code = s.http.sendRequest(method, (uint8_t*)body.c_str(), body.length());

56	  if (out) *out = code > 0 ? s.http.getString() : String();

57	  else if (code > 0) s.http.getString();

58	  s.http.end();

59	  return code;

60	}

61

62	// After a failure, wait 5, 15, 30 then 60 s before the next try and close the

63	// connection so the next attempt starts fresh.

64	static void backoff(Svc& s) {

65	  static const uint16_t steps[] = {5, 15, 30, 60};

66	  s.retryAt = millis() + steps[min<uint8_t>(s.fails, 3)] * 1000UL;

67	  if (s.fails < 10) s.fails++;

68	  s.tls.stop();

69	  s.plain.stop();

70	}

71

72	static bool due(const Svc& s) { return (int32_t)(millis() - s.retryAt) >= 0; }

73

74	// Tracks whether Firebase is reachable; the change beeps, logs and shows on

75	// the Info screen as "Firebase OK" or "NOT SENDING".

76	static void setFbOk(bool ok) {

77	  if (ok == fbOk) return;

78	  fbOk = ok;

79	  lockState();

80	  st.cloudOk = ok;

81	  unlockState();

82	  buzz(ok ? T_CLOUD_UP : T_CLOUD_DOWN);

83	  Serial.printf("[firebase] %s\n", ok ? "reachable" : "cannot be reached");

84	  publish();

85	}

86

87	// URL-encodes keys and tokens for query strings.

88	static String enc(const String& s) {

89	  String o;

90	  const char* hex = "0123456789ABCDEF";

91	  for (size_t i = 0; i < s.length(); i++) {

92	    char ch = s[i];

93	    if (isalnum((unsigned char)ch) || ch == '-' || ch == '_' || ch == '.' || ch == '~') o += ch;

94	    else { o += '%'; o += hex[(ch >> 4) & 15]; o += hex[ch & 15]; }

95	  }

96	  return o;

97	}

98

99	// Email/password sign-in through Firebase Auth's REST API. The ID token lasts

100	// about an hour; it is renewed 5 minutes early. A "sign-in failed (400)" line

101	// usually means a wrong API key, email or password, or that Email/Password

102	// sign-in is not enabled in the Firebase console. The http:// branch is for

103	// the local mock server used in testing.

104	static bool signIn() {

105	  String url = net.fbUrl.startsWith("http://")

106	                   ? net.fbUrl + "/__auth/signInWithPassword?key=" + enc(net.fbKey)

107	                   : "https://identitytoolkit.googleapis.com/v1/accounts:signInWithPassword?key=" + enc(net.fbKey);

108	  JsonDocument d;

109	  d["email"] = net.fbEmail;

110	  d["password"] = net.fbPass;

111	  d["returnSecureToken"] = true;

112	  String body, resp;

113	  serializeJson(d, body);

114	  auth.http.setReuse(false);

115	  Client& c = url.startsWith("https://") ? (Client&)auth.tls : (Client&)auth.plain;

116	  auth.http.setTimeout(10000);

117	  if (!auth.http.begin((NetworkClient&)c, url)) return false;

118	  auth.http.addHeader("Content-Type", "application/json");

119	  int code = auth.http.POST(body);

120	  if (code > 0) resp = auth.http.getString();

121	  auth.http.end();

122	  auth.tls.stop();

123	  if (code != 200) {

124	    Serial.printf("[firebase] sign-in failed (%d) %s\n", code, resp.substring(0, 120).c_str());

125	    return false;

126	  }

127	  JsonDocument r;

128	  if (deserializeJson(r, resp)) return false;

129	  idToken = r["idToken"] | "";

130	  uint32_t secs = atoi(r["expiresIn"] | "3600");

131	  tokenUntil = millis() + (secs > 400 ? secs - 300 : secs) * 1000UL;

132	  return idToken.length() > 0;

133	}

134

135	static String fbUrl(const String& path, const String& query = "") {

136	  return net.fbUrl + "/" + path + ".json?auth=" + enc(idToken) + query;

137	}

138

139	// Every Firebase call goes through here: signs in when needed, retries once

140	// on 401 (expired token), backs off on network or 5xx errors. Writes are

141	// counted to estimate monthly download (each write is downloaded once per

142	// app or dashboard watching).

143	static int fbCall(const char* method, const String& path, const String& body, String* out, const String& query = "",

144	                  bool count = true) {

145	  if (!idToken.length() || (int32_t)(millis() - tokenUntil) >= 0) {

146	    if (!signIn()) {

147	      backoff(fb);

148	      setFbOk(false);

149	      return -1;

150	    }

151	  }

152	  int code = request(fb, fbUrl(path, query), method, body, out);

153	  if (code == 401) {

154	    idToken = "";

155	    code = request(fb, fbUrl(path, query), method, body, out);

156	  }

157	  if (code <= 0 || code >= 500) {

158	    backoff(fb);

159	    setFbOk(false);

160	    return code;

161	  }

162	  fb.fails = 0;

163	  setFbOk(code < 400);

164	  if (count && (!strcmp(method, "PUT") || !strcmp(method, "PATCH") || !strcmp(method, "POST")))

165	    monthBytes += (uint64_t)body.length() * max(1, watchers);

166	  return code;

167	}

168

169	static String currentMonth() {

170	  char b[12];

171	  monthKeyUtc(b, sizeof(b));

172	  return b;

173	}

174

175	// Restores this month's byte count after a reboot.

176	static void loadUsage() {

177	  String out;

178	  if (fbCall("GET", String("sb/usage/by_bin/") + BIN_ID, "", &out, "", false) != 200) return;

179	  JsonDocument d;

180	  if (deserializeJson(d, out) || !d.is<JsonObject>()) return;

181	  String m = d["month"] | "";

182	  monthKey = currentMonth();

183	  if (m == monthKey) monthBytes = d["bytes"] | (uint64_t)0;

184	}

185

186	// Saves this bin's count, then sums both bins and publishes the shared

187	// usage level (ok / warn 70% / near 90% / bad 100%) for the apps to show.

188	static void writeUsage() {

189	  String m = currentMonth();

190	  if (!m.length()) return;

191	  if (m != monthKey) { monthKey = m; monthBytes = 0; }

192	  JsonDocument me;

193	  me["month"] = m;

194	  me["bytes"] = monthBytes;

195	  String b;

196	  serializeJson(me, b);

197	  fbCall("PUT", String("sb/usage/by_bin/") + BIN_ID, b, nullptr, "", false);

198	  String out;

199	  if (fbCall("GET", "sb/usage/by_bin", "", &out, "", false) != 200) return;

200	  JsonDocument all;

201	  if (deserializeJson(all, out)) return;

202	  uint64_t total = 0;

203	  for (JsonPair p : all.as<JsonObject>())

204	    if (m == (p.value()["month"] | "")) total += p.value()["bytes"] | (uint64_t)0;

205	  usageFraction = (float)((double)total / (double)FB_MONTH_LIMIT);

206	  JsonDocument u;

207	  u["month"] = m;

208	  u["bytes_month"] = total;

209	  u["down_bytes"] = total;

210	  u["limit_bytes"] = FB_MONTH_LIMIT;

211	  u["fraction"] = round(usageFraction * 10000.0) / 10000.0;

212	  u["level"] = usageFraction >= 1 ? "bad" : usageFraction >= 0.9 ? "near" : usageFraction >= 0.7 ? "warn" : "ok";

213	  u["estimate"] = true;

214	  b = "";

215	  serializeJson(u, b);

216	  fbCall("PATCH", "sb/usage", b, nullptr, "", false);

217	}

218

219	// Counts apps/dashboards that sent a heartbeat in the last 30 s. Uses the

220	// "ts" index when the clock is set; falls back to reading the whole list.

221	static void readWatchers() {

222	  String out;

223	  int code = -1;

224	  if (clockSet()) {

225	    uint64_t since = nowMs() - 30000ULL;

226	    code = fbCall("GET", "sb/watchers", "", &out, "&orderBy=%22ts%22&startAt=" + String((unsigned long long)since), false);

227	  }

228	  if (code != 200) code = fbCall("GET", "sb/watchers", "", &out, "", false);

229	  if (code != 200) return;

230	  JsonDocument d;

231	  if (deserializeJson(d, out) || !d.is<JsonObject>()) { watchers = 0; return; }

232	  int n = 0;

233	  uint64_t now = nowMs();

234	  for (JsonPair p : d.as<JsonObject>()) {

235	    uint64_t t = p.value()["ts"] | (uint64_t)0;

236	    if (!now || (t && now - t <= 30000ULL)) n++;

237	  }

238	  watchers = n;

239	}

240

241	// Reads queued commands and deletes each one first, so a command never runs

242	// twice after a reboot. Only LOCK, UNLOCK and COLLECT are accepted from the

243	// cloud; calibration and simulation need the bin's own Wi-Fi. Commands older

244	// than CMD_MAX_AGE_MS are refused, so a lost phone cannot replay old taps.

245	static void pollCommands() {

246	  String out;

247	  if (fbCall("GET", String("sb/commands/") + BIN_ID, "", &out, "", false) != 200) return;

248	  if (out == "null" || out.length() < 3) return;

249	  JsonDocument d;

250	  if (deserializeJson(d, out) || !d.is<JsonObject>()) return;

251	  uint64_t now = nowMs();

252	  for (JsonPair p : d.as<JsonObject>()) {

253	    String key = p.key().c_str();

254	    const char* cmd = p.value()["cmd"] | "";

255	    uint64_t t = p.value()["ts"] | (uint64_t)0;

256	    fbCall("DELETE", String("sb/commands/") + BIN_ID + "/" + key, "", nullptr, "", false);

257	    bool stale = now && t && now > t && now - t > CMD_MAX_AGE_MS;

258	    bool allowed = !strcmp(cmd, "LOCK") || !strcmp(cmd, "UNLOCK") || !strcmp(cmd, "COLLECT");

259	    if (stale || !allowed) {

260	      JsonDocument r;

261	      r["ok"] = false;

262	      r["cmd"] = cmd;

263	      r["why"] = stale ? "Too old, ignored" : "Needs the bin's Wi-Fi";

264	      r["ts"][".sv"] = "timestamp";

265	      String b;

266	      serializeJson(r, b);

267	      fbCall("PUT", "sb/results/" + key, b, nullptr);

268	      continue;

269	    }

270	    Serial.printf("[firebase] command %s\n", cmd);

271	    if (!sendCommand(cmd, SRC_FIREBASE, 0, nullptr, key.c_str())) {

272	      JsonDocument r;

273	      r["ok"] = false;

274	      r["cmd"] = cmd;

275	      r["why"] = "Busy";

276	      String b;

277	      serializeJson(r, b);

278	      fbCall("PUT", "sb/results/" + key, b, nullptr);

279	    }

280	  }

281	}

282

283	// Writes queued command results to sb/results/<ref>.

284	static void flushResults() {

285	  CmdResult r;

286	  while (xQueueReceive(resultQueue, &r, 0) == pdTRUE) {

287	    if (!r.ref[0]) continue;

288	    JsonDocument d;

289	    d["ok"] = r.ok;

290	    d["cmd"] = r.cmd;

291	    if (r.why[0]) d["why"] = r.why;

292	    d["ts"][".sv"] = "timestamp";

293	    String b;

294	    serializeJson(d, b);

295	    fbCall("PUT", String("sb/results/") + r.ref, b, nullptr);

296	  }

297	}

298

299	// Posts queued alerts (POST makes a time-ordered key). Always logged to Serial.

300	static void flushAlerts(bool fbOn) {

301	  AlertEv a;

302	  while (xQueueReceive(alertQueue, &a, 0) == pdTRUE) {

303	    Serial.printf("[alert] %s: %s (%s)\n", a.title, a.meta, a.sev);

304	    if (!fbOn) continue;

305	    JsonDocument d;

306	    d["kind"] = a.kind;

307	    d["sev"] = a.sev;

308	    d["title"] = a.title;

309	    d["meta"] = a.meta;

310	    d["ts"][".sv"] = "timestamp";

311	    String b;

312	    serializeJson(d, b);

313	    fbCall("POST", String("sb/alerts/") + BIN_ID, b, nullptr);

314	  }

315	}

316

317	// Keeps the newest 50 alerts. Keys made by POST sort by time, so the lowest

318	// keys are the oldest.

319	static void trimAlerts() {

320	  String out;

321	  if (fbCall("GET", String("sb/alerts/") + BIN_ID, "", &out, "&shallow=true", false) != 200) return;

322	  JsonDocument d;

323	  if (deserializeJson(d, out) || !d.is<JsonObject>()) return;

324	  JsonObject o = d.as<JsonObject>();

325	  int n = o.size();

326	  if (n <= 50) return;

327	  std::vector<String> keys;

328	  for (JsonPair p : o) keys.push_back(p.key().c_str());

329	  std::sort(keys.begin(), keys.end());

330	  for (int i = 0; i < n - 50; i++) fbCall("DELETE", String("sb/alerts/") + BIN_ID + "/" + keys[i], "", nullptr, "", false);

331	}

332

333	// What was last sent to a service, used to skip uploads when nothing changed.

334	struct Sent {

335	  float fill = -100, weight = -1000, temp = -100, hum = -100, iaq = -100, tilt = -100;

336	  uint8_t status = 255, tamper = 255, acc = 255;

337	  bool lid = false, motion = false, full = false, env = false;

338	  uint8_t faults = 255;

339	};

340

341	static uint8_t faultBits(const BinState& s) { return (!s.okHC) | (!s.okHX << 1) | (!s.okBME << 2) | (!s.okIMU << 3); }

342

343	// A change in status, lid, tamper, alarms or faults: sent right away.

344	static bool importantChange(const Sent& a, const BinState& s) {

345	  return a.status != s.status || a.lid != s.lidOpen || a.tamper != s.tamperPhase || a.full != s.alarmFull ||

346	         a.env != s.alarmEnv || a.faults != faultBits(s);

347	}

348

349	// Sensor drift above the same steps used in sensorTask: sent on the slower pace.

350	static bool minorChange(const Sent& a, const BinState& s) {

351	  const Reading& v = s.v;

352	  return fabsf(a.fill - v.fill) >= 1 || fabsf(a.weight - v.weight_g) >= 20 || fabsf(a.temp - v.temp) >= 0.2f ||

353	         fabsf(a.hum - v.humidity) >= 1 || fabsf(a.iaq - v.iaq) >= 5 || fabsf(a.tilt - v.tilt) >= 1 ||

354	         a.motion != v.motion || a.acc != v.iaq_acc;

355	}

356

357	static void remember(Sent& a, const BinState& s) {

358	  a.fill = s.v.fill; a.weight = s.v.weight_g; a.temp = s.v.temp; a.hum = s.v.humidity; a.iaq = s.v.iaq;

359	  a.tilt = s.v.tilt; a.status = s.status; a.tamper = s.tamperPhase; a.acc = s.v.iaq_acc; a.lid = s.lidOpen;

360	  a.motion = s.v.motion; a.full = s.alarmFull; a.env = s.alarmEnv; a.faults = faultBits(s);

361	}

362

363	// One ThingSpeak channel update. The reply is the entry number; "0" means the

364	// write was refused (wrong key, or less than 15 s since the last write).

365	static bool tsWrite(const BinState& s) {

366	  const Reading& v = s.v;

367	  int flags = (v.motion ? 1 : 0) + (s.lidOpen ? 2 : 0) + 4 * s.status;

368	  String url = net.tsServer + "/update?api_key=" + enc(net.tsWriteKey) + "&field1=" + String(v.fill, 1) +

369	               "&field2=" + String(v.weight_g, 0) + "&field3=" + String(v.temp, 2) + "&field4=" + String(v.humidity, 1) +

370	               "&field5=" + String(v.iaq, 0) + "&field6=" + String(v.pressure, 1) + "&field7=" + String(v.tilt, 1) +

371	               "&field8=" + String(flags) + "&status=" + enc(localIp());

372	  String out;

373	  int code = request(ts, url, "GET", "", &out);

374	  out.trim();

375	  bool ok = code == 200 && out != "0";

376	  if (code <= 0 || code >= 500) backoff(ts);

377	  else ts.fails = 0;

378	  lockState();

379	  st.tsOk = ok;

380	  unlockState();

381	  if (!ok) Serial.printf("[thingspeak] write failed (%d) %s\n", code, out.c_str());

382	  return ok;

383	}

384

385	static Svc tb;

386

387	// ThingSpeak TalkBack: takes the next queued command (LOCK, UNLOCK or COLLECT).

388	static void talkBack() {

389	  String url = net.tsServer + "/talkbacks/" + enc(net.tbId) + "/commands/execute.json";

390	  String out;

391	  int code = request(tb, url, "POST", "api_key=" + enc(net.tbKey), &out, "application/x-www-form-urlencoded");

392	  if (code <= 0 || code >= 500) { backoff(tb); return; }

393	  tb.fails = 0;

394	  if (code != 200 || out.length() < 3) return;

395	  JsonDocument d;

396	  if (deserializeJson(d, out)) return;

397	  String cmd = d["command_string"] | "";

398	  cmd.trim();

399	  cmd.toUpperCase();

400	  if (cmd == "LOCK" || cmd == "UNLOCK" || cmd == "COLLECT") {

401	    Serial.printf("[talkback] command %s\n", cmd.c_str());

402	    sendCommand(cmd.c_str(), SRC_TALKBACK);

403	  }

404	}

405

406	// Main cloud loop, every 500 ms or when woken. Paused while Wi-Fi is down or

407	// Bluetooth setup is running (the radio is shared), in which case alerts are

408	// only printed to Serial.

409	void cloudTask(void*) {

410	  for (Svc* s : {&fb, &ts, &auth, &tb}) {

411	    // The server certificate is not checked (no CA bundle on the board). The

412	    // traffic is still encrypted.

413	    s->tls.setInsecure();

414	    s->tls.setHandshakeTimeout(12);

415	  }

416	  Sent fbSent, tsSent;

417	  uint32_t lastStatus = 0, lastCmd = 0, lastWatch = 0, lastUsage = 0, lastTrim = 0, lastTs = 0, lastTb = 0, lastCalVer = 0;

418	  bool usageLoaded = false, wasConnected = false;

419	  bool calSent = false;

420

421	  for (;;) {

422	    uint32_t ev = 0;

423	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(500));

424	    // New network or new Firebase settings: sign in again from scratch.

425	    if (ev & EV_WIFI) {

426	      idToken = "";

427	      fb.retryAt = 0;

428	      fb.fails = 0;

429	      calSent = false;

430	    }

431	    BinState s = snapshot();

432	    bool online = WiFi.isConnected() && !s.btOn;

433	    bool fbOn = cloudFirebaseOn();

434	    if (!online) {

435	      if (wasConnected) {

436	        for (Svc* x : {&fb, &ts, &tb}) { x->tls.stop(); x->plain.stop(); }

437	      }

438	      wasConnected = false;

439	      AlertEv a;

440	      while (xQueueReceive(alertQueue, &a, 0) == pdTRUE) Serial.printf("[alert] %s: %s\n", a.title, a.meta);

441	      continue;

442	    }

443	    wasConnected = true;

444	    uint32_t now = millis();

445

446	    // --- Firebase ---

447	    // Status pace: right away on an important change, every 5 s on minor

448	    // changes while someone watches (15 s above 90% of the free quota,

449	    // 60 s at 100%), and a 60 s heartbeat otherwise.

450	    if (fbOn && due(fb)) {

451	      if (!usageLoaded && clockSet()) {

452	        usageLoaded = true;

453	        loadUsage();

454	      }

455	      flushResults();

456	      flushAlerts(true);

457

458	      if (now - lastWatch >= FB_WATCHERS_MS || lastWatch == 0) {

459	        lastWatch = now;

460	        readWatchers();

461	      }

462	      bool watched = watchers > 0 || !FB_REQUIRE_WATCHERS;

463	      uint32_t every = !watched ? FB_IDLE_MS : usageFraction >= 0.9f ? 15000 : FB_WATCHED_MS;

464	      if (usageFraction >= 1.0f) every = FB_IDLE_MS;

465	      bool imp = importantChange(fbSent, s);

466	      bool minor = minorChange(fbSent, s);

467	      if (imp || (minor && now - lastStatus >= every) || now - lastStatus >= FB_IDLE_MS || lastStatus == 0) {

468	        String body = statusJson(false, true);

469	        if (fbCall("PUT", String("sb/bins/") + BIN_ID + "/status", body, nullptr) == 200) {

470	          remember(fbSent, s);

471	          lastStatus = now;

472	        }

473	      }

474	      if (!calSent || s.calVersion != lastCalVer) {

475	        JsonDocument d;

476	        calToJson(calSnapshot(), d.to<JsonObject>());

477	        String b;

478	        serializeJson(d, b);

479	        if (fbCall("PUT", String("sb/bins/") + BIN_ID + "/cal", b, nullptr) == 200) {

480	          calSent = true;

481	          lastCalVer = s.calVersion;

482	        }

483	      }

484	      if (now - lastCmd >= (watched ? FB_CMD_WATCHED_MS : FB_CMD_IDLE_MS)) {

485	        lastCmd = now;

486	        pollCommands();

487	      }

488	      if (now - lastUsage >= FB_USAGE_MS || lastUsage == 0) {

489	        lastUsage = now;

490	        writeUsage();

491	      }

492	      if (now - lastTrim >= 600000 || lastTrim == 0) {

493	        lastTrim = now;

494	        trimAlerts();

495	      }

496	    } else if (!fbOn) {

497	      flushAlerts(false);

498	      CmdResult r;

499	      while (xQueueReceive(resultQueue, &r, 0) == pdTRUE) {}

500	    }

501

502	    // --- ThingSpeak: every 30 s on change, 15.5 s on an alarm edge, and a

503	    // 5 minute heartbeat so the charts never show a gap. ---

504	    if (net.tsWriteKey.length() && due(ts)) {

505	      bool alertEdge = s.alarmFull != tsSent.full || s.tamperPhase != tsSent.tamper || s.status != tsSent.status;

506	      uint32_t since = now - lastTs;

507	      bool write = lastTs == 0 || (alertEdge && since >= TS_MIN_GAP_MS) ||

508	                   (since >= TS_WRITE_MS && (minorChange(tsSent, s) || importantChange(tsSent, s))) || since >= TS_HEARTBEAT_MS;

509	      if (write) {

510	        lastTs = now;

511	        if (tsWrite(s)) remember(tsSent, s);

512	      }

513	    }

514

515	    // --- TalkBack polling every 5 s (15 s when nobody watches) ---

516	    if (net.tbId.length() && net.tbKey.length() && due(tb)) {

517	      uint32_t every = (fbOn && watchers == 0 && FB_REQUIRE_WATCHERS) ? TB_IDLE_MS : TB_WATCHED_MS;

518	      if (now - lastTb >= every) {

519	        lastTb = now;

520	        talkBack();

521	      }

522	    }

523	  }

524	}

firmware/SmartBin/ble_setup.h

1	// ble_setup.h - Bluetooth LE Wi-Fi setup (started from WI-FI SETUP on the screen).

2	#pragma once

3	#include <Arduino.h>

4

5	enum BtPhase : uint8_t { BT_IDLE = 0, BT_WAIT, BT_CODE, BT_LOCKOUT, BT_PAIRED, BT_CONN, BT_OK, BT_FAIL, BT_OFF };

6

7	struct BtView {

8	  BtPhase phase;

9	  uint32_t code;

10	  uint32_t phaseMs;      // millis() when the phase began

11	  uint32_t offAtMs;      // millis() when Bluetooth switches itself off

12	  char ssid[33];

13	  char why[40];

14	  char ip[16];

15	  uint8_t netCount;

16	  char nets[6][33];

17	  int8_t rssi[6];

18	};

19

20	void bleStart();

21	void bleStop(bool fromRemote);

22	BtView bleView();

23	void bleRetry();

firmware/SmartBin/ble_setup.cpp

1	// ============================================================================

2	// ble_setup.cpp - give the bin a new Wi-Fi network over Bluetooth LE

3	// Flow: WI-FI SETUP + Collect PIN on the screen -> advertise "SmartBin-0x" ->

4	// the phone or laptop connects -> the bin shows a 6-digit passkey -> user

5	// types it -> paired -> app writes SSID and password, then 1 to CTRL -> the

6	// bin tries to join for up to 15 s and reports on STAT. Three wrong passkeys

7	// stop pairing for 60 s. Bluetooth turns itself off after 5 minutes idle.

8	// Characteristics (UUIDs in config.h): SSID w, PASS w, CTRL w (1 join, 2 stop),

9	// STAT read/notify JSON, SCAN read JSON of the 6 strongest networks.

10	// ============================================================================

11	#include "ble_setup.h"

12	#include "state.h"

13	#include "buzzer.h"

14	#include <WiFi.h>

15	#include <NimBLEDevice.h>

16

17	// view is read by the UI task and written by BLE callbacks, so vLock guards it.

18	static SemaphoreHandle_t vLock;

19	static BtView view;

20	static volatile bool running = false, stopReq = false, stopRemote = false, joinReq = false, retryReq = false;

21	static String newSsid, newPass;

22	static uint8_t wrongCodes = 0;

23	static volatile bool codeShown = false;

24	static NimBLEServer* server = nullptr;

25	static NimBLECharacteristic *chStat = nullptr, *chScan = nullptr;

26	static uint16_t connHandle = 0xFFFF;

27	static TaskHandle_t hBle = nullptr;

28

29	// Changes the setup phase and wakes the UI task to redraw the BT screen.

30	static void setPhase(BtPhase p) {

31	  xSemaphoreTake(vLock, portMAX_DELAY);

32	  view.phase = p;

33	  view.phaseMs = millis();

34	  xSemaphoreGive(vLock);

35	  if (hUi) xTaskNotify(hUi, EV_BT, eSetBits);

36	}

37

38	BtView bleView() {

39	  if (!vLock) { BtView v{}; return v; }

40	  xSemaphoreTake(vLock, portMAX_DELAY);

41	  BtView v = view;

42	  xSemaphoreGive(vLock);

43	  return v;

44	}

45

46	static void notifyStat(const String& json) {

47	  if (!chStat) return;

48	  chStat->setValue(json);

49	  chStat->notify();

50	}

51

52	// Connection and pairing callbacks from the NimBLE stack.

53	class ServerCb : public NimBLEServerCallbacks {

54	  void onConnect(NimBLEServer*, NimBLEConnInfo& info) override {

55	    connHandle = info.getConnHandle();

56	    codeShown = false;

57	    xSemaphoreTake(vLock, portMAX_DELAY);

58	    view.offAtMs = millis() + BT_IDLE_OFF_MS;

59	    xSemaphoreGive(vLock);

60	  }

61	  void onDisconnect(NimBLEServer*, NimBLEConnInfo&, int) override {

62	    connHandle = 0xFFFF;

63	    BtPhase p = bleView().phase;

64	    if (p == BT_CODE || p == BT_PAIRED) setPhase(BT_WAIT);

65	    if (p != BT_LOCKOUT && p != BT_OFF && running) NimBLEDevice::startAdvertising();

66	  }

67	  // Called when the phone asks to pair: pick a random 6-digit code, show it

68	  // on the screen and return it to the stack.

69	  uint32_t onPassKeyDisplay() override {

70	    uint32_t code = esp_random() % 1000000;

71	    xSemaphoreTake(vLock, portMAX_DELAY);

72	    view.code = code;

73	    xSemaphoreGive(vLock);

74	    codeShown = true;

75	    setPhase(BT_CODE);

76	    buzz(T_BT_CODE);

77	    return code;

78	  }

79	  void onAuthenticationComplete(NimBLEConnInfo& info) override {

80	    if (info.isEncrypted() && info.isAuthenticated()) {

81	      wrongCodes = 0;

82	      setPhase(BT_PAIRED);

83	      buzz(T_BT_PAIRED);

84	      return;

85	    }

86	    server->disconnect(info.getConnHandle());

87	    // Only a code that was shown and then typed wrong counts. A phone or laptop that

88	    // still remembers an old pairing fails here without any code, and that must not

89	    // use up the 3 tries.

90	    bool counted = codeShown;

91	    codeShown = false;

92	    if (!counted) {

93	      setPhase(BT_WAIT);

94	      return;

95	    }

96	    if (++wrongCodes >= 3) {

97	      NimBLEDevice::stopAdvertising();

98	      setPhase(BT_LOCKOUT);

99	      buzz(T_BT_STOPPED);

100	    } else {

101	      setPhase(BT_WAIT);

102	      buzz(T_BT_WRONG);

103	    }

104	  }

105	};

106

107	// Writes from the app. Every write pushes the auto-off time back 5 minutes.

108	class CharCb : public NimBLECharacteristicCallbacks {

109	  void onWrite(NimBLECharacteristic* c, NimBLEConnInfo&) override {

110	    std::string v = c->getValue();

111	    NimBLEUUID u = c->getUUID();

112	    if (u == NimBLEUUID(BT_SSID_UUID)) newSsid = String(v.c_str());

113	    else if (u == NimBLEUUID(BT_PASS_UUID)) newPass = String(v.c_str());

114	    else if (u == NimBLEUUID(BT_CTRL_UUID) && v.size()) {

115	      uint8_t b = (uint8_t)v[0];

116	      if (b == 1 || b == '1') joinReq = true;

117	      else if (b == 2 || b == '2') { stopRemote = true; stopReq = true; }

118	    }

119	    xSemaphoreTake(vLock, portMAX_DELAY);

120	    view.offAtMs = millis() + BT_IDLE_OFF_MS;

121	    xSemaphoreGive(vLock);

122	  }

123	  // The SCAN value is built fresh on each read from the last Wi-Fi scan.

124	  void onRead(NimBLECharacteristic* c, NimBLEConnInfo&) override {

125	    if (c != chScan) return;

126	    BtView v = bleView();

127	    JsonDocument d;

128	    JsonArray a = d.to<JsonArray>();

129	    for (int i = 0; i < v.netCount; i++) {

130	      JsonObject o = a.add<JsonObject>();

131	      o["ssid"] = v.nets[i];

132	      o["rssi"] = v.rssi[i];

133	    }

134	    String s;

135	    serializeJson(d, s);

136	    c->setValue(s);

137	  }

138	};

139

140	static ServerCb serverCb;

141	static CharCb charCb;

142

143	// Keeps the 6 strongest unique network names from the last async scan,

144	// strongest first.

145	static void collectScan() {

146	  int n = WiFi.scanComplete();

147	  if (n < 0) return;

148	  struct Net { String s; int r; };

149	  Net best[6];

150	  int k = 0;

151	  for (int i = 0; i < n; i++) {

152	    String s = WiFi.SSID(i);

153	    if (!s.length()) continue;

154	    bool dup = false;

155	    for (int j = 0; j < k; j++) if (best[j].s == s) dup = true;

156	    if (dup) continue;

157	    int r = WiFi.RSSI(i);

158	    if (k < 6) best[k++] = {s, r};

159	    else {

160	      int w = 0;

161	      for (int j = 1; j < 6; j++) if (best[j].r < best[w].r) w = j;

162	      if (r > best[w].r) best[w] = {s, r};

163	    }

164	  }

165	  for (int i = 0; i < k; i++)

166	    for (int j = i + 1; j < k; j++)

167	      if (best[j].r > best[i].r) { Net t = best[i]; best[i] = best[j]; best[j] = t; }

168	  xSemaphoreTake(vLock, portMAX_DELAY);

169	  view.netCount = k;

170	  for (int i = 0; i < k; i++) {

171	    strlcpy(view.nets[i], best[i].s.c_str(), sizeof(view.nets[i]));

172	    view.rssi[i] = best[i].r;

173	  }

174	  xSemaphoreGive(vLock);

175	  WiFi.scanDelete();

176	}

177

178	// Tries the new network for 15 s. On success it is saved and the IP is sent

179	// to the app; on failure the bin goes back to its old network. "Wrong

180	// password" and "Network not found" come from the ESP32's own status codes.

181	static void doJoin() {

182	  String ssid = newSsid, pass = newPass;

183	  xSemaphoreTake(vLock, portMAX_DELAY);

184	  strlcpy(view.ssid, ssid.c_str(), sizeof(view.ssid));

185	  xSemaphoreGive(vLock);

186	  setPhase(BT_CONN);

187	  notifyStat("{\"state\":\"CONNECTING\"}");

188	  String oldSsid = net.ssid, oldPass = net.pass;

189	  WiFi.disconnect(false, false);

190	  vTaskDelay(pdMS_TO_TICKS(200));

191	  WiFi.begin(ssid.c_str(), pass.c_str());

192	  uint32_t t0 = millis();

193	  wl_status_t s = WL_IDLE_STATUS;

194	  while (millis() - t0 < 15000) {

195	    s = WiFi.status();

196	    if (s == WL_CONNECTED || s == WL_CONNECT_FAILED || s == WL_NO_SSID_AVAIL) break;

197	    vTaskDelay(pdMS_TO_TICKS(250));

198	  }

199	  if (s == WL_CONNECTED) {

200	    net.ssid = ssid;

201	    net.pass = pass;

202	    saveNet();

203	    String ip = WiFi.localIP().toString();

204	    xSemaphoreTake(vLock, portMAX_DELAY);

205	    strlcpy(view.ip, ip.c_str(), sizeof(view.ip));

206	    view.offAtMs = millis() + BT_IDLE_OFF_MS;

207	    xSemaphoreGive(vLock);

208	    setPhase(BT_OK);

209	    buzz(T_BT_JOINED);

210	    notifyStat("{\"state\":\"CONNECTED\",\"ip\":\"" + ip + "\",\"ssid\":\"" + ssid + "\"}");

211	    return;

212	  }

213	  const char* why = s == WL_NO_SSID_AVAIL ? "Network not found" : s == WL_CONNECT_FAILED ? "Wrong password" : "Could not join";

214	  xSemaphoreTake(vLock, portMAX_DELAY);

215	  strlcpy(view.why, why, sizeof(view.why));

216	  xSemaphoreGive(vLock);

217	  setPhase(BT_FAIL);

218	  buzz(T_BT_JOIN_FAIL);

219	  notifyStat(String("{\"state\":\"FAILED\",\"why\":\"") + why + "\"}");

220	  WiFi.disconnect(false, false);

221	  if (oldSsid.length()) WiFi.begin(oldSsid.c_str(), oldPass.c_str());

222	}

223

224	// Stops advertising and releases the whole BLE stack (deinit(true) frees its

225	// memory). cloudTask resumes once st.btOn is false again.

226	static void shutdown(bool remote, bool automatic) {

227	  NimBLEDevice::stopAdvertising();

228	  NimBLEDevice::deinit(true);

229	  server = nullptr;

230	  chStat = chScan = nullptr;

231	  running = false;

232	  lockState();

233	  st.btOn = false;

234	  unlockState();

235	  publish();

236	  if (remote || automatic) {

237	    setPhase(BT_OFF);

238	    buzz(automatic ? T_BT_AUTO_OFF : T_BT_OFF);

239	  } else {

240	    setPhase(BT_IDLE);

241	  }

242	}

243

244	// Watches for stop, timeout, join and retry requests every 200 ms, refreshes

245	// the scan every 30 s, and deletes itself when Bluetooth turns off.

246	static void bleTask(void*) {

247	  uint32_t lastScan = 0;

248	  for (;;) {

249	    vTaskDelay(pdMS_TO_TICKS(200));

250	    uint32_t now = millis();

251	    if (stopReq) {

252	      bool remote = stopRemote;

253	      stopReq = false;

254	      stopRemote = false;

255	      shutdown(remote, false);

256	      break;

257	    }

258	    BtView v = bleView();

259	    if ((int32_t)(now - v.offAtMs) >= 0 && v.phase != BT_CONN) {

260	      shutdown(false, true);

261	      break;

262	    }

263	    if (joinReq) {

264	      joinReq = false;

265	      doJoin();

266	    }

267	    if (retryReq) {

268	      retryReq = false;

269	      setPhase(connHandle != 0xFFFF ? BT_PAIRED : BT_WAIT);

270	    }

271	    if (v.phase == BT_CODE && now - v.phaseMs >= BT_CODE_MS && connHandle != 0xFFFF) {

272	      server->disconnect(connHandle);

273	      setPhase(BT_WAIT);

274	    }

275	    if (v.phase == BT_LOCKOUT && now - v.phaseMs >= BT_LOCKOUT_MS) {

276	      wrongCodes = 0;

277	      setPhase(BT_WAIT);

278	      NimBLEDevice::startAdvertising();

279	      buzz(T_BT_WAIT);

280	    }

281	    collectScan();

282	    if (now - lastScan > 30000 && v.phase != BT_CONN && WiFi.scanComplete() != WIFI_SCAN_RUNNING) {

283	      lastScan = now;

284	      WiFi.scanNetworks(true);

285	    }

286	  }

287	  hBle = nullptr;

288	  vTaskDelete(nullptr);

289	}

290

291	// Security: bonding, MITM protection and secure connections, with the bin as

292	// a display-only device. Every characteristic needs an encrypted,

293	// authenticated link, so nothing can be written before the passkey step.

294	void bleStart() {

295	  if (!vLock) vLock = xSemaphoreCreateMutex();

296	  if (running) return;

297	  running = true;

298	  wrongCodes = 0;

299	  stopReq = false;

300	  stopRemote = false;

301	  joinReq = false;

302	  retryReq = false;

303	  xSemaphoreTake(vLock, portMAX_DELAY);

304	  memset(&view, 0, sizeof(view));

305	  view.offAtMs = millis() + BT_IDLE_OFF_MS;

306	  xSemaphoreGive(vLock);

307	  setPhase(BT_WAIT);

308	  lockState();

309	  st.btOn = true;

310	  unlockState();

311	  publish();

312

313	  NimBLEDevice::init(BIN_BLE_NAME);

314	  NimBLEDevice::setSecurityAuth(true, true, true);

315	  NimBLEDevice::setSecurityIOCap(BLE_HS_IO_DISPLAY_ONLY);

316	  server = NimBLEDevice::createServer();

317	  server->setCallbacks(&serverCb, false);

318	  NimBLEService* svc = server->createService(BT_SVC_UUID);

319	  const uint32_t W = NIMBLE_PROPERTY::WRITE | NIMBLE_PROPERTY::WRITE_ENC | NIMBLE_PROPERTY::WRITE_AUTHEN;

320	  const uint32_t R = NIMBLE_PROPERTY::READ | NIMBLE_PROPERTY::READ_ENC | NIMBLE_PROPERTY::READ_AUTHEN;

321	  svc->createCharacteristic(BT_SSID_UUID, W)->setCallbacks(&charCb);

322	  svc->createCharacteristic(BT_PASS_UUID, W)->setCallbacks(&charCb);

323	  svc->createCharacteristic(BT_CTRL_UUID, W)->setCallbacks(&charCb);

324	  chStat = svc->createCharacteristic(BT_STAT_UUID, R | NIMBLE_PROPERTY::NOTIFY);

325	  chStat->setValue("{\"state\":\"WAITING\"}");

326	  chScan = svc->createCharacteristic(BT_SCAN_UUID, R);

327	  chScan->setCallbacks(&charCb);

328	  server->start();

329	  NimBLEAdvertising* adv = NimBLEDevice::getAdvertising();

330	  adv->setName(BIN_BLE_NAME);

331	  adv->addServiceUUID(BT_SVC_UUID);

332	  adv->enableScanResponse(true);

333	  adv->start();

334	  WiFi.scanNetworks(true);

335	  buzz(T_BT_WAIT);

336	  xTaskCreatePinnedToCore(bleTask, "ble", 6144, nullptr, 1, &hBle, 0);

337	}

338

339	// Asks bleTask to shut Bluetooth down on its next pass.

340	void bleStop(bool fromRemote) {

341	  if (!running) return;

342	  stopRemote = fromRemote;

343	  stopReq = true;

344	}

345

346	void bleRetry() { retryReq = true; }

firmware/SmartBin/ui.h

1	// ui.h - 2.4-inch ILI9341 touchscreen (240 x 320, portrait) and its task.

2	#pragma once

3	#include <Arduino.h>

4

5	void uiInit();

6	void uiTask(void*);

7	void isrTouch();

8	void uiStartTouchCal();

9	void uiResetTouchCal();

firmware/SmartBin/ui.cpp

1	// ============================================================================

2	// ui.cpp - touchscreen user interface

3	// uiTask (core 1, priority 2) redraws about every 100 ms. Each screen is drawn

4	// by one draw*() function; pickScreen() decides which one is shown.

5	// To avoid flicker, nothing is redrawn unless it changed: every widget passes

6	// a short "signature" string to dirty(), and only a different signature

7	// triggers a redraw. If part of a screen does not update, its signature is

8	// probably missing the value that changed.

9	// Touches are matched against the hit boxes that buttons register while they

10	// are drawn (addHit), so a button that is not drawn cannot be pressed.

11	// ============================================================================

12	#include "ui.h"

13	#include "state.h"

14	#include "buzzer.h"

15	#include "ble_setup.h"

16	#include "sensors.h"

17	#include <SPI.h>

18	#include <WiFi.h>

19	#include <Adafruit_GFX.h>

20	#include <Adafruit_ILI9341.h>

21	#include <XPT2046_Touchscreen.h>

22	#include <Fonts/FreeSans9pt7b.h>

23	#include <Fonts/FreeSansBold9pt7b.h>

24	#include <Fonts/FreeSansBold12pt7b.h>

25	#include <Fonts/FreeSansBold18pt7b.h>

26	#include <Fonts/FreeSansBold24pt7b.h>

27

28	// The display and the touch controller share one SPI bus (separate CS pins).

29	static Adafruit_ILI9341 tft(&SPI, PIN_TFT_DC, PIN_TFT_CS, -1);

30	static XPT2046_Touchscreen touch(PIN_TOUCH_CS);

31

32	// Touch IRQ (GPIO 36, active low). Wakes uiTask so a press is read within

33	// 30 ms instead of waiting for the next 100 ms frame.

34	void IRAM_ATTR isrTouch() {

35	  BaseType_t woke = pdFALSE;

36	  if (hUi) xTaskNotifyFromISR(hUi, EV_TOUCH, eSetBits, &woke);

37	  portYIELD_FROM_ISR(woke);

38	}

39

40	// Converts a web color 0xRRGGBB to the display's 16-bit RGB565.

41	static constexpr uint16_t C(uint32_t rgb) {

42	  return (uint16_t)(((rgb >> 8) & 0xF800) | ((rgb >> 5) & 0x07E0) | ((rgb >> 3) & 0x001F));

43	}

44

45	// Color palette: background, surface, border, text, the five level colors

46	// (green, amber, red, purple, teal), blue, cyan, the text colors used on top

47	// of each, light tints, and dim/rule grays. Colors match the dashboard and app.

48	struct Pal {

49	  uint16_t bg, surf, bd, tx, tx2, g, a, r, p, t, bl, cy, on_g, on_a, on_r, on_p, on_t, on_bl, on_cy, tg, ta, tr, tp, tt, dim, rule;

50	};

51	static const Pal LIGHT = {C(0xF5F0EB), C(0xFFFFFF), C(0x1A1A1A), C(0x1A1A1A), C(0x6B635B), C(0x1E8E3E), C(0xD98A1F),

52	                          C(0xD93025), C(0x7A45B5), C(0x0D8A8A), C(0x1A62C4), C(0x0E7F9E), C(0xFFFFFF), C(0x1A1A1A),

53	                          C(0xFFFFFF), C(0xFFFFFF), C(0xFFFFFF), C(0xFFFFFF), C(0xFFFFFF), C(0xCADCC8), C(0xEFDCC2),

54	                          C(0xEFCAC3), C(0xDCCEE0), C(0xC7DCD8), C(0xB5AFA9), C(0xC6C2BE)};

55	static const Pal DARK = {C(0x2A2520), C(0x3A3530), C(0xE8E0D8), C(0xEFE7DC), C(0xA89E94), C(0x4FBF6E), C(0xE8A94A),

56	                         C(0xFF6B5E), C(0xB98BE8), C(0x3FC7C2), C(0x79AEF5), C(0x5FD0E8), C(0x13100D), C(0x13100D),

57	                         C(0x13100D), C(0x13100D), C(0x13100D), C(0x13100D), C(0x13100D), C(0x324731), C(0x544229),

58	                         C(0x59342E), C(0x493B4C), C(0x2F4944), C(0x6E6862), C(0x5E5852)};

59	static const Pal* P = &LIGHT;

60	static uint8_t darkTheme = 0;

61

62	// Level colors: LG green (OK), LA amber (filling), LR red (full/alarm),

63	// LP purple (locked), LT teal (system).

64	enum Lv : uint8_t { LG, LA, LR, LP, LT };

65	static uint16_t fillOf(Lv k) { const uint16_t v[] = {P->g, P->a, P->r, P->p, P->t}; return v[k]; }

66	static uint16_t onOf(Lv k) { const uint16_t v[] = {P->on_g, P->on_a, P->on_r, P->on_p, P->on_t}; return v[k]; }

67	static uint16_t tintOf(Lv k) { const uint16_t v[] = {P->tg, P->ta, P->tr, P->tp, P->tt}; return v[k]; }

68

69	// Fonts from smallest to largest. CAP[] is each font's cap height in pixels,

70	// used to center text vertically.

71	enum Ft : uint8_t { F_TINY = 0, F_SR, F_S, F_M, F_L, F_XL };

72	static const GFXfont* const FONTS[] = {nullptr, &FreeSans9pt7b, &FreeSansBold9pt7b, &FreeSansBold12pt7b,

73	                                       &FreeSansBold18pt7b, &FreeSansBold24pt7b};

74	static const uint8_t CAP[] = {7, 13, 13, 17, 25, 34};

75

76	static Ft fontFor(int px, bool regular = false) {

77	  if (px <= 12) return F_TINY;

78	  if (px <= 17) return regular ? F_SR : F_S;

79	  if (px <= 24) return F_M;

80	  if (px <= 36) return F_L;

81	  return F_XL;

82	}

83

84	static void useFont(Ft f, uint8_t mag) {

85	  tft.setFont(FONTS[f]);

86	  tft.setTextSize(mag);

87	  tft.setTextWrap(false);

88	}

89

90	// Width of a string in pixels for a given font.

91	static int textW(const char* s, Ft f, uint8_t mag = 1) {

92	  useFont(f, mag);

93	  int16_t x1, y1;

94	  uint16_t w, h;

95	  tft.getTextBounds(s, 0, 50, &x1, &y1, &w, &h);

96	  return f == F_TINY ? (int)strlen(s) * 6 * mag : (int)w + x1;

97	}

98

99	// Draws text in a box w pixels wide; px is the wanted text height. If the

100	// text does not fit it steps down to a smaller font. align: 'l', 'c', 'r'.

101	static void txt(int x, int y, int w, const char* s, int px, uint16_t col, char align = 'l', bool regular = false, uint8_t mag = 1) {

102	  Ft f = fontFor(px, regular);

103	  int tw = textW(s, f, mag);

104	  while (tw > w && f > F_TINY) {

105	    f = (Ft)(f - 1);

106	    if (f == F_SR && !regular) f = F_TINY;

107	    tw = textW(s, f, mag);

108	  }

109	  int cx = align == 'c' ? x + (w - tw) / 2 : align == 'r' ? x + w - tw : x;

110	  useFont(f, mag);

111	  tft.setTextColor(col);

112	  if (f == F_TINY) tft.setCursor(cx, y + (px - 7 * mag) / 2);

113	  else tft.setCursor(cx, y + (px + CAP[f] * mag) / 2);

114	  tft.print(s);

115	}

116

117	// Like txt() but cuts long text and adds "..", for Wi-Fi names.

118	static void txtFit(int x, int y, int w, const char* s, int px, uint16_t col, char align = 'l', bool regular = false) {

119	  char b[40];

120	  strlcpy(b, s, sizeof(b));

121	  Ft f = fontFor(px, regular);

122	  size_t n = strlen(b);

123	  if (textW(b, f) > w) {

124	    while (n > 2) {

125	      b[--n] = 0;

126	      char t[44];

127	      snprintf(t, sizeof(t), "%s..", b);

128	      if (textW(t, f) <= w) { strlcpy(b, t, sizeof(b)); break; }

129	    }

130	  }

131	  txt(x, y, w, b, px, col, align, regular);

132	}

133

134	// Word-wraps centered text over several lines lh pixels apart.

135	static void wrapTxt(int x, int y, int w, const char* s, int px, uint16_t col, int lh) {

136	  char line[64] = "", word[40];

137	  const char* p = s;

138	  int ly = y;

139	  while (*p) {

140	    int n = 0;

141	    while (*p == ' ') p++;

142	    while (*p && *p != ' ' && n < 39) word[n++] = *p++;

143	    word[n] = 0;

144	    if (!n) break;

145	    char test[104];

146	    snprintf(test, sizeof(test), "%s%s%s", line, line[0] ? " " : "", word);

147	    if (textW(test, fontFor(px, true)) > w && line[0]) {

148	      txt(x, ly, w, line, px, col, 'c', true);

149	      ly += lh;

150	      strlcpy(line, word, sizeof(line));

151	    } else {

152	      strlcpy(line, test, sizeof(line));

153	    }

154	  }

155	  if (line[0]) txt(x, ly, w, line, px, col, 'c', true);

156	}

157

158	// --- Drawing helpers in the app's neo-brutalist style: thick dark border

159	// and a hard offset shadow. ---

160	static void frame(int x, int y, int w, int h, uint16_t fill, int border, uint16_t bd) {

161	  tft.fillRect(x, y, w, h, bd);

162	  tft.fillRect(x + border, y + border, w - 2 * border, h - 2 * border, fill);

163	}

164

165	static void box(int x, int y, int w, int h, uint16_t fill, int border = 3, int shadow = 0) {

166	  if (shadow) tft.fillRect(x + shadow, y + shadow, w, h, P->bd);

167	  frame(x, y, w, h, fill, border, P->bd);

168	}

169

170	static void thick(int x0, int y0, int x1, int y1, int t, uint16_t c) {

171	  for (int dx = -(t / 2); dx <= t / 2; dx++)

172	    for (int dy = -(t / 2); dy <= t / 2; dy++) tft.drawLine(x0 + dx, y0 + dy, x1 + dx, y1 + dy, c);

173	}

174

175	static void arc(int cx, int cy, int r, int a0, int a1, int t, uint16_t c) {

176	  for (int a = a0; a <= a1; a += 4) {

177	    float rad = a * DEG_TO_RAD;

178	    tft.fillCircle(cx + cosf(rad) * r, cy + sinf(rad) * r, t / 2, c);

179	  }

180	}

181

182	// Line icons drawn with thick lines on a 24 x 24 grid scaled to size s.

183	enum Icon : uint8_t { I_BIN, I_LIDOPEN, I_SPEAKER, I_LOCK, I_TRI, I_CHECK, I_X, I_BT, I_TRUCK, I_SUN, I_MOON, I_RING, I_WIFI };

184

185	static void icon(Icon k, int x, int y, int s, uint16_t c, uint16_t bgc, int phase = 0) {

186	  float u = s / 24.0f;

187	  int t = max(2, (int)(s / 9));

188	  auto X = [&](float v) { return x + (int)(v * u); };

189	  auto Y = [&](float v) { return y + (int)(v * u); };

190	  switch (k) {

191	    case I_BIN:

192	      thick(X(5), Y(8), X(19), Y(8), t, c);

193	      thick(X(3), Y(8), X(21), Y(8), t, c);

194	      thick(X(5), Y(8), X(6.4), Y(20), t, c);

195	      thick(X(19), Y(8), X(17.6), Y(20), t, c);

196	      thick(X(6.4), Y(20), X(17.6), Y(20), t, c);

197	      thick(X(9), Y(4.5), X(15), Y(4.5), t, c);

198	      break;

199	    case I_LIDOPEN:

200	      thick(X(6), Y(11), X(18), Y(11), t, c);

201	      thick(X(6), Y(11), X(7.2), Y(20), t, c);

202	      thick(X(18), Y(11), X(16.8), Y(20), t, c);

203	      thick(X(7.2), Y(20), X(16.8), Y(20), t, c);

204	      thick(X(3.5), Y(8.5), X(16.5), Y(4.3), t, c);

205	      break;

206	    case I_SPEAKER:

207	      tft.fillTriangle(X(3), Y(12), X(12), Y(5.5), X(12), Y(18.5), c);

208	      tft.fillRect(X(3), Y(9.5), (int)(5 * u), (int)(5 * u) + 1, c);

209	      if (phase != 1) arc(X(13), Y(12), (int)(4 * u), -45, 45, t, c);

210	      if (phase != 2) arc(X(13), Y(12), (int)(8 * u), -50, 50, t, c);

211	      break;

212	    case I_LOCK:

213	      frame(X(4.5), Y(10.5), (int)(15 * u), (int)(10.5 * u), bgc, t, c);

214	      arc(X(12), Y(8.5), (int)(4 * u), 180, 360, t, c);

215	      thick(X(8), Y(8.5), X(8), Y(10.5), t, c);

216	      thick(X(16), Y(8.5), X(16), Y(10.5), t, c);

217	      thick(X(12), Y(14.5), X(12), Y(17.5), t, c);

218	      break;

219	    case I_TRI:

220	      thick(X(12), Y(3.2), X(22), Y(20.5), t, c);

221	      thick(X(22), Y(20.5), X(2), Y(20.5), t, c);

222	      thick(X(2), Y(20.5), X(12), Y(3.2), t, c);

223	      thick(X(12), Y(9.5), X(12), Y(14.5), t + 1, c);

224	      tft.fillCircle(X(12), Y(17.3), t, c);

225	      break;

226	    case I_CHECK:

227	      thick(X(4), Y(12.5), X(9), Y(17.5), t + 1, c);

228	      thick(X(9), Y(17.5), X(20), Y(6.5), t + 1, c);

229	      break;

230	    case I_X:

231	      thick(X(6), Y(6), X(18), Y(18), t + 1, c);

232	      thick(X(18), Y(6), X(6), Y(18), t + 1, c);

233	      break;

234	    case I_BT:

235	      thick(X(7), Y(7.5), X(17), Y(16.5), t, c);

236	      thick(X(17), Y(16.5), X(12), Y(21), t, c);

237	      thick(X(12), Y(21), X(12), Y(3), t, c);

238	      thick(X(12), Y(3), X(17), Y(7.5), t, c);

239	      thick(X(17), Y(7.5), X(7), Y(16.5), t, c);

240	      break;

241	    case I_TRUCK:

242	      frame(X(2), Y(7), (int)(11 * u), (int)(9 * u), bgc, max(2, t - 1), c);

243	      thick(X(13), Y(10), X(18), Y(10), t - 1, c);

244	      thick(X(18), Y(10), X(21.5), Y(13), t - 1, c);

245	      thick(X(21.5), Y(13), X(21.5), Y(16), t - 1, c);

246	      thick(X(13), Y(16), X(21.5), Y(16), t - 1, c);

247	      tft.fillCircle(X(6.5), Y(18.5), (int)(2.2 * u), c);

248	      tft.fillCircle(X(17), Y(18.5), (int)(2.2 * u), c);

249	      break;

250	    case I_SUN:

251	      tft.fillCircle(X(12), Y(12), (int)(4.5 * u), c);

252	      for (int a = 0; a < 360; a += 45) {

253	        float r = a * DEG_TO_RAD;

254	        thick(X(12 + cosf(r) * 7), Y(12 + sinf(r) * 7), X(12 + cosf(r) * 9.5f), Y(12 + sinf(r) * 9.5f), max(1, t - 1), c);

255	      }

256	      break;

257	    case I_MOON:

258	      tft.fillCircle(X(11), Y(12.5), (int)(8.5 * u), c);

259	      tft.fillCircle(X(15.5), Y(8.5), (int)(7 * u), bgc);

260	      break;

261	    case I_RING:

262	      for (int a = 0; a < 360; a += 40) arc(X(12), Y(12), (int)(9 * u), a + phase, a + phase + 22, t + 1, c);

263	      break;

264	    case I_WIFI:

265	      arc(X(12), Y(20), (int)(5 * u), 220, 320, t, c);

266	      arc(X(12), Y(20), (int)(10 * u), 220, 320, t, c);

267	      tft.fillCircle(X(12), Y(20), t, c);

268	      break;

269	  }

270	}

271

272	// Button IDs used by the touch handler (onPress). B_K0..B_K9 are PIN keys.

273	enum BtnId : uint8_t {

274	  B_NONE = 0, B_COLLECT, B_INFO, B_THEME, B_DONE, B_DONE_BACK, B_CAL_CANCEL, B_BACK, B_WIFI_SETUP, B_BT_CANCEL,

275	  B_BT_DONE, B_BT_RETRY, B_PIN_CANCEL, B_K0, B_K1, B_K2, B_K3, B_K4, B_K5, B_K6, B_K7, B_K8, B_K9, B_KDEL, B_KOK

276	};

277	struct Hit { int16_t x, y, w, h; uint8_t id; };

278	static Hit hits[24];

279	static uint8_t hitN = 0;

280	static void addHit(int x, int y, int w, int h, uint8_t id) {

281	  for (uint8_t i = 0; i < hitN; i++)

282	    if (hits[i].id == id && hits[i].x == x && hits[i].y == y) return;

283	  if (hitN < 24) hits[hitN++] = {(int16_t)x, (int16_t)y, (int16_t)w, (int16_t)h, id};

284	}

285

286	// A button: shadowed box, optional icon, label. If the label and icon do not

287	// fit, the icon shrinks and is dropped last.

288	static void btn(int x, int y, int w, int h, const char* label, uint16_t fill, uint16_t on, int px, int ic, int isz, uint8_t id,

289	                bool pressed = false) {

290	  int o = pressed ? 4 : 0;

291	  if (pressed) tft.fillRect(x, y, w + 4, h + 4, P->bg == fill ? P->surf : P->bg);

292	  box(x + o, y + o, w, h, fill, 3, pressed ? 0 : 4);

293	  int lw = textW(label, fontFor(px));

294	  int gap = 5;

295	  while (ic >= 0 && lw + isz + gap > w - 12 && isz > 14) {

296	    isz -= 2;

297	    gap = 3;

298	  }

299	  if (ic >= 0 && lw + isz + gap > w - 12) ic = -1;

300	  int tw = lw + (ic >= 0 ? isz + gap : 0);

301	  int sx = x + o + (w - tw) / 2;

302	  if (ic >= 0) {

303	    icon((Icon)ic, sx, y + o + (h - isz) / 2, isz, on, fill);

304	    sx += isz + gap;

305	  }

306	  txt(sx, y + o + (h - px) / 2, x + o + w - 6 - sx, label, px, on);

307	  if (id) addHit(x, y, w + 4, h + 4, id);

308	}

309

310	static void chip(int x, int y, int w, int h, const char* label, uint16_t fill, uint16_t on, int px = 12, bool sh = false, int border = 2) {

311	  if (sh) tft.fillRect(x + 3, y + 3, w, h, P->bd);

312	  frame(x, y, w, h, fill, border, P->bd);

313	  txt(x + 2, y + (h - px) / 2, w - 4, label, px, on, 'c');

314	}

315

316	// FNV-1a hash of a widget signature.

317	static uint32_t hashOf(const char* s) {

318	  uint32_t h = 2166136261u;

319	  while (*s) { h ^= (uint8_t)*s++; h *= 16777619u; }

320	  return h;

321	}

322

323	// wcache holds the last signature hash of each widget slot (id 0..39).

324	// fullDraw forces every widget to redraw, used after a screen is cleared.

325	static bool fullDraw = true;

326	static uint32_t wcache[40];

327	static bool dirty(uint8_t id, const char* sig) {

328	  uint32_t h = hashOf(sig);

329	  if (fullDraw || wcache[id] != h) { wcache[id] = h; return true; }

330	  return false;

331	}

332

333	// Screens and overlays. Overlays (Info, PIN, Bluetooth) sit above the normal

334	// screens and close by themselves after SCREEN_TIMEOUT_MS without a touch.

335	enum Screen : uint8_t { S_BOOT, S_CAL, S_MAIN, S_URGENT, S_PIN, S_COLLECT, S_DONE, S_TAMPER, S_LOCKED, S_INFO, S_BT, S_TOUCHCAL };

336	enum Overlay : uint8_t { O_NONE, O_INFO, O_PIN, O_BT };

337	static Screen cur = S_BOOT;

338	static Overlay overlay = O_NONE;

339	static uint32_t overlayMs = 0;

340	static bool pinForBt = false;

341	static char pinBuf[5] = "";

342	static uint32_t pinWrongUntil = 0;

343	static uint32_t doneShownAt = 0;

344	static uint32_t lastTouchMs = 0;

345	static uint8_t blink = 0, spin = 0;

346	static uint32_t lastAnim = 0;

347

348	static const char* lidText(bool open) { return open ? "LID: OPEN" : "LID: CLOSED"; }

349

350	// Top bar: bin name, Info and theme buttons, optional BACK button and the lid chip.

351	static void topBar(const BinState& s, bool icons, int lidMode, const char* extraBack = nullptr) {

352	  char sig[48];

353	  snprintf(sig, sizeof(sig), "%d%d%d", icons, lidMode, darkTheme);

354	  if (dirty(0, sig)) {

355	    tft.fillRect(0, 0, 240, 34, P->surf);

356	    tft.fillRect(0, 34, 240, 3, P->bd);

357	    int nw = icons ? 60 : lidMode >= 0 ? 130 : 150;

358	    txt(8, 9, nw, BIN_LABEL, 16, P->tx);

359	    if (icons) {

360	      tft.fillRect(76, 6, 26, 26, P->bd);

361	      frame(74, 4, 26, 26, P->bg, 2, P->bd);

362	      txt(74, 10, 26, "i", 14, P->tx, 'c');

363	      addHit(0, 0, 72, 34, B_INFO);

364	      addHit(72, 0, 30, 34, B_INFO);

365	      tft.fillRect(106, 6, 26, 26, P->bd);

366	      frame(104, 4, 26, 26, P->bg, 2, P->bd);

367	      icon(darkTheme ? I_MOON : I_SUN, 107, 7, 20, P->tx, P->bg);

368	      addHit(102, 0, 34, 34, B_THEME);

369	    }

370	    if (extraBack) {

371	      frame(168, 4, 64, 26, P->bg, 2, P->bd);

372	      txt(168, 11, 64, extraBack, 12, P->tx, 'c');

373	      addHit(160, 0, 80, 34, B_BACK);

374	    }

375	  }

376	  if (lidMode >= 0) {

377	    snprintf(sig, sizeof(sig), "lid%d%d", lidMode, darkTheme);

378	    if (dirty(1, sig)) {

379	      bool open = lidMode == 1;

380	      chip(142, 6, 90, 22, lidText(open), open ? P->a : P->bg, open ? P->on_a : P->tx);

381	    }

382	  }

383	}

384

385	// Status chips under the top bar: WI-FI/OFFLINE, MOTION, SIM, CAL.

386	static void drawChips(const BinState& s, const Sim& sm, uint16_t tint) {

387	  char sig[24];

388	  snprintf(sig, sizeof(sig), "%d%d%d%d%d", s.wifi, s.v.motion, sm.mask != 0, s.calibrating, darkTheme);

389	  if (!dirty(2, sig)) return;

390	  tft.fillRect(0, 40, 240, 24, tint);

391	  int wx;

392	  if (s.wifi) { chip(8, 42, 52, 20, "WI-FI", P->g, P->on_g); wx = 66; }

393	  else { chip(8, 42, 70, 20, "OFFLINE", P->r, P->on_r); wx = 84; }

394	  addHit(4, 38, wx - 4, 28, B_INFO);

395	  if (s.v.motion) chip(wx, 42, 62, 20, "MOTION", P->cy, P->on_cy);

396	  else chip(wx, 42, 62, 20, "MOTION", P->surf, P->dim);

397	  int sx = wx + 68;

398	  if (sm.mask) chip(sx, 42, 40, 20, "SIM", P->p, P->on_p);

399	  else chip(sx, 42, 40, 20, "SIM", P->surf, P->dim);

400	  if (s.calibrating) chip(sx + 46, 42, 40, 20, "CAL", P->t, P->on_t);

401	  else chip(sx + 46, 42, 40, 20, "CAL", P->surf, P->dim);

402	}

403

404	// Vertical fill bar with marks at 50% and at the fill threshold.

405	static void fillBar(int x, int y, int w, int h, float pct, uint16_t col, float th) {

406	  box(x, y, w, h, P->surf, 3, 0);

407	  int inner = h - 6;

408	  int fh = (int)roundf(inner * constrain(pct, 0.0f, 100.0f) / 100.0f);

409	  if (fh > 0) {

410	    tft.fillRect(x + 3, y + 3 + inner - fh, w - 6, fh, col);

411	    tft.fillRect(x + 3, y + 3 + inner - fh, w - 6, min(2, fh), P->bd);

412	  }

413	  for (float t : {50.0f, th}) {

414	    int ty = y + 3 + (int)roundf(inner * (1 - t / 100.0f));

415	    tft.fillRect(x, ty - 1, w, 2, P->bd);

416	  }

417	}

418

419	// One sensor row: label and value. Turns red when over its limit and shows

420	// "--" with a "!" when the sensor is faulty.

421	static void reading(uint8_t id, int y, const char* label, const char* value, bool alarm, bool fault, int dot, uint16_t rowBg) {

422	  char sig[64];

423	  snprintf(sig, sizeof(sig), "%s|%d%d%d%d", value, alarm, fault, dot, darkTheme);

424	  if (!dirty(id, sig)) return;

425	  tft.fillRect(52, y - 2, 188, 22, rowBg);

426	  if (fault) {

427	    txt(56, y + 2, 90, label, 13, P->tx2, 'l', true);

428	    frame(178, y + 1, 12, 15, rowBg, 2, P->tx2);

429	    txt(178, y + 2, 12, "!", 12, P->tx2, 'c');

430	    txt(196, y + 1, 36, "--", 15, P->tx2, 'r');

431	    return;

432	  }

433	  if (alarm) {

434	    tft.fillRect(57, y + 2, 180, 18, P->bd);

435	    frame(54, y - 1, 180, 18, P->r, 2, P->bd);

436	    txt(60, y + 2, 90, label, 13, P->on_r, 'l', true);

437	    txt(120, y + 1, 106, value, 15, P->on_r, 'r');

438	    return;

439	  }

440	  txt(56, y + 2, 90, label, 13, P->tx2, 'l', true);

441	  int vw = textW(value, fontFor(15));

442	  if (dot >= 0) tft.fillRect(232 - vw - 15, y + 5, 9, 9, fillOf((Lv)dot));

443	  txt(110, y + 1, 122, value, 15, P->tx, 'r');

444	}

445

446	// A row with a shrinking progress line (lid closing, buzzer time left).

447	static void timerRow(uint8_t id, int y, int ic, const char* label, float frac, uint16_t col, uint16_t rowBg) {

448	  char sig[48];

449	  snprintf(sig, sizeof(sig), "%s|%d|%d", label, (int)(frac * 50), darkTheme);

450	  if (!dirty(id, sig)) return;

451	  frame(8, y, 224, 20, P->surf, 2, P->bd);

452	  icon((Icon)ic, 11, y + 2, 14, P->tx, P->surf);

453	  txt(29, y + 3, 200, label, 12, P->tx);

454	  tft.fillRect(10, y + 15, 220, 3, P->bg);

455	  tft.fillRect(10, y + 15, (int)(220 * constrain(frac, 0.0f, 1.0f)), 3, col);

456	}

457

458	static void clearRow(uint8_t id, int y, int h, uint16_t bgc) {

459	  if (dirty(id, "none")) tft.fillRect(0, y, 240, h, bgc);

460	}

461

462	// Bottom row: bin status box and the COLLECT button.

463	static void bottomRow(const char* status, uint16_t fill, uint16_t on, int px) {

464	  char sig[40];

465	  snprintf(sig, sizeof(sig), "%s%d", status, darkTheme);

466	  if (!dirty(3, sig)) return;

467	  tft.fillRect(8 + 4, 274 + 4, 112, 36, P->bd);

468	  frame(8, 274, 112, 36, fill, 3, P->bd);

469	  txt(10, 274 + 11, 108, status, px, on, 'c');

470	  btn(128, 270, 104, 44, "COLLECT", P->surf, P->tx, 15, I_TRUCK, 20, B_COLLECT);

471	}

472

473	// "Full in ~x h" text from the fill rate. Hidden above 72 h or when unknown.

474	static void fmtTtf(const BinState& s, const Cal& c, char* out, size_t n) {

475	  out[0] = 0;

476	  if (s.fillRatePerHour <= 0 || s.v.fill >= c.fill_th) return;

477	  float h = (c.fill_th - s.v.fill) / s.fillRatePerHour;

478	  if (h > 72) return;

479	  if (h < 1) snprintf(out, n, "Full in ~%d min", max(1, (int)roundf(h * 60)));

480	  else snprintf(out, n, "Full in ~%.1f h", h);

481	}

482

483	static void lidRows(const BinState& s, const Cal& c, uint32_t now, int y, uint16_t rowBg, uint8_t id) {

484	  char label[40];

485	  if (s.lidOpen && s.v.motion && s.status == ST_OK) {

486	    timerRow(id, y, I_LIDOPEN, "LID HOLDING (motion)", 1, P->cy, rowBg);

487	  } else if (s.lidOpen && s.lidCloseAtMs) {

488	    float left = max<int32_t>(0, (int32_t)(s.lidCloseAtMs - now)) / 1000.0f;

489	    snprintf(label, sizeof(label), "LID CLOSES IN %.1f s", left);

490	    timerRow(id, y, I_LIDOPEN, label, left / max(1, (int)c.pir_hold_s), P->cy, rowBg);

491	  }

492	}

493

494	static bool lidRowActive(const BinState& s) { return s.lidOpen && (s.v.motion || s.lidCloseAtMs) && s.status == ST_OK; }

495	static bool buzzRowActive(const BinState& s, uint32_t now) { return s.buzzerOn || (int32_t)(s.buzzerUntilMs - now) > 0; }

496

497	static void buzzRow(const BinState& s, const Cal& c, uint32_t now, int y, uint16_t rowBg, uint8_t id) {

498	  char label[32];

499	  int32_t left = (int32_t)(s.buzzerUntilMs - now);

500	  if (left > 0) {

501	    snprintf(label, sizeof(label), "BUZZER %.1f s", left / 1000.0f);

502	    timerRow(id, y, I_SPEAKER, label, left / (max(1, (int)c.buzz_s) * 1000.0f), P->r, rowBg);

503	  } else {

504	    timerRow(id, y, I_SPEAKER, "BUZZER ON", 1, P->r, rowBg);

505	  }

506	}

507

508	// --- Main screen: fill level, weight, temperature, humidity, air ---

509	// The background tint follows the level: green under 50%, amber from 50%,

510	// red at the fill threshold.

511	static void drawMain(const BinState& s, const Cal& c, const Sim& sm, uint32_t now) {

512	  bool hcFault = !(s.okHC || (sm.mask & 1));

513	  int fill = (int)roundf(s.v.fill);

514	  Lv lvl = fill >= c.fill_th ? LR : fill >= 50 ? LA : LG;

515	  uint16_t tint = tintOf(lvl);

516	  char sig[64];

517	  snprintf(sig, sizeof(sig), "main%d%d", lvl, darkTheme);

518	  if (dirty(4, sig)) {

519	    fullDraw = true;

520	    tft.fillScreen(tint);

521	  }

522	  topBar(s, true, s.lidOpen ? 1 : 0);

523	  drawChips(s, sm, tint);

524

525	  snprintf(sig, sizeof(sig), "%d|%d|%d|%.0f", fill, hcFault, darkTheme, c.fill_th);

526	  if (dirty(5, sig)) {

527	    tft.fillRect(8, 66, 224, 78, tint);

528	    fillBar(10, 68, 36, 150, hcFault ? 0 : fill, fillOf(lvl), c.fill_th);

529	    txt(56, 68, 100, "FILL LEVEL", 12, P->tx2);

530	    if (hcFault) {

531	      chip(150, 64, 82, 20, "NO READING", P->surf, P->tx2, 11);

532	      txt(56, 82, 176, "--", 56, P->tx);

533	    } else {

534	      chip(172, 64, 60, 20, fill >= c.fill_th ? "FULL" : fill >= 50 ? "FILLING" : "OK", fillOf(lvl), onOf(lvl));

535	      char n[8];

536	      snprintf(n, sizeof(n), "%d", fill);

537	      txt(56, 82, 176, n, 56, P->tx);

538	      int nw = textW(n, F_XL);

539	      txt(56 + nw + 2, 98, 40, "%", 26, P->tx);

540	    }

541	  }

542

543	  char val[24];

544	  bool hx = s.okHX || (sm.mask & 2), bme = s.okBME || (sm.mask & 8), bmeGas = s.okBME || (sm.mask & 4), gasOk = bmeGas && s.v.iaq_acc > 0;

545	  snprintf(val, sizeof(val), "%.2f kg", s.v.weight_g / 1000.0f);

546	  reading(6, 146, "Weight", val, hx && s.v.weight_g >= c.weight_cap, !hx, -1, tint);

547	  snprintf(val, sizeof(val), "%.1f C", s.v.temp);

548	  reading(7, 164, "Temperature", val, bme && s.v.temp >= c.temp_th, !bme, -1, tint);

549	  snprintf(val, sizeof(val), "%d%%", (int)roundf(s.v.humidity));

550	  reading(8, 182, "Humidity", val, bme && s.v.humidity >= c.hum_th, !bme, -1, tint);

551	  float gp = s.v.iaq / max(1.0f, c.gas_th) * 100;

552	  Lv al = gp >= 80 ? LR : gp >= 50 ? LA : LG;

553	  snprintf(val, sizeof(val), "%s %d", gp >= 80 ? "Poor" : gp >= 50 ? "Moderate" : "Good", (int)roundf(s.v.iaq));

554	  // During BME680 warm-up the Air row shows the countdown and the raw gas

555	  // resistance in kOhm instead of an index.

556	  if (bmeGas && s.v.iaq_acc == 0) {

557	    int left = airWarmLeft();

558	    if (left > 0) snprintf(val, sizeof(val), "%d:%02d  %.0fk", left / 60, left % 60, s.v.gas_k);

559	    else if (s.v.gas_k > 0) snprintf(val, sizeof(val), "Starting %.0fk", s.v.gas_k);

560	    else strcpy(val, "No gas reading");

561	    reading(9, 200, "Air (warm)", val, false, false, -1, tint);

562	  }

563	  else reading(9, 200, "Air", val, gasOk && gp >= 100, !gasOk, al, tint);

564

565	  // Up to two timer rows (lid, buzzer); otherwise the "Full in" line.

566	  bool lr = lidRowActive(s), br = buzzRowActive(s, now);

567	  if (lr && br) {

568	    lidRows(s, c, now, 222, tint, 10);

569	    buzzRow(s, c, now, 244, tint, 11);

570	  } else if (lr || br) {

571	    if (lr) lidRows(s, c, now, 224, tint, 10);

572	    else buzzRow(s, c, now, 224, tint, 10);

573	    clearRow(11, 244, 22, tint);

574	  } else {

575	    char t[32];

576	    fmtTtf(s, c, t, sizeof(t));

577	    snprintf(sig, sizeof(sig), "ttf%s%d", hcFault ? "" : t, darkTheme);

578	    if (dirty(10, sig)) {

579	      tft.fillRect(0, 220, 240, 26, tint);

580	      if (!hcFault && t[0]) txt(8, 230, 224, t, 13, P->tx);

581	    }

582	    clearRow(11, 246, 22, tint);

583	  }

584	  if (hcFault) bottomRow("BIN: ?", P->surf, P->tx2, 14);

585	  else bottomRow(fill >= c.fill_th ? "BIN: FULL" : "BIN: OK", fillOf(lvl), onOf(lvl), 14);

586	}

587

588	// --- Urgent screen: shown when fill passes the threshold ---

589	// Blinking border and speaker icon every 500 ms.

590	static void drawUrgent(const BinState& s, const Cal& c, const Sim& sm, uint32_t now) {

591	  char sig[48];

592	  snprintf(sig, sizeof(sig), "urg%d", darkTheme);

593	  if (dirty(4, sig)) {

594	    fullDraw = true;

595	    tft.fillScreen(P->r);

596	  }

597	  topBar(s, true, s.lidOpen ? 1 : 0);

598	  if (fullDraw) {

599	    txt(12, 46, 170, "URGENT", 40, P->on_r);

600	    txt(12, 98, 200, "FILL LEVEL", 12, P->on_r);

601	    txt(12, 196, 216, "Bin is almost full.", 16, P->on_r, 'l', true);

602	    txt(12, 216, 216, "Collect it now!", 20, P->on_r);

603	  }

604	  snprintf(sig, sizeof(sig), "%d%d", (int)roundf(s.v.fill), sm.mask != 0);

605	  if (dirty(5, sig)) {

606	    tft.fillRect(10, 112, 176, 82, P->r);

607	    char n[8];

608	    snprintf(n, sizeof(n), "%d", (int)roundf(s.v.fill));

609	    txt(12, 112, 160, n, 80, P->on_r, 'l', false, 2);

610	    int nw = textW(n, F_XL, 2);

611	    txt(14 + nw, 146, 50, "%", 36, P->on_r);

612	    tft.fillRect(186, 94, 50, 26, P->r);

613	    if (sm.mask) chip(188, 96, 44, 20, "SIM", P->p, P->on_p);

614	  }

615	  snprintf(sig, sizeof(sig), "b%d", blink);

616	  if (dirty(12, sig)) {

617	    uint16_t bc = blink ? P->bd : P->r;

618	    for (int i = 0; i < 5; i++) tft.drawRect(i, 37 + i, 240 - 2 * i, 283 - 2 * i, bc);

619	    tft.fillRect(190, 46, 38, 38, P->r);

620	    if (blink) icon(I_SPEAKER, 190, 46, 38, P->on_r, P->r);

621	  }

622	  bool lr = lidRowActive(s), br = buzzRowActive(s, now);

623	  if (br) buzzRow(s, c, now, 244, P->r, 10);

624	  else if (lr) lidRows(s, c, now, 244, P->r, 10);

625	  else clearRow(10, 242, 24, P->r);

626	  bottomRow("BIN: FULL", P->surf, P->tx, 14);

627	}

628

629	// --- Collect PIN keypad. Also used to unlock WI-FI SETUP. ---

630	// A wrong PIN shows the red banner for 1.6 s.

631	static void drawPin(uint32_t now) {

632	  bool wrong = (int32_t)(pinWrongUntil - now) > 0;

633	  char sig[24];

634	  snprintf(sig, sizeof(sig), "pin%d%d", wrong, darkTheme);

635	  if (dirty(4, sig)) {

636	    fullDraw = true;

637	    tft.fillScreen(P->bg);

638	    tft.fillRect(0, 0, 240, 34, P->surf);

639	    tft.fillRect(0, 34, 240, 3, P->bd);

640	    frame(6, 4, 84, 26, P->r, 2, P->bd);

641	    icon(I_X, 13, 10, 13, P->on_r, P->r);

642	    txt(30, 11, 56, "CANCEL", 12, P->on_r);

643	    addHit(0, 0, 96, 36, B_PIN_CANCEL);

644	    txt(98, 9, 134, "COLLECT PIN", 16, P->tx, 'r');

645	    const char* keys[] = {"1", "2", "3", "4", "5", "6", "7", "8", "9", "<", "0", "OK"};

646	    const uint8_t ids[] = {B_K1, B_K2, B_K3, B_K4, B_K5, B_K6, B_K7, B_K8, B_K9, B_KDEL, B_K0, B_KOK};

647	    for (int i = 0; i < 12; i++) {

648	      int r = i / 3, cc = i % 3;

649	      bool ok = i == 11;

650	      btn(8 + cc * 76, 98 + r * 46, 68, 42, keys[i], ok ? P->g : P->surf, ok ? P->on_g : P->tx, 22, -1, 0, ids[i]);

651	    }

652	    if (wrong) {

653	      frame(8, 288, 224, 26, P->r, 3, P->bd);

654	      txt(8, 294, 224, "WRONG PIN. Try again.", 14, P->on_r, 'c');

655	      for (int i = 0; i < 5; i++) tft.drawRect(i, 37 + i, 240 - 2 * i, 283 - 2 * i, P->r);

656	    } else {

657	      txt(8, 296, 224, "4 digits, then OK", 13, P->tx2, 'c', true);

658	    }

659	  }

660	  int n = wrong ? 4 : strlen(pinBuf);

661	  snprintf(sig, sizeof(sig), "%d", n);

662	  if (dirty(5, sig)) {

663	    for (int i = 0; i < 4; i++) {

664	      box(20 + i * 52, 46, 44, 40, P->surf, 3, 3);

665	      if (i < n) tft.fillCircle(20 + i * 52 + 22, 66, 6, P->tx);

666	    }

667	  }

668	}

669

670	// --- Collecting screen: lid held open until DONE ---

671	static void drawCollect(uint32_t now) {

672	  char sig[24];

673	  snprintf(sig, sizeof(sig), "col%d", darkTheme);

674	  if (dirty(4, sig)) {

675	    fullDraw = true;

676	    tft.fillScreen(P->g);

677	    BinState s = snapshot();

678	    topBar(s, false, 1);

679	    icon(I_LIDOPEN, 88, 84, 64, P->on_g, P->g);

680	    const char* rows[] = {"Lid: open", "Motion: ignored", "Tamper: paused", "Alerts: muted"};

681	    for (int i = 0; i < 4; i++) {

682	      tft.fillRect(28, 172 + i * 22, 9, 9, P->on_g);

683	      txt(44, 164 + i * 22, 180, rows[i], 16, P->on_g, 'l', true);

684	    }

685	    btn(8, 262, 224, 48, "DONE", P->surf, P->tx, 22, I_CHECK, 24, B_DONE);

686	  }

687	  char dots[24];

688	  int d = (now / 300) % 4;

689	  snprintf(dots, sizeof(dots), "COLLECTING%.*s", d, "...");

690	  if (dirty(5, dots)) {

691	    tft.fillRect(0, 44, 240, 34, P->g);

692	    txt(8, 48, 224, dots, 26, P->on_g, 'c');

693	  }

694	}

695

696	// --- Collection complete: goes back to the main screen after 30 s ---

697	static void drawDone(uint32_t now) {

698	  char sig[24];

699	  snprintf(sig, sizeof(sig), "done%d", darkTheme);

700	  if (dirty(4, sig)) {

701	    fullDraw = true;

702	    tft.fillScreen(P->tg);

703	    BinState s = snapshot();

704	    topBar(s, false, 0);

705	    txt(8, 50, 224, "COLLECTION", 22, P->tx, 'c');

706	    txt(8, 76, 224, "COMPLETE", 22, P->tx, 'c');

707	    box(84, 108, 72, 72, P->g, 3, 4);

708	    icon(I_CHECK, 94, 118, 52, P->on_g, P->g);

709	    btn(8, 198, 224, 56, "DONE", P->g, P->on_g, 26, -1, 0, B_DONE_BACK);

710	  }

711	  int left = 30 - (int)((now - doneShownAt) / 1000);

712	  if (left < 0) left = 0;

713	  snprintf(sig, sizeof(sig), "%d", left);

714	  if (dirty(5, sig)) {

715	    char t[48];

716	    snprintf(t, sizeof(t), "Back to the main screen in %d s", left);

717	    tft.fillRect(0, 264, 240, 20, P->tg);

718	    txt(8, 266, 224, t, 13, P->tx, 'c', true);

719	    frame(8, 286, 224, 16, P->surf, 2, P->bd);

720	    tft.fillRect(10, 288, 220 * left / 30, 12, P->g);

721	  }

722	}

723

724	// --- Tamper screen: countdown, then latched until reset from the dashboard ---

725	static void drawTamper(const BinState& s, const Cal& c, uint32_t now) {

726	  bool latched = s.tamperPhase == 2;

727	  char sig[24];

728	  snprintf(sig, sizeof(sig), "tmp%d%d", latched, darkTheme);

729	  if (dirty(4, sig)) {

730	    fullDraw = true;

731	    tft.fillScreen(P->r);

732	    txt(8, 82, 224, latched ? "TAMPER LATCHED" : "TAMPER DETECTED", 22, P->on_r, 'c');

733	    txt(8, 112, 224, "Bin moved or tilted", 15, P->on_r, 'c', true);

734	    if (latched) {

735	      box(16, 160, 208, 84, P->surf, 3, 4);

736	      txt(16, 176, 208, "RESET ON", 22, P->tx, 'c');

737	      txt(16, 204, 208, "DASHBOARD", 22, P->tx, 'c');

738	      txt(8, 252, 224, "Stays on until it is cleared", 13, P->on_r, 'c', true);

739	    } else {

740	      txt(8, 236, 224, "SECONDS, CLEARS IF STABLE", 12, P->on_r, 'c');

741	    }

742	    tft.fillRect(53, 279, 140, 32, P->bd);

743	    frame(50, 276, 140, 32, P->surf, 3, P->bd);

744	    icon(I_SPEAKER, 70, 281, 22, P->tx, P->surf);

745	    txt(98, 284, 90, latched ? "ALARM ON" : "ALARM", 15, P->tx);

746	  }

747	  char t[24];

748	  snprintf(t, sizeof(t), "Tilt: %.0f deg", s.v.tilt);

749	  if (dirty(5, t)) {

750	    tft.fillRect(8, 130, 224, 20, P->r);

751	    txt(8, 132, 224, t, 15, P->on_r, 'c', true);

752	  }

753	  if (!latched) {

754	    int left = (int)TAMPER_COUNTDOWN_S - (int)((now - s.tamperStartMs) / 1000);

755	    if (left < 0) left = 0;

756	    snprintf(t, sizeof(t), "%d", left);

757	    if (dirty(6, t)) {

758	      tft.fillRect(8, 150, 224, 84, P->r);

759	      txt(8, 152, 224, t, 84, P->on_r, 'c', false, 2);

760	    }

761	    float frac = 1.0f - (now - s.tamperStartMs) / (TAMPER_COUNTDOWN_S * 1000.0f);

762	    snprintf(t, sizeof(t), "%d", (int)(frac * 52));

763	    if (dirty(7, t)) {

764	      frame(16, 254, 208, 14, P->surf, 2, P->bd);

765	      tft.fillRect(18, 256, (int)(204 * constrain(frac, 0.0f, 1.0f)), 10, P->bd);

766	    }

767	  }

768	  snprintf(t, sizeof(t), "%d", blink);

769	  if (dirty(12, t)) {

770	    tft.fillRect(88, 16, 64, 58, P->r);

771	    icon(I_TRI, 88 + (blink ? 2 : -2), 14, 60, P->on_r, P->r);

772	    uint16_t bc = blink ? P->bd : P->r;

773	    for (int i = 0; i < 8; i++) tft.drawRect(i, i, 240 - 2 * i, 320 - 2 * i, bc);

774	  }

775	}

776

777	// --- Locked screen: lid stays closed, unlock from the app or dashboard ---

778	static void drawLocked(const BinState& s) {

779	  char sig[24];

780	  snprintf(sig, sizeof(sig), "lock%d", darkTheme);

781	  if (dirty(4, sig)) {

782	    fullDraw = true;

783	    tft.fillScreen(P->p);

784	    icon(I_LOCK, 80, 46, 80, P->on_p, P->p);

785	    txt(8, 134, 224, "LOCKED", 40, P->on_p, 'c');

786	    txt(8, 184, 224, "Lid stays closed.", 15, P->on_p, 'c', true);

787	    txt(8, 204, 224, "Motion is ignored.", 15, P->on_p, 'c', true);

788	    frame(8, 228, 224, 34, P->surf, 3, P->bd);

789	    txt(8, 238, 224, "Unlock on the app or dashboard.", 13, P->tx, 'c');

790	  }

791	  topBar(s, true, 0);

792	  bottomRow("BIN: LOCKED", P->surf, P->tx, 13);

793	}

794

795	// --- Boot screen: progress bar and the result of each sensor check ---

796	// A FAIL row here matches a "faults" entry in /api/status.

797	static void drawBoot(const BinState& s, uint32_t now) {

798	  if (dirty(4, "boot")) {

799	    fullDraw = true;

800	    tft.fillScreen(P->t);

801	    box(16, 18, 208, 66, P->surf, 3, 4);

802	    icon(I_BIN, 28, 30, 38, P->tx, P->surf);

803	    txt(76, 30, 140, "SMART", 20, P->tx);

804	    txt(76, 54, 140, "BIN", 20, P->tx);

805	    frame(16, 100, 208, 18, P->surf, 3, P->bd);

806	  }

807	  tft.fillRect(19, 103, min(202, (int)(now / 20)), 12, P->bd);

808	  const char* names[] = {"HC-SR04", "BME680", "HX711", "BMI160"};

809	  bool ok[] = {s.okHC, s.okBME, s.okHX, s.okIMU};

810	  for (int i = 0; i < 4; i++) {

811	    char sig[24];

812	    bool done = now > 1500u + i * 500u;

813	    snprintf(sig, sizeof(sig), "%d%d%d", i, done, ok[i]);

814	    if (!dirty(5 + i, sig)) continue;

815	    int y = 136 + i * 26;

816	    tft.fillRect(16, y, 210, 22, P->t);

817	    txt(20, y + 3, 120, names[i], 14, P->on_t);

818	    if (!done) chip(168, y, 52, 18, "...", P->surf, P->tx);

819	    else if (ok[i]) chip(168, y, 52, 18, "OK", P->surf, P->tx);

820	    else chip(168, y, 52, 18, "FAIL", P->r, P->on_r);

821	  }

822	  char ip[40];

823	  snprintf(ip, sizeof(ip), "IP: %s", s.wifi ? localIp().c_str() : "joining Wi-Fi...");

824	  if (dirty(9, ip)) {

825	    box(16, 252, 208, 56, P->surf, 3, 4);

826	    txt(26, 261, 190, ip, 16, P->tx);

827	    txt(26, 284, 190, "Firmware v" FW_VERSION, 13, P->tx2, 'l', true);

828	  }

829	}

830

831	// --- Calibration screen: three steps ticked by TARE, d_empty and d_full ---

832	static void drawCal(const BinState& s, const Cal& c, uint32_t now) {

833	  if (dirty(4, darkTheme ? "cal1" : "cal0")) {

834	    fullDraw = true;

835	    tft.fillScreen(P->t);

836	    frame(8, 222, 224, 44, P->surf, 3, P->bd);

837	    txt(8, 228, 224, "Keep the bin empty and", 13, P->tx, 'c');

838	    txt(8, 245, 224, "do not touch it.", 13, P->tx, 'c');

839	    btn(60, 268, 120, 42, "CANCEL", P->r, P->on_r, 15, -1, 0, B_CAL_CANCEL);

840	  }

841	  topBar(s, false, 0);

842	  char sig[40];

843	  snprintf(sig, sizeof(sig), "CALIBRATING%.*s", (int)((now / 300) % 4), "...");

844	  if (dirty(5, sig)) {

845	    tft.fillRect(0, 44, 240, 34, P->t);

846	    txt(8, 48, 224, "CALIBRATING", 26, P->on_t, 'c');

847	  }

848	  snprintf(sig, sizeof(sig), "%d", spin);

849	  if (dirty(6, sig)) {

850	    tft.fillRect(90, 80, 60, 60, P->t);

851	    icon(I_RING, 92, 82, 56, P->on_t, P->t, spin * 15);

852	  }

853	  char rows[3][24];

854	  strcpy(rows[0], "Weight zero");

855	  snprintf(rows[1], 24, "Empty: %.0f cm", c.d_empty);

856	  snprintf(rows[2], 24, "Full: %.0f cm", c.d_full);

857	  for (int i = 0; i < 3; i++) {

858	    bool ok = s.calSteps & (1 << i);

859	    char rs[40];

860	    snprintf(rs, sizeof(rs), "%s%d", rows[i], ok);

861	    if (!dirty(7 + i, rs)) continue;

862	    int y = 150 + i * 24;

863	    tft.fillRect(16, y, 210, 22, P->t);

864	    txt(20, y + 4, 140, rows[i], 15, P->on_t, 'l', true);

865	    chip(168, y, 52, 20, ok ? "OK" : "...", P->surf, P->tx);

866	  }

867	}

868

869	static String uptimeText() {

870	  uint32_t m = millis() / 60000;

871	  char b[24];

872	  if (m < 60) snprintf(b, sizeof(b), "%lu m", (unsigned long)m);

873	  else snprintf(b, sizeof(b), "%lu h %lu m", (unsigned long)(m / 60), (unsigned long)(m % 60));

874	  return b;

875	}

876

877	extern bool cloudFirebaseOn();

878

879	// --- Info screen: IP address, Wi-Fi, signal, cloud, Bluetooth, firmware,

880	// uptime. First place to look when the bin does not show up online. ---

881	static void drawInfo(const BinState& s) {

882	  if (dirty(4, darkTheme ? "info1" : "info0")) {

883	    fullDraw = true;

884	    tft.fillScreen(P->bg);

885	    btn(8, 266, 164, 44, "WI-FI SETUP", P->bl, P->on_bl, 15, I_BT, 20, B_WIFI_SETUP);

886	    box(180, 266, 52, 44, P->surf, 3, 4);

887	    icon(darkTheme ? I_MOON : I_SUN, 194, 275, 24, P->tx, P->surf);

888	    addHit(176, 262, 64, 58, B_THEME);

889	  }

890	  topBar(s, false, -1, "BACK");

891	  String ip = s.wifi ? localIp() : String("Not connected");

892	  if (dirty(5, ip.c_str())) {

893	    box(8, 46, 224, 66, P->surf, 3, 4);

894	    txt(18, 54, 200, "IP ADDRESS", 12, P->tx2);

895	    txt(18, 72, 204, ip.c_str(), 27, P->tx);

896	  }

897	  char rows[6][2][28];

898	  strcpy(rows[0][0], "Wi-Fi");

899	  strlcpy(rows[0][1], s.wifi ? WiFi.SSID().c_str() : net.ssid.c_str(), 28);

900	  strcpy(rows[1][0], "Signal");

901	  if (s.wifi) snprintf(rows[1][1], 28, "%d dBm", s.rssi);

902	  else strcpy(rows[1][1], "--");

903	  strcpy(rows[2][0], "Cloud");

904	  strcpy(rows[2][1], !cloudFirebaseOn() ? "OFF" : s.cloudOk ? "Firebase OK" : "NOT SENDING");

905	  strcpy(rows[3][0], "Bluetooth");

906	  strcpy(rows[3][1], s.btOn ? "ON" : "OFF");

907	  strcpy(rows[4][0], "Firmware");

908	  strcpy(rows[4][1], "v" FW_VERSION);

909	  strcpy(rows[5][0], "Uptime");

910	  strlcpy(rows[5][1], uptimeText().c_str(), 28);

911	  for (int i = 0; i < 6; i++) {

912	    char sig[40];

913	    snprintf(sig, sizeof(sig), "%s", rows[i][1]);

914	    if (!dirty(6 + i, sig)) continue;

915	    int y = 124 + i * 23;

916	    tft.fillRect(8, y, 224, 20, P->bg);

917	    txt(10, y + 3, 90, rows[i][0], 13, P->tx2, 'l', true);

918	    txt(100, y + 2, 130, rows[i][1], 15, P->tx, 'r');

919	    tft.drawFastHLine(8, y + 20, 224, P->rule);

920	  }

921	}

922

923	static void btTop() {

924	  tft.fillRect(0, 0, 240, 34, P->bl);

925	  tft.fillRect(0, 34, 240, 3, P->bd);

926	  txt(8, 9, 170, "WI-FI SETUP", 16, P->on_bl);

927	  chip(190, 8, 42, 18, "BT", P->surf, P->tx);

928	}

929

930	static void mmss(char* b, size_t n, int32_t ms) {

931	  if (ms < 0) ms = 0;

932	  int s = ms / 1000;

933	  snprintf(b, n, "%d:%02d", s / 60, s % 60);

934	}

935

936	// --- Bluetooth Wi-Fi setup screens, one layout per phase (see ble_setup.cpp) ---

937	static void drawBt(uint32_t now) {

938	  BtView v = bleView();

939	  char sig[64];

940	  snprintf(sig, sizeof(sig), "bt%d%d%s%lu%d", v.phase, darkTheme, v.ip, (unsigned long)v.code, v.netCount);

941	  bool cancel = v.phase != BT_OK && v.phase != BT_FAIL && v.phase != BT_OFF;

942	  if (dirty(4, sig)) {

943	    fullDraw = true;

944	    tft.fillScreen(P->bg);

945	    btTop();

946	    if (cancel) btn(8, 266, 224, 44, "CANCEL", P->r, P->on_r, 15, -1, 0, B_BT_CANCEL);

947	    switch (v.phase) {

948	      case BT_IDLE:

949	      case BT_WAIT:

950	        txt(8, 120, 224, BIN_BLE_NAME, 22, P->tx, 'c');

951	        wrapTxt(16, 152, 208, "On the dashboard or app press PAIR A BIN and pick this name.", 13, P->tx2, 17);

952	        break;

953	      case BT_CODE: {

954	        txt(8, 46, 224, "PAIRING CODE", 14, P->tx2, 'c');

955	        box(8, 66, 224, 72, P->surf, 3, 4);

956	        char code[12];

957	        snprintf(code, sizeof(code), "%03lu %03lu", (unsigned long)(v.code / 1000), (unsigned long)(v.code % 1000));

958	        txt(8, 76, 224, code, 46, P->tx, 'c');

959	        wrapTxt(16, 146, 208, "Type it in the pairing window that pops up on the laptop or phone.", 13, P->tx2, 17);

960	        txt(8, 242, 224, "3 wrong codes stop pairing for 60 s.", 12, P->tx2, 'c');

961	        break;

962	      }

963	      case BT_LOCKOUT:

964	        box(8, 48, 224, 130, P->r, 3, 4);

965	        txt(16, 62, 208, "PAIRING STOPPED", 22, P->on_r, 'c');

966	        txt(16, 98, 208, "3 wrong codes", 20, P->on_r, 'c');

967	        wrapTxt(16, 192, 208, "Only someone who can see this screen can pair. The phone that tried was turned away.", 13, P->tx2, 17);

968	        break;

969	      case BT_PAIRED:

970	        box(96, 44, 48, 48, P->g, 3, 4);

971	        icon(I_CHECK, 102, 50, 36, P->on_g, P->g);

972	        txt(8, 98, 224, "PAIRED", 22, P->tx, 'c');

973	        wrapTxt(16, 126, 208, "Pick the network on the dashboard.", 13, P->tx2, 17);

974	        for (int i = 0; i < min<int>(v.netCount, 3); i++) {

975	          int y = 168 + i * 27;

976	          frame(8, y, 224, 23, P->surf, 2, P->bd);

977	          txtFit(14, y + 4, 156, v.nets[i], 13, P->tx);

978	          char r[12];

979	          snprintf(r, sizeof(r), "%d dBm", v.rssi[i]);

980	          txt(172, y + 4, 54, r, 12, P->tx2, 'r');

981	        }

982	        break;

983	      case BT_CONN:

984	        txt(8, 128, 224, "CONNECTING...", 22, P->tx, 'c');

985	        box(24, 166, 192, 34, P->surf, 3, 3);

986	        txtFit(32, 174, 176, v.ssid, 16, P->tx, 'c');

987	        txt(20, 214, 200, "Password received.", 13, P->tx2, 'c', true);

988	        break;

989	      case BT_OK:

990	        box(8, 48, 224, 140, P->g, 3, 4);

991	        txt(16, 58, 208, "CONNECTED", 22, P->on_g, 'c');

992	        txt(16, 88, 208, "IP ADDRESS", 12, P->on_g, 'c');

993	        txt(16, 102, 208, v.ip, 28, P->on_g, 'c');

994	        txtFit(16, 140, 208, v.ssid, 14, P->on_g, 'c', true);

995	        txt(16, 162, 208, "Type it in the dashboard.", 13, P->on_g, 'c', true);

996	        wrapTxt(16, 200, 208, "Bluetooth turns off by itself in 5 minutes.", 13, P->tx2, 17);

997	        btn(8, 266, 224, 44, "DONE", P->surf, P->tx, 18, I_CHECK, 20, B_BT_DONE);

998	        break;

999	      case BT_FAIL:

1000	        box(8, 48, 224, 126, P->r, 3, 4);

1001	        txt(16, 62, 208, "FAILED", 28, P->on_r, 'c');

1002	        txt(16, 102, 208, v.why, 20, P->on_r, 'c');

1003	        txt(16, 132, 208, "Check it and try again.", 14, P->on_r, 'c', true);

1004	        wrapTxt(16, 190, 208, "Other reasons: network not found, or a 5 GHz network.", 13, P->tx2, 17);

1005	        btn(8, 266, 108, 44, "RETRY", P->bl, P->on_bl, 16, -1, 0, B_BT_RETRY);

1006	        btn(124, 266, 108, 44, "CANCEL", P->surf, P->tx, 16, -1, 0, B_BT_CANCEL);

1007	        break;

1008	      case BT_OFF:

1009	        box(84, 60, 72, 72, P->surf, 3, 4);

1010	        icon(I_CHECK, 94, 70, 52, P->tx, P->surf);

1011	        txt(8, 150, 224, "BLUETOOTH OFF", 24, P->tx, 'c');

1012	        txt(20, 184, 200, "Setup finished. Wi-Fi stays on.", 14, P->tx2, 'c', true);

1013	        txt(8, 262, 224, "Back to the main screen", 12, P->tx2, 'c');

1014	        break;

1015	    }

1016	  }

1017	  // Parts of the BT screens that change every second (countdowns, spinner).

1018	  char t[32], mm[12];

1019	  switch (v.phase) {

1020	    case BT_IDLE:

1021	    case BT_WAIT:

1022	      snprintf(sig, sizeof(sig), "%d%d", blink, (int)((v.offAtMs - now) / 1000));

1023	      if (dirty(5, sig)) {

1024	        tft.fillRect(84, 44, 76, 76, P->bg);

1025	        int o = blink ? 0 : 2;

1026	        box(88 + o, 48 + o, 64 - 2 * o, 64 - 2 * o, P->surf, 3, 4);

1027	        icon(I_BT, 96 + o, 56 + o, 48 - 2 * o, P->bl, P->surf);

1028	        mmss(mm, sizeof(mm), (int32_t)(v.offAtMs - now));

1029	        snprintf(t, sizeof(t), "BT OFF IN %s", mm);

1030	        tft.fillRect(20, 220, 204, 36, P->bg);

1031	        box(24, 222, 192, 30, P->surf, 3, 3);

1032	        txt(32, 229, 176, t, 14, P->tx, 'c');

1033	      }

1034	      break;

1035	    case BT_CODE:

1036	      mmss(mm, sizeof(mm), (int32_t)(v.phaseMs + BT_CODE_MS - now));

1037	      if (dirty(5, mm)) {

1038	        snprintf(t, sizeof(t), "NEW CODE IN %s", mm);

1039	        tft.fillRect(20, 202, 204, 34, P->bg);

1040	        box(24, 204, 192, 28, P->surf, 3, 3);

1041	        txt(32, 210, 176, t, 14, P->tx, 'c');

1042	      }

1043	      break;

1044	    case BT_LOCKOUT:

1045	      mmss(mm, sizeof(mm), (int32_t)(v.phaseMs + BT_LOCKOUT_MS - now));

1046	      if (dirty(5, mm)) {

1047	        snprintf(t, sizeof(t), "Try again in %s", mm);

1048	        tft.fillRect(12, 128, 216, 40, P->r);

1049	        txt(8, 134, 224, t, 16, P->on_r, 'c', true);

1050	      }

1051	      break;

1052	    case BT_CONN:

1053	      snprintf(sig, sizeof(sig), "%d", spin);

1054	      if (dirty(5, sig)) {

1055	        tft.fillRect(86, 48, 68, 68, P->bg);

1056	        icon(I_RING, 88, 50, 64, P->bl, P->bg, spin * 15);

1057	      }

1058	      break;

1059	    case BT_OFF: {

1060	      int32_t left = (int32_t)(v.phaseMs + 1500 - now);

1061	      snprintf(sig, sizeof(sig), "%d", max<int32_t>(0, left / 100));

1062	      if (dirty(5, sig)) {

1063	        frame(60, 282, 120, 12, P->surf, 2, P->bd);

1064	        tft.fillRect(62, 284, 116 * max<int32_t>(0, left) / 1500, 8, P->bd);

1065	      }

1066	      if (left <= 0) overlay = O_NONE;

1067	      break;

1068	    }

1069	    default:

1070	      break;

1071	  }

1072	}

1073

1074	// --- Touch calibration ---

1075	// Four crosses at the corners. Each press is averaged over at least 3 samples.

1076	// Started by the TOUCH_CAL command or by holding the screen for 5 s.

1077	static volatile bool tcReq = false, tcResetReq = false;

1078	static bool tcActive = false, tcWaitUp = false;

1079	static uint8_t tcStep = 0;

1080	static uint32_t tcLastMs = 0, tcDoneMs = 0;

1081	static long tcSx = 0, tcSy = 0;

1082	static int tcN = 0;

1083	static int tcRaw[4][2];

1084	static const int TC_X[4] = {20, 219, 20, 219};

1085	static const int TC_Y[4] = {20, 20, 299, 299};

1086

1087	void uiStartTouchCal() {

1088	  tcReq = true;

1089	  if (hUi) xTaskNotify(hUi, EV_STATE, eSetBits);

1090	}

1091

1092	void uiResetTouchCal() {

1093	  tcResetReq = true;

1094	  if (hUi) xTaskNotify(hUi, EV_STATE, eSetBits);

1095	}

1096

1097	// Calibration screen: cross for the current step, or "TOUCH SAVED".

1098	static void drawTouchCal() {

1099	  char sig[16];

1100	  snprintf(sig, sizeof(sig), "tc%d%d", tcStep, darkTheme);

1101	  if (!dirty(4, sig)) return;

1102	  fullDraw = true;

1103	  tft.fillScreen(P->bg);

1104	  if (tcStep < 4) {

1105	    txt(8, 112, 224, "TOUCH CALIBRATION", 16, P->tx, 'c');

1106	    char b[32];

1107	    snprintf(b, sizeof(b), "Press the cross  %d of 4", tcStep + 1);

1108	    txt(8, 142, 224, b, 14, P->tx, 'c', true);

1109	    wrapTxt(20, 170, 200, "Press firmly on the center, then let go. Wait 30 s to cancel.", 13, P->tx2, 17);

1110	    int x = TC_X[tcStep], y = TC_Y[tcStep];

1111	    tft.drawFastHLine(x - 14, y, 29, P->r);

1112	    tft.drawFastVLine(x, y - 14, 29, P->r);

1113	    tft.drawCircle(x, y, 7, P->r);

1114	  } else {

1115	    txt(8, 120, 224, "TOUCH SAVED", 22, P->tx, 'c');

1116	    txt(8, 160, 224, "Buttons should line up now", 14, P->tx2, 'c', true);

1117	  }

1118	}

1119

1120	// Which screen to show, highest priority first: touch calibration, boot,

1121	// calibration, Bluetooth, Info/PIN overlays, collection complete, tamper,

1122	// collecting, locked, urgent, main.

1123	static Screen pickScreen(const BinState& s, const Cal& c, const Sim& sm, uint32_t now) {

1124	  if (tcActive) return S_TOUCHCAL;

1125	  if (s.booting) return S_BOOT;

1126	  if (s.calibrating) return S_CAL;

1127	  if (overlay == O_BT) return S_BT;

1128	  if ((overlay == O_INFO || overlay == O_PIN) && now - overlayMs > SCREEN_TIMEOUT_MS) {

1129	    overlay = O_NONE;

1130	    buzz(T_BACK_MAIN);

1131	  }

1132	  if (overlay == O_INFO) return S_INFO;

1133	  if (overlay == O_PIN) return S_PIN;

1134	  if (doneShownAt && !s.collecting) {

1135	    if (now - doneShownAt < 30000) return S_DONE;

1136	    doneShownAt = 0;

1137	    buzz(T_BACK_MAIN);

1138	  }

1139	  if (s.tamperPhase) return S_TAMPER;

1140	  if (s.collecting) return S_COLLECT;

1141	  if (s.status == ST_LOCKED) return S_LOCKED;

1142	  if ((s.okHC || (sm.mask & 1)) && s.v.fill >= c.fill_th) return S_URGENT;

1143	  return S_MAIN;

1144	}

1145

1146	// Clears the widget cache and hit boxes whenever the screen changes, then

1147	// draws the current screen.

1148	static void render(uint32_t now) {

1149	  BinState s = snapshot();

1150	  Cal c = calSnapshot();

1151	  Sim sm = simSnapshot();

1152	  if (s.doneAtMs && !doneShownAt && now - s.doneAtMs < 2000) doneShownAt = s.doneAtMs;

1153	  Screen want = pickScreen(s, c, sm, now);

1154	  if (want != cur) {

1155	    cur = want;

1156	    memset(wcache, 0, sizeof(wcache));

1157	    hitN = 0;

1158	  }

1159	  fullDraw = false;

1160	  switch (cur) {

1161	    case S_BOOT: drawBoot(s, now); break;

1162	    case S_CAL: drawCal(s, c, now); break;

1163	    case S_MAIN: drawMain(s, c, sm, now); break;

1164	    case S_URGENT: drawUrgent(s, c, sm, now); break;

1165	    case S_PIN: drawPin(now); break;

1166	    case S_COLLECT: drawCollect(now); break;

1167	    case S_DONE: drawDone(now); break;

1168	    case S_TAMPER: drawTamper(s, c, now); break;

1169	    case S_LOCKED: drawLocked(s); break;

1170	    case S_INFO: drawInfo(s); break;

1171	    case S_BT: drawBt(now); break;

1172	    case S_TOUCHCAL: drawTouchCal(); break;

1173	  }

1174	}

1175

1176	static void resetScreen() {

1177	  memset(wcache, 0, sizeof(wcache));

1178	  hitN = 0;

1179	}

1180

1181	static void openOverlay(Overlay o) {

1182	  overlay = o;

1183	  overlayMs = millis();

1184	  resetScreen();

1185	}

1186

1187	// What each button does. PIN digits are collected here; OK compares them with

1188	// the Collect PIN from the calibration settings.

1189	static void onPress(uint8_t id) {

1190	  BinState s = snapshot();

1191	  if (id >= B_K0 && id <= B_K9) {

1192	    if (strlen(pinBuf) < 4) {

1193	      size_t n = strlen(pinBuf);

1194	      pinBuf[n] = '0' + (id - B_K0);

1195	      pinBuf[n + 1] = 0;

1196	    }

1197	    pinWrongUntil = 0;

1198	    buzz(T_PIN_KEY);

1199	    overlayMs = millis();

1200	    return;

1201	  }

1202	  switch (id) {

1203	    case B_COLLECT:

1204	      pinForBt = false;

1205	      pinBuf[0] = 0;

1206	      buzz(T_OPEN);

1207	      openOverlay(O_PIN);

1208	      break;

1209	    case B_INFO:

1210	      buzz(T_OPEN);

1211	      openOverlay(O_INFO);

1212	      break;

1213	    case B_THEME:

1214	      darkTheme = !darkTheme;

1215	      P = darkTheme ? &DARK : &LIGHT;

1216	      saveTheme(darkTheme);

1217	      buzz(darkTheme ? T_THEME_DARK : T_THEME_LIGHT);

1218	      resetScreen();

1219	      break;

1220	    case B_BACK:

1221	    case B_PIN_CANCEL:

1222	      buzz(T_CANCEL);

1223	      pinBuf[0] = 0;

1224	      openOverlay(O_NONE);

1225	      break;

1226	    case B_KDEL: {

1227	      size_t n = strlen(pinBuf);

1228	      if (n) pinBuf[n - 1] = 0;

1229	      buzz(T_PIN_DEL);

1230	      overlayMs = millis();

1231	      break;

1232	    }

1233	    case B_KOK: {

1234	      char want[8];

1235	      snprintf(want, sizeof(want), "%04u", calSnapshot().collect_pin);

1236	      if (strlen(pinBuf) == 4 && !strcmp(pinBuf, want)) {

1237	        pinBuf[0] = 0;

1238	        if (pinForBt) {

1239	          buzz(T_OPEN);

1240	          openOverlay(O_BT);

1241	          bleStart();

1242	        } else {

1243	          buzz(T_PIN_OK);

1244	          openOverlay(O_NONE);

1245	          sendCommand("COLLECT", SRC_SCREEN);

1246	        }

1247	      } else {

1248	        pinBuf[0] = 0;

1249	        pinWrongUntil = millis() + 1600;

1250	        buzz(T_PIN_WRONG);

1251	        overlayMs = millis();

1252	      }

1253	      break;

1254	    }

1255	    case B_DONE:

1256	      sendCommand("COLLECT_DONE", SRC_SCREEN);

1257	      doneShownAt = millis();

1258	      break;

1259	    case B_DONE_BACK:

1260	      doneShownAt = 0;

1261	      buzz(T_BACK_MAIN);

1262	      resetScreen();

1263	      break;

1264	    case B_CAL_CANCEL:

1265	      sendCommand("CALIBRATE_END", SRC_SCREEN);

1266	      break;

1267	    case B_WIFI_SETUP:

1268	      pinForBt = true;

1269	      pinBuf[0] = 0;

1270	      buzz(T_OPEN);

1271	      openOverlay(O_PIN);

1272	      break;

1273	    case B_BT_CANCEL:

1274	      buzz(T_CANCEL);

1275	      bleStop(false);

1276	      openOverlay(O_NONE);

1277	      break;

1278	    case B_BT_DONE:

1279	      buzz(T_KEY);

1280	      openOverlay(O_NONE);

1281	      break;

1282	    case B_BT_RETRY:

1283	      buzz(T_KEY);

1284	      bleRetry();

1285	      resetScreen();

1286	      break;

1287	  }

1288	  (void)s;

1289	}

1290

1291	// Reads the touch controller and maps raw values to screen pixels using the

1292	// saved calibration. Pressure z > 200 counts as a press. Set TOUCH_DEBUG to 1

1293	// in config.h to print raw values when the touch is offset.

1294	static bool readTouch(int& x, int& y, int& rawX, int& rawY) {

1295	  if (!touch.touched()) return false;

1296	  TS_Point p = touch.getPoint();

1297	  rawX = p.x;

1298	  rawY = p.y;

1299	  int rx = p.x, ry = p.y;

1300	  if (touchCal.swap) { rx = p.y; ry = p.x; }

1301	#if TOUCH_DEBUG

1302	  Serial.printf("[touch] raw %d %d z %d\n", rx, ry, p.z);

1303	#endif

1304	  x = constrain(map(rx, touchCal.x0, touchCal.x1, 0, 239), 0, 239);

1305	  y = constrain(map(ry, touchCal.y0, touchCal.y1, 0, 319), 0, 319);

1306	  return p.z > 200;

1307	}

1308

1309	static void tcBegin(uint32_t now) {

1310	  tcActive = true;

1311	  tcWaitUp = true;

1312	  tcStep = 0;

1313	  tcN = 0;

1314	  tcSx = tcSy = 0;

1315	  tcLastMs = now;

1316	  overlay = O_NONE;

1317	  resetScreen();

1318	  buzz(T_CAL_START);

1319	}

1320

1321	static void tcEnd() {

1322	  tcActive = false;

1323	  resetScreen();

1324	}

1325

1326	// Works out the raw value at each screen edge from the four presses. If the

1327	// presses changed more along the other axis, the X and Y wires are swapped.

1328	// Presses less than 800 raw units apart are rejected (probably a missed press).

1329	static void tcFinish(uint32_t now) {

1330	  float dxx = (tcRaw[1][0] + tcRaw[3][0]) / 2.0f - (tcRaw[0][0] + tcRaw[2][0]) / 2.0f;

1331	  float dxy = (tcRaw[1][1] + tcRaw[3][1]) / 2.0f - (tcRaw[0][1] + tcRaw[2][1]) / 2.0f;

1332	  bool sw = fabsf(dxy) > fabsf(dxx);

1333	  float px[4], py[4];

1334	  for (int i = 0; i < 4; i++) {

1335	    px[i] = sw ? tcRaw[i][1] : tcRaw[i][0];

1336	    py[i] = sw ? tcRaw[i][0] : tcRaw[i][1];

1337	  }

1338	  float left = (px[0] + px[2]) / 2, right = (px[1] + px[3]) / 2;

1339	  float top = (py[0] + py[1]) / 2, bottom = (py[2] + py[3]) / 2;

1340	  if (fabsf(right - left) < 800 || fabsf(bottom - top) < 800) {

1341	    Serial.println("[touch] calibration rejected, presses too close together");

1342	    buzz(T_CAL_CANCEL);

1343	    raiseAlert("setup", "warning", "Touch calibration failed", "Presses were too close together. Try again.");

1344	    tcEnd();

1345	    return;

1346	  }

1347	  float kx = (right - left) / (TC_X[1] - TC_X[0]);

1348	  float ky = (bottom - top) / (TC_Y[2] - TC_Y[0]);

1349	  TouchCal t;

1350	  t.swap = sw;

1351	  t.x0 = lroundf(left - kx * TC_X[0]);

1352	  t.x1 = lroundf(right + kx * (239 - TC_X[1]));

1353	  t.y0 = lroundf(top - ky * TC_Y[0]);

1354	  t.y1 = lroundf(bottom + ky * (319 - TC_Y[2]));

1355	  touchCal = t;

1356	  saveTouchCal();

1357	  Serial.printf("[touch] saved swap %d x %d..%d y %d..%d\n", t.swap, t.x0, t.x1, t.y0, t.y1);

1358	  buzz(T_SAVED);

1359	  char meta[72];

1360	  snprintf(meta, sizeof(meta), "%s touchscreen calibrated", BIN_LABEL);

1361	  raiseAlert("setup", "success", "Touch calibrated", meta);

1362	  publish(EV_CAL);

1363	  tcStep = 4;

1364	  tcDoneMs = now;

1365	}

1366

1367	// Touch calibration input: wait for release, collect samples while pressed,

1368	// store the average on release. Gives up after 30 s with no press.

1369	static void tcHandle(bool pressed, int rx, int ry, uint32_t now) {

1370	  if (tcStep >= 4) {

1371	    if (now - tcDoneMs > 2500) tcEnd();

1372	    return;

1373	  }

1374	  if (tcWaitUp) {

1375	    if (!pressed) tcWaitUp = false;

1376	    tcLastMs = now;

1377	    return;

1378	  }

1379	  if (pressed) {

1380	    tcSx += rx;

1381	    tcSy += ry;

1382	    tcN++;

1383	    tcLastMs = now;

1384	    return;

1385	  }

1386	  if (tcN >= 3) {

1387	    tcRaw[tcStep][0] = tcSx / tcN;

1388	    tcRaw[tcStep][1] = tcSy / tcN;

1389	    tcStep++;

1390	    if (tcStep == 4) tcFinish(now);

1391	    else buzz(T_CAL_STEP);

1392	  }

1393	  tcSx = tcSy = 0;

1394	  tcN = 0;

1395	  if (tcActive && tcStep < 4 && now - tcLastMs > 30000) {

1396	    buzz(T_CAL_CANCEL);

1397	    tcEnd();

1398	  }

1399	}

1400

1401	// The touch CS is driven high before the display starts so the touch chip

1402	// stays off the bus. SPI at 40 MHz for the display.

1403	void uiInit() {

1404	  SPI.begin(PIN_SPI_SCK, PIN_SPI_MISO, PIN_SPI_MOSI);

1405	  pinMode(PIN_TOUCH_CS, OUTPUT);

1406	  digitalWrite(PIN_TOUCH_CS, HIGH);

1407	  tft.begin(40000000);

1408	  tft.setRotation(TFT_ROTATION);

1409	  touch.begin(SPI);

1410	  touch.setRotation(TFT_ROTATION);

1411	  darkTheme = loadTheme();

1412	  P = darkTheme ? &DARK : &LIGHT;

1413	  pinMode(PIN_TOUCH_IRQ, INPUT);

1414	  tft.fillScreen(P->t);

1415	}

1416

1417	// Polls touch every 30 ms while pressed, 100 ms otherwise. A press is accepted

1418	// at most every 220 ms (debounce). Holding anywhere for 5 s opens touch

1419	// calibration, a way out when the touch is too far off to hit any button.

1420	void uiTask(void*) {

1421	  bool down = false;

1422	  uint32_t lastDraw = 0, holdSince = 0;

1423	  for (;;) {

1424	    uint32_t ev = 0;

1425	    xTaskNotifyWait(0, UINT32_MAX, &ev, pdMS_TO_TICKS(down ? 30 : 100));

1426	    uint32_t now = millis();

1427	    if (now - lastAnim >= 500) {

1428	      lastAnim = now;

1429	      blink = !blink;

1430	    }

1431	    spin = (now / 120) % 24;

1432

1433	    if (tcReq) {

1434	      tcReq = false;

1435	      tcBegin(now);

1436	    }

1437	    if (tcResetReq) {

1438	      tcResetReq = false;

1439	      resetTouchCal();

1440	      buzz(T_SAVED);

1441	      publish(EV_CAL);

1442	    }

1443	    {

1444	      int x, y, rx = 0, ry = 0;

1445	      bool pressed = readTouch(x, y, rx, ry);

1446	      if (!pressed) holdSince = 0;

1447	      else if (!holdSince) holdSince = now;

1448	      if (tcActive) {

1449	        tcHandle(pressed, rx, ry, now);

1450	        down = pressed;

1451	        holdSince = 0;

1452	      } else if (pressed && holdSince && now - holdSince > 5000) {

1453	        holdSince = 0;

1454	        tcBegin(now);

1455	      } else if (pressed && !down && now - lastTouchMs > 220) {

1456	        down = true;

1457	        lastTouchMs = now;

1458	        // Last hit box first, so a box drawn on top wins.

1459	        for (int i = hitN - 1; i >= 0; i--) {

1460	          const Hit& h = hits[i];

1461	          if (x >= h.x && x < h.x + h.w && y >= h.y && y < h.y + h.h) {

1462	            if (h.id < B_K0 && h.id != B_KOK && h.id != B_KDEL) buzz(T_KEY);

1463	            onPress(h.id);

1464	            break;

1465	          }

1466	        }

1467	      } else if (!pressed) {

1468	        down = false;

1469	      }

1470	    }

1471	    if ((ev & ~EV_TOUCH) || now - lastDraw >= 100) {

1472	      lastDraw = now;

1473	      render(now);

1474	    }

1475	  }

1476	}

firmware/firebase.rules.json

1	{

2	  "rules": {

3	    "sb_users": {

4	      "$uid": {

5	        ".read": "auth != null && auth.uid === $uid"

6	      }

7	    },

8	    "sb": {

9	      ".read": "auth != null && (root.child('sb_users').child(auth.uid).exists() || auth.token.email.matches(/^bin.*@/))",

10	      "bins": {

11	        "$b": {

12	          ".write": "auth != null && auth.token.email.matches(/^bin.*@/)"

13	        }

14	      },

15	      "commands": {

16	        "$b": {

17	          "$c": {

18	            ".write": "auth != null && ((!data.exists() && root.child('sb_users').child(auth.uid).exists()) || (!newData.exists() && auth.token.email.matches(/^bin.*@/)))",

19	            ".validate": "!newData.exists() || (newData.hasChildren(['cmd', 'ts']) && newData.child('cmd').isString())"

20	          }

21	        }

22	      },

23	      "results": {

24	        "$r": {

25	          ".write": "auth != null"

26	        }

27	      },

28	      "alerts": {

29	        "$b": {

30	          ".write": "auth != null && auth.token.email.matches(/^bin.*@/)"

31	        }

32	      },

33	      "watchers": {

34	        ".indexOn": "ts",

35	        "$w": {

36	          ".write": "auth != null && root.child('sb_users').child(auth.uid).exists()"

37	        }

38	      },

39	      "usage": {

40	        ".write": "auth != null && auth.token.email.matches(/^bin.*@/)"

41	      }

42	    }

43	  }

44	}

firmware/tools/embed_dashboard.py

1	#!/usr/bin/env python3

2	"""Packs Web Dashboard/smartbin-dashboard.html into firmware/SmartBin/dashboard_gz.h

3	so each bin serves the dashboard at http://<bin-ip>/ . Run it again after changing the

4	dashboard, then upload the firmware.

5

6	    python firmware/tools/embed_dashboard.py

7	"""

8	import gzip

9	import pathlib

10

11	root = pathlib.Path(__file__).resolve().parents[2]

12	src = root / "Web Dashboard" / "smartbin-dashboard.html"

13	dst = root / "firmware" / "SmartBin" / "dashboard_gz.h"

14

15	data = gzip.compress(src.read_bytes(), compresslevel=9, mtime=0)

16	rows = [", ".join("0x%02x" % b for b in data[i:i + 20]) for i in range(0, len(data), 20)]

17	dst.write_text(

18	    "#pragma once\n// Generated by firmware/tools/embed_dashboard.py from Web Dashboard/smartbin-dashboard.html. Do not edit.\n"

19	    "#include <Arduino.h>\n"

20	    "const size_t DASHBOARD_GZ_LEN = %d;\n"

21	    "const uint8_t DASHBOARD_GZ[] PROGMEM = {\n  %s\n};\n" % (len(data), ",\n  ".join(rows)),

22	    encoding="ascii",

23	)

24	print("%s: %d bytes of HTML -> %d bytes gzipped" % (dst.name, src.stat().st_size, len(data)))